SIDS initial assessment PROFILE

| Category Name | Copper and copper compounds |
| --- | --- |
| CAS No(s) and chemical name | CAS 7440-50-8 (copper powder and massive) CAS 7758-99-8 (copper sulphate pentahydrate) CAS 1317-38-0 (copper oxide) CAS 1317-39-1 (dicopper oxide) CAS 1332-65-6 (dicopper chloride trihydroxide) |
| Chemical Formula(s) | Cu CuSO4.5 H2O CuO Cu2O Cu2Cl(OH)3 |
| SUMMARY CONCLUSIONS OF THE SIAR This document covers the category assessments for copper metal, massive and powder, coated copper flakes and several copper compounds, assessed under the EU Risk Assessment. Substance specific aspects are provided where relevant. The SIAR is based on the EU Risk Assessment of copper and various copper compounds used as general chemicals, under the Existing Substance Regulation, completed in 2008 as an industry initiative. The copper risk assessment report and opinions from EU technical and scientific committees are available from http://echa.europa.eu/copper-voluntary-risk-assessment-reports. Considering the comments from the EU technical and scientific committees and additional information obtained from industry, updates were done for the 2010-2013 REACH registrations and this SIAR. Nevertheless, a limitation of this SIAR is the small number of recent open literature data (beyond 2006). Rationale for copper category approach The category includes a group of commonly used copper substances whose ecotoxicological and systemic human hazard profiles are related to the release of copper-ions. Nanoform copper substances are excluded from this assessment because the biological effects of nanoform metals can differ from the ionic forms. The category includes five substances: soluble copper sulphate pentahydrate and dicopper chloride trihydroxide, less soluble CuO and Cu2O, and three forms of zero valent copper materials (Cu°): copper massive, copper powder and coated copper flakes,. Coated copper flakes are added to the category because human health hazards are available for these coated copper flakes. Where relevant for read-across purposes, information from other copper compounds is included in the assessment. For the environmental endpoints and systemic health endpoints, the copper ion is considered as the reactive functional group within the category. The counter-ions of the copper salts (i.e. oxygen, sulphate, chloride and hydroxide), are due to their ubiquitous presence in environmental media and human fluids not considered to contribute to the environmental nor systemic human toxicity of copper salts. The category implies that the effects assessment is expressed as Cu-ions and that further read-across to other copper compounds, not assessed in this SIAR, is possible on condition that the copper ions are driving the toxicity. Copper is an essential nutrient for humans and non-human organisms and, therefore, low concentrations may lead to deficiency while high concentrations of copper ions may lead to copper toxicity. Once released to the environment, copper ions have more than one oxidation state. The principal ionic forms are cuprous (Cu(I), Cu+) and cupric (Cu(II), Cu2+). The trivalent form (Cu(III), Cu3+) occurs but is relatively unimportant in physical and biological systems. Cu+ is unstable in aqueous media and Cu1+-ions readily transform into Cu2+-ions. Depending on the chemistry of the receiving environment, soluble and/or insoluble copper compounds are formed. Hence Cu1+-ions are, due to their instability, considered as a source of Cu2+ ions for environmental and systemic toxicity. Environment: The assessment aims at defining the basic hazards profile of soluble and sparingly soluble compounds, copper metal (powder and massive forms) and coated copper flakes. Human health: The human health hazard assessment addresses a large variety of effects and different administration routes. In assessing the human health effects of copper and copper compounds, the essentiality and homeostatic mechanisms have to be considered. Acute effects are based on tests with the individual substances. Short term and long term systemic effects are based on tests with the individual soluble substances belonging to the category and/or read-across where such data are not available Physical-chemical properties Physico-chemical properties that are important for the human health and environmental hazard profiles are granulometry and solubility. - Copper metal (Cu°) is insoluble and needs to be transformed to Cu(I) or Cu(II) to become bio-available/bio-accessible. Typical copper powders have a diameter of around 100 µm but small production volumes of fine powders are also reported. The surface area of fine powders (10-50 µm) is 67-107 mm2/mg. The surface area of a massive copper material (sphere of 1 mm) is 0.67 mm2/mg. The melting point of copper metal is 1059 - 1069°C. Its boiling point has not been determined in view of the high melting point. - Copper flakes, coated with aliphatic acids have a particle size of 8 – 11 µm and surface area of 2,900 mm2/mg. Cu is in the form of Cu° and therefore needs to be transformed to Cu(I) or Cu(II) to become bio-available/bio-accessible. The melting point of coated copper flakes is 1057 - 1058°C. Its boiling point has not been determined in view of the high melting point. - Copper sulphate pentahydrate is a blue crystalline powder. Its water solubility at 25°C is 220 g/L. Copper sulphate pentahydrate decomposes at 110°C without melting or boiling. - Dicopper oxide is an orange-red powder. The water solubility at 20°C is ≥ 28.6 g/L at pH 4.0, 6.39 x 10-4 g/L at pH 6.5 - 6.6, and < 5.39 x 10-4 g/L at pH 9.8. The melting point of dicopper oxide is in excess of 400°C (the maximum temperature tested). Its boiling point has not been determined in view of the high melting point. - Copper oxide is a dark grey powder. The water solubility at 20°C is > 0.23 g/L at pH 5.1 - 5.5, 3.94 x 10-4 g/L at pH 6 and < 1.0 x 10-5 g/L at pH 9. The melting point of copper oxide is 1326°C. Its boiling point has not been determined in view of the high melting point. - Dicopper chloride trihydroxide is a light green powder. The water solubility at 20°C is > 101 g/L at pH 3.1, 1.19 x 10-3 g/L at pH 6.5 and ≤ 5.25x10-4 g/L at pH 10. Dicopper chloride trihydroxide decomposes from 240°C without melting or boiling. Remark 1: The octanol-water partition coefficient (log Kow) is not relevant (the mechanisms of absorption of Cu2+ into organic matter and living cells are different from those traditionally attributed to carbon-based substances and the parameter therefore has little relevance to ionic copper). pKa is also not considered a relevant parameter and is thus not mentioned above. Remark 2: Data on vapour pressure are only relevant for coated copper flakes (7.5e-9 Pa (20°C), they are negligible for Cu-compounds and Cu-metal. Vapour pressure is therefore considered as not relevant to the category members Essentiality Copper is an essential micronutrient, needed for optimal growth and development of micro-organisms, plants, animals and humans. Human Health Toxicokinetics (absorption, metabolism, distribution and elimination) Introduction The toxicokinetics of essential elements such as copper are regulated to a large degree by homeostatic mechanisms. Homeostasis can be described as the maintenance of a constant internal environment in response to changes in internal and external environments. Homeostatic maintenance requires the tightly coordinated control of copper uptake, distribution and efflux in cells and the organism as a whole. As a result of the presence of a homeostatic mechanism for copper, rat and human metabolism of copper are very similar and are therefore discussed together in the following sections. Essentiality Copper is an essential metal present in human body tissues and fluids at concentrations of parts per million or parts per billion. In common with other trace metals, copper has a number of physiological roles that may be classified as regulatory, structural and/or protective. In the regulatory role they are an essential part of metalloenzymes, acting either as electron donors or acceptors at the active site, or by shaping the enzyme to the configuration necessary for its activity. The structural functions of trace metals may be in, for example, membrane integrity or bone structure, and the protective function may involve antioxidant defence or the immune system. Copper is involved in the reactions and functions of many enzymes, including angiogenesis, neurohormone release, oxygen transport and regulation of genetic expression. Copper is an allosteric component of several enzymes that have oxidation and reduction activity, functioning as an electron transfer intermediate in redox reactions. Absorption Oral A large quantity of oral absorption data are available for animals, specifically rats, and humans. These data enable an estimation of true absorption at the relatively high copper intakes used in toxicity studies. True absorption was determined as there is a large quantity of data available on the absorption of copper in animals and humans, predominantly relating to oral exposure. In these studies, quantitative data on the absorption of copper have been based on faecal monitoring, as faecal excretion is the main excretory route for copper. In several of these studies, the amount absorbed has been determined as the difference between oral intake and faecal excretion. This absorption value represents a measure of apparent absorption only, as faecal copper does not distinguish between unabsorbed copper and endogenous copper losses. Endogenous copper losses may occur from (1) biliary excretion of systemically-absorbed copper that mixes with the endogenous pool and is subsequently excreted, and (2) the fraction absorbed by intestinal mucosa and subsequently eliminated into the GI tract as cells are sloughed off (i.e. without systemic absorption). Apparent absorption thus represents a somewhat crude measure of copper absorption. In order to measure true absorption, which provides a more accurate measure of copper absorption following oral exposure, the percentage of copper intake recovered in the faeces was corrected for endogenous copper losses. Based on these absorption data, an absorption factor of 25% is taken to be the best estimate of true absorption in rats at the high copper intakes. The most reliable human data currently available on copper absorption following oral exposure come from volunteer studies. Based on the available true absorption data, oral absorption rates in humans have been derived.The available data have been fitted to two functions giving a continuous distribution with mostly similar results: Equation 1: oral absorption% = -15.0 ln(x) + 63.2777 Equation 2: oral absorption% =72.287 e-0.1167x x= copper intake (mg/day) For a given dose in the GIT, absorption in humans is calculated based on the mean result for these two functions. In humans, this method of calculation is applied to the sum of the oral intake and copper arising from inhalation exposure and subsequently translocated to the GIT. The minimum oral absorption is set to 20%. Oral absorption data for humans and rats show qualitative and quantitative similarities between the two species. In both species, absorption of copper over the range of intakes studied is inversely related to copper intake, illustrating the important role of absorption in copper homeostasis. In both species, true absorption of copper from diets containing what are considered as adequate levels of copper (1-10 mg/day in humans; 0.3-0.6 mg/kg bw/day in animals) is in the 30-60% range. The above oral absorption data, and corresponding functions, are based on copper sulphate. Assuming that orally-administered copper will occur in the GIT, at least in part, in the ionic form and therefore be available for absorption, and in view of the solubility of copper sulphate, it is considered appropriate to adopt a conservative approach and to use the oral absorption data for copper sulphate for other less soluble copper species. Dermal absorption and penetration In the two reviewed studies, the copper compounds were applied in an aqueous medium (suspension). There is uncertainty about the applicability of these absorption data to exposures of dry copper compounds as encountered in occupational exposure scenarios. However, in view of the limitations of the studies on which this dermal absorption factor is based (absence of mass balance data and large standard deviations); the value of 0.3% is considered to represent the best estimate based on data currently available. Given the available studies provide no consistent evidence that dermal absorption is greater for soluble than for insoluble copper substances, a dermal absorption factor of 0.3% is also proposed for both soluble and insoluble copper substances. For dry exposure scenarios, a 10-fold lower dermal absorption value is proposed (0.03%), consistent with the approach used in the OECD Cooperative Chemicals Assessment of Zinc (and EU risk assessment). Distribution On entering interstitial fluid and blood plasma, absorbed copper initially becomes bound to two proteins, albumin and transcuprein. Most of the copper bound to albumin and transcuprein is rapidly transported via portal blood to the liver. Once in the liver, copper is incorporated into ceruloplasmin, which is subsequently released into the systemic circulation for delivery to other tissues. Excretion Quantitative data on excretion were reported in a bioequivalence study. The fate of excess copper was examined in bile-cannulated male Sprague Dawley rats (five per group) following oral administration of a single dose of copper (nominal dose 20 mg Cu/kg bw; actual dose 22-24 mg Cu/kg bw). Six inorganic copper salts were investigated including three category members. Copper levels in excreta during the 24-h period after dosing were as follows: bile 1.54-2.48% of dose; urine 0.20-0.39% of dose; faeces 64-76% of dose (although it is noted that faecal copper will also comprise some absorbed copper). Values were found to be similar for all six substances tested. The results showed faecal excretion to be the main route of elimination for orally-administered copper, with urinary excretion as a relatively minor route. Comparative bioavailability In mammalian toxicity, it is also considered that the most toxic form of any copper salt is the Cu2+ ion. This can be shown through the comparison of the most soluble (e.g. copper sulphate pentahydrate, copper nitrate) and relatively insoluble copper salts, where the solubility, bioavailability and hence toxicity of these salts can vary – with copper sulphate pentahydrate representing the worst-case scenario. As all suitable short- to long-term available animal copper toxicity studies are derived from oral administration, the use of copper sulphate pentahydrate data would represent a worst case scenario for the determination of the effect of relatively insoluble copper compounds in mammalian toxicity. In addition, the use of copper sulphate pentahydrate data would minimise the number of animal studies. In vivo bioavailability For the oral exposure route, in a series of bioavailability studies, conducted by several authors the bioavailability of copper sulphate to other relatively insoluble copper salts including copper oxide was compared. Although the species tested are not usual species used in regulatory guidelines, the results are consistent when evaluating a number of studies and appear to be reproducible. In addition, the studies have measured copper levels in the most important organ and body fluid in determining copper absorption from the gastro-intestinal tract, namely the liver and bile. Relative bioavailability of supplemental copper sources Source of copper; Species. Poultry; Swine; Cattle. Copper sulphate; 100; 100; 100. Copper oxide; 0 (3); 30 (4); 15 (2). Average numbers rounded to the nearest ‘5’ and expressed relative to response obtained with copper sulphate. Number of studies or samples involved indicated within parenthesis. The low bioavailability of copper in copper oxide, relative to that of copper in the sulphate salt, was also demonstrated in the rat following administration at adequate dietary levels. In vitro bioavailability Several studies assessed the release/dissolution of metal ions from metal bearing materials (minerals, soils, substances) in simulated biological fluids. The release/dissolution of copper ions from copper materials and copper compounds was assessed from in vitro tests using biological fluids simulating oral exposure. The in vitro tests follow the ASTM D 5517-07 protocol, using HCl 0.07N (pH 1.5) as a gastric mimetic fluid. The copper materials tested include: copper wires (representing massive copper materials), copper powder (130 µm median diameter), biocidal and non-biocidal coated copper flakes (ca 8.5 µm), copper oxide, cuprous chloride and dicopper sulphide. Loading rates between 100 mg/L and 2 g/L were assessed. The results are expressed as % copper dissolved at the end of the bioelution test. Copper, in the form of Cu°, is insoluble and needs to be transformed to Cu+ or Cu 2+ to demonstrate solubility. Such transformation/dissolution takes place at the surface of the copper particles and therefore, for the copper massive, copper powder and coated copper flakes, the influence of surface area on bioaccessibility was also evaluated. The results are summarized in the Table below. All copper present in CuSO4 was solubilised (99.95%) in the gastric fluid while “massive” copper materials, tested as wires at different mass loadings (59 to 478 mg/L) and surface loadings (67 – 516 mm2/L) consistently showed low solubility (0.1%). Relative bio-accessibility of copper and copper compounds, assessed from the recovery of copper after bioelution tests in gastric fluids (pH 1.5, 2 hours) in accordance to ASTM D 5517-07. Material Tested; Composition; Bioelution recovery (as % of Cu content). Cu massive; >99.9% Cu; 0.096 – 0.105. Cu powder; 99.7% Cu, 0.3% Cu2O; 1.1 – (7.3*). Dicopper sulphide; 79.9% Cu; 3.3. Cu flake – biocidal product; 93.7% Cu, 2.6% Cu2O, 3.89% LOI**; 42 – 71. Cu flake – non-biocidal product; 96.3% Cu, 1% Cu2O, 2.8% LOI**; 44 – 60. CuCl; 63.78% Cu; 77 – 94. CuO; 80% Cu; 68-84%. CuSO4; 25.45% Cu; 100. *The results at the higher loading rate show unacceptably high variability (CV of 66%), possibly related to abrasion of the particles during the test. The results of this test are therefore not considered as reliable. ** Loss if ignition, as a measure of the organic content (coating by aliphatic acid). For the Cu° materials, the relation between the release of Cu-ions and the surface area exposed is well described by a power function: Bioaccessible copper ions = 1.94 surface loading (mm2/L) 0.84 In conclusion, the in vitro gastric bioelution tests with various copper-bearing materials demonstrate important differences in bio-accessibility in gastric fluids: CuS04.5H20 (100%) > CuCl > CuO > coated copper flakes > Cu2S > copper powder > copper massive (0.1%). The data also show that bio-accessibility of Cu° is related to surface area, described by the power function. Measured and modelled (from the power function) bio-soluble copper concentrations released from 200 mg copper powders/L are respectively 1.1% and 2.8% for a typical copper powder (0.024 m2/g) and reasonable worst case 10 µm powder (of 0.067 m2/g). Using the coated copper flakes as reference substance (with average bio-accessibility of 57%), the relative bio-accessibility of a typical copper powder (0.024 m2/g) and reasonable worst case 10µm powder (of 0.067 m²/g) are therefore respectively 1.9% and 4.9%. These data can be used to predict the acute oral toxicity of copper massive and copper powder using available toxicity studies on other forms of copper and copper compounds and this is presented below in the acute toxicity section. Acute toxicity Available key study data for acute toxicity: Key studies for acute toxicity by inhalation, dermal and oral exposure route Route of administration / endpoint / test guideline; Test substance; Clinical effects; Endpoint value. Inhalation (US 173.132). LC50, rat (m/f) – nose only; Dicopper chloride trihydroxide; >11.4 mg/L air (m/f). Inhalation (OECD 403). LC50, rat (m/f) – nose only; Dicopper chloride trihydroxide; 2.83 mg/L air (m) > 2.77 mg/L air (f) 4.74 mg/L air (m/f). LC50, rat (m/f); Dicopper oxide; Lung abnormalities, wet fur, staining, hunched posture, piloerection, ptosis, altered respiration.; 2.92 mg/L air (m) 3.69 mg/L air (f) 3.34 mg/L air (m/f). LC50, rat (m/f); Dicopper oxide; Respiratory depression, discolored lungs.; 5.36 mg/L air (m/f). LC50, rat (m/f); Dicopper oxide; Apathy, sedation, difficult respiration, squat position, reduced reflexes, tremors, disturbed coordination.; > 30 mg/L air (m/f). LC50, rat (m/f); Dicopper oxide; > 5 mg/L air (m/f). LC50, rat (m/f); Dicopper oxide; Respiratory reduction, enlarged lungs, subdued, hunched posture, piloerection, hypothermia, ataxia, staining of the fur, discoloration peri-anal region, slightly emaciated and unkempt condition. reduced weight gain.; ca. 5 mg/L air (m/f). Inhalation (OECD 436). LC50, rat (m/f); Coated copper flakes; Ataxia, tremor, dyspnoe, reduced motility, reduced body weight gain. Grey-stained discolored lungs.; > 5.11 mg/L air (nose only-dry aerosol). LC50, rat (m/f); Coated copper flakes; Decreased respiratory rate, laboured respiration, noisy respiration, hunched posture, pilo-erection; Males 0.733 mg/L Female 1.67 mg/L. Dermal (OECD 402). LD50, dermal, rat (m/f); Coated copper flakes; > 2,000 mg/kg bw. LD50, dermal, rat (m/f); Copper oxide; None; > 2,000 mg/kg bw. LD50, dermal, rat (m/f); Copper sulphate pentahydrate pentahydrate; > 2,000 mg/kg bw. LD50, dermal, rat (m/f); Dicopper chloride trihydroxide; > 2,000 mg/kg bw. LD50, dermal, rat (m/f); Dicopper oxide; None; > 2,000 mg/kg bw. LD50, dermal, rabbit (m/f); Dicopper chloride trihydroxide; > 2,000 mg/kg bw. Dermal (EPA OPP 81-2) equivalent to OECD 402. LD50, dermal, rabbit (m/f); Dicopper oxide; Reddened skin, weight gain, liver lesion; > 2,000 mg/kg bw. LD50, dermal, rabbit (m/f); Dicopper chloride trihydroxide; > 2,000 mg/kg bw. Oral (EPA OPP 81-1). LD50, oral, rat (m/f); Dicopper chloride trihydroxide; 1,200 mg/kg bw (m) 950 mg/kg bw (f). Oral (OECD 423). LD50, oral, rat (m); Copper oxide; > 2,000 mg/kg bw. LD50, oral, rat (m/f); Coated copper flakes; 300-500 mg/kg bw. Oral (OECD 401). LD50, oral, mice (m/f); Dicopper chloride trihydroxide; 299 mg/kg bw. LD50, oral, rat (m/f); Copper sulphate pentahydrate; 481 mg/kg bw. LD50, oral, rat (m/f); Copper sulphate pentahydrate; Lethargy, prostate posture, green coloured diarrhea, voiding few faeces and moribundity; 482 mg/kg bw. LD50, oral, rat (m/f); Dicopper oxide; Piloerection, hunched posture, lethargy, decreased respiratory rate and diarrhea.; 1,625 mg/kg bw (m) 928-2,000 mg/kg dw (f) 1,340 mg/kg bw (m/f). LD50, oral, rat (m/f); Dicopper chloride trihydroxide; 1,796 mg/kg bw (m) 2,006 mg/kg bw (f) 1,862 mg/kg bw (m/f). LD50, oral, rat (m/f); Dicopper chloride trihydroxide; 1,083 mg/kg bw (m) 1,854 mg/kg bw (f) 1,398 mg/kg bw (m/f). Read across and bridging for acute oral toxicity For copper massive and uncoated copper powder, the acute oral toxicity can be predicted using in vitro studies and available toxicity studies on other forms of copper and copper compounds. In vivo acute oral toxicity tests are available for several copper bearing materials. The results of the appropriate oral toxicity studies are summarised below. These include data on 2 substances, dicopper chloride and dicopper sulphide, which are not included in this OECD CoCam review programme. - The acute oral effects - LD50 - observed for coated copper flakes are between 300 and 500 mg/kg. - The acute oral effects - LD50 - observed for copper sulphate pentahydrate: 481 mg/kg. - The acute oral lethal effects - LD50 - observed for copper chloride: 336 mg/kg. - The acute oral lethal effects - LD50 - observed for copper oxide is >2000 mg/kg - The acute oral lethal effects - LD50 - observed for dicopper sulphide is >2000 mg/kg The acute oral data indicate the importance of bio-availability/bio-accessibility: in vivo exposure to highly bio-accessible copper compounds (coated copper flakes, copper sulphate pentahydrate and copper chloride) results in a higher acute toxicity than in vivo exposure to less bioaccessible/bioavailable copper oxide and dicopper sulphide. As read-across approach, the measured LD50 values of the all source materials (coated flakes, CuS04.5H2O and CuCl) were expressed as mg bio-accessible Cu/kg bw and combined with the % bio-accessible Cu released from the various copper bearing materials to predict the LD50 values. The results are given below. Oral LD50 values for the various copper materials Cu Material; Copper content %; Bio-accessible/ bio-availability Cu %(1); LD50 as bio-accessible Cu (mg Cu/kg bw) (2); Measured LD50 LD50 (mg substance/kg bw); Calculated LD50 LD50 (mg substance/kg bw) (3). Coated flakes (2.9m2/g); 99.7; 57; 227; 400 (mid value of 300 -500 mg/kg); 215-401. CuSO4.5H20; 25.4; 25; 122; 481; 480-894. CuCl; 63.78; 51; 173; 336; 232-442. CuO*; 80; 15-76; >2000; 160-1513. Cu2S; 78.9; 2.7; >2000; 4685-8718. Copper powder (0.024m2/g); 100; 1.1; 11091-20636. 10 µm copper powder (0.067m2/g); 100; 2.8; 4357-8107. (1) Obtained from bioelution test or in vivo bioavailability test (2) Measured LD50 / (1) (3) Min-Max range (2) / (1) The above Table shows comparable bioavailable/bioaccessible LD50 values for the 3 substances with bound LD50 data (122 to 227 mg bio-available mg Cu/kg bw). The calculated LD50 values for these materials (copper coated flakes, CuSO4.5H20 and CuCl) correspond to the measured hazard profile of these materials (LD50 < 2,000 mg/kg and > 200 mg/kg bw). Correct predictions are also observed for Cu2S (i.e. LD50 > 2,000 mg/kg). For CuO, the predicted LD50 values are below the observed LD50 and therefore, calculations are conservative. The data therefore support the concept that bio-available copper ions are responsible for the observed acute toxicity profiles of CuSO4.5H20, CuCl, Coated copper flakes CuO and Cu2S and that in vitro bioaccessibility data can be used as read-across parameter. Therefore, following the read-across approach, the in vivo toxicity of coated copper flakes and copper compounds were combined with the relative bio-accessibility of copper powders and massive to derive the classification of copper powders/massive. From the assessment it was concluded that copper powder (typical powder and worst case 10µm powder) and therefore also copper massive follow the same hazard profile as CuO and Cu2S (LD50 > 2,000 mg/kg) : they do not merit acute oral classification. Consideration of available acute oral toxicity data leads to the conclusion that coated copper flakes have the same hazard as the soluble copper compounds (CuSO4.5H20 and CuCl). CuO and Cu2S and copper powder/massive do not present a hazard by the oral route. Conclusion on the acute oral, dermal and inhalation routes - Oral: the release of copper ions drive the acute oral toxicity. Dicopper oxide, copper sulphate pentahydrate, dicopper chloride hydroxide and coated copper flakes present a hazard by the oral route. Copper oxide, copper powder and copper massive are not hazardous by the oral route. - Dermal: all tested substances within the category are not hazardous by the dermal route. Considering the lower solubility and bioaccessability of copper powders/massives compared to the tested copper substances, copper powders/massive forms do not present a hazard by the dermal route. - Inhalation: Dicopper oxide, dicopper chloride hydroxide and copper flakes are considered hazardous by the inhalation route. Copper sulphate pentahydrate, copper oxide and copper powder/massive are not hazardous by inhalation route Skin, eye and respiratory irritation Available key study data for irritation (skin, eye and respiratory tract): Test substance; Guideline; Result; Symptoms. Dicopper oxide; OECD 404, rabbit; Not skin irritant; Very slight erythema and very slight oedema observed in the abraded skin. OECD 405, rabbit; Eye irritant; Ocular irritation. OECD 405, rabbit; Not eye irritant; /. EPA OPP 81.4, rabbit; Not eye irritant; Conjunctival redness and chemosis. Copper oxide; OECD 404, rabbit; Not skin irritant; /. OECD 405, rabbit; Not eye irritant; Scattered or diffuse corneal opacity, iridial inflammation. Dicopper chloride trihydroxide; OECD 404, rabbit; Not skin irritant; /. EPA OPP 81.5, rabbit; Not skin irritant; /. OECD 405, rabbit; Not eye irritant; Corneal effects. EPA OPP 81.4, rabbit; Not eye irritant; Corneal and conjunctival effects. OECD 405, rabbit; Not an eye iritant; Conjunctival effects. OECD 405, rabbit; Not an eye irritant; /. Copper sulphate pentahydrate; OECD 404, rabbit; Not skin irritant; /. OECD 405, rabbit; Severe eye irritant; Lesions. Coated copper flakes; OECD 404, rabbit; Not skin irritant; /. Coated copper flakes; OECD 405, rabbit; Slight eye irritation; corneal and conjunctival effects. Of the category substances, dicopper oxide, copper sulphate pentahydrate and coated copper flakes are considered an eye irritation hazard. Copper oxides and dicopper chloride hydroxide do not present a hazard for eye irritation. No data are available for copper powders. None of the category substances are considered a skin irritation hazard. Skin sensitization Available key study data for skin sensitization: Test substance; Guideline; Result; Symptoms. Dicopper oxide; OECD 406, guinea pig; Not sensitizing; No skin response after challenge.. Copper oxide; OECD 406, guinea pig; Not sensitizing; Mild skin response: 10% w/w: discrete or patchy erythrema at challenge sites in 4/10 animals) at 24h; no skin reaction at 48h. 5% w/w: discrete or patchy erythrema at challenge sites in 2/10 animals) at 24h; no skin reaction at 48h.. Dicopper chloride trihydroxide; OECD 406, guinea pig; Not sensitizing; No skin response after challenge.. EU B.6, guinea pig; Not sensitizing; No skin response after challenge.. Copper sulphate pentahydrate; OECD 406, guinea pig; Not sensitizing; Slight erythema in 1/20 animals at 24h observation only.. Coated copper flakes; OECD 406, guinea pig; Not sensitizing; No skin response after challenge.. The available animal data suggest the chemicals are of low hazard Repeated dose toxicity Available key study data for repeated dose: Test substance; Toxicity; Method and study type details; Symptoms; NOAEL / LOAEL. Copper sulphate pentahydrate; Oral; B.26 Rat m/f subchronic Dose: 0, 8, 17, 34, 67 or 138 mg Cu/kg bw/day, for 92 days for 7d/week, administrated in the feed; hyperplasia of the squamous, liver inflammation, altered clinical chemistry and urinary parameters, increased cytoplasmic droplets; NOAEL: 16.7 mgCu/kg bw/day. Copper sulphate pentahydrate; Oral; B.26 Mice m/f subchronic Dose: 0, 44, 97, 187, 398 and 815 mg Cu/kg bw/day in males, and 0, 52, 126, 267, 536 and 1058 mg Cu/kg bw/day in females for 92 days for 7d/week, administrated in the feed; hyperplasia of the squamous, liver inflammation, altered clinical chemistry and urinary parameters, increased cytoplasmic droplets; NOAEL: 97.2 mgCu/kg bw/day (m) 125.7 mgCu/kg bw/day (f). Dicopper oxide (Cu2+); Inhalation; OECD 412 Rat m/f 0.21, 0.41, 0.8, 2.0 mg/m3 (analytical conc.) Vehicle: air Exposure: 28 days, 6 hours per day (5 days per week.); macrophages in the lung, increase in neutrophil number in BALF and blood, increase in LDH and protein levels in the BALF. Neutrophil-dominated inflammation in the lung. Most test substance-related effects at 2.0 mg/m3 appeared to show a peak in the effect prior to completion of exposure. Decreased wet/dry lung weight ratio (highest exposure level only); NOAEC: >= 2 mg dicopper oxide/m3 air. Studies have shown that copper and copper compounds are considered equally or less bioavailable to a number of animal species when compared to copper sulphate, therefore the use of copper sulphate studies is justified on scientific grounds. The copper sulphate studies indicate that there is no evidence to indicate that copper or copper compounds present a hazard for repeat dose oral toxicity. The inhalation study on dicopper oxide in the rat performed to a standard guideline is considered the most relevant. In this study, no serious adverse effects were observed up to the maximum test concentration (2 mg Cu/m3 as dicopper oxide). Therefore dicopper oxide does not present a hazard for repeated dose inhalation toxicity. Genotoxicity Available key study data for genotoxicity: Test substance; Method and study type details; Test results; Evaluation of results. Copper sulphate pentahydrate; OECD 471 In vitro Bacterial reverse mutation assay Salmonella typhimurium Doses: 1.6, 8, 40, 200, 1000 μg/plate (exp. 1) and 50, 100, 200, 400, 800 μg/plate (exp. 2); Negative for Salmonella typhimurium Strains TA98, TA100, TA1535, TA1537, TA102 (all strains/cell types tested); metabolic activation: with and without; Cytotoxicity: yes; Negative. Copper sulphate pentahydrate; EU B.12 In vivo micronucleus assay mouse; Genotoxicity: negative; Negative. Copper sulphate pentahydrate; OECD 486 In vivo Oral: gavage unscheduled DNA synthesis rat; Genotoxicity: negative (m); Negative. There is no human data available on the genotoxic potential of copper and copper compounds in humans. Copper and copper compounds are not genotoxic. Carcinogenicity Dietary copper/copper compounds have been administered orally to rats in long-term studies. None of the studies presented below meets exactly the requirements of the International Guidelines, but they do show conclusively that copper has no carcinogenic activity. Two types of studies have been performed: - Investigative toxicity studies demonstrating no tumor formation or long term effects even at very high dose levels. - Co-administration with known carcinogens to demonstrate that copper is effective at reducing the incidence and delaying the onset of tumours. Toxicity to reproductive organs and fertility and developmental toxicity Available key study data for toxicity to reproductive organs and fertility: Test substance; Method and study type details; NOAEL. Copper hydroxide; OECD 414 Teratogenicity Rabbit, f Oral: gavage Day 7-28 of gestation Dose: 0, 6, 9, 18 mg Cu/kg bw/day; Maternal toxicity reported at 9 mg/kg bw/d (inappetance and initial weight loss) and 18 mg/kg bw/d (deaths, weight loss). Effects on foetus (increased incidence of some common skeletal variants and 9 and 18 mg/kg d. NOAEL maternal toxicity 6 mg/kg bw/day NOAEL teratogenicity 6 mg/kg bw/day. Copper sulphate pentahydrate; OECD 416 Multi-generation Rat m/f 0, 100, 500, 1000, 1500 ppm in diet Two-generation study Exposure >= 70d before mating; 1500 ppm or 23.6 mg Cu/kg bw/day (m) (P) 1000 ppm (F1 and F2) or 26.7 mg Cu/kg bw/day – reduced spleen weight (f). Dicopper chloride; OECD 422 Rat m/f 0, 0.8, 3.2, 12.9, 51.7 mgCu/kg bw/day Reprotoxcity/developmental toxicity screening test; 12.9 mg Cu/kg bw/day based on systemic and reprotoxic effects. In the teratogenicity study, maternal toxicity was represented by initial weight loss. These effects are considered to be local effects on the stomach in rabbits which result from gavage administration of copper hydroxide. Consequently, it is considered inappropriate to use data on maternal toxicity from this study as the basis of a repeat-dose NOAEL for copper. The spleen effect cannot be considered a reproductive effect. The existing toxicology data therefore supports the conclusion that copper has no reproductive or developmental toxicity potential. The results of multi-generation study indicates that under the conditions of this study, the no-observed-adverse-effect level (NOAEL) for reproductive toxicity was 1500 ppm (23.6 mg Cu/kg bw/day), the highest concentration tested. The NOAEL for P1 and F1 rats and F1 and F2 offspring during lactation was 1000 ppm, based on reduced spleen weight in P1 adult females, and F1 and F2 male and female weanlings at 1500 ppm however the transient reduced spleen weights are not considered a reproductive endpoint as it did not affect growth or fertility. Three of the substances and one form of zero valent copper in this category present a hazard for human health, based notably on the release/bioaccessibility of copper ions. - Copper sulphate pentahydrate: severe eye irritation and acute hazard by the oral route. - Dicopper oxide: severe eye irritation, acute hazard by the oral and inhalation routes. - Dicopper chloride hydroxide: acute hazard by the oral and inhalation route. - Coated copper flakes: eye irritation, acute hazard by the oral and inhalation routes. Copper oxide, copper powder and copper massive do not pose a hazard to human health. In addition, the currently available evidence on the substances in the category do not cause concern for repeated dose toxicity, genotoxicity, reprotoxicity and carcinogenicity. Adequate screening-level data are available to characterize the human health hazard for the purposes of the Cooperative Chemicals Assessment Programme. Note: A voluntary risk assessment of copper and copper compounds was performed in the context of the EU Existing Substances Regulation Environment Essentiality and copper background level Copper is an essential micronutrient, needed for optimal growth and development of micro-organisms, plants, animals and humans. Copper deficiency and copper toxicity was experimentally observed in freshwater, marine water and soil organisms. Considering that copper is a natural element, essential for all life forms, the safe upper threshold levels need to be compared with copper background levels, the optimal concentration ranges and essentiality levels. As an example, typical copper ambient background levels, reported for European surface waters, range between 0.1 and 14 µg Cu/L with an EU-wide median value of 0.9 µg Cu/L. Some studies demonstrated copper deficiency for freshwater organisms at levels ranging between < 1 and 10 µg dissolved Cu/L. Environmental fate properties Copper is a natural element and transition metal. The release of copper ions (eg Cu(II)) depends on the substance, particle size and characteristics of the receiving medium. The solubility of Cu2O and CuO is dependent on pH. Data on solubility and/or transformation/dissolution (OECD 29) indicate that copper sulphate pentahydrate, dicopper chloride hydroxide and Cu2O have higher solubility than CuO (Annex 1). Copper metal (Cu°) needs to be transformed to its ionic forms (Cu+/Cu2+) to dissolve copper-ions. Copper metal (Cu°) transformation/dissolution to ionic copper (Cu+/Cu2+) takes place at the surface of the copper particles and is related to the pH and surface area exposed. Transformation/dissolution tests (OECD 29) on CuO and Cu°, demonstrate important pH dependent copper release rates of ionic copper. At pH 6, 7days transformation/dissolution of CuO releases more than 10 times more copper ions than at pH 7. Similarly, at pH 6, 7 days transformation/dissolution of Cu° releases 5 times more copper ions than at pH 7. The results from the transformation/dissolution tests of CuO, copper massive materials (spheres of 1-1.5 mm diameter), copper powders and coated copper flakes are summarised in the table below. Transformation/dissolution of CuO and Cu° materials, in accordance to OECD 29 Material; pH; Mass loading (mg/L); Surface loading (mm2/L); µg soluble Cu/L. 7 days; 28 days. CuO; 6; 1; 49; 210. 7; 5; 9. 8; 0; 1. Copper massive - wire and epoxy mounted (1.5 mm); 6; 100; 43-47; 6-19. Copper massive (1 mm) - read-across from wire and epoxy-mounted; 6; 1; 0.67; <1. 6; 10; 6.7; 1-3. 6; 100; 67; 9-27. Copper massive (1 mm) (epoxy mounted); 6; 1; 0.67; 1; 3. Fine Copper powders (10-50µm) -read-across from wire and epoxy-mounted; 6; 1; 67-107; 9-44; 37-176*. Coated copper flakes; 6; 1; 2900; 721; 773. 7; 1; 2900; 363; 639. *From linear extrapolation of the7 days transformation/dissolution data In conclusion, the information from solubility (see physico-chemistry) and transformation/dissolution tests with various copper-bearing materials demonstrate important differences in solubilisation properties ranging from fully soluble CuSO4.5H2O to copper massive granules (1 mm diameter), with only 0.3% transformed/dissolved in 28 days at pH 6. Hydrolysis, biodegradation and phototransformation are not applicable endpoints for copper and the inorganic copper salts. The occurrence of various copper species will depend on the characteristics of the receiving environment. After being released into the environment, the Cu(II) ions typically bind to inorganic and organic ligands contained within water, soil and sediments. In water, Cu(II) binds to dissolved organic matter (e.g., humic or fulvic acids). The Cu(II) ion forms stable complexes with -NH2, -SH, and, to a lesser extent, -OH groups in these organic acids. Cu(II) will also bind with varying affinities to inorganic and organic components in sediments and soils. For example, Cu(II) binds strongly to sulphides in sediments and to hydrous manganese and iron oxides in clay, and to humic acids, but much less strongly to aluminosilicates in sand. In all environmental compartments (water, sediment, soil), the binding affinities of Cu(II) with inorganic and organic matter is dependent on pH, the oxidation-reduction potential in the local environment, and the presence of competing metal ions and inorganic anions. The results of comparing the bio-availability of the Cu-ions in the receiving compartment must therefore be integrated in effects and risk characterisations. Typical Kd-values for copper to freshwater suspended matter, freshwater sediment and soil are 30,246, 24,409 and 2,120 L/kg, respectively. Typical Kd-values for copper to marine and estuarine suspended matter are 131,826 and 56,234 L/kg, respectively. Typical Kd values correspond to 50th percentiles of distribution of values from available monitoring data. Scientific information on copper bioaccumulation factors (BCF, BAF, TTF) does not support the use of BAF or BCF values when they are used as traditional generic threshold criteria for the hazard potential since they are not an intrinsic property for copper. Therefore, for inorganic copper compounds, bioaccumulation factors should be used with caution (Document ENV/JM/MONO(2001). Indeed, for copper, acclimation and homeostatic regulation mechanisms are induced after longer exposure times. Furthermore, some organisms accumulate copper in a non-bioavailable form by using copper binding and sequestrations mechanisms as regulation system. These adaptation and regulation systems play a role in the copper essentiality/toxicity profile and therefore, the BCF/BAF is not independent of exposure concentration and has no meaning for a hazard assessment. Aquatic toxicity according to standard species/protocols Different metal species are used as test substances in various ecotoxicity tests (e.g. CuSO4.5H20, CuCl2, CuO). The metal ion (Cu2+) drives the aquatic ecotoxicity and therefore, only tests with soluble inorganic copper compounds are retained. Acute and chronic toxicity data from the various soluble compounds (e.g. CuSO4.5H20, CuCl2) were combined and expressed as soluble metal ion concentrations (μg dissolved Cu/L) causing a specific effect. To derive a reliable baseline data-set, the high quality measured toxicity values (µg dissolved Cu/L), obtained from the standard OECD test species and endpoints were retained. High quality, acute L(E)C50 values were retained from short term standard freshwater tests for 10 standard fish, invertebrate and algae test species (Pseudokirchneriella subcapitata, Chlamydomonas reinhardtii, Chlorella vulgaris, Ceriodaphnia dubia, Daphnia magna, Oncorhynchus mykiss, Pimephales promelas, Lepomis macrochirus, Brachydanio rerio and Cyprinus carpio). Reliable values range between 3 and 9,150 µg Cu/L. The variability has been attributed to species-specific differences in sensitivity as well as water-characteristics such as pH, DOC and hardness. Species-specific geometric mean values range between 34 µg Cu/L (Ceriodaphnia dubia) and 2837 µg Cu/L (Lepomis macrochirus). High quality, chronic NOEC(s) / EC10 values were retained from standard freshwater tests for 9 standard fish, invertebrate and aquatic plant test species (Pseudokirchneriella subcapitata, Chlorella vulgaris, Chlamydomonas reinhardti and Lemna minor, Ceriodaphnia dubia, Daphnia magna, Oncorhynchus mykiss, Pimephales promelas and Salvelinus fontinalis). Reliable values range between 2 and 337 µg Cu/L. The variability has been attributed to species and endpoint specific differences in sensitivity as well water-characteristics such as pH, DOC and hardness. Species-specific geometric mean values range between 12 µg Cu/L (Ceriodaphnia dubia and Oncorhynchus mykiss) and 137 µg Cu/L (Chlorella vulgaris). The information further indicates: - The cellular mechanism of copper toxicity/deficiency, as well as the cellular mechanisms of copper homeostasis, has been largely preserved through evolution. - The key indicator of copper toxicity is disturbance of the sodium homeostasis. The key target tissue for copper toxicity is therefore the water/organism interface, with cell wall and gill-like surfaces acting as target biotic ligands in all species investigated. - The information on the ecotoxicity following exposures, through water and/or food and the information on metal bio-accumulation and homeostasis explain the observed small ratios between mortality and sub-lethal endpoints (typically a factor of 1 to 3). - Large intra-species variability is observed and has been related to differences in copper species formed and different bio-availability in the various test media. The hazard assessment of the substances in the category, derived from the acute L(E)C50 or chronic NOEC(s) / EC10 values (µg dissolved Cu/L) and a molecular weight translation or the results from transformation/dissolution tests, shows that the copper compounds and the fine copper powders do pose a hazard to the environment. Coarse granules and massive copper materials release less copper ions and therefore pose less or no hazard to the environment under typical conditions. Deriving the HC5,50% from Species Sensitivity Distributions for freshwaters, marine waters and soils. Several phenomena on the ecotoxicity of copper were considered for deriving HC5,50% for freshwater, marine and soil compartments: - The toxicity response is species-specific - The toxicity response is dependent on the receiving environment - The toxicity response is dependent on background levels - The toxicity response is changing with time : e.g. ageing of copper in soils. Freshwater To derive a HC5,50% value for the freshwater compartment, the standard chronic toxicity database was extended to include standard and non-standard species/protocols/endpoints. This resulted in a single-species chronic toxicity data–base of more than 200 NOEC/L(E)C10 values and 3 high quality, multi-species mesocosm studies. The observed single species NOEC/L(E)C10 values range between 2 and 510 µg Cu/L. The observed mesocosm NOEAECs and LOEAEC values were, respectively, 4 to 20 µg Cu/L and 9 to 40 µg Cu/L. Accounting for species –specific differences : The copper aquatic effects database contains high quality, single-species chronic NOEC/L(E)C10 values for 27 species, representing algae, invertebrates (cladocerans, rotifer, molluscs, insects and amphipods), fish and higher plants. Species-specific geometric mean values range between 6 µg Cu/L (Juga plicifera)) and 137 µg Cu/L (Chlorella vulgaris). Considering the large number of species assessed, the statistical extrapolation method, applied to all NOEC/L(E)C10 values, is used to derive the HC5,50%.. Such Species Sensitivity Distribution was constructed using the non-normalised species-mean NOEC values for the most sensitive endpoints and resulted in a log normal HC5,50% of 6 µg Cu/L. Accounting for dependence on the water type: The species-specific NOECs observed are often characterised by large variability because Cu bioavailability and toxicity to aquatic organisms is influenced by abiotic parameters, such as pH, hardness and dissolved organic carbon (DOC). This raised the need to develop/use a bioavailability normalisation process for the HC5,50% derivation. Chronic Biotic Ligand Models were developed for Pseudokirchneriella subcapitata, Daphnia magna, Pimephales promelas and Oncorhynchus mykiss in order to provide a mechanistic basis for understanding and predicting bioavailability through integration of chemical parameters (e.g. pH, hardness, DOC) and biological parameters (receptor sites on organism, mode of action). The BLM models developed were further validated to represent the three basic trophic levels (algae, invertebrates and fish): (1) a unified chronic model for the algae (Pseudokirchneriella subcapitata, Chlamydomonas reinhardtii and Chlorella vulgaris). The applicability of the model for predicting higher plant ecotoxicity (hydrocultures of barley) was demonstrated (2) a chronic BLM for invertebrates (Daphnia magna). The capacity of the BLM for predicting copper toxicity to other invertebrates was demonstrated from copper toxicity studies with Brachionus calyciflorus, Lampilis siliquoidea, Hyridella depressa and Hyalella azteca (3) a unified chronic model for 2 fish species (Pimephales promelas and Oncorhynchus mykiss). The boundaries of the BLM applicability across species have been defined for pH (6-8.5), hardness (12-360 mg CaCO3/L), dissolved organic carbon (DOC) (0-20 mg/L). The database showed under prediction for one field water with high Fe and Al and therefore boundaries were set as 332 µg dissolved Al/L and 332 µg dissolved Fe/L. The BLMs developed for chronic fish (P. promelas and O. mykiss), invertebrates (D. magna) and algae (P. subcapitata) were used for normalising all retained chronic NOEC values of respectively fish, invertebrates and algae/plant species. Briefly, the bioavailability normalisation process normalises the NOEC/(L(E)C10 values to site-specific physicochemical conditions (i.e. pH, hardness and DOC). The BLM normalised NOEC/(L(E)C10 values were used to construct Species Sensitivity Distributions for a range of physico-chemical conditions in European surface waters. Typical HC5,50% s range between 7 and 30 µg Cu/L. Validation of the HC5,50% derivation for multi-species systems: Species-specific NOECs and mesocosm specific NOEAECs / LOEAEC, protective of ecosystem structures and functions, are obtained from three distinct, high quality mesocosm studies, representing lentic and lotic system. Detailed comparisons between the BLM predicted and observed mesocosm effects, demonstrate that BLM could adequately predict the mesocosm sensitivity within a factor of 2. Marine The differences in physiology between freshwater and marine organisms, and the related differences in ecotoxic behaviour, led to the derivation of a separate HC5,50% value for freshwater and marine environments. The estuarine compartment is not covered in this assessment. The copper marine effects database contains more than 50 high quality, chronic NOECs/EC10s values varying between 3 µg/L (Phaeodactylum tricornutum) and 145 µg/L (Penaeus monodon). Accounting for species –specific differences: The copper marine effects database contains high quality, chronic NOECs/EC10s values for 24 species. Species-specific geometric mean values range between 4 µg/L (Phaeodactylum tricornutum) and 145 µg/L (Penaeus monodon). A Species Sensitivity Distribution was constructed using the species-specific NOECs and resulted in an HC5,50% of 5 µg Cu/L. Accounting for the characteristics of the marine water: Marine waters are characterised by high pH (typically around 8.3), high salinity (35%o) and high ionic strength. Unlike the inorganic composition of marine waters, DOC levels may vary considerable between marine water bodies. Open ocean waters usually have lower DOC, ranging between 0.5 and 1.8 mg/L. As for the freshwater system, Cu-availability and toxicity to marine organisms is therefore influenced by the strong binding of copper to the dissolved organic carbon (DOC). This raised the need to use an availability normalisation process. A relationship between the EC50s or NOEC/EC10, values and the DOC levels were assessed for 6 species: Fucus vesiculosus, Crassostreas gigas, Mytilus galloprovincialis, Dendraster excentricus, Strongylocentrotus purpuratus. Since the six data sets are statistically equivalent, these were combined for deriving an overall descriptor of the protective effects of DOC. This equation was used to translate all NOEC data to standard DOC levels of 2 mg DOC/L for coastal waters and 0.2 mg DOC/L for the open sea. An organic carbon normalisation was carried out and the HC5,50% was derived at a DOC levels representative of coastal and open ocean areas (2 and 0.2 mg/L). From the high quality data, HC5,50% values of respectively 5.2 and 1.3 µg Cu/l, were derived. Validation of the HC5,50% derivation for a marine mesocosm: A marine mesocosm study resulted in NOEAEC and LOEAEC values of respectively 5.7 and 9.9 µg dissolved Cu/L. The mesocosm study therefore supports the HC5,50% obtained from the single-species study. The assessment also confirmed that the DOC- normalised single species HC5,50% is protective to the ecosystem structure and function. Accounting for acclimation: copper deficiency is a well known phenomenon in open ocean but occurs at levels below the derived HC5,50% for open oceans. Terrestrial The copper terrestrial effects database contains more than 250 high quality, chronic NOEC/EC10 values. The chronic NOECs/EC10s vary between 8.4 mg/kg for Eisenia andrei (cocoon production) and 2,402 mg/kg (maize respiration). The lowest value is actually below the limit for essentiality for the species. Information on 8 single species studies, in field contaminated soils, and 5 multi-species studies (freshly spiked and field contaminated) were used as an additional weight of evidence for the terrestrial compartment. Accounting for species –specific differences: The copper terrestrial effects database contains high quality, chronic NOEC values for 19 species and 9 microbial functions. Accounting for bioavailability dependence on the soil characteristics: To normalise the bio-availability data for soil type, a total of seven regression models were derived to predict toxicity of copper to terrestrial organisms for a wide range of soil types. For plants, the L. esculentum model (endpoint yield) was applied only on data for tomatoes, while all other plant data were normalised using the H. vulgare root elongation model because this endpoint is the most sensitive for plants. For invertebrates, the E. fetida model was used to normalise all soft-bodied species, while the F. candida model was used to normalise all hard-bodied species. For the microbial processes, all NOEC values related to the N-cycle were normalised based on the CEC slope of the nitrifying micro-organisms. The maize respiration model was used for normalisation of all microbial processes using a natural substrate. All other microbial processes were normalised using the substrate induced respiration model. Accounting for soil leaching and ageing : Observed differences in toxicity of copper to terrestrial organisms, between lab spiked soils and field contaminated soils, allowed for the derivation of a leaching-ageing factor of 2, based on the 25-percentile of the ecotoxicity database. This factor was further supported by the mechanistic research on ageing and ionic strength (leaching) effects. Deriving HC5,50% values: Considering bio-availability and ageing, information from a large monitoring database allows calculating four HC5,50% values of copper for soil samples taken from grazing land in European countries. The HC5,50% values range between 13 and 205 mg Cu/kg dry weight, depending on the soil chemistry. A reasonable worst case 10th percentile of 69.6 mg Cu/kg dry weight is retained for the grazing land. A reasonable worst case 10th percentile of 59.5 mg Cu/kg dry weight is retained for the arable land. The overall soil median reasonable worst case value across the two land-types is 64.6 mg Cu/kg dry weight. Copper and copper compounds may present a hazard for the environment depending on the release/bioaccessibility of copper ions and on the conditions of the receiving environment (pH, hardness, presence and type of organic matter, anions and competing cations). Adequate screening-level data are available to characterize the environmental hazard for the purposes of the Cooperative Chemicals Assessment Programme. Note: A voluntary risk assessment of copper and copper compounds was performed in the context of the EU Existing Substances Regulation. Exposure Production and uses - copper - worldwide The 2012 global production of copper was 25.7 million tonnes. Approximately 20.2 million tonnes was produced from mining (primary production) and the smelting/refining of complex, end of life materials (secondary production). Another 5.5 million tonnes, combing offcuts from the value chain and clean, end-of- life scrap, were recycled directly by the producers of semi-fabricated products. Recognising the locations of today’s copper mines, Latin American is the main exporting region. The major importers are Asia, (particularly China, India and Korea) and, to a lesser extent, Europe. North America is reasonably balanced between production and demand. Production and uses - copper – Europe EU production volume in 2012 of copper has stabilised at around 1.5 million tonnes for smelting and 1.8 million tonnes for refining. Additionally, around 1.3 million tonnes of scrap (secondary recycled raw material) is used as feedstock material. Besides the production, about 0.6 million tonnes of refined copper are imported in the EU. Copper massive forms account for +/- 99.6% of the market. Principal uses include copper wire, for power cables, building wire, electric motors and voltage transformers, copper tubes and fittings, for domestic water, gas distribution, water heating systems and air conditioning, strip for the electronics industry and sheets for roofing, gutters and down-pipes. Extrapolating EU data, copper powders are estimated to account for +/- 0.4%. They are mainly used for friction materials such as for vehicle brake pads, carbon brushes for electrical motors and sintered parts for engineering components. Coated copper flakes represent less than 0.1% and are used mainly as pigments and as an active ingredient in antifouling paints. Copper oxide is used as wood preservative (biocidal/antimicrobial), catalyst, brake pads, industrial (glass, ceramics). Dicopper oxide is used as antifouling (biocidal/antimicrobial), fungicide (agrochemical), catalyst, “hot” industrial processes. Copper sulphate pentahydrate is used as algaecide (biocidal/antimicrobial), fertiliser, raw material use, general industrial uses (dyes, mineral flotation, ceramics, glass), animal feed and foodstuffs. Dicopper chloride hydroxide is used as fungicide (agrochemical), fertiliser and industrial uses e.g. ceramics EU- exposure Sources Copper is a naturally occurring element that can be found at background levels in water, sediment and soil. Total copper releases were dominated by agricultural uses (feed additives and fertilizers, 39%) and traffic (mainly brake pads, 43%). Massive copper uses (wear of overhead wires, corrosion of copper tubes, fittings and taps and external building applications (roofs, gutters, down pipes, facades)) contribute to 15% of the total anthropogenic EU emissions. The relative contribution from waste incineration plants and landfill facilities is minor (0.4%). Other minor copper releases, that have been observed include, among others, industry releases, domestic and industrial heating, fireworks and domestic wastewaters . Monitoring Background levels of copper in water, sediment and soil are reported in the EU FOREGS Geochemical Atlas (Forum of European Geological Surveys). Median natural background concentration levels in Europe are 0.88 µg dissolved Cu/L for surface water (rivers and lakes), 14 mg/kg dry weight for river and lake sediment and 12 mg/kg dry weight for topsoil. Region-specific dissolved Cu freshwater PEC values, derived for Austria, Belgium, Denmark, Finland Barentz area, Germany Elbe, Ireland, Portugal, The Netherlands, Sweden, England, Wales and Scotland ranged between 0.5 and 4.7 μg dissolved Cu/L with a median of 2.7 μg dissolved Cu/L. Measured PECs for the EU-15 are 67,5 mg/kg dw for sediment and 31 mg/kg dw for agricultural soil. Humans exposed via the environment The contribution to dietary intake (copper from anthropogenic origin in fruit and vegetables, locally grown, and other foodstuffs) in the local environment was rather low due to the impact of industrial air pollution control measures and the effective homeostatic control of copper uptake by plants. External exposure through inhalation is even more limited. Occupational exposure The sectors that have been identified for estimating local exposures include: Smelting and Refining, Wirerod & Cables production, Casting Billets and Plates, Production of semi-finished copper shapes, Production of Copper Powders and Copper Chemicals. For melting and casting of billets and further processing, respirable copper as a function of total copper ranges from 6-25% with a median value of 9%. For smelting a single value of 13% is given while for non-foundry operations in the manufacture of copper powders the single value of 4%. Consumer exposure Consumer exposure to copper may occur via dermal or oral routes or via inhalation. Dermal exposure occurs mainly through the use of toiletries and cosmetics face cream and hair care products, through coin handling or jewellery. Additional dermal exposure is possible from the use of special paints or from copper containing wood preservatives and pesticides. Oral exposure (other than from food and water) occurs in particular by ingestion of dietary supplements containing copper, inhalation exposure occurs mainly through cigarette smoke. Internal exposure may also occur with the use of intra-uterine devices. There are no known exposures of consumers to other copper compounds covered in this category. |
| Limitations The hazard profile is based on intensive literature searches and targeted research programs carried out for the European hazard classification and risk assessment regulation during the period 2000-2006. Recent findings are not included. |  |

ANNEX 1: Standard OECD Solubility of copper compounds

| Compound | pH range |  |  |  |
| --- | --- | --- | --- | --- |
|  | 5.5-6.5 | >6.5-7.5 | >7.5-8.5 | >8.5-10 |
|  | Solubility (mg/L) |  |  |  |
| CuSO4.5H2O | 220000 |  |  |  |
| Cu2Cl(OH)3 | 1.19 | - | - | 0.525 |
| Cu2O | - | 0.639 | - | 0.539 |
| CuO | 0.394 | - | - | 0.01 |

SIDS Initial Assessment Report

For

CoCAM 6

30 - 03 October, 2014

| 1{a}. Chemical Name(s): | Copper (copper powder and massive) Copper sulphate pentahydrate Copper (II) oxide Copper (I) oxide Dicopper chloride trihydroxide |
| --- | --- |
| {1b. Category Name:} | Copper and copper compounds |
| 2. CAS Number(s) and chemical name: | CAS 7440-50-8 (copper powder and massive1) CAS 7758-98-7 (copper sulphate pentahydrate) CAS 1317-38-0 (copper (II) oxide) CAS 1317-39-1 (copper (I) oxide) CAS 1332-65-6 (dicopper chloride trihydroxide) |
| 3. Sponsor: | Istituto Superiore di Sanita Viale Regina Elena, 299, I-00161 Rome, Italy Leonello Attias |
| 4. Shared Partnership with: | European Copper Institute, Tervurenlaan, 168 - box 10 1150 Brussels, Belgium. Stijn Baken, Katia Lacasse, and Katrien Delbeke, RCL – Regulatory Compliance Limited, secretartiat for the copper compounds consortium. 5 Telford Gardens Brewood, Staffordshire, UK ST19 9ED. Carol Mackie, |
| 5. Review and Quality Check Process Prior to CoCAM: | TCNES, SCHER and the Technical Committee for the Biocidal product Directive (http://echa.europa.eu/web/guest/information-on-chemicals/transitional-measures/voluntary-risk-assessment-reports); all data prior to 2008 were part of the EU voluntary risk assessment for copper, and were thoroughly evaluated on their quality and usefulness. Information from 2008 onwards were not part of this revaluation process |
| 8. Comments: | The summaries and evaluations contained in this assessment report may be based on unpublished proprietary data submitted for the purpose of the OECD assessment. In many cases, national/regional regulatory authorities will not accept a regulatory submission based on the summaries and evaluation of unpublished proprietary data contained in this [these] assessment report[s] unless they have received the data on which the summaries and evaluation are based, either from: - the owner of the data; or - a second party that has obtained permission from the owner of the data for this purpose; or alternatively - the applicant has received permission from the data owner that the summary and evaluation contained in this [these] assessment report[s] may be used in lieu of the data; or - following expiry of any period of exclusive use, mandatory compensation, where required, has been offered unless the period of protection for the proprietary data concerned has expired. Applicants who wish to use data in this assessment report should ensure that the use is consistent with the legal requirements in the country/region in which it is proposed to use the data. |

SIDS Initial Assessment Report

# identity

## Identification of the Substance

| Property | Copper 1 | Copper sulphate pentahydrate | Copper (II) oxide | Copper (I) oxide | Dicopper chloride trihydroxide |
| --- | --- | --- | --- | --- | --- |
| CAS N umber: | 7440-50-8 | 7758-98-8 | 1317-38-0 | 1317-39-1 | 1332-65-6 |
| IUPAC Name: | Copper | Copper (II) sulphate | Copper(II) oxide | Copper(I) oxide | Dicopper chloride trihydroxide |
| Molecular Formula: | Cu | CuSO4; 5H2O | CuO | Cu2O | Cu2Cl(OH)3 |
| Structural Formula: |  |  |  |  |  |
| Molecular Weight: | 63.546 | 249.685 | 79.545 | 143.09 | 213.56 |
| Synonyms: |  | Cupric sulfate | Cupric oxide | - Cuprous oxide - Cuprite - Red copper oxide | - Tribasic copper chloride (TBCC) - - Copper Hydroxychloride - Copper trihydroxyl chloride -Dicopper chloride trihydroxide |

1 Copper flakes, coated with aliphatic acids are also included in the dossier

## Purity/Impurities/Additives

Copper Massive

Purity: 99.9%  Cu

Possible Impurities: trace amounts of inorganic elements

Copper Powder

Purity: >95% Cu

Impurities: Cu2O <5%.   Other relevant impurities: Pb, Ni, As, Cd

Coated copper flake

Purity : 96% Cu, additive : <3% aliphatic acids

Impurities: CuO (<3%). Other relevant impurities: Pb, Ni, As, Cd

Copper (II) sulphate pentahydrate

Purity:  98.5% CuSO4.5H20 (25% Cu)

Main relevant impurities: Pb, Ni, As, Cd

Copper (I) oxide

Purity: 97.2% Cu(I)O (87.4% Cu)

Main relevant impurities: Pb, Ni, As, Cd

Copper (II) oxide

Purity: 97.5-98.5% CuO (78% Cu)

Main relevant impurities: Pb, Ni, As, Cd

Dicopper chloride trihydroxide

Purity: 96-97% dicopper chloride trihydroxide  (57-58% Cu)

Main relevant impurities: Pb, Ni, As, Cd

## 1.3	Physical-Chemical Properties

The physico-chemical properties of Cu, CuSO4.5H2O, CuO, Cu2O and Cu2Cl(OH)3, given in the Table 1 are mainly provided by copper producers.

Table 1 : Physico-chemical properties of the substances

- copper metal; (2) Coated flakes

|  | Cu metal (1) and coated copper flakes (2) |  | Copper(II) sulphate pentahydrate | Copper (I) oxide |  | Copper (II) oxide |  |  | Dicopper chloride trihydroxide |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Physical state | Solid (1) Liipo et al 2010a (2) Liipo et al. 2010b, Messershmidt 2006a | Solid Lide DR 2009 |  |  | Solid (O'Connor B and Mullee D 2003) |  | Solid (Desai LS 1992) | Solid (O’Connor BJ and Mullee DM 2003) |  |
| Boiling point | Not required * | Decomposes without boiling at 110°C (Lide DR 2009) |  |  | Boiling point cannot be determined because of decomposition to at >332°C |  | not applicable | Boiling point cannot be determined because of decomposition (onset temperature 240°C |  |
| Melting point | 1059-1069°C - Liipo et al. 2010a - Liipo et al. 2010b, Franke 2006a | Decomposes without boiling at 110°C (Lide DR 2009) |  |  | Melting point cannot be determined because of decomposition to CuO at >332°C |  | 1326°C (Lide DR 2003) | Melting point cannot be determined because of decomposition (onset temperature 240°C) (O’Connor BJ and Mullee DM 2003) |  |
| Relative density | - Relative density Cu metal : 8.94 g.cm-3 at 20 °C (Liipo 2010a) (2) Tap density Coated Cu flakes 0.47 g/ml (Messershmidt 2006a) | 2.29 g cm-3 at 20°C (Lide DR 2009) |  |  | 5.87 g cm-3 at 20°C (O’Connor B and Mullee D 2003) |  | 6.32 g cm-3 at 20°C (Lide DR 2003) | 3.64 g cm-3 at 20°C (O’Connor BJ and Mullee DM 2003) |  |
| Vapour pressure | - Negligible - 7.5e-9 Pa (20°C), 1.8e-8 Pa (25°C) and 8.4e-7 Pa (50°C ( Franke 2006b) | Negligible |  |  | Negligible |  | Negligible at room temperature | Negligible |  |
| Water solubility (H2O) | Insoluble ** | 220g/L (Lide DR 2009) |  |  | pH 4.0: 28.6 g/L at 20°C pH 6.5-6.6: 6.39 x 10-4 g/L at 20°C pH 9.7-9.8: <5.39 x 10-4 g/L at 20°C (O’Connor B and Mullee D 2003) |  | pH 5.1-5.5: >0.23 g/L at 20°C pH 9: <1.0 x 10-5 g/L at 20°C pH 6: 3.94 x 10-4 g/L at 20°C (O’Connor BJ and Mullee DM 2004) | pH 3.1: >101 g/L at 20°C pH 6.5: 1.19 x 10-3 g/L at 20°C pH 10.1: ≤5.25 x 10-4 g/L at 20°C (O’Connor BJ and Mullee DM 2003) |  |
| Flash point | Not required * | Not required * |  |  | Not required * |  | Not required * | Not required * |  |
| Autoignition and Flammability | Non flammable - Liipo 2010a (2) Liipo et al. 2010b | Non flammable |  |  | Relative self ignition temperature is 234°C |  | Non flammable | Non flammable |  |
| Explosive | Not explosive | Not explosive |  |  | Not explosive |  | Not explosive | Not explosive |  |
| Granulometry *** | Dependent on the form*** |  | Ranging from stone size to microcrystals. | Not available |  | Not available |  |  | Not available |

* Flash point is not required since the active substances are solids and do not melt below 55°C. Boiling point is not required since the substance is a solid that melts above 300°C

** Copper metal (Cu°) needs to be transformed to Cu+ or Cu++ to become soluble. A special solubility test (transformation/dissoluiotn test) was developed for the classification of metals for the environment.

***Powders, and particulates d50values>10 µm to < 1 mm. Massives are categorizes as forms with diameter>1mm and thus include large particulates Representative powder samples were characterised with d50 of <50 and 135 µm and specific surface area of 0.024 - 0.107 m2/g, were used as testing material.

Coated copper flakes granulometric d50 values range between 5 and <100 µm. They are characterised by a high surface area.  A representative sample of a coated copper flake with a d50of 8-11µm and specific surface area of 2.9 m2/g was used for testing.

Partition coefficient (log Kow)”, “Flash point”, “Explosive”, and “Oxidising properties”, information is not substance-specific and is not critical from the both environmental and human health view-points.  These properties were therefore excluded from Table1.

## 1.4	Category Justification

The category includes a group of copper substances whose ecotoxicological and systemic human hazard profiles are related to the release of copper-ions. The proposed category was limited to specific copper compounds as they were considered as European priorities and were included in the European Copper Voluntary Risk Assessment (2008). The basis of this dossier and database is the Copper Voluntary Risk Assessment report and the use of copper and copper compounds as general chemicals.

Nanoform copper substances are excluded from this assessment because the biological effects of nanoform metals can differ from the ionic forms.

The category includes 5 substances: soluble copper sulphate, dicopperchloride trihydroxide, less soluble copper (I) oxide and copper (II) oxide, and three forms of zero valent copper materials (Cu°): copper massive, copper powder and coated copper flake. Coated copper flakes are added to the category because human health hazards are available for these coated copper flakes.  Where relevant for read-across purposes, information from other copper compounds is included in the assessment.

For the environmental endpoints and systemic health endpoints, the copper ion is considered as the reactive functional group within the category. The counter-ions of the copper salts (i.e. oxygen, sulphate, chloride and  hydroxide), are due to their ubiquitous presence in environmental media and human fluids not considered to contribute to the environmental nor systemic human toxicity of copper salts.

The category implies that the effects assessment is expressed as Cu-ions and that further read-across to other copper compounds is possible on condition that copper ions are driving the toxicity.

Copper is an essential nutrient for humans and non-human organisms and, therefore, low concentrations may lead to deficiency while high concentrations of copper ions may lead to copper toxicity.

Once released to the environment, copper ions have more than one oxidation state and copper is thus characterized as transition metal. The principal ionic forms are cuprous (Cu(I), Cu+) and cupric (Cu(II), Cu2+). The trivalent form (Cu(III), Cu3+) occurs but is relatively unimportant in physical and biological systems. Cu+ is unstable in aqueous media and soluble Cu1+-compounds readily transform into soluble Cu2+-ions, compounds and/or insoluble Cu2+-ions, compounds (e.g. copper sulphides) that precipitate. Hence Cu1+-ions are, due to their instability, considered as a source of Cu2+ ions for environmental and systemic toxicity.

Environment:

The assessment aims at defining the basic hazards profile of soluble and sparingly soluble compounds, copper metal (powder and massive forms) and coated copper flakes. The assessment also aims at deriving safe threshold values for Cu-ions for environmental compartments (freshwater, marine, soil) with due consideration of the “data-richness” and applications of bio-availability models.

Annex I provides an overview of the category rational matrix for environmental endpoints

Human health:

The human health hazard assessment addresses a large variety of effects and different administration routes. In assessing the human health effects of copper and copper compounds, the essentiality and homeostatic mechanisms have to be considered.

Acute effects are based on tests with the individual substances.  Short term and long term systemic effects are based on tests with the individual soluble substances belonging to the category and/or read-across where such data are not available.

Annex II provides an overview of the category rational matrix for human health endpoints

# general information on exposure

## Production Volumes and Use Pattern

Production and uses - copper - worldwide

According to the International Copper Study Group (http://www.icsg.org/), the 2012 global production of copper was 25.7 million tonnes. Approximately 20.2 million tonnes was produced from mining (primary production) and the smelting/refining of complex, end of life materials (secondary production). Another 5.5 million tonnes, combing offcuts from the value chain and clean, end-of- life scrap, were recycled directly by the producers of semi-fabricated products.

Recognising the locations of today’s copper mines, Latin American is the main exporting region. The major importers are Asia, particularly China, India and Korea) and, to a lesser extent, Europe. North America is reasonably balanced between production and demand.

Production and uses- copper – Europe

EU production tonnage

EU production volume in 2012 of copper has stabilised at around 1.5 million tonnes for smelting and 1.8 million tonnes for refining. Additionally, around 1.3 million tonnes of scrap (secondary recycled raw material) is used as feedstock material. Besides the production, about 0.6 million tonnes of refined copper are imported in the EU.

EU Production process and production sites

- Mining : EU mines are situated in Poland, Portugal, Spain and Sweden

- Hydrometallurgical route: The primary and secondary hydrometallurgical copper production route involves the crushing of the ore followed by leaching using sulphuric acid, sometimes in the presence of biological species, using heap, vat or agitation processes. The liquor produced from leaching is then clarified and purified and concentrated by solvent extraction. A further variation of leaching is possible where black copper is produced from in an Ausmelt/ISA Smelt furnace. The black copper granules are dissolved in an autoclave and the pure copper is electro-won from the resulting solution. The copper is then removed by electro-winning. The electrowinning process differs from the electrorefining process in the form of the anode. Electrowinning uses an inert anode such as lead titanium and the metal ions are removed from the solution and deposited on the cathode in the same manner as electro-refining. Cathodes are then stripped in the same way if permanent cathode blanks are used. The electrolyte is circulated through a series of cells and finally is depleted of copper. Electrolyte is then returned to the solvent extraction circuit. Some electrolyte is normally bled for the control of impurities, which might have been carried over during solvent extraction.

- Pyro metallurgical production route: The production of primary copper starts with the mining of copper ore. In the next step, concentrates are produced at the mining site by crushing, grinding, flotation, filtering and drying ores. Concentrates contain besides copper – usually present as copper sulphides - various other substances and during the production process, these are separated in a stepwise process, thus increasing the copper content of the remaining material. The process consists of the following steps:

Step 1: roasting and smelting of concentrate in a furnace to blister (98% Cu) and further into anodes (99.6% Cu); The concentrate is converted to blister copper in two sub-steps. Both sub-steps are usually performed on the same site. First, copper is concentrated by partial oxidation of the concentrate at around 1250 ºC (smelting). The aim of this step is the removal of Fe and other impurities. Streams of concentrate and product are usually a liquid at this temperature. FeS is partially converted to FeO and SO2. Copper sulphides (various forms) are converted to Cu2S and SO2. The SO2 is carried off in the exhaust gases. A Cu2S rich phase (the matte) does not mix with the remaining phase (FeO/ SiO2 phase), being tapped off as slag. In the second step, the matte produced is further oxidised in a second pyrometallurgical step (conversion) in a converter at 1250 ºC to metallic Cu, FeO and SO2. The conversion takes place in two stages. In the first stage the remaining FeS is oxidised to FeO/Fe3O4, which after mixing with SiO2 is again separated as slag. In the second stage Cu2S is converted into raw metallic copper (blister) and SO2. A small fraction of the copper present in the concentrate comes through to the slag from the smelter or the converter or appears as fly-ash in the waste gases from both processes. If there is a sufficiently high concentration of copper in the slag it is trapped and recirculated to the smelter, thereby increasing the yield of the process.

Step 2: electro-refining of the anode copper into cathode copper of 99.9 % pure copper. During this step, the copper is purified further by electrolysis in a sulphuric acid solution to a purity of > 99.9%; it is made into cathodes in the electrolysis cell

Secondary copper is produced through the same process but using scrap as feed material.

- Production of copper powder: Copper powders are produced mainly by water or air atomisation and by oxide reduction of molten copper. They can also be made by hydrogen reduction, chemical and electrolytic processes.

- Production of coated copper flake is a special production of copper powders in which the copper has been coated with aliphatic acid (considered as non-hazardous) to stabilise the flake in small particle sizes, creating a higher surface area and higher Cu-ion releases (needed for specific applications).

- Production of Copper (II) oxide: Two methods of Copper (II) oxide production have been identified:

1.  Recovery/recycling of various industrial waste streams and secondary materials containing copper.

2.  Metal acidification.  Copper is dissolved in nitric acid to produce the nitrate solution, or the process starts with a metal nitrate solution, which is treated to precipitate the insoluble carbonate.  The precipitate is heat-treated to convert it to the dry oxide powder.

-   Production of Copper (I) oxide: Production of copper (I) oxide involves oxidation of copper metal in a batch process.  The copper (I) oxide is generated as a solid in a water-suspension.  The product is filtered, stabilized and dried.

-    Production of Copper sulphate: Copper sulphate is manufactured using the following processes:

1.  As a by-product of electrolytic purification of crude copper.

2.  By direct reaction of copper metal and sulphuric acid in a hot and oxidizing media. The resulting solution is then cooled to produce copper sulphate crystals.

3.  By ‘chemical synthesis’.

- Production of Dicopper chloride trihydroxide: Dicopper chloride trihydroxide is produced by the acidification of copper metal prior to oxidation.  Two variations have been identified:

1. Copper metal is reacted with hydrochloric acid solution in the presence of oxygen. The resulting suspension is then filtered, dried and milled.

2. Copper metal is mixed with water, to which hydrochloric acid is added to oxidize the copper.  The solution is aerated for 24 hours.  Dicopper chloride trihydroxide is formed once the acid has lixiviated the copper oxide.  The resulting suspension is centrifuged and dried.

EU use pattern

The 2012 EU-27 copper market was estimated at 3.75 Million tonnes.   The use patterns of copper are illustrated in its supply chain in Figure 1.

Figure 1 : Copper supply and use chain (from EU REACH registration dossiers -2013)

Copper massive forms account for 99.6% of the market. Principal end uses include copper wire, for power cables, building wire, electric motors and voltage transformers, copper tubes and fittings, for domestic water, gas distribution, water heating systems and air conditioning, strip for the electronics industry and sheets for roofing, gutters and down-pipes).

Figure 2 : Use categories of copper massive and alloys (from EU REACH registration dossiers -2013)

Copper powders are estimated to account for 0.4% of the total EU production. They are mainly used for friction materials such as for vehicle brake pads, carbon brushes for electrical motors and sintered parts for engineering components.

Figure 3 : Use categories of copper powders

Coated copper flakes represent less than <0.1% and are used mainly as pigments and as an active ingredient in antifouling paints.

The main uses of the copper compounds are as follows:

- Copper (II) oxide – wood preservative (biocidal/antimicrobial), catalyst, brake pads, industrial (glass, ceramics).

- Copper (I) oxide – antifouling (biocidal/antimicrobial), fungicide (agrochemical), catalyst, “hot” industrial processes.

- Copper sulphate – algaecide (biocidal/antimicrobial), fertiliser, raw material use, general industrial uses (dyes, mineral floatation, ceramics, glass), animal feed and foodstuffs.

- Dicopper chloride trihydroxide – fungicide (agrochemical), fertiliser and industrial uses e.g. ceramics

## Environmental Exposure and Fate

Copper is a natural element and transition metal with more than one oxidation state. Copper in its metallic form (Cu°) is not bioavailable. Copper needs to be transformed to its ionic forms to become available for uptake by living organisms.

It is also an essential element and, in common with other trace metals, copper has a number of physiological roles that may be classified as regulatory, structural and protective.  In the regulatory role they are an essential part of metalloenzymes, acting either as electron donors or acceptors at the active site, or by shaping the enzyme to the configuration necessary for its activity.  The structural functions of copper include, for example, membrane integrity or bone structure, and the protective function may involve antioxidant defence or the immune system (Ralph & Mc Ardle, 2001). Copper is involved in the reactions and functions of many enzymes, including angiogenesis, neurohormone release, oxygen transport and regulation of genetic expression.  Copper is an allosteric component of several enzymes that have oxidation and reduction activity, functioning as an electron transfer intermediate in redox reactions.

### Sources of Environmental Exposure

Copper is a natural elemental that can be found at background levels in water, sediment, soil and biota.

A summary of geological copper levels are available from e.g. Reimann et al. (2014a,b): copper is enriched in mafic rock (89-90 mg/kg), while intermediate rocks (20 mg/kg) and granite (12 mg/kg) contain less copper. Unmineralized sediments contain low copper levels (5-20 mg/kg). Black shales are enriched in copper (50 mg/kg).

European baseline levels of copper in water, sediment and soil are reported in the EU FOREGS Geochemical Atlas (Forum of European Geological Surveys). Typical background concentration levels in European surface waters range between 0.1 and 14 µg Cu/L, with a 10th-90th percentile range of 0.23-2.45 µg Cu/L. The median value of 0.88 µg Cu/L is in line with the dissolved copper background level around Europe of 0.8 µg Cu/L that was reported by Zuurdeeg (2002). For the OSPAR regions, dissolved background copper levels, estimated from off shore samples, range between 0.050 and 0.360 µg/L (OSPAR, 2001). For freshwater sediment, ambient baseline levels for copper range between 1.0 and 998 mg Cu/kgdw, with a 10th/90th percentile of 4 and 44 mg/kgdw and a 50th percentile of 14 mg Cu/kgdw, respectively. Soil baseline levels for copper in FOREGS are situated between 3 and 33 mg Cu/kgdw (10th and 90th percentile, respectively), with a 50th percentile of 12 mg Cu/kgwd. The EU Geological maps for Soils (Reimann et al., 2014a,b) reported similar copper concentrations: 10th and 90th percentile of 4 and 38 mg/kgdw), with a 50th percentile of 13-15 mg Cu/kgdw.  Similar concentrations were recorded for agricultural soils and grazing land (13-15 mg/kg in both).  Considerable concentration differences were noted between northern (median of 9.4 mg/kg) and southern Europe (median of 19 mg/kg).

Most important use-related copper releases are agricultural uses (copper sulphate added as feed additives and fertilizers, 39% of total EU emissions) and traffic (mainly from copper powder uses in brake pads, 43% of total EU emissions). Massive copper uses (wear of overhead wires, corrosion of copper tubes, fittings and taps and external building applications (roofs, gutters, down pipes, facades)) contribute to 15% of the total anthropogenic EU emissions. The relative contribution from waste incineration plants and landfill facilities is minor (0.4%). Other minor copper releases, that have been observed, include, among others, industry releases, domestic and industrial heating, fireworks and domestic wastewaters (excl. corrosion).

The anthropogenic input of copper and copper compounds to the environmental compartments have been attributed to agricultural soil: 38% of total emission; air: 31%; surface water: 18% and industrial soils: 15%.

Extensive datasets with ambient Cu-levels in the environment were received from different EU-countries (Austria, Belgium, Denmark, Finland, France, Germany, Ireland, Norway, Portugal, The Netherlands, Spain, Sweden, United Kingdom), and country-specific reasonable worst-case (RWC) ambient levels in freshwater were situated between 0.5 and 4.7 µg Cu/L (typical EU-value: 2.9 µg Cu/L). The RWC ambient level represents the 90th percentiles of ambient waters that are not directly affected by point source contamination (diffuse sources only). Similar data were derived for other environmental compartments: 1.1 µg Cu/L (min-max: 0.8-2.7) for coastal waters, 67 mg Cu/kgdw (min-max: 46-88) for river and lake freshwater sediment, 16.1 mg Cu/kgdw (min-max: 4-55) for coastal sediment, 24.4 mg Cu/kgdw (min-max: 7-40) for forest soils, 32.8 mg Cu/kgdw (min-max: 28-44) for grassland soils, and 31.2 mg Cu/kgdw (min-max: 16-58) for agricultural soils.

### Degradation and stability in water

In traditional terms, degradation (i.e., hydro-, photo-, and biodegradation) are not applicable to the inorganic Cu compounds which are assessed in this report.

Copper metal and inorganic copper compounds are not subject to degradation by hydrolysis. The soluble substances in this category will dissociate upon dissolution, and the less soluble category member will release copper ions in aqueous environments over time at rates dependent on both their properties and the characteristics of the receiving environment.

Once released to the environment, copper ions have more than one oxidation state and copper is thus characterized as transition metal. The principal ionic forms are cuprous (Cu(I), Cu+) and cupric (Cu(II), Cu2+). The trivalent form (Cu(III), Cu3+) occurs but is relatively unimportant in physical and biological systems. Cu+ is unstable in aqueous media and soluble Cu1+ ions readily transform to Cu2+ ions. This transformation of Cu+ to Cu2+ is a result of a redox reaction initiated through atmospheric water vapour as well as in aqueous solution. However, monovalent copper cations are only susceptible to such transformation when they are not chemically bound in insoluble compounds or stabilised in complexed forms.

Cu (II) is thus the most environmental relevant species. It is further recognised that Cu (II) ions - commonly named free cupric ions- are the most active copper species and that total Cu or Cu(II) concentrations are usually not directly related to ecological effects since exposure of biota may be limited by processes that render Cu unavailable for uptake (ICPS, 1998).

### Transport between Environmental Compartments

Transport and distribution of naturally-occurring elements such as copper over the different environmental compartments are determined by their solubility, their speciation, and their binding affinities to organic matter, sulphides and particles that occur in water, soil and sediment (e.g., clay particles, particulate organic carbon, precipitated hydroxides). Metal ions do not degrade over time but changes in speciation affects their toxicity profile. The presence of a metal in either a solution or solid phase is determined by its solubilization under environmentally relevant conditions, and its distribution coefficient.

After being released into the environment, the Cu(II) ions typically bind to inorganic and organic ligands contained within water, soil, and sediments. In water Cu(II) binds to dissolved organic matter (e. g., humic or fulvic acids). The Cu(II) ion forms stable complexes with -NH2, -SH, and, to a lesser extent, -OH groups in these organic acids. Cu(II) will also bind with varying affinities to inorganic (e.g. high for Mn/Fe oxides in clay, but low for aluminosilicates is salt) and organic components in sediments and soils.

Binding affinities depend on pH, oxidation-reduction potential and the presence of competing metal ions and inorganic anions).

Several publication in open literature have reported relevant partitioning coefficients for the different environmental compartments, and these have been summarized in Table 2.  The detailed overview of the Kds is provided in Annex III.

Table 2: Summary of the partitioning coefficients for different environmental compartments.

|  | Log Kd (l/kg) -50thP (range) |
| --- | --- |
| Freshwater - suspended solids/dissolved | 4.48 (3.3-5.8) |
| Estuarine - suspended solids/dissolved | 4.75 (3-6.5) |
| Marine - suspended solids/dissolved | 5.12 (3.7-5.9) |
| Freshwater -sediment solids/porewater | 4.39 (2.7-5.0) |
| Soil - solids/porewater | 3.33 * |

*Based on the data compilation from Sauvé et al., 2000.  An empirical regression was derived to predict Kd for Cu in soil:    Log Kd = 1.75 + 0.21pH(soil solution) + 0.51log(OC%) (R² = 0.42)

### Bioaccumulation

Aquatic environment

Bioconcentration/bioaccumulation factors (BCF/BAF)

For inorganic compounds, bioaccumulation factors are not applicable (Document ENV/JM/MONO(2001).

Indeed, for copper as well as for other essential metals, acclimation and homeostatic regulation mechanisms are induced after longer exposure times and these play a role in the copper essentiality/toxicity profile. Literature data demonstrated that the species’ homeostatic capacity allows an organism to regulate actively its metabolically required tissues concentrations and maintain optimal levels under varying external concentrations of the essential element (e.g.review papers of Adams et al., 2003).

Furthermore, some organisms accumulate copper in a non-bioavailable form by using copper binding and sequestration mechanisms as regulation system. These adaptation and regulation system play a role in the copper essentiality/toxicity profile and therefore, the BCF/BAF are not independent of exposure concentration and have no meaning for a hazard assessment.

A more mechanistic understanding of copper regulations of accumulations as well as internal copper binding mechanisms and sequestrations are provided by e.g. Borgmann, 1993 and Rainbow (1980, 1985, 1989).

Secondary poisoning - aquatic

Additional information of relevance to the absence of secondary poisoning is available from several publications (De Schamphelaere et al, 2004, Kamunde et al, 2005; Taylor, 2000; Kamunde, 2001; Hansen et al, 2004; Allinson, 2002; Nott,1994). The results are clearly relating copper toxicity to waterborne and not dietary exposure route. Importantly, the copper mesocosm studies from Roussel (2007) demonstrated a low sensitivity of the predating fish and did not show a concern from secondary poisoning.

Secondary poisoning of birds and mammals via fish or mussels was investigated for metals, including copper and reviewed by RIVM (Smit et al, 2000) who concluded that for copper it was not necessary to integrate secondary poisoning aspects into the copper assessment for the aquatic environment.

Aquatic biomagnification factors (BMF)

The absence of copper biomagnification, with consistent BMFs<1, was shown from several papers, covering seagrass, several benthic invertebrates, planktonic food chain and insects (Barwick and Maher, 2003; Farag et al., 1998; Weis & Weis, 1999; Wang, 2002; Quinn et al., 2003)

Terrestrial bioaccumulation

Terrestrial BCF and BAF and secondary poisoning and BMF

As for the aquatic environment, the data on metals bioaccumulation factors (BCF, BAF and Trophic Transfer factors (TTF)) do not support the use of BAF or BCF values when they are used as traditional generic threshold criteria for the hazard potential of inorganic forms of metals since they are not an intrinsic property. Therefore a fixed BCF, BAF is considered as not applicable for inorganic copper compounds.

The pattern of biomagnification of Cu in the terrestrial invertebrates’ food web was explored by Lakowski (1991). Based on 37 biomagnification factors representing herbivores, carnivores and detrivores, the slope of the linear regression was less than one suggesting regulation of Cu concentration. Other authors came to a similar conclusion, and some specifically stated that predatory invertebrates had high Cu concentrations only when their prey concentrations were high (Hunter and Johnson, 1982; Heikens et al, 2001; Wittassek, 1987).

Conclusion on bio-accumulation, biomagnifications and secondary poisoning

Available information in open literature demonstrates that copper is well regulated in all living organisms and that the BCF and BAF values have no meaning for a hazard assessment. The data also demonstrate that copper is not biomagnified in the aquatic or terrestrial ecosystems and consequently, secondary poisoning is not considered to be an issue for copper.

### Other Information on Environmental Fate

Copper compounds have variable degrees of solubility.  Copper sulphate and copper oxchloride are fully soluble under relevant environmental conditions (pH 5.5-8.5) and concentrations. CuO and Cu2O have a lower solubility. Copper metal (Cu°) is insoluble and needs to be transformed to its ionic forms (Cu+/Cu2+) to dissolve copper-ions. To define the basic hazard profile of sparingly soluble CuO and various copper forms, transformation/dissolution tests (in accordance to the OECD guidance, Annex 10 of the GHS) were therefore performed.

The study records are included in the IUCLID Section “Additional information on fate and pathways” and summarized below.

Transformation/dissolution of CuO

Transformation/dissolution of CuO was assessed at pH 6, 7 and 8 from transformation/dissolution tests, in accordance to the OECD guidance, Annex 10 of the GHS (Rodriguez et al., 2000).

Dissolved copper concentrations resulting from 7 and 28 days transformation/dissolution tests at 1mg/L at pH 6 are respectively 0.049 and 0.21 mg/L.   The results show 10 times higher copper releases at pH 6 compared to pH 7.  Copper releases at pH 8 are around detection limit for 1 mg/L loading (see also Table 3).

Transformation/dissolution of Cu°

Copper metal (Cu°) is insoluble and needs to be transformed to its ionic forms (Cu+/Cu2+) to dissolve copper-ions. Such transformation/dissolution takes place at the surface of the copper particles and, therefore copper-ion releases are related to the pH and surface area exposed .

Surface area and pH dependent releases of dissolved copper from Cu°

For pure copper metal materials, transformation/dissolution tests were carried out at pH 6, 7 and 8, in accordance to the OECD guidance (Annex 10 of GHS) (Rodriguez et al, 2007, 2008, 2011 and 2012)

Rodriguez et al. (2007) performed during 7 and 28 days Transformation/Dissolution (T/D) tests on copper wire pieces (99.9% purity) with varying diameters.  The data demonstrated that:

- Copper ion releases during transformation/dissolution tests are related to the surface area tested (Figure 4).

- Copper ion releases during transformation/dissolution tests are pH-dependent. At similar loading (100 mg/L), 5 times higher releases were measured at pH 6 compared to pH 7 and 8 (Figure 4).

- Higher stirring rates increased the intra-vessel coefficient of variation and was attributed to abrasion of the materials (Figure 5). Rodriguez et al., 2008 subsequently confirmed the abrasion issue from additional tests and measured 6 times lower releases with copper wire mounted on polypropylene discs (no abrasion) compared to the original classic test-set-up (with abrasion).

Figure 4: Copper releases during transformation/dissolution of copper wire pieces at various loadings and surface areas (Rodriguez et al., 2007)

Figure 5: Transformation-dissolution of copper (0.16 mm diameter wires) at pH 6.0, 100 mg/L loading for 24 hours, plotted versus the stirring rate (Rodriguez et al., 2007)

Rodriguez et al. (2011 and 2012) avoided abrasion with 2 non-abrasion devices: copper wire mounted on polypropylene discs and copper massive mounted in epoxy and polished. The epoxy mounted tests demonstrated the lowest coefficients of variation and are therefore preferred for further assessments/evaluations.

- Rodriguez et al., 2011 measured 19 µg dissolved Cu/L after 7 days transformation/dissolution of polished epoxy-mounted copper surfaces at relative high surface loading (43 mm2/L) and at pH 6, corresponding to a surface-specific releases of 0.14 µg Cu/ 7 days.mm2.  When using the copper wire mounted on polypropylene discs (at 47 mm2/L), they measured 6 µg dissolved Cu/L, corresponding to a surface-specific releases of 0.14 µg Cu/ 7 days.mm2.

- At a low surface area loading (0.67 mm2/L - epoxy-mounted), Rodriguez et al., 2012  measured 1 µg dissolved Cu/L (around detection limit) after 7 days T/D tests at pH 6 and 3.4 µg dissolved Cu/L after 28 days T/D test at pH 6. The 7 and 28 days release data at relative low mass loading therefore corresponds to a surface-specific releases of respectively 1.5 µg Cu / 7 days.mm2 and 5 µg Cu/ 28 days.mm2.

The surface area-specific releases can therefore be used to assess the copper released from various copper forms at various surface and mass loadings. The tests conducted at high surface loading have a slightly lower copper release per unit surface. A similar trend was also noted by Skeaff and Hardy (2005) (see below). For environmental classification, this shows that the most relevant choice is to use transformation-dissolution data obtained at a surface loading relevant to the copper form under consideration.

Releases from copper massive (> 1 mm, according to GHS guidance)

The surface loadings of copper spheres with diameter of 1mm, at mass loadings of 1, 10 and 100 mg/L, are calculated as 0.67, 6.7 and 67 mm2/L (Rodriguez et al., 2007).

- At low loading (e.g. 1 mg/L, 0.67 mm2 /L) and pH 6 the data from Rodriguez et al., 2012 can be used to assess the release from a copper sphere of 1 mm: around the detection limit (1 µg dissolved Cu/L) after 7 days T/D tests and 3 µg dissolved Cu/L after 28 days T/D test.

- At higher loading (43-47 mm2/L) and pH 6, 7 days release rates of 0.14 and 0.41 µg Cu/mm2 were measured (Rodriguez et.al, 2011). These data can be used to read across and assess the releases of dissolved copper from a sphere of 1 mm at 10 and 100 mg/L (surface loadings of 6.7 and 67 mm2/L): 0.9-2.7 µg dissolved Cu /L at 10 mg/L and 9-27 µg dissolved Cu /L at 100 mg/L.

The comparison of the pH-specific release data (Rodriguez, 2007) would indicate that at pH 7 and 8 dissolved copper concentrations would be 5 times lower.

Copper powder (10 µm to 1mm)

The surface areas of copper powders (10 µm – 1 mm), assumed to be spheres, theoretically range between 67.5 and 0.67 mm2/mg.  This surface area agrees well with the surface area of a fine representative powder (77.3% < 50 m), which was measured as 47 or 107 mm2/mg, depending on the analytical technique (measured by sieving and Brunauer–Emmett–Teller theory (BET)) (Skeaff and Hardy, 2005).

Skeaff and Hardy, 2005 conducted 168 hours (7 days) transformation/dissolution tests (pH 6) on these fine copper powders, at different loadings and reported a non-linear relationship between the surface loading and the dissolved copper concentration. They also reported suspected particle abrasion. The observed release of copper at 1 mg/L loading (7 days) was 82 µg/L.

Considering the observed particle abrasion in Skeaff and Hardy, 2005 and Rodriguez (2007 and 2008), the non-abrasive surface area-specific release rates (0.14-0.41 µg Cu/mm2 as measured by Rodriguez et al. 2011 at a surface area of 47 mm2/L), are considered as relevant for read-across to worst case copper powders (10 µm).

- At pH 6, 1 mg/L and 7 days TD, a copper release of 9–44 µg Cu/L is derived for very fine copper powders (surface loading 67 mm2/L). Releases of copper ions from powders at different specific surface area’s and different surface loadings (e.g. 10 and 100 mg/L) can be calculated accordingly using the most appropriate surface area-specific releases (Rodriguez et al. 2011 and 2012).

- At pH 6, 1 mg/L and 28 days TD releases are derived following linear extrapolation of the above retained 7 days transformation/dissolution test results: 37-176 µg Cu/L (pH 6).

The releases at pH 6 can be extrapolated to releases at pH 7 and pH 8 by considering the pH dependent difference in copper releases (Rodriguez et al., 2007).

Coated copper flakes (5µm to 100µm)

The surface coating of the flakes results in the formation of high particle-specific surface areas (2900 mm2/mg). Seven and twenty-eight days transformation/dissolution tests at pH 6 and pH 7 were conducted with a 1 mg/L loading (Schaefer, 2013), and the following copper releases were observed:

- At pH 6: at a loading of 1 mg/L, the dissolved mean copper concentrations at day 7 (acute) and at day 28 (chronic) were respectively 721 µg/l and 773 µg/l.

- At pH 7: at a loading of 1 mg/L, dissolved mean copper concentrations at day 7 (acute) and at

day 28 (chronic) were respectively 363 µg/l and 639 µg/l.

Conclusion transformation/dissolution of copper and copper compounds

For copper and copper compounds copper ions release is always higher at lower pHs. For CuO, the release is 10 times higher at pH 6 than at pH 7. For copper powders/massive the release is 5 times higher at pH 6 than at pH 7 and pH 8 (Rodriguez, 2007). It is only for coated copper flakes that the difference between releases at pH 6 and pH 7 is less important, after 28 days, the value at pH 6 being only 20% higher than the value at pH 7.

Table 3:  Comparative releases of copper-ions during transformation/dissolution tests (pH 6)

| Substance | pH | Mass loading (mg/L) | Surface loading (mm2/L) | µg soluble Cu/L |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | 7 days | 28 days |
| CuO (1) | 6 | 1 |  | 49 | 21 |
|  | 7 |  |  | 5 | 9 |
|  | 8 |  |  | <1 | 1 |
| Copper massive (>1mm) (non-abrasive devices) (2) | 6 | 100 | 43-47 | 6-19 |  |
| Copper massive (1 mm) (epoxy mounted) (3) | 6 | 1 | 0.67 | 1 | 3 |
| Copper massive (>1 mm) (read-across from non-abrasive devices) (2) | 6 | 1 | 0.67 | <1 |  |
|  | 6 | 10 | 6.7 | <1-2.7 |  |
|  | 6 | 100 | 67 | 9-27 |  |
| Fine Copper powders (10-50µm) (read-across from -across from non-abrasive devices) (2) | 6 | 1 | 67-107 | 9-44 | 37-176* |
| Coated copper flakes (4) | 6 | 1 | 2900 | 721 | 773 |
|  | 7 | 1 | 2900 | 363 | 639 |

*linear extrapolation from non-abrasive 7 days data

(1) Rodriguez et al., 2000 - Report available from the copper RAR (2008) (Annex K3)

(2) Rodriguez et al., 2011

(3) Rodriguez et al., 2012

(4) Schaefers and Klawonn, 2013

In conclusion, the information from solubility and transformation/dissolution tests with various copper-bearing materials therefore demonstrate important differences in potential copper releases: Coated copper flakes (77 % transformed/dissolved in 28 days) > CuO > copper powder > 1mm copper massive spheres (<0.3% transformed/dissolved in 28 days).

The results of comparing the bio-availability of the Cu-ions in the receiving compartment must therefore be integrated in effects and risk characterisations. The solubility and transformation/dissolution data can be used to predict the ecotoxic potential of copper containing materials using available ecotoxicity studies on soluble copper compounds and this is presented further in the environmental toxicity section. It should be noted that the evaluation of biocidal copper and copper compounds in the EU is still ongoing (2014—2017 and onwards) and was not included in this report.

## 2.3. Human Exposure

Exposure to copper arises mainly through ingestion of food and drinking water. Median daily dietary intake for adults is 1.2 mg/day. Typically, copper levels in drinking water are very low with an estimated median intake of 0.11 mg/day, less where bottled water is consumed. However, copper may leach from copper pipes into water depending on a number of factors including the physico-chemical properties of the water, the condition of the distribution network and standing time.

Young children also ingest dust by hand to mouth contact. This exposure may consist of a mixture of house dust and garden dust and is considered to be relevant to the exposure of children and adolescents. Exposure estimates are calculated using the IEUBK dust ingestion data and the copper in household dust data in UK cities with a median value of 250 mg/kg for the city of Birmingham. The assumed typical copper intake is between 3.4 and 21 µg/day (0-12yrs old).

It is assumed that a typical consumer is exposed via the dermal route to 0.14 mg Cu/day to coins and to 4.3x10-6 mg Cu/day via hair-care products and via the inhalation route to 0.001 mg Cu/person/day by smoking cigarettes. The contribution to dietary intake (copper of anthropogenic origin from fruit and vegetables, locally grown, and other foodstuffs) in the local environment was rather low due to the impact of industrial air pollution control measures and the effective homeostatic control of copper uptake by plants. External exposure through inhalation is even more limited.

## 2.3.1 Occupational Exposure

Workers can be exposed to dusts of copper substances during their manufacture, packaging and cleaning. Primary routes of exposure at the workplace are via inhalation and dermal contact.  Best available techniques and good hygiene measures are assumed to prevent/limit releases, dispersion and exposure to copper and copper compounds. At such workplaces direct exposure of the worker is reduced by risk management measures, such as automation or enclosure of the process or installation of exhaust ventilation systems. If such measures are not applicable – or where necessary, in addition to those measures - personal respiratory protective equipment is used.

The sectors that have been identified for estimating local exposures include: Smelting and refining, wirerod & cables production, casting billets and plates, production of semi finished copper shapes, production of copper powders and copper chemicals. For melting and casting of billets and further processing, respirable copper as a function of total copper ranges from 6-25% with a median value of 9%. For smelting a single value of 13% is given while for non-foundry operations in the manufacture of copper powders the single value of 4% is proposed.

## 2.3.2 Consumer exposure

Consumer exposure to copper may occur via dermal or oral routes or via inhalation. Dermal expo-sure occurs mainly through the use of toiletries and cosmetics, face cream and hair care products, through coin handling or jewelry. Additional dermal exposure is possible from the use of special paints or from copper containing wood preservatives and pesticides. Oral exposure (other than from food and water) occurs in particular by ingestion of dietary supplements containing copper, inhalation exposure occurs through cigarette smoke. Internal exposure may also occur with the use of intra-uterine devices. The contribution of inhaled air/dust is generally neglible.

Dermal route

- Use of face cream and hair care products

- Wearing of jewellery

- Handling of coins

- Use of special paints

- Copper-containing wood preservatives

- Pesticides

Inhalation route

- Cigarette smoke

Oral route (other than food and water)

- Dietary supplements containing copper

# human health hazards

Copper is an essential micronutrient, needed for optimal growth and development of microorganisms, plants, animals and humans. In the regulatory role they are an essential part of metalloenzymes, acting either as electron donors or acceptors at the active site, or by shaping the enzyme to the configuration necessary for its activity. Copper is also involved in the reactions and functions of many enzymes, including angiogenesis, neurohormone release, oxygen transport and regulation of genetic expression. Furthermore, copper is an allosteric component of several enzymes that have oxidation and reduction activity, functioning as an electron transfer intermediate in redox reactions.

NOTE: There are many studies in the public domain dealing with the repeat and chronic toxicity of copper compounds to several animal species. However, these studies do not meet the higher quality criteria and will therefore not be described in this report.

## Effects on Human Health

Copper is an essential trace element and its toxicokinetics are regulated by homeostasis. A review of copper toxicokinetics therefore requires consideration of the essentiality of copper and the mechanisms of copper homeostasis. The database on copper toxicokinetics is extensive. Studies included in this section were selected on the basis of (1) relevance to human health and (2) providing relevant information on current understanding of copper toxicokinetics and copper homeostasis.

### Toxicokinetics

The toxicokinetics of essential elements such as copper are regulated to a large degree by homeostatic mechanisms. Homeostasis can be described as the maintenance of a constant internal environment in response to changes in internal and external environments. Homeostatic maintenance requires the tightly coordinated control of copper uptake, distribution and efflux in cells and the organism as a whole. As a result of the presence of a homeostatic mechanism for copper, rat and human metabolism of copper are very similar and are therefore discussed together in the following sections.

Most of the data on toxicokinetics comes from studies in which copper was administered as copper sulphate, the most soluble of the copper substances. Read-across from copper sulphate to other less soluble substances is a conservative approach that may result in over-estimation of effects for poorly soluble substances.

#### Essentiality and homeostasis

Copper is an essential metal present in human body tissues and fluids at concentrations of parts per million or parts per billion. In common with other trace metals, copper has a number of physiological roles that may be classified as regulatory, structural and protective. In the regulatory role they are an essential part of metalloenzymes, acting either as electron donors or acceptors at the active site (key role in fundamental redox-reactions), or by shaping the enzyme to the configuration necessary for its activity. (Linder and Hazagh-Azam, 1996; Lindley 2000; Goldstein et al, 1979; Bordo et al., 2001; Opsahl et al, 1982; IPCS 1998). The structural functions of trace metals may be in, for example, membrane integrity or bone structure, and the protective function may involve antioxidant defence or the immune system. Copper is involved in the reactions and functions of many enzymes, including angiogenesis, neurohormone release, oxygen transport and regulation of genetic expression. Copper is an allosteric component of several enzymes that have oxidation and reduction activity, functioning as an electron transfer intermediate in redox reactions.

Major homeostatic control mechanisms, which help to maintain a constant internal environment in response to changes in internal and external environments, involve a regulatory loop between gastrointestinal absorption and biliary excretion via first-pass metabolism in the liver. Because of the homeostatis of body copper, indices of copper status remain stable except under extreme exposure conditions. Since copper is an essential trace element, it is necessary for the maintenance of health and an adequate dietary intake is needed to meet nutritional requirements. Studies conducted in young, healthy adult male volunteers indicate that a dietary intake of 1 mg Cu/day is sufficient to maintain copper balance and normal levels of some cuproenzymes and putative markers of copper status. With regard to chronic copper deficiency, a range of adverse outcomes has been demonstrated in animal studies including anaemia, neutropenia, bone abnormalities, neurological and cardiovascular effects. There are a number of hereditary diseases, which are a result of disorders in copper homeostasis. These diseases are rare and include Menkes disease, a deficiency disorder, and Wilson disease, a toxicity disorder (Mercer et al, 1993; Bull et al, 1993).

#### Oral absorption

In vivo studies in animals

A large quantity of oral absorption data are available for animals, specifically rats, and humans. Most reliable data are shown in Table 4. Van den Berg et al (1994, 1995) reported reliable apparent and true absorption values for orally administered copper in rats. Somewhat less reliable absorption data for rats were reported by Johnson (1988); reported true absorption values related only to a 3-4 day period, which occurred several days after administration of 67Cu and therefore may not reflect complete excretion of the administered copper. In addition, 67Cu was administered via intramuscular injection which may represent different toxicokinetics. Apparent absorption data have been reported in different other animal studies (Wolf et al, 1998; Rimbach et al, 1995; Yu et al, 1994; Johnson & Murphy, 1988; Johnson and Hove, 1986; Johnson and Flagg, 1986; Johnson and Gratzek, 1986; Johnson and Lee, 1988; Himmelstein, 2003). Overall the findings show that rats adjust to low copper diets by increasing absorption efficiency and reducing biliary excretion.  Increasing dietary copper levels are associated with decreased percentage absorption and increased faecal and endogenous excretion of copper. All these data enable an estimation of true absorption at the relatively high copper intakes used in toxicity studies (see Figure 6).

True absorption was determined as there is a large quantity of data available on the absorption of copper in animals and humans, predominantly relating to oral exposure. In these studies, quantitative data on the absorption of copper have been based on faecal monitoring, as faecal excretion is the main excretory route for copper. In several of these studies, the amount absorbed has been determined as the difference between oral intake and faecal excretion. This absorption value represents a measure of apparent absorption only, as faecal copper does not distinguish between unabsorbed copper and endogenous copper losses. Endogenous copper losses may occur from (1) biliary excretion of systemically-absorbed copper that mixes with the endogenous pool and is subsequently excreted, and (2) the fraction absorbed by intestinal mucosa and subsequently eliminated into the GI tract as cells are sloughed off (i.e. without systemic absorption). Apparent absorption thus represents a somewhat crude measure of copper absorption. In order to measure true absorption, which provides a more accurate measure of copper absorption following oral exposure, the percentage of copper intake recovered in the faeces was corrected for endogenous copper losses.

Figure 6: True absorption data from pooled animal studies (Van den Berg et al, 1994; Johnson, 1988; Van den Berg & Benyen, 1992; Johnson & Lee, 1988)

Based on these absorption data, an absorption factor of 25% is taken to be the best estimate of true absorption in rats at the high copper intakes.

Table 4: Summary of key animal studies reporting absorption data for dietary copper

| Reference | Animal model | Identity of copper | Dietary Cu |  |  | Duration of treatment/ measurement | Analytical method | Absorption of Cu (% of intake) |  | Other information |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Mg/kg feed | µg/d | Mg/kg bw/d |  |  | Apparent | True |  |
| Van den Berg et al., (1994) | Wistar rat (male) (n=3 to 6 rats/dietary group) | CuSO4 + 64Cu acetate (i.p. or oral) | 5.2 1.1 | (a) 74 (0-14d) (b) 88 (21-35d) (c) 80 (42-56d) 15 17 16 | 0.30 0.35 0.32 0.06 0.07 0.06 | Up to 56d (treatment) 4d (whole-body 64Cu) 14d (faeces collection) | Whole-body counting of 64Cu AAS analysis of feed/faecal Cu | 32 ± 2 31± 2 22 ± 3 46 ± 4 48 ± 3 53 ± 3 | 55 ± 5 56 ± 13 52 ± 10 (a) 72 ± 6 (b) 66 ± 3 (c) 58 ± 7 | Cu T1/2: 5.6 ± 0.4 (i.p.) 18.8 ± 1.1 (i.p.) |
| Johnson (1988) | Long-Evans rat (male) (n= 25 rats/dietary group) | NR + 67Cu (i.m.) | 6.15 0.42 | 113 7.6 | 0.45 0.03 | 4wk (treatment) 3-4d (faeces collection) | Radioactivity (liver/kidney & faeces) + isotope dilution AAS analysis of tissue/ faecal Cu | NR | 39 ± 7 32 ± 10 | Balance: 20 ± 7 μg/d 0.6 ± 0.5 μg/d |
| Van den Berg & Beynen (1992) | Wistar rat (male) (n=6 rats/dietary group) | NR+ 64Cu acetate (i.p. or oral) | 5.0 1.0 | 74 12 | 0.30 0.05 | Up to 26d (treatment) 2x3d (faeces collection) | Whole-body counting of 64Cu AAS analysis of feed/faecal Cu | 42 70 | 62 ± 2 NR | Cu T1/2: 4.7 ± 0.3 (oral) 5.2 ± 0.4 (i.p.) |
| Johnson and Lee (1988) | Sprague Dawley rat (male) |  | 0.4 1.7 3.5 5.4 10.6 21.1 | 7 33 63 92 180 403 |  | 5d (faeces collection) | AAS analysis of feed/faecal Cu |  | 46 40 41 30 23 22 |  |

Values reported are mean values ± (if specified) standard deviation

NR – not reported

In vivo studies in humans

The most reliable human data currently available on copper absorption following oral exposure come from volunteer studies. A number of different methods have been employed in these studies.  The most reliable and informative studies are considered to be those using the stable isotope tracers 63Cu and 65Cu, in conjunction with TIMS or ICP-MS analytical techniques.

Studies reported in this summary (Table 5) are limited to those which employed parallel administration of a faecal marker (polyethylene glycol (PEG), dysprosium or holmium), ensuring complete faecal collection, or where a minimum faecal collection period of 12 days has been used.  Key studies are provided by Turnlund et al (1998; 2005), which use the most reliable methodology currently available to determine both apparent and true absorption data for copper in humans, and by Turnlund et al (1989), which provides reliable apparent absorption data. Other reliable studies are by Harvey et al (2003; 2005), although these are considered to be of secondary importance to the Turnlund studies.

Turnlund et al (1989) determined copper absorption and retention (balance) in eleven young men at three levels of dietary copper. The outcome of these study indicated that similar homeostatic mechanisms operate in humans as in animals, and that absorption appears to represent the initial point of regulation of copper. Data from this study also indicate that prolonged rebound effects following depletion/repletion events are unlikely.  Turnlund et al (1998) further provides reliable apparent absorption data for copper in young men and the best available estimates for true absorption that are currently available. The study also showed that regulation of absorption and endogenous losses are both involved in protecting the body against copper deficiency, although it was noted that these controls did not prevent subjects being in negative balance during the early part of the low-copper periods. In a second study, Turnlund (2005) provided reliable true absorption data associated with relatively high, long-term copper intakes and important findings on copper retention and homeostasis.

Harvey and Majsak-Newman (2003) reported apparent absorption rates of 41-45% and true absorption rates of 45-48% in twelve adult males receiving dietary copper intakes of 0.7, 1.6 and 6.0 mg/d, but the lack of any significant effect of copper intake on absorption (apparent or true) and the similarity between apparent and true absorption values at each dietary level are inconsistent with findings of Turnlund et al (1989; 1998; 2005) and available animal data (e.g. Van den Berg et al, 1994).

In Harvey and Majsak-Newman (2003) the methodology used was less well validated and therefore not retained in the database and the true absorption data of Turnlund et al (1989; 1998; 2005) were therefore retained and integrated in exposure related absorption equations (Figure 7).

Equation 1:  oral absorption% = -15.04 ln(x) + 63.176

Equation 2:  oral absorption% = 72.287 e-0.1167x

Figure 7: Oral copper absorption - human data

Conclusion on oral absorption

Oral absorption data for humans and rats presented in Figure 6 and Figure 7 show qualitative and quantitative similarities between the two species. In both species, absorption of copper over the range of intakes studied is inversely related to copper intake, illustrating the important role of absorption in copper homeostasis. In both species, true absorption of copper from diets containing what are considered as adequate levels of copper (1-10 mg/day in humans; 0.3-0.6 mg/kg bw/day in animals) is in the 30-60% range. The above oral absorption data, and corresponding functions, are based on copper sulphate. Assuming that orally-administered copper will occur in the GIT, at least in part, in the ionic form and therefore be available for absorption, and in view of the solubility of copper sulphate, it is considered appropriate to adopt a conservative approach and to use the oral absorption data for copper sulphate for other less soluble copper species.

Table 5: Summary of human studies reporting absorption data for copper from Cu-containing diets

| Reference | Study population (age) | Dietary copper (mg/d) | Stable isotope (route) | Duration of treatment or measurement | Analysis | Absorption (% of intake) |  | Cu balance a (mg/d) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Apparent | True |  |
| Turnlund et al (2005) | 9 males (26 – 49y) | (a) 1.6 (b) 7.8 | 63Cu (oral in diet or i.v.) | Treatment (a) 18d; (b) 129d; (c) 18d | ICPMS | a) 29 b) 16 | 40 29 | a)0.06 b)0.67 |
| Turnlund et al (1998) | 11 males (26 ± 4y) | (a) 0.66 (b) 0.38 (c) 2.49 | 65Cu (oral in diet or i.v.) | Treatment: (a) 24d; (b) 42d; (c) 24d 12d (faeces collection) | TIMS (total Cu & 65Cu) | (a) 54 (b) 67 (c) 44 | 73 77 66 | (a) -0.13 (b) -0.015 (c) [511] |
| Turnlund et al (1989) | 11 males (22-35y) | (a) 1.68 (b) 0.79 (c) 7.53 | 65Cu (oral in diet) | Treatment: (a) 24d; (b) 42d; (c) 24d 12d (faeces collection) | AAS (Cu in diet/faeces) TIMS (65Cu in diet & faeces) | (a) 36.3 (b) 56.2 (early) 55.0 (late) (c) 12.4 | NR | (a) 0.17 (b) 0.002 (c) 0.94 |
| Harvey and Majsak-Newman (2003) | 12 males (32y ± 11) | 3 dietary periods: 0.7 1.6 (c) 6.0 | 65Cu (oral in drink) | Treatment: 42d each dietary period, separated by >4 wks. 14d (faeces collection). | ICPMS | (a) 41 (b) 42 (c) 45 | 48 45 48 | (a) 0.13 (b) 0 (c) 0.75 |
| Harvey et al (2005) | 6 males (mean 39y; range 34-57y) | 1.4 | 65Cu (oral or i.v.) | 14d (faeces collection) 7d (urine collection) | ICPMS | 33 ± 3 | 49 ± 4 (oral) 48 ± 5 (i.v.) 8 ± 2 (simple model) | NR |

NR - not reported.  Absorption and balance values represent mean values

#### Dermal absorption

Skin penetration by copper has been reviewed by Hostynek and Maibach (2003).  Available data on this subject show that copper compounds and copper metal are capable of penetrating the stratum corneum of human skin. With regard to dermal absorption of copper, two in vitro studies using human skin provide the best data currently available (Table 6). The copper compounds were applied in an aqueous medium (slurry). Roper (2003 – unpublished) reports dermal absorption rates near zero and dermal delivery rates of <0.1% for both copper (I) and dicopper chloride trihydroxide. It should be noted that this study has several limitations, including the lack of mass balance data (applied dose of copper is not fully accounted for) and large standard deviations in the absorption and delivery data. Cage (2003 – unpublished) reported dermal absorption rates of <0.2% for copper (I) oxide and copper sulphate, and <0.1% for copper powder. Similar to the Roper (2003) study, this study was limited by the absence of mass balance data and the occurrence of fairly large standard deviations. In addition, data concerning copper retained in the skin was also lacking. It is notable that absorption values reported in the Roper study are lower than those reported by Cage. For example, values for copper (I) oxide are 0.001% and 0.157%, respectively.

Table 6: Summary of reliable in vitro dermal absorption data

| Reference | Substance | Guideline and analytical method | Applied dose (mg Cu/cm2) | Absorption |  | Dermal delivery* |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | % of applied dose | Ng/cm2 | % of applied dose | Ng/cm2 |
| Roper (2003) | Copper oxychloride | OECD 428 ICPMS | 3.34 | 0.002 ± 0.01 | 82.98 ± 316.07 | 0.08 ± 0.06 | 2601.4 ± 1870.2 |
| Roper (2003) | Copper (I) oxide | OECD 428 ICPMS | 5.4 | 0.001 ± 0.01 | 55.56 ± 397.7 | 0.06 ± 0.04 | 3074.1 ± 2261.6 |
| Cage (2003) | Copper (I) oxide | OECD 428 AAS | 1.05 | 0.157 ± 0.026 | 1647 | ND | ND |
| Cage (2003) | Copper powder | OECD 428 AAS | 0.974 | 0.038 ± 0.006 | 372 | ND | ND |
| Cage (2003) | Copper sulphate | OECD 428 AAS | 0.942 | 0.136 ± 0.108 | 1277 | ND | ND |

*Dermal delivery was determined as the sum of copper in receptor fluid and stripped dermis; mean values were 3074 ng/cm2 ± 2261 and 2601 ng/cm2 ± 1870, respectively (reported as 0.06% ± 0.04 and 0.08% ± 0.06 of the applied dose, respectively).

Quantitative dermal absorption data for humans are also available from two published studies by Pirot et al (1996a; 1996b). Their research showed that penetration resulting from topical formulations (emulsions or ointments) with CuSO4 showed some dependence on vehicle.

Conclusion on dermal absorption

No data are available on dermal absorption of copper in animals. Based on the Roper (2003) and Cage (2003) studies, a dermal absorption factor of 0.3% is derived for insoluble and soluble copper substances in solution or suspension. This value is derived from the highest value for copper measured in receptor fluid (0.157% for copper (I) oxide reported by Cage (2003), plus a value for copper retained in the skin (mean of 0.07% and 0.05% reported by Roper (2003).  This results in a dermal absorption value of 0.217% ± 0.06. As a conservative approach this value was rounded up to 0.3%.

For dry exposure scenarios, consistent with the approach used in the OECD Cooperative chemicals Assessment of zinc (and EU risk assessment), a 10-fold lower dermal absorption value is proposed (0.03%). In the absence of data to indicate whether copper retained in the skin will become bioavailable or not, the inclusion of copper retained in the skin can be considered as a worst-case approach.

#### Inhalation

Published studies that provide substantial relevant information on uptake of copper via inhalation were not identified. Contradictory results on the uptake of copper by humans due to smoking were reported by Fisher et al (1990) and McMaster et al (1992). The former concluded that smoking had no effect on serum copper or other indices of copper status among male or female subjects of all ages. McMaster et al (1992), however concluded that smoking increased serum copper levels incrementally. Other inconclusive studies on this route of exposure (i.e. via smoking) were conducted by Tsuchiyama et al (1997) and Takemoto et al (1991). Srivastava et al (1984) evaluated systemic absorption of copper (in suspended fly ash) from the respiratory tract was investigated in thirty male Wistar rats. This study demonstrates the potential for systemic absorption of copper following inhalation, but provides no reliable quantitative data.

In the absence of relevant inhalation absorption data, the inhalation absorption can be calculated using the Multiple Path Model of Particle Deposition (MPPD) developed by RIVM, and the particle size distribution data of the copper and copper compounds. The fractional and regional deposition of copper in the respiratory tract is modelled using the MPPD model (v1.0) (Asgharian & Freijer, 1999). For these calculations, the following model assumptions were used in assessing conditions reflective of workplace conditions (Table 7):

Table 7: MPPD model parameters using the Respicon particle size data

| Airway morphometry |  | Human Yeh Schum symmetric model |
| --- | --- | --- |
| Particle density | 1 g/cm |  |
| Particle diameter | MMAD = 4 µm (respirable) 10 µm (tracheobronchial) 50 µm (extrathoracic, nominal) |  |
| Inhalability adjustment | Off |  |
| GSD | 2.0 |  |
| Exposure conditions | Constant |  |
| Aerosol concentration | 1000 µg/m |  |
| Breathing mode | Oronasal normal augmenter |  |
| Shift breathing volume | 10m/8h* |  |
| Breathing frequency | 18 breaths/min |  |
| Tidal volume | 1150 ml |  |

* Occup. breathing volume defined by ICRP as 9.6m/h shift, composed of 7h light exercise, plus 1h heavy exercise

Copper deposited in the alveolar region was assumed to be 100% absorbed (conservative default), and copper deposited in the upper respiratory tracts (ET and TB fractions) was assumed to be translocated to the gut (an overestimation, other clearance mechanisms such as expulsion are more relevant there). Based on these data and the information provided in Table 8, the MPPD model was used to predict fractional deposition behaviour in the human respiratory tract for workers.

Table 8: MPPD model predictions of deposition behaviour in the respiratory tract, and derivation of proposed inhalation absorption factors for five different copper compounds using the particle size distribution data collected in the dustiness test.

ET =Extrathoracic; TB= Tracheobronchial; PU=Pulmonary

| Substance | CAS | rel. density [g/cm³] | D50 [µm] MMAD(1) | D50 GSD(1) | Predicted fractional deposition |  |  | Inhalation absorption(2) [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ET | TB | PU |  |
| Copper powder | 7440-50-8 | 5.9 | 71.7 | 3.9 | 49.2 | 0.9 | 1.2 | 14 |
| Copper (I) oxide | 1317-39-1 | 6.3 | 9.9 | 3.3 | 67.5 | 1.4 | 3.8 | 21 |
| Copper (II) oxide | 1317-38-0 | 2.3 | 60.7 | 3.8 | 49.4 | 0.9 | 1.1 | 14 |
| Copper (II) sulphate pentahydrate | 7758-99-8 | 3.6 | 90.3 | 5.2 | 48.6 | 1.0 | 1.2 | 14 |
| Dicopper chloride trihydroxide | 1332-40-7; 1332-65-7 | 8.9 | 12.2 | 4.1 | 56.2 | 1.4 | 3.5 | 18 |

(1): MMAD/GSD values above the linear range of the model are reset by default to max. values of 20 µm and 4.0 (GSD); (2): rounded values

The predicted inhalation absorption factors vary between 14 and 21%. These values may be considered as conservative for copper powder, copper (II) oxide and copper sulphate pentahydrate. Absorption rates similar to those of skin (i.e., 0.3%) are commonly considered appropriate. As a consequence, the predicted absorption factors of 14% entail an overestimation in assuming full translocation to the GI tract.

Conclusion on inhalation absorption

In absence of relevant systemic absorption data of copper, the MPPD model can be used to predict fractional deposition behaviour in the human respiratory tract.

#### Distribution

Distribution and metabolism in animals

Marceau et al (1970) observed that plasma transport of copper in rats is a saturable process (decline of Cu-levels at high doses). Experiments conducted by Weiss and Linder (1985) support the generally accepted two-stage model of copper transport. The liver is the major organ involved in copper distribution and exerts homeostatic control over extra-hepatic copper by regulating its release (Harris 2000).

Distribution and metabolism in humans

Results from studies investigating copper levels in human tissues and organs are summarised in Table 9.  The data indicate that liver and brain are the organs containing the highest concentrations of copper.

Summary of distribution and metabolism

On entering interstitial fluid and blood plasma, absorbed copper initially becomes bound to two proteins; albumin and transcuprein. Most of the copper bound to albumin and transcuprein is rapidly transported via portal blood to the liver. Once in the liver, copper is incorporated into ceruloplasmin, which is subsequently released into the systemic circulation for delivery to other tissues.

Table 9: Summary of copper levels in human tissues

| Reference | Liver | Heart | Kidney | Adrenals | Pancreas | Spleen | Intestine | Muscle | Rib | Lung | Brain | Testis | Ovary | Skin |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cartwright & Wintrobe (1964) | 5.1 (5) | 3.0 (5) | 2.0 (5) |  |  | 0.8 (5) |  | 0.9 (5) |  |  | 6.3 (5) |  |  |  |
| Hamilton et al (1972) | 14.7±10.3 (7) |  |  |  |  |  |  | 0.7±0.1 (6) |  | 2.2±0.7 (11) |  | 0.8±0.5 (5) | 1.2±0.7 (6) |  |
| Sumino et al (1975) | 9.9±5.5 (30) | 3.3±0.7 (29) | 2.6±0.4 (29) | 2.1±0.6 (24) | 1.5±0.3 (20) | 1.2±0.2 (20) | 0.5 (26) | 0.9±0.3 (27) | 0.5±0.2 (8) | 1.3±0.2 (30) | 6.2±1.2 (17) | 0.9±0.2 (11) | 1.0±0.3 (13) | 0.7±0.3 (26) |
| Bem et al (1988) | 5.3±1.9 (11) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Xia et al (1989) | 7.9±4.3 (20) | 2.3±1.7 (20) | 3.2±1.8 (20) |  | 1.4±0.6 (20) | 1.4±0.7 (20) |  | 1.0±0.4 (20) | 2.7±1.6 (20) | 1.5±0.8 (20) |  |  |  |  |
| Takemoto et al (1991) |  |  |  |  |  |  |  |  |  | 2.6±1.2# (2274) |  |  |  |  |
| Bem et al (1993a) | 4.5 (75) |  | 2.2 (75) |  |  |  |  |  |  |  |  |  |  |  |
| Bem et al (1993b) | 4.0±2.1 (79) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Piotrowski et al (1996) | 4.6±2.2 (60) |  | 2.3±1.2 (60) |  |  |  |  |  |  |  |  |  |  |  |
| Pinheiro et al (1997) | 4.8±0.7 (8) |  | 2.2±0.5 (8) |  |  |  |  |  |  | 1.5±0.4 (8) |  |  |  |  |
| Tsuchiyama et al (1997) |  |  |  |  |  |  |  |  |  | 0.9±0.2# (17) |  |  |  |  |
| Treble et al (1998) | 3.7±1.8# (71) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Copper concentrations are presented as μg Cu/g wet tissue.  Values presented are mean values ± standard deviation.  Number of subjects is in parentheses.

#Copper concentrations in wet tissue estimated from reported dry tissue values using conversion factors in Pinheiro et al (1997)

#### Excretion

Excretion in animals

Quantitative data on excretion were reported in the bioequivalence study by Himmelstein (2003 – unpublished). This OECD 417 guideline study showed faecal excretion to be the main route of elimination for orally-administered copper, with urinary excretion as a relatively minor route. Biliary copper is recognised to be the main excretory route for orally-administered copper in humans and animals, resulting in measurement of faecal copper in many toxicokinetic/absorption studies. The fate of excess copper was examined in bile-cannulated male Sprague Dawley rats (five per group) following oral administration of a single dose of copper (nominal dose 20 mg Cu/kg BW; actual dose 22-24 mg Cu/kg BW). The substances investigated were copper (I) oxide, copper sulphate, dicopper chloride trihydroxide, copper hydroxide, tribasic copper sulphate and Bordeaux mixture. Copper levels in excreta during the 24-h period after dosing were as follows: bile 1.54-2.48% of dose; urine 0.20-0.39% of dose; faeces 64-76% of dose (although it is noted that faecal copper will also comprise some absorbed copper). Values were found to be similar for all six substances tested. The results showed faecal excretion to be the main route of elimination for orally-administered copper, with urinary excretion as a relatively minor route.

No quantitative data appear to be available for other routes of copper excretion (saliva, sweat and hair) in animals.

Excretion in humans

Many studies reporting on the toxicokinetics of copper, specifically its absorption, have measured faecal copper and biliary copper is recognised to be the main excretory route for orally-administered copper in humans and animals.  Turnlund et al (2005) noted that other excretory routes, such as urine, saliva, sweat and integumentary losses, are minor routes of excretion for copper in humans. This was further confirmed in other studies by the same authors (Turnlund et al, 1990, 1997) when evaluating urinary and salivary copper levels as well as copper levels in sweat.

Summary of excretion

Biliary excretion, with subsequent elimination in the faeces, represents the main route of excretion for copper in humans and animals. Copper is excreted into the bile in a relatively inabsorbable form. Consequently, little enterohepatic absorption takes place.

#### Comparative biology

In mammalian toxicity, it is considered that the most toxic form of any copper salt is the Cu2+ ion. This can be shown through the comparison of the most soluble (e.g. copper sulphate, copper nitrate) and relatively insoluble copper salts, where the solubility, bioavailability and hence toxicity of these salts can vary – with copper sulphate representing the worst-case scenario. As all suitable short- to long-term available animal copper toxicity studies are derived from oral administration, the use of copper sulphate data would represent a worst-case scenario for the determination of the effect of relatively insoluble copper compounds in mammalian toxicity.

In vivo bioavailability

Himmelstein (2003 – unpublished) provides relevant information on copper absorption in animals. In this study, absorption, distribution and excretion of several copper substances were investigated in thirty-five male Sprague Dawley rats following oral administration. The copper substances investigated were copper (I) oxide, copper sulphate and dicopper chloride trihydroxide, copper hydroxide, tribasic copper sulphate and Bordeaux mixture. The results indicate that absorption, distribution and excretion of a single, orally administered dose of 20 mg Cu/kg BW in male rats was similar for each of the six copper substances tested. Using copper hydroxide as the representative copper compound, plasma copper levels remained fairly stable during 48h post-dosing for treated and control. Liver copper levels increased rapidly after dosing, peaking at 12h, and returning to control levels by 48h. In addition to the study by Himmelstein (2003), several other studies have investigated comparative bioavailability of copper substances, albeit focusing on agricultural animals (cattle, pigs, sheep, poultry), leading to consistent and reproducible results (Kegley and Spears, 1994, Cromwell et al, 1989; Baker et al, 1991; Aoyagi and Baker, 1993; Buescher et al, 1961; Allen et al, 1961) (Table 10). Overall, these studies demonstrate that copper (II) oxide administered in feed has poor bioavailability compared to copper sulphate, and that copper (I) oxide is similar to copper sulphate. The low bioavailability of copper in copper (II) oxide, relative to that of copper in the sulphate salt, was also demonstrated in the rat following administration at adequate dietary levels (Rojas et al, 1996). In addition, the studies have measured copper levels in the most important organ and body fluid in determining copper absorption from the gastro-intestinal tract, namely the liver and bile.

Table 10: Relative bioavailability of supplemental copper sources

| Source of copper | Species |  |  | References (and applicable species) |
| --- | --- | --- | --- | --- |
|  | Poultry | Swine | Cattle |  |
| Copper sulphate | 100 | 100 | 100 | Aoyagi and Baker, 1993 (poultry) Baker et al, 1991 (poultry) Buescher et al, 1961 (swine) Cromwell et al, 1989 (swine) Kegley and Spears, 1994 (cattle) |
| Copper (II) oxide | 0 (3) | 30 (4) | 15 (2) |  |
| Cuprous oxide | 95.2 (2) |  |  | Aoyagi and Baker, 1993 (poultry) Baker et al, 1991 (poultry) |
| Basic copper carbonate | 113(1) | 60 (1) |  | Allen et al, 1961 (swine) Aoyagi and Baker, 1993 (poultry) |
| Copper carbonate |  | 95 (1) |  | Buescher et al, 1961 (swine) |

Average numbers rounded to the nearest ‘5’ and expressed relative to response obtained with copper sulphate. Number of studies or samples involved indicated within parenthesis.

In vitro bioavailability

The comparative bioavailability between copper salts can also be observed by the dermal route when evaluating the in vitro dermal penetration study of Cage (2003). Findings are presented in Table 11. These data indicate that it is possible to read-across toxicity data on copper sulphate as a worst case in comparison to copper (I) oxide, copper thiocyanate and copper powder. In addition, it will be possible to calculate the route-to-route exposure from available oral toxicity studies on copper sulphate and using dermal penetration studies on insoluble copper compounds. Therefore an equivalent but realistic determination of dermal toxicity may be derived from available sub-chronic oral exposure studies on copper sulphate, permissible systemic copper levels and in vitro dermal penetration studies on copper sulphate and insoluble copper compounds (NTP, 1993; Cage, 2003).

Table 11: Comparative bioavailability between copper salts (data from Cage, 2003)

| Product name | % applied dose | Ratio |
| --- | --- | --- |
| Cuprous oxide composite | 0.157 | 1.15 |
| Cuprous thiocyanate | 0.126 | 0.92 |
| Coated copper flakes | 0.038 | 0.28 |
| Cupric sulphate | 0.136 | 1.00 |

Rodriguez et al, 2010a, assessed the release/dissolution of copper ions from copper materials and copper compounds in biological fluid, simulating oral exposure. The results are summarized in Table 12, and demonstrate the highest solubility of CuSO4 and CuCl. Comparison of the results for CuO from the in vitro gastric biosolubility tests () with in vivo bioavailability studies (Table 10) demonstrate the conservative nature of the gastric biosolubility tests.

Table 12: Relative bio-accessibility of copper and copper compounds, assessed from the recovery of copper after bioelution tests in gastric fluids (pH 1.5, 2 hours) in accordance to ASTM D 5517-07

| Material Tested | Composition | Bioelution recovery (as % of Cu content) |
| --- | --- | --- |
| Cu massive | >99.9% Cu | 0.096 – 0.105 |
| Cu powder | 99.7% Cu, 0.3% Cu2O | 1.1 – (7.3*) |
| Cu flake – biocidal product | 93.7% Cu, 2.6% Cu2O, 3.9% LOI** | 42 – 71 |
| Cu flake – non-biocidal product | 96.3% Cu, 1% Cu2O, 2.8% LOI** | 44 – 60 |
| CuO | 79.89% Cu | 68 – 84 |
| CuCl | 63.78% Cu | 77 – 94 |
| CuSO4 | 25.45% Cu | 100 |

*The results at the higher loading rate show unacceptably high variability (CV of 66%), possibly related to abrasion of the particles during the test.  The results of this test are therefore not considered as reliable.

** Loss if ignition, as a measure of the organic content

For the Cu° materials the data in Rodriguez et al, 2010 further show (Figure 8) that the relation between the release of Cu-ions and the surface area exposed is well described by a power function:

Bioaccessible copper ions = 1.94 surface loading (mm2/L) 0.84

Figure 8: Release of copper with different amounts of surface exposed to the gastric mimetic fluid (Square = copper powder tested at 5 g/L. excluded from the regression because of high variability among replica’s (see text))

Conclusion

The in vitro gastric bioelution tests with various copper-bearing materials demonstrate important differences in bio-accessibility in gastric fluids with CuS04 (100%) being the most bio-accessible copper compound and copper massive the least bio-accessible one. Bio-accessibility of Cu° is related to surface area. Using the coated copper flakes as reference substance, the relative bio-accessibility of a typical copper powder (0.024 m2/g) and reasonable worst-case 10 µm powder (of 0.067 m²/mg) are therefore respectively 1.9% and 4.9% (data taken from section 2.2.7). These data can be used to predict the acute oral toxicity of copper and copper powder using available toxicity studies on other forms of copper and copper compounds and this is presented further in the acute toxicity section.

#### Conclusion on toxicokinetics

Copper is an essential metal present in human body tissues and fluids at concentrations of parts per million or parts per billion. From the toxicokinetics studies, a value of 25% oral absorption was chosen for risk assessment purposes. Dermal absorption factors of respectively 0.3% and 0.03% are proposed for wet and dry exposure scenarios.

Comparative bioavailability, solubility and toxicity studies have shown that relatively insoluble copper and sparingly soluble copper (II) oxide are less bioavailable than the more soluble copper salts, e.g. copper sulphate pentahydrate and copper (I) oxide. The available toxicological information on copper-sulphate, copper (II) oxide and copper are used in combination with the toxicokinetics to allow for read- across where necessary.

### Acute Toxicity

#### Inhalation

Studies in Animals

Several high quality studies on copper compounds and copper are available. Three additional studies on coated copper flakes were submitted thereafter.  The summary of the acute inhalation toxicity data in animals is provided in Annex IV.

Acute inhalation toxicity data are available for copper (I) oxide, dicopper chloride trihydroxide and copper sulphate. Published data on the acute inhalation toxicity of copper compounds in animals are limited to two studies (Skornik and Brain, 1983; Drummond et al, 1986). Seven unpublished reports of acute inhalation toxicity tests in rats are also available, five using cuprous (I) oxide, one using dicopper chloride trihydroxide and one using copper sulphate.  A summary of these studies are presented in Annex IV.  Key studies in this section are considered to be those of Greenough and McDonald (1985 a and b), Blagden (1999), Wesson (2003) and Holbert (1994), which were conducted in accordance with OECD (No. 403) Test Guideline.

Coated copper flakes

Coated Copper flakes were found to be non-toxic to rats with an LC50 >5.11 mg/L (Leuschner, 2011). However, Wesson (2001) reported that coated copper flake was toxic by inhalation and reported LC50 (4 h) of 1.03 mg/L air (analytical) (male/female combined) in rats [LC50 (4 h): 0.733 mg/L air (analytical) (male); LC50 (4 h): 1.67 mg/L air (analytical) (female)}.

Both studies used copper flakes, coated with aliphatic acid. For the Leuschner (2011) study the MMAD (3.477 µm) and GSD (2.61-2.77) were within the required OECD specifications. In the Wesson (2001) study a particle size MMAD of around 5 µm (4.49-5.86 µm, geometric mean standard deviation (GSD) 2.73-3.25) was tested, but with a proportion of very fine particles present,).  The test concentrations in the Wesson (2001) study were not within the guideline requirements: all exceeded the stated ±15 or ±25% of the mean achieved concentration at various time points during the exposure period, and were also high (15-70% above the mean value) at the beginning of the study.   In making a final decision on the classification of coated copper flake, the uncertainties around the Wesson study not considered sufficient to dismiss the results and coated copper flakes are considered hazardous by the inhalation route, according to GHS.

Copper sulphate

An acute inhalation toxicity study for copper sulphate has been made available (Holbert, 1994 – unpublished), and an LC50 of >2.95 mg/L was reported for males and females.  Adverse effects of inhaled CuSO4 on male Syrian golden hamsters (Skornik and Brain, 1983) and CD1 mice (Drummond et al, 1986) were also investigated (endpoints: abundance and function of pulmonary macrophages, respiratory responses to bacterial challenge). Taking into consideration all available data, it was concluded that copper sulphate pentahydrate is not toxic by inhalation.

Copper (I) oxide

The two most reliable inhalation rat studies with copper (I) oxide resulted in a 4h-LC50 of 5.63 mg/L for rats (Greenough and McDonald, 1985b – unpublished), and in a 4h-LC50 of 3.34 mg/L (Blagden 1999 – unpublished). Endpoints included respiratory depression during the nose-only exposure, lung effects (enlargement, discolouration and presence of fluid) and some effects in the kidneys, liver and intestines. Difference between these two values may be attributed to the different particle sizes used in the two studies. Exposure by inhalation to smaller particles would be expected to be associated with greater toxicity and a lower LC50 value. A less reliable study (lack of proper description on test substance properties) resulted in a 4h- LC50 of >5 mg/litre (Fulfs 1990 – unpublished. Other reported 4h-LC50s for rats was >30 mg/L (Dickhaus 1988b - unpublished).

Taking the results of the Blagden (1999) study as representing the worst case, animal studies indicate that copper (I) oxide is hazardous by the inhalation route.

Copper (II) oxide

Acute inhalation toxicity of copper (II) oxide is determined to a large extent by particle size and solubility. Copper (II) oxide and copper powder are sparingly soluble substances. Experimental particle size distribution data (reported in an unpublished study) showed that <5 % of the samples tested had a particle size of 10 μm or below (Battersby, 2004). Therefore, based on physical and chemical properties, copper (II) oxide is considered not toxic by inhalation.

Dicopper chloride trihydroxide

An acute whole body inhalation 1h-LC50 for dicopper chloride trihydroxide was reported to be > 11.4 mg/L (Holbert, 1993 - unpublished). Observed effects included discoloration and swelling of lungs, discoloured mucous abdomen and discoloured contents of the GI tract. Another acute nose-only inhalation toxicity study with rats, fully conformed to OECD 403 Test Guidelines, was conducted by Wesson (2003 – unpublished) and resulted in 4h-LC50 of 2.83 mg/L for males and >2.77 mg/L females (combined: 4.74 mg/L). Observed effects were lung abnormalities. The data for dicopper chloride trihydroxide show that dicopper chloride trihydroxide is hazardous by the inhalation route.

Copper powder

Acute inhalation toxicity of copper powder will be determined to a large extent by particle size and solubility. Based on physical and chemical properties (Battersby 2004 – unpublished study).

Considering the low solubility and bioaccessibility and the higher particle size of copper powders, compared to coated copper flakes and copper (II) oxide and copper sulphate, the available information leads to the conclusion that copper powder (and copper massive) is not hazardous by the inhalation route.

Little information is available on acute effects in humans and inhalation of copper-containing materials. Borak et al (2000) extensively reviewed seven reports metal fume fever (MFF) –related effects and its possible association with copper exposure, but none of the seven studies provided adequate exposure data, qualitative or quantitative, to enable identification of the causative agent(s) associated with the reported symptoms. Therefore there is insufficient evidence to conclude that exposure to copper dust or fume is hazardous.

#### Dermal

Studies in animals

No published data are available on the acute dermal toxicity of copper compounds in animals.  Unpublished reports of acute dermal toxicity tests for several copper compounds and for coated copper flakes, which have been conducted according to OECD (No. 402) guidelines, have been made available. This type of test the test involves the assessment of toxicity in rats or rabbits following single application of the test substance for 24 hours to intact, shaved skin amounting to approximately 10% of the total body surface.  Effects are assessed during the 14-day period after exposure. Studies were conducted with coated copper flakes, copper (II) oxide, copper sulphate, copper (I) oxide, copper (II) oxide and dicopper chloride trihydroxide (e.g., Nitka, 1991a; Sterner and Chibanguza, 1988a and b; Lheritier, 1993; Kukulinski, 1976 and Forster and Luperi, 1987b). In each of the studies, the LD50 value was > 2000 mg/kg bw, irrespective of their sexes. The summary of the acute dermal toxicity data in animals is provided in Annex V.

Studies in Humans

There are no published data on acute dermal effects of copper or copper compounds.

#### Oral

Studies in animals

Acute oral toxicity data are available for copper (I) oxide, copper (II) oxide, copper sulphate and dicopper chloride trihydroxide.  No acute oral toxicity data are available for copper powder.

Oral LD50 values for several copper compounds in rats have been reported in a number of published studies, some of which have been summarised elsewhere (IPCS 1998).  In addition to these published data, several unpublished study reports of acute oral toxicity tests are available.  These tests have been conducted according to OECD guideline or Fixed Dose methods, and provide good quality data for several copper compounds. The summary of the acute oral toxicity data in animals is provided in Annex VI.

Copper (I) oxide

The most reliable data on oral toxicity of copper (I) oxide on rats were reported by Collier (1984 c- unpublished) and Cuthbert and Carr (1986 - unpublished). LD50-values of 1625 mg/kg bw (male LD50), 928-2000 mg/kg bw (female LD50), and 1340 mg/kg bw (male/female LD50). A similar LD50 value of 1291 mg/kg BW was reported by Cuthbert and Carr (1986) (male and female combined). Driscoll (1999a - unpublished) estimated an LD50 value between 300 and 500 mg/kg BW (male and female combined).  This LD50 range is much lower than the LD50 value reported in the previous studies and may be explained by different particle sizes of the different grades of copper (I) oxide tested and/or different vehicle used for administering the test substance.

Current studies confirm copper (I) oxide is harmful after acute oral administration.

Copper (II) oxide

Available oral toxicity data for copper (II) oxide on rats indicate no harmful effects (Sanders, 2002b – unpublished) as a single oral dose of substance of 2500 mg/kg BW caused no deaths, signs of systemic toxicity, effect on weight gain or abnormalities at necropsy.

Coated copper flakes

Based on the available acute oral toxicity data, coated copper flakes are harmful after acute oral administration.

Copper sulphate pentahydrate

Data from animal studies indicate that copper sulphate pentahydrate is harmful if swallowed. Lehritier (1994) reported a LD50 value in rats (m/f) of 482 mg/kg BW. Manciaux (1998) reported a slightly higher LD50 value of 666 mg/kg BW in female rats.

Dicopper chloride trihydroxide

In rats, LD50 values range from 950 to 2006 mg/kg (Deenihan 1987; Forster and Luperi, 1987a; Jackson 1994a). A lower LD50 value (299 mg/kg) was reported in mice exposed to dicopper chloride trihydroxide (Haynes 1998). From these results it is concluded that dicopper chloride trihydroxide is hazardous by the oral route.

Copper powder and copper massive

No acute oral toxicity data for copper powder were identified.

Studies in Humans

Araya et al (2001, 2003) conducted two well-controlled, oral exposure studies with human volunteers (Table 13). Both studies showed a concentration-related increase in gastrointestinal symptoms which were associated with single oral exposure to copper in drinking water.

Table 13: Summary of acute oral toxicity in humans

| Reference | Substance | Method | Dose levels | Results |
| --- | --- | --- | --- | --- |
| Araya et al, 2001 | CuSO4 | m/f | 0, 2, 4, 6, 8 mg Cu/L | LOAEL: 6 mg Cu/L NOAEL: 4 mg Cu/L |
| Araya et al, 2003 | CuSO4 | f only | 0, 2, 4, 6 and 8 mg Cu/L | LOAEL: 6 mg Cu/L NOAEL: 4 mg Cu/L |

Nausea was the most frequently reported symptom in both studies, and generally occurred within 15 minutes of administration. Other gastrointestinal symptoms that were tracked and observed included vomiting, diarrhea, and abdominal pain. A single dose-response curve for subjects reporting one or more gastrointestinal symptoms or nausea alone was generated using a logistic regression model. Reported gastrointestinal symptoms or nausea alone was shown to be significantly increased in subjects consuming water containing copper at 6 mg/L and above, compared to no copper (logistic regression model, p<0.05). The LOAEL and NOAEL for gastrointestinal symptoms and for nausea alone were 6 mg Cu/L and 4 mg Cu/L, respectively, for copper in drinking water. Derived LOAEL and NOAEL were identical in both studies.

A slightly lower LOAEL of 4 mg/L was determined in a later study by the same authors (Araya et al, 2004)., but in view of the well-controlled methodology used in Araya et al (2001 and 2003), the LOAEL and NOAEL identified in these studies (6 mg/L and 4 mg/L, respectively) are considered to represent the most robust quantitative data on acute effects of copper currently available.

Olivares et al (2001) conducted a study with the objective of determining a nausea threshold in adults who received drinking water containing graded concentrations of copper and concluded that masking the copper taste (addition of orange-flavour) increased the NOAEL from 2 mg/L to 6 mg/L (endpoint: nausea). Zacharias et al (2001) reported a taste threshold for copper in drinking water of 2.6 mg/L (Zacarias et al, 2001), which is in line with the findings of Olivares et al (2001).

In an earlier study, the same research group examined GIT effects associated with different ratios of soluble/insoluble copper compounds in drinking water (Pizarro et al, 2001). Using a similar study design to a previous study (Pizarro et al, 1999), adult female volunteers received drinking water containing 5 mg Cu/L for one-week study periods. The drinking water was prepared using different ratios of copper sulphate and copper (II) oxide. The findings show that acute GIT symptoms of a similar nature can be caused by copper sulphate, a soluble compound, or by copper (II) oxide, an insoluble compound.  The results indicate that the ionic form of copper formed in the stomach is responsible for acute GIT effects.

In line with all of these studies, Wyllie (1957) reported gastrointestinal disturbances resulting from an estimated copper dose of 5.3 mg/L.

The results of a population-based study (Pettersson et al, 2003) showed no association between the mean daily intake of copper or the maximal copper concentration in drinking water and the risk of either diarrhoea or vomiting in young children (mean age: 11.8 months).  Copper levels in consumed water ranged from 0.01 to 5.0 mg/L.

Conclusion

The following is concluded from the animal test data

- Copper (I) oxide, dicopper chloride trihydroxide and coated copper flakes are hazardous by the inhalation route. Copper sulphate pentahydrate is not considered hazardous by the inhalation route.

- Coated copper flakes, copper sulphate pentahydrate, copper (I) oxide and copper oxychloride are hazardous by the oral administration route. Copper (II) oxide is not considered hazardous by the inhalation route.

Read across approach for acute oral toxicity is applied to copper

For copper massive and uncoated copper powder, the acute oral toxicity can be predicted using in vitro studies and available toxicity studies on other forms of copper and copper compounds.

In vivo acute oral toxicity tests are available for several copper bearing materials. The results of the appropriate oral toxicity studies are summarised below.  These include data on 2 substances, dicopper chloride and dicopper sulphide, which are not included in this OECD CoCam review programme.

- The acute oral effects - LD50 - observed for coated copper flakes are between 300 and 500 mg/kg.

- The acute oral effects - LD50 - observed for copper sulphate pentahydrate:  481 mg/kg.

- The acute oral lethal effects - LD50 - observed for copper chloride:  336 mg/kg.

- The acute oral lethal effects - LD50 - observed for copper (II) oxide is >2000 mg/kg

- The acute oral lethal effects - LD50 - observed for dicopper sulphide is >2000 mg/kg

The acute oral data indicate the importance of bio-availability/bio-accessibility:  in vivo exposure to highly bio-accessible copper compounds (coated copper flakes, copper sulphate pentahydrate and copper chloride) results in a higher acute toxicity than in vivo exposure to less bioaccessible/bioavailable copper (II) oxide and dicopper sulphide.  As read-across approach, the measured LD50 values of the all source materials (coated flakes, CuS04.5H2O and CuCl) were expressed as mg bio-accessible Cu/kg bw and combined with the % bio-accessible Cu released from the various copper bearing materials to predict the LD50 values. The results are given below.

Table 14 : Oral LD50 values for the various copper materials

| Cu Material | Copper content % | Bio-accessible/ bio-availability Cu %(1) | LD50 as bio-accessible Cu (mg Cu/kg bw) (2) | Measured LD50 LD50 (mg substance/kg bw) | Calculated LD50 LD50 (mg substance/kg bw) (3) |
| --- | --- | --- | --- | --- | --- |
| Coated flakes (2.9m2/g) | 99.7 | 57 | 227 | 400 (mid value of 300 -500 mg/kg) | 215-401 |
| CuSO4.5H20 | 25.4 | 25 | 122 | 481 | 480-894 |
| CuCl | 63.78 | 51 | 173 | 336 | 232-442 |
| CuO* | 80 | 15-76 |  | >2000 | 160-1513 |
| Cu2S | 78.9 | 2.7 |  | >2000 | 4685-8718 |
| Copper powder (0.024m2/g) | 100 | 1.1 |  |  | 11091-20636 |
| 10 µm copper powder (0.067m2/g) | 100 | 2.8 |  |  | 4357-8107 |

(1) Obtained from bioelution test or in vivo bioavailability test

(2) Measured LD50  / (1)

(3) Min-Max range (2) / (1)

Table 14 shows comparable bioavailable/bioaccessible LD50 values for the 3 substances with bound LD50 data (122 to 227 mg bio-available mg Cu/kg bw).  The calculated LD50 values for these materials (copper coated flakes, CuSO4.5H20 and CuCl) correspond to the measured hazard profile of these materials (LD50 < 2,000 mg/kg and > 200 mg/kg bw), according to GHS. Correct predictions are also observed for Cu2S (i.e. LD50 > 2,000 mg/kg). For CuO, the predicted LD50 values are below the observed LD50 and therefore, calculations are conservative.

The data therefore support the concept that bio-available copper ions are responsible for the observed acute toxicity profiles of CuSO4.5H20, CuCl, Coated copper flakes CuO and Cu2S and that in vitro bioaccessibility data can be used as read-across parameter.  Therefore, following the read-across approach,  the in vivo toxicity of coated copper flakes and copper compounds were combined with the relative bio-accessibility of copper powders and massive to derive the classification of copper powders/massive.   From the assessment it was concluded that copper powder (typical powder and worst case 10µm powder) and therefore also copper massive follow the same hazard profile as CuO and Cu2S (LD50 > 2,000 mg/kg), according to GHS: they do not present an acute oral hazard. Consideration of available acute oral toxicity data leads to the conclusion that coated copper flakes have the same hazard as the soluble copper compounds (CuSO4.5H20 and CuCl).   CuO and Cu2S and copper powder/massive do not present a hazard by the oral route.

Conclusion on the acute oral, dermal and inhalation routes

- Oral: the release of copper ions drive the acute oral toxicity. Copper (I) oxide, copper sulphate pentahydrate, dicopper chloride hydroxide and coated copper flakes present a hazard by the oral route. Copper (II) oxide, copper powder and copper massive are not hazardous by the oral route.

- Dermal: all tested substances within the category are not hazardous by the dermal route. Considering the lower solubility and bioaccessability of copper powders/massives compared to the tested copper substances, copper powders/massive forms do not present a hazard by the dermal route.

- Inhalation: Copper (I) oxide, dicopper chloride hydroxide and copper flakes are considered hazardous by the inhalation route.  Copper sulphate pentahydrate, copper (II) oxide and copper powder/massive are not hazardous by inhalation route

### Irritation

Unpublished data, comprising a number of complete study reports of dermal irritation tests, have been made available. These tests have mostly been conducted in accordance with EU Method B4 and OECD 404 guidelines and provide good quality data on the skin irritant properties of several copper compounds and copper powder. A summary of the relevant studies is reported below.

#### Skin Irritation

Studies in Animals

No published data are available on skin irritant properties of copper compounds in animals. Unpublished data, comprising a number of complete study reports of dermal irritation tests, have been made available. A summary of these studies is presented in Table 15.

Table 15: Overview of experimental studies on skin irritation

| Reference | Substance | Method | Test animals | Dose levels | Results |
| --- | --- | --- | --- | --- | --- |
| Dickhaus and Heisler, 1988a | Copper (I) oxide | OECD 404 | Rabbit, New Zeland white | 0.5 ml of test substance applied. Test 1: substance applied as powder and wetted with water 1:1. Test 2: substance tested as 10% suspension in tylose | Erythema: 0 Oedema: 0 |
| Collier & Wilson (1984b)# | Copper (I) oxide | EU Method B4 | Rabbit, New Zeland white |  | Erythema: 0 Oedema: 0 |
| Driscoll (1999c)# | Copper (I) oxide | OECD 404 EU Method B4 | Rabbit, New Zeland white |  | Erythema: 0 Oedema: 0 |
| Sanders, 2002c | Copper (II) oxide | OECD 404 | Rabbit, New Zeland white, m | 0.5g test substance moistened with 0.5ml distilled water | Erythema: 0 Oedema: 0 |
| Sanders, 2001c | Coated copper flakes | OECD 404 EU Method B4 | Rabbit, New Zeland white |  | Erythema: 0 Oedema: 0 |
| Mercier, 1994c | Copper sulphate pentahydrate | OECD 404 EU Method B4 | Rabbit, New Zeland white, m | 0.5g with 0.07g of purified water | Erythema: 0.22 Oedema: 0 |
| Dickhaus and Heisler, 1987 | Dicopper chloride trihydroxide | OECD 404 | Rabbit, New Zealand White |  | Erythema: 0 (mean) Oedema: 0 (mean) |
| Forster and Luperi, 1987c | Dicopper chloride trihydroxide | OECD 404 EU Method B4 EPA OTS 798.4470 | Rabbit, New Zealand White |  | Erythema: 0.1 (mean) Oedema: 0 (mean) |
| Jackson, 1994c | Dicopper chloride trihydroxide | OECD 404 EU Method B4 | Rabbit, New Zealand White |  | Erythema: 0 (mean) Oedema: 0 (mean) |
| Deenihan, 1987 | Dicopper chloride trihydroxide | EPA OPP 81-5 | Rabbit, New Zealand White |  | Erythema: 0 (mean) Oedema: 0 (mean) |
| Sterner and Chibanguza 1988c | Dicopper chloride trihydroxide | EU Method B4 OECD 404 | Rabbit, New Zealand White |  | Erythema: 0 (mean) Oedema: 0 (mean) |

#  unpublished study.

Copper sulphate pentahydrate

Copper sulphate pentahydrate caused a very slight skin response in New Zealand white rabbits (Mercier 1994c – unpublished). Very slight erythema/eschar formation (grade 1) occurred in 1/3 animals after 1 hour and in 2/3 animals after 24 hours. Absence of lesions at 72 hours showed this weak irritant effect to be reversible.  Copper sulphate was therefore not considered skin irritant.

Copper (II) oxide

Copper (II) oxide produced no signs of skin irritation (erythema/eschar formation, oedema formation ) in three New Zealand White rabbits in a dermal irritation study (Sanders 2002 – unpublished). Copper (II) oxide was therefore not considered skin irritant.

Copper (I) oxide

Copper (I) oxide is not considered skin irritant. Application of cuprous (I) oxide to intact, shaved skin of New Zealand White rabbits caused very slight erythema in all three animals and very slight oedema in one animal at one hour post-exposure (Collier and Wilson, 1984 b- unpublished).  At all other time-points after exposure there was no sign of either erythema/eschar formation or oedema formation. Other studies with rabbits produced no signs of skin irritation (Dickhaus, 1988a; Driscoll 1999c – unpublished; Hixson 1973). Dickhaus (1988a), however, recorded grade 1 oedema in 2/5 animals at 24 hours when administered to abraded skin.

Dicopper chloride trihydroxide

Application of dicopper chloride trihydroxide to intact, shaved skin of New Zealand rabbits caused slight erythema in one animal after 30 min and in two animals after 1 and 24 hours (Forster and Luperi, 1987c - unpublished). No erythema occurred at 48 and 72 hours. There were no signs of oedema at any time point. In four other skin irritation studies, dicopper chloride trihydroxide failed to cause any signs of skin irritation (erythema or oedema) in rabbits (Deenihan 1987; Jackson 1994c; Sterner and Chibanguza, 1988c – unpublished; Dickhaus and Heisler, 1987a - unpublished). Dicopper chloride trihydroxide is not considered skin irritant.

Coated copper flake

Coated copper flakes caused no signs of skin irritation (erythema/eschar formation, oedema formation) in New Zealand White rabbits (Sanders 2001c – unpublished). Consequently, coated copper flake is not considered a skin irritant.

Studies in Humans

There is no published information available concerning the skin irritant properties of copper or copper compounds in humans.

Summary of skin irritation

There are no published data available concerning the skin irritant properties of copper compounds in animals. Several unpublished reports of skin irritation tests conducted in animals have been made available. These studies provide good quality data on the skin irritant properties of copper (I) oxide, copper (II) oxide, copper sulphate, dicopper chloride trihydroxide and coated copper flakes. To summarise the results of these studies, copper (II) oxide, dicopper chloride trihydroxide and coated copper flakes all caused no signs of skin irritation in rabbits during the 72-hour period after exposure. Copper (I) oxide caused very slight, transient skin irritation (erythema and oedema) soon after exposure in one study. Copper sulphate pentahydrate caused very slight irritation in one study which had resolved by 72 hours. There are no data available to indicate that copper compounds are skin irritant.

#### Eye Irritation

Studies in Animals

No published data are available on the eye irritant properties of copper or copper compounds in animals. Unpublished data have been made available, comprising complete reports of eye irritation tests (Table 16).

Table 16: Summary of eye irritation studies in animals

| Reference | Substance Identity | Test method | Ocular effects |  |  |  | Persistent effects (duration) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Corneal opacity | Iris congestion | Conjunctival redness | Conjunctival chemosis |  |
| Collier & Wilson (1984 a) # | Copper (I) oxide | OECD 405 | 2, 2, 2 | 1, 1, 1 | 2.7, 2.7, 2.7 | 2, 3, 2 | Ocular irritation was seen in all 3 test rabbits |
| Kuhn (1994) # | Copper (I) oxide | EPA OPP 81-4, equivalent to OECD 405 | 0.4 | 0 | 0.9 | 0.9 | Slight potential to induce eye irritation. Conjunctival redness is all washed eyes after 1h, conjunctioval redness and chemosis in all non-washed eyes. |
| Kuhn (1994) # | Copper (I) oxide | Annex V B.5 (washout after 30s) | 0, 0, 0 | 0, 0, 0 | 0, 0, 0 | 0, 0, 0 | None |
| Dickhaus & Heisler (1988b) # | Copper (I) oxide | OECD 405 | 0, 0, 0 | 0, 0, 0 | 2, 2, 2 | 1.7, 2, 1 | Slight potential to induce eye irritation. Reversibility has not been fully evaluated. |
| Driscoll (1999d) # | Copper (I) oxide | Annex V B.5; OECD 405 (no washout) | 0.7, 0.7, 0.7 | 0.7, 0.3, 0.3 | 1.7, 2, 2 | 1.3, 1.7, 1.7 | None |
| Sanders (2002d) # | Copper (II) oxide | EU Method B5 OECD 405 (no washout) | 0, 0, 1 | 0, 0, 0.7 | 0.3, 0.3, 1.7 | 0, 0, 1.3 | One application of test material resulted in scattered or diffuse corneal opacity in one treated eye up to the 72h time point, as well as iridial inflammation up to the 48h time point. |
| Mercier (1994b) # | Copper sulphate pentahydrate | OECD 405 EU Method B5 (no washout) | 2.3, 2, 3.3 | 1, 1, 1 | 2, 2, 2 | 4, 4, 3.3 | Lesions observed at 72h were still present in the 3 rabbits after 21d. The test material showed severe ocular irritation and other lesions. |
| Luperi & Forster (1987d) # | Dicopper chloride trihydroxide | EU Method B5 OECD 405 EPA OTS 798.4500 | 1.0 | 0.4 | 0.9 | 1.3 | Corneal effects (day 14) |
| Deenihan (1987d) # | Dicopper chloride trihydroxide | US EPA OPP 81-4 (washing after 24h) | 0.8 | 0.4 | 1.9 | 1.2 | Corneal and conjunctival effects (day 7) |
| Jackson (1994d) # | Dicopper chloride trihydroxide | EU Method B5 OECD 405 (no washing) | 0, 0, 0 | 0.3, 0, 1 | 0.3, 0.3, 0.7 | 0, 0, 0 | Conjunctival effects (day 5) |
| Dickhaus & Heisler (1987b) # | Dicopper chloride trihydroxide | OECD 405 (no washing) | 0 | 0 | 1.2 | 0.3 | None |
| Sanders (2001d) # | Coated copper flake | Annex V B.5 (no washout) EU Method B5 OECD 405 | 0, 1, 2 | 0, 0.7, 1 | 1.7, 1.7, 2 | 0.7, 1, 1.7 | Corneal effects (day 7); conjunctival effects (day 7) |

#  unpublished study

Copper (I) oxide

Copper (I) oxide caused eye irritation in rabbits in a number of tests, mostly of mild to moderate severity and involving the conjunctiva and, to a lesser extent, the cornea and iris. Instillation of cuprous (I) oxide in one eye of each of three New Zealand White rabbits produced ocular lesions affecting the cornea, iris and conjunctiva in all treated eyes (Collier and Wilson 1984 a - unpublished). In another study, cuprous (I) oxide was similarly placed in the conjunctival sac of the left eye of male and female rabbits (Kuhn 1994 – unpublished).No ocular effects were observed. Eye irritant properties of copper (I) oxide were also investigated in another study (Dickhaus and Heisler 1988b). Driscoll (1999d) examined eye irritant properties of copper (I) oxide in three rabbits in a further study. Based on the available data, copper (I) oxide is irritant to the eye.

Copper (II) oxide

Eye irritant properties of copper (II) oxide were investigated in rabbits (Sanders 2002d – unpublished). No ocular effects occurred at later time-points. Copper (II) oxide is not considered eye irritant.

Copper sulphate

Copper sulphate pentahydrate was investigated in an eye irritation test by Mercier (1994b – unpublished). Lesions in the cornea, iris and conjunctiva occurred in all treated eyes. Copper sulphate was considered to be an eye irritant.

Dicopper chloride trihydroxide

Eye irritant properties of dicopper chloride trihydroxide were examined in rabbits (Luperi and Forster, 1987d – unpublished). Corneal opacity (grade 1) persisted in one animal up to day 7 and another animal up to day 14. All ocular lesions had resolved by day 21. Several other authors reported on eye irritant properties of dicopper chloride trihydroxide (e.g., Deenihan, 1987d – unpublished ; Jackson, 1994d; Dickhaus and Heisler, 1987b), but reported effects such as corneal opacity, conjunctival redness  and ocular lesions resolved over time. Dicopper chloride trihydroxide was thus considered not an eye irritant.

Coated copper flake

Eye irritant properties of the coated copper flakes were investigated in rabbits (Sanders 2001d – unpublished).  Coated copper flakes caused signs of irritation in the available eye irritation study. All effects were shown to be reversible within 21 days. Whereas the mean scores over 24-72 h were below the threshold values for classification for iritis, conjunctival redness and chemosis (≥1, ≥2 and ≥2, respectively), the mean score for corneal opacity over 24-72 h was ≥1 in 2 of the 3 tested animals. The mean score over all three animals (1) was also at this threshold value. Coated copper flake is therefore considered as an eye irritant

Studies in Humans

No published data appear to be available regarding the eye irritant properties of copper or copper compounds in humans.

#### Respiratory Tract Irritation

No published studies are available which report on the potential respiratory irritant effects of copper or its compounds.

#### Conclusion

It is concluded that coated copper flakes is not a skin irritant but is an eye irritant.. Copper (II) oxide and Dicopper chloride trihydroxide are not skin nor eye irritants. Copper sulphate is not skin irritant, however, it is severe eye irritant. Copper (I) oxide, is an eye irritant but not a skin irritant. Considering the lower solubility and bioaccessibility of copper powders compared to copper (II) oxide and coated copper flakes, the data from copper (II) oxide are read-across to copper powders and massive forms.

### Sensitisation

#### Skin Sensitisation

-Studies in Animals

No published studies are available which report on the potential of copper or its compounds to cause skin sensitisation in animals. A number of available unpublished reports of skin sensitisation provide good quality data on the skin sensitising potential of copper and some copper compounds. In most of these studies, the test method used was the guinea pig maximisation test (GPMT) of Magnusson and Kligman. In one study, the Buehler test was used (Walker 1990).  In most cases, tests were conducted according to EU Method B6 and OECD guidelines.  A summary of these studies is presented in Table 17.

Table 17: Overview of experimental studies on skin sensitisation

| Reference | Substance | Method | Test animals | Dose levels | Results |
| --- | --- | --- | --- | --- | --- |
| Bien, 1993 | Copper (I) oxide | OECD 406 | Guinea pig, Pirbright-Hartley, m/f (20 controls, 20 test animals) One animal in each group died during challenge | 0.1 ml FCZ 50% (w/w) diluted in aqua ad inject, 0.1 ml test article diluted in CMC (final conc 0.25%), 0.1 ml test article diluted in FCA/CMC (final conc 0.25%) | 0/19 animals sensitized |
| Driscoll (1999e) | Copper (I) oxide | OECD 406 EU Method B6 | Guinea pig (Dunkin-Hartley) | 0.05% and topical application of 75% (w/w in distilled water) topical challenge was 25% and 10% w/w in distilled water | No skin response after challenge |
| Smith (1986) | Copper (I) oxide | EPA guidances - modification of Buehler | Guinea pig | 500 mg of test substance (100%; moistened with propylene glycol) | Very slight erythema at 24h (3/10 animals) and 48h (1/10 animals). Slight erythrema also reported in 2/10 negative control animals |
| Leuschner, 2005 | Dicopper chloride trihydroxide | OECD 406 EU Method B6 | Male Guinea pig (Dunkin-Hartley) | 50% v/v | 1st reading: 0/10 2nd reading: 0/10 negative control: 0/5 |
| Walker, 1990 | Dicopper chloride trihydroxide | Equivalent to EU Method B6 | Female Guinea pig (Dunkin-Hartley) | 50% w/w | 1st reading: 0/20 2nd reading: 0/20 negative control: 0/20 |
| Mercier, 1994a | Copper sulphate pentahydrate | OECD 406 EU Method B6 EPA OTS 798.4100 | Guinea pig (Hartley), m/f | Induction concentration: 0.1% w/w in water or 10% w/w formulation in water Challenge concentration: 10% w/w | 0/20 animals sensitised Slight skin response (1 animal) at 24h; no response at 48h |
| Sanders, 2001e | Coated copper flakes | OECD 406 EU Method B6 | Guinea pig (Dunkin-Hartley), m |  | 1st reading: 0/10 2nd reading: 0/10 negative control: 0/5 |
| Sanders, 2002e | Copper (II) oxide | OECD 406 EU Method B6 | Guinea pig (Dunkin-Hartley), m/f | Induction concentration: 0.1% w/w in distilled water and 75% w/w formulation in distilled water Challenge concentration: 10% w/w and 5% w/w in arachis oil BP | 0/10 animals sensitised Mild skin response: - 10% w/w: discrete or patchy erythrema at challenge sites in 4/10 animals) at 24h; no skin reaction at 48h. - 5% w/w: discrete or patchy erythrema at challenge sites in 2/10 animals) at 24h; no skin reaction at 48h. |

Copper (I) oxide

Skin sensitising potential of copper (I) oxide was investigated (Bien 1993 – unpublished). No skin reaction was seen in treated or control animals at both the first induction phase (intradermal injection with cuprous oxide on t=0), and the second induction phase (topical application of copper (I) oxide for 48 hours on day 7). The findings of this study show that copper (I) oxide is not a skin sensitiser under the conditions of this test. Copper (I) oxide similarly failed to induce skin sensitisation in another GPMT test with guinea pigs (Driscoll 1999e - unpublished) and in a Bueller test conducted to EPA requirements (Smith 1986).

Copper (II) oxide

Copper (II) oxide failed to induce skin sensitisation in a guinea pig maximisation test (Sanders 2002e – unpublished). It is concluded that copper (II) oxide produced a 0% sensitisation rate in this test and is therefore considered to have no skin sensitising potential in guinea pigs.

Copper sulphate

Copper sulphate pentahydrate failed to induce skin sensitisation in a GPMT (Mercier 1994 a– unpublished). Copper sulphate pentahydrate produced a 0% sensitisation rate under the conditions of this test and is therefore considered to have no skin sensitising potential in guinea pigs.

Dicopper chloride trihydroxide

Dicopper chloride trihydroxide was negaitive in a  skin sensitisation test (24h or 48h after challenge) in guinea pigs in a Buehler test (Walker 1990 – unpublished).    Another study (Leuschner 2005-– unpublished) reported that dicopper chloride trihydroxide did not induce skin sensitisation in the guinea pig in the Magnusson and Kligman Maximisation test.

Coated copper flakes

No skin reaction (0% sensitisation rate) in a GPMT was observed with guinea pigs at challenge sites in any test or control animals at 24 or 48 hours (Sanders 2001d – unpublished).  In Copper is therefore considered to have no skin sensitising potential in guinea pigs.

Copper powder

Considering the lower solubility and bioaccessibility compared to copper (II) oxides and coated copper flakes (see section 3.1.1.7), read-across to copper (powders and massive forms) is applied.

Studies in Humans

The few cases of skin sensitisation from exposure to copper or its compounds reported in the literature are restricted to clinical case reports involving small numbers of patients, and in evaluation of a case-series of patients from dermatology clinics.  A review by Barranco (1972) showed that only six cases of allergic contact dermatitis to copper had been reported at that point in time.  Three cases occurred as a result of contact with brass (copper and zinc alloy). The other cases were due in each case to CuSO4, copper metal, and copper in jewellery respectively. Motolese et al (1993) assessed 190 enamellers and decorators by patch tests, and only a single case of a positive patch test to red copper oxide in the group was observed. Sterry and Schmoll (1985) described contact urticaria with a positive patch test in a patient exposed to copper (II)-acetyl acetonate used in self-adhesive disinfection pads applied to the skin.

Cross-reactivity

Metal objects such as spectacle frames have caused dermatitis (Gaul, 1958), but the role of copper in these cases is uncertain, as there is often concomitant exposure to other known sensitisers such as nickel compounds. Some cases of cross-reactivity between copper and other metal sensitisers have been documented between Pd-NiSO4-CuSO4 (Hackel et al, 1991) and between CuSO4 and mercuric chloride (Nordlind and Liden, 1992).

Skin reactions following use of copper IUD

Several studies reported adverse skin effects (dermatitis, urticaria, papulo-erythematous skin lesions, non-pruritic skin eruption, abdominal pain) upon insertion of a copper-based intra-uterine contraceptive device (IUD) (Barkoff, 1976; Romaguera and Grimalt, 1981; Barranco, 1972; Frenz and Teilum, 1980; Rongioletti et al, 1985; Pujol et al, 1998), and relation to the Cu-content was further demonstrated with either patch tests or scratch tests with CuSO4 as test substance.

Prevalence of allergic dermatitis from copper salts

Findings by Lisi et al (1987) and Karlberg (1983) give clear indication on the relative rarity of copper compounds in comparison to other metals as a cause of allergic contact dermatitis. The former observed no irritant reaction to CuSO4 in 564 subjects while studying allergic contact dermatitis from pesticides, whereas the latter observed a reaction to CuSO4 in only 13 eczema patients out of the 1190 that were exposed over a three-year period.

#### Respiratory tract

No information is available from animal/human studies concerning respiratory sensitisation and exposure to copper.

#### Conclusion

Available unpublished studies on the skin sensitising potential of copper (I) oxide, copper (II) oxide, copper sulphate pentahydrate, dicopper chloride trihydroxide and coated copper flakes demonstrated the absence of skin sensitisation in guinea pigs. Given the extensive use of copper, and the potential for continuous and/or intermittent skin contact, human case-reports of skin sensitisation due to copper or its compounds are rare and may be due to cross-reactivity.

### Repeated Dose Toxicity

When interpreting studies of essential trace elements, it needs to be remembered that these elements also play a role in many biological functions, have tight homeostatic control, and are closely linked to physiology with (haematological) effects caused by excess exposure as well as deficiency (e.g., Wang et al. 2012).

Repeated dose (oral) toxicity data in animals are based largely on copper sulphate.  No data are available for copper (I) oxide, copper (II) oxide, dicopper chloride trihydroxide and copper metal.  With regard to repeated oral exposure, it is assumed that copper in the gastro-intestinal tract will occur, at least in part, in the ionic form and will therefore be available for absorption.  Whilst bioavailability data exist for several copper substances in addition to copper sulphate (copper (I) oxide, copper (II) oxide, dicopper chloride trihydroxide), the database is limited in terms of quality and/or relevance to human risk assessment (reviewed in ‘Toxicokinetics’). it is considered appropriate to read-across from copper sulphate, on the basis that this is the most soluble and apparently bioavailable substance covered here, and the most well-characterised with respect to toxicokinetics and repeated dose toxicity.

Animal Studies

A large number of studies are available which have investigated the effects of repeated exposure in animals.

Inhalation

Kirkpatrick (2010) reports a 28 days repeated dose inhalation study on Cu2O, carried out according to OECD Guideline 412, with the addition of a 13 week recovery period and evaluation of adaptation to the test substance (intermediate time points at 0, 1 and 2 weeks). Further additional study endpoints were measurements of copper levels in lung tissue, lung lavage fluid, liver, brain, as well as wet/dry lung weight ratio and clinical chemistry and cytology of bronchoalveolar lavage fluid of all animals. The additional study endpoints were designed to aid in the interpretation of any test substance effects.

Table 18: Studies on repeated dose toxicity after inhalation exposure

| Reference | Substance | Method | Test animals | Dose levels | Results |
| --- | --- | --- | --- | --- | --- |
| Kirkpatrick D (2010) | Cu2+ as cuprous oxide | OECD 412 - subactute whole body inhalation vehicle: air | Rat, Sprague-Dawley, | 0.2, 0.4, 0.8 and 2.0 mg Cu2O/m3 (nominal conc) 0.18, 0.36, 0.71 and 1.8 mg Cu/m3 (nominal conc) | NOAEL: ≥ 2 mgCu2O/ m3 air (m/f) ; 1.8 mg Cu /m3 air (m/f) LOEL: 0.2 mg Cu2O /m3 air (m/f); 0.18 mg/m3 air (m/f) |

There were no histopathological findings in the lung (e.g. no tissue damage). The occurrence of increased collagen levels in high dose animals (an indication of fibrosis) was defined as not toxicologically relevant, as there was minimal and occasionally mild staining also in the control groups. Re-examination for a qualitative histopathological peer-review by German pathologists (Or. Ernst and Or. Rittinghausen, Fraunhofer Institute and Böttner, Histovia 2010) led to the conclusion that the effects on increased collagen formation were "very slight" (Fraunhofer) and "mild to moderate" (Histovia) in the high dose animals, with full reversal of the findings after the recovery period. Only the Histovia report concluded that these findings were statistically significant. Computerized analysis confirmed that there is no significant effect on collagen content of the lung.

In this study there was also no indication of persistent, chronic inflammation. The pattern of responses in the lung and lung-draining lymph nodes in this study following copper (I) oxide exposure is typical for inhalation exposure to aerosol particles in general (i.e., no substance-specific response). An effect on neutrophil numbers (blood or BALF) was noted in all exposure levels, but the observed increase in numbers in the absence of an immunotoxic endpoint or evidence of injury to lung epithelium, is not necessarily adverse. Neutrophils are highly motile and attracted by various factors, including the presence of macrophages, and have a number of mechanisms to defend the host, such as phagocytosis, release of granule proteins, or "respiratory burst". It is therefore concluded that the overall NOAEL for this study is >2 mg Cu2O/m3 (>1.8 mg Cu/m3). No serious adverse effects were observed at the maximum tested concentration (2 m Cu2O/m3 or >1.8 mg Cu/m3).

In addition there are two animal studies available which have investigated effects following repeated exposure to copper compounds by inhalation (Drummond et al, 1986; Johansson et al, 1984), but neither study was considered reliable and no NOAEL/LOAEL could be determined for these studies.

Effects on the respiratory system were investigated by in CD1 mice and Syrian hamsters repeatedly exposed by inhalation to CuSO4 aerosol (0.12- 0.13 mg Cu.m-3) (Drummond et al, 1986). In hamsters exposed for five or ten days, there appeared to be no significant effect on tracheal ciliary activity or on tracheal morphology compared to controls. Repeated exposure of mice by inhalation to CuSO4 aerosol for five or ten days had an effect on a number of respiratory effect parameters (bacteria-induced mortality rate, tracheal and alveolar morphology).  However, poor respiratory health of control animals raises uncertainties regarding the reliability of these findings and the authors themselves concluded that the strain of mice used (CD1) did not appear to be appropriate for this type of study.

An inhalation study for 4-6 wk with rabbits was conducted by Johansson et al (1984); using copper chloride as test substance (0.6 mg Cu.m-3  0.3). Histopathological examination of lung tissue showed no significant differences between CuCl2-treated animals and controls. Some changes in the alveolar region of the lungs could only be detected by electron microscopy. The results of this study, are not useful as the study design did not meet some critical quality criteria (e.g. number of animals, species, number of exposure concentrations, absence of haematological and clinical biochemistry data, and extent of histopathological examination).

Dermal

The dermal route of exposure is the most significant route there is no evidence to indicate that copper or copper compounds can cause toxicity or indeed pass through intact skin at significant levels. Acute dermal toxicity studies showed no toxic effects up to and including the highest dose tested. Therefore an accurate and realistic determination of dermal toxicity can be derived from available sub-chronic oral exposure studies, permissible systemic copper levels and in vitro dermal penetration studies on copper and copper compounds.

Oral

The effects of repeated ingestion of copper sulphate pentahydrate were investigated in two-week drinking water and feeding studies and in a 13-week feeding study using Fischer 344/N rats and B6C3F1 mice, conducted under the National Toxicology Program (NTP).  Tests were similar to EU Method B.26 (Sub-Chronic Oral Toxicity Test: Repeated Dose 90-Day Oral Toxicity Study in Rodents) (Hébert et al 1993; Hébert 1993).  Dose levels for the 13-week study were based on findings of the two-week dietary study.  Repeated administration of copper sulphate pentahydrate in the drinking water for two weeks caused clinical signs of toxicity and fatalities in rats and mice and histopathological changes in mice at 762 mg Cu/litre and above (45 and 31 mg Cu/kgBW/day in male and female rats, respectively; 58 and 62 mg Cu/kgBW/day in male and female mice, respectively).  These effects appear to be a result of dehydration due to reduced intake of treated drinking water and associated reduced weight gain rather than direct toxicity of the compound. Repeated administration of CuSO4 in the diet for two weeks produced effects in the forestomach, liver, kidney and bone marrow in rats and in the forestomach in mice.  When expressed as a dietary concentration, the lowest NOAEL in both species occurred in respect of forestomach lesions corresponding to a dietary concentration of 1000 mg CuSO4/kg diet (255 mg Cu/kg).  When expressed as dietary copper consumption as a function of body mass, the NOAEL was lower for rats than for mice (23.4 mg Cu/kgBW/day added to feed for male rats; 22.7 mg Cu/kgBW/day for females).

The 13wk feeding study described repeated administration of CuSO4 in the feed for 13 weeks produced effects in the forestomach, liver and kidney in rats. Effects also occurred in the haematopoietic system, although these were considered to be transient changes secondary to alterations in iron metabolism that were within the range of homeostatic control. The treatment had no effect on a number of reproductive parameters. Forestomach lesions consisted of hyperplasia of the squamous mucosa of the limiting ridge at the junction of the forestomach and glandular stomach. Hepatic changes consisted of histopathological changes (chronic active inflammation) plus significant alterations in several clinical chemistry parameters. Hepatic changes appeared most pronounced in males. Renal toxicity consisted of histopathological changes (increase in cytoplasmic droplets), together with significant alterations in some urinary parameters. Kidney changes were most marked in males. The NOAEL for effects in the forestomach, liver and kidney was 1000 mg CuSO4/kg diet [255 mg Cu/kg diet; 16.3 and 17.3 mg Cu/kg bw/day added to feed in males and females, respectively]. The liver is the critical organ for copper. Minimal to moderate effects (forestomach, liver, kidney) were observed at 2000 mg CuSO4/kg diet [509 mg Cu/kg diet/; 33 and 34 mg Cu/kg bw/day, added to feed in males and females, respectively]. The incidence and severity of the effects were dose-dependent. Inflammation of the liver occurred in male and female animals at 4000 mg CuSO4/kg diet [1018 mg Cu/kg diet 260 mg CuSO4/kg bw/day; 66 and 68 mg Cu/kg bw/day, added to feed in males and females respectively] and above.

No exposure-related deaths occurred in the 13-week mouse feeding study with  B6C3F1 mice.  Final body weight gain was significantly depressed in both sexes at 4000 mg CuSO4/kg diet  [1018 mg Cu/kg diet; 187 and 267 mg Cu/kgBW/day] and above, and in male mice also at 2000 mg CuSO4/kg diet [509 mg Cu/kg; 97  and 126 mg Cu/kgBW/day], added to feed in males and females. Forestomach lesions in mice were similar to those seen in the rat (NOAELmice, lesions: 2000 mg CuSO4/kg diet  [97 and 126 mg Cu/kgBW/day for males and females respectively] ), but in contrast to the rat, no effects were seen in the liver or kidneys.   No effect on reproductive parameters were measured at doses up to and including 68 (rats) and 536 mg Cu/kg  bw/day (mice).

The results of these studies (Table 19) therefore demonstrate minimal to moderate effects at >100 mg CuSO4/ kg bw/day and severe effects of CuSO4 in the kidney and liver > 200 mg CuSO4 /kg bw/day. This study is considered to be the pivotal study for Cu2+ presented as copper sulphate pentahydrate and results in an NOAEL of 16.7 mg Cu /kg/bw/day in the rat.

In repeated dose toxicity study performed according to OECD TG 422, copper monochloride was given orally (gavage) to Sprague-Dawley rats for 30 days to males and for 39 – 51 days to females at concentrations of 0, 1.3, 5.0, 20, and 80 mg copper monochloride /kg bw/day (Chung et al., 2009). With regard to general effects of repeated exposure, systemic toxicity was evident at 80 mg copper monochloride /kg bw/d  (corresponding to 48 mg Cu /kg bw/d),  consisting of some altered haematological parameters (males and females), changes in some biochemical parameters (males only) and increased incidence of femoral marrow hyperplasia (males only). Squamous cell hyperplasia of the stomach showed a significant, dose-related increase in males at 20 mg copper monochloride /kg bw/d and above and in females at 5 mg copper monochloride /kg bw/d and above. These effects, which were considered to be a direct effect of the corrosive properties of copper chloride, are not consider relevant for the hazard profile of other copper containing materials.

Aburto et al (2001) investigated the hepatic effects of repeated exposure to dietary copper (administered as CuSO4) in Fischer 344 rats. The results show that repeated exposure to dietary copper at 1500 mg Cu/kg and above for three months caused significant hepatotoxicity (increased number of parenchymal necro-inflammatory foci in a defined area) and reduced weight gain in rats.  Based on these conclusions, the NOAEL for this effect would be 1250 mg Cu/kg (103 mg/kgBW/day).   However, in view of the dose-related increase in histopathological changes which occurred with all treatments (750 to 2000 mg Cu/kg), it is not possible to identify a NOAEL from this study; The derived LOAEL for this study with a Reliability Score of 2 was 62 mg/kgBW/d.

Other, less reliable studies investigating the effects of repeated oral exposure to copper confirm the findings of the Hébert studies, namely that the liver is the target organ for copper-related toxicity, and that effects are the same irrespective of strain. Cristofori et al (1992) demonstrated significant copper accumulation in liver, kidney and paw tissue in female Sprague Dawley rats fed a diet containing 200 mg Cu/kg diet  (20 mg Cu/kgBW/day) copper without any signs of associated toxicity.  Based on the absence of toxicity at the dose level investigated, a NOAEL of 20 mg Cu/kgBW/day can be determined for this study. Other dietary studies with limited reliability for which no NOAEL could be derived, were published by Haywood (1980; effects of repeated ingestion of copper on the liver and kidney of male rats), Haywood and Comerford  (1980; effects of repeated ingestion of copper on blood and plasma levels of copper, ceruloplasmin activity, and biochemical indicators of liver damage and haemolysis), Haywood and Loughran (1985; induced tolerance to high copper levels in rats, leading to reduced liver damage), Fuentealba et al (2000; copper-induced liver damage as a function of age and sex differences), Fuentealba et al (1989; copper accumulation in liver and associated effects),  and by Liu and Medeiros (1986; the effects of excess dietary copper on blood pressure and haematological parameters). All these studies have reported LOAELs for hepatotoxicity in rats of 40 mg Cu/kgBW/day and above, which are not at variance with the findings of the NTP study

Finally, Rana and Kumar (1980) investigated effects on liver and kidney histopathology and haematological parameters in male albino rats repeatedly treated by gavage with a single fixed dose of CuSO4 during 20 days (40 mg Cu/kgBW/day). This study demonstrated hepatotoxicity, renal toxicity and some haemotological changes in rats, but due to the absence of a no-effect dosis, no NOAEL could be determined for this study.

Table 19: Summary of studies investigating repeated dose toxicity in animals

| Reference | Species/strain | Exposure Route | Treatment | Oral dose of copper |  |  | Results | LOAEL/NOAEL | Study ratinga |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | mg/kg diet | mg/kgBW/day |  |  |  |  |
| Hébert et al (1993); Hébert (1993) | Fischer rat | Oral | CuSO4 in drinking water for 15d | 76 mg/L 254 762 2543 7629 | Male 10 29 45 36 97 | Female 10 26 31 31 71 | Increased mortality and clinical signs of toxicity at 762 mg/L and above. Effects attributed to reduced water consumption. | ND | 1 |
| Hébert et al (1993); Hébert (1993) | B6C3F1 mouse | Oral | CuSO4 in drinking water for 15d | 76 mg/L 254 762 2543 7629 | 10 24 58 n/a 367 | 15 36 62 n/a 330 | Increased mortality and clinical signs of toxicity at 762 mg/L and above. Effects attributed to reduced water consumption. | ND | 1 |
| Hébert et al (1993); Hébert (1993) | Fischer rat | Oral | CuSO4 in feed for 15d | 255 509 1018 2036 4072 | 23.4 45.8 92.4 197.8 324.5 | 22.7 44.3 93.4 195.7 285.3 | Histopathological changes in forestomach, liver, kidney and bone marrow. | NOAEL (forestomach lesions): 23.4 mg/kgBW/d (M) 22.7 mg/kgBW/d (F) | 1 |
| Hébert et al (1993); Hébert (1993) | B6C3F1 mouse | Oral | CuSO4 in feed for 15d | 255 509 1018 2036 4072 | 42.8 92.1 196.8 293.7 717 | 53.5 103.8 216.1 397.8 780.8 | Histopathological changes in forestomach. | NOAEL (forestomach lesions): 42.8 mg/kgBW/d (M) 53.5 mg/kgBW/d (F) | 1 |
| Hébert et al (1993); Hébert (1993) | Fischer rat | Oral | CuSO4 in feed for 92d | 127 255 509 1018 2036 | 8.1 16.3 32.8 65.9 140.2 | 8.7 17.3 34.4 68.0 134.4 | Histopathological changes in forestomach, liver and kidney. Changes in clinical chemistry and urine analysis. No effects on reproductive parameters. | NOAEL (forestomach lesions): 16.3 mg/kgBW/d (M) 17.3 mg/kgBW/d (F) NOAEL (liver toxicity): 16.3 mg/kgBW/d (M) NOAEL (kidney toxicity): 16.3 mg/kgBW/d (M) | 1 |
| Hébert et al (1993); Hébert (1993) | B6C3F1 mouse | Oral | CuSO4 in feed for 92d | 255 509 1018 2036 4072 | 44 97.2 187.3 397.8 814.7 | 52.2 125.7 266.7 536 1058 | Histopathological changes in forestomach. No effects on reproductive parameters | NOAEL (forestomach lesions): 97.2 mg/kgBW/d (M) 125.7 mg/kgBW/d (F) | 1 |

Studies in Humans

Inhalation

Three studies are available concerning the health of vineyard sprayers exposed to ‘Bordeaux mixture’ fungicide by inhalation. Pimental and Menezes (1975, 1977) described pulmonary and liver lesions related to the exposure of vineyard sprayers to ‘Bordeaux mixture’ fungicide for 3 to 15 years. Data on exposure (mixture composition, exposure concentrations), were not available, and taking into account some of the reported confounding factors (“alcoholic”, “moderate drinking habits”, no information on smoking habits), this report provides no clear indication of the causative agent(s) which may be associated with the reported health effects (hepatic changes such as proliferation and swelling of Kupffer cells, development of histiocytic or sarcoid granulomas, liver fibrosis, angiosarcoma, and micronodular cirrhosis or fatty liver which are related to alcohol consumption. Consequently, it is difficult to draw conclusions based on the findings of this study.

A study by Plamenac et al (1985) provides evidence of cytological changes in the respiratory epithelium among vineyard sprayers in the former republic of Yugoslavia who were exposed on a regular basis to ‘Bordeaux mixture’ by inhalation.  The reported cytological changes do not provide a clear indication as to whether the underlying mechanism for the respiratory changes has an immunological (increased eosinophilia) or irritant (increased respiratory spirals) basis.  As no exposure data were presented in this study, the respiratory effects reported cannot be attributed to either a specific causal agent in the ‘Bordeaux mixture’ or to specific levels of exposure.

No significant effects on respiratory health was observed by Ostiguy et al (1995) among copper refinery workers exposed by inhalation to metal dusts including copper dust, asbestos, other dusts and foundry fumes. Srivastava et al (1992) conducted a cross-sectional study of paper mill workers potentially exposed to copper, chromium and zinc fumes, xylol and sulphuric acid. The study provided some indication of increased prevalence of respiratory effects in exposed subjects compared to controls. Clinical examination showed significantly increased prevalence of emphysema, decreased red blood cell count and decreased erythrocyte sedimentation However, given their exposure to several chemicals with recognised toxicities, together with the absence of quantitative exposure data, this study does not provide a clear indication of the causative agent(s) or the levels of exposure associated with such effects.

Finally, Askergen and Mellgren (1975) focussed on changes in the mucous membranes of the nose and throat of Sheet-metal workers exposed to dust from patinated copper. The study does provides some evidence of atrophic changes in the nasal mucosa of workers associated with occupational exposure to patinated copper dust, but provides no supportive quantitative exposure data.

Dermal

No studies are available which have investigated the effects of repeated dermal exposure to copper compounds in humans.

Oral

Investigations of copper toxicity in children, with emphasis on hepatotoxicity

Several publications are available, including cohort studies, cluster analysis studies and case-reports, which have investigated liver toxicity among young children associated with chronic ingestion of copper.  Interest in this subject has been prompted by reported cases of childhood cirrhosis associated with excessive accumulation of copper.  One form of the disease is referred to as ‘Indian childhood cirrhosis’ (ICC), as a result of its occurrence in India where it was once a major cause of infant mortality.  Other terms have been used for what appears to be a similar condition, including Non-Indian Childhood Cirrhosis (NICC), Copper-Associated Childhood Cirrhosis (CACC), Copper Storage Disease (CDS), Idiopathic Copper Toxicosis (ICT), Early Childhood Cirrhosis (ECC), ‘copper-associated liver disease in childhood’ and ‘exogenous infantile copper intoxication’.

Tanner et al (1979), for instance, provided evidence of very high copper concentrations in liver tissue of Indian children with ICC, but not with other types of liver disease. In a follow-up study by Tanner et al (1983), there were significant indications that relate the occurrence of ICC with the use of copper-containing vessels for the storage and heating of milk/water. One of the conclusions of these authors was that copper intake would be 6- to 20 fold lower for breast-fed infants (absence of copper-containing vessels). Increase of Cu-content in milk due to Cu-release from brass containers was also confirmed by O’Neill and Tanner (1989).

Current understanding of the aetiology of these conditions is that they involve elevated exposure to copper from drinking water or food, possibly combined with a, as yet unproven, genetic predisposition to copper accumulation (IPCS 1998).  The relative importance of the role of these two factors is currently undetermined.

Cohort studies in children

The effect of copper supplementation in the drinking water at the then provisional WHO Guideline level of 2 mg/L was investigated in formula-fed and breast-fed infants from 3 to 12 months old in Chile (Olivares et al., 1998). Estimated total copper consumption (excluding breast milk at 4-6 months) in the control group ranged from 0.8 to 1.6 mg/day, comparable to adult intakes. This study failed to demonstrate any adverse effects in infants who had consumed water with a copper content of 2 mg/litre during the first 12 months of life. Based on this repeat-dosing study, a NOAEL of 2 mg Cu/litre drinking water can be identified for adverse effects in infants, including liver toxicity and GIT effects. Copper in drinking water at concentrations above 2 mg/L was not investigated in this study.

Two German epidemiological studies with infants from household with copper plumbing (Zietz et al., 2003a; 2003b) demonstrated no evidence of liver disease among infants living in households where copper levels in drinking water were ≥0.8 mg/litre, with maximum values of ~4 mg/litre reported in the wider study. These authors also conducted a fairly similar investigation in Lower Saxony with a smaller population of infants (aged up to 12 months). Among these infants, no evidence of liver malfunction was detected and none were affected by corresponding symptoms of nausea or vomiting. This additional investigation failed to demonstrate any signs of liver disease among infants living in households where copper levels in water were ≥0.8 mg/litre, with maximum levels of ~6 mg/litre reported in the wider study.

Some other individual case-studies of copper toxicity in young children were identified in open literature, but are not discussed here in detail (Salmon and Wright, 1971; Walker-Smith and Blomfield, 1973; Bhandariand Sharda, 1982; Müller-Höcker et al, 1988; Adamson et al, 1992; Baker et al, 1995; Trollman et al, 1999) (see Annex VII)

Investigations of hepatotoxicity, gastrointestinal and/or other effects in adults

Several studies have been published which have investigated hepatotoxicity, GIT and/or other effects in adults, exposed repeatedly to copper in drinking water, feed or copper supplementation.  Whilst GIT effects in relation to copper exposure are recognised to be acute effects, the pattern of exposure in the following studies is that of repeated exposure.  Consequently, these studies are reviewed both in this section and in the Acute toxicity section.  Only studies which have included quantitative exposure data for copper have been reviewed here.

Cohort studies in adults

An investigation of the effects of copper supplementation in drinking water among healthy Chilean women was conducted by Pizarro et al (1999). The findings of this study resulted in a NOAEL of 5 mg/litre for copper in drinking water for chronic effects, specifically effects on liver function as indicated by enzyme markers.  At 5 mg/litre, total copper consumption (including copper from water and copper from diet) was reported to be 9.8 mg/day.

Pratt et al, 1985 assessed, as part of a double-blind study of back pain management, 7 adult patients, receiving an oral dose of 5 mg copper (as copper gluconate capsules) twice a day for 12 weeks.  Seven others received a placebo capsule over the exposure period.  Subjects were seen every 2 weeks to evaluate their progress.  There was no toxicologically significant change in the level of copper, zinc or magnesium of the serum, urine or hair samples of the seven subjects during the 12 weeks of the study.  Similarly, there was no toxicologically significant change in the haematocrit, mean corpuscular volume, serum cholesterol, serum triglyceride, SGOT, serum alkaline phosphatase, serum GGT, or serum LDH.Serum potassium changed from a mean of 4.3 mEq/L to 4.0 mEq/L.  The incidence of nausea, diarrhoea, and heartburn was the same in subjects receiving copper gluconate as it was in the control group. The briefly reported study by Pratt et al (1985) therefore concluded that a repeated oral intake of 10 mg Cu/day as a supplement in adults does not cause any health effects (acute and chronic NOAEL), but the report presented few details of the methodology and only reported few results. It is noted that the NOAEL of 10 mg Cu/day from this study has been used as the basis of the Scientific Committee on Food’s Tolerable Upper Intake Level for copper.Two other studies with low reliability were conducted by Buchanan et al (1999) and Turnlund et al (2004), and no high-quality NOAELs  could be derived from these studies.

Cluster analysis reports

A series of cluster analysis studies in the USA have been reported by (Knobeloch et al, 1994). Most studies reported on adverse gastro-intestinal effects (abdominal cramps, nausea, diarrhoea, vomiting) related to elevated concentrations of copper in drinking water. The different cases on this publication reported maximum Cu-levels 1.3 mg/Cu/L, 5.0-87.7 mg Cu/L, 1.3 mg Cu/L, 3.6 mg Cu/L; some of these values exceed the WHA guideline value of 2 mg Cu/L;

Reported individual cases

O’Donohue et al (1993) described a case of chronic copper intoxication (intake of Cu tablets on a regular basis for approximately three years) leading to liver failure.This single case study provides evidence of hepatotoxicity in an adult associated with long-term oral exposure to copper at 30 or 60 mg/day.  This dose corresponds to 0.43 or 0.86 mg/kg/day, assuming a bodyweight of 70 kg.

A study in the US identified a family where the father and two daughters suffered repeated gastrointestinal symptoms, mainly vomiting and abdominal pains, for a period in excess of one year (Spitalny et al, 1984) and effects were associated with the consumptions of tap water containing a median concentration of 3.07 mg Cu/L. Complete remission from the recurrent symptoms occurred when consumption of the tap water was terminated.

Investigations of cardiovascular effects in adults

A large number of studies are available which have investigated copper and its involvement in cardiovascular disease.  The only study that provides quantitative exposure data for copper is that of Punsar et al (1975), which present data on copper levels in drinking water.  Consequently, this appears to be a key study in this sub-section.

A 10-year follow-up study was conducted to search for a hypothetical factor in drinking-water that might explain the markedly different coronary heart disease (CHD) mortality rates in two regions of Finland and that might influence the development of CHD (Punsar et al, 1975).  Copper was one of 22 drinking water factors investigated in the study. The study, however, revealed no significant correlation between CHD risks factors and copper in drinking water, although some significant correlations were demonstrated for other drinking water constituents.

Investigations of neurotoxic effects in adults

A number of studies have been published by a research group from Japan which have investigated the combined effects of copper, lead and zinc on the nervous system in occupationally-exposed humans (Murata et al 1987; Araki et al 1987; Murata and Araki, 1991; Araki et al 1993).  In all of these studies, the focus has been on lead-induced neurotoxicity and the possible antagonistic effects of copper and zinc, rather than on the neurological effects of copper per se.  None of these studies provide any useful information concerning the effects of repeated exposure to copper.

Conclusion

The NTP studies by Hébert et al (1993) and Hébert (1993) are considered to be the pivotal study for Cu2+ presented as copper sulphate pentahydrate and results in an NOAEL of 16.7 mg Cu Cu2+/kg/bw/day in the rat. With regard to repeated dose oral toxicity of other copper substances, it is considered appropriate to adopt a conservative approach and use the data from copper sulphate, on the basis that this is the most soluble and apparently bioavailable copper substance covered here, and the most well-characterised with respect to toxicokinetics and repeated dose toxicity.

The data from Chung et al (2009), with an observed general effects of repeated exposure, systemic toxicity at 48 mg Cu /kg bw/d  provide supportive evidence. The  observed local effects at lower doses (squamous cell hyperplasia of the stomach), are considered to be a direct effect of the corrosive properties of copper chloride and are therefore not relevant for the repeat dose hazard profile of other copper containing materials in the category.

Repeated dose NOAELs derived from human oral-dosing studies, namely 5 mg Cu/L drinking water (Pizarro et al, 1999) and 10 mg Cu/day as a dietary supplement (Pratt et al, 1985), are considered to be less robust than the animal data from Hébert et al (1993) and are viewed as supporting data.

With regard to repeated dose toxicity following either inhalation or dermal exposure, no reliable animal or human data are available which clearly indicate a hazard via these exposure routes. In the 4-weeks study by inhalation in rat (Kirkpatrick 2010), no serious adverse effects were observed at the maximum tested concentration (2 mg/m3).

### Mutagenicity

Mechanistic in vitro studies have shown that free copper (II) ions are able to redox cycle and catalyse the generation of reactive oxygen species (Kadiska et al, 1992; Cai et al, 1995; Milne et al, 1993; Toyokuni et al, 1994).  These reactive oxygen species are capable of damaging DNA, providing a mechanism by which copper may indirectly cause genetic damage in cells.  In vitro experiments have also shown that copper can affect the fidelity of DNA-repair (Porter et al, 1997).

Under normal physiological conditions which occur in vivo, the concentration of free copper is extremely low, i.e., less than one free copper atom per cell (Rae et al 1999). Consequently, under normal physiological conditions it seems unlikely that copper-induced oxidative damage to DNA will occur.

In vitro in cell mutation assays (bacterial, mammalian)

Available data on the mutagenicity of copper substances are largely based on studies of copper sulphate. In view of current understanding of copper-associated DNA damage, namely that it may be caused indirectly by free copper ions, and taking into consideration toxicokinetic data and acute/repeated-dose toxicity data for copper sulphate, it is considered appropriate to read across from copper sulphate to other copper substances. The most reliable studies (Reliability Scores of 1 and 2) are shown in Table 20.  A total of eleven bacterial cell studies showed a negative result with regard to mutagenicity of copper (Ballantyne, 1994; Marzin and Phi, 1985; Moriya et al, 1983; Tso and Fung, 1981; Wong, 1988; Codina et al, 1995; Kanematsu et al, 1980; Nishioka, 1975; Riley, 1994; Ward, 1994; Tinwell and Ashby, 1990). Dose-dependent increase in UDS was noted in one study with primary rat hepatocytes (Denizeu and Marion, 1989), whereas effect only at cytotoxic concentrations were found in three studies with non-transformed human fibroblasts and Chinese Hamster Ovary cells, rat hepatocytes (Olin et al, 1996; Sina et al, 1983; Bhunya and Pati, 1987)). Dose-dependent increase in chromosome aberrations were observed by Agarwal et al (1990) Bhunya and Pati (1987). All the bacterial cell studies were conducted with either copper sulphate or copper nitrate.

Table 20: Summary of reliable (R1, R2) studies investigating mutagenicity of copper compounds

| Reference | Substance | In vitro test |  |  | Result | Study rating |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Method | Cell type | Route of administration |  |  |
| Ballantyne (1994) | Copper sulphate | Reverse mutation assay | S. typh. (TA98, 100, 1535, 1537, 102) | - | Negative | 1c |
| Moriya et al (1983) | Copper sulphate | Reverse mutation assay | S.typh. (TA98, 100) | - | Negative | 2 |
| Codina et al (1995) | Copper chloride | Reverse mutation assay | S.typh. (TA98, 100) E.Coli (WP2) | - | Negative | 2 |
|  |  | In vivo test |  |  |  |  |
|  |  | Method | Animal model | Route of administration |  |  |
| Riley (1994) | Copper sulphate | Micronucleus test | CD-1 mice | Oral (gavage) | No increase in micronucleus formation | 1 |
| Ward (1994) | Copper sulphate | UDS test (hepatocytes) | Wistar rats | Oral (gavage) | No induction of UDS | 1 |
| Agarwal et al (1990) | Copper sulphate | Bone marrow cytogenetic test | Swiss albino mice | i.p. injection | Dose-dependent increase in chromosome aberrations | 2 |
| Tinwell and Ashby (1990) | Copper sulphate | Micronucleus test | CBA mice | i.p. injection | No increase in micronucleus formation | 2 |

Two publications reported on in vitro cell mutagenicity studies with another copper compound: Sideris et al (1988) (reliability rating: R3) reported dose-dependent increase in mutation frequency for copper nitrate in Chinese hamster V79 cells, whereas Rusov et al (1997) reported (reliability rating: R3) dose-dependent increase in micronucleus formation for copper acetate without any indication of cytotoxicity.

In vivo studies

The most reliable in vivo data come from two unpublished, oral-dosing studies with copper sulphate as test substance. An unpublished, full report (Riley, 1994 – unpublished) concluded that copper sulphate failed to increase micronucleus formation in polychromatic erythrocytes from bone marrow of CD-1 mice (5 males, 5 females) treated orally with 447 mg/kg/day copper sulphate (113.76 mg Cu/kg/day) for two consecutive days, a dose which caused some toxicity. A second unpublished study of an in vivo UDS test for copper sulphate with male Wistar rats (Ward, 1994) - unpublished) showed that oral treatment of rats with copper sulphate at doses up to 2000 mg/kg (509 mg Cu/kg) failed to induce UDS in hepatocytes isolated ex vivo 12-14 or 2-4 hours after dosing (no DNA damage was observed).

No high quality human data available on the genotoxic potential of copper and copper compounds in humans were identified in open literature

Table 21: Overview of experimental studies on in vivo genotoxicity

| Reference | Substance | Method | Test animals | Dose levels | Results |
| --- | --- | --- | --- | --- | --- |
| Ward PJ (1994) | Cu2+ as copper sulphate pentahydrate | Equivalent to OECD 486 - unscheduled DNA synthesis | Rat, Wistar, 6m | Oral: gavage 632.5 or 2000 mg/kg | Negative |
| Riley SE (1994) | Cu2+ as copper sulphate pentahydrate | EU method B.12 - micronucleus assay | Mouse CD-1 15m and 15f | Oral: gavage 447 mg/kg | Negative |

Based on these findings it can be concluded that copper sulphate pentahydrate, copper and other copper compounds are not considered genotoxic.

Additional in vivo genotoxicity studies have been identified in open literature, but these studies do not meet the high-quality criteria and are only mentioned for reasons of completeness, e.g. studies conducted by Agarwal et al, (1990), Tinwell and Ashby (1990), Bhunya and Pati (1987), and Rusov et al (1997) who administered cupric (I) acetate via ip injections. When toxicity studies are conducted with i.p. route of administration, they bypass the normal uptake and distribution mechanism that is specifically designed to protect the animal from the toxic/reactive copper ion. The VRAR (2008) provides a full review of these studies and the discussion on the unsuitability of these studies.

Conclusion

Consideration of the weight of evidence from in vitro and in vivo tests, with greatest emphasis being placed on those in vivo tests which had the highest study rating, leads to the conclusion that copper sulphate is not mutagenic. Results from in vitro mammalian cell tests show that copper sulphate is genotoxic only at high, cytotoxic concentrations.  Taking into account the acute and repeated dose toxicity data reviewed in this report, and in consideration of toxicokinetic data, read across from copper sulphate to the other copper substances is justified.

### Carcinogenicity

In vitro Studies

There are no data available for this type of tests

In vivo Studies in Animals

The potential carcinogenicity of copper compounds in animals has not been well studied.  Available studies on the carcinogenicity of copper compounds in animals are listed in Table 22 together with a rating based on the evaluation criteria developed by Klimisch et al (1997). The poor design of these studies (inadequate numbers of animals, inappropriate exposure route, insufficient dose levels, and/or limited histopathological investigations) severely limits the conclusions that can be drawn.  Nonetheless, the available studies give no indication that copper compounds are carcinogenic in rats or mice.

Table 22: Summary of carcinogenicity studies

| Reference | Species | Conclusions |
| --- | --- | --- |
| Stoner et al (1976) (Reliability Score 3) | Strain A mice (groups of 10 males and 10 females) Intraperitoneal injection of copper acetate, 3 times a week for 8 weeks at total doses of 0, 36, 90 and 180mg/kg body weight. Mice sacrificed 22 weeks after final injection. | Inadequate evidence to suggest an effect of copper on the frequency of spontaneous lung cancers in this highly susceptible strain of mouse |
| Llewellyn et al (1985) (Reliability Score 3) | Holzman rats(groups of 10) Control group fed 18mg Cu/kg diet. Treated group fed 2600mg Cu/kg diet for 21 weeks | Only one toxic dose tested. No evidence of neoplastic effects |
| Greene et al (1987) (Reliability Score 3) | Sprague Dawley rats (groups of 10) rats fed 0.6, 25, 100mg Cu/kg diet for 25 weeks. | No conclusions regarding the potential carcinogenicity of copper can be drawn from this study |

Table 23 gives an overview of available studies on potential carcinogenic effects via oral exposure. It should be noted that no high quality data are available with regard to inhalation or dermal exposure. The presented oral studies indicate that copper sulphate and other copper salts do not appear to have carcinogenic potential even at very high dose levels of up to 120 mg Cu/kg/bw/day (Harrison et al., 1954).  Based on the observations in some of these (and other) studies  it can be concluded that excess copper may have a protective effect on known carcinogens (e.g. Carlton and Price, 1973). In addition, the available genotoxicity studies support the indication that copper compounds have no carcinogenic potential.

Table 23: Overview of experimental studies on carcinogenicity

| Reference | Route | Test animals | Dose levels | Tumours |
| --- | --- | --- | --- | --- |
| Carlton and Price (1973) | Oral, diet, 9 months | Rat, Sprague-Dawley, m | 1 ppm, 800 ppm | one kidney tumour (1/42, though not significant) was observed in the low copper group |
| Burki & Okita (1969) | Oral, drinking water, 46 weeks | Mouse, C57BL/6J, f | 198 mg/L | Data suggest that copper sulphate may possibly inhibit DMBA-induced tumour development. CuSO4 had no effect on the incidence of DMBA-induced adenomas of the lung, lymphomas and breast tumours. |
| Harrison et al (1954) | Oral, diet, 30-44 weeks | Rat, Sprague-Dawley, m/f | 0, 530 or 1600 ppm Cu | Several histopathological effects were observed at the highest dose, but there were no reports of evidence of neoplasms in any treatment group. |

The findings of these studies do not raise concerns with respect to carcinogenic activity.

Studies in Humans

A number of epidemiological studies have investigated the health hazards, including cancers (most frequently lung cancer) and non-malignant diseases, associated with occupational exposures in the copper mining, copper smelting and refining, and copper alloy industries.  Most of these studies are confounded by numerous factors including co-occurring exposures to known carcinogenic compounds (e.g., arsenic compounds, SO2, Pb, Se, Cad oxide fumes, acid mist,…) or other parameters that may increase the incidence of cancer (e.g. failure to consider smoking status) (Chen et al, 1993, 1995; Pinto et al, 1978; Welch et al., 1982; Viren & Silvers, 1994; Lubin et al., 2000; Enterline et al., 1995; Karjalainen et al., 1992;  Logue et al. 1982; Sorahan et al, 1995). For example, ten of these studies examined cancer risk in copper smelter workers, but effects were related to As-exposure, and potential involvement of copper in cancer mortality did not feature in any of these studies.

Other human studies have looked at the association between serum copper levels and cancer risk, but the few prospective studies where copper serum levels were measured prior to diagnosis, there is no convincing evidence that dietary intake or serum-copper levels play an aetiological role in carcinogenesis (Coates et al, 1989; Overvad et al, 1993). Elevated copper serum levels, however, may be a potential marker of disease-state.

Conclusion

None of the available studies provide convincing evidence that copper plays an aetiological role in the development of cancer in humans.  It is acknowledged that available animal and human data on the carcinogenicity of copper and its compounds are deficient in several respects, but the findings do not raise concerns with respect to carcinogenic activity. It is concluded that the available data for copper compounds do not meet the criteria requiring classification for carcinogenicity. These results further indicate that the NOAEL values derived from the sub-chronic effects observed in the NTP study, 1993 could be regarded as worst case

### Toxicity for Reproduction

There are several studies in the public domain that investigate the reproductive toxicity potential of copper and copper compounds. In many of these studies, positive or equivocal findings have been reported. However, on investigation, it has been shown that these positive findings have been the result of inappropriate routes of administration. These are namely intra peritoneal (i.p.) and intravenous (i.v.) routes. Neither of these routes of administration is representative of exposure of copper and copper compounds through normal production and use. When toxicity studies are conducted with either i.p. and i.v. routes of administration, they bypass the normal uptake and distribution mechanism that is specifically designed to protect the animal from the toxic/reactive Cu2+ ion.

The most reliable data on the effects of reproduction come from an oral-dosing two-generation reproduction toxicity study using copper sulphate and an oral-dosing teratogenicity study using copper hydroxide.  Some supporting data are provided by studies using copper sulphate.

It is assumed that copper in the gastro-intestinal tract will occur, at least in part, in the ionic form and will therefore be available for absorption, and thus potentially available to cause effects on reproduction. The section on toxicokinetics refers to data for copper hydroxide from a bioequivalence study, indicating similarities with several copper substances (copper (I) oxide, dicopper chloride trihydroxide and copper sulphate). Hence, read-across from copper hydroxide to these substances is considered appropriate.  In the case of data on reproductive toxicity from copper sulphate, it is also considered appropriate to adopt a conservative approach and read-across to less soluble substances.

Studies in Animals

Effects on Fertility

Available public domain studies on the fertility of copper are of limited value to ascertain the reprotoxic potential copper compounds over multi-generations. These studies have a lower reliability rating but are reported in the table below.

Mylchreest (2005) conducted a reliable (reliability rating R1) two-generation oral reproduction study (performed in accordance with OECD test guidance 416, see Table 24), and reported a reduced spleen weight. The OECD document ENV/JM/MONO(2001) “Definition of reproductive toxicity” states that the spleen effect cannot be considered a reproductive effect; for a compound to be considered to be a reproductive toxin ‘data for animal studies ideally should provide clear evidence of specific reproductive toxicity in the absence of other, systemic, toxic effects’. As the results of this study do not indicate specific reproductive toxicity at the highest dose level tested, it is proposed that copper sulphate and, after read-across, copper and copper compounds, are not toxic to reproduction.

The results of the multigeneration study should be interpreted in conjunction with the rest of the toxicology database for copper. The following findings are considered relevant when evaluating the reproductive and developmental toxicity potential of the test substance:

- Subchronic and chronic studies show no adverse effects on reproductive organs or endocrine functions,

- Copper salts show no indication of genotoxicity.

In addition, a repeated dose toxicity study by Hébert (1993) showed no effect on reproductive parameters measured at doses up to and including 68 (rats) and 536 mg Cu/kg diet (mice) (endpoints: male reproductive organ weights, spermatid or spematozoal measurements, oestrous cycle length or proportion of oestrous cycle spent in each stage).

Table 24: Overview of experimental studies on reproduction

| Reference | Substance | Method | Test animals | Dose levels | Results |
| --- | --- | --- | --- | --- | --- |
| Mylchreest (2005) | Cu2+ as copper sulphate pentahydrate | EPA OPPTS 870.3800 OECD 416 oral: feed | Holzman rats (Crl:CD®(SD)IGS BR), m/f 10 animals/group | 0, 100, 500, 1000 and 1500 ppm | LOAEL (P): >1500 ppm (m) LOAEL (P): 1500 ppm (f) LOAEL (F1): 1500 ppm (m) LOAEL (F1): 1500 ppm (f) LOAEL (F2): 1500 ppm LOAEL (F2): 1500 ppm (f) NOAEL (P): 1500 ppm (m) NOAEL (P): 1000 ppm (f) NOAEL (F1): 1000 ppm (f) NOAEL (F1): 1000 ppm (f) NOAEL (F2): 1000 ppm NOAEL (F2): 1000 ppm (f) |

A further high quality study is available by Chung et al (2009).  This study is a combined repeat dose and reproduction/developmental screening test, which is intended to investigate effects of repeated exposure over a relatively limited time-period and to provide initial information on effects on male and female reproductive performance. The study was conducted according to OECD test Guideline (no. 422) using Sprague-Dawley rats and is considered to be of high quality. In this study, male rats were treated with copper chloride two weeks prior to mating and during the mating period (total of 30 days). Female rats were treated two weeks prior to mating, throughout mating and gestation, and until day 3 of lactation (estimated duration ~50 days). Copper chloride was administered daily as an aqueous suspension by gavage at 1.3, 5, 20 and 80 mg/kg bw/d (calculated as 0.8, 3.2, 12.9 and 51.7 mg Cu/kg bw/d). With regard to general effects of repeated exposure, systemic toxicity was evident at 80 mg/kg bw/d, consisting of some altered haematological parameters (males and females), changes in some biochemical parameters (males only) and increased incidence of femoral marrow hyperplasia (males only). Squamous cell hyperplasia of the stomach, which was considered to be a direct effect of the irritant properties of copper chloride, showed a significant, dose-related increase in males at 20 mg/kg bw/d and above and in females at 5 mg/kg bw/d and above (12.9 and 3.2 mg Cu/kg bw/d, respectively). With regard to reproduction/developmental effects, there were no effects on any male or female reproductive parameters investigated in this study. Effects on two developmental parameters were reported: numbers of pups with gross (icteric) lesions and numbers of runt pups, both of which were significantly increased at 80 mg/kg bw/d (51.7 mg Cu/kg bw/d).  In conclusion, reproductive toxicity was only observed when systemic maternal toxicity was evident (at 80 mg/kg bw/d.)

Developmental Toxicity

Several animal studies investigated the effects of copper on fetal or embryonic development, but only one of these studies, an unpublished study by Munley (2003), conforms to OECD methods for a teratogenicity test (OECD method 414, see Table 25).  This study provides the most reliable animal data concerning the developmental toxicity of copper.

Groups of 22 female NZW rabbits were treated orally by gavage on days 7 to 28 of pregnancy with copper hydroxide in this developmental toxicity study. The outcome of this study demonstrated maternal toxicity (initial weight loss and reduced food intake) and effects on the fetus (increased incidence of a common skeletal abnormality) following oral exposure of rabbits to copper hydroxide at 9 mg Cu/kgBW/day and above during pregnancy.  There were no indications of fetal abnormalities associated with treatment at up to maternally toxic levels (endpoint: initial weight loss and inappetance).  The NOAEL for maternal toxicity and developmental effects in rabbits in this study was 6 mg Cu/kgBW/day.  Effects on the fetus were considered to be secondary to maternal toxicity and consequently not a specific effect of copper on reproduction.

Table 25: Summary of a reliable experimental study on developmental toxicity

| Reference | Substance | Method | Test animals | Dose levels | Results |
| --- | --- | --- | --- | --- | --- |
| Munley SM (2003) | Copper hydroxide | OECD 414 EPA OPPTS 870.3700 MAFF 59 EU Method B.31 NohSan NP. 4200 | Rabbit, New Zealand White, f | 6, 9, 18 mg Cu/kg bw/day | NOAEL maternal toxicity: 6 mg/kg bw/day NOAEL teratogenicity: 6 mg/kg bw/day |

Other studies investigating developmental effects in animals have significant deficiencies in methodology and/or reporting reliability ratings 3 or 4) (Lecyk, 1980; Keen et al, 1982; Haddad et al, 1991; Aulerich et al, 1982; Ferm and Hanlon, 1974; Di Carlo, 1980; Reinstein et al, 1984; Barlow et al, 1981; Chang and Tatum, , 1973). Observed effects could be related to confounding factors other than copper exposure (e.g., maternal toxicity, reduced food intake, irrelevant exposure route, and zinc deficiency). Consequently, these studies provide little useful information concerning the developmental effects of copper compounds which are relevant to human hazard assessment.

Studies in Humans

Effects on Fertility

A limited amount of epidemiological data for humans on Cu-related reproductive toxicity have identified. Scott and Speakman (1991) reported a negative correlation between serum copper and sperm count in a study of male patients attending a fertility clinic. The study report, however, provided too few details to enable further evaluation of the results.  Aschengrau et al (1989; 1993) published on the relationship between drinking water quality and adverse pregnancy outcomes in two studies. Both studies failed to demonstrate any association between copper in drinking water and adverse pregnancy outcomes.

Developmental Toxicity

A brief report by Morton et al (1976) investigated the relationship between the incidence of CNS malformations in the 48 local authorities and trace elements in water, and observed significant negative correlations for three elements (copper, calcium, and barium).  However, the brevity of this report precludes critical evaluation of the study and the reliability of the results is therefore uncertain. A brief study by Barash et al (1990) reported no evidence of teratogenic effects associated with copper IUDs, nor any evidence that copper is distributed to placental or embryonic tissue.

Conclusion

Available animal and human data on the reproductive effects of copper and its compounds do not raise concerns for developmental or reproductive toxicity.  The most reliable data for developmental toxicity indicate that developmental effects occur only in the presence of maternal toxicity. Results of a two-generation reproduction toxicity test showed no treatment-related reproductive effects up to 1500 ppm dietary copper sulphate pentahydrate, the highest concentration tested.

The available data concerning reproductive effects of copper in humans are sparse.  Of the four human studies which have been reviewed here, two have major deficiencies which make the reliability of their findings uncertain.  The remaining two studies, both drinking water studies, failed to demonstrate any association between copper levels in drinking water and adverse pregnancy outcome.

It is also important to consider that copper is an essential element and many countries recommend an increased dietary intake of copper during pregnancy. This increased recommendation is because a foetus requires copper levels up to 10 times adult levels. The copper is absorbed across the placenta and is required for healthy growth and development, especially in blood maturation, bone development, heart development and function, brain development and function and the function of 20 key enzymes (Ralph & McArdle, 2001). The existing toxicology data package therefore supports the conclusion that copper has no reproductive or developmental toxicity potential.

## Conclusion

Copper is an essential nutrient present in human body fluids and tissues at concentrations of parts per mil-lion or parts per billion. It has a number of physiolgical roles that may be classified as regulatory, structural and protective. From the available toxicokinetic studies, a value of 25% true oral absorption was determined in the rat and 20% true oral absorption in humans.

Both in vitro and in vivo bioavailability studies have shown that relatively insoluble copper powder and sparingly soluble copper compounds e.g. dicopper sulphide and copper (II) oxide, are less bioavailable than the more soluble copper salts e.g. copper sulphate pentahydrate and cuprous oxide.

Copper and copper compounds are not mutagenic, carcinogenic or a reproductive toxicant. There is no specific target organ toxicity effects observed that would warrant classification as an acute or repeat STOT under Global Harmonisation System

Three of the substances and one form of zero valent copper in this category present a hazard for human health, based notably on the release/bioaccessibility of copper ions.

- Copper sulphate pentahydrate: severe eye irritationand acute hazard by the oral route.

- Copper (I) oxide: severe eye irritation, acute hazard by the oral and inhalation routes.

- Dicopper chloride hydroxide: acute hazard by the oral and inhalation route.

- Coated copper flakes: eye irritation, acute hazard by the oral and inhalation routes.

Copper (II) oxide, copper powder and copper massive do not pose a hazard to human health.

# hazards to the environment

Different metal species are used as test substances in various ecotoxicity tests (e.g. CuSO4, CuCl2, CuO). Considering that the metal ion (Cu2+) drives the ecotoxicity, only tests with soluble inorganic copper compounds were retained. Acute and chronic toxicity data from the various soluble com-pounds (e.g. CuSO4, CuCl2) were combined and expressed as metal ion concentrations (μg dis-solved Cu/L).

The selected studies were further screened, using reliability and quality criteria. All data reported are measured values (µg dissolved Cu/L).

The core database was developed between 2000-2006 as a co-operation between consultants, the copper industry and the Italian Istituto Superiori Di Sanita. The data-set and conclusions were reviewed in 2004-2007 by the European Technical Committee for New and Existing Substances (TCNES). Further reviews were done by the European Scientific Committee for Health and Environment Risk (SCHER) in 2008. The Technical Meeting on Biocidal Products in 2008 reviewed the data for copper sulphate pentahydrate, copper (II) oxide and dicopper chloride trihydroxide. As follow-up of the final comments from TCNES, SCHER and the Biocidal technical meetings, some focussed follow-up research was carried out and included in the REACH and CoCam dossiers.It should be noted that the data on hazard classification are further discussed during the discussions on harmonized classification of biocidal copper forms and compounds (2014—2017 and onwards) in the EU.

## Effects to Freshwater

### Introduction

Considering that copper is a natural element, essential for all life forms, different studies have quantified levels of homeostasis and deficiency in invertebrates, fish and amphibian – examples are provided below

- Bossuyt et al (2004) performed acclimation experiments with D. magna and showed that after three generation of acclimation, the optimal concentration ranges (from energy reserves and number of offsprings) remained constant between 1 and 35 µg Cu/L. Below 1 µg Cu/L (a concentration often used as background copper concentration in the ecotoxicity media), copper deficiency was clearly observed, and the optimal concentration (highest energy reserve) was observed at 11 µg Cu/L. Toxicity was observed above 35 µg Cu/L.

- A decreased growth of O. mykiss below 7.8 µg Cu/l and above 16 µg Cu/l indicated a copper optimal concentration range between 8 & 16 µg Cu /l (Seim et al, 1984).

- Fort et al (2000) determined optimal copper concentration ranges for the early life stages of the amphibia X. laevis : 1 to 10 µg Cu/L

- On the other hand, for example, Johnson et al (2007) showed that neither of the two acclimated microalgae (Nitzschia closterium acclimated to 25 µg Cu/L and Chlorella sp. acclimated to 4 µg/L)   had increased copper tolerance compared with that of the non-acclimated algae, nor were there any changes in control growth rates.

The cellular mechanism of copper toxicity/deficiency, as well as the cellular mechanisms of copper homeostasis are largely preserved through evolution. The molecular mechanism of copper homeo-stasis, is related to 2 key elements: P-type ATPases that can pump copper across biological mem-branes in either direction and copper chaperones are important for the intracellular copper homeostasis (Odermatt et al, 1992). This cellular copper homeostasis mechanism is considered as being univer-sal as the sequences of copper chaperones are highly conserved between species (Wunderli et al. 1999).  Besides these active regulation mechanisms, some groups of organisms have developed ad-ditional internal regulation mechanism (molecular binding and sequestration) as a strategy to cope against copper excess (Rainbow, 1998; Marsden and Rainbow, 2004). Vertebrate dietary copper exposure studies (fish, mammals and birds) further demonstrated that intestinal absorption/biliary excretion of copper is regulated with varying dietary intakes (WHO, 1998).

The key indicator of copper toxicity is disturbance of the sodium homeostasis. The key target tissue for copper toxicity is therefore the water/organism interface, with cell wall and gill-like surfaces acting as target biotic ligands (Playle, 1993).

The aquatic hazard assessment related to copper excess is based on the results of tests that were carried out with soluble copper compounds, having Reliability  scores 1 or 2, and expressed as Cu2+ in a quantitative way (i.e., based on measured values). Tests carried out with organisms that were acclimated to low copper concentration have not been used as a selection criterium. Other parameters that were considered for assigning the Reliability score included the use of soluble test compound, use of standard test species in test that are conducted in line with standard guidances, the presence of clear concentration-response relationships, the use of ecotoxicological relevant endpoints (e.g. mortality, reproduction, growth and development) and the absence of confounding parameters that may affect the outcome of the test.

### Acute Toxicity Test Results and hazard identification

To identify the acute aquatic hazard of copper and copper compounds, literature searches have including key citation indices (Biological abstracts, Poltox, Science citation index 1988-2001) as well searches through the Copper Research Information Forum (Univ. British Colombia, USA –up to 2006). Over 5000 separate acute copper values, ranging between 0.2 µg/L (Daphna magna 72hr LC50) and 20,000 µg/L (Pimephales promelas 96 h LC50) were retained.

To derive a reliable baseline data-set, only high quality data obtained for the standard OECD test species and endpoints were retained for further assessment:

- Fish Oncorhynchus mykiss, Brachydanio rerio, Cyprinus carpio, Pimephales promelas, Lepomis macrochirus: LC50 values from 96 hours exposure duration

- Crustacaea  Daphnia magna and Ceriodaphnia dubia:  L(I)C50 values from 48 hours exposure duration

- Algae and higher plants: Pseudokirchneriella subcapitata, Chlamydomonas reinhardtii, Chlorella vulgaris, Lemna minor: EC50 (growth rate) from minimal exposure duration of 24 hour.

The quality criteria used for retaining acute toxicity test include: well described test set-up and test media characteristics, pH and hardness within the range of the OECD test-protocols, quantified dose-response relationship and reported L(E)C50 value as measured test concentrations.  Annex VIII provides an overview of the retained acute toxicity data.

In total, 451 high quality acute data points were retained. For the fish 262 individual data points were selected for 5 standard species. For the invertebrates 123 individual data points were selected for 2 standard species and for the algae 66 individual data points were selected for 3 standard species. Considering that the data were obtained from open literature instead of GLP tests, their reliability is described as reliability 2.

Reliable L(E)C50 values for the standard test species range between 3 and 9,150 µg Cu/L (Pseudokirchneriella subcapitata, Chlamydomonas reinhardtii, Chlorella vulgaris, Ceriodaphnia dubia, Daphnia magna, Oncorhynchus mykiss, Pimephales promelas, Lepomis macrochirus, Brachydanio rerio and Cyprinus carpio).

For most species a large variation among effect values for the same test species and effect parameter is observed, and this variation can be explained by the observed variation of the abiotic conditions of the test media such as pH, hardness, and dissolved organic carbon. Each of these parameters affect the bioavailability and toxicity of free copper ion, and should therefore be considered when interpreting copper effects data. Several studies investigated the relationship between (speciation-dependent) copper toxicity to fish and physicochemical parameters (such as pH, dissolved organic carbon, hardness), and concluded that observed variation in copper toxicity was related to changes of one or more of these properties of the test medium (e.g. Waiwood et al, 1978; Ryan et al, 2004; Erikson et al, 2004; Grosell et al, 2007).

Table 26 summarizes the acute toxicity for 10 standard species under standard pH conditions (between pH 5.5 and 8.5). The data across pH ranges (pH 5.5-8.5) show species-specific geometric mean values ranging between 34 µg Cu/L (Ceriodaphnia dubia) and 2837 µg Cu/L (Lepomis macrochirus).  The data also show that intra-species acute hazards from soluble copper can vary with orders of magnitude.

Table 26 : Overview of the acute freshwater L(E)C50  values  at different pH ranges

| Acute L(E)C50 – values for standard species and endpoints - µg dissolved Cu/L Geomean values (ranges, number of data) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Species & Endpoint | pH 5.5-6.5 | pH 6.5-7.5 | pH 7.5-8.5 | pH 5.5-8.5 |
| Pseudokircheriella subcapitata |  |  |  |  |
| Biomass growth | 116.6 (84-194, n=3) | 118.1 (32-245, n=4) | 50.8 (16.5-113, n=7) | 77.2 (84-245, n=14) |
| Growth rate – BLM data | 347.8 (122-824, n=8) | 131.4 (35-685, n=15) | 138.8 (51-346, n=10) | 169.2 (35-824, n=33) |
| Chlamydomonas reinhardtii |  |  |  |  |
| Growth rate | 380.0 (---, n=1) | 315.0 (---, n=1) | 146.0 (---, n=1) | 259.5 (146-380, n=3) |
| Chlorella vulgaris |  |  |  |  |
| Growth rate | 582.9 (333-987, n=5) | 232.7 (60-446, n=7) | 214.4 (99-506, n=4) | 296.1 (60-987, n=16) |
| Plants Biomass Growth rate | (84-194, n=3) (122-987, n=14) | 118.1 (32-245, n=4) (35-685, n=23) | (16.5-113, n=7) (51-506, n=15) | (84-245, n=14) (35-987, n=52) |
| Daphnia magna |  |  |  |  |
| Mortality | 36.9 (33.8-40.9, n=4) | 92.9 (7-792, n=10) | 52.1 (9.8-529, n=63) | 55.1 (7-792, n=77) |
| Mortality – BLM data | 279.0 (106-465, n=4) | 203.1 (92.6-1213, n=9) | 298.4 (100-826, n=12) | 259.5 (92.6-1213, n=25) |
| Ceriodaphnia dubia |  |  |  |  |
| Mortality | 25.0 (9.5-56, n=4) | 47.3 (28-84, n=4) | 29.8 (8.5-200, n=13) | 34.4 (8.5-200, n=21) |
| Invertebrates Mortality | (9.5-465, n=12) | (7-1213, n=23) | (8.5-826, n=88) | (7-1213, n=123) |
| Oncorhynchus mykiss |  |  |  |  |
| Mortality | 29.6 (4.2-82.2, n=7) | 61.25(2.8-890, n=18) | 95.4 (9.5-516, n=25) | 69.2 (2.8-890, n=50) |
| Brachydanio rerio |  |  |  |  |
| Mortality | --- | 35.0 (---, n=1) | 149.0 (---, n=1) | 72.2 (35-149, n=2) |
| Cyprinus carpio |  |  |  |  |
| Mortality | --- | --- | 805 (800-810, n=2) | 805 (800-810, n=2) |
| Pimephales promelas |  |  |  |  |
| Mortality | 8.1 (4.4-15, n=2) | 151.2 (5.9-1400, n=33) | 225.2 (12.4-1060, n=161) | 203.5 (4.4-1400, n=196) |
| Lepomis macrochirus |  |  |  |  |
| Mortality | --- | 1005 (1000-1010, n=2) | 5509 (4250-9150, n=3) | 2837.5 (1000-9150, n=5) |
| Fish Mortality | (4.2-82.2, n=9) | (2.8-1400, n=54) | (9.5-9150, n=192) | (2.8-9150, n=255) |

Table 27 summarizes the pH-specific ecotoxicity data obtained from soluble copper compounds (Table 26), with the transformation/dissolution data of sparingly soluble CuO and Copper-metal (Table 3). The lowest species-specific geometric mean L(E)C50 values recorded at pH 5.5-6.5, 6.5-7.5 and 7.5-8.5 (Table 26) are respectively 25, 35 and 30 µg Cu-ions/L and considered as acute Environmental Reference Values for Cu-ions (acute ERVs Cu-ion).  The comparative acute environmental hazard profile of the soluble copper compounds can then be derived through molecular weight translations of the acute ERVs Cu-ion.  The comparative acute environmental hazard profile for sparingly soluble CuO and copper can be derived from the ratio between the transformation/dissolution and the acute ERV-Cu-ion.

The hazard assessment of the substances in the category, derived from the L(E)C50 values (µg dissolved Cu/L) and a molecular weight translation or the results from transformation/dissolution tests, show that the copper compounds and the fine copper powders do pose a hazard to the environment. Coarse granules and massive copper materials release less copper ions and therefore pose less or no hazard to the environment under typical environmental conditions. The assessment shows that due to the differences in copper releases from the various copper compounds, coated flakes, copper powder and copper massive distinct hazard profiles can be distinguished.

Table 27 :  Acute hazard identification of Cu materials considering molecular weight translation or transformation/dissolution tests (from

Table 3 and Figure 4) and acute ecotoxicity reference values (from Table 26).

|  |  | Acute ERV at different pHs between 5.5 and 8.5 | 7 days transformation/dissolution |  |
| --- | --- | --- | --- | --- |
| Material | Molecular weight | L(E)C50 | Loading | Test result |
|  |  | µg/L | mg Cu/l | µg dissolved Cu/l |
| CuSO4.5H2O | 249.7 | 98 (pH6), 137(pH7),118 (pH8) | Fully soluble |  |
| Cu2Cl(OH)3 | 213.6 | 42(pH6), 59(pH7),50 (pH8) | Fully soluble |  |
| Cu2O | 143.09 | 28(pH6), 39(pH7), 34(pH8) |  |  |
| CuO | 79.6 | 25(pH6), 35(pH7), 30 (pH8) | 1 | 49(pH6), 5(pH7), 1 (H8) |
| Coated copper flakes (2000-3000 mm2/mg) | 63.55 | 25(pH6), 35(pH7), 30 (pH8) | 1 | 721 (pH6), 363 (pH7) |
| Fine 10µm Copper powder (67 m2/mg) | 63.55 | 25(pH6), 35(pH7), 30 (pH8) | 1 | 9- 44 (pH6)** |
| Copper massive (<0.67 mm2/mg) | 63.55 | 25(pH6), 35(pH7), 30 (pH8) | 1 | 1 (pH6)** |
| Copper granules/massive (<0.43 mm2/mg) | 63.55 | 25(pH6), 35(pH7), 30 (pH8) | 100 | 6-19 (pH6)** |
| * acute ERV of the Cu-ion (lowest species specific geometric mean LC(50) and molecular weight translation in absence of transformation/dissolution |  |  |  |  |
| ** 5 times lower values are expected at pH 7 and 8 |  |  |  |  |

### Chronic Toxicity Test Results and hazard identification

The chronic ecotoxicity dataset (Annex IX to XI) for copper was developed for hazard and risk assessment purposes and includes standard and non-standard species/protocols/endpoints.  The data-quality was assessed, considering: information on test set-up and test medium (within the boundaries of OECD and BLMs), measured copper concentrations in the test media, information on controls, dose-response, reported NOEC or L(E)C10 as measured concentration.

Data are grouped per trophic level and when data on more than one endpoint were available for an organism, a geometric mean per endpoint was determined and the lowest value was put forward as species mean value. These values have already been used in a regulatory context for setting generic safe water quality criteria (e.g. Cu EU-voluntary risk assessment, REACH Chemical safety reports for copper and copper compounds).

The ecotoxicity data-set covers literature searches and focused research carried between 1988 and 2006.  The key quality criteria are summarized below:

Description of test material and methods: Tests should be performed according standard operational procedures. A detailed description of methods employed in the study should be provided. Including, but not limited to preparation of the test solutions (environment), timing of administration and observations recorded, etc.

Test duration: The relevance of specific exposure times for the estimation of chronic effects for organisms with relative long life times (fish) was evaluated in more details on a case by case basis considering sensitive life stages.

Chemical analysis:  Only effect levels based on actual (measured) concentrations have been found reliable.

Test acceptability:  Minimal requirements for endpoints such as mortality, growth, reproduction (e.g. control mortality for chronic exposure < 20 %). In addition there must be evidence that the concentration of the substance being tested has been satisfactorily maintained, and preferably it should be least 80% of the nominal concentration throughout the test (OECD, 1983).

Concentration-effect relationships: Clear dose response relationships should be observed. Because effect concentrations are statistically derived values, information concerning the statistics should be used as a criterion for data selection.

High quality chronic NOEC/(L(E)C10 values are retained for 10 fish species: Ictalurus punctatus, Oncorhynchus kisutch, Oncorhynchus mykiss, Salvelinus fontinalis, Pimephales promelas, Pimephales notatus, Perca fluviatilis, Noemacheilus barbatulus, Catostomus commersoni and  Esox Lucius (see Annex IX). The following endpoints were evaluated for one or more different species: mortality, growth rate, reproduction (nr offspring, #eggs per female (or per spawn), and % of eggs hatched). Individual high quality NOEC/(L(E)C10 values from different studies range between 2.2 μg/l Cu for the rainbow trout Oncorhynchus mykiss (endpoint: growth) to 188 μg/l Cu for the perch Perca fluviatilis (endpoint: mortality) .

With regard to chronic toxicity, it is generally recommended to focus  on the studies where sub-lethal effect concentrations are reported. For copper, the mechanism of action (osmo-regulatory interference) often results in small differences between mortality and sub-lethal effects (e.g. for D.magna). Hence the inclusion of chronic mortality data is relevant for copper compounds.

Reliable chronic data were identified for 13 different invertebrate species, representing different classes: 1 rotifer species (Brachionus calyciflorus); 3 insect species (Clistoronia magnifica; Chironomus riparius; Paratanytarsus parthenogeneticus), 4 mollusc species (Juga plicifera, Campeloma decisum; Villosa iris; Dreissenia polymorpha), 5 crustacean species (Ceriodaphnia dubia; Daphnia magna; Daphnia pulex; Hyalella azteca; Gammarus pulex) (see Annex X). Individual NOEC/(L(E)C10 values range between 4 μg Cu/L (Ceriodaphnia dubia) and 181 μg Cu/L (Daphnia magna reproduction). Covered endpoints for this trophic level are predominantly growth, reproduction and mortality, but also include e.g. life cycle (C. magnifica), population response (G. pulex) and filtration rate (D. polymorpha). The species geometric mean values for freshwater invertebrates are situated between 6 µg Cu/L (J .plicifera) and 50.3 µg Cu/L (H. azteca).

High quality chronic NOEC/(L(E)C10 algae values are available for 3 species: Pseudokirchnerella subcapitata, Chlamydomonas reinhardtii and Chlorella vulgaris (see Annex XI). One high quality NOEC is available for a higher plant, i.e. duckweed Lemna minor. For these species the conventional endpoints biomass and growth rate were covered and growth rates were retained. Species mean values for algae/higher plants range from 30 µg Cu/L (L. minor) to 138 µg Cu/L (C. vulgaris).

Table 28 summarizes the ecotoxicity data for the standard OECD species/ endpoints under standard pH conditions (pH 5.5-8.5), whereby the information is split by pH to allow for a direct comparison between the ecotoxicity data and the transformation/dissolution data as input into GHS hazard assessments.   The lowest pH-specific, geometric mean values (by species and endpoint) are considered as Chronic Environmental Reference Value (Chronic ERV-Cu-ions):  20, 6.3 and 11.4 µg Cu/L at respectively pH 6, 7 and 8.

The chronic environmental hazard identity of the soluble compounds such as CuSO4 is derived through molecular weight translation of these chronic ERVs-Cu-ions (Table 29). The chronic environmental hazard identity for CuO and copper can be derived from comparison between the results from the transformation/dissolution with the chronic ERVs-Cu-ions (Table 29).

Table 28 : Species and pH specific geometric mean NOEC/L(E)C10 values for the most sensitive endpoints (µg Cu/L). R: reproduction, M= mortality, G= growth; n= number of data-points for that endpoint.

|  | Geomean NOEC or L(E)C10 (µg Cu/L) |  |  |  |
| --- | --- | --- | --- | --- |
| Endpoint | pH 5.5-6.5 | pH 6.5-7.5 | pH 7.5-8.5 | pH 5.5-8.5 |
| Ceriodaphnia dubia |  |  |  |  |
| Mortality |  | 8.7 (4-19, n=2) | 49.4 (20-122,n=2) | 20.8 (4-122, n=4) |
| Reproduction | 20 (n=1) | 6.3 (4-10, n=2) | 14.8 (6.3-31.6, n=4) | 12.1 (4-31.6, n=7) |
| Daphnia magna |  |  |  |  |
| Mortality |  |  | 36.8 (n=1) | 36.8 (n=1) |
| Reproduction | 24.5 (21.5-28, n=2) | 181 (n=1) | 64.7 (36.8-106, n=4) | 56.8 (21.5-181, n=7) |
| Reproduction - BLM develop | 54.5 (29.4-93.1, n=9) | 96.4 (31.8-213, n=18) | 112 (78.6-153.1, n=11) | 88 (29.4-213, n=38) |
| Growth |  |  | 12.6 ( n=1) | 12.6 (n=1) |
| Oncorhynchus mykiss |  |  |  |  |
| Mortality |  | 25.9 (24-28,n=2) | 11.4 (n=1) | 19.7 (11.4-28, n=3) |
| Growth |  | 9.9 (2.2-45, n=2) | 13.5 (11.4-16 , n=2) | 11.6 (2.2-45, n=4) |
| Pimephales promelas |  |  |  |  |
| Mortality |  | 7.1 (4.8-10.6, n=2 | 44.9 (33-61) ( n=2) | 17.9 (4.8-61, n=4) |
| Reproduction |  | 10.6 (n=1) | 26.3 (14.5-66, n=5) | 22.6 (14.5-10.6, n=6) |
| Growth |  | 7.1 (4.8-10.6, n=2) | 44.3 (33-59.5, n=2) | 17.8 (4.8-59.5, n=4) |
| Salvelinus fontanilis |  |  |  |  |
| Mortality |  | 14.6 (9.5-21, n=4) | 22 (n=1) | 15.9 (9.5-22.3, n=5) |
| Reproduction |  | 18.1 (7-49, n=3) |  | 18.1 (7-49, n= 3) |
| Growth |  | 12.5 (7-21, n=4) | 22 (n=1) | 14 (7-22.3, n=5) |
| Lemna minor |  |  |  |  |
| Mortality | 30 (n=1) |  |  | 30 (n=1) |
| Chlamydomonas reinhardtii |  |  |  |  |
| Growth | 62.6 (22-178, n=2) | 108 (n=1) | 96 (n=1) | 79.8 (22-178, n=4) |
| Chlorella vulgaris |  |  |  |  |
| Growth |  |  |  |  |
| Pseudokirchneriella subcapitata |  |  |  |  |
| Growth | 67.6 (53-95, n=3) | 104 (65-164, n=2) | 28.5 (15.7-56.4, n=6) | 45.6 (15.7-164, n=11) |
| Growth BLM development | 128.2 (49-337, n=9) | 56.1 (17-203, n=15) | 61.3 (25-171, n=11) | 71.8 (17-337, n=35) |

Table 29 : Chronic hazard identification of Cu materials considering results from molecular weight translation or transformation/dissolution tests (from Table 3) and chronic ecotoxicity reference values (from Table 28) at various pH.

|  | Molecular weight | Chronic ERV* at different pHs between 5.5 and 8.5 | Solubility or 28 days transformation/dissolution (TD) |  |
| --- | --- | --- | --- | --- |
| Material |  | NOEC/L(E0)C10 | Loading | Result TD test |
|  |  | µg/L | mg Cu/l | µg dissolved Cu/l |
| CuSO4.5H2O | 249.7 | 79(pH6), 25(pH7), 45(pH8) | fully soluble |  |
| Cu2Cl(OH)3 | 213.6 | 34(pH6), 10(pH7), 21(pH8) | fully soluble |  |
| Cu2O | 143.09 | 22(pH6), 7(pH7), 14(pH8) |  |  |
| CuO | 79.5 | 20(pH6), 6(pH7), 11(pH8) | 1 | 210(pH6), 9(pH7) , 1 (pH8) |
| Coated copper flakes (2000-3000 mm2/mg) | 63.55 | 20(pH6), 6(pH7), 11(pH8) | 1 | 773(pH6), 639 (pH7) |
| Copper powder (107 mm2/mg - 0.067 mm2/mg) | 63.55 | 20(pH6), 6(pH7), 11(pH8) | 1 | 37-176(pH6)** |
| Copper massive (<0.67 mm2/mg) | 63.55 | 20(pH6), 6(pH7), 11(pH8) | 1 | 3(pH6)** |
| * chronic ERV of the Cu-ion (lowest species and endpoint specific geometric mean L(E)C10 or NOEC and molecular weight translation in absence of transformation/dissolution. |  |  |  |  |
| ** 5 times lower values are expected at pH 7 and 8 compared to pH 6 |  |  |  |  |

The assessment shows a similar relative ecotoxicity profile as observed from acute toxicity data: due to the differences in copper releases from the various copper compounds, coated flakes, copper powder and copper massive, distinct chronic hazard profiles need to be distinguished.

The hazard assessment of the substances in the category, derived from the chronic L(E)C10 (µg dissolved Cu/L) and a molecular weight translation or the results from transformation/dissolution tests, shows that the copper compounds and the fine copper powders do pose a chronic hazard to the environment. Coarse granules and massive copper materials release less copper ions and therefore pose less or no chronic hazard to the environment under typical environmental conditions.

### Derivation of safe threshold values for various aquatic environment, including bio-availability assessment and correction of effects data

Considering the standard acute and chronic hazard profiles of copper and copper compounds, environmental safe threshold values were derived, considering the results from single-species chronic ecotoxicity studies (standard and non-standard species/protocols/endpoints) and multi-species mesocosm studies.

Single-species studies

As previously indicated and elaborated in the OECD monograph (ENV/JM/MONO(2012), copper bioavailability and toxicity to aquatic organisms is influenced by abiotic parameters, such as pH, hardness and dissolved organic carbon (DOC).

It is therefore critical to consider the impact of abiotic factors on observed effects when deriving environmental relevant safe threshold values and conducting risk assessments for copper. The abiotic conditions for each of the chronic ecotoxicity data are therefore provided in Annex 9 to Annex 11.

For the risk characterisation of copper, the significant impact of abiotic factors on copper toxicity has raised the need to use a bioavailability normalisation process when using chronic ecotoxicity data for setting water quality standards (e.g. PNEC in EU legislation, CRV in an US regulatory framework). Acute and chronic Biotic Ligand Models were developed in order to provide a mechanistic basis for understanding and predicting bioavailability from chemical parameters (e.g. pH, hardness, DOC) and biological parameters (receptor sites on organism, mode of action).  These models allow the translation of an existing ecotoxicity value that was generated under specific physical-chemical conditions (pH, hardness, DOC), to a value that is relevant for another test medium.

Chronic BLM models have been developed/validated for 10 species, representing the three basic trophic levels (algae, invertebrates and fish):

- a unified chronic model for the algae (Pseudokirchneriella subcapitata, Chlamydomonas reinhardtii and Chlorella vulgaris). The applicability of the model for predicting higher plant ecotoxicity (hydrocultures of Barley) was demonstrated

- a chronic BLM for invertebrates (Daphnia magna). The potential of the BLM for predicting copper toxicity to other invertebrates was demonstrated from copper toxicity studies with Brachyonus calyciflorus, Lampilis siliquoidea, Hyridella depressa and Hyalella azteca

- a unified chronic model for 2 fish species (Pimephales promelas and Oncorhynchus mykiss).

The boundaries of the BLM applicability are provided in Table 30.

Table 30:  Applicability of the copper freshwater BLM (hardness in mg CaCO3/L; DOC in mg/L) (from http://echa.europa.eu/web/guest/copper-voluntary-risk-assessment-reports)

.

The BLMs developed for chronic fish (P. promelas and O. mykiss), invertebrates (D. magna) and algae (P. subcapitata) can be used for the normalisation of all retained chronic NOEC values of respectively fish, invertebrates and algae/plant species. Briefly, the bioavailability normalisation process normalises the NOEC/(L(E)C10 values to site-specific physicochemical conditions (i.e. pH, hardness and dissolved organic carbon (DOC). With the normalised NOEC/(L(E)C10 values it is possible to construct Species Sensitivity Distributions for a range of physico-chemical conditions in different types of surface waters, and HC5,50%  values can be derived from these SSDs. Within the context of REACH (EU-regulation) several HC5,50% values were determined for different types of EU surface waters. The BLM derived HC5,50% values as low as 1 µg/L but typical values for EU Surface waters range between 7 and 30 µg Cu/L. More information is available from ENV/JM/MONO (2012).

Mesocosm-studies

Schaefers et al (2003) investigated the effects of a permanent copper sulphate exposure to a lentic aquatic system for a period of 110 days simulating early summer to autumn in indoor semi-realistic microcosms including phyto- and zooplankton, macrophytes, and benthic invertebrates. A total number of sixty different taxonomic groups were assessed in this study. With respect to water qualities and bioavailable exposure comparable to that of the microcosms tested the NOEAC (No Observed Ecologically Acceptable Concentration) and LOEAEC (Lowest Observed Ecologically Adverse Effects Concentration) for this study were set to 20 µg/L  and 40 µg Cu/L total dissolved copper. At the NOEAC, some significant effects occurred. However, all these effects were slight and mainly temporary effects without permanent impact on ecosystem quality, neither in structure nor in function (standing crop).

Markedly lower NOECs of 4.0 µg/L (total copper)/ 3.6 µg/L (dissolved copper) were reported by Hedtke (1984) for primary producers in the water column. At the end of the study (32 weeks after the starting exposition), gross primary production was significantly lower at 9.3 µg/L total Cu, respectively 8.8 µg/L dissolved Cu. The macroalgae Vaucheria, showed significantly lower abundances at 8.8 µg dissolved Cu/L (reduction around 71 % compared to controls). However, based on observed abundances of different taxonomic groups (including cladoceran zooplankton groups), the authors concluded that macro-invertebrate abundance, microbial population activities and periphyton growth were less sensitive than primary producers or zooplankton species. The lowest concentration that affected a species of these taxonomic groups was 25 µg dissolved Cu/L (reduced survival of snails Viviparus).  The main conclusion of this study was that substantial structural effects were observed only at 25 µg dissolved Cu/L.

A third, 18 month mesocosm study was conducted by Roussel (2005). This study allowed the evaluation of the effects of copper on the structure and function of freshwater ecosystems using experimental streams that allowed the realisation of an ecologically realistic study while controlling many parameters (contaminant exposure, water and sediment quality, antecedent of biotic and abiotic material, etc.). Community structure of phytoplankton, periphyton, macrophytes, zooplankton, macroinvertebrates, emerging insects, aquatic hyphomycètes and population dynamics of three-spined sticklebacks were monitored. The 75 µg/L treatment revealed a decreased abundance of macrophytes, zooplankton, macro-invertebrates and an increased abundance of periphyton, emerging insects and fish, and taxa richness was lowered in all communities in the 75 µg/L treatment. This study highlighted the interest of studying both ecosystem structure and function to identify a range of responses as symptoms of ecosystem dysfunctions. Considering all results, a NOEAEC ecosystem was set at 4 µg/l (as dissolved), with a LOAEC of 20 µg Cu/L.

In addition, it is noteworthy that detailed comparisons between the BLM predicted and observed mesocosm effects, demonstrated that the developed BLM could adequately predict the mesocosm sensitivity within a factor of 2.

It can thus be concluded that the ecotoxicity of copper to aquatic organisms is explained by:

- The toxicity response is species-specific and related to water-borne exposure.

- The mechanism of copper-induced toxicity is disturbance of the sodium homeostasis. The key target tissue for copper toxicity is therefore the water/organism interface, with cell wall and gill-like surfaces acting as target biotic ligands (e.g. Playle, 1993, Grosell et al., 2002; De Schamphelaere and Jannsen, 2004).

- The information on the ecotoxicity following exposures, through water and/or food and the information on metal bio-accumulation and homeostasis explain the observed small ratios between mortality and sub-lethal endpoints (typically a factor of 1 to 3).

- A Species Sensitivity Distribution constructed, using the non-normalised species-mean NOEC values for the most sensitive endpoints, resulted in a log normal HC5,50%  of 6 µg Cu/L. The toxicity response strongly depends on the abiotic properties of the test medium (pH, hardness, dissolved organic carbon).   BLM derived safe threshold values, typically ranging between 7 and 27 µg/L (see further)

- Considering that a clear “definition” of deficiency is difficult to provide and deficiency levels ranging between 1 and 10 µg Cu/l have been reported.  Acclimation was not used as a selection criterium for copper toxicity data.

- Toxicity responses, dependent on background levels and thus acclimation of the organisms, has been observed  for e.g. Daphnia magna: Research related to copper acclimation and deficiency (Bossuyt et al, 2004) demonstrated that after three generation of acclimation, the optimal concentration ranges (from energy reserves and number of offspring) remained constant between 1 and 35 µg Cu/L. Below 1 µg Cu/L (a concentration often used as background copper concentration in the ecotoxicity media), copper deficiency was clearly observed.

Figure 9: Energy reserves (Ea) of first (triangles) and 4th to 15th  (squares) generation D. magna acclimated to different copper concentrations.  Error bars represent standard deviations. Mean levels for same letter are not significantly different at p<00.5. Ovals represent the optimal concentration range.  From Bossuyt et al., 2004

Johnson et al. (2007), however,  reported that  Chlorella sp., acclimated to 3.4 µg dissolved/L  (3.8 µg  total copper),  did not show  increased copper tolerance compared with that of the non-acclimated algae (1.4 µg/L total and dissolved copper), nor were there any changes in control growth rates.

Uncertainty analysis

The ecotoxicity data-set covers literature searches and focused research carried between 1988 and 2006.

A summary and evaluation of the ecotoxicity database and derived safe threshold values (HC5,50%) values therefore is provided below.

Data quality: From the evaluation of the NOEC data quality and quantity it can be concluded that the Cu-baseline database is of high quality, covers complete life stages and has built-in factors providing additional degrees of protection.

Important built-in conservative factors are:

- Copper is homeostatically regulated. Acclimation to changes in copper concentrations in the aquatic environment have however not been accounted for in this assessment. Further, it is worthwhile noting that 60% of the laboratory test waters had copper background levels below the range of copper background levels as reported by Zuurdeeg et al. (1992) for European lowlands. This leads to further protection and conservatism as several studies have demonstrated that organisms acclimated to low copper levels in culture waters were 1-3 times more sensitive. It should be noted, however, that this is not always the case; Johnson et al (2007) did not observe such a difference between acclimated and non-acclimated micro-algae species (N.closterium, Chlorella sp.)

- It has been demonstrated that overestimation of toxicity values often occur for tests performed in flow through systems, characterized by separate inflow of test substance and test medium, because the Cu-DOC complexes in such systems have not yet been fully formed (Ma et al., 1999).  In comparison to natural surface waters, the laboratory flow-through test set-up will indeed have higher free cupric ion activity, which leads to lower NOECs (µg dissolved copper/L). In this effects data-set, 43% of the retained single species NOECs were derived from flow through systems: 100% of the fish tests, 16% of the invertebrate tests and 3% of the algae tests were carried out in flow-through system. The difference in sensitivity due to the non-equilibrated Cu-DOC in laboratory systems versus natural surface waters therefore leads to a build-in conservative factor that has not been accounted for.

Higher toxicity values in algal studies can also be related to the increasing pH during the tests, often happens in algal tests (at static conditions).

The mechanism of action and taxonomic groups covered: The copper aquatic effects database contains a large number of high quality chronic NOEC values for 27 species.  The use of statistical extrapolation, using all NOECs in the ecotoxicity database to derive a reference value for setting water quality criteria (e.g. HC5,50 ) is therefore preferred rather than the use of the assessment factor method on the lowest NOEC.

The information further shows similar modes of actions across species:

  - The cellular mechanism of copper toxicity/deficiency as well as the cellular mechanisms of copper homeostasis have been largely preserved through evolution. The key indicator of copper toxicity is disturbance of the sodium homeostasis. The key target tissue for copper toxicity is therefore the water/organism interfaces with cell wall and gill-like surfaces acting as target biotic ligands (Playle, 1993). This explains the relative low acute to chronic ratio’s observed for sensitive species such as D. magna (typically a factor of 1 to 3).

  - The observed differences in copper toxicity among species were further shown to be related to the organisms body size (Grosell et al., 2007).  The copper database includes taxonomic groups/endpoints with small sizes (eg crustaceans and rotifers, larval stages of insects and molluscs), indicating that sensitive organisms are covered in the database.

  - Additional, it is useful to mention that the information on the mechanism of action, the homeostatic regulatory mechanisms, the observed small acute L(E)C50 to chronic NOEC ratio’s, the comparison of the effects data obtained from exposures through respectively water and food and the information on copper accumulations across trophic chains do not indicate towards concern from secondary poisoning nor trophic chain transfer.

Based on the taxonomic groups assessed, the small inter-species variability (after BLM normalization) and the mode of action, it can be concluded that the effects database covers key taxonomic groups and endpoints.

Treatment of data for one species: Detailed investigation of the ecotoxicity database covers realistic environmental conditions. The original database showed large variations among multiple values from the same species.  A sensitivity analysis was performed to understand the relative importance of bioavailability versus acclimation/adaptation of the organisms to a range of copper concentrations.  The results clearly showed that bioavailability was the most important factor explaining the large intra-species variability.  Safe threshold values may be over- or underestimated without due considerations of local/regional bio-availability.

Statistical uncertainties around the 5th percentile estimate (HC5,50) : The log-normal and beta distribution function results in the ’best fitting’ SSD. Similar to the Ni-SIAR, based on the statistical uncertainty analysis, it can be concluded that the HC5,50% is a statistical robust determination.

HC5,50% versus HC5-5 values was also determined from individual NOECs instead of the species mean NOEC values (log-normal distribution). The individual NOEC-HC5,50% values are similar to the HC5,50% values derived from species mean NOECS. A consistent HC5-50/HC5-5 ratio of 1.2 is observed across all scenarios for the log-normal distribution, based on all individual NOEC values.

Evaluation of NOEC values below the HC5,50%:  Only one or two out of the 27 species-specific NOECs falls under the HC5,50% value.  The observed similarity in mechanism of action among invertebrates, including the most sensitive species, Brachionus calyciflorus, as well as the large ecotox database and small difference between the NOEC for Brachionus calyciflorus and the SSD-HC5,50% (<-factor 1.4) does demonstrate the applicability of statistical extrapolation to the copper database

Summary of the derived HC5,50% and conservative elements built into the HC5,50% : From the above summary, it is concluded that the copper aquatic effects database contains a large number of high quality chronic NOEC values for 27 species, covering sensitive life stages and taxonomic groups.  The Biotic ligand models have been validated and applied to the toxicity database. The uncertainty and sensitivity analysis of the data and data treatment clearly demonstrate the robustness of the derived copper HC5,50% values

Some key conservative elements have been incorporated in the HC5,50% settings proposed:

- The 50th % confidence limit is used for hazard and risk assessment purposes

- Overestimation of toxicity values often occur for tests performed in flow through systems because the Cu-DOC complexes have not yet been formed in the tests media.

- No acclimation of the organisms (no limit was set to the copper in the test media).  The influence of background values was tested and demonstrated that acclimation may decrease the sensitivity up to a factor of 3.

Some key further uncertainties have also been for addressed are:

- The HC5,50% is conservative, especially as acclimation was not accounted for

- The HC5,50%  can protect sensitive waters as demonstrated by BLM calculations for sensitive waters

- The HC5,50% is also protective to multi-species systems as demonstrated from mesocosm studies

Remaining uncertainties and further recommendations

- The SIAR has limited data on sensitivity of fish at low pHs.  Additional investigations at pH below 7 would be useful.

- The ecotoxicity data were derived under broad ranges of pH and hardness, even outside of the ranges recommended by OECD.  Test organisms were often not pre-acclimation to these sub-optimal test conditions and this may have influenced the ecotoxicity results relevant to copper exposures.  Appropriate acclimation of organisms to the test media are recommended for any further testing

- The data-base covers ecotoxicity searches till 2006.  Including more recent ecotoxicity data would benefit the assessment.

- The database is further discussed and updated during the discussions on harmonized classifcations of biocidal copper forms and compounds in the EU (2014—2017 and onwards).

## Marine Effects

Considering the differences in physiology of marine versus freshwater organisms as well as the differences in physicochemistry of marine versus freshwater environments, the ecotoxicity data for marine organisms has been collected separately and used as a basis for the derivation of a marine HC5-50.

The single-species ecotoxicity data-set covers literature searches and focused research carried between 1988 and 2006. The marine mesocosm study was finalized in 2010.

Marine environmental safe threshold values were derived, considering the results from single-species chronic ecotoxicity studies (standard and non-standard species/protocols/endpoints) and multi-species mesocosm studies.

Single-species studies

The copper marine effects database contains high quality, chronic NOECs/EC10s values for 24 species, representing three trophic levels; fish (n=2), invertebrates (n=18) and micro/macro-algae (n=4) (see annexes XII to XIV).  Where appropriate a species mean value was calculated, and species-specific geometric mean values range between 2.9 µg Cu/ L (Phaeodactylum tricornutum) and 145 µg Cu/L (Penaeus monodon).

High quality chronic single-species NOEC/L(E)C10 values are available for 2 species of marine fish (Table 31). Individual NOEC/L(E)C10 values range between 55 and 123μg Cu/L, both values for Atherinops affinis, reproduction (hatchling growth parameters).

High quality marine invertebrate NOEC/L(E)C10 were retained for 18 different individual species belonging to different taxonomic groups: 6 mollusc species (Mytilus edulis; Prototheca staminea; Crassostrea gigas, Mercenaria mercenaria, Mytilus galloprovincialis, Placopecten magellanicus), 1 annelid species (Neanthes arenaceodentata) 3 decapod (crustacean) species (Pandalus danae; Penaeus mergulensis; Penaeus monodon), 3 copepod (crustacean) species(Eurytemora affinis; Tisbe battagliai, Tisbe furcata), 1 arthropod (crustacean) species (Artemia franciscana), 1 echinoderm species (Paracentrotus lividus), 3 cnidaria species (Acropora tenuis, Goniastrea aspera, Lobophytum compactum) . Individual data are summarized in Table 32. The lowest NOEC/EC10 for each invertebrate species ranged between 5.9 µg Cu/L (M.galloprovincialis, endpoint development) and 51.1 µg Cu/L (E.affinis, endpoint mortality, fecundity). The species- specific NOEC values range from 6.0 μg Cu/L for bivalve Mytilus edulis to 145 μg Cu/L for the crustacean Penaeus monodon.

High quality chronic NOEC values have been identified for four marine algal species, namely two diatoms (Phaeodactylum tricornutum and Skeletonema costatum) and 2 macroalgae (Macrocystis pyrifera and Fucus vesiculosis) (Table 33). The individual NOEC values range between 2.9 µg Cu/L (Phaeodactylum tricornutum, endpoint growth) to 50 µg Cu/L (Macrocystis pyrifera, endpoint germination).

High quality “species mean” NOEC values are derived for the most sensitive endpoint for each of the 4 species of marine algae. The  “species mean" NOEC value range from 2.9 µg/l Cu for the diatom P. tricornutum (endpoint: growth rate) to 11 µg/l for F. vesiculosis (endpoint: growth rate). It should be noted that higher effect values for F. vesiculosus (ranging between 18.5 and 46 µg Cu/L) were not considered as dissolved organic carbon (as humic acid) was added to the test medium (natural filtered seawater), hereby directly affecting the typical bioavailability  (and toxicity) of copper in the marine environment.

Table 31: Summary of reliable chronic Cu toxicity data for marine fish; reported values represent measured concentration levels.

| Species | Min-max range of individual values (µg Cu/L), species mean1 and number of data points | References & Reliability scoring |
| --- | --- | --- |
| Atherinops affinis | Min-max: 55 – (123) (embryo/young abnormalities, hatchability ; n=9) (Species meanhatchability: 55 ; n=1) | Anderson et al, 1991 (R2) |
| Cyprinodon variegatus | Min-max: 57.8 – (109) (survival, embryo development, hatchability ; n=4) (Species meandevelopment: 57.8; n=1) | Hurd, 2006b (R1) |

1: Species mean :  most sensitive endpoint-specific geometric mean value for the species

Table 32: Summary of reliable chronic Cu toxicity data for marine invertebrates; reported values represent measured concentration levels

| Species | Min-max range of individual values (µg Cu/L), species mean1 and number of data points | References & Reliability scoring |
| --- | --- | --- |
| Penaeus monodon | 145 (growth) (n=1) | Ahsanullah and Ying, 1995 (R2) |
| Penaeus mergulensis | 33 (growth) (n=1) | Ahsanullah and Ying, 1995 (R2) |
| Mytilus edulis | Min-max: 6.0 – 6.2 (embryo development, growth rate; n=2) (Species meangrowth rate: 6.0 ; n=1) | Brooks, 2006a (K2) ; Redpath, 1985 (R2) |
| Mytilus galloprovincialis | Min-max: 5.9 – 9.7 (embryo development ; n=4) (Species meandevelopment: 7.93) | Rosen, 2005 (R2) |
| Crassostrea gigas | Min-max: 10.4 – 47.1 (embryo development ; n=6) (Species meandevelopment: 18.3) | Brooks, 2006 (R2) |
| Paracentrotus lividus | Min-max: 8.8 – 16.5 (embryo development ; n=2) (Species meandevelopment: 12.1) | Lorenzo et al, 2006 (K2) ; Hurd, 2006a (R1) |
| Neanthes arenaceodentata | Min-max: 12.1 – 13.5 (growth ; n=2) (Species meangrowth: 12.8) | Pesch et al, 1986 (R2) |
| Protothaca staminea | 18 (mortality : n=1) | Roesijida, 1980 (R2) |
| Pandalus danae | 9.9 (development, mortality ; n=2) | Young et al, 1979 (R2) |
| Eurytemora affinis | 51.1 (mortality, fecundity, maturation) ; n=1) | Hall, 1997 (R2) |
| Tisbe battagliai | 18 (reproduction, development, mortality) ; n=3) | Williams and Hayfiend, 2006 (R1) |
| Tisbe furcate | 19.1 (reproduction, survival ; n=1) | Bechmann, 1994 (R2) |
| Placopecten magellanicus | 10 (gonad development) ; n=1) | Gould et al, 1988 (R2) |
| Mercenaria mercenaria | 7 (development ; n=1) | LaBreche et al, 2002 (R2) |
| Artemia franciscana | 6.6 (hatching success ; n=1) | Brix 2006 (R2) |
| Acropora tenuis | 17.3 (larval settlement ; n=1) | Reichelt-Brushett and Harrison, 2004 (R2) |
| Goniastrea aspera | 14.2 (motility ; n=1) | Reichelt-Brushett and Harrison, 2004 (R2) |
| Lobophytum compactum | 36 (fertilisation success ; n=1) | Reichelt-Brushett and Harrison, 2004 (R2) |

1: Species mean :  most sensitive endpoint-specific geometric mean value for the species

Table 33: Summary of reliable chronic Cu toxicity data for marine algae and higher plants; reported values represent measured concentration levels.

| Species | Min-max range of individual values (µg Cu/L), species mean1 and number of data points | References & Reliability scoring |
| --- | --- | --- |
| Macrocystis pyrifera | Min-max: 10.2 – (50.1) (sporophyte growth, germ tube growth, germination ; n=3) (Species meangrowth: 10.2) | Anderson et al, 1990 (R2) |
| Fucus vesiculosis | Min-max: 11 – 46 (growth ; n=4) (Species meangrowth: 112) | Brooks, 2006 (R2) |
| Phaeodactylum tricornutum | Min-max: 2.9 – 5.7 (growth rate ; n=2) (Species meangrowth: 4.07) | Simpson et al, 2003 (R2) ; Smyth and Kent, 2006 (R1) |
| Skeletonema costatum | 7.5 (growth rate ; n=1) | Smyth, 2006 (R1) |

1: Species mean :  most sensitive endpoint-specific geometric mean value for the species

2: other tests were conducted in modified seawater

Bioavailability

Similar to the findings for the freshwater environment, it can be seen that there is sometimes a large intra-species variability for the same endpoint, and variability of test medium properties may lay at the basis of these differences.

Marine waters are characterised by high pH (typically around 8.3), high salinity (35%) and high ionic strength. Unlike the inorganic composition of marine waters, DOC levels may vary considerable between marine water bodies.  Baltic Sea, North Sea and Mediterranean coastal waters have DOC level ranging between 1-10 mg/L.  Open Ocean waters usually has lower DOC, ranging between 0.5 and 1.8 mg/L. Therefore, a DOC correction has already been implemented for marine waters.

A full marine/estuarine BLM is under development.

A relation between the observed NOEC and organic carbon content was established for Mytillus edulis. Its applicability to other invertebrate species was demonstrated for the molluscs Crassostreas gigas, Paracentrotus lividus and Mytilus galloprovincialis, and for the echinoderms Dendraster exentricus and Strongylocentrotus purpuratus, (Arnold et al, 2008 and 2010, Hall 2010 and Brooks 2006). For the marine macro-algae Fucus vesiculosis it was also demonstrated that changes in dissolved organic carbon content of the test medium could explain this variation (Brooks 2006) .

With all the species mean values that are provided in Table 31 to Table 33, a Species Sensitivity Distribution was constructed. The outcome of this analysis resulted in an HC5,50% of 4.6 µg Cu/L.

An organic carbon normalisation was carried out and the HC5,50% was derived at a DOC levels representative of coastal and open ocean areas (2 and 0.2 mg/L). From the high quality data, HC5,50% values of respectively 5.2 and 1.3 µg Cu/l, were derived.

Mesocosm-study

An outdoor marine mesocosm study was performed by Foekema (2010) to investigate the effects of continuous exposure to a series of actively maintained concentrations of dissolved copper. Various macro invertebrate species were added to each mesocosm in known numbers, and the different populations were allowed to acclimatise for 33 days. No indication of effect of the treatment was seen at the lowest two test concentrations. Clear direct adverse effects, were observed in the two highest treatments (16 and 31 µg Cu/L, respectively). Affected species/endpoints were zooplankton, bivalves and sponges and, to a lesser extent, the periphyton (sessile algae) development and the shell growth of gastropods. The phytoplankton concentration and related primary production, however, increased as a response to the reduced grazing pressure. In the intermediate treatment (9.9 µg Cu/L) some effects on periphyton development and the zooplankton community were observed, but these were of short-term character. However, the reproduction success of the bivalve cockle (Cerastoderma edule) was strongly negatively affected. This adverse ecological effect formed the most sensitive endpoint of this study. Therefore the NOECmesocosm and the NOEAECmesocosm for this study are equal; i.e., 5.7 µg Cu/L. The LOEC is 9.9 µg Cu/L.

As DOC has an important impact on copper toxicity, the quality of the DOC was assessed (Smith et al. 2010). The main conclusions from the DOC characterisation were that based on these analyses the water from the mesocosms looked very similar to water from pristine natural environments with low anthropogenic or terrestrial inputs.

Uncertainty analysis.

Data quality : From the evaluation of the NOEC data quality and quantity it can be concluded that the Cu-marine database is of high quality, covers complete or sensitive life stages and has built-in factors providing additional degrees of protection. Two important built-in conservative factors are:

- Copper is homeostatically regulated. Acclimation to changes in copper concentrations in the marine environment have however not been accounted for.   Some of the test organisms have been retained from open ocean - copper deficient- systems and their susceptibility to increased copper exposures is thus expected to be higher than the ones observed in organisms acclimated to coastal systems.

- As for the freshwater compartment, most marine ecotoxicity studies were carried out without pre-equilibration between Cu and DOC

The mechanism of action and taxonomic groups covered: Information on copper’s mode of action, having a similar basis (iono-regulation) as observed for freshwater organisms, provides confidence in the taxonomic groups covered:

- Grosell et al., 2007 demonstrated that the sensitivity of fish across a full salinity gradient is related to osmoregulatory disturbance (similar to what is observed for freshwater organisms).  The authors demonstrated further that not the absolute ionic concentrations but the sodium gradient from the blood plasma to the water,  influences the copper sensitivity across the salinity gradient  The latter mechanism explained the observed higher copper sensitivity of Fundulus heteroclitus in freshwater, followed by seawater and waters of intermediate salinity.

- The key indicator of copper toxicity is thus, as for freshwater, the disturbance of the sodium homeostasis. The key target tissue for copper toxicity is therefore the water/organism interface with cell wall and gill-like surfaces acting as target ligands in all species investigated. This results in relative small observed overall inter-species variability in marine NOECs (max/min ratio of 34 (after normalization for DOC) for 24 species).

- The copper database includes taxonomic groups/endpoints with small sizes (eg crustaceans, larval stages of molluscs and fish), indicating that sensitive organisms and life stages are covered in the marine database.

Based on the taxonomic groups assessed, the small inter-species variability (after organic carbon normalization) and the mode of action, it can be concluded that the effects database covers key taxonomic groups and endpoints.

Treatment of data for one species: Detailed investigation of the ecotoxicity database covers realistic environmental conditions in marine waters, covering DOC ranges between 0.2 mg DOC/L (open ocean) and 2 mg DOC/L (typical coastal).   The data demonstrate that there is no statistical difference between the observed copper effects-DOC relationships among taxonomic groups and thus allows to perform a DOC normalization, applicable to all taxonomic groups.  The application of the DOC normalization to all taxonomic groups investigated, allows the prediction of all observed NOEC/EC10 values (derived under varying DOC levels) within a factor of 2.

Statistical uncertainties around the 5th percentile estimate: Parametric statistical analysis was undertaken using the BestFit software (Palisade Inc.) and the ETX model.  The different parametric curve fittings were ranked by statistical tests of fitness.  The data were also assessed with the semi-parametric kernel density estimation.  Based on the parametric tests, HC5,50% values, derived from the Q1 data-set, coastal zone (2 mg DOC/L) scenario range between of 4.4 µg Cu/L (log normal) to 5.2 µg Cu/L( Best-fit). The HC5,50% value derived from the semi-parametric Kernel Density Estimation was 4.8 µg Cu/L.

To evaluate the robustness of the HC5,50% values derived, several sensitivity analysis were performed: (1) using only NOECs/EC10s from “truly filtered” systems; (2) inclusion of the Q2 and Q3 NOECs/EC10s; (3) considering varying DOC quality.  The HC5,50% values obtained from non-rejected parametric distributions range between 4.3 and 6.0 µg Cu/L with a median value of 5.2 µg Cu/L  (at 2mg DOC/L). Based on the statistical uncertainty and sensitivity analysis, it was concluded that an HC5,50% value of 5.2 µg Cu/L is a robust, statistically sound HC5,50% determination, applicable to coastal waters. The sensitivity analysis shows that the uncertainty around this value is very low.

Evaluation of NOEC values below the HC5,50%: The lowest species-specific NOEC (4 µg Cu/L for Phaeodactylum) is only a slightly below the HC5,50% value (5.2 µg Cu/l), derived with the best fitting.  Having one NOEC below the HC5,50% is thus merely a statistical phenomenon related to the large number of species in the database (24 species).  Comparing the HC5,50% values with the species-specific NOECs obtained from the Q1+Q2+Q3 dataset shows that the species (geometric) mean NOEC values for the most sensitive endpoints have increased: for Phaeodactylum 4 µg l-1 (Q1 only) to 13.4 µg l-1 (all data).  The Q3 dataset (64 species-mean NOECs) returns another species with species mean NOEC at or below the HC5,50% (Spisula solidissima, with a NOEC of 5 µg Cu/L). Having one NOEC below the HC5,50% is thus again merely a statistical phenomenon related to the large number of species in the database (64 species).

Comparison to background levels and copper deficiency levels

Nutrient deficiency, including copper deficiency is documented in open ocean scenarios.  For the OSPAR regions, dissolved background copper levels, estimated from off shore samples, range between 0.050 and 0.360 µg/L (OSPAR,2001). The HC5,50% derived for the open ocean (0.2 mg/L) is 1.3 µg Cu/L and thus slightly above the background levels of an open ocean system.

Copper levels in the controls of the retained Q1 ecotoxicity tests were < 4.3 µg Cu/l with a median value of 2.4 µg Cu/L.   The Q1 HC5,50% values derived from best fitting distributions (1.3 to 5.2 µg Cu/l ) is thus around the copper levels in control test media.

Summary of the derived HC5,50% and conservative elements built into the HC5,50%: From the above summary, it is concluded that the copper marine effects database contains a large number of high quality chronic NOEC values (NOECs for 24 species), covering sensitive life stages and taxonomic groups.  The organic carbon normalization was validated for 6 species and applied to the whole toxicity database. The uncertainty and sensitivity analysis of the data and data treatment clearly demonstrate the robustness of the derived copper HC5,50% values and do not indicate for an additional need of an AF on the HC5,50% values.

Some key conservative elements have been incorporated in the HC5,50% settings proposed:

- The 50th % confidence limit is used

- Overestimation of toxicity values often occur for tests performed in non-equilibrated systems because the Cu-DOC complexes have not yet been formed in the tests media.

- No acclimation of the organisms

Additional remaining uncertainties

- The assessment covers a DOC correction.  To be applicable to estuarine waters further investigation on the influence of salinity would be useful.

- The data-base covers ecotoxicity searches till 2006.  Including more recent ecotoxicity data would benefit the assessment.

## Terrestrial Effects

The ecotoxicity data-set covers literature searches and focused research carried between 1988 and 2005. Detailed monitoring of the EU soil characteristics was performed in 2010 and the soil threshold values revised accordingly between 2008 and 2010.

Soil safe threshold values were derived, considering the results from single-species chronic ecotoxicity lab and field studies (standard and non-standard species/protocols/endpoints) and multi-species mesocosm studies (Table 34, Table 35).  Bio-availability was incorporated in the assessment.

Chronic Toxicity Test Results

The terrestrial effects data set for copper covers 252 good quality single-species chronic NOEC/EC10 values representing different trophic groups (micro-organisms, plants, invertebrates). Additional information on 8 single species studies in field contaminated soils and 5 multi-species studies (freshly spiked and field contaminated) provide additional information on the toxicity of copper under field conditions. A summary of the individual data on terrestrial organisms is given in Table 34 and Table 35. Data for microbial processes a given in Annexes XV to XVII.  Detailed information, including the physical-chemical properties of the tested soil samples (pH, organic carbon content, clay content, CEC) is also provided in Annex.  Similar to the findings that were demonstrated in the aquatic compartment, soil properties have a major impact on terrestrial copper bioavailability and toxicity, and explain the observed intra-species variability. In this context several copper bio-availability regression models have been developed which can be used for the normalisation of existing ecotoxicity data to the same environmental conditions.

Considering the importance of differences in toxicity of copper to terrestrial organisms between lab spiked soils and field contaminated soils, information from freshly spiked soils and aged soils are both included in the database.

Chronic terrestrial invertebrate data:

The invertebrate (including arthropods) effects data set includes 108 bounded NOEC/(L(E)C10 values (Table 34); hard and soft bodied organisms with different exposure routes and feeding strategies belonging to 10 different species and 6 different families (i.e. Eisenia andrei, Eisenia fetida, Lumbricus rubellus belonging to the family of the Lumbricidae; Cognettia sphagnetorum to the family of the Enchytraedae; Isotoma viridis, Folsomia candida, Folsomia fimetaria to the family of the Isotomidae; Hypoaspis aculeiferto the family of the Laelapidae, Platynothrus peltifer to the family of the Camisiidae , Plectus acuminatus to the family of the Plectidae).

Individual high quality NOEC/(L(E)C10 values from different studies range from 8.4 mg/kg for Eisenia andrei (endpoint: cocoon production) to 1,460 mg/kg for Folsomia candida reproduction. Remarkably low NOEC values are found in some tests that used Eisenia species. The lowest value is found for E. andrei reproduction (8.4 mg total Cu/kg) in a natural soil (a German standard soil often used in toxicity tests, LUFA 2.2). This value is below the limit for essentiality. Van Gestel et al (1989) actually warn against use of cocoon production as reliable endpoint for Eisena fetida.

Two bioavailability regression models have been developed for terrestrial invertebrates: the E. fetida model, representing soft-bodied species and the F. candida model representing hard-bodied species. With this model it was possible to explain the observed intra-species variability, and both models have been used in a regulatory contexts for setting safe copper concentration levels in different types of soil.

Table 34: Summary of reliable chronic Cu toxicity data for terrestrial invertebrate species; reported values represent total measured concentration levels.

| Species | Min-Max range (mg Cu/kgdw), endpoints and #of data points | Reference & Reliability scoring |
| --- | --- | --- |
| Cognettia sphagnetorum | Min-max: 73.7 – 465.7 (growth, fragmentation ; n=4) (Species meangrowth: | Augustsson and Rundgren, 1998 |
| Eisenia andrei | Min-max: 8.3 – >325.2 (cocoon production, juvenile production, growth, reproduction ; n=9) (Species meangrowth: | Van Dis et al, 1988; Van Gestel et al, 1989,1991; Svendsen and Weeks, 1997a; Kula and Larink, 1997; Criel et al, 2005 |
| Eisenia fetida | Min-max: 12.4 – 728.2 (mortality, cocoon production, juvenile production, growth, reproduction ; n=27) (Species meangrowth: | Spurgeon et al, 1994; Spurgeon and Hopkin, 1995; Kula and Larink, 1997; Scotts-Fordsmand et al, 2000; Criel et al, 2005 |
| Folsomia candida | Min-max: 31 – 1460 (mortality, growth, reproduction, instantaneous rate of population increase ; n=34) (Species meangrowth: 12.8) | Sandifer and Hopkin, 1996,1997; Rundgren and Van Gestel, 1988; Herbert et al, 2004; Criel et al, 2005 |
| Folsomia fimetaria | Min-max: 141 – 1015 (mortality, growth, reproduction ; n=17) | Scotts-Fordsmand et al, 1997,2000a,Pedersen et al, 2000,2001 |
| Hypoaspis aculeifer | 179.2 (reproduction ; n=1) | Krogh and Axelsen, 1998 |
| Isotoma viridis | 55.2 – 403.2 (growth ; n=2) | Rundgren and Van Gestel, 1988 |
| Lumbricus rubellus | Min-max: 54 – 162 (mortality, growth, reproduction, , litter breakdown ; n=10) (Species meangrowth: 12.8) | Ma, 1984 ; Svendsen and Weeks, 1997 ; Spurgeon et al, 2004 |
| Plectus acuminatus | 35.2 (juvenile production ; n=1) | Kammenga et al, 1996 |
| Platynothrus peltifer | 68.2 (juvenile production ; n 3) | Van Gestel and Doornekamp, 1998 |

Chronic terrestrial plant data:

The plant effect records include 67 high quality single-species chronic NOEC/EC10 values covering monocotyle and dicotyle plants including agricultural and wild species belonging to 9 different species and 5 different families: (Polygonum convolvulus– family of the Polyonaceae; Lycopersicon esculentum– family of the Solanaceae; Hordeum vulgare, Avena sativa, Pao annua– family of the Poaceae; Senecio vulgaris, Andryala integrifolia, Hypochoeris radicata– family of the Asteraceae; Lolium perenne– family of the Gramineae) (Table 35).

Individual high quality NOEC/(L(E)10 values from different studies range between ranging from 18 mg/kg for Hordeum vulgare to 698 mg/kg for Lycopersicon esculentum. Important intra-species variability in NOEC/L(E)C10 values are observed due to differences in the physico-chemistry of the soils.   For plants, 2 models were developed (Rooney et al, 2004 and 2006):  L. esculentum model (endpoint yield) and H. vulgare root elongation model. These models can be used for the normalization of the plant NOEC data to the same environmental conditions.

Additional research on the influence of soil chemistry, soil ageing and soil leaching on the plant toxicity has been conducted and reported in open literature (Ginochio et al., 2006; Zhao et al., 2006; Chander and Brooks, 1993; Oorts 2006b), and should also be taken into account when assessing copper toxicity in the terrestrial compartment. Strandberg et al. (2006) further showed that plant community composition was significantly correlated with soil copper concentration and community composition at soil copper concentrations above 200 mg/kg differed significantly from community composition at lower copper levels.

The studies on soil attenuation, reported by Ma et al (2006, 2006b) are also of relevance for correctly assessing copper toxicity to terrestrial plants

Table 35: Summary of reliable chronic Cu toxicity data for terrestrial plant species; reported values represent total measured concentration levels.

| Species | Min-Max range (mg Cu/kgdw), endpoints and #of data points | Reference & Reliability scoring |
| --- | --- | --- |
| Polygonum convolvulus / Fallopia convolvulus | Min-max: 137 – 222 (mortality, total plant yield, reproductive dry matter, seed biomass, shoot yield, root yield) ; n=6) | Kjaer and Elmegaard, 1996; Pedersen et al, 2000 |
| Avena sativa | Min-max: 204 – 258 (grain yield ; n=5) | De Haan et al, 1985 |
| Lolium perenne | 106 (stem yield, root yield ; n=2) | Jarvis, 1978 |
| Hordeum vulgare | Min-max: 18 – 322 (stem growth/yield, root length, yield, leaves growth, seedling emergence ; n=22) | Ali et al, 2004 ; Rooney et al, 2004 |
| Lycopersicon esculentum | Min-max: 36 – 698 (stem yield ; n=21) | Rooney et al, 2004 ; Rhoads et al, 19891 |
| Senecio vulgaris | Min-max: 186– 339 (mortality, reproductive dry matter, seedling emergence ; n=3) | Brun et al, 2003 |
| Poa annua | Min-max: 200– 537 (mortality, reproductive dry matter, seedling emergence ; n=3) | Brun et al, 2003 |
| Andryala integrifolia | Min-max: 234 – 236 (mortality, seedling emergence ; n=2) | Brun et al, 2003 |
| Hypochoeris radicata | Min-max: 339– 350 (mortality, reproductive dry matter, seedling emergence ; n=3) | Brun et al, 2003 |

1: 3 values reported by Rhoads et al (1989) represent added Cu-levels (range: 175-350 mg Cu/kgdw)

Chronic data on microbial processes in soil:

The effect records related to microbial processes include 77 NOEC/EC10 values; 9 different endpoints representing the C- and N-cycle and measurement of microbial biomass are available (i. e. maize induced respiration, substrate induced respiration, litter decomposition, glutamic acid decomposition, N-mineralisation, denitrification, nitrification, ammonification, biomass C, biomass N). All data are presented in Annex.

Individual high quality NOEC/(L(E)C10 values from different studies range from 30 mg/kg (glucose respiration)) to 2,402 mg/kg (maize respiration). Important intra-species variability in NOEC/L(E)C10 values are observed due to differences in the physico-chemistry of the soils.   For microbial processes, 3 bioavailability models were developed: the nitrification process model, the maize respiration model (using a natural substrate) and the substrate induced respiration model (Smolder and Oorts 2004 and Oorts, 2006a). These models can be used for the normalization of the NOEC data for microbial processes to similar environmental conditions.

Derivation of soil safe threshold values

To derive reasonable worst case HC5,50% value for soils across Europe, the data from GEMAS-project (Geochemical Mapping of Agricultural and Grazing Land Soil project, Reimann et al.,2014) allow calculating site-specific predicted HC5,50% for Cu in 4331 soil samples taken from arable and grazing lands across Europe (Reimann, et al, 2009).

The HC5,50% values range between 13 and 205 mg Cu/kg dry weight, depending on the soil chemistry.  A reasonable worst case 10th percentile of 69.6 mg Cu/kg dry weight is retained for the grazing land. A reasonable worst case 10th percentile of 59.5 mg Cu/kg dry weight is retained for the arable land.   The overall soil median reasonable worst case value across the two land-types is 64.6 mg Cu/kg dry weight (Oorts and van Nederkassel, 2010)

Uncertainty analysis.

Database: 252 chronic NOEC/EC10 values are available:  67 plant data, 108 invertebrates’ data, 77 micro-organisms data.  Data cover 3 trophic group, include 19 species from several taxonomic groups and 9 microbial endpoints; data cover the range of EU soils.

-9 different plant species –including agricultural and wild species, monocotyls and dicotyls- from 5 different families

-10 different invertebrate species –including hard- and soft-bodied organisms with different feeding strategies- from 6 different families

-measurement of microbial biomass and 6 different microbial functions representing the C- and N-cycle

There is no specific group which is significantly more sensitive to copper, giving confidence in the representativeness of the database for the terrestrial ecosystem. This is also confirmed by the additional species tested in field and multi-species studies.

Treatment of data from one species:  For several species multiple data are available for the same endpoint. Large differences in effects of copper can be explained by differences in bioavailability.

A total of 7 regression models were derived to predict toxicity of copper to terrestrial organisms for a wide range of soil types: 2 for plants (1 monocotyle and 1 dicotyle plant); 2 for invertebrates (1 for soft-bodied and 1 for hard-bodied invertebrates); 3 for micro-organisms functions (1 related to the N-cycle and 2 related to the C-cycle).

The parameter explaining best the variability in toxicity for most of the endpoints is the CEC.

Application of these models on the database significantly lowered the variability in the toxicity data were multiple data are available for the same endpoint. The intra-species and functions variability after normalization was drastically reduced to remaining levels of variability which can be expected between laboratories (inter-lab variability assessed in ringtests) and allowed the derivation of meaningful geometric mean NOEC/EC10 values for each endpoint. For each species, the lowest endpoint-specific geometric mean value was used as input into the SSD.

A sensitivity analysis was made to assess the influence of applying one single model for invertebrates and one single model for plants. For invertebrates, this resulted in a smaller reduction of the variability and less good predictions of the toxicity values. For plants, no differences were found.

An additional benefit of these models is that it allows the derivation of soil or soil type specific SSDs and so HC5,50%.

A conservative leaching-ageing factor is available to correct for differences between lab and field due to leaching-ageing effects. This factor is based on the 25-P of an extensive ecotoxicity dataset.

Statistical uncertainty: A comparison was made of the uncertainty around the HC5 between the best fitting distribution and the log-normal distribution. The actual model for the best-fitting distribution depends on the soil scenario.

The statistical uncertainty related to the HC5 value is reflected in the small differences between the HC5 and the HC5,50%, ranging between a factor 1.0 to 1.3, depending on the soil scenario and fitting distribution.

Evaluation of NOEC values below the HC5,50%: Only 1 single datapoint out of 28 is found below the HC5,50%. The LOEC value (14% effect) is however well above the HC5,50%.

Comparison of the HC5,50%, with the EU background and essentiality levels: A comparison of the HC5,50% with the background (natural forest soils) and essentiality levels indicates the HC5,50% to be above the background levels and essentiality levels.

Mesocosm and field data: A total of 8 single species studies are available in which the toxicity of Cu to micro-organisms, invertebrates and plants in field contaminated aged soils was investigated for a wide range of European soil types (peaty, sandy, clay).

A total of 5 multi-species studies are available, 3 of which studied the effects of copper in freshly spiked soils and 2 in field contaminated aged soils. Several of the soils are expected to be more sensitive to copper (sandy soils, low pH, low CEC). Invertebrates, plants and micro-organisms were studied.

Single species and multi-species studies indicate the HC5,50% to be protective in the field.

Additional remaining uncertainties

- The data-base covers ecotoxicity searches till 2006.  Including more recent ecotoxicity data would benefit the assessment.

# ConCLUSIONS and Risk Characterisation

Copper is a natural element and an essential nutrient to all life forms. In freshwater, marine waters and soils, copper deficiencies and copper toxicities may be observed, depending on the exposure levels. Copper is well regulated, not biomagnified, and BCF and BAF values have no meaning for a hazard assessment.

The release and bio-availability of copper ions are critical to the essentiality and hazard profiles of copper containing materials. Information on molecular weight, solubility, rates of environmental transformation/dissolution and bio-elution in body fluids are useful parameters for read-across between substances of the category.

Environment

Copper is a data-rich category, with information from single-species and multi-species tests. The information allows to build a weight of evidence for the derivation of safe threshold values based on statistical extrapolation (HC5,50%  values).

Considering the importance of bio-availability, these can be considered for freshwater, marine and terrestrial compartments.  For soil, the ageing of copper was observed and allowed the derivation of a lab to field ageing factor.

Copper and copper compounds may present a hazard for the environment based on the release/bioaccessibility of copper ions, which depends on the conditions of the receiving environment (pH, hardness, presence and type of organic matter, anions and competing cations). Adequate screening-level data are available to characterize the environmental hazard for the purposes of the Cooperative Chemicals Assessment Programme.

Notes: A voluntary risk assessment of copper and copper compounds was performed in the context of the EU Existing Substances Regulation. The data-review under EU biocidal and plant-protection products was not finalized in September 2014.

Human health

Three of the substances and one form of zero valent copper in this category present a hazard for human health, based on the release/bioaccessibility of copper ions (copper sulphate pentahydrate: severe eye irritation, acute toxicity (oral); copper(I) oxide: eye irritation, acute toxicity (oral and inhalation), repeated dose toxicity (inhalation); dicopper chloride trihydroxide: acute toxicity (oral & inhalation); copper flakes: acute toxicity (oral and inhalation) and eye irritation. Adequate screening-level data are available to characterize the human health hazard for the purposes of the Cooperative Chemicals Assessment Programme.

Notes: A voluntary risk assessment of copper and copper compounds was performed in the context of the EU Existing Substances Regulation9 .The data-review under EU biocidal and plant-protection products was not finalized in September 2014.

# references

Note: Several studies are “unpublished”. For more information on ownership, please contact K. Lacasse at ECI or Carol Mackie at Regulatory Compliance Limited

Abrahams PW, Thornton I, 1994. The contamination of agricultural land in the metalliferous province of southwest England – implications to livestock. Agric.Ecosyst.Environ. 48:125-137.

Aburto EM, Cribb AE, Fuentealba IC, Ikede BO, Kibenge FS, Markham F, 2001. Morphological and biochemical assessment of the liver response to excess dietary copper in Fischer 344 rats. Canadian J.Vet.Res. 65:97-103.

Adams WJ et al, 2003. Evaluating copper bioaccumulation in aquatic risk assessments. Cobre 2003 - Proceedings of the Symposium, Dec 2003, Santiago, 1-11.

Adamson M, Reiner B, Olson JL, Goodman Z, Plotnick L, Bernardini I, Gahl WA, 1992. Indian childhood cirrhosis in an American child. Gastroenterol. 102:1771-1777.

Agarwal K, Sharma A, Talukder G, 1990. Clastogenic effects of copper sulphate on the bone marrow chromosomes of mice in vivo. Mutation Res. 243:1-6.

Ahsanullah M, Ying W, 1995. Toxic effects of dissolved copper on Penaeus mergulensis and Penaeus monodon. Bull.Environ.Contam.Toxicol. 55:81-88.

Ali NA et al, 2004. Phytotoxicity and bioaccumulation of copper and chromium using barley (Hordeum vulgare L.) in spiked artificial and natural forest soils. Ecotox.Environ.Safe. 57(3):363-374.

Allen, MM, Barber RS, Braude R, Mitchell KG, 1961. Further studies on various aspects of the use of high-copper supplements for growing pigs. Brit.J.Nutr. 15:507-522.

Allinson G, Laurenson LJB, Pistone G, Stagnitti F, Jones PL, 2000. Effects of dietary copper on the Australian freshwater crayfish Cherax destructor. Ecotox.Safe. 46:117-123

Alva AK, Graham JH, Tucker DPH, 1993. Role of calcium in amelioration of copper phytotoxicity for citrus. Soil Sci. 155:211–218.

Amiard JC, Amiard-Triquet C, Metayer C, 1985. Experimental study of bioaccumulation, toxicity and regulation of some trace metals in various estuarine and coastal organisms. Symp.Biologica.Hung. 29:313-323.

Anderson BS, Hunt JW, Turpen SL, Coulon AR, Martin M, 1990. Copper toxicity to microscopic stages of giant kelp Macrocystis pyrifera: interpopulation comparisons and temporal variability. Mar.Ecol.Prog.Ser. 68:147-156.

Anderson BS, Middaugh, DP, Hunt JW, Turpen SL, 1991. Copper toxicity to sperm, embryos, and larvae of|topsmelt Atherinops affinis, with notes on induced spawning. Mar.Environ.Res. 31:17-35.

Aoyagi S, Baker DH, 1993. Bioavailability of copper in analytical-grade and feed grade inorganic copper sources when fed to provide copper at levels below the chick's requirement. Poultry Science 72:1075-1083.

Araki S, Murata K, Aono H, 1987. Central and peripheral nervous system dysfunction in workers exposed to lead, zinc and copper. A follow-up study of visual and somatosensory evoked potentials. Int.Arch.Occup.Environ.Health 59:177-187.

Araki S, Murata K, Uchida E, Aono H, Ozawa H, 1993. Radial and median nerve conduction velocities in workers exposed to lead, copper, and zinc: a follow-up study for 2 years.  Environ.Res. 61;308-316.

Araya M, Chen B, Klevay LM, Strain JJ, Johnson L-A, Robson P, Shi W, Nielsen F, Zhu H, Olivares M, Pizarro F, Haber LT, 2003. Confirmation of an acute no-observed-adverse-effect and low-observed- adverse-effect level for copper in bottled drinking water in a multisite international study. Reg.Tox.Pharmacol. 38:389-399.

Araya M, McGoldrick MC, Klevay LM, Strain JJ, Robson P, Nielsen F, Olivares M, Pizarro F, Johnson L, Poirier K, 2001. Determination of an acute no-observed-adverse- effect level (NOAEL) for copper in water. Reg.Tox.Pharmacol. 34:137-145.

Araya M, Olivares M, Pizarro F, Llanos A, Figueroa G, Uauy R, 2004. Community-based randomized double-blind study of gastrointestinal effects and copper exposure in drinking water. Environ.Health Persp. 1068-1073.Arnold W. Ray, J. S. Cotsifas, D. S. Smith, S. Le Page and K. M. Gruenthal (2008). A comparison of the copper sensitivity of two economically important saltwater mussel species and a review of previously reported copper toxicity data for mussels: important implications for determining future ambient copper saltwater criteria for the USA. published online in Wiley InterScience (www. interscience. wiley. com).

Arnold W. Ray; J. S. Cotsifas, R. S. Ogle, S. G. S. De Palma, D. S. Smith (2010). A comparison of the copper sensitivity of six invertebrate species in ambient salt water of varying dissolved organic matter concentrations. Env. Tox. and Chem. Vol.29, No2, pp 311-319.

Arshad M, Frankenberger WT, 1991. Microbial production of plant hormones. Plant Soil 133:1-8.

Arthur JW, Leonard EN, 1970. Effects of copper on Gammarus pseudolimnaeus, Physa integra and Campeloma decisum in soft water. J.Fish.Res.Bd.Canada. 27:1277-1283.

Aschengrau A, Zierler S, Cohen A, 1989. Quality of community drinking water and the occurrence of spontaneous abortion. Arch.Environ.Health: An International Journal 44(5):283-290.

Aschengrau A, Zierler S, Cohen A, 1993. Quality of community drinking water and the occurrence of late adverse pregnancy outcomes. Arch.Environ.Health 48:105-113.

Asgharian, Freijer, 1999. Multiple-path particle deposition model (MPPDep), Chemical Industry Institute of Toxicology (CIIT), USA; National Institute of Public Health and the Environment (RIVM), the Netherlands, and The Ministry of Housing, Spatial Planning and the Environment, the Netherlands, version 1.11

Askergren A, Mellgren M, 1975. Changes in the nasal mucosa after exposure to copper salt dust. A preliminary report. Scand.J.Work Environ.Health 1:45-49.

Augustsson AK, Rundgren S, 1998. The enchytraeid Cognettia sphagnetorum in risk assessment: advantages and disadvantages. Ambio 27:62-69.

Aulerich RJ, Ringer RK, Bleavins MR, Napolitano A, 1982. Effects of supplemental dietary copper on growth, reproductive performance and kit survival of standard dark mink and the acute toxicity of copper to mink. J.Animal Sci. 55:337-343.

Baker DH, Ammerman CB, (1995). Copper bioavailability. In: Ammerman CB, Baker DH, Lewis AJ (eds). Bioavailability of nutrients for animals: Amino acids, minerals and vitamins. Academic Press, Inc., pp. 127-156.

Baker DH, Odle J, Funk MA, Wieland TM, 1991. Research note: Bioavailability of copper in cupric oxide, cuprous oxide, and in a copper-lysine complex. Poultry Sci. 70:177-179.

Badilla-Ohlbaum, R. et al. (2001). Relationship between soil copper content and copper content of selected crop plants in central Chile. Environ Toxicol Chem 2001; 20(12):2749-57.

Ballatyne M, 1994. Study to determine the ability of copper II sulphate pentahydrate to induce mutation in five histidine-requiring strains of Salmonella typhimurium. Hazleton Europe. Report No. 456/31. Unpublished.

Barash A, Shoham Z, Borenstein R, Nabel L, 1990. Development of human embryos in the presence of a copper intrauterine device. Gynecol.Obstet.Invest. 29:203-206.

Barkoff JR, 1976. Urticaria secondary to a copper intrauterine device. Int.J.Dermatol. 15, 594-595.

Barlow SM, Knight AF, House I, 1981. Intrauterine exposure to copper IUDs and prenatal development in the rat. J.Repro.Fertility 62:123-30.

Barranco VP, 1972. Eczematous dermatitis caused by internal exposure to copper. Arch.Derm. 106:386-387.

Battersby R., 2004. Summary report - Dustiness and particle size testing of copper and copper compounds. EBRC Consulting GmbH, Hannover, 20.11.2004. Unpublished.

Barwick M, Maher W, 2003. Biotransference and biomagnification of selenium copper, cadmium, zinc, arsenic and lead in a temperate seagrass ecosystem from Lake Macquarie Estuary, NSW, Australia. Mar.Environ.Res. 56:471-502.

Bechmann RK, 1994. Use of life tables and LC50 tests to evaluate chronic and acute toxicity effects of copper on the marine copepod Tisbe furcata (Baird). Environ.Toxicol.Chem. 13(9):1509-1517.

Beck T, 1981. Untersuchungen über die toxische Wirkung der in Siedlungsabfällen häufigen Schwermetalle auf die Bodenmikroflora. Journal Z. Pflanzenernähr. Bodenk. 144:613-627.

Belanger, A., Levesque, M. P. & Mathur, S. P. (1987). The Influence of Variation in Soil Copper on the Yield and Nutrition of Radishes Grown in Microplots on Two Peat Soils. International Peat Journal. 2; 65-73.

Belanger SC, Cherry DS, 1990. Interacting effects of pH acclimation, pH and heavy metals on acute and chronic toxicity to Ceriodaphnia dubia (cladocera). J.Crust.Biol. 10(2):225-235.

Belanger SE et al, 1989. Effects of diet, water hardness and population source on acute and chronic copper toxicity to Ceriodaphnia dubia. Arch.Environ.Contam.Toxicol. 18:601-611.

Bem EM, Orlowski C, Piotrowski JK, Januszewski K, Pajak J, 1993a. Cadmium, zinc, copper, and metallothionein levels in the kidney and liver of inhabitants of Upper Silesia (Poland). Int.Arch.Occup.Environ.Health 65:57-63.

Bem EM, Kaszper BW, Orłowski C, Piotrowski JK, Wójcik G, Żołnowska E, 1993b. Cadmium, zinc, copper and metallothionein levels in the kidney and liver of humans from central Poland. Environ.Health Monitor.Assess. 25(1):1-13.

Bem EM, Piotrowski JK, Sobczak-Kozlowska M. Dmuchowski C, 1988. Cadmium, zinc, copper and metallothionein levels in human liver. Int.Arch.Occup.Environ. Health 60:413-417.

Bengtsson G et al, 1986. Effects of metal pollution on the earthworm Dendrobaena rubida (Sav.) in acidified soils. Water Air Soil Pollut. 28:361-383.

Benoit DA, 1975. Chronic effects of copper on survival, growth and reproduction of the bluegill (Lepomis macrochirus). Trans.Am.Fish.Soc. 104(2):353-358.

Benoit G, Oktay-Marshall SD, Cantu A, Hood EM, Coleman CH, Corapcioglu MO, Santschi PH, 1994. Partitioning of Cu, Pb, Ag, Zn, Fe, Al and Mn between filter-retained particles, colloids and solution in six Texas estuaries. Marine Chem. 45:307-336.

Bhandari R, Sharda B, 1982. Copper from cooking utensils as a cause of Indian childhood cirrhosis?  Arch.Dis.Child. 57:323.

Bhunya SP, Pati PC, 1987. Genotoxicity of an inorganic pesticide, copper sulfate in mouse in vivo test system. Cytologia 52:801-808.

Bien E, 1993. Guinea pig maximization test of skin sensitisation with “URA 17030”. Testing laboratory: International Bio Research. Report no.: 10-05-1961/00-92. Report date: 1993-03-02. Unpublished

Biesinger KA, Christensen GM, 1972. Effects of various metals on survival, growth, reproduction and metabolism of Daphnia magna. J.Fish.Res.Bd.Can. 29:1691-1700.

Birge, W. J. and Black, J. A. (1979). Effects of copper on embryonic and juvenile stages of aquatic animals. In: J. O. Nriagu (eds.). Copper in the Environment. Part 2. Health Effects. John Wiley & Sons; New York, NY, USA, p. 373-399.

Birge WJ, Benson WH, Black JA, 1983. Induction of tolerance to heavy metals in natural and laboratory populations of fish.

Blagden, S. M. (1999). Nordox Agro Grade: Acute Inhalation Toxicity (Nose only) Study in the Rat. Testing laboratory: Safepharm Laboratories Limited. Report no.: 148/014. Unpubmished

Bogomolov, D. M., Chen, S. K., Parmalee, R. W, Subler, S. & Edwards, C. A. (1996). An Ecosystem Approach to Soil Toxicity Testing: A Study of Copper Contamination in Laboratory Soil Microcosms. Applied Soil Ecology. 4: 95-105

Bollag J-M, Barabasz W, 1979. Effect of heavy metals on the denitrification process in soil. J.Environ.Qual. 8(2):196-201.

Borak J, Cohen H, Hethmon TA, 2000. Copper exposure and metal fume fever: lack of evidence for a causal relationship. Am.Ind.Hyg.Assoc.J. 61:832-836.

Bordo D, Pesce A, Bolognesi M, Stroppolo ME, Falconi M, Desideri A, 2001. Copper-zinc superoxide dismutase in prokaryotes and eukaryotes. In: Handbook of Metalloproteins (Vol 2), Messerschmidt A et al (Eds), John Wiley and Sons, Chichester.

Borgmann U (2005). Effect of major ions on the toxicity of copper to Hyalella azteca and implications for the biotic ligand model. Aquat Toxicol. 2005; 73(3):268-87.

Borgmann U, Norwood WP, Clarke C, 1993. Accumulation, regulation and toxicity of copper, zinc, lead and mercury in Hyalella azteca. Hydrobiologia 259:79-89.

Borgmann U, Ralph KM, 1983. Complexation and toxicity of copper and the free metal bioassay technique. Water Res. 17(11):1697-1703.

Bossuyt BT, Janssen CR, 2004a. Influence of multi-generation acclimation to copper on tolerance, energy reserves and homeostasis of Daphnia magna. Environ.Toxicol.Chem. 23(8):2029-2037.

Bossuyt BT, Janssen CR, 2004b. Long-term acclimation of Pseudokirchneriella subcapitata (Korshikov) Hindak to different copper concentrations: changes in tolerance and physiology. Aquatic Toxicol. 68(1):61-74.

Bresch, 1982. Investigation of the long-term action of xenobiotics on fish with special regard to reproduction. Ecotox.Environ.Safe. 6:102-112.

Bridges, C. M. et al. (2002). Comparative Contaminant Toxicity: Are Amphibian Larvae More Sensitive than Fish?. Bull. Environ. Contam. Toxicol.; 69:562–569.

Brix, K. V. and DeForest, D. K. (2000). Critical review of the use of bioconcentration factors for hazard classification of metals and metal compounds. Final Report: 1-71.

Brix KV et al, 2006) Effects of copper, cadmium and zinc on the hatching success of brine shrimp (Artemia franciscana). Arch.Environ.Contam.Toxicol. 51r:580-583.

Brooks S, 2006b. The effects of dissolved organic carbon on copper toxicity to the development of the oyster embryo. Cefas contract report C2548-1. Unpublished.

Brooks S, 2006c. The effects of dissolved organic carbon on the toxicity of copper to the marine macroalgae Fucus vesiculosis. Report no.: C2548-2. Unpublished.

Brooks S; 2006a. Copper speciation and toxicity to the development of mussel embryo. Cefas contract report C1921. Unpublished.

Brun LA, Le Corff J, Maillet J, 2003. Effects of elevated soil copper on phenology, growth and reproduction of five ruderal plant species. Environ.Pollut. 122(3):361-8.

Brungs WA, Geckle JR and Gast M (1976). Acute and chronic toxicity of copper to the fathead minnow in a surface water of variable quality. Water research, vol 10, pp37-43. Testing laboratory: US environmental protection agency, Newton Fish Toxicologu station, 3411 Church street, Cincinnati, OH 45244, USA.

Brungs WA et al, 1976. Acute and chronic toxicity of copper to the fathead minnow in a surface water of variable quality. Water Res. 10:37-43.

Buchanan SD, Diseker RA, Sinks T, Olson DR, Daniel J, Flodman T, 1999. Copper in drinking water, Nebraska, 1994. Int.J.Occup.Environ.Health 5:256-261.

Buckley JA, 1983. Complexation of copper in the effluent of a sewage treatment plant and an estimate of its influence on toxicity to Coho salmon. Water Res. 7(12):1929-1934

Buescher RG, Griffin SA, Bell MC, 1961. Copper availability to swine from Cu64 labelled inorganic compounds. J.Animal Sci. 20:529-531.

Bull PC, Thomas GR, Rommens JM, Forbes JR, Cox DW, 1993. The Wilson disease gene is a putative copper transporting P-type ATPase similar to the Menkes gene. Nat. Genet. 5:327-337.

Burki, H. R. and Okita, G. T. (1969). Effect of oral copper sulfate on 7,12-dimethylbenz(a) anthracene carcinogenesis in mice. Br. J. Cancer Sep; 23(3): 591-596.

Cage, S. (2003a). Copper compounds : In vitro dermal penetration study through human skin. Am. J. Clin. Nutr. 63, 797-811. Testing laboratory: Huntingdon Life Sciences Ltd. Study number: CSV 004/023929. Report date: 2003-07-14. Unpublished

Cage S, 2003. Copper compounds: In vitro dermal penetration study through human skin. Testing laboratory: Huntingdon Life Science Ltd., Huntingdon, UK. Report no.: CVS 004/023929. Report date: 2003-07-15. Unpublished.

Cai L, Koropatnick J, Cherian MG, 1995. Metallothienein protects DNA from copper-induced but not iron-induced cleavage in vitro. Chem-Biol. Interact. 96:143-155.

Cairns J Jr, Buikema AL Jr, Heath AG, Parker BC, 1978. Effects of temperature on aquatic organism sensitivity to selected chemicals. Bulletin 106.

Calamari D, Marchetti R, 1973. The toxicity of mixtures of metals and surfactants to rainbow trout (Salmo gairdneri rich.). Water Res. 7:1453-1464.

Carlton WW, Price PS, 1973. Dietary copper and the induction of neoplasms in the rat by acetylaminofluorene and dimethylnitrosamine. Fd.Cosmet.Toxicol. 11:827-840.

Cartwright GE, Wintrobe MM, 1964. Copper metabolism in normal subjects.  Am.J.Clin.Nutr. 14:224-232.

Cerda B, Olive JH, 1993. Effects of diet on seven-day Ceriodaphnia dubia toxicity tests. Ohio J. Sci. 93(3):44-47

Chakoumakos C, Russo RC, Thurston RV, 1979. Toxicity of copper to cutthroat trout (Salmo clarki) under different conditions of alkalinity, pH and hardness. Am.Chem. Soc. 13(2).

Chander K, Brookes PC, 1993. Residual effects of zinc, copper and nickel in sewage sludge on microbial biomass in a sandy loam. Soil Biol.Biochem. 25(9):1231-1239.

Chaney RL, 1983 .  Fate of toxic substances in sludge applied to cropland. In: Proc. International Symposium on Land Application of Sewage Sludge. Oct. 13-15, 1982, Tokyo, pp.259-324.

Chang CC, Tatum HJ, 1973. Absence of teratogenicity of intrauterine copper wire in rats, hamsters and rabbits. Contraception 7:413-434.

Chapman GA, 1978. Toxicities of cadmium, copper and zinc to four juvenile stages of chinook salmon and steelhead. Trans.Am.Fish.Soc. 107(6):841-847.

Chapman GA, Ota S, Recht F, 1980. Effects of water hardness on the toxicity of metals to Daphnia magna. Testing laboratory: Corvallis Environmental Research laboratory, Office of Research and Development, US Environmental Protection Agency Corvallis, Oregon 97330. Report no.: E3621. Unpublished

Chapman GA, Stevens DG, 1978. Acutely lethal levens of cadmium, copper and zinc to adult male coho salmon and steelhead. Trans.Am.Fish.Soc. 107(6):837-840.

Chen R, Wei L, Chen RL, 1995. Lung cancer mortality update and prevalence of smoking among copper miners and smelters. Scand.J.Work Environ.Health 21:513-516.

Chen RL, Wei L, Huang H, 1993. Mortality from lung cancer among copper miners. Br.J.Ind.Med. 50:505-09.

Chhibba IM et al, 1994. Upper critical-level of copper in wheat (Triticum aestivum) raised on Typic Ustipsamment soil. Ind.J.Agric.Sci. 64:285-289.

Chung MK, Baek SS, Lee SH, Kim H, Choi K, Kim JC, 2009. Combined repeated dose and reproductive/developmental toxicities of copper monochloride in rats. Environmental Toxicology 24(4):315-326.

Coates RJ, Weiss NS, Daling JR, Rettmer RL, Warnick GR, 1989. Cancer risk in relation to serum copper levels. Cancer Res. 49:4353-4356.

Codina JC, Perez-Torrente C, Perez-Garcia A, Cazorla FM, de Vicente A, 1995. Comparison of microbial tests for the detection of heavy metal genotoxicity. Arch.Environ.Contam.Toxicol. 29:260-265.

Collier JA, Wilson JC, 1984a. OECD Eye irritation test. Determination of the degree of ocular irritation caused by cuprous oxide in the rabbit. Testing laboratory: SafePharm Laboratories Limited. Report no.: 105/8404. Report date: 1984-05-18. Unpublished

Collier TA, Wilson JC, 1984b. OECD skin irritation test: Determination of the degree of primary cutaneous irritation caused by cuprous oxide in the rabbit.

Collier TA, Wilson JC, 1984c. OECD Acute oral toxicity test: Determination of the acute oral median Lethal Dose (LD50) of cuprous oxide in the rat. Testing laboratory: SafePharm Laboratories Limited. Report no.: 296/8404.Report date: 1984-05-18. Unpublished

Collvin L, 1984. The effects of copper on maximum respiration rate and growth rate of perch, Perca fluviatilis. Water Res. 18(2):139-144.

Collvin, L. (1985). The Effect of Copper on Growth, Food Consumption of Perch Perca Fluviatilis Offered Maximal Food Rations. Aquatic Toxicol. 6, 105-113.

Criel P, De Schamphelaere KAC, Janssen CR, 2005. Development of a predictive model of bioavailability and toxicity of copper in soils Invertebrate toxicity. Laboratory of Environmental Toxicology and Aquatic Ecology, Ghent University.

Cristofori P, Terron A, Marella M, Moretti U, Pasqualicchio M, Velo GP, Milanino R, 1992. Copper supplementation in the rat: preliminary observations on the clinical hematological and histopathological profile. Agents Actions, Conference issue.

Cromwell GL, Stahly TS, Monegue HJ, 1989. Effects of source and level of copper on performance and liver copper stores in weanling pigs. J.Animal Sci. 67:2996-3002.

Curtis MW, Copeland TL, Ward CH, 1979. Acute toxicity of 12 industrial chemicals to freshwater and saltwater organisms. Water Res. 13:137-174.

Curtis MW, Ward CH, 1981. Aquatic toxicity of forty industrial chemicals: testing in support of hazardous substance spill prevention regulation. J.Hydrol. 51:359-367.

Cusimano RF, Brakke DF, Chapman GA, 1986. Effects of pH on the toxicities of cadmium, copper, and zinc to steelhead trout (Salmo gairdneri). Can.J.Fish.Aq.Sci. 43:1497-1503.

Cuthbert JA, Carr SMA, 1986. Cuprous oxide: Acute oral and dermal toxicity tests in rats. Inveresk Research International. Report No. 3464. Unpublished.

De Groot AC, Peijnenburg WGJM, van den Hoop MAGT, Ritsema R, van Veen RPM, 1998. Heavy metals in Dutch field soils: an experimental and theoretical study on equilibrium partitioning. Testing laboratory: Laboratory for Ecotoxicology, National Institute of Public Health and the Environment. Report no.: 607220001.

De Haan S, Rethfeld H, van Driel W, 1985. Acceptable levels of heavy metals (Cd, Cr, Cu, Ni, Pb, Zn) in soils, depending on their clay and human content and cation-exchange capacity. Instituut Voor Bodemvruchtbaarheid Haren-Gr. Report no.: 0434-6793.

De Schamphelaere KAC et al, 2006. Cross–phylum comparison of a chronic biotic ligand model to predict chronic toxicity of copper to a freshwater rotifer, Brachionus calyciflorus. Ecotox.Environ.Safe. 63:189-195.

De Schamphelaere KAC, Janssen CR, 2002b. A biotic ligand model predicting acute copper toxicity to Daphnia magna: the effects of calcium, magnesium, sodium, potassium and pH. Environ.Sci.Technol. 36:48- 84.

De Schamphelaere KAC, Janssen CR, 2004a. Development and field validation of a biotic ligand model predicting chronic copper toxicity to Daphnia magna. Environ.Toxicol.Chem. 23(6):1365-75.

De Schamphelaere KAC, Janssen CR, 2004b. Modelling copper bioavailability and toxicity in freshwater: uncertainty reduction for risk assessment (Chronic fish-BLM). Final Report for the Voluntary risk assessment of copper, copper II sulphate, pentahydrate, copper(I) oxide, copper(II) oxide, dicopper chloride trihydroxide, Appendix U. Testing laboratory: Laboratory of Environmental Toxicology and Aquatic Ecology, Ghent University. Unpublished

De Schamphelaere KAC, Janssen CR, 2004c. Effects of chronic dietary copper exposure on growth and reproduction of Daphnia magna. Environ.Toxicol.Chem. 23(8):2038-2047.

De Schamphelaere KAC, Janssen CR, 2006. Bioavailability models for predicting copper toxicity to freshwater green microalgae as a function of water chemistry. Environ.Sci.Technol. 40:4514-4522.

De Schamphelaere KAC, Vasconcelos F, Heijerick D, Tack F, Delbeke K, Janssen C, 2003. Development and field validation of a predictive copper toxicity model for the green alga Pseudokirchneriella subcapitata. Environ.Toxicol.Chem. 22(10):2454-2465.

De Schamphelaere KAC, Heijerick DG and Janssen CR (2002a). Refinement and field validation of a biotic ligand model predicting acute copper toxicity to Daphnia magna. Comparative Biochemistry and Physiology, part C, nr 133, p 243-258. Testing laboratory: Laboratory of Environmental Toxicology and Aquatic Ecology, Ghent University, J. plateaustraat 22, 9000 Gent, Belgium.

Deaver E, Rodgers JH, 1996. Measuring bioavailable copper using anodic stripping voltammetry. Environ.Toxicol.Chem. 15(11):1925-1930.

Deenihan MJ, 1987. Copper oxychloride technical grade – acute toxicology testing. Northview PacificLaboratories, Inc. Report No. X7G039G. Unpublished.

Denizeau F, Marion M, 1989. Genotoxic effects of heavy metals in rat hepatocytes. Cell Biol. Toxicol. 5:15-25.

Di Carlo FJ, 1980. Syndromes of cardiovascular malformations induced by copper citrate in hamsters.  Teratol. 21:89-101.

Dickhaus S, 1988a. Irritant effects of Kupfer-I-Oxid on rabbit skin according to Draize.  Pharmatox. Report No. 1-3-42-88. Unpublished.

Dickhaus S, Heisler E, 1987a. Prüfung der Substanz Kupferoxychlorid auf Primäre Hautreizwirkung beim Kaninchen. Pharmatox.Report No. 1-3-34-87. Unpublished.

Dickhaus S, Heisler E, 1987b. Augenreiztest am kaninchen mit der Substanz Kupferoxychlorid nach Draize. Pharmatox. Report No. 1-3-33-87. Unpublished.

Dickhaus S, Heisler E, 1988a. Irritant effects of Kupfer-I-Oxide on rabbit skin acc. to draize (OECD-Guidance 404). Testing laboratory: Pharmatox GmbH. Report no.: E. H. /B. 1-3-42-88. Unpublished.

Dickhaus S, Heisler E, 1988b. Eye irritation test with kupfer-I-oxid according to Draize and OECD guidances no. 405. Pharmatox. Report No. 1-3-41-88. Unpublished.

Dickhaus S. 1988b. Acute Toxicological Study of Kupfer-I-Oxid After Oral Application to the Quail. Pharmatox GmbH, project no. 1-8-43-88.

Doelman P, Haanstra L, 1984. Short term and long term effects of cadmium, chromium, copper, nickel, lead and zinc on soil microbial respiration in relation to abiotic soil factors. Plant Soil 79:317-327.

Doelman P, Haanstra L, 1989. Short- and long-term effects of heavy metals on phosphatase activity: An ecological dose-response model approach. Biol. Fertil.Soils. 8:235–241.

Driscoll R, (1999d. Nordox Agro-grade: acute eye irritation test in the rabbit. Safepharm Laboratories. Report No. 148/016. Unpublished.

Driscoll R, 1999a. Nordox Agro-grade: acute oral toxicity study in the rat – acute toxic class method. Safepharm Laboratories Limited. Report No. 148/012. Unpublished.

Driscoll R, 1999c. Nordox Agro-grade: acute dermal irritation test in the rabbit. Safepharm Laboratories Limited. Report No. 148/015. Unpublished.

Driscoll R, 1999e. Nordox Agro-grade: Magnusson and Kligman maximisation study in the guinea pig. Safepharm Laboratories Limited. Report No. 148/017. Unpublished.

Drummond JG, Aranyi C, Schiff LG, Fenters JD, Graham JA, 1986. Comparative study of various methods used for determining health effects of inhaled sulfates. Environ. Res. 41, 514-528.

Eckard Helmers, 1996. Trace metals in suspended particulate matter of Atlantic Ocean surface water (40°N to 20°S). Mar.Chem. 53:51-67.

Eisler R, 1984. Trace metal changes associated with age of marine vertebrates. Biol.Trace Elem.Res. 6:165-180.

Elbaz-Poulichet F, Garnier J-M, Guan DM, Martin J-M, Thomas AJ, 1996. The conservative behaviour of trace metals (Cd, Cu, Ni and Pb) and As in the surface plume of stratified estuaries: example of the Rhöne River (France). Est.Coast.Shelf Sci. 42:289-310.

Enterline PE, Day R, Marsh GM, 1995. Cancers related to exposure to arsenic at a copper smelter. Occup.Environ.Med. 52:28-32.

Erickson RJ et al, 1996. The effects of water chemistry on the toxicity of copper to fathead minnows. Environ.Toxicol.Chem. 15:181-193.

Erickson RJ, Benoit DA, Mattson VR, 1996. A protoype toxicity factors model for site-specific copper water quality criteria. Testing laboratory: US Environmental Protection Agency, environmental research laboratory, 6201 Congdon blvd, Duluth, Minnesota 55804, USA. Owner company: US Environmental Protection Agency. Report date: 1987-08-19.

Ernst and Rittinghausen (2010). Re-examination of the sudy "A four-week inhalation toxicity study of cuprous oxide in sprague dawley rats with a time course evaluation and a 13-week recovery evaluation" by Kirkpatrick (2010). Testing laboratory: Fraunhofer Institute and Or. Böttner, Histovia. Unpublished

Fageria VD, 2001. Nutrient interactions in crop plants. J.Plant Nutr. 24(8):1269-1290.

Farag AM et al, 1998. Concentrations of metals associated with mining waste in sediments, biofilm, benthic macroinvertebrates, and fish from the Coeur d'Alene River Basin, Idaho. Arch.Environ.Contam.Toxicol. 34(2):119-127.

Ferm HF, Hanlon DP, 1974. Toxicity of copper salts in hamster embryonic development. Biol. Reprod. 11:97-101.

Fischer PWF, L’Abbé MR, Giroux A, 1990. Effects of age, smoking, drinking, exercise and estrogen use on indices of copper status in healthy adults. Nutr.Res. 10:1081-1090.

Foekema EM, Kramer KJM, Kaag NHBM, Sneekes AC, Hoornsman G, Lewis WE, van der Vlies EM, 2010. Determination of the biological effects and fate of dissolved copper in outdoor marine mesocosms, IMARES Report C105/10, IMARES, Wageningen, pp. 113  Unpublished

Fogels A, Sprague JB, 1977. Comparative short-term tolerance of zebrafish, flagfish and rainbow trout to five poisons including potential reference toxicants. Water Res. 11:811-817.

Forster R, Luperi L, 1987a. Acute oral toxicity in rats. Test substance: copper oxychloride tech. Life Science Research. Report No. 128001-ST-01487. Unpublished.

Forster, R. and Luperi, L. (1987b). Acute dermal toxicity in rabbits. Test substance: copper oxychloride tech., unpublished report. Testing laboratory: Life Science Research. Report no.: 128004-ST-01787

Forster R, Luperi L, 1987c. Primary skin irritation in rabbits. Test substance: copper oxychloride. Life Science Research. Report No. 128002-ST-01587. Unpublished.

Fort, D. J. and Stover, E. L. (1996). Effect of Low-Level Copper and Pentachlorophenol Exposure on Various Early Life Stages of Xenopus Laevis. In: Environmental Toxicology and Risk Assessment: Biomarkers and Risk Assessment (Fifth Volume), ASTM STP 1306, David A. Bengtson and Diane S. Henshel, Eds., American Society of Testing and Materials, Philadelphia, 1997.

Fort, D. J. at al. (2000). Chronic boron or copper deficiency induces limb teratogenesis in Xenopus. Biol Trace Elem Res. 2000; 77(2):173-87.

Fort, D. J. et al. (2000). Adverse developmental and reproductive effects of copper deficiency in Xenopus laevis. Biol Trace Elem Res. 2000; 77(2):159-72.

Franke J. 2005. Copper powder; Vapour pressure A.4 (OECD 104). Test at Siemens AG. Study report nr GAB 20050986.02. Unpublished

Frenz G, Teilum D, 1980. Cutaneous eruptions and intrauterine contraceptive copper device. Acta Derm-ven (Stockholm) 60:631-637

Frostegård Å et al, 1993. Phospholipid fatty-acid composition, biomass, and activity of microbial communities from 2 soil types experimentally exposed to different heavy-metals. Appl.Environ.Microbiol. 59:3605-3617.

Fuentealba I, Haywood S, Trafford J, 1989. Variations in the intralobular distribution of copper in the livers of copper-loaded rats in relation to the pathogenesis of copper storage diseases. J.Comp.Path. 100:1-11.

Fuentealba IC, Mullins JE, Aburto EM, Lau JC, Cherian GM, 2000. Effect of age and sex on liver damage due to excess dietary copper in Fischer 344 rats. Clin.Toxicol. 38:709-717.

Fulfs JC, 1990. Acute inhalation toxicity in the rat.  Single level – limit test. Inhausen Research Institute Inc. Report No. 131.003. Unpublished.

Gaul LE (1958). Dermatitis from metal spectacles. Arch Dermatol 78:475-478.

Gazzotti, L.2006. Water solubility of copper powder, Final report. Test Lab GAB Analisi S.r.l; study nr GAB 20051384/01-PCSB. Unpublished

Geckler JR, Honring WB, Neiheisel TM, Pickering QH, Robinson EL, 1976. Validity of laboratory tests for predicting copper toxicity in streams. Testing laboratory: Newtown Fish Toxicology Station, Environmental Research Laboratory, Duluth, Cincinnati, Ohio 45244. Report no.: EPA-600/3-76-116. Owner company: US Environmental Protection Agency, Duluth, Minnesota 55804.

Ginocchio R et al, 2002. Effect of soil Cu content and pH on copper uptake of selected vegetables grown under controlled conditions. Environ.Toxicol.Chem. 21(8):1736–1744.

George, S. G., Pirie, B. J. S., Cheyne, A. R., Coombs, T. L. & Grant, P. T. (1978). Detoxication of Metals by Marine Bivalves. An Ultrastrutural Study of the Compartmentation of Copper and Zinc in the Oyster Ostrea edulis. Marine Biology, 45; 147-156.

Ginocchio R, Sanchez P, de la Fuente LM, Camus I, Bustamante E, Silva Y, Urrestarazu P, Torres JC, Rodriguez PH, 2006. Agricultural soils spiked with copper mine wastes and copper concentrate: implications for copper bioavailability and bioaccumulation. Environ.Toxicol.Chem. 25(3):712- 718

Ginocchio, R. et al. (2002). Effect of soil Cu content and pH on copper uptake of selected vegetables grown under controlled conditions. Environ Toxicol Chem., 21(8), 1736–1744.

Goldstein IM, Kaplan HB, Edelson HS, Weissmann G, 1979. Ceruloplasmin a scavenger of superoxide anion radicals. J.Biol.Chem. 254:4040-4045.

Golimowski J, Merks AGA, Valenta P, 1990. Trends in heavy metal levels in the dissolved and particulate phase in the Dutch Rhine-Meuse (Maas) delta. Sci.Total Environ. 92:113-127.

Gonzales SP, 1991. Upper critical levels of copper to alfalfa in ten Chilean soils. Water Air Soil Pollut. 57-58:201-208.

Gould E et al, 1988. Uptake and effects of copper and cadmium in the gonad of the scallop Placopecten magellanicus: concurrent metal exposure. Mar.Biol. 97:212-233.

Graney, R. L., Cherry, D. S. & Cairns, J. (1983). Heavy Metal Indicator Potential of the Asiatic Clam (Corbicula fluminea) in Artificial Stream Systems. Hydrobiologia. 102: 81-88.

Greene FL, Lamb LS, Barwick M, Pappas NJ, 1987. Effect of dietary copper on colonic tumour and aortic integrity in the rat. J.Surg.Res. 42:503-512.

Greenough RJ, McDonald P, 1985a. Cuprous oxide: Acute inhalation toxicity study in rats. Testing laboratory: Inveresk Research International. Report no.: Report No.: 3401. Unpublished

Greenough RJ, McDonald P, 1985b. Cuprous oxide: Acute inhalation toxicity study in rats. Testing laboratory: Inveresk Research International. Report no.: Report No.: 3398. Unpublished

Grosell M., C. Nielsen, A. Bianchini (2002). Sodium turnover rate determines sensitivity to acute copper and silver exposure in freshwater animals. Comparative Biochemistry and Physiology Part C 133 (2002) 287-303.

Grosell M et al, 2007. Physiology is pivotal for interactions between salinity and acute copper toxicity to fish and invertebrates. Aquat.Toxicol. 84(2):162-72.

Haanstra L, Doelman P, 1984. Glutamic acid decomposition as a sensitive measure of heavy metal pollution in soil method: Soil Biol.Biochem. 16:595-600.

Haanstra L, Doelman P, 1991. An ecological dose-response model approach to short- and long-term effects of heavy metals on arylsulfatase activity. Biol.Fertil.Soils 11:18-23.

Hackel H, Miller K, Elsner P, Burg G, 1991. Unusual combined sensitization to palladium and other metals. Contact Dermatitis 24:131-132.

Haddad DS, Al-Alousin LA, Kantarjian AH, 1991. The effects of copper loading on pregnant rats and their offspring. Funct. Dev. Morphol. 1:17-22.

Hale JG (1977). Toxicity of metal mining wastes. Buletin of Environmental contamination & toxicology, vol 17, nr 1, p66-73. Testing laboratory: EPA National Enforcement Investigations Center, Denver, Colorado.

Hall LW Jr, Anderson RD, Kilian JV, 1997. Acute and chronic toxicity of copper to the estuarine copepod Eurytemora affinis: influence of organic complexation and speciation. Chemosphere 35(7):1567-1597.

Hamilton EI, Minski MJ, Cleary JJ, 1972. The concentration and distribution of some stable elements in healthy human tissue from the United Kingdom. Sci.Total Environ. 1:341-374.

Handy, R. D. (2003). Chronic effects of copper exposure versus endocrine toxicity: two sides of the same toxicological process?. Comp Biochem Physiol A Mol Integr Physiol. 2003; 135(1):25-38.

Hansen JA, Lipton J, Welsh PG, Cacela D, MacConnell B, 2004. Reduced growth of rainbow trout (Oncorhynchus mykiss) fed a live invertebrate diet pre-exposed to metal contaminated sediments. Environ.Toxicol.Chem. 23(8):1902-1911.

Harris ED, 2000. Cellular copper transport and metabolism. Ann.Rev.Nutr. 20:291-310.

Harrison, JWE, Levin SE, Trabin B, 1954. The safety and fate of potassium sodium copper chlorophyllin and other copper compounds. J.Am.Pharmac.Assoc. 43(12):722-737.

Harvey L, Dainty JR, Hollands WJ, Bull VJ, Beattie JH, Venelinov TI; Hoogewerff JA, Davies IM, Fairweather-Tait SJ, 2005. Use of mathematical modeling to study copper metabolism in humans. Am.J.Clin.Nutr. 81: 807-813.

Harvey LJ, Majsak-Newman G, 2003. Adaptive responses in men fed low-and high-copper diets. Brit.J.Nutrit. 90:161-168.

Hatakeyama S, Yasuno M, 1981. A method for assessing chronic effects of toxic substance on the midge, Paratanytarsus parthenogeneticus - effects of copper. Arch.Environ.Contam.Toxicol. 10:705-713.

Haynes G, 1998. Copper oxychloride acute oral toxicity study in the mouse. Research Toxicology Centre. Report No. 6166/T/248/97. Unpublished.

Haywood S, 1980. The effect of excess dietary copper on the liver and kidney of the male rat. J.Comp.Pathol. 90:217-232.

Haywood S, Comerford B, 1980. The effect of excess dietary copper on plasma enzyme activity and on the copper content of the blood of the male rat. J.Comp.Pathol 90:233-238.

Haywood S, Loughran M, 1985. Copper toxicosis and tolerance in the rat. II. Tolerance – a liver protective adaptation. Liver 5:267-275.

Hedtke SF, 1984. Structure and function of copper-stressed aquatic microcosms. Aquat.Toxicol. 5:227-44.

Hébert, C. D. (1993). NTP Technical Report on toxicity studies of cupric sulphate (CAS No. 7758-99-8) administered in drinking water and feed to F344/N rats and B6C3F1 mice. National Toxicology Program, Toxicity Report Series No. 29, United States Department of Health and Human Services

Hébert C.D., Elwell M.R., Travlos G.S., Fitz C.J. and Bucher J.R. (1993). Subchronic toxicity of cupric sulfate administered in drinking water and feed to rats and mice. Fund. Appl. Toxicol. 21, 461-475

Heijerick D, Bossuyt B, De Schamphelaere K, Indeherberg M, Mingazzini M, Janssen C, 2005. Effect of varying physicochemistry of European surface waters on the copper toxicity to the green alga Pseudokirchneriella subcapitata. Ecotoxicology 14:661-670.

Heijerick D, Bossuyt B, Janssen C, 2001. EURO-ECOLE Assessment of the bioavailability and potential ecological effects of copper in european surface waters – Subproject 4: Evaluation and improvement of the ecological relevance of laboratory generated toxicity data. Report no.: no report number.

Heijerick D, Van Sprang P, 2005. Review of copper partitioning coefficients in the aquatic environment and processes causing the observed variation. EU risk assessment of copper, copper II sulphate pentahydrate, copper(I) oxide, copper(II) oxide, dicopper chloride trihydroxide. Environment-exposure - Voluntary risk assessment, Appendix F.

Heijerick D, Van Sprang P, 2008. Evaluation of partition coefficients for copper in the marine environment (estuaries and marine conditions). EU risk assessment of copper, copper II sulphate pentahydrate, copper(I) oxide, copper(II) oxide, dicopper chloride trihydroxide. Voluntary risk assessment, Appendix M.

Heikens A et al, 2001. Bioaccumulation of heavy metals in terrestrial invertebrates. Environ.Pollut. 113:385-93.

Hemida SK, Omar SA, Abdel-Mallek AY, 1997. Microbial populations and enzyme activity in soil treated with heavy metals. Water Air Soil Pollut. 95:13-22.

Herbert IN, Svendsen C, Hankard PK, Spurgeon DJ, 2004. Comparison of instantaneous rate of population increase and critical-effect estimates in Folsomia candida exposed to four toxicants. Ecotox.Environ.Safe. 57:175-183.

Hernández, L. M. et al. (1999). Accumulation of heavy metals and As in wetland birds in the area around Doñana National Park affected by the Aznalcollar toxic spill. Sci. Total Environ. 1999; 242(1-3):293-308.

Himmelstein MW, 2003. Five copper substances- absorption, distribution and excretion in male rats. Testing laboratory: Haskell Laboratory for Health and Environmental Sciences. Report no.: 11784. Report date: 2003-11-18. Unpublished

Hixson, O.F., 1973. Purple Copp 97. To determine the oral toxicity and skin irritation potential in accordance with Federal Hazardous Substances Labeling Act Regulations.  Rosner-Hixson Laboratories.  Laboratory No. PT73-40.  Unpublished.

Holbert MS, 1993. Copper oxychloride MUP. Acute inhalation toxicity study in rats. Stillmeadow, Inc. Report No. 0567-93. Unpublished.

Holbert MS, 1994. Copper sulphate instant powder, acute inhalation toxicity study in rats. Stillmeadow, Inc. Report  No. 0693-93. Unpublished.

Horning WB, Neiheisel TW, 1979. Chronic effect of copper on the bluntnose minnow, Pimephales notatus. Arch.Environ.Contam.Toxicol. 8:545-552.

Hostynek JJ, Maibach HI, 2003. Copper hypersensitivity: Dermatologic aspects - an overview. Rev.Environ.Health  18(3):153-188.

Hunter BA, Johnson MS, 1982. Food chain relationships of copper and cadmium in contaminated grassland ecosystems. Oikos 38:108-117.

Hurd KS, 2006a. Copper determination of the toxicity to the larvae of the sea urchin (Paracentrotus lividus). Brixham Environmental Lab study number 05-0033/N. Testing laboratory: Brixham Environmental Laboratory, AstraZeneca UK Limited, Brixham, UK. Report no.: BEL report N° BL8354/B.

Hurd, KS, 2006b. Copper: Determination of the effects on the embryo larval development of the sheepshead minnow (Cyprinodon variegatus). Testing laboratory: Brixham Environmental Laboratory, AstraZeneca UK Limited, Brixham, UK. Report no.: report N° BL8353/B. Unpublished

IPCS, 1998. Copper: Environmental Health Criteria 200. Geneva: WHO Publication.

Jackson D, 1994a. Technical copper oxychloride. Acute oral toxicity (LD50) test in rats. Inveresk Research International. Report No. 9911. Unpublished.

Jackson D, 1994c. Technical copper oxychloride. Acute dermal irritation test in rabbits. Inveresk Research International. Report No. 9913. Unpublished.

Jackson D, 1994d. Technical copper oxychloride. Acute eye irritation test in rabbits. Inveresk Research International. Report No. 9914. Unpublished.

Jacobson PJ, Neves RJ, Cherry DS Farris JL, 1997. Sensitivity of glochidial stages of freshwater mussels (Bivalvia: Unionidae) to copper. Environ.Toxicol.Chem. 16(11):2384-2392.

Jäggy A, Streit B, 1982. Toxic effects of soluble copper on Octolasium cyaneum sav. (Lumbricidae). Rev.Suisse De Zool. 89:881-889.

Janssen RPT et al, 1997a. Equilibrium partitioning of heavy metals in Dutch field soils. I. relationship between metal partition coefficients and soil characteristics. Environ.Toxicol.Chem. 16(12):2470-2478.

Janssen RPT et al, 1997b. Equilibrium partitioning of heavy metals in Dutch field soils. II. Prediction of metal accumulation in earthworms. Environ.Toxicol.Chem. 16(12):2479-2488.

Jarvis SC, 1978. Copper uptake and accumulation by perennial ryegrass grown in soil and solution culture. J.Sci.Fd.Agric. 29:12-18.

Johansson A, Curstedt T, Robertson B, Camner P, 1984. Lung morphology and phospholipids after experimental inhalation of soluble cadmium, copper and cobalt. Environ.Res. 34:295-309.

Johnson MA, Flagg EW, 1986. Effects of sucrose and cornstarch on the development of copper deficiency in rats fed high levels of zinc. Nutr.Res. 6:1307-1319.

Johnson MA, Gratzek JM, 1986. Influence of sucrose and starch on the development of anemia in copper- and iron-deficient rats. J.Nutr. 116:2443-2452.

Johnson MA, Hove SS, 1986. Development of anemia in copper-deficient rats fed high levels of dietary iron and sucrose. In: Copper Deficiency, Iron and Sucrose, pp 1225 – 1238. American Institute of Nutrition.

Johnson MA, Murphy CL, 1988. Adverse effects of high dietary iron and ascorbic acid on copper status in copper-deficient and copper-adequate rats. Am.J.Clin.Nutr. 47:96-101.

Johnson PE, 1988. Effect of various dietary carbohydrates on absorption and excretion of copper in the rat as measured by isotope dilution. J.Trace Elem.Exp.Med. 1:143-55.

Johnson, P. E. and Lee, D. -Y. (1988). Copper Absorption and Excretion Measured by Two Methods in Rats Fed Varying Concentrations of Dietary Copper. J. Trace Elem. Exp. Med. 1: 129-141.

Johnson HL, Stauber JL, Adams MS, Jolley DF, 2007. Copper and zinc tolerance of two tropical microalgae after copper acclimation. Environmental Toxicology 22(3):234-244.

Jop KM et al, 1995. Development of a water-effect ratio for copper, cadmium and lead for the Great Works River in Maine using Ceriodaphnia dubia and Salvelinus fontinalis. Bull.Environ.Contam.Toxicol. 54:29-35.

Kadiska MB, Hanna PM, Hernandez L, Mason RP, 1992. In vivo evidence of hydroxyl radical formation after acute copper and ascorbic acid intake: electron spin resonance spin-trapping investigation. Mol.Pharmacol. 42:723-729.

Kaland T,  Andersen T, Hylland K, 1993. Accumulation and subcellular distribution of metals in the marine gastropod Nassarius reticulatus. p37-53. In: Dallinger R, Rainbow PS (Eds). Ecotoxicology of Metals in Invertebrates Proceedings of the 1st SETAC European Conference. Lewis Publications; Boca Raton, Fl. USA 461.

Kalyanaraman SB, Sivagurunathan P, 1993. Effect of cadmium, copper, and zinc on the growth of blackgram. J.Plant Nutrit. 16:2029-2042.

Kammenga JE, Van Koert PHG, Riksen JAG, Korthals GW, Bakker J, 1996. A toxicity test in artificial soil based on the life-history strategy of the nematode Plectus acuminatus. Environ.Toxicol.Chem. 15(5):722-727.

Kamunde CN et al, 2001. Copper metabolism and gut morphology in rainbow trout (Oncorhynchus mykiss) during chronic sublethal dietary copper exposure. Can.J.Fish.Aquat.Sci. 58:293–305.

Kamunde CN et al, 2005. Interaction of dietary sodium chloride and waterborne copper in rainbow trout (Oncorhynchus mykiss): copper toxicity and sodium and chloride homeostasis. Can.J.Fish.Aquat.Sci. 62:390–399.

Kanematsu N, Hara M, Kada T, 1980. REC assay and mutagenicity studies on metal compounds. Mutation Res. 77:109-116.

Karjalainen S, Raimo K, Pukkala E, 1992. Cancer risk among workers at a copper/nickel smelter and nickel refinery in Finland. Int.Arch.Occup.Environ.Health 63:547-551.

Karlberg A-T, Boman A, Wahlberg JE, 1983. Copper - a rare sensitizer. Contact Dermatitis 9; 134-139.

Keen CL, Lönnerdal B, Hurley LS, 1982. Teratogenic effects of copper deficiency and excess. Dis. Copper (Proc Symp) 1981:109-121.

Kegley EB, Spears JW, 1994. Bioavailability of feed-grade copper sources (oxide, sulfate, or lysine) in growing cattle. J. Animal Sci. 72: 2728-2734.

Khalil MA et al, 1996a. Analysis of separate and combined effects of heavy metals on the growth of Aporrectodea caliginosa (Oligochaeta; Annelida), using the toxic unit approach. Appl.Soil Ecol. 4:213- 219.

Khalil MA et al, 1996b. Effects of metals and metal mixtures on survival and cocoon production of the earthworm Aporrectodea caliginosa. Pedobiologia 40:548-556.

Khan M, Scullion J, 2002. Effects of metal (Cd, Cu, Ni, Pb or Zn) enrichment of sewage-sludge on soil micro-organisms and their activities. Appl.Soil Ecol. 20:145-155.

Kirkpatrick D, 2010. A four-week inhalation toxicity study of cuprous oxide in Sprague Dawley rats with a time course evaluation and a 13-week recovery evaluation. Testing laboratory: WIL Research Laboratories, LLC. Report no.: WIL-708003. Report date: 2010-08-19. Unpublished

Khangarot, B. S. and Bhakin, M. K. (1981). “Man and Biosphere” – Studies in the Sikkim Himalayas. Part 4: Effects of Chelating Agent EDTA on the Acute Toxicity of Copper and Zinc on Tadpoles of the Frog Rana hexadactyla. Acta hydrochim. hydrobiol. 1985; 13(1):121-125.

Khangarot, B. S. et al. (1981). Studies on the acute toxicity of copper on selected freshwater organisms. Sci. Cult. 1981; 47:429-431

Kjær C, Elmegaard N, 1996. Effects of copper sulphate on black bindweed (Polygonum convolvulus L.). Ecotox.Environ.Safe. 33:110-117.

Knobeloch L, Ziarnik M, Howard J, Theis B, Farmer D, Anderson H, Proctor M, 1994. Gastrointestinal upsets associated with ingestion of copper-contaminated water. Environ.Health Perspect. 102:958-961.

Knobeloch L, Ziarnik M, Howard J, Theis B, Farmer D, Anderson H, Proctor M, (1994). Gastrointestinal upsets associated with ingestion of copper-contaminated water. Environ. Health Perspect. 102:958-961.

Koelmans AA, Radovanovic H, 1998. Prediction of trace metal distribution coefficients (Kd) for aerobic sediments. Wat.Sci.Tech. 37(6-7):71-78.

Korthals, G. W., Alexiev, A. D., Lexmond, T. M., Kammenga, J. E. & Bongers, T. (1996). Long Term Effects of Copper and pH on the Nematode Community in an Agrosystem. Environ. Toxicol. Chem. 15 (6): 979-985.

Kraak MHS, Schoon H, Peeters WHM, Van Straalen NM, 1994. Chronic ecotoxicity of mixtures of Cu, Zn and Cd to the zebra mussel Dreissena polymorpha. Ecotox.Environ.Safe. 25:315-327.

Kraak, M. H. S., Lavy, D, Peeters, W. J. M & Davids, C. (1992). Chronic Ecotoxicity of Copper and Cadmium to the Zebra Mussel Dreissena polymorpha. Arch. Environ. Contam. Toxicol. 23; 363-369.

Krogh PH, Axelsen JA, 1998. Test on the predatory mite Hypoaspis aculeifer preying on the collembolan Folsomia fimetaria. In: Handbook of Soil Invertebrate Toxicity Tests. pp 239-251 Ed. Lokke H, Van Gestel CAM. J. Wiley and Sons Ltd, Chichester, UK.

Kuhn JO, 1994. Purple Copp 97N cuprous oxide - Primary eye irritation study in the rabbit. Testing laboratory: Stillmeadow Inc. Report no.: 0634-93. Report date: 1994-02-07. Unpublished

Kukulinski M, 1976. Acute dermal toxicity and eye irritation tests. Rosner-Hixson Laboratories. Report no. PT76-298. Unpublished.

Kula H, Larink O, 1997. Development and standardisation of test methods for the prediction of sublethal effects of chemicals on earthworms. Soil Biol.Biochem. 29(3/4):635-639.

Kula H, Larink O, 1998. Tests on the earthworms Eisenia fetida and Aporrectodea caliginosa. In: Lokke H, Van Gestel CaM (Eds.), Handbook of Soil invertebrate toxicity tests. Chapter 7, Wiley, New York , p.95-112.

LaBreche TMC et al, 2002. Copper toxicity to larval Mercenaria mercenaria (hard clam). Environ.Toxicol.Chem. 21(4):760-766.

Laskowski R, 1991. Are the top carnivores endangered by heavy metal biomagnification? Oikos, 60(3):387-390.

Law RJ et al, 1992. Trace metals in the livers of marine mammals from the Welsh coast and the Irish Sea. Mar.Pollut.Bull. 24:296-304.

Lazorchak JM, 1987. The significance of weight loss of Daphnia magna straus during acute toxicity tests with copper. Testing laboratory: University of Texas, Dallas. Report no.: 8719909. Owner company: UMI 300N, Zzeeb road, Ann arbor, MI 48106. Unpublished

Lazorchak JM, Waller WT, 1993. The relationship of total copper 48H LC50s to Daphnia magna dry weight. Environ.Toxicol.Chem. 12:903-911.

Lecyk M, 1980. Toxicity of CuSO4 in mice embryonic development. Zoologica Poloniae 28:101-105.

Lehritier M, 1993. Test to evaluate the acute toxicity following a single cutaneous administration (limit test) in the rat. Testing laboratory: Pharmakon Europe. Report no.: 44293. Unpublished

Lheritier M (1994). Test to evaluate the acute toxicity following a single oral administration(LD50) in the rat. Testing laboratory: Pharmakon Europe. Report no.: 44193. Unpublished

Leuschner J, 2005. Examination of copper hydroxide in a skin sensitisation test in guinea pigs according to Magnusson and Kligman (maximisation test). Report No. 19184/05. LPT Laboratory of Pharmacology and Toxicology KG, Germany. Unpublished

Leuschner J, 2011. Acute Inhalation Toxicity Study of Copper Powder KU 7600 Standard Material in Rats. Testing laboratory: LPT, Laboratory of Pharmacology and Toxicity GmbH & Co. KG, Redderweg 8, 21147 Hamburg, Germany. Report no.: 27371. Guntersthal 4, 91235 Hartenstein, Germany. Report date: 2011-12-21. Unpublished

Lewis MA, 1983. Effect of loading density on the acute toxicities of surfactants, copper and phenol to Daphnia magna Straus. Arch.Environ.Contam.Toxicol. 12:51-55.

Lexmond, T. M. (1980). The Effect of Soil pH on Copper Toxicity to Forage Maize Grown Under Field Conditions. Neth. J. Agric. Sci. 28: 164-183.

Liang J, Zhou Q, 2003. Single and binary-combined toxicity of methamidophos, acetochlor and copper on earthworm Eisenia foetida. Chin. J. Appl.Ecol. 14(4):593-596.

Liipo J. , Maija-Leena Metsärinta, Päivi Kinnunen, Kirsi Virta,  Elina Wiik, Matti Santala, and Säde Harle (2010a). Characterization of copper powder. Testing laboratory: Outotec. Report no.: 10113 ORC-T. Unpublished

Liipo J. , Maija-Leena Metsärinta, Päivi Kinnunen, Kirsi Virta,  Elina Wiik, Matti Santala, and Säde Harle (2010b). Characterization of 2 coated copper flakes. Testing laboratory: Outotec. Report no.: 10111 ORC-T. Unpublished

Lide 2009 CRC Handbook of Chemistry and Physics, 90th Edition

Linder MC, Hazegh-Azam M, 1996. Copper biochemistry and molecular biology. Am.J.Clin.Nutr. 63:797S-811S.

Lindley PF, 2000. Ceruloplasmin. In: Handbook of Metalloproteins. Vol. 2. Messerschmidt A et al. (eds), Chichester, John Wiley and Sons.

Lisi P, Caraffini S, Assalve D, 1987. Irritation and sensitisation potential of pesticides. Contact Dermatitis 17:212-218.

Liu C.-CF, Medeiros DM, 1986. Excess diet copper increases systolic blood pressure in rats. Biol.Trace Elem.Res. 9:15-24.

Llewellyn GC, Floyd EA, Hoke GD, Weekley LB, Kimbrough TD, 1985. Influence of aflatoxin, zinc and copper on bone size, organ weight and body weight in hamsters and rats. Bull.Environ.Contam.Toxicol. 35:149-156.

Lock JW et al, 1992. Metal concentrations in seabirds of the New Zealand region. Environ.Pollut. 75:289-300.

Lock K, Janssen C, 2002. Mixture toxicity of zinc, cadmium, copper, and lead to the potworm Enchytraeus albidus. Ecotox.Environ.Safe. 52(1):1-7.

Lofts S, Tipping E, 2000. Solid-solution metal partitioning in the Humber rivers: application of WHAM and SCAMP. Sci.Total Environ. 251/252:381-399.

Logue JN, Koontz MD, Hattwick MAW, 1982. A historical prospective mortality study of workers in copper and zinc refineries. J.Occup.Med. 5:398-408.

Lorenzo JI, Nieto O, Beiras R, 2006. Anodic stripping voltammetry measures copper bioavailability for sea urchin larvae in the presence of fulvic acids. Environ.Toxicol.Chem. 25(1).

Lubin JH, Pottern LM, Stone BJ, Fraumenti JF, 2000. Respiratory cancer in a cohort of copper smelter workers: results from more than 50 years of follow-up. Am.J.Epi. 6:554-65.

Luperi L, Forster R, 1987. Acute eye irritation in rabbits. Life Science Research. Report No. 128003-ST-01687. Unpublished.

Ma W-C, 1984. Sublethal toxic effects of copper on growth, reproduction and litter breakdown activity in the earthworm Lumbricus rubellus, with observations on the influence of temperature and soil pH. Environ.Pollut.(Series A) 33:207-219.

Ma W, 1988. Toxicity of copper to lumbricid earthworms in sandy agricultural soils amended with Cu-enriched organic waste materials. Ecol.Bull. 39:53-56.

Ma YB et al, 2006a. Determination of labile copper in soils and isotopic exchangeability of colloid copper complexes. Europ.J.Soil Sci. 57(2):147-153.

Ma YB et al, 2006b. Short-term natural attenuation of copper in soils: effects of time, temperature, and soil characteristics. Environ.Toxicol.Chem. 25(3):652-658.

Maliszewska W, Stanislawa D, Wierzbicka H, Wozniakowska A, 1985. The influence of various heavy metals compounds on the development and activity of soil micro-organisms. Environ.Pollut. 37:195-215.

Manciaux X, 1998. Acute oral toxicity in rats. Centre International de Toxicologie. Report No. 16141 TAR. Unpublished.

Marceau N, Aspin N, Sass-Kortsak A, 1970. Absorption of copper 64 from gastrointestinal tract of the rat. Am.J.Physiol. 218:377-383.

Markich, S. J. et al. (2003). The Effects of pH and Dissolved Organic Carbon on the Toxicity of Cadmium and Copper to a Freshwater Bivalve: Further Support for the Extended Free Ion Activity Model. Arch. Environ. Contam. Toxicol., 2003; 45:479–491.

Marr JCA et al, 1996. Relationship between copper exposure duration, tissue copper concentration and rainbow trout growth. Aq.Toxicol. 36:17-30.

Marr JCA, Hansen JA, Meyer JS, Cacela D, Podrabsky T, Lipton J, Bergman HL, 1998. Toxicity of cobalt and copper to rainbow trout: application of a mechanistic model for predicting survival. Aq.Toxicol. 43:225-238.

Marr JCA, Lipton J, Cacela D, Hansen JA, Meyer JS, Bergman HL, 1999. Bioavailability and acute toxicity of copper to rainbow trout (Oncorhynchus mykiss) in the presence of organic acids simulating natural dissolved organic carbon. Can.J.Fish.Aq.Sci. 56:1471-1483.

Marsh GM, Stone RA, Esmen NA, Gula MJ, Gause CK, Petersen NJ, Meaney F, Prybylski D (1998). A case-control study of lung cancer mortality in four rural Arizona smelter towns. Arch. Environ. Health 53(1):15-28.

Marsh GM, Stone RA, Esmen NA, Gula MJ, Gause CK, Petersen NJ, Meaney FJ, Rodney S, Prybylski D (1997). A Case-Control Study of Lung Cancer Mortality in Six Gila Basin, Arizona Smelter Towns. Environ. Res. 75(1):56-72. Report date: 1996-12-22.

Martin JM, Guan DM, Elbaz-Poulichet F, Thomas AJ, Gordeev VV, 1993. Preliminary assessment of the distributions of some trace elements (As, Cd, Cu, Fe, Ni, Pb and Zn) in a pristine aquatic environment: the Lena River estuary (Russia). Mar.Chem. 43:185-199.

Marzin DR, Phi HV, 1985. Study of the mutagenicity of metal derivatives with Salmonella typhimurium. Mutation Res. 155:49-51.

Maund SJ et al, 1992. Population responses of the freshwater amphipod crustacean Gammarus pulex to copper. Freshwater Biol. 28:29-36.

McBride MB,  2001. Cupric ion activity in peat soil as a toxicity indicator for Maize. J.Environ.Qual. 30(1):78-84.

McGeer JC et al, 2002. The role of dissolved organic carbon in moderating the bioavailability and toxicity of Cu to rainbow trout during chronic waterborne exposure. Comp.Biochem.Physiol.C Toxicol.Pharmacol. 133(1-2):147-60.

McGeer JC et al, 2003. Inverse relationship between bioconcentration factor and exposure concentration for metals: implications for hazard assessment of metals in the aquatic environment. Environ.Toxicol.Chem. 22(5):1017-37.

McKim JM, Benoit DA, 1971. Effects of long-term exposures to copper on survival, growth and reproduction of brook Trout (Salvelinus fontalis). J.Fish Res.Bd.Can. 28:655-662.

McKim JM, Eaton JG, Holcombe GW, 1978. Metal toxicity to embryos and larvae of eight species of freshwater fish - II: copper. 0007-4861/78/0019-0608 Springer-Verlag New York Inc.

McLusky, D. S. & Phillips, C. N. K. (1975). Some Effects of Copper on the Polychaete Phyllodoce maculata. Estuarine and Coastal Marine Science. 3: 103-108.

McManus JP, Prandle D, 1996. Determination of source concentrations of dissolved and particulate trace metals in the Southern North Sea. Mar.Pollut.Bull. 32(6):504-512.

McMaster D, McCrum E, Patterson CC, Kerr MM, O’Reilly D, Evans AE, Love AHG, 1992. Serum copper and zinc in random samples of the population of Northern Ireland. Am.J.Clin.Nutr 56:440-446.

Mercer JF, Livingston J, Hall B, et al., 1993. Isolation of a partial candidate gene for Menkes disease by positional cloning. Nat.Genet. 3:20-25.

Mercier O, 1994a. Test to evaluate the sensitising potential in the guinea-pig maximisation test. Pharmakon Europe. Report No. 44093. Unpublished.

Mercier O, 1994b. Test to evaluate the acute ocular irritation and reversibility in the rabbit. Pharmakon Europe. Report No. 44393. Unpublished.

Mercier O, 1994c. Test to evaluate the acute primary cutaneous irritation and corrosivity in the rabbit. Pharmakon Europe. Report No. 44493. Unpublished.

Mersch J, Morhain E, Mouvet C, 1993. Laboratory accumulation and depuration of copper and cadmium in the freshwater mussel Dreissena polymorpha and the aquatic moss Rhynchostegium riparioides. Chemosphere 27(8):1475-1485.

Messerschmidt, S. 2006. Coated copper flakes. Tap density of copper powder. Tested by Eurofins. Report nr  GAB 20051384/01-PCTD. Unpublished

Milne DB, Johnson PE, 1993. Assessment of copper status: effect of age and gender on reference ranges in healthy adults. Clin.Chem. 39:883-887.

Monteny F, Elskens M, Baeyens W, 1993. The behaviour of copper and zinc in the Scheldt estuary. Neth.J.Aq.Ecol. 27(2-4):279-286.

Moriya M, Ohta T, Watanabe K, Miyazawa T, Kato K, Shirasu Y, 1983. Further mutagenicity studies on pesticides in bacterial reversion assay systems. Mutation Res. 116:185-216.

Morton MS, Elwood PC, Abernethy M, 1976. Trace elements in water and congenital malformations of the central nervous system in South Wales. Brit.J.Prev.Soc.Med. 30:36-39.

Motolese A, Truzzi M, Giannini A, Seidenari S, 1993. Contact dermatitis and contact sensitization among enamellers and decorators in the ceramic industry. Contact Dermatitis 28:59-62.

Mount DI, 1968. Chronic toxicity of copper to fathead minnows (Pimephales promelas rafinesque). Water Res. 2:215-223.

Mount DI, Stephan CE, 1969. Chronic toxicity of copper to the fathead minnow (Pimephales promelas) in soft water. J.Fish.Res.Bd.Can. 26(9).

Mount DI, Stephan CE, 1969. Chronic toxicity of copper to the fathead minnow. J.Fish.Res.Bd.Can. 26:2449-2457.

Mozaffari M et al, 1996. Relation of copper extractable from soil and pH to copper content and growth of two citrus rootstocks. Soil Sci. 161(11):786-792.

Mudge JE, Northstrom TE, Jeane GS, Davis W, Hickam JL, 1993. Effect of varying environmental conditions on the toxicity of copper to salmon. Environ.Toxicol.Risk Assess.: 2nd edition STP 1216. Testing laboratory: SE Technologies inc, 1370 Washington Pike, Bridgeville, PA15017.

Müller-Höcker J, Meyer U, Wiebecke B, Hübner G, Eife R, Kellner M, Schramel P, 1988. Copper storage disease of the liver and chronic dietary copper intoxication in two further German infants mimicking Indian Childhood Cirrhosis. Path.Res.Pract. 183:39-45.

Munksgaard NC, Parry DL, 2001. Trace metals, arsenic and lead isotopes in dissolved and particulate phases of North Australian coastal and estuarine seawater. Mar.Chem. 75:165-184.

Munley SM, 2003a. Copper hydroxide –developmental toxicity study in rabbits. Testing laboratory: E. I. du Pont de Nemours and Company, Haskell Laboratory for Health and Environmental Sciences, Newark, Delaware. Report no.: DuPont-11862. Report date: 2003-11-17. Unpublished

Munley, S. M. (2003b). Copper hydroxide – pilot developmental toxicity study in rabbits. Testing laboratory: E. I. du Pont de Nemours and Company, Haskell Laboratory for Health and Environmental Sciences, Newark, Delaware. Report no.: DuPont-11861. Unpublished

Munley, S. M. (2003c). Copper – A 23-day tolerability study in non-pregnant rabbits. Testing laboratory: E. I. du Pont de Nemours and Company, Haskell Laboratory for Health and Environmental Sciences, Newark, Delaware. Report no.: DuPont-11762. Report date: 2003-11-16. Unpublished

Munley, S. M. (2003d). Five copper substances – repeated dose toxicity and tolerability study in non-pregnant rabbits. Testing laboratory: E. I. du Pont de Nemours and Company, Haskell Laboratory for Health and Environmental Sciences, Newark, Delaware. Report no.: DuPont-11638. Report date: 2003-11-16. Unpublished

Murata K, Araki S, 1991. Autonomic nervous system dysfunction in workers exposed to lead, zinc and copper in relation to peripheral nerve conduction: a study of R-R interval variability. Am.J.Ind.Med. 20:663-671.

Murata K, Araki S, Aono H, 1987. Effects of lead, zinc and copper absorption on peripheral nerve conduction in metal workers. Int.Arch.Occup.Environ.Health 59:11-20.

Mylchreest E, 2005. Copper sulfate pentahydrate: multigeneration reproduction study in rats. Testing laboratory: DuPont Haskell Laboratory for Health and Environmental Sciences. Report no.: Laboratory Project ID: DuPont-14226. Unpublished

Nebeker A et al, 1984. Effects of copper, nickel and zinc on the life cycle of the caddisfly Clistoronia magnifica. Environ.Toxicol.Chem. 3:645-649.

Nebeker AV, Stinchfield A, Savonen C, Chapman GA, 1986. Effects of copper, nickel and zinc on three species of Oregon freshwater snails. Environ.Toxicol.Chem. 5(9):807-811.

Nelson H, Benoit D, Erickson R, Mattson V, Lindberg J, 1985. The effects of variable hardness, pH, alkalinity, suspended clay and humics on the chemical speciation and aquatic toxicity of copper. Report no.: EPA/600/3-86/023. Owner company: US. EPA Duluth, Minnesota. Report date: 1985-09-25. Unpublished

Nishioka H, 1975. Mutagenic activities of metal compounds in bacteria. Mutation Res. 31:185-189.

Nitka S, 1991a. Acute Dermal Toxicity in Rabbits (FIFRA). Testing laboratory: Consumer Products Testing. Report no.: Experimental reference no.: 91048-1. Report date: 1991-04-17. Unpublished

Nitka S, 1991b. Acute oral LD50 in rats. Consumer Product Testing. Report No. 91048-2. Unpublished.

Nolting RF, Helder W,  de Baar HJW, Gerringa LJA, 1999. Contrasting behaviour of trace metals in the Scheldt estuary in 1978 compared to recent years. J.Sea Res.42:275-290.

Nordlind K, Lidén S, 1992. Patch test reactions to metal salts in patients with oral mucosal lesions associated with amalgam restorations. Contact Dermatitis 27:157-160.

Nott JA, Nicolaidou A, 1994. Variable transfer of detoxified metals from snails to hermit crabs in marine food chains. Mar.Biol. 120:369-377.

O’Donohue JW, Reid MA, Varghese A, Portmann B, Williams R, 1993. Micronodular cirrhosis and acute liver failure due to chronic copper self-intoxication. Eur.J.Gastroenterol. 5:561-562.

O’Neill NC, Tanner MS, 1989. Uptake of copper from brass vessels by bovine milk and its relevance to Indian Childhood Cirrhosis. J.Pediatr.Gastroenterol.Nutr. 9:167-172.

Olin KL, Cherr GN, Rifkin E, Keen CL, 1996. The effects of some redox-active metals and reactive aldehydes on DNA-protein cross-links in vitro. Toxicol. 110:1-8.

Olivares M, Araya M, Pizarro F, Uauy R, 2001. Nausea threshold in apparently healthy individuals who drink fluids containing graded concentrations of copper. Reg.Toxicol.Pharmacol. 33:271-275.

Olivares M, Pizarro F, Speisky H, Lönnerdal B, Uauy R, 1998. Copper in infant nutrition: safety of World Health Organization provisional guidance value for copper content of drinking water. J.Pediatr.Gastroenterol.Nutr. 26(3):251-257.

Oorts K, H. Bronckaers and E. Smolders (2006a). Discrepance of the microbial response to elevated copper between freshly spiked and long-term contaminated soils. Environ Toxicol Chem., Vol. 25, No. 3, pp. 845-853, 2006.

Oorts K, Ghesquiere U, Swinnen K, Smolders E, 2006b. Soil properties affecting the toxicity of CuCl2 and NiCl2 for soil microbial processes in freshly spiked soils. Environ.Toxicol.Chem,  25(3):836-844.

Opsahl W, Zeronian H, Ellison M, Lewis D, Rucker RB, Riggins RS, 1982. Role of copper in collagen cross-linking and its influence on selected mechanical properties of chick bone and tendon. J.Nutr. 112:708-716.

Ostiguy G, Vaillancourt C, Begin R, 1995. Respiratory health of workers exposed to metal dusts and foundry fumes in a copper refinery. Occup.Environ.Med. 52:204-210.

Othman MS, Pascoe D, 2002. Reduced recruitment in Hyalella azteca (Saussure, 1858) exposed to copper. Ecotox.Environ.Safe. 53(1):59-64.

Overvad K, Wang DY, Olsen J, Allen DS, Thorling EB, Bulbrook RD, Hayward JL, 1993. Copper in human mammary carcinogenesis: A case-cohort study. Am.J.Epi. 137:409-14.

Owens RE, Balls PW, Price NB, 1997. Physicochemical processes and their effects on the composition of suspended particulate material in estuaries: Implications for monitoring and modelling. Mar.Pollut.Bull. 34(1):51-60.

Papadimitriou, E. and Loumbourdis, N. S. (2002). Exposure of the Frog Rana ridibunda to Copper: Impact on Two Biomarkers, Lipid Peroxidation, and Glutathione. Bull. Environ. Contam. Toxicol. 2002; 69:885–891.

Parmelee, R. W., Wentsel, R. S., Phillips, C. T., Simini, M. & Checkai, R. T. (1993). Soil Microcosm for Testing the Effects of Chemical Pollutants on Soil Fauna Communities and Trophic Structure. Environ. Toxicol. Chem. 12; 1477-1486.

Paucot H, Wollast R, 1997. Transport and transformation of trace metals in the Scheldt estuary. Mar.Chem. 58:229-244.

Pedersen MB, Kjær C, Elmegaard N, 2000. Toxicity and bioaccumulation of copper to black bindweed (Fallopia convolvulus) in relation to bioavailability and the age of soil contamination. Arch.Environ.Contam.Toxicol. 39:431-439.

Pedersen MB, Van Gestel CAM, 2001. Toxicity of copper to the collembolan Folsomia fimetaria in relation to the age of soil contamination. Ecotox.Environ.Safe. 49:54-59.

Pedersen MB, van Gestel CAM, Elmegaard N, 2000. Effects of copper on reproduction of two collembolan species exposed through soil, food and water. Environ.Toxicol.Chem. 19(10):2579-2588.

Peres I, Pihan JC, 1991. Study of accumulation of copper by carp (Cyprinus carpio L.): Adaptation analysis of bioconcentration by the gills. Environ.Technol. 12:167-177.

Pesch CE, Schauer PS, Balboni MA, 1986. Effect of diet on copper toxicity to Neanthes arenaceodentata. NTIS PB 87-167128. pp. 369-383. In: Poston TM, Purdy R (eds.). ASTM Special Technical Publication N°921: Aquatic Toxicology and Environmental Fate, Vol. 9. A Symposium Amer. Soc. Test. Mater. Philadelphia, PA, USA, 530 p.

Pesch, C. & Morgan, D. (1978). Influence of Sediment in Copper Toxicity Tests with the Polychaete Neanthes arenaceodentata. Water Reseach. 12; 747-751.

Pettersson R, Rasmussen F, Oskarsson A (2003). Copper in drinking water: not a strong risk factor for diarrhoea among young children. A population-based study from Sweden. Acta Paediatr 92: 473-480, 2003 [Taylor & Francis, ISSN 0803-5253].

Pettine M, Camusso M, Martinotti W, Marchetti R, Passino R, Queirazza G, 1994. Soluble and particulate metals in the Po River: factors affecting concentrations and partitioning. Sci.Total Environ. 145:243-265.

Pickering Q et al, 1977. Effect of exposure, time and copper concentration on reproduction of the fathead minnow (Pimephales promelas). Water Res. 11:1079-1083.

Pimental JC, Menezes AP, 1975. Liver granulomas containing copper in vineyard sprayer’s lung. Am.Rev.Respir. Dis. 111:189-195.

Pimental JC, Menezes AP, 1977. Liver disease in vineyard sprayers. Gastroenterology 72:275-283.

Pinheiro T, Bugalho de Almeida A, Vieira JR, Fernandes A, Alves LC, Reis M,  Carvalho Soares J, 1997. Elemental distribution in the human respiratory system and excretion organs: Absorption and accumulation. X-Ray Spectrom. 26:217-222.

Pinto SS, Henderson V, Enterline PE, 1978. Mortality experience of arsenic-exposed workers. Arch. Environ. Health 33:325-31.

Piotrowski JK, Orlowski C, Bem EM, Brys M, Baran E, 1996. The monitoring of cadmium, zinc and copper in the kidneys and liver of humans deceased in the region of Cracow (Poland). Environmental Monitoring and Assessment 43:227-236.

Pirot F, Millet J, Kalia YN, Humbert P, 1996b. In vitro study of percutaneous absorption, cutaneous bioavailability and bioequivalence of zinc and copper from five topical formulations. Skin Pharmacol. 9:259-269.

Pirot F, Panisset F, Agache P, Humbert P, 1996a. Simultaneous absorption of copper and zinc through human skin in vitro: Influence of counter-ion and vehicle. Skin Pharmacol. 9:43-52.

Pizarro F, Olivares M, Araya M, Gidi V, Uauy R, 2001. Gastrointestinal effects associated with soluble and insoluble copper in drinking water. Environ.Health Perspect. 109:949-952.

Pizarro F, Olivares M, Uauy R, Contreras P, Rebelo A, Gidi V, 1999. Acute gastrointestinal effects of graded levels of copper in drinking water. Environ.Health Perspect. 107:117-121.

Plamenac P, Santic Z, Nikulin A, Serdarevic H, 1985. Cytologic changes of the respiratory tract in vineyard spraying workers. Eur.J.Respir.Dis. 67:50-55.

Playle RC et al, 1993a. Copper and cadmium binding to fish gills: Estimates of metal-gill stability constants and modelling of metal accumulation. Can.J.Fish.Aquat.Sci. 50(12):2678-2687.

Playle RC et al, 1993b. Copper and cadmium binding to fish gills: Modification by dissolved organic carbon and synthetic ligands. Can.J.Fish.Aquat.Sci. 50(12):2667-2677.

Pohl C, Hennings U, 1999. The effect of redox processes on the partitioning of Cd, Pb, Cu and Mn between dissolved and particulate phases in the Baltic Sea. Mar.Chem. 65:41-53.

Porter DW, Yakushiji H, Nakabeppu Y, Sekiguchi M, Fivash MJ, Kasprzak KS, 1997. Sensitivity of Escherichia coli (MutT) and human (MTH1) 8-oxo-dGTPases to in vitro inhibition by the carcinogenic metals, nickel(II), copper(II), cobalt(II) and cadmium(II). Carcinogenesis 18:1785-1791.

Pratt WB, Omdahl, JL, Sorenson, RJ, 1985. Lack of effects of copper gluconate supplementation. Am.J.Clin.Nutr. 42:681- 682.

Premi PR, Cornfield AH, 1969. Effects of addition of copper, manganese, zinc and chromium compounds on ammonification and nitrification during incubation of soil. Plant Soil 31(2):345-352.

Pujol RM, Randazzo L, Miralles J, Alomar A, 1998. Perimenstrual dermatitis secondary to a copper-containing intrauterine contraceptive device. Contact Dermatitis 38:288.

Punsar S, Erämetsä O, Karvonen MJ, Ryhänen A, Hilska P, Vormano H, 1975. Coronary heart disease and drinking water. J.Chron.Dis. 28:259-287.

Quinn MR et al, 2003. Analyzing trophic transfer of metals in stream food webs using nitrogen isotopes. Sci.Total Environ. 317:73–89.

Quraishi MSI, Cornfield AH (1973. Incubation study of nitrogen mineralisation and nitrification in relation to soil pH and level of copper (II) addition. Environ.Pollut. 4:159-163.

Rae TD, Schmidt PJ, Pufahl RA, Culotta VC, O’Halloran TV, 1999. Undetectable intracellular free copper: The requirement of a copper chaperone for superoxide dismutase. Science 284, 805-808.

Rainbow PS, 1985. Accumulation of Zn, Cu and Cd by crabs and barnacles. Est.Coast.Shelf Sci. 21:669-686.

Rainbow PS, Scott AG, Wiggins ES, Jackson RW, 1980. Effect of chelating agents on the accumulation of cadmium by the barnacle Semibalanus balanoides, and complexation of soluble Cd, Zn and Cu. Mar.Ecol. 2:143-152.

Rainbow PS, White SL, 1989. Comparative strategies of heavy metal accumulation by crustaceans: zinc, copper and cadmium in a decapod and amphipod and a barnacle. Hydrobiologia 174:245-262.

Ralph A, McArdle H, 2001. Copper metabolism and copper requirements in the pregnant mother, her fetus, and children. a critical review. International Copper Association, Ltd., New York.

Rana SVS, Kumar A, 1980. Biological, haematological and histological observations in copper poisoned rats. Ind.Health 18:9-17.

Redick, M. S. and La Point, T. W. (2004). Effects of Sublethal Copper Exposure on Behavior and Growth of Rana pipiens Tadpoles. Bull. Environ. Contam. Toxicol. 2004; 72:706–710.

Redpath KJ, 1985. Growth inhibition and recovery in mussels (Mytilus edulis) exposed to low copper concentrations. J.Mar.Biol.Assoc.UK. 65(2):421-31.

Regnier P, Hoenig M, Chou L, Wollast R, 1990. Trace metals in the suspended matter collected in the mixing zone of the Rhone estuary. Water Pollut.Res.Rep. 20:388-396.

Rehwoldt R, Bida G, Nerrie B, 1971. Acute toxicity of copper, nickel and zinc ions to some Hudson River fish species. Bull.Environ.Contam.Toxicol. 6(5).

Rehwoldt R, Menapace LW, Nerrie B, Alessandrello D, 1972. The effect of increased temperature upon the acute toxicity of some heavy metal ions. Bull.Environ.Contam.Toxicol. 8(2).

Reichelt-Brushett AJ, Harrison PL, 2000. The effect of copper on the settlement success of larvae from the scleractinian coral Acropora tenuis. Mar.Pollut.Bull. 41(7-12):385-391.

Reichelt-Brushett AJ, Harrison PL, 2004. Development of a sublethal test to determine the effects of copper and lead on scleractinian Coral Larvae. Arch.Environ.Contam.Toxicol. 47:40-55.

Reichelt-Brushett AJ, Michalek-Wagner K, 2005. Effects of copper on the fertilization success of the soft coral Lobophytum compactum. Aq.Toxicol. 74:280-284.

Reimann C, Birke M, Demetriades A, Filzmoser P, O’Connor P, 2014a. Chemistry of Europe’s Agricultural Soils. Part A: Methodology and Interpretation of the GEMAS data set. Geol.Jb. B102: 528 pp., Hannover, Germany.

Reimann C, Birke M, Demetriades A, Filzmoser P, O’Connor P, 2014b. Chemistry of Europe’s Agricultural Soils. Part B: General background information and further analysis of the GEMAS data set. Geol.Jb. B103: 352 pp., Hannover, Germany.

Reinstein NH, Lönnerdal B, Keen CL, Hurley LS, 1984. Zinc-copper interactions in the pregnant rat: fetal outcome and maternal and fetal zinc, copper and iron. J.Nutr.114:1266-1279.

Rhoads FM et al, 1992. Copper toxicity and phosphorus concentration in Florida-502 oats. Soil Crop Sci.Soc.Fl.Proc. 51:18-20.

Rhoads FM, 1989. Copper toxicity in tomato plants. J.Environ.Qual. 18:195-197.

Richards Vl, Beitinger TL, 1995. Reciprocal influences of temperature and copper on survival of fathead minnows, Pimepales promelas. Bull.Environ.Contam.Toxicol. 55:230-236.

Riley, J. & Roth, I. (1971). The Distribution of Trace Elements in Some Species of Phytoplankton Grown in Culture. J. Mar. Biol. Ass. 51; 63-72.

Riley S E, 1994. Copper II sulphate pentahydrate: induction of micronuclei in the bone marrow of treated Mice. Testing laboratory: Hazleton Europe. Report no.: 456/33. Unpublished

Rimbach G, Pallauf J, Brandt K, Most E, 1995. Effect of phytic acid and microbial phytase on Cd accumulation, Zn status, and apparent absorption of Ca, P, Mg, Fe, Zn, Cu, and Mn in growing rats. Ann.Nutr.Metab. 39:361-370.

Rodriguez et al, 2010. Copper and copper compounds: Bio-elution in gastric mimetic fluids. Testing laboratory: CIMM.

Rodriguez PH, Adams W, Delbeke K, 2007.  Methodology for aquatic hazard classification of massive metal forms: the copper case.  Proceeding of the sixth International Copper Conference, Copper 2007, Volume VI pp 217-228.  Available from the VRAR, 2008, annex K2 (http://echa.europa.eu/chem_data/transit_measures/vrar_en.asp), slightly amended in August 2001. Unpublished

Rodriguez PH, Arbildua J, Urrestarazu P,  2012.  Massive copper samples,  28 days transformation/dissolution study   at pH 6 of a metallic sample mounted in epoxy resin at pH 6. Report to ECI dated 25 January 2012.  Unpublished

Rodriguez PH, Urrestarazu P, Opazo M, 2011.  Massive copper samples, transformation/dissolution protocol at pH 6, in the presence of anti-abrasion devices. Report to ECI dated 25 August 2011. Unpublished

Roesijadi G, 1980. Influence of copper on the clam Protothaca staminea: effects on gills and occurrence of copper-binding proteins. Biol.Bull. 158:233-247.

Rojas, L. X., McDowell, L. R., Cousins, R. J., Martin, F. G., Wilkinson, N. S., Johnson, A. B. and Velasquez, J. B. (1996). Interaction of different organic and inorganic zinc and copper sources fed to rats. J. Trace Elements Med. Biol. 10: 139-144.

Romaguera C, Grimalt F, 1981. Contact dermatitis from a copper containing intrauterine contraceptive device. Contact Dermatitis 7:163-164.

Rongioletti F, Rivara G, Rebora A, 1985. Contact dermatitis to a copper containing intra-uterine device. Contact Dermatitis 13:343.

Rooney CP et al, 2004. Development of a predictive model of bioavailability and toxicity of copper in soils: biological endpoints: Plant toxicity and effects of shock on microbial communities. Agricultural and Environment Division, Rothamsted Research. Report no.: No report number.

Rooney CP et al, 2006. Soil factors controlling the expression of copper toxicity to plants in a wide range of European soils. Environ.Toxicol.Chem. 25(3):726-32.

Roper CS, 2003. The in vitro percutaneous absorption of copper from various formulations through human skin. Testing laboratory: Inveresk Research, Tranent, Scotland. Report no.: 22829. Report date: 2003-08-27. Unpublished

Rosen G, Rivera-Duarte I, Kear-Padilla L, Chadwick DB, 2005. Use of laboratory toxicity tests with bivalve and echinoderm embryos to evaluate the bioavailability of copper in San Diego Bay, California, USA. Environmental Toxicology and Chemistry, Vol 24, No 2, pp 415-422, 2005.

Roth JA et al, 1971. Uptake by oats and soybeans of copper and nickel added to a peat soil. Soil Sci. 112:338-342.

Roussel H, 2005. The effects of copper on structure and function of freshwater ecosystems: a lotic mesocosm study. Ph.D. Thesis. University of Toulouse, p. 249.

Roussel, H, Ten-Hage L, Joachim S, Le Cohu R, Gauthier L, Bonzom J-M, 2007. A long-term copper exposure on a freshwater ecosystem using lotic mesocosms: primary producer community responses. Aquat. Toxicol. 81:168-182.

Ruchards Vl and Beitinger TL (1995). Reciprocal influences of temperature and copper on survival of fathead minnows, Pimepales promelas. Bulletin of Environmental Contamination toxicology, nr 55, p230-236. Testing laboratory: Department of Biological Sciences, University of North Texas, Post office box 5218, Denton, Texas 76203 USA.

Rundgren S, Van Gestel CAM, 1998. Comparison of species sensitivity. In: Handbook of Soil Invertebrate Toxicity Tests. pp 41-55 Ed. Lokke K, Van Gestel CAM. J. Wiley and Sons Ltd, Chichester, UK.

Rusov C, Zivkovic R, Jojic-Malicevic L, 1997. A study of copper genotoxicity in the micronucleus test on mice. Acta Vet. (Belgrade) 47:371-376.

Ryan AC et al, 2004. Influence of natural organic matter source on copper toxicity to larval fathead minnows (Pimephales promelas): implications for the biotic ligand model. Environ.Toxicol.Chem. 23(6):1567-1574.

Salmon MA, Wright T, 1971. Chronic copper poisoning presenting as pink disease. Arch.Dis.Child. 46:108-110.

Sample, B. E. et al. (1999). Literature-derived bioaccumulation models for earthworms: Development and validation. Environ. Toxicol. Chem. 1999; 18(9):2110-2120.

Sanders A, 2001a. Copper powder: Acute oral toxicity in the rat - Acute toxic class method. Testing laboratory: SafePharm Laboratories. Report no.: 1451/001. Unpublished

Sanders, A. (2001b). Acute dermal toxicity (limit test) in the rat. Testing laboratory: Safepharm Laboratories Limited. . SStudy number: SPL No: 1451/002. Unpublished

Sanders A, 2001c. Copper powder - Acute dermal irritation in t he rabbit. Testing laboratory: Safepharm Laboratory. Study number: project n° 1451/004. Unpublished

Sanders, A. (2001d). Copper powder: acute eye irritation in the rabbit. Testing laboratory: SafePharm Laboratories Limited. Study number: project number: 1451/005. Unpublished

Sanders A, 2001e. Copper powder: skin sensitisation in the guinea pig - Magnusson and Kligman Maximisation Method. Testing laboratory: SafePharm Laboratories Limited. Report no.: 1451/006. Unpublished

Sanders A, 2002a. Cupric Oxide: Acute dermal toxicity (Limit Test) in the rat. Testing laboratory: SafePharm Laboratories Limited. Report no.: 1645/002. Unpublished

Sanders A, 2002b Cupric oxide acute oral. Unpublished

Sanders A, 2002c. Cupric Oxide: Acute dermal irritation in the rabbit. Testing laboratory: SafePharm Laboratories Limited. Report no.: 1645/003 Unpublished

Sanders A, 2002d. Cupric oxide: Acute eye irritation in the rabbit. Testing laboratory: SafePharm Laboratories Limited. Report no.: 1645/004. Unpublished

Sanders A, 2002e. Cupric oxide: skin sensitisation in the guinea pig - Magnusson and Kligman Maximisation Method. Testing laboratory: SafePharm Laboratories Limited. Report no.: 1645/005. Unpublished

Sandifer RD, Hopkin SP, 1996. Effects of pH on the toxicity of cadmium, copper, lead and zinc to Folsomia candida Willem, 1902 (Collembola) in a standard laboratory test system. Chemosphere 33(12):2475-2486.

Sandifer RD, Hopkin SP, 1997. Effects of temperature on the relative toxicities of Cd, Cu, Pb and Zn to Folsomia candida (Collembola). Ecotox.Environ.Safe. 37:125-130.

Sanudo-Wilhelmy SA, Rivera-Duarte I, Flegal AR, 1996. Distribution of colloidal trace metals in the San Francisco Bay estuary. Geochim.Cosmochim.Acta, 60(24): 4933-4944.

Santore, R. C. et al. (2001). Biotic ligand model of the acute toxicity of metals. 2. Application to acute copper toxicity in freshwater fish and Daphnia. Environ Toxicol Chem. 2001; 20(10):2397-402.

Santore R.C., Mathew R., Paquin R.P. and DiToro D. (2002). Application of the biotic ligand model to predicting zinc toxicity to rainbow trout, fathead minnow and Daphnia magna . Comparative Biochemistry and Physiology, C133, 271-285.

Sauter S et al, 1976. Effects of exposure to heavy metals on selected freshwater fish. Toxicity of copper, cadmium, chromium and lead to eggs and fry of seven fish species. Ecological Research Series EPA-600/3-76- 105. U. S. Environmental Protection Agency, Environmental Research Laboratory, Duluth, Minnesota.

Sauvé S, Hendershot W, Allen HE, 2000. Solid-solution partitioning of metals in contaminated soils: dependence on pH, total metal burden and organic matter. Environ.Sci.Technol. 34(7):1125-1131.

Saviozzi A, LeviMinzi R, Cardelli R, Riffaldi R, 1997. The influence of heavy metals on carbon dioxide evolution from a typic xerochrept soil‘, Water Air Soil Pollut. 93:409-417.

Schäfer et al. (2003). Study report: Community level study with copper in aquatic microcosms. Study for ICA, Schmallenberg, Germany. 116 p. Unpublished.

Schäfer H, Hettler H, Fritsche U, Pitzen G, Röderer G, Wenzel A 1994. Biotests using unicellular algae and ciliates for predicting long-term effects of toxicants. Ecotox.Environ.Safe. 27:64- 81.

Scheifler R., A. de Vaufleury, M. Coeurdassier, N. Crini and PM Badot (2006). Transfer of Cd, Cu, Ni, Pb and Zn in a soil-plant-invertebrate food chain: a microcosm study. Env Toxicol & Chem., 25 (3): 815-822.

Schramel P, Müller-Höcker J, Meyer U, Weiß M, Eife R, 1988. Nutritional copper intoxication in three German infants with severe liver cell damage (features of Indian childhood cirrhosis). J.Trace Elem.Electrolytes Health Dis. 2:85-89.

Schubauer-Berigan MK, Dierkes JR, Monson PD, Ankley GT, 1993. pH dependent toxicity of Cd, Cu, Ni, Pb and Zn to Ceriodaphnia dubia, Pimepales promelas, Hyalella azteca and Lumbriculus variegatus. Environ.Toxicol.Chem. 12:1261-1266.

Scott K, Turnlund JR (1994). Compartmental model of copper metabolism in adult men. J. Nutr. Biochem. vol 5. Testing laboratory: United States Department of Agriculture~Agricultural Research Service Western Human Nutrition Research Center, Presidio of San Francisco, CA USA.

Scott R, Speakman MJ, 1991. Trace metals and endocrine studies: Their role in male infertility. In: TEMA 7 Trace Elements in Man and Animals. Momcilovic B. (ed), p30-4, Seventh International Symposium Inst Med Res Occup Health, Univ Zagreb, Croatia.

Scott-Fordsmand JJ, Krogh PH, Weeks JM, 1997. Sublethal toxicity of copper to a soil-dwelling springtail (Folsomia fimetaria) (Collembola: Isotomidae). Environ.Toxicol.Chem. 16(12):2538-2542.

Scott-Fordsmand JJ, Krogh PH, Weeks JM, 2000b. Responses of Folsomia fimetaria (Collembola: Isotomidae) to copper under different soil copper contamination histories in relation to risk assessment. Environ.Toxicol.Chem. 19(5):1297-1303.

Scott-Fordsmand JJ, Weeks JM, Hopkins SP, 2000a. Importance of contamination history for understanding toxicity of copper to earthworm Eisenia fetida (Oligochaeta: Annelida), using neutral red retention assay. Environ.Toxicol.Chem. 19(7):1774-1780.

Scudder B et al, 1988. Effects of copper on development of the fathead minnow, Pimephales promelas. Aq.Toxicol. 12:107-124.

Seim WK et al, 1984. Growth and survival of developing steelhead trout (Salmo gairdneri) continuously or intermittently exposure to copper. Can.J.Fish.Aquat.Sci. 41:433-438.

Shuster, C. N. & Pringle, B. H. (1969). Trace Metal Accumulation by the American Eastern Oyster, Crassostrea virginica. Proceedings of the National Shellfisheries Association. 59; 91-103.

Shuster, C. N and Pringle, B. H. (1969). Effects of Trace Metals on Estuarine Molluscs. Proceedings of the 1st Mid-Atlantic Industrial Waste Conference. November 13-15, 1976.

Sideris EG, Charalambous SC, Tsolomyty A, Katsaros N, 1998. Mutagenesis, carcinogenesis and the metal elements – DNA interaction. Prog.Clin.Biol.Res. 259:13-25.

Simpson S et al, 2003. Effect of declining toxicant concentrations on algal bioassay endpoints. Environ.Toxicol.Chem. 22(9):2073-2079.

Sina JF, Bean CL, Dysart GR, Taylor VI, Bradley MO, 1983. Evaluation of the alkaline elution/rat hepatocyte assay as a predictor of carcinogenic/mutagenic potential. Mutation Res. 113:357-391

Skeaff JM, Hardy DJ, 2005.  Bioavailability test studies of copper and iron powder and of sintered alloys at pH 6.  CANMET-MMSL 04-041(CR)/Contract No. 602785.  Available from the VRAR, 2008, annex K4  (http://echa.europa.eu/chem_data/transit_measures/vrar_en.asp)

Skornik WA, Brain JD, 1983. Relative toxicity or inhaled metal sulphate salts for pulmonary macrophages. Am.Rev.Respir.Dis. 128:297-303.

Skujins J, Nohrstedt H-O, Odén S, 1986. Development of a sensitive biological method for the determination of a low-level toxic contamination in soils.  Swed.J.Agric. 16:113-118.

Smit CE, van Wezel AP, Jager T, Traas TP, 2000. RIVM Report n° 601501009 Secondary poisoning of Cd, Cu, Hg for the maximum permissible concentration in water, sediments and soils

Smith SH (1986). Dermal sensitization study in guinea pigs with purple copper 97N cuprous oxide. Testing laboratory: American Biogenics Corporation. Report no.: Study No.: 480-2833. Unpublished

Smolders E, Oorts K, 2004. Development of a predicitive model of bioavailability and toxicity of copper in soils: microbial toxicity. Testing laboratory: Laboratory of Soil and Water Management, Katholieke Universiteit Leuven. Report no.: no report number.

Smyth DV, 2006. Copper: toxicity to the marine alga Skeletonema costatum. Testing laboratory: Brixham Environmental Laboratory, AstraZeneca UK Limited, Brixham, UK. Report no.: BEL report no BL8337/B. Unpublished

Smyth DV, Kent S, 2006. Copper: toxicity to the marine algae Phaeodactylum tricornutum. Testing laboratory: Brixham Environmental Laboratory, AstraZeneca UK Limited, Brixham, UK. Report no.: BEL report no. BL8338/B. Unpublished

Solbé JF. de LG, Cooper VA, 1976. Studies on the toxicity of copper sulphate to stone loach Noemacheilus barbatulus in hard water. Water Res. 10:523-527.

Sorahan T, Lister A, Gilthorpe MS, Harrington JM, 1995. Mortality of copper cadmium alloy workers with special reference to lung cancer and non-malignant diseases of the respiratory system, 1946-92. Occup.Environ.Med. 52:804-12.

Spear P, 1977. Copper accumulation kinetics and lethal tolerance in relation to fish size. Masters thesis biological science, 1977. Report date: 1977-04-01.

Spehar RL,  Fiandt JT, 1986. Acute and chronic effects of water quality criteria-based metal mixtures on three aquatic species. Environ.Toxicol.Chem. 5:917-931.

Spehar RL, Fiandt JT, 1985. Acute and chronic effects of water quality criteria based metal mixtures on three aquatic species. Environmental Research Laboratory, USEPA. Report No. EPA/600/3-85/074.

Speir TW, Kettles HA, Percival HJ, Parshotam A, 1999. Is soil acidification the cause of biochemical response when soils are amended with heavy metal salts? Soil Biol.Biochem. 31:1953-1961.

Spitalny KC, Brondum J, Vogt RL, Sargent HE, Kappel S, 1984. Drinking-water-induced copper intoxication in a Vermont family. Pediatrics 74:1103-1106.

Spurgeon D, Svendsen C, Kille P, Morgan A, Weeks J, 2004. Responses of earthworms (Lumbricus rubellus) to copper and cadmium as determined by measurement of juvenile traits in a specifically designed test system. Ecotoxicol.Environ.Safe. 57:54-64.

Spurgeon DJ, Hopkin SP, 1995. Extrapolation of the laboratory-based OECD earthworm toxicity test to metal-contaminated field sites. Ecotoxicity 4:190-205.

Spurgeon DJ, Hopkin SP, Jones DT, 1994. Effects of cadmium, copper, lead and zinc on growth, reproduction and survival of the earthworm Eisenia fetida (Savigny): assessing the environmental impact of point-source metal contamination in terrestrial ecosystems. Environ.Pollut. 84:123-130.

Srivastava AK, Gupta BN, Bihari V, Mathur N, Gaur JS, Mahendra PN, Kumar P, Bharti RS, 1992. Clinical studies in workers engaged in maintenance of watermark moulds in a paper mills. Int.Arch.Occup.Environ.Health 64:141-145.

Srivastava VK, Sengupta S, Kumar R, Misra UK, 1984) Distribution of metals of inhaled fly ash in various organs of rats at various periods after exposure. J.Environ.Sci.Health 19A:663-677.

Stagg, R. M. and Shuttleworth, T. J. (1982). The accumulation of copper in Platichthys flesus L. and its effects on plasma electrolyte concentrations. J Fish Biol 1982; 20:491-500.

Sterner W, Chibanguza G, 1988a. Akute dermale Toxizität an Ratten mit Kupferoxychlorid – Limit Test. IBR. Report No. 1-4-971-88. Unpublished.

Sterner W, hibanguza G, 1988b. Acute dermal toxicity in rats with Copper-I-Oxide. Testing laboratory: International Bio Research. Report no.: Project No.: 1-4-1602-88. Report date: 1998-01-04. Unpublished

Sterner W.  Chibanguza G, 1988c. Testing copper oxychloride on rabbits as a skin irritant.  IBR. Report No. 1-3-1314-88. Unpublished.

Sterry W, Schmoll W, 1985. Contact urticaria and dermatitis from self-adhesive pads. Contact Dermatitis. 13(4):284-5.

Stoner GD, Shimkin MB, Troxell MC, Thompson TL, Terry LS, 1976. Test for carcinogenicity of metallic compounds by the pulmonary tumour response in strain A mice. Cancer Res. 36:1744-1747.

Strandberg B, Axelsen JA, Pedersen MB, Jensen J, Attrill MJ, 2006. Effect of a copper gradient on plant community structure. Environ.Toxicol.Chem. 25(3):743-753.

Sumino M, Hayakawa K, Shibata T, Kitamura S, 1975. Heavy metals in normal Japanese tissues. Arch.Environ.Health. 30:487-494.

Svendsen C, Weeks JM, 1997a. Relevance and applicability of a simple earthworm biomarker of copper exposure: I. Links to ecological effects in a laboratory study with Eisenia andrei. Ecotoxicol.Environ.Safe. 36:72-79.

Svendsen C, Weeks JM, 1997b. Relevance and applicability of a simple earthworm biomarker of copper exposure. II Validation and applicability under field conditions in a mesocosm experiment with Lumbricus rubellus. Ecotoxicol.Environ.Safe. 36:80-88.

Takemoto K, Kawai H, Kuwahara T, Nishina M, Adachi S, 1991. Metal concentrations in human lung tissue, with special reference to age, sex, cause of death, emphysema and contamination of lung tissue. Int.Arch.Occup.Environ.Health 62:579-586.

Tambasco G et al, 2000. Phytoavailability of Cu and Zn to lettuce (Lactuca sativa) in contaminated urban soils. Can.J.Soil Sci. 80:309–317.

Tanner MS, Kantarjian AH, Bhave SA, Pandit AN, 1983. Early introduction of copper-contaminated animal milk feeds as a possible cause of Indian childhood Cirrhosis. Lancet 2:992-995.

Tanner MS, Portman B, Mowat AP, Williams R, Pandit AN, Mills CF, Bremner I, 1979. Increased hepatic copper concentration in Indian childhood cirrhosis. Lancet 1:1203-1205.

Tappin AD, Millward GE, Statham PJ, Burton JD, Morris A, 1995. Trace metals in the Central and Southern North Sea. Est.Coast.Shelf Sci. 41:275-323.

Taylor EJ et al, 1991. Evaluation of a chronic toxicity test using growth of the insect Chironomus riparius Meigen. In: Bioindicators and Environmental Management. Jeffrey DW, Madden (Eds). Academic Press, p343-352.

Taylor LN et al, 2000. Physiological effects of chronic copper exposure to rainbow trout (Oncorhynchus mykiss) in hard and soft water: evaluation of chronic indicators. Environ.Toxicol.Chem. 19(9):2298–2308.

Teisseire H et al, 1998. Toxic responses and catalase activity of Lemna minor exposed to folpet, copper and their combination. Ecotoxicol.Environ.Safe. 40:194-200.

Thompson KW, Hendricks AC, Cairns J Jr, 1980. Acute toxicity of zinc and coper singly and in combination to the Bluegill (Lepomis macrochirus). Bull.Environ.Contam.Toxicol. 25:122-129.

Timmermans KR, Walker PA, 1989. The fate of trace metals during metamorphosis of chrionomids (Diptera, Chironomidae). Environ.Pollut. 62:73-85.

Tinwell H, Ashby J, 1990. Inactivity of copper sulphate in a mouse bone-marrow micronucleus assay. Mutation Res. 245:223-226.

Tipping E, Lofts S, Lawlor AJ, 1998. Modelling the chemical speciation of trace metals in the surface waters of the Humber system. Sci.Total Environ. 210/211:63-77.

Toyokuni S, Sagripanti JL, 1994. Increased 8-hydroxydeoxyguanosine in kidney and liver of rats continuously exposed to copper. Toxicol.Applied Pharmacol. 126:91-97.

Trama FB, 1954. The acute toxicity of copper to the common bluegill. Not.Natur.Ac. Nat.Sci.Phil. 257:1-13.

Treble RG, Thompson TS, Lynch HR, 1998. Determination of copper, manganese and zinc in human liver. BioMetals 11:49-53.

Trollmann R, Neureiter D, Lang T, Dörr HG, Behrens R, 1999. Late manifestation of Indian childhood cirrhosis in a 3-year-old German girl. Eur.J.Pediatr. 158:373-378.

Tso W-W, Fung W-P, 1981. Mutagenicity of metallic cations. Toxicol.Lett. 8:195-200.

Tsuchiyama F, Hisanaga N, Shibata E, Aoki T, Takagi H, Ando T, Takeuchi Y, 1997. Pulmonary metal distribution in urban dwellers. Int.Arch.Occup.Environ.Health 70:77-84.

Turner A, Martino M, Le Roux SM, 2002. Trace metal distribution coefficients in the Mersey estuary, UK: evidence for salting out of metal complexes. Environ.Sci.Technol. 36:4578-4584.

Turner A, Millward GE, Schuchardt B, Schirmer M, Prange A, 1992. Trace metal distribution coefficients in the Weser Estuary. Cont.Shelf Res. 12(11):1277-1292.

Turnlund JR, Jacob RA, Keen CL, Strain JJ, Kelley DS, Domek JM, Keyes WR, Ensunsa JL, Lykkesfeldt J, Coulter J, 2004. Long-term high copper intake: effects on indexes of copper status, antioxidant status, and immune function in young men. Am.J.Clin.Nutr. 79:1037-1044.

Turnlund JR, Keyes WR, Anderson HL, Acord LL, 1989. Copper absorption and retention in young men at three levels of dietary copper by use of the stable isotope 65Cu. Am.J.Clin.Nutr. 49:870-878.

Turnlund JR, Keyes WR, Kim SK, Domek JM, 2005. Long-term high copper intake: effects on copper absorption, retention, and homeostasis in men. Am.J.Clin.Nutr. 81(4):822-828.

Turnlund JR, Keyes WR, Peiffer GL, Scott KC, 1998a. Copper absorption, excretion and retention by young men consuming low dietary copper determined by using the stable isotope 65Cu. Am.J.Clin.Nutr. 67:1219-1225.

Turnlund J, Keen C, Smith R (1990). Copper status and urinary and salivary copper in young men at three levels of dietary copper. Am J Clin Nutr 51:658-664.

Turnlund J, Scott K, Peiffer G, Jang A, Keyes W, Keen C, Sakanashi T (1997). Copper status of young men consuming a low-copper diet. Am. J. Clin. Nutr. 65:72-78.

Valenta P, Duursma EK, Merks AGA, Rutzel H, Nurnberg HW, 1986. Distribution of Cd, Pb and Cu between the dissolved and particulate phase in the eastern Scheldt and western Scheldt estuary. Sci.Total Environ. 53:41-76.

Van den Berg GJ, Beynen A, 1992. Influence of ascorbic acid supplementation on copper metabolism in rats. British Journal of Nutrition 68:701-715.

Van den Berg GJ, Wolterbeek HT, de Goeij JJM Beynen AC, 1995. Absorption and retention studies of trace elements and minerals in rats using radiotracers and whole-body counting. Lab.Anim. 29:66-77.

Van den Berg GJ, Yu S, Lemmens AG, Beynen AC, 1994. Ascorbic acid feeding of rats reduces copper absorption, causing impaired copper status and depressed biliary copper excretion. Biol.Trace Elem.Res. 41(1-2):47-58.

Van den Berg, G. J. and McArdle, H. J. (1994). A plasma membrane NADH oxidase is involved in copper uptake by plasma membrane vesicles isolated from rat liver. Biochimica et Biophysica Acta 1195 (1994), 276-280

Van Der Kooij LA, Van De Meent D, Van Leeuwen CJ, Bruggeman WA, 1991. Deriving quality criteria for water and sediment from the results of aquatic toxicity tests and product standards: application of the equilibrium partitioning method. Wat.Res. 25(6):697-705.

Van Dis WA, Van Gestel CAM, Sparenburg PM, 1988. Ontwikkeling van een toets ter bepaling van sublethale effecten van chemische stoffen op regenwormen. RIVM expert report. Report no.: 718480002.

Van Gestel CAM et al, 1989. Toxiciteit en bioaccumulatie van chroom (III) nitraat i de regenworm Eisenia andrei in de kunstgrond. RIVM Biltshoven Rapp. 758707001.

Van Gestel CAM, Doornekamp A, 1998. Tests on the oribatid mite, Platynothrus peltifo. In: Handbook of Soil Invertebrate Toxicity Tests. pp 113-130 Ed. Lokke H, Van Gestel CAM. J. Wiley and Sons Ltd, Chichester, UK.

Van Gestel CAM, Van Dis WA, Breemen EM, Sparenburg PM, 1989a. Development of a standardised reproduction toxicity test with the earthworm species Eisenia fetida andrei using copper, pentachlorophenol and 2,4-Dichloroaniline. Ecotoxicol.Environ.Safe. 18:305-312.

Van Gestel CAM, Van Dis WA, Dirven-van Breeman EM, Sparenburg PM, Baerselman R, 1991. Influence of cadmium, copper and pentachlorophenol on growth and sexual development of Eisenia andrei (Oligochaeta; Annelida). Biol.Fert.Soils. 12:117-121.

Van Leeuwen CJ et al, 1988. Intermittent flow system for population studies demonstrated with Daphnia and copper. Bull.Environ.Contam.Toxicol. 40:496-502.

Vesely J, Majer V, Kucera J, Havranek V, 2001. Solid-water partitioning of elements in Czech freshwaters. Appl.Geochem. 16:437-450.

Viren JR, Silvers A, 1994. Unit risk estimates for airborne arsenic exposure: an updated view based on recent data from two copper smelter cohorts. Reg.Toxicol.Pharmacol 20:125-138.

Waiwood KG, Beamish FWH, 1978a. The effect of copper, hardness and pH on the growth of rainbow trout, Salmo gairdneri. J.Fish Biol. 13:591-598.

Waiwood KG, Beamish FWH, 1978b. Effects of copper, pH and hardness the critical swimming performance of rainbow trout (Salmo gairdneri Richardson). J.Fish Biol. 12:611-619.

Walker DJ, 1990. Copper oxychloride tech: Buehler delayed contact hypersensitivity study in the guinea pig. Safepharm Laboratories Limited. Report No. 205/43. Unpublished.

Walker-Smith JA, Blomfield J, 1973. Wilson’s disease or chronic copper poisoning? Arch.Dis.Child. 48:958-962.

Wang WX, 2002. Interactions of trace metals and different marine food chains. Mar.Ecol.Progr.Ser. 243:259-309.

Wang Y, Ou Y_L, Liu Y-Q, Xie Q, Liu Q-F, Wu Q, Fan T-Q, Yan L-L, Wang J-Y, 2012. Correlations of trace element levels in the diet, blood, urine, and feces in the Chinese male. Biological Trace Element Research 145(2):127:135.

Ward PJ, 1994. Copper II sulphate pentahydrate: measurement of unscheduled DNA synthesis in rat liver using an in vivo/in vitro procedure. Testing laboratory: Hazleton Europe. Report no.: 456/32. Unpublished

Weis P, Weis JS, 1999. Accumulation of metals in consumers associated with chromated copper arsenate-treated wood panels. Mar.Environ.Res. 48:73-81.

Weiss KC, Linder MC, 1985. Copper transport in rats involving a new plasma protein. Am.J.Physiol. 249, E77-E88.

Welch K, Higgins I., Oh M, Burshfiel C, 1982. Arsenic exposure, smoking and respiratory cancer in copper smelter workers. Arch.Environ.Health 37:325-335.

Wesson CM, 2003. Copper oxychloride: Acute inhalation toxicity (nose-only) study in the rat. SafePharm Laboratories. Report no. 1632/006. Unpublished.

Wesson, CM. (2001). Copper powder: Acute inhalation toxicity (nose only) study in the rat. Testing laboratory: SafePharm Laboratories. Report no.: 1451/003.

Williams TD, Hayfield AJ, 2006. Copper effects on the survival, development and reproduction of the marine copepod Tisbe battagliai. Testing laboratory: Brixham Environmental Laboratory, AstraZeneca UK Limited, Freshwater Quarry, Brixham, UK. Report no.: BEL report N° BL8295/B.

Winner RW, 1985. Bioaccumulation and toxicity of copper as affected by interactions between humic acid and water hardness. Water Res. 19(4):449-455.

Winner, R. W. (1984). The Toxicity and Bioaccumulation of Cadmium and Copper as Affected by Humic Acid. AquaticToxicology. 5: 267-274.

Wittassek R, 1987. Kupferaufnahme bei verschiedenen Bodenwirbellosen in kupferbelasteten Weinbergsböden. Verhandlungen der Gesellschaft für Ökologie 16:383-391.

Wolf BW, Firkins JL, Zhang X-S, 1998. Varying dietary concentrations of fructo-oligosaccharides affect apparent absorption and balance of minerals in growing rats. Nutr.Res. 18:1791-1806.

Wong MH, Cheung YH, 1986. Heavy metal concentrations in caterpillars fed with waste-grown vegetables. Agric.Wastes 18: 61-68.

Wong PK (1988). Mutagenicity of heavy metals. Toxicol. 40:597-603.

Wyllie J, 1957. Copper poisoning at a cocktail party. Am.J.Public Health 47:617.

Xia L, Liang S, Zhou X, Xia Y, 1989. Concentration of trace elements in human tissues in Hunan, China, and the relation of ratios of mutual metals to human health. Trace Elem.Med. 6:114-118.

Young JS, Gurtisen JM, Apts CW, Crecelius EA, 1979. The relationship between the copper complexing capacity of sea water and copper toxicity in shrimp zoeae. Mar.Environ.Res. 2:344-348.

Yu S, West CE, Beynen AC, 1994. Increasing intakes of iron reduce status, absorption and biliary excretion of copper in rats. Brit.J.Nutrit. 71:887-895.

Zacarias I, Yanez CG, Araya M, Oraka C, Olivares M, Uauy R, 2001. Determination of the taste threshold of copper in water. Chem.Senses 26:85-89.

Zhao F-J, Rooney CP, Zhang H, McGrath SP, 2006. Comparison of soil solution speciation and diffusive gradients in thin-films measurement as an indicator of copper bioavailability to plants. Environ.Toxicol.Chem. 25(3):733-742.

Zhou JL, Liu YP, Abrahams P, 2003a. Trace metal behaviour in the Conwy estuary, North Wales. Chemosphere 51:429-440.

Zietz BP, Dieter HH, Lakomek M, Schneider H, Keßler-Gaedtke B, Dunkelberg H, 2003a. Epidemiology investigation on chronic copper toxicity to children exposed via the public drinking water supply. Sci.Total Environ. 302:127-144.

Zietz BP, J. Dassel de Vergara and H. Dunkelberg (2003b). Copper concentration in tap water and possible effects on infant’s health – Results of a study in Lower Saxony, Germany. Environmental Research 92 (2003) 129-138.

Zuurdeeg BW, Van Enk RJ, Vriend SP, 1992. Natuurlijke achtergrond gehalten van zware metalen en enkele andere sporenelementen in Nederlands oppervlaktewater. Geochem-Research, Utrecht (in Dutch). Abrahams PW, Thornton I, 1994. The contamination of agricultural land in the metalliferous province of southwest England – implications to livestock. Agric.Ecosyst.Environ. 48:125-137.

ANNEXES

# Annex I : Overview of ecotoxicological data (reliable study results) for five copper substances and category rational matrix for environmental endpoints

| Endpoint | Coated Copper Flakes, copper powders and copper massive | Copper sulphate pentahydrate | Copper (II) oxide | Copper (I) oxide | Dicopper chloride trihydroxide |
| --- | --- | --- | --- | --- | --- |
| Biodegradation | In traditional terms, degradation (i.e., hydro-, photo-, and biodegradation) are not applicable to inorganic Cu compounds |  |  |  |  |
| Bioaccumulation | Not applicable to essential nutrient | Not applicable to essential nutrient | Not applicable to essential nutrient | Not applicable to essential nutrient | Not applicable to essential nutrient |
| Acute environmental toxicity | Acute ecotoxicity of soluble copper salts and Transformation/dissolution data | Acute ecotoxicity of soluble copper salts and Molecular weight translation | Acute ecotoxicity of soluble copper salts and Transformation/dissolution data | Acute ecotoxicity of soluble copper salts and Molecular weight translation | Acute ecotoxicity of soluble copper salts and Molecular weight translation |
| Chronic environmental toxicity | Chronic ecotoxicity of soluble copper salts and Transformation/dissolution data | Chronic ecotoxicity of soluble copper salts and Molecular weight translation | Chronic ecotoxicity of soluble copper salts and Transformation/dissolution data | Chronic ecotoxicity of soluble copper salts and Molecular weight translation | Chronic ecotoxicity of soluble copper salts and Molecular weight translation |

# Annex II : Overview of toxicological data (reliable study results) for five copper substances  and category rational matrix for human health

|  | Copper powder | Copper flakes | Copper sulphate pentahydrate | Copper (I) oxide | Copper (II) oxide | Dicopper chloride trihydroxide |
| --- | --- | --- | --- | --- | --- | --- |
| Acute toxicity, oral | OECD TG 401: LD50 > 2000 mg/kg(1) | OECD TG 401: LD50 > 300-500 mg/kg | OECD TG 401: LD50 = 482 mg/kg | OECD TG 401: LD50 = 1340 mg/kg | OECD TG 401: LD50 > 2000 mg/kg | OECD TG 401: LD50 = 299 mg/kg (mice) LD50 = 1398 mg/kg (rats) |
| Acute toxicity, dermal | No data (read-across) | OECD TG 402: LD50 > 2000 mg/kg | OECD TG 402: LD50 > 2000 mg/kg | OECD TG 402: LD50 > 2000 mg/kg | OECD TG 402: LD50 > 2000 mg/kg | OECD TG 402: LD50 > 2000 mg/kg |
| Acute toxicity, inhalation | Considered not toxic based on physical (particles size) and chemical properties of copper powder/massive | OECD TG 403: LC50 = 0.733 mg/L and > 5 mg/L | Considered not toxic by inhalation based on particle size of the substance | OECD TG 403: LC50 = 3.34 mg/L | Considered not toxic based on physical and chemical properties of copper (II) oxide | OECD TG 403: LC50 = 2.83 mg/L |
| Skin irritation | No data (read-across) | OECD TG 404; EU Method B4: not irritating | OECD TG 404: not irritating | OECD TG 404; EU Method B4: not irritating | OECD TG 404: not irritating | OECD TG 404; EU Method B4: not irritating |
| Eye irritation | no data | OECD TG 405; EU Method B5: irritating | OECD TG 405; EU Method B5: Irritating to the eye | OECD TG 405: Irritating to the eye | OECD TG 405; EU Method B5: not irritating | OECD TG 405; EU Method B5: not irritating |
| Respiratory irritation | no data | no data | no data | no data | no data | no data |
| Skin Sensitisation | No data (read-across) | OECD TG 406: not sensitising | OECD TG 406: not sensitising | OECD TG 406: not sensitising | OECD TG 406: not sensitising | OECD TG 406: not sensitising |
| Mutagenicity/Genetic toxicity | no data (read-across from copper sulphate) | no data (read-across from copper sulphate) | Not mutagenic based on 3 GLP and OECD guideline studies | no data (read-across from copper sulphate) | no data (read-across from copper sulphate) | no data (read-across from copper sulphate) |
| Repeated dose toxicity, oral | no data (read-across from copper sulphate) | no data (read-across from copper sulphate) | OECD TG xx: NOAEL= 16.7 mg/kg/bw/day (Cu added as copper sulphate pentahydrate) | no data (read-across from copper sulphate) | no data (read-across from copper sulphate) | no data (read-across from copper sulphate) |
| Repeated dose toxicity, inhalation | no data (read-across from copper (I) oxide) | no data (read-across from copper (I) oxide)) | no data (read-across from copper (I) oxide) | OECD TG 412: NOAEL >1.78 mg Cu/m3 (added as cuprous oxide) | no data (read-across from copper (I) oxide) | no data (read-across from copper (I) oxide)) |
| Reproductive toxicity: effects on fertility | no data (read-across from copper sulphate) | no data (read-across from copper sulphate) | 2-gen study (OECD TG 416) NOAEL, fertility = 23.6 mg Cu/kg/day) | no data (read-across from copper sulphate) | no data (read-across from copper sulphate) | no data (read-across from copper sulphate) |
| Reproductive toxicity: developmental toxicity | Read-across from Cu(OH)2: OECD TG 414: NOAEL maternal toxicity = 6 mgCu/kg/day | Read-across from Cu(OH)2: OECD TG 414: NOAEL maternal toxicity = 6 mgCu/kg/day | Read-across from Cu(OH)2: OECD TG 414: NOAEL maternal toxicity = 6 mgCu/kg/day | Read-across from Cu(OH)2: OECD TG 414: NOAEL maternal toxicity = 6 mgCu/kg/day | Read-across from Cu(OH)2: OECD TG 414: NOAEL maternal toxicity = 6 mgCu/kg/day | Read-across from Cu(OH)2: OECD TG 414: NOAEL maternal toxicity = 6 mgCu/kg/day |
| Carcinogenicity | no data indicating carcinogenicity (data for repeated dose considered as worst-case) | no data indicating carcinogenicity (data for repeated dose considered as worst-case) | no data indicating carcinogenicity (data for repeated dose considered as worst-case) | no data indicating carcinogenicity (data for repeated dose considered as worst-case) | no data indicating carcinogenicity (data for repeated dose considered as worst-case) | no data indicating carcinogenicity (data for repeated dose considered as worst-case) |

(1): read-accross, based on oral toxicity of Cu2+ and  bi-elution tests

# Annex III : Overview of reported partition coefficients for copper for different environmental compartments

| Values | Remarks | References & Reliability scoring |
| --- | --- | --- |
| Suspended particulate matter – fresh water Typical Kd (50th percentile): 30,246 L/kg (log Kd: 4.48) |  |  |
| Log Kd: 4.17, 4.4, 3.87, 3.85, 4.4, 3.92-5, 3.36-5.46, 4.91-5.15, 3.79-5.1, 4.31, 4.5, 4.11-4.91, 3.7-5.3, 4.7-5.79, 4.55, 4.38, 4.54 | Review – different publications that report reliable Kd values and, where possible, provides information on parameters that most likely affect the copper binding | Heijerick and Van Sprang, 2008 (R2) |
| Log Kd: 3.3-3.7 | Estimated using modeling | Tipping et al, 1998 (R2) |
| Log Kd: 4.31, 4.5 | Field measurements | Elbaz-Poulichet et al, 1996 (R2) |
| Log Kd: 4.18-4.95, 4.11-4.91, 4.11-5.3, 3.7-4.84, 4.78-5.79, 4.7-5 | Field measurements | Golimowski et al, 1990 (R2) |
| Log Kd: 3.92-5, 3.36-5.46, 4.91-5.15, 3.79-5.1 | Field measurements / estimated using modeling | Lofts and Tipping, 2000 (R2) |
| Log Kd (median): 5 Log Kd (Average): 4.6 | Field measurements | Pettine et al, 1994 (R2) |
| Log Kd: 4.5 | Field measurements | Regnier et al, 1990 |
| Log Kd: 4.15 | Field measurements | Turner et al, 1992 (R2) |
| Log Kd: 4.66 | Field measurements | Turner et al, 2002 (R2) |
| Log Kd: 4.43 | Field measurements | Vesely et al, 2001 (R2) |
| Suspended particulate matter – brackish water Typical Kd (50th percentile): 56,234 L/kg (log Kd: 4.75) |  |  |
| Log Kd: 4.76, 4.65, 4.67, 5.01, 5.53, 4.1, 4.61, 4.88, 3.85, 5.22, 4.74, 4.5, 4.6, 4.74, 6.16, 4.55, 4.8 | Review – measurements of dissolved and suspended particulate matter copper concentrations | Heijerick and Van Sprang, 2008 (R2) |
| Log Kd: 5.2 – 6.3 | Field measurements | Paucot and Wollast, 1997 (R2) |
| Log Kd: 4.2 – 4.9 | Field measurements | Sanudo-Wilhelmy et al, 1996 (R2) |
| Log Kd: 4.78, 4.64 | Field measurements | Owens et al, 1997 (R2) |
| Log Kd: 4.54 – 6.46 | Field measurements | Zhou et al, 2003 (R2) |
| Log Kd: 4.8, 4.7, 4.5-4.8 | Field measurements (salinity 5-15 g/L) | Nolting et al, 1999 (R2) |
| Log Kd: 5.14, 4.74 | Field measurements (salinity 0-15 g/L) | Monteney et al, 1993 (R2) |
| Log Kd (median values): 5.49, 5.47, 5.23, 5.31, 5.2 | Field measurements | Pohl and Hennings, 1999 (R2) |
| Log Kd: 3-5.1 | Field measurements | Benoit et al, 1994 |
| Log Kd: 4.4-5.3, 6.1 | Field measurements | Valenta et al, 1986 (R2) |
| Log Kd: 4.6 | Field measurements | Martin et al, 1993 5R2) |
| Suspended particulate matter – salt water Typical Kd (50th percentile): 131,826 L/kg (log Kd: 5.12) |  |  |
| Log Kd: 5.62, 5.11, 4.6, 5, 5.6, 4.78 | Review – measurements of dissolved and suspended particulate matter copper concentrations | Heijerick and Van Sprang, 2008 (R2) |
| Log Kd: 4.78 | Field measurements | Tappin et al, 1995 (R2) |
| Log Kd: 4.5 – 5.9 | Field measurements | Eckhard Helmers, 1996 (R2) |
| Log Kd: 5.11 – 5.92 | Field measurements | McManus and Prandle, 1996 5R2) |
| LogKd: 3.7 – 5.4 | Field measurements | Munksgaard and Parry, 2001 (R2) |
| Log Kd: 4.5, 4.8-4.9, 4.5 | Field measurements (salinity 20-30 g/L) | Nolting et al, 1999 (R2) |
| Log Kd: 4.36 | Field measurements (salinity 25-35 g/L) | Monteney et al, 1993 (R2) |
| Log Kd: 5 | Field measurements | Martin et al, 1993 5R2) |
| Sediment – fresh water Typical Kd (50th percentile): 24,409 L/kg (log Kd: 4.39) |  |  |
| Log Kd: 4.67, 4.51-4.65, 4.56, 4.75, 4.74, 4.7, 4.68, 4.6, 4.67, 4.67, 4.38, 4.4, 3.92, 4.98, 4.95, 5.04 | Review – different publications that report reliable Kd values and, where possible, provides information on parameters that most likely affect the copper binding to sediment | Heijerick and Van Sprang, 2008 (R2) |
| Mean Log Kd: 2.68 (aqua regia destruction) Mean Log Kd: 2.84 (HNO3 destruction) | Field measurements | De Groot et al, 1998 (R2) |
| Log Kd: 4.66 | Calculated from published data | Van Der Kooij et al, 1991 (R2) |
| Kd: 7100-25200 Log Kd: 3.85-4.4 | Modeling | Koelmans and Radanovic, 1998 (R2) |
| Soil Typical Kd (50th percentile): 2,120 L/kg (log Kd: 3.33) |  |  |
| Average Kd: 500 L/kg (aqua regia) Average Kd: 700 L/kg (HNO3 extract) | Dutch soils | De Groot 1998 |
| Kd: 25-135 (forest soil) Kd: 92-4318 soil from river bank | Batch equilibrium method | Janssen et al, 1997ab (R2) |
| Mean Log Kd: 3.68 Median Log Kd: 3.3 | Compilation of 70 studies | Sauvé et al, 2000 (R2) |

Annex IV: Summary of acute inhalation toxicity studies in animals

| Reference | Substance | Method | Test animals | Dose levels | Results |
| --- | --- | --- | --- | --- | --- |
| Blagden, 1999 | Copper (I) oxide Nordox Agro Grade | OECD 403, nose only Exposure duration: 4 hours. Post exposure period: 14 days | Rat, Sprague-Dawley, male / female (10 animals per dose group, 5 males and 5 females). | Group 1: Mean achieved 5.25 mg/l (SD 0.44); Nominal 12.2 mg/l. Group 2: Mean achieved 2.37 mg/l (SD 0.42); Nominal 4.90 mg/l. Group 3: Mean achieved 1.28 mg/l (SD 0.36); Nominal 2.58 mg/l. | Male LC50: 2.92 mg/l air. Female LC50: 3.69 mg/l air. Male / female LC50: 3.34mg/l air. |
| Dickhaus, 1988 | Copper (I) oxide. | OECD 403 Exposure duration: 4 hours. Post exposure period: 14 days | Rat, Wistar, male / female (10 animals per dose group, 5 males and 5 females). | Group 1 (powder) - 60g/h. Group 2 (suspension) - 60 ml/h at 10%. Group 3 (suspension) - 60 ml/h at 40%. Nominal concentrations: 30 mg/l. | Male / female LC50: > 30 mg/l air (nominal). |
| Fulfs, 1990 | Copper (I) oxide | OECD 403 Exposure duration: 4 hours. Post exposure period: 14 days. | Rat, Sprague-Dawley, 5 males and 5 females. | 5.075 mg/l (5 mg/l nominal). | Male / female LC50: > 5mg/l air. |
| Greenough and McDonald, 1985a. | Copper (I) oxide | OECD 403 Exposure duration: 4 hours. Post exposure period: 14 days. | Rat, Sprague-Dawley, male / female (5 males and 5 females). | Nominal concentration = 15.02 mg/l Analytical concentration = 5.78 ± 0.94 mg/l | Male / female LC50: ca. 5mg/l air (analytical). |
| Greenough and McDonald, 1985b. | Copper (I) oxide. | OECD 403 Exposure duration: 4 hours. Post exposure period: 14 days. | Rat, Sprague-Dawley, male / female (5 males and 5 females). | Group 1: 20.10 mg/ml (nominal), 5.09 mg/l (analytical). Group 2: 11.53 mg/ml (nominal), 3.45 mg/l (analytical). Group 3: 12.54 mg/ml (nominal), 4.43 mg/l (analytical). | Male / female LC50: 5.63mg/l air. |
| Wesson, CM, 2003 | dicopper chloride trihydroxide | inhalation: dust (nose only) EU Method B.2 (Acute Toxicity (Inhalation)) OECD Guidance 403 (Acute Inhalation Toxicity) | rat (Crl:CD (SD) IGS BR) male/female |  | LC50 (4 h): 2.83 mg/L air (male) based on: test mat. LC50 (4 h): > 2.77 mg/L air (female) based on: test mat. LC50 (4 h): 4.74 mg/L air (male/female) based on: test mat. |
| Holbert, MS, 1993 | dicopper chloride trihydroxide | inhalation: dust (whole body) US Department of Transport 49 CFR 173.132 | rat (HSD:SD) male/female |  | LC50 (1 h): > 11.4 mg/L air (male/female) based on: test mat. |
| Leuschner J, 2011 | Coated copper flakes | Inhalation: nose only OECD 436 | Rat (Sprague-Dawley), m/f |  | LC50 (4h): >5.11 mg/L air (m/f) Based on: test mat. |
| Holbert, MS, 1994 | Copper sulphate pentahydrate | inhalation: dust (whole body) US Department of Transport 49 CFR 173.132 | rat (HSD:SD) male/female |  | LC50 (1 h): > 2.95 mg/L air (male/female) based on: test mat. |
| Leuschner J, 2011 | Coated copper flakes | Inhalation: nose only OECD 403 | Rat (Sprague-Dawley), m/f |  | LC50 (4 h): 1.03 mg/L air (analytical) (male/female) LC50 (4 h): 0.733 mg/L air (analytical) (male) LC50 (4 h): 1.67 mg/L air (analytical) (female) |

# Annex V: Summary of acute dermal toxicity studies in animals

| Reference a | Substance |  |  | Reported test method | Vehicle | Species (strain) | Sex | LD50 value (mg/kg BW) | Study ratingb |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Identity | Purity | Cu content (% w/w Cu) |  |  |  |  |  |  |
| Cuthbert & Carr (1986) a | Copper (I) oxide | NR | NR | Annex V B.3 | Water | Rat (SD) | M+F | >2000 | 1c |
| Driscoll (1999b) a | Copper (I) oxide | NR | NR | Annex V B.3; OECD 402 | Distilled water | Rat (SD) | M+F | >2000 | 1c |
| Chibanguza (1988) a | Copper (I) oxide | NR | NR | OECD (limit test) | Water | Rat (Wistar) | M+F | >2000 | 1c |
| Sterner & Chibanguza (1988b) a | Copper (I) oxide | NR | NR | Annex V B.3; OECD 402 | Water | Rat (Wistar) | M+F | >2000 | 1c |
| Nitka (1991a) a | Copper (I) oxide | NR | NR | Annex V B.3 | Water | Rabbit (NZ White) | M+F | >2000 | 1c |
| Kukulinski (1976) a | Copper (I) oxide | NR | NR | Not specified |  | Albino rabbit | M | >2000 | 2 |
| Sanders (2002b) a | Copper (II) oxide | 97.7 | NR | Annex V B.3 | Water | Rat (SD) | M+F | >2000 | 1 |
| Lheritier (1993) a | Copper sulphate | >99 | NR | Annex V B.3 | Water | Rat (SD) | M+F | >2000 | 1 |
| Sterner & Chibanguza (1988a) a | Dicopper chloride trihydroxide | NR | NR | Annex V B.3; OECD 402 | Water | Rat (Wistar) | M+F | >2000 | 1c |
| Deenihan (1987) a | Dicopper chloride trihydroxide | NR | NR | US EPA 81-2 | Deionised water | Rabbit (NZ White) | M+F | >2000 | 1c |
| Forster & Luperi (1987b) a | Dicopper chloride trihydroxide | NR | 57.72 | Annex V B.3; OECD 402 | Distilled water | Rabbit (NZ White) | M+F | >2000 | 1c |
| Jackson (1994) a | Dicopper chloride trihydroxide | NR | NR | Annex V B.3; OECD 402 | Distilled water | Rat (SD) | M+F | >2000 | 1c |
| Sanders (2001a) a | Copper powder | NR | NR | Annex V B.3 | Arachis oil | Rat (SD) | M+F | >2000 | 1c |

NR - not reported.      a  unpublished report.      b  based on evaluation criteria developed by Klimisch et al (1997).

c  a study rating of ‘1’ was considered appropriate for this study, even though purity data were not provided.

# Annex VI: Summary of acute oral toxicity studies in animals

| Reference a | Substance |  |  | Reported test method | Vehicle | Species (strain) | Sex | LD50 value (mg/kg BW) | Study ratingb |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Identity | Purity (%) | Cu content (% w/w Cu) |  |  |  |  |  |  |
| Collier & Wilson (1984c) a | Copper (I) oxide | NR | NR | Annex V B.1 | Arachis oil | Rat (SD) | M+F | 1340 | 1d |
| Cuthbert & Carr (1986) a | Copper (I) oxide | NR | NR | Annex V B.1 | Carboxymethyl cellulose | Rat (SD) | M F M+F | 1295 1209 1291 | 1 d |
| Driscoll (1999a) a | Copper (I) oxide (Nordox agro grade) | NR | NR | Annex V B.1 (tris); OECD 423 | Distilled water | Rat (SD) | M+F | 300-500 c | 1 d |
| Nitka (1991b) a | Copper (I) oxide | NR | NR | Annex V B.1 | Corn oil | Rat (Wistar) | M+F | 5400 | 2 |
| Hixson (1973) a | Copper (I) oxide | NR | NR | NR | 10% Tween | Rat (SD) | NR | >5000 | 3 |
| Smyth et al (1969) in IPCS (1998) | Copper (I) oxide | NR | NR | NR | NR | Rat | NR | 470 | 4 |
| Sanders (2002a) a | Copper (II) oxide | 97.7 | NR | Annex V B.1 (Fixed Dose) | Arachis oil | Rat (SD) | M+F | >2500 | 1 |
| Lheritier (1994) a | Copper sulphate (pentahydrate) | >99 | NR | Annex V B.1 | Water | Rat (SD) | M+F | 482 | 1 |
| Manciaux (1998) a | Copper sulphate (pentahydrate) | NR | 25.4% | Annex V B.1; OECD 401 | Purified water | Rat (SD) | F | 666 | 1 d |
| Lehman (1951) | Copper sulphate (anhydrous) | NR | NR | NR | NR | Rat | NR | 300 | 3 |
| Smyth et al (1969) in IPCS (1998) | Copper sulphate (pentahydrate) | NR | NR | NR | NR | Rat | NR | 960 | 4 |
| Deenihan (1987) a | Dicopper chloride trihydroxide | NR | NR | US EPA 81-1 | Deionised water | Rat (SD) | F M | 950 1200 | 1 d |
| Forster & Luperi (1987a) a | Dicopper chloride trihydroxide | NR | 57.72 | Annex V B.1; OECD 401 | Corn oil | Rat (SD) | M F M+F | 1796 2006 1862 | 1 d |
| Jackson (1994) a | Dicopper chloride trihydroxide | NR | NR | Annex V B.1; OECD 401 | Carboxymethyl cellulose | Rat (SD) | M F M+F | 1083 1854 1398 | 1 d |
| Haynes (1998) a | Dicopper chloride trihydroxide | NR | 57.3 | Annex V B.1; OECD 401 | Alembicol D | Mouse (CD-1) | M+F | 299 | 1 d |

NR - not reported.

a  unpublished report

b  based on evaluation criteria developed by Klimisch et al (1997)

c  estimated range based on Fixed Dose method.

d  a study rating of ‘1’ was considered appropriate for this study, even though purity data were not provided.

# Annex VII: Summary of case-reports of copper toxicity in children

| Reference | Subject(s) | Details of condition | Outcome/treatment | Possible causative factors investigated | Other information |
| --- | --- | --- | --- | --- | --- |
| Salmon and Wright (1971) | Child (15 months) | Diarrhoea, weight loss, red extremities, elevated serum Cu, liver disease (elevated enzyme markers). No histopathology reported. | Improvement following Cu-chelation treatment | Cu in water (hot - 0.79 mg/L; cold 0.35 mg/L) cf 0.04-0.08 mg/L in other locations in study. |  |
| Walker-Smith and Blomfield (1973) | Boy (14 months) | Abdominal swelling, ascites, micronodular cirrhosis of liver, elevated plasma and urinary Cu, elevated liver Cu | Deterioration and death (liver failure) | Cu in water 6.75 mg/L (cold) and 9.7 mg/L (hot; first draw). |  |
| Bhandari and Sharda (1982) | One twin (10 months) | Hepatosplenomegaly, micronodular cirrhosis of liver, elevated serum and urinary Cu | NR | Infant fed on cows’ milk (boiled and stored in Cu utensils for up to 6h); Cu levels 3.04 mg/L. | Second twin (breast-fed until 10 months) was unaffected |
| Müller-Höcker et al (1988) | 2 children (5 and 9 months) | Abdominal distention, hepatosplenomegaly. Liver biopsy (micronodular cirrhosis, ballooning hepatocytes, Mallory bodies, intracellular Cu). Serum Cu normal. Elevated Cu in liver (698 µg/g wet wt; 2154 µg/dry wt). | Death in one case from liver failure | Condition attributed to Cu in drinking water, but no supportive data provided. |  |
| Schramel et al (1988) | 3 children (<1y) from 2 families | Elevated Cu in liver (1240, 1870 and 1450 µg/g dry wt) | Two deaths (liver failure) | Cu in drinking water (from well, supplied through Cu pipes) 0.43 and 3.4 mg/L in two affected households. |  |
| Adamson et al (1992) | Boy (2 y) | Weight loss, abdominal distention, abnormal liver enzyme markers, hepatosplenomegaly, jaundice, ascites, oedema. Elevated urinary Cu. Liver biopsy (micronodular cirrhosis, ballooning degeneration, Mallory bodies, Cu-containing granules). Elevated Cu in liver (1500 µg/g dry wt). | Death at 32 months from gastrointestinal bleed | Cu in drinking water 1.13 mg/L |  |
| Baker et al (1995) | 3 children (10, 14 and 29 months) | Abdominal distention, mild jaundice, ascites, hepatosplenomegaly, hypoglycaemia, elevated serum bilirubin. Abnormal liver histopathology (micronodular cirrhosis, ballooning hepatocytes, Mallory bodies, Cu-containing granules). Elevated Cu in liver (1245, 1310 and 1100 µg/g dry wt). | Case 1 and 2 - deterioration in liver function and development of encephalopathy, followed by death. Case 3 - liver transplant followed by successful recovery. | Cu levels in hot standing and cold running water were: Case 1 – 8.0 and 3.9 mg/L, respectively. Case 2 – 6.3 and 2.3 mg/L, respectively. Case 3 - < 0.1 mg/L. | Case 2 – other siblings exposed to same water did not develop symptoms |
| Trollman et al (1999) | Girl (3.5 y) | Symptoms first occurred at 3.5y. Abdominal distention, jaundice, hepatomegaly. Liver biopsy (micronodular cirrhosis, Cu deposits). Elevated Cu in liver (1400 µg/g dry wt). | Death resulted from progressive liver failure | Cu levels in standing water were 8.7 mg/L. Child drank ~1litre of tap water per day. | Family moved to house with water from well, supplied through Cu pipes when child was 2 y. |

# Annex VIII: Overview of acute ecotoxicity values for standard freshwater species and physico-chemical characteristics of the test media.

# Reported LC50values represent measured concentration levels, unless specified otherwise.

| Organism | LC50 (µg/L) | Temperature (°C) | pH | DOC (mg/L) | Hardness (mg CaCO3/L) | Medium | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudokirchneriella subcapitata | EbC50: 152.0 | 20-25 | 7.5 (pH at sampling time was 5.94, pH variation during test >0.5) | 8.91 | 10.1 | field water | Heijerick et al, 2005 |
| Pseudokirchneriella subcapitata | EbC50: 97.4 | 20-25 | 6.31 | 2.72 | 10.1 | field water | Heijerick et al, 2005 |
| Pseudokirchneriella subcapitata | EbC50: 194.0 | 20-25 | 5.52 | 10.3 | 7.9 | field water | Heijerick et al, 2005 |
| Pseudokirchneriella subcapitata | EbC50: 84.0 | 20-25 | 6.10 | 2.34 | 12.4 | field water | Heijerick et al, 2005 |
| Pseudokirchneriella subcapitata | EbC50: 32.0 | 20-25 | 7.4 (pH at sampling time was 6.39, pH variation during test >0.5)) | 1.55 | 27.8 | field water | Heijerick et al, 2005 |
| Pseudokirchneriella subcapitata | EbC50: 163 | 20-25 | 7.5 | 20.4 | 134.0 | field water | Heijerick et al, 2005 |
| Pseudokirchneriella subcapitata | EbC50: 52.7 | 20-25 | 8.23 | 2.52 | 48.5 | field water | Heijerick et al, 2005 |
| Pseudokirchneriella subcapitata | EbC50: 113 | 20-25 | 7.55 | 6.13 | 132.0 | field water | Heijerick et al, 2005 |
| Pseudokirchneriella subcapitata | EbC50: 245 | 20-25 | 7.3 | 17.8 | 166.0 | field water | Heijerick et al, 2005 |
| Pseudokirchneriella subcapitata | EbC50: 36.9 | 20-25 | 8.20 | 1.7 | 169.0 | field water | Heijerick et al, 2005 |
| Pseudokirchneriella subcapitata | EbC50: 33.9 | 20-25 | 8.06 | 1.98 | 191.0 | field water | Heijerick et al, 2005 |
| Pseudokirchneriella subcapitata | EbC50: 108.0 | 20-25 | 8.26 | 6.42 | 220.0 | field water | Heijerick et al, 2005 |
| Pseudokirchneriella subcapitata | EbC50: 65.4 | 20-25 | 8.30 | 8.24 | 238.0 | field water | Heijerick et al, 2005 |
| Pseudokirchneriella subcapitata | EbC50: 16.5 | 20-25 | 7.8 | < 1.0 | 23.8 | reconstituted water | Heijerick et al, 2005 |
| Pseudokirchneriella subcapitata | ErC50: 230 |  | 6.19 | 5.23 | 100 | reconstituted water | De Schamphelaere, 2006 |
| Pseudokirchneriella subcapitata | ErC50: 824 |  | 6.22 | 15.8 | 100 | reconstituted water | De Schamphelaere, 2006 |
| Pseudokirchneriella subcapitata | ErC50: 93 |  | 7.92 | 5.99 | 400 | reconstituted water | De Schamphelaere, 2006 |
| Pseudokirchneriella subcapitata | ErC50: 35 |  | 7.05 | 2.21 | 250 | reconstituted water | De Schamphelaere, 2006 |
| Pseudokirchneriella subcapitata | ErC50: 268 |  | 8.01 | 15.1 | 400 | reconstituted water | De Schamphelaere, 2006 |
| Pseudokirchneriella subcapitata | ErC50: 156 |  | 7.09 | 9.99 | 250 | reconstituted water | De Schamphelaere, 2006 |
| Pseudokirchneriella subcapitata | ErC50: 190 |  | 7.04 | 9.89 | 500 | reconstituted water | De Schamphelaere, 2006 |
| Pseudokirchneriella subcapitata | ErC50: 219 |  | 8.37 | 10.3 | 250 | reconstituted water | De Schamphelaere, 2006 |
| Pseudokirchneriella subcapitata | ErC50: 462 |  | 7.17 | 18.5 | 250 | reconstituted water | De Schamphelaere, 2006 |
| Pseudokirchneriella subcapitata | ErC50: 199 |  | 6.2 | 5.31 | 100 | reconstituted water | De Schamphelaere, 2006 |
| Pseudokirchneriella subcapitata | ErC50: 811 |  | 6.2 | 15.6 | 100 | reconstituted water | De Schamphelaere, 2006 |
| Pseudokirchneriella subcapitata | ErC50: 92 |  | 8.07 | 5.75 | 400 | reconstituted water | De Schamphelaere, 2006 |
| Pseudokirchneriella subcapitata | ErC50: 35 |  | 7.08 | 2.06 | 250 | reconstituted water | De Schamphelaere, 2006 |
| Pseudokirchneriella subcapitata | ErC50: 346 |  | 8.05 | 16.1 | 400 | reconstituted water | De Schamphelaere, 2006 |
| Pseudokirchneriella subcapitata | ErC50: 178 |  | 7.09 | 11.1 | 250 | reconstituted water | De Schamphelaere, 2006 |
| Pseudokirchneriella subcapitata | ErC50: 281 |  | 7.12 | 10.5 | 500 | reconstituted water | De Schamphelaere, 2006 |
| Pseudokirchneriella subcapitata | ErC50: 161 |  | 8.25 | 10.3 | 250 | reconstituted water | De Schamphelaere, 2006 |
| Pseudokirchneriella subcapitata | ErC50: 685 |  | 7.11 | 19.9 | 250 | reconstituted water | De Schamphelaere, 2006 |
| Pseudokirchneriella subcapitata | ErC50: 122 |  | 6.18 | 5.07 | 100 | reconstituted water | De Schamphelaere, 2006 |
| Pseudokirchneriella subcapitata | ErC50: 368 |  | 6.17 | 14.9 | 100 | reconstituted water | De Schamphelaere, 2006 |
| Pseudokirchneriella subcapitata | ErC50: 51 |  | 7.92 | 5.46 | 400 | reconstituted water | De Schamphelaere, 2006 |
| Pseudokirchneriella subcapitata | ErC50: 30 |  | 7.02 | 1.95 | 250 | reconstituted water | De Schamphelaere, 2006 |
| Pseudokirchneriella subcapitata | ErC50: 151 |  | 7.78 | 15.2 | 400 | reconstituted water | De Schamphelaere, 2006 |
| Pseudokirchneriella subcapitata | ErC50: 99 |  | 7.04 | 10.1 | 250 | reconstituted water | De Schamphelaere, 2006 |
| Pseudokirchneriella subcapitata | ErC50: 105 |  | 7.02 | 10.4 | 500 | reconstituted water | De Schamphelaere, 2006 |
| Pseudokirchneriella subcapitata | ErC50: 55 |  | 8.58 | 10.5 | 250 | reconstituted water | De Schamphelaere, 2006 |
| Pseudokirchneriella subcapitata | ErC50: 174 |  | 6.95 | 18.2 | 250 | reconstituted water | De Schamphelaere, 2006 |
| Pseudokirchneriella subcapitata | ErC50: 205 |  | 5.99 | 5.64 | 400 | reconstituted water | De Schamphelaere, 2006 |
| Pseudokirchneriella subcapitata | ErC50: 59 |  | 8.02 | 5.42 | 100 | reconstituted water | De Schamphelaere, 2006 |
| Pseudokirchneriella subcapitata | ErC50: 209 |  | 8.05 | 15.3 | 100 | reconstituted water | De Schamphelaere, 2006 |
| Pseudokirchneriella subcapitata | ErC50: 756 |  | 5.68 | 9.84 | 250 | reconstituted water | De Schamphelaere, 2006 |
| Pseudokirchneriella subcapitata | ErC50: 193 |  | 7.19 | 10.4 | 25 | reconstituted water | De Schamphelaere, 2006 |
| Pseudokirchneriella subcapitata | ErC50: 102 |  | 7.03 | 9.98 | 250 | reconstituted water | De Schamphelaere, 2006 |
| Pseudokirchneriella subcapitata | ErC50: 100 |  | 7.01 | 10.2 | 250 | reconstituted water | De Schamphelaere, 2006 |
| Chlamydomonas reinhardtii | ErC50: 380 |  | 6.02 | 9.84 | 250 | field water | De Schamphelaere, 2006 |
| Chlamydomonas reinhardtii | ErC50: 315 |  | 7.03 | 9.84 | 250 | field water | De Schamphelaere, 2006 |
| Chlamydomonas reinhardtii | ErC50: 146 |  | 8.11 | 9.84 | 250 | field water | De Schamphelaere, 2006 |
| Chlorella vulgaris | ErC50: 333 |  | 6.03 | 5.17 | 100 | field water | De Schamphelaere, 2006 |
| Chlorella vulgaris | ErC50: 773 |  | 6.04 | 15.49 | 100 | field water | De Schamphelaere, 2006 |
| Chlorella vulgaris | ErC50: 99 |  | 7.92 | 5.04 | 400 | field water | De Schamphelaere, 2006 |
| Chlorella vulgaris | ErC50: 506 |  | 7.97 | 15.82 | 400 | field water | De Schamphelaere, 2006 |
| Chlorella vulgaris | ErC50: 296 |  | 7.03 | 10.81 | 250 | field water | De Schamphelaere, 2006 |
| Chlorella vulgaris | ErC50: 254 |  | 7.01 | 10.03 | 500 | field water | De Schamphelaere, 2006 |
| Chlorella vulgaris | ErC50: 60 |  | 7.04 | 1.5 | 250 | field water | De Schamphelaere, 2006 |
| Chlorella vulgaris | ErC50: 200 |  | 8.75 | 9.9 | 250 | field water | De Schamphelaere, 2006 |
| Chlorella vulgaris | ErC50: 446 |  | 7.05 | 19.9 | 250 | field water | De Schamphelaere, 2006 |
| Chlorella vulgaris | ErC50: 440 |  | 6.01 | 5.03 | 400 | field water | De Schamphelaere, 2006 |
| Chlorella vulgaris | ErC50: 987 |  | 6.05 | 15.24 | 400 | field water | De Schamphelaere, 2006 |
| Chlorella vulgaris | ErC50: 111 |  | 7.88 | 5.31 | 100 | field water | De Schamphelaere, 2006 |
| Chlorella vulgaris | ErC50: 380 |  | 7.88 | 15.66 | 100 | field water | De Schamphelaere, 2006 |
| Chlorella vulgaris | ErC50: 602 |  | 5.5 | 10.27 | 250 | field water | De Schamphelaere, 2006 |
| Chlorella vulgaris | ErC50: 238 |  | 7.07 | 10.26 | 25 | field water | De Schamphelaere, 2006 |
| Chlorella vulgaris | ErC50: 364 |  | 7.03 | 10.81 | 250 | field water | De Schamphelaere, 2006 |
| Chlorella vulgaris | ErC50: 208 |  | 7.04 | 10.23 | 250 | field water | De Schamphelaere, 2006 |
| Daphnia magna | 100 |  | 7.59 | 1.5 |  | reconstituted water | De Schamphelaere et al, 2002 |
| Daphnia magna | 200 |  | 7.29 | 3.4 |  | reconstituted water | De Schamphelaere et al, 2002 |
| Daphnia magna | 106 |  | 6.4 | 2.2 |  | reconstituted water | De Schamphelaere et al, 2002 |
| Daphnia magna | 276 |  | 7.65 | 3.6 |  | reconstituted water | De Schamphelaere et al, 2002 |
| Daphnia magna | 292 |  | 6.11 | 6.9 |  | reconstituted water | De Schamphelaere et al, 2002 |
| Daphnia magna | 92.6 |  | 6.77 | 1.4 |  | reconstituted water | De Schamphelaere et al, 2002 |
| Daphnia magna | 210 |  | 8.39 | 3.8 |  | reconstituted water | De Schamphelaere et al, 2002 |
| Daphnia magna | 152 |  | 7.67 | 1.7 |  | reconstituted water | De Schamphelaere et al, 2002 |
| Daphnia magna | 526 |  | 7.71 | 7.8 |  | reconstituted water | De Schamphelaere et al, 2002 |
| Daphnia magna | 826 |  | 8.27 | 14.2 |  | reconstituted water | De Schamphelaere et al, 2002 |
| Daphnia magna | 388 |  | 7.74 | 4.0 |  | reconstituted water | De Schamphelaere et al, 2002 |
| Daphnia magna | 157 |  | 8.46 | 2.5 |  | reconstituted water | De Schamphelaere et al, 2002 |
| Daphnia magna | 136 |  | 6.95 | 2.5 |  | reconstituted water | De Schamphelaere et al, 2002 |
| Daphnia magna | 229 |  | 7.35 | 3.7 |  | reconstituted water | De Schamphelaere et al, 2002 |
| Daphnia magna | 244 |  | 8.39 | 4.4 |  | reconstituted water | De Schamphelaere et al, 2002 |
| Daphnia magna | 100 |  | 6.85 | 1.3 |  | reconstituted water | De Schamphelaere et al, 2002 |
| Daphnia magna | 1213 |  | 7.41 | 14.8 |  | reconstituted water | De Schamphelaere et al, 2002 |
| Daphnia magna | 421 |  | 6.29 | 9.0 |  | reconstituted water | De Schamphelaere et al, 2002 |
| Daphnia magna | 300 |  | 7.43 | 4.6 |  | reconstituted water | De Schamphelaere et al, 2002 |
| Daphnia magna | 289 |  | 7.81 | 4.8 |  | reconstituted water | De Schamphelaere et al, 2002 |
| Daphnia magna | 117 |  | 6.98 | 1.4 |  | reconstituted water | De Schamphelaere et al, 2002 |
| Daphnia magna | 109 |  | 7.0 | 2.2 |  | reconstituted water | De Schamphelaere et al, 2002 |
| Daphnia magna | 465 |  | 6.31 | 8.3 |  | reconstituted water | De Schamphelaere et al, 2002 |
| Daphnia magna | 798 |  | 8.46 | 10.9 |  | reconstituted water | De Schamphelaere et al, 2002 |
| Daphnia magna | 380 |  | 7.83 | 4.2 |  | reconstituted water | De Schamphelaere et al, 2002 |
| Daphnia magna | 35.2 |  | 5.52 | 10.3 | 8.0 | field water | De Schamphelaere et al, 2002 |
| Daphnia magna | 37.9 |  | 6.31 | 2.72 | 10.1 | field water | De Schamphelaere et al, 2002 |
| Daphnia magna | 33.8 |  | 6.1 | 2.34 | 12.4 | field water | De Schamphelaere et al, 2002 |
| Daphnia magna | 40.9 |  | 6.35 | 4.87 | 18.1 | field water | De Schamphelaere et al, 2002 |
| Daphnia magna | 295 |  | 7.02 | 10.1 | 52.7 | field water | De Schamphelaere et al, 2002 |
| Daphnia magna | 648 |  | 7.5 | 20.4 | 133.4 | field water | De Schamphelaere et al, 2002 |
| Daphnia magna | 792 |  | 7.35 | 22.8 | 139.7 | field water | De Schamphelaere et al, 2002 |
| Daphnia magna | 686 |  | 7.3 | 17.8 | 165.4 | field water | De Schamphelaere et al, 2002 |
| Daphnia magna | 281 |  | 7.59 | 7.81 | 69.7 | field water | De Schamphelaere et al, 2002 |
| Daphnia magna | 484 |  | 7.7 | 10.0 | 92.0 | field water | De Schamphelaere et al, 2002 |
| Daphnia magna | 366 |  | 7.72 | 7.88 | 108.1 | field water | De Schamphelaere et al, 2002 |
| Daphnia magna | 332 |  | 7.85 | 11.8 | 112.7 | field water | De Schamphelaere et al, 2002 |
| Daphnia magna | 399 |  | 7.87 | 14.3 | 116.6 | field water | De Schamphelaere et al, 2002 |
| Daphnia magna | 529 |  | 7.6 | 18.4 | 125.1 | field water | De Schamphelaere et al, 2002 |
| Daphnia magna | 175 |  | 7.55 | 6.13 | 131.7 | field water | De Schamphelaere et al, 2002 |
| Daphnia magna | 276 |  | 7.93 | 6.60 | 135.7 | field water | De Schamphelaere et al, 2002 |
| Daphnia magna | 119 |  | 8.06 | 1.98 | 190.2 | field water | De Schamphelaere et al, 2002 |
| Daphnia magna | 188 |  | 8.26 | 6.42 | 219.3 | field water | De Schamphelaere et al, 2002 |
| Daphnia magna | 257 |  | 8.3 | 8.24 | 236.4 | field water | De Schamphelaere et al, 2002 |
| Daphnia magna | 7.0 | 25 | 7.5 |  | 45 | tap water | Cairns et al, 1978 |
| Daphnia magna | 10.0 | 20 | 7.5 |  | 45 | tap water | Cairns et al, 1978 |
| Daphnia magna | 40.0 | 15 | 7.5 |  | 45 | tap water | Cairns et al, 1978 |
| Daphnia magna | 70.0 | 10 | 7.5 |  | 45 | tap water | Cairns et al, 1978 |
| Daphnia magna | 90.0 | 5 | 7.5 |  | 45 | tap water | Cairns et al, 1978 |
| Daphnia magna | 26 | 21.6 | 7.5 |  | 143 | well water | Lewis, 1983 |
| Daphnia magna | 9.8 | 18 | 7.7 |  | 45.3 | field water | Biesinger and Christensen, 1972 |
| Daphnia magna | 60.0 | 18 | 7.7 |  | 45.3 | field water | Biesinger and Christensen, 1972 |
| Daphnia magna | 26 | 18.2 | 7.8 |  | 52 | well water | Chapman et al, 1980 |
| Daphnia magna | 30 | 20.3 | 7.9 |  | 105 | well water | Chapman et al, 1980 |
| Daphnia magna | 38 | 19.7 | 8.1 |  | 106 | well water | Chapman et al, 1980 |
| Daphnia magna | 69 | 19.9 | 8.3 |  | 207 | well water | Chapman et al, 1980 |
| Daphnia magna | 52 | 20 | 8.2 |  | 170 | reconstituted water | Lazorchak, 1987 |
| Daphnia magna | 51 | 20 | 8.2 |  | 170 | reconstituted water | Lazorchak, 1987 |
| Daphnia magna | 65 | 20 | 8.2 |  | 170 | reconstituted water | Lazorchak, 1987 |
| Daphnia magna | 58 | 20 | 8.2 |  | 170 | reconstituted water | Lazorchak, 1987 |
| Daphnia magna | 35 | 20 | 8.2 |  | 170 | reconstituted water | Lazorchak, 1987 |
| Daphnia magna | 37 | 20 | 8.2 |  | 170 | reconstituted water | Lazorchak, 1987 |
| Daphnia magna | 46 | 20 | 8.2 |  | 170 | reconstituted water | Lazorchak, 1987 |
| Daphnia magna | 28 | 20 | 8.2 |  | 170 | reconstituted water | Lazorchak, 1987 |
| Daphnia magna | 31 | 20 | 8.2 |  | 170 | reconstituted water | Lazorchak, 1987 |
| Daphnia magna | 50 | 20 | 8.2 |  | 170 | reconstituted water | Lazorchak, 1987 |
| Daphnia magna | 39 | 20 | 8.2 |  | 170 | reconstituted water | Lazorchak, 1987 |
| Daphnia magna | 31 | 20 | 8.2 |  | 170 | reconstituted water | Lazorchak, 1987 |
| Daphnia magna | 38 | 20 | 8.2 |  | 170 | reconstituted water | Lazorchak, 1987 |
| Daphnia magna | 20 | 20 | 8.2 |  | 170 | reconstituted water | Lazorchak, 1987 |
| Daphnia magna | 22 | 20 | 8.2 |  | 170 | reconstituted water | Lazorchak, 1987 |
| Daphnia magna | 23 | 20 | 8.2 |  | 170 | reconstituted water | Lazorchak, 1987 |
| Daphnia magna | 25 | 20 | 8.2 |  | 170 | reconstituted water | Lazorchak, 1987 |
| Daphnia magna | 26 | 20 | 8.2 |  | 170 | reconstituted water | Lazorchak, 1987 |
| Daphnia magna | 27 | 20 | 8.2 |  | 170 | reconstituted water | Lazorchak, 1987 |
| Daphnia magna | 27 | 20 | 8.2 |  | 170 | reconstituted water | Lazorchak, 1987 |
| Daphnia magna | 28 | 20 | 8.2 |  | 170 | reconstituted water | Lazorchak, 1987 |
| Daphnia magna | 28 | 20 | 8.2 |  | 170 | reconstituted water | Lazorchak, 1987 |
| Daphnia magna | 32 | 20 | 8.2 |  | 170 | reconstituted water | Lazorchak, 1987 |
| Daphnia magna | 33 | 20 | 8.2 |  | 170 | reconstituted water | Lazorchak, 1987 |
| Daphnia magna | 34 | 20 | 8.2 |  | 170 | reconstituted water | Lazorchak, 1987 |
| Daphnia magna | 36 | 20 | 8.2 |  | 170 | reconstituted water | Lazorchak, 1987 |
| Daphnia magna | 37 | 20 | 8.2 |  | 170 | reconstituted water | Lazorchak, 1987 |
| Daphnia magna | 39 | 20 | 8.2 |  | 170 | reconstituted water | Lazorchak, 1987 |
| Daphnia magna | 42 | 20 | 8.2 |  | 170 | reconstituted water | Lazorchak, 1987 |
| Daphnia magna | 43 | 20 | 8.2 |  | 170 | reconstituted water | Lazorchak, 1987 |
| Daphnia magna | 44 | 20 | 8.2 |  | 170 | reconstituted water | Lazorchak, 1987 |
| Daphnia magna | 46 | 20 | 8.2 |  | 170 | reconstituted water | Lazorchak, 1987 |
| Daphnia magna | 30 | 19 | 8.2 |  | 170 | reconstituted water | Lazorchak, 1993 |
| Daphnia magna | 31 | 19 | 8.2 |  | 170 | reconstituted water | Lazorchak, 1993 |
| Daphnia magna | 31 | 19 | 8.2 |  | 170 | reconstituted water | Lazorchak, 1993 |
| Daphnia magna | 35 | 19 | 8.2 |  | 170 | reconstituted water | Lazorchak, 1993 |
| Daphnia magna | 37 | 19 | 8.2 |  | 170 | reconstituted water | Lazorchak, 1993 |
| Daphnia magna | 38 | 19 | 8.2 |  | 170 | reconstituted water | Lazorchak, 1993 |
| Daphnia magna | 39 | 19 | 8.2 |  | 170 | reconstituted water | Lazorchak, 1993 |
| Daphnia magna | 46 | 19 | 8.2 |  | 170 | reconstituted water | Lazorchak, 1993 |
| Daphnia magna | 50 | 19 | 8.2 |  | 170 | reconstituted water | Lazorchak, 1993 |
| Daphnia magna | 51 | 19 | 8.2 |  | 170 | reconstituted water | Lazorchak, 1993 |
| Daphnia magna | 52 | 19 | 8.2 |  | 170 | reconstituted water | Lazorchak, 1993 |
| Daphnia magna | 58 | 19 | 8.2 |  | 170 | reconstituted water | Lazorchak, 1993 |
| Daphnia magna | 63 | 19 | 8.2 |  | 170 | reconstituted water | Lazorchak, 1993 |
| Daphnia magna | 31.8 | 20 | 8.4 |  |  | reconstituted water | Borgmann and Ralph, 1983 |
| Ceriodaphnia dubia | 66 | 25 | 8.2 |  | 100 | field water | Sphehar and Fiandt, 1985 |
| Ceriodaphnia dubia | 9.5 | 25 | 6.3 |  | 290 | reconstituted water | Schubauer-Berigan et al, 1993 |
| Ceriodaphnia dubia | 28 | 25 | 7.3 |  | 290 | reconstituted water | Schubauer-Berigan et al, 1993 |
| Ceriodaphnia dubia | 200 | 25 | 8.3 |  | 290 | reconstituted water | Schubauer-Berigan et al, 1993 |
| Ceriodaphnia dubia | 8.5 | 23 | 7.6 |  | 85 | reconstituted water | Cerda and Olive, 1993 |
| Ceriodaphnia dubia | 8.5 | 23 | 7.6 |  | 85 | reconstituted water | Cerda and Olive, 1993 |
| Ceriodaphnia dubia | 10.8 | 23 | 7.6 |  | 85 | reconstituted water | Cerda and Olive, 1993 |
| Ceriodaphnia dubia | 10.8 | 23 | 7.6 |  | 85 | reconstituted water | Cerda and Olive, 1993 |
| Ceriodaphnia dubia | 36.9 | 23 | 7.6 |  | 85 | reconstituted water | Cerda and Olive, 1993 |
| Ceriodaphnia dubia | 39 | 23 | 7.6 |  | 85 | reconstituted water | Cerda and Olive, 1993 |
| Ceriodaphnia dubia | 46.9 | 23 | 7.6 |  | 85 | reconstituted water | Cerda and Olive, 1993 |
| Ceriodaphnia dubia | 46.3 | 23 | 7.6 |  | 85 | reconstituted water | Cerda and Olive, 1993 |
| Ceriodaphnia dubia | 14 | 25 | 6.0 | 2.9 (2) | 97.6 | field water | Belanger and Cherry, 1990 |
| Ceriodaphnia dubia | 52 | 25 | 6.0 | 2 (3) | 113.6 | field water | Belanger and Cherry, 1990 |
| Ceriodaphnia dubia | 56 | 25 | 6.0 | 3 (4) | 182 | field water | Belanger and Cherry, 1990 |
| Ceriodaphnia dubia | 28 | 25 | 7.0 | 2.9 (2) | 97.6 | field water | Belanger and Cherry, 1990 |
| Ceriodaphnia dubia | 76 | 25 | 7.0 | 2 (3) | 113.6 | field water | Belanger and Cherry, 1990 |
| Ceriodaphnia dubia | 84 | 25 | 7.0 | 3 (4) | 182 | field water | Belanger and Cherry, 1990 |
| Ceriodaphnia dubia | 31 | 25 | 8.0 | 2.9 (2) | 97.6 | field water | Belanger and Cherry, 1990 |
| Ceriodaphnia dubia | 91 | 25 | 8.0 | 2 (3) | 113.6 | field water | Belanger and Cherry, 1990 |
| Ceriodaphnia dubia | 93 | 25 | 8.0 | 3 (4) | 182 | field water | Belanger and Cherry, 1990 |
| Oncorhynchus mykiss | 80 | 12 | 7.7 |  | 120 | well water | Seim et al, 1984 |
| Oncorhynchus mykiss | 4.2 | 15.8 | 5.7 |  | 9.2 | well water | Cusimano et al, 1986 |
| Oncorhynchus mykiss | 2.8 | 15.8 | 7.0 |  | 9.2 | well water | Cusimano et al, 1986 |
| Oncorhynchus mykiss | 57 | 9 | 7.6 |  | 42 | well water | Chapman and Stevens, 1978 |
| Oncorhynchus mykiss | 17 | 12.2 | 7.1 |  | 22 | field water | Chapman, 1978 |
| Oncorhynchus mykiss | 18 | 12.2 | 7.1 |  | 22 | field water | Chapman, 1978 |
| Oncorhynchus mykiss | 28 | 12.2 | 7.1 |  | 22 | field water | Chapman, 1978 |
| Oncorhynchus mykiss | 29 | 12.2 | 7.1 |  | 22 | field water | Chapman, 1978 |
| Oncorhynchus mykiss | 28.9 | 15 | 6.0 |  | 31 | well water | Howard and Sprague, 1978 |
| Oncorhynchus mykiss | 22.4 | 15 | 6.0 |  | 32 | well water | Howard and Sprague, 1978 |
| Oncorhynchus mykiss | 40 | 15 | 6.0 |  | 101 | well water | Howard and Sprague, 1978 |
| Oncorhynchus mykiss | 70 | 15 | 6.0 |  | 366 | well water | Howard and Sprague, 1978 |
| Oncorhynchus mykiss | 82.2 | 15 | 6.0 |  | 371 | well water | Howard and Sprague, 1978 |
| Oncorhynchus mykiss | 31.9 | 15 | 6.0 |  | 99 | well water | Howard and Sprague, 1978 |
| Oncorhynchus mykiss | 81.1 | 15 | 7.0 |  | 100 | well water | Howard and Sprague, 1978 |
| Oncorhynchus mykiss | 47.4 | 15 | 7.0 |  | 101 | well water | Howard and Sprague, 1978 |
| Oncorhynchus mykiss | 298 | 15 | 7.0 |  | 361 | well water | Howard and Sprague, 1978 |
| Oncorhynchus mykiss | 30 | 15 | 8.0 |  | 31 | well water | Howard and Sprague, 1978 |
| Oncorhynchus mykiss | 309 | 15 | 8.0 |  | 360 | well water | Howard and Sprague, 1978 |
| Oncorhynchus mykiss | 516 | 15 | 8.0 |  | 371 | well water | Howard and Sprague, 1978 |
| Oncorhynchus mykiss | 83.3 | 12.8 | 7.8 |  | 194 | well water | Chakoumakos et al, 16979 |
| Oncorhynchus mykiss | 85.3 | 12.8 | 7.8 |  | 194 | well water | Chakoumakos et al, 16979 |
| Oncorhynchus mykiss | 103 | 12.8 | 7.8 |  | 194 | well water | Chakoumakos et al, 16979 |
| Oncorhynchus mykiss | 128 | 12.8 | 7.8 |  | 194 | well water | Chakoumakos et al, 16979 |
| Oncorhynchus mykiss | 165 | 12.8 | 7.8 |  | 194 | well water | Chakoumakos et al, 16979 |
| Oncorhynchus mykiss | 169 | 12.8 | 7.8 |  | 194 | well water | Chakoumakos et al, 16979 |
| Oncorhynchus mykiss | 197 | 12.8 | 7.8 |  | 194 | well water | Chakoumakos et al, 16979 |
| Oncorhynchus mykiss | 221 | 12.8 | 7.8 |  | 194 | well water | Chakoumakos et al, 16979 |
| Oncorhynchus mykiss | 243 | 12.8 | 7.8 |  | 194 | well water | Chakoumakos et al, 16979 |
| Oncorhynchus mykiss | 274 | 12.8 | 7.8 |  | 194 | well water | Chakoumakos et al, 16979 |
| Oncorhynchus mykiss | 514 | 12.8 | 7.8 |  | 194 | well water | Chakoumakos et al, 16979 |
| Oncorhynchus mykiss | 102 | 15 | 8.2 |  | 362 | well water | Fogels and Sprague, 1977 |
| Oncorhynchus mykiss | 164 | 13.5 | 7.3 |  | 33 | field water | Buckley, 1983 |
| Oncorhynchus mykiss | 286 | 13.5 | 7.5 |  | 41 | field water | Buckley, 1983 |
| Oncorhynchus mykiss | 94 | 6.0 | 6.8 |  | 18.3 | field water | Mudge et al, 1993 |
| Oncorhynchus mykiss | 89 | 9.5 | 7.1 |  | 18.6 | field water | Mudge et al, 1993 |
| Oncorhynchus mykiss | 93 | 4.4 | 6.9 |  | 23.7 | field water | Mudge et al, 1993 |
| Oncorhynchus mykiss | 68 | 15.3 | 7.2 |  | 28.6 | field water | Mudge et al, 1993 |
| Oncorhynchus mykiss | 890 |  | 7.4 |  | 300 | lab water | Calamari and Marchetti, 1973 |
| Oncorhynchus mykiss | 253 |  | 7.4 |  |  | field water | Hale, 1977 |
| Oncorhynchus mykiss | 9.5 | 10 | 7.6 |  | 24.2 | well water | Marr et al, 1999 |
| Oncorhynchus mykiss | 10.5 | 10 | 7.6 |  | 24.2 | well water | Marr et al, 1999 |
| Oncorhynchus mykiss | 12.5 | 10 | 7.6 |  | 24.2 | well water | Marr et al, 1999 |
| Oncorhynchus mykiss | 16.5 | 10 | 7.6 |  | 24.2 | well water | Marr et al, 1999 |
| Oncorhynchus mykiss | 21.5 | 10 | 7.6 |  | 24.2 | well water | Marr et al, 1999 |
| Oncorhynchus mykiss | 18 | 9.8 | 7.5 |  | 25.1 | well water | Marr et al, 1998 |
| Oncorhynchus mykiss | 190 | 10 | 7.8 |  | 125 | tap water | Spear, 1977 |
| Oncorhynchus mykiss | 200 | 10 | 7.8 |  | 125 | tap water | Spear, 1977 |
| Oncorhynchus mykiss | 210 | 10 | 7.8 |  | 125 | tap water | Spear, 1977 |
| Oncorhynchus mykiss | 20 | 15 | 7.5 |  | 54 | tap water | Skidmore and Firth, 1983 |
| Brachydanio rerio | 149 | 25 | 8;2 |  | 362 | well water | Fogels and Sprague, 1977 |
| Brachydanio rerio | 35 | 24 | 7.2 |  | 100 | reconstituted water | Bresch, 1982 |
| Cyprinus carpio | 800 | 28 | 8.0 |  | 55 | field water | Rehwoldt, 1972 |
| Cyprinus carpio | 810 | 17 | 7.8 |  | 53 | field water | Rehwoldt, 1971 |
| Pimephales promelas | 15 | 25 | 6.3 |  | 290 | reconstituted water | Schubauer-Berigan et al, 1993 |
| Pimephales promelas | 75 | 22 | 7.1 |  | 31.4 | field water | Mount and Stephan, 1969 |
| Pimephales promelas | 84 | 22 | 7.1 |  | 31.4 | field water | Mount and Stephan, 1969 |
| Pimephales promelas | 96 | 25 | 7.4 |  | 43.9 | field water | Spehart and Fiandt, 1985 |
| Pimephales promelas | 600 | 9.5 | 8.5 |  | 212 | field water | Brungs et al, 1976 |
| Pimephales promelas | 690 | 21.5 | 8.3 |  | 120 | field water | Brungs et al, 1976 |
| Pimephales promelas | 750 | 6.5 | 8.2 |  | 244 | field water | Brungs et al, 1976 |
| Pimephales promelas | 830 | 17.5 | 8.1 |  | 228 | field water | Brungs et al, 1976 |
| Pimephales promelas | 930 | 21.5 | 8.1 |  | 150 | field water | Brungs et al, 1976 |
| Pimephales promelas | 980 | 18 | 8.0 |  | 224 | field water | Brungs et al, 1976 |
| Pimephales promelas | 820 | 12.5 | 8.1 |  | 260 | field water | Brungs et al, 1976 |
| Pimephales promelas | 630 | 25.5 | 8.2 |  | 280 | field water | Brungs et al, 1976 |
| Pimephales promelas | 750 | 3 | 8.0 |  | 280 | field water | Brungs et al, 1976 |
| Pimephales promelas | 770 | 21.5 | 8.4 |  | 280 | field water | Brungs et al, 1976 |
| Pimephales promelas | 730 | 23.5 | 8.0 |  | 294 | field water | Brungs et al, 1976 |
| Pimephales promelas | 860 | 22.5 | 8.3 |  | 298 | field water | Brungs et al, 1976 |
| Pimephales promelas | 840 | 26 | 8.3 |  | 308 | field water | Brungs et al, 1976 |
| Pimephales promelas | 870 | 18.5 | 8.2 |  | 310 | field water | Brungs et al, 1976 |
| Pimephales promelas | 44 | 25 | 7.3 |  | 290 | reconstituted water | Schubauer-Berigan et al, 1993 |
| Pimephales promelas | >200 | 25 | 8.3 |  | 290 | field water | Schubauer-Berigan et al, 1993 |
| Pimephales promelas | 430 | 20.8 | 7.9 |  | 198 | field water | Mount, 1968 |
| Pimephales promelas | 470 | 20.8 | 7.9 |  | 198 | field water | Mount, 1968 |
| Pimephales promelas | 460 | 22 | 7.8 |  | 202 | field water | Pickering et al, 1977 |
| Pimephales promelas | 490 | 22 | 7.8 |  | 202 | field water | Pickering et al, 1977 |
| Pimephales promelas | 1090 | 11 | 7.4 |  | 260 | field water | Geckler et al, 1977 |
| Pimephales promelas | 920 | 15 | 7.5 |  | 240 | field water | Geckler et al, 1977 |
| Pimephales promelas | >610 | 24.5 | 7.4 |  | 266 | field water | Geckler et al, 1977 |
| Pimephales promelas | <640 | 25.5 | 7.5 |  | 280 | field water | Geckler et al, 1977 |
| Pimephales promelas | >780 | 21.5 | 7.5 |  | 280 | field water | Geckler et al, 1977 |
| Pimephales promelas | 960 | 24.5 | 7.5 |  | 282 | field water | Geckler et al, 1977 |
| Pimephales promelas | 1400 | 25 | 7.4 |  | 298 | field water | Geckler et al, 1977 |
| Pimephales promelas | 970 | 26 | 7.2 |  | 308 | field water | Geckler et al, 1977 |
| Pimephales promelas | 810 | 18.5 | 7.3 |  | 310 | field water | Geckler et al, 1977 |
| Pimephales promelas | 780 | 21 | 7.5 |  | 324 | field water | Geckler et al, 1977 |
| Pimephales promelas | >800 | 23.5 | 7.3 |  | 336 | field water | Geckler et al, 1977 |
| Pimephales promelas | 465 | 24 | 5.0 |  | 200 | lab water | Geckler et al, 1977 |
| Pimephales promelas | 600 | 3 | 8.2 |  | 206 | field water | Geckler et al, 1977 |
| Pimephales promelas | 640 | 8.5 | 8.1 |  | 242 | field water | Geckler et al, 1977 |
| Pimephales promelas | 660 | 9.5 | 8.5 |  | 212 | field water | Geckler et al, 1977 |
| Pimephales promelas | 690 | 3 | 8.0 |  | 210 | field water | Geckler et al, 1977 |
| Pimephales promelas | 750 | 6.5 | 8.0 |  | 244 | field water | Geckler et al, 1977 |
| Pimephales promelas | 820 | 17.5 | 7.9 |  | 228 | field water | Geckler et al, 1977 |
| Pimephales promelas | 940 | 21.5 | 7.9 |  | 150 | field water | Geckler et al, 1977 |
| Pimephales promelas | 1060 | 18 | 8.0 |  | 224 | field water | Geckler et al, 1977 |
| Pimephales promelas | 580 | 6.5 | 7.8 |  | 252 | field water | Geckler et al, 1977 |
| Pimephales promelas | 830 | 23 | 7.6 |  | 252 | field water | Geckler et al, 1977 |
| Pimephales promelas | 820 | 4.5 | 7.9 |  | 260 | field water | Geckler et al, 1977 |
| Pimephales promelas | 950 | 12.5 | 8.0 |  | 260 | field water | Geckler et al, 1977 |
| Pimephales promelas | 680 | 2.5 | 8.1 |  | 262 | field water | Geckler et al, 1977 |
| Pimephales promelas | 560 | 18.5 | 8.1 |  | 272 | field water | Geckler et al, 1977 |
| Pimephales promelas | 645 | 17 | 8.3 |  | 274 | field water | Geckler et al, 1977 |
| Pimephales promelas | 650 | 19.5 | 8.0 |  | 276 | field water | Geckler et al, 1977 |
| Pimephales promelas | 750 | 3 | 8.0 |  | 280 | field water | Geckler et al, 1977 |
| Pimephales promelas | 820 | 21 | 7.6 |  | 284 | field water | Geckler et al, 1977 |
| Pimephales promelas | 830 | 24 | 7.7 |  | 284 | field water | Geckler et al, 1977 |
| Pimephales promelas | 490 | 15 | 7.9 |  | 290 | field water | Geckler et al, 1977 |
| Pimephales promelas | 1000 | 24 | 8.0 |  | 303 | field water | Geckler et al, 1977 |
| Pimephales promelas | 750 | 7 | 7.8 |  | 308 | field water | Geckler et al, 1977 |
| Pimephales promelas | 810 | 22.5 | 7.9 |  | 310 | field water | Geckler et al, 1977 |
| Pimephales promelas | 760 | 14.5 | 7.6 |  | 312 | field water | Geckler et al, 1977 |
| Pimephales promelas | 540 | 24 | 8.0 |  | 314 | field water | Geckler et al, 1977 |
| Pimephales promelas | 865 | 19 | 7.2 |  | 316 | field water | Geckler et al, 1977 |
| Pimephales promelas | 650 | 15 | 7.2 |  | 318 | field water | Geckler et al, 1977 |
| Pimephales promelas | 920 | 8 | 8.0 |  | 322 | field water | Geckler et al, 1977 |
| Pimephales promelas | 136.5 | 22 | 7.6 |  | 44 | reconstituted water | Curtis et al, 1979 |
| Pimephales promelas | 12.4 | 22 | 7.0 |  | 24 | field/lab water | Erickson et al, 1996 |
| Pimephales promelas | 4.4 | 22 | 6.0 |  | 22.5 | field/lab water | Erickson et al, 1996 |
| Pimephales promelas | 21 | 22 | 7.2 |  | 45 | field water | Erickson et al, 1996 |
| Pimephales promelas | 19.7 | 22 | 7.4 |  | 45.5 | field water | Erickson et al, 1996 |
| Pimephales promelas | 18.2 | 22 | 7.2 |  | 46 | field water | Erickson et al, 1996 |
| Pimephales promelas | 5.9 | 22 | 6.53 |  | 49 | field water | Erickson et al, 1996 |
| Pimephales promelas | 79.7 | 22 | 7.3 |  | 85.1 | field/lab water | Erickson et al, 1996 |
| Pimephales promelas | 71.2 | 22 | 7.1 |  | 87.1 | field/lab water | Erickson et al, 1996 |
| Pimephales promelas | 663 | 22 | 7.2 |  | 251.2 | lab water | Erickson et al, 1996 |
| Pimephales promelas | 20.5 | 22 | 7.5 |  | 19 | field/lab water | Erickson et al, 1996 |
| Pimephales promelas | 26.9 | 22 | 8.0 |  | 23 | field/lab water | Erickson et al, 1996 |
| Pimephales promelas | 12.4 | 22 | 7.6 |  | 25 | field/lab water | Erickson et al, 1996 |
| Pimephales promelas | 17.2 | 22 | 7.9 |  | 45.5 | field water | Erickson et al, 1996 |
| Pimephales promelas | 20.3 | 22 | 7.6 |  | 27 | field/lab water | Erickson et al, 1996 |
| Pimephales promelas | 21.6 | 22 | 7.9 |  | 45.5 | field water | Erickson et al, 1996 |
| Pimephales promelas | 23.2 | 22 | 7.5 |  | 34 | field/lab water | Erickson et al, 1996 |
| Pimephales promelas | 26.7 | 22 | 7.5 |  | 27 | field/lab water | Erickson et al, 1996 |
| Pimephales promelas | 26.7 | 22 | 7.6 |  | 30 | field/lab water | Erickson et al, 1996 |
| Pimephales promelas | 28 | 22 | 7.6 |  | 29 | field/lab water | Erickson et al, 1996 |
| Pimephales promelas | 35 | 22 | 7.5 |  | 51 | field/lab water | Erickson et al, 1996 |
| Pimephales promelas | 36.9 | 22 | 8.2 |  | 47 | field water | Erickson et al, 1996 |
| Pimephales promelas | 41.1 | 22 | 8.0 |  | 44 | field water | Erickson et al, 1996 |
| Pimephales promelas | 42.1 | 22 | 7.7 |  | 27 | field/lab water | Erickson et al, 1996 |
| Pimephales promelas | 42.6 | 22 | 8.0 |  | 44.5 | field water | Erickson et al, 1996 |
| Pimephales promelas | 44.7 | 22 | 7.8 |  | 44 | field water | Erickson et al, 1996 |
| Pimephales promelas | 46.3 | 22 | 8.0 |  | 44.5 | field water | Erickson et al, 1996 |
| Pimephales promelas | 52.7 | 22 | 8.1 |  | 45 | field water | Erickson et al, 1996 |
| Pimephales promelas | 58.5 | 22 | 8.4 |  | 48 | field water | Erickson et al, 1996 |
| Pimephales promelas | 62.3 | 22 | 7.9 |  | 45.5 | field water | Erickson et al, 1996 |
| Pimephales promelas | 62.5 | 22 | 7.8 |  | 41 | field/lab water | Erickson et al, 1996 |
| Pimephales promelas | 68.6 | 22 | 8.0 |  | 44.5 | field water | Erickson et al, 1996 |
| Pimephales promelas | 70.1 | 22 | 7.9 |  | 50 | field/lab water | Erickson et al, 1996 |
| Pimephales promelas | 70.5 | 22 | 8.1 |  | 47 | field water | Erickson et al, 1996 |
| Pimephales promelas | 70.8 | 22 | 8.0 |  | 45 | field water | Erickson et al, 1996 |
| Pimephales promelas | 75.8 | 22 | 8.1 |  | 45 | field water | Erickson et al, 1996 |
| Pimephales promelas | 77 | 22 | 8.0 |  | 44 | field water | Erickson et al, 1996 |
| Pimephales promelas | 77.5 | 22 | 8.0 |  | 51 | field/lab water | Erickson et al, 1996 |
| Pimephales promelas | 78.9 | 22 | 7.8 |  | 37 | field water | Erickson et al, 1996 |
| Pimephales promelas | 81.6 | 22 | 8.0 |  | 51 | field/lab water | Erickson et al, 1996 |
| Pimephales promelas | 83.9 | 22 | 8.0 |  | 45.5 | field water | Erickson et al, 1996 |
| Pimephales promelas | 84.3 | 22 | 8.0 |  | 52 | field/lab water | Erickson et al, 1996 |
| Pimephales promelas | 92.9 | 22 | 8.2 |  | 43 | field water | Erickson et al, 1996 |
| Pimephales promelas | 94 | 22 | 8.0 |  | 44.5 | field water | Erickson et al, 1996 |
| Pimephales promelas | 96.7 | 22 | 8.1 |  | 60.1 | field/lab water | Erickson et al, 1996 |
| Pimephales promelas | 97.9 | 22 | 8.0 |  | 51 | field/lab water | Erickson et al, 1996 |
| Pimephales promelas | 99.6 | 22 | 7.6 |  | 88.1 | field/lab water | Erickson et al, 1996 |
| Pimephales promelas | 101.5 | 22 | 7.9 |  | 59.1 | field/lab water | Erickson et al, 1996 |
| Pimephales promelas | 103.8 | 22 | 7.8 |  | 45 | field water | Erickson et al, 1996 |
| Pimephales promelas | 110.9 | 22 | 8.0 |  | 53 | field/lab water | Erickson et al, 1996 |
| Pimephales promelas | 111.8 | 22 | 7.8 |  | 47 | field water | Erickson et al, 1996 |
| Pimephales promelas | 112 | 22 | 8.1 |  | 45 | field water | Erickson et al, 1996 |
| Pimephales promelas | 113.4 | 22 | 7.8 |  | 47 | field water | Erickson et al, 1996 |
| Pimephales promelas | 114.6 | 22 | 7.9 |  | 46 | field water | Erickson et al, 1996 |
| Pimephales promelas | 117.4 | 22 | 7.9 |  | 75.1 | field water | Erickson et al, 1996 |
| Pimephales promelas | 122.9 | 22 | 7.8 |  | 138.1 | field water | Erickson et al, 1996 |
| Pimephales promelas | 122.9 | 22 | 8.1 |  | 243.2 | field water | Erickson et al, 1996 |
| Pimephales promelas | 123.3 | 22 | 8.0 |  | 46.5 | field water | Erickson et al, 1996 |
| Pimephales promelas | 125.4 | 22 | 7.8 |  | 60.1 | field water | Erickson et al, 1996 |
| Pimephales promelas | 125.7 | 22 | 7.9 |  | 46 | field water | Erickson et al, 1996 |
| Pimephales promelas | 126.6 | 22 | 7.9 |  | 74.1 | field water | Erickson et al, 1996 |
| Pimephales promelas | 127.1 | 22 | 8.1 |  | 91.1 | field/lab water | Erickson et al, 1996 |
| Pimephales promelas | 129.9 | 22 | 7.8 |  | 45 | field water | Erickson et al, 1996 |
| Pimephales promelas | 131.1 | 22 | 7.8 |  | 45 | field water | Erickson et al, 1996 |
| Pimephales promelas | 132.8 | 22 | 7.7 |  | 87;1 | field/lab water | Erickson et al, 1996 |
| Pimephales promelas | 137.2 | 22 | 8.1 |  | 87.1 | field/lab water | Erickson et al, 1996 |
| Pimephales promelas | 143.0 | 22 | 8.0 |  | 45.5 | field water | Erickson et al, 1996 |
| Pimephales promelas | 148.7 | 22 | 8.2 |  | 90.1 | field/lab water | Erickson et al, 1996 |
| Pimephales promelas | 150.3 | 22 | 8.2 |  | 91.1 | field/lab water | Erickson et al, 1996 |
| Pimephales promelas | 151.0 | 22 | 7.8 |  | 46 | field water | Erickson et al, 1996 |
| Pimephales promelas | 152.0 | 22 | 8.0 |  | 53 | field/lab water | Erickson et al, 1996 |
| Pimephales promelas | 155.7 | 22 | 7.8 |  | 76.1 | field water | Erickson et al, 1996 |
| Pimephales promelas | 158.1 | 22 | 7.8 |  | 107.1 | field water | Erickson et al, 1996 |
| Pimephales promelas | 160.3 | 22 | 7.8 |  | 45 | field water | Erickson et al, 1996 |
| Pimephales promelas | 166.7 | 22 | 7.8 |  | 103.1 | field water | Erickson et al, 1996 |
| Pimephales promelas | 167.3 | 22 | 7.8 |  | 103.1 | field water | Erickson et al, 1996 |
| Pimephales promelas | 167.4 | 22 | 7.9 |  | 76.1 | field water | Erickson et al, 1996 |
| Pimephales promelas | 167.4 | 22 | 7.8 |  | 45 | field water | Erickson et al, 1996 |
| Pimephales promelas | 168.4 | 22 | 8.0 |  | 44.5 | field water | Erickson et al, 1996 |
| Pimephales promelas | 172.4 | 22 | 7.8 |  | 151.1 | field water | Erickson et al, 1996 |
| Pimephales promelas | 172.6 | 22 | 7.9 |  | 43 | field water | Erickson et al, 1996 |
| Pimephales promelas | 172.8 | 22 | 7.9 |  | 133.1 | field water | Erickson et al, 1996 |
| Pimephales promelas | 175.4 | 22 | 7.9 |  | 52 | field/lab water | Erickson et al, 1996 |
| Pimephales promelas | 176.9 | 22 | 7.8 |  | 46 | field water | Erickson et al, 1996 |
| Pimephales promelas | 181.0 | 22 | 7.8 |  | 45 | field water | Erickson et al, 1996 |
| Pimephales promelas | 182.4 | 22 | 8.4 |  | 87.1 | field/lab water | Erickson et al, 1996 |
| Pimephales promelas | 183.0 | 22 | 7.8 |  | 44 | field water | Erickson et al, 1996 |
| Pimephales promelas | 183.0 | 22 | 8.0 |  | 90.1 | field/lab water | Erickson et al, 1996 |
| Pimephales promelas | 183.0 | 22 | 8.1 |  | 120.1 | field/lab water | Erickson et al, 1996 |
| Pimephales promelas | 189.1 | 22 | 8.1 |  | 87.1 | field/lab water | Erickson et al, 1996 |
| Pimephales promelas | 190.6 | 22 | 7.8 |  | 45.5 | field water | Erickson et al, 1996 |
| Pimephales promelas | 190.8 | 22 | 8.1 |  | 180.2 | field/lab water | Erickson et al, 1996 |
| Pimephales promelas | 191.7 | 22 | 7.8 |  | 189.2 | field water | Erickson et al, 1996 |
| Pimephales promelas | 193.0 | 22 | 8.1 |  | 45.5 | field water | Erickson et al, 1996 |
| Pimephales promelas | 199.8 | 22 | 7.8 |  | 134.1 | field water | Erickson et al, 1996 |
| Pimephales promelas | 223.7 | 22 | 8.1 |  | 93.1 | field/lab water | Erickson et al, 1996 |
| Pimephales promelas | 226.8 | 22 | 7.8 |  | 134.1 | field water | Erickson et al, 1996 |
| Pimephales promelas | 229.9 | 22 | 8.3 |  | 47 | field water | Erickson et al, 1996 |
| Pimephales promelas | 230 | 22 | 8.1 |  | 45 | field water | Erickson et al, 1996 |
| Pimephales promelas | 242.6 | 22 | 7.8 |  | 139.1 | field water | Erickson et al, 1996 |
| Pimephales promelas | 253.5 | 22 | 8.1 |  | 92.1 | field/lab water | Erickson et al, 1996 |
| Pimephales promelas | 256.2 | 22 | 8.1 |  | 45.5 | field water | Erickson et al, 1996 |
| Pimephales promelas | 262.3 | 22 | 8.0 |  | 90.1 | field/lab water | Erickson et al, 1996 |
| Pimephales promelas | 268.4 | 22 | 8.1 |  | 138.1 | field water | Erickson et al, 1996 |
| Pimephales promelas | 271.4 | 22 | 8.1 |  | 107.1 | field/lab water | Erickson et al, 1996 |
| Pimephales promelas | 283.3 | 22 | 8.1 |  | 92.1 | field/lab water | Erickson et al, 1996 |
| Pimephales promelas | 289.1 | 22 | 8.1 |  | 43 | field water | Erickson et al, 1996 |
| Pimephales promelas | 289.1 | 22 | 7.8 |  | 45 | field water | Erickson et al, 1996 |
| Pimephales promelas | 292.3 | 22 | 8.0 |  | 88.1 | field/lab water | Erickson et al, 1996 |
| Pimephales promelas | 370.3 | 22 | 8.0 |  | 140.1 | field/lab water | Erickson et al, 1996 |
| Pimephales promelas | 405.1 | 22 | 8.0 |  | 140.1 | field/lab water | Erickson et al, 1996 |
| Pimephales promelas | 496.0 | 22 | 8.1 |  | 194.2 | field/lab water | Erickson et al, 1996 |
| Pimephales promelas | 521.0 | 22 | 8.1 |  | 235.2 | lab water | Erickson et al, 1996 |
| Pimephales promelas | 644.9 | 22 | 8.4 |  | 144.1 | field/lab water | Erickson et al, 1996 |
| Pimephales promelas | 646.8 | 22 | 8.3 |  | 218.2 | field/lab water | Erickson et al, 1996 |
| Pimephales promelas | 653.8 | 22 | 8.3 |  | 217.2 | field/lab water | Erickson et al, 1996 |
| Pimephales promelas | 758.1 | 22 | 8.1 |  | 200.2 | field/lab water | Erickson et al, 1996 |
| Pimephales promelas | 940.4 | 22 | 8.3 |  | 212.2 | field/lab water | Erickson et al, 1996 |
| Pimephales promelas | 953.0 | 22 | 8.4 |  | 226.2 | field/lab water | Erickson et al, 1996 |
| Pimephales promelas | 892.1 | 22 | 8.3 |  | 251.2 | lab water | Erickson et al, 1996 |
| Pimephales promelas | 905.4 | 22 | 7.6 |  | 252.2 | lab water | Erickson et al, 1996 |
| Pimephales promelas | 996.3 | 22 | 7.9 |  | 252.2 | lab water | Erickson et al, 1996 |
| Pimephales promelas | 38.4 | 22 | 8.1 |  | 255.7 | field water | Erickson et al, 1996 |
| Pimephales promelas | 698.0 | 22 | 8.3 |  | 292.3 | field/lab water | Erickson et al, 1996 |
| Pimephales promelas | 752.9 | 22 | 8.2 |  | 440.4 | field/lab water | Erickson et al, 1996 |
| Pimephales promelas | 297 | 12 | 8.2 |  | 164 | reconstituted water | Richards and Beitinger, 1995 |
| Pimephales promelas | 311 | 22 | 8.2 |  | 164 | reconstituted water | Richards and Beitinger, 1995 |
| Pimephales promelas | 450 | 5 | 8.2 |  | 164 | reconstituted water | Richards and Beitinger, 1995 |
| Pimephales promelas | 513 | 32 | 8.2 |  | 164 | reconstituted water | Richards and Beitinger, 1995 |
| Pimephales promelas | 232 | 25 | 7.5 |  | 55 | field water | Nelson et al, 1985 |
| Pimephales promelas | 363 | 25 | 7.5 |  | 68 | field water | Nelson et al, 1985 |
| Pimephales promelas | >449 | 25 | 7.3 |  | 82 | field water | Nelson et al, 1985 |
| Pimephales promelas | 427 | 25 | 7.3 |  | 90 | field water | Nelson et al, 1985 |
| Pimephales promelas | 52 | 25 | 7.7 |  | 52 | field water | Nelson et al, 1985 |
| Pimephales promelas | 171 | 25 | 7.4 |  | 36 | field water | Nelson et al, 1985 |
| Pimephales promelas | 210 | 22 | 7.4 |  | 103 | reconstituted water | Birge et al, 1983 |
| Pimephales promelas | 360 | 22 | 7.4 |  | 103 | reconstituted water | Birge et al, 1983 |
| Pimephales promelas | 390 | 22 | 7.4 |  | 262.5 | reconstituted water | Birge et al, 1983 |
| Pimephales promelas | 410 | 22 | 7.4 |  | 262.5 | reconstituted water | Birge et al, 1983 |
| Lepomis macrochirus | 4250 | 15 | 8.2 |  | 318 | field water | Geckler et al, 1976 |
| Lepomis macrochirus | 4300 | 19 | 8.2 |  | 316 | field water | Geckler et al, 1976 |
| Lepomis macrochirus | 9150 | 24 | 8.0 |  | 200 | lab water | Geckler et al, 1976 |
| Lepomis macrochirus | 1000 | 22 | 7.2 |  | 40.2 | tap water | Thompson et al, 1980 |
| Lepomis macrochirus | 1100 | 20 | 7.5 |  | 45 | field water | Benoit, 1975 |
| Lepomis macrochirus | 710 | 20 | 5.4 |  | 46 | reconstituted water | Trama, 1954 |
| Lepomis macrochirus | 770 | 20 | 5.4 |  | 46 | reconstituted water | Trama, 1954 |

Annex IX: Overview of Overview of the NOEC values (µg Cu/L) and physico-chemical parameters for freshwater fish.

Reported LC50values represent measured concentration levels, unless specified otherwise. FT= flow through, S= static, R= renewal; H= Hardness; d= days ; Cb= copper background concentration expressed as measured µg Cu/L

| Organism |  | Age/size of organisms | Exposure time | Endpoint | NOEC (µg/L) | Testtype | Cb (µg Cu/l) | Physico-chemical conditions | Medium | Reference |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ictalurus punctatus |  | fry | 60 d | growth | 13 | FT | 3 | T: 22°C; pH: 7.65; H: 186.3 mg/l CaCO3; DOC: 1.3 mg/l (9) | Well | Sauter et al., 1976 |  |
| Ictalurus punctatus |  | fry | 60 d | mortality | 13 | FT | 3 | T: 22°C; pH: 7.65; H: 186.3 mg/l CaCO3; DOC: 1.3 mg/L (9) | Well | Sauter et al., 1976 |  |
| Noemacheilus barbatulus |  | adult (8.7 - 12.1 cm) | 64 d | mortality | 120 | FT | 2 | T 11.9°C; pH: 8.26; H: 249 mg/L CaCO3; DOC: 1.3 mg/L (9) | Well | Solbe & Cooper, 1976 |  |
| Oncorhynchus kisutch |  | parr | 61 d | growth | 22 | FT | / | T: 9.5 °C; pH: 7.10; H: 24.4 mg/L CaCO3; DOC: 2.9 mg/L (11) | River (Chehalis River) | Mudge et al., 1993 |  |
| Oncorhynchus kisutch |  | fry | 60 d | growth | 21 | FT | / | T: 16.7 °C; pH: 7.30; H: 31.8 mg/L CaCO3; DOC: 2.9 mg/L (11) | River (Chehalis River) | Mudge et al., 1993 |  |
| Oncorhynchus kisutch |  | parr | 61 d | growth | 28 | FT | / | T: 8.7 °C; pH: 7.0; H: 28.7 mg/L CaCO3; DOC: 2.9 mg/L (11) | River (Chehalis River) | Mudge et al., 1993 |  |
| Oncorhynchus kisutch |  | parr | 61 d | mortality | 24 | FT | / | T: 9.5 °C; pH: 7.10; H: 24.4 mg/L CaCO3; DOC: 2.9 mg/L (11) | River (Chehalis River) | Mudge et al., 1993 |  |
| Oncorhynchus kisutch |  | fry | 60 d | mortality | 18 | FT | / | T: 16.7 °C; pH: 7.30; H: 31.8 mg/L CaCO3; DOC: 2.9 mg/L (11) | River (Chehalis River) | Mudge et al., 1993 |  |
| Oncorhynchus mykiss |  | fry (0.12 g; 2.6 cm) | 60 d | growth | 2.2 | FT | 0.45* | T: 9.8 °C; pH: 7.5; H: 25 mg/L CaCO3; DOC: 0.2 mg/L (12) | Well + deionised water | Marr et al., 1996 |  |
| Oncorhynchus mykiss |  | parr | 61 d | growth | 45 | FT | / | T: 9.5 °C; pH: 7.10; H: 24.4 mg/L CaCO3; DOC: 2.9 mg/L (11) | River (Chehalis River) | Mudge et al., 1993 |  |
| Oncorhynchus mykiss |  | eggs | 63 d | growth | 16 | FT | 3 | T: 12 °C; pH: 7.65; H: 120 mg/L CaCO3; DOC: 1.3 mg/L (9) | Well | Seim et al., 1984 |  |
| Oncorhynchus mykiss |  | parr | 61 d | mortality | 24 | FT | / | T: 9.5 °C; pH: 7.10; H: 24.4 mg/L CaCO3; DOC: 2.9 mg/L (11) | River (Chehalis River) | Mudge et al., 1993 |  |
| Oncorhynchus mykiss |  | parr | 61 d | mortality | 28 | FT | / | T: 8.7 °C; pH: 7.0; H: 28.7 mg/L CaCO3; DOC: 2.9 mg/L (11) | River (Chehalis River) | Mudge et al., 1993 |  |
| Oncorhynchus mykiss |  | embryo | 30 d | Growth | 11.4 | FT | 3 | T: 10.8 °C; pH: 7.6; H: 45.4 mg/L CaCO3; DOC: 1.0 mg/L (13) | Lake (Lake Superior) | McKim et al., 1978 |  |
| Oncorhynchus mykiss |  | embryo | 30 d | mortality | 11.4 | FT | 3 | T: 10.8 °C; pH: 7.6; H: 45.4 mg/L CaCO3; DOC: 1.0 mg/L (13) | Lake (Lake Superior) | McKim et al., 1978 |  |
| Catostomus commersoni |  | embryo | 30 d | Growth | 12.9 | FT | 3 | T: 14.9 °C; pH: 7.6; H: 45.4 mg/L CaCO3; DOC: 1.0 mg/L (13) | Lake (Lake Superior) | McKim et al., 1978 |  |
| Catostomus commersoni |  | embryo | 30 d | mortality | 12.9 | FT | 3 | T: 14.9 °C; pH: 7.6; H: 45.4 mg/L CaCO3; DOC: 1.0 mg/L (13) | Lake (Lake Superior) | McKim et al., 1978 |  |
| Esox lucius |  | embryo | 30 d | Growth | 34.9 | FT | 3 | T: 15.6 °C; pH: 7.6; H: 45.4 mg/L CaCO3; DOC: 1.0 mg/L (13) | Lake (Lake Superior) | McKim et al., 1978 |  |
| Esox lucius |  | embryo | 30 d | mortality | 34.9 | FT | 3 | T: 15.6 °C; pH: 7.6; H: 45.4 mg/L CaCO3; DOC: 1.0 mg/L (13) | Lake (Lake Superior) | McKim et al., 1978 |  |
| Perca fluviatilis |  | juvenile (3.8 - 4.3 g) | 30 d | growth | 39 | FT | 1 | T: 17.5 °C; pH: 7.8; H: 194 mg/L CaCO3; DOC: 1 mg/L (7) | Tap | Collvin, 1985 |  |
| Perca fluviatilis |  | juvenile (3.8 g) | 30 d | mortality | 188 | FT | 3 | T: 15.1 °C; pH: 7.8; H: 178 mg/L CaCO3; DOC: 1mg/L (7) | Tap | Collvin, 1984 |  |
| Pimephales notatus |  | fry (15 - 16 mm) -second generation | 30 d | growth | 44 | FT | 4.3 | T: 25 °C; pH: 8.1; H: 201 mg/L CaCO3; DOC: 0.55 mg/L (14) | Spring + demineralised tap | Horning & Neiheisel, 1979 |  |
| Pimephales notatus |  | fry (15 - 16 mm) | 60 d | growth | 71.8 | FT | 4.3 | T: 25 °C; pH: 8.1; H: 201 mg/L CaCO3; DOC: 0.55 mg/L (14) | Spring + demineralised tap | Horning & Neiheisel, 1979 |  |
| Pimephales notatus |  | fry (15 - 16 mm) | 60 d | mortality | 71.8 | FT | 4.3 | T: 25 °C; pH: 8.1; H: 201 mg/L CaCO3; DOC: 0.55 mg/L (14) | Spring + demineralised tap | Horning & Neiheisel, 1979 |  |
| Pimephales promelas |  | fry (10 - 15 mm) | 330 d | growth | 33 | FT | 3.5 | T: 21°C; pH: 8.0; H: 198 mg/L CaCO3; DOC: 0.55 mg/L (14) | Spring+ deionised tap | Mount, 1968 |  |
| Pimephales promelas |  | fry (10 - 20 mm) | 327 d | growth | 10.6 | FT | 4.4 | T: 22°C; pH: 6.9; H: 31.4 mg/L CaCO3; DOC: 0.55 mg/L (14) | Spring+ deionised tap | Mount & Stephan, 1969 |  |
| Pimephales promelas |  | larvae (4 weeks old) | 187 d | growth | 59.5 | FT | 4.2 | T: 23°C; pH: 7.85; H: 202 mg/L CaCO3; DOC: 0.55 mg/L (14) | Spring+ demineralised tap | Pickering et al., 1977 |  |
| Pimephales promelas |  | embryo-larval | 32 d | growth | 4.8 | FT | 1.25* | T: 25°C; pH: 7.05; H: 43.9 mg/L CaCO3; DOC: 1 mg/L (13) | Lake (Lake Superior) | Spehar & Fiandt, 1985 |  |
| Pimephales promelas |  | fry (10 - 15 mm) | 330 d | mortality | 33 | FT | 3.5 | T: 21°C; pH: 8.0; H: 198 mg/L CaCO3; DOC: 0.55 mg/L (14) | Spring+ deionised tap | Mount, 1968 |  |
| Pimephales promelas |  | fry (10 - 20 mm) | 327 d | mortality | 10.6 | FT | 4.4 | T: 22°C; pH: 6.9; H: 31.4 mg/L CaCO3; DOC: 0.55 mg/L (14) | Spring+ deionised tap | Mount & Stephan, 1969 |  |
| Pimephales promelas |  | larvae | 28 d | mortality | 61 | FT | 0.6 | T: 21°C; pH: 8.17; H: 202 mg/L CaCO3; DOC: 1.3 mg/L (9) | Ground water | Scudder et al., 1988 |  |
| Pimephales promelas |  | embryo-larval | 32 d | mortality | 4.8 | FT | 1.25* | T: 25°C; pH: 7.05; H: 4 mg/L CaCO3; DOC: 1 mg/L (13) | Lake (Lake Superior) | Spehar & Fiandt, 1985 |  |
| Pimephales promelas |  | juvenile (32 - 38 mm; 5 months old) | 270 d | reproduction | 66 | FT | 7 | T: 23°C; pH: 8.1; H: 274 mg/L CaCO3; DOC: 2 mg/L (3) | River | Brungs et al., 1976 |  |
| Pimephales promelas |  | fry (10 - 15 mm) | 330 d | reproduction | 14.5 | FT | 3.5 | T: 21°C; pH: 8.0; H: 198 mg/L CaCO3; DOC: 0.55 mg/L (14) | Spring+ deionised tap | Mount, 1968 |  |
| Pimephales promelas |  | fry (10 - 20 mm) | 327 d | reproduction | 10.6 | FT | 4.4 | T: 22°C; pH: 6.9; H: 31.4 mg/L CaCO3; DOC: 0.55 mg/L (14) | Spring+ deionised tap | Mount & Stephan, 1969 |  |
| Pimephales promelas |  | larvae (4 weeks old) | 187 d | reproduction | 25.5 | FT | 4.2 | T: 23°C; pH: 7.85; H: 202 mg/L CaCO3; DOC: 0.55 mg/L (14) | Spring+ deionised tap | Pickering et al., 1977 |  |
| Pimephales promelas |  | larvae (4 weeks old) | 97 d | reproduction | 23 | FT | 4.2 | T: 23°C; pH: 7.85; H: 202 mg/L CaCO3; DOC: 0.55 mg/L (14) | Spring+ deionised tap | Pickering et al., 1977 |  |
| Pimephales promelas |  | larvae (4 weeks old) | 7 d | reproduction | 22.5 | FT | 4.2 | T: 23°C; pH: 7.85; H: 202 mg/L CaCO3; DOC: 0.55 mg/L (14) | Spring+ deionised tap | Pickering et al., 1977 |  |
| Salvelinus fontinalis |  | embryo | 30-60 d | Growth | 22.3 | FT | / | T: 5.6 °C; pH: 7.6; H: 45.4 mg/L CaCO3; DOC: 1.0 mg/L (13) | Lake (Lake Superior) | McKim et al., 1978 |  |
| Salvelinus fontinalis |  | embryo | 30-60 d | mortality | 22.3 | FT | / | T: 5.6 °C; pH: 7.6; H: 45.4 mg/L CaCO3; DOC: 1.0 mg/L (13) | Lake (Lake Superior) | McKim et al., 1978 |  |
| Salvelinus fontinalis |  | Alevins/juveniles | 189 d | Growth | 9.5 | FT | / | T: 10.6 °C; pH: 7.45; H: 45.4 mg/L CaCO3; DOC: 1 mg/L (7) | Tap | McKim & Benoit, 1971 |  |
| Salvelinus fontinalis |  | Alevins/juveniles | 189 d | mortality | 9.5 | FT | / | T: 10.6 °C; pH: 7.5; H: 45.4 mg/L CaCO3; DOC: 1 mg/L (7) | Tap | McKim & Benoit, 1971 |  |
| Salvelinus fontinalis |  | yearling | 244 d | growth | 17.4 | FT | / | T: 10.6 °C; pH: 7.45; H: 45.4 mg/L CaCO3; DOC: 1 mg/L (7) | Tap | McKim & Benoit, 1971 |  |
| Salvelinus fontinalis |  | fry | 30 d | Growth | 7 | FT | 3 | T: 10 °C; pH: 6.85; H: 37.5 mg/L CaCO3; DOC: 1.3 mg/L (9) | Well | Sauter et al., 1976 |  |
| Salvelinus fontinalis |  | fry | 30 d | growth | 21 | FT | 3 | T: 10 °C; pH:6.9; H: 187 mg/L CaCO3; DOC: 1.3 mg/L (9) | Well | Sauter et al., 1976 |  |
| Salvelinus fontinalis |  | yearling | 244 d | mortality | 17.4 | S | 1.9 | T: 10.6 °C; pH: 7.45; H: 45.4 mg/L CaCO3; DOC: 1 mg/L (7) | Tap | McKim & Benoit, 1971 |  |
| Salvelinus fontinalis |  | fry | 60 d | mortality | 13 | FT | 3 | T: 10 °C; pH: 6.85; H: 37.5 mg/L CaCO3; DOC: 1.3 mg/L (9) | Well | Sauter et al., 1976 |  |
| Salvelinus fontinalis |  | fry | 30 d | mortality | 21 | FT | 3 | T: 10 °C; pH:6.9; H: 187 mg/L CaCO3; DOC: 1.3 mg/L (9) | Well | Sauter et al., 1976 |  |
| Salvelinus fontinalis |  | yearling | 244 d | reproduction | 17.4 | FT | 1.9 | T: 10.6 °C; pH: 7.45; H: 45.4 mg/L CaCO3; DOC: 1 mg/L (7) | Tap | McKim & Benoit, 1971 |  |
| Salvelinus fontinalis |  | fry | 60 d | reproduction | 7 | FT | 3 | T: 10 °C; pH: 6.85; H: 37.5 mg/L CaCO3; DOC: 1.3 mg/L (9) | Well | Sauter et al., 1976 |  |
| Salvelinus fontinalis |  | fry | 30 d | reproduction | 49 | FT | 3 | T: 10 °C; pH:6.9; H: 187 mg/L CaCO3; DOC: 1.3 mg/L (9) | Well | Sauter et al., 1976 |  |
| Footnote Cb : * = estimated. (1): DOC estimation of reconstituted water is 0.5 mg/L (De Schamphelaere and Janssen, 2002 (0.3 mg DOC/L); Ryan eta al., 2004 (0.4-0.5 mg DOC/L); Karman et al., 2004 (<0.1 mg DOC/L); Hollis et al, 1997 (0.4-0.6 mg DOC/L).. (2): DOC estimation for New River (USA) water extracted from the United States Geological Survey records (USGS). The USGS database reports TOC concentration of 3.65 mg/L, and assuming a DOC/TOC ratio of 0.8.. (3): DOC estimation for unknown river/lake water or for which no reliable DOC concentration could be estimated is 2.0 mg/L (Santore et al., 2002). (4): DOC estimation for Clinch River (USA) water extracted from the United States Geological Survey records (USGS). The USGS database reports TOC concentration of 3.7 mg/L, and assuming a DOC/TOC ratio of 0.8.. (5): DOC estimation for Lester River (USA) water extracted from the United States Geological Survey records (USGS). The USGS database reports TOC concentration of 7.1 mg/L, and assuming a DOC/TOC ratio of 0.8.. (6): DOC estimation for deionized water (= 0.1 mg/L according to Santore et al., 2002) with addition of artificial humic acids (no addition; 0.15 mg/L; 0.75 mg/L; 1.5 mg/L). Conversion from humic acid content to organic carbon content was performed after using a factor of 2.. (7): DOC estimation for tap water is 1.0 mg/L (Santore et al., 2002). (8): DOC estimation for spring water is 1.0 mg/L (Santore et al., 2002). (9): DOC estimation for well water is 1.3 mg/L (Santore et al., 2002). (10): DOC level of Markermeer (origin of the test water) was used as a basis for the DOC estimation; the Markermeer water was however filtered extensively over a sand bed to reduce the TOC (pers. communication)) and the resulting DOC value is therefore < 7.3 mg/L.. (11): DOC estimation for Chehalis River (USA) water extracted from the United States Geological Survey records (USGS). The USGS database reports TOC concentration of 3.6 mg/L, and assuming a DOC/TOC ratio of 0.8.. (12): DOC estimation for ultrapure deionized water (0.1 mg/L Santore et al., 2002) and well water (1.3 mg/L according to Santore et al., 2002) in a ratio of 90%/10% is 0.45 mg/L.. (13): DOC estimation for Lake Superior water is 1.0 mg/L (Santore et al., 2002). |

Annex X: Overview of Overview of the NOEC values (µg Cu/L) and physico-chemical parameters for freshwater invertebrates.

Reported LC50values represent measured concentration levels, unless specified otherwise.; H= Hardness; d= days ; Cb= copper background concentration expressed as measured µg Cu/L.

Footnotes see Annex IX

| OrganismI | Age/size of organisms | Exposure time | Endpoint | NOEC (µg/L) | Test type | Cb (µg Cu/l) | Physico-chemical conditions | Medium | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ceriodaphnia dubia | neonates (< 24 h) | 7 d | reproduction | 10 | R | 0.5* | T: 23°C; pH: 7.6; H: 85 mg/L CaCO3; DOC: 0.5 mg/L (1) | Reconstitued | Cerda & Olive, 1993 |
| Ceriodaphnia dubia | neonates (< 24 h) | 7 d | mortality | 20 | R | 0.5* | T: 23°C; pH: 7.6; H: 85 mg/L CaCO3; DOC: 0.5 mg/L (1) | Reconstitued | Cerda & Olive, 1993 |
| Ceriodaphnia dubia | neonates (< 24 h) | 7 d | reproduction | 20 | S | 1.5* | T: 25°C; pH: 6.0; H: 182 mg/L CaCO3; DOC: 3.70 mg/L (4) | River (Clinch River) | Belanger & Cherry, 1990 |
| Ceriodaphnia dubia | neonates (< 8 h) | 7 d | mortality | 19 | S | / | T: 25°C; pH: 7.0; H: 22 mg/L CaCO3; DOC: 3.20 mg/L (3) | River | Jop et al., 1995 |
| Ceriodaphnia dubia | neonates (< 8 h) | 7 d | mortality | 4 | S | / | T: 25°C; pH: 6.95; H: 20 mg/L CaCO3; DOC: 0.5 mg/L (1) | Reconstituted | Jop et al., 1995 |
| Ceriodaphnia dubia | neonates (< 24 h) | 7 d | mortality | 122 | R | 3.4 | T: 25°C; pH: 8.25; H: 100 mg/L CaCO3; DOC: 5.7 mg/L (5) | River (Lester River) | Spehar & Fiandt, 1985 |
| Ceriodaphnia dubia | neonates (2-8 h) | 7 d | reproduction | 6.3 | S | 1.5 | T: 25°C; pH: 8.15; H: 94.1 mg/L CaCO3; DOC: 3.70 mg/L (2) | River (New River) | Belanger et al., 1989 |
| Ceriodaphnia dubia | neonates (2-8 h) | 7 d | reproduction | 24.1 | S | 4.7 | T: 25°C; pH: 8.31; H: 179 mg/L CaCO3; DOC: 3.70 mg/L (4) | River (Clinch River) | Belanger et al., 1989 |
| Ceriodaphnia dubia | neonates (< 8 h) | 7 d | reproduction | 4 | S | / | T: 25°C; pH: 6.95; H: 20 mg/L CaCO3; DOC: 0.5 mg/L (1) | Reconstituted | Jop et al., 1995 |
| Ceriodaphnia dubia | neonates (< 8 h) | 7 d | reproduction | 10 | S | / | T: 25°C; pH: 7.00; H: 22 mg/L CaCO3; DOC: 3.20 mg/L (3) | River | Jop et al., 1995 |
| Ceriodaphnia dubia | neonates (< 24 h) | 7 d | reproduction | 31.6 | S | 3.4 | T: 25°C; pH: 8.25; H: 100 mg/L CaCO3; DOC: 5.7 mg/L (5) | River (Lester River) | Spehar & Fiandt, 1985 |
| Daphnia magna | neonates | 21 d | growth | 12.6 | R | 2.6 | T: 20°C; pH: 8.1; H: 225 mg/L CaCO3; DOC: 2.0 mg/L (3) | Lake (Lake Ijssel) | Van Leeuwen et al., 1988 |
| Daphnia magna | neonates | 21 d | Survival, population growth | 36.8 | FT | 2.6 | T: 20°C; pH: 8.1; H: 225 mg/L CaCO3; DOC: 2.0 mg/L (3) | Lake (Lake Ijssel) | Van Leeuwen et al., 1988 |
| Daphnia magna | neonates | 21 d | reproduction | 28 | R | 0.5* | T: 20°C; pH: 6.31; H: 10 mg/L CaCO3; DOC: 2.72 mg/L | Lake | Heijerick et al., 2002 |
| Daphnia magna | neonates | 21 d | reproduction | 21.5 | R | 0.5* | T: 20°C; pH: 6.1; H: 12.4 mg/L CaCO3; DOC: 2.34 mg/L | Lake | Heijerick et al., 2002 |
| Daphnia magna | neonates | 21 d | reproduction | 71.4 | R | 0.5* | T: 20°C; pH: 8.3; H: 238 mg/L CaCO3; DOC: 8.24 mg/L | Lake | Heijerick et al., 2002 |
| Daphnia magna | neonates | 21 d | reproduction | 68.8 | R | 0.5* | T: 20°C; pH: 8.06; H: 191 mg/L CaCO3; DOC: 1.99 mg/L | River | Heijerick et al., 2002 |
| Daphnia magna | neonates | 21 d | reproduction | 106 | R | 0.5* | T: 20°C; pH: 7.55; H: 132 mg/L CaCO3; DOC: 6.13 mg/L | River | Heijerick et al., 2002 |
| Daphnia magna | neonates | 21 d | reproduction | 181 | R | 0.5* | T: 20°C; pH: 7.5; H: 134 mg/L CaCO3; DOC: 20.4 mg/L | Lake | Heijerick et al., 2002 |
| Daphnia pulex | neonates (< 24 h) | 42 d | mortality | 4 | R | 0.5* | T: 20°C; pH: 8.6; H: 57.5 mg/L CaCO3; DOC: 0.1 mg/L (6) | Deionized reconstituted | Winner, 1985 |
| Daphnia pulex | neonates (< 24 h) | 42 d | mortality | 20 | R | 0.5* | T: 20°C; pH: 8.5; H: 57.5 mg/L CaCO3; DOC: 0.475 mg/L (6) | Deionized reconstituted + DOC | Winner, 1985 |
| Daphnia pulex | neonates (< 24 h) | 42 d | mortality | 30 | R | 0.5* | T: 20°C; pH: 8.7; H: 57.5 mg/L CaCO3; DOC: 0.85 mg/L (6) | Deionized reconstituted + DOC | Winner, 1985 |
| Daphnia pulex | neonates (< 24 h) | 42 d | mortality | 5 | R | 0.5* | T: 20°C; pH: 8.7; H:115 mg/L CaCO3; DOC: 0.1 mg/L (6) | Deionized reconstituted | Winner, 1985 |
| Daphnia pulex | neonates (< 24 h) | 42 d | mortality | 20 | R | 0.5* | T: 20°C; pH: 8.55; H: 115 mg/L CaCO3; DOC: 0.475 mg/L (6) | Deionized reconstituted + DOC | Winner, 1985 |
| Daphnia pulex | neonates (< 24 h) | 42 d | mortality | 40 | R | 0.5* | T: 20°C; pH: 8.55; H:115 mg/L CaCO3; DOC: 0.85 mg/L (6) | Deionized reconstituted + DOC | Winner, 1985 |
| Daphnia pulex | neonates (< 24 h) | 42 d | mortality | 10 | R | 0.5* | T: 20°C; pH: 8.55; H: 230 mg/L CaCO3; DOC: 0.175 mg/L (6) | Deionized reconstituted + DOC | Winner, 1985 |
| Daphnia pulex | neonates (< 24 h) | 42 d | mortality | 15 | R | 0.5* | T: 20°C; pH: 8.6; H: 230 mg/L CaCO3; DOC: 0.475 mg/L (6) | Deionized reconstituted + DOC | Winner, 1985 |
| Daphnia pulex | neonates (< 24 h) | 42 d | mortality | 20 | R | 0.5* | T: 20°C; pH: 8.6; H: 230 mg/L CaCO3; DOC: 0.85 mg/L (6) | Deionized reconstituted + DOC | Winner, 1985 |
| Brachionus calyciflorus | neonates (< 2 h) | 2 d | reproduction | 8.2 | S | 0.3 | T: 25°C; pH: 6.0; H: 100 mg/L CaCO3; DOC: 4.91 mg/L | Reconstituted | De Schamphelaere et al., 2006 (13) |
| Brachionus calyciflorus | neonates (< 2 h) | 2 d | reproduction | 31.2 | S | 0.3 | T: 25°C; pH: 6.0; H: 100 mg/L CaCO3; DOC: 14.5 mg/L | Reconstituted | De Schamphelaere et al., 2006 (13) |
| Brachionus calyciflorus | neonates (< 2 h) | 2 d | reproduction | 47.8 | S | 0.3 | T: 25°C; pH: 7.8; H: 100 mg/L CaCO3; DOC: 4.83 mg/L | Reconstituted | De Schamphelaere et al., 2006 (13) |
| Brachionus calyciflorus | neonates (< 2 h) | 2 d | reproduction | 103 | S | 0.3 | T: 25°C; pH: 7.8; H: 100 mg/L CaCO3; DOC: 14.7 mg/L | Reconstituted | De Schamphelaere et al., 2006 (13) |
| Gammarus pulex | mixed sizes (1.5-14 mm) | 100 d | population response | 11 | FT | 2.6 | T: 11°C; pH: ; H: 103 mg/L CaCO3; DOC: 1 mg/L (7) | Tap | Maund et al., 1992 |
| Hyalella azteca | 2 - 3 weeks old | 10 d | mortality | 50 | S | / | T: 20°C; pH: 7.65; H: 36 mg/L CaCO3; DOC: 1 mg/L (8) | Spring | Deaver & Rodgers, 1996 |
| Hyalella azteca | 2 - 3 weeks old | 10 d | mortality | 50 | S | / | T: 20°C; pH: 7.8; H: 50 mg/L CaCO3; DOC: 1 mg/L (8) | Spring | Deaver & Rodgers, 1996 |
| Hyalella azteca | 2 - 3 weeks old | 10 d | mortality | 82 | S | / | T: 20°C; pH: 8.05; H: 64 mg/L CaCO3; DOC: 1 mg/L (8) | Spring | Deaver & Rodgers, 1996 |
| Hyalella azteca | 2 - 3 weeks old | 10 d | mortality | 82 | S | / | T: 20°C; pH: 7.5; H: 22 mg/L CaCO3; DOC: 1 mg/L (8) | Spring | Deaver & Rodgers, 1996 |
| Hyalella azteca | 2 - 3 weeks old | 10 d | mortality | 30 | S | / | T: 20°C; pH: 6.95; H: 5 mg/L CaCO3; DOC: 1 mg/L (8) | Spring | Deaver & Rodgers, 1996 |
| Hyalella azteca | <7 days old | 35 d | mortality | 32 | R | 3 | T: 22°C; pH: 7.6; H: 127.7 mg/L CaCO3; DOC: 1 mg/L (7) | Tap | Othman & Pascoe, 2002 |
| Chironomus riparius | eggs (< 12 h) | 10 d | growth | 16.9 | R | 0.5* | T: 20°C; pH: 7.0; H: 151.2 mg/L CaCO3; DOC: 0.5 mg/L (1) | Reconstituted | Taylor et al., 1991 |
| Clistoronia magnifica | larvae 1st generation | 240 d | Life cycle | 8.3 | FT | / | T: 15°C; pH: 7.3; H: 26 mg/L CaCO3; DOC: 1.3 mg/L (9) | Well | Nebeker et al., 1984 |
| Clistoronia magnifica | larvae- 2nd generation | 240 d | Life cycle | 13 | FT | / | T: 15°C; pH: 7.3; H: 26 mg/L CaCO3; DOC: 1.3 mg/L (9) | Well | Nebeker et al., 1984 |
| Paratanytarsus parthenogeneticus | larvae (7 days old) | 16 d | growth | 40 | / | 0.5* | T: 23°C; pH: 7.0; H: 25 mg/L CaCO3; DOC: 0.5 mg/L (1) | Reconstituted | Hatakeyama & Yasuno, 1981 |
| Paratanytarsus parthenogeneticus | larvae (7 days old) | 16 d | reproduction | 40 | / | 0.5* | T: 23°C; pH: 7.0; H: 25 mg/L CaCO3; DOC: 0.5 mg/L (1) | Reconstituted | Hatakeyama & Yasuno, 1981 |
| Dreissenia polymorpha | 18-22 mm | 63-77 d | Filtration rate | 16 | S | 3 | T: 15°C; pH: 7.9; H: 150 mg/L CaCO3; DOC: 7.10 mg/L (10) | Lake (Lake Markermeer) | Kraak et al., 1994 |
| Dreissenia polymorpha | 18-22 mm | 27 d | Filtration rate | 21 | R | / | T: 13.4°C; pH: 7.8; H: 296 mg/L CaCO3; DOC: 1.0 mg/L (7) | Tap | Mersch et al., 1993 |
| Villosa iris | glochidia | 30 d | mortality | 19.1 | FT | 3.2 | T: 20.8°C; pH: 8.39; H: 152 mg/L CaCO3; DOC: 3.7 mg/L (4) | River (Clinch River) | Jacobson et al., 1997 |
| Campeloma decisum | 11 to 27 mm snail | 42 d | mortality | 8 | FT | 1.9 | T: 15°C; pH: 8.15; H: 45 mg/L CaCO3; DOC: 1 mg/L (7) | Tap | Arthur & Leonard, 1970 |
| Campeloma decisum | 11 to 27 mm snail | 42 d | mortality | 8 | FT | 1.9 | T: 15°C; pH: 8.15; H: 45 mg/L CaCO3; DOC: 1 mg/L (7) | Tap | Arthur & Leonard, 1970 |
| Juga plicifera | mature | 30 d | mortality | 6 | FT | 0.5* | T: 15°C; pH: 7.1; H: 23 mg/L CaCO3; DOC: 1.3 mg/L (9) | Well | Nebeker et al., 1986 |

Annex XI: Overview NOECs for freshwater algae.

NOEC values are expressed as measured µg Cu/L  H= Hardness; d= days ; Cb= copper background concentration expressed as measured µg Cu/L

Footnotes see Annex IX

| Organism | Age/size of organisms | Exposure time | Endpoint | NOEC (µg.L) | Testtype | Cb (µg Cu/l) | Physico-chemical conditions | Medium | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Chlamydomonas reinhardtii | Inoculum: 1,000 c/ml | 10 d | growth | 22 | FT | 0.5* | T: 24°C; pH: 6.20; H: 25 mg/L CaCO3; DOC: 0.5 mg/L (1) | Reconstituted | Schäfer et al., 1994 |
| Chlamydomonas reinhardtii | Inoculum: 10,000 c/ml | 3 d | growth | 178 | S | 0.5* | T: 20°C; pH: 6.02; H: 250 mg/L CaCO3; DOC: 9.84 mg/L | Reconstituted | De Schamphelaere et al., 2006 |
| Chlamydomonas reinhardtii | Inoculum: 10,000 c/ml | 3 d | growth | 108 | S | 0.5* | T: 20°C; pH: 7.03; H: 250 mg/L CaCO3; DOC: 9.84 mg/L | Reconstituted | De Schamphelaere et al., 2006 |
| Chlamydomonas reinhardtii | Inoculum: 10,000 c/ml | 3 d | growth | 96 | S | 0.5* | T: 20°C; pH: 8.11; H: 250 mg/L CaCO3; DOC: 9.84 mg/L | Reconstituted | De Schamphelaere et al., 2006 |
| Chlorella vulgaris | Inoculum: 10,000 c/ml | 3 d | growth | 108.3 | S | 0.5* | T: 20°C; pH: 6.03; H: 100 mg/L CaCO3; DOC: 5.17 mg/L | Reconstituted | De Schamphelaere et al., 2006 |
| Chlorella vulgaris | Inoculum: 10,000 c/ml | 3 d | growth | 407.4 | S | 0.5* | T: 20°C; pH: 6.04; H: 100 mg/L CaCO3; DOC: 15.5 mg/L | Reconstituted | De Schamphelaere et al., 2006 |
| Chlorella vulgaris | Inoculum: 10,000 c/ml | 3 d | growth | 55.6 | S | 0.5* | T: 20°C; pH: 7.92; H: 400 mg/L CaCO3; DOC: 5.0 mg/L | Reconstituted | De Schamphelaere et al., 2006 |
| Chlorella vulgaris | Inoculum: 10,000 c/ml | 3 d | growth | 36.4 | S | 0.5* | T: 20°C; pH: 7.04; H: 250 mg/L CaCO3; DOC: 1.5 mg/L | Reconstituted | De Schamphelaere et al., 2006 |
| Chlorella vulgaris | Inoculum: 10,000 c/ml | 3 d | growth | 172.9 | S | 0.5* | T: 20°C; pH: 7.97; H: 400 mg/L CaCO3; DOC: 15.8 mg/L | Reconstituted | De Schamphelaere et al., 2006 |
| Chlorella vulgaris | Inoculum: 10,000 c/ml | 3 d | growth | 98.9 | S | 0.5* | T: 20°C; pH: 7.03; H: 250 mg/L CaCO3; DOC: 10.8 mg/L | Reconstituted | De Schamphelaere et al., 2006 |
| Chlorella vulgaris | Inoculum: 10,000 c/ml | 3 d | growth | 85.4 | S | 0.5* | T: 20°C; pH: 7.01; H: 500 mg/L CaCO3; DOC: 10.0 mg/L | Reconstituted | De Schamphelaere et al., 2006 |
| Chlorella vulgaris | Inoculum: 10,000 c/ml | 3 d | growth | 161.9 | S | 0.5* | T: 20°C; pH: 8.75; H: 250 mg/L CaCO3; DOC: 9.9 mg/L | Reconstituted | De Schamphelaere et al., 2006 |
| Chlorella vulgaris | Inoculum: 10,000 c/ml | 3 d | growth | 282.9 | S | 0.5* | T: 20°C; pH: 7.05; H: 250 mg/L CaCO3; DOC: 19.10 mg/L | Reconstituted | De Schamphelaere et al., 2006 |
| Chlorella vulgaris | Inoculum: 10,000 c/ml | 3 d | growth | 187.8 | S | 0.5* | T: 20°C; pH: 6.01; H: 400 mg/L CaCO3; DOC: 5.0 mg/L | Reconstituted | De Schamphelaere et al., 2006 |
| Chlorella vulgaris | Inoculum: 10,000 c/ml | 3 d | growth | 510.2 | S | 0.5* | T: 20°C; pH: 6.05; H: 400 mg/L CaCO3; DOC: 15.2 mg/L | Reconstituted | De Schamphelaere et al., 2006 |
| Chlorella vulgaris | Inoculum: 10,000 c/ml | 3 d | growth | 31 | S | 0.5* | T: 20°C; pH: 7.88; H: 100 mg/L CaCO3; DOC: 5.3 mg/L | Reconstituted | De Schamphelaere et al., 2006 |
| Chlorella vulgaris | Inoculum: 10,000 c/ml | 3 d | growth | 188 | S | 0.5* | T: 20°C; pH: 7.88; H: 100 mg/L CaCO3; DOC: 15.7 mg/L | Reconstituted | De Schamphelaere et al., 2006 |
| Chlorella vulgaris | Inoculum: 10,000 c/ml | 3 d | growth | 404.1 | S | 0.5* | T: 20°C; pH: 5.5; H: 250 mg/L CaCO3; DOC: 10.3 mg/L | Reconstituted | De Schamphelaere et al., 2006 |
| Chlorella vulgaris | Inoculum: 10,000 c/ml | 3 d | growth | 158.7 | S | 0.5* | T: 20°C; pH: 7.07; H: 25 mg/L CaCO3; DOC: 10.3 mg/L | Reconstituted | De Schamphelaere et al., 2006 |
| Chlorella vulgaris | Inoculum: 10,000 c/ml | 3 d | growth | 83.9 | S | 0.5* | T: 20°C; pH: 7.03; H: 250 mg/L CaCO3; DOC: 10.8 mg/L | Reconstituted | De Schamphelaere et al., 2006 |
| Chlorella vulgaris | Inoculum: 10,000 c/ml | 3 d | growth | 132.3 | S | 0.5* | T: 20°C; pH: 7.04; H: 250 mg/L CaCO3; DOC: 10.2 mg/L | Reconstituted | De Schamphelaere et al., 2006 |
| Pseudokirchneriella subcapitata | Inoculum: 10,000 c/ml | 3 d | growth | 52.9 | S | 0.5* | T: 23°C; pH: 6.31; H: 10.0 mg/L CaCO3; DOC: 2.72 mg/L | Lake | Heijerick et al., 2002 |
| Pseudokirchneriella subcapitata | Inoculum: 10,000 c/ml | 3 d | growth | 61.8 | S | 0.5* | T: 23°C; pH: 6.1; H: 12.4 mg/L CaCO3; DOC: 2.34 mg/L | Lake | Heijerick et al., 2002 |
| Pseudokirchneriella subcapitata | Inoculum: 10,000 c/ml | 3 d | growth | 94.7 | S | 0.5* | T: 23°C; pH: 5.52; H: 7.9 mg/L CaCO3; DOC: 10.3 mg/L | Lake | Heijerick et al., 2002 |
| Pseudokirchneriella subcapitata | Inoculum: 10,000 c/ml | 3 d | growth | 17.9 | S | 0.5* | T 23°C; pH: 8.23; H: 48.7 mg/L CaCO3; DOC: 2.52 mg/L | Lake | Heijerick et al., 2002 |
| Pseudokirchneriella subcapitata | Inoculum: 10,000 c/ml | 3 d | growth | 49 | S | 0.5* | T: 23°C; pH: 8.26; H: 220 mg/L CaCO3; DOC: 6.42 mg/L | Lake | Heijerick et al., 2002 |
| Pseudokirchneriella subcapitata | Inoculum: 10,000 c/ml | 3 d | growth | 35.4 | S | 0.5* | T: 23°C; pH: 8.30; H: 238 mg/L CaCO3; DOC: 8.24 mg/L | Lake | Heijerick et al., 2002 |
| Pseudokirchneriella subcapitata | Inoculum: 10,000 c/ml | 3 d | growth | 23.1 | S | 0.5* | T: 20°C; pH: 7.93; H: 191 mg/L CaCO3; DOC: 1.99 mg/L | River | Heijerick et al., 2002 |
| Pseudokirchneriella subcapitata | Inoculum: 10,000 c/ml | 3 d | growth | 19.3 | S | 0.5* | T: 23°C; pH: 8.06; H: 191 mg/L CaCO3; DOC: 1.98 mg/L | River | Heijerick et al., 2002 |
| Pseudokirchneriella subcapitata | Inoculum: 10,000 c/ml | 3 d | growth | 56.4 | S | 0.5* | T: 23°C; pH: 7.55; H: 132 mg/L CaCO3; DOC: 6.13 mg/L | River | Heijerick et al., 2002 |
| Pseudokirchneriella subcapitata | Inoculum: 10,000 c/ml | 3 d | growth | 164 | S | 0.5* | T: 23°C; pH: 7.30; H: 166 mg/L CaCO3; DOC: 17.8 mg/L | Lake | Heijerick et al., 2002 |
| Pseudokirchneriella subcapitata | Inoculum: 10,000 c/ml | 3 d | growth | 65.5 | S | 0.5* | T: 23°C; pH: 7.50; H: 134 mg/L CaCO3; DOC: 20.4 mg/L | Lake | Heijerick et al., 2002 |
| Pseudokirchneriella subcapitata | Inoculum: 10,000 c/ml | 3 d | growth | 15.7 | S | 0.5* | T: 23°C; pH: 8.2016; H: 169 mg/L CaCO3; DOC: 1.7 mg/L | Lake | Heijerick et al., 2002 |
| Lemna minor | Double fronded colonies | 7 d | growth | 30 | S | 0.5* | T: 25°C; pH: 6.5; H : 26.8 mg/L CaCO3; DOC: 0.5 mg/L (1) | artificial | Teisseire et al., 1998 |

Annex XII: High quality NOEC values for marine fish and related chemistry of the test waters.

e: value was estimated; m: value was measured and reported; NR :  Background concentration not reported, but since results are based on measured concentrations, this does not affect the validity of the final result. NOEC values between brachets are not included in the derivation of a species mean NOEC, because they are not the most sensitive biological endpoint for the species

| Species | Age and/or size of test organism | Test duration | Effect parameter | NOEC Value (µg/l) | Administration of test substance | Cu back-ground (µg/l) | DOC mg/L | Salinity (g/l) | Test water | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Atherinops affinis | early blastula embryo | 12 days | embryo abnormalities | (123) | static | <3m | 2.0e | 33 | filtered natural seawater | Anderson et al., 1991 |
| Atherinops affinis | early blastula embryo | 12 days | hatchability | (123- | static | <3m | 2.0e | 33 | filtered natural seawater | Anderson et al., 1991 |
| Atherinops affinis | early blastula embryo | 12 days | young abnormalities | 63 | static | <3m | 2.0e | 33 | filtered natural seawater | Anderson et al., 1991 |
| Atherinops affinis | early blastula embryo | 12 days | embryo abnormalities | (115) | static | <3m | 2.0e | 33 | filtered natural seawater | Anderson et al., 1991 |
| Atherinops affinis | early blastula embryo | 12 days | hatchability | (115) | static | <3m | 2.0e | 33 | filtered natural seawater | Anderson et al., 1991 |
| Atherinops affinis | early blastula embryo | 12 days | young abnormalities | 68 | static | <3m | 2.0e | 33 | filtered natural seawater | Anderson et al., 1991 |
| Atherinops affinis | early blastula embryo | 12 days | embryo abnormalities | 55 | static | <3m | 2.0e | 33 | filtered natural seawater | Anderson et al., 1991 |
| Atherinops affinis | early blastula embryo | 12 days | hatchability | 55 | static | <3m | 2.0e | 33 | natural seawater | Anderson et al., 1991 |
| Atherinops affinis | early blastula embryo | 12 days | young abnormalities | 55 | static | <3m | 2.0e | 33 | natural seawater | Anderson et al., 1991 |
| Cyprinodon variegates | Egg | 7 days | hatchability | (109) | flowthrough | <0.4m | 1.19m | 23.5-27 | natural seawater | Hurd, K., 2006b |
| Cyprinodon variegates | Embryo-larval stages | 32 days | Survival | (109) | flowthrough | <0.4m | 1.19m | 23.5-27 | natural seawater | Hurd, K., 2006b |
| Cyprinodon variegates | Embryo-larval stages | 32 days | Embryo development (weight) | 57.8 | flowthrough | <0.4m | 1.19m | 23.5-27 | natural seawater | Hurd, K., 2006b |
| Cyprinodon variegates | Embryo-larval stages | 32 days | Embryo development (length) | 57.8 | flowthrough | <0.4m | 1.19m | 23.5-27 | natural seawater | Hurd, K., 2006b |

Annex XIII: High quality NOEC values for marine invertebrates and related chemistry of the test waters.

e: value was estimated; m: value was measured and reported; NR :  Background concentration not reported, but since results are based on measured concentrations, this does not affect the validity of the final result

NOEC values between brachets are not included in the derivation of a species mean NOEC, because they are not the most sensitive biological endpoint for the species

| Species | Age and/or size of test organism | Test duration | Effect parameter | NOEC Value (µg/l) | Administra-tion of test substance | Cu back-ground (µg l-1) | DOC (mg/L) | Salinity (g/l) | Test water | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Penaeus mergulensis | Juvenile | 14 days | growth | 33 | Flowthrough | <1m | 2.0e | 20 | natural seawater | Ahsanullah, M. Et al 1995 |
| Penaeus monodon | Juvenile | 14 days | growth | 145 | Flowthrough | <1m | 2.0e | 20 | natural seawater | Ahsanullah, M. Et al 1995 |
| Tisbe furcata | Life cycle | 100 days max | Survival / reproduction | 19.1 | Static renewal | NR | 2.0e | 34 | Natural seawater | Bechmann R.K., 1994 |
| Artemia franciscana | Cysts | 48 hours | Hatching success | 6.6 | Static | 0.2m | 0.48m | Not reported | Artificial seawater | Brix, 2006 |
| Mytilus edulis | Embryo | 48 hours | Development | 6.2 | Flowthrough | 1.8m | 1.51m | 32 | natural seawater | Brooks, S. 2006 |
| Crassostreas gigas | Embryo | 24 hour | Development | 10.89 | Flowthrough | 2.8m | 2.19m | 31.1 - 34.2 | natural seawater + 0.1 mg DOC/L, added as humic acids | Brooks, S. 2006 |
| Crassostreas gigas | Embryo | 24 hour | Development | 10.42 | Flowthrough | 2.5m | 3.36m | 31.1 - 34.2 | natural seawater, + 0.81 mg DOC/L, added as humic acids | Brooks, S. 2006 |
| Crassostreas gigas | Embryo | 24 hour | Development | 12.83 | Flowthrough | 3.0m | 3.36m | 31.1 - 34.2 | natural seawater+ 1.02 mg DOC/L, added as humic acids | Brooks, S. 2006 |
| Crassostreas gigas | Embryo | 24 hour | Development | 19.53 | Flowthrough | 3.6m | 3.88m | 31.1 - 34.2 | natural seawater, + 1.85 mg DOC/L, added as humic acids | Brooks, S. 2006 |
| Crassostreas gigas | Embryo | 24 hour | Development | 28.19 | Flowthrough | 1.1m | 4.66m | 31.1 - 34.2 | natural seawater; + 2.77 mg DOC/L, added as humic acids | Brooks, S. 2006 |
| Crassostreas gigas | Embryo | 24 hour | Development | 47.13 | Flowthrough | 3.2m | 5.19m | 31.1 - 34.2 | natural seawater,+ 3.13 mg DOC/L, added as humic acids | Brooks, S. 2006 |
| Placopecten magellanicus | Adult | 8 weeks | gonad development | 10.0 | Flowthrough | 2.5-3.4m | 2.0e | 25 | Natural seawater | Gould, 1988 |
| Eurytemora affinis | <24 hrs | 8 days | Mortality, fecundity and maturation | 51.1 | Semi-static | <3m | 2.0e | 14 - 17 | natural estuarine water | Hall, L., 1997 |
| Paracentrotus lividus | Embryo | 48 hours | Development | 8.8 | Static | <0.4m | 1.83m | 34.4 | natural seawater | Hurd, K. 2006a |
| Mercenaria mercenaria | Larvae | 96 hours | Development | 7.0 | Static | 1m | 0.5e | 26.5 | Artificial seawater | LaBreche, 2002 |
| Paracentrotus lividus | Embryo | 48 hours | Development | 16.5 | Static | 0.32 – 1.45m | 2.0e | 35 | natural seawater | Lorenzo, J. 2006 |
| Neanthes arenaceodentata | 3-4 week larva | 28 days | growth | 13.5 | Flow through | 2±1m | 2.0m | 32 | filtered natural seawater | Pesch et al., 1986 |
| Neanthes arenaceodentata | 3-4 week larva | 28 days | growth | 12.1 | Flow through | 2±1m | 2.0m | 32 | filtered natural seawater | Pesch et al., 1986 |
| Mytilus edulis | 1.0-1.5 cm individuals | 10 days | growth rate | 6.0 | daily renewal of solutions | 2.0 – 2.4m | 2.0m | not reported | Filtered seawater | Redpath, K.J. 1985 |
| Goniastrea aspera | Larvae | 72 hours | Motility | 14.2 | Static | 1.2m | 2.0e | not reported | Natural seawater | Reichelt-Brushett, 2004 |
| Acropora tenuis | Larvae | 48 hours | Settlement success | 17.3 | Static | 0.63m | 2.0e | not reported | Natural seawater | Reichelt-Brushett, 2000 |
| Lobophytum compactum | Eggs/sperm | 5 hours | Fertilisa-tion success | 36.0 | Static | NR | 2.0e | not reported | Natural seawater | Reichelt-Brushett, 2005 |
| Protothaca staminea | 5.2 to 5.8 cm total length | 30 days | Mortality | 18 | Flowthrough | 0.35m | 2.0e | 32 | natural seawater | Roesijida, G. 1980 |
| Mytilus galloprovincialis | Embryo | 48 hours | Development | 5.9 | Static | 0.6m | 0.9m | Not reported | Filtered seawater | Rosen, 2005 |
| Mytilus galloprovincialis | Embryo | 48 hours | Development | 7.5 | Static | 1.5m | 0.9m | Not reported | Filtered seawater | Rosen, 2005 |
| Mytilus galloprovincialis | Embryo | 48 hours | Development | 9.2 | Static | 0.7m | 1.5m | Not reported | Filtered seawater | Rosen, 2005 |
| Mytilus galloprovincialis | Embryo | 48 hours | Development | 9.7 | Static | 1.0m | 0.9m | Not reported | Filtered seawater | Rosen, 2005 |
| Tisbe battagliai | <24 hrs | 21 days | Survival | 18 | Semi-Static | 2.0m | 2.79m | 35 | natural seawater | Williams, T. 2006 |
| Tisbe battagliai | <24 hrs | 21 days | Development | 18 | Semi-Static | 2.0m | 2.79m | 35 | natural seawater | Williams, T. 2006 |
| Tisbe battagliai | <24 hrs | 21 days | Reproduction | 18 | Semi-Static | 2.0m | 2.79m | 35 | natural seawater | Williams, T. 2006 |
| Pandalus danae | Larvae | >42 days | Mortality | 9.9 | Flow through | 0.47m | 2.0e | 29.8-30.6 | natural seawater | Young et al., 1979 |
| Pandalus danae | Larvae | >42 days | Development | 9.9 | Flow through | 0.47m | 2.0e | 29.8-30.6 | natural seawater | Young et al., 1979 |

Annex XIV: High quality NOEC values for marine algae and related chemistry of the test waters.

e: value was estimated; m: value was measured and reported; NR :  Background concentration not reported, but since results are based on measured concentrations, this does not affect the validity of the final result. NOEC values between brackets are not included in the derivation of a species mean NOEC, because they are not the most sensitive biological endpoint for the species

| Species | Age and/or size of test organism | Test duration | Effect parameter | NOEC value (µg/l) | Administration of test substance | Cu background (µg/L) | DOC (mg/L) | Salinity (g/l) | Test water | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Macrocystis pyrifera, motile zoospore | Zoospores | 19 days | sporophyte growth | 10.2 | static renewal | <0.6m | 2.0e | 35-37 | artificial filtered seawater | Anderson et al., 1990 |
| Macrocystis pyrifera, motile zoospore | Zoospores | 19 days | germination | (50.1) | static renewal | <0.6m | 2.0e | 35-37 | artificial filtered seawater | Anderson et al., 1990 |
| Macrocystis pyrifera, motile zoospore | Zoospores | 19 days | germ tube growth | 10.2 | static renewal | <0.6m | 2.0e | 35-37 | artificial filtered seawater | Anderson et al., 1990 |
| Fucus vesiculosis | Zoospore | 14 days | Growth | 11 | flow through | 4.2m | 1.67m | 30.9 | natural filtered seawater | Brooks, S., 2006d |
| Fucus vesiculosis | zoospore | 14 days | Growth | 18.5 | flow through | 2.3m | 2.11m | 31 | natural filtered seawater,+ 0.56 mg DOC/L added as humic acids | Brooks, S., 2006d |
| Fucus vesiculosis | zoospore | 14 days | Growth | 32 | flow through | 2.9m | 2.56m | 31.4 | natural filtered seawater,,+ 1.65 mg DOC/L added as humic acids | Brooks, S., 2006d |
| Fucus vesiculosis | zoospore | 14 days | Growth | 46 | flow through | 2.8m | 2.88m | 30.9 | natural filtered seawater,,+ 2.03 mg DOC/L added as humic acids | Brooks, S., 2006d |
| Phaeodactylum tricornutum | 103 cells/ml | 72 hours | Growth rate | 2.9 | static | NR | 1.0m | 31 | natural filtered seawater | Simpson, 2003 |
| Skeletonema costantum |  | 72 hours | Growth rate | 7.5 | static | <0.4m | 2.19m | 31 | natural filtered seawater | Smyth, D. 2006a |
| Phaeodactylum tricornutum | 104 cells/ml | 72 hours | Growth rate | 5.7 | static | <0.4m | 2.19m | 31 | natural filtered seawater | Smyth, D. 2006b |

Annex XV: Overview of the NOEC values for soil invertebrates.

Values selected for the effects assessment are underlined.  See IUCLID/RAR (2008) for reasons on the selection.

NOEC indices: m: mortality, r: reproduction (based on cocoon production (cp), juvenile production (jp)); h: hatching success, g: growth, ab: abundance, f: fragmentation, lb: litter breakdown, mi: maturity index; ri: Instantaneous rate of population increase. Estimated background copper concentrations and CEC** are indicated in italics.

*measured concentration-Cb; ** If the CEC was missing from a test with plants/invertebrates/micro-organisms, then it was estimated from % clay, pH and %organic matter using an experimentally derived regression model: CEC=(30+4.4 pH)*clay/100+(-34.66+29.72 pH)*OM/100; the clay is the % clay in the soil (Helling et al., 1964; regression based on CEC measured at various pH values on 60 different soils; CEC refers to the soil pH).

|  |  |  |  |  |  |  |  |  |  | Added NOEC |  | Total NOEC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Organism | Medium | pH | OM | clay | Cb | CEC | Equil. Period | Durat. | Endpoint | NOEC or EC10 | Unbounded NOEC | NOEC or EC10 | Unbounded NOEC |  |
|  |  |  | % | % | mg/kgdw | cmol/kg | days | days |  | mg/kgdw | mg/kgdw | mg/kgdw | mg/kgdw |  |
| Allobophora chlorotica | Sandy soil | 4.8-5.2 | 04-Jun | 02-Apr | 12 | 05-Sep | 0 | 28 | NOECr(cp) | 28 |  | 40 |  | Ma, 1988 |
| Allobophora caliginosa | Non-EU sandy soil |  |  |  | 10.7 |  | 0 | 14 | NOECm | 500 |  | 510.7 |  | Martin, 1986 |
| Allobophora caliginosa | Non-EU sandy soil |  |  |  | 10.7 |  | 0 | 14 | NOECr(cp) | 50 |  | 60.7 |  | Martin, 1986 |
| Aporrectodea caliginosa | Sandy soil | 4.8-5.2 | 04-Jun | 02-Apr | 10.7 |  | 0 | 28 | NOECr(cp) | 27 |  | 37.7 |  | Ma, 1988 |
| Aporrectodea caliginosa | Non-EU soil | 7.05 | 21.6 |  | 10.7 |  | 0 | 42 | NOECg | 25 |  | 35.7 |  | Khalil et al., 1996a and b |
| Aporrectodea caliginosa | Non-EU soil | 7.05 | 21.6 |  | 10.7 |  | 0 | 56 | NOECr(cp) | 70 |  | 80.7 |  | Khalil et al., 1996a and b |
| Cognettia sphagnetorum | LUFA 2.2 + peat + fungus | 4.1 | 66 | 5.1 | 10.7 | 60.6 | 0 | 63 | NOECg | 20 |  | 30.7 |  | Augustsson and Rundgren, 1998 |
| Cognettia sphagnetorum | LUFA 2.2 + peat + fungus | 4.1 | 66 | 5.1 | 10.7 | 60.6 | 0 | 35 | NOECg | 63 |  | 73.7 |  | Augustsson and Rundgren, 1998 |
| Cognettia sphagnetorum | LUFA 2.2 + peat + algae | 4.1 | 66 | 5.1 | 10.7 | 60.6 | 0 | 63 | NOECg | 441 |  | 451.7 |  | Augustsson and Rundgren, 1998 |
| Cognettia sphagnetorum | LUFA 2.2 + peat + algae | 4.1 | 66 | 5.1 | 10.7 | 60.6 | 0 | 42 | NOECg | 312 |  | 322.7 |  | Augustsson and Rundgren, 1998 |
| Cognettia sphagnetorum | LUFA 2.2 + peat + fungus | 4.1 | 66 | 5.1 | 10.7 | 60.6 | 0 | 70 | NOECf | 23 |  | 33.7 |  | Augustsson and Rundgren, 1998 |
| Cognettia sphagnetorum | LUFA 2.2 + peat + algae | 4.1 | 66 | 5.1 | 10.7 | 60.6 | 0 | 70 | NOECf | 455 |  | 465.7 |  | Augustsson and Rundgren, 1998 |
| Dendrobaena rubida | Sand+cattle dung | 4.5 | 7.7-11.7 |  | 3.6 |  | 28 | 90 | NOECh | 100 |  | 103.6 |  | Bengtsson et al., 1986 |
| Dendrobaena rubida | Sand+cattle dung | 5.5 | 7.7-11.7 |  | <0.5 |  | 28 | 90 | NOECh | 100 |  | 100 |  | Bengtsson et al., 1986 |
| Dendrobaena rubida | Sand+cattle dung | 5.5 | 7.7-11.7 |  | <0.5 |  | 28 | 90 | NOECr(cp) | 100 |  | 100 |  | Bengtsson et al., 1986 |
| Dendrobaena rubida | Sand+cattle dung | 6.5 | 7.7-11.7 |  | 1.3 |  | 28 | 90 | NOECh | 100 |  | 101.3 |  | Bengtsson et al., 1986 |
| Dendrobaena rubida | Sand+cattle dung | 6.5 | 7.7-11.7 |  | 1.3 |  | 28 | 90 | NOECr(cp) | 100 |  | 101.3 |  | Bengtsson et al., 1986 |
| Eisenia andrei | OECD soil | 6.2 | 10 | 20 | 3.2 | 15.1 | 0 | 84 | NOECg | 56 |  | 59.2 |  | Van Dis et al., 1988 |
| Eisenia andrei | OECD soil | 6.3-7.1 | 10 | 20 | 3.2 | 16.6 | 0 | 28 | NOECr(cp) | 120 |  | 123.2 |  | Van Gestel et al., 1989 |
| Eisenia andrei | OECD soil | 6.2 | 10 | 20 | 6.1 | 15.1 | 0 | 84 | NOECg | 56 |  | 62 |  | Van Gestel et al., 1991 |
| Eisenia andrei | Forest soil | 5.6 | <1 | 4 | 3.7 | 2.9 | 3 | 28 |  | 188* |  | 192 |  | Svendsen & Weeks, 1997a |
| Eisenia andrei | Forest soil | 5.6 | <1 | 4 | 3.7 | 2.9 | 3 | 28 | NOECr | 188* |  | 192 |  | Svendsen & Weeks, 1997a |
| Eisenia andrei | OECD soil | 6 | 10 | 20 | 3.2 | 14.5 | 0 | 28 | NOECr(cp) | 100 |  | 103.2 |  | Kula and Larink, 1997 |
| Eisenia andrei | OECD soil | 6 | 10 | 20 | 3.2 | 14.5 | 0 | 28 | NOECr(jp) | 100 |  | 103.2 |  | Kula and Larink, 1997 |
| Eisenia andrei | OECD soil | 6 | 10 | 20 | 3.2 | 14.5 | 0 | 28 | NOECg |  | ≥ 320 |  | ≥ 323.2 | Kula and Larink, 1997 |
| Eisenia andrei | LUFA 2.2 | 5.8 | 3.9 | 5.1 | 5.2 | 8.3 | 0 | 28 | NOECr(cp) | 3.2 |  | 8.4 |  | Kula and Larink, 1997 |
| Eisenia andrei | LUFA 2.2 | 5.8 | 3.9 | 5.1 | 5.2 | 8.3 | 0 | 28 | NOECg |  | ≥ 320 |  | ≥ 325.2a | Kula and Larink, 1997 |
| Eisenia fetida | OECD soil | 6.3 | 10 | 20 | 2.4 | 15.4 | 0 | 56 | NOECm | 200 |  | 202.4 |  | Spurgeon et al., 1994 |
| Eisenia fetida | OECD soil | 6.3 | 10 | 20 | 2.4 | 15.4 | 0 | 56 | NOECr(cp) | 10 |  | 12.4 |  | Spurgeon et al., 1994 |
| Eisenia fetida | OECD soil | 6.1 | 10 | 20 | 3.2 | 14.8 |  | 21 | NOECr(cp) | 29 |  | 32.3 |  | Spurgeon and Hopkin, 1995 |
| Eisenia fetida | OECD soil | 6.1 | 10 | 20 | 3.2 | 14.8 |  | 21 | NOECg | 725 |  | 728.2 |  | Spurgeon and Hopkin, 1995 |
| Eisenia fetida | OECD soil | 6.1 | 10 | 20 | 3.2 | 14.8 |  | 14 | NOECm | 293 |  | 296.2 |  | Spurgeon and Hopkin, 1995 |
| Eisenia fetida | OECD soil | 6 | 10 | 20 | 3.2 | 14.5 | 0 | 28 | NOECr(cp) | 10 |  | 13.2 |  | Spurgeon and Hopkin, 1995 |
| Eisenia fetida | OECD soil | 6 | 10 | 20 | 3.2 | 14.5 | 0 | 28 | NOECr(jp) | 32 |  | 35.2 |  | Kula and Larink, 1997 |
| Eisenia fetida | OECD soil | 6 | 10 | 20 | 3.2 | 14.5 | 0 | 28 | NOECg |  | ≥ 320 |  | ≥ 323.2 | Kula and Larink, 1997 |
| Eisenia fetida | LUFA 2.2 | 5.8 | 3.9 | 5.1 | 5.2 | 8.3 | 0 | 28 | NOECr(cp) | 10 |  | 15.2 |  | Kula and Larink, 1997 |
| Eisenia fetida | LUFA 2.2 | 5.8 | 3.9 | 5.1 | 5.2 | 8.3 | 0 | 28 | NOECr(jp) | 32 |  | 37.2 |  | Kula and Larink, 1997 |
| Eisenia fetida | LUFA 2.2 | 5.8 | 3.9 | 5.1 | 5.2 | 8.3 | 0 | 28 | NOECg |  | ≥ 320 |  | ≥ 325.2 | Kula and Larink, 1997 |
| Eisenia fetida | OECD soil | 6 | 10 | 20 | 3.2 | 14.5 | 0 | 28 | NOECm |  | ≥ 320 |  | ≥ 323.2 | Kula and Larink, 1998 |
| Eisenia fetida | LUFA 2.2 | 5.8 | 3.9 | 5.1 | 5.2 | 8.3 | 0 | 28 | NOECm |  | ≥ 320 |  | ≥ 325.2 | Kula and Larink, 1998 |
| Eisenia fetida | Sandy clay | 6.5-7.0 | 3.9-5.5 | 13-16 | 15 | 16.6 | 1 | 21 | NOECm |  | ≥ 1,400 |  | ≥ 1,415 | Scott-Fordsmand et al., 2000 |
| Eisenia fetida | Sandy clay | 6.5-7.0 | 3.9-5.5 | 13-16 | 15 | 16.6 | 1 | 21 | NOECg | 700 |  | 715 |  | Scott-Fordsmand et al., 2000 |
| Eisenia fetida | Sandy clay | 6.5-7.0 | 3.9-5.5 | 13-16 | 15 | 16.6 | 1 | 21 | NOECr | 100 |  | 115 |  | Scott-Fordsmand et al., 2000 |
| Enchytraeus albidus | OECD soil | 6 | 10 | 20 | 3.2 | 14.5 | 0 | 42 | NOECr – P- |  | <175.7 |  | <178.9 | Lock and Janssen (2002 |
| Enchytraeus albidus | OECD soil | 6 | 10 | 20 | 3.2 | 14.5 | 0 | 42 | NOECr-F1 |  | <175.7 |  | <178.9 | Lock and Janssen (2002 |
| Eisenia fetida | natural soil | / | / | / | 10.7 | / | 0 | 14 | NOECm | 650 |  | 660.7 |  | Liang and Zhou, 2003 |
| Folsomia candida | OECD soil | 6 | 10 | 20 | 3.2 | 14.5 | 2 | 28 | NOECm |  | ≥ 3000 |  | ≥ 3003.2 | Sandifer & Hopkin, 1996 |
| Folsomia candida | OECD soil | 6 | 10 | 20 | 3.2 | 14.5 | 2 | 28 | NOECr | 200 |  | 203.2 |  | Sandifer & Hopkin, 1996 |
| Folsomia candida | OECD soil | 5 | 10 | 20 | 3.2 | 11.5 | 2 | 28 | NOECm | 40 |  | 43.2 |  | Sandifer & Hopkin, 1996 |
| Folsomia candida | OECD soil | 5 | 10 | 20 | 3.2 | 11.5 | 2 | 28 | NOECr | 200 |  | 203.2 |  | Sandifer & Hopkin, 1996 |
| Folsomia candida | OECD soil | 4.5 | 10 | 20 | 3.2 | 10 | 2 | 28 | NOECm |  | ≥ 3000 |  | ≥ 3003.2 | Sandifer & Hopkin, 1996 |
| Folsomia candida | OECD soil | 4.5 | 10 | 20 | 3.2 | 10 | 2 | 28 | NOECr | 1,000 |  | 1,003.20 |  | Sandifer & Hopkin, 1996 |
| Folsomia candida | OECD soil | 6 | 10 | 20 | 3.2 | 14.5 | 2 | 28 | NOECr | 200 |  | 203.2 |  | Sandifer & Hopkin, 1997 |
| Folsomia candida | OECD soil | 6 | 10 | 20 | 3.2 | 14.5 | 2 | 42 | NOECr | 200 |  | 203.2 |  | Sandifer & Hopkin, 1997 |
| Folsomia candida | OECD soil | 6 | 10 | 20 | 3.2 | 14.5 | 2 | 28 | NOECm |  | ≥ 3,000 |  | ≥ 3,003 | Sandifer & Hopkin, 1997 |
| Folsomia candida | OECD soil | 6 | 10 | 20 | 3.2 | 14.5 | 2 | 28 | NOECm | 1,000 |  | 1,003.20 |  | Sandifer & Hopkin, 1997 |
| Folsomia candida | OECD soil | 6 | 10 | 20 | 3.2 | 14.5 | 2 | 42 | NOECm | 1,000 |  | 1,003.20 |  | Sandifer & Hopkin, 1997 |
| Folsomia candida | LUFA 2.2 | 5.8 | 3.9 | 5.1 | 5.2 | 8.3 | 0 | 21 | NOECg | 200 |  | 205.2 |  | Rundgren and Van Gestel, 1988 |
| Folsomia candida | LUFA 2.2 | 5.8 | 3.9 | 5.1 | 5.2 | 8.3 | 0 | 21 | NOECr | 400 |  | 405.2 |  | Rundgren and Van Gestel, 1988 |
| Folsomia candida | OECD | 6 | 10 | 20 | 3.2 | 14.5 | 0 | 56 | NOECg | 800 |  | 803.2 |  | Rundgren and Van Gestel, 1988 |
| Folsomia candida | OECD | 6 | 10 | 20 | 3.2 | 14.5 | 0 | 56 | NOECr | 400 |  | 403.2 |  | Rundgren and Van Gestel, 1988 |
| Folsomia fimetaria | LUFA 2.2 | 5.5 | 3.91 | 5 | 5.2 | 7.8 | 1 | 21 | NOECm | 800 |  | 805.2 |  | Scott-Fordsmand et al., 1997 |
| Folsomia fimetaria | LUFA 2.2 | 5.5 | 3.91 | 5 | 5.2 | 7.8 | 1 | 21 | NOECm |  | ≥1,000 |  | ≥1,005 | Scott-Fordsmand et al., 1997 |
| Folsomia fimetaria | LUFA 2.2 | 5.5 | 3.91 | 5 | 5.2 | 7.8 | 1 | 21 | NOECg | 542 |  | 547.2 |  | Scott-Fordsmand et al., 1997 |
| Folsomia fimetaria | LUFA 2.2 | 5.5 | 3.91 | 5 | 5.2 | 7.8 | 1 | 21 | NOECg | 845 |  | 850.2 |  | Scott-Fordsmand et al., 1997 |
| Folsomia fimetaria | LUFA 2.2 | 5.5 | 3.91 | 5 | 5.2 | 7.8 | 1 | 21 | NOECg | 400 |  | 405.2 |  | Scott-Fordsmand et al., 1997 |
| Folsomia fimetaria | Sandy clay | 6.5-7.0 | 3.9-5.5 | 13-16 | 15 | 16.6 | 1 | 21 | NOECm | 1,000 |  | 1,015 |  | Scott-Fordsmand et al., 2000a |
| Folsomia fimetaria | Sandy clay | 6.5-7.0 | 3.9-5.5 | 13-16 | 15 | 16.6 | 1 | 21 | NOECm | 600 |  | 615 |  | Scott-Fordsmand et al., 2000a |
| Folsomia fimetaria | Sandy clay | 6.5-7.0 | 3.9-5.5 | 13-16 | 15 | 16.6 | 1 | 21 | NOECm | 1,000 |  | 1,015 |  | Scott-Fordsmand et al., 2000a |
| Folsomia fimetaria | Sandy clay | 6.5-7.0 | 3.9-5.5 | 13-16 | 15 | 16.6 | 1 | 21 | NOECg | 1,000 |  | 1,015 |  | Scott-Fordsmand et al., 2000a |
| Folsomia fimetaria | Sandy clay | 6.5-7.0 | 3.9-5.5 | 13-16 | 15 | 16.6 | 1 | 21 | NOECg | 1,000 |  | 1,015 |  | Scott-Fordsmand et al., 2000a |
| Folsomia fimetaria | Sandy clay | 6.5-7.0 | 3.9-5.5 | 13-16 | 15 | 16.6 | 1 | 21 | NOECg | 1,000 |  | 1,015 |  | Scott-Fordsmand et al., 2000a |
| Folsomia fimetaria | Sandy clay | 6.5-7.0 | 3.9-5.5 | 13-16 | 15 | 16.6 | 1 | 21 | NOECr | 400 |  | 415 |  | Scott-Fordsmand et al., 2000a |
| Folsomia fimetaria | Sandy clay | 6.7 | 4.5 | 13.8 | 19 | 15.6 | 0 | 21 | EC10r | 122 |  | 141 |  |  |
|  | Hygum | 6.7 | 4.5 | 13.8 | 19 | 15.6 | 0 | 21 |  |  |  |  |  | Pedersen et al., 2000 |
| Folsomia candida | Sandy clay | 6.7 |  |  |  |  |  |  | EC10r | <31 |  |  | <50 | Pedersen et al., 2000 |
| Folsomia fimetaria | Sandy clay | 6.7 |  |  |  |  |  |  | EC10r | 698 |  | 717 |  |  |
|  | Hygum | 6.7 |  |  |  |  |  |  |  | 776 |  | 795 |  |  |
|  |  | 6.7 | 4.5 |  |  |  |  |  |  | 888 |  | 907 |  | Pedersen et al., 2001 |
|  |  | 4.5 |  |  |  |  |  |  |  | 648 |  | 667 |  |  |
|  |  | 4.5 |  |  |  |  |  |  |  | 688 |  | 707 |  |  |
| Folsomia candida | artificial OECD soil | 4.5 | 13.8 |  |  |  |  |  | NOECri | 796.8 |  | 800 |  | Herbert et al, 2004. |
| Hypoaspis aculeifer | LUFA 2.2 | 13.8 |  |  |  |  |  |  | NOECr | 174 |  | 179.2 |  | Krogh and Axelsen, 1998 |
| Hypoaspis aculeifer | LUFA 2.2 | 13.8 |  |  |  |  |  |  | NOECm |  | ≥ 1,000 |  | ≥ 1,005 | Krogh and Axelsen, 1998 |
| Isotoma viridis | LUFA 2.2 | 19 |  |  |  |  |  |  | NOECg | 50 |  | 55.2 |  | Rundgren and Van Gestel, 1988 |
| Isotoma viridis | OECD | 19 |  |  |  |  |  |  | NOECg | 400 |  | 403.2 |  | Rundgren and Van Gestel, 1988 |
| Lumbricus rubellus | Sandy loam | 19 | 15.6 |  |  |  |  |  | NOECm | 150 |  | 162 |  | Ma, 1982 |
| Lumbricus rubellus | Loamy sand | 15.6 |  |  |  |  |  |  | NOECr | 40* |  | 54 |  | Ma, 1984 |
| Lumbricus rubellus | Loamy sand | 15.6 |  |  |  |  |  |  | NOEClb | 40* |  | 54 |  | Ma, 1984 |
| Lumbricus rubellus | Loamy sand | 15.6 | 1 |  |  |  |  |  | NOECg | 117* |  | 131 |  | Ma, 1984 |
| Lumbricus rubellus | Loamy sand | 7 |  |  |  |  |  |  | NOECm | 117* |  | 131 |  | Ma, 1984 |
| Lumbricus rubellus | Calcareous sandy loam | 35 |  |  |  |  |  |  | NOEClb | 50* |  | 63 |  | Ma, 1984 |
| Lumbricus rubellus | Calcareous sandy loam | 84 | 21 |  |  |  |  |  | NOECg |  | ≥ 360 |  | ≥ 373 | Ma, 1984 |
| Lumbricus rubellus | Calcareous sandy loam | 21 |  |  |  |  |  |  | NOECm | 123* |  | 136 |  | Ma, 1984 |
| Lumbricus rubellus | Sandy soil | 21 |  |  |  |  |  |  | NOECr(cp) | 80 |  | 90.7 |  | Ma, 1988 |
| Lumbricus rubellus | Forest soil |  |  |  |  |  |  |  | NOECg | 73* |  | 76 |  | Svendsen & Weeks, 1997b |
| Lumbricus rubellus | Forest soil | 6 | 10 | 20 | 3.2 | 14.5 | 7 | 28 | NOECm | 150* |  | 153 |  | Svendsen & Weeks, 1997b |
| Lumbricus rubellus | Clay loam | 5.8 | 3.9 | 5.1 | 5.2 | 8.3 | 0 | 21 | NOECg | 139.6 |  | 154 |  | Spurgeon et al (2004) |
| Octalasium cyaneum | brown soil |  |  |  |  |  |  |  | NOECm | 100 |  | 153 |  | Spurgeon et al (2004) |
| Octalasium cyaneum | peaty soil | 5.8 | 3.9 | 5.1 | 5.2 | 8.3 | 0 | 56 | NOECm | 1,200 |  | 1,214 |  | Jäggy & Streit, 1982 |
| Plectus acuminatus | OECD |  |  |  |  |  |  |  | NOECr(jp) | 32 |  | 35.2 |  | Kammenga et al., 1996 |
| Platynothrus peltifer | LUFA 2.2 |  |  |  |  |  |  |  | NOECr(jp) | 63 |  | 68.2 |  | Van Gestel and Doornekamp, 1998 |
| Platynothrus peltifer | LUFA 2.2 | 4.8 | 5.7 | 2 | 14 | 7.2 | 0 | 42 | NOECr(jp) | 63 |  | 68.2 |  | Van Gestel and Doornekamp, 1998 |
| Platynothrus peltifer | LUFA 2.2 | 4.8 | 5.7 | 2 | 14 | 7.2 | 0 | 42 | NOECr(jp) | 63 |  | 68.2 |  | Van Gestel and Doornekamp, 1998 |
| Folsomia candida | loamy sand | 4.8 | 5.7 | 2 | 14 | 7.2 | 0 | 42 | NOECr |  | < 29.7 |  | <31.7 | Criel et al, 2005 |
|  | Gudow | 7.3 | 3.4 | 17 | 13 | 16.9 | 0 | 42 |  |  |  |  |  |  |
| Folsomia candida | Sandy loam | 7.3 | 3.4 | 17 | 13 | 16.9 | 0 | 42 | NOECr | 174 |  | 191 |  | Criel et al, 2005 |
|  | Nottingham | 7.3 | 3.4 | 17 | 13 | 16.9 | 0 | 42 |  |  |  |  |  |  |
| Folsomia candida | loamy sand |  |  |  |  |  |  |  | NOECr | 28.2 |  | 31 |  | Criel et al, 2005 |
|  | Houthalen | 4.8-5.2 | 04-Jun | 02-Apr | 10.7 |  | 0 | 28 |  |  |  |  |  |  |
| Folsomia candida | loamy sand |  |  |  |  |  |  |  | NOECr | 279 |  | 293 |  | Criel et al, 2005 |
|  | Rhydtalog | 5.6 | <1 | 4 | 3 | 2.9 | 5 | 110 |  |  |  |  |  |  |
| Folsomia candida | Sandy clay loam | 5.6 | <1 | 4 | 3 | 2.9 | 5 | 110 | EDr | 1390 |  | 1460 |  | Criel et al, 2005 |
|  | Zegveld |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Folsomia candida | Loamy sand | 7.2-7.8 | 9.6-9.95 | 41 | 14.4 | 44.2 | 14 | 294 | NOECr | 55.5 |  | 61.5 |  | Criel et al, 2005 |
|  | Kovlinge |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Folsomia candida | Sandy clay | 4.78 | 5.4 |  | 53 |  | 0 | 30 | NOECr | 53.1 |  | 84.1 |  | Criel et al, 2005 |
|  | Souli | 4.5 | 72 |  | 14 |  | 0 | 14 |  |  |  |  |  |  |
| Folsomia candida | loamy sand |  |  |  |  |  |  |  | NOECr | 172 |  | 177 |  | Criel et al, 2005 |
|  | Montpellier | 5.5 | 10 | 20 | 3.2 | 13 | 5h | 21 |  |  |  |  |  |  |
| Folsomia candida | Clay |  |  |  |  |  |  |  | NOECr | 276 |  | 297 |  | Criel et al, 2005 |
|  | Aluminusa | 5.8 | 3.9 | 5.1 | 5.2 | 8.3 | 0 | 70 |  |  |  |  |  |  |
| Folsomia candida | Sandy clay loam | 5.8 | 3.9 | 5.1 | 5.2 | 8.3 | 0 | 70 | NOECr | 244 |  | 266 |  | Criel et al, 2005 |
|  | Woburn | 5.8 | 3.9 | 5.1 | 5.2 | 8.3 | 0 | 70 |  |  |  |  |  |  |
| Folsomia candida | Silt loam |  |  |  |  |  |  |  | NOECr | 237 |  | 259 |  | Criel et al, 2005 |
|  | Ter Munck | 3 | 8.2 | 7 | 2 | 5.8 | 7 | 28 |  |  |  |  |  |  |
| Folsomia candida | Silty clay loam | 3.4 | 8.3 | 13 | 17 | 6.7 | 7 | 28 | NOECr | 534 |  | 555 |  | Criel et al, 2005 |
|  | Vault de lugny | 3.4 | 3 | 5 | 2 | 1.9 | 7 | 28 |  |  |  |  |  |  |
| Folsomia candida | Silty clay loam | 4.2 | 20.7 | 13 | 14 | 15.2 | 7 | 28 | NOECr | 160 |  | 174 |  | Criel et al, 2005 |
|  | Rots | 4.7 | 37.3 | 24 | 70 | 35.3 | 7 | 28 |  |  |  |  |  |  |
| Folsomia candida | Clay | 4.8 | 2.6 | 7 | 6 | 2.4 | 7 | 28 | NOECr | 887 |  | 921 |  | Criel et al, 2005 |
|  | Souli | 4.8 | 0.7 | 38 | 31 | 11.2 | 7 | 28 |  |  |  |  |  |  |
| Folsomia candida | Silt loam | 5.2 | 1.2 | 9 | 5 | 2.5 | 7 | 28 | NOECr | 453 |  | 471 |  | Criel et al, 2005 |
|  | Marknesse | 5.4 | 1.4 | 51 | 21 | 22.6 | 7 | 28 |  |  |  |  |  |  |
| Folsomia candida | Loam | 6.4 | 7 | 21 | 22 | 23.4 | 7 | 28 | NOECr | 139 |  | 227 |  | Criel et al, 2005 |
|  | Barcelona | 6.8 | 1.6 | 15 | 22 | 8.9 | 7 | 28 |  |  |  |  |  |  |
| Folsomia candida | Clay | 7.3 | 2.5 | 38 | 21 | 26.2 | 7 | 28 | NOECr | 632 |  | 663 |  | Criel et al, 2005 |
|  | Brécy | 7.4 | 2 | 27 | 14 | 20 | 7 | 28 |  |  |  |  |  |  |
| Folsomia candida | Loam | 7.4 | 4.2 | 46 | 34 | 36.3 | 7 | 28 | NOECr | 538 |  | 545 |  | Criel et al, 2005 |
|  | Guadalajara | 7.5 | 2 | 26 | 18 | 20.1 | 7 | 28 |  |  |  |  |  |  |
| Folsomia candida | Sandy clay | 7.5 | 2.4 | 21 | 88 | 14.3 | 7 | 28 | NOECr | 493 |  | 511 |  | Criel et al, 2005 |
|  | Hygum | 7.5 | 2.4 | 50 | 31 | 23.5 | 7 | 28 |  |  |  |  |  |  |
| Folsomia candida | Loamy sand Wageningen A | 7.5 | 0.6 | 25 | 7 | 16.9 | 7 | 28 | NOECr | 27.9 |  | 45.4 |  | Criel et al, 2005 |
| Folsomia candida | Loamy sand Wageningen D | 5.4 | 3.3 | 23 | 21 | 6.7 | 7 | 28 | NOECr | 48 |  | 65.4 |  | Criel et al, 2005 |
| Folsomia candida | Sand | 4.3 | 2.2 | 9 | 19 | 1.2 | 7 | 28 | NOECr |  |  |  |  | Criel et al, 2005 |
|  | Woburn salt | 5 | 2.3 | 9 | 19 | 1.9 | 7 | 28 |  |  |  |  |  |  |
| Folsomia candida | Sand | 6.5 | 0.2 | 8 | 13 | 8.4 | 7 | 28 | NOECr | 132 |  | 167 |  | Criel et al, 2005 |
|  | Woburn cake | 6.5 | 0.3 | 8 | 35 | 11.6 | 7 | 28 |  |  |  |  |  |  |
| Eisenia fetida | loamy sand | 3 | 8.2 | 7 | 2 | 5.8 | 7 | 28 | NOECr | 177 |  | 179 |  | Criel et al, 2005 |
|  | Gudow | 3.4 | 8.3 | 13 | 17 | 6.7 | 7 | 28 |  |  |  |  |  |  |
| Eisenia fetida | Sandy loam | 4.7 | 37.3 | 24 | 70 | 35.3 | 7 | 28 | NOECr | 93.6 |  | 110.6 |  | Criel et al, 2005 |
|  | Nottingham | 4.8 | 2.6 | 7 | 6 | 2.4 | 7 | 28 |  |  |  |  |  |  |
| Eisenia fetida | Sandy clay loam | 4.8 | 0.7 | 38 | 31 | 11.2 | 7 | 28 | NOECr | 56.4 |  | 126 |  | Criel et al, 2005 |
|  | Zegveld | 5.1 | 3.8 | 9 | 8 | 4.7 | 7 | 28 |  |  |  |  |  |  |
| Eisenia fetida | Loamy sand | 5.2 | 1.2 | 9 | 5 | 2.5 | 7 | 28 | NOECr | 48.2 |  | 54 |  | Criel et al, 2005 |
|  | Kovlinge | 5.4 | 1.4 | 51 | 21 | 22.6 | 7 | 28 |  |  |  |  |  |  |
| Eisenia fetida | Sandy clay | 6.4 | 7 | 21 | 22 | 23.4 | 7 | 28 | NOECr | 179 |  | 210 |  | Criel et al, 2005 |
|  | Souli | 6.8 | 1.6 | 15 | 22 | 8.9 | 7 | 28 |  |  |  |  |  |  |
| Eisenia fetida | Sandy loam | 7.3 | 2.3 | 38 | 21 | 26.2 | 7 | 28 | NOECr | 86.8 |  | 95 |  | Criel et al, 2005 |
|  | Kovlinge | 7.4 | 2 | 27 | 14 | 20 | 7 | 28 |  |  |  |  |  |  |
| Eisenia fetida | Loamy sand | 7.4 | 4.2 | 46 | 34 | 36.3 | 7 | 28 | NOECr | 54.9 |  | 60 |  | Criel et al, 2005 |
|  | Montpellier | 7.5 | 2 | 26 | 18 | 20.1 | 7 | 28 |  |  |  |  |  |  |
| Eisenia fetida | Clay | 7.5 | 2.4 | 50 | 31 | 23.5 | 7 | 28 | NOECr |  | < 91.9 |  | < 113 | Criel et al, 2005 |
|  | Aluminusa | 5.4 | 3.3 | 23 | 21 | 6.7 | 7 | 28 |  |  |  |  |  |  |
| Eisenia fetida | Sandy clay loam | 5 |  |  | 5.7 | 7.88 | 7 | 28 | NOECr | 177 |  | 199 |  | Criel et al, 2005 |
|  | Woburn | 6.45 |  |  | 2.3 | 16.74 | 7 | 28 |  |  |  |  |  |  |
| Eisenia fetida | Silt loam | 5 |  |  | 5.7 | 7.88 | 7 | 28 | ED10r | 91.8 |  | 114 |  | Criel et al, 2005 |
|  | Ter Munck |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Eisenia fetida | Silty clay loam | 7.3 | 2.3 | 38 | 21 | 26.2 | 7 | 28 | NOECr | 303 |  | 324 |  | Criel et al, 2005 |
|  | Vault de lugny |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Eisenia fetida | Silty clay loam | 7.4 | 2 | 27 | 14 | 20 | 7 | 28 | NOECr | 289 |  | 303 |  | Criel et al, 2005 |
|  | Rots |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Eisenia fetida | Clay | 7.4 | 4.2 | 46 | 34 | 36.3 | 7 | 28 | NOECr | 287 |  | 321 |  | Criel et al, 2005 |
|  | Souli |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Eisenia fetida | Silt loam | 7.5 | 2 | 26 | 18 | 20.1 | 7 | 28 | NOECr | 153 |  | 171 |  | Criel et al, 2005 |
|  | Marknesse |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Eisenia fetida | Clay | 7.5 | 2.4 | 50 | 31 | 23.5 | 7 | 28 | NOECr | 164 |  | 195 |  | Criel et al, 2005 |
|  | Brécy |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Eisenia fetida | Sandy clay | 5.4 | 3.3 | 23 | 21 | 6.7 | 7 | 28 | NOECr | 91.6 |  | 112.6 |  | Criel et al, 2005 |
|  | Hygum |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Eisenia fetida | Lufa 2.2 | 5 |  |  | 5.7 | 7.88 | 7 | 28 | NOEC | 81.9 |  | 87.6 |  | Criel et al, 2005 |
| Eisenia fetida | OECD | 6.45 |  |  | 2.3 | 16.74 | 7 | 28 | NOEC | 186 |  | 188 |  | Criel et al, 2005 |
| Eisenia andrei | Lufa 2.2 | 5 |  |  | 5.7 | 7.88 | 7 | 28 | NOEC | 154 |  | 159 |  | Criel et al, 2005 |

Annex XVI: Overview of the NOEC values for plants

Values selected for the effects assessment are underlined.  See IUCLID/RAR (2008) for reasons on the selection. NOEC indices: m = mortality; y = yield (based on root (r), shoot (s), leaves (l), stem (st), grain (g), tubers (tub) or total plant (tp) dry weight); rep = reproductive dry matter; sb = seed biomass; ; se=seedling emergence; rl = root length. Estimated background copper concentrations and CEC** are indicated in italics.

|  |  |  |  |  |  |  |  |  |  | Added NOEC |  | Total NOEC |  | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Organism | Medium | pH | OM | clay | Cb | CEC | Equil. Period | Duration | Endpoint | NOEC or EC10 | Unbounded NOEC | NOEC or EC10 | Unbounded NOEC |  |
|  |  |  | % | % | mg/kgdw | cmol/kg | d | d |  | mg/kgdw | mg/kgdw | mg/kgdw | mg/kgdw |  |
| Polygonum convolvulus = Fallopia convolvulus | Field soil: clayey sand | 6.4 | 1.7 | 11.1 | 12 | 9.2 | 11 | 105 | NOECm | 125 |  | 137 |  | Kjær and Elmegaard, 1996 |
| Polygonum convolvulus | Field soil: clayey sand | 6.4 | 1.7 | 11.1 | 12 | 9.2 | 11 | 34 | NOECy(tp) | 200 |  | 212 |  | Kjær and Elmegaard, 1996 |
| Polygonum convolvulus | Field soil: clayey sand | 6.4 | 1.7 | 11.1 | 12 | 9.2 | 11 | 34 | NOECrep | 200 |  | 212 |  | Kjær and Elmegaard, 1996 |
| Polygonum convolvulus | Field soil: clayey sand | 6.4 | 1.7 | 11.1 | 12 | 9.2 | 11 | 105 | NOECsb | 200 |  | 212 |  | Kjær and Elmegaard, 1996 |
| Fallopia convolvulus | Field soil : Hygum site | 6.7 | 4.5 | 13.8 | 22 | 15.7 | 84 | 35 | NOECy(s) | 200 |  | 222 |  | Pedersen et al., 2000 |
| Fallopia convolvulus | Field soil : Hygum site | 6.7 | 4.5 | 13.8 | 22 | 15.7 | 84 | 35 | NOECy(r) | 200 |  | 222 |  | Pedersen et al., 2000 |
| Vigna mungo | Unspecified | 6.2 |  |  | 10.7 |  |  | 45 | NOECy(st) | 50 |  | 60.7 |  | Kalyanaraman and Sivagurunathan, 1993 |
| Vigna mungo | Unspecified | 6.2 |  |  | 10.7 |  |  | 45 | NOECy(l) | 100 |  | 110.7 |  | Kalyanaraman and Sivagurunathan, 1993 |
| Triticum aestivum | Loamy sand | 7.8 | 0.2 |  | 10.7 |  |  | To flag leaf stage | NOECy(s) | 40 |  | 50.7 |  | Chhibba et al., 1994 |
| Triticum aestivum | Loamy sand | 7.8 | 0.2 |  | 10.7 |  |  | To maturity | NOECy(g) | 40 |  | 50.7 |  | Chhibba et al., 1994 |
| Citrus reticulata | Sandy soil | 5.8 | 0.8 |  | 10.7 | 1.78 | 45 | 220 | NOECy(s) |  | <50 |  | <60.7 | Alva et al., 1993 |
| Citrus reticulata | Sandy soil | 5.8 | 0.8 |  | 10.7 | 1.78 | 45 | 220 | NOECy(s) |  | <50 |  | <60.7 | Alva et al., 1993 |
| Citrus limon | Sandy soil | 5.8 | 0.8 |  | 10.7 | 1.78 | 45 | 220 | NOECy(s) |  | <50 |  | <60.7 | Alva et al., 1993 |
| Citrus limon | Sandy soil | 5.8 | 0.8 |  | 10.7 | 1.78 | 45 | 220 | NOECy(s) |  | <50 |  | <60.7 | Alva et al., 1993 |
| Avena sativa | Loamy fine sand | 6.5 |  |  | 10.7 |  | 14 | 49 | NOECy(s) |  | <100 |  | <110.7 | Rhoads et al., 1992 |
| Avena sativa | Loamy fine sand | 7.1 |  |  | 10.7 |  | 14 | 49 | NOECy(s) |  | <100 |  | <110.7 | Rhoads et al., 1992 |
| Avena sativa | Loamy fine sand | 6.7 |  |  | 10.7 |  | 14 | 49 | NOECy(s) |  | <100 |  | <110.7 | Rhoads et al., 1992 |
| Avena sativa | Loamy fine sand | 6.5 |  |  | 10.7 |  | 14 | 49 | NOECy(r) | 100 |  | 110.7 |  | Rhoads et al., 1992 |
| Avena sativa | Loamy fine sand | 6.7 |  |  | 10.7 |  | 14 | 49 | NOECy(r) | 100 |  | 110.7 |  | Rhoads et al., 1992 |
| Avena sativa | Loamy fine sand | 7.1 |  |  | 10.7 |  | 14 | 49 | NOECy(r) | 100 |  | 110.7 |  | Rhoads et al., 1992 |
| Avena sativa | Clay soil | 5.6 | 1.6 | 12 | 6 | 8.7 |  | 150 | NOECy(g) | 200 |  | 206 |  | De Haan et al., 1985 |
| Avena sativa | Clay soil | 5.4 | 2.4 | 40 | 7 | 24.7 |  | 150 | NOECy(g) | 200 |  | 207 |  | De Haan et al., 1985 |
| Avena sativa | Clay soil | 5.2 | 3.2 | 58 | 58 | 34.8 |  | 150 | NOECy(g) | 200 |  | 258 |  | De Haan et al., 1985 |
| Avena sativa | Sandy soil | 5 | 3.4 | 4 | 4 | 6 |  | 150 | NOECy(g) | 200 |  | 204 |  | De Haan et al., 1985 |
| Avena sativa | Sandy soil | 5.4 | 6.8 | 5 | 19 | 11.3 |  | 150 | NOECy(g) | 200 |  | 219 |  | De Haan et al., 1985 |
| Avena sativa | Sandy soil | 4.6 | 19.4 | 4 | 21 | 22 |  | 150 | NOECy(g) |  | ≥ 400 |  | ≥ 421 | De Haan et al., 1985 |
| Avena sativa | Peaty muck | 7 | 56 |  | 86 |  | 21 | 31 | NOECy(s) |  | ≥ 4705* |  | ≥ 4791 | Roth et al., 1971 |
| Glycine max | Peaty muck | 7 | 56 |  | 86 |  | 21 | 46 | NOECy(s) | 1946* |  | 2032 |  | Roth et al., 1971 |
| Lolium perenne | Loamy soil | 7.5 | 3.1 | 12.8 | 10.7 | 14 |  | 102 | NOECy(s) | 95.3 |  | 106 |  | Jarvis, 1978 |
| Lolium perenne | Loamy soil | 7.5 | 3.1 | 12.8 | 10.7 | 14 |  | 102 | NOECy(r) | 95.3 |  | 106 |  | Jarvis, 1978 |
| Hordeum vulgare | Forest soil | 7.6 | 3.8 | 8 | 17.2 | 12.4 | 0 | 14 | NOECg(s) | 304.8 |  | 322 |  | Ali et al, 2004 |
|  |  |  |  |  |  |  |  |  | NOECg(r) | 20.2 |  | 37.4 |  |  |
|  |  |  |  |  |  |  |  |  | NOECse | 111.8 |  | 129 |  |  |
| Hordeum vulgare | Artificial soil | 7.8 | 0.3 | 0 | 0 | 0.6 | 0 | 14 | NOECg(s) | 11.2 |  | 11.2 |  | Ali et al, 2004 |
|  |  |  |  |  |  |  |  |  | NOECg(r) | 1 |  | 1 |  |  |
|  |  |  |  |  |  |  |  |  | NOECse | 11 |  | 11 |  |  |
| Hordeum vulgare | Sandy loam Nottingham | 3.4 | 8.3 | 13 | 17 | 6.7 | 7 | 4 | EC10rl | 58 |  | 75 |  | Rooney et al.(2004) |
| Hordeum vulgare | Loamy sand Houthalen | 3.4 | 3.2 | 5 | 2 | 1.9 | 7 | 4 | EC10rl | 16 |  | 18 |  | Rooney et al.(2004) |
| Hordeum vulgare | Loamy sand Rhydtalog | 4.2 | 20.7 | 13 | 14 | 15.2 | 7 | 4 | NOECrl | 30 |  | 44 |  | Rooney et al.(2004) |
| Hordeum vulgare | Sandy clay loam Zegveld | 4.7 | 37.3 | 24 | 70 | 35.3 | 7 | 4 | NOECrl | 80 |  | 150 |  | Rooney et al.(2004) |
| Hordeum vulgare | Loamy sand Kovlinge I | 4.8 | 2.6 | 7 | 6 | 2.4 | 7 | 4 | NOECrl | 45 |  | 51 |  | Rooney et al.(2004) |
| Hordeum vulgare | Sandy clay | 4.8 | 0.7 | 38 | 31 | 11.2 | 7 | 4 | NOECrl | 77 |  | 108 |  | Rooney et al.(2004) |
|  | Souli I |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hordeum vulgare | Sandy loam Kovlinge II | 5.1 | 3.8 | 9 | 8 | 4.7 | 7 | 4 | NOECrl | 37 |  | 45 |  | Rooney et al.(2004) |
| Hordeum vulgare | Loamy sand Montpellier | 5.2 | 1.2 | 9 | 5 | 2.5 | 7 | 4 | EC10rl | 38 |  | 43 |  | Rooney et al.(2004) |
| Hordeum vulgare | Clay Aluminosa | 5.4 | 1.4 | 51 | 21 | 22.6 | 7 | 4 | NOECrl | 252 |  | 273 |  | Rooney et al.(2004) |
| Hordeum vulgare | Sandy clay loam Woburn | 6.4 | 7 | 21 | 22 | 23.4 | 7 | 4 | NOECrl | 144 |  | 166 |  | Rooney et al.(2004) |
| Hordeum vulgare | Silt loam | 6.8 | 1.6 | 15 | 22 | 8.9 | 7 | 4 | NOECrl | 55 |  | 77 |  | Rooney et al.(2004) |
|  | Ter Munck |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hordeum vulgare | Silty clay loam Vault de Lugny | 7.3 | 2.3 | 38 | 21 | 26.2 | 7 | 4 | NOECrl | 154 |  | 175 |  | Rooney et al.(2004) |
| Hordeum vulgare | Silty clay loam Rots | 7.4 | 2 | 27 | 14 | 20 | 7 | 4 | NOECrl | 47 |  | 61 |  | Rooney et al.(2004) |
| Hordeum vulgare | Clay | 7.4 | 4.2 | 46 | 34 | 36.3 | 7 | 4 | EC10rl | 120 |  | 154 |  | Rooney et al.(2004) |
|  | Souli II |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hordeum vulgare | Silt loam Marknesse | 7.5 | 2 | 26 | 18 | 20.1 | 7 | 4 | NOECrl | 37 |  | 55 |  | Rooney et al.(2004) |
| Hordeum vulgare | Loam | 7.5 | 2.4 | 21 | 88 | 14.3 | 7 | 4 | NOECrl | 77 |  | 165 |  | Rooney et al.(2004) |
|  | Barcelona |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hordeum vulgare | Clay | 7.5 | 2.4 | 50 | 31 | 23.5 | 7 | 4 | NOECrl | 44 |  | 75 |  | Rooney et al.(2004) |
|  | Brecy |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hordeum vulgare | Loam | 7.5 | 0.6 | 25 | 7 | 16.9 | 7 | 4 | NOECrl |  | <37 |  | <44 | Rooney et al.(2004) |
|  | Guadalajara |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hordeum vulgare | Sandy clay | 5.4 | 3.3 | 23 | 21 | 6.7 | 7 | 4 | NOECrl | 114 |  | 135 |  | Rooney et al.(2004) |
|  | Hygum |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hordeum vulgare | Loamy sand Wageningen A | 4.3 | 2.2 | 9 | 19 | 1.2 | 7 | 4 | NOECl |  | <70 |  | <89 | Rooney et al.(2004) |
| Hordeum vulgare | Loamy sand Wageningen D | 5 | 2.3 | 9 | 19 | 1.9 | 7 | 4 | NOECrl |  | >138 |  | >157 | Rooney et al.(2004) |
| Hordeum vulgare | Sand | 6.5 | 1.7 | 8 | 13 | 8.4 | 7 | 4 | NOECrl | 44 |  | 57 |  | Rooney et al.(2004) |
|  | Woburn salt |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hordeum vulgare | Sand | 6.5 | 2.5 | 8 | 35 | 11.6 | 7 | 4 | NOECrl |  | <50 |  | <85 | Rooney et al.(2004) |
|  | Woburn cake |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Lycopersicon esculentum | Sandy loam Nottingham | 3.4 | 8.3 | 13 | 17 | 6.7 | 7 | 28 | NOECy(s) | 19 |  | 36 |  | Rooney et al.(2004) |
| Lycopersicon esculentum | Loamy sand Houthalen | 3.4 | 3.2 | 5 | 2 | 1.9 | 7 | 28 | NOECy(s) |  | <20 |  | <22 | Rooney et al.(2004) |
| Lycopersicon esculentum | Loamy sand Rhydtalog | 4.2 | 20.7 | 13 | 14 | 15.2 | 7 | 28 | NOECy(s) | 357 |  | 371 |  | Rooney et al.(2004) |
| Lycopersicon esculentum | Sandy clay loam Zegveld | 4.7 | 37.3 | 24 | 70 | 35.3 | 7 | 28 | NOECy(s) | 628 |  | 698 |  | Rooney et al.(2004) |
| Lycopersicon esculentum | Loamy sand Kovlinge I | 4.8 | 2.6 | 7 | 6 | 2.4 | 7 | 28 | NOECy(s) | 85 |  | 91 |  | Rooney et al.(2004) |
| Lycopersicon esculentum | Sandy clay | 4.8 | 0.7 | 38 | 31 | 11.2 | 7 | 28 | NOECy(s) | 43 |  | 74 |  | Rooney et al.(2004) |
|  | Souli I |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Lycopersicon esculentum | Sandy loam Kovlinge II | 5.1 | 3.8 | 9 | 8 | 4.7 | 7 | 28 | NOECy(s) | 197 |  | 205 |  | Rooney et al.(2004) |
| Lycopersicon esculentum | Sandy loam Montpellier | 5.2 | 1.2 | 9 | 5 | 2.5 | 7 | 28 | NOECy(s) |  | <41 |  | <46 | Rooney et al.(2004) |
| Lycopersicon esculentum | Clay | 5.4 | 1.4 | 51 | 21 | 22.6 | 7 | 28 | NOECy(s) | 176 |  | 197 |  | Rooney et al.(2004) |
|  | Aluminosa |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Lycopersicon esculentum | Sandy clay loam Woburn | 6.4 | 7 | 21 | 22 | 23.4 | 7 | 28 | NOECy(s) | 91 |  | 113 |  | Rooney et al.(2004) |
| Lycopersicon esculentum | Silt loam Ter Munck | 6.8 | 1.6 | 15 | 22 | 8.9 | 7 | 28 | NOECy(s) | 198 |  | 220 |  | Rooney et al.(2004) |
| Lycopersicon esculentum | Silty clay loam Vault de Lugny | 7.3 | 2.3 | 38 | 21 | 26.2 | 7 | 28 | NOECy(s) | 311 |  | 332 |  | Rooney et al.(2004) |
| Lycopersicon esculentum | Silty clay loam Rots | 7.4 | 2 | 27 | 14 | 20 | 7 | 28 | NOECy(s) | 660 |  | 674 |  | Rooney et al.(2004) |
| Lycopersicon esculentum | Clay Souli II | 7.4 | 4.2 | 46 | 34 | 36.3 | 7 | 28 | NOECy(s) | 628 |  | 662 |  | Rooney et al.(2004) |
| Lycopersicon esculentum | Silt loam Marknesse | 7.5 | 2 | 26 | 18 | 20.1 | 7 | 28 | NOECy(s) | 227 |  | 245 |  | Rooney et al.(2004) |
| Lycopersicon esculentum | Loam Barcelona | 7.5 | 2.4 | 21 | 88 | 14.3 | 7 | 28 | NOECy(s) | 315 |  | 403 |  | Rooney et al.(2004) |
| Lycopersicon esculentum | Clay Brecy | 7.5 | 2.4 | 50 | 31 | 23.5 | 7 | 28 | NOECy(s) | 100 |  | 131 |  | Rooney et al.(2004) |
| Lycopersicon esculentum | Loam Guadalajara | 7.5 | 0.6 | 25 | 7 | 16.9 | 7 | 28 | NOECy(s) | 313 |  | 320 |  | Rooney et al.(2004) |
| Lycopersicon esculentum | Sandy clay Hygum | 5.4 | 3.3 | 23 | 21 | 6.7 | 7 | 28 | NOECy(s) | 106 |  | 127 |  | Rooney et al.(2004) |
| Lycopersicon esculentum | Loamy sand Wageningen A | 4.3 | 0.2 | 9 | 19 | 1.2 | 7 | 28 | NOECy(s) |  | <70 |  | <89 | Rooney et al.(2004) |
| Lycopersicon esculentum | Loamy sand Wageningen D | 5 | 0.3 | 9 | 19 | 1.9 | 7 | 28 | NOECy(s) | 71 |  | 90 |  | Rooney et al.(2004) |
| Lycopersicon esculentum | loamy fine sand (thermic Typic Paleudult) | 4.8-5.5 | 2.7 | 9 | 10.7 | 7.9 | 14 | 42 | NOECy(s) |  | <175 |  | <185.7 | Rhoads et al (1989) |
|  |  | 5.9-6.5 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 6.5-6.6 | 2.7 | 9 | 10.7 | 9.2 | 14 | 42 |  | 175 |  | 185.7 |  |  |
|  |  | 7.1-7.4 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 2.7 | 9 | 10.7 | 9.6 | 14 | 42 |  | 350 |  | 360.7 |  |  |
|  |  |  | 2.7 | 9 | 10.7 | 10.5 | 14 | 42 |  | 350 |  | 360.7 |  |  |
| Oryza sativa | Oxysol | 6 | 1.5 |  | 1 |  |  | 28 | NOECy(s) | 66 |  | 67 |  | Fageria, 2001 |
| Phaseolus vulgaris | Oxysol | 6 | 1.5 |  | 1 |  |  | 21 | NOECy(s) | 40 |  | 41 |  | Fageria, 2001 |
| Zea mays | Oxysol | 6 | 1.5 |  | 1 |  |  | 21 | NOECy(s) | 56 |  | 57 |  | Fageria, 2001 |
| Glycine max | Oxysol | 6 | 1.5 |  | 1 |  |  | 28 | NOECy(s) | 13 |  | 14 |  | Fageria, 2001 |
| Triticum aestivum | Oxysol | 6 | 1.5 |  | 1 |  |  | 28 | NOECy(s) | 63.5 |  | 64.5 |  | Fageria, 2001 |
| Medicago sativa | Silty loam | 7.1 | 0.3 |  | 39 |  |  |  | NOECy(stp | 1,500 |  | 1,539 |  | Gonzales, 1991 |
| Medicago sativa | clayey | 7.8 | 1.8 |  | 37 |  |  |  | NOECy(stp | 1,253 |  | 1,290 |  | Gonzales, 1991 |
| Medicago sativa | Silty loam | 6.1 | 14.5 |  | 40 |  |  |  | NOECy(stp | 860 |  | 900 |  | Gonzales, 1991 |
| Medicago sativa | Sandy loam | 7.5 | 1.1 |  | 57 |  |  |  | NOECy(stp | 821 |  | 878 |  | Gonzales, 1991 |
| Medicago sativa | loamy | 5.3 | 12 |  | 4 |  |  |  | NOECy(stp | 816 |  | 820 |  | Gonzales, 1991 |
| Zea mays | Peat soil | 5.2 | 75 |  | 76 |  | 20 | 21 | NOECy(s) |  | <200 |  | <276 | McBride, 2001 |
| Senecio vulgaris | regolithic acidic | 4.1 | 1.9 | 17.5 | 158 | 10.1 | 28 | >105 | LC10m | 67 |  | 225 |  | Brun et al (2003) |
|  | regolithic acidic | 4.1 | 1.9 | 17.5 | 158 | 10.1 | 28 |  | EC10rep | 28 |  | 186 |  |  |
|  | regolithic acidic | 4.1 | 1.9 | 17.5 | 158 | 10.1 | 28 |  | EC10se | 181 |  | 339 |  |  |
| Poa annua | regolithic acidic | 4.1 | 1.9 | 17.5 | 158 | 10.1 | 28 | 210 | LC10m | 379 |  | 537 |  | Brun et al (2003) |
|  | regolithic acidic | 4.1 | 1.9 | 17.5 | 158 | 10.1 | 28 |  | EC10rep | 42 |  | 200 |  |  |
|  | regolithic acidic | 4.1 | 1.9 | 17.5 | 158 | 10.1 | 28 |  | EC10se | 158 |  | 316 |  |  |
| Andryala integrifolia | regolithic acidic | 4.1 | 1.9 | 17.5 | 158 | 10.1 | 28 | >175 | LC10m | 76 |  | 234 |  | Brun et al (2003) |
|  | regolithic acidic | 4.1 | 1.9 | 17.5 | 158 | 10.1 | 28 |  | EC10rep | / |  | / |  |  |
|  | regolithic acidic | 4.1 | 1.9 | 17.5 | 158 | 10.1 | 28 |  | EC10se | 78 |  | 236 |  |  |
| Hypochoeris radicata | regolithic acidic | 4.1 | 1.9 | 17.5 | 158 | 10.1 | 28 | 196 | LC10m | 192 |  | 350 |  | Brun et al (2003) |
|  | regolithic acidic | 4.1 | 1.9 | 17.5 | 158 | 10.1 | 28 |  | EC10rep | 192 |  | 350 |  |  |
|  | regolithic acidic | 4.1 | 1.9 | 17.5 | 158 | 10.1 | 28 |  | E10Cse | 181 |  | 339 |  |  |
| Cleopatra mandarin | fine sand (Typic Quartzipsamments) from topsoil (0-15 cm) of an orange grove | 5 |  |  | 10.7 |  | 90 | 106 | NOECy(r) |  | >200 |  | >210.7 | Mozaffari et al (1996) |
|  |  | 5 |  |  | 10.7 |  | 90 | 106 | NOECy(s) |  | >200 |  | >210.7 |  |
|  |  | 6 |  |  | 10.7 |  | 90 | 106 | NOECy(r) | 100 |  | 110.7 |  |  |
|  |  | 6 |  |  | 10.7 |  | 90 | 106 | NOECy(s) | 100 |  | 110.7 |  |  |
|  |  | 7 |  |  | 10.7 |  | 90 | 106 | NOECy(r) |  | >200 |  | >210.7 |  |
|  |  | 7 |  |  | 10.7 |  | 90 | 106 | NOECy(s) |  | >200 |  | >210.7 |  |
| Swingle citrumelo |  | 5 |  |  | 10.7 |  | 90 | 106 | NOECy(r) | 50 |  | 60.7 |  | Mozaffari et al (1996) |
|  |  | 5 |  |  | 10.7 |  | 90 | 106 | NOECy(s) |  | >200 |  | >210.7 |  |
|  |  | 6 |  |  | 10.7 |  | 90 | 106 | NOECy(r) | 100 |  | 110.7 |  |  |
|  |  | 6 |  |  | 10.7 |  | 90 | 106 | NOECy(s) |  | >200 |  | >210.7 |  |
|  |  | 7 |  |  | 10.7 |  | 90 | 106 | NOECy(r) | 100 |  | 110.7 |  |  |
|  |  | 7 |  |  | 10.7 |  | 90 | 106 | NOECy(s) |  | >200 |  | >210.7 |  |

Annex XVII: Overview of the NOEC values for soil micro-organisms

If the CEC was missing from a test, then it was estimated from % clay, pH and %organic matter using an experimentally derived regression model: CEC=(30+4.4 pH)*clay/100+(-34.66+29.72 pH)*OM/100; the clay is the % clay in the soil (Helling et al., 1964; regression based on CEC measured at various pH values on 60 different soils; CEC refers to the soil pH). Estimated background copper concentrations and CEC** are indicated in italics.

|  |  |  |  |  |  |  |  |  | Added NOEC |  | Total NOEC |  | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test substance | Process | Medium | pH | %OM | %clay | Cb | CEC | Duration | NOEC or EC10 | Unbounded NOEC | NOEC or EC10 | Unbounded NOEC |  |
|  |  |  |  |  |  | mg/kgdw | cmol/kg |  | mg/kgdw | mg/kgdw | mg/kgdw | mg/kgdw |  |
| CuCl2 | phosphatase activity | sandy loam | 5.1 | 5.7 | 9 | 6.5 |  | 540 | 438 |  | 445 |  | Doelman & Haanstra, 1989 |
| CuCl2 | phosphatase activity | silty loam | 7.4 | 2.4 | 19 | 22 |  | 540 | 170 |  | 192 |  | Doelman & Haanstra, 1989 |
| CuCl2 | phosphatase activity | clay | 6.8 | 3.2 | 60 | 52 |  | 540 | 960 |  | 1,012 |  | Doelman & Haanstra, 1989 |
| CuCl2 | phosphatase activity | sandy peat | 4.3 | 12.8 | 5 | 5.5 |  | 540 | 58 |  | 64 |  | Doelman & Haanstra, 1989 |
| CuCl2 | urease activity | silty loam | 7.4 | 2.4 | 19 | 22 |  | 540 | 340 |  | 362 |  | Doelman & Haanstra, 1986 |
| CuCl2 | urease activity | clay | 6.8 | 3.2 | 60 | 52 |  | 540 | 520 |  | 572 |  | Doelman & Haanstra, 1986 |
| CuCl2 | urease activity | sandy peat | 4.3 | 12.8 | 5 | 5.5 |  | 540 | 210 |  | 216 |  | Doelman & Haanstra, 1986 |
| CuCl2 | respiration | sand | 7.7 | 1.6 | 2 | 4 | 4.4 | 490 | 150 |  | 154 |  | Doelman & Haanstra, 1984 |
| CuCl2 | respiration | sandy loam | 5.1 | 5.7 | 9 | 6.5 |  | 301 |  | <150 |  | <156.5 | Doelman & Haanstra, 1984 |
| CuCl2 | respiration | silty loam | 7.4 | 2.4 | 19 | 22 |  | 630 |  | ≥ 8,000 |  | ≥ 8,022 | Doelman & Haanstra, 1984 |
| CuCl2 | respiration | clay | 6.8 | 3.2 | 60 | 52 |  | 560 |  | ≥ 8,000 |  | ≥ 8,052 | Doelman & Haanstra, 1984 |
| CuCl2 | respiration | sandy peat | 4.3 | 12.8 | 5 | 5.5 | 14.5 | 574 | 400 |  | 406 |  | Doelman & Haanstra, 1984 |
| CuCl2 | arylsulphatase activity | sandy loam | 5.1 | 5.7 | 9 | 6.5 |  | 540 | 347 |  | 354 |  | Haanstra & Doelman, 1991 |
| CuCl2 | arylsulphatase activity | silty loam | 7.4 | 2.4 | 19 | 22 |  | 540 | 289 |  | 311 |  | Haanstra & Doelman, 1991 |
| CuCl2 | arylsulphatase activity | clay | 6.8 | 3.2 | 60 | 52 |  | 540 | 2,669 |  | 2,721 |  | Haanstra & Doelman, 1991 |
| CuCl2 | arylsulphatase activity | sandy peat | 4.3 | 12.8 | 5 | 5.5 |  | 540 | 3,323 |  | 3,329 |  | Haanstra & Doelman, 1991 |
| CuSO4 | N-mineralisation | sandy loam | 5.9 | 3.4 | 16 | 33 | 13.8 | 21 | 100 |  | 133 |  | Quraishi & Cornfield, 1973 |
| CuSO4 | N-mineralisation | sandy loam | 7.3 | 3.4 | 16 | 33 | 16.3 | 21 |  | <100 |  | <133 | Quraishi & Cornfield, 1973 |
| CuSO4 | nitrification | sandy loam | 5.9 | 3.4 | 16 | 33 | 13.8 | 21 | 100 |  | 133 |  | Quraishi & Cornfield, 1973 |
| CuSO4 | nitrification | sandy loam | 7.3 | 3.4 | 16 | 33 | 16.3 | 21 | 100 |  | 133 |  | Quraishi & Cornfield, 1973 |
| CuSO4 | ammonification (aeroob) | sandy loam | 7.1 | 3.4 | 17 | 33 | 16.5 | 21 | 1,000 |  | 1,033 |  | Premi & Cornfield, 1969 |
| CuSO4 | nitrification | sandy loam | 7.1 | 3.4 | 17 | 33 | 16.5 | 21 | 1,000 |  | 1,033 |  | Premi & Cornfield, 1969 |
| CuSO4 | ammonification (anaeroob) | sandy loam | 7.1 | 3.4 | 17 | 33 | 16.5 | 21 |  | ≥ 10,000 |  | ≥ 10,033 | Premi & Cornfield, 1969 |
| CuCl2 | glutamic acid decomposition | sand | 7.7 | 1.6 | 2 | 4 |  | 540 |  | <55 |  | <59 | Haanstra & Doelman, 1984 |
| CuCl2 | glutamic acid decomposition | silty loam | 7.4 | 2.4 | 19 | 22 | 16.5 | 540 | 55 |  | 77 |  | Haanstra & Doelman, 1984 |
| CuCl2 | glutamic acid decomposition | clay | 6.8 | 3.2 | 60 | 52 | 41.6 | 540 | 55 |  | 107 |  | Haanstra & Doelman, 1984 |
| CuCl2 | glutamic acid decomposition | sandy peat | 4.3 | 12.8 | 5 | 5.5 | 14.5 | 540 | 400 |  | 406 |  | Haanstra & Doelman, 1984 |
| CuSO4 | amidase activity | clay | 7.5 |  | 17.7 | 10.7 |  | 84 | 200 |  | 210.7 |  | Hemida et al., 1997 |
| CuSO4 | amidase activity | sand | 7.4 |  | 2.2 | 10.7 |  | 84 |  | ≥ 2,000 |  | ≥ 2,010.7 | Hemida et al., 1997 |
| CuSO4 | N-mineralisation | silty loam | 6.9 | 2.2 |  | 9.4 | 27.3 | 84 |  | <100 |  | <109 | Chang & Broadbent, 1982 |
| CuSO4 | respiration | silty loam | 6.9 | 2.3 |  | 9.4 | 27.3 | 90 |  | <40.6 |  | <50 | Chang and Broadbent (1981) |
| CuSO4 | ethylene production | Hanford soil | 7.15 | 8.16 | 18 | 10.7 |  | 7 | 10 |  | 20.7 |  | Arshad & Frankenberger, 1991 |
| CuSO4 | microbial biomass | Lösslehm | 6.1 | 2.7 |  | 50 |  | 30 | 100 |  | 150 |  | Beck, 1981 |
| CuSO4 | microbial biomass | Auengleye | 7.5 | 4.9 |  | 80 |  | 30 | 100 |  | 180 |  | Beck, 1981 |
| CuSO4 | microbial biomass | Aueboden | 7.2 | 3.3 |  | 50 |  | 30 | 100 |  | 150 |  | Beck, 1981 |
| Cu-salt | respiration | Grassland soil | 6.3 | 10.1 | 29.8 | 32 | 61.4 | 49 |  | >768 |  | >800 | Khan and Scullion, 2002 |
|  | Microbial biomass C | Grassland soil | 6.3 | 10.1 | 29.8 | 32 | 61.4 | 49 | 118 |  | 150 |  | Khan and Scullion, 2002 |
|  | Microbial biomass N | Grassland soil | 6.3 | 10.1 | 29.8 | 32 | 61.4 | 49 | 468 |  | 500 |  | Khan and Scullion, 2002 |
|  | N-mineralisation | Grassland soil | 6.3 | 10.1 | 29.8 | 32 | 61.4 | 49 | 268 |  | 300 |  | Khan and Scullion, 2002 |
| Cu(NO3)2 | denitrification | silty loam | 6.75 | 3.1 | 28.1 | 10.7 | 22.1 | 21 | 100 |  | 110.7 |  | Bollag & Barabasz, 1979 |
| CuSO4 | ATP content | forest soil | 4.5 | 80 |  | 10.7 |  | 182 | 337 |  | 347.7 |  | Frostegard et al., 1993 |
| CuSO4 | ATP content | sandy loam | 7.8 | 4.4 |  | 10.7 |  | 182 | 197 |  | 207.7 |  | Frostegard et al., 1993 |
| CuSO4 | respiration | forest soil | 4.5 | 80 |  | 10.7 |  | 182 | 763 |  | 773.7 |  | Frostegard et al., 1993 |
| CuSO4 | dehydrogenase activity | alluvial soil | 7.1 | 1.9 |  | 3 |  | 182 | 10 |  | 13 |  | Maliszewska et al., 1985 |
|  |  | Sandy soil | 6.9 | 3 |  | 0.9 |  | 182 |  | <10 |  | <10.9 | Maliszewska et al., 1985 |
| CuCl2 | respiration | sandy loam | 5.2 | 2.4 | 8 | 10.7 |  | 28 |  | <50 |  | <60.7 | Saviozzi et al., 1997 |
| CuCl2 | respiration | forest soil: sandy clay loam | 7 |  |  | 10.7 |  | 20 | 50 |  | 60.7 |  | Skujins et al., 1986 |
| CuCl2 | urease activity | forest soil: sandy clay loam | 7 |  |  | 10.7 |  | 20 | 200 |  | 210.7 |  | Skujins et al., 1986 |
| Cu(NO3)2 | phosphatase activity | Loam | 6.1 | 20.4 | 32 | 10.7 |  | 7 |  | <635 |  | <640.7 | Speir et al., 1999 |
| Cu(NO3)2 | phosphatase activity | Silt loam | 6.3 | 13.8 | 2.5 | 10.7 |  | 7 |  | <635 |  | <640.7 | Speir et al., 1999 |
| Cu(NO3)2 | sulphatase activity | loam | 6.1 | 20.4 | 32 | 10.7 |  | 7 |  | <635 |  | <640.7 | Speir et al., 1999 |
| Cu(NO3)2 | sulphatase activity | silt loam | 6.3 | 13.8 | 2.5 | 10.7 |  | 7 |  | <635 |  | <640.7 | Speir et al., 1999 |
| Cu(NO3)2 | substrate induced respiration | loam | 6.1 | 20.4 | 32 | 10.7 | 48.5 | 7 | 635 |  | 645.7 |  | Speir et al., 1999 |
| Cu(NO3)2 | substrate induced respiration | silt loam | 6.3 | 13.8 | 2.5 | 10.7 | 22.7 | 7 | 635 |  | 645.7 |  | Speir et al., 1999 |
| Cu(NO3)2 | substrate induced respiration | Loamy sand | 5.8 | 7.8 | 3 | 10.7 |  | 7 |  | <635 |  | <640.7 | Speir et al., 1999 |
| CuCl2 | Nitrification | Sandy loam Nottingham | 3.4 | 8.3 | 13 | 17 | 6.7 | 28 | 200 |  | 217 |  | Smolders and Oorts, 2004 |
| CuCl2 | Nitrification | Sandy clay loam Zegveld | 4.7 | 37.3 | 24 | 70 | 35.3 | 4 | 1200 |  | 1270 |  | Smolders and Oorts, 2004 |
| CuCl2 | Nitrification | Loamy sand Kovlinge I | 4.8 | 2.6 | 7 | 6 | 2.4 | 28 | 25 |  | 31 |  | Smolders and Oorts, 2004 |
| CuCl2 | nitrification | Sandy clay Souli I | 4.8 | 0.7 | 38 | 31 | 11.2 | 28 | 25 |  | 56 |  | Smolders and Oorts, 2004 |
| CuCl2 | nitrification | Sandy loam Kovlinge II | 5.1 | 3.8 | 9 | 8 | 4.7 | 14 | 50 |  | 58 |  | Smolders and Oorts, 2004 |
| CuCl2 | nitrification | Clay Aluminosa | 5.4 | 1.4 | 51 | 21 | 22.6 | 28 | 100 |  | 121 |  | Smolders and Oorts, 2004 |
| CuCl2 | nitrification | Sandy clay loam Woburn | 6.4 | 7 | 21 | 22 | 23.4 | 4 | 300 |  | 322 |  | Smolders and Oorts, 2004 |
| CuCl2 | nitrification | Silt loam Ter Munck | 6.8 | 1.6 | 15 | 22 | 8.9 | 7 | 200 |  | 222 |  | Smolders and Oorts, 2004 |
| CuCl2 | nitrification | Silty clay loam Vault de Lugny | 7.3 | 2.3 | 38 | 21 | 26.2 | 4 | 800 |  | 821 |  | Smolders and Oorts, 2004 |
| CuCl2 | nitrification | Silty clay loam Rots | 7.4 | 2 | 27 | 14 | 20 | 7 | 400 |  | 414 |  | Smolders and Oorts, 2004 |
| CuCl2 | nitrification | Clay Souli II | 7.4 | 4.2 | 46 | 34 | 36.3 | 14 | 600 |  | 634 |  | Smolders and Oorts, 2004 |
| CuCl2 | nitrification | Silt loam Marknesse | 7.5 | 2 | 26 | 18 | 20.1 | 7 | 800 |  | 818 |  | Smolders and Oorts, 2004 |
| CuCl2 | nitrification | Loam Barcelona | 7.5 | 2.4 | 21 | 88 | 14.3 | 11 | 300 |  | 388 |  | Smolders and Oorts, 2004 |
| CuCl2 | nitrification | Clay Brécy | 7.5 | 2.4 | 50 | 31 | 23.5 | 4 | 400 |  | 431 |  | Smolders and Oorts, 2004 |
| CuCl2 | nitrification | Loam Guadalajara | 7.5 | 0.6 | 25 | 7 | 16.9 | 7 | 52 |  | 59 |  | Smolders and Oorts, 2004 |
| CuCl2 | nitrification | Sandy clay | 5.4 | 3.3 | 23 | 21 | 6.7 | 14 | 127 |  | 148 |  | Smolders and Oorts, 2004 |
|  |  | Hygum |  |  |  |  |  |  |  |  |  |  |  |
| CuCl2 | nitrification | Loamy sand Wageningen A | 4.3 | 2.2 | 9 | 19 | 1.2 | 18 |  | <12 |  | <29 | Smolders and Oorts, 2004 |
| CuCl2 | nitrification | Loamy sand Wageningen D | 5 | 2.3 | 9 | 19 | 1.9 | 18 | 65 |  | 84 |  | Smolders and Oorts, 2004 |
| CuCl2 | nitrification | Sand Woburn salt | 6.5 | 0.2 | 8 | 13 | 8.4 | 14 | 100 |  | 113 |  | Smolders and Oorts, 2004 |
| CuCl2 | nitrification | Sand Woburn cake | 6.5 | 0.3 | 8 | 35 | 11.6 | 14 | 50 |  | 85 |  | Smolders and Oorts, 2004 |
| CuCl2 | Glucose respiration | Loamy sand Gudow | 3 | 8.2 | 7 | 2 | 5.8 | 4 | 1200 |  | 1202 |  | Smolders and Oorts, 2004 |
| CuCl2 | Glucose respiration | Sandy loam Nottingham | 3.4 | 8.3 | 13 | 17 | 6.7 | 4 | 150 |  | 167 |  | Smolders and Oorts, 2004 |
| CuCl2 | Glucose respiration | Loamy sand Houthalen | 3.4 | 3 | 5 | 2 | 1.9 | 4 | 50 |  | 52 |  | Smolders and Oorts, 2004 |
| CuCl2 | Glucose respiration | Loamy sand Rhydtalog | 4.2 | 20.7 | 13 | 14 | 15.2 | 4 | 600 |  | 614 |  | Smolders and Oorts, 2004 |
| CuCl2 | Glucose respiration | Sandy clay loam Zegveld | 4.7 | 37.3 | 24 | 70 | 35.3 | 4 | 100 |  | 170 |  | Smolders and Oorts, 2004 |
| CuCl2 | Glucose respiration | Loamy sand Kovlinge I | 4.8 | 2.6 | 7 | 6 | 2.4 | 4 | 25 |  | 31 |  | Smolders and Oorts, 2004 |
| CuCl2 | Glucose respiration | Sandy clay Souli I | 4.8 | 0.7 | 38 | 31 | 11.2 | 4 | 100 |  | 131 |  | Smolders and Oorts, 2004 |
| CuCl2 | Glucose respiration | Sandy loam Kovlinge II | 5.1 | 3.8 | 9 | 8 | 4.7 | 4 | 50 |  | 58 |  | Smolders and Oorts, 2004 |
| CuCl2 | Glucose respiration | Loamy sand Montpellier | 5.2 | 1.2 | 9 | 5 | 2.5 | 4 | 25 |  | 30 |  | Smolders and Oorts, 2004 |
| CuCl2 | Glucose respiration | clay Aluminosa | 5.4 | 1.4 | 51 | 21 | 22.6 | 4 | 400 |  | 421 |  | Smolders and Oorts, 2004 |
| CuCl2 | Glucose respiration | Sandy clay loam Woburn | 6.4 | 7 | 21 | 22 | 23.4 | 4 | 300 |  | 321 |  | Smolders and Oorts, 2004 |
| CuCl2 | Glucose respiration | Silt loam Ter Munck | 6.8 | 1.6 | 15 | 22 | 8.9 | 4 | 50 |  | 72 |  | Smolders and Oorts, 2004 |
| CuCl2 | Glucose respiration | Silt clay loam Vault de Lugny | 7.3 | 2.3 | 38 | 21 | 26.2 | 4 | 102 |  | 123 |  | Smolders and Oorts, 2004 |
| CuCl2 | Glucose respiration | Silt clay loam Rots | 7.4 | 2 | 27 | 14 | 20 | 4 | 200 |  | 214 |  | Smolders and Oorts, 2004 |
| CuCl2 | Glucose respiration | Clay Souli II | 7.4 | 4.2 | 46 | 34 | 36.3 | 4 | 89 |  | 123 |  | Smolders and Oorts, 2004 |
| CuCl2 | Glucose respiration | Silt loam Marknesse | 7.5 | 2 | 26 | 18 | 20.1 | 4 | 23 |  | 41 |  | Smolders and Oorts, 2004 |
| CuCl2 | Glucose respiration | loam Barcelona | 7.5 | 2.4 | 21 | 88 | 14.3 | 4 | 300 |  | 388 |  | Smolders and Oorts, 2004 |
| CuCl2 | Glucose respiration | clay Brécy | 7.5 | 2.4 | 50 | 31 | 23.5 | 4 | 200 |  | 231 |  | Smolders and Oorts, 2004 |
| CuCl2 | Glucose respiration | loam Guadalajara | 7.5 | 0.6 | 25 | 7 | 16.9 | 4 | 50 |  | 57 |  | Smolders and Oorts, 2004 |
| CuCl2 | Glucose respiration | Sandy clay Hygum | 5.4 | 3.3 | 23 | 21 | 6.7 | 4 | 170 |  | 191 |  | Smolders and Oorts, 2004 |
| CuCl2 | Glucose respiration | Loamy sand Wageningen A | 4.3 | 2.2 | 9 | 19 | 1.2 | 4 | 12 |  | 31 |  | Smolders and Oorts, 2004 |
| CuCl2 | Glucose respiration | Loamy sand Wageningen D | 5 | 2.3 | 9 | 19 | 1.9 | 4 | 25 |  | 44 |  | Smolders and Oorts, 2004 |
| CuCl2 | Glucose respiration | Sand Woburn salt | 6.5 | 0.2 | 8 | 13 | 8.4 | 4 | 100 |  | 113 |  | Smolders and Oorts, 2004 |
| CuCl2 | Glucose respiration | Sand Woburn cake | 6.5 | 0.3 | 8 | 35 | 11.6 | 4 | 27 |  | 62 |  | Smolders and Oorts, 2004 |
| CuCl2 | Maize respiration | Loamy sand Gudow | 3 | 8.2 | 7 | 2 | 5.8 | 28 | 2400 |  | 2402 |  | Smolders and Oorts, 2004 |
| CuCl2 | Maize respiration | Sandy loam Nottingham | 3.4 | 8.3 | 13 | 17 | 6.7 | 28 | 1200 |  | 1217 |  | Smolders and Oorts, 2004 |
| CuCl2 | Maize respiration | Loamy sand Rhydtalog | 4.2 | 20.7 | 13 | 14 | 15.2 | 28 | 1200 |  | 1214 |  | Smolders and Oorts, 2004 |
| CuCl2 | Maize respiration | Sandy clay loam Zegveld | 4.7 | 37.3 | 24 | 70 | 35.3 | 28 | 300 |  | 370 |  | Smolders and Oorts, 2004 |
| CuCl2 | Maize respiration | Loamy sand Kovlinge I | 4.8 | 2.6 | 7 | 6 | 2.4 | 28 | 50 |  | 56 |  | Smolders and Oorts, 2004 |
| CuCl2 | Maize respiration | Sandy clay Souli II | 4.8 | 0.7 | 38 | 31 | 11.2 | 28 | 200 |  | 231 |  | Smolders and Oorts, 2004 |
| CuCl2 | Maize respiration | Sandy loam Kovlinge II | 5.1 | 3.8 | 9 | 8 | 4.7 | 28 | 100 |  | 108 |  | Smolders and Oorts, 2004 |
| CuCl2 | Maize respiration | Loamy sand Montpellier | 5.2 | 1.2 | 9 | 5 | 2.5 | 28 | 50 |  | 55 |  | Smolders and Oorts, 2004 |
| CuCl2 | Maize respiration | Clay Aluminosa | 5.4 | 1.4 | 51 | 21 | 22.6 | 28 | 400 |  | 421 |  | Smolders and Oorts, 2004 |
| CuCl2 | Maize respiration | Sandy clay loam Woburn | 6.4 | 7 | 21 | 22 | 23.4 | 28 | 150 |  | 172 |  | Smolders and Oorts, 2004 |
| CuCl2 | Maize respiration | Silt loam Ter Munck | 6.8 | 1.6 | 15 | 22 | 8.9 | 28 | 50 |  | 72 |  | Smolders and Oorts, 2004 |
| CuCl2 | Maize respiration | Silty clay loam Vault de Lugny | 7.3 | 2.3 | 38 | 21 | 26.2 | 28 | 400 |  | 421 |  | Smolders and Oorts, 2004 |
| CuCl2 | Maize respiration | clay Souli II | 7.4 | 4.2 | 46 | 34 | 36.3 | 28 | 600 |  | 634 |  | Smolders and Oorts, 2004 |
| CuCl2 | Maize respiration | Silt loam Marknesse | 7.5 | 2 | 26 | 18 | 20.1 | 28 | 150 |  | 168 |  | Smolders and Oorts, 2004 |
| CuCl2 | Maize respiration | Loam Barcelona | 7.5 | 2.4 | 21 | 88 | 14.3 | 28 | 150 |  | 238 |  | Smolders and Oorts, 2004 |
| CuCl2 | Maize respiration | Sandy clay Hygum | 5.4 | 3.3 | 23 | 21 | 6.7 | 28 |  | >804 |  | >825 | Smolders and Oorts, 2004 |
| CuCl2 | Maize respiration | Loamy sand Wageningen A | 4.3 | 2.2 | 9 | 19 | 1.2 | 28 | 51 |  | 70 |  | Smolders and Oorts, 2004 |
| CuCl2 | Maize respiration | Loamy and Wageningen D | 5 | 2.3 | 9 | 19 | 1.9 | 28 | 83 |  | 102 |  | Smolders and Oorts, 2004 |
| CuCl2 | Maize respiration | Sand Woburn salt | 6.5 | 0.2 | 8 | 13 | 8.4 | 28 |  | >200 |  | >213 | Smolders and Oorts, 2004 |
| CuCl2 | Maize respiration | Sand Woburn cake | 6.5 | 0.3 | 8 | 35 | 11.6 | 28 | 100 |  | 135 |  | Smolders and Oorts, 2004 |

Includes copper flakes, coated with aliphatic acid, because the toxicity data on this material is used to read across to copper powders and massives. The same CAS number is thus used to cover three different forms of copper in this assessment (massive, powder and coated flakes).

Aliphatic acids are added for the production of coated copper flakes, to stabilise the copper flake in small particle sizes with higher surface area (needed for specific niche applications – biocides and pigments).

The same CAS number is used to cover three forms of  zero valence copper materials : massive, powders and coated copper flakes.

NOEAEC : No Observed Ecological Adverse Effects Concentration

LOEAEC : Lowest Observed Ecological Adverse Effects Concentration

http://echa.europa.eu/copper-voluntary-risk-assessment-reports

1 Copper flakes, coated with aliphatic acids are also included in the dossier

4 https://echa.europa.eu/copper-voluntary-risk-assessment-reports

The same CAS number is used to cover three forms of  zero valence copper materials : massive, powders and coated copper flakes.

The aliphatic acids are added to stabilise the copper flake in small particle sizes with higher surface area (needed for specific niche applications – biocides and pigments).

Primary copper : copper produced from primary raw materials (copper ores).

Secondary copper : copper produced from secondary  raw materials (scrap)

Black copper is a metallic  intermediate formed during secondary copper production. Black copper is produced by melting and/or processing of metallic scrap and/or oxidic bearing materials (slag, oxides, ashes).

More information is available at http://echa.europa.eu/web/guest/copper-voluntary-risk-assessment-reports -   environmental exposure part 1 and appendix  (copper emission inventory)

The transformation-dissolution data are further discussed during the discussions on harmonized classification of biocidal copper forms and compounds (2014—2017 and onwards) in the EU.

Provided by the EU Copper Task Force to the European Copper Institute under a specific secrecy agreement.

Provided by the EU Copper Task Force to the European Copper Institute under a specific secrecy agreement.

By the Wood Preservative Copper Task Force, EU Antifouling Copper Task Force, EU Copper Task Force and Landis International to the European Copper Institute under a specific secrecy agreement.

Particle size distribution of airborne fraction of five different copper substances were determined using the Heubach  method.  Summary report provided by EBRC Consulting GmbH under specific secrecy agreement.

Metal fume fever (MFF) is a transient illness which appears to develop 4-12 hours after occupational exposure to metal fume.  MFF presents as an influenza-like illness with cough and dyspnoea followed by fever, sweating and shivering.  Other accompanying clinical signs and symptoms are nausea, headache, weakness, a sweet metallic taste, and muscle and joint pain.

By the wood preservative copper task force, EU antifouling copper task force and EU copper task force to the European Copper Institute under a specific secrecy agreement.

By the Wood Preservative Copper Task Force, EU Antifouling Copper Task Force and EU Copper Task Force to the European Copper Institute under a specific secrecy agreement.

By the Wood Preservative Copper Task Force, EU Antifouling Copper Task Force and EU Copper Task Force to the European Copper Institute under a specific secrecy agreement.

By the Wood Preservative Copper Task Force, EU Antifouling Copper Task Force and EU Copper Task Force to the European Copper Institute under a specific secrecy agreement.

a solution of copper sulphate neutralised with hydrated lime.  According to available information, CuSO4 in ‘Bordeaux mixture’ reacts during mixing prior to application to form the stable hydroxy-sulphate hydrates brochantite, posnjakite, devellite and anterlite (Leeming, personal communication)

http://europa.eu.int/comm/food/fs/sc/scf/index_en.html

Reported in the copper monochloride OECD SIAR and SIAP

The database is still under further discussion and review in view of the EU harmonized hazard classification discussions of copper and copper compounds in EU (2014-2017).

Data “higher than”, “lower than”, or with a pH not within the 5-5-8.5 range were not included in this overview table

The applicability of the species mean/lowest value for P. promelas at pH 6 to natural surface water waters is questioned.  The lowest value of 4.4 μg/l Cu is extracted from Erickson et al. (1996). This test was performed in a flow-through with a very short retention time (± 45 min.) using a diluted reconstituted medium (prepared from Lake Superior water through reverse osmosis) with a low hardness (22 mg/l CaCO3) and DOC concentration (reverse osmosis). Moreover, a higher acute sensitivity of P. promelas compared to O. mykiss towards copper is not expected when considering the data at pH 6.5-7.5 and 7.5-8.5.

HC5,50%  is the 5th percentile of the SSD, and this value represents a concentration level is protective for 95% of the species that are included in the distribution

unbounded value

http://echa.europa.eu/copper-voluntary-risk-assessment-reports