SIDS Dossier

OECD HPV Chemical Programme, SIDS Dossier, approved at SIAM 32 (19/04/2011)

| Printing Date | 2013-03-05 13:24:37 EST |
| --- | --- |

Restriction of specific regulatory purposes

EU: REACH, other:

Confidentiality

| Name | 13952-84-6_master_sec-butylamine (IUC4 DSN 410) |
| --- | --- |
| Legal entity owner | BASF SE |

Substance: 13952-84-6_master_sec-butylamine (IUC4 DSN 410)

| UUID |  | IUC4-a5f279aa-200d-3d80-936b-42241024dfcf |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | Collette / Epona Associates, LLC / Willington / United States |
| Date |  | 2012-07-20 14:19:16 EDT |
| Remarks |  | Renamed endpoint study record 5.4.3 Distribution modelling |

0 Related Information

0.1 Templates

0.2 Categories

0.3 Mixtures

1 General Information

1.1 Identification

Substance identification

| Chemical name | 13952-84-6_master_sec-butylamine (IUC4 DSN 410) |
| --- | --- |
| EU: REACH |  |
| Legal entity | BASF SE |
| EU: REACH |  |

Role in the supply chain

EU: REACH

Reference substance

| sec-butylamine |  |
| --- | --- |
| EC number; EC name. 237-732-7; sec-butylamine. CAS number; CAS name. IUPAC name. |  |

Type of substance

| Composition | other: Existing Chemical |
| --- | --- |
| Origin | organic |

Other names

| Flags | Name Type | Name | Country | Remarks |
| --- | --- | --- | --- | --- |
| EU: REACH | Trade name | sec-Butylamin |  |  |
| EU: REACH | Trade name | sec-Butylamine (6CI, 8CI) |  |  |
| EU: REACH | Trade name | 2-Butylamine |  |  |
| EU: REACH | Trade name | 1-Methylpropanamine |  |  |
| EU: REACH | Trade name | Butafume |  |  |
| EU: REACH | Trade name | 2-AB |  |  |
| EU: REACH | Trade name | 1-Methylpropylamine |  |  |
| EU: REACH | Trade name | Tutane |  |  |
| EU: REACH | Trade name | 2-Aminobutane |  |  |
| EU: REACH | Trade name | 2-Butanamine (9CI) |  |  |

Contact persons

| Person flags | EU: REACH |
| --- | --- |

1.2 Composition

1.3 Identifiers

1.4 Analytical information

1.5 Joint submission

1.6 Sponsors

1.7 Suppliers

1.8 Recipients

1.9 Product and process oriented research and development

2 Classification & Labelling and PBT assessment

2.1 GHS

2.2 DSD - DPD

Classification and Labelling according 67/548/EEC (DSD)

EU: REACH. General information. Remarks; RE-Freetext: BASF AG, Safety data sheet SEC.-BUTYLAMINE, 10.02.2005 RE-Freetext: Commission Directive 2004/73/EC, 29 April 2004 (29th adaption to the technical progress of 67/548/EEC). Classification. Classification; Reason for no classification. Explosiveness; data lacking. Oxidising properties; data lacking. Flammability; data lacking. Thermal stability; data lacking. Acute toxicity; data lacking. Acute toxicity - irreversible damage after single exposure; data lacking. Repeated dose toxicity; data lacking. Irritation / Corrosion; data lacking. Sensitisation; data lacking. Carcinogenicity; data lacking. Mutagenicity - Genetic Toxicity; data lacking. Toxicity to reproduction - fertility; data lacking. Toxicity to reproduction - development; data lacking. Toxicity to reproduction - breastfed babies; data lacking. Environment; N; R50 Dangerous for the environment; Very toxic to aquatic organisms.....

EU: REACH. General information. Name; 67/548/EEC annex 1. Status; 67/548/EEC annex 1. Remarks; Specific limits: no RE-Freetext: BASF AG, Safety data sheet SEC.-BUTYLAMINE, 10.02.2005 RE-Freetext: Commission Directive 2004/73/EC, 29 April 2004 (29th adaption to the technical progress of 67/548/EEC) RE-Freetext: BASF AG, Safety data sheet SEC.-BUTYLAMINE, 10.02.2005 RE-Freetext: Commission Directive 2004/73/EC, 29 April 2004 (29th adaption to the technical progress of 67/548/EEC). Classification. Classification; Reason for no classification. Explosiveness; data lacking. Oxidising properties; data lacking. Flammability; F; R11 Highly flammable.. Thermal stability; data lacking. Acute toxicity; data lacking. Acute toxicity - irreversible damage after single exposure; data lacking. Repeated dose toxicity; data lacking. Irritation / Corrosion; data lacking. Sensitisation; data lacking. Carcinogenicity; data lacking. Mutagenicity - Genetic Toxicity; data lacking. Toxicity to reproduction - fertility; data lacking. Toxicity to reproduction - development; data lacking. Toxicity to reproduction - breastfed babies; data lacking. Environment; data lacking... Labelling. Indication of danger. F - highly flammable. C - corrosive. N - dangerous for the environment. Risk phrases. R11 - Highly flammable. R20/22 - Harmful by inhalation and if swallowed. R35 - Causes severe burns. R50 - Very toxic to aquatic organisms. Safety phrases. Code; S9 - keep container in a well-ventilated place.. Code; S16 - keep away from sources of ignition - No smoking.. Code; S26 - in case of contact with eyes, rinse immediately with plenty of water and seek medical advice.. Code; S28 - After contact with skin, wash immediately with plenty of ... (to be specified by the manufacturer).. Code; S36/37/39 - wear suitable protective clothing, gloves and eye/face protection.. Code; S45 - in case of accident or if you feel unwell, seek medical advice immediately (show the label where possible).. Code; S61 - avoid release to the environment. refer to special instructions/safety data sheets. Additional text; water... Notes. Note C. Some organic substances may be marketed either in a specific isomeric form or as a mixture of several isomers. In Annex I, a general designation of the following type is sometimes used: "xylenol". In this case the manufacturer or any other person who markets such a substance must state on the label whether the substance is a specific isomer (a) or a mixture of isomers (b). Example: (a) 2,4-dimethylphenol (b) xylenol (mixture of isomers)....

EU: REACH. General information. Remarks; RE-Freetext: BASF AG, Safety data sheet SEC.-BUTYLAMINE, 10.02.2005 RE-Freetext: Commission Directive 2004/73/EC, 29 April 2004 (29th adaption to the technical progress of 67/548/EEC). Classification. Classification; Reason for no classification. Explosiveness; data lacking. Oxidising properties; data lacking. Flammability; data lacking. Thermal stability; data lacking. Acute toxicity; data lacking. Acute toxicity - irreversible damage after single exposure; data lacking. Repeated dose toxicity; data lacking. Irritation / Corrosion; C; R35 Causes severe burns... Sensitisation; data lacking. Carcinogenicity; data lacking. Mutagenicity - Genetic Toxicity; data lacking. Toxicity to reproduction - fertility; data lacking. Toxicity to reproduction - development; data lacking. Toxicity to reproduction - breastfed babies; data lacking. Environment; data lacking...

EU: REACH. General information. Remarks; RE-Freetext: BASF AG, Safety data sheet SEC.-BUTYLAMINE, 10.02.2005 RE-Freetext: Commission Directive 2004/73/EC, 29 April 2004 (29th adaption to the technical progress of 67/548/EEC). Classification. Classification; Reason for no classification. Explosiveness; data lacking. Oxidising properties; data lacking. Flammability; data lacking. Thermal stability; data lacking. Acute toxicity; Xn; R20/22 Harmful by inhalation and if swallowed... Acute toxicity - irreversible damage after single exposure; data lacking. Repeated dose toxicity; data lacking. Irritation / Corrosion; data lacking. Sensitisation; data lacking. Carcinogenicity; data lacking. Mutagenicity - Genetic Toxicity; data lacking. Toxicity to reproduction - fertility; data lacking. Toxicity to reproduction - development; data lacking. Toxicity to reproduction - breastfed babies; data lacking. Environment; data lacking...

3 Manufacture, use and exposure

3.1 Technological process

Technological process

3.2 Estimated quantities

3.3 Sites

3.4 Information on mixtures

3.5 Life Cycle description

3.6 Uses advised against

3.7 Exposure Scenarios, exposure and risk assessment

3.7.2 Environmental assessment for aggregated sources

3.7.3 Generic exposure potential

3.8 Biocidal information

3.10 Application for authorisation of uses

4 Physical and chemical properties

4.1 Appearance/physical state/colour

Endpoint study record: Appearance/physical state/colour, IUC4#2/Ch.1.1.1

| UUID |  | IUC5-609452fe-4729-4b86-9e4e-b40a5635572b |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-13 10:50:27 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BASF AG, Safety data sheet SEC.-BUTYLAMINE, 10.02.2005 |  |  |  |  |  |  |  |  |

Materials and methods

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 13952-84-6 |
| EC number | 237-732-7 |
| EC name | sec-butylamine |

Results and discussion

Physical state at 20°C and 1013 hPa

liquid

Colour

colourless to yellow

Odour

other: amine-like

Substance type

organic

4.2 Melting point/freezing point

Endpoint summary: Melting point/freezing point

| UUID |  | IUC5-66cc6d48-bef8-4384-9584-edf224d748d1 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2009-10-23 15:15:46 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

EU: REACH

Short description of key information

not applicable

Discussion

The melting/freezing point is below -20°C

Endpoint study record: Melting point/freezing point, IUC4#2/Ch.2.1

| UUID |  | IUC5-d7610cbb-bc0c-4ea6-b000-5bae9756571e |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | Collette / Epona Associates, LLC / Willington / United States |
| Date |  | 2012-07-20 13:06:01 EDT |
| Remarks |  |  |

Administrative Data

| Purpose flag | key study |
| --- | --- |
| Reliability | 2 (reliable with restrictions) |
| Rationale for reliability incl. deficiencies | Peer reviewed database |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| review article or handbook | Budavari, S. (ed.) | 1996 | The Merck Index | page 256, cited by HSDB 21 Sept 2006 |  |  |  |  |  |

Materials and methods

Principles of method if other than guideline

Method: other

GLP compliance

no data

Test materials

Details on test material

IUCLID4 Test substance: other TS TS-Freetext: sec-butylamine pure

Results and discussions

Melting / freezing point

| Melt./Freez. pt. | Atm. pressure | Decomposition | Decomp. temp. | Sublimation | Subl. temp. | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| -104 °C |  |  |  |  |  |  |

Endpoint study record: expert judgement 2008. Melting point/freezing point

| UUID |  | IUC5-dbc62f34-95e4-4f5f-a5f6-8645eecead3f |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2009-10-23 15:15:47 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| EU: REACH |  |
| --- | --- |
| Data waiving | other justification |
| Justification for data waiving | In accordance with column 2 of REACH Annex VII, the melting/freezing point does not need to be tested, because the melting point is below -20°C. |
| Reliability | 2 (reliable with restrictions) |
| Rationale for reliability incl. deficiencies | expert judgement |

Data source

Data access

data submitter is data owner

Materials and methods

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Endpoint study record: Melting point/freezing point, IUC4#1/Ch.2.1

| UUID |  | IUC5-51e2d15d-6928-4534-87ca-67cc4d1a48b2 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-13 10:48:50 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Reliability | 2 (reliable with restrictions) |
| --- | --- |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BASF AG, Sicherheitsdatenblatt Sek.-BUTYLAMIN (1/91) |  |  |  |  |  |  |  |  |

Materials and methods

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Results and discussions

Melting / freezing point

| Melt./Freez. pt. | Atm. pressure | Decomposition | Decomp. temp. | Sublimation | Subl. temp. | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| < -100 °C |  |  |  |  |  |  |

4.3 Boiling point

Endpoint study record: BASF study-187.0002.1-1987. Boiling point

| UUID |  | IUC5-dcf9a620-f1dc-483d-8a1c-4c3c57565237 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-13 10:48:52 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Study result type | experimental result |  |  |
| --- | --- | --- | --- |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | guideline study |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | BASF AG | 1987 | Dampfdruck-(p,t-) Messung |  | Department of Central Analytics | 187.0002.1 | BASF SE | 187.0002.1 | 1987-03-19 |

Data access

data submitter is data owner

Materials and methods

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
|  | other guideline: BASF standard method |  |

Type of method

dynamic method (with nitrogen atmosphere)

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 13952-84-6 |

Details on test material

- Name of test material (as cited in study report): 2-Butylamin - Molecular weight (if other than submission substance): 73.14 g/mol - Analytical purity: 99.22 %

Results and discussions

Boiling point

| Boiling pt. | Atm. pressure | Decomposition | Decomp. temp. | Remarks |
| --- | --- | --- | --- | --- |
| 63 °C | 1013.3 hPa |  |  | calculated with Antoine equation |

Endpoint study record: Boiling point, IUC4#1/Ch.2.2

| UUID |  | IUC5-e79dbfbe-df45-42fe-b0bd-4c4b9bdaf68b |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-13 10:48:53 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Reliability | 2 (reliable with restrictions) |
| --- | --- |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BASF AG, Sicherheitsdatenblatt Sek.-BUTYLAMIN (1/91) |  |  |  |  |  |  |  |  |

Materials and methods

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Results and discussions

Boiling point

| Boiling pt. | Atm. pressure | Decomposition | Decomp. temp. | Remarks |
| --- | --- | --- | --- | --- |
| 63 — 64 °C |  |  |  |  |

Endpoint study record: Boiling point, IUC4#2/Ch.2.2

| UUID |  | IUC5-542d4645-c1d7-4b0d-9708-4c4afa7b4090 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-13 10:48:55 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Reliability | 2 (reliable with restrictions) |
| --- | --- |
| Rationale for reliability incl. deficiencies | Peer reviewed database |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Budavari, S. (ed.). The Merck Index (1996), p 256, cited by HSDB 21 Sep 2006 |  |  |  |  |  |  |  |  |

Materials and methods

Principles of method if other than guideline

Method: other

GLP compliance

no data

Test materials

Details on test material

IUCLID4 Test substance: other TS TS-Freetext: sec-butylamine pure

Results and discussions

Boiling point

| Boiling pt. | Atm. pressure | Decomposition | Decomp. temp. | Remarks |
| --- | --- | --- | --- | --- |
| 63 °C |  |  |  |  |

4.4 Density

Endpoint study record: Density, IUC4#1/Ch.2.3

| UUID |  | IUC5-d83d4aa7-98ed-45b8-901a-5c3bd97def50 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-13 10:48:56 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Reliability | 2 (reliable with restrictions) |
| --- | --- |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BASF AG, Sicherheitsdatenblatt Sek.-BUTYLAMIN (1/91) |  |  |  |  |  |  |  |  |

Materials and methods

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Results and discussion

Density

| Type | Density | Temp. | Remarks |
| --- | --- | --- | --- |
| density | ca. 0.72 g/cm³ | 20 °C |  |

Overall remarks, attachments

Remarks on results including tables and figures

RM-Freetext: DIN 51757

Endpoint study record: Density, IUC4#2/Ch.2.3

| UUID |  | IUC5-c2f4f1ba-f5ce-4787-90b8-9f2ffd1f684b |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-13 10:48:57 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Reliability | 2 (reliable with restrictions) |
| --- | --- |
| Rationale for reliability incl. deficiencies | Peer reviewed database |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Budavari, S. (ed.). The Merck Index (1996), p 256, cited by HSDB 21 Sep 2006 |  |  |  |  |  |  |  |  |

Materials and methods

Principles of method if other than guideline

Method: other

GLP compliance

no data

Test materials

Details on test material

IUCLID4 Test substance: other TS TS-Freetext: sec-butylamine pure

Results and discussion

Density

| Type | Density | Temp. | Remarks |
| --- | --- | --- | --- |
|  | 0.72 g/cm³ | 20 °C |  |

4.5 Particle size distribution (Granulometry)

Endpoint summary: Particle size distribution (Granulometry)

| UUID |  | IUC5-9a7b695d-a950-4023-b174-0e0df2136533 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2009-10-23 15:16:11 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

EU: REACH

Short description of key information

not applicable

Discussion

Substance is marketed or used in a non solid or granular form.

Endpoint study record: expert judgement 2008. Particle size distribution (Granulometry)

| UUID |  | IUC5-02a99860-ed4b-4e99-a61a-4b8890752ddf |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2009-10-23 15:16:12 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| EU: REACH |  |
| --- | --- |
| Purpose flag | key study; used for MSDS |
| Data waiving | other justification |
| Justification for data waiving | In accordance with column 2 of REACH Annex VII, the particle size distribution (Granulometrie) study does not need to be performed as the substance is marketed or used in a non solid or granular form. |
| Reliability | 2 (reliable with restrictions) |
| Rationale for reliability incl. deficiencies | expert judgement |

Data source

Data access

data submitter is data owner

Materials and methods

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

4.6 Vapour pressure

Endpoint study record: Key.BASF study-187.0002.1-1987. Vapour pressure

| UUID |  | IUC5-72b3fa7b-b948-4feb-b2b4-a2237f2d9a18 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | Collette / Epona Associates, LLC / Willington / United States |
| Date |  | 2012-06-04 12:06:03 EDT |
| Remarks |  |  |

Administrative Data

| Purpose flag | key study |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | guideline study |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | BASF AG | 1987 | Dampfdruck-(p,t-) Messung |  | Department of Central Analytics | 187.0002.1 | BASF SE | 187.0002.1 | 1987-03-19 |

Data access

data submitter is data owner

Materials and methods

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
|  | other guideline: BASF standard method |  |

Type of method

dynamic method (with nitrogen atmosphere)

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 13952-84-6 |

Details on test material

- Name of test material (as cited in study report): 2-Butylamin - Molecular weight (if other than submission substance): 73.14 g/mol - Analytical purity: 99.22 %

Results and discussions

Vapour pressure

183.1 hPa

20 °C

230.5 hPa

25 °C

Any other information on results incl. tables

measured values: temperature (°C) vapour pressure (hPa) -1.50 60.000 3.68 80.000 7.79 100.100 15.78 150.000 25.90 240.000 34.68 349.960 43.47 498.910 52.43 700.080 57.89 850.070 62.64 1000.140 The regression of the results leads with a mean deviation of 0.18 % to the following equation: ln(p/bar) = 9.3112 - 2574.83 / (213.88 + t/°C) The vapour pressure at 20 °C and 25 °C was calculated from the regression equation.

Endpoint study record: Vapour pressure, IUC4#2/Ch.2.4

| UUID |  | IUC5-8223510b-07f8-4c11-b840-4daa129fe1c4 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | Collette / Epona Associates, LLC / Willington / United States |
| Date |  | 2012-06-04 12:07:37 EDT |
| Remarks |  |  |

Administrative Data

| Reliability | 2 (reliable with restrictions) |
| --- | --- |
| Rationale for reliability incl. deficiencies | Peer reviewed database |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Daubert, T.E., Danner, R,P., Physical and Thermodynamic Properties of Pure Chemicals Data Compilation. Washington D.C. (1989), cited by HSDB 21 Sep 2006 |  |  |  |  |  |  |  |  |

Materials and methods

Principles of method if other than guideline

Method: other (measured)

GLP compliance

no data

Test materials

Details on test material

IUCLID4 Test substance: other TS TS-Freetext: sec-butylamine pure

Results and discussions

Vapour pressure

| 237 hPa |  |
| --- | --- |
| 25 °C |  |
| Remarks | 178 mm Hg |

Endpoint study record: Vapour pressure, IUC4#1/Ch.2.4

| UUID |  | IUC5-4ed5a6c2-ecb8-47d2-81c5-2963b99a9647 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-13 10:48:59 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BASF AG, Sicherheitsdatenblatt Sek.-BUTYLAMIN (1/91) |  |  |  |  |  |  |  |  |

Materials and methods

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Results and discussions

Vapour pressure

182 hPa

20 °C

4.7 Partition coefficient

Endpoint study record: KEY.BASF study-BRU 88.034-1988. Partition coefficient

| UUID |  | IUC5-f4ccb417-b187-4324-813b-6a49be75f1f4 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | Collette / Epona Associates, LLC / Willington / United States |
| Date |  | 2012-06-04 12:08:47 EDT |
| Remarks |  |  |

Administrative Data

| Purpose flag | key study |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | guideline study |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | BASF AG | 1988 | Verteilungskoeffizient von Methylamin, Dimethylamin, Trimethylamin und 2-Butylamin bei 25 °C |  | Department of Central Analytics | BRU 88.034 | BASF SE | BRU 88.034 | 1988-02-23 |

Data access

data submitter is data owner

Materials and methods

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
|  | other guideline: BASF standard method |  |

Type of method

shake-flask method

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 13952-84-6 |

Details on test material

- Name of test material (as cited in study report): 2-Butylamin - Molecular weight (if other than submission substance): 73.14 g/mol - Analytical purity: 99.9 %

Results and discussions

Partition coefficient

| Type | Partition coefficient | Temp. | pH | Remarks |
| --- | --- | --- | --- | --- |
| log Pow | 0.601 | 25 °C | 10.9 |  |

Details on results

mean value of three measurements

Any other information on results incl. tables

Mean value of three measurements.

Endpoint study record: Rekker 1988. Partition coefficient

| UUID |  | IUC5-e409128d-582c-4b02-92a1-6b5265a0070b |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-13 10:49:02 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Rekker R.F., The Hydrophobic Fragmental Constant, p. 43,¦Elsevier, Amsterdam (1988) |  |  |  |  |  |  |  |  |

Materials and methods

Principles of method if other than guideline

Method: other (calculated)

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Results and discussions

Partition coefficient

| Type | Partition coefficient | Temp. | pH | Remarks |
| --- | --- | --- | --- | --- |
| log Pow | 0.74 |  |  |  |

Endpoint study record: Partition coefficient, IUC4#3/Ch.2.5

| UUID |  | IUC5-020c5cc7-a86f-4832-9718-458025b2a41a |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-13 10:49:03 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BASF AG, Labor fuer Umweltanalytik; unveroeffentlichte¦Untersuchung (1/89) |  |  |  |  |  |  |  |  |

Materials and methods

Principles of method if other than guideline

Method: other (calculated): Inkrementenmethode von Rekker mit Computerprogramm der Firma CompuDrug Ltd.

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Results and discussions

Partition coefficient

| Type | Partition coefficient | Temp. | pH | Remarks |
| --- | --- | --- | --- | --- |
| log Pow | 0.74 |  |  |  |

Endpoint study record: HSDB 2006. Partition coefficient

| UUID |  | IUC5-824b3c50-d4ef-4ecd-b1fd-4aa7158865fa |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-13 10:49:04 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Reliability | 2 (reliable with restrictions) |
| --- | --- |
| Rationale for reliability incl. deficiencies | Peer reviewed database |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Hansch, C. et al., Exploring QSAR - Hydrophobic, Electronic, and Steric Constants, Washington D.C., American Chem. Soc. (1995), p.10, cited by HSDB 21 Sep 2006 |  |  |  |  |  |  |  |  |

Materials and methods

Partition coefficient type

octanol-water

Principles of method if other than guideline

Method: other (calculated)

GLP compliance

no data

Test materials

Details on test material

IUCLID4 Test substance: other TS TS-Freetext: sec-butylamine pure

Results and discussions

Partition coefficient

| Type | Partition coefficient | Temp. | pH | Remarks |
| --- | --- | --- | --- | --- |
| log Pow | 0.74 |  |  |  |

Endpoint study record: logD calculation.Partition coefficient

| UUID |  | IUC5-0514dfa3-ef36-4c30-b724-f79f96f8c1fa |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2009-01-21 11:09:30 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Study result type | estimated by calculation |  |  |
| --- | --- | --- | --- |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Scientifically acceptable calculation method |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| other: log D calculation | BASF SE | 2009 |  |  |  |  | BASF SE |  | 2009-01-20 |

Data access

data submitter is data owner

Materials and methods

Principles of method if other than guideline

log D calculation based on the formula: logD(alkaline) = log P - log [1 + 10^(pKa-pH)]

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 13952-84-6 |
| EC number | 237-732-7 |
| IUPAC name | butan-2-amine |

Any other information on materials and methods incl. tables

Input parameters used for calculation: pKa = 10.56 (derived from SPARC database search, value from Perrin) log Pow = 0.74 as stated in Rekker R.F., The Hydrophobic Fragmental Constant, p. 43, Elsevier, Amsterdam (1988)

Results and discussions

Partition coefficient

| Type | Partition coefficient | Temp. | pH | Remarks |
| --- | --- | --- | --- | --- |
|  | -5.82 |  | 4 |  |
|  | -2.8 |  | 7 |  |
|  | -0.83 |  | 9 |  |

4.8 Water solubility

Endpoint study record: Water solubility, IUC4#2/Ch.2.6.1

| UUID |  | IUC5-938e1f5f-c85f-46bb-8a60-4f99b14b3c38 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | Collette / Epona Associates, LLC / Willington / United States |
| Date |  | 2012-07-20 12:40:13 EDT |
| Remarks |  |  |

Administrative Data

| Purpose flag | key study |
| --- | --- |
| Reliability | 2 (reliable with restrictions) |
| Rationale for reliability incl. deficiencies | Peer reviewed database |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Yalkowsky SH, Dannenfelser RM, The Aquasol Database of Aqueous Solubility, Fifth ed, Tuscon, AZ (1992) |  |  |  |  |  |  |  |  |

Materials and methods

Principles of method if other than guideline

Method: other

GLP compliance

no data

Test materials

Details on test material

IUCLID4 Test substance: other TS TS-Freetext: sec-butylamine pure

Results and discussions

Water solubility

| 11200 mg/L |  |
| --- | --- |
| Temp. | 20 °C |

Endpoint study record: Water solubility, IUC4#1/Ch.2.6.1

| UUID |  | IUC5-e31d0805-cf43-4ba0-93e8-6d54d1bb593f |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-13 10:49:06 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Reliability | 4 (not assignable) |
| --- | --- |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BASF AG, Sicherheitsdatenblatt Sek.-BUTYLAMIN (1/91) |  |  |  |  |  |  |  |  |

Materials and methods

Principles of method if other than guideline

Method: other

GLP compliance

no data

Test materials

Details on test material

IUCLID4 Test substance: other TS

Results and discussions

Water solubility

| Temp. | 20 °C |
| --- | --- |

Applicant's summary and conclusion

Interpretation of results

miscible

4.10 Surface tension

Endpoint summary: Surface tension

| UUID |  | IUC5-84d2da2e-88a2-4037-ba33-8d395e667745 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2009-10-23 15:15:49 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

EU: REACH

Short description of key information

not surface active

Discussion

Based on chemical structure, no surface activity is to be expected.

Endpoint study record: Surface tension, IUC4#1/Ch.2.6.2

| UUID |  | IUC5-9a4ce026-823d-4b1c-803c-e3177b3bbad8 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-13 10:49:08 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Reliability | 2 (reliable with restrictions) |
| --- | --- |
| Rationale for reliability incl. deficiencies | Peer reviewed database |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | U.S.Coast Guard, Department of Transportation. CHRIS - Hazardous Chemicals Data. Volume II. Washington, DC (1984-5), cited by HSDB 21 Sep 2006 |  |  |  |  |  |  |  |  |

Materials and methods

Principles of method if other than guideline

Method: other

GLP compliance

no data

Test materials

Details on test material

IUCLID4 Test substance: other TS TS-Freetext: sec-butylamine pure

Results and discussions

Surface tension

22.42 mN/m

Endpoint study record: expert judgement 2008. Surface tension

| UUID |  | IUC5-dacf1470-5a2e-4c1c-847b-276788e3548b |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2009-10-23 15:15:50 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| EU: REACH |  |
| --- | --- |
| Purpose flag | key study; used for MSDS |
| Data waiving | other justification |
| Justification for data waiving | In accordance with column 2 of REACH Annex VII, the surface tension does not need to be tested as based on chemical structure, no surface activity is to be expected. |
| Reliability | 2 (reliable with restrictions) |
| Rationale for reliability incl. deficiencies | expert judgement |

Data source

Data access

data submitter is data owner

Materials and methods

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

4.11 Flash point

Endpoint study record: Flash point, IUC4#1/Ch.2.7

| UUID |  | IUC5-4541ff00-9b27-46b3-9981-0c0aebba48bd |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-13 10:49:09 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Reliability | 2 (reliable with restrictions) |
| --- | --- |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BASF AG, Sicherheitsdatenblatt Sek.-BUTYLAMIN (1/91) |  |  |  |  |  |  |  |  |

Materials and methods

Type of method

closed cup

Principles of method if other than guideline

Method: other: DIN 51755

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Results and discussions

Flash point

-19.5 °C

Endpoint study record: Flash point, IUC4#2/Ch.2.7

| UUID |  | IUC5-aa2d5e92-aafc-4d83-a259-1f1e5ab7e230 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-13 10:49:10 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Reliability | 2 (reliable with restrictions) |
| --- | --- |
| Rationale for reliability incl. deficiencies | Peer reviewed database |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Quincy, MA: National Fire Protection Association (1997), p. 325-21, cited by HSDB 21 Sep 2006 |  |  |  |  |  |  |  |  |

Materials and methods

Type of method

closed cup

Principles of method if other than guideline

Method: other

GLP compliance

no data

Test materials

Details on test material

IUCLID4 Test substance: other TS TS-Freetext: sec-butylamine pure

Results and discussions

Flash point

-9 °C

4.12 Auto flammability

Endpoint study record: Auto flammability, IUC4#1/Ch.2.8

| UUID |  | IUC5-7cefb2a8-2dfe-40dd-9bf5-96eecc359ee9 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-16 03:01:17 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Reliability | 2 (reliable with restrictions) |
| --- | --- |
| Rationale for reliability incl. deficiencies | Peer reviewed database |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Keith, L.H., D.B. Walters, (eds.). Compendium of Safety Data Sheets for Research and Industrial Chemicals. Part I, II, and III. Deerfield Beach, FL (1985), cited by HSDB 21 Sep 2006 |  |  |  |  |  |  |  |  |

Materials and methods

Principles of method if other than guideline

Method: other

GLP compliance

no data

Test materials

Details on test material

IUCLID4 Test substance: other TS TS-Freetext: sec-butylamine pure

Results and discussions

Autoflammability / Self-ignition temperature

377.8 °C

4.13 Flammability

Endpoint summary: Flammability

| UUID |  | IUC5-70e98e1a-9c23-48ce-a4bc-8e871835df9c |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2009-10-23 15:15:52 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

EU: REACH

Short description of key information

Highly flammable liquid The substance has no pyrophoric properties and does not liberate flammable gases on contact with water.

Discussion

Flammability derived from flash point (and boilling point) Based on chemical structure pyrophoric properties and flammability in contact with water are not to be expected.

Endpoint study record: expert judgement 2008. Flammability

| UUID |  | IUC5-4e3f5175-d172-42a1-8535-0464fed34c16 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2009-10-23 15:15:53 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| EU: REACH |  |
| --- | --- |
| Purpose flag | key study; used for classification; used for MSDS |
| Data waiving | study scientifically unjustified |
| Justification for data waiving | In accordance with section 1 of REACH Annex XI, the flammability is deduced from flash point and boiling point. |
| Reliability | 2 (reliable with restrictions) |
| Rationale for reliability incl. deficiencies | expert judgement |

Data source

Data access

data submitter is data owner

Materials and methods

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Results and discussions

Solid/liquid: Ignition on contact with air

no

Endpoint study record: expert judgement_ pyrophoric. Flammability

| UUID |  | IUC5-23524e6b-137c-4dd7-875c-4e9d92a65dc9 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2009-10-23 15:15:54 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| EU: REACH |  |
| --- | --- |
| Purpose flag | key study; used for classification; used for MSDS |
| Data waiving | study scientifically unjustified |
| Justification for data waiving | In accordance with section 1 of REACH Annex XI, the study does not need to be performed as based on chemical structure pyrophoric properties are not to be expected. |
| Reliability | 2 (reliable with restrictions) |
| Rationale for reliability incl. deficiencies | expert judgement |

Data source

Data access

data submitter is data owner

Materials and methods

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Endpoint study record: expert judgement_ water contact. Flammability

| UUID |  | IUC5-bfa181a3-b7aa-40d6-8714-b2d21033b22d |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2009-10-23 15:15:56 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| EU: REACH |  |
| --- | --- |
| Purpose flag | key study; used for classification; used for MSDS |
| Data waiving | study scientifically unjustified |
| Justification for data waiving | In accordance with section 1 of REACH Annex XI, the study does not need to be performed as based on chemical structure water reactivity is not to be expected. |
| Reliability | 2 (reliable with restrictions) |
| Rationale for reliability incl. deficiencies | expert judgement |

Data source

Data access

data submitter is data owner

Materials and methods

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Endpoint study record: Flammability, IUC4#1/Ch.2.9

| UUID |  | IUC5-04955376-8411-43b3-bef3-10a84673bde9 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-16 03:03:33 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Reliability | 2 (reliable with restrictions) |
| --- | --- |
| Rationale for reliability incl. deficiencies | Peer reviewed database |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Quincy, MA: National Fire Protection Association (1997), p. 325-21, cited by HSDB 21 Sep 2006 |  |  |  |  |  |  |  |  |

Materials and methods

Principles of method if other than guideline

Method: other

GLP compliance

no data

Test materials

Details on test material

IUCLID4 Test substance: other TS TS-Freetext: sec-butylamine pure

Applicant's summary and conclusion

Interpretation of results

highly flammable

4.14 Explosiveness

Endpoint summary: Explosiveness

| UUID |  | IUC5-4798b301-295e-4baf-965d-3d08da0fcb99 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2009-10-23 15:16:13 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

EU: REACH

Short description of key information

non explosive

Key value for chemical safety assessment

| Explosiveness | non explosive |
| --- | --- |

Discussion

There are no chemical groups associated with explosive properties present in the molecule.

Endpoint study record: expert judgement 2008. Explosiveness

| UUID |  | IUC5-549e14e9-98dd-4e85-901e-6dcd9c49c479 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2009-10-23 15:16:15 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| EU: REACH |  |
| --- | --- |
| Purpose flag | key study; used for classification; used for MSDS |
| Data waiving | other justification |
| Justification for data waiving | In accordance with column 2 of REACH Annex VII, the explosiveness of the substance does not need to be tested, because there are no chemical groups associated with explosive properties in the molecule |
| Reliability | 2 (reliable with restrictions) |
| Rationale for reliability incl. deficiencies | expert judgement |

Data source

Data access

data submitter is data owner

Materials and methods

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

4.15 Oxidising properties

Endpoint summary: Oxidising properties

| UUID |  | IUC5-5f586c4d-1505-4f73-8fe3-a60d6a176c10 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2009-10-23 15:16:16 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

EU: REACH

Short description of key information

no oxidising properties

Key value for chemical safety assessment

| Oxidising | no |
| --- | --- |

Discussion

The substance is highly flammable.

Endpoint study record: expert judgement_highly flammable.Oxidising properties

| UUID |  | IUC5-623cb2af-1dd3-4e09-a1d7-8d0d4b876bf3 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2009-10-23 15:16:17 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| EU: REACH |  |
| --- | --- |
| Purpose flag | key study; used for classification; used for MSDS |
| Data waiving | other justification |
| Justification for data waiving | In accordance with column 2 of REACH Annex VII, the oxidising properties does not need to be conducted as the substance is highly flammable. |
| Reliability | 2 (reliable with restrictions) |
| Rationale for reliability incl. deficiencies | expert judgement |

Data source

Data access

data submitter is data owner

Materials and methods

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

4.17 Stability in organic solvents and identity of relevant degradation products

Endpoint summary: Stability in organic solvents and identity of relevant degradation products

| UUID |  | IUC5-76c9cba0-72fa-4721-b179-a2a0ecf90d0b |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2009-10-23 15:16:22 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

EU: REACH

Short description of key information

not applicable

Discussion

The stability of the substance is not considered as critical.

Endpoint study record: expert judgement_not considered as critical. Stability in organic solvents and identity of relevant degradation products

| UUID |  | IUC5-0dbb705f-389c-4251-bc63-218b038fe43f |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2009-10-23 15:16:23 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| EU: REACH |  |
| --- | --- |
| Purpose flag | key study; used for MSDS |
| Data waiving | other justification |
| Justification for data waiving | In accordance with column 1 of REACH Annex IX, the test does not need to be conducted because the stability of the substance is not considered as critical |
| Reliability | 2 (reliable with restrictions) |
| Rationale for reliability incl. deficiencies | expert judgement |

Data source

Data access

data submitter is data owner

Materials and methods

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

4.21 Dissociation constant

Endpoint study record: KEY.SPARC database.Dissociation constant

| UUID |  | IUC5-0f018244-c3dd-4a22-8ccf-5ba471e9e70d |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | Collette / Epona Associates, LLC / Willington / United States |
| Date |  | 2012-06-04 12:09:48 EDT |
| Remarks |  |  |

Administrative Data

| Purpose flag | key study |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Data from US EPA peer reviewed database |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| secondary source | SPARC database | 2009 |  | http://ibmlc2.chem.uga.edu/sparc/ |  |  |  |  |  |

Data access

data published

Materials and methods

GLP compliance

no data

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 13952-84-6 |

Results and discussions

Dissociation constant

| No. | pKa | at | Remarks |
| --- | --- | --- | --- |
| #1 | 10.56 |  | temperature not stated |

4.23 Additional physico-chemical information

Endpoint study record: Additional physico-chemical information, IUC4#1/Ch.2.14

| UUID |  | IUC5-eb8cbb96-37a6-4b4e-a353-7173b15d7872 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-13 10:49:13 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BASF AG, Sicherheitsdatenblatt Sek.-BUTYLAMIN (1/91) |  |  |  |  |  |  |  |  |

Materials and methods

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Overall remarks, attachments

Remarks on results including tables and figures

RM-Freetext: Explosionsgrenzen in Luft: 1.8 - 9 Vol. % Zuendtemperatur: 290 Grad C (DIN 51794) Gefaehrliche Reaktionen: mit Saeuren stark exotherme Reaktion

5 Environmental fate and pathways

5.1 Stability

5.1.1 Phototransformation in air

Endpoint study record: key.Phototransformation in air Epinwin 2010

| UUID |  | IUC5-2184063f-8b09-4506-ba01-db9f12562eaa |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | Collette / Epona Associates, LLC / Willington / United States |
| Date |  | 2012-06-04 12:11:52 EDT |
| Remarks |  |  |

Administrative Data

| Purpose flag | key study |  |  |
| --- | --- | --- | --- |
| Study result type | estimated by calculation |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | accepted calculation method |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| other: EPIWin calculation |  | 2010 |  | U.S. Environmental Protection Agency (U.S. EPA). Estimation Programs Interface Suite,™. for Microsoft® Windows, v4.00. United States Environmental Protection Agency, Washington, DC, USA. |  |  |  |  |  |

Materials and methods

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| other: | SMILES : C(C(C)N)C CHEM : sec-Butylamine MOL FOR: C4 H11 N1 MOL WT : 73.14 |

Study design

Estimation method (if used)

PHOTOCHEMICAL REACTION WITH OH RADICALS - Concentration of OH radicals: 1.5E6 OH/cm3 - Degradation rate constant: 44.6750 E-12 cm3/molecule-sec - Temperature for which rate constant was calculated: 25 deg C - Computer programme: AOP Program (v1.92)

Any other information on materials and methods incl. tables

SMILES : C(C(C)N)C CHEM: sec-Butylamine MOL FOR: C4 H11 N1 MOL WT : 73.14 ------------------------------ EPI SUMMARY (v4.00) -------------------------- Physical Property Inputs: Log Kow (octanol-water):0.60 Boiling Point (deg C):63.00 Melting Point (deg C):-104.00 Vapor Pressure (mm Hg) :178 Water Solubility (mg/L):1.12E+005 Henry LC (atm-m3/mole) :------.

Results and discussions

% Degradation of test substance

| %Degr. | St. dev. | Sampling time | Test condition |
| --- | --- | --- | --- |
| 50 |  | 0.24 d |  |

Any other information on results incl. tables

AOP Program (v1.92) Results: =========================== ------------------- SUMMARY (AOP v1.92): HYDROXYL RADICALS (25 deg C) -------- Hydrogen Abstraction = 23.6750 E-12 cm3/molecule-sec Reaction with N, S and -OH = 21.0000 E-12 cm3/molecule-sec Addition to Triple Bonds = 0.0000 E-12 cm3/molecule-sec Addition to Olefinic Bonds = 0.0000 E-12 cm3/molecule-sec Addition to Aromatic Rings = 0.0000 E-12 cm3/molecule-sec Addition to Fused Rings = 0.0000 E-12 cm3/molecule-sec OVERALL OH Rate Constant = 44.6750 E-12 cm3/molecule-sec HALF-LIFE = 0.239 Days (12-hr day; 1.5E6 OH/cm3) HALF-LIFE = 2.873 Hrs ------------------- SUMMARY (AOP v1.91): OZONE REACTION (25 deg C) ----------- ****** NO OZONE REACTION ESTIMATION ****** (ONLY Olefins and Acetylenes are Estimated) Experimental Database: NO Structure Matches Fraction sorbed to airborne particulates (phi): 7.34E-009 (Junge-Pankow, Mackay avg) 1.25E-008 (Koa method) Note: the sorbed fraction may be resistant to atmospheric oxidation

5.1.2 Hydrolysis

Endpoint study record: Waiving.Hydrolysis

| UUID |  | IUC5-562ee9cd-f41f-4081-b209-76d537b74700 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2009-10-23 15:16:20 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| EU: REACH |  |
| --- | --- |
| Data waiving | study scientifically unjustified |
| Justification for data waiving | According to Kollig et al. (1993) and Boethling & Mackay (2000) ethylamine does not contain any functional groups liable to hydrolysis. |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| review article or handbook | Boethling RS, Mackay D | 2000 | Handbook of property estimation methods for chemicals: Environmental health and sciences | CRC Press LLC, Boca Raton, Florida. |  |  |  |  |  |
| publication | Kollig HP et al. | 1993 | Environmental fate constants for organic chemicals under consideration for EPA’s hazardous waste identification projects | EPA/600/R-93/132 |  |  |  |  |  |

Materials and methods

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 13952-84-6 |
| EC number | 237-732-7 |
| IUPAC name | butan-2-amine |

5.2 Biodegradation

5.2.1 Biodegradation in water: screening tests

Endpoint study record: KEY. Painter and King 1985/KS K2/Biodegradation in water: screening tests

| UUID |  | IUC5-b45ac014-13b6-4b23-b3f7-eec626884b19 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | Collette / Epona Associates, LLC / Willington / United States |
| Date |  | 2012-06-04 15:49:11 EDT |
| Remarks |  |  |

Administrative Data

| Purpose flag | key study; robust study summary |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Modified guideline study; biodegradation test conducted at 10 different labs; not GLP |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Painter, H.A. and E.F. King | 1985 | Ring Test Programme 1983-1984: Assessment of Biodegradability of Chemicals in Water by Manometric Respirometry. | Directorage-General Environment, Consumer Protection and Nuclear Safety, Commission of the European Communities. Environment and Quality of Life. | Ten laboratories participated in testing | Contract No. XI/W/83/238 |  |  |  |

Data access

data published

Materials and methods

Test type

ready biodegradability

Principles of method if other than guideline

The objective of the ring-test programme was to extend the experience of the use of the EEC respirometric method (DG X1/283/82) to laboratories in OECD countries. Also it was intended to broaden the scope of the programme by testing substances which were known to behave abnormally in some way in the original 14-day MITI I method. Twenty-six laboratories participated in the programme. Sec-butylamine (1-methylpropylamine ) was evaluated at 10 of the participating laboratories. The EEC manometric respirometry method (DG X1/283/81, rev. 5; equivalent or similar to OECD 301C) was used to assess ready biodegradability. It is similar to the Modified MITI I method but differs in that it employs an activated sludge inoculum and a more buffered medium containing an increased concentration of ammonium salts.

GLP compliance

no data

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Details on test material

1-methyl-propylamine Tested at 100 mg/L ThOD (mg O2/100 mg TS) Proportion of (%) NH3 NO3 C N 263 350 66 19

Study design

Oxygen conditions

aerobic

Inoculum or test system

activated sludge, domestic (adaptation not specified)

Details on inoculum

The sludges used as source of inocula by the different laboratories were compared for metabolic activity

Duration of test (contact time)

ca. 28 d

Parameter followed for biodegradation estimation

DOC removal

Details on analytical methods

% biodegradation was calculated by measuring %ThOD by %NH3 or %OxN

Details on study design

Aniline used as a control substance in all testing. In addition to results obtained in the ring test by the 10 laboratories, one laboratory (#14) conducted a Repetitive Die-Away (RDA) test for comparison.

Reference substance

aniline

Results and discussions

% Degradation of test substance

| %Degr. | St. dev. | Parameter | Sampling time | Remarks |
| --- | --- | --- | --- | --- |
| ca. 75 |  | DOC removal |  |  |

BOD5 / COD results

Results with reference substance

Aniline behaved as expected in over 97% of testing

Any other information on results incl. tables

Characteristics of sludges used as source of inocula compared with metabolic activity: Activated sludge plant conditions. Lab No.; Rank position; Sludge age (d); Retention time of liquid phase (h). 3; 1; 2.6; 5.0. 4; 2; 3.7; 7.6. 5; 3; 14.1; 10.1. 19; 4; -; -. 13; 5; 10.0; 6.0. 1; 6; 18.0; 19.0. 21; 7; -; 4.0. 6; 8; -; -. 20; 9; 11.0; 14.0. 9; 10; -; -. The inocula used by the different laboratories were ranked in order of activity by three methods and it was concluded that some sludges were more generally metabolically active than others.No significant correlation was found between the sludge ages of the activated sludge plants from which the inocula were obtained and metabolic activity.. The EEC manometric respirometric method was concluded to be satisfactory for determining ready biodegradability.The importance of allowing for nitrification in calculating the %ThOD of nitrogen-containing test substances was noted.It was also concluded that changes in the concentration of ammonia and oxidized nitrogen should be determined, even with test substances not containing nitrogen.The effects of nitrification could be lessened by using less ammonium chloride in the medium.Evidence was also obtained to show that the “pass” level could be reduced to 50% or even 40% ThOD, with concomitant high removal of DOC. Results from all laboratories reporting on 1-methylpropylamine: values in ascending order of magnitude. Lab No.; After 28 d. %ThOD by NH3; %ThOD by OxNa; %DOC removed. 9; 5; 5; -. 19; 27*; 27*; 27. 21; 50*; 50*; 94. 6; 68; 52; 74. 20; 82; 62; 97. 1; 83; 63; 82. 4; 101; 76; 93. 13; 103*; 78*; 99. 3; 110; 84; 96. 5; 116*; 86*; 101. Median; 83; 63. Range; 5-116; 5-86. Mean; 75; 58; 85. n; 10; 10; 9. sd; 37; 26; 23. >60%; 7/10; 6/10. >50%; 8/10; 8/10. >40%; 8/10; 8/10. Where (a)=oxidized nitrogen; * = disparate duplicates If NH3 is the end product, 7/10 laboratories reported > 60% ThOD at 28 d; if nitrate is taken into account only 5/10 and 6/10, respectively, would “pass” at 60%.If the “pass” level were reduced to 50% a further two laboratories would succeed, both being confirmed by its %DOC removal.. Summary of Results for 1-methyl-propylamine: No. of Labs; After 28 days; DOC removed. Mean %ThOD; %Labs >60%. 10; 75; 70(80); 85%. Where ( ) = values over 50% ThOD. Percentage oxygen uptake for 1-methylpropylamine Number of Laboratories; Delay time (td, days: median); Oxygen uptake (%ThOD); DOC removed (%). (td + 10)d; After 14 d (mean); 28 d. 10; 0; 66; 53*; 45; 34*; 75; 58*; 85. (7); (33); (24); (37); (27); (37); (26); (23). ( ) = standard deviations *allows for nitrification Nitrification in the presence of 1-methylpropylamine: No. of laboratories; Inhibition of nitrification reported. Reporting; OxN found; OxN complete. 6; 5; 5; no.. Oxygen uptake and DOC removed using MITI inoculum: Lab no.; Delay time (td); %ThOD at 28 d; %DOC; Nitrification. 13; -; 0; 6; inhibited. 20; 3; 52 (39); 66; -. Values in brackets allow for nitrification. Oxygen uptake and DOC removal reported by laboratories also conducting the “Repetitive Die-Away” Test: Laboratory No.; Delay time td (days); After 28 d; After a longer period. %ThOD; %DOC; (days); %ThOD; %DOC. 14; 6; 74; 98; 42; >100; 99. In the Die-Away test, 1-methylpropylamine was found to be readily biodegradable after 28 days (no additonal details available)...

Applicant's summary and conclusion

Interpretation of results

readily biodegradable

Conclusions

Based on results of testing in 10 different laboratories, 1-methylpropylamine was concluded to be readily biodegradable based on mean %ThOD values of more than 60% by more than two-thirds of the reporting laboratories.

Endpoint study record: BASF AG 1250.Biodegradation in water: screening tests

| UUID |  | IUC5-23d61257-ac74-4c50-8bdb-c1585e21cb06 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2009-01-21 10:26:18 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Purpose flag | robust study summary |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Non GLP guideline study |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | BASF AG | 1981 | sec. Butylamin: Standversuch | unpublished data | Department of Ecology |  | BASF SE | 1250 | 1981-01-07 |

Data access

data submitter is data owner

Materials and methods

Test type

inherent biodegradability

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | OECD Guideline 302 B (Inherent biodegradability: Zahn-Wellens/EMPA Test) |  |

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 13952-84-6 |

Details on test material

- Name of test material (as cited in study report): sec. Butylamin

Study design

Oxygen conditions

aerobic

Inoculum or test system

activated sludge, industrial, non-adapted

Duration of test (contact time)

18 d

Initial test substance concentration

| Initial conc. | Based on |
| --- | --- |
| 400 mg/L |  |

Parameter followed for biodegradation estimation

O2 consumption

Results and discussions

% Degradation of test substance

| %Degr. | St. dev. | Parameter | Sampling time | Remarks |
| --- | --- | --- | --- | --- |
| 100 |  | TOC removal | 18 d |  |

Details on results

- kinetic of elimination: time DOC-elimination (days) (%) 3 (hours) -8 1 -4 4 -3 7 6 11 99 13 100 15 100 18 100 - the sigmoidal shape of the elimination curve is a strong indication for biodegradation

Applicant's summary and conclusion

Interpretation of results

other: easily eliminated from water

Endpoint study record: MITI 1992.302C. Biodegradation in water: screening tests

| UUID |  | IUC5-ea76af3e-c31e-44ad-9206-09a991018b9d |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2009-02-11 04:08:34 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Study result type | experimental result |  |  |
| --- | --- | --- | --- |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Guideline study with acceptable restrictions |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Japan Chemical Industry Ecology-Toxicology & Information Center (JETOC) | 1980 | Biodegradation and Bioaccumulation Data of Existing Chemicals based on the CSCL Japan |  | Chemicals Inspection & Testing Institute Japan |  |  |  | 1980-12-24 |

Data access

data published

Materials and methods

Test type

inherent biodegradability

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | OECD Guideline 302 C (Inherent Biodegradability: Modified MITI Test (II)) |  |

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 13952-84-6 |

Details on test material

- Name of test material (as cited in study report): 1-metzhylpropylamine

Study design

Oxygen conditions

aerobic

Inoculum or test system

mixture of sewage, soil and natural water

Details on inoculum

- Source of inoculum/activated sludge (e.g. location, sampling depth, contamination history, procedure): In March, June, September, and December, sludge was sampled at the following 10 places in Japan: 1. Fukogawa city sewage plant, 2. Fukashiba industry sewage plant, 3. Nakahama city sewage plant, 4. Ochiai city sewage plant, 5. Kitakami river, 6. Shinano river, 7. Yoshino river, 8. Lake Biwa, 9. Hiroshima bay, 10. Dookai bay; sampling: 1. City sewage: Returned sludge from sewage plants was taken. 2. Rivers, lake and sea: Surface water and surface soil which were in contact with atmosphere were collected. - Method of cultivation: About 30 minutes after ceasing aeration to the sludge mixture, supernatant corresponding to about 1/3 of the whole volume was removed. Then the equal volume of dechlorinated water was added to the remaining portion and aerated again, followed by addition of synthetic sewage at a concentration of 0.1% (w/v). This procedure was repeated once every day. The culturing was carried out at 25 ± 2 °C. 5 L of the filtrate of the supernatant of old activated sludge was mixed with 500 mL of the filtrate of the supernatant of new sludge and cultured at pH 7.0 ± 1.0 under sufficient aeration using prefiltered open air. During the cultivation, appearance of the supernatant, precipitability, formation of flock, pH, dissolved oxygen concentration in the solution and temperature were checked and necessary adjustments were made, Microflora in the activated sludge was microscopically observed and sludge with no abnormal symptom was used for the test. - Concentration of sludge: 100 mg/L

Duration of test (contact time)

28 d

Initial test substance concentration

| Initial conc. | Based on |
| --- | --- |
| 30 mg/L | test mat. |

Parameter followed for biodegradation estimation

O2 consumption

Details on study design

TEST CONDITIONS - Composition of medium: 3 mL each of four stock solutions, as described in JIS K 0102-1986-21, are diluted to 1000 mL with purified water - pH: 7.0 - pH adjusted: yes - Suspended solids concentration: determined according to Method Japanese Industrial Standards (JIS) K 0102-1986-14.1 TEST SYSTEM - Culturing apparatus: Closed system oxygen consumption measuring apparatus (Coulometer: Ohkura Electric Co., Ltd.); 300 mL vessel, absorbent for evolving carbon dioxide Soda lime No .l (extra pure reagent, Wako Pure Chemical Industries, Ltd.). - Number of culture flasks/concentration: 1 - Measuring equipment: Coulometer, Okhura Electric Co., Ltd. - Test performed in open system: no - Details of trap for CO2 and volatile organics if used: soda lime, extra pure, Wako Pure Chemical Industries, Ltd.) CONTROL AND BLANK SYSTEM - Inoculum blank: yes - Abiotic sterile control: yes - Toxicity control: no

Reference substance

aniline

Results and discussions

% Degradation of test substance

| %Degr. | St. dev. | Parameter | Sampling time | Remarks |
| --- | --- | --- | --- | --- |
| 68 — 100 |  | O2 consumption | 28 d |  |

Applicant's summary and conclusion

Interpretation of results

inherently biodegradable

Endpoint study record: Hoechst AG (+1980; Abw.-biol Unters, Zahn Wellens; RL2)

| UUID |  | IUC5-78bab218-871b-41e7-a84e-ba49d796f292 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2009-01-12 08:35:50 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Study result type | experimental result |  |  |
| --- | --- | --- | --- |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Pre-guideline, but comparable to guideline study. Detailed study report not available. |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| other company data | Hoechst AG | 1980 | Abwasserbiologische Untersuchung von 2-Aminobutan | unpublished (compilation of test results) | Hoechst AG, Abteilung Reinhaltung von Wasser und Luft, Frankfurt/Main, Germany |  | OXEA Group | T02467 | 1980-04-16 |
| publication | Zahn R and Wellens H | 1974 | Ein einfaches Verfahren zur Prüfung der biologischen Abbaubarkeit von Produkten und Abwasserinhaltsstoffen | Chemiker-Zeitung 98(5): 228-232 |  |  |  | T04679 | 1973-12-31 |
| publication | Zahn R and Wellens H | 1980 | Prüfung der biologischen Abbaubarkeit im Standversuch - weitere Erfahrungen und neue Einsatzmöglichkeiten | Z Wasser Abwasser Forsch 13(1): 1-7 |  |  |  | T02014 | 1979-12-31 |

Data access

data submitter is data owner

Data protection claimed

yes, but willing to share

Materials and methods

Test type

inherent biodegradability

Principles of method if other than guideline

Pre-guideline study; method: other: original Zahn-Wellens test (see reference 2) and 3)). Comparable to OECD test guideline 302 B ((Zahn-Wellens/EMPA Test)

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 13952-84-6 |
| EC number | 237-732-7 |
| EC name | sec-butylamine |

Details on test material

- Name of test material (as cited in study report): 2-Aminobutan - Analytical purity: no data - Stability under test conditions: no data - Storage condition of test material: no data

Study design

Oxygen conditions

aerobic

Inoculum or test system

activated sludge, non-adapted

Details on inoculum

- Source of inoculum/activated sludge: source of inoculum/activated sludge unknown - Preparation of inoculum for exposure: centrifugation for 5 min at 2000 R/min - Pretreatment: no data - Concentration of sludge in test medium: 1 g dry substance/L (ca. 12 g wet sludge after centrifugation for 2 L reaction medium) - Water filtered: no, tap water

Duration of test (contact time)

20 d

Initial test substance concentration

| Initial conc. | Based on |
| --- | --- |
| ca. 1000 mg/L | COD |

Parameter followed for biodegradation estimation

other: COD

Details on study design

TEST CONDITIONS - Composition of medium: mineral medium: 215 mg urea (alternatively 385 mg NH4Cl) and 89 mg NaH2PO4 x H2O in 2 L of water - Additional substrate: no - Solubilising agent (type and concentration if used): no - Test temperature: 22 ± 3°C - pH: 6.5 - 7.5 - pH adjusted: if necessary - Aeration of dilution water: yes - Suspended solids concentration: 1 g dry substance (activated sludge)/L - Continuous darkness: no data TEST SYSTEM - Culturing apparatus: 4 L cylindric glas vessel (beaker) - Number of culture flasks/concentration: 1 - Method used to create aerobic conditions: stirring and introduction of compressed air passed through a cotton wool strainer and a wash-bottle - Measuring equipment: COD or TOC analyser - Test performed in closed vessels due to significant volatility of test substance: no data - Test performed in open system: no data SAMPLING - Sampling frequency: daily or other time intervals - Sampling method: withdrawal of sample from test vessel and filtration of sample through paper filter (Schleicher und Schüll, Blauband Nr. 589) washed three times with distilled water CONTROL AND BLANK SYSTEM - Inoculum blank: yes

Reference substance

no data

Results and discussions

% Degradation of test substance

| %Degr. | St. dev. | Parameter | Sampling time | Remarks |
| --- | --- | --- | --- | --- |
| 85 |  | other: COD | 20 d |  |
| 32 |  | other: COD | 15 d |  |
| 13 |  | other: COD | 10 d |  |
| 10 |  | other: COD | 5 d |  |

Applicant's summary and conclusion

Validity criteria fulfilled

no data

Interpretation of results

inherently biodegradable

Conclusions

Sec-butylamine undergoes 85% biodegration within 20 days (10 days: 13%, 15 days: 32%) based on determination of COD. It meets the criteria for inherent biodegradability.

Executive summary

Sec-butylamine was tested for biodegradability in a pre-guideline test following the original method of Zahn-Wellens. Test realization ist reported with no specific details, but the general procedure followed is described in reference 2. The test method is comparable to OECD test guideline 302 B (Zahn-Wellens/EMPA Test). Non-adapted activated sludge of a sewage treatment plant served as inoculum. Test substance was incubated for 20 days at 22°C. No parallels were set up. Samples were analyzed for COD at various time points. After 20 days sec-butylamine was degraded to 85% (day 15: 32%, day 10: 13%). In a Zahn-Wellens test for biodegradability,sec-butylamine was shown to be inherently biodegradable(Hoechst AG, 1980). This biodegradability study is classified as acceptable. The method used is comparable to OECD test guideline 302 B (Zahn-Wellens/EMPA Test) with minor restrictions (no duplicates, different test medium, restricted reporting).

Endpoint study record: Biodegradation in water: screening tests, IUC4#3/Ch.3.5

| UUID |  | IUC5-a8684252-211c-4bf1-b6ea-0818a385c475 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | Collette / Epona Associates, LLC / Willington / United States |
| Date |  | 2012-06-04 15:54:12 EDT |
| Remarks |  |  |

Administrative Data

| Reliability | 2 (reliable with restrictions) |
| --- | --- |
| Rationale for reliability incl. deficiencies | test procedure following precursor of OECD 301 F |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Water Research Centre, Dec 1986, as cited in a letter of Covance Laboratories, UK, addressed to C. Hamilton, Air Products and Chemicals, Inc., USA, 19 Dec 2001 |  |  |  |  |  |  |  |  |

Materials and methods

Principles of method if other than guideline

Method: other: precursor of OECD 301 F

GLP compliance

no data

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Study design

Oxygen conditions

aerobic

Inoculum or test system

other: activated sludge (Wargrave/Thames; UK)

Duration of test (contact time)

28 d

Initial test substance concentration

| Initial conc. | Based on |
| --- | --- |
| 100 mg/L | test mat. |

Any other information on materials and methods incl. tables

ME-Freetext: UK/EC respirometric method, basically the MITI method, but with a straight activated sludge inoculum, which was later (1992) adopted by OECD as guideline 301 F

Results and discussions

% Degradation of test substance

| %Degr. | St. dev. | Parameter | Sampling time | Remarks |
| --- | --- | --- | --- | --- |
| > 60 |  |  | 28 d |  |

Details on results

RS-Freetext: DOC-elimination after 28 days: >80 % Kinetic of test substance (in %): = 10 after 15 > 60 after 25

BOD5 / COD results

Any other information on results incl. tables

RM-Freetext: - test was performed in the presence of 10 mg/L TCMP (2-chloro-6-(trichloromethyl)pyridine) to suppress nitrification - activated sludge: 30 mg/L dry weight

Applicant's summary and conclusion

Interpretation of results

readily biodegradable

Endpoint study record: Biodegradation in water: screening tests, IUC4#2/Ch.3.5

| UUID |  | IUC5-8945b301-9374-47a6-9ff6-b1166b4c3659 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2009-02-11 03:58:04 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Study result type | experimental result |  |  |
| --- | --- | --- | --- |
| Reliability | 4 (not assignable) |  |  |
| Rationale for reliability incl. deficiencies | original reference not available |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| other: Ringtest reference | BASF AG | 1983 | EG-Respirometer-Ringtest |  | Department of Ecology |  |  |  |  |

Data access

data submitter is data owner

Materials and methods

Test type

ready biodegradability

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | OECD Guideline 301 F (Ready Biodegradability: Manometric Respirometry Test) |  |

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Details on test material

IUCLID4 Test substance: no data

Study design

Oxygen conditions

aerobic

Inoculum or test system

activated sludge, domestic (adaptation not specified)

Duration of test (contact time)

26 d

Results and discussions

% Degradation of test substance

| %Degr. | St. dev. | Parameter | Sampling time | Remarks |
| --- | --- | --- | --- | --- |
| ca. 70 — ca. 80 |  |  | 26 d |  |

Details on results

RS-Freetext: 80-90 % DOC-elimination after 26 days

BOD5 / COD results

Any other information on results incl. tables

RM-Freetext: lag phase: 17 days, plateau phase reached on day 26

Applicant's summary and conclusion

Interpretation of results

readily biodegradable

5.3 Bioaccumulation

5.3.1 Bioaccumulation: aquatic / sediment

Endpoint study record: KEY.Bioaccumulation: aquatic / sediment Epiwin 2010

| UUID |  | IUC5-c543c326-407e-4689-82b2-09a05aabad41 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | Collette / Epona Associates, LLC / Willington / United States |
| Date |  | 2012-06-04 12:30:06 EDT |
| Remarks |  |  |

Administrative Data

| Purpose flag | key study |  |  |
| --- | --- | --- | --- |
| Study result type | estimated by calculation |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Accepted calculation method |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| other: EPIWin calculation |  | 2010 |  | U.S. Environmental Protection Agency (U.S. EPA). Estimation Programs Interface Suite,™. for Microsoft® Windows, v4.00. United States Environmental Protection Agency, Washington, DC, USA. |  |  |  |  |  |

Materials and methods

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| other: | SMILES : C(C(C)N)C CHEM : sec-Butylamine MOL FOR: C4 H11 N1 MOL WT : 73.14 |

Test conditions

Any other information on materials and methods incl. tables

SMILES : C(C(C)N)C CHEM: sec-Butylamine MOL FOR: C4 H11 N1 MOL WT : 73.14 ------------------------------ EPI SUMMARY (v4.00) -------------------------- Physical Property Inputs: Log Kow (octanol-water):0.60 Boiling Point (deg C):63.00 Melting Point (deg C):-104.00 Vapor Pressure (mm Hg) :178 Water Solubility (mg/L):1.12E+005 Henry LC (atm-m3/mole) :------.

Results and discussions

Bioaccumulation factor

| Conc. in environment / dose | Type | Value | Basis | Time of plateau | Calculation basis | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
|  | BCF | 3.1262 L/kg |  |  |  |  |

Any other information on results incl. tables

BCFBAF Program (v3.00) Results: ============================== --------------------------------- BCFBAF v3.00 -------------------------------- Summary Results: Log BCF (regression-based estimate): 0.50 (BCF = 3.16 L/kg wet-wt) Biotransformation Half-Life (days) : 0.155 (normalized to 10 g fish) Log BAF (Arnot-Gobas upper trophic): 0.10 (BAF = 1.26 L/kg wet-wt) Log Kow (experimental): 0.74 Log Kow used by BCF estimates: 0.60 (user entered) Equation Used to Make BCF estimate: Log BCF = 0.50 Correction(s): Value Correction Factors Not Used for Log Kow < 1 Estimated Log BCF = 0.500 (BCF = 3.162 L/kg wet-wt) =========================================================== Whole Body Primary Biotransformation Rate Estimate for Fish: =========================================================== ------+-----+--------------------------------------------+---------+--------- TYPE ¦ NUM ¦ LOG BIOTRANSFORMATION FRAGMENT DESCRIPTION ¦ COEFF ¦ VALUE ------+-----+--------------------------------------------+---------+--------- Frag ¦ 1 ¦ Aliphatic amine [-NH2 or -NH-] ¦ 0.4067 ¦ 0.4067 Frag ¦ 2 ¦ Methyl [-CH3] ¦ 0.2451 ¦ 0.4902 Frag ¦ 1 ¦ -CH2- [linear] ¦ 0.0242 ¦ 0.0242 Frag ¦ 1 ¦ -CH- [linear] ¦ -0.1912 ¦ -0.1912 L Kow¦ * ¦ Log Kow = 0.60 (user-entered ) ¦ 0.3073 ¦ 0.1847 MolWt¦ * ¦ Molecular Weight Parameter ¦ ¦ -0.1876 Const¦ * ¦ Equation Constant ¦ ¦ -1.5058 ============+============================================+=========+========= RESULT ¦ LOG Bio Half-Life (days) ¦ ¦ -0.8100 RESULT ¦ Bio Half-Life (days) ¦ ¦ 0.1549 NOTE ¦ Bio Half-Life Normalized to 10 g fish at 15 deg C ¦ ============+============================================+=========+========= Biotransformation Rate Constant: kM (Rate Constant): 4.475 /day (10 gram fish) kM (Rate Constant): 2.517 /day (100 gram fish) kM (Rate Constant): 1.415 /day (1 kg fish) kM (Rate Constant): 0.7958 /day (10 kg fish) Arnot-Gobas BCF & BAF Methods (including biotransformation rate estimates): Estimated Log BCF (upper trophic) = 0.100 (BCF = 1.258 L/kg wet-wt) Estimated Log BAF (upper trophic) = 0.100 (BAF = 1.258 L/kg wet-wt) Estimated Log BCF (mid trophic) = 0.074 (BCF = 1.185 L/kg wet-wt) Estimated Log BAF (mid trophic) = 0.074 (BAF = 1.185 L/kg wet-wt) Estimated Log BCF (lower trophic) = 0.066 (BCF = 1.165 L/kg wet-wt) Estimated Log BAF (lower trophic) = 0.066 (BAF = 1.165 L/kg wet-wt) Arnot-Gobas BCF & BAF Methods (assuming a biotransformation rate of zero): Estimated Log BCF (upper trophic) = 0.121 (BCF = 1.32 L/kg wet-wt) Estimated Log BAF (upper trophic) = 0.122 (BAF = 1.325 L/kg wet-wt)

5.4 Transport and distribution

5.4.1 Adsorption / desorption

Endpoint study record: Adsorption / desorption, IUC4#2/Ch.3.3.1

| UUID |  | IUC5-6b40c3be-4407-4771-a535-8e79d217ed2d |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-13 10:49:19 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Rationale for reliability incl. deficiencies | scientifically acceptable method |
| --- | --- |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BASF AG, Department of Product Safety, unpublished¦calculation, 01.02.2002 |  |  |  |  |  |  |  |  |

Materials and methods

Study type

adsorption

Media

soil

Principles of method if other than guideline

Method: other: calculated: PCKOCWIN v1.66

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Results and discussions

Results: Batch equilibrium or other method

Any other information on results incl. tables

RS-Freetext: log Koc = 1.7064 (Koc = 50.9)

5.4.2 Henry's Law constant

Endpoint study record: KEY.Henry's Law constant Epiwin 2010

| UUID |  | IUC5-a0c8afa6-4f2c-4af9-9e29-329e6bceea88 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | Collette / Epona Associates, LLC / Willington / United States |
| Date |  | 2012-06-04 12:30:55 EDT |
| Remarks |  |  |

Administrative Data

| Purpose flag | key study |  |  |
| --- | --- | --- | --- |
| Study result type | estimated by calculation |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Accepted calculation method |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| other: EPIWin calculation |  | 2010 |  | U.S. Environmental Protection Agency (U.S. EPA). Estimation Programs Interface Suite,™. for Microsoft® Windows, v4.00. United States Environmental Protection Agency, Washington, DC, USA. |  |  |  |  |  |

Materials and methods

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| other: | Name : SEC-BUTYLAMINE CAS Num : 013952-84-6 |

Any other information on materials and methods incl. tables

SMILES : C(C(C)N)C CHEM: sec-Butylamine MOL FOR: C4 H11 N1 MOL WT : 73.14 ------------------------------ EPI SUMMARY (v4.00) -------------------------- Physical Property Inputs: Log Kow (octanol-water):0.60 Boiling Point (deg C):63.00 Melting Point (deg C):-104.00 Vapor Pressure (mm Hg) :178 Water Solubility (mg/L):1.12E+005 Henry LC (atm-m3/mole) :------.

Results and discussions

Henry's Law constant H

| H | 1.8 Pa m³/mol |
| --- | --- |
| Temp. (°C) | 25 |
| Remarks | Bond estimate; 1.77E-005 atm-m3/mole |

Any other information on results incl. tables

HENRYWIN (v3.20) Program Results: ============================= Bond Est : 1.77E-005 atm-m3/mole (1.80E+000 Pa-m3/mole) Group Est: Incomplete --------------------------- HENRYWIN v3.20 Results -------------------------- Experimental Database Structure Match: Name : SEC-BUTYLAMINE CAS Num : 013952-84-6 Exp HLC : 1.53E-04 atm-m3/mole (15.5 Pa-m3/mole) Temper : 25 deg C Exp Ref : VP/WSOL ----------+---------------------------------------------+---------+---------- CLASS ¦ BOND CONTRIBUTION DESCRIPTION ¦ COMMENT ¦ VALUE ----------+---------------------------------------------+---------+---------- HYDROGEN ¦ 9 Hydrogen to Carbon (aliphatic) Bonds ¦ ¦ -1.0771 HYDROGEN ¦ 2 Hydrogen to Nitrogen Bonds ¦ ¦ 2.5670 FRAGMENT ¦ 3 C-C ¦ ¦ 0.3489 FRAGMENT ¦ 1 C-N ¦ ¦ 1.3010 ----------+---------------------------------------------+---------+---------- RESULT ¦ BOND ESTIMATION METHOD for LWAPC VALUE ¦ TOTAL ¦ 3.140 ----------+---------------------------------------------+---------+---------- HENRYs LAW CONSTANT at 25 deg C = 1.77E-005 atm-m3/mole = 7.25E-004 unitless = 1.80E+000 Pa-m3/mole --------+-----------------------------------------------+------------+-------- ¦ GROUP CONTRIBUTION DESCRIPTION ¦ COMMENT ¦ VALUE --------+-----------------------------------------------+------------+-------- ¦ 2 CH3 (X) ¦ ¦ -1.24 ¦ 1 CH2 (C)(C) ¦ ¦ -0.15 ¦ 1 NH2 (C) ¦ ¦ 4.15 ¦ MISSING Value for: CH (C)(N)(C) --------+-----------------------------------------------+------------+-------- RESULT ¦ GROUP ESTIMATION METHOD for LOG GAMMA VALUE ¦ INCOMPLETE ¦ 2.76 --------+-----------------------------------------------+------------+-------- For Henry LC Comparison Purposes: Exper Database: 1.53E-04 atm-m3/mole (1.55E+001 Pa-m3/mole) User-Entered Henry LC: not entered Henrys LC [via VP/WSol estimate using User-Entered or Estimated values]: HLC: 1.529E-004 atm-m3/mole (1.550E+001 Pa-m3/mole) VP: 178 mm Hg (source: User-Entered) WS: 1.12E+005 mg/L (source: User-Entered)

5.4.3 Distribution modelling

Endpoint study record: Key.Distribution modelling.Epiwin 2012

| UUID |  | IUC5-d895a6e1-10de-40a3-b37e-f962dd0bd969 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | Collette / Epona Associates, LLC / Willington / United States |
| Date |  | 2012-07-20 14:28:40 EDT |
| Remarks |  |  |

Administrative Data

| Purpose flag | key study |  |  |
| --- | --- | --- | --- |
| Study result type | estimated by calculation |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Accepted calculation method |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| other: EPIWin calculation |  | 2012 |  | U.S. Environmental Protection Agency (U.S. EPA). Estimation Programs Interface Suite,™. for Microsoft® Windows, v4.10. United States Environmental Protection Agency, Washington, DC, USA. |  |  |  |  |  |

Materials and methods

Model

Calculation according to Mackay, Level III

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| other: | Chem Name : sec-Butylamine Molecular Wt: 73.14 |

Test substance input data

Level III Fugacity Model (Full-Output): ======================================= Chem Name : 2-Butanamine Molecular Wt: 73.14 Henry's LC : 1.77e-005 atm-m3/mole (user-entered; Bond estimate) Vapor Press : 173 mm Hg (user-entered) Log Kow : 0.601 (user-entered) Soil Koc : 37.2 (KOCWIN MCI method)

Results and discussions

Percent distribution in media

Air (%)

0.743

Water (%)

32.2

Soil (%)

67

Sediment (%)

0.109

Any other information on results incl. tables

Level III Fugacity Model (Full-Output): ======================================= Mass Amount Half-Life Emissions (percent) (hr) (kg/hr) Air 0.743 5.75 1000 Water 32.2 360 1000 Soil 67 720 1000 Sediment 0.109 3.24e+003 0 Fugacity Reaction Advection Reaction Advection (atm) (kg/hr) (kg/hr) (percent) (percent) Air 2.91e-011 1.05e+003 87.2 35 2.91 Water 4.57e-010 727 378 24.2 12.6 Soil 8.86e-009 756 0 25.2 0 Sediment 4.07e-010 0.273 0.0255 0.00909 0.00085 Persistence Time: 391 hr Reaction Time: 463 hr Advection Time: 2.52e+003 hr Percent Reacted: 84.5 Percent Advected: 15.5 Half-Lives (hr), (based upon Biowin (Ultimate) and Aopwin): Air: 5.746 Water: 360 Soil: 720 Sediment: 3240 Biowin estimate: 3.062 (weeks) Advection Times (hr): Air: 100 Water: 1000 Sediment: 5e+004

Endpoint study record: Distribution modelling, IUC4#1/Ch.3.3.2

| UUID |  | IUC5-36366c88-4ca1-4812-98b5-7b21e7525596 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-13 10:49:20 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Reliability | 2 (reliable with restrictions) |
| --- | --- |
| Rationale for reliability incl. deficiencies | scientifically acceptable method |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BASF AG, Department of Product Safety, unpublished¦calculation, 01.02.2002 |  |  |  |  |  |  |  |  |

Materials and methods

Model

Calculation according to Mackay, Level I

Media

air - biota - sediment(s) - soil - water

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Results and discussions

Percent distribution in media

Any other information on results incl. tables

RS-Freetext: over time the substance will preferentially distribute into the compartments water and air: - water: 61.0 % - air: 38.9 %

Overall remarks, attachments

Remarks on results including tables and figures

RM-Freetext: the following physico-chemical properties of the substance were used to calculate the distribution: - molecular mass: 73 g/mol - data temperature: 25 °C - log Kow: 0.6 - water-solubility: 7.2E5 g/m3 - Henry´s law constant: 1.85 Pa*m3/mol - vapour pressure: 18200 Pa - melting point: -100 °C

Endpoint study record: Distribution modelling, IUC4#2/Ch.3.3.2

| UUID |  | IUC5-b448de0c-93c0-4796-b869-3252c7a380f8 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-13 10:49:21 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Reliability | 2 (reliable with restrictions) |
| --- | --- |
| Rationale for reliability incl. deficiencies | scientifically acceptable method |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BASF AG, Department of Product Safety, unpublished calculation, 25.03.2003 |  |  |  |  |  |  |  |  |

Materials and methods

Model

Calculation according to Mackay, Level I

Media

air - biota - sediment(s) - soil - water

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Any other information on materials and methods incl. tables

ME-Freetext: Level I V 2.11 model

Results and discussions

Percent distribution in media

Any other information on results incl. tables

RS-Freetext: over the time, the substance will preferentially distribute into the water: 60.9% (air: 39.1%)

Overall remarks, attachments

Remarks on results including tables and figures

RM-Freetext: the following physico-chemical properties of the substance were used to calculate the distribuion: - moleculare mass: 73 g/mol - data temperature: 20 °C - log Kow: -0.74 - water-solubility: 7.20E+06 g/m3 - Henry's law constant: 1.83 Pa*m3/mol - vapour pressure: 18000 Pa - melting point: -104 °C

5.4.4 Other distribution data

Endpoint study record: Other distribution data, IUC4#1/Ch.3.3.1

| UUID |  | IUC5-2d71d4a4-df0f-4ad9-9369-7d7c65f23464 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-13 10:49:22 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Reliability | 2 (reliable with restrictions) |
| --- | --- |
| Rationale for reliability incl. deficiencies | scientifically acceptable method |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BASF AG, Department of Product Safety, unpublished¦calculation, 01.02.2002 |  |  |  |  |  |  |  |  |

Materials and methods

Type of study

volatility

Media

water - air

Principles of method if other than guideline

Method: other: HENRYWIN v.3.1

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Results and discussions

Any other information on results incl. tables

RS-Freetext: Henry´s law constant: 1.79 Pa*m3/mol (according to experimental database structure match: 15.50 Pa*m3/mol)

5.6 Additional information on environmental fate and behaviour

Endpoint study record: Additional information on environmental fate and behaviour, IUC4#1/Ch.1.9.1

| UUID |  | IUC4-4071dd78-bc62-3624-bac0-01e8ee900a46 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 1992-07-05 18:00:00 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BASF AG, Safety data sheet SEC.-BUTYLAMINE, 10.02.2005 |  |  |  |  |  |  |  |  |

Materials and methods

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 13952-84-6 |
| EC number | 237-732-7 |
| EC name | sec-butylamine |

Any other information on materials and methods incl. tables

ME-Freetext: DTA.

Results and discussions

Any other information on results incl. tables

EC Name: Thermal decomposition: > 500°C.

6 Ecotoxicological Information

6.1 Aquatic toxicity

6.1.1 Short-term toxicity to fish

Endpoint study record: KEY.BASF AG 85/144.Short-term toxicity to fish

| UUID |  | IUC5-544256be-5137-4c95-a157-ce59db29b4f6 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | Collette / Epona Associates, LLC / Willington / United States |
| Date |  | 2012-06-06 08:40:47 EDT |
| Remarks |  |  |

Administrative Data

| Purpose flag | key study; robust study summary; used for classification; used for MSDS |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Non-GLP guideline study |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | BASF AG | 1986 | sec-butylamin: Report on the study of the acute toxicity | unpublished data | Department of Toxicology |  | BASF SE | 85/144 | 1986-01-06 |

Data access

data submitter is data owner

Materials and methods

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | other guideline: German standard DIN 38412, part L15 |  |

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 13952-84-6 |

Details on test material

- Name of test material (as cited in study report): sec.-Butylamin - Analytical purity: not stated

Analytical monitoring

no

Vehicle

no

Details on test solutions

PREPARATION AND APPLICATION OF TEST SOLUTION (especially for difficult test substances) - Method: An aqueous solution of the test substance was prepared (1 g/100 ml).

Test organisms

Test organisms (species)

Leuciscus idus

Details on test organisms

TEST ORGANISM - Common name: Golden orfe - Source: Fischzucht Paul Eggers, Hohenwestedt, Germany - Length at study initiation (length definition, mean, range and SD): 7.3 cm (range: 6.0 - 8.2 cm) - Weight at study initiation (mean and range, SD): 3.2 g (range: 1.7 - 4.5) ACCLIMATION - Acclimation period: 3 days - Type and amount of food: "Fukosalm" Growing Feed 1, Club Kraftfutterwerke, Hamburg, Germany; ad libitum - Feeding frequency: daily - Health during acclimation (any mortality observed): no mortality observed

Study design

Test type

static

Water media type

freshwater

Limit test

no

Total exposure duration

96 h

Test conditions

Hardness

2.5 mmol/l

Test temperature

21 °C

pH

7.4 - 10.3 (not neutralised) 7.5 - 7.7 (neutralised)

Dissolved oxygen

6.6 - 9.0 mg/l

Nominal and measured concentrations

Nominal test concentrations (mg/l): 10, 14.7, 21.5, 31.6, 46.4, 68.1, 100, 100 (neutralised), control

Details on test conditions

TEST SYSTEM - Test vessel: glass aquaria (30 x 22 x 24 cm) - Type (delete if not applicable): open - Fill volume: 10 l - No. of organisms per vessel: 10 - No. of vessels per concentration (replicates): 1 - No. of vessels per control (replicates): 1 TEST MEDIUM / WATER PARAMETERS - Source/preparation of dilution water: reconstituted test water according to DIN 38412, part 11 - Ca/mg ratio: 4:1 - Conductivity: 10 µ MHO OTHER TEST CONDITIONS - Adjustment of pH: at highest test substance concentration (100 mg/l) - Photoperiod: 8:16 hours day night regime EFFECT PARAMETERS MEASURED (with observation intervals if applicable) : Mortality and symptoms of intoxication TEST CONCENTRATIONS - Spacing factor for test concentrations: 1.5 - Range finding study - Results used to determine the conditions for the definitive study: LC50 after 72 h >10 <100 mg/l

Results and discussions

Effect concentrations

| Duration | Endpoint | Effect conc. | Nominal/Measured | Conc. based on | Basis for effect | Remarks (e.g. 95% CL) |
| --- | --- | --- | --- | --- | --- | --- |
| 96 h | NOEC | 21.5 mg/L | nominal | test mat. | mortality |  |
| 96 h | LC0 | 31.6 mg/L | nominal | test mat. | mortality |  |
| 96 h | LC50 | 46 — 68 mg/L | nominal | test mat. | mortality |  |
| 96 h | LC100 | 68 mg/L | nominal | test mat. | mortality |  |

Details on results

symptoms: apathy, gasping, narcotic-like state, tumbling. The toxicity of sec. butylamine is probably due to changes in the ph caused by the substance since no effect was observed when the test substance was neutralised.

Any other information on results incl. tables

Cumulated mortality Nominal conc.(mg/l); Number of fish; Mortality (cumulated) after. 1 h; 4 h; 24 h; 48 h; 72 h; 96 h. 0; 10; 0; 0; 0; 0; 0; 0. 10; 10; 0; 0; 0; 0; 0; 0. 14.7; 10; 0; 0; 0; 0; 0; 0. 21.5; 10; 0; 0; 0; 0; 0; 0. 31.6; 10; 0; 0; 0; 0; 0; 0. 46.4; 10; 0; 0; 1; 1; 2; 2. 68.1; 10; 0; 10; 10; 10; 10; 10. 100; 10; 10; 10; 10; 10; 10; 10. 100*; 10; 0; 0; 0; 0; 0; 0. *: pH-adjusted symptoms: apathy, gasping, narcotic-like state, tumbling. The toxicity of sec. butylamine is probably due to changes in the ph caused by the substance since no effect was observed when the test substance was neutralised. pH values Nominal conc.(mg/l); pH values after. 1 h; 24 h; 48 h; 72 h; 96 h. 0; 7.2; 7.4; 7.6; 7.6; 7.6. 10; 8.2; 7.5; 7.6; 7.6; 7.6. 14.7; 8.7; 7.3; 7.4; 7.5; 7.5. 21.5; 9.0; 7.6; 7.6; 7.7; 7.7. 31.6; 9.3; 7.7; 7.8; 7.8; 7.8. 46.4; 9.6; 7.7; 7.7; 7.9; 7.8. 68.1; 10.1; 9.7. 100; 10.3; 10.0. 100*; 7.5; 7.5; 7.6; 7.6; 7.7. * pH adjusted

Endpoint study record: EPIWIN 2010, Short-term toxicity to fish

| UUID |  | IUC5-0513b0ca-2113-4dac-8058-17b38464a9f5 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | Collette / Epona Associates, LLC / Willington / United States |
| Date |  | 2012-07-02 16:10:12 EDT |
| Remarks |  |  |

Administrative Data

| Purpose flag | supporting study |  |  |
| --- | --- | --- | --- |
| Study result type | estimated by calculation |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Accepted calculation method |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| other: modelling |  | 2010 |  | U.S. Environmental Protection Agency (U.S. EPA) Estimation Programs Interface Suite,™. for Microsoft® Windows, v4.00. United States Environmental Protection Agency, Washington, DC, USA. |  |  |  |  |  |

Materials and methods

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test conditions

Any other information on materials and methods incl. tables

ECOSAR Program (v1.00): ============================== SMILES : C(C(C)N)C CHEM : sec-Butylamine MOL FOR: C4 H11 N1 MOL WT : 73.14 Log Kow: 0.76 (KowWin estimate) Melt Pt: -104.00 deg C Wat Sol: 1.12E+005 mg/L (measured)

Results and discussions

Effect concentrations

| Duration | Endpoint | Effect conc. | Nominal/Measured | Conc. based on | Basis for effect | Remarks (e.g. 95% CL) |
| --- | --- | --- | --- | --- | --- | --- |
| 96 h | LC50 | 71.5 mg/L |  |  |  |  |

Details on results

ECOSAR Program (v1.00) Results: ============================== ECOSAR v1.00 Class(es) Found ------------------------------ Aliphatic Amines Predicted ECOSAR Class Organism Duration End Pt mg/L (ppm) ============================================= Aliphatic Amines : Fish 96-hr LC50 71.458 ============================================= Neutral Organic SAR : Fish 96-hr LC50 629.561 (Baseline Toxicity) Aliphatic Amines: ---------------- For Fish 96-hr LC50: For aliphatic amines with log Kow greater than 7.0, a test duration of greater than 96 hrs may be required for proper expression of toxicity. Also, if the toxicity value obtained by the use of this equation exceeds the water solubility (measured or estimated), mortalities greater than 50% would not be expected in a saturated solution during an exposure period of 96 hrs. ECOSAR v1.00 SAR Limitations: ---------------------------- Maximum LogKow: 6.0 (Fish LC50) Maximum Mol Wt: 1000 Baseline Toxicity SAR Limitations: --------------------------------- Maximum LogKow: 5.0 (Fish 96-hr LC50) Maximum Mol Wt: 1000

Endpoint study record: Short-term toxicity to fish, IUC4#2/Ch.4.1

| UUID |  | IUC5-98b3d1e1-20df-43a9-ac35-77c16c12b6b6 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-13 10:49:24 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Worthing, Charles R. (ed.) in: The Pesticide Manual - A¦world compendium, 8th ed. Suffolk, S.1660, (1987) |  |  |  |  |  |  |  |  |

Materials and methods

Principles of method if other than guideline

Method: other: no data

GLP compliance

no data

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Details on test material

IUCLID4 Test substance: as prescribed by 1.1 - 1.4

Analytical monitoring

no data

Test organisms

Test organisms (species)

Lepomis macrochirus

Study design

Test type

other: no data

Results and discussions

Effect concentrations

| Duration | Endpoint | Effect conc. | Nominal/Measured | Conc. based on | Basis for effect | Remarks (e.g. 95% CL) |
| --- | --- | --- | --- | --- | --- | --- |
|  | LC50 | > 50 mg/L |  |  |  |  |

Overall remarks, attachments

Remarks on results including tables and figures

RM-Freetext: Literatur ohne Primaerquelle zitiert. Expositionszeit nicht angegeben.

Endpoint study record: Short-term toxicity to fish, IUC4#3/Ch.4.1

| UUID |  | IUC5-cb7ce5ab-c4d1-4bfd-bdd1-47e1e4e055ba |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-13 10:49:26 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Gillette, L.A. et al.: Water environ. research (Sewage Ind.¦Wastes) 24, 1397-1401 (1952) |  |  |  |  |  |  |  |  |

Materials and methods

Principles of method if other than guideline

Method: other: no data

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Details on test material

IUCLID4 Test substance: as prescribed by 1.1 - 1.4

Analytical monitoring

no

Test organisms

Test organisms (species)

Semolitus atromaculatus

Study design

Test type

static

Results and discussions

Effect concentrations

| Duration | Endpoint | Effect conc. | Nominal/Measured | Conc. based on | Basis for effect | Remarks (e.g. 95% CL) |
| --- | --- | --- | --- | --- | --- | --- |
| 24 h | LC50 | 20 — 50 mg/L |  |  |  |  |

Overall remarks, attachments

Remarks on results including tables and figures

RM-Freetext: Das Ergebnis von 20 bis 60 mg/l wurde als "critical range" bezeichnet, wobei unterhalb der angegebenen Spanne alle Fische ueberleben wuerden, oberhalb vollstaendige Mortalitaet eintreten wuerde.

6.1.3 Short-term toxicity to aquatic invertebrates

Endpoint study record: BASF AG 1/0177/2/88-0177/88.Short-term toxicity to aquatic invertebrates, IUC4#1/Ch.4.2

| UUID |  | IUC5-5e69a41a-04f1-49d2-aa18-b1545804557c |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2009-10-23 15:15:59 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| EU: REACH |  |  |  |
| --- | --- | --- | --- |
| Purpose flag | key study; robust study summary; used for classification; used for MSDS |  |  |
| Study result type | experimental result |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Guideline study; However, test item concentrations and stability not verified |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | BASF AG | 1988 | Bestimmung der akuten Wirkung von sec. Butylamin gegenueber dem Wasserfloh Daphnia magna Straus . | unpublished data | Department of Ecology |  | BASF SE | 1/0177/2/88-0177/88 | 1988-04-20 |

Data access

data submitter is data owner

Materials and methods

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | other guideline: Directive 79/831/EEC, Annex V, Part C |  |

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 13952-84-6 |

Details on test material

- Name of test material (as cited in study report): sec-butylamine - Analytical purity: >99 %

Analytical monitoring

no

Test organisms

Test organisms (species)

Daphnia magna

Details on test organisms

TEST ORGANISM - Source: originally obtained from the Institute National de Recherche Chimique Appliquee, France and cultured in the laboratories of BASF since 1978. - Age at study initiation (mean and range, SD): 2 - 24 h - Weight at study initiation (mean and range, SD): - Length at study initiation (length definition, mean, range and SD): - Valve height at study initiation, for shell deposition study (mean and range, SD): - Peripheral shell growth removed prior to test initiation: - Method of breeding: - Feeding during test - Food type: - Amount: - Frequency: ACCLIMATION - Acclimation period: - Acclimation conditions (same as test or not): - Type and amount of food: green algae daily, bakers yeast when water was changed - Feeding frequency: - Health during acclimation (any mortality observed): QUARANTINE (wild caught) - Duration: - Health/mortality:

Study design

Test type

static

Water media type

freshwater

Limit test

no

Total exposure duration

48 h

Test conditions

Hardness

2.7 +/- 0.5 mmol/l

pH

start: 8.31 - 10.83 termination: 7.61 - 8.75

Dissolved oxygen

8.44 - 8.86 mg/l

Nominal and measured concentrations

Nominal test concentration (mg/l): 1.95, 3.9, 7.8, 15.6, 31.2, 62.4, 125, 250, control

Details on test conditions

TEST SYSTEM - Test vessel: flatt bottom glass tubes - Fill volume: 10 ml - No. of organisms per vessel: 5 - No. of vessels per concentration (replicates): 4 - No. of vessels per control (replicates): 4 TEST MEDIUM / WATER PARAMETERS - Source/preparation of dilution water: reconstituted tap water - Ca/mg ratio: 4:1 - Conductivity: 550 - 650 µSiemens/cm OTHER TEST CONDITIONS - Photoperiod: 16:8 day-night regime - Light intensity: diffuse light EFFECT PARAMETERS MEASURED (with observation intervals if applicable) : Mobility after 0, 3, 6, 24 and 48 hours

Results and discussions

Effect concentrations

| Duration | Endpoint | Effect conc. | Nominal/Measured | Conc. based on | Basis for effect | Remarks (e.g. 95% CL) |
| --- | --- | --- | --- | --- | --- | --- |
| 48 h | EC0 | 7.8 mg/L | nominal | test mat. | mobility |  |
| 48 h | EC50 | 40.35 mg/L | nominal | test mat. | mobility |  |
| 48 h | EC100 | 125 mg/L | nominal | test mat. | mobility |  |

Details on results

The toxic effect at 62.4 - 250 mg/L may be due to changes in pH in the test vessels by the test substance.

Any other information on results incl. tables

Immobile daphnids after 48 hours (n = 20) Nominal conc.(mg/l); Immobile daphnids after 48hours. 0; 0. 1.95; 0. 3.9; 0. 7.8; 0. 15.6; 3. 31.2; 4. 62.4; 17. 125; 20. 250; 20. pH values Nominal conc.(mg/l); pH values after. 0 h; 48 h. 0; 8.06; 7.61. 1.95; 8.31; 7.65. 3.9; 8.53; 7.64. 7.8; 8.85; 7.67. 15.6; 9.22; 7.7. 31.2; 9.6; 7.79. 62.4; 10.04; 8.12. 125; 10.46; 8.25. 250; 10.83; 8.75.

Endpoint study record: EPIWIN 2010, Short-term toxicity to aquatic invertebrates

| UUID |  | IUC5-a18964c5-48f2-442d-836f-166ec4eceacf |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | Collette / Epona Associates, LLC / Willington / United States |
| Date |  | 2012-07-03 07:58:27 EDT |
| Remarks |  |  |

Administrative Data

| Purpose flag | supporting study |  |  |
| --- | --- | --- | --- |
| Study result type | estimated by calculation |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Accepted calculation method |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| other: modelling |  | 2010 |  | U.S. Environmental Protection Agency (U.S. EPA) (2010). Estimation Programs Interface Suite,™. for Microsoft® Windows, v4.00. United States Environmental Protection Agency, Washington, DC, USA. |  |  |  |  |  |

Materials and methods

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test conditions

Any other information on materials and methods incl. tables

ECOSAR Program (v1.00): ============================== SMILES : C(C(C)N)C CHEM : sec-Butylamine MOL FOR: C4 H11 N1 MOL WT : 73.14 Log Kow: 0.76 (KowWin estimate) Melt Pt: -104.00 deg C Wat Sol: 1.12E+005 mg/L (measured)

Results and discussions

Effect concentrations

| Duration | Endpoint | Effect conc. | Nominal/Measured | Conc. based on | Basis for effect | Remarks (e.g. 95% CL) |
| --- | --- | --- | --- | --- | --- | --- |
| 48 h | EC50 | 5.6 mg/L |  |  |  |  |

Details on results

ECOSAR Program (v1.00) Results: ============================== ECOSAR v1.00 Class(es) Found ------------------------------ Aliphatic Amines Predicted ECOSAR Class Organism Duration End Pt mg/L (ppm) ============================================= Aliphatic Amines : Daphnid 48-hr LC50 5.580 ============================================= Neutral Organic SAR (Baseline Toxicity) : Daphnid 48-hr LC50 312.756 Aliphatic Amines: ---------------- For Daphnid 48-hr LC50: For aliphatic amines with log Kow greater than 5.0, a test duration of greater than 48 hrs may be required for proper expression of toxicity. Also, if the toxicity value obtained by the use of this equation exceeds the water solubility (measured or estimated), significant mortalities would not be expected in a saturated solution during an exposure period of 48 hrs. ECOSAR v1.00 SAR Limitations: ---------------------------- Maximum LogKow: 5.0 (Daphnid LC50) Maximum Mol Wt: 1000 Baseline Toxicity SAR Limitations: --------------------------------- Maximum LogKow: 5.0 (Daphnid LC50) Maximum Mol Wt: 1000

6.1.5 Toxicity to aquatic algae and cyanobacteria

Endpoint study record: BASF AG 2/w177/88.Toxicity to aquatic algae and cyanobacteria

| UUID |  | IUC5-ac236423-ae36-48ae-8cac-1a4666b6ef67 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | Collette / Epona Associates, LLC / Willington / United States |
| Date |  | 2012-06-04 12:35:43 EDT |
| Remarks |  |  |

Administrative Data

| EU: REACH |  |  |  |
| --- | --- | --- | --- |
| Purpose flag | key study; robust study summary; used for classification; used for MSDS |  |  |
| Study result type | experimental result |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Non-GLP guideline study |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | BASF AG | 1989 | sec. Butylamin Tank 67: Algentest | unpublished data | Department of Ecology |  | BASF SE | 2/w177/88/t96 | 1989-02-02 |
| other company data | ECT Oekotoxikologie GmbH | 2007 | Alga, growth inhibition test (OECD 201, EEC, ISO): BASF AG, Algae test, Project No. 2/w177/88 | unpublished calculation | Department of Product Safety |  | BASF SE | 2/w177/88/t96 | 2007-11-04 |

Data access

data submitter is data owner

Materials and methods

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | other guideline: German standard DIN 38412, part 9 |  |

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 13952-84-6 |

Details on test material

- Name of test material (as cited in study report): sec-butylamine Tank 67

Analytical monitoring

no

Details on test solutions

PREPARATION AND APPLICATION OF TEST SOLUTION (especially for difficult test substances) - Method: stock solution No.: 10 mg/L (bidest. water; conductivity: 21 µS/cm, pH 10.2); stock solution was diluted in test medium to obtain the final test concentrations

Test organisms

Test organisms (species)

Scenedesmus subspicatus (new name: Desmodesmus subspicatus)

Details on test organisms

TEST ORGANISM - Strain: CHODAT 86.81

Study design

Test type

static

Water media type

freshwater

Limit test

no

Total exposure duration

96 h

Test conditions

Test temperature

23 °C

pH

pH range after 96 h: 8.1 (5 mg/L, 2.5 mg/L, inoculated) - 10.1 (control, inoculated)

Nominal and measured concentrations

Nominal test concentrations (mg/l): 0.0195, 0.039, 0.078, 0.156, 0.313, 0.625, 1.25, 2.5, 5.0 mg/L

Details on test conditions

TEST SYSTEM - Test vessel: 20 ml tubes plugged with gas permeable silicon-sponge caps - Type (delete if not applicable): open - Fill volume: 10 ml - Initial cells density: 10000 - No. of vessels per concentration (replicates): 5 - No. of vessels per control (replicates): 5 GROWTH MEDIUM - Standard medium used: OECD medium OTHER TEST CONDITIONS - Photoperiod: permanent artificial light - Light intensity and quality: 120 µE/(m2*s) EFFECT PARAMETERS MEASURED (with observation intervals if applicable) : - Determination of cell concentrations: fluorescence (in vivo chlorophyll-a fluorescence at 685 nm (pulsed excitation with light flashes having a wavelength of 435 nm)) after 0, 24, 48, 72 and 96 hours

Results and discussions

Effect concentrations

| Duration | Endpoint | Effect conc. | Nominal/Measured | Conc. based on | Basis for effect | Remarks (e.g. 95% CL) |
| --- | --- | --- | --- | --- | --- | --- |
| 72 h | NOEC | 0.156 mg/L | nominal | test mat. | growth rate |  |
| 72 h | LOEC | 0.313 mg/L | nominal | test mat. | growth rate |  |
| 72 h | EC10 | 0.255 mg/L | nominal | test mat. | growth rate | recalculated value |
| 72 h | EC50 | 2.031 mg/L | nominal | test mat. | growth rate | recalculated value |
| 72 h | NOEC | 0.156 mg/L | nominal | test mat. | biomass |  |
| 72 h | LOEC | 0.313 mg/L | nominal | test mat. | biomass |  |
| 72 h | EC10 | 0.123 mg/L | nominal | test mat. | biomass | recalculated value |
| 72 h | EC50 | 0.716 mg/L | nominal | test mat. | biomass | recalculated value |
| 72 h | EC50 | 0.52 mg/L | nominal | test mat. | other: yield |  |

Any other information on results incl. tables

Treatment [mg/L]; Growth rate inhibition [%]; Biomass decrease [%]; Yield Decrease [%]. Control; 0; 0; 0. 0.0195; -0.2; 2.3; -0.6. 0.039; 1.0; 3.9; 3.6. 0.078; 1.4; 7.2; 7.8. 0.156; 1.8; 7.6; 10.1. 0.313; 10.6; 27.9; 32.8. 0.625; 19.6; 45.5; 56.2. 1.25; 44.8; 71; 81.8. 2.5; 61.2; 83.1; 91. 5; 62.6; 83.5; 91.4. pH values Concentration [mg/L]; uninoculated; inoculated. 0 h; 96 h; 96 h. Control; 8.0; 8.0; 10.1. 0.0195; 8.0; 8.0; 10.0. 0.039; 8.0; 8.0; 9.9. 0.078; 8.0; 8.0; 9.5. 0.156; 8.0; 8.0; 9.5. 0.313; 8.0; 8.0; 9.2. 0.625; 8.0; 8.0; 8.6. 1.25; 8.0; 8.0; 8.2. 2.5; 8.0; 8.0; 8.1. 5; 8.0; 8.0; 8.1.

Endpoint study record: BASF AG 2/0177/88_a.Toxicity to aquatic algae and cyanobacteria

| UUID |  | IUC5-4a56b172-9824-4e2f-8471-1895e58e37a6 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | Collette / Epona Associates, LLC / Willington / United States |
| Date |  | 2012-06-04 12:40:17 EDT |
| Remarks |  |  |

Administrative Data

| EU: REACH |  |  |  |
| --- | --- | --- | --- |
| Purpose flag | supporting study; robust study summary; used for classification; used for MSDS |  |  |
| Study result type | experimental result |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Non-GLP guideline study |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | BASF AG | 1989 | sec. Butylamin Tank 67: Algentest | unpublished data | Department of Ecology |  | BASF SE | 2/0177/88/t96 | 1989-01-05 |
| other company data | ECT Oekotoxikologie GmbH | 2002 | Alga, growth inhibition test (OECD 201, EEC, ISO): BASF AG, Algae test, Project No. 2/0177/88 | unpublished calculation | ECT Oekotoxikologie GmbH |  | BASF SE |  | 2007-11-04 |

Data access

data submitter is data owner

Materials and methods

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | other guideline: German standard DIN 38412, part 9 |  |

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 139952-84-6 |

Details on test material

- Name of test material (as cited in study report): sec-butylamine Tank 67

Analytical monitoring

no

Details on test solutions

PREPARATION AND APPLICATION OF TEST SOLUTION (especially for difficult test substances) - Method: stock solution No.: 10 ml of stock solution (1000 mg/l) diluted in 1l (bidest. water; conductivity: 26 µS/cm, pH 9.6); stock solution was diluted in test medium to obtain the final test concentrations

Test organisms

Test organisms (species)

Scenedesmus subspicatus (new name: Desmodesmus subspicatus)

Details on test organisms

TEST ORGANISM - Strain: CHODAT 86.81

Study design

Test type

static

Water media type

freshwater

Limit test

no

Total exposure duration

96 h

Test conditions

pH

pH range after 96 h: 8.1 (5 mg/L, 2.5 mg/L, inoculated) - 10.1 (control, inoculated)

Nominal and measured concentrations

Nominal test concentrations (mg/l): 0.0195, 0.039, 0.078, 0.156, 0.313, 0.625, 1.25, 2.5, 5.0 mg/L

Details on test conditions

TEST SYSTEM - Test vessel: 20 ml tubes plugged with gas permeable silicon-sponge caps - Type (delete if not applicable): open - Fill volume: 10 ml - Initial cells density: 10000 - No. of vessels per concentration (replicates): 5 - No. of vessels per control (replicates): 5 GROWTH MEDIUM - Standard medium used: OECD medium OTHER TEST CONDITIONS - Photoperiod: permanent artificial light - Light intensity and quality: 120 µE/(m2*s) EFFECT PARAMETERS MEASURED (with observation intervals if applicable) : - Determination of cell concentrations: fluorescence (in vivo chlorophyll-a fluorescence at 685 nm (pulsed excitation with light flashes having a wavelength of 435 nm)) after 0, 24, 48, 72 and 96 hours

Results and discussions

Effect concentrations

| Duration | Endpoint | Effect conc. | Nominal/Measured | Conc. based on | Basis for effect | Remarks (e.g. 95% CL) |
| --- | --- | --- | --- | --- | --- | --- |
| 72 h | NOEC | 0.625 mg/L | nominal | test mat. | growth rate |  |
| 72 h | LOEC | 1.25 mg/L | nominal | test mat. | growth rate |  |
| 72 h | EC10 | 1.684 mg/L | nominal | test mat. | growth rate | recalculated value |
| 72 h | EC50 | 7.786 mg/L | nominal | test mat. | growth rate | recalculated value |
| 72 h | NOEC | 0.625 mg/L | nominal | test mat. | biomass |  |
| 72 h | LOEC | 1.25 mg/L | nominal | test mat. | biomass |  |
| 72 h | EC10 | 1.014 mg/L | nominal | test mat. | biomass | recalculated value |
| 72 h | EC50 | 5.006 mg/L | nominal | test mat. | biomass | recalculated value |
| 72 h | EC50 | 3.14 mg/L | nominal | test mat. | other: yield |  |

Any other information on results incl. tables

Treatment [mg/L]; Growth rate inhibition [%]; Biomass decrease [%]; Yield Decrease [%]. Control; 0; 0; 0. 0.02; -5.7; -17.1; -24. 0.039; -1.6; -9.3; -12.1. 0.078; -5.1; -13.9; -21.9. 0.156; -8.3; -19.4; -33. 0.313; -0.9; -0.4; -7.5. 0.625; 0.6; 1.5; 1.5. 1.147; 8.2; 14.3; 20. 2.5; 15.9; 30.9; 40.1. 5; 35.9; 48.6; 68.2. pH values Concentration [mg/L]; uninoculated; inoculated. 0 h; 96 h; 96 h. Control; 8.3; 8.1; 9.7. 0.0195; 8.3; 8.0; 10.1. 0.039; 8.3; 8.0; 10.0. 0.078; 8.3; 8.0; 10.1. 0.156; 8.3; 8.0; 9.9. 0.313; 8.3; 8.0; 9.9. 0.625; 8.3; 8.0; 9.4. 1.25; 8.3; 8.0; 8.9. 2.5; 8.3; 8.0; 8.5. 5; 8.5; 8.1; 9.7.

Endpoint study record: BASF AG 2/0177/88_b.Toxicity to aquatic algae and cyanobacteria

| UUID |  | IUC5-a5d4a65e-0fb2-4879-91c9-af0fac6862f0 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | Collette / Epona Associates, LLC / Willington / United States |
| Date |  | 2012-06-04 12:38:57 EDT |
| Remarks |  |  |

Administrative Data

| EU: REACH |  |  |  |
| --- | --- | --- | --- |
| Purpose flag | supporting study; robust study summary; used for classification; used for MSDS |  |  |
| Study result type | experimental result |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Non-GLP guideline study |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | BASF AG | 1988 | sec. Butylamin Tank 67: Algentest | unpublished data | Department of Ecology |  | BASF SE | 2/0177/88/t96 | 1988-10-20 |
| other company data | ECT Oekotoxikologie GmbH | 2002 | Alga, growth inhibition test (OECD 201, EEC, ISO): BASF AG, Algae test, Project No. 2/0177/88 | unpublished calculation | ECT Oekotoxikologie GmbH |  | BASF SE |  | 2007-11-01 |

Data access

data submitter is data owner

Materials and methods

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | other guideline: German standard DIN 38412, part 9 |  |

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 139952-84-6 |

Details on test material

- Name of test material (as cited in study report): sec-butylamine Tank 67

Analytical monitoring

no

Details on test solutions

PREPARATION AND APPLICATION OF TEST SOLUTION (especially for difficult test substances) - Method: stock solution conductivity: 62 µS/cm, pH 10.5; stock solution was diluted in test medium to obtain the final test concentrations

Test organisms

Test organisms (species)

Scenedesmus subspicatus (new name: Desmodesmus subspicatus)

Details on test organisms

TEST ORGANISM - Strain: CHODAT 86.81

Study design

Test type

static

Water media type

freshwater

Limit test

no

Total exposure duration

96 h

Test conditions

pH

pH range after 96 h: 8.1 (5 mg/L, 2.5 mg/L, inoculated) - 10.1 (control, inoculated)

Nominal and measured concentrations

Nominal test concentrations (mg/l): 0.156, 0.313, 0.625, 1.25, 2.5, 5.0, 10, 20 mg/L

Details on test conditions

TEST SYSTEM - Test vessel: 20 ml tubes plugged with gas permeable silicon-sponge caps - Type (delete if not applicable): open - Fill volume: 10 ml - Initial cells density: 10000 - No. of vessels per concentration (replicates): 5 - No. of vessels per control (replicates): 5 GROWTH MEDIUM - Standard medium used: OECD medium OTHER TEST CONDITIONS - Photoperiod: permanent artificial light - Light intensity and quality: 120 µE/(m2*s) EFFECT PARAMETERS MEASURED (with observation intervals if applicable) : - Determination of cell concentrations: fluorescence (in vivo chlorophyll-a fluorescence at 685 nm (pulsed excitation with light flashes having a wavelength of 435 nm)) after 0, 24, 48, 72 and 96 hours

Results and discussions

Effect concentrations

| Duration | Endpoint | Effect conc. | Nominal/Measured | Conc. based on | Basis for effect | Remarks (e.g. 95% CL) |
| --- | --- | --- | --- | --- | --- | --- |
| 72 h | NOEC | 0.156 mg/L | nominal | test mat. | growth rate |  |
| 72 h | LOEC | 0.313 mg/L | nominal | test mat. | growth rate |  |
| 72 h | EC10 | 0.301 mg/L | nominal | test mat. | growth rate | recalculated value |
| 72 h | EC50 | 4.376 mg/L | nominal | test mat. | growth rate | recalculated value |
| 72 h | EC10 | 0.144 mg/L | nominal | test mat. | biomass | recalculated value |
| 72 h | EC50 | 1.096 mg/L | nominal | test mat. | biomass | recalculated value |
| 72 h | NOEC | 0.156 mg/L | nominal | test mat. | biomass |  |
| 72 h | LOEC | 0.313 mg/L | nominal | test mat. | biomass |  |
| 72 h | EC50 | 0.69 mg/L | nominal | test mat. | other: yield |  |

Any other information on results incl. tables

Treatment [mg/L]; Growth rate inhibition [%]; Biomass decrease [%]; Yield Decrease [%]. Control; 0; 0; 0. 0.156; 2.8; -1.9; 9.0. 0.313; 9.2; 26.9; 34.2. 0.625; 15.4; 36; 45.9. 1.25; 26.2; 53.8; 66.3. 2.5; 43.1; 74.4; 83.3. 5; 56.7; 83.7; 91.3. 10; 66.3; 87.5; 93.6. 20; 70.3; 86.6; 95. pH values Concentration [mg/L]; uninoculated; inoculated. 0 h; 96 h; 96 h. Control; 7.9; 8.3; 9.9. 0.156; 8.0; 8.3; 9.8. 0.313; 8.0; 8.2; 9.5. 0.625; 8.0; 8.2; 9.3. 1.25; 8.0; 8.2; 9.1. 2.5; 8.1; 8.2; 8.8. 5; 8.3; 8.2; 8.5. 10; 8.5; 8.2; 8.4. 20; 8.9; 8.2; 8.4.

Endpoint study record: EPIWIN 2010, Toxicity to aquatic algae and cyanobacteria

| UUID |  | IUC5-c925215f-951a-47a3-ad3c-8a1978971058 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | Collette / Epona Associates, LLC / Willington / United States |
| Date |  | 2012-07-02 16:23:47 EDT |
| Remarks |  |  |

Administrative Data

| Purpose flag | supporting study |  |  |
| --- | --- | --- | --- |
| Study result type | estimated by calculation |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Accepted calculation method |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| other: modelling |  | 2010 |  | U.S. Environmental Protection Agency (U.S. EPA) (2010). Estimation Programs Interface Suite,™. for Microsoft® Windows, v4.00. United States Environmental Protection Agency, Washington, DC, USA. |  |  |  |  |  |

Materials and methods

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test conditions

Any other information on materials and methods incl. tables

ECOSAR Program (v1.00) Results: ============================== SMILES : C(C(C)N)C CHEM : sec-Butylamine MOL FOR: C4 H11 N1 MOL WT : 73.14 Log Kow: 0.76 (KowWin estimate) Melt Pt: -104.00 deg C Wat Sol: 1.12E+005 mg/L (measured)

Results and discussions

Effect concentrations

| Duration | Endpoint | Effect conc. | Nominal/Measured | Conc. based on | Basis for effect | Remarks (e.g. 95% CL) |
| --- | --- | --- | --- | --- | --- | --- |
| 96 h | EC50 | 1.6 mg/L |  |  |  |  |

Details on results

ECOSAR Program (v1.00) Results: ============================== ECOSAR v1.00 Class(es) Found ------------------------------ Aliphatic Amines Predicted ECOSAR Class Organism Duration End Pt mg/L (ppm) ============================================= Aliphatic Amines : Green Algae 96-hr EC50 1.601 ============================================= Neutral Organic SAR (Baseline Toxicity) : Green Algae 96-hr EC50 81.109 Aliphatic Amines: ---------------- For Green Algae Acute Toxicity Values: If the log Kow of the chemical is greater than 7, or if the compound is solid and the EC50 exceeds the water solubility by 10X, no effects at saturation are predicted for these endpoints. ECOSAR v1.00 SAR Limitations: ---------------------------- Maximum LogKow: 7.0 (Green Algae EC50) Maximum Mol Wt: 1000 Baseline Toxicity SAR Limitations: --------------------------------- Maximum LogKow: 6.4 (Green Algae EC50) Maximum Mol Wt: 1000

6.1.7 Toxicity to microorganisms

Endpoint study record: BASF AG 9/0177/88.Toxicity to microorganisms

| UUID |  | IUC5-99a2519c-1ef1-41b0-8f7a-48dd8ae9b71c |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-13 10:49:32 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Purpose flag | robust study summary |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Non-GLP guideline study |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | BASF AG | 1988 | sec. Butylmamin: Wachstumshemmtest in Anlehnung an Bringmann-Kuehn | unpublished data | Department of Ecology |  | BASF SE | 9/0177/88 | 1988-03-22 |

Data access

data submitter is data owner

Materials and methods

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| equivalent or similar to | other guideline: German standard DIN 38412, part 8 |  |

Principles of method if other than guideline

Method: other: DIN 38412 Part 8, zum Gelbdruck verabschiedet, Bestimmung der Hemmwirkung von Wasserinhaltsstoffen auf Bakterien

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 13952-84-6 |

Details on test material

- Name of test material (as cited in study report): sec-butylamine - Analytical purity: >99 %

Analytical monitoring

no

Test organisms

Test organisms (species)

Pseudomonas putida

Details on inoculum

- Initial biomass concentration: bacterial solution 10 TE/F

Study design

Test type

static

Water media type

freshwater

Limit test

no

Total exposure duration

17 h

Test conditions

Nominal and measured concentrations

Nominal test concentrations (mg/l): 7.81, 15.63, 31.25, 62.5, 125, 250, 500, 1000, control

Details on test conditions

TEST SYSTEM - Test vessel: Erlenmeyer vessel (300 ml) - Fill volume: 100 ml - No. of vessels per concentration (replicates): 4 - No. of vessels per control (replicates): 4 TEST MEDIUM / WATER PARAMETERS - Source/preparation of dilution water: Ak Medium according to DIN 38412, part 8, draft

Results and discussions

Effect concentrations

| Duration | Endpoint | Effect conc. | Nominal/Measured | Conc. based on | Basis for effect | Remarks (e.g. 95% CL) |
| --- | --- | --- | --- | --- | --- | --- |
| 17 h | EC10 | 70 mg/L | nominal | test mat. | growth inhibition |  |
| 17 h | EC50 | 100 mg/L | nominal | test mat. | growth inhibition |  |
| 17 h | other: EC90 | 160 mg/L | nominal | test mat. | growth inhibition |  |

Any other information on results incl. tables

pH values Concentration [mg/L]; uninoculated; inoculated. 0 h; 17 h; 17 h. Control; 7.0; 5.0; 7.0. 7.81; 7.2; 4.9; 7.1. 15.63; 7.4; 5.0; 7.3. 31.25; 7.9; 5.0; 7.5. 62.5; 9.2; 5.9; 7.8. 125; 10.1; 7.3; 8.1. 250; 10.4; 8.4; 8.5. 500; 10.7; 9.7; 9.3. 1000; 10.9; 9.9; 9.9.

Endpoint study record: key BASF AG 1250.Toxicity to microorganisms

| UUID |  | IUC5-1c51a711-7226-4f01-aea8-706075901d5e |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2009-10-23 15:16:09 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| EU: REACH |  |  |  |
| --- | --- | --- | --- |
| Purpose flag | key study; robust study summary; used for classification; used for MSDS |  |  |
| Study result type | experimental result |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Non GLP guideline study |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | BASF AG | 1980 | sec. Butylamin: Kurzzeitatmungstest | unpublished data | Department of Ecology |  | BASF SE | 1250 | 1980-11-24 |

Data access

data submitter is data owner

Materials and methods

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | OECD Guideline 209 (Activated Sludge, Respiration Inhibition Test) |  |

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 13952-84-6 |

Details on test material

- Name of test material (as cited in study report): sec. Butylamine

Analytical monitoring

no

Test organisms

Test organisms (species)

activated sludge, industrial

Details on inoculum

Activated sludge taken from a industrial waste water treatment plant. - Initial biomass concentration: 1000 mg/l

Study design

Test type

static

Water media type

freshwater

Limit test

no

Total exposure duration

30 min

Test conditions

Nominal and measured concentrations

Nominal test concentrations (mg/l): 150, 1995, control

Results and discussions

Effect concentrations

| Duration | Endpoint | Effect conc. | Nominal/Measured | Conc. based on | Basis for effect | Remarks (e.g. 95% CL) |
| --- | --- | --- | --- | --- | --- | --- |
| 30 min | other: EC20 | > 1995 mg/L | nominal | test mat. | respiration rate |  |

Endpoint study record: Toxicity to microorganisms, IUC4#3/Ch.4.4

| UUID |  | IUC5-28631200-30b0-4015-a1af-0da2aeee78a9 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-13 10:49:34 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Hockenbury,M.R., Grady Jr.,C.P.L., JWPCF 49(1), 768-777,¦(1977) |  |  |  |  |  |  |  |  |

Materials and methods

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test organisms

Test organisms (species)

Nitrosomonas sp.

Results and discussions

Effect concentrations

| Duration | Endpoint | Effect conc. | Nominal/Measured | Conc. based on | Basis for effect | Remarks (e.g. 95% CL) |
| --- | --- | --- | --- | --- | --- | --- |
| 2 h | EC10 | > 100 mg/L |  |  |  |  |

Overall remarks, attachments

Remarks on results including tables and figures

RM-Freetext: Keine Hemmung der Ammonium-Oxidation bei 100 mg/l.

Endpoint study record: Toxicity to microorganisms, IUC4#4/Ch.4.4

| UUID |  | IUC5-67af1f37-b012-4a9d-86c8-77150ef2f46a |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-13 10:49:35 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BASF AG, Sicherheitsdatenblatt (Sek.-Butylamin) |  |  |  |  |  |  |  |  |

Materials and methods

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test organisms

Test organisms (species)

activated sludge

Overall remarks, attachments

Remarks on results including tables and figures

RM-Freetext: Bei sachgemaesser Einleitung in adaptierte biologische Klaer- anlagen sind keine Stoerungen der Abbauaktivitaet des Belebt- schlamms zu erwarten.

6.6 Additional ecotoxicological information

Endpoint study record: Additional ecotoxicological information, IUC4#1/Ch.4.9

| UUID |  | IUC5-44635e36-15d6-4d5c-8317-0dcc7c6d5ee7 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-13 10:49:36 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BASF AG, Labor Oekologie; unveroeffentlichte Untersuchung,¦(0177/88) |  |  |  |  |  |  |  |  |

Materials and methods

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Results and discussions

Any other information on results incl. tables

Memo: Der konzentrationsabhaengig steigende pH-Wert duerfte fuer die Toxizitaet mitverantwortlich sein.

7 Toxicological information

7.1 Toxicokinetics, metabolism and distribution

7.1.1 Basic toxicokinetics

Endpoint study record: Basic toxicokinetics, IUC4#2/Ch.5.11

| UUID |  | IUC5-7dda2ae2-cd4c-4375-8bf6-124036f6923f |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-13 10:49:37 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Eli Lilly Co., unveroeffentlichte Untersuchung (1965).¦Zitiert in: Pesticide residues in food, FAO plant ¦protection paper 1, 59-82, (1975) |  |  |  |  |  |  |  |  |

Materials and methods

Objective of study

metabolism

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Details on test material

TS-Freetext: sec-Butylamin

Overall remarks, attachments

Remarks on results including tables and figures

RM-Freetext: Es wurden Urinproben von zwei Hunden, die taeglich mit 5000 bzw. 10000 ppm der Testsubstanz im Futter behandelt wurden, angesaeuert und destilliert. Es konnte ein Diphenylhydrazon nachgewiesen werden, das auf Methylethylketon als Stoffwechselprodukt (Deaminierung der Testsubstanz) schliessen liess.

7.2 Acute Toxicity

7.2.1 Acute toxicity: oral

Endpoint study record: BASFAGXVII/313.Acute toxicity, oral to rat

| UUID |  | IUC5-027a3e5e-b797-4845-9f6a-17e57e5ff43d |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | Collette / Epona Associates, LLC / Willington / United States |
| Date |  | 2012-06-06 15:44:19 EDT |
| Remarks |  |  |

Administrative Data

| Purpose flag | supporting study |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Meets generally accepted scientific standards, well documented and acceptable for assessment |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | BASF AG | 1967 | Bericht über die gewerbetoxikologische Grundprüfung |  | BASF AG, Department of Toxicology | XVII/313 | BASF SE |  | 1967-11-13 |

Data access

data submitter is data owner

Materials and methods

Test type

standard acute method

Limit test

no

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| equivalent or similar to | OECD Guideline 401 (Acute Oral Toxicity) |  |

Principles of method if other than guideline

In principle, the methods described in the OECD Guideline 401 were used. Young adult laboratory rats were purchased from breeder. Several groups of 5 rats per sex and dose were treated simultaneously by gavage with preparations of the test substance in suitable vehicle. The concentrations of these preparations were used to achieve comparable volumes per kg body weight. Group-wise documentation of clinical signs was performed over the 14 day study period.

GLP compliance

no (; GLP was not compulsory at the time the study was conducted )

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 13952-84-6 |

Details on test material

- Name of test material (as cited in study report): sec. Butylamin - Physical state: liquid - Analytical purity: 98.5 %

Test animals

Species

rat

Strain

other: US rats

Sex

male/female

Details on test animals and environmental conditions

TEST ANIMALS - Weight at study initiation: average body weight, males: ca. 207 g average body weight, females: ca. 165 g

Administration / exposure

Route of administration

oral: gavage

Vehicle

other: aqueous suspension with Traganth, Cremophor EL

Details on oral exposure

VEHICLE - Concentration in vehicle: 20 % for 1600 µl/kg bw, 10 % for 1000 µl/kg bw, 8 % for 800 and 640 µl/kg bw, 4 % for 400 µg/kg bw and 2 % for 200 µl/kg bw - Amount of vehicle: 10 ml/kg bw for 1000, 800, 400 and 200 µl/kg bw and 8 ml/kg bw for 1600 and 640 µl/kg bw.

Doses

1600, 1000, 800, 640, 400 and 200 µl/kg bw => 1163, 727, 581, 465, 291 and 145 mg/kg bw

No. of animals per sex per dose

5 animals per sex per dose

Control animals

no

Details on study design

Duration of observation period following administration: 14 days - Frequency of observations and weighing: Animals were observed and examined for clinical signs of toxicity during the first hour following application, after 5 hours and further on day 1, 3, 4, 5 after dosing. On day 7 after dosing the surviving animals were sacrificed. Body weights were determined prior to application of the test material only. - Necropsy of survivors performed: yes Deceased animals and those sacrificed at the end of the observation period were necropsied.

Results and discussions

Effect levels

| Sex | Endpoint | Effect level | Based on | 95% CL | Remarks |
| --- | --- | --- | --- | --- | --- |
| male/female | LD50 | 545 mg/kg bw |  |  |  |

Mortality

1163 mg/kg bw: Within 24 h after administration of the test material 10/10 animals were found dead. 727 mg/kg bw: Within 24 h after administration of the test material 4/10 (1 m and 3 f) animals were dead. After 48 hours 6/10 (2 m and 4 f) animals had died and after 7 days 7/10 (2 m and 5 f) animals were found dead. 581 mg/kg bw: Within 24 h after administration of the test material 2/10 (2 m and o f) animals were found dead. After 48 hours 6/10 (3 m and 3 f) animals had died. No further dead animals were found until day 7. 465 mg/kg bw: Within 48 h after administration of the test material 2/10 (0 m and 2 f) animals were found dead. No further dead animals were found until day 7. 291 mg/kg bw: No mortality occured until day 7. 145 mg/kg bw: No mortality occured until day 7.

Clinical signs

After application of the test material the animals were in a ventral position, dyspnea, atony and slight apathy were seen. After 5 hours the animals were found in a crouched down position, partly in a lateral position and with a toddling, staggering walk. Apathy was seen and at the high dose groups an anesthesia-like condition was observed. After one day the surviving animals were still found in a crouched down position, intermittent breath, bloody incrusted eyes and noses and slight apathy were seen. The findings were of lower intensity in the lower dose groups. The surviving animals recovered soon and after 3 to 5 days they were without any findings.

Body weight

The body weight of the individual animals was recorded prior to administration of the test material only.

Gross pathology

At gross pathology of the animals that died during the study, the following findings were seen: All animals had bloody and serous scab around the snouts, the intestines were expanded and there was thin fluid found inside. Inside the stomach aqueous, bloody content was found. Diahrrea was seen and in one case each agglutination and gastrorrhagia were observed. At gross pathology of the surviving animals that were sacrificed after 7 days, the following findings were seen: In 3 cases inflamed adhesions of the stomach, spleen and liver were seen with the abdominal wall. It was unclear according to the authors, if this effect was caused by the gavage procedure.

Any other information on results incl. tables

Mortality Dose (µl/kg); Conc. (%); No. of animls; died within 1 hour 24 hours 48 hours 7 days. 1600; 20; 10; 0/10; 10/10; 10/10; 10/10. 1000; 10; 10; 0/10; 4/10; 6/10; 7/10. 800; 8; 10; 0/10; 2/10; 6/10; 6/10. 640; 8; 10; 0/10; 0/10; 2/10; 2/10. 400; 4; 10; 0/10; 0/10; 0/10; 0/10. 2000; 2; 10; 0/10; 0/10; 0/10; 0/10.

Overall remarks, attachments

Remarks on results including tables and figures

Under the conditions of the BASF test (similar to OECD guideline 401) for acute toxicity after oral application the LD50 was 545 mg/kg body weight for male and female rats. Therefore, the test material has to be classified as Xi R22 according to EU criteria and as Category 4 according to GHS criteria. Original data: LD50= 700 µl/kg body weight.

Endpoint study record: Acute toxicity: oral, IUC4#2/Ch.5.1.1

| UUID |  | IUC5-33e3a878-17ef-4925-8bdd-b4076e42b7c2 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | Collette / Epona Associates, LLC / Willington / United States |
| Date |  | 2012-06-06 15:41:29 EDT |
| Remarks |  |  |

Administrative Data

| Purpose flag | supporting study |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Peer-reviewed publication |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Cheever, K.L. et al. | 1982 | The acute oral toxicity of isomeric monobutylamines in the adult male and female rat. | Toxicology and Applied Pharmacology 63, 150-152 |  |  |  |  |  |
|  | RTECS, update 8910: Toxicol. Appl. Pharmacol. 63, 150 (1982) |  |  |  |  |  |  |  |  |

Materials and methods

Test type

standard acute method

Principles of method if other than guideline

Method: other

GLP compliance

no data

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Details on test material

IUCLID4 Test substance: as prescribed by 1.1 - 1.4

Test animals

Species

rat

Administration / exposure

Details on oral exposure

Rats dosed with sec-butylamine in corn oil via gavage

Doses

100, 200, 300, 400, 500 or 600 mg/kg bw

No. of animals per sex per dose

10/sex/dose

Details on study design

Post dosing observation period of 14 days

Results and discussions

Effect levels

| Sex | Endpoint | Effect level | Based on | 95% CL | Remarks |
| --- | --- | --- | --- | --- | --- |
| male/female | LD50 | 152 mg/kg bw |  |  |  |

Mortality

The actual number of deaths was not reported, but they generally occurred within 1-3 hours post-dosing.

Clinical signs

Clinical signs included sedation, ataxia, nasal discharge, gasping, salivation and convulsions.

Gross pathology

At necropsy, decedent animals exhibited pulmonary edema.

Endpoint study record: Acute toxicity: oral, IUC4#3/Ch.5.1.1

| UUID |  | IUC5-1d6b3d1b-1dec-497d-914f-d8b25ea3ea2d |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-13 10:49:41 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | RTECS, update 8910 |  |  |  |  |  |  |  |  |
|  | The Pesticide Manual, Seite 64 (1977) |  |  |  |  |  |  |  |  |

Materials and methods

Principles of method if other than guideline

Method: other: no data

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Details on test material

IUCLID4 Test substance: as prescribed by 1.1 - 1.4

Test animals

Species

dog

Results and discussions

Effect levels

| Sex | Endpoint | Effect level | Based on | 95% CL | Remarks |
| --- | --- | --- | --- | --- | --- |
|  | LD50 | 225 mg/kg bw |  |  |  |

Endpoint study record: Acute toxicity: oral, IUC4#4/Ch.5.1.1

| UUID |  | IUC5-5824a6d2-798f-4bae-81a0-f7737089c08a |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-13 10:49:42 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Worthing, Charles R. (ed.) in: The Pesticide Manual - A¦world compendium, 8th ed. Suffolk, S.1660, (1987) |  |  |  |  |  |  |  |  |

Materials and methods

Principles of method if other than guideline

Method: other: no data

GLP compliance

no data

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Details on test material

IUCLID4 Test substance: as prescribed by 1.1 - 1.4

Test animals

Species

rat

Results and discussions

Effect levels

| Sex | Endpoint | Effect level | Based on | 95% CL | Remarks |
| --- | --- | --- | --- | --- | --- |
|  | LD50 | 380 mg/kg bw |  |  |  |

Endpoint study record: Acute toxicity: oral, IUC4#5/Ch.5.1.1

| UUID |  | IUC5-f9e42ee6-ee16-4a25-b88e-89c9d7a6ed45 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-13 10:49:43 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Eli, Lilly and Co., unveroeffentlichte Untersuchung,¦(1965);¦Zitiert in: Pesticide residues in food, FAO plant¦protection paper 1, 59-82, (1975) |  |  |  |  |  |  |  |  |

Materials and methods

Principles of method if other than guideline

Method: other: no data

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Details on test material

IUCLID4 Test substance: as prescribed by 1.1 - 1.4

Test animals

Species

mouse

Results and discussions

Effect levels

| Sex | Endpoint | Effect level | Based on | 95% CL | Remarks |
| --- | --- | --- | --- | --- | --- |
|  | LD50 | 660 mg/kg bw |  |  |  |

Endpoint study record: Acute toxicity: oral, IUC4#6/Ch.5.1.1

| UUID |  | IUC5-e9cfbcfe-8e8a-455e-9020-95fc93b0805a |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-13 10:49:44 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Eli, Lilly and Co., unveroeffentlichte Untersuchung,¦(1965);¦Zitiert in: Pesticide residues in food, FAO plant¦protection paper 1, 59-82, (1975) |  |  |  |  |  |  |  |  |

Materials and methods

Principles of method if other than guideline

Method: other: no data

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Details on test material

IUCLID4 Test substance: as prescribed by 1.1 - 1.4

Test animals

Species

rat

Results and discussions

Effect levels

| Sex | Endpoint | Effect level | Based on | 95% CL | Remarks |
| --- | --- | --- | --- | --- | --- |
|  | LD50 | 350 mg/kg bw |  |  |  |

Endpoint study record: Acute toxicity: oral, IUC4#7/Ch.5.1.1

| UUID |  | IUC5-ba46fc7d-ed0b-4f80-b067-c46107ab198b |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-13 10:49:45 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Eli, Lilly and Co., unveroeffentlichte Untersuchung,¦(1965);¦Zitiert in: Pesticide residues in food, FAO plant¦protection paper 1, 59-82, (1975) |  |  |  |  |  |  |  |  |

Materials and methods

Principles of method if other than guideline

Method: other: no data

GLP compliance

no

Test materials

Details on test material

IUCLID4 Test substance: other TS: sec-Butylamin Acetat, - Phosphat, - Carbonat und - Hydrochlorid

Test animals

Species

mouse

Results and discussions

Effect levels

| Sex | Endpoint | Effect level | Based on | 95% CL | Remarks |
| --- | --- | --- | --- | --- | --- |
|  | LD50 | 1750 — 2470 mg/kg bw |  |  |  |

Endpoint study record: Acute toxicity: oral, IUC4#8/Ch.5.1.1

| UUID |  | IUC5-783580e2-7c10-4aac-a3c4-d2148bff2402 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-13 10:49:46 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Eli, Lilly and Co., unveroeffentlichte Untersuchung,¦(1965);¦Zitiert in: Pesticide residues in food, FAO plant¦protection paper 1, 59-82, (1975) |  |  |  |  |  |  |  |  |

Materials and methods

Principles of method if other than guideline

Method: other: no data

GLP compliance

no

Test materials

Details on test material

IUCLID4 Test substance: other TS: sec-Butylamin Acetat, - Phosphat, - Carbonat und - Hydrochlorid

Test animals

Species

guinea pig

Results and discussions

Effect levels

| Sex | Endpoint | Effect level | Based on | 95% CL | Remarks |
| --- | --- | --- | --- | --- | --- |
|  | LD50 | 880 mg/kg bw |  |  |  |

Endpoint study record: Acute toxicity: oral, IUC4#9/Ch.5.1.1

| UUID |  | IUC5-2de41a2c-b355-4810-ae12-dd5375bbd2b7 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-13 10:49:48 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Eli, Lilly and Co., unveroeffentlichte Untersuchung,¦(1965);¦Zitiert in: Pesticide residues in food, FAO plant¦protection paper 1, 59-82, (1975) |  |  |  |  |  |  |  |  |

Materials and methods

Principles of method if other than guideline

Method: other: no data

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Details on test material

IUCLID4 Test substance: as prescribed by 1.1 - 1.4

Test animals

Species

dog

Results and discussions

Effect levels

| Sex | Endpoint | Effect level | Based on | 95% CL | Remarks |
| --- | --- | --- | --- | --- | --- |
|  | LD50 | 250 mg/kg bw |  |  |  |

Endpoint study record: Acute toxicity: oral, IUC4#10/Ch.5.1.1

| UUID |  | IUC5-7d319a11-861d-442d-b39b-e46a5ac19dd7 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-13 10:49:49 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Eli, Lilly and Co., unveroeffentlichte Untersuchung,¦(1965);¦Zitiert in: Pesticide residues in food, FAO plant¦protection paper 1, 59-82, (1975) |  |  |  |  |  |  |  |  |

Materials and methods

Principles of method if other than guideline

Method: other: no data

GLP compliance

no

Test materials

Details on test material

IUCLID4 Test substance: other TS: sec-Butylamin Acetat, - Phosphat, - Carbonat und - Hydrochlorid

Test animals

Species

dog

Results and discussions

Effect levels

| Sex | Endpoint | Effect level | Based on | 95% CL | Remarks |
| --- | --- | --- | --- | --- | --- |
|  | LD50 | 250 — 500 mg/kg bw |  |  |  |

Endpoint study record: Acute toxicity: oral, IUC4#11/Ch.5.1.1

| UUID |  | IUC5-28571d95-c5ee-4c0c-8acf-14079c1aa98d |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-13 10:49:50 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Eli, Lilly and Co., unveroeffentlichte Untersuchung,¦(1965);¦Zitiert in: Pesticide residues in food, FAO plant¦protection paper 1, 59-82, (1975) |  |  |  |  |  |  |  |  |

Materials and methods

Principles of method if other than guideline

Method: other: no data

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Details on test material

IUCLID4 Test substance: as prescribed by 1.1 - 1.4

Test animals

Species

rat

Results and discussions

Effect levels

| Sex | Endpoint | Effect level | Based on | 95% CL | Remarks |
| --- | --- | --- | --- | --- | --- |
|  | LD50 | 380 mg/kg bw |  |  |  |

Overall remarks, attachments

Remarks on results including tables and figures

RM-Freetext: Erwachsene Tiere.

Endpoint study record: Acute toxicity: oral, IUC4#12/Ch.5.1.1

| UUID |  | IUC5-188b8e25-475f-4e00-ac14-35892764ecfe |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-13 10:49:51 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Eli, Lilly and Co., unveroeffentlichte Untersuchung,¦(1965);¦Zitiert in: Pesticide residues in food, FAO plant¦protection paper 1, 59-82, (1975) |  |  |  |  |  |  |  |  |

Materials and methods

Principles of method if other than guideline

Method: other: no data

GLP compliance

no

Test materials

Details on test material

IUCLID4 Test substance: other TS: sec-Butylamin Acetat, - Phosphat, - Carbonat und - Hydrochlorid

Test animals

Species

rat

Results and discussions

Effect levels

| Sex | Endpoint | Effect level | Based on | 95% CL | Remarks |
| --- | --- | --- | --- | --- | --- |
|  | LD50 | 430 — 690 mg/kg bw |  |  |  |

Overall remarks, attachments

Remarks on results including tables and figures

RM-Freetext: Neugeborene Tiere

Endpoint study record: Acute toxicity: oral, IUC4#13/Ch.5.1.1

| UUID |  | IUC5-45330cdb-4c3b-4b44-8d90-7b6cc49aac03 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-13 10:49:52 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Eli, Lilly and Co., unveroeffentlichte Untersuchung,¦(1965);¦Zitiert in: Pesticide residues in food, FAO plant¦protection paper 1, 59-82, (1975) |  |  |  |  |  |  |  |  |

Materials and methods

Principles of method if other than guideline

Method: other: no data

GLP compliance

no

Test materials

Details on test material

IUCLID4 Test substance: other TS: sec-Butylamin Acetat, - Phosphat, - Carbonat und - Hydrochlorid

Test animals

Species

rat

Results and discussions

Effect levels

| Sex | Endpoint | Effect level | Based on | 95% CL | Remarks |
| --- | --- | --- | --- | --- | --- |
|  | LD50 | 1270 — 1660 mg/kg bw |  |  |  |

Overall remarks, attachments

Remarks on results including tables and figures

RM-Freetext: Jungtiere

Endpoint study record: Acute toxicity: oral, IUC4#14/Ch.5.1.1

| UUID |  | IUC5-78be61c3-a055-4ac3-9aa2-ff41229b4c46 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-13 10:49:53 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Eli, Lilly and Co., unveroeffentlichte Untersuchung,¦(1965);¦Zitiert in: Pesticide residues in food, FAO plant¦protection paper 1, 59-82, (1975) |  |  |  |  |  |  |  |  |

Materials and methods

Principles of method if other than guideline

Method: other: no data

GLP compliance

no

Test materials

Details on test material

IUCLID4 Test substance: other TS: sec-Butylamin Acetat, - Phosphat, - Carbonat und - Hydrochlorid

Test animals

Species

rat

Results and discussions

Effect levels

| Sex | Endpoint | Effect level | Based on | 95% CL | Remarks |
| --- | --- | --- | --- | --- | --- |
|  | LD50 | 1510 — 4600 mg/kg bw |  |  |  |

Overall remarks, attachments

Remarks on results including tables and figures

RM-Freetext: Erwachsene Tiere

Endpoint study record: Acute toxicity: oral, IUC4#15/Ch.5.1.1

| UUID |  | IUC5-1e799d63-ace3-49f2-b17c-252d6a84a26d |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-13 10:49:54 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Worthing, Charles R. in: The Pesticide Manual - A world¦compendium, 8th ed., Suffolk, S.1660, (1987) |  |  |  |  |  |  |  |  |

Materials and methods

Principles of method if other than guideline

Method: other: no data

GLP compliance

no data

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Details on test material

IUCLID4 Test substance: as prescribed by 1.1 - 1.4

Test animals

Species

hen

Results and discussions

Effect levels

| Sex | Endpoint | Effect level | Based on | 95% CL | Remarks |
| --- | --- | --- | --- | --- | --- |
|  | LD50 | 250 mg/kg bw |  |  |  |

7.2.2 Acute toxicity: inhalation

Endpoint study record: BASFAGXVII/313.Acute inhalation toxicity, IHT to rat

| UUID |  | IUC5-a4cf9812-798f-44f2-a59c-d0a8e2e4de25 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2009-08-27 01:50:49 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Study result type | experimental result |  |  |
| --- | --- | --- | --- |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Acceptable study which meets basic scientific principles. |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | BASF AG | 1967 | Bericht über die Gewerbetoxikologische Vorprüfung |  | BASF AG, Department of Toxicology | XVII/313 | BASF SE |  | 1967-10-09 |

Data access

data submitter is data owner

Materials and methods

Test type

other: Inhalation hazard test

Limit test

no

Principles of method if other than guideline

BASF-Test The inhalation hazard test demonstrates the toxicity of an atmosphere saturated with vapors of the volatile components of a test substance at the temperature chosen for vapor generation (usually 20°C). Several groups of usually 3 rats per sex were sequentially exposed to the vapors, generated by bubbling 200 l/h air through a substance column of about 5 cm above a fritted disc in a glass cylinder for different time periods. No analytical determination of the atmosphere concentrations was performed. The nominal concentration usually can be calculated as quotient of the amount of the test substance weight loss during exposure. Group-wise documentation of clinical signs was performed over the 14 day study period. Body weight of groups was determined before the start of the study and at the end of the observation period in surviving animals.

GLP compliance

no (; GLP was not compulsory at the time the study was conducted )

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 13952-84-6 |

Details on test material

- Name of test material (as cited in study report): sec. Butylamin - Physical state: liquid - Analytical purity: 98.5 %

Test animals

Species

rat

Strain

no data

Sex

male/female

Administration / exposure

Route of administration

inhalation: vapour

Type of inhalation exposure

whole body

Vehicle

other: The animals were exposed to atmosphere saturated with the volatile parts of the test material in the air at 20 °C.

Details on inhalation exposure

Saturation of the atmosphere was generated by conducting an air stream through a substance layer of 5 cm height. The rats were exposed to the test atmosphere for 3 min in the first experiment and for 5 min in the second experiment. Air flow was 200 l/h in the exposure chamber.

Analytical verification of test atmosphere concentrations

no

Duration of exposure

| min | Remarks | 3 or 5 minutes |
| --- | --- | --- |

Concentrations

The mean concentration was 495 mg/l (20 °C), the nominal concentration was calculated as quotient of the amount of the test substance weight loss during exposure.

No. of animals per sex per dose

3 animals per sex per experiment; in each experiment there were 6 animals (3 males, 3 females)

Control animals

no

Details on study design

The exposure time not causing lethality was tested twice in 2 experiments. In the first experiment the exposure duration was 3 min, in the second experiment the exposure duration was 5 min. - Duration of post exposure period until necropsy: 14 days - Frequency of weighing: Test animals were weighed at test initiation only - Frequency of observations: The animals were observed following exposure after 3 min, 10 min, 30 min, 1 hour, 3 hours, 7 hours and thereafter daily for the first 3 days after test initiation. - Necropsy of survivors performed: yes. The necropsy of the animals was done on day 14 following exposure for the survivors or shortly after death.

Results and discussions

Effect levels

| Sex | Endpoint | Effect level | Based on | 95% CL | Exp. duration | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| male/female | LC100 | 700 mg/L air |  |  | 3 min | All 12 animals exposed to the test material died within 5 and also within 3 minutes. |

Mortality

All animals exposed to the test material died. 12/12 animals died within the experiment during exposure (6 animals within 2-3 min in the 1st experiment and 6 animals wihtin 2-4 min in the 2nd experiment).

Clinical signs

Signs of corrosive action were recorded. The animals vigorously attended to escape, eyelid closure was seen. After 1 - 2 minutes gasping was observed and aqueous, bloody nasal was seen. The animals were cyanotic and convulsions, dyspnea and corneal opacity were seen.

Body weight

No data given.

Gross pathology

Slight serous scab arond the snouts, unkempt fur, exhalation of the lungs and edema of the lungs were seen in both experiments.

Overall remarks, attachments

Remarks on results including tables and figures

Under the experimental conditions of the inhalation hazard test chosen, the LC100 for acute toxicity after inhalation of the test material for 3 minutes to male and female rats was 700 mg/l air.

Endpoint study record: BASFAG.77/0516(IFREB).Acute toxicity, inhalation to rat

| UUID |  | IUC5-0594e0bf-c10c-4da7-9d7a-e4ac1f69301d |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2009-08-27 01:56:36 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Study result type | experimental result |  |  |
| --- | --- | --- | --- |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Meets generally accepted scientific principles. |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | IFREB, Institut Francais de Recherches et Essais Biologiques | 1977 | Acute Toxicitx by Inhalation of Vapors in the Rat, Test Substance 77/516 (B.A.S.F.) |  | IFREB - Institut Francais de Rechereches et Essais Biologiques | IFREB-R 710216 | BASF SE | 77/0516 | 1977-10-09 |

Data access

data submitter is data owner

Materials and methods

Test type

fixed concentration procedure

Limit test

no

Principles of method if other than guideline

A glass cube of a volume of one cubic meter (100 litres) is ventilated permanently with an atmosphere containing the vapor of the compound. The vapor is produced from an aerosol generated in a glass column. This column is supplied with compressed air at an adjustable rate and with the liquid compound, also delivered at an adjustable rate by a "Braun " precision infusion pump. The chamber of 100 litres will be supplied with an air stream of approx. 1 m3 per hour, 10 air changes per hour. In the assay under discussion the concentration to be investigated will be 2 grams per m3 and the column will therefore be supplied with 2 grams of the compound per hour. The air flow which supplies the colum is adjusted through a "BROOKS" rotameter with a needle screw valve and controlled by a volumeter placed in series at the outflow of the chamber. Depending on the density of the liquid chemical and the pre-set infusion rate of the perfusion pump using a precision 5 ml syringe, an adjustment of the air flow rate is required in order to achieve results which can be expressed in grams per m3.

GLP compliance

no (; GLP was not compulsory at the time the study was conducted.)

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 13952-84-6 |

Details on test material

- Name of test material (as cited in study report): Butylamin (or 1 amino 1 methyl propan, or 2 amino butan) - Physical state: liquid - Analytical purity: 99 %

Test animals

Species

rat

Strain

Sprague-Dawley

Sex

male/female

Details on test animals and environmental conditions

TEST ANIMALS Mean body weight at study initiation: males: 206 g; females: 205 g

Administration / exposure

Route of administration

inhalation: vapour

Type of inhalation exposure

whole body

Vehicle

other: unchanged (no vehicle)

Details on inhalation exposure

Chamber of 100 litres Open circuit no . 4 Desired concentration (C): 2 g/m3 Infusion rae to produce C: 9 ml/hour Infusion rate to maintain C: 2.25 ml/hour Theoretical air flow : 0.812 m3/hour

Analytical verification of test atmosphere concentrations

no data

Duration of exposure

63 min

Concentrations

Mean concentration of vapour in atmosphere was 2.07 mg/m3.

No. of animals per sex per dose

5 animals per sex per dose

Control animals

no

Details on study design

The test had as object the determination of the acute toxicity of the vapor of the compound at a concentration of 2 mg/litre air in rats exposed to this atmosphere for 60 minutes. The toxic effects, if any, will be observed and recorded over a 48 hour period after exposure. The rats (5 males and 5 females of Sprague-Dawley strain) will be placed in stainless steel wire grid cages in the chamber of 100 litres. The chamber is connected on the one side to the vapor generator and on the other to a slight negative pressure for extraction of the potentially toxic atmosphere. A gas-meter is placed between the chamber and the extraction system. Depending on the density of the liquid and the infusion rate, the air flow will be determined which is necessary to produce a concentration of 2 mg compound per litre air. Once this flow rate has been set the chamber is closed and the air flow will be adjusted according to the air volume registered by the gasmeter. A glass syringe of 5 ml, filled with the liquid chemical, will be attached to the precision perfusion pump and connected with the vaporization column through a very fine tubing (polyethylene). The infusion pump will be set to the speed which will deliver five times the calculated final infusion rate. The moment the liquid reaches the column a stop watch will be set and the infusion rate at five times the final one will be maintained for the time (in seconds) derived from the scales on the table attached. The ventilator inside the chamber assists in the rapid and homogeneous distribution of the vapor in air and once the point in time is reached at which the final concentration has been achieved in the chamber the infusion speed is reduced to the one which gives the desired concentration of 2 g per m3 . The air flow will be checked through the measurements taken with the gasmeter during 3 minute periods at 15, 30, 40 and 55 minutes. This in order to detect any fluctuations. Similarly the distance travelled by the piston in the graduated syringe will be measured . Measurements of the total air volume which has flown through the chambar and of the volume of liquid which is left in the syringe during the one hour of the test will detect any failure in the system.

Results and discussions

Effect levels

| Sex | Endpoint | Effect level | Based on | 95% CL | Exp. duration | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| male/female | LC50 | > 2.07 mg/L air |  |  | 63 min |  |

Mortality

No mortality occured during the whole study.

Clinical signs

During exposure agitation was observed at the beginning of the exposure, thereafter prostration was seen. After exposure no anomaly was observed.

Body weight

At day 0, males weighed 206 g and females weighed 205 g. At day 2, the mean weight of male animals was 222 g and the mean weight of females was 209 g.

Gross pathology

Autopsy revealed no relevant findings.

Overall remarks, attachments

Remarks on results including tables and figures

Under the experimental conditions of the test, no deaths occured after 63 min exposure to 2.07 mg/l air of the test material.

Endpoint study record: BASFAG23I0024/927002.Respiratory Tract Irritation to mice

| UUID |  | IUC5-dff729c8-fbbb-4aed-8d52-c4e42a6ed5df |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2009-08-27 02:05:22 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Study result type | experimental result |  |  |
| --- | --- | --- | --- |
| Reliability | 1 (reliable without restriction) |  |  |
| Rationale for reliability incl. deficiencies | Study was conducted under GLP conditions and is well documented. |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | BASF AG | 1998 | Report sec. Butylamine - Study on Respiratory Tract Irritation in mice - Vapor exposure - |  | BASF AG,Department of Toxicology | 23I0024/927002 | BASF SE |  | 1998-12-17 |

Data access

data submitter is data owner

Materials and methods

Test type

other: Study on Respiratory Tract Irritation

Limit test

no

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| no guideline available |  |  |

Principles of method if other than guideline

The test method was in based on "ASTM : E 981-84: Standard Test Method for Estimating Sensory Irritancy of Airborne Chemicals" (ASTM, 1984) .

GLP compliance

yes

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 13952-84-6 |

Details on test material

- Name of test material (as cited in study report): sec. Butylamine - Physical state: liquid - Analytical purity: 99.3 % - Batch No.: Tank 17 - Storage condition of test material: was stored at room temperature, covered with N2 - Stability under test conditions: stability of the test substance has been proven by reanalysis

Test animals

Species

mouse

Strain

CD-1

Sex

male

Details on test animals and environmental conditions

TEST ANIMALS - Source: Charles River Wiga GmbH - Age at study initiation: 6-7 weeks - Average weight at study initiation: 28.2 g - Housing: singly in cages type MK I of Becker, with bedding - Diet: KLIBA rat/mouse/hamster laboratory diet 24-343-4 10 mm pellets (Klingentalmühle AG, Kaiseraugst, Switzerland), ad libitum - Water: ad libitum ENVIRONMENTAL CONDITIONS - Temperature: 20 - 24 °C - Humidity: 30 - 70 % - Air changes (per hr): fully air-conditioned rooms - Photoperiod (hrs dark / hrs light): 12/12

Administration / exposure

Route of administration

inhalation: vapour

Type of inhalation exposure

nose/head only

Vehicle

other: unchanged (no vehicle)

Details on inhalation exposure

Procedure A vapor was generated. By means of the infusion pump suitable amounts of the test substance were supplied to the heated vaporizer for each test group. The vapors that developed were mixed with supply air and passed via the mixing vessel and the 3-way valve either to the exhaust air system (pre- and post exposure) or into the inhalation system (exposure). Exposure system Head-nose inhalation system INA 10, (stainless steel construction, BASF-Aktiengesellschaft, volume approx. 7.5 l); the animals were restrained in plethysmographs and their snouts projected into the inhalation system. Supply air flows were set to 1200 l/h. An air change of about 160 times per hour can be calculated by dividing the supply air flows through the volume of the inhalation system. The exposure system was located in an air conditioned laboratory. By means of an exhaust air system the pressure ratios in the inhalation system were adjusted in such a way that the amount of exhaust air was about 17 % lower (positive pressure). This ensured that the mixture of test substance and air was riot diluted with laboratory air in the breathing zones of the animals. Exposure was started and stopped by switching the 3-way valve .

Analytical verification of test atmosphere concentrations

yes

Duration of exposure

45 min

Concentrations

93, 229, 360 mg/m3 (31.5, 77.6 and 122 ppm) (analytical concentrations)

No. of animals per sex per dose

4 male animals per concentration.

Control animals

yes

Details on study design

Measurement of respiratory parameters: System for respiratory measurements: Body plethysmographs equipped with pressure transducers (Validyne) were used. The signals from the transducers were processed. by a "Non Invasive Respiratory Analyzer" (Buxco Electronics, Inc.). The data of each parameter from the analyzer were supplied via a data logger as minute values to a personal computer for further evaluation using tabular calculation software. During the different phases of the study breathing curves were recorded with an Omnilight high speed recorder (Sanei Instruments) for intervals of about 1 minute. The system allows for the simultaneous measurement of 4 animal. Breathing Parameters: The Analyzer calculates the breathing parameters from the pressure changes in the plethysmographs over a given time-period. - Respiratíon Rate (RR ) The respiration rate is given in number of breaths per minute [min-1]. - Tidal Volume (TV ) The tidal volume is the air volume of a single breath(i .e . the TV is inspired and expired during one breath). Its unit is milliliter [ml]. - Inspiration Time (TI ) The inspiration time is the duration of inspiration in a single breath. Its unit is seconds [s]. - Expiration Time (TE ) The expiration time is the duration of expiration in a single breath. Its unit is seconds [s]. The analyzer calculates TE as the difference TT - TI = TE, in which TT is the time from the beginning of a breath to the beginning of the next breath . - Relaxation Time (RT ) The relaxation time is the duration needed during expiration to 25 % of the tidal volume. Its unit is seconds [s]. - Peak Expiratory Flow (PEF ) The peak expiratory flow reflects the maximum expiration-flow [ml/sec] of the single breaths from breath to breath. - Minute Volume (MV ) The minute volume is the product of the tidal volume (TV) and the respiration rate (RR). Its unit is milliliter per minute [ml/min]. For quantitative evaluation of the response only the respiration rate measurements were used. Conduct of measurements: After the four animals of the respective test group were placed into the plethysmographs, the test was started. The test was structured into 4 phases 1 . In the "Adaptation phase", lasting approx . 10 minutes, the animals could adapt to the exposure conditions, while being exposed to supply air. 2 . In the "Control phase", lasting 15 minutes (min 1 to 15 of the measurement period), recording of respiratory measurement was started to establish the parameters for "normal breathing". The "baseline" of respiratory rate, that is the respiratory rate used as the 100 % value in comparing respiratory rates during exposure, was calculated as the mean value of minutes 6 to 15. 3 . During the "Exposure phase", lasting 45 minutes (min 16 to 60 of the measurement period) substance-air mixtures were offered to the animals and the effects on respiration was measured. Mean values of the percentage of decrease in respiration rate were calculetes from minutes 21 to 30, 36 to 45 and 51 to 60 as early, middle and late response. 4 . The "Recovery phase", lasting at least 15 minutes (min 61 to 75 of the measurement period) served for measuring the ability of the animals to restore normal breathing . The animals of the test group 0 served as "control group". They underwent the same procedure without being exposed to the test substance. Post-exposure-observation period, clinical examination and pathology: Following exposure, the animals were observed for seven days. The body weight of the animals was checked at random before the test and three times during the postexposure-observation period. Clinical signs were checked during exposure, after exposure on exposure day and once per workday during the post-exposure-observation period. A check for dead or moribund animals was made daily. At the end of the post-exposure observation period the animals were killed by exsanguination under Nembutal narcosis. The lungs were evaluated macroscopically and weighed with trachea attached (without larynx).

Statistics

Calculations and statistical evaluation: The decrease in respiration rate was calculated for the time intervals specified below in the following way: For each animal of a test group the mean baseline respiration rate (RR) was calculated from the measurements of minute 6-15 of the control phase. The mean respiration rates for early, middle, late response and recovery were calculated for the time intervals specified below . These means were compared to the baseline mean as % of baseline rate (RR%) for each animal, and mean RR for each test groups were calculated using the single animal RR% values. The mean respiratory depression (RD%) for each test animal and each time interval was calculated as 100 - RR%. These individual values were used to calculate the RD5O values. Additionally mean RD% for each test groups were calculated using the single animal RR% values. The following time intervals were used: 1 .) For calculation of the early decrease the values of minutes 6 to 15 of the exposure phase (minutes 21 to 30 of the measurement period) were used. 2 .) For calculation of the middle decrease the values of the minutes 21 to 30 of the exposure phase (minutes 36 to 45 of the measurement period) were used. 3 .) For calculation of the late decrease the values of the minutes 36 to 45 of the exposure phase (minutes 51 to 60 of the measurement period) were used. 4 .) For calculation of the recovery the values of the minutes 6 to 15 of the recovery phase (minutes 66 to 75 of the measurement period) were used. The substance-concentration leading to a respiration rate Decrease of 50 % (RD50) was calculated from the logarithm of the analytical concentration and the percentage of decrease of the respiratory rate (RD%) of each animal and each time interval using linear regression analysis. The concentration and the 95 % confidence interval was estimated by inverse regression.

Results and discussions

Effect levels

| Sex | Endpoint | Effect level | Based on | 95% CL | Exp. duration | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| male | other: RD50 | 389 mg/m³ air |  |  | 45 min | early response |
| male | other: RD50 | 242 mg/m³ air |  |  | 45 min | middle response |
| male | other: RD50 | 273 mg/m³ air |  |  | 45 min | late response |

Mortality

No deaths occured.

Clinical signs

No abnormalities were detected in the animals of the test groups 0 - 3 during exposure and in the post exposure observation period. No clinical observations were performed on public holidays or weekend.

Body weight

The body weight gain of the treated groups was comparable to the respective controls.

Gross pathology

Necropsy findings at termination of the study: No abnormalities were detected in any of the animals. The mean lung weights also did not differ between the treated animals and the controls.

Other findings

The respiratory depression ameliorated during the recovery phase, but was not fully reversible at 360 mg/m3. The RD50 was of the same magnitude during the whole exposure period. The breathing curves clearly show a prolongation of exspiration time.

Any other information on results incl. tables

Table 1: Body weight gain Conc. (mg/m3); day 0; day 1; day 5; day 7. Control; 0; 29.5; 29.3; 31.1 (day 3); 33.6. Test group 1; 93; 27.0; 28.4 (day 3); 29.2; 30.5. Test group 2; 229; 26.8; 26.5; 29.2; 30.1. Test group 3; 360; 29.3; 29.4; 30.9 (day 3); 33.0. Table 2: Mean lung weights Conc. (mg/m3); terminal body weight (g); lung weight (g); lung / body weight (%). Control; 0; 33.6; 210.8; 0.63. Test group 1; 93; 30.5; 201.0; 0.66. Test group 2; 229; 30.1; 200.5; 0.67. Test group 3; 360; 33.0; 216.8; 0.66. Table 3: Respiration rate decrease Respiration rate decrease (RD%). Conc. (mg/m3); early; middle; late; recovery. Control; 0; 10.2; 7.9; 15.7; 5.8. Test group 1; 93; 20.1; 25.4; 32.4; 22.7. Test group 2; 229; 37.3; 53.0; 47.9; 22.8. Test group 3; 360; 49.3; 56.6; 53.9; 41.4.

Overall remarks, attachments

Remarks on results including tables and figures

The test substance effects sensory irritation in mice. The RD50 obtained was in the range of 300 mg/m3 (~ 100 ppm) under the experimental conditions chosen.

Endpoint study record: Acute toxicity: inhalation, IUC4#2/Ch.5.1.2

| UUID |  | IUC5-58329bee-1fd5-4bcb-89a1-4361db168ee0 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-13 10:49:57 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Eli, Lilly and Co., unveroeffentlichte Untersuchung,¦(1965);¦Zitiert in: Pesticide residues in food, FAO plant¦protection paper 1, 59-82, (1975) |  |  |  |  |  |  |  |  |

Materials and methods

Principles of method if other than guideline

Method: other: no data

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Details on test material

IUCLID4 Test substance: as prescribed by 1.1 - 1.4

Test animals

Species

rat

Administration / exposure

Duration of exposure

| Remarks | Exposure unspecifiedExposure unspecified |
| --- | --- |

Results and discussions

Effect levels

| Sex | Endpoint | Effect level | Based on | 95% CL | Exp. duration | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
|  | LC50 | 3.5 mg/L air |  |  |  | Exposure unspecifiedExposure unspecified |

Overall remarks, attachments

Remarks on results including tables and figures

RM-Freetext: Substanz geprueft als: Base, Salze (Acetat, Phosphat, Carbonat und HCl-Salz)

Endpoint study record: Acute toxicity: inhalation, IUC4#4/Ch.5.1.2

| UUID |  | IUC5-11fb93f6-5b19-45e2-93bd-bc10ecc18430 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-13 10:49:59 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Plunkett E.R. (ed.): Handboook of Industrial Toxicology,¦71-72, (1976) |  |  |  |  |  |  |  |  |

Materials and methods

Principles of method if other than guideline

Method: other: no data

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Details on test material

IUCLID4 Test substance: as prescribed by 1.1 - 1.4

Test animals

Species

other: no data

Administration / exposure

Duration of exposure

| Remarks | Exposure unspecifiedExposure unspecifiedExposure unspecified |
| --- | --- |

Overall remarks, attachments

Remarks on results including tables and figures

RM-Freetext: Bei Inhalation (keine Angabe zur Konzentration, Spezies) werden als Symptome Reizung der Atemwege und Lunge angegeben.

Endpoint study record: Acute toxicity: inhalation, IUC4#5/Ch.5.1.2

| UUID |  | IUC5-8b909095-6960-43a7-b3a0-ea5e6fe96ce2 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-13 10:50:00 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Gage J.C.: Br. J. industr. Med. 27, 1-18, (1979) |  |  |  |  |  |  |  |  |

Materials and methods

Principles of method if other than guideline

Method: other: no data

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Details on test material

IUCLID4 Test substance: as prescribed by 1.1 - 1.4

Test animals

Species

rat

Administration / exposure

Duration of exposure

5 h

Overall remarks, attachments

Remarks on results including tables and figures

RM-Freetext: Bei 5stuendiger Exposition in einer gesaettigten Atmosphaere (280 mg/l) starben alle Ratten.

Endpoint study record: Acute toxicity: inhalation, IUC4#6/Ch.5.1.2

| UUID |  | IUC5-9ff9a277-6209-4d72-92af-01636ad1129e |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-13 10:50:01 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Gagnaire F. et al.: J. of Applied Toxicology 9, 301-304,¦(1989) |  |  |  |  |  |  |  |  |

Materials and methods

Principles of method if other than guideline

Method: other: nach American Society for Testing and Materials (ASTM), Standard test method for estimating sensory irritancy of airborne chemicals, Annual Book of ASTM Standards, E 981-84

GLP compliance

no data

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Details on test material

IUCLID4 Test substance: as prescribed by 1.1 - 1.4

Test animals

Species

mouse

Administration / exposure

Duration of exposure

15 min

Results and discussions

Effect levels

| Sex | Endpoint | Effect level | Based on | 95% CL | Exp. duration | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
|  | other: RD50 (Verminderung der Respirationsrate um 50%) | 0.28 mg/L air |  |  | 15 min |  |

7.2.3 Acute toxicity: dermal

Endpoint study record: BASFAG77/0516.Acute toxicity, dermal to rabbit

| UUID |  | IUC5-f95c7cb4-cf94-4cee-8425-cf62838941ff |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2009-09-22 03:39:06 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Study result type | experimental result |  |  |
| --- | --- | --- | --- |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Meets generally accepted scientific principles. |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | BASF AG | 1979 | Bericht über die Prüfung der akuten dermalen Toxizität von "sek.-Butylamin" an der Rückenhaut weißer Kaninchen |  | BASF AG, Department of Toxicology | 77/0516 | BASF SE |  | 1979-11-25 |

Data access

data submitter is data owner

Materials and methods

Test type

standard acute method

Limit test

no

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| equivalent or similar to | OECD Guideline 402 (Acute Dermal Toxicity) | yes (application time 24 h; occlusive conditions; only one dose was tested.) |

GLP compliance

no (; GLP was not compulsory at the time the study was conducted.)

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 13952-84-6 |

Details on test material

- Name of test material (as cited in study report): sek.-Butylamin - Physical state: liquid - Analytical purity: 99 %

Test animals

Species

rabbit

Strain

Vienna White

Sex

male/female

Details on test animals and environmental conditions

TEST ANIMALS - Source: M. Gaukler, Offenbach, Germany - Average weight at study initiation: males ca. 2.3 kg; females ca. 2.3 kg - Diet: Ssniff K, standard diet for rabbits and guinea pigs (Intermast Gmbh, Soest, Germany), ad libitum - Water: ad libitum

Administration / exposure

Type of coverage

occlusive

Vehicle

unchanged (no vehicle)

Details on dermal exposure

TEST SITE - Area of exposure: ca. 50 cm2 (the skin of the animals was clipped with an electric clipper 15-24 h prior to application of the test material. Only animals with an intact skin were used). - Type of wrap if used: inert foil which was fixed with adhesive tape (occlusive) REMOVAL OF TEST SUBSTANCE - Washing: the treated skin site was washed with warm water / lutrol and thereafter drying of the skin with pulp. - Time after start of exposure: 24 h TEST MATERIAL - Amount(s) applied: 200 mg/kg bw - Concentration: undiluted - Constant volume or concentration used: no

Duration of exposure

24 h

Doses

200 mg/kg bw

No. of animals per sex per dose

5 male and 5 female rabbits

Control animals

other: the untreated skin of the same animal served as control.

Details on study design

- Duration of observation period following administration: 8 days - Frequency of observations and weighing: Body weight was determined prior to application of the test material and on day 8. Clinical signs of toxicity and local irritation efeects were recorded daily on working days. - Necropsy of survivors performed: yes, the animals were sacrificed with CO2 on day 8 and thereafter a necropsy was conducted.

Results and discussions

Effect levels

| Sex | Endpoint | Effect level | Based on | 95% CL | Remarks |
| --- | --- | --- | --- | --- | --- |
| male/female | LD50 | > 200 mg/kg bw |  |  |  |

Mortality

No deaths occured at any time point during the study.

Clinical signs

No clinical signs of toxicity were seen.

Body weight

The body weight of the animals increased within the normal range during the study period, except for 1 female which body weight on day 8 was slightly reduced.

Gross pathology

Autopsy of the animals revealed no relevant findings.

Other findings

24 hours after application, necroses were seen on the treated sited of all animals, which remained on the skin until the end of the observation period on day 8.

Any other information on results incl. tables

Table 1: Body weight of the animals Animal No.; Sex; Body weight on day 1 (kg); Body weight on day 8 (kg). 1; m; 2.43; 2.51. 2; m; 2.22; 2.31. 3; m; 2.26; 2.3. 4; m; 2.17; 2.34. 5; m; 2.34; 2.35. 1; f; 2.07; 2.2. 2; f; 2.33; 2.4. 3; f; 2.54; 2.58. 4; f; 2.15; 2.13. 5; f; 2.52; 2.2.

Overall remarks, attachments

Remarks on results including tables and figures

Under the experimental conditions of the test a LD50 of > 200 mg/kg bw was obtained for acute toxicity after dermal application to rabbit.

Endpoint study record: Acute toxicity: dermal, IUC4#1/Ch.5.1.3

| UUID |  | IUC5-c9806b5c-3e82-4e7f-aac1-94ba193f65f4 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-13 10:50:03 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Eli, Lilly and Co., unveroeffentlichte Untersuchung,¦(1965);¦Zitiert in: Pesticide residues in food, FAO plant¦protection paper 1, 59-82, (1975) |  |  |  |  |  |  |  |  |
|  | RTECS, update 8910 |  |  |  |  |  |  |  |  |

Materials and methods

Principles of method if other than guideline

Method: other: no data

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Details on test material

IUCLID4 Test substance: as prescribed by 1.1 - 1.4

Test animals

Species

rabbit

Results and discussions

Effect levels

| Sex | Endpoint | Effect level | Based on | 95% CL | Remarks |
| --- | --- | --- | --- | --- | --- |
|  | LD50 | 2500 mg/kg bw |  |  |  |

Overall remarks, attachments

Remarks on results including tables and figures

RM-Freetext: Substanz geprueft als: Base, Salze (Acetat, Phosphat, Carbonat und HCl-Salz)

7.2.4 Acute toxicity: other routes

Endpoint study record: BASFXVII/313.Acute toxicity i.p. to mouse

| UUID |  | IUC5-6ad000ef-a947-41a4-8b7d-a0c64827a332 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2009-09-22 04:20:40 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Study result type | experimental result |  |  |
| --- | --- | --- | --- |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Meets generally accepted scientific standards, well documented and acceptable for assessment |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | BASF AG | 1967 | Bericht über die gewerbetoxikologische Vorprüfung |  | BASF AG, Department of Toxicology | XVII/313 | BASF SE |  | 1967-11-13 |

Data access

data submitter is data owner

Materials and methods

Limit test

no

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| no guideline available |  |  |

Principles of method if other than guideline

BASF-Test: Young adult laboratory mice were purchased from breeder. Several groups of 5 mice per sex and dose were treated simultaneously with preparations of the test substance in suitable vehicle. The substance was injected into the peritoneal cavity. The concentrations of the preparations were used to achieve comparable volumes per kg body weight. Group-wise documentation of clinical signs was performed over the study period of 14 days.

GLP compliance

no (; GLP was not compulsory at the time the study was conducted)

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 13952-84-6 |

Details on test material

- Name of test material (as cited in study report): sec. Butylamin - Physical state: liquid - Analytical purity: 98.5 %

Test animals

Species

mouse

Strain

other: Kisslegg-mice

Sex

male/female

Details on test animals and environmental conditions

TEST ANIMALS - Weight at study initiation: mean body weight, males: 27.5 g mean body weight, females: 23.5 g

Administration / exposure

Route of administration

intraperitoneal

Vehicle

water

Details on exposure

DOSAGE PREPARATION: 0.5 % for the 18.1 mg/kg bw dose group, dose volume applied: 5 ml/kg bw 1 % for the 36.3 mg/kg bw dose group, dose volume applied: 5 ml/kg bw 1 % for the 72.7 mg/kg bw dose group, dose volume applied: 10 ml/kg bw 2 % for the 145 mg/kg bw dose group, dose volume applied: 10 ml/kg bw 20 % for the 1163 mg/kg bw dose group, dose volume applied: 8 ml/kg bw

Doses

25, 50, 100, 200 and 1600 µl/kg bw => 18.175, 36.35, 72.7, 145 and 1163 mg/kg bw

No. of animals per sex per dose

5 animals per sex per dose

Control animals

no

Details on study design

Duration of observation period following administration: 14 days - Frequency of observations and weighing: Animals were observed and examined for clinical signs of toxicity during the first hour following application, after 2, 4 and 5 hours and further daily on working days after dosing. Body weights were determined prior to application of the test material. - Necropsy of survivors performed: yes Deceased animals and those sacrificed at the end of the observation period were necropsied.

Results and discussions

Effect levels

| Sex | Endpoint | Effect level | Based on | 95% CL | Remarks |
| --- | --- | --- | --- | --- | --- |
| male/female | LD50 | > 36.35 — < 72.7 mg/kg bw |  |  |  |

Mortality

No mortalities were seen in the 18.1 mg/kg bw dose group. 4/10 mortalities were seen (3/10 animals were dead after 7 days, one further animal died within the following 24 hours) in the 36.35 mg/kg bw dose group. In the 72.7 mg/kg bw group 2/10 animals died within the first 48 hours after application of the test material. After 7 days 8/10 animals had died. In the 145 mg/kg bw group, 10/10 animals were found dead after 24 hours. In the 1163 mg/kg bw group, 3/10 animals died within the first hour, and after 24 hours 10/10 animals had died.

Clinical signs

36.35 - 1163 mg/kg bw groups: Immediately after application of the test material, a staggering walk and accelerated breath were seen, the eye lids of the animals were closed, the flanks were haggard and tremor was seen. After 5 hours the animals were in a ventral position, partly crouched down, and slight cyanosis, dyspnea, apathy and ruffled coat was seen. After one day all animals of the two highest doses were dead. In the surviving animals, the symptoms attenuated in the following days only slowly. After seven days intermittent breath, closed eye lids and a croched down position were still seen. The surviving animals were without any findings 12 days after application of the test material. 18.1 mg/kg bw group: Immediately after application of the test material staggering walk and accelerated breath were seen, one day later accelerated breath was still observed. Three days after application of the test material the animals were without any findings.

Body weight

The body weight of the animals was determined prior to application of the test material only.

Gross pathology

Animals that died during the experiment: In most animals slight adhesions were seen, in two cases substance incorporation was found, in one case a distinct serous injection into the small intestine was seen. Surviving animals that were sacrificed at the end of the study: The organs examined were without any findings.

Any other information on results incl. tables

Mortality Dose (cmm/kg); Conc. (%); No. of animals; Died within 1h; 24 h; 48 h; 7 days; 14 days. 1600; 20; 10; 3/10; 10/10; 10/10; 10/10. 200; 2; 10; 0/10; 10/10; 10/10; 10/10. 100; 1; 10; 0/10; 0/10; 2/10; 8/10; 8/10. 50; 1; 10; 0/10; 0/10; 0/10; 3/10; 4/10. 25; 0.5; 10; 0/10; 0/10; 0/10; 0/10.

Overall remarks, attachments

Remarks on results including tables and figures

Under the conditions of the test for acute toxicity after intraperitoneal application the LD50 was >36.35 and <72.7 mg/kg body weight for male and female mice.

Endpoint study record: Acute toxicity: other routes, IUC4#2/Ch.5.1.4

| UUID |  | IUC5-b82c7fcb-497b-47de-8357-347e62146e31 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-13 10:50:06 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Eli, Lilly and Co., unveroeffentlichte Untersuchung,¦(1965);¦Zitiert in: Pesticide residues in food, FAO plant¦protection paper 1, 59-82, (1975) |  |  |  |  |  |  |  |  |

Materials and methods

Principles of method if other than guideline

other: no data

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Details on test material

IUCLID4 Test substance: as prescribed by 1.1 - 1.4

Test animals

Species

mouse

Administration / exposure

Route of administration

intravenous

Results and discussions

Effect levels

| Sex | Endpoint | Effect level | Based on | 95% CL | Remarks |
| --- | --- | --- | --- | --- | --- |
|  | LD50 | 225 mg/kg bw |  |  |  |

Endpoint study record: Acute toxicity: other routes, IUC4#3/Ch.5.1.4

| UUID |  | IUC5-302b4b1e-f1b1-4400-b90d-8039612a630c |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-13 10:50:07 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Goldenthal E.I.: Toxicol. and Appl. Pharmacology 18,¦185-207, (1971) |  |  |  |  |  |  |  |  |

Materials and methods

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test animals

Species

rat

Overall remarks, attachments

Remarks on results including tables and figures

RM-Freetext: Die LD50 junger (2 Tage) und adulter Ratten (max. 49 Tage) ist vergleichbar.

7.3 Irritation / corrosion

7.3.1 Skin irritation / corrosion

Endpoint study record: BASFXVII/313.Skin irritation/corrosion to rabbit

| UUID |  | IUC5-928503bb-bb96-44af-a401-ca8ea900da41 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2009-05-14 09:14:50 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Study result type | experimental result |  |  |
| --- | --- | --- | --- |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Acceptable study report which meets generally accepted scientific principles. |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | BASF AG | 1967 | Bericht über die Gewerbetoxikologische Vorprüfung |  | Gewerbehyg.-Pharmakol. Institut der BASF | XVII/313 | BASF SE |  | 1967-11-13 |

Data access

data submitter is data owner

Materials and methods

Type of method

in vivo

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| equivalent or similar to | OECD Guideline 404 (Acute Dermal Irritation / Corrosion) | yes : application time: 1, 5, 15 min (OECD: 4 h); 2 animals used (OECD: 3); observation period 8 days (OECD: 14); skin changes were recorded on working days; dose ~1 ml (OECD: 0.5 ml) |

Principles of method if other than guideline

BASF-Test: Before OECD guideline 404 was established, skin irritation was tested using an internal method. White Vienna rabbits were used. Usually, 2 animals were treated for 1, 5, 15 min and 20 h using occlusive conditions. An application site of 2.5 cm x 2.5 cm was covered with the liquid test substance. After the application time, the skin was washed with water which sometimes contained a mild detergent. The animals were observed for 8 days. Skin reactions were recorded after 24h and after 8 days of exposure. In the raw data, skin reactions were additionally recorded at reading time points 1 h, 48h, 72h, 96h and 7 days after application.

GLP compliance

no (; GLP was not compulsory at the time the study was conducted.)

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 13952-84-6 |

Details on test material

- Name of test material (as cited in study report): sec. Butylamin - Physical state: liquid - Analytical purity: 98.5 %

Test animals

Species

rabbit

Strain

Vienna White

Details on test animals and environmental conditions

TEST ANIMALS - Weight at study initiation: animal 1: 3.37 kg; animal 2: 2.8 kg

Test system

Type of coverage

occlusive

Preparation of test site

shaved

Vehicle

unchanged (no vehicle)

Amount/concentration applied

~1 ml of the test substance was applied to the skin using a patch soaked with undiluted test material.

Duration of treatment / exposure

Dorsal skin: 1, 5, 15 min

Observation period

The animals were observed following exposure for 8 days on working days. Skin changes were recorded after removal of the bandage as well as after 1 h, 24 h, 48 h, 72 h, and after 6 and 8 days.

Number of animals

2 male animals.

Control animals

other: Untreated, clipped skin of the same animal served as control.

Details on study design

Undiluted test substance (~1 ml) was applied to a 2.5 cm x 2.5 cm patch and placed on the clipped and intact skin of two Vienna White rabbits for 1 min, 5 min, 15 min application time under occlusive conditions. The test substance was removed by washing once with 100 % Lutrol and subsequently with 50 % Lutrol in water. Skin reactions were recorded after 1 h, 24 h, 48 h, 72 h, and after 6 and 8 days. The corresponding reading time points used for assessment were 24, 48 and 72 hours after application. Dermal reactions were classified using a BASF-scoring system, which is convertible into the current OECD Draize scheme. For the evaluation the BASF scores given in the raw data were converted to Draize scores.

Results and discussions

Irritant/corrosive response data

Hard, movable necroses were seen on the treated sites of both animals 24 h after application of the test material for 1 min, 5 min or 15 min. After 8 days hard, movable necroses were still present on the skin of both animals treated with the test material for one minute. For the application time 5 min and 15 min hard, but non-movable necroses were seen in both animals.

Overall remarks, attachments

Remarks on results including tables and figures

Under the experimental conditions chosen, the test material caused hard necroses on the treated skin of the animals after an application time of 1 min, 5 min and 15 min. Therefore the test material has to be classified as corrosive C R35 according to EU criteria and as Category 1B according to GHS criteria.

Applicant's summary and conclusion

Interpretation of results

corrosive

Criteria used for interpretation of results

EU

7.3.2 Eye irritation

Endpoint study record: BASFXVII/313.Eye irritation to rabbit

| UUID |  | IUC5-5a866e67-82ce-4ec8-b3a7-f2e09f969ea6 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2009-05-14 09:26:27 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Study result type | experimental result |  |  |
| --- | --- | --- | --- |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Meets basic scientific principles. |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | BASF AG | 1967 | Bericht über die Gewerbetoxikologische Vorprüfung |  | Gewerbehyg.-Pharmakol. Institut der BASF | XVII/313 | BASF SE |  | 1967-11-13 |

Data access

data submitter is data owner

Materials and methods

Type of method

in vivo

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| equivalent or similar to | OECD Guideline 405 (Acute Eye Irritation / Corrosion) | yes : volume of 0.05 ml applied (OECD: volume of 0.1 ml); 2 animals were used (OECD: 3); observation period was 8 days (OECD: 21) |

Principles of method if other than guideline

BASF-TEST: Before OECD guideline 405 was established, eye irritation was tested using an internal method. White Vienna rabbits were used. Usually approximately 50 µl of the test substance were applied to the conjunctival sac of one eye of each of 2 animals. The non- or saline- treated eye served as control. The animals were observed for several times on the treatment day and up to 8 days afterwards, and findings were recorded on working days. The eyes were not washed out after 24 h as specified in OECD Guideline 405.

GLP compliance

no (; GLP was not compulsory at the time the study was conducted.)

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 13952-84-6 |

Details on test material

- Name of test material (as cited in study report): sec. Butylamin - Physical state: liquid - Analytical purity: 98.5 %

Test animals

Species

rabbit

Strain

Vienna White

Details on test animals and environmental conditions

TEST ANIMALS - Weight at study initiation: animal 1: 2.91 kg; animal 2: 3.28 kg

Test system

Vehicle

unchanged (no vehicle)

Amount/concentration applied

Amount applied: 0.05 ml undiluted test material

Duration of treatment / exposure

24 h; the substance was not washed out

Observation period

8 days

Number of animals

2 male animals

Control animals

other: the second eye of the same rabbit was treated with 50 µl NaCl saline

Details on study design

50 µl of the undiluted test substance was applied to the right conjuntival sac of two Vienna White rabbits. The left eye, treated with physiological saline served as a control. The substance was not washed out. Ocular reactions were recorded at 10 min, 1, 3 and 24 hours, as well as dailay on working days until 8 days after instillation using a BASF-scoring system, which is convertible into the current OECD Draize scheme. For the evaluation, the BASF scores given in the raw data were converted to Draize scores. The reading time points used for assessment were 24, 48 and 72 hours after application.

Results and discussions

Irritant/corrosive response data

One hour after application of the test material in both animals corrosion of the eyeball, of the nictitating membrane and the eye lids was seen as well as bloody secretion and moderate redness of the conjunctiva. After 24 hours corrosion of the eyeball and of the lids was seen and hard necroses were present. This finding was still present after 8 days and the eye ball of one rabbit was leaking. The control eyes were without any findings at any time point during the study.

Overall remarks, attachments

Remarks on results including tables and figures

Under the experimental conditions of the test, the tested material caused severe and irreversible eye damage to the eyes of the test animals. Therefore the test material has to be classified as R41 according to EU criteria and as Category 1 according to GHS criteria.

Applicant's summary and conclusion

Interpretation of results

irritating

7.5 Repeated dose toxicity

7.5.1 Repeated dose toxicity: oral

Endpoint study record: Repeated dose toxicity: oral, IUC4#2/Ch.5.4

| UUID |  | IUC5-aefd2ce5-6b50-4162-a20e-85b568c0e080 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-13 10:50:11 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Eli, Lilly and Co., unveroeffentlichte Untersuchung,¦(1965);¦Zitiert in: Pesticide residues in food, FAO plant¦protection paper 1, 59-82, (1975) |  |  |  |  |  |  |  |  |

Materials and methods

Principles of method if other than guideline

Method: other: no data

GLP compliance

no

Test materials

Details on test material

IUCLID4 Test substance: other TS: sec-Butylamin Acetat

Test animals

Species

rat

Strain

no data

Sex

male/female

Administration / exposure

Route of administration

oral: feed

Duration of treatment / exposure

3 Monate

Frequency of treatment

kontinuierlich im Futter

Doses/concentrations

312.5, 625, 1250, 2500, 5000 ppm im Futter (ca. 14, 18, 57, 114, 227 mg/kg/d)

Control animals

yes, concurrent no treatment

Details on study design

Post-exposure period: keine Angaben

Results and discussions

Results of examinations

Any other information on results incl. tables

RS-Freetext: Die Substanz wurde als Acetatsalz verfuettert. Die Koerpergewichtsentwicklung war bei der obersten Dosierung verzoegert. Weiterhin wurde eine dosisabhaengige Leukopenie bei beiden Geschlechtern beschrieben. Andere klinisch chemische, haematologische, makroskopische bzw. histopathologischen Befunde traten nicht auf. Ein NOEL ist in der Sekundaerquelle "FAO plant protection paper 1, (1975)" nicht angegeben.

Endpoint study record: Repeated dose toxicity: oral, IUC4#3/Ch.5.4

| UUID |  | IUC5-ef276ff4-83ea-491e-9341-69079badf5d3 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-13 10:50:12 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Eli, Lilly and Co., unveroeffentlichte Untersuchung,¦(1965);¦Zitiert in: Pesticide residues in food, FAO plant¦protection paper 1, 59-82, (1975) |  |  |  |  |  |  |  |  |
|  | Worthing, Charles R. (ed.) in: The Pesticide Manual - A¦world compendium, 8th ed. Suffolk, S. 1660, (1987) |  |  |  |  |  |  |  |  |

Materials and methods

Principles of method if other than guideline

Method: other: no data

GLP compliance

no

Test materials

Details on test material

IUCLID4 Test substance: other TS: sec-Butylamin Acetat

Test animals

Species

rat

Strain

no data

Sex

male/female

Administration / exposure

Route of administration

oral: feed

Duration of treatment / exposure

2 Jahre

Frequency of treatment

kontinuierlich im Futter

Doses/concentrations

1250, 2500, 5000 ppm im Futter (ca. 50, 100, 200 mg/kg/d)

Control animals

yes, concurrent no treatment

Details on study design

Post-exposure period: keine Angaben

Results and discussions

Effect levels

| Endpoint | Effect level | Based on | Sex | Basis for effect level / Remarks |
| --- | --- | --- | --- | --- |
| NOAEL | ca. 100 mg/kg bw/day (nominal) |  | male/female |  |

Results of examinations

Any other information on results incl. tables

RS-Freetext: Die Substanz wurde als Acetatsalz verabreicht. Die Mortalitaetsrate blieb unbeeinflusst. Bei maennlichen Tieren wurde eine verzoegerte Koerpergewichtsentwicklung sowie erhoehtes Nieren- und erniedrigtes Prostata- bzw. Hodengewicht beschrieben. In der Sekundaerquelle "FAO plant protection paper 1, (1975)" fehlt ein Hinweis, ob die Organe histopathologisch untersucht wurden. Weiterhin schliesst die Sekundaerquelle aufgrund der Daten eine Leukopenie als Substanzbezug nicht aus.

7.5.2 Repeated dose toxicity: inhalation

Endpoint study record: Repeated dose toxicity: inhalation, IUC4#5/Ch.5.4

| UUID |  | IUC5-fa2b2d77-471f-43be-89c6-e435f4c1bdfd |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-13 10:50:14 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Gage J.C.: Br. J. industr. Med. 27, 1-18, (1979) |  |  |  |  |  |  |  |  |

Materials and methods

Principles of method if other than guideline

Method: other

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Details on test material

IUCLID4 Test substance: as prescribed by 1.1 - 1.4

Test animals

Species

rat

Strain

other: Alderley Park

Sex

male

Administration / exposure

Route of administration

inhalation

Duration of treatment / exposure

13 Tage

Frequency of treatment

6.5 Stunden/Tag

Doses/concentrations

0.7 mg/l

Control animals

other: no data specified

Details on study design

Post-exposure period: keine

Results and discussions

Results of examinations

Any other information on results incl. tables

RS-Freetext: Es wurden u.a. Lethargie und verzoegerte Koerpergewichtsentwicklung beschrieben. Bei der Autopsie waren die Organe ohne Befund. Detaillierte Informationen fehlen.

7.5.3 Repeated dose toxicity: dermal

Endpoint study record: Repeated dose toxicity: dermal, IUC4#1/Ch.5.4

| UUID |  | IUC5-aa96f8c2-84f3-46fc-aff7-c36684382aee |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-13 10:50:13 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Eli, Lilly and Co., unveroeffentlichte Untersuchung,¦(1965);¦Zitiert in: Pesticide residues in food, FAO plant¦protection paper 1, 59-82, (1975) |  |  |  |  |  |  |  |  |

Materials and methods

Principles of method if other than guideline

Method: other: no data

GLP compliance

no

Test materials

Details on test material

IUCLID4 Test substance: other TS: sec-Butylamin Acetat

Test animals

Species

rabbit

Strain

no data

Sex

male/female

Administration / exposure

Details on exposure

Route of Administration: dermal

Duration of treatment / exposure

20 Tage

Frequency of treatment

taeglich

Doses/concentrations

2 ml/kg einer 10 %igen Loesung

Control animals

other: no data specified

Details on study design

Post-exposure period: keine Angaben

Results and discussions

Results of examinations

Any other information on results incl. tables

RS-Freetext: Die Substanz wurde als Acetatsalz an der intakten und skarifizierten Haut geprueft. Als einziger Befund wurde von der Sekundaerquelle "FAO plant protection paper 1, (1975)" Durchfall bei einem Tier berichtet. Hautbefunde traten offensichtlich nicht auf.

7.5.4 Repeated dose toxicity: other routes

Endpoint study record: Repeated dose toxicity: other routes, IUC4#4/Ch.5.4

| UUID |  | IUC5-86975dfa-a689-45c9-b191-90c9519eb1ff |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-13 10:50:15 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Eli, Lilly and Co., unveroeffentlichte Untersuchung,¦(1965);¦Zitiert in: Pesticide residues in food, FAO plant¦protection paper 1, 59-82, (1975) |  |  |  |  |  |  |  |  |
|  | Worthing, Charles R. (ed.) in: The Pesticide Manual - A¦world compendium, 8th ed., Suffolk, S.1660, (1987) |  |  |  |  |  |  |  |  |

Materials and methods

Principles of method if other than guideline

Method: other: no data

GLP compliance

no

Test materials

Details on test material

IUCLID4 Test substance: other TS: sec-Butylamin Acetat

Test animals

Species

dog

Strain

other: Beagle und Mongrel

Sex

male/female

Administration / exposure

Route of administration

other: als Kapsel bzw. im Futter

Duration of treatment / exposure

2 Jahre

Frequency of treatment

taeglich

Doses / concentrations

31, 62.5, 125, 250 mg/kg/d

Control animals

yes, concurrent no treatment

Details on study design

Post-exposure period: keine Angaben

Results and discussions

Effect levels

| Endpoint | Effect level | Based on | Sex | Basis for effect level / Remarks |
| --- | --- | --- | --- | --- |
| NOAEL | 125 mg/kg bw/day |  | male/female |  |

Results of examinations

Any other information on results incl. tables

RS-Freetext: Die Substanz wurde als Acetatsalz verabreicht. 2 Tiere pro Geschlecht erhielten die Substanz zusaetzlich im Futter (10000 ppm). Die oberste Dosierung (Kapselgabe) war innerhalb von 3 Wochen letal. Es kam zu Traenenfluss und Hornhautentzuendung bei 3/4 Tieren. Pupillenerweiterungen traten bei allen behandelten Hunden auf. Bei Kontrolltieren war dies nur einmal der Fall. Der Tod von je einem maennlichen Tier in der 125 und 63 mg/kg Behandlungsgruppe wurde als nicht substanzbedingt gewertet. Hunde, die die Substanz im Futter erhielten, hatten ebenfalls Keratitis. Darueber hinaus waren Haemoglobin und Haematokritwerte erniedrigt sowie das Milz- und Lebergewicht erhoeht. Es wurden keinerlei histopathologische Veraenderungen beobachtet.

7.6 Genetic toxicity

7.6.1 Genetic toxicity in vitro

Endpoint study record: Genetic toxicity in vitro, IUC4#1/Ch.5.5

| UUID |  | IUC5-ce330f8a-ea75-40ca-b420-f969b7563e90 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-13 10:50:16 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Klopman G. et al.: Mutation Research, 343-356, (1985) |  |  |  |  |  |  |  |  |
|  | Mortelmans K. und Riccio E.S., unveroeffentlichte¦Untersuchung, unbekannte Auftraggeber, (1980);¦Zitiert in: Pesticide residues in food, FAO plant protection¦paper 42, 25-26, (1982) |  |  |  |  |  |  |  |  |
|  | Waters M.D.: Basic Life Science 21, 275-326, (1982) |  |  |  |  |  |  |  |  |

Materials and methods

Type of genotoxicity

gene mutation

Type of study

bacterial reverse mutation assay (e.g. Ames test)

Principles of method if other than guideline

Method: other: nach Ames, B.N. et al.: Mutation Research 31, 347

GLP compliance

no data

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Details on test material

IUCLID4 Test substance: as prescribed by 1.1 - 1.4

Method

Species/strain

| Species/strain | other: Salmonella typhimurium TA 1535, TA 1537, TA 1538, TA 98, TA 100 und Escherichia coli WP2 uvr A |
| --- | --- |
| Metabolic activation | with and without |

Test concentrations

0.07 - 36.5 mg/Platte

Details on test system and conditions

IUCLID4 Type: Ames test

Results and discussions

Test results

| Metabolic activation | with and without |
| --- | --- |
| Test system | other: Salmonella typhimurium TA 1535, TA 1537, TA 1538, TA 98, TA 100 und Escherichia coli WP2 uvr A |
| Genotoxicity | negative |

Overall remarks, attachments

Remarks on results including tables and figures

Year: 1975 (unclear if "Year of test guideline" or "Year of study completion".) RM-Freetext: Studie nicht identisch mit Klopman G. et al., (1985) und Waters M.D., (1982). Das zusaetzlich getestete Phosphatsalz war ebenfalls nicht mutagen.

Endpoint study record: Genetic toxicity in vitro, IUC4#2/Ch.5.5

| UUID |  | IUC5-87b08a27-4262-474f-b566-eb310d9619b0 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | Collette / Epona Associates, LLC / Willington / United States |
| Date |  | 2013-03-05 13:24:06 EST |
| Remarks |  |  |

Administrative Data

| Purpose flag | key study |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Publication |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Zeiger E. et al | 1987 | Salmonella Mutagenicity Tests III. Results from the Testing of 255 Chemicals | Environmental Mutagenesis 9, Supp. 9, 1-110, (1987) |  |  |  |  |  |
| publication | Klopman G. et al. | 1985 |  | Mutation Research, 343-356, (1985) |  |  |  |  |  |
|  | Waters M.D. | 1982 |  | Basic Life Science 21, 275-326, (1982) |  |  |  |  |  |

Materials and methods

Type of genotoxicity

gene mutation

Type of study

bacterial reverse mutation assay (e.g. Ames test)

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| equivalent or similar to | OECD Guideline 471 (Bacterial Reverse Mutation Assay) |  |

Principles of method if other than guideline

Method: Haworth, S. et al.: Environ. Mutagen. 5, Suppl. 1, 3-142

GLP compliance

no data

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 13952-84-6 |

Details on test material

Purchased from Aldrich, 99% reported purity

Method

Species/strain

| Species/strain | S. typhimurium TA 1535, TA 1537, TA 98 and TA 100 |
| --- | --- |
| Metabolic activation | with and without |
| Metabolic activation system | S-9 fraction of Aroclor 1254-induced male Sprague-Dawley rat and male Syrian hamster livers |

Test concentrations

up to 3333 ug/plate

Vehicle

distilled water

Controls

| Solvent / vehicle controls | yes |
| --- | --- |
| Positive controls | yes |
| Positive control substance | sodium azide. 9-aminoacridine. other: 4-nitro-o-phenylenediamine; 2-aminoanthracene. |

Details on test system and conditions

preincubation assay. Five doses tested in triplicate with independent repeat.

Evaluation criteria

Mutagenic (+) = if a dose related increase over the corresponding solvent control was seen. weakly mutagenic (+W) = low level dose response. questionable (?) = dose related increase insufficienlty high to justify +W (single dose elevated over control, non dose related increase).

Statistics

Not reported

Results and discussions

Test results

| Species/strain | S. typhimurium TA 1535, TA 1537, TA 98 and TA 100 |
| --- | --- |
| Metabolic activation | with and without |
| Test system | all strains/cell types tested (Salmonella typhimurium TA 1535, TA 1537, TA 1538, ) |
| Genotoxicity | negative |
| Cytotoxicity | no |
| Vehicle controls valid | yes |
| Positive controls valid | yes |

Any other information on results incl. tables

Mutagenic responses of Salmonella tester strains (mean + SEM; three plates) to sec-Butylamine 10% HLI= Aroclor 1254-induced hamster liver S-9 10% RLI= Aroclor 1254-induced rat liver S-9 NA = not activated (-) = nonmutagenic Solvent = H2O, distilled water [0.000 = solvent control] POS = positive control Test conducted at Case Western Reserve University Dose; TA100; TA1535. NA; 10% HLI; 10% RLI; NA; 10% HLI; 10% RLI. (-); (-); (-); (-); (-); (-); (-); (-); (-); (-); (-); (-). µg/plate; MEAN; SEM; MEAN; SEM; MEAN; SEM; MEAN; SEM; MEAN; SEM; MEAN; SEM. 0.000; 100; 6.7; 157; 12.2; 147; 6.4; 21; 1.5; 17; 2.0; 17; 2.3. 10.000; 19; 2.0; 24; 1.2; 21; 1.2. 33.000; 100; 8.5; 141; 1.5; 146; 0.9; 19; 2.5; 23; 2.8; 19; 2.1. 100.000; 97; 3.8; 144; 7.7; 127; 9.6; 14; 1.5; 22; 1.7; 14; 0.6. 333.000; 99; 4.6; 135; 8.0; 154; 6.6; 21; 3.1; 14; 1.5; 20; 2.7. 1000.000; 99; 1.2; 124; 8.3; 131; 5.5; 10; 2.0; 7; 1.3; 4; 2.1. 3333.000; 93; 4.3; 123; 4.4; 131; 1.5. POS; 1011; 117.4; 880; 68.6; 1534; 246.9; 1340; 152.5; 502; 11.3; 611; 65.3. Dose; TA1537; TA98. NA; 10% HLI; 10% RLI; NA; 10% HLI; 10% RLI. (-); (-); (-); (-); (-); (-); (-); (-); (-); (-); (-); (-). µg/plate; MEAN; SEM; MEAN; SEM; MEAN; SEM; MEAN; SEM; MEAN; SEM; MEAN; SEM. 0.000; 10; 4.1; 12; 2.0; 16; 1.5; 30; 1.7; 40; 6.0; 29; 2.3. 10.000; 7; 0.9; 17; 1.5; 11; 2.2. 33.000; 7; 1.2; 12; 1.2; 12; 1.2; 28; 4.7; 31; 3.4; 27; 3.2. 100.000; 7; 1.5; 15; 2.5; 10; 0.6; 30; 2.8; 35; 2.0; 27; 0.0. 333.000; 6; 1.5; 13; 0.3; 14; 5.5; 26; 3.2; 39; 2.8; 28; 2.5. 1000.000; 5; 0.9; 1; 0.7; 3; 1.2; 22; 0.9; 37; 2.3; 31; 1.9. 3333.000; 21; 2.3; 26; 2.4; 26; 1.7. POS; 678; 121.2; 431; 21.4; 521; 68.6; 186; 25.4; 516; 66.2; 1044; 133.0.

Applicant's summary and conclusion

Interpretation of results

negative with metabolic activation

negative without metabolic activation

Endpoint study record: Genetic toxicity in vitro, IUC4#3/Ch.5.5

| UUID |  | IUC5-a59e99c0-59ae-4ac4-94cc-071d0708d8c0 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-13 10:50:19 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Klopman G. et al.: Mutation Research, 343-356, (1985) |  |  |  |  |  |  |  |  |
|  | Mortelmans K. und Riccio E.S., unveroeffentlichte¦Untersuchung, unbekannte Auftraggeber, (1980);¦Zitiert in: Pesticide residues in food, FAO plant protection¦paper 42, 25-26, (1982) |  |  |  |  |  |  |  |  |
|  | Waters M.D.: Basic Life Science 21, 275-326, (1982) |  |  |  |  |  |  |  |  |

Materials and methods

Type of genotoxicity

DNA damage and/or repair

Type of study

mitotic recombination in Saccharomyces cerevisiae

Principles of method if other than guideline

Method: other: no data

GLP compliance

no data

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Details on test material

IUCLID4 Test substance: as prescribed by 1.1 - 1.4

Method

Species/strain

| Species/strain | other: Saccharomyces cerevisae D3 |
| --- | --- |
| Metabolic activation | with and without |

Test concentrations

0.05 - 0.5%

Results and discussions

Test results

| Metabolic activation | with and without |
| --- | --- |
| Test system | other: Saccharomyces cerevisae D3 |
| Genotoxicity | negative |

Overall remarks, attachments

Remarks on results including tables and figures

RM-Freetext: Studie nicht identisch mit Klopman G. et al., (1985) und Waters M.D., (1982). Das zusaetzlich getestete Phosphatsalz war ebenfalls nicht mutagen.

Endpoint study record: Genetic toxicity in vitro, IUC4#4/Ch.5.5

| UUID |  | IUC5-96d019bc-f0db-4fd7-9dc8-5b2962f22001 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-13 10:50:20 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Klopman G. et al.: Mutation Research, 343-356, (1985) |  |  |  |  |  |  |  |  |
|  | Waters M.D.: Basic Life Science 21, 275-326, (1982) |  |  |  |  |  |  |  |  |
|  | Zeiger E. et al.: Environmental Mutagenesis 9, Supp. 9,¦1-110, (1987) |  |  |  |  |  |  |  |  |

Materials and methods

Type of genotoxicity

DNA damage and/or repair

Type of study

mitotic recombination in Saccharomyces cerevisiae

Principles of method if other than guideline

Method: other: nach Waters, M.D. et al., In: Fleck, R.A. und Hollaender, A. (eds.), Genetic Toxicology, Plenum, New York, Seite 275-326

GLP compliance

no data

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Details on test material

IUCLID4 Test substance: as prescribed by 1.1 - 1.4

Method

Species/strain

| Species/strain | other: Saccharomyces cerevisae D3 |
| --- | --- |
| Metabolic activation | with and without |

Test concentrations

keine Angaben

Results and discussions

Test results

| Metabolic activation | with and without |
| --- | --- |
| Test system | other: Saccharomyces cerevisae D3 |
| Genotoxicity | negative |

Overall remarks, attachments

Remarks on results including tables and figures

Year: 1982 (unclear if "Year of test guideline" or "Year of study completion".) RM-Freetext: Bei den Referenzen Klopman G. et al., (1985), Waters M.D., (1982) und Zeiger E. et al., (1987) handelt es sich um verschiedene Publikationen, welche die Ergebnisse einer Studie aufzeigen, die von der Case Western Reserve University im Rahmen des Gene Tox Programms der US EPA durchgefuehrt wurden.

Endpoint study record: Genetic toxicity in vitro, IUC4#5/Ch.5.5

| UUID |  | IUC5-66d6ee73-f051-4c1e-825e-8c4f31dbc606 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-13 10:50:21 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Klopman G. et al.: Mutation Research, 343-356, (1985) |  |  |  |  |  |  |  |  |
|  | Waters M.D.: Basic Life Science 21, 275-326, (1982) |  |  |  |  |  |  |  |  |

Materials and methods

Type of genotoxicity

DNA damage and/or repair

Type of study

DNA damage and repair assay, unscheduled DNA synthesis in mammalian cells in vitro

Principles of method if other than guideline

Method: other: nach Waters, M.D. et al., In: Fleck, R.A. und Hollaender, A. (eds.), Genetic Toxicology, Plenum, New York, Seite 275-326

GLP compliance

no data

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Details on test material

IUCLID4 Test substance: as prescribed by 1.1 - 1.4

Method

Species/strain

| Species/strain | other: Escherichia coli pol A + und pol A- |
| --- | --- |
| Metabolic activation | no data |

Test concentrations

keine Angaben

Details on test system and conditions

IUCLID4 Type: DNA damage and repair assay

Results and discussions

Test results

| Metabolic activation | no data |
| --- | --- |
| Test system | other: Escherichia coli pol A + und pol A- |
| Genotoxicity | positive |

Overall remarks, attachments

Remarks on results including tables and figures

Year: 1982 (unclear if "Year of test guideline" or "Year of study completion".) RM-Freetext: Die im Escherichia coli pol A Differential Toxicity Assay verwendeten Konzentrationen sowie andere Versuchsdetails wurden nicht berichtet.

Endpoint study record: Genetic toxicity in vitro, IUC4#6/Ch.5.5

| UUID |  | IUC5-3216580d-200f-4d63-a870-74fc2890545a |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-13 10:50:22 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Klopman G. et al.: Mutation Research, 343-356, (1985) |  |  |  |  |  |  |  |  |
|  | Waters M.D.: Basic Life Science 21, 275-326, (1982) |  |  |  |  |  |  |  |  |

Materials and methods

Type of genotoxicity

DNA damage and/or repair

Type of study

DNA damage and repair assay, unscheduled DNA synthesis in mammalian cells in vitro

Principles of method if other than guideline

Method: other: nach Waters, M.D. et al., In: Fleck, R.A. und Hollaender, A. (eds.), Genetic Toxicology, Plenum, New York, Seite 275-326

GLP compliance

no data

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Details on test material

IUCLID4 Test substance: as prescribed by 1.1 - 1.4

Method

Species/strain

| Species/strain | other: Bacillus subtilis rec + und rec - |
| --- | --- |
| Metabolic activation | no data |

Test concentrations

keine Angaben

Details on test system and conditions

IUCLID4 Type: DNA damage and repair assay

Results and discussions

Test results

| Metabolic activation | no data |
| --- | --- |
| Test system | other: Bacillus subtilis rec + und rec - |
| Genotoxicity | negative |

Overall remarks, attachments

Remarks on results including tables and figures

Year: 1982 (unclear if "Year of test guideline" or "Year of study completion".)

7.8 Toxicity to reproduction

7.8.1 Toxicity to reproduction

Endpoint study record: Toxicity to reproduction, IUC4#1/Ch.5.8.1

| UUID |  | IUC5-b6b2d00b-b998-4e1f-8993-21c39faa6e3b |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-13 10:50:23 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Clegg D.J.: Toxicol. Environ. Sci. 4, 45-59, (1979) |  |  |  |  |  |  |  |  |
|  | Elanco Company, unveroeffentlichte Untersuchung, (Aug.¦1969); ¦zitiert in: Pesticide residues in food, FAO plant¦protection paper 1, 59-82, (1975) |  |  |  |  |  |  |  |  |
|  | Eli, Lilly and Co., unveroeffentlichte Untersuchung, (1970);¦zitiert in: Pesticide residues in food, FAO plant¦protection paper 1, 59-82, (1975) |  |  |  |  |  |  |  |  |

Materials and methods

Test type

fertility

Principles of method if other than guideline

Method: other: no data

GLP compliance

no

Test materials

Details on test material

IUCLID4 Test substance: other TS: sec-Butylamin Acetat

Test animals

Species

rat

Strain

no data

Sex

male/female

Administration / exposure

Route of administration

oral: feed

Duration of treatment / exposure

Exposure period: Mehrgenerationsstudie (4 Generationen) Duration of test: 4 Generationen

Frequency of treatment

kontinuierlich im Futter

Doses / concentrations

500 ppm und 2500 ppm im Futter (ca. 20 und 100 mg/kg/d)

Control animals

yes, concurrent no treatment

Results and discussions

Results of examinations: offspring

Any other information on results incl. tables

RS-Freetext: parentale Toxizitaet: Die Koerpergewichtsentwicklung bei den mit 2500 ppm behandelten Tieren war ueber den gesamten Studienverlauf verzoegert. Keine Beeinflussung der Reproduktionsparameter. Die Substanz wurde als Acetatsalz verabreicht. Es wurden je 2 Wuerfe bei den 4 Generationen durchgefuehrt. In der F0 Generation wurden zusaetzlich 6 Wuerfe durchgefuehrt. Das Reproduktionsverhalten von Kaninchen, die maximal 2500 ppm sekundaeres Butylamin als Phosphatsalz im Futter erhielten, war ebenfalls unbeeinflusst.

7.8.2 Developmental toxicity / teratogenicity

Endpoint study record: Developmental toxicity / teratogenicity, IUC4#1/Ch.5.8.2

| UUID |  | IUC5-7c2feda6-af39-45ca-920d-b04978c8ebbf |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-13 10:50:24 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Eli, Lilly und Co., unveroeffentlichte Untersuchung, (1965);¦zitiert in: Pesticide residues in food, FAO plant¦protection paper 1, 59-82, (1975) |  |  |  |  |  |  |  |  |

Materials and methods

Principles of method if other than guideline

Method: other: no data

GLP compliance

no

Test materials

Details on test material

IUCLID4 Test substance: other TS: sec-Butylamin Acetat

Test animals

Species

rabbit

Strain

Dutch

Administration / exposure

Route of administration

oral: gavage

Duration of treatment / exposure

8.-18. Tag der Traechtigkeit

Frequency of treatment

taeglich

Doses / concentrations

75 mg/kg und 150 mg/kg

Control animals

yes, concurrent no treatment

Further details on study design

Sex: female Duration of test: 28 Tage

Results and discussions

Any other information on results incl. tables

RS-Freetext: - nicht teratogen Die Substanz wurde als Acetatsalz verabreicht. Erniedrigung von Fetengewicht und Anzahl der lebenden Feten bei 150 mg/kg wurden als einziger Befund in Sekundaerquelle zitiert.

7.12 Additional toxicological information

Endpoint study record: Additional toxicological information, IUC4#1/Ch.5.11

| UUID |  | IUC5-895f95e3-cdf7-443a-8af4-a99cad5a96f1 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2008-12-13 10:50:26 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Gage J.C.: Br. J. industr. Med. 27, 1-18, (1979) |  |  |  |  |  |  |  |  |
|  | Pesticide residues in food, FAO plant protection paper 1,¦59-82, (1975) |  |  |  |  |  |  |  |  |

Materials and methods

Type of information

Type: other

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Overall remarks, attachments

Remarks on results including tables and figures

RM-Freetext: Eine sympathomimetische Wirkung wird aus akuten toxikologischen und pharmakologischen Studien abgeleitet.

14 Information requirements

14.2 Alternative name request

Legal entity: BASF SE

| UUID |  | IUC4-fa6b5320-73c6-367e-9073-49dcfd348866 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-29 10:26:05 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

General information

| Legal entity name | BASF SE |
| --- | --- |
| Legal entity type | company |

Reference substance: sec-butylamine

| UUID |  | ECB5-ee388862-8c11-4885-be00-db9a1e260862 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2009-10-26 10:39:38 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

General information

| Reference substance name | sec-butylamine |
| --- | --- |

EC inventory

| EC number | 237-732-7 | CAS number | 13952-84-6 |
| --- | --- | --- | --- |
| EC name | sec-butylamine |  |  |
| Molecular formula | C4H11N |  |  |