SIDS Initial Assessment Report

For

SIAM 31

Oxford, United Kingdom, 19-22 October, 2010

| 1. Category Name: | Alkyl chlorosilanes (Chlorotrimethylsilane, Dichlorodimethylsilane, Trichloromethylsilane, and Trichloroethylsilane) Category |
| --- | --- |
| 2. CAS Number and name: | 75-77-4 Chlorotrimethylsilane 75-78-5 Dichloro(dimethyl)silane 75-79-6 Trichloro(methyl)silane 115-21-9 Trichloro(ethyl)silane |
| 3. Sponsor Country: | United States Dr. Oscar Hernandez Director, Risk Assessment Division (7403M) U.S. Environmental Protection Agency 1200 Pennsylvania Ave, N.W. Washington, DC 20460 Phone: 202-564-7641 |
| 4. Shared Partnership with: | Silicones Environmental Health and Safety Council (SEHSC): Clariant LSM (Florida), Inc. Evonik Dow Corning Corporation Momentive Performance Materials Bluestar Silicones Shin-Etsu Silicones of America Wacker Silicones, A Division of Wacker Chemical Corporation |
| 5. Roles/Responsibilities of the Partners: |  |
| Name of industry sponsor /consortium | Silicones Environmental Health and Safety Council Contact point: Tracy Hill SEHSC (703) 788-6562 |
| Process used | The SEHSC produced the documents; EPA reviewed the documents and provided additional information where there were data gaps. |
| 6. Sponsorship History |  |
| How was the chemical or category brought into the OECD HPV Chemicals Programme ? | Documents were prepared and reviewed by industry prior to submission to sponsor country. Sponsor country conducted reviews of submitted data and offered comments to industry. Industry prepared and resubmitted documents for consideration at SIAM 31. no testing ( X ) testing ( ) |
| 7. Review Process Prior to the SIAM: | Industry prepared the documents, conducted an independent third party expert review then submitted the documents to EPA for submission to the OECD. The U.S. EPA conducted a review of this case. Discussions were held. Documents were then finalized for submission to the OECD. |
| 8. Quality check process: | Literature searches were conducted by sponsor country to determine if all relevant data have been included in this submission. |
| 9. Date of Submission: |  |
| 10. Date of last Update: |  |
| 11. Comments: | The Alkyl Chlorosilanes Category uses data from the hydrolysis product hydrogen chloride (HCl; CAS No. 7647-01-0), which was presented and agreed upon under the OECD HPV Chemicals Program. These documents are available for review at http://cs3-hq.oecd.org/scripts/hpv/http://www.chem.unep.ch/irptc/sids/OECDSIDS/64175.pdf. The Alkyl Chlorosilanes Category also used data from one structurally similar, highly reactive alkylsilane, methyltrimethoxysilane (CAS No. 1185-55-3) and two silanol hydrolysis products including trimethylsilanol (CAS No. 1066-40-6) and dimethylsilanediol (CAS No. 1066-42-8). Methyltrimethoxysilane (CAS No. 1185-55-3) was sponsored under the OECD SIDS Program. The summaries and evaluations contained in this assessment report may be based on unpublished proprietary data submitted for the purpose of the OECD assessment. In many cases, national/regional regulatory authorities will not accept a regulatory submission based on the summaries and evaluation of unpublished proprietary data contained in this [these] assessment report[s] unless they have received the data on which the summaries and evaluation are based, either from: - the owner of the data; or - a second party that has obtained permission from the owner of the data for this purpose; or alternatively - the applicant has received permission from the data owner that the summary and evaluation contained in this [these] assessment report[s] may be used in lieu of the data; or - following expiry of any period of exclusive use, mandatory compensation, where required, has been offered unless the period of protection for the proprietary data concerned has expired. Applicants who wish to use data in this assessment report should ensure that the use is consistent with the legal requirements in the country/region in which it is proposed to use the data. |

SIDS INITIAL ASSESSMENT PROFILE

| Category Name | Alkyl chlorosilanes (Chlorotrimethylsilane, Dichlorodimethylsilane, Trichloromethylsilane, and Trichloroethylsilane) Category |
| --- | --- |
| CAS Nos. | Sponsored Substances 75-77-4 75-78-5 75-79-6 115-21-9 Hydrolysis Products 1066-40-6 1066-42-8 2445-53-6 5651-16-1 7647-01-0 Analogue Substance 1185-55-3 |
| Chemical Names | Sponsored Substances Chlorotrimethylsilane (CTMS) Dichlorodimethylsilane (C2DMS) Trichloromethylsilane (C3MS) Trichloroethylsilane (C3ES) Hydrolysis Products Trimethylsilanol (TMS) Dimethylsilanediol (DMSD) Methylsilanetriol (MST) Ethylsilanetriol (EST) Hydrogen Chloride (HCl) Analogue Substance Methyltrimethoxysilane (MTMS) |
| Structural Formula | 75-77-4. 75-78-5. 75-79-6. - 115-21-9. Hydrolysis Products 1066-40-6 1066-42-8 2445-53-6 5651-16-1 7647-01-0 Analogue substance 1185-55-3 |
| SUMMARY CONCLUSIONS OF THE SIAR Category/Analogue Justification The category consists of four sponsored chlorosilanes: chlorotrimethylsilane (CTMS, CAS No. 75-77-4), dichlorodimethylsilane (C2DMS, CAS No. 75-78-5), trichloromethylsilane (C3MS, CAS No. 75-79-6), and trichloroethylsilane (C3ES, CAS No. 115-21-9). These chemicals are grouped into a category based on similar molecular structure, high reactivity, physicochemical and toxicological properties. Similar High Reactivity The chlorine group is the most active functional group on these molecules and determines many aspects of the behaviour of the category members. These chlorosilanes undergo rapid hydrolysis in the presence of water to form one to three moles of hydrochloric acid and one mole of silanol, depending on the parent substance. Hydrolysis is the primary reaction in aqueous systems and has been shown to occur very quickly (half-life <17 seconds) for CTMS, C2DMS, and C3MS. No data is available for C3ES. Similar Chemical/Physical Properties All category members are liquids with melting points, boiling points and vapour pressures that are largely dependent on molecular weight. Due to their high reactivity with water, water solubility and partition coefficient are not relevant endpoints. Similar Toxicological Properties These category members are severely irritating and corrosive at the site of contact (i.e., respiratory tract, skin, and eyes). The approach to address the SIDS endpoints for these chlorosilanes is to utilize available data from the four category members and three hydrolysis products, including hydrogen chloride. Chlorosilanes react rapidly when exposed to water or polar reagents, producing hydrogen chloride (HCl; CAS No. 7647-01-0) and the corresponding silanols. The primary hydrolysis products for these sponsored chlorosilanes are hydrogen chloride (HCl; CAS No. 7647-01-0), common to the hydrolysis of all chlorosilanes and the corresponding silanol. Data are available for two of the silanol hydrolysis products: trimethylsilanol (TMS; CAS No. 1066-40-6) and dimethylsilanediol (DMSD; CAS No. 1066-42-8). Repeated-dose and reproductive toxicity endpoints for the category members are characterized through the use of data from HCl, and two of the silanol hydrolysis products; TMS and DMSD. Aquatic toxicity endpoints are also characterized with data from the hydrolysis products (HCl, TMS and DMSD) and the structural analogue methyltrimethoxysilane (MTMS: CAS No. 1185-55-3) (which produces methylsilanetriol on hydrolysis, the hydrolysis product of C3MS). MTMS has previously been assessed in the OECD HPV programme: http://webnet.oecd.org/hpv/ui/Search.aspx. Hydrogen chloride has also previously been assessed in the OECD HPV programme: http://www.chem.unep.ch/irptc/sids/oecdsids/sidspub.html. Physical-chemical properties The EPISuite program (v 4.0) developed by the U.S. Environmental Protection Agency and Syracuse Research Corporation has not been validated for chemicals that contain silanes in their molecular structure (although some measured data are included in the training set); therefore, there is greater uncertainty associated with the calculated values and they should be used with caution whenever they are reported below. These chlorosilanes are very reactive and hydrolytically unstable; they are liquids at normal temperature and pressure. Hydrolysis studies (OECD TG 111) were conducted on three category members. The melting points range from -57.7oC (CTMS) to -105.6 oC (C3ES); boiling points range from 57.6 oC at 1013 hPa (CTMS) to 97.9oC at 1013 hPa (C3ES). Vapor pressures of these chlorosilanes range from 47.83 hPa at 20oC (C3ES) to 250.3 hPa at 25oC (CTMS). The water solubility and partition coefficient estimates are not reliable because these chlorosilanes are hydrolytically unstable. The water solubility values of the silanol hydrolysis products TMS and DMSD are 920 mg/L and 2790 mg/L, respectively. The water solubility of HCl is 673 g/L at 30°C. The log of the 1-octanol/water partition coefficient (Kow) of the silanol hydrolysis product, TMS is 1.19. Human Health There are no data for toxicokinetics of these chlorosilanes. However, chlorosilanes rapidly hydrolyze on contact with water. The hydrolysis products, TMS and DMSD, penetrated through human skin when applied neat in an in vitro test system. Also, the systemic effects found in an acute dermal toxicity study in rabbits with CTMS (discoloration of liver and lungs) provide evidence that chlorosilanes or their hydrolysis products can penetrate through the skin. These chlorosilanes may be absorbed via inhalation at high doses based on discoloration in the liver (and spleen for C2DMS) observed in acute inhalation studies. HCl -, a hydrolysis product of all chlorosilanes, will rapidly dissociate in the presence of water; its effects on skin are thought to be a result of pH change. The range of 1 hour acute inhalation (OECD 403) LC50’s for the category members is 8.35 mg/L (C3MS) to 18.92 mg/L (CTMS). The 4-hr nose-only LC50 for C3MS was 5.43 mg/L in rats. The acute inhalation hazard posed by an individual chlorosilane, as defined by an LC50 value, is directly proportional to its chlorine content and subsequently to the HCl that is liberated during hydrolysis. The principal clinical signs were indicative of respiratory and ocular effects. The dermal LD50 for CTMS applied without vehicle using methods similar to OECD TG 402 in rabbits was 1513 (females) and 2030 (males) mg/kg bw; for C3MS (no vehicle; similar to OECD TG 402) the dermal LD50 in rabbits was 1068 (females) and 1719 (males) mg/kg bw. Necrosis, irritation and loss of body weight were common clinical signs for both substances. The primary necropsy findings for CTMS included various discolorations of the liver or lungs; no remarkable findings were noted for C3MS. The oral (similar to OECD TG 401) LD50 for CTMS in rats was 5636 (females) and 4811 (males) mg/kg bw; for C3MS the oral LD50 in rats was 3594 (females) and 2057 (males) mg/kg bw. Necrosis, irritation and loss of body weight were common clinical signs for both substances; chlorosilanes are widely recognized as corrosive and considered irritants. The primary necropsy findings for CTMS included various discolorations of the liver or lungs; no remarkable findings were noted for C3MS. Reported signs of toxicity for both substances in surviving animals included sluggishness, salivation, dyspnea, rales, prostration, and/or staining/greasy texture of the fur. Body weight gains were reported for all groups of surviving animals exposed to either CTMS or C3MS. There were no remarkable findings at necropsy for surviving animals for either CTMS or C3MS with the exception of the finding of lungs with dark red patches in females dosed with CMS at 2540 mg/kg bw. Acute toxicity of the hydrolysis products TMS and DMSD resulted in a 4-hour LC50 of 3151 ppm (11.6 mg/L) and an LD50 of >2000 mg/kg respectively. For TMS, common clinical signs included effects on respiration, activity, urogenital and facial staining, increased lacrimation, partial closer of eyes, and decreased urination and defecation. Necropsy findings included discoloration of the lungs, kidney, adrenal gland, liver, spleen and urinary bladder, distension of the urinary bladder, and clear fluid in uterus. For DMSD, clinical signs included impaired equilibrium, hypoactivity, and decreased respiration. There were no remarkable findings at necropsy. These sponsored chlorosilanes are corrosive and highly irritating to the skin, eyes and respiratory tract; these effects are likely due to the rapid production of HCl following hydrolysis. Similar effects are seen following exposure to HCl. The hydrolysis product, TMS, is non-irritating to the skin and slightly irritating to the eyes. No data are available on the sensitisation potential of the alkyl chlorosilanes or their hydrolysis products. Repeated dose toxicity data for these chlorosilanes are not available. Based on their rapid hydrolysis, data from the primary hydrolysis products are used to fill the repeated dose toxicity endpoint. In 90-day inhalation studies in rats and mice exposed to 0, 0.015, 0.030 or 0.075 mg/L HCl, local effects of irritation were observed at 0.015 mg/L and above. The NOAEL for systemic toxicity for HCl, excluding the local effects of irritation, has been determined to be 0.030 mg/L for rats and mice based on decreased body weight gain (mice; male rats) and decreased liver weights (male mice). In a combined inhalation repeated-dose/ reproductive/developmental toxicity screening test in rats exposed to 0.22, 1.10 or 2.20 mg/L/day hydrolysis product, TMS, there were no treatment-related effects observed up to 2.2 mg/L/day. The NOAEL for inhalation systemic toxicity of TMS was 2.20 mg/L (highest concentration tested). In a combined oral gavage repeated-dose/reproductive/developmental toxicity screening test in rats treated with 0, 80, 250 or 750 mg/kg-bw/day hydrolysis product, TMS, liver effects (increased relative liver weights of 21% in females and histopathological changes) and clinical signs of toxicity (staggering gait) were observed at 750 mg/kg-bw/day. Liver weights were increased by 17% in females at 250 mg/kg-bw/day. The NOAEL for oral systemic toxicity of TMS was 250 mg/kg-bw/day. In a combined oral gavage repeated-dose/reproductive/developmental toxicity screening test in rats treated with 0, 50, 250 or 500 mg/kg-bw/day hydrolysis product, DMSD, clinical signs of toxicity included abdominal, urogenital and muzzle soiling in males at 250 and 500 mg/kg-bw/day and muzzle soiling in females at 500 mg/kg-bw/day. Based on liver porphyria in male rats and liver vacuolation in toxicity group female rats at 500 mg/kg-bw/day, the NOAEL for oral systemic toxicity of DMSD was 250 mg/kg-bw/day. Data for mutagenicity in vitro are available for all category members. In vitro and in vivo chromosome aberration data are available for CTMS, C2DMS and C3MS. In reverse-mutation assays, CTMS, C2DMS, C3MS and C3ES were negative for mutagenicity in all Salmonella typhimurium and E. coli strains tested. CTMS, C2DMS and C3MS were negative for mutagenicity in DNA damage and repair assays and/or mammalian cell gene mutation tests with L5178Y mouse lymphoma cells. CTMS and C2DMS were also negative in a sister chromatid exchange (SCE) assay with L5178Y mouse lymphoma cells; C3MS was positive for SCEs with and without metabolic activation. CTMS, C2DMS and C3MS were negative in in vivo chromosome aberration studies. CTMS and C2DMS were negative in a chromosome aberration test with L5178Y mouse lymphoma cells; C3MS was considered to have weak clastogenic activity at cytotoxic concentrations. Based on these data, these chlorosilanes are not expected to be genotoxic. No data are available for carcinogenicity on these chlorosilanes. No data are available on the reproductive toxicity of these chlorosilanes. However; data are available for hydrolysis products, TMS (inhalation route), DMSD (oral route) and HCl. In a combined repeated-dose/reproductive/developmental toxicity screening test (OECD TG 422), no treatment-related effects on reproduction or development were observed in rats exposed whole-body to TMS at concentrations up to 2.20 mg/L. The NOAEC for reproductive toxicity was 2.20 mg/L (highest concentration tested). The NOAEC for maternal and developmental toxicity was 2.20 mg/L (highest concentration tested). In a combined repeated-dose/reproductive/developmental toxicity screening test (OECD TG 422), no treatment-related effects on reproduction or development were observed in rats exposed via oral gavage to DMSD at doses up to 500 mg/kg bw/day. The NOAEL for reproductive toxicity was 500 mg/kg bw/day (highest dose tested). The NOAEL for maternal and developmental toxicity was 500 mg/kg bw/day (highest dose tested). No reliable studies have been reported regarding toxicity to reproduction and development in animals after oral, dermal or inhalation exposure to hydrogen chloride/hydrochloric acid. However, no effects on the gonads were observed in a 90-day repeated-dose inhalation study. Based on data for the hydrolysis products, these chlorosilanes are not expected to be reproductive toxicants and are unlikely to result in developmental toxicity. These sponsored chlorosilanes possess properties indicating a hazard for human health (lethality from acute inhalation, severe skin, eye and respiratory tract irritation, corrosivity). Adequate screening-level data are available to characterize the human health hazard for the purposes of the OECD HPV Programme. Environment The chlorine group is the most active functional group on these molecules and determines many aspects of the behaviour of the category members. These chlorosilanes undergo rapid hydrolysis in the presence of water to form one to three moles of hydrochloric acid and one mole of silanol, depending on the parent substance. Hydrolysis is the primary reaction in aqueous systems and has been shown to occur very quickly (half-life <17 seconds) for CTMS, C2DMS, and C3MS. No data are available for C3ES but the reaction is expected to be as quick as for the other category members. The overall rate constants for reaction with OH radicals in the atmosphere for these chlorosilanes and resulting half-lives (12 hr day) due to indirect photolysis are estimated to range from 0.15 x 10-12 cm3/molecule-sec and 71 d (C3MS) to 1.2 x 10-12 cm3/molecule-sec and 8.9 d (Cl3ES). Photodegradation as a mode of removal is unlikely as the alkyl chlorosilanes are hydrolytically unstable. It is assumed that reaction with water vapor is the predominant degradation process in air. CTMS, C2DMS and C3MS hydrolyzed to HCl and the corresponding silanol in less than 17 seconds at pH 4, 7, and 9 and 1.5ºC These chlorosilanes hydrolyze to form one mole of their respective silanol hydrolysis products (TMS, DMSD, methylsilanetriol, and ethylsilanetriol and one to three moles of HCl.The products resulting from hydrolysis (silanol hydrolysis products) in the atmosphere are expected to further react with hydroxyl radicals. The half-lives (12 hr day) due to the atmospheric oxidation from indirect photolysis of the silanol hydrolysis products were 0.9 d (Ethylsilanetriol) to 2.5 d (TMS); the overall OH rate constants were 3.95 x 10-12 cm3/molecule-sec (TMS) and 7.19 x 10-12 cm3/molecule-sec (DMSD). HCl can react with hydroxyl radicals to form chloride free radical and water and its half-life time is calculated as 11 d. Level III Fugacity modeling, using loading rates of 1000 kg/h each for air, soil, and water, shows the following percent distribution range for the alkyl chlorosilanes: Air = 40.2 (C3ES) to 53.2 (CTMS); Soil = 1.74 (CTMS) to 17.3 (C3ES cm3/molecule-sec and 71 d (C3MS); Water = 39.7 (C3MS) to 44.9 (CTMS); Sediment = 0.145 (C3MS) to 0.225 (C3MS). However, because these chlorosilanes are very reactive and hydrolytically unstable, the substances are unlikely to be found in the environment. Therefore, Level III Fugacity modeling for the hydrolysis products (TMS, DMSD, methylsilanetriol, and ethylsilanetriol) was conducted using loading rates of 1000 kg/h each for air, soil, and water. The model estimated the following percent distribution ranges, when the silanol hydrolysis products are released simultaneously to all three compartments: Air <1.0 (all, except TMS) to 5.26 (TMS); Soil = 64.7 (TMS) to 80.1 (Ethylsilanetriol), Water = 19.8 (Ethylsilanetriol) to 29.9 (TMS); and Sediment <1.0 (all silanols). The Fugacity model cannot be applied for ionized substances such as HCl. The biodegradation of these chlorosilanes was not determined due to their rapid hydrolysis; any potential for biodegradation is likely to be of the hydrolysis products. Consequently, the only potentially biodegradable materials in the test system will be silanols. The biodegradation of MTMS was not determined due to its rapid hydrolysis; methylsilanetriol is not expected to be readily biodegradable In an OECD Guideline 310, TMS was not readily biodegradable (0% degradation after 29 days). HCl is an inorganic compound and OECD guideline tests for biodegradation are not applicable. Bioaccumulation is not anticipated since these chlorosilanes are hydrolytically unstable. The estimated BCF values for these chlorosilanes range from 9.8 to 20.6 L/kg wet-wt, suggesting low bioaccumulation potential of the substances. The estimated BCF values for these chlorosilane hydrolysis products range from 2.6 to 3.2 L/kg wet-wt. Aquatic toxicity data are not available for these chlorosilanes; the substances undergo rapid hydrolysis, which occurs during testing; exposures to the parent chlorosilane is most likely irrelevant and observed toxicity is likely due to HCl and the corresponding silanol hydrolysis product. The following acute toxicity test results for similar materials and hydrolysis products have been determined for aquatic species: Test substance; Guideline; Test type; Species; Result (mg/L). Fish. Analogous substance – similar structure. MTMS; OECD TG 203; flow-through; Rainbow trout (Oncorhynchus mykiss); 96-hr LC50 >110 (measured) 96-h LC50 >62 (calculated methylsilanetriol concentration). MTMS; ECOSAR; Fish; 96-hr LC50 = 45 (alkoxy silane category) 96-hr LC50 = 9130 (neutral organic category). Analogous substance – hydrolysis product. TMS; OECD TG 203; semi-static; Rainbow trout (Oncorhynchus mykiss); 96-hr LC50 = 271(measured). TMS; Environment Federal Bureau Berlin; static; Zebra-fish (Brachydanio rerio); 96-hr LC50 >519 (measured). TMS; ECOSAR; Fish; 96-hr LC50 = 395. DMSD; OECD TG 203; static; Rainbow trout (Oncorhynchus mykiss); 96-hr LC50 >126 (measured). DMSD; ECOSAR; Fish; 96-hr LC50 = 9320. Ethylsilanetriol; ECOSAR; Daphnid; 48-hr EC50 = 13,000. Methylsilanetriol; ECOSAR; Daphnid; 48-hr EC50 = 1000. HCl; OECD TG 203; semi-static; Rainbow trout (Oncorhynchus mykiss); 96-hr LC50 = pH 4.3 (4.92 mg/L). Aquatic invertebrates. Analogous substance – similar structure. MTMS; OECD TG 202; flow-through; Daphnia magna; 48-hr EC50>122 (measured) 48-h EC50 >57 (calculated methylsilanetriol concentration). MTMS; ECOSAR; Daphnid; 96-hr LC50 = 16 (alkoxy silane category) 96-hr LC50 = 7990 (neutral organic category). Analogous substance – hydrolysis product. TMS; OECD TG 202; semi-static; Daphnia magna; 48-hr EC50 = 124 (measured). TMS; ECOSAR; Daphnid; 48-hr EC50 = 196. DMSD; OECD TG 202; static; Daphnia magna; 48-hr EC50 >117 (measured). DMSD; ECOSAR; Daphnid; 48-hr EC50 = 3680. Ethylsilanetriol; ECOSAR; Daphnid; 48-hr EC50 = 7700. Methylsilanetriol; ECOSAR; Daphnid; 48-hr EC50 = 48,000. HCl; OECD TG 202; Daphnia magna; 48-hr EC50 = pH 5.3 (0.492 mg/L). Algae. Analogous substance – similar structure. MTMS; OECD TG 201; static; Pseudokirchneriella subcapitata; 72-hr EC50>120 (nominal) 72-hr EC50>3.6 (measured); 72-hr EC50>80 (calculated methylsilanetriol concentration). Analogous substance – hydrolysis product. TMS; OECD TG 201; static; Pseudokirchneriella subcapitata; 72-hr EC50 = 555 mg/L, 72-hr EbC50 = 566 mg/L, 72-hr ErC50 > 1053 mg/L (measured). TMS; Similar to OECD TG 201; static; Desmodesmus subspicatus; 72-hr EbC50 = 806 mg/L (nominal). TMS; ECOSAR; Green Algae; 96-hr EC50 = 64. DMSD; OECD TG 201; static; Pseudokirchneriella subcapitata; 72-hr EyC50 and ErC50 >118. DMSD; ECOSAR; Green Algae; 96-hr EC50 = 615. Ethylsilanetriol; ECOSAR; Green Algae; 96-hr EC50 = 5981. Methylsilanetriol; ECOSAR; Green Algae; 96-hr EC50 = 577. HCl; OECD TG 201; static; Selenastrum capricornutum; 72-hr EC50 =pH 5.3 (0.492 mg/L). Hydrogen Chloride (HCl) The hazard of hydrochloric acid for the environment is caused by the proton (pH effect). For this reason the effect of hydrogen chloride on the organisms depends on the buffer capacity of the aquatic ecosystem. Also the variation in acute toxicity for aquatic organisms can be explained for a significant extent by the variation in buffer capacity of the test medium. For example, LC50 values of acute fish toxicity tests varied from 4.92 to 282 mg/L. The toxicity values to Selenastrum capricornutum 72h-EC50 is pH 5.1 (0.780 mg/L) for biomass, pH 5.3 (0.492 mg/L) for growth rate and the 72h-NOEC is pH 6.0 (0.097 mg/L) for biomass and growth rate. The 48h-EC50 for Daphnia magna is pH 5.3 (0.492 mg/L) based on immobilization. These chlorosilanes exhibit very rapid hydrolysis in the environment, hence any ecotoxicity will result from their hydrolysis products. Structural analogues of the parent chlorosilane compounds, and silanol hydrolysis products show low toxicity. Another byproduct of the hydrolysis of these chlorosilanes, HCl, has properties that can result in toxicity of < 1 mg/L to aquatic organisms in poorly buffered systems, mainly due to acidification of the test medium. These sponsored chlorosilanes and their hydrolysis products have low potential for bioaccumulation. Silanol degradation products are not biodegradable . Adequate screening-level data are available to characterize the hazard for the environment for the purposes of the OECD HPV Programme. Exposure Chlorosilanes react rapidly when exposed to moisture or polar reagents; this reaction is normally heterogeneous, highly exothermic, and potentially difficult to control, requiring close control of the reaction. To prevent the rapid hydrolysis and subsequent loss of material or exposure of the reactants to atmospheric moisture, chlorosilanes are handled in closed pipes or containers. The following summarizes the 2005 production volumes in tonnes of these chlorosilanes for the North America, Europe and Japan; ranges are provided in order to protect confidential business information. Sponsored substance; North America; Europe; Japan. CTMS (CAS No. 75-77-4); 13608 - 27216; 22680 - 34019; 4536-13608. C2DMS (CAS No. 75-78-5); 272155 - 680389; 226796- 340194; 111130-290299. C3MS (CAS No 75-79-6); 22680 - 45359; 11398- 294835; 9072 -20412. C3ES (CAS No. 115-21-9); 907-2041; 227-680; 0. These chlorosilanes are produced and processed in closed systems. Due to the dynamic and exothermic nature of the hydrolysis processes for producing siloxane oligomers and polymers from these chlorosilanes, many engineering controls are in place to prevent occupational exposure such as local ventilation; water scrubber devices and related equipment; and closed sampling systems. Employees involved in chlorosilane production and application are required to use personal protective equipment (PPE) such as safety glasses or goggles, steel-tipped shoes, flame-resistant clothing, hard hat, chemical resistant gloves, and respirator mask. For any situation (e.g. equipment maintenance and repair) where potential exposure to chlorosilanes is expected, the use of acid resistant protective equipment, respiratory equipment and face shield is recommended because of their irritating or corrosive properties. Drivers who transport these chemicals must always have this equipment ready for immediate use. Potential routes of occupational exposure include inhalation and dermal exposure. All of these chlorosilanes are used as chemical intermediates in closed system and worker, and general public exposure is expected to be low. There are no consumer uses of these chlorosilanes. |

SIDS Initial Assessment Report

# identity

The Alkyl Chlorosilanes Category consists of 4 sponsored chlorosilanes: chlorotrimethylsilane (CAS No. 75-77-4), dichloro(dimethyl)silane (CAS No. 75-78-5), trichloromethylsilane) (CAS No. 75-79-6), and trichloroethylsilane (CAS No. 115-21-9).  These chemicals can be represented by a similar general molecular formula:

(Cl)xSi (Alkyl)y

Where Cl = chlorine, x = 1, 2, or 3

Si = silicon,

Alkyl = alkyl group (ethyl or methyl), y = 4-x

The chlorine group is the most active functional group on these molecules and determines many aspects of the behaviour of the category members.  The alkyl chlorosilanes undergo rapid hydrolysis in the presence of water to form one to three moles of hydrochloric acid and one mole of silanol, depending on the parent substance.  Silanols can react to form oligomers and polymer but the rate is dependent on concentration and pH.  There are no studies currently available to define these processes under environmentally relevant conditions.

## Identification of the Category

Tables 1, 2 and 3 provide the chemical identifiers, structural formulae and synonyms of the category members.

Table 1     Chemical Identifiers

| CAS No. | Chemical Name (abbreviation) | IUPAC Name | MW | Molecular Formula |
| --- | --- | --- | --- | --- |
| Sponsored Substances |  |  |  |  |
| 75-77-4 | Chlorotrimethylsilane (CTMS) | Chloro (trimethyl) silane | 108.64 | (CH3)3SiCl |
| 75-78-5 | Dichloro(dimethyl)silane (C2DMS) | Dichloro (dimethyl) silane | 129.06 | (CH3)2Si(Cl)2 |
| 75-79-6 | Trichloro(methyl)silane (C3MS) | Trichloro (methyl) silane | 149.48 | CH3Si(Cl)3 |
| 115-21-9 | Trichloro(ethyl)silane (C3ES) | Trichloro (ethyl) silane | 163.51 | CH3CH2Si(Cl)3 |

Table 2     Structural Formula

| CAS No. | Chemical Name | Structure |
| --- | --- | --- |
| Sponsored Substances |  |  |
| 75-77-4 | CTMS |  |
| 75-78-5 | C2DMS |  |
| 75-79-6 | C3MS |  |
| 115-21-9 | C3ES |  |

Table 3     Synonyms of Category Members

| CAS No. | Chemical Name | Synonyms |
| --- | --- | --- |
| 75-77-4 | CTMS | Chlorotrimethylsilicon Monochlorotrimethylsilicon NSC 15750 Silane, chlorotrimethyl- Silane, trimethylchloro- Silicane, chlorotrimethyl- Silylium, trimethyl-, chloride TL 1163 Trimethyl chlorosilanes Trimethylchlorosilane Trimethylsilylchloride |
| 75-78-5 | C2DMS | Dichlorodimethylsilane Dichlorodimethylsilicon Dimethyl-dichlorsilan Dimethyl-dichlorsilan [Czech] Dimethyldichlorosilane Dimethylsilane dichloride EINECS 200-901-0 HSDB 361 Inerton AW-DMCS Inerton dw-dmc LS 130 NSC 77070 Repel-Silan Silane, dichlorodimethyl- |
| 75-79-6 | C3MS | Methyl-trichlorsilan Methyl-trichlorsilan [Czech] Methylsilicochloroform Methylsilyl trichloride Methyltrichlorosilane Monomethyltrichlorosilane NSC 77069 Silane, methyltrichloro- Silane, trichloromethyl- Trichlor-methylsilan Trichlor-methylsilan [Czech] Trichloromethylsilane Trichloromethylsilicon |
| 115-21-9 | C3ES | Ethyl silicon trichloride Ethyl Trichlorosilane Ethyltrichlorosilane HSDB 884 Silane, ethyl(trichloro)- Silane, trichloroethyl- Silicane, trichloroethyl- Trichloroethylsilane Trichloroethylsilicane Trichloroethylsilicon |

## Purity/Impurities/Additives

Data on individual chlorosilanes are considered confidential business information by manufacturers, and are unavailable.  However, based on a survey conducted by the SEHSC, the purity for all four chlorosilanes is 99%-100% (Guerrero, 2010).

Table 4     Summary of the purity of the Alkyl Chlorosilanes

| Name | Purity |
| --- | --- |
| CTMS(CAS No 75-77-4) | 99-100% |
| C2DMS (CAS No 75-78-5) | 99.3-100% |
| C3MS (CAS No 75-79-6) | 99-99.3% |
| C3ES (CAS No 115-21-9) | 100% |

Table 5     Summary of the impurities of the alkyl chlorosilanes

|  | ClTMS(CAS No 75-77-4) | Cl2DMS (CAS No 75-78-5) | Cl3MS (CAS No 75-79-6) | Cl3ES (CAS No 115-21-9) | CL3OS (CAS No 5283-66-9) |
| --- | --- | --- | --- | --- | --- |
| Silicon Tetrachloride (CAS No. 10026-04-7) | <.03 % | -- | -- | <.0.01 % | -- |
| 3-Methylpentane (CAS No. 96-14-0) | <0.2 % | -- | -- | -- | -- |
| Methyltrichlorosilane (CAS No. 75-79-6) | <0.2% | -- | -- | -- | -- |
|  | ClTMS(CAS No 75-77-4) | Cl2DMS (CAS No 75-78-5) | Cl3MS (CAS No 75-79-6) | Cl3ES (CAS No 115-21-9) |
| Silicon Tetrachloride (CAS No. 10026-04-7) | <.03 % | -- | -- | <.0.01 % |
| 3-Methylpentane (CAS No. 96-14-0) | <0.2 % | -- | -- | -- |
| Methyltrichlorosilane (CAS No. 75-79-6) | <0.2% | -- | -- | -- |
| Methyldichlorosilane (CAS No. 75-54-7) | <0.1% | -- | -- | -- |
| Hydrocarbons (C6) | <0.5% | -- | -- | -- |
| Hexamethyldisiloxane (CAS No. 107-46-0) | <0.1% | -- | -- | -- |
| Me3SiCl/Me2SiCl2/EtMe2SiCl/EtHSiCl2/MeSiCl3 | -- | <.0.05 % | -- | -- |
| Trichloromethylsilane (CAS No. 75-79-6) | -- | <0.4% | -- | -- |
| Me2SiCl2 | -- | -- | <.0.6% | -- |
| Hydrocarbons (C6-C9) (CAS No. 68920-06-9) | -- | -- | <1% | -- |
| Dimethyldichlorosilane (cAS No. 75-78-5) | -- | -- | <7 – 0.2% | -- |

## Physico-Chemical properties

Table 6     Summary of physico-chemical properties

| Property | CTMS (CAS No.75-77-4) | C2DMS (CAS No.75-78-5) | C3MS (CAS No.75-79-6) | C3ES (CAS No.115-21-9) |
| --- | --- | --- | --- | --- |
| Physical state | liquid | liquid | liquid | liquid |
| Melting point | Value:-57.7oC (measured) Ref: DIPPR, 1988 | Value: -76.1°C (measured) Ref: DIPPR, 1990 | Value: -77.8 °C (measured) Ref: DIPPR, 1946 | Value: -105.6 °C (measured) Ref: DIPPR, 1946 |
| Boiling point | Value: 57.6 °C at 1013 hPa (measured) Ref: DIPPR, 1954 | Value: 70.2 °C at 1013 hPa (measured) Ref: DIPPR, 1990 | Value: 66.4 °C at 1013 hPa (measured) Ref: DIPPR, 1947 | Value: 97.9 oC at 1013 hPa (measured) Ref: DIPPR, 1950 |
| Liquid density | Value: 0.85 g/cm³ at 25 °C (measured) Ref: DIPPR, 1987 | Value: 1.06 g/cm³ at 25 °C (measured) Ref: DIPPR, 1948 | Value: 1.27 g/cm³ at 25 °C (measured) Ref: DIPPR, 1986 | Value: 1.24 g/cm³ at 20 °C (measured) Ref: DIPPR, 1988 |
| Vapour pressure | Value: 250.3 hPa at 20 °C (measured) Ref: DIPPR, 2001 | Value: 191.18 hPa at 25 °C (measured) Ref: DIPPR, 1990 | Value: 185.62 hPa at 20 °C (measured) Ref: DIPPR, 2001 | Value: 47.83 hPa at 20 °C (measured) Ref: DIPPR, 1954 |
| Water solubility (b, c) | 1013 mg/L at 25 °C: (estimated) Ref: USEPA, 2010 | 1754 mg/L at 25 °C: (estimated) Ref: USEPA, 2010 | 2959 mg/L at 25 °C: (estimated) Ref: USEPA, 2010 | 987.8 mg/L at 25 °C: (estimated) Ref: USEPA, 2010 |
| Partition coefficient n-octanol/water (log value) (b, c) | 2.48: (estimated) Ref: USEPA, 2010 | 2.24: (estimated) Ref: USEPA, 2010 | 2.01: (estimated) Ref: USEPA, 2010 | 2.50: (estimated) Ref: USEPA, 2010 |
| Henry’s law constant (c) | 6.48E+003 Pa-m3/mole: (estimated) Bond method Ref: USEPA, 2010 | 1.87E+003 Pa-m3/mole: (estimated) Bond method Ref: USEPA, 2010 | 5.41E+002 Pa-m3/mole (estimated) Bond method Ref: USEPA, 2010 | 7.18E+002 Pa-m3/mole: (estimated) Bond method Ref: USEPA, 2010 |

- Tg = glass transition temperature

- The water solubility and log Kow values may not be accurate because the chemicals are hydrolytically unstable.

- Reliability =4; The EPI Suite program (v 4.0) developed by the U.S. Environmental Protection Agency and Syracuse Research Corporation has not been validated for chemicals that contain silanes in their molecular structure (although some measured data are included in the training data set); therefore, there is uncertainty associated with the calculated values and they should be used with caution whenever they are reported.

## Analogue/Category Justification

The category consists of four sponsored alkyl chlorosilanes, CTMS (CAS No. 75-77-4), C2DMS (CAS No. 75-78-5),	C3MS (CAS No. 75-79-6), and C3ES (CAS No. 115-21-9). These chemicals are grouped into a category based on similar molecular structure and functionality, high reactivity, physicochemical and toxicological properties.

Similar Molecular Structure and Functionality

The alkyl chlorosilanes can be represented by a similar general molecular formula:

(Cl)xSi (Alkyl)y

Where Cl = chlorine, x = 1, 2, or 3

Si = silicon,

Alkyl = alkyl group (ethyl or methyl, etc.), y = 4-x

Similar High Reactivity

The chlorine group is the most active functional group on these molecules and determines many aspects of the behaviour of the category members.  The alkyl chlorosilanes undergo rapid hydrolysis in the presence of water to form one to three moles of hydrochloric acid and one to three moles of silanol, depending on the parent substance.  Hydrolysis is the primary reaction in aqueous systems and has been shown to occur very quickly (half-life <17 seconds) for CTMS (CAS No. 75-77-4), C2DMS (CAS No. 75-78-5), and C3MS (CAS No. 75-79-6).  No data are available for C3ES (CAS No. 115-21-9). but hydrolysis is expected to occur just as quickly (Baker and Watling, 2004; USEPA, 2009).

Similar Chemical/Physical Properties

All category members are liquids with melting points, boiling points and vapor pressures that are largely dependent on molecular weight.  Due to their high reactivity with moisture, water solubility and partition coefficient are not applicable endpoints.

Similar Toxicological Properties

These category members are severely irritating and corrosive at the point of contact (i.e., respiratory tract, skin, and eyes).

Alkyl chlorosilanes react rapidly when exposed to moisture or polar reagents, producing hydrogen chloride (HCl; CAS No. 7647-01-0) and the corresponding silanols (siloxane oligomers and polymers; Bayer et al. 1991, Bazant et al. 1965, Noll 1968, and Tomanek 1990).  The silanols can condense spontaneously to form highly cross-linked polymeric gels in uncontrolled environments.  Hydrogen chloride is the only hydrolysis product released into the air (USEPA, 2009; Baker and Watling, 2004).  Data are available for hydrogen chloride (previously assessed in the OECD HPV programme:  http://www.chem.unep.ch/irptc/sids/oecdsids/sidspub.html) and two hydrolysis products; hydroxytrimethylsilane (TMS; CAS No. 1066-40-6) and dimethylsilanediol (DMSD; CAS No. 1066-42-8).  The primary hydrolysis products of the sponsored alkyl chlorosilanes are listed in Table 7.

Repeated-dose and reproductive toxicity endpoints for the category are fulfilled through the use of data from HCl, and two silanol hydrolysis products, TMS and DMSD, respectively.  Aquatic toxicity endpoints are fulfilled with data for the hydrolysis products (HCl, TMS and DMSD) and the structural analogue methyltrimethoxysilane (MTMS: CAS No. 1185-55-3).  MTMS has previously been assessed in the OECD HPV programme:  http://webnet.oecd.org/hpv/ui/Search.aspx.

Table 7     Summary of the primary silanol hydrolysis products

| CAS No. |  | Hydrolysis Product Name (acronym) | Structure | CAS No. of parent |
| --- | --- | --- | --- | --- |
| # 1066-40-6 |  | # Trimethylsilanol (TMS) |  | 75-77-4 |
| 1066-42-8 |  | Dimethylsilandiol (DMSD) |  | # 75-78-5 |
| # 2445-53-6 |  | # Methylsilanetriol (MST) |  | # 75-79-6 |
| # 5651-16-1 |  | # Ethylsilantriol (EST) |  | # 115-21-9 |

# general information on exposure

Alkyl chlorosilanes react rapidly when exposed to moisture or polar reagents, producing hydrogen chloride (HCl; CAS No. 7647-01-0) and silanols (siloxane oligomers and polymers) (Bayer et al. 1991, Bazant et al. 1965, Noll 1968, and Tomanek 1990).  This reaction is normally heterogeneous, highly exothermic, and potentially difficult to control, requiring close control of the reaction.  To prevent the rapid hydrolysis and subsequent loss of material or exposure of the reactants to atmospheric moisture, alkyl chlorosilanes are handled in closed pipes or containers.  An essential component is the reactor jacket and/or internal coils containing a heat-transfer fluid, which is also used for initial heating of the bed.  This system allows recovery of the reaction heat via steam, and greatly limits environmental exposure. Alkyl chlorosilanes are handled in closed systems as once they are exposed to moisture through accidental release they no longer have commercial value.  Exposure to alkyl chlorosilanes is not expected under normal use conditions for this reason.  If there is an accidental release the relevant routes of exposure are inhalation and dermal.

The silicone industry has a long history of safe transportation of these alkyl chlorosilanes.  They are transported in rail cars or cargo tanks in accordance with U.S. Department of Transportation (U.S. DOT) Hazardous Materials Transportation Regulations  (49 CFR 171-180).  These regulations specify acceptable modes of transport, hazard placarding, container types and construction and handling practices.  U.S. DOT classifies chlorosilanes as Hazard Class 8, UN1760.  This classification requires “corrosive” labeling and packaging in United Nations (U.N.) certified containers.  Such containers meet rigorous performance standards as agreed to by the International Transportation Community.  The SEHSC prepared an ASTM manual (ASTM Stock #: MN L33), “Chlorosilane Emergency Response Guidelines,” and continues working with U.S. DOT, international transportation bodies, and chlorosilane producers to ensure the safe transportation of chlorosilanes.

## Production Volumes and Use Pattern

100% of alkyl chlorosilanes, by volume, are used as intermediates in the manufacture of commercial silicones (polymers and silanes).  These commercial silicones made from alkyl chlorosilanes are used as ingredients in a wide variety of industrial and consumer products, or sold as fluids, resins, or specialty products from the silicone industry.  Alkyl chlorosilanes are not expected to be found in the resultant commercial silicone products, as they are hydrolyzed in this process (Guerrero, 2010).  Production volumes are summarized in Table 8 (Guerrero, 2010).  Ranges are provided in order to protect confidential business information.

Table 8     Production volumes (tonnes) by region (2005)

| Sponsored substance | North America | Europe | Japan |
| --- | --- | --- | --- |
| CTMS (CAS No. 75-77-4) | 13608 - 27216 | 22680 - 34019 | 4536-13608 |
| C2DMS (CAS No. 75-78-5) | 272155 - 680389 | 226796- 340194 | 111130-290299 |
| C3MS (CAS No 75-79-6) | 22680 - 45359 | 11398- 294835 | 9072 -20412 |
| C3ES (CAS No. 115-21-9) | 907-2041 | 227-680 | 0 |

## Environmental Release and Fate

The EPISuite program (v 4.0) developed by the U.S. Environmental Protection Agency and Syracuse Research Corporation has not been validated for chemicals that contain silanes in their molecular structure (although some measured data are included in the training set); therefore, there is greater uncertainty associated with the calculated values and they should be used with caution whenever they are reported below (USEPA, 2010).

### Sources of Environmental Release

The alkyl chlorosilanes are handled in closed systems, and are transported from the production site as the parent chemical, and then consumed during the generation of silicones.  Transfer of this material is in closed pipe, drums, or tanks rather than in open systems to minimize loss of this material (through hydrolysis).  There are no intentional releases to the environment from manufacturing processes.

The alkyl chlorosilanes hydrolyse rapidly; the half-lives for CTMS, C2DMS and C3MS are <17 seconds.  This limits exposure to the parent materials but results in greater exposure to hydrolysis products (HCl and silanols).  Hydrolysis of ClTMS results in the expected production of 1 mole of HCl and one mole of TMS.  TMS has been shown to be stable under experimental conditions and does not readily polymerize or condense.  Hydrolysis of C2DMS results in the expected production of 2 moles of HCl and one mole of DMSD.  DMSD has been shown to be stable under experimental conditions and does not readily polymerize or condense.  Hydrolysis of C3MS and C3ES results in the expected production of 3 moles of HCl and 1 mole of corresponding silanetriol.  Depending on pH and concentration in water, silanetriols can condense to form highly cross-linked, high molecular weight polymers, further reducing the potential for exposure.  If the substance is slowly released such that the concentration of the resulting silanol is not high enough to result in polymerization, the silanol will exist largely as a monomer.  The monomer is known to be water soluble due to the hydroxy groups on the silicon.  Thus, the silanols will have low log Kow values and are not expected to bioaccumulate.  The water solubility of the silanetriol cannot be measured because of the tendency to condense at concentrations greater than approximately 500 mg/L (Chandra, 1977).  It is known, however, that the silanetriol and small condensation products will only precipitate out of water due to formation of larger, water insoluble polymeric resins (Chandra, 1977).

### Photodegradation

The alkyl chlorosilanes in air are not expected to undergo direct photolysis, but may undergo indirect photolysis through hydroxyl radical oxidation.  The hydroxyl radical reaction was calculated using a 12-hr day in AOPWIN® ver. 1.92 (EPI Suite (v4.0).  The overall OH rate constants and estimated half-lives are provided in Table 9.  Photodegradation as a removal mechanism from air is unlikely as the alkyl chlorosilanes are reactive and hydrolytically unstable, such that HCl and the likely hydrolysis products are assumed to be rapidly generated upon contact with atmospheric water vapour.

Table 9     Summary of Photodegradation Modeling

|  | Overall OH rate constant (E-12 cm3/molecule-sec) | Estimated half-life | Reference |
| --- | --- | --- | --- |
| CTMS (CAS No. 75-77-4) | 0.44 x 10-12 | 23.8 days | USEPA, 2010 |
| C2DMS (CAS No. 75-78-5) | 0.29 x 10-12 | 35.7 days | USEPA, 2010 |
| C3MS (CAS No. 75-79-6) | 0.15 x 10-12 | 71.4 days | USEPA, 2010 |
| C3ES (CAS No. 115-21-9) | 1.19 x 10-12 | 8.9 days | USEPA, 2010 |

### Stability in Water

Hydrolysis studies were conducted on three category members using OECD TG 111. CTMS, C2DMS and C3MS hydrolyzed to HCl and the corresponding silanol in less than 17 seconds at pH 4, 7, and 9 and 1.5ºC (Miller and Kozerski, 2001; Table 10).  No additional silanol testing was conducted as you cannot isolate the other silanols formed.   No data are available for C3ES but hydrolysis is expected to occur just as quickly (Baker and Watling, 2004; USEPA, 2009).  The hydrolysis mechanism is the same for all chlorosiloanes.  The hydrolysis reactions of chlorosilanes proceed by nucleophilic attack of water at the Si atom, resulting in substitution of an –OH group for each –Cl atom attached to Si in the parent silane.  Due to the strong silicon-oxygen bonds formed during hydrolysis, this is a highly exothermic process.  The –Cl atoms are eliminated as HCl, which will ionize in water to form H+ and Cl- ions, leading to a pH reduction in unbuffered systems.

Table 10     Summary of Hydrolysis

| Substance | Half-life in seconds, at 1.5 +/- 0.5 oC, pH 4.0, 7.0, 9.0 | Expected hydrolysis products | Reference |
| --- | --- | --- | --- |
| Sponsored substances |  |  |  |
| CTMS (CAS No. 75-77-4) | 7, 11, 8 | HCl, TMS | Miller and Kozerski , 2001 |
| C2DMS (CAS No. 75-78-5) | 10, 17, 7 | HCl, DMSD | Miller and Kozerski , 2001 |
| C3MS (CAS No. 75-79-6) | 7, 9, 6 | HCl, Methylsilanetriol | Miller and Kozerski , 2001 |
| C3ES (CAS No. 115-21-9) | No data – expected to be similar to other category members: < 30 s | HCl, Ethylsilanetriol | -- |

- The chromatograms at pH 9.0 showed no peak at relevant retention even directly after preparation of the samples.

### 2.2.4	Transport between Environmental Compartments

The EQC Level III Fugacity model was used to evaluate the fate, transport and distribution of the alkyl chlorosilanes and the hydrolysis products between environmental matrices (USEPA, 2008). Alkyl chlorosilanes are expected to distribute primarily to air.  Table 11 provides the results of Level III fugacity modelling, using equal loading rates of 1000 kg/h each for air, soil and water and continuous distributions.

Table 11     Summary of Fugacity Modeling

| Substance | % Distribution | Reference |
| --- | --- | --- |
| Sponsored Substances |  |  |
| CTMS (CAS No 75-77-4) | Air = 53.2 Water = 44.9 Soil = 1.74 Sediment = 0.164 | USEPA, 2008 |
| C2DMS (CAS No 75-78-5) | Air = 52.2 Water = 43.1 Soil = 4.53 Sediment = 0.157 | USEPA, 2008 |
| C3MS (CAS No 75-79-6) | Air = 47.6 Water = 39.7 Soil = 12.5 Sediment = 0.145 | USEPA, 2008 |
| C3ES (CAS No 115-21-9) | Air = 40.2 Water = 42.4 Soil = 17.3 Sediment = 0.225 | USEPA, 2008 |
| Hydrolysis products |  |  |
| TMS (CAS No 1066-40-6) | Air = 5.26 Water = 29.9 Soil = 64.7 Sediment <1 | USEPA, 2008 |
| DMSD (CAS No 1066-42-8) Methylsilanetriol | Air <1 Water = 23.1 Soil = 76.6 Sediment <1 Air <1 Water = 22.5 Soil = 77.4 | USEPA, 2008 USEPA, 2008 |
| (CAS No 2445-53-6) Ethylsilanetriol | Sediment <1 Air <1 Water = 19.8 Soil = 80.1 Sediment <1 | USEPA, 2008 USEPA, 2008 |
| HCl (CAS No 7647-01-0) | Not Applicable | http://www.chem.unep.ch/irptc/sids/oecdsids/sidspub.html |

### 2.2.5	Biodegradation

Sponsored substances

The biodegradation of the alkyl chlorosilanes was not determined due to their rapid hydrolysis; any potential for biodegradation is likely to be of the hydrolysis products.  Consequently, the only biodegradable materials in the test system will be silanols, and condensed silanol materials (high molecular weight polymers) (Chandra, 1977).

Hydrolysis products

In an OECD TG 310 (Ready Biodegradability - CO2 in Sealed Vessels (Headspace Test), TMS was not readily biodegradable (0% degradation after 29 days; Clarke, 2008).  Based on studies of DMSD (14C-dimethylsilanediol) in four soils at 25 ºC, the substance is not readily biodegradable (Sabourin, 1996). HCl is an inorganic compound and biodegradation tests are not applicable.  Based on available data for hydrolysis products, the alkyl chlorosilanes are not readily biodegradable.

### 2.2.6	Bioaccumulation

Sponsored substances

The bioaccumulation potential of the alkyl chlorosilanes was not determined due to their rapid hydrolysis.  Estimated BCFs for the category members are listed in Table 12.

Table 12     Summary of Bioaccumulation of Sponsored Substances

| Substance | BCF (L/kg wet-wt) | Expected hydrolysis products | Reference |
| --- | --- | --- | --- |
| Sponsored substances |  |  |  |
| CTMS (CAS No 75-77-4) | 20.1 | HCl, TMS | USEPA, 2010 |
| C2DMS (CAS No 75-78-5) | 14.0 | HCl, DMSD | USEPA, 2010 |
| C3MS (CAS No 75-79-6) | 9.8 | HCl, methylsilanetriol | USEPA, 2010 |
| C3ES (CAS No 115-21-9) | 20.6 | HCl, ethylsilanetriol | USEPA, 2010 |

## 2.3	Human Exposure

### 2.3.1	Occupational Exposure

Alkyl chlorosilanes are produced and processed in closed systems.  Due to the dynamic and exothermic nature of the processes incorporating alkyl chlorosilanes, many engineering controls are in place to prevent occupational exposure such as local ventilation; water scrubber devices and related equipment; and closed sampling systems.  Employees involved in chlorosilane production and application are required to use personal protective equipment (PPE) such as safety glasses or goggles, steel-tipped shoes, flame-resistant clothing, hard hat, chemical resistant gloves, and respirator mask.  For any situation (e.g. equipment maintenance and repair) where potential exposure to alkyl chlorosilanes is expected, the use of acid resistant protective equipment, respiratory equipment and face shield is recommended because of their irritating or corrosive properties.  Drivers who transport these chemicals must always have this equipment ready for immediate use (Guerrero, 2010).  Potential routes of exposure include inhalation and dermal exposure.

### 2.3.2	Consumer Exposure

There are no consumer uses of the alkyl chlorosilanes (Guerrero, 2010).

# human health hazards

## 3.1	Effects on Human Health

Due to their reactivity, the sponsored alkyl chlorosilanes CTMS, C2DMS, C3MS and C3ES are expected to hydrolyse prior to exposure, or locally at the port of entry, to form HCl and corresponding silanol hydrolysis products.  The levels of alkyl chlorosilane required to generate concentrations near those tested for the silanols would result in severely corrosive HCl concentrations.  Two potential silanol hydrolysis products for which toxicity testing has been conducted include TMS and DMSD.  Although these silanols may not represent hydrolysis products of each sponsored alkyl chlorosilane, they have been included to supplement characterization of the human health hazard.  The primary human health hazard for each of the sponsored alkyl chlorosilanes is considered to be exposure to the hydrogen chloride hydrolysis product.  Given the corrosivity of hydrogen chloride, no additional testing of the alkyl chlorosilanes is being considered.

### 3.1.1	Toxicokinetics, Metabolism and Distribution

Sponsored substances

No data are available on the toxicokinetics, metabolism and distribution of the alkyl chlorosilanes.  However, alkyl chlorosilanes rapidly hydrolyze on contact with moisture to form HCl and silanols.  Also, the systemic effects found in an acute dermal toxicity study in rabbits with CTMS (discoloration of liver and lungs) provide evidence that chlorosilanes or their hydrolysis products can penetrate through the skin.  These chlorosilanes or the associated hydrolysis products may be absorbed via inhalation at high doses based on discoloration in the liver (and spleen for C2DMS) observed in acute inhalation studies.

Hydrolysis products

Studies in Animals

No data available.

Studies in Humans

In vitro Studies

In vitro percutaneous penetration of 14C-labeled trimethylsilanol (14C-TMS) was evaluated when applied neat to human skin, under in vitro semi-occluded conditions for 24 hours (Dow Corning Corporation (DCC), 2008a).  The majority of the absorbed dose penetrated through the skin into receptor fluid with the estimated mean cumulative penetration of 9.1 +/- 0.9 g equivalents of 14C-TMS per cm2 of exposed skin over the 24 hr diffusion period (0.08% of applied dose).  In vitro percutaneous penetration of 14C-labeled dimethylsilanediol (14C-DMSD) was evaluated when applied in aqueous solution to human skin for 24 hours (DCC, 2008b).  At the end of the assay, a considerable portion (59.5%) of 14C-DMSD volatilized from the skin surface, 18.3% was found on the skin surface, 2.5% remained in the skin after washing and tape stripping, and the dose absorbed was 13.9%. The majority of this absorbed dose penetrated through the skin (82.1%).

In vivo Studies

Hydrogen chloride will rapidly dissociate and its effects are thought to be a result of pH change (local deposition of H+) (http://www.chem.unep.ch/irptc/sids/oecdsids/sidspub.html).

Conclusion

No data are available on the toxicokinetics, metabolism and distribution of the alkyl chlorosilanes.  However, alkyl chlorosilanes rapidly hydrolyze on contact with moisture.  Data from the hydrolysis products, TMS and DMSD, show that the hydrolysis products penetrated human skin in vitro. Hydrogen chloride dissociates; its effects are thought to be a result of pH change.  Based on observed systemic effects after dermal exposure to high doses of CTMS and inhalation of high concentrations of C2DMS, absorption of the parent or hydrolysis products can occur via these exposure routes.

### 3.1.2	Acute Toxicity

Acute inhalation toxicity data are available for all the sponsored alkyl chlorosilanes.  Acute dermal and oral toxicity data are available for CTMS and C3MS. Acute inhalation, dermal and/or oral toxicity data are presented for the hydrolysis products, TMS and DMSD.  Table 13, 14, and 15 present details from inhalation, dermal, and oral studies, respectively.

Studies in Animals

Inhalation

Sponsored substances

The acute inhalation hazard posed by an individual alkyl chlorosilane, as defined by an LC50 value, is directly proportional to its chlorine content and subsequently to the HCl that is liberated during hydrolysis (Jean et al., 2006).  The acute inhalation toxicity of 10 chlorosilanes was investigated in Fischer 344 rats using a 1-h whole-body vapor inhalation exposure and a 14-day recovery period. The median lethal concentration (LC50) for each material was calculated from the nominal exposure concentrations and mortality.  Experimentally derived LC50 values for monochlorosilanes (17.32-18.92 mg/L) were greater than those for dichlorosilanes (8.39-11.04 mg/L), which were greater than those for trichlorosilanes (8.41-10.64 mg/L).  Apparent was a strong structure-activity relationship between chlorine content and LC50 value. Estimated LC50 values for mono-, di-, and trichlorosilanes were determined utilizing this relationship and the lower limit of the 95% prediction interval.  The LC50 values determined in this series of studies were greater than that reported for HCl (4.59 mg/L) when expressed on a chlorine equivalence basis, demonstrating that the acute toxicity of these chlorosilanes is similar to or less than that for HCl.  The good correlation between chlorine content and LC50 provides a sound basis for estimation of LC50 for chlorosilanes.

CTMS.  In a guideline study (OECD TG 403), Fisher 344 rats (5/sex/concentration) were exposed by whole body vapour inhalation for one hour to concentrations of 14.09, 18.39, 18.96, and 22.75 mg/L (nominal) (DCC, 1999; Jean et al., 2006).  All animals died at 22.75 mg/L, 6 died at 18.96 mg/L, 2 died at 18.39 mg/L and there were no deaths at 14.09 mg/L.  The principal clinical signs were indicative of respiratory and ocular effects.  Laboured breathing, rales and gasping were seen in essentially all rats, most frequently early in the study period; some signs of respiratory distress persisted well into the post-exposure observation period.  Hypoactivity, salivation and lacrimation were seen early in the observation period.  Nearly all rats in the two higher exposure groups had corneal opacity.  Many signs indicative of stress and/or general unthriftiness (staining, soiling and encrustation) were observed in all groups. Lung and upper respiratory tract injury were observed at necropsy.  In general, this consisted of pulmonary consolidation, congestion and/or hemorrhage in rats that died and ectasia and atelectasis in many of the rats that survived to the scheduled necropsy.  The acute one-hour combined male/female LC50, based on nominal concentrations, was determined to be 18.92 mg/L.  In a study conducted following OECD Short-term and Long-term Toxicology Groups Final Report adopted May 1961, groups of 5 Sprague-Dawley rats/sex were exposed via nose-only for a 1 hour to CTMS at 10.28, 12.69, 14.61, and 16.63 mg/L (measured) (DCC, 1987).   A separate group of animals was exposed to HCL gas (8.30, 8.95, 12.76 and 14.69 mg/L).   Mortalities by group were 1/10, 3/10, 10/10 and 10/10 for 10.28, 12.69, 14.61, and 16.63 mg/L, respectively. No deaths were observed in the HCL exposed animals.  Nasal crust and sores on the nose and face were the main clinical signs for HCL exposed animals with corneal opacity and respiratory difficulty the main clinical signs observed following CTMS exposure.  Necropsy of the animals that died as a result of exposure to the CTMS showed gas extended G.I. tracts and apparent hemorrhaging in the lungs.  HCl appeared to be reacting in the upper respiratory tract while the CTMS was most likely reacting with cells of the deeper respiratory tissues.  Reduction in body weights was observed in all exposed animals.  Gross necropsy findings included changes in external nares, pulmonary edema, fluid in the thoracic cavity and GI tract extended with gas in animals exposed to HCL.  Following CTMS exposure, gross necropsy findings included corneal opacity, diffuse or focal dark colored patched on lungs, exudates in the nasal turbinate region and in some cases gas in the GI tract. The acute one-hour combined male/female LC50, based on measured concentrations, was determined to be 13.01 mg/L.

C2DMS.  In a guideline study (OECD TG 403), Fisher 344 rats (5/sex/concentration) were exposed by whole body vapour inhalation for one hour to concentrations of 6.91, 10.96, 12.42 and 14.39 mg/L (nominal) (DCC, 1997a; Jean et al., 2006).  All animals died at 14.39 mg/L, 8 died at 12.42 mg/L, 5 died at 10.96 mg/L and there were no deaths at 6.91 mg/L.  All rats on the study exhibited various staining/soiling around the nose, mouth and/or eyes and other body surfaces, lacrimation and salivation.  Respiratory effects (labored breathing, rales) were not seen at 6.91 mg/L, but did occur in all other groups on the day of exposure; these effects persisted for increasing periods of time as exposure concentration increased. Partial or complete eye closure was noted for essentially all rats in the 10.96, 12.42, and 14.39 mg/L groups, and for one 6.91 mg/L rat, on the day of exposure.  At least six rats/group in the 6.91, 10.96, and 12.42 mg/L groups exhibited ocular/corneal opacities; this effect was not observed in the 14.39 mg/L group.  Prostration, hypothermia and abdominal distension were observed in all groups except the 6.91 mg/L group; occurrence generally preceded death. Various ocular abnormalities (opacity, hemorrhage, dark red areas, rupture, etc.), pulmonary abnormalities, primarily consisting of areas of gray discoloration and red foci, and various external stains, hair losses and misshapen, shrunken or missing extremities was seen in surviving rats.  There were no deaths in the 6.91 mg/L group.  Necropsy findings observed for rats in all groups except 6.91 mg/L included  various external stains, mattings and hair losses, discolored, encrusted, firm and/or obstructed nares/nostrils, pulmonary abnormalities  consisting of ectasia, congestion, hemorrhage and discoloration, gastrointestinal findings consisting of abnormal contents, gaseous distension and empty, decreased or absent body fat, various ocular abnormalities including corneal opacities, pannus and abnormal anterior chamber content, liver findings of dark red/congested, small and dark or pale and soft and small and/or dark spleen.  In addition, encrustation on the forefeet was observed in all groups except the 12.42 mg/L group. The one-hour combined male/female LC50 based on nominal concentrations, was determined to be 11.04 mg/L.  In a study conducted following OECD Short-term and Long-term Toxicology Groups Final Report adopted May 1961, groups of 5 Sprague-Dawley rats/sex were exposed via nose-only for a 1 hour to C2TMS at 9.90, 13.32 and 15.34 mg/L (measured) (DCC, 1987).  Seven animals died at 15.34 mg/L, 7 died at 13.32 mg/L, and 2 died at 9.90 mg/L.  Nasal crust, rough coat, sores on the face and corneal opacity were observed in all groups. Mouth breathing was seen in males of the higher exposure group. Gross necropsy findings included corneal opacity and dark areas on the lungs.  There were no other necropsy findings.  The acute one-hour combined male/female LC50, based on measured concentrations, was determined to be 12.36 mg/L.

C3MS. In a guideline study (OECD TG 403), Sprague-Dawley rats (5/sex/concentration) were exposed by nose only vapour inhalation for four hours to concentrations of 3.50, 5.34, 8.30 and 14.22 mg/L (measured) (DCC, 1989).  All animals died at 14.22 mg/L, 9 died at 8.30 mg/L, 6 died at 5.34 mg/L and there were no deaths at 3.50 mg/L.  Clinical signs included laboured breathing, nasal discharge, mouth breathing, and blackened tip of nose.  Rats exposed to 5.34, 8.30 and 14.22 mg/L had gross lung changes and lesions as a result of treatment.  Pulmonary lesions included mottling, atelectasis and congestion.  Nasal passages were obstructed in many animals and facial soiling was common.  The lesions were of a nature and severity to account for the death of many of these rats, probably by asphyxiation.  No test article-related effects were seen in rats exposed to 3.50 mg/L.  The 4-hour combined male/female nose-only LC50 based on measured concentrations, was determined to be 5.43 mg/L.

In another guideline study (OECD TG 403) Fisher 344 rats (5/sex/concentration) were exposed by whole body vapour inhalation for one hour to concentrations of 3.80, 6.40, 8.80 and 18.80 mg/L (nominal) (DCC, 1997b; Jean et al., 2006).  All animals died at 18.80 mg/L, 6 died at 8.80 mg/L and 1 died at 6.40 mg/L.  There were no deaths at 3.80 mg/L.  Clinical signs observed in all groups included red nose, lacrimation, salivation, red and/or swollen paws and various staining/soiling around the nose, mouth and/or eyes and other body surfaces.  Respiratory distress (laboured breathing, gasping, and/or rales) was observed starting at 6.40 mg/L.  Hunched posture, ocular opacity, ataxia and emaciation were noted for animals in the 8.80 mg/L group.  There were no gross necropsy findings for all survivors from the 3.80 and 6.40 mg/L groups, and for two of the four survivors from the 8.80 mg/L group.  For the animals in the 8.80 and 18.80 mg/L groups that died, gross necropsy findings included hemorrhage and/or congestion of the lungs, congestion of the liver, and discoloration and/or staining of the fur.  Hemorrhage of the thymus was frequently observed in animals in the 18.80 mg/L group and was noted in one animal in both the 6.4 and 8.8 mg/L groups.  Corneal opacities were noted at the scheduled necropsy for two 8.80 mg/L rats.  Clinical signs and necropsy findings were consistent with the lungs and eyes being target organs.  The one-hour combined male/female LC50 was determined to be 8.35 mg/L (nominal).

In a study conducted following OECD Short-term and Long-term Toxicology Groups Final Report adopted May 1961, groups of five Sprague-Dawley rats/sex were exposed by nose only vapour inhalation for one hour to concentrations of 5.44, 7.90, 11.02 and 15.02 mg/L (measured) (DCC, 1987).  All animals died at 15.02 mg/L, 6 died at 11.01 mg/L and 2 died at 7.90 mg/L and one died at 5.44 mg/L.  Clinical signs seen in all groups included nasal crust, rough coat, sores on the face and rales. Mouth breathing was seen in all groups except in the 5.44 mg/L exposure group.  Other less frequently observed signs included ocular discharge.  There were no other clinical signs.  Necropsy findings noted were corneal opacity and dark areas of the lungs. Exudate on the nasal turbinates and hemorrhagic intestines were seen in a number of animals but not in all groups.  There were no other necropsy findings.  The one-hour combined male/female LC50 based on measured concentrations, was determined to be 9.46 mg/L.

Sprague-Dawley rats (20/sex/concentration) were exposed to C3MS for one hour by nose only vapour inhalation (DCC, 1992).  Exposure concentrations were 0, 3.06 and 6.11 mg/L; two additional groups of animals were exposed to 2.20 and 4.40 mg/L.  The objective of this study was to investigate the differences in toxicity between C3MS and gaseous hydrogen chloride at low concentrations and also to identify the target area of the respiratory tract, nature of lesions, and sequence of events involved in the response and recovery of animals.  There were 3 deaths at 6.11 mg/L and 2 deaths at 3.06 mg/L.  Tissues and organs from C3MS exposed groups did not significantly differ grossly from HCl exposed animals. Mean terminal body weights of C3MS exposed groups were generally lower as compared to HCl exposed animals and organ weights, except lung, were proportionally lower as well.  Lung to body weight values were generally elevated for C3MS exposed groups.  Marked local toxic changes in the upper respiratory tract were observed for C3MS exposed groups.  Ciliated respiratory epithelium was predominately affected but olfactory epithelium was also damaged in most animals.  Histopathologic manifestations of toxicity ranged from attenuation to squamous metaplasia, necrosis and desquamation of respiratory tract mucosa.  Accompanying acute inflammation with edema and exudation was a prominent feature of the upper respiratory tract of all C3MS exposed animals.  The pattern of distribution of tissue change centered about the middle to posterior nasal cavity, with the most severe alteration in the nasopharynx for C3MS and the rostral portion of the nasal cavity for HCL.  Mucosal changes diminished in frequency and severity with posterior progression along with the tracheo-bronchial tree. Lungs of C3MS exposed rats had occasional acute or resolving haemorrhages and increased inflammatory changes, compared to unexposed controls.  Several liver changes were also observed in rats of both sexes exposed C3MS. An LC50 value was not reported.

C3ES. In a guideline study (OECD TG 403)  Fisher 344 rats (5/sex/concentration) were exposed by whole body vapour inhalation for one hour to concentrations of 7.73, 8.87 and 9.46 mg/L (nominal) (DCC, 1997c; Jean et al., 2006).  All animals died at 9.46 mg/L, 6 died at 8.87 mg/L and 2 died at 7.73 mg/L.  Respiratory (laboured breathing, rales), activity (hypoactivity, ataxia) and ocular (closed eyes, lacrimation, porphyrin staining, corneal opacity, dried material and alopecia around the eyes) effects were seen in all groups, as was various facial and body soiling.  These effects were first seen on the day of or the day following exposure and generally persisted until death, or throughout the first week of the study in rats that survived.  A majority of the rats that survived appeared normal by the end of the 14-day observation period.  There were no remarkable gross findings in rats that survived to the scheduled necropsy.  Necropsy findings in the rats that died (all groups) included ectasia of the lungs, obstruction of the nostrils, staining, fluid and/or crusts on the nares, dark red/congested livers, hemorrhage, congestion and/or consolidation of the lungs and discoloration or staining of the fur, blood in the GI tract, gaseous distension of the GI tract, corneal opacity, dehydration and a decrease or absence of body fat.  The one-hour combined male/female LC50 (nominal concentrations) was determined to be 8.41 mg/L.

Hydrolysis products

TMS.  In a guideline study (OECD TG 403), the 4-hr LC50 of TMS as a vapour in male and female, Sprague-Dawley rats was 11.6 mg/L.  Clinical signs included urogenital staining, decreased, shallow respiration, rales, hypoactivity, prostration, ataxia, lacrimation, partial closure of eyes, decreased defecation and urination, ocular opacities.

HCl. Inhalation LC50 values were determined to be 23.7-60.9 mg/L/5 min, 5.7-7.0 mg/L/30 min and 4.2-4.7 mg/L/60 min for rats, 20.9 mg/L/5min, 3.9 mg/L/30min and 1.7 mg/L/30min for mice, depending on exposure period.

Table 13     Summary of Acute Inhalation Toxicity Studies

| Species | Sex | Exposure duration | Comments | LC50 | Reference |
| --- | --- | --- | --- | --- | --- |
| Sponsored substance |  |  |  |  |  |
| CTMS (CAS No. 75-77-4) |  |  |  |  |  |
| Fisher 344 rat | Male/Female | 1 hr | Whole body | 18.92 mg/L (nominal) | DCC, 1999 |
| Sprague-Dawley rat | Male/Female | 1 hr | Nose only | 13.01 mg/L (measured) | DCC, 1987 |
| C2DMS (CAS No. 75-78-5) |  |  |  |  |  |
| Fisher 344 rat | Male/Female | 1 hr | Whole body | 11.04 mg/L (nominal) | DCC, 1997a |
| Sprague-Dawley rat | Male/Female | 1 hr | Nose only | 12.36 mg/L (measured) | DCC, 1987 |
| C3MS (CAS No. 75-79-6) |  |  |  |  |  |
| Sprague-Dawley rat | Male/Female | 4 hr | Nose only | 5.43 mg/L (measured) | DCC, 1989 |
| Fisher 344 rat | Male/Female | 1 hr | Whole body | 8.35 mg/L (nominal) | DCC, 1997b |
| Sprague-Dawley rat | Male/Female | 1 hr | Nose only | 9.46 mg/L (measured) | DCC, 1987 |
| Sprague-Dawley rat | Male/Female | 1 hr | Nose only | Not reported | DCC, 1992 |
| C3ES (CAS No. 115-21-9) |  |  |  |  |  |
| Fisher 344 rat | Male/Female | 1 hr | Whole body | 8.41 mg/L (nominal) | DCC, 1997c |
| Hydrolysis product |  |  |  |  |  |
| TMS (CAS No. 1066-40-6) |  |  |  |  |  |
| Rat | Male/Female | 4 hr | Whole body | 11.6 mg/L | WIL, 2007 |
| HCl (CAS No. 7647-01-0) |  |  |  |  |  |
| Rat | Not specified | 5 min 30 min 60 min | Not specified | 23.7-60.9 mg/L 5.7-7.0 mg/L 4.2-4.7 mg/L | http://www.chem.unep.ch/irptc/sids/oecdsids/sidspub.html |

Dermal

Studies in Animals

Sponsored substances

CTMS. In a study similar to OECD TG 402, New Zealand White rabbits (5/sex/dose) were exposed by the dermal route to 850, 1700 and 3400 mg/kg bw under occlusive cover for 24 hours (Bushy Run Research Center (BRRC), 1982).  All animals died at 3400 mg/kg bw.  Three of 8 animals died at 1700 mg/kg bw; necrosis and irritation and loss of body weight was noted for all animals.  One of 8 animals died at 850 mg/kg bw; necrosis and irritation and loss of body weight was noted for all animals.  The primary necropsy findings included various discolourations of the liver or lungs.  The dermal LD50s were 2023 (males) mg/kg bw and 1513 (females) mg/kg bw.

C3MS. Groups of five New Zealand white rabbits/sex were exposed by the dermal route to 635, 1270, and 2540 mg/kg bw under occlusive cover for 24 hours (BRRC, 1981).  The test protocol was not specified but was similar to OECD TG 402.  All animals died at 2540 mg/kg bw.  Three of 8 animals died at 1270 mg/kg bw; necrosis and irritation and loss of body weight were noted.  One of 8 animals died at 635 mg/kg bw; necrosis and irritation and loss of body weight were noted.  There were no remarkable necropsy findings for the surviving animals.  The dermal LD50s were 1791 (males) mg/kg bw and 1068 (females) mg/kg bw.

Hydrolysis products

No data are available for the hydrolysis products.

Table  14     Summary of Acute Dermal Toxicity Studies

| Species | Sex | LD50 (mg/kg bw) | Reference |
| --- | --- | --- | --- |
| Sponsored substance |  |  |  |
| CTMS (CAS No 75-77-4) |  |  |  |
| New Zealand White rabbit | Male/Female | males: 2023; females: 1513 | BRRC, 1982 |
| C3MS (CAS No 75-79-6) |  |  |  |
| New Zealand White rabbit | Male/Female | males: 1791; females: 1068 | BRRC, 1981 |

Oral

Studies in Animals

Sponsored substances

CTMS. In a study similar to OECD TG 401, Hilltop-Wistar albino rats (5/sex/dose) were administered the test material by gavage as a 25% dilution in Silicone L-45 Oil (BRRC, 1982).  The dosage levels used in this study were 1700, 3400, and 6800 mg/kg bw in males and 3400, 4845, and 6800 mg/kg bw in females.  All animals died at 6800 mg/kg bw.  There were no deaths in the remaining groups with the exception of a single male animal at 1700 mg/kg bw.  Signs of toxicity in surviving animals included slight sluggishness and staining/greasy appearance of the fur.  Body weight gains were reported for all groups of surviving animals.  There were no remarkable findings at necropsy for surviving animals. The LD50s were 4811 mg/kg bw (males) and 5636 mg/kg bw (females).

C3MS. In a study similar to OECD TG 401, Hilltop-Wistar albino rats (5/sex/dose) were administered the test material by gavage as a 25% dilution in Silicone L-45 Oil (BRRC, 1981).  The dosage levels used in this study were 635, 1270, 5080 and 10,160 mg/kg bw in males; 1270, 2540, 5080 and 10,160 mg/kg bw in females.  The test protocol was not specified but was similar to OECD TG 401.  There were 0 (males only), 2, 4, 8 and 10 deaths at doses of 635, 1270, 2540, 5080 and 10,160 mg/kg bw, respectively.  Body weight gains were reported for all groups of surviving animals.  There were few clinical signs in surviving animals with sporadic occurrence of sluggishness or ruffled, greasy fur.  There were no remarkable findings at necropsy for surviving animals with the exception of reported lungs with dark red patches in females dosed at 2540 mg/kg bw.  The LD50s were 2057 mg/kg bw (males) and 3594 mg/kg bw (females).

Hydrolysis products

TMS. In a guideline study (OECD TG 401), Wistar rats (5/sex/dose) were administered TMS by gavage at doses of 0.5, 1.0, 2.0, 3.1, 5.0, and 6.3 mL/kg bw (Ramm, 1985).  Clinical signs included an effect on general condition, prone/lateral position, growth retardation, and ruffled fur. Body weight data was not reported.  There were no remarkable findings at necropsy for animals that survived.  The LD50 was 3.5 mL/kg bw (sexes combined; based on a density of 0.81 the LD50 = 2835 mg/kg bw).

DMSD. In a guideline study (OECD TG 425) Crj: CD(SD) female rats (5/dose) were administered DMSD by gavage at a single (limit) dose of 2000 mg/kg bw (DCC, 2007).  There was one death. Clinical findings for surviving animals included wet yellow material on the urogenital, anogential area and /or ventral trunk.  There were no remarkable body weight changes or necropsy findings. The LD50 was greater than 2000 mg/kg bw (females).

HCl. The acute oral LD50 values were determined to be 238-277 mg/kg bw for female rats.

Table 15     Summary of Acute Oral Toxicity Studies

| Species | Sex | LD50 (mg/kg bw) | Reference |
| --- | --- | --- | --- |
| Sponsored substance |  |  |  |
| CTMS (CAS No 75-77-4) |  |  |  |
| Hilltop-Wistar albino rats | Male/Female | males: 4811; females: 5636 | BRRC, 1982 |
| C3MS (CAS No 75-79-6) |  |  |  |
| Hilltop-Wistar albino rats | Male/Female | males: 2057; females: 3594 | BRRC, 1981 |
| Hydrolysis product |  |  |  |
| TMS (CAS No 1066-40-6) |  |  |  |
| Wistar | Male/Female | 2835 mg/kg bw (sexes combined) | Ramm, 1985 |
| DMSD (CAS No 1066-42-8) |  |  |  |
| Crj: CD(SD) | Female | >2000 mg/kg bw | DCC, 2007 |
| HCl (CAS No 7647-01-0) |  |  |  |
| Rat | Female | 238-277 mg/kg bw | http://www.chem.unep.ch/irptc/sids/oecdsids/sidspub.html |

Conclusion

The range of 1-hour acute inhalation LC50’s for the category members is 8.35 mg/L (C3MS) to 18.92 mg/L (CTMS).  The 4-hour nose-only LC50 for C3MS was 5.43 mg/L in rats.  The principal clinical signs were indicative of respiratory and ocular effects.  The acute inhalation hazard posed by an individual chlorosilane, as defined by an LC50 value, is directly proportional to its chlorine content and subsequently to the HCl that is liberated during hydrolysis.  The dermal LD50 for CTMS in rabbits was 1513 (females) and 2030 (males) mg/kg bw; for C3MS the dermal LD50 in rabbits was 1068 (females) and 1719 (males) mg/kg bw.  Necrosis, irritation and loss of body weight were common clinical signs for both substances.  The primary necropsy findings for CTMS included various discolorations of the liver or lungs; no remarkable findings were noted for C3MS.

The oral LD50 for CTMS in rats was 5636 (females) and 4811 (males) mg/kg bw; for C3MS the oral LD50 in rats was 3594 (females) and 2057 (males) mg/kg bw.  Necrosis, irritation and loss of body weight were common clinical signs for both substances; chlorosilanes are widely recognized as corrosive and considered irritants.  The primary necropsy findings for CTMS included various discolorations of the liver or lungs; no remarkable findings were noted for C3MS.  Reported signs of toxicity for both substances in surviving animals included sluggishness, salivation, dyspnea, rales, prostration, and/or staining/greasy texture of the fur. Body weight gains were reported for all groups of surviving animals exposed to either CTMS or C3MS.  There were no remarkable findings at necropsy for surviving animals for either CTMS or C3MS with the exception of the finding of lungs with dark red patches in females dosed with CMeS at 2540 mg/kg bw.

### 3.1.3	Irritation

Skin and eye irritation data are available for CTMS and C3MS.  Respiratory tract irritation data are available for the sponsored substances and hydrolysis product, HCl.

Skin Irritation

Studies in Animals

Sponsored substances

CTMS. 0.5 ml ClTMS was applied under occlusive cover to the skin of three New Zealand White rabbits/sex (BRRC, 1982). The procedure followed the Draize test. All 6 rabbits had erythema, edema and necrosis.  Desquamation was apparent after 7 days.  A reliable PDII could not be calculated.

C3MS. 0.5 ml Cl3MS was applied under occlusive cover for four hours to the skin of one male and five female New Zealand White rabbits (BRRC, 1981).  The procedure followed the Draize test. Reddening, swelling and necrosis were observed at the site of contact.  Necrosis occurred within a day, and often was present at 7 days (the end of the period of observation).  Desquamation and scab formation were also noted during the study.  C3MS was considered corrosive in this study.  0.5 ml C3MS was applied under occlusive cover for three minutes to the skin of one male and one female New Zealand White rabbit (BRRC, 1989).  The procedure followed the Draize test.  Moderate edema and full thickness necrosis was observed for both animals within one hour after contact.  Ulceration was observed on both animals at 24 hours with the tissue of the dose site of one animal apparently completely destroyed; the remaining animal had severe ulceration (tissue destruction) at 48 hours.  C3MS was considered corrosive in this study.

Hydrolysis products

TMS. In an OECD Guideline 404 (Acute Dermal Irritation / Corrosion) study, 0.5 ml TMS was applied under semi-occlusive cover for four hours to the skin of 3 female New Zealand white rabbits (HC:NZW) and observed for 7 days(Suberg, 1986).  There were no signs of erythema or edema; the substance was considered non-irritating.

HCl. Hydrogen chloride is corrosive to the skin.

Eye Irritation

Studies in Animals

Sponsored substances

CTMS. One eye of male or female New Zealand White rabbits was dosed with volumes of 0.1 (two animals), 0.01 (six animals) and 0.005 (six animals) mL (BRRC, 1982).  The eyelids were held together for one second and the eyes were not rinsed.  The procedure followed the Draize test.  Two eyes dosed with 0.1 mL of the undiluted material had severe irritation of the cornea, iris and conjunctivae which persisted up to 21 days.  Less severe injury resulted from administration of 0.01 and 0.005 mL per eye, but it persisted for several days after dosing.  All dosed rabbits had immediate signs of discomfort. CTMS was considered highly irritating.

C3MS. One eye of six New Zealand White rabbits (sex not specified) was dosed with 0.005 mL; the eyelids were held together for one second and the eyes were not rinsed (BRRC, 1981).  Moderate to severe reddening and swelling of the conjunctivae with mild to severe corneal injury was observed.  The corneal opacities prevented an accurate assessment of the condition of the iris.  Effects on the conjunctivae and cornea appeared within an hour of the application and persisted in many cases for 14 days.  One dosed rabbit died at 13 days.  The cause of death was uncertain.  Injury to the remaining rabbits’ eyes persisted for more than 21 days, although it had become less severe. C3MS was considered highly irritating.

Hydrolysis products

TMS. In a guideline study (OECD TG  405), one eye of 3 male New Zealand White rabbits (HC:NZW) was dosed with 0.1 mL (Suberg, 1985) of test substance.  Eyes were rinsed after 24 hours.  The substance was slightly irritating; all effects were reversible within 7 days.

HCl. Severe effects can be expected from exposure to the eyes.

Respiratory Tract Irritation

Studies in Animals

Sponsored substances

Data for the category members from acute toxicity inhalation studies (described in Section 3.1.2) suggest that the category members are highly irritating to the respiratory tract.

Hydrolysis products

HCl. Animals died from respiratory failure (alveolar emphysema, atelectasis, and oedema of the lungs) shortly after acute inhalation exposure.

Conclusion

The sponsored alkyl chlorosilanes are corrosive and highly irritating to the skin, eyes and respiratory tract; these effects are likely due to the rapid production of HCl following hydrolysis.  Similar effects are seen following exposure to HCl.  The hydrolysis product, TMS, is non-irritating to the skin and slightly irritating to the eyes.

### 3.1.4	Sensitisation

No data are available for the sponsored materials or their associated hydrolysis products.

### 3.1.5	Repeated Dose Toxicity

Reliable repeated dose toxicity data for the alkyl chlorosilanes are not available. Inhalation data for the alkyl chlorosilanes in single concentration or 10 day inhalation studies are considered unassignable (reliability 4), but are presented in the tables for information purposes.  Inhalation and oral data are available for the hydrolysis products.  Tables 16 and 17 report details of inhalation and oral studies, respectively.

Studies in Animals

Inhalation

Sponsored substances

CTMS. No test article-related effects occurred when male and female Sprague-Dawley rats were exposed to 0.15 mg/L (nominal) of CTMS vapor five day per week for two weeks (DCC, 1993; RL=4).

C2DMS. No test article-related effects occurred when male and female Sprague-Dawley rats were exposed to 0.079 mg/L (nominal) of C2DMS vapor, five day per week for two weeks (DCC, 1993; RL=4).

C3MS. No test article-related effects occurred when male and female Sprague-Dawley rats were exposed to 0.073 mg/L (nominal) of C3MS vapors five day per week for two weeks (DCC, 1993; RL=4).

Hydrolysis products

TMS. In a combined repeated-dose/reproductive/developmental toxicity screening test (OECD TG 422) , three groups of Crl:CD(SD) rats, each group consisting of 10 males, 10 toxicity phase females and 10 reproductive phase females, were exposed via whole-body inhalation to vapor atmospheres of the test article 6 hours/day, 7 days/week; males were dosed for ~ 34 days and toxicity phase females for 28 days. Target exposure concentrations were 0, 60, 300 or 600 ppm (equivalent to 0, 0.22, 1.10, and 2.20 mg/L).  Mean measured test article exposure concentrations were 0, 61, 303 or 602 ppm for males and 0, 61, 304 or 604 ppm for females.  A concurrent control group was exposed to filtered air on a comparable regimen (WIL Research, 2008).  One F0 toxicity phase female in the 1.10 mg/L group was found dead on day of scheduled necropsy; since there were no mortalities in the 600 ppm group, this death was not attributed to test article exposure.  There were no test article-related clinical signs, macroscopic findings, or effects on body or organ weights.  There were no test article-related effects on FOB parameters and no effects on locomotor activity patterns. Test article-related effects were limited to changes in hematology (lower eosinophil and lymphocyte counts for males) and serum chemistry (higher alanine aminotransferase for males and toxicity phase females) at 2.20 mg/L.  These changes occurred in the absence of correlating histologic changes and were not considered adverse.  The NOAEC for systemic toxicity for TMS in rats was 2.20 mg/L (highest concentration tested).

HCl. Several 90-day repeated dose inhalation toxicity studies, compliant with FDA-GLP, have been conducted resulting in  local effects of irritation in groups of 0.015 mg/L and above.  The NOAELs have been determined to be 0.030 mg/L for rats and mice.

Table 16    Summary of Repeated-Dose Inhalation Toxicity

| Test substance | Species | Test guideline | Study duration | Doses | NOAEC | Reference |
| --- | --- | --- | --- | --- | --- | --- |
| Sponsored substances |  |  |  |  |  |  |
| CTMS (CAS No 75-77-4) | Rat | Not specified | 10 days | 0.15 mg/L | 0.15 mg/L | DCC, 1993; RL = 4 |
| C2DMS (CAS No 75-78-5) | Rat | Not specified | 10 days | 0.079 mg/L | 0.079 mg/L | DCC, 1993; RL = 4 |
| C3MS (CAS No 75-79-6) | Rat | Not specified | 10 days | 0.073 mg/L | 0.073 mg/L | DCC, 1993; RL = 4 |
| Hydrolysis products |  |  |  |  |  |  |
| TMS (CAS No 1066-40-6) | Rat | OECD TG 422 | 28 days | 0.22, 1.10 and 2.20 mg/L/day | 2.20 mg/L (highest concentration tested) | WIL Research, 2008 |
| HCl (CAS No 7647-01-0) | Rats and mice | -- | 6 hrs/d, 5 d/wk; 90 d | 0, 0.015, 0.030 and 0.075 mg/L/day | 0.030 mg/L | http://www.chem.unep.ch/irptc/sids/oecdsids/sidspub.html |

Dermal

No data available.

Oral

Sponsored substances

No data available.

Hydrolysis products

TMS. In a guideline study (OECD TG 407). Wistar; Bor:WISW (SPF Cpb) (5/sex/concentration) were administered TMS by gavage at doses of 0 (peanut oil), 80, 250 and 750 mg/kg bw/day for 7 days a week for 4 weeks (28 days) (Ramm and Bomhard, 1987).  Animals were monitored for mortality, clinical signs, food/water intake, body weight changes, hematology, and organ weight changes.  Gross necropsy and histopathology was conducted of all animals at study termination. There were no deaths and no effects on food/water consumption.  After daily dosing, the 750 mg/kg bw/day animals showed symptoms of alcohol staggering gait; this clinical sign diminished in the late afternoon.  Slight growth retardation in all 750 mg/kg bw/day animals was observed.  There was no effect on hematology.  Relative liver weights were significantly increased in the 250mg/L (17%) and 750 mg/L (21%) group females respectively.  No significant increase was noted for males at any dose level; there was no indication of liver damage (clinical chemical, pathological, histopathological) at 80 or 250 mg/kg bw/day and changes were considered to be adaptive in nature.  Slight changes in the bile ducts (pigment deposit; proliferation) in the 750 mg/kg bw groups were observed.  There was no change in function and morphology of the other organs and tissues examined.  The NOAEL was = 250 mg/kg bw/day for male and female animals based on clinical signs, growth retardation, and changes in the liver at 750 mg/kg bw/day.

DMSD. In a combined repeated-dose/reproductive/developmental toxicity screening test (OECD TG 422), the substance was administered via gavage to 10 Sprague-Dawley rats/sex/dose at 0 (corn oil), 50, 250 and 500 mg/kg bw/day (DCC, 2008c).  Males were treated during pre-mating and mating periods.  Males and toxicity group females were sacrificed after they had been treated for 28 days.  All but one animal survived to the scheduled necropsy; it was euthanized on the third day of dosing due to an oral gavage error.  Clinical signs included abdominal, urogenital and/or muzzle soiling in males at 250 mg/kg bw/day and both sexes at 500 mg/kg bw/day. There was no effect on body weights; body weight gains decreased (45%) for males at week 4, 500 mg/kg bw and decreased (49%) for toxicity group females at week 3, 500 mg/kg bw.  There were no effects on food consumption, FOB/motor activity or clinical pathology.  Absolute liver weights were increased in males and females at 250 (19% and 24%, respectively) and 500 mg/kg bw (31% and 61%, respectively).  Liver to body weight ratios increased in males and females at 250 (22% and 25%) and 500 mg/kg bw (41% and 65%, respectively). Absolute adrenal weights were decreased in males at 500 mg/kg bw (24%) and relative weights decreased (29%).  Absolute thymus weights were decreased by 31% in males at 500 mg/kg bw and relative weights were decreased by 30%.  Absolute testes weights were decreased in males at 500 mg/kg bw (12%). Liver centrilobular hypertrophy was observed in males at 250 (8/10 animals) and 500 mg/kg bw (8/10) and females at 250 (8/10) and 500 mg/kg bw (8/10).  Severity was minimal to mild and considered to be an adaptive change.  Liver porphyria accompanied by chronic inflammation and bile duct hyperplasia was observed in males only at 500 mg/kg bw (9/10, minimal to mild).  Liver periportal vacuolation was observed in females at 500 mg/kg bw (10/10, grade was mild to moderate).  Thyroid gland follicular hypertrophy was observed in males at 250 (8/10) and 500 mg/kg bw (9/10), with minimal to mild severity and a significant shift in grade as compared to control animals.  This was considered an adaptive secondary effect and although adverse in the rat, the mechanism is generally not applicable to species with significant levels of thyroid binding globulin (Capen, et. al., 2002).  A NOAEL of 250 mg/kg bw/day was assigned for systemic toxicity for males and toxicity group females based on liver porphyria in male rats and liver vacuolation in toxicity group female rats.

Table 17    Summary of Repeated-Dose Oral Toxicity

| Test substance | Species | Test guideline | Study duration | Doses (mg/kg bw) | NOAEL | Reference |
| --- | --- | --- | --- | --- | --- | --- |
| Hydrolysis products |  |  |  |  |  |  |
| TMS (CAS No. 1066-40-6) | Wistar | OECD TG 407 | 28 days | 80, 250 and 750 | 250 | Ramm and Bomhard, 1987 |
| DMSD (CAS No. 1066-42-8) | Sprague-Dawley rat | OECD TG 422 | 28-29 days | 0, 50, 250 or 500 | 250 | DCC, 2008c |

Conclusion

Repeated dose toxicity data for the alkyl chlorosilanes are not available.  Based on their rapid hydrolysis, data from the primary hydrolysis products are used to fill the repeated-dose toxicity endpoint.  By the inhalation route, the lowest NOAEC was for hydrolysis product HCl; during repeated dose toxicity studies, the local effects of irritation were observed in the groups of 0.015 mg/L and above in the 90-day inhalation study.  The NOAEC for systemic toxicity for HCl, excluding the local effects of irritation, has been determined to be 0.030 mg/L for rats and mice.  The NOAEC for repeated dose inhalation of the silanol hydrolysis product, TMS, was 2.20 mg/L (the highest concentration tested).  The NOAELs for the silanol hydrolysis products, TMS and DMSD were each 250 mg/kg bw with liver as the primary target organ.

### 3.1.6	Mutagenicity [rearranged to match the SIAR template]

Bacterial mutagenicity studies have been conducted with all sponsored alkyl chlorosilanes.  In vitro chromosome aberration assays have been conducted with CTMS and C3MS. In vivo micronucleus studies have been conducted with CTMS, C2DMS and C3MS.  In vitro gene mutation, chromosomal aberration and other assays are outlined in Table 18 and in vivo chromosomal aberrations studies are detailed in Table 19.

Studies in Animals

In vitro Studies - Gene mutations

Sponsored substances

CTMS. In two separate tests, CTMS was negative for mutagenicity in bacterial reverse mutation assays with Salmonella typhimurium TA-1535, TA-1537, TA-1538, TA-98 and TA-100 and Escherichia coli WP2 uvrA or W3110/pol A+ and P3478/pol A-, in the presence and absence of metabolic activation at concentrations up to 5 g/plate or 5 L/plate (Litton Bionetics, 1979; Wacker Chemie, 1987).  All bacterial strains exhibited mutagenic responses to the positive control substances, as appropriate.  The cytotoxic concentration in one study was 2500 µg/plate and in the second was 5000 µL/plate.  In another Salmonella typhimurium (TA 98, TA 100, TA 1535, TA 1537 and TA 1538) assay, a preincubation bacterial reverse mutation study following the standard NTP protocol, CTMS was negative for mutagenicity in the presence and absence of metabolic activation (Zeiger et al., 1992).  Concentrations tested were 0.5, 5, 50, 250 or 500 µg/plate.  Additional details were not available for this study.  A Salmonella (strains not identified) preincubation bacterial reverse mutation study following the standard national toxicology program (NTP) protocol showed that CTMS was weakly positive for mutagenicity in the presence and absence of metabolic activation (Mortelmans et al., 1986).  Finally, data from the NTP website (http://ntp-apps.niehs.nih.gov/ntp_tox/index.cfm?fuseaction=ntpsearch.searchhome) showed that CTMS in Salmonella typhimurium strains TA 97, TA98, TA 100, TA 1535 and TA 1537 were weakly positive (cytotoxicity observed at 1.66 g/plate) in one study and negative in the other when tested up to 3.33 mg/plate.  The positive controls reacted appropriately.

In a mammalian cell gene mutation assay, L5178Y Fischer mouse lymphoma cells were exposed to CTMS at concentrations of 0.02, 0.04, 0.08, 0.16, and 0.32 µL/mL in the presence and absence of metabolic activation (Litton Bionetics, 1979).  Appropriate concurrent negative and positive controls were included and the expected responses were observed.  The test substance did not induce forward mutation in L5178Y mouse lymphoma cells either without activation or in the presence of the mouse liver S9 activation system.  A second trial was conducted in order to test the compound at less than 50% survival in the presence of the S9 activation system. This trial supported the observations in Trial 1.  CTMS did not induce significant forward mutation under the conditions of this assay.

C2DMS. In a bacterial reverse mutation assay Salmonella typhimurium TA-98, TA-100, TA-1535, TA-1537, TA-1538 and Escherichia coli WP2uvrA were exposed to C2DMS at concentrations up to 10,000  µg/plate in the presence and absence of metabolic activation (Pharma Research, 1987).  Cytotoxicity was observed at doses greater than 1500 µg/plate.  Appropriate concurrent negative and positive controls were included and the expected responses were observed.  C2DMS did not demonstrate genetic activity with or without metabolic activation.  C2DMS was negative for mutagenicity in other bacterial reverse mutation assays with Salmonella typhimurium TA-1535, TA-1537, TA1538, TA-98 and TA-100 and Escherichia coli W3110/pol A+ and P3478/pol A-, in the presence and absence of metabolic activation at concentrations up to 5 µL/plate (Litton Bionetics, 1978b).  The cytotoxic concentration was 5 µL/plate.  In a mammalian cell gene mutation assay, L5178Y Fischer mouse lymphoma cells were exposed to C2DMS at concentrations of 0.02 - 0.32 µL/mL in the absence of metabolic activation  and 0.04 - 0.64 µL/mL in the presence of metabolic activation (Litton Bionetics, 1978b).  Appropriate concurrent negative and positive controls were included and the expected responses were observed.  The test substance did not induce forward mutation in L5178Y mouse lymphoma cells either without activation or in the presence of the mouse liver S9 activation system.

C3MS. In a bacterial reverse mutation assay, Salmonella typhimurium TA-98, TA-100, TA-1537, TA-1538 were exposed to concentrations up to 12,500 µg/plate in one study and up to 2400 µg/plate in a second study the presence and absence of metabolic activation (Bayer, 1988).  Cytotoxicity was observed at 12500 µg/plate.  Appropriate concurrent positive and negative controls were included and even at the low doses used, the expected responses were observed. C3MS was not mutagenic in any of the tests conducted in this evaluation, both with and without metabolic activation.  In another bacterial reverse mutation assay, Salmonella typhimurium TA-1535, TA-1537, TA-1538, TA-98 and TA-100 and Escherichia coli W3110/polA+ and P3478/polA- were exposed to concentrations of C3MS up to 5 µL/plate in the presence and absence of metabolic activation (Litton Bionetics, 1978a). Cytotoxicity was observed at 5 µL /plate (TA 1535, TA 1537, TA 1538) and 1 and 5 µL /plate (E. coli).  Appropriate concurrent negative and positive controls were included, and the expected responses were observed. There was no evidence of mutagenic activity in any of the assays conducted, both with and without metabolic activation.

In a mammalian cell gene mutation assay, L5178Y Fischer mouse lymphoma cells were exposed to concentrations up to 0.32 µL/mL in the presence and absence of metabolic activation (Litton Bionetics, 1978a).  Cytotoxicity was observed at concentrations > 0.16 µL/mL.  Appropriate concurrent negative and positive controls were included and the expected responses were observed.  There was no evidence of gene mutation in mammalian cells following exposure to C3MS.

C3ES. In a bacterial reverse mutation assay, Salmonella typhimurium TA-98, TA-100, TA-102, TA-1535 and TA-1537 were exposed to C3ES at concentrations up to 5000 g/plate in the presence and absence of metabolic activation (Wacker Chemie, 2002).  Cytotoxicity was observed at 5000 g/plate.  Appropriate concurrent negative and positive controls were included and the expected responses were observed.  C3ES caused no mutagenic effect in the Salmonella typhimurium test strains when tested either in the plate incorporation or in the preincubation test with or without metabolic activation.

Table 18    Summary of In Vitro Studies

| Test substance | Test guideline | Strain/species | Dose | Conclusion | Reference |
| --- | --- | --- | --- | --- | --- |
| CTMS (CAS No. 75-77-4) | Ames et al., 1973; Ames et al., 1975; Green and Muriel, 1976 (1) | S typhimurium TA-1535, TA-1537, TA-1538, TA-98 and TA-100 and E coli WP2 uvrA | up to 5 g/plate | Negative | Wacker Chemie, 1987 |
|  | National Toxicology Program protocol | S typhimurium TA 98, TA 100, TA 1535, TA 1537 and TA 1538 | 0.5, 5, 50, 250 or 500 µg/plate. | Negative | Zeiger et al 1992 |
|  | National Toxicology Program protocol | Not identified | Not indentified | Weakly positive | Mortelmans et al 1986 |
|  | National Toxicology Program protocol | S typhimurium TA97, TA98, TA100, TA102, TA104, TA1535, TA1537, and TA1538 | Cytotoxic at 1.66 µg/plate. | Weakly positive | http://ntp-apps.niehs.nih.gov/ntp_tox/index.cfm?fuseaction=ntpsearch.searchhome |
|  | National Toxicology Program protocol | S typhimurium TA97, TA98, TA100, TA102, TA104, TA1535, TA1537, and TA1538 | Up to 3.33p to 3. | Negative | http://ntp-apps.niehs.nih.gov/ntp_tox/index.cfm?fuseaction=ntpsearch.searchhome |
|  | Ames et al., 1975; McCann et al., 1975 (2) | S typhimurium TA-1535, TA-1537, TA1538, TA-98 and TA-100 and E coli W3110/pol A+ and P3478/pol A- | 0.001, 0.01, 0.1, 1.0 and 5 µl/plate | Negative | Litton Bionetics, 1979 |
|  | Mammalian cell gene mutation assay (3) | L5178Y Fischer mouse lymphoma cells | 0.02, 0.04, 0.08, 0.16, and 0.32 µl/ml | Negative | Litton Bionetics, 1979 |
|  | Cytogenetics (6) | L5178Y Fischer mouse lymphoma cells | 0.02, 0.04, 0.08, 0.16, and 0.32 µL/mL | Negative | Litton Bionetics, 1979 |
|  | SCE (3) | L5178Y Fischer mouse lymphoma cells | 0.02, 0.04, 0.08, 0.16, and 0.32 µl/ml | Negative | Litton Bionetics, 1979 |
|  | DNA damage and repair assay (3) | L5178Y Fischer mouse lymphoma cells | 0.08, 0.16, 0.32, and 0.64 µl/ml | Negative | Litton Bionetics, 1979 |
| C2DMS (CAS No. 75-78-5) | Ames test et al., 1973 | S typhimurium TA 98, TA 100, TA 1535, TA1537, TA1538 and E coli WP2uvrA | 4, 20, 100, 500, 2500, 5000, 10000 µg/plate | Negative | Pharma Research, 1987 |
|  | Ames et al., 1975; McCann et al., 1975 (2) | S typhimurium TA-1535, TA-1537, TA1538, TA-98 and TA-100 and E coli W3110/pol A+ and P3478/pol A- | 0.001, 0.01, 0.1, 1.0 and 5 µl/plate | Negative | Litton Bionetics, 1978b |
|  | Mammalian cell gene mutation assay (3) | L5178Y Fischer mouse lymphoma cells | 0.02-0.64 µl/ml | Negative | Litton Bionetics, 1978b |
|  | Cytogenetics (6) | L5178Y Fischer mouse lymphoma cells | 0.02 - 0.32 uL/mL (without activation) and 0.04 - 0.64 uL/mL (with activation) | Negative | Litton Bionetics, 1978b |
|  | SCE (3) | L5178Y Fischer mouse lymphoma cells | 0.02-0.64 µl/ml | Negative | Litton Bionetics, 1978b |
|  | DNA damage and repair assay (3) | L5178Y Fischer mouse lymphoma cells | 0.08, 0.16, 0.32, 0.64 µl/ml | Negative | Litton Bionetics, 1978b |
| C3MS (CAS No. 75-79-6) | Ames test | S typhimurium TA 98, TA 100, TA 1537, TA 1538 | 20,100,500,2500,12,500 µg/plate and (2) 2nd Study:75,150,300,600,1200,2400 µg/plate | Negative | Bayer, 1988 |
|  | (4) | S typhimurium TA 1535, TA 1537, TA 1538, TA 98 and TA100 and E coli W3110/polA+ and P3478/polA- | 0.001, 0.01, 0.1, 1.0 and 5 µl / plate | Negative | Litton Bionetics, 1978a |
|  | Mammalian cell gene mutation assay (3) | L5178Y Fischer mouse lymphoma cells | 0.01, 0.02, 0.04, 0.08, 0.16, 0.32 µl/ml | Negative | Litton Bionetics, 1978a |
|  | Cytogenetics (6) | L5178Y Fischer mouse lymphoma cells | 0.01, 0.02, 0.04, 0.08, 0.16, 0.32 µL/mL | Weak positive | Litton Bionetics, 1978a |
|  | SCE (3) | L5178Y Fischer mouse lymphoma cells | 0.01, 0.02, 0.04, 0.08, 0.16, 0.32 µl/ml | Weak positive | Litton Bionetics, 1978a |
|  | DNA damage and repair assay (3) | L5178Y Fischer mouse lymphoma cells | 0.01, 0.02, 0.04, 0.08, 0.16, 0.32 µl/ml | Negative | Litton Bionetics, 1978a |
| C3ES (CAS No. 115-21-9) | (5) | S typhimurium TA 98, TA 100; TA 102; TA 1535; TA 1537 | 100 to 5000 µg/plate | Negative | Wacker Chemie, 2002 |

(1) Ames et al. (1973) Proc Nat Acad Sci 70:2281-2285; Ames et al. (1975) Mutat Res 31:347-364; Green and Muriel (1976) Mutat Res 3-32.

(2) Ames et al. (1975) Mutat Res 31:347-364; McCann et al. (1975) 72:5135-5139.

(3) Clive and Spector, Mutation Research 31:17-29, 1975; Korenberg and Freedlander, Chromosomes 48:355, 1974; Swenberg et al., BBRC 13:4134-4139, 1974

(4) Litton Bionetics standard procedure: Screening Program for the Identification of Potential Mutagens and Carcinogens. Protocol Number: DMT 100.

(5) EC Directive 92/69/EEC, L 383 A: Part B; OECD Guideline No. 471; ICH Guideline S2A: Genotoxicity (CPMP/ICH/141/95); ICH Guideline S2B: Genotoxicity (CPMP/ICH/174/95).\

(6) Clive and Spector, Mutation Research 31:17-29, 1975; Korenberg and Freedlander, Chromosomes 48:355, 1974; Swenberg et al., BBRC 13:4134-4139, 1974

In vitro Studies - Chromosomal aberrations

Sponsored substances

CTMS. In a chromosome aberration test, L5178Y Fischer mouse lymphoma cells were exposed to CTMS at concentrations of 0.02, 0.04, 0.08, 0.16, and 0.32 µL/mL in the presence and absence of metabolic activation (Litton Bionetics, 1979).  Concurrent negative and positive control agents were included. The expected responses were observed for the appropriate concurrent negative and positive controls.  Some chromosomal aberrations were observed.  However, they were dose independent and not considered positive under the conditions of this assay.  CTMS was considered negative for chromosomal aberrations in this assay.

C2DMS. In a chromosome aberration test, L5178Y Fischer mouse lymphoma cells were exposed to C2DMS at doses of 0.02 - 0.32 µl/mL (without activation) and 0.04 - 0.64 µL/mL (with activation) (Litton Bionetics, 1978b).  Concurrent negative and positive control agents were included.  The expected responses were observed for the appropriate concurrent negative and positive controls.  There was no significant increase in chromosomal aberrations either with or without metabolic activation.

C3MS. In a chromosomal aberration test, L5178Y Fischer mouse lymphoma cells were exposed to 0.01, 0.02, 0.04, 0.08, 0.16, and 0.32 µL/mL Cl3MS in the presence and absence of metabolic activation (Litton Bionetics, 1978a). Cytotoxicity was observed at concentrations greater than 0.16 uL/mL.  Appropriate concurrent negative and positive controls were included and the expected responses were observed.  Cytogenetic analysis for chromosome aberrations showed test substance-related increases at 0.08 and 0.16 µL/mL dose levels under activation conditions; no increase was observed in the absence of activation.  C3MS was considered to have weak clastogenic activity in this assay.

In vitro Studies – Other

Sponsored Substances

CTMS.  In a Sister Chromatid Exchange (SCE) assay, L5178Y Fischer mouse lymphoma cells were exposed to concentrations of 0.02, 0.04, 0.08, 0.16, and 0.32 µL/mL CTMS in the presence and absence of metabolic activation (Litton Bionetics, 1979).  Appropriate concurrent negative and positive controls were included and the expected responses were observed.  No evidence of significant induced increases in the number of SCEs per cell or chromosomes with SCEs or in the mean number of SCEs per chromosome with SCE's was observed either with or without activation.  None of the values at any concentration were significantly different from either the concurrent or historical controls; nor was there a dose-related response.

In a DNA damage and repair assay, L5178Y Fischer mouse lymphoma cells were exposed to concentrations of CTMS at 0.02, 0.04, 0.08, 0.16, and 0.32 µL/mL in the absence of metabolic activation and 0.16, 0.32 0.64 and 1.28 µL/mL in the presence of metabolic activation (Litton Bionetics, 1979).  Appropriate concurrent negative and positive controls were included and the expected responses were observed.  A significant change in the amount of radiolabeled DNA retained on the filter was observed at 0.64 µL/mL without activation and with activation at 1.28 µL/mL.  This loss was due to the cell toxicity observed at and above 0.64l/ml.  This loss was due to cell toxicity, however, rather than primary DNA damage of a repairable type.  The test substance did not induce significant primary DNA damage in L5178Y mouse lymphoma cells.

C2DMS.  In an SCE assay, L5178Y Fischer mouse lymphoma cells were exposed to concentrations of 0.02 - 0.32 µL/mL C2DMS in the absence of metabolic activation and 0.04 – 0.64 µL/mL in the presence of metabolic activation (Litton Bionetics, 1978b).  The solvent control and positive controls gave the expected responses.  A very slight increase in SCE per cell (<2X the negative control vales) was observed in a non-dose-responsive fashion in mouse lymphoma cells without activation.  The suggestion of clastogenic activity appeared to be related to cytotoxic effects of the test material.  There was no statistically significant increase with activation. In a DNA damage and repair assay, L5178Y Fischer mouse lymphoma cells were exposed to concentrations of C2DMS at 0.08, 0.16, and 0.32 µL/mL in the absence of metabolic activation and 0.16, 0.32 and 0.64 µL/mL in the presence of metabolic activation (Litton Bionetics, 1978b).  Appropriate concurrent negative and positive controls were included and the expected responses were observed.  The test substance did not induce significant primary DNA damage in L5178Y mouse lymphoma cells.

C3MS.  In an SCE assay, L5178Y Fischer mouse lymphoma cells were exposed to concentrations of 0.01, 0.02, 0.04, 0.08, 0.16, 0.32 µL/mL in the presence and absence of metabolic activation (Litton Bionetics, 1978a).  Cytotoxicity was observed at doses equal to and greater than 0.16 µL/mL.  Appropriate concurrent negative and positive controls were included and the expected responses were observed.  Appropriate concurrent negative and positive controls were included and the expected responses were observed. C3MS induced significant levels of SCEs under both activation and non activation test conditions.  The levels of increases were higher with activation and may indicate some type of metabolic activation.  C3MS was considered to have weak clastogenic activity in this assay.

In an alkaline elution (DNA damage) test, L5178Y Fischer mouse lymphoma cells were exposed to concentrations of 0.01, 0.02, 0.04, 0.08, 0.16, 0.32 µL/mL in the presence and absence of metabolic activation (Litton Bionetics, 1978a). Cytotoxicity was observed at doses equal to and greater than 0.16 µL/mL.  Appropriate concurrent negative and positive controls were included and the expected responses were observed.  The results of the alkaline elution test for DNA damage were negative in both the activation and non activation tests.

In vivo Studies

Sponsored substances

CTMS. Groups of 6 male Sprague-Dawley rats/group were administered CTMS in paraffin oil by i.p. injection at 10, 19, 37 and 74 mg/kg bw; only the top three dose groups were used for cytogenetic analysis.  Animals were sacrificed at 6, 24, or 48 hours after injection.  The negative control, paraffin oil, was used in each assay, but the positive control, cyclophosphamide (22 mg/kg) was included only in the 24-hour group (Bioassay Systems, 1982a; Isquith et al., 1988).  Expected responses were observed with appropriate concurrent negative and positive controls.  CTMS did not cause an increase in the frequency of chromosomal breaks or aberrations in bone marrow cells of rats.

C2DMS. Groups of 6 Sprague-Dawley rats/group were administered C2DMS in paraffin oil by i.p. injection at 5, 11, 22, 32, 43, 54 mg/kg bw; only the top four dose groups were used for cytogenetic analysis.  Animals were sacrificed at 6, 24, or 48 hours after injection.  The negative control, paraffin oil, was used in each assay, but the positive control, cyclophosphamide (22 mg/kg) was included only in the 24-hour group (Bioassay Systems, 1981).  Appropriate concurrent negative and positive controls were included and the expected responses were observed.  C2DMS did not cause an increase in the frequency of chromosomal breaks or aberrations in bone marrow cells of rats.

C3MS. Groups of 6 Sprague-Dawley rats/group were administered C3MS in paraffin oil by i.p. injection at 12, 16 and 23 mg/kg bw.  Animals were sacrificed at 6, 24, or 48 hours after injection.  The negative control, paraffin oil, was used in each assay, but the positive control, cyclophosphamide (22 mg/kg) was included only in the 24-hour group (Bioassay Systems, 1982b).  Appropriate concurrent negative and positive controls were included and the expected responses were observed.  C3MS did not cause an increase in the frequency of chromosomal breaks or aberrations in bone marrow cells of rats.

Table 19    Summary of In Vivo Chromosomal Aberrations Studies

| Test substance | Test guideline | Strain/species | Dose | Conclusion | Reference |
| --- | --- | --- | --- | --- | --- |
| CTMS (CAS No. 75-77-4) | (1) | Sprague-Dawley rats | 10, 19*, 37* and 74* mg/kg bw | Negative | Bioassay Systems, 1982a; Isquith, 1988 |
| C2DMS (CAS No. 75-78-5) | (1) | Sprague-Dawley rats | 5, 11, 22*, 32*, 43*, 54* mg/kg bw | Negative | Bioassay Systems, 1981 |
| C3MS (CAS No. 75-79-6) | (1) | Sprague-Dawley rats | 12*, 16* and 23* mg/kg bw | Negative | Bioassay Systems, 1982b |

- Rodent Bone Marrow Cytogenetic Assay as recommended by the Ad Hoc Committee on Chromosome Methodologies in Mutagen Testing (TAP 22: 269-275, 1972) with modifications per the EPA Gene-Tox Program Cytogenetics Committee (12/3 to 12/5, 1980, Washington, D.C.).

- *Dose used for cytogenetic analysis

Conclusion

Gene mutations. CTMS, C2DMS, C3MS and C3ES were negative for mutagenicity in all Salmonella typhimurium and E coli strains tested. CTMS, C2DMS and C3MS were negative for mutagenicity in DNA damage and repair assays and/or mammalian cell gene mutation tests with L5178Y Fisher mouse lymphoma cells. CTMS and C2DMS were also negative in a sister chromatid exchange assay with L5178Y Fisher mouse lymphoma cells; C3MS was positive for  SCEs with and without metabolic activation.

Chromosome aberrations. CTMS and C2DMS were negative in a chromosome aberration test with L5178Y Fischer mouse lymphoma cells; C3MS was considered to have weak clastogenic activity.  CTMS, C2DMS and C3MS were negative in rat in vivo chromosomal abberation test.  Based on the available data, the alkyl chlorosilanes are not expected to be genotoxic.

### 3.1.7	Carcinogenicity

No data available.

### 3.1.8	Toxicity for Reproduction

Toxicity for reproduction data for the alkyl chlorosilanes category is not available; however, data are available for the hydrolysis products.  Toxicity for reproduction data is summarized in Table 23; developmental toxicity data are summarized in Table 24.

Studies in Animals

Effects on Fertility

Sponsored substances

No data available.

Hydrolysis products

TMS. In a combined repeated-dose/reproductive/developmental toxicity screening test(OECD TG 422) described previously in Section 3.1.5, Crl:CD(SD) rats, were exposed via whole-body inhalation to vapor atmospheres of 0, 0.22, 1.10 or 2.20 mg/L TMS, 6 hours/day, 7 days/week . A concurrent control group was exposed to filtered air on a comparable regimen (WIL Research, 2008).  F0 males and reproductive phase females were exposed daily for 14 days prior to mating, throughout the mating period and continuing through the day prior to euthanasia (males) or gestation day 20 (reproductive phase females, for a total of 35-48 consecutive days).  F0 reproductive phase females with no evidence of mating or that failed to deliver were exposed through post-mating or post-cohabitation day 24 (the day prior to euthanasia) for a total of 52 consecutive days.  Individual reproductive phase female body weights and food consumption was recorded.  During the period of expected parturition, the females were observed twice daily for initiation and completion of parturition and for signs of dystocia.  Individual gestation length was calculated using the date delivery started. Females that delivered were euthanized on lactation day 4; the number of former implantation sites and corpora lutea was recorded.  Females that failed to deliver were euthanized on post-mating day 25 (females with evidence of mating) or post-cohabitation day 25 (females with no evidence of mating). Uteri with no macroscopic evidence of implantation were opened and subsequently placed in 10% ammonium sulfide solution for detection of early implantation loss.  Special emphasis was placed on stages of spermatogenesis in the male gonads and histopathology of interstitial cell structure.  There were no remarkable clinical observations noted at any exposure level. Mean body weights, body weight gains and food consumption for reproductive females at all exposure levels were similar to the control group.  Mean mating, fertility and copulation/conception indices for all exposure concentrations were similar to the control group.  The NOAEC for reproductive toxicity of TMS in rats was 2.20 mg/L (nominal; highest concentration tested).

DMSD. In a combined repeated-dose/reproductive/developmental toxicity screening test (OECD TG 422) described previously in Section 3.1.5, DMSD was administered via gavage to Sprague-Dawley rats (10/sex/dose) at 0 (corn oil), 50, 250 and 500 mg/kg bw/day (DCC, 2008c).  Males were treated during pre-mating and mating periods.  Reproductive toxicity group females were treated for 51 days; 2 weeks pre-mating, mating, and through post partum day 3.  General clinical observations were made daily after the exposure period to assess external condition, behaviour and mortality/morbidity.  Detailed clinical observations were conducted weekly and were performed after the exposure period.  Mean body weight and food consumption of dams were recorded weekly.  Duration of gestation, evidence of parturition and parturition difficulties were observed. Each litter was examined to determine the number of fetuses, sex, still births, runts and the presence of any gross abnormalities.  Reproductive females were not subjected to macroscopic examination at necropsy, however the number of corpora lutea and implantation sites was recorded.  No test article-related effects were observed in any of the reproductive parameters evaluated.The NOAEL for reproductive toxicity of DMSD in rats was 500 mg/kg bw/day (highest dose tested).

HCl. No reliable studies have been reported regarding toxicity to reproduction and development in animals after oral, dermal or inhalation exposure to hydrogen chloride/hydrochloric acid.  Because proton and chloride ion are the normal constituents in the body fluid of animal species, lower concentration of hydrogen chloride gas/mist or solution does not seem to cause adverse effects to animals.  In addition, no effects on the gonads were observed in a 90-day inhalation repeated-dose study up to concentrations of 0.073 mg/L.

Table 20     Summary of Reproductive Toxicity Data

| Test substance | Test guideline | Species | Route | Dose Levels | Duration | NOAEL | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Hydrolysis product |  |  |  |  |  |  |  |
| TMS (CAS No. 1066-40-6) | OECD TG 422 | Crl:CD (SD) rats | Whole-body inhalation | 0.22, 1.10 and 2.20 mg/L/day | Up to 45 days | 2.20 mg/L (highest concentration tested) | WIL Research, 2008 |
| DMSD (CAS No. 1066-42-8) | OECD TG 422 | Sprague-Dawley rat | Oral (gavage) | 0, 50, 250, 500 mg/kg bw/day | Up to 45 days | 500 mg/kg bw/d (highest dose tested) | DCC, 2008c |
| HCl (CAS No. 7647-01-0) | 90-day | Crl:CD (SD)Br rat | Inhalation | 0, 0.014, 0.028 and 0.073 mg/L/day | 90 day | No effects on the gonads observed | http://www.chem.unep.ch/irptc/sids/oecdsids/sidspub.html |

Developmental Toxicity

Sponsored substances

No data available.

Hydrolysis products

TMS. In a combined repeated-dose/reproductive/developmental toxicity screening test (OECD TG 422) previously described in Sections 3.1.5 and 3.1.8,  Crl:CD(SD) rats were exposed via whole-body inhalation to vapor atmospheres of 0, 0.22, 1.10 or 2.20 mg/L TMS for 6 hours/day, 7 days/week.  A concurrent control group was exposed to filtered air on a comparable regimen (WIL Research, 2008). On the day parturition was initiated (PND 0), pups were sexed and examined for gross malformations, and the numbers of stillborn and live pups were recorded. Individual gestation length was calculated using the date delivery started.  F1 clinical observations and body weights were recorded on postnatal days (PND) 1 and 4.  Each litter was examined daily for survival, and all deaths were recorded. All pups were individually identified by application of tattoo markings on the digits following completion of parturition.  A daily record of litter size was maintained. Standardization of litter size was not performed.  Pups were individually sexed on PND 0 and 4.  Pups were euthanized on PND 4.  Intact offspring that died were necropsied using a fresh dissection technique including the heart and major vessels and examination of the brain by a mid-coronal slice. If a skeletal anomaly was suspected, the pups were eviscerated, cleared, stained with Alizarin Red S and examined. Cannibalized pups were discarded without necropsy.  Tissues were preserved in 10% neutral-buffered formalin for possible future histopathologic examination only as deemed necessary by the gross findings.  The carcass of each pup was then discarded. Histopathologic examination was deemed not to be necessary.  There were no remarkable clinical observations noted at any exposure level.  Mean body weights, body weight gains and food consumption for reproductive females at all exposure levels were similar to the control group.  There were no test article-related macroscopic findings observed for pups.  The NOAEC for maternal and developmental toxicity in rats was 2.20 mg/L (highest concentration tested).

DMSD. In a combined repeated-dose/reproductive/developmental toxicity screening test (OECD TG 422) described previously in Sections 3.1.5 and 3.1.8, Sprague-Dawley rats (10/sex/dose) were administered DMSD at 0 (corn oil), 50, 250 and 500 mg/kg bw/day via oral gavage (DCC, 2008c).  Detailed clinical observations were conducted weekly and were performed after the exposure period.  Mean body weight and food consumption of dams were recorded weekly.  Each litter was examined to determine the number of fetuses, sex, still births, runts and the presence of any gross abnormalities.  Pups found dead and pups sacrificed on day four postpartum were examined for external gross abnormalities only.  No pup tissues were collected and the carcasses were discarded.  No test article-related effects were observed in any of the developmental parameters evaluated.  The NOAEL for maternal and developmental toxicity of DMSD in rats was 500 mg/kg bw/day (highest dose tested).

HCl. No reliable studies have been reported regarding toxicity to reproduction and development in animals after oral, dermal or inhalation exposure to hydrogen chloride/hydrochloric acid.  Because proton and chloride ion are the normal constituents in the body fluid of animal species, lower concentration of hydrogen chloride gas/mist or solution does not seem to cause adverse effects to animals.  In fact, the cells of gastric glands secrets hydrochloric acid into the cavity of stomach and orally administered sulfuric acid, which result in pH change as well, did not cause developmental toxicity to laboratory animals.  These facts indicate that hydrogen chloride/hydrochloric acid is not expected to have developmental toxicity.

Table 21     Summary of Developmental Toxicity Data

| Test substance | Test guideline | Species | Route | Dose Levels | Duration | NOAEL or NOAEC | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Hydrolysis product |  |  |  |  |  |  |  |
| TMS (CAS No. 1066-40-6) | OECD TG 422 | Crl:CD (SD) rats | Whole-body inhalation | 0.22, 1.10 and 2.20 mg/L/day | Up to 45 days | 2.20 mg/L (highest concentration tested) | WIL Research, 2008 |
| DMSD (CAS No. 1066-42-8) | OECD TG 422 | Sprague-Dawley rat | Oral (gavage) | 0, 50, 250, 500 mg/kg bw/day | Up to 45 days | 500 mg/kg bw/day (highest dose tested) | DCC, 2008c |
| HCl (CAS No. 7647-01-0) | Not expected to have developmental toxicity. |  |  |  |  |  | http://www.chem.unep.ch/irptc/sids/oecdsids/sidspub.html |

Conclusion

No data are available on the reproductive toxicity of alkyl chlorosilanes.  However; data are available for hydrolysis products, TMS (inhalation route), DMSD (oral route) and HCl.

In a combined repeated-dose/reproductive/developmental toxicity screening test (OECD TG 422), no treatment-related effects on reproduction or development were observed in rats exposed whole-body to TMS at concentrations up to 2.20 mg/L.  The NOAEC for reproductive toxicity was 2.20 mg/L (highest concentration tested).  The NOAEC for maternal and developmental toxicity was 2.20 mg/L (highest concentration tested).

In a combined repeated-dose/reproductive/developmental toxicity screening test (OECD TG 422), no treatment-related effects on reproduction or development were observed in rats exposed via oral gavage to DMSD at doses up to 500 mg/kg bw/day.  The NOAEL for reproductive toxicity was 500 mg/kg bw/day (highest dose tested).  The NOAEL for maternal and developmental toxicity was 500 mg/kg bw/day (highest dose tested).

No reliable studies have been reported regarding toxicity to reproduction and development in animals after oral, dermal or inhalation exposure to hydrogen chloride/hydrochloric acid.  However, no effects on the gonads were observed in a 90-day repeated-dose inhalation study.  Based on data for the hydrolysis products, the alkyl chlorosilanes are not expected to be reproductive or developmental toxicants.

# hazards to the environment

## 4.1	Aquatic Effects

Data are not available for the alkyl chlorosilanes; the substances undergo rapid hydrolysis, which occurs during testing; exposures to parent alkyl chlorosilanes are likely to be transient and observed toxicity is likely due its hydrolysis products, HCl and the corresponding silanol hydrolysis product. Data are available for the silanol hydrolysis products TMS and DMSD; data for the remaining silanol hydrolysis products was not located. TMS and DMSD can be used as analogues for CTMS and C2DMS, respectively, because of the rapid hydrolysis of the parent substances.  Aquatic toxicity endpoints for the sponsored substances CTMS and C2DMS are fulfilled through the use of data from HCl, and the corresponding silanol hydrolysis products TMS and DMSD, respectively.

Data are available for the analogous substance with a similar structure (MTMS); these alkylsilanes rapidly hydrolyze to produce three moles of alcohol (methanol or ethanol, respectively) and one mole of the corresponding silanol.  MTMS hydrolyzes to form the same silanol as the sponsored substances C3MS.  Aquatic toxicity endpoints are fulfilled through the use of data from HCl, as well MTMS for sponsored substances C3MS and C3ES.  ; MTMS is expected to be applicable based on structural similarity between the methoxy- and ethoxy- moiety.  The contribution of three moles of corresponding alcohol to the toxicity of the parent silane is expected to be negligible; based on laboratory studies MTMS poses a low hazard to the aquatic environment.

These data are summarized in Table 23.

Acute Toxicity Test Results

Fish

Sponsored substances

No data available.

Analogue substances – similar structure

MTMS. In an OECD TG 203 "Fish, Acute Toxicity Test" groups of ten rainbow trout (Oncorhynchus mykiss) were exposed for 96 hours under flow-through conditions to nominal MTMS concentrations of 0 (negative and solvent controls), 13, 25, 50, 100 and 200 mg/L.  No mortality or abnormal responses were observed.  The 96-hour LC50 value for rainbow trout (Oncorhynchus mykiss) exposed to MTMS was >110 mg/L (measured), the highest concentration tested.

Analogous substances – hydrolysis products

TMS. In an OECD TG 203 "Fish, Acute Toxicity Test", groups of ten rainbow trout (Oncorhynchus mykiss) were exposed to TMS for 96 hours under semi-static conditions to measured concentrations of 0 (negative control), 65, 128, 250, 481 and 949 mg/L (Wildlife International, 2004a).  The 96-hour LC50 value for rainbow trout (Oncorhynchus mykiss) exposed to TMS was 271 mg/L and the NOEC was 128 mg/L.  Following the method of the Environment Federal Bureau Berlin, groups of ten Zebra-fish (Brachydanio rerio) were exposed for 96 hours to TMS under static conditions to 0 or 1000 mg/L in a limit test (nominal; Caspers, 1992).  There was no mortality and abnormal swimming was noted in 2 fish at 1 hour, with recovery by 2 hours.  The 96-hour LC50 value for Zebra-fish (Brachydanio rerio) exposed to TMS was >519 mg/L (measured), the highest concentration tested.

DMSD. In an OECD TG 203 "Fish, Acute Toxicity Test", groups of ten rainbow trout (Oncorhynchus mykiss) were exposed to DMSD for 96 hours under static conditions to nominal concentrations of 0 (negative control) and 120 mg/L (limit test; Drottar and Miller, 2009a).  There was no mortality and no abnormal responses.  The 96-hour LC50 value for rainbow trout (Oncorhynchus mykiss) exposed to TMS was >126 mg/L (measured), the highest concentration tested.

HCl. The hazard of hydrochloric acid for the environment is caused by the proton (pH effect).  For this reason the effect of hydrochloric acid on the organisms depends on the buffer capacity of the aquatic ecosystem.  Also the variation in acute toxicity for aquatic organisms can be explained for a significant extent by the variation in buffer capacity of the test medium.  For example, LC50 values of acute fish toxicity tests varied from 4.92 to 282 mg/L.  Several studies on acute toxicity to fish have been published.  The lowest 96h LC50 reported is pH 4.3 (=4.92 mg/L) for Cyprinus carpio, which was conducted according to OECD TG 203.

Aquatic invertebrates

Sponsored substances

No data available.

Analogue substances – similar structure

MTMS. In an OECD TG 202 study, groups of Daphnia magna were exposed for 48 hours under flow-through conditions to nominal MTMS concentrations of 0 (negative and solvent controls), 13, 25, 50, 100 and 200 mg/L.  The 48-hour EC50 value for Daphnia magna exposed to MTMS was >122 mg/L (measured), the highest concentration tested

Analogous substances – hydrolysis products

TMS. In an OECD TG 202 study, groups of Daphnia magna (10 daphnids/replicate, 2 replicates/concentration) were exposed for 48 hours under semi-static conditions to TMS concentrations of 0 (negative control), 63, 125, 250, 500 and 1000 mg/L (nominal; Wildlife International, 2004b).  After 48 hours of exposure, mortality/immobility in the 60, 116, 207, 447 and 905 mg/L (measured) treatment groups was 30, 65, 45, 80 and 85%, respectively.  At 48 hours, daphnia trapped on the water surface were observed in every group except the negative control and the 60 mg/L treatment group, with more daphnia being trapped at the higher concentrations.  The 48-hour EC50 value for Daphnia magna exposed to TMS was 124 mg/L (measured).

DMSD. In an OECD TG 202 study, groups of Daphnia magna (10 daphnids/replicate, 2 replicates/concentration) were exposed for 48 hours under static conditions to DMSD concentrations of 0 (negative control) and 120 mg/L (nominal; Drottar and Miller, 2009b).  No mortality or immobility was observed.  The 48-hour EC50 value for Daphnia magna exposed to DMSD was >117 mg/L (measured), the highest concentration tested.

HCl. The hazard of hydrochloric acid for the environment is caused by the proton (pH effect).  For this reason the effect of hydrochloric acid on the organisms depends on the buffer capacity of the aquatic ecosystem.  Also the variation in acute toxicity for aquatic organisms can be explained for a significant extent by the variation in buffer capacity of the test medium.  The results conducted according to OECD TG 202 are a 48h EC50 of pH 5.3 (0.492 mg/L) for Daphnia magna.

Algae

Sponsored substances

No data available.

Analogue substances – similar structure

MTMS. In an OECD TG 201 "Algae, Growth Inhibition Test", Pseudokirchneriella subcapitata were exposed under static conditions to MTMS at concentrations of 0 (negative control), 7.5, 15, 30, 60 and 120 mg/L.  EC10, EC20, and EC50 values for Pseudokirchneriella subcapitata exposed to MTMS were estimated to be >120 mg/L (expressed as nominal based on the rapid hydrolysis of the test substance in a static system) for yield and growth rate.  The 72-hour NOEC values for yield and growth rate were 120 mg/L (nominal), the highest concentration tested.

Analogous substances – hydrolysis products

TMS. In an OECD TG 201 "Algae, Growth Inhibition Test", Pseudokirchneriella subcapitata were exposed under static conditions to TMS at concentrations of 0 (negative control), 31, 63, 125, 250, 500 and 1000 mg/L (nominal; Wildlife International, 2004c).  Results are based on mean measured concentrations of 0, 33, 70, 135, 257, 522 and 1057 mg/L.  Based on cell density, the 72-hour EC50 was 555 mg/L.  Based on area under the growth curve (biomass), the 72-hour EbC50 value was 566 mg/L.  Based on growth rate, the 72-hour ErC50 value was > 1053 mg/L.  The 72-hour NOEC, based on cell density, biomass and growth rate was 70 mg/L (measured).  In a study conducted following DIN 38 412 (similar to OECD TG 201 "Algae, Growth Inhibition Test"), Scenedesmus subspicatus (new name: Desmodesmus subspicatus) were exposed under static conditions to TMS at concentrations of 0 (negative control), 10, 31.6, 100, 316, and 1,000 mg/l (nominal) mg/L (Caspers, 1992).  The 72-hour EbC50 value was 806 mg/L (nominal).

DMSD. In an OECD TG 201 "Algae, Growth Inhibition Test", Pseudokirchneriella subcapitata were exposed under static conditions to DMSD at concentrations of 0 (negative control), 7.5, 15, 30, 60, 120 mg/L (Jezowski, 2008).  The EyC50 and ErC50 values for Pseudokirchneriella subcapitata exposed to dimethylsilanediol were estimated to be >118 mg/L, the highest concentration tested.  The 72-hour NOEC values for yield and growth rate were 118 mg/L.

HCl. The hazard of hydrochloric acid for the environment is caused by the proton (pH effect). For this reason the effect of hydrochloric acid on the organisms depends on the buffer capacity of the aquatic ecosystem.  Also the variation in acute toxicity for aquatic organisms can be explained for a significant extent by the variation in buffer capacity of the test medium.  For Selenastrum capricornutum, 72h EC50 and NOEC based on growth rate are pH 5.3 (giving substance concentration 0.492 mg/L) and pH 6.0 (0.097 mg/L), respectively.  This study was conducted according to OECD TG 201.

Table 22     Summary of Acute Aquatic Toxicity

| Test substance | Guideline; Test type | Species | Result (mg/L) | Reference |
| --- | --- | --- | --- | --- |
| Fish |  |  |  |  |
| Analogous substance – similar structure |  |  |  |  |
| MTMS | OECD TG 203; flow-through | Rainbow trout (Oncorhynchus mykiss) | 96-hr LC50 >110 (measured) 96-h LC50 >62 (estimated methylsilanetriol concentration) | http://webnet.oecd.org/hpv/ui/Search.aspx. |
| MTMS | ECOSAR | Fish | 96-hr LC50 = 45 (alkoxy silane category) 96-hr LC50 = 9130 (neutral organic category) | USEPA, 2010 |
| Analogous substance – hydrolysis product |  |  |  |  |
| TMS | OECD TG 203; semi-static | Rainbow trout (Oncorhynchus mykiss) | 96-hr LC50 = 271(measured) | Wildlife International, 2004a |
| TMS | Environment Federal Bureau Berlin; static | Zebra-fish (Brachydanio rerio) | 96-hr LC50 >519 (measured) | Caspers, 1992 |
| TMS | ECOSAR | Fish | 96-hr LC50 = 395 | USEPA, 2010 |
| DMSD | OECD TG 203; static | Rainbow trout (Oncorhynchus mykiss) | 96-hr LC50 >126 (measured) | Drottar and Miller, 2009a |
| DMSD | ECOSAR | Fish | 96-hr LC50 = 9320 | USEPA, 2010 |
| Ethylsilanetriol | ECOSAR | Daphnid | 48-hr EC50 = 13,000 | USEPA, 2010 |
| Methylsilane-triol | ECOSAR | Daphnid | 48-hr EC50 = 1000 | USEPA, 2010 |
| HCl | OECD TG 203; semi-static | Rainbow trout (Oncorhynchus mykiss) | 96-hr LC50 = pH 4.3 (4.92 mg/L) | http://webnet.oecd.org/hpv/ui/Search.aspx. |
| Aquatic invertebrates |  |  |  |  |
| Analogous substance – similar structure |  |  |  |  |
| MTMS | OECD TG 202; flow-through | Daphnia magna | 48-hr EC50>122 (measured) 48-h EC50 >57 (estimated methylsilanetriol concentration) | http://webnet.oecd.org/hpv/ui/Search.aspx. |
| MTMS | ECOSAR | Daphnid | 96-hr LC50 = 16 (alkoxy silane category) 96-hr LC50 = 7990 (neutral organic category) | USEPA, 2010 |
| Analogous substance – hydrolysis product |  |  |  |  |
| TMS | OECD TG 202; semi-static | Daphnia magna | 48-hr EC50 = 124 (measured) | Wildlife International, 2004b |
| TMS | ECOSAR | Daphnid | 48-hr EC50 = 196 | USEPA, 2010 |
| DMSD | OECD TG 202; static | Daphnia magna | 48-hr EC50 >117 (measured) | Drottar and Miller, 2009b |
| DMSD | ECOSAR | Daphnid | 48-hr EC50 = 3680 | USEPA, 2010 |
| Ethylsilanetriol | ECOSAR | Daphnid | 48-hr EC50 = 7700 | USEPA, 2010 |
| Methylsilane-triol | ECOSAR | Daphnid | 48-hr EC50 = 48,000 | USEPA, 2010 |
| HCl | OECD TG 202 | Daphnia magna | 48-hr EC50 = pH 5.3 (0.492 mg/L) | http://webnet.oecd.org/hpv/ui/Search.aspx. |
| Algae |  |  |  |  |
| Analogous substance – similar structure |  |  |  |  |
| MTMS | OECD TG 201; static | Pseudokirchneriella subcapitata | 72-hr EC50>120 (nominal) 72-hr EC50>3.6 (measured); 72-hr EC50>80 (estimated methylsilanetriol concentration) | http://webnet.oecd.org/hpv/ui/Search.aspx. |
| Analogous substance – hydrolysis product |  |  |  |  |
| TMS | OECD TG 201; static | Pseudokirchneriella subcapitata | 72-hr EC50 = 555 mg/L (measured) | Wildlife International, 2004c |
| TMS | Similar to OECD TG 201; static | Desmodesmus subspicatus | 72-hr EbC50 = 806 mg/L (nominal) | Caspers, 1992 |
| TMS | ECOSAR | Green Algae | 96-hr EC50 = 64 | USEPA, 2010 |
| DMSD | OECD TG 201; static | Pseudokirchneriella subcapitata | 72-hr EyC50 and ErC50 >118 | Jezowski, 2008 |
| DMSD | ECOSAR | Green Algae | 96-hr EC50 = 615 | USEPA, 2010 |
| Ethylsilanetriol | ECOSAR | Green Algae | 96-hr EC50 = 5981 | USEPA, 2010 |
| Methylsilane-triol | ECOSAR | Green Algae | 96-hr EC50 = 577 | USEPA, 2010 |
| HCl | OECD TG 201; static | Selenastrum capricornutum | 72-hr EC50 =pH 5.3 (0.492 mg/L) | http://webnet.oecd.org/hpv/ui/Search.aspx. |

Chronic Toxicity Test Results

Algae

Analogous substances – hydrolysis products

TMS. In a study conducted following Algal Assay Procedure: Bottle Test published in the Environmental Protection Agency’s Manual, “Biological Field and Laboratory (EPA-67014-73-00), Anabaena flos-aquae were exposed under static conditions to TMS at concentrations of 0, 10, 100, 500, and 1000 mg/L for 14 days (Annelin, 1976a).  The 14 day NOEC was 1000 mg/L (nominal).

DMSD. In a study conducted following Algal Assay Procedure: Bottle Test published in the Environmental Protection Agency’s Manual, “Biological Field and Laboratory (EPA-67014-73-00), Anabaena flos-aquae were exposed under static conditions to DMSD at concentrations of 0, 100, 500, 1000, and 2000 mg/L for 14 days (Annelin, 1976b).  The 14 day NOEC was 2000 mg/L (nominal).

Chronic Toxicity Estimated Results - Hydrolysis Products

Environmental toxicity values for hydrolysis products can be estimated from EPIWIN although the validity of these values is unknown.

TMS. EPIWIN predicts the following chronic toxicity values for trimethylsilanol:

Fish	14-day LC50 = 394 mg/L

Fish	30-day ChV = 38.9 mg/L;  (ChV = geometric mean of predicted NOEC and LOEC)

Daphnid ChV = 19.9 mg/L

Green Algae ChV = 21.3 mg/L

DMSD.  EPIWIN predicts the following chronic toxicity values for dimethylsilanediol:

Fish	14-day LC50 = 9070 mg/L

Fish	30-day ChV = 827 mg/L;

Daphnid ChV = 292 mg/L

Green Algae ChV = 157 mg/L

Methylsilanetriol. EPIWIN predicts the following chronic toxicity values for methylsilanetriol:

Fish	14-day LC50 = 472,000 mg/L

Fish	30-day ChV = 39,000 mg/L;

Daphnid ChV = 8600 mg/L

Green Algae ChV = 1950 mg/L

Ethylsilanetriol. EPIWIN predicts the following chronic toxicity values for ethylsilanetriol:

Fish	14-day LC50 = 200,000 mg/L

Fish	30-day ChV = 17,000 mg/L

Daphnid ChV = 4250 mg/L

Green Algae ChV = 1200 mg/L

## 4.2	Terrestrial Effects

No data available.

Conclusion

These chlorosilanes exhibit very rapid hydrolysis in the environment, hence any ecotoxicity will result from their hydrolysis products.   Structural analogues of the parent chlorosilane compounds, and silanol hydrolysis products show low toxicity.  Another byproduct of the hydrolysis of these chlorosilanes, HCl, has properties that can result in toxicity of < 1 mg/L to aquatic organisms in poorly buffered systems, mainly due to acidification of the test medium.  These sponsored chlorosilanes and their hydrolysis products have low potential for bioaccumulation.  Silanol degradation products are not biodegradable.  Adequate screening-level data are available to characterize the environmental hazard for the purposes of the OECD HPV Programme.

# references

Ames, B.N., McCann, J. and Yamasaki, E. (1975) Methods for detecting carcinogens and mutagens with the Salmonella/mammalian-microsome mutagenicity test. Mutation Research 31, 347-364.

Annelin, R. (1976a) Algal Assay: Bottle Test: Trimethylsilanol derived from Trimethylethoxysilane.  Dow Corning Corporation.  Report No.  1976-I0005-0447.

Annelin, R. (1976b) Algal Assay: Bottle Test: Dimethylsilanediol derived from Dimethyldiethoxysilane.  Dow Corning Corporation.  Report No.  1976-I0005-0436.

Bayer AG Fachbereich Toxicology (1988) Study Number: T 4027186; Bayer AG Internal Report Number: 17233.

Bayer AG, Goldschmidt, and Wacker-Chemie (1991)  Silicones: Chemistry and Technology.  CRC Press, Boca Raton.  pp. 23-25.

Bazant, V., V. Chvalovsky, and J. Rathousky (1965)  Organosilicon Compounds.  Academic Press, New York and London.  pp. 40-75.

Bioassay Systems Corporation (1981) Project No. 10367, Dow Corning internal report number 1991-I0000-36073, 1981.

Bioassay Systems Corporation (1982a) Project No.10367-4; Industrial Health Foundation.

Bioassay Systems Corporation (1982b) Project No.10367-3; Industrial Health Foundation.

Blankinship, A.S.; Kendall, T.Z.; Krueger, H.O. (2004) Wildlife International, Ltd Trimethylsilanol: A 96-Hour Static-Renewal Acute Toxicity Test with The Rainbow Trout (Oncorhynchus mykiss) Wildlife International, Ltd. Report No.  570A-105A.

Bushy Run Research Center (1981) Organochlorosilane A-154: Acute Toxicity and Primary Irritancy Studies, Bushy Run Research Center, Project Report 44-128, December 21, 1981.

Bushy Run Research Center (1982) Organochlorosilane A-161 Acute Toxicity and Primary Irritancy Studies, Bushy Run Research Center, Project Report 45-1, February 26, 1982.

Caspers (1992) Untersuchungen Zum Ökologischen Verhalten Von Trimethylsilanol.  Bayers AG  Report date  1992-08-03.

Chandra, G. (Editor) (1997) Organosilicon Materials; The Handbook of Environmental Chemistry Volume 3 Anthropogenic Compounds, Part H. Springer.

Clarke, N. (2008) Trimethylsilanol (CAS No.1066-40-6): Assessment of Ready Biodegradability: CO2 in Sealed Vessels (CO2 Headspace Test); Final Study Report to the Centre Européen des Silicones (CES).  Harlan Laboratories Ltd: Derbyshire, UK.  Report number 2581/0002.

Design Institute for Physical Property Data (DIPPR) (1946, 1947, 1950, 1948, 1954, 1986, 1987, 1988, 1990, 2001) American Institute of Chemical Engineers (AIChE), New York, N.Y., U.S.A.

Dow Corning Corporation (1987) Internal Report No. 1987-I0005-1665.

Dow Corning Corporation (1989) The Acute Inhalation Toxicity of Methyltrichlorosilane in Rats.  Report number 1989-I0005-2534

Dow Corning Corporation (1992) Report No. 1992-I0000-37420.

Dow Corning Corporation (1993) Internal Report No. 1993-I0000-38257.

Dow Corning Corporation (1997a) Internal Report No. 1997-I0000-43537.

Dow Corning Corporation  (1997b) Internal Report No. 1997-I0000-43381.

Dow Corning Corporation (1997c) Internal Report No.: 1997-I0000-43174.

Dow Corning Corporation (1999) Internal Report No. 1999-I0000-47271.

Dow Corning Corporation (2007) Report No.  2007-I0000-58054.

Dow Corning Corporation (2008a) In Vitro Percutaneous Absorption of 14C-trimethylsilanol (14C-TMS) Through Human Skin.  Dow Corning Corporation.  Report No.  10373-102.

Dow Corning Corporation (2008b) In Vitro Percutaneous Absorption of 14C-Dimethylsilanediol (14C-DMSD) Applied in Aqueous Solution to Human Skin. Dow Corning Corporation Testing laboratory Report No.  10374-102.

Dow Corning Corporation (2008c) Combined Repeated Dose Toxicity Study with the Reproductive/Developmental Toxicity Screening Test for Dimethylsilanediol (DMSD) in Sprague-Dawley Rats via Oral Gavage.  Dow Corning Corporation Report No.  10872-102.

Drottar, K.R. and J.A. Miller (2009a) Dimethylsilanediol (CAS Number 1066-42-8): Acute Toxicity to the Rainbow Trout (Oncorhynchus mykiss) Under Static Test Conditions. Dow Corning Corporation Health and Environmental Sciences Report No.  10892-102. Sponsored by Silicones Environmental Health and Safety Council (SEHSC).

Drottar, K.R. and J.A. Miller (2009b) Dimethylsilanediol (CAS Number 1066-42-8): Acute Toxicity to the Cladoceran (Daphnia magna) Under Static Test Conditions. Dow Corning Corporation Health and Environmental Sciences Report No.  10893-102. Sponsored by Silicones Environmental Health and Safety Council (SEHSC).

Infracor Chemistry Services (2002a) Determination of the Melting Temperature and Boiling Temperature of Dynasylan OCTCS. Final Report AN-ASB 0222.1/0222.2 Degussa AG-Nr.: 2002-0013-DGP. August 2, 2002.

Infracor Chemistry Services (2002b) Determination of the Relative Density of Dynasylan OCTCS. Final Report AN-ASB 0222.3 Degussa AG-Nr.: 2002-0015-DGP August 2, 2002.

Isquith, A., Slesinski, R., and D. Matheson (1988) Genotoxicity Studies on Selected Organosilicon Compounds: In Vivo Assays. Food Chem. Toxic. 26(3):263-266.

Jean, P.A., Gallavan, R.H., Kolesar, G.B., Siddiqui, W.H., Oxley, J.A., and Meeks, R.G. (2006) Chlorosilane Acute Inhalation Toxicity and Development of an LC50 Prediction Model. Dow Corning Corporation, Midland, Michigan, USA. Inhalation Toxicology, 18:515–522, 2006 Copyright Taylor and Francis Group, LLC.

Jezowski, R.L. (2008) Dimethylsilanediol (CAS Number 1066-42-8): Freshwater Alga, Pseudokirchneriella subcapitata, Growth Inhibition Test. Dow Corning Corporation Health and Environmental Sciences Report no.  10971-102. Sponsored by Silicones Environmental Health and Safety Council (SEHSC).

Guerrero, T. (2010) Senior SEHSC Scientific Programs Manager. Correspondence with SEHSC member companies.

Litton Bonetics, Inc. (1978a) Project No. 20893, Industrial Health Foundation Final Report, May 1978.

Litton Bonetics, Inc. (1978b) Project No. 20893, Industrial Health Foundation Final Report, July 1978.

Litton Bonetics, Inc. (1979) Project No. 20893, Industrial Health Foundation Final Report, June 1979.

McCann, J., Choi, E., Yamasaki, E. and Ames, B.N. (1975). Detection of carcinogens as mutagens in the Salmonella/microsome test Assay of 300 chemicals. Proc. Nat. Acad. Sci. 72, 5135-5139.

Miller, J. A.; Kozerski, G. E. (2001) "Hydrolysis Screening Studies of HPV Chlorosilanes"; final report submitted to the Silicones Environmental, Health and Safety Council of North America (SEHSC); Dow Corning Corporation Technical Report No. 2001-I0000-50823.

Miller, J.A.  (2009) Quantitation of Dimethylsilanediol in Test Media for the Rainbow Trout (Oncorhynchus mykiss), During an Acute Toxicity Study Under Static Conditions, Appendix C--Contributing Scientist Report contained in Drottar, K.D. (2009) Dimethylsilanediol (CAS Number 1066-42-8): Acute Toxicity to the Rainbow Trout (Oncorhynchus mykiss) Under Static Test Conditions.  Testing laboratory Report no.  Appendix C in Study No. 10892-102.

Mortelmans, K., Haworth, S., Lawlor, T., Speck, W., Tainer, B., and Zeiger, E. Salmonella

mutagenicity tests. II. Results from the testing of 270 chemicals Environ. Mutagen. Vol. 8 (Suppl 7)

(1986)1-119.http://ntp.niehs.nih.gov/index.cfm?objectid=BD5E2185-123F-7908-7B22C9902D10C6D8

Baker, M.V. and Watling, J.D. (2004).  NMR study of the hydrolysis and oligomerization of alkyltrichlorosilanes in silanizing solutions used to prepare alkylsiloxane self-assembled monolayers. J. Sol-Gel Science and Technology, 30, 101-115.

Noll, W.  (1968)  Chemistry and Technology of Silicones.  Academic Press, New York and London.  pp. 94-95, 208-219.

Pharma Research (1987) Toxicology and Pathology study no. 87.0447, Report No. 87.0983, conducted for Hoechst AG.

Ramm, W.  (1985) Trimethylsilanol: Untersuchung Zur Akuten Oralen Toxizität An Männlichen Und Weiblichen Wistar-Ratten.  Bayer AG .

Ramm, W. and Bomhard, E.  (1987)  Trimethylsilanol: Subakute Toxikologische Untersuchungen An Ratten (Versuch Mit Magensondeapplikation Über 4 Wochen). Bayer AG.  Report date 1987-12-14.

Sabourin CL. 1996. Study of the  Biodegradation of Dimethylsilanediol and Methylsilanediol in Soil. Final Report, June 24, 1996. General Electric, Corporate Research and Development, Schenectady, NY.

Suberg, H. (1985) Trimethylsilanol: Prüfung Auf Primäre Reizende/Ätzende Wirkung Am Kaninchenauge,  Bayer AG  1985-07-25.

Suberg, H. (1986) Trimethylsilanol: Prüfung Auf Primäre Reizende/Ätzende Wirkung An Der Kaninchenhaut, Bayer AG 1986-01-07.

Tomanek, A.  (1990)  Silicones and Industry.  Hauser, Munich.  pp. 20-23.

USEPA (2009) Interim acute exposure guideline levels (AEGLs) for 26 selected chlorosilanes, available at:  http://www.epa.gov/oppt/aegl/pubs/chlorosilanes_interim_apr_2009.v2.pdf

USEPA (2008) Estimations Programs Interface SuiteTM for Microsoft® Windows, v [4.0].  United States Environmental Protection Agency, Washington, DC, USA.

Wacker Chemie (1987) Pharma Reasearch Toxicology and Pathology; Hoechst Aktiengesellschaft; Silane M3 Study of the Mutagenic Potential in Strains of Salmonella typhimurium (Ames Test) and Escherichia coli; Study No.87.0388.

Wacker Chemie (2002) Laboratory of Pharmacology and Toxicology KG; Mutagenicity study of A1 in the Salmonella Typhimurium Reverse Mutation Assay (In Vitro); Study No. 15425/46/02.

WIL Research Laboratory (2008)   A Whole-body Inhalation Repeated Dose Toxicity Study with the Reproductive/Developmental Toxicity Screening Test of Trimethylsilanol in Rats. WIL 401009; 2008-11-21.

Wildlife International, Ltd.  (2004a)  Trimethylsilanol: A 96-Hour Static-Renewal Acute Toxicity Test with the Rainbow Trout (Oncorhynchus mykiss).  Study No. 570A-105. April 19, 2004.

Wildlife International, Ltd.  (2004b)  Trimethylsilanol: A 48-Hour Static-Renewal Acute Toxicity Test with the Cladoceran (Daphnia magna).  Study No. 570A-104. June 2, 2004.

WIL Research Laboratory (2007) Trimethylsilanol:  Acute whole body inhalation toxicity study.

Wildlife International, Ltd.  (2004c)  Trimethylsilanol: A 96-Hour Toxicity Test with the Freshwater Alga (Selenastrum capricornutum).  Study No. 570A-106. April 22, 2004.

Zeiger, E., Anderson, B., Haworth, S., Lawlor, T., and Mortelman, K. Salmonella

Mutagenicity Tests V. Results from the Testing of 311 Chemicals Environ. Molec. Mutagen. Vol. 19 (Suppl 21) (1992) 2-141.http://ntp.niehs.nih.gov/index.cfm?objectid=BD5E2185-123F-7908-7B22C9902D10C6D8)