SIDS Dossier

OECD HPV Chemical Programme, SIDS Dossier approved at SIAM 19, 19-22 October 2004

Template: But-1-ene LOAT

| UUID |  | IUC5-783091fd-1f88-4250-bf05-c4a233ed38af |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | Jenny Odum / LOA Reach Services Team / Tervuren / Belgium |
| Date |  | 2010-09-14 12:40:48 BST |
| Remarks |  | Renamed endpoint study record 7.6.1 Genetic toxicity in vitro |

General information

| Template name | But-1-ene LOAT |
| --- | --- |
| Type of template | inherit |
| Remarks | The content of this dossier is designed to meet the requirements of Regulation (EC) No 1907/2006. This requires that information regarding the objectives, methods, results and conclusions from published and unpublished sources be presented in sufficient detail so as to make independent assessment of the original material unnecessary. This information is presented here solely for the purpose of compliance with the Regulation. There is no intention to infringe the copyright of the sources contributing to this work. |
| EU: REACH |  |
| Legal entity | LOA Reach Services Team / Tervuren / Belgium |

4 Physical and chemical properties

4.1 Appearance/physical state/colour

Endpoint study record: Appearance/physical state/colour. Key study - CRC 2008

| UUID |  | IUC5-0018e7c2-45cc-426a-abc8-6acbaa232a1e |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V1.0 Plug-In |
| Date |  | 2009-11-27 11:08:31 GMT |
| Remarks |  | Successfully migrated to IUCLID 5.2 format. |

Administrative Data

| EU: REACH |  |  |  |
| --- | --- | --- | --- |
| Purpose flag | key study; robust study summary; used for MSDS |  |  |
| Study result type | experimental result |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Non-GLP, non-guideline, Information from published handbook, acceptable with restrictions. |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| review article or handbook | Lide D (Ed.) | 2008 | CRC Handbook of Chemistry and Physics, 89th Edition. | CRC Press Inc. Boca Raton. USA. |  |  |  |  |  |

Data access

data published

Materials and methods

Principles of method if other than guideline

The use of secondary sources of data is acceptable when they are based on a critical evaluation of peer-reviewed data and a consequent selection of a reliable and representative value for the property under investigation. The introduction to the CRC Handbook of Chemistry and Physics states that data for physical constants have been taken from many sources, including both compilations and the primary literature. Where conflicts were found, the value deemed most reliable was chosen. Therefore, although the method used is not known, the values presented here are acceptable as they are from a reliable secondary source of phys chem. data.

GLP compliance

no data (No data reported)

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 106-98-9 |

Details on test material

No data reported

Confidential details on test material

No data reported

Results and discussion

Physical state at 20ºC and 1013 hPa

gaseous

Colour

colourless gas

Odour

other: No data reported

Substance type

organic

Applicant's summary and conclusion

Conclusions

But-1-ene is a colourless gas

Executive summary

But-1-ene is a colourless gas. No information on the test method used is available in the handbook. However, this information is taken from a reliable peer reviewed handbook and so can be considered reliable and suitable for use as the key study for this endpoint.

4.2 Melting point/freezing point

Endpoint study record: Melting point/freezing point.Key study-CRC 2008

| UUID |  | IUC5-ed498aa5-dd05-4641-9d5d-11f460c427b6 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V1.0 Plug-In |
| Date |  | 2009-11-27 11:07:57 GMT |
| Remarks |  | Successfully migrated to IUCLID 5.2 format. |

Administrative Data

| EU: REACH |  |  |  |
| --- | --- | --- | --- |
| Purpose flag | key study; robust study summary; used for MSDS |  |  |
| Study result type | experimental result | Study period | Up to 2008 |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Non-GLP, non-guideline, Information from published handbook, acceptable with restrictions. |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| review article or handbook | Lide D (Ed.) | 2008 | CRC Handbook of Chemistry and Physics, 89th Edition. | CRC Press Inc. Boca Raton. USA. |  |  |  |  |  |

Data access

data published

Materials and methods

Type of method

other: Not reported

Principles of method if other than guideline

The use of secondary sources of data is acceptable when they are based on a critical evaluation of peer-reviewed data and a consequent selection of a reliable and representative value for the property under investigation. The introduction to the CRC Handbook of Chemistry and Physics states that data for physical constants have been taken from many sources, including both compilations and the primary literature. Where conflicts were found, the value deemed most reliable was chosen. Therefore, although the method used is not known, the values presented here are acceptable as they are from a reliable secondary source of phys chem. data.

GLP compliance

no data (No data reported)

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS name | 106-98-9 |

Details on test material

No data reported

Confidential details on test material

No data reported

Results and discussions

Melting / freezing point

| Melt./Freez. pt. | Atm. pressure | Decomposition | Decomp. temp. | Sublimation | Subl. temp. | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| -185 °C |  |  |  |  |  | At standard temperature and pressure |

Applicant's summary and conclusion

Conclusions

The melting point of but-1-ene is -185˚C

Executive summary

No information on the test method used is available in the handbook. However, this information is taken from a reliable peer reviewed handbook and so can be considered reliable and suitable for use as the key study for this endpoint. The melting point of but-1 -ene is therefore taken as -185 ˚C

4.3 Boiling point

Endpoint study record: Boiling point.Key study- CRC 2008

| UUID |  | IUC5-e6437585-1011-4c54-9ec9-0d0eb1ec2b71 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V1.0 Plug-In |
| Date |  | 2010-02-24 14:24:01 GMT |
| Remarks |  | Successfully migrated to IUCLID 5.2 format. |

Administrative Data

| EU: REACH |  |  |  |
| --- | --- | --- | --- |
| Purpose flag | key study; robust study summary; used for MSDS |  |  |
| Study result type | experimental result | Study period | Up to 2008 |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Non-GLP, non-guideline, Information from published handbook, acceptable with restrictions. |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| review article or handbook | Lide D (Ed.) | 2008 | CRC Handbook of Chemistry and Physics, 89th Edition. | CRC Press Inc. Boca Raton. USA. |  |  |  |  |  |

Data access

data published

Materials and methods

Type of method

other: No data reported

Principles of method if other than guideline

The use of secondary sources of data is acceptable when they are based on a critical evaluation of peer-reviewed data and a consequent selection of a reliable and representative value for the property under investigation. The introduction to the CRC Handbook of Chemistry and Physics states that data for physical constants have been taken from many sources, including both compilations and the primary literature. Where conflicts were found, the value deemed most reliable was chosen. Therefore, although the method used is not known, the values presented here are acceptable as they are from a reliable secondary source of phys chem. data.

GLP compliance

no data (No data reported)

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 106-98-9 |

Details on test material

No data reported

Confidential details on test material

No data reported

Results and discussions

Boiling point

| Boiling pt. | Atm. pressure | Decomposition | Decomp. temp. | Remarks |
| --- | --- | --- | --- | --- |
| -6.26 | 1013 hPa |  |  | At standard temperture and pressure |

Any other information on results incl. tables

No further information given.

Applicant's summary and conclusion

Conclusions

The boiling point of but-1-ene is -6.26˚C

Executive summary

No information on the test method used is available in the handbook. However, this information is taken from a reliable peer reviewed handbook and so can be considered reliable and suitable for use as the key study for this endpoint. The boiling point of but-1-ene is -6.26˚C

4.4 Density

Endpoint study record: Density Key study- CRC 2008

| UUID |  | IUC5-26d2c2b2-6c89-4124-a356-4516ea8b464d |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V1.0 Plug-In |
| Date |  | 2009-07-02 11:07:55 BST |
| Remarks |  | Successfully migrated to IUCLID 5.2 format. |

Administrative Data

| EU: REACH |  |  |  |
| --- | --- | --- | --- |
| Purpose flag | key study; robust study summary |  |  |
| Study result type | experimental result | Study period | Up to 2008 |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Non-GLP, non-guideline, Information from published handbook, acceptable with restrictions. |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| review article or handbook | Lide D (Ed.) | 2008 | CRC Handbook of Chemistry and Physics, 89th Edition. | CRC Press Inc. Boca Raton. USA. |  |  |  |  |  |

Data access

data published

Materials and methods

Type of method

other: Not reported

Principles of method if other than guideline

The use of secondary sources of data is acceptable when they are based on a critical evaluation of peer-reviewed data and a consequent selection of a reliable and representative value for the property under investigation. The introduction to the CRC Handbook of Chemistry and Physics states that data for physical constants have been taken from many sources, including both compilations and the primary literature. Where conflicts were found, the value deemed most reliable was chosen. Therefore, although the method used is not known, the values presented here are acceptable as they are from a reliable secondary source of phys chem. data.

GLP compliance

no data (Not reported)

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 106-98-9 |

Details on test material

No data reported

Confidential details on test material

No data reported

Results and discussion

Density

| Type | Density | Temp. | Remarks |
| --- | --- | --- | --- |
| density | 0.59 g/cm³ | 25 °C |  |

Applicant's summary and conclusion

Conclusions

The density of but-1-ene is 0.588 g.cm-3

Executive summary

No information on the test method used is available in the handbook. However, this information is taken from a reliable peer reviewed handbook and so can be considered reliable and suitable for use as the key study for this endpoint. The density of but-1 -ene is therefore 0.588 g.cm-3.

4.7 Partition coefficient

Endpoint study record: Partition coefficient.Key study-Hansch 1995

| UUID |  | IUC5-f55aac5d-c7db-428a-bac6-7d43e3f8586b |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V1.0 Plug-In |
| Date |  | 2010-02-24 14:24:47 GMT |
| Remarks |  | Successfully migrated to IUCLID 5.2 format. |

Administrative Data

| EU: REACH |  |  |  |
| --- | --- | --- | --- |
| Purpose flag | key study; robust study summary |  |  |
| Study result type | experimental result | Study period | Up to 1995 |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Non-GLP, non-guideline, Information from published handbook, acceptable with restrictions. |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| review article or handbook | Hansch, C. Et al. | 1995 | Exploring QSAR: Hydrophobic, Electronic, and Steric Constants. | American Chemical Society, Washington DC. |  |  |  |  |  |

Data access

data published

Materials and methods

Partition coefficient type

octanol-water

Type of method

shake-flask method (No further details reported)

Principles of method if other than guideline

The use of secondary sources of data is acceptable when they are based on a critical evaluation of peer-reviewed data and a consequent selection of a reliable and representative value for the property under investigation. The introduction to Exploring QSAR, Hydrophobic, Electronic and Steric Constants states that it is an extensive compilation of published data. Each record is evaluated and the preferred value is identified for each substance. Therefore, although the method used is not known, the values presented here are acceptable as they are from a reliable secondary source of phys chem. data

GLP compliance

no data (No data reported)

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 106-98-9 |

Details on test material

No data reported

Confidential details on test material

No data reported

Details on methods

No data reported

Results and discussions

Partition coefficient

| Type | Partition coefficient | Temp. | pH | Remarks |
| --- | --- | --- | --- | --- |
| log Pow | 2.4 |  |  |  |

Details on results

No further information provided

Any other information on results incl. tables

No further information provided.

Applicant's summary and conclusion

Conclusions

The partition coefficient (Log Pow) of but-1-ene is 2.4

Executive summary

The value cited by the authors is a measured and preferred value. Little information is given on the methods used. However, this information is taken from a reliable peer reviewed handbook and so can be considered reliable and suitable for use as the key study for this endpoint. The partition coefficient (Log Pow) ofbut-1-ene is 2.4

4.8 Water solubility

Endpoint study record: Water solubility. Key study-McAuliffe 1966

| UUID |  | IUC5-1d074d9c-4e58-4019-b132-4b7a3ddd15bb |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V1.0 Plug-In |
| Date |  | 2010-02-24 14:25:23 GMT |
| Remarks |  | Successfully migrated to IUCLID 5.2 format. |

Administrative Data

| EU: REACH |  |  |  |
| --- | --- | --- | --- |
| Purpose flag | key study; robust study summary |  |  |
| Study result type | experimental result |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Non-GLP; non-guideline; experimental study; predates implementation of GLP and development of study guidelines but otherwise acceptable for assessment. |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | McAuliffe C. J | 1966 | Solubility in water of paraffin, cycloparaffin, oldfin, acetylene, cyclo-olefin and aromatic hydrocarbons | J Phys Chem 70, 1267-75 |  |  |  |  |  |

Data access

data published

Materials and methods

Type of method

analysis

Principles of method if other than guideline

But-1-ene at one atmosphere was added to a glass equilibraton bottle filled with distilled water by displacing water until the volume of the aqueous phase was 3/4 of the volume of the bottle. The gas was maintained over the water. The test solution was stirred the test solution at 25 +-1.5 °C. A rubber balloon in the line from the gas cylinder to the bottle served as a resevoir and was remained in-line until the aqueous phase was sampled. The 3/4 full bottle was shaken for 5 to 10 minutes and then allowed to stand at least 30 minutes prior to analysis. Aqueous samples were taken using a Hamilton syringe and transferred to the gas chromatograph (GC). Analysis was by gas chromatograph (GC) with a hydrogen-flame ionization detecter (Beckman). The chromatographic column was 12 ft. x 0.25 in., stainless steel tubing packed with 25% SE 30 gum rubber on 30-60 mesh firebrick. Helium flow through the column was 65cc/min. Aqueous samples were taken from the equilibration bottle through a septum in tubing at the base of the bottle using a Hamilton syringe and transferred to the GC. Aqueous phase concentrations of but-1-ene were determined against a standard curve prepared from the gas sample used in the aqueous equilibration procedure

GLP compliance

no data (No data reported)

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 106-98-9 |

Details on test material

No data reported

Confidential details on test material

No data reported

Details on methods

But-1-ene at one atmosphere was added to a glass equilibraton bottle filled with distilled water by displacing water until the volume of the aqueous phase was 3/4 of the volume of the bottle. The gas was maintained over the water. A rubber balloon in the line from the gas cylinder to the bottle served as a resevoir and was remained in-line until the aqueous phase was sampled. The 3/4 full bottle was shaken for 5 to 10 minutes and then allowed to stand at least 30 minutes prior to analysis. Aqueous samples were taken using a Hamilton syringe and transferred to the gas chromatograph (GC). Analysis was by gas chromatograph (GC) with a hydrogen-flame ionization detecter (Beckman). The chromatographic column was 12 ft. x 0.25 in., stainless steel tubing packed with 25% SE 30 gum rubber on 30-60 mesh firebrick. Helium flow through the column was 65cc/min. Aqueous samples were taken from the equilibration bottle through a septum in tubing at the base of the bottle using a Hamilton syringe and transferred to the GC. Aqueous phase concentrations of but-1-ene were determined against a standard curve prepared from the gas sample used in the aqueous equilibration procedure

Results and discussions

Water solubility

| 0.222 g/L |  |
| --- | --- |
| Temp. | 25 °C |

Details on results

It should be noted that in this test equilibrium between the gas and water phase was achieved in a closed system. In reality e.g. in surface water or in an aquatic test system which is not closed, solubility could be less.

Any other information on results incl. tables

The gas chromatograph technique of measuring concentration of dissolved hydrocarbons in water has a principle advantage in that impurities do not interfere in the result, except in instances where the impurity has the same relative retention time on the column as the hydrocarbon being measured.

Applicant's summary and conclusion

Interpretation of results

moderately soluble (100-1000 mg/L)

Conclusions

The water solubility of But-1-ene is 222±10 m g/l at 25˚C

Executive summary

The water solubility of But-1-ene is 222±10 mg/l at 25˚C. The study predates the implementation of GLP and standard guidelines. However, the method is described in detail and is considered appropriate for determining the solubility of a gaseous substance.

4.12 Auto flammability

Endpoint study record: Auto flammability. Key study-CRC 2008

| UUID |  | IUC5-c4735e8e-7055-470d-87a7-516a1bb5b905 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V1.0 Plug-In |
| Date |  | 2010-01-04 13:31:52 GMT |
| Remarks |  | Successfully migrated to IUCLID 5.2 format. |

Administrative Data

| EU: REACH |  |  |  |
| --- | --- | --- | --- |
| Purpose flag | key study; robust study summary |  |  |
| Study result type | experimental result |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Non-GLP, non-guideline, Information from published handbook, acceptable with restrictions. |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| review article or handbook | Lide D (Ed.) | 2008 | CRC Handbook of Chemistry and Physics, 89th Edition. | CRC Press Inc. Boca Raton. USA. |  |  |  |  |  |

Data access

data published

Materials and methods

Principles of method if other than guideline

The use of secondary sources of data is acceptable when they are based on a critical evaluation of peer-reviewed data and a consequent selection of a reliable and representative value for the property under investigation. The introduction to the CRC Handbook of Chemistry and Physics states that data for physical constants have been taken from many sources, including both compilations and the primary literature. Where conflicts were found, the value deemed most reliable was chosen. Therefore, although the method used is not known, the values presented here are acceptable as they are from a reliable secondary source of phys chem. data.

GLP compliance

no data (Not reported)

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 106-98-9 |

Details on test material

No data reported

Confidential details on test material

No data reported

Results and discussions

Autoflammability / Self-ignition temperature

385 °C

Any other information on results incl. tables

No further information reported

Applicant's summary and conclusion

Conclusions

The autoflammability of but-1-ene is 385˚C

Executive summary

No information on the test method used is available in the handbook. However, this information is taken from a reliable peer reviewed handbook and so can be considered reliable and suitable for use as the key study for this endpoint. The autoflammability of but-1-ene is 385˚C.

4.13 Flammability

Endpoint study record: Flammability. Key study -CRC 2008

| UUID |  | IUC5-dfba5289-8e03-41cb-a30f-ff8ff3d2638c |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V1.0 Plug-In |
| Date |  | 2009-07-02 11:12:00 BST |
| Remarks |  | Successfully migrated to IUCLID 5.2 format. |

Administrative Data

| EU: REACH |  |  |  |
| --- | --- | --- | --- |
| Purpose flag | key study; robust study summary |  |  |
| Study result type | experimental result |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Non-GLP, non-guideline, Information from published handbook, acceptable with restrictions. |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| review article or handbook | Lide D (Ed.) | 2008 | CRC Handbook of Chemistry and Physics, 89th Edition. | CRC Press Inc. Boca Raton. USA. |  |  |  |  |  |

Data access

data published

Materials and methods

Principles of method if other than guideline

The use of secondary sources of data is acceptable when they are based on a critical evaluation of peer-reviewed data and a consequent selection of a reliable and representative value for the property under investigation. The introduction to the CRC Handbook of Chemistry and Physics states that data for physical constants have been taken from many sources, including both compilations and the primary literature. Where conflicts were found, the value deemed most reliable was chosen. Therefore, although the method used is not known, the values presented here are acceptable as they are from a reliable secondary source of phys chem. data.

GLP compliance

no data (Not reported)

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 106-98-9 |

Details on test material

No data reported

Confidential details on test material

No data reported

Results and discussions

Gas: Lower explosion limit (%)

1.6

Gas: Upper explosion limit (%)

10

Any other information on results incl. tables

No further information provided

Applicant's summary and conclusion

Interpretation of results

extremely flammable

Conclusions

The lower and upper explosive limits of but-1-ene are 1.6-10 %

Executive summary

No information on the test method used is available in the handbook. However, this information is taken from a reliable peer reviewed handbook and so can be considered reliable and suitable for use as the key study for this endpoint. The lower and upper explosive limits of but-1-ene are 1.6-10 %.

5 Environmental fate and pathways

5.1 Stability

5.1.1 Phototransformation in air

Endpoint study record: Phototransformation in air. Key study - Atkinson 1985

| UUID |  | IUC5-dfaccad9-b2e7-45fb-97a4-4addda7bfe87 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V1.0 Plug-In |
| Date |  | 2009-07-02 11:12:55 BST |
| Remarks |  | Successfully migrated to IUCLID 5.2 format. |

Administrative Data

| EU: REACH |  |  |  |
| --- | --- | --- | --- |
| Purpose flag | key study; robust study summary |  |  |
| Study result type | estimated by calculation |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Non-GLP, information from a review article, acceptable with restrictions |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| review article or handbook | Atkinson R | 1985 | Kinetics and mechanisms of the gas-phase reactions of the hydroxyl radical with organic compounds under atmospheric conditions. | Chem. Rev. 85, 69-201 |  |  |  |  |  |

Data access

data published

Materials and methods

Principles of method if other than guideline

Measured data from author and other investigators were quality assessed and then used to develop rate constants for different chemicals. The author applied a least squares analysis of degradation rate constants for organic chemicals by hydroxyl radicals developed by other investigators.

GLP compliance

no data (No data reported)

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 106-98-9 |

Details on test material

No data reported

Confidential details on test material

No data reported

Details on properties of test surrogate or analogue material

No data reported

Study design

Estimation method (if used)

No data reported

Light source

other: No data reported

Details on light source

No data reported

Details on test conditions

No data reported

Results and discussions

Preliminary study

No data reported

Test performance - Remarks

No data reported

Dissipation half-life of parent compound

| DT50 | 0.5 d |
| --- | --- |
| Test condition | No further details |

Results with reference substance

No data reported

Any other information on results incl. tables

Based on a calculated rate constant of 3.14E-11 cm3molecule-1sec-1, and a hydroxyl radical concentration of 5E5 molecule.cm-3

Applicant's summary and conclusion

Validity criteria fulfilled

not applicable (Review article)

Conclusions

The half-life of but-1-ene is 0.5 days

Executive summary

The half-life of but-1-ene is 0.5days. Measured data from author and other investigators were quality assessed and then used to develop rate constants for different chemicals. Therefore, this value represents a valid rate constant for but-1-ene based on all valid studies at the time of publication.

5.2 Biodegradation

5.2.1 Biodegradation in water: screening tests

Endpoint study record: Biodegradation in water: screening tests. Key study - QSAR (1)

| UUID |  | IUC5-feb63024-4110-4ac2-9703-fd4b9463a3f8 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V1.0 Plug-In |
| Date |  | 2010-04-06 14:16:28 BST |
| Remarks |  | Successfully migrated to IUCLID 5.2 format. |

Administrative Data

| EU: REACH |  |  |  |
| --- | --- | --- | --- |
| Purpose flag | key study; robust study summary |  |  |
| Study result type | (Q)SAR |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | QSAR calculation, acceptable with restrictions |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Howard, P.H., W.M., Meylan, Aronson, D., Stiteler,W.M., Tunkel, J., Comber, M. and Parkerton, F. | 2005 | A New Biodegradation Prediction Model Specific to Petroleum Hydrocarbons. | Environ. Toxicol. Chem. 24(8): 1847-1860. |  |  |  |  |  |

Data access

data published

Materials and methods

Test type

other: Not applicable

Principles of method if other than guideline

BioHCwin v1.01 in EPISuite 4 (2009). BioHCwin is a predictive model for determining quantitative primary biodegradation half-lives for individual petroleum hydrocarbons. This model uses a fragment-based approach that is similar to several other biodegradation models, such as those within the Biodegradation Probability Program (Biowin) estimation program. A half-life in days is estimated using a multiple linear regression against counts of 31 distinct molecular fragments. The model was developed using a data set consisting of 175 compounds with environmentally-relevant experimental data that was divided into training and validation sets.

GLP compliance

no (Not applicable)

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 106-98-9 |

Details on test material

Not applicable

Confidential details on test material

Not applicable

Details on properties of test surrogate or analogue material

Not applicable

Study design

Oxygen conditions

other: Not applicable

Inoculum or test system

other: Not applicable

Details on inoculum

Not applicable

Details on analytical methods

Not applicable

Details on study design

Not applicable

Results and discussions

Preliminary study

Not applicable

Test performance

Not applicable

Details on results

DT50 2.8 days. Predicted on the basis of the presence of an alkenyl hydrogen functional group, a linear [-CH2-] and a methyl group

BOD5 / COD results

Any other information on results incl. tables

No further information available. QSAR gives result in days, not percentage.

Applicant's summary and conclusion

Validity criteria fulfilled

not applicable (QSAR)

Interpretation of results

other: Estimated half life of 2.8 days.

Conclusions

The results of the BioHCwin predictions for but-1-ene indicate that it will degrade rapidly, with an estimated half life of 2.8 days.

Executive summary

As but-1-ene is a gas at standard temperature and pressure conducting a standard ready biodegradability test is technically difficult and based on exposure estimates the test may not be relevant. Therefore, the use of a QSAR to predict the biodegradability of but-1-ene is an appropriate technique to use a part of a weight of evidence approach. The use of BioHCwin is appropriate for but-1-ene as this compound falls within the applicability domain of the model. The results indicate that but-1-ene is expected to biodegrade rapidly.

Endpoint study record: Biodegradation in water: screening tests. Supporting study - QSAR (2)

| UUID |  | IUC5-070869a8-5a36-4ea0-8004-01dbe357bf1b |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V1.0 Plug-In |
| Date |  | 2010-04-06 14:17:34 BST |
| Remarks |  | Successfully migrated to IUCLID 5.2 format. |

Administrative Data

| EU: REACH |  |  |  |
| --- | --- | --- | --- |
| Purpose flag | supporting study; robust study summary |  |  |
| Study result type | (Q)SAR |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | QSAR calculation, acceptable with restrictions |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Howard, P.H., Boethling, R.S., Stiteler, W.M., Meylan, W.M., Hueber, A.E., Beauman, J.A., and M.E. Larosche. | 1992 | Predictive model for aerobic biodegradability developed from a file of evaluated biodegradation data. | Environ. Toxicol. Chem. 11: 593-603. |  |  |  |  |  |

Data access

data published

Materials and methods

Test type

other: Not applicable

Principles of method if other than guideline

Biowin v4.1 in EPISuite 4 (2009). The Biodegradation Probability Program (Biowin) estimates the probability for the rapid aerobic biodegradation of an organic chemical in the presence of mixed populations of environmental microorganisms. Estimates are based upon fragment constants that were developed using multiple linear and non-linear regression analyses.

GLP compliance

no (Not applicable)

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 106-98-9 |

Details on test material

Not applicable

Confidential details on test material

Not applicable

Details on properties of test surrogate or analogue material

Not applicable

Study design

Oxygen conditions

other: Not applicable

Inoculum or test system

other: Not applicable

Details on inoculum

Not applicable

Details on analytical methods

Not applicable

Details on study design

Not applicable

Results and discussions

Preliminary study

Not applicable

Test performance

Not applicable

Details on results

Biowin 5 and 6 models contain the most molecular fragment predictors that are relevant to but-1-ene (an alkenyl hydrogen, a methyl and a linear [-CH2-]. The results of Biowin 1,2,3 and 4 are based on the molecular mass and linear C4 terminal chain [CCC-CH3]of but-1-ene. Biowin 1-2 predict a probability of between 0.83 and 0.98 for ready biodegradability. Biowin 5 predicts a probability of 0.61 for ready biodegradability. Biowin 6 predicts a probability of 0.81 for ready biodegradability

BOD5 / COD results

Results with reference substance

Not applicable

Any other information on results incl. tables

Biowin1 (Linear Model Prediction) : Biodegrades Fast, Biowin2 (Non-Linear Model Prediction): Biodegrades Fast, Biowin3 (Ultimate Biodegradation Timeframe): Days-Weeks, Biowin4 (Primary Biodegradation Timeframe): Days, Biowin5 (MITI Linear Model Prediction) : Readily degradable, Biowin6 (MITI Non-Linear Model Prediction): Readily degradable, Ready Biodegradability Prediction: YES

Applicant's summary and conclusion

Validity criteria fulfilled

not applicable (QSAR)

Interpretation of results

readily biodegradable

Conclusions

The results of the BIOWIN 1,2, 5 and 6 predictions for but-1-ene indicate that it is expected to biodegrade fast.

Executive summary

As but-1-ene is a gas at standard temperature and pressure conducting a standard ready biodegradability test is technically difficult and based on exposure estimates the test may not be relevant. Therefore, the use of a QSAR to predict the biodegradability of but-1-ene is an appropriate technique to use . The use of Biowin 5 and 6 is appropriate for but-1-ene as this compound falls within the applicability domain of the model. The results indicate that but-1-ene is expected to biodegrade rapidly.

Endpoint study record: Biodegradation in water: screening tests. weight of evidence - Hou 1983 (3)

| UUID |  | IUC5-4ca7e48b-5132-41ce-973f-6964c9fed368 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V1.0 Plug-In |
| Date |  | 2010-04-06 14:22:31 BST |
| Remarks |  | Successfully migrated to IUCLID 5.2 format. |

Administrative Data

| EU: REACH |  |  |  |
| --- | --- | --- | --- |
| Purpose flag | supporting study; robust study summary |  |  |
| Study result type | experimental result |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Non-GLP, non-guideline, published in peer reviewed literature, acceptable with restrictions |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Hou CT, Patel R, Laskin AI, Barnabe N, Barist I | 1983 | Epoxidation of short-chain alkenes by resting-cell suspensions of propane-grown bacteria | Appl Environ Microbiol. 46; 171-177 |  |  |  |  |  |

Data access

data published

Materials and methods

Test type

no data

Principles of method if other than guideline

Propane degrading bacteria are isolated from soil and lake water samples. Their ability to degrade short chain gaseous alkenes is tested.

GLP compliance

no data (No data reported)

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 106-98-9 |

Details on test material

obtained from Matheson Gas Products

Confidential details on test material

No data reported

Details on properties of test surrogate or analogue material

No data reported

Study design

Oxygen conditions

aerobic

Inoculum or test system

other: See Details on inoculum

Details on inoculum

Isolated strains of Arthrobacter sp, Alcaligenes sp, Brevibacterium sp, Mycobacterium sp, Nocardia sp and Pseudomonas sp.

Duration of test (contact time)

1 h

Details on analytical methods

No data reported

Results and discussions

Preliminary study

No data reported

Test performance

No data reported

Details on results

Degradation rates are not reported in this paper. 27 strains of bacteria were able to degrade but-1-ene to its corresponding 1,2-epoxide.

BOD5 / COD results

Results with reference substance

No data reported

Any other information on results incl. tables

Degradation rates are not reported in this paper. 27 strains of bacteria were able to degrade but-1-ene to its corresponding 1,2-epoxide.

Applicant's summary and conclusion

Interpretation of results

other: No data reported

Conclusions

This paper identifies 27 strains of bacteria that were capable of biodegrading but-1-ene in aerobic condititions.

Executive summary

Although this paper does not follow standard guidelines for ready biodegradability tests it does demonstrate that microorganisms isolated from soil and lake water samples can biodegrade but-1 -ene. This adds to the weight of evidence that but-1 -ene can be expected to biodegrade if released to the environment.

5.4 Transport and distribution

5.4.1 Adsorption / desorption

Endpoint study record: Adsorption / desorption. Key study - Sabljic and Güsten 1995

| UUID |  | IUC5-954dfd97-cac1-4de6-84c1-d498b42dfbbe |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V1.0 Plug-In |
| Date |  | 2010-01-04 13:46:29 GMT |
| Remarks |  | Successfully migrated to IUCLID 5.2 format. |

Administrative Data

| EU: REACH |  |  |  |
| --- | --- | --- | --- |
| Purpose flag | key study; robust study summary |  |  |
| Study result type | estimated by calculation | Study period | Up to 2004 |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | TGD Guideline, QSAR calculation, acceptable with restrictions |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | Sabljic A and Güsten H | 1995 | QSARs for soil sorption. In: Overview of Structure-Activity Relationships for Environmental Endpoints. | Environmental Technologies RTD Programme (DG XII/D-1) of the European Commission under contract number EV5V-CT92- 0211. |  |  |  |  |  |

Data access

data published

Materials and methods

Study type

other: QSAR

Media

other: Not applicable

Type of method

other: Not applicable

Principles of method if other than guideline

According to Sabljic and Gusten (1995) as recommended in Part 2 Section 2.3 of the TGD (2003) for hydrophobic compounds. Koc is calculated as log Koc = 0.81*log Kow + 0.1. Based on a measured log Kow of 2.4 for But-1-ene.

GLP compliance

no data

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 106-98-9 |

Radiolabelling

no data

Details on test material

Not applicable

Confidential details on test material

Not applicable

Details on properties of test surrogate or analogue material

Not applicable

Study design

Test temperature

Not applicable

HPLC method

Details on study design: HPLC method

Not applicable

Batch equilibrium or other method

Analytical monitoring

not required

Details on sampling

Not applicable

Details on analytical methods

Not applicable

Details on matrix

Not applicable

Details on test conditions

Not applicable

Computational methods

Not applicable

Results and discussions

Adsorption coefficient

| Type | Value | Temperature | % Org. carbon | Remarks |
| --- | --- | --- | --- | --- |
| Koc | 109.6 |  |  |  |
| log Koc | 2.04 |  |  |  |

Results: HPLC method

Details on results (HPLC method)

Not applicable

Results: Batch equilibrium or other method

Adsorption and desorption constants

Not applicable

Recovery of test material

Not applicable

Concentration of test substance at end of adsorption equilibration period

Not applicable

Concentration of test substance at end of desorption equilibration period

Not applicable

Transformation products

no data

Details on results (Batch equilibrium method)

Not applicable

Statistics

Not applicable

Applicant's summary and conclusion

Validity criteria fulfilled

not applicable (Calcualted value)

Conclusions

The estimated adsorption coefficient koc of but-1-ene is 109.6 and the log koc is 2.04

Executive summary

The adsorption coefficient has been calculated according to the appropriate equation in the TGD (EC, 2003) and is considered reliable for this endpoint. The Koc of but-1 -ene is 10 9.6

Cross-reference to other study

None

6 Ecotoxicological Information

6.1 Aquatic toxicity

6.1.1 Short-term toxicity to fish

Endpoint study record: Short-term toxicity to fish. QSAR

| UUID |  | IUC5-7a0f2c2e-312c-4988-b1df-1850f0a45fee |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V1.0 Plug-In |
| Date |  | 2009-07-10 12:12:07 BST |
| Remarks |  | Successfully migrated to IUCLID 5.2 format. |

Administrative Data

| EU: REACH |  |  |  |
| --- | --- | --- | --- |
| Purpose flag | key study; robust study summary |  |  |
| Study result type | (Q)SAR |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | QSAR calculation, reliable with restrictions |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| other: computor model | Nabholz V and Mayo-Bean K | 2009 | ECOWIN v1 ECOSAR Classes for Microsoft Windows | USEPA OPPT Risk Assessment Division |  |  |  |  |  |

Data access

not applicable

Materials and methods

Principles of method if other than guideline

The Ecosar class program has been developed primarily for the evaluation of neutral organic compounds and organic classes with excess toxicity. The QSARs in the Ecosar program are developed for chemical classes based on measured test data that have been submitted by industry or they are developed by other sources for chemicals with similar structures, e.g., phenols. Using the measured aquatic toxicity values and estimated Kow values, regression equations can be developed for a class of chemicals. Toxicity values for new chemicals may then be calculated by inserting the Kow into the regression equation and correcting the resultant value for the molecular weight of the compound.

GLP compliance

no (Not applicable)

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 106-98-9 |

Details on test material

Not applicable

Confidential details on test material

Not applicable

Details on properties of test surrogate or analogue material

Not applicable

Analytical monitoring

not required

Details on sampling

Not applicable

Details on analytical methods

Not applicable

Details on test solutions

Not applicable

Test organisms

Test organisms (species)

other: Fish

Details on test organisms

Not applicable

Study design

Test type

other: Not applicable

Water media type

freshwater

Post exposure observation period

Not applicable

Test conditions

Hardness

Not applicable

Test temperature

Not applicable

pH

Not applicable

Dissolved oxygen

Not applicable

Salinity

Not applicable

Nominal and measured concentrations

Not applicable

Details on test conditions

Not applicable

Results and discussions

Effect concentrations

| Duration | Endpoint | Effect conc. | Nominal/Measured | Conc. based on | Basis for effect | Remarks (e.g. 95% CL) |
| --- | --- | --- | --- | --- | --- | --- |
| 96 h | LC50 | 19 mg/L | estimated | test mat. | mortality |  |

Details on results

calculated using the neutral organic QSAR. Data entered: log Kow 2.4, melting point = -185, water solubility = 222mg/l

Results with reference substance (positive control)

Not applicable

Reported statistics and error estimates

Not applicable

Applicant's summary and conclusion

Validity criteria fulfilled

not applicable (QSAR)

Conclusions

The estimated 96hr LC50 for fish is 19 mg/l.

Executive summary

The estimated 96hr LC50 for fish is 19 mg/l. Measured data are not available for but-1-ene for this endpoint. Due to the practical difficulties associated with the ecotoxicity testing of gases (i.e. maintaining exposure concentrations) the use of QSAR toxicity estimates is an appropriate alternative. The ECOSAR model is a reliable and appropriate QSAR model to apply to but-1-ene as it is based on a related chemical dataset that calculates the toxicity of neutral organic hydrocarbons whose mode of action is non-polar narcosis. The endpoint calculated here is therefore a reasonable estimate of its baseline toxicity.

6.1.2 Long-term toxicity to fish

Endpoint study record: Long-term toxicity to fish. QSAR

| UUID |  | IUC5-59c3e5e9-4ab0-43e8-96a7-b1aef06c7f00 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V1.0 Plug-In |
| Date |  | 2010-04-15 09:30:54 BST |
| Remarks |  | Successfully migrated to IUCLID 5.2 format. |

Administrative Data

| EU: REACH |  |  |  |
| --- | --- | --- | --- |
| Purpose flag | key study; robust study summary |  |  |
| Study result type | (Q)SAR |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | QSAR calculation, reliable with restrictions |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| other: computer model | Nabholz V and Mayo-Bean K | 2009 | ECOWIN v1 ECOSAR Classes for Microsoft Windows | USEPA OPPT Risk Assessment Division |  |  |  |  |  |

Data access

data published

Materials and methods

Life stage / endpoint studied

other: calculated

Principles of method if other than guideline

The Ecosar class program has been developed primarily for the evaluation of neutral organic compounds and organic classes with excess toxicity. The QSARs in the Ecosar program are developed for chemical classes based on measured test data that have been submitted by industry or they are developed by other sources for chemicals with similar structures, e.g., phenols. Using the measured aquatic toxicity values and estimated Kow values, regression equations can be developed for a class of chemicals. Toxicity values for new chemicals may then be calculated by inserting the Kow into the regression equation and correcting the resultant value for the molecular weight of the compound.

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 106-98-9 |

Details on test material

not applicable

Confidential details on test material

not applicable

Details on properties of test surrogate or analogue material

not applicable

Analytical monitoring

no

Details on sampling

not applicable

Details on analytical methods

not applicable

Vehicle

no

Details on test solutions

not applicable

Test organisms

Test organisms (species)

other: fish

Details on test organisms

not applicable

Study design

Test type

no data

Water media type

freshwater

Limit test

no

Total exposure duration

30 d

Post exposure observation period

not applicable

Test conditions

Hardness

not applicable

Test temperature

not applicable

pH

not applicable

Dissolved oxygen

not applicable

Salinity

not applicable

Nominal and measured concentrations

not applicable

Details on test conditions

not applicable

Reference substance (positive control)

no

Results and discussions

Effect concentrations

| Duration | Endpoint | Effect conc. | Nominal/Measured | Conc. based on | Basis for effect | Remarks (e.g. 95% CL) |
| --- | --- | --- | --- | --- | --- | --- |
| 30 d | other: ChV | 2 mg/L | estimated | test mat. | mortality |  |

Details on results

calculated using the neutral organic QSAR. Data entered: log Kow 2.4, melting point = -185, water solubility = 222mg/l

Results with reference substance (positive control)

not applicable

Reported statistics and error estimates

not applicable

Applicant's summary and conclusion

Validity criteria fulfilled

not applicable (QSAR)

Conclusions

The estimated 30 day ChV for fish is 2 mg/l

Executive summary

The estimated 30 day ChV for fish is 2 mg/l. Measured data are not available for but-1-ene for this endpoint. Due to the practical difficulties associated with the ecotoxicity testing of gases (i.e. maintaining exposure concentrations) the use of QSAR toxicity estimates is an appropriate alternative. The ECOSAR model is a reliable and appropriate QSAR model to apply to but-1-ene as it is based on a related chemical dataset that calculates the toxicity of neutral organic hydrocarbons whose mode of action is non-polar narcosis. The endpoint calculated here is therefore a reasonable estimate of its baseline toxicity.

6.1.3 Short-term toxicity to aquatic invertebrates

Endpoint study record: Short-term toxicity to aquatic invertebrates. QSAR

| UUID |  | IUC5-c7d5b89d-b2a7-4542-8a36-23c8cba6d036 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V1.0 Plug-In |
| Date |  | 2010-04-15 09:31:05 BST |
| Remarks |  | Successfully migrated to IUCLID 5.2 format. |

Administrative Data

| EU: REACH |  |  |  |
| --- | --- | --- | --- |
| Purpose flag | key study; robust study summary |  |  |
| Study result type | (Q)SAR |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | QSAR calculation, reliable with restrictions |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| other: computor model | Nabholz V and Mayo-Bean K | 2009 | ECOWIN v1 ECOSAR Classes for Microsoft Windows | USEPA OPPT Risk Assessment Division |  |  |  |  |  |

Data access

not applicable

Materials and methods

Principles of method if other than guideline

The Ecosar class program has been developed primarily for the evaluation of neutral organic compounds and organic classes with excess toxicity. The QSARs in the Ecosar program are developed for chemical classes based on measured test data that have been submitted by industry or they are developed by other sources for chemicals with similar structures, e.g., phenols. Using the measured aquatic toxicity values and estimated Kow values, regression equations can be developed for a class of chemicals. Toxicity values for new chemicals may then be calculated by inserting the Kow into the regression equation and correcting the resultant value for the molecular weight of the compound.

GLP compliance

no (Not applicable)

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 106-98-9 |

Details on test material

Not applicable

Confidential details on test material

Not applicable

Details on properties of test surrogate or analogue material

Not applicable

Details on sampling

Not applicable

Details on analytical methods

Not applicable

Details on test solutions

Not applicable

Test organisms

Test organisms (species)

Daphnia sp.

Details on test organisms

Not applicable

Study design

Test type

other: Not applicable

Water media type

no data

Post exposure observation period

Not applicable

Test conditions

Hardness

Not applicable

Test temperature

Not applicable

pH

Not applicable

Dissolved oxygen

Not applicable

Salinity

Not applicable

Nominal and measured concentrations

Not applicable

Details on test conditions

Not applicable

Results and discussions

Effect concentrations

| Duration | Endpoint | Effect conc. | Nominal/Measured | Conc. based on | Basis for effect | Remarks (e.g. 95% CL) |
| --- | --- | --- | --- | --- | --- | --- |
| 48 h | LC50 | 11 mg/L | estimated | test mat. |  |  |

Details on results

calculated using the neutral organic QSAR. Data entered: log Kow 2.4, melting point = -185, water solubility = 222mg/l

Results with reference substance

Not applicable

Reported statistics and error estimates

Not applicable

Applicant's summary and conclusion

Validity criteria fulfilled

not applicable (QSAR)

Conclusions

The estimated 48hr LC50 for Daphnids is 11 mg/l

Executive summary

The estimated 48hr LC50 for Daphnids is 11 mg/l. Measured data are not available for but-1-ene for this endpoint. Due to the practical difficulties associated with the ecotoxicity testing of gases (i.e. maintaining exposure concentrations) the use of QSAR toxicity estimates is an appropriate alternative. The ECOSAR model is a reliable and appropriate QSAR model to apply to but-1-ene as it is based on a related chemical dataset that calculates the toxicity of neutral organic hydrocarbons whose mode of action is non-polar narcosis. The endpoint calculated here is therefore a reasonable estimate of its baseline toxicity.

6.1.4 Long-term toxicity to aquatic invertebrates

Endpoint study record: Long-term toxicity to aquatic invertebrates. QSAR

| UUID |  | IUC5-11a5c34c-1764-4a29-8328-d05486ec1f8a |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V1.0 Plug-In |
| Date |  | 2010-04-15 09:32:48 BST |
| Remarks |  | Successfully migrated to IUCLID 5.2 format. |

Administrative Data

| EU: REACH |  |  |  |
| --- | --- | --- | --- |
| Purpose flag | key study; robust study summary |  |  |
| Study result type | (Q)SAR |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | QSAR calculation, reliable with restrictions |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| other: computer model | Nabholz V and Mayo-Bean K | 2009 | ECOWIN v1 ECOSAR Classes for Microsoft Windows | USEPA OPPT Risk Assessment Division |  |  |  |  |  |

Data access

data published

Materials and methods

Principles of method if other than guideline

The Ecosar class program has been developed primarily for the evaluation of neutral organic compounds and organic classes with excess toxicity. The QSARs in the Ecosar program are developed for chemical classes based on measured test data that have been submitted by industry or they are developed by other sources for chemicals with similar structures, e.g., phenols. Using the measured aquatic toxicity values and estimated Kow values, regression equations can be developed for a class of chemicals. Toxicity values for new chemicals may then be calculated by inserting the Kow into the regression equation and correcting the resultant value for the molecular weight of the compound.

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 106-98-9 |

Details on test material

not applicable

Confidential details on test material

not applicable

Details on properties of test surrogate or analogue material

not applicable

Analytical monitoring

no

Details on sampling

not applicable

Details on analytical methods

not applicable

Vehicle

no

Details on test solutions

not applicable

Test organisms

Test organisms (species)

Daphnia sp.

Details on test organisms

not applicable

Study design

Test type

no data

Water media type

freshwater

Limit test

no

Post exposure observation period

not applicable

Test conditions

Hardness

not applicable

Test temperature

not applicable

pH

not applicable

Dissolved oxygen

not applicable

Salinity

not applicable

Nominal and measured concentrations

not applicable

Details on test conditions

not applicable

Reference substance (positive control)

no data (Not reported)

Results and discussions

Effect concentrations

| Duration | Endpoint | Effect conc. | Nominal/Measured | Conc. based on | Basis for effect | Remarks (e.g. 95% CL) |
| --- | --- | --- | --- | --- | --- | --- |
| 96 h | other: ChV | 1.4 mg/L | estimated | test mat. | mortality |  |

Details on results

calculated using the neutral organics QSAR. Data entered: log Kow 2.4, melting point = -185, water solubility = 222mg/l

Results with reference substance (positive control)

not applicable

Reported statistics and error estimates

not applicable

Applicant's summary and conclusion

Validity criteria fulfilled

not applicable (Calculated value)

Conclusions

The estimated ChV for Daphnia is 1.4 mg/l.

Executive summary

The estimated ChV for Daphnia is 1.4 mg/l. Measured data are not available for but-1-ene for this endpoint. Due to the practical difficulties associated with the ecotoxicity testing of gases (i.e. maintaining exposure concentrations) the use of QSAR toxicity estimates is an appropriate alternative. The ECOSAR model is a reliable and appropriate QSAR model to apply to but-1-ene as it is based on a related chemical dataset that calculates the toxicity of neutral organic hydrocarbons whose mode of action is non-polar narcosis. The endpoint calculated here is therefore a reasonable estimate of its baseline toxicity.

6.1.5 Toxicity to aquatic algae and cyanobacteria

Endpoint study record: Toxicity to aquatic algae and cyanobacteria.QSAR (acute)

| UUID |  | IUC5-e320a3ae-6d27-436b-92b0-e82f0573fd35 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V1.0 Plug-In |
| Date |  | 2009-09-08 10:40:38 BST |
| Remarks |  | Successfully migrated to IUCLID 5.2 format. |

Administrative Data

| EU: REACH |  |  |  |
| --- | --- | --- | --- |
| Purpose flag | key study; robust study summary |  |  |
| Study result type | (Q)SAR |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | QSAR calculation, reliable with restrictions |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| other: computer model | Nabholz V and Mayo-Bean K | 2009 | ECOWIN v1 ECOSAR Classes for Microsoft Windows | USEPA OPPT Risk Assessment Division |  |  |  |  |  |

Data access

not applicable

Materials and methods

Principles of method if other than guideline

The Ecosar class program has been developed primarily for the evaluation of neutral organic compounds and organic classes with excess toxicity. The QSARs in the Ecosar program are developed for chemical classes based on measured test data that have been submitted by industry or they are developed by other sources for chemicals with similar structures, e.g., phenols. Using the measured aquatic toxicity values and estimated Kow values, regression equations can be developed for a class of chemicals. Toxicity values for new chemicals may then be calculated by inserting the Kow into the regression equation and correcting the resultant value for the molecular weight of the compound.

GLP compliance

no data (Not applicable)

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 106-98-9 |

Details on test material

Not applicable

Confidential details on test material

Not applicable

Details on properties of test surrogate or analogue material

Not applicable

Details on sampling

Not applicable

Details on analytical methods

Not applicable

Details on test solutions

Not applicable

Test organisms

Test organisms (species)

other: Green algae

Details on test organisms

Not applicable

Study design

Test type

other: Not applicable

Water media type

no data

Post exposure observation period

Not applicable

Test conditions

Hardness

Not applicable

Test temperature

Not applicable

pH

Not applicable

Dissolved oxygen

Not applicable

Salinity

Not applicable

Nominal and measured concentrations

Not applicable

Details on test conditions

Not applicable

Results and discussions

Effect concentrations

| Duration | Endpoint | Effect conc. | Nominal/Measured | Conc. based on | Basis for effect | Remarks (e.g. 95% CL) |
| --- | --- | --- | --- | --- | --- | --- |
| 96 h | EC50 | 6.5 mg/L | estimated | test mat. | no data |  |

Details on results

calculated using the neutral organic QSAR. Data entered: log Kow 2.4, melting point = -185, water solubility = 222mg/l

Results with reference substance (positive control)

Not applicable

Reported statistics and error estimates

Not applicable

Applicant's summary and conclusion

Validity criteria fulfilled

not applicable (QSAR)

Conclusions

The estimated 96hr EC50 for green algae is 6.5 mg/l

Executive summary

The estimated 96hr EC50 for green algae is 6.5 mg/l. Measured data are not available for but-1-ene for this endpoint. Due to the practical difficulties associated with the ecotoxicity testing of gases (i.e. maintaining exposure concentrations) the use of QSAR toxicity estimates is an appropriate alternative. The ECOSAR model is a reliable and appropriate QSAR model to apply to but-1-ene as it is based on a related chemical dataset that calculates the toxicity of neutral organic hydrocarbons whose mode of action is non-polar narcosis. The endpoint calculated here is therefore a reasonable estimate of its baseline toxicity.

6.3 Terrestrial toxicity

6.3.1 Toxicity to soil macroorganisms except arthropods

Endpoint study record: Toxicity to soil macroorganisms except arthropods. QSAR

| UUID |  | IUC5-994d2e78-fac8-4a60-a906-fdb8f98a327a |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V1.0 Plug-In |
| Date |  | 2010-04-06 16:18:31 BST |
| Remarks |  | Successfully migrated to IUCLID 5.2 format. |

Administrative Data

| EU: REACH |  |  |  |
| --- | --- | --- | --- |
| Purpose flag | supporting study; robust study summary |  |  |
| Study result type | (Q)SAR |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | QSAR calculation, reliable with restrictions |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| other: Computer model | Nabholz V and Mayo-Bean K | 2009 | ECOWIN v1 ECOSAR Classes for Microsoft Windows | USEPA OPPT Risk Assessment Division |  |  |  |  |  |

Data access

not applicable

Materials and methods

Principles of method if other than guideline

The Ecosar class program has been developed primarily for the evaluation of neutral organic compounds and organic classes with excess toxicity. The QSARs in the Ecosar program are developed for chemical classes based on measured test data that have been submitted by industry or they are developed by other sources for chemicals with similar structures, e.g., phenols. Using the measured aquatic toxicity values and estimated Kow values, regression equations can be developed for a class of chemicals. Toxicity values for new chemicals may then be calculated by inserting the Kow into the regression equation and correcting the resultant value for the molecular weight of the compound.

GLP compliance

no (Not applicable)

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 106-98-9 |

Details on test material

Not applicable

Confidential details on test material

Not applicable

Details on properties of test surrogate or analogue material

Not applicable

Details on sampling

Not applicable

Details on analytical methods

Not applicable

Details on preparation and application of test substrate

Not applicable

Test organisms

Test organisms (species)

other: Earthworm

Animal group

annelids

Details on test organisms

Not applicable

Study design

Study type

other: Not applicable

Test duration type

short-term toxicity

Substrate type

other: Not applicable

Post exposure observation period

Not applicable

Test conditions

Test temperature

Not applicable

pH

Not applicable

Moisture

Not applicable

Nominal and measured concentrations

Not applicable

Details on test conditions

Not applicable

Results and discussions

Effect concentrations

| Duration | Endpoint | Effect conc. | Nominal/Measured | Conc. based on | Basis for effect | Remarks (e.g. 95% CL) |
| --- | --- | --- | --- | --- | --- | --- |
| 14 d | LC50 | 88 mg/kg soil dw | estimated | test mat. | mortality |  |

Details on results

calculated using the neutral organic QSAR. Data entered: log Kow 2.4, melting point = -185, water solubility = 222mg/l

Results with reference substance (positive control)

Not applicable

Reported statistics and error estimates

Not applicable

Applicant's summary and conclusion

Validity criteria fulfilled

not applicable (QSAR)

Conclusions

The estimated 14 day LC50 for but-1-ene is 88 mg/kg

Executive summary

The estimated 14 day LC50 for but-1-ene is 88 mg/kg. Measured data are not available for but-1-ene for this endpoint. Due to the practical difficulties associated with the ecotoxicity testing of gases (i.e. maintaining exposure concentrations) the use of QSAR toxicity estimates is an appropriate alternative. The ECOSAR model is a reliable and appropriate QSAR model to apply to but-1-ene as it is based on a related chemical dataset that calculates the toxicity of neutral organic hydrocarbons whose mode of action is non-polar narcosis. The endpoint calculated here is therefore a reasonable estimate of its baseline toxicity

7 Toxicological information

7.1 Toxicokinetics, metabolism and distribution

7.1.1 Basic toxicokinetics

Endpoint study record: Basic toxicokinetics. Key study, Eide et al, 1995

| UUID |  | IUC5-eae47424-f85d-48a2-a281-66f42169964c |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V1.0 Plug-In |
| Date |  | 2010-04-11 11:27:28 BST |
| Remarks |  | Successfully migrated to IUCLID 5.2 format. |

Administrative Data

| EU: REACH |  |  |  |
| --- | --- | --- | --- |
| Purpose flag | key study; robust study summary |  |  |
| Study result type | experimental result |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Non-GLP, non-guideline study, published in peer reviewed literature, minor restrictions in design and/or reporting but otherwise adequate for assessment |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Eide I, Hagemann R, Zahlsen K, Tareke E, Törnqvist M, Kumar R, Vodicka P and Hemminki K. | 1995 | Uptake, distribution, and formation of hemoglobin and DNA adducts after inhalation of C2-C8 1-alkenes (olefins) in the rat. | Carcinogenesis 16:1603-1609 |  |  |  |  |  |

Data access

data published

Cross-reference to same study

none

Materials and methods

Type of method

in vivo

Objective of study

absorption

distribution

excretion

other: haemoglobin and DNA adduct formation

Principles of method if other than guideline

A homologous series of C2-C8 1-alkenes (olefins) were investigated in inhalation experiments with rats. The structure activity approach gave information about uptake, distribution and formation of haemoglobin and DNA adducts in relation to the number of carbon atoms in the olefins.

GLP compliance

no data

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| Common name | 1-butene |

Radiolabelling

no

Test materials

Details on test material

1-butene (>99%) was obtained from HydroGas AS, Norway

Confidential details on test material

none

Test animals

Species

rat

Strain

Sprague-Dawley

Sex

male

Details on test animals and environmental conditions

TEST ANIMALS - Source: Mǿllgaard A/S, Ll. Skensved, Denmark - Age at study initiation: no data - Weight at study initiation: 180-225 g. - Housing: There were eight animals in each cage and a maximum of four cages in each inhalation chamber. - Diet: ad libitum except during exposure. - Water: ad libitum except during exposure. - Acclimation period: At least 5 days ENVIRONMENTAL CONDITIONS - Temperature: 21±1°C - Humidity: 65± 5% - Air changes (per hr): 10 per hr - Photoperiod: 10 hrs dark / 14 hrs light IN-LIFE DATES: no data

Administration / exposure

Route of administration

inhalation

Vehicle

other: air

Details on exposure

TYPE OF INHALATION EXPOSURE: whole body GENERATION OF TEST ATMOSPHERE / CHAMBER DESCRIPTION - Exposure apparatus: conically shaped 0.7 m3 steel chambers with glass front doors and walls - System of generating test atmosphere: The aimed concentrations of C2 to C4 gases in the exposure chambers were generated by introducing a controlled stream of pure gas delivered by a two-stage precision pressure regulator. TEST ATMOSPHERE - Monitored hourly by gas chromatography

Duration and frequency of treatment / exposure

12 h/day for 3 consecutive days

Doses / concentrations

300 ppm

No. of animals per sex per dose

A minimum of 8 males

Control animals

yes, concurrent no treatment

Positive control

none

Details on study design

see below

Details on dosing and sampling

see below

Statistics

none

Any other information on materials and methods incl. tables

The concentration of 1-alkenes was measured in blood, brain, liver, kidney, lung and peri-renal fat in groups of four animals immediately after each 12 h exposure and after a recovery period of 12 h after termination of exposure on day 3. DNA adducts were determined by32P-post-labeling in liver and lymphocytes sampled immediately after the last exposure. Erythrocytes were isolated from blood sampled immediately after the last exposure and haemoglobin adducts were determined by GC/MS and GC/MS/MS.

Results and discussions

Preliminary studies

not determined

Pharmacokinetic studies

Details on absorption

not determined

Details on distribution in tissues

see below

Transfer into organs

| Transfer type | other: |
| --- | --- |

Details on excretion

not determined

Toxicokinetic parameters

| other: |  |
| --- | --- |

Metabolite characterisation studies

Metabolites identified

not measured

Details on metabolites

Concentrations of 1-alkenes in blood and organs reached a steady-state level after the first 12 h exposure, and the concentrations 12 h after the last exposure were generally low, except in fat tissue. Concentrations of 1-alkenes in blood and the different tissues increased with increasing number of carbon atoms. Levels of haemoglobin and DNA adducts decreased with increasing number of carbon atoms with the most pronounced decrease from C2 to C3. The decrease through the whole homologous series from C2 to C8 was most pronounced for haemoglobin adducts followed by the DNA adducts in the lymphocytes. Detectable levels of haemoglobin and DNA adducts were produced by all 1-alkenes, although the levels of haemoglobin adducts after C4-C8 exposure were low.

Any other information on results incl. tables

Concentrations of 1-alkenes in tissues after the final 12 h exposure to 300 ppm blood; liver; lung; brain; kidneys; fat. Ethane; 0.3±0.1; 0.4±0.1; 2.3±0.5; 0.7±0.1; 0.7±0.1; 7±1. Propene; 1.1 ±0.2; 0.3±0.2; 2.9± 1.3; 1.7±0.3; 1.8±0.2; 36±4. 1-butene; 1.9±0.1; 0.8±0.3; 4.9± 1.1; 3.0±0.3; 5.7±1.4; 70±8. 1-pentene; 8.6±1.4; 51.6±12.9; 31.4±10.6; 41.0±4.9; 105.7±13.7; 368±79. 1-hexene; 18.2±1.1; 66.8±1.2; 59.7± 17.7; 59.7±4.6; 188.0±21.7; 1031±54. 1-heptene; 37.0±1.8; 138.3±9.7; 85.6±8.2; 109.3±5.5; 269.3±21.8; 2598±253. 1-octene; 60.1±2.2; 443.7±26.9; 202.4±29.5; 270.0±9.5; 385.1±32.6; 4621±468. N=4, mean +/- SD All concentrations are in µmol/kg Adjustment factors for exhalatory loss of C2 -C4 1 -alkenes ranged from 1.00 to 1.10. Mean levels of in haemoglobin adducts (N-(2-hydroxyalkyl)valine) and DNA adducts (7-alkylguanine) in lymphocytes and liver after exposure of rats to 1-alkenes Haemoglobin (pmol/g); Lymphocytes (adducts/107normal nucleotides); Liver (adducts/107normal nucleotides). Ethane; 2730±100; 5.8±2.2; 7.4±1.0. Propene; 740 ±50; 1.8 ±0.9; 2.8 ±0.9. 1-butene; 20±1; 0.8±0.4; 2.1±0.5. 1-pentene; 51±3; 0.5±0.2; 1.8±0.6. 1-hexene; 39±1; 0.3±0.3; 1.4±0.2. 1-heptene; 11±1; 0.4±0.1; 1.3±0.4. 1-octene; 1.7±0.5; 0.2±0.2; 1.3±0.6. Background values are subtracted N = 3-8 for haemoglobin and 4 for DNA adduct analysis

Overall remarks, attachments

Overall remarks

REACH requires inhalation exposure units to be in mg/m3, ppm have been converted into mg/m3using molecular weight of 56.11 and constant of 24.45 (molar vol of a gas at 25oC and 1 atm pressure).

Applicant's summary and conclusion

Interpretation of results

low bioaccumulation potential based on study results

Conclusions

Rats exposed to 300ppm of 1-alkenes of from C2-C8 (including 1-butene) for 12h for 3 days had increased concentrations of the alkenes in blood and tissues proportional to increasing numbers of carbon atoms. In contrast, levels of haemoglobin and DNA adducts decreased with increasing numbers of carbon atoms. The 1-alkenes were widely distributed within the body with the lowest concentrations in blood and the highest in fat. Concentrations of 1-butene (micromol/kg tissue) were: blood 1.9, liver 0.8, lung 4.9, brain 3.0, kidneys 5.7, fat 70. DNA adducts (N-7 alkyl guanine) (adducts/107 nucleotides) were: lymphocytes 0.8, liver 2.1 after exposure to 1-butene.

Executive summary

The absorption, distribution, elimination, haemoglobin adduct formation and DNA adduct formation of individual C2-C8 1-alkenes was studied in the rat after exposure to 300 ppm (688mg/m3), 12 h a day for 3 consecutive days. The concentrations of the alkenes were measured in blood, lung, brain, liver, kidney and peri-renal fat immediately after each exposure and 12 h after the third exposure. DNA adducts were determined by 32P-postlabeling in liver. Haemoglobin adducts were determined in erythrocytes by GC/MS and GC/MS/MS. Concentrations of 1-alkenes in blood and organs reached a steady-state level after the first 12 h exposure, and the concentrations 12 h after the last exposure were generally low, except in fat. Concentrations of 1-alkenes in blood and the different tissues increased with increasing number of carbon atoms. However, DNA adducts and haemoglobin adducts decreased with increasing number of carbon atoms with the most pronounced decrease being from C2 to C3. The decrease in haemoglobin adducts was more pronounced than DNA adducts. All 1-alkenes caused formation of detectable levels of haemoglobin and DNA adducts, although the levels of haemoglobin adducts after C4-C8 exposure were low. These results also indicate that extrapolation within the homologous series is possible.

Cross-reference to other study

none

7.2 Acute Toxicity

7.2.2 Acute toxicity: inhalation

Endpoint study record: Acute toxicity: inhalation. Key study, TNO, 1992a

| UUID |  | IUC5-9a977d35-dd56-4ede-868d-d0f05a81adf0 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | Thomas Penman / LOA Reach Services Team / Tervuren / Belgium |
| Date |  | 2010-07-14 22:43:34 BST |
| Remarks |  |  |

Administrative Data

| EU: REACH |  |  |  |
| --- | --- | --- | --- |
| Purpose flag | key study; robust study summary |  |  |
| Study result type | experimental result |  |  |
| Reliability | 1 (reliable without restriction) |  |  |
| Rationale for reliability incl. deficiencies | GLP, guideline study, available as unpublished report, no restrictions, fully adequate for assessment |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | TNO | 1992 | Acute (4-hour) inhalation toxicity study of butene-2 in rats |  | Departments of Biological Toxicology and Experimental Biology, TNO-Toxicology and Nutrition Institute, PO Box 360, 3700 AJ Zeist, The Netherlands | V 92.183 / 352130 | Dow Europe, Horgen, Switzerland |  | 1992-02-25 |

Data access

other: The data rights for this study have been acquired by the Lower Olefins and Aromatics REACH Consortium. The registrant has a letter of access for all studies within this dossier that are considered proprietary.

Data protection claimed

yes, but willing to share (A letter of access to the studies is available on application to Lower Olefins and Aromatics REACH Consortium. )

Cross-reference to same study

not applicable

Materials and methods

Test type

standard acute method

Limit test

yes

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | OECD Guideline 403 (Acute Inhalation Toxicity) | no |

Principles of method if other than guideline

not applicable

GLP compliance

yes (incl. certificate)

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| Common name | butene-2 |

Details on test material

- Name of test material (as cited in study report): Butene-2 - Physical state: gas - Analytical purity: ≥ 95% cis and trans-2-butene - Composition of test material, percentage of components: n-butane (1.11 vol%), 1-butene (0.935 vol%), isobutylene (+/- 200 vol ppm), ethylacetylene (< 20 vol ppm), 1,3 butadiene (240 vol ppm), unknown C4 (0.218 vol% as C4H10), trans-2-butene 55.3 vol%), cis-2-butene (42.4 vol%). - Storage condition of test material: ambient temperature

Confidential details on test material

none

Test animals

Species

rat

Strain

other: Crl:WI (WU) BR

Sex

male/female

Details on test animals and environmental conditions

TEST ANIMALS - Source: Charles River, Wiga, Sulzfeld, FRG - Age at study initiation: 9-10 weeks - Weight at study initiation: mean weight males 277g, mean weight females 171 g - Housing: 5/cage - Diet: TNO Institute stock diet ad libitum - Water: ad libitum - Acclimation period: 27 days ENVIRONMENTAL CONDITIONS - Temperature: 21.5-23.0°C - Humidity: 38-67% (up to 87% for short periods during cleaning) - Air changes: 10 per hr - Photoperiod: 12hrs dark / 12hrs light IN-LIFE DATES: From: 30 March 1992 To: 13 April 1992

Administration / exposure

Route of administration

inhalation

Type of inhalation exposure

whole body

Vehicle

other: air

Details on inhalation exposure

GENERATION OF TEST ATMOSPHERE / CHAMBER DESCRIPTION - Exposure apparatus: modified H 1000 multi-tiered inhalation chamber with pyramidal top and bottom, constructed of stainless steel with 2 glass doors. - Exposure chamber volume: 1.2 m3 - Method of holding animals in test chamber: Individually held in wire mesh stainless steel cages - Rate of air-flow: 21.4 ± 0.6 m3 per hour - Treatment of exhaust air: - Temperature, humidity, pressure in air chamber: Mean temp = 23.1±0.1°C, mean RH = 49±2%, negative pressure = 1-4mm water column. TEST ATMOSPHERE - Brief description of analytical method used: total carbon analyser - Samples taken from breathing zone: yes

Analytical verification of test atmosphere concentrations

yes (10,057 ± 276 ppm)

Duration of exposure

4 h

Concentrations

Target concentration of 10,000 ppm (23 g/m3), analytical concentration 10,057 ± 276 ppm

No. of animals per sex per dose

5

Control animals

no

Details on study design

- Duration of observation period following administration: 14 days - Frequency of observations and weighing: observed just before, during and immediately after exposure and once daily thereafter. Bodyweights recorded just prior to exposure and on days 7 and 14., - Necropsy of survivors performed: yes

Statistics

none

Any other information on materials and methods incl. tables

none

Results and discussions

Preliminary study (if fixed dose study)

none

Effect levels

| Sex | Endpoint | Effect level | Based on | 95% CL | Exp. duration | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| male/female | LC50 | > 10000 ppm (22,948 mg/m3, 23 mg/L nominal) |  |  | 4 h | no effects observed at the highest concentration tested |

Mortality

none

Clinical signs

Restless during 1st hour of exposure. No abnormalities during 14 day observation period.

Body weight

Normal bodyweight gain except for two females that showed reduced bodyweight gain.

Gross pathology

no abnormalities

Other findings

none

Any other information on results incl. tables

none

Overall remarks, attachments

Overall remarks

REACH requires inhalation exposure units to be in mg/m3. Units of exposure in this report are given as ppm, ppm have been converted into mg/m3 using a molecular weight of 56.11 and constant of 24.45 (molar vol of a gas at 25°Cand 1 atm pressure).

Applicant's summary and conclusion

Interpretation of results

not classified

Criteria used for interpretation of results

EU

Conclusions

From the results of this study, it was concluded that the 4-hour acute inhalation LC50 value of butene-2 was greater than 10,000 ppm ( 22,948 mg/m3, 23mg/L).

Executive summary

Rats were exposed for 4 hrs to 2-butene at a nominal concentration of 10,000 ppm ( 22,948 mg/m3). No clinical signs were seen and normal growth occurred over the 14d observation period. No abnormalities were observed at gross necropsy. Only one concentration was tested. This concentration was at the explosive limit and therefore higher concentrations could not be tested.

Cross-reference to other study

none

Endpoint study record: Acute toxicity: inhalation. Supporting study. Virtue, 1950

| UUID |  | IUC5-70c3547a-e2f3-4246-aedb-20b30ad26fe3 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V1.0 Plug-In |
| Date |  | 2010-02-23 14:40:27 GMT |
| Remarks |  | Successfully migrated to IUCLID 5.2 format. |

Administrative Data

| EU: REACH |  |  |  |
| --- | --- | --- | --- |
| Purpose flag | supporting study; robust study summary |  |  |
| Study result type | experimental result |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Non-GLP, non-guideline study, published in peer reviewed literature, minor restrictions in design and/or reporting but otherwise adequate for assessment |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Virtue RW | 1950 | Anaesthetic effects in mice and dogs of some unsaturated hydrocarbons and carbon-oxygen ring compounds | Proc Soc Exptl Biol Med, 1950 Vol 73, pp 259-262 |  |  |  |  |  |

Data access

data published

Cross-reference to same study

This study also contains data on other butene isomers (but-1-ene, 2-butene and isobutene), these are listed as separate IUCLID entries under the relevant isomer.

Materials and methods

Test type

other:

Limit test

no

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| no guideline available |  |  |

Principles of method if other than guideline

To show the relationship between chemical constitution and physiological activity. The concentration required to induce surgical anaesthesia and respiratory arrest in mice and dogs was measured.

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| Common name | butene-1 |

Details on test material

- Name of test material (as cited in study report): butene-1 - Analytical purity: 99.88% (by mass spectrometer) - Boiling point: -6°C

Confidential details on test material

none

Test animals

Species

other: mouse and dog

Strain

no data

Sex

no data

Details on test animals and environmental conditions

no data

Administration / exposure

Route of administration

inhalation

Type of inhalation exposure

whole body

Vehicle

other: air

Details on inhalation exposure

The experiments were carried out in a large jar equipped through the rubber stopper with apparatus that provided for introduction of known quantities of oxygen or anaesthetics compound while maintaining atmospheric pressure. Carbon dioxide was absorbed by soda lime. The oxygen concentration was kept at 21%. The experiments were carried out between 25 and 27° C.

Analytical verification of test atmosphere concentrations

no data

Duration of exposure

< 20 min

Concentrations

Varying levels of concentration of compounds were used. In order to standardize results the experiments were limited to concentrations of anaesthetics agents that would afford surgical anaesthesia within 10 minutes. Experiments were terminated after 20 minutes of anaesthesia.

No. of animals per sex per dose

Not reported. At least 64 mice were used to establish the endpoints given below.

Control animals

no data

Details on study design

see below

Statistics

Probit analysis (Finney, 1947)

Any other information on materials and methods incl. tables

Surgical anaesthesia was accepted as the condition in which the righting reflex had been lost and in which the mice failed to respond to pinching a leg against the side of the jar with a stirring rod. To test the degree of sensitization of cardiac tissue to epinephrine by anaesthetic, dogs were anesthetized and epinephrine was injected according to the technique of Meek, et al (1937).

Results and discussions

Preliminary study (if fixed dose study)

see below

Effect levels

| Sex | Endpoint | Effect level | Based on | 95% CL | Exp. duration | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| no data | other: respiratory arrest occurred in mice | ca. 27.2 % (624,209 mg/m3 or 624 mg/L) |  |  | 10 min |  |

Mortality

Respiratory arrest occurred at levels of 27.2% in atmosphere

Clinical signs

Surgical anaesthesia was observed at levels of 22.7% in atmosphere

Body weight

Not reported

Gross pathology

not reported

Other findings

Not reported

Any other information on results incl. tables

Induction of anaesthesia in mice with butene-1 was accompanied by tachypnea and mild tremors of legs and head. A few tremors of hind legs were seen during anaesthesia. Sleep was generally quiet but the mice were not well relaxed. Recovery was characterized by wild excitement lasting from 15 to 30 seconds. Respiratory arrest in mice preceded circulatory failure. All mice appeared healthy for a period of at least 2 weeks after exposure. Electrocardiograms were made during injection of epinephrine when dogs were anesthetized with butene-1. Arrhythmias were produced.

Overall remarks, attachments

Overall remarks

REACH requires inhalation exposure units to be in mg/m3. Units of exposure in this paper are given as percentages. 1% is equivalent to 10 000 ppm, ppm have been then converted into mg/m3using molecular weight of 56.11 and constant of 24.45 (molar vol of a gas at 25oC and 1 atm pressure).

Applicant's summary and conclusion

Interpretation of results

not classified

Criteria used for interpretation of results

EU

Conclusions

In a non-guideline study conducted in mice, inhalation of 1-butene for 10 minutes produced anaesthesia at a concentration of 22.7% and respiratory arrest at a concentration of 27.2%. The anaesthetic index was 1.2.

Executive summary

This is an old, non-guideline study, with the main part of the study conducted in mice, but there are few acute toxicity studies available on all butene isomers. (No) effect levels cannot be derived from this study. However, inhalation of 1-butene for 10 minutes produced anaesthesia in mice at a concentration of 22.7% (520,939 mg/m3 or 520 mg/L) and respiratory arrest at 27.2% in atmosphere (624,209 mg/m3 or 624 mg/L).

Cross-reference to other study

none

7.5 Repeated dose toxicity

7.5.2 Repeated dose toxicity: inhalation

Endpoint study record: Repeated dose toxicity: inhalation, Key study, Huntingdon, 2003

| UUID |  | IUC5-f6f35bab-ba15-49b0-89ab-1af545657c8a |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V1.0 Plug-In |
| Date |  | 2010-04-11 17:39:12 BST |
| Remarks |  | Successfully migrated to IUCLID 5.2 format. |

Administrative Data

| EU: REACH |  |  |  |
| --- | --- | --- | --- |
| Purpose flag | key study; robust study summary |  |  |
| Study result type | experimental result |  |  |
| Reliability | 1 (reliable without restriction) |  |  |
| Rationale for reliability incl. deficiencies | GLP compliant, guideline study, available as unpublished report, fully adequate for assessment |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | Huntingdon | 2003 | 1-Butene: A combined repeated exposure toxicity, reproduction and neurotoxicity screening in rats via wholebody inhalation exposures. |  | Huntingdon Life Sciences | 02-4224 | American Chemistry Council Olefins Panel |  | 2003-08-01 |

Data access

other: The data rights for this study have been acquired by the Lower Olefins and Aromatics REACH Consortium. The registrant has a letter of access for all studies within this dossier that are considered proprietary.

Data protection claimed

yes, but willing to share (A letter of access to the studies is available on application to Lower Olefins and Aromatics REACH Consortium. )

Cross-reference to same study

The reproductive element of this study is recorded in Toxicity to Reproduction Section 7.8.1

Materials and methods

Test type

other: Combined repeated exposure toxicity, reproduction and neurotoxicity screening in rats via whole-body inhalation exposures

Limit test

no

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | OECD Guideline 422 (Combined Repeated Dose Toxicity Study with the Reproduction / Developmental Toxicity Screening Test) | no |

Principles of method if other than guideline

Not applicable

GLP compliance

yes

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 106-98-9 |
| Common name | 1-butene |

Details on test material

- Name of test material (as cited in study report): 1-butene - Analytical purity: ≥99%

Confidential details on test material

none

Test animals

Species

rat

Strain

other: Crl:CD® (Sprague-Dawley) IGS BR

Sex

male/female

Details on test animals and environmental conditions

TEST ANIMALS - Source: Charles River Laboratories, Kingston, New York, USA - Age at study initiation: c.a. 8 weeks - Weight at study initiation: Males 200-300g, females 150-250 g - Housing: Individually in stainless steel, wire mesh cages within a 1.0 m3 glass and stainless steel whole body exposure chamber - Diet: Certified Rodent Diet No. 5002 (PMI Nutrition International, St. Louis, MO, USA) ad libitum except during exposure - Water: Mains water ad libitum except during exposure. - Acclimation period: 2 weeks ENVIRONMENTAL CONDITIONS - Temperature: 20-25°C - Humidity: 36-82% - Air changes: Not reported - Photoperiod: 12 hrs dark / 12 hrs light IN-LIFE DATES: From: 1 July 2002 To: 25 August 2002

Administration / exposure

Route of administration

inhalation: gas

Type of inhalation exposure

whole body

Vehicle

other: air

Details on inhalation exposure

GENERATION OF TEST ATMOSPHERE / CHAMBER DESCRIPTION The whole-body exposure chambers each had a volume of approximately 1000 L. The chambers were operated at a minimum flow rate of 200 L/minute. The final airflow was set to provide at least one air change in 5.0 minutes (12 air changes/hour) and a T99 equilibrium time of at most 23 minutes At the end of the 6-hour exposure, all animals remained in the chambers for a minimum of 30 minutes. During this time, the chambers were operated at approximately the same flow rate using clean air only. The chambers were exhausted through the in housed filtering system, which consisted of a coarse filter, a HEPA filter and activated charcoal. For the treated groups, the test article was delivered from the cylinder via a regulator with a backpressure gauge via 1/4" tubing through a flowmeter via 1/4" tubing. The outlet of the flowmeter was regulated by a built-in metering valve. The test article laden airstream was directed into the turret of a 1.0 m3 glass and stainless steel exposure chamber via 1/4" tubing, where it was diluted with room air as it was drawn into the chamber. For controls, room air was drawn into the turret of the 1.0 m3 glass and stainless steel exposure chamber. TEST ATMOSPHERE During each exposure, measurements of airborne concentrations were performed in the animals' breathing zone at least 4 times using an appropriate sampling procedure and on-line GC analytical method. During each week of exposure, particle size determinations were performed using a TSI Aerodynamic Particle Sizer to characterize the aerodynamic particle size distribution of any aerosol present.

Analytical verification of doses or concentrations

yes

Details on analytical verification of doses or concentrations

The mean (± standard deviation) analytical (GC) concentrations for the control and the exposure groups were as follows: 0 ± 0, 524 ± 40, 2062 ± 126 and 8271 ± 683 ppm. The analytically measured exposure levels of the airborne test article were reasonably close to the targeted exposure levels.

Duration of treatment / exposure

28 days

Frequency of treatment

6 hours/day, 7 days/week

Doses/concentrations

| 0, 500, 2000, or 8000 ppm |  |
| --- | --- |
| Basis | other: target concentrations |

| 0 ± 0, 524 ± 40, 2062 ± 126 and 8271 ± 683 ppm |  |
| --- | --- |
| Basis | analytical conc. |

MMAD / GSD

MMAD: 0.822 ± 0.1, 0.893 ± 0.2, 0.7986 ± 0.0343 and 3.6503 ± 7.0 for 0, 500, 2000 and 8000 ppm respectively GSD: 2.019 ± 0.4, 1.869 ± 0.3, 1.893 ± 0.3 and 2.181 ± 0.8 for 0, 500, 2000 and 8000 ppm respectively

No. of animals per sex per dose

12

Control animals

yes, sham-exposed

Details on study design

The study design included the main study for repeated dose toxicity end points and reproductive/ developmental toxicity satellite groups (summarized separately). The reproductive and developmental toxicity satellite groups (12 females per exposure level) were exposed for two weeks prior to breeding, during breeding, and continuing through day 19 of gestation. Males from the main study were used to breed these females.

Positive control

None

Examinations

Observations and examinations performed and frequency

Effects on general toxicity, neurobehavioural activity, clinical chemistry, coagulation and haematology were evaluated. CAGE SIDE OBSERVATIONS: Yes - Time schedule: at least twice/day DETAILED CLINICAL OBSERVATIONS: Yes - Time schedule: prior to randomisation and then once/week BODY WEIGHT: Yes - Time schedule for examinations: weekly FOOD CONSUMPTION: Yes - Time schedule for examinations: weekly FOOD EFFICIENCY: No WATER CONSUMPTION: No OPHTHALMOSCOPIC EXAMINATION: No HAEMATOLOGY: Yes - Time schedule for collection of blood: at termination - Anaesthetic used for blood collection: Yes (carbon dioxide/oxygen) - Animals fasted: Yes - How many animals: 12/sex/group - Parameters examined: erythrocyte count, haematocrit, haemoglobin, MCV, MCHC, leucocyte count (total and differential), platelet count, reticulocyte count, erythrocyte morphology, prothrombin time, activated partial thromboplastin CLINICAL CHEMISTRY: Yes - Time schedule for collection of blood: at termination - Anaesthetic used for blood collection: Yes (carbon dioxide/oxygen) - Animals fasted: Yes - How many animals: 12/sex/group - Parameters examined: alanine aminotransferase, aspartate aminotransferase, alkaline phosphatase, gamma glutamyl transpeptidase, urea nitrogen, creatinine, glucose, total cholesterol, triglycerides, total protein, albumin, globulin, albumin/globulin ratio, total bilirubin, sodium, potassium, chloride, calcium, phosphorus URINALYSIS: No NEUROBEHAVIOURAL EXAMINATION: Yes - Time schedule for examinations: pre-test and during final week of exposure - Dose groups that were examined: All - Battery of functions tested: sensory activity / grip strength / motor activity / rectal temperature

Sacrifice and pathology

GROSS PATHOLOGY: Yes (all animals including those dying spontaneously or killed moribund). The nasal pharynx was preserved but not examined microscopically. ORGAN WEIGHTS: Yes (testes, epididymides, ovaries with oviducts, uterus with vagina, adrenals, brain with brain stem, heart, kidneys, liver, lungs, spleen, thymus) HISTOPATHOLOGY: Yes (control and high dose only initially. Tissues examined: adrenals, bone marrow (femur), brain (medulla/pons, cerebrum and cerebellum), epididymides, heart, kidneys, large intestine (caecum, colon and rectum), liver, lungs (with mainstem bronchi), lymph nodes (mesenteric and mediastinal), mammary gland, ovary with oviduct, prostate, seminal vesicles, small intestine (duodenum, ileum and jejunum), spinal cord, spleen, stomach, testes, thymus, thyroid with parathyroids, tibial nerve, trachea, urinary bladder, uterus with vagina, all macroscopic lesions and tissue masses.

Other examinations

none

Statistics

Mean values of all exposure groups were compared to the mean value for the control group at each time interval. For all parameters except for organ weights, the standard one-way analysis of variance (ANOVA) using the F ratio to assess significance was used. If significant differences among the means were indicated, additional testing was performed using Dunnett's t-test to determine which means were significantly different from the control. Organ weight data was analyzed only by parametric methods. Bartlett's test was performed to determine if groups had equal variances. The standard one-way analysis of variance (ANOVA) using the F ratio to assess significance was used. If significant differences among the means were indicated, additional tests were used to determine which means were significantly different from the control: Dunnett's t-test for homogeneous data, or Cochran and Cox's modified t-test for non-homogeneous data. All t-tests were conducted at the 5% and 1% significance levels. Motor Activity Data was analyzed using split-plot repeated measures ANOVA with model terms for group, animal within group, interval and group by interval interaction. If the group x interval interaction was statistically significant (p=0.05), indicating non-parallelism in the behavioural profile between groups, a separate one-way ANOVA for group effects was performed at each interval. If the response data passed on the parallel hypothesis, an ANOVA (using summed responses over intervals) was used to test for the overall treatment effect which constituted the level hypothesis. If any significant overall treatment group effect was found by any of the above ANOVAs, Dunnett's t-test was used to find groups that differed from control. Analyses were performed for sexes separately and combined. Treatment group effects were deemed significant at the p=0.05 level. Plots, tables, listings, and analyses were generated using SAS version 6.12 for WINDOWS.

Any other information on materials and methods incl. tables

none

Results and discussions

Effect levels

| Endpoint | Effect level | Based on | Sex | Basis for effect level / Remarks |
| --- | --- | --- | --- | --- |
| NOAEC (Toxicity) | 8000 ppm (18359 mg/m3, 18 mg/L) nominal |  | male/female | there were no adverse effects at 8000 ppm, the highest concentration tested |

Results of examinations

Clinical signs and mortality

no effects

Body weight and weight gain

no effects

Food consumption

no effects

Food efficiency

no data

Water consumption

not examined

Ophthalmoscopic examination

not examined

Haematology

no effects

Clinical chemistry

no effects

Urinalysis

not examined

Neurobehaviour

no effects

Organ weights

no effects

Gross pathology

no effects

Histopathology: non-neoplastic

no effects

Histopathology: neoplastic

no effects

Details on results

See below

Any other information on results incl. tables

There were no microscopic findings considered to be related to exposure to 1-butene. In comparison with controls, there was a slightly increased incidence and severity of mixed inflammatory cells in the caecal mucosa of rats exposed to 1-butene at exposure levels of 2000 ppm and above. The caecal mucosa normally contains a small population of mixed inflammatory cells, which acts as a natural defence mechanism against ingested substances or organisms. Increased numbers of inflammatory cells are sometimes seen as a normal spontaneous finding, and this was evident in a few males and females from the control group in this study. Since the finding was present in the control group and there was no clear exposure level response relationship in the treated groups, the increased incidence is considered to be fortuitous and unlikely to be related to treatment with 1 -butene. Other microscopic findings occurred sporadically or showed a similar incidence in control and 1-butene-treated animals. None were considered to be associated with exposure to 1-butene.

Overall remarks, attachments

Overall remarks

REACH requires inhalation exposure units to be in mg/m3. Units of exposure in this report are given as ppm, ppm have been converted into mg/m3 using molecular weight of 56.11 and constant of 24.45 (molar vol of a gas at 25°Cand 1 atm pressure).

Applicant's summary and conclusion

Conclusions

Exposure of male and female rats to target concentrations of 500, 2000 and 8000 ppm of 1-butene resulted in no general systemic effects.

Executive summary

Exposure to 1-butene at target concentrations of 500, 2000, 8000 ppm (approximately 1147, 4589, 18359 mg/m3) did not induce systemic toxicity in male and female rats exposed for 28 days or in pregnant female rats exposed for 14 days pre-mating, through mating and gestation to day 19. No treatment-related effects on body weight, clinical chemistry, organ weights or histopathology were found. Neurotoxicity screening also showed no effects on motor activity or functional observation battery. The NOAECs were at the highest concentration level tested

Cross-reference to other study

none

7.6 Genetic toxicity

7.6.1 Genetic toxicity in vitro

Endpoint study record: Genetic toxicity in vitro. Key study (Ames), Araki et al 1994

| UUID |  | IUC5-92cc4878-bdf6-4f97-b42b-9358181ad1e8 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V1.0 Plug-In |
| Date |  | 2010-02-23 15:05:46 GMT |
| Remarks |  | Successfully migrated to IUCLID 5.2 format. |

Administrative Data

| EU: REACH |  |  |  |
| --- | --- | --- | --- |
| Purpose flag | key study; robust study summary |  |  |
| Study result type | experimental result |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Non-GLP, non-guideline study, published in peer reviewed literature, minor restrictions in design and/or reporting but otherwise adequate for assessment |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Araki A, Noguchi T, Kato F & Matsushima T | 1994 | Improved method for mutagenicity testing of gaseous compounds by using a gas sampling bag | Mutation Research, 307, 335-344 |  |  |  |  |  |

Data access

data published

Cross-reference to same study

This study also contains data on other butene isomers (but-1-ene, 2-butene and isobutene), these are listed as separate IUCLID entries under the relevant isomer.

Materials and methods

Type of genotoxicity

gene mutation

Type of study

bacterial reverse mutation assay (e.g. Ames test)

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| no guideline followed |  | not applicable |

Principles of method if other than guideline

Mutagenicity test on S. Typhimurium TA98, TA100, TA1535 and TA1537 and E. Coli WP2 uvrA using the developed gas exposure method (using a gas sampling bag as an exposure vessel and a preparation vessel of diluted gas).

GLP compliance

no data

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 106-98-9 |
| Common name | 1-butene |

Details on test material

1-butene was obtained from Tokyo Kasei Co. Ltd, Tokyo, Japan

Confidential details on test material

none

Method

Target gene

not applicable

Species/strain

| Species/strain | S. typhimurium TA 1535, TA 1537, TA 98 and TA 100 |
| --- | --- |
| Additional strain characteristics | no data |
| Metabolic activation | with and without |
| Metabolic activation system | S9 from Phenobarbitol and 5,6-benzoflavone induced rat liver |

| Species/strain | E. coli WP2 uvr A |
| --- | --- |
| Additional strain characteristics | no data |
| Metabolic activation | with and without |
| Metabolic activation system | S9 from Phenobarbitol and 5,6-benzoflavone induced rat liver |

Test concentrations

Toxic concentration levels or about 50% of the maximum exposure concentration.

Vehicle

Vehicle(s)/solvent(s): HEPA-filtered air)

Controls

| Negative controls | no data |
| --- | --- |
| Solvent / vehicle controls | no data |
| True negative controls | no data |
| Positive controls | no data |
| Positive control substance | no data |
| Remarks | The objective was to develop an improved method for testing gases. Use of standard positive and negative control substances was not stated, buta,-1,3-diene was used as a positive substance & the data presented indicates that an air control was used |

Details on test system and conditions

Mutagenicity testing was performed by the gas exposure method using a gas sampling bag with or without metabolic activation. The exposure condition was : bacterial plates made by the agar overlay method with 2mL top agar per plate, 100 microlitres of S9 per plate, exposure temperature of 37°C, exposure period of 48 h and exposure volume of gas was 500 mL per plate. Tests were performed at toxic concentration levels or about 50% levels as the maximum concentration. The conditions listed above were chosen based on the results of a preliminary study with 1,3 butadiene in which the effect of the volume of gas per plate (357, 625, 1250, 2500 or 5000 mL/plate, which corresponded to 14, 8, 4, 2 or 1 plates/bag), the amount of S9 (50, 100, 200 or 400 microlitres/plate), the exposure temperature (30 or 37 degrees C), exposure period (2, 4, 14, 24 or 48 hours), the amount of top agar (0, 1, or 2 mL/plate) were examined.

Evaluation criteria

After incubation the number of revertant colonies was counted.

Statistics

none

Any other information on materials and methods incl. tables

Preparation of gas:A fixed volume of test substance gas was pumped out from the gas sampling bag into another gas sampling bag (gas dilution bag), which had been filled with a fixed amount of dilution gas. Exposure: The bacterial plates were placed in a 10L gas sampling bag which was resealed tightly. A flow meter and pump were connected to the bag and after washing out the air it was filled with test substance gas at appropriate concentration at a fixed amount per plate. The bacterial plates in the bag were kept at 25 or 37°C for a fixed period. At the end of exposure period the test gas was replaced by sterile air and then plates were removed and left to stand for 30 min in order to evaporate test substance, then lids replaced. Incubation after exposure: The plates were then transferred to a container or vinyl bag, separately according to the concentration and method used, and were then incubated for a total exposure period and incubation period of 48 h. In case of 24 h exposure at 37°C, the incubation period was 24 h. After incubation revertant colonies were counted.

Results and discussions

Test results

| Species/strain | S. typhimurium TA 1535, TA 1537, TA 98 and TA 100 |
| --- | --- |
| Metabolic activation | with and without |
| Test system | all strains/cell types tested |
| Genotoxicity | negative |
| Cytotoxicity | no, but tested up to limit concentrations |
| Vehicle controls valid | no data |
| Negative controls valid | no data |
| Positive controls valid | no data |

| Species/strain | E. coli WP2 uvr A |
| --- | --- |
| Metabolic activation | with and without |
| Test system | all strains/cell types tested |
| Genotoxicity | negative |
| Cytotoxicity | no, but tested up to limit concentrations |
| Vehicle controls valid | no data |
| Negative controls valid | no data |
| Positive controls valid | no data |

Additional information on results

none

Any other information on results incl. tables

none

Overall remarks, attachments

Overall remarks

none

Applicant's summary and conclusion

Interpretation of results

negative with metabolic activation

negative without metabolic activation

Conclusions

1-butene was not mutagenic with and without metabolic activation, in S. Typhimurium strains: TA 1535, TA 1537, TA 98 and TA 100 and E. Coli WP2 uvr A.

Executive summary

1-butene was not mutagenic to S Typhimuirium TA98, TA100, TA1535, TA 1537 and E.coli WP2uvrA, with or without metabolic activation, using a gas sampling bag method. Concentrations were not specified but tests were carried out at increasing levels until toxicity was observed. Buta-1,3-diene was mutagenic under these conditions indicating that the method was robust.

Cross-reference to other study

none

Endpoint study record: Genetic toxicity in vitro. Key study (Chomosome Aberration), Safepharm, 1992

| UUID |  | IUC5-c9290751-8aa6-44ad-a66f-a0d29add7285 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | Thomas Penman / LOA Reach Services Team / Tervuren / Belgium |
| Date |  | 2010-07-14 22:43:25 BST |
| Remarks |  |  |

Administrative Data

| EU: REACH |  |  |  |
| --- | --- | --- | --- |
| Purpose flag | key study; robust study summary |  |  |
| Study result type | experimental result |  |  |
| Reliability | 1 (reliable without restriction) |  |  |
| Rationale for reliability incl. deficiencies | GLP, guideline study, available as unpublished report, no restrictions, fully adequate for assessment |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | Safepharm | 1992 | Butene-2: Metaphase analysis in rat lymphocytes in vitro |  | SafePharm Laboratories Limited, PO Box No. 45, Derby, DE1 2BT, UK | 44/813 | Dow Europe S.A., Bachtobelstrasse 3, CH-8810 Horgen, Switzerland |  | 1992-10-19 |

Data access

other: The data rights for this study have been acquired by the Lower Olefins and Aromatics REACH Consortium. The registrant has a letter of access for all studies within this dossier that are considered proprietary.

Data protection claimed

yes, but willing to share (A letter of access to the studies is available on application to Lower Olefins and Aromatics REACH Consortium. )

Cross-reference to same study

not applicable

Materials and methods

Type of genotoxicity

chromosome aberration

Type of study

in vitro mammalian chromosome aberration test

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | OECD Guideline 473 (In vitro Mammalian Chromosome Aberration Test) | no |
| according to | EU Method B.10 (Mutagenicity - In Vitro Mammalian Chromosome Aberration Test) | no |

Principles of method if other than guideline

Not applicable

GLP compliance

yes (incl. certificate)

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| Common name | Butene-2 |

Details on test material

- Name of test material (as cited in study report): Butene-2 (cis and trans-2-butene) - Physical state: gas - Supplied by: Union Carbide Industrial Gases - Analytical purity: Not reported - Lot/batch No.: Not reported - Storage condition of test material: room temperature

Confidential details on test material

none

Method

Target gene

not applicable

Species/strain

| Species/strain | lymphocytes: rat |
| --- | --- |
| Details on mammalian cell lines (if applicable) | - Type and identity of media: RPMI, supplemented with 10% foetal calf serum (FCS), 25mM Hepes and antibiotics |
| Metabolic activation | with and without |
| Metabolic activation system | S9 from Arochlor induced rat liver |

Test concentrations

40, 50 and 60% (selected from 0, 10, 20, 40, 50, 60, 80 and 100% -initial series of dose levels)

Vehicle

- Vehicle(s)/solvent(s) used: none (air only)

Controls

| Solvent / vehicle controls | yes (clean air) |
| --- | --- |
| Positive controls | yes |
| Positive control substance | other: EMS 500- 500µg/mL without S9; cyclophoshamide (CP) 4.2µg/mL with S9. Vinyl chloride was used as a positive control at an atmospheric concentration of approx 50%. |

Details on test system and conditions

METHOD OF APPLICATION: in medium in culture flasks DURATION - Exposure duration: 4 hours with S9 activation; 20 hours without S9 activation. - Expression time (cells in growth medium): With S9: cell harvest after 16 and 26 hour expression periods (i.e. 20 and 30 hours after treatment initiation). Without S9: 20 hours after treatment initiated) NUMBER OF REPLICATIONS: 2 NUMBER OF CELLS EVALUATED: 2000 lymphocyte cell nuclei for each culture DETERMINATION OF CYTOTOXICITY - Method: mitotic index OTHER:

Evaluation criteria

Positive if % cells with aberrations (gaps, chromatid breaks, chromosome breaks, exchanges, multiple aberrations and chromosome number) markedly exceeds those seen in concurrent control with/without clear dose response relationship. For modest increases in aberration frequency, dose-response generally required and statistical tests employed.

Statistics

Fischer's Exact test

Any other information on materials and methods incl. tables

Rat lymphocytes, exposed to butene-2 were evaluated for chromosome aberrations at three dose levels, in duplicate, together with negative and positive controls. Three treatment conditions were used, i.e. 4 hours exposure with the addition of an induced rat liver homogenate metabolising system at 20% in standard co-factors with cell harvest after 16 and 26 hour expression periods and a 20 hour continuous exposure in the absence of activation. The dose range was selected from a series of 7 dose levels and was 40-60%.

Results and discussions

Test results

| Species/strain | lymphocytes: |
| --- | --- |
| Metabolic activation | with and without |
| Genotoxicity | negative |
| Cytotoxicity | yes (At a target concentration of 80% for 20-hour harvest group) |
| Vehicle controls valid | yes |
| Positive controls valid | yes |

Additional information on results

Butene-2 induced a very steep dose-related decrease in the mitotic index both with and without S9. Butene-2 was therefore considered to be toxic to rat lymphocytes in vitro at the target concentration of 80% for the 20-hour harvest group.

Any other information on results incl. tables

Butene-2 produced no significant increases in the frequency of chromosome aberrations either in the presence or absence of a liver enzyme metabolising system. Butene-2 is, therefore, considered to be non-clastogenic to rat lymphocytes in vitro.

Overall remarks, attachments

Overall remarks

none

Applicant's summary and conclusion

Interpretation of results

negative with metabolic activation

negative without metabolic activation

Conclusions

2-Butene was not clastogenic to rat lymphocytes in vitro.

Executive summary

A chromosome aberration study was conducted with butene-2. No significant increases were seen in the frequency of chromosome aberrations either in the presence or absence of a liver enzyme metabolising system. Positive control compounds gave the expected results demonstrating the validity of the test. Butene-2 was therefore, considered to be non-clastogenic to rat lymphocytes in vitro.

Cross-reference to other study

none

7.8 Toxicity to reproduction

7.8.1 Toxicity to reproduction

Endpoint study record: Toxicity to reproduction, Key study, Huntingdon, 2003

| UUID |  | IUC5-d6509775-7eca-4ff8-a6ca-929472f6775e |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V1.0 Plug-In |
| Date |  | 2010-02-02 16:09:44 GMT |
| Remarks |  | Successfully migrated to IUCLID 5.2 format. |

Administrative Data

| EU: REACH |  |  |  |
| --- | --- | --- | --- |
| Purpose flag | key study; robust study summary |  |  |
| Study result type | experimental result |  |  |
| Reliability | 1 (reliable without restriction) |  |  |
| Rationale for reliability incl. deficiencies | GLP compliant, guideline study, available as unpublished report, fully adequate for assessment |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | Huntingdon | 2003 | 1-Butene: A combined repeated exposure toxicity, reproduction and neurotoxicity screening in rats via wholebody inhalation exposures. |  | Huntingdon Life Sciences | 02-4224 | American Chemistry Council Olefins Panel |  | 2003-08-01 |

Data access

other: The data rights for this study have been acquired by the Lower Olefins and Aromatics REACH Consortium. The registrant has a letter of access for all studies within this dossier that are considered proprietary.

Data protection claimed

yes, but willing to share (A letter of access to the studies is available on application to Lower Olefins and Aromatics REACH Consortium. )

Cross-reference to same study

The details of the repeat dose toxicity element of this study is recorded in Repeated Dose Toxicity Section 7.5.3

Materials and methods

Test type

screening

Limit test

no

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | OECD Guideline 422 (Combined Repeated Dose Toxicity Study with the Reproduction / Developmental Toxicity Screening Test) | no |

Principles of method if other than guideline

not applicable

GLP compliance

yes

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 106-98-9 |
| Common name | 1-butene |

Details on test material

- Name of test material (as cited in study report): 1-butene - Analytical purity: ≥99%

Confidential details on test material

none

Test animals

Species

rat

Strain

other: Crl:CD® (Sprague-Dawley) IGS BR

Sex

male/female

Details on test animals and environmental conditions

- Source: Charles River Laboratories, Kingston, New York, USA. - Age at study initiation: c.a. 8 weeks - Weight at study initiation: Males 200-300g, females 150-250 g - Housing: Individually in stainless steel, wire mesh cages within a 1.0 m3 glass and stainless steel whole body exposure chamber except during the mating period when one male and one female were housed together in a cage of suitable size. - Diet: Certified Rodent Diet No. 5002 (PMI Nutrition International, St. Louis, MO, USA) ad libitum except during exposure - Water: Mains water ad libitum except during exposure. - Acclimation period: Approximately 2 weeks ENVIRONMENTAL CONDITIONS - Temperature: 20-25°C - Humidity: 36-82% - Air changes: Not reported - Photoperiod: 12 hrs dark / 12 hrs light IN-LIFE DATES: From: 1 July 2002 To: 25 August 2002

Administration / exposure

Route of administration

inhalation: gas

Type of inhalation exposure (if applicable)

whole body

Vehicle

other: air

Details on exposure

GENERATION OF TEST ATMOSPHERE / CHAMBER DESCRIPTION The whole-body exposure chambers each had a volume of approximately 1000 L. The chambers were operated at a minimum flow rate of 200 L/minute. The final airflow was set to provide at least one air change in 5.0 minutes (12 air changes/hour) and a T99 equilibrium time of at most 23 minutes At the end of the 6-hour exposure, all animals remained in the chambers for a minimum of 30 minutes. During this time, the chambers were operated at approximately the same flow rate using clean air only. The chambers were exhausted through the in housed filtering system, which consisted of a coarse filter, a HEPA filter and activated charcoal. For the treated groups, the test article was delivered from the cylinder via a regulator with a backpressure gauge via 1/4" tubing through a flowmeter via 1/4" tubing. The outlet of the flowmeter was regulated by a built-in metering valve. The test article laden airstream was directed into the turret of a 1.0 m3 glass and stainless steel exposure chamber via 1/4" tubing, where it was diluted with room air as it was drawn into the chamber. For controls, room air was drawn into the turret of the 1.0 m3 glass and stainless steel exposure chamber. TEST ATMOSPHERE During each exposure, measurements of airborne concentrations were performed in the animals' breathing zone at least 4 times using an appropriate sampling procedure and on-line GC analytical method. During each week of exposure, particle size determinations were performed using a TSI Aerodynamic Particle Sizer to characterize the aerodynamic particle size distribution of any aerosol present.

Details on mating procedure

- M/F ratio per cage: 1 male and 1 female - Length of cohabitation: until evidence of mating seen or for 2 weeks - Proof of pregnancy: vaginal plug / sperm in vaginal smear referred to as day 0 of pregnancy - After successful mating each pregnant female was caged Individually

Analytical verification of doses or concentrations

yes

Details on analytical verification of doses or concentrations

The mean (± standard deviation) analytical (GC) concentrations for the control and the exposure groups were as follows: 0 ± 0, 524 ± 40, 2062 ± 126 and 8271 ± 683 ppm. The analytically measured exposure levels of the airborne test article were reasonably close to the targeted exposure levels.

Duration of treatment / exposure

Two weeks prior to breeding, during breeding, and continuing through day 19 of gestation. The dams were then allowed to deliver their litters, which were retained until lactation day 4.

Frequency of treatment

6 hours/day, 7 days/week

Details on study schedule

see below

Doses / concentrations

| 0, 500, 2000, or 8000 ppm |  |
| --- | --- |
| Basis | other: target concentrations |

| 0 ± 0, 524 ± 40, 2062 ± 126 and 8271 ± 683 ppm |  |
| --- | --- |
| Basis | analytical conc. |

No. of animals per sex per dose

12

Control animals

yes, sham-exposed

Further details on study design

MMAD: 0.822 ± 0.1, 0.893 ± 0.2, 0.7986 ± 0.0343 and 3.6503 ± 7.0 for 0, 500, 2000 and 8000 ppm respectively GSD: 2.019 ± 0.4, 1.869 ± 0.3, 1.893 ± 0.3 and 2.181 ± 0.8 for 0, 500, 2000 and 8000 ppm respectively

Positive control

None

Examinations

Parental animals: Observations and examinations

CAGE SIDE OBSERVATIONS: Yes - Time schedule: At least twice/day DETAILED CLINICAL OBSERVATIONS: Yes - Time schedule: Females weekly during pre-mating and on gestations days 0, 7, 14, 20 and lactation days 0, 1 and 4. BODY WEIGHT: Yes - Time schedule: Females day treatment initiated, twice/week until copulation confirmed, gestations days 0, 7, 14 and 20, lactation days 1 and 4. Females without evidence of mating were weighed weekly. All weighed at termination. FOOD CONSUMPTION: Yes - Time schedule: Females weekly during pre-mating period, gestation days 0-7, 7-14, 14-20 and lactation days 1-4.

Estrous cyclicity (Parental animals)

not determined

Sperm parameters (Parental animals)

not determined

Litter observations

STANDARDISATION OF LITTERS - Performed on day 4 postpartum: No PARAMETERS EXAMINED The following parameters were examined as soon as possible after parturition and twice/day thereafter: number of live and dead pups and sex of pups, physical or behavioural abnormalities. GROSS EXAMINATION OF DEAD PUPS: yes, for external and internal abnormalities and presence/absence of milk in stomach

Postmortem examinations (Parental animals)

SACRIFICE - Maternal animals: Mated animals on day 5 post partum, animals when evidence of mating was detected but failed to deliver 26 days after evidence of mating, animals with no evidence of mating 26 days after completion of mating period. GROSS NECROPSY - Maternal animals: Gross necropsy consisted of external and internal examinations including the cervical, thoracic, and abdominal viscera. Number of implantation sites and corpora lutea were assessed. ORGAN WEIGHTS Maternal animals: ovaries with oviducts, uterus with vagina, adrenals, brain with brainstem, heart, kidneys, liver, thymus, spleen, lungs. MICROSCOPIC EXAMINATION: No

Postmortem examinations (Offspring)

External macroscopic postmortem examinations performed on all F1 pups found dead during lactation and at termination on lactation day 4. No microscopic examination of pups.

Statistics

Mean values of all exposure groups were compared to the mean value for the control group at each time interval. For all parameters except for organ weights, the standard one-way analysis of variance (ANOVA) using the F ratio to assess significance was used. If significant differences among the means were indicated, additional testing was performed using Dunnett's t-test to determine which means were significantly different from the control. Organ weight data was analyzed only by parametric methods. Bartlett's test was performed to determine if groups had equal variances. The standard one-way analysis of variance (ANOVA) using the F ratio to assess significance was used. If significant differences among the means were indicated, additional tests were used to determine which means were significantly different from the control: Dunnett's t-test for homogeneous data, or Cochran and Cox's modified t-test for non-homogeneous data. All t-tests were conducted at the 5% and 1% significance levels. For incidence data, a Fisher Exact Test with Bonferonni correction was performed to identify differences between the control and treatment groups.

Reproductive indices

Gonadal function, mating behaviour, implantation, and general fertility were evaluated.

Offspring viability indices

Litter size, pup survival, sex, body weight, and the presence of gross external malformations were assessed in the offspring.

Any other information on materials and methods incl. tables

The study design included a main study for repeated dose toxicity end points (summarized separately) and reproductive / developmental toxicity satellite groups of 12 females per exposure level. The reproductive and developmental toxicity satellite groups were exposed for two weeks prior to breeding, during breeding, and continuing through day 19 of gestation. Males from the main study were used to breed these females. The dams were allowed to deliver their litters, which were retained until lactation day 4. Effects on general toxicity, gonadal function, mating behaviour, implantation, and general fertility were evaluated, followed by a gross necropsy of the females on lactation day 4. Litter size, pup survival, sex, body weight, and the presence of gross external malformations were assessed in the offspring.

Results and discussions

Effect levels

| Endpoint | Generation | Sex | Effect level | Based on | Basis for effect level / Remarks |
| --- | --- | --- | --- | --- | --- |
| NOAEC | P | male/female | 8000 (18359 mg/m3, 18.4 mg/m3) nominal |  | no treatment-related effects on dams at the highest concentration tested, no effects on reproductive performance or fertility |
| NOAEC | F1 | male/female | 8000 (18359 mg/m3, 18.4 mg/L) nominal |  | no treatment-related effects on pups at the highest concentration tested, no effects on pup viability, body weight or development |

Results of examinations: parental animals

Clinical signs (parental animals)

no effects

Body weight and food consumption (parental animals)

no effects

Test substance intake (parental animals)

no data

Reproductive function: estrous cycle (parental animals)

not examined

Reproductive function: sperm measures (parental animals)

not examined

Reproductive performance (parental animals)

no effects

Organ weights (parental animals)

no effects

Gross pathology (parental animals)

no effects

Histopathology (parental animals)

no effects

Details on results (parental animals)

see below

Results of examinations: offspring

Viability (offspring)

no effects

Clinical signs (offspring)

no effects

Body weight (offspring)

no effects

Sexual maturation (offspring)

not examined

Organ weights (offspring)

not examined

Gross pathology (offspring)

no effects

Details on results (offspring)

see below

Any other information on results incl. tables

All mated female animals (except one animal in the 2000 ppm group) were found pregnant and delivered live pups. Mating indices for the male rats treated with the test article were comparable to controls. Mating, fertility and gestation indices for the female rats treated with the test article were comparable to controls. Most of the females mated at the first opportunity. There were also no treatment-related differences in the other reproductive parameters up to the time of parturition including the percent of females completing delivery and the duration of gestation, when compared to controls. There were no treatment related differences in all parturition parameters including the total number of pups delivered, the number of pups dying, the viability (4 day survival) index, the number of implantation sites and corpora lutea per dam, the pup sex ratio and the number of live pups/litter, when compared to controls.

Overall remarks, attachments

Overall remarks

REACH requires inhalation exposure units to be in mg/m3. Units of exposure in this report are given as ppm, ppm have been converted into mg/m3 using molecular weight of 56.11 and constant of 24.45 (molar vol of a gas at 25°Cand 1 atm pressure).

Applicant's summary and conclusion

Conclusions

Repeated inhalation exposure of 1-butene to female Sprague Dawley rats at levels of 0, 500, 2000, or 8000 ppm produced no evidence of adverse effects on any measures of reproductive function.

Executive summary

Male and female rats were exposed to 1-butene at target concentrations of 500, 2000, 8000 ppm (1147, 4589, 18359 mg/m3) for two weeks prior to breeding, during breeding and until day 19 of gestation. The dams were then allowed to deliver their litters, which were retained until post-natal day 4. There was no evidence of systemic toxicity in the parents.There were no effects on mating behaviour, fertility and gestation indices, the number of implantation sites and corpora lutea per dam, numbers of pups delivered, viability of pups at and after birth and the pup sex ratio when compared to the control group. Based on these data, the NOAEC for reproductive toxicity was the highest concentration tested (18359 mg/m3). There were no treatment-related effects on the development of pups. There were no effects on body weight gain or observed during macroscopic examination of pups at post mortem. Based on these data, the NOAEC for developmental toxicity was also the highest concentration tested (18359 mg/m3).

Cross-reference to other study

none

7.8.2 Developmental toxicity / teratogenicity

Endpoint study record: Developmental toxicity / teratogenicity. Supporting study. Huntingdon, 2003

| UUID |  | IUC5-76c99e92-7f78-4032-b8c4-00189e09ef23 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V1.0 Plug-In |
| Date |  | 2010-04-11 18:03:41 BST |
| Remarks |  | Successfully migrated to IUCLID 5.2 format. |

Administrative Data

| EU: REACH |  |  |  |
| --- | --- | --- | --- |
| Purpose flag | supporting study; robust study summary |  |  |
| Study result type | experimental result |  |  |
| Reliability | 1 (reliable without restriction) |  |  |
| Rationale for reliability incl. deficiencies | GLP compliant, guideline study, available as unpublished report, no restrictions, fully adequate for assessment |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | Huntingdon | 2003 | 1-Butene: A combined repeated exposure toxicity, reproduction and neurotoxicity screening in rats via wholebody inhalation exposures. |  | Huntingdon Life Sciences | 02-4224 | American Chemistry Council Olefins Panel |  | 2003-08-01 |

Data access

other: The data rights for this study have been acquired by the Lower Olefins and Aromatics REACH Consortium. The registrant has a letter of access for all studies within this dossier that are considered proprietary.

Data protection claimed

yes, but willing to share (A letter of access to the studies is available on application to Lower Olefins and Aromatics REACH Consortium. )

Cross-reference to same study

none

Materials and methods

Limit test

no

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | other guideline: OECD Guideline 422 (Combined Repeated Dose Toxicity Study with the Reproduction / Developmental Toxicity Screening Test) | no |

Principles of method if other than guideline

not applicable

GLP compliance

yes

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 106-98-9 |
| Common name | 1-butene |

Details on test material

- Name of test material (as cited in study report): 1-butene - Analytical purity: ≥99% - Molecular weight: 56.1 - Boiling point: 4°C - Melting point: -139°C - Vapour pressure: 2.5 bars at 20°C - Explosive limits in air: 1.6 - 10%

Confidential details on test material

none

Test animals

Species

rat

Strain

other: Crl:CD® (Sprague-Dawley) IGS BR

Details on test animals and environmental conditions

- Source: Charles River Laboratories, Kingston, New York, USA. - Age at study initiation: c.a. 8 weeks - Weight at study initiation: Males 200-300g, females 150-250 g - Housing: Individually in stainless steel, wire mesh cages within a 1.0 m3 glass and stainless steel whole body exposure chamber except during the mating period when one male and one female were housed together in a cage of suitable size. - Diet: Certified Rodent Diet No. 5002 (PMI Nutrition International, St. Louis, MO, USA) ad libitum except during exposure - Water: Mains water ad libitum except during exposure. - Acclimation period: Approximately 2 weeks ENVIRONMENTAL CONDITIONS - Temperature: 20-25°C - Humidity: 36-82% - Air changes: Not reported - Photoperiod: 12 hrs dark / 12 hrs light IN-LIFE DATES: From: 1 July 2002 To: 25 August 2002

Administration / exposure

Route of administration

inhalation: gas

Type of inhalation exposure (if applicable)

whole body

Vehicle

other: air

Details on exposure

GENERATION OF TEST ATMOSPHERE / CHAMBER DESCRIPTION The whole-body exposure chambers each had a volume of approximately 1000 L. The chambers were operated at a minimum flow rate of 200 L/minute. The final airflow was set to provide at least one air change in 5.0 minutes (12 air changes/hour) and a T99 equilibrium time of at most 23 minutes At the end of the 6-hour exposure, all animals remained in the chambers for a minimum of 30 minutes. During this time, the chambers were operated at approximately the same flow rate using clean air only. The chambers were exhausted through the in housed filtering system, which consisted of a coarse filter, a HEPA filter and activated charcoal. For the treated groups, the test article was delivered from the cylinder via a regulator with a backpressure gauge via 1/4" tubing through a flowmeter via 1/4" tubing. The outlet of the flowmeter was regulated by a built-in metering valve. The test article laden airstream was directed into the turret of a 1.0 m3 glass and stainless steel exposure chamber via 1/4" tubing, where it was diluted with room air as it was drawn into the chamber. For controls, room air was drawn into the turret of the 1.0 m3 glass and stainless steel exposure chamber. TEST ATMOSPHERE During each exposure, measurements of airborne concentrations were performed in the animals' breathing zone at least 4 times using an appropriate sampling procedure and on-line GC analytical method. During each week of exposure, particle size determinations were performed using a TSI Aerodynamic Particle Sizer to characterize the aerodynamic particle size distribution of any aerosol present.

Analytical verification of doses or concentrations

yes

Details on analytical verification of doses or concentrations

The mean (± standard deviation) analytical (GC) concentrations for the control and the exposure groups were as follows: 0 ± 0, 524 ± 40, 2062 ± 126 and 8271 ± 683 ppm. The analytically measured exposure levels of the airborne test article were reasonably close to the targeted exposure levels.

Details on mating procedure

- M/F ratio per cage: 1 male and 1 female - Length of cohabitation: until evidence of mating seen or for 2 weeks - Proof of pregnancy: vaginal plug / sperm in vaginal smear referred to as day 0 of pregnancy - After successful mating each pregnant female was caged Individually

Duration of treatment / exposure

Two weeks prior to breeding, during breeding, and continuing through day 19 of gestation. The dams were then allowed to deliver their litters, which were retained until lactation day 4.

Frequency of treatment

6 hrs/day, 7 days/week

Duration of test

After an initial exposure period of 2 weeks, males and females were mated for 1 week. Females were allowed to litter and to rear their progeny until day 4 of lactation; after which the parent animals and the pups were killed

Doses / concentrations

| 0, 500, 2000, or 8000 ppm |  |
| --- | --- |
| Basis | other: target concentrations . ( 0 ± 0, 524 ± 40, 2062 ± 126 and 8271 ± 683 ppm, analytical conc.) |

No. of animals per sex per dose

12

Control animals

yes, sham-exposed

Further details on study design

none

Examinations

Maternal examinations

CAGE SIDE OBSERVATIONS: Yes - Time schedule: At least twice/day DETAILED CLINICAL OBSERVATIONS: Yes - Time schedule: Females weekly during pre-mating and on gestations days 0, 7, 14, 20 and lactation days 0, 1 and 4. BODY WEIGHT: Yes - Time schedule: Females day treatment initiated, twice/week until copulation confirmed, gestations days 0, 7, 14 and 20, lactation days 1 and 4. Females without evidence of mating were weighed weekly. All weighed at termination. FOOD CONSUMPTION: Yes - Time schedule: Females weekly during pre-mating period, gestation days 0-7, 7-14, 14-20 and lactation days 1-4. SACRIFICE - Maternal animals: Mated animals on day 5 post partum, animals when evidence of mating was detected but failed to deliver 26 days after evidence of mating, animals with no evidence of mating 26 days after completion of mating period.

Ovaries and uterine content

GROSS NECROPSY - Maternal animals: Gross necropsy consisted of external and internal examinations including the cervical, thoracic, and abdominal viscera. Number of implantation sites and corpora lutea were assessed. ORGAN WEIGHTS Maternal animals: ovaries with oviducts, uterus with vagina, adrenals, brain with brainstem, heart, kidneys, liver, thymus, spleen, lungs.

Fetal examinations

STANDARDISATION OF LITTERS - Performed on day 4 postpartum: No PARAMETERS EXAMINED The following parameters were examined as soon as possible after parturition and twice/day thereafter: number of live and dead pups and sex of pups, physical or behavioural abnormalities. GROSS EXAMINATION OF DEAD PUPS: yes, for external and internal abnormalities and presence/absence of milk in stomach Postmortem examinations (Offspring) External macroscopic postmortem examinations performed on all F1 pups found dead during lactation and at termination on lactation day 4. No microscopic examination of pups.

Statistics

Mean values of all exposure groups were compared to the mean value for the control group at each time interval. For all parameters except for organ weights, the standard one-way analysis of variance (ANOVA) using the F ratio to assess significance was used. If significant differences among the means were indicated, additional testing was performed using Dunnett's t-test to determine which means were significantly different from the control. Organ weight data was analyzed only by parametric methods. Bartlett's test was performed to determine if groups had equal variances. The standard one-way analysis of variance (ANOVA) using the F ratio to assess significance was used. If significant differences among the means were indicated, additional tests were used to determine which means were significantly different from the control: Dunnett's t-test for homogeneous data, or Cochran and Cox's modified t-test for non-homogeneous data. All t-tests were conducted at the 5% and 1% significance levels. For incidence data, a Fisher Exact Test with Bonferonni correction was performed to identify differences between the control and treatment groups. Reproductive indices Gonadal function, mating behaviour, implantation, and general fertility were evaluated. Offspring viability indices Litter size, pup survival, sex, body weight, and the presence of gross external malformations were assessed in the offspring.

Indices

none

Historical control data

not cited

Results and discussions

Effect levels

| Endpoint | Effect type | Effect level | Based on | Basis for effect level / Remarks |
| --- | --- | --- | --- | --- |
| NOAEC | maternal toxicity | 8000 ppm (18,359 mg/m3, 18.4 mg/L) nominal |  | no treatment-related effects on dams at the highest concentration tested |
| NOAEC | developmental toxicity | 8000 ppm (18,359 mg/m3, 18.4 mg/L) nominal. |  | no treatment-related effects on pup body weight or development at the highest concentration tested |

Maternal toxic effects

no effects

Details on maternal toxic effects

no effects

Embryotoxic / teratogenic effects

no effects

Details on embryotoxic / teratogenic effects

no effects

Any other information on results incl. tables

BODY WEIGHT AND WEIGHT GAIN (PARENTS) No effects BODY WEIGHT (OFFSPRING) No effects

Overall remarks, attachments

Overall remarks

Reach requires inhalation exposure units to be in mg/m3. Units of exposure in this paper are given as ppm, ppm have been converted into mg/m3using molecular weight of 56.11 and constant of 24.45 (molar vol of a gas at 25°C and 1 atm pressure).

Applicant's summary and conclusion

Conclusions

No effects on or pup development were seen when male and female rats were exposed to 1-butene at concentrations of 2500, 2000, or 8000 ppm for two weeks prior to breeding, during breeding and until day 19 of gestation.

Executive summary

Male and female rats were exposed to 1-butene at target concentrations of 500, 2000, 8000 ppm (1147, 4589, 18,359 mg/m3) for two weeks prior to breeding, during breeding and until day 19 of gestation. The dams were then allowed to deliver their litters, which were retained until post-natal day 4. There was no evidence of significant systemic toxicity in the parents. There were no treatment-related effects on the development of pups. There were no effects on body weight gain or observed during macroscopic examination of pups at post mortem. Based on these data, the NOAEC for reproductive and developmental toxicity was 8000 ppm (18,359 mg/m3), the highest concentration tested.

Cross-reference to other study

none

Endpoint study record: Developmental toxicity / teratogenicity. Key study. CTL, 2002

| UUID |  | IUC5-ebe9823b-505e-431f-b15a-a61a17666849 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | Thomas Penman / LOA Reach Services Team / Tervuren / Belgium |
| Date |  | 2010-07-14 22:43:13 BST |
| Remarks |  |  |

Administrative Data

| EU: REACH |  |  |  |
| --- | --- | --- | --- |
| Purpose flag | key study; robust study summary |  |  |
| Study result type | experimental result |  |  |
| Reliability | 1 (reliable without restriction) |  |  |
| Rationale for reliability incl. deficiencies | GLP compliant, guideline study, available as unpublished report, no restrictions, fully adequate for assessment |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | CTL | 2002 | Isobutylene: prenatal developmental toxicity study in the rat |  | Central Toxicology Laboratory (CTL), Alderley Park, Macclesfield, Cheshire, UK | CTL/RR0907/Regulatory Report | CEFIC Lower Olefins Sector Group, Ave. E Van Nieuwenhuyse 4, Box 2, Brussels, B-1060, Belgium |  | 2002-02-07 |

Data access

other: The data rights for this study have been acquired by the Lower Olefins and Aromatics REACH Consortium. The registrant has a letter of access for all studies within this dossier that are considered proprietary.

Data protection claimed

yes, but willing to share (A letter of access to the studies is available on application to Lower Olefins and Aromatics REACH Consortium. )

Cross-reference to same study

none

Materials and methods

Limit test

no

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | OECD Guideline 414 (Prenatal Developmental Toxicity Study) | no |

Principles of method if other than guideline

not applicable

GLP compliance

yes

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| Common name | isobutylene |
| other: Aldrich product name | 2-methylpropene |

Details on test material

- Name of test material (as cited in study report): isobutylene - Physical state: colourless gas - Analytical purity: 99.9% - Lot/batch No.: 295469 - Stability under test conditions: not specified - Storage condition of test material: Cylinders kept tightly closed, away from heat, sparks or open flame, in a ventilated area.

Confidential details on test material

none

Test animals

Species

rat

Strain

other: Alpk:APfSD (Wistar-derived)

Details on test animals and environmental conditions

TEST ANIMALS - Source: Rodent Breeding Unit (RBU), Alderley Park, Macclesfield, Cheshire, UK - Age at study initiation: 10-12 weeks - Weight at study initiation: 215-293g - Housing: individually housed in metal cages with solid bottoms - Diet: CT1 diet (supplied by Special Diets Services Limited, Stepfield, Witham, Essex, UK ad libitum except during exposure - Water: Mains water ad libitum except during exposure - Acclimation period: Animals were acclimatised to the holding chambers for 4 days before the start of exposure. However, as the animals were time-mated, the day of arrival was the first day of the study. ENVIRONMENTAL CONDITIONS - Temperature: 22±3°C - Humidity: 30-70% - Air changes (per hr): At least 12 changes/hr - Photoperiod: 12 hrs dark / 12 hrs light) IN-LIFE DATES: From: 28 February 2001 To: 6 June 2002

Administration / exposure

Route of administration

inhalation: vapour

Type of inhalation exposure (if applicable)

whole body

Vehicle

other: air

Details on exposure

GENERATION OF TEST ATMOSPHERE / CHAMBER DESCRIPTION - Exposure apparatus: The animals were housed in circular whole body exposure chambers. The chambers consisted of a perspex circular section subdivided into 10 equal segments by wire mesh partitions. The chamber was covered by an aluminium lid and stood on an aluminium base plate. The internal volume of each chamber was approximately 150 litres. - System of generating atmosphere: Test atmospheres were generated by metering appropriate amounts of test substance through a copper coil heated to 40°C using a Techne TE-WA water bath. The resultant vapour was passed through a distribution plenum to which flowmeters were attached. The vapour was passed through the flowmeters to a copper transfer line to the top of each exposure chamber for groups 2, 3 and 4. Clean dry air was used to dilute to the required concentration for each group. - Temperature/humidity in air chamber: Temperatures were within the range of 22±3°C and the relative humidity within 15-70%. - Air flow rate: Flow rates through the chamber gave at least 12 air changes per hour. The air flow rates were monitored continuously using variable area flowmeters and were recorded at approximately 30 minute intervals during the study. The optimum air flow rates through the chamber necessary to conduct the study were determined during trial generations. TEST ATMOSPHERE - Brief description of analytical method used: Grab samples were taken from each exposure chamber using a 5 mL gas-tight syringe equipped with integral on-off valve. Samples were directly introduced into a gas chromatograph equipped with flame ionisation detection using an automated gas-sampling valve. Absolute atmosphere concentrations of isobutylene (in units of ppm v/v) were directly calculated after suitable system calibration and reprocessing. - Samples taken from breathing zone: yes, approximately hourly, during each exposure period. Clean, dry air (dried and filtered using equipment supplied by ATLAS-COPCO, Sweden), was supplied to the control group.

Analytical verification of doses or concentrations

yes

Details on analytical verification of doses or concentrations

The mean analysed atmosphere concentrations were found to be 505 ± 18.2, 1963 ± 56.4 and 8014 ± 183 ppm for target concentrations of 500, 2000 and 8000 ppm respectively. No isobutylene was detected in the control or room atmospheres. The limit of detection was calculated to be approximately 4 ppm (v/v) isobutylene in the atmosphere concentration.

Details on mating procedure

At the Rodent Breeding Unit, female rats were paired overnight with males of the same strain. The following morning, vaginal smears from the females were examined for the presence of spermatozoa. The day on which sperm were detected in a vaginal smear was designated as day 1 of gestation.

Duration of treatment / exposure

6 hours per day

Frequency of treatment

On days 5 to 21 (inclusive) of gestation

Duration of test

The day of mating was designated day 1 and the rats were terminated on day 22 of gestation

Doses / concentrations

| 500, 2000 or 8000 ppm |  |
| --- | --- |
| Basis | other: target concentration |

No. of animals per sex per dose

24

Control animals

yes, concurrent no treatment

Further details on study design

The exposure concentrations were selected by the Sponsor on the basis of the results obtained from an inhalation toxicity study in which groups of 10 male and 10 female rats were exposed to 250, 1000 or 8000 ppm isobutylene for 6 hrs/day, 5 days/wk for 13 consecutive wks. Although no toxicologically significant effects were observed at 8000 ppm, higher exposure concentrations were not possible because of the risk of explosion. Isobutylene forms explosive mixtures with air. On day 1 of gestation, females were delivered to CTL. The females were supplied over 4 consecutive days in each of 2 weeks.

Examinations

Maternal examinations

DETAILED CLINICAL OBSERVATIONS: Yes - Time schedule: Detailed observations were recorded for each animal at least once a day throughout the study (days 1 - 22 of gestation). On exposure days, clinical observations were recorded prior to exposure. The rats were observed during and following exposure. BODY WEIGHT: Yes - Time schedule for examinations: The bodyweight of each rat was recorded on days 1, 5, 6, 7, 10, 13, 16, 19 and 22 of gestation. Bodyweights were recorded prior to daily exposure. FOOD CONSUMPTION: Yes - The amount of food consumed by each rat was measured by giving a weighed quantity of food on days 1, 5, 7, 10, 13, 16 and 19 and calculating the amount consumed from the residue on days 5, 7, 10, 13, 16, 19 and 22 respectively. Food was not available to the rats during each 6 hour period of exposure. POST-MORTEM EXAMINATIONS: Yes - Killed on gestation day 22 of gestation - Organs examined: All rats were given an examination post mortem which involved an external examination and an examination of the thoracic and abdominal viscera and uterine contents. All macroscopic findings were recorded. Pregnancy status was determined.

Ovaries and uterine content

The ovaries and uterine content was examined after termination: Yes Examinations included: - Gravid uterus weight: Yes - Number of corpora lutea: Yes - Number of implantations: Yes - Position of implantations: Yes - Number of live foetuses: Yes - Number of intra-uterine deaths, early and late

Fetal examinations

- External examinations: Yes: [all per litter] including examination of the oral cavity - Soft tissue examinations: Yes: [all per litter] - Skeletal examinations: Yes: [all per litter] including assessment of ossification of the manus and pes - Head examinations: Yes: [all per litter]

Statistics

All analyses were carried out in SAS (1996). For Fisher’s Exact Tests the proportion in each treated group was compared to the control group proportion. The percentage of male foetuses was analysed by analysis of variance following double arcsine transformations of Freeman and Tukey (1950). All statistical tests were two sided.

Indices

none

Historical control data

not cited

Any other information on materials and methods incl. tables

Classification of foetal observations: The observations were classified as major (permanent structural or functional deviations considered likely to be incompatible with survival or rarely seen) or minor defects or variants (small, generally transient deviations considered compatible with survival). The difference between the minor defect and variant classification is the frequency of occurrence in the control population of rats of this strain.

Results and discussions

Effect levels

| Endpoint | Effect type | Effect level | Based on | Basis for effect level / Remarks |
| --- | --- | --- | --- | --- |
| NOAEC | maternal toxicity | 8000 ppm (18359 mg/m3, 18.4 mg/L) nominal |  | no effects at the highest concentration tested, isobutylene at exposure concentrations of 500, 2000 and 8000 ppm had no effect on the females during gestation |
| NOAEC | developmental toxicity | 8000 ppm (18359 mg/m3, 18.4 mg/L) nominal. |  | no effects at the highest concentration tested, there was no effect of isobutylene on the number, growth or survival of the foetuses in utero and no effect on foetal development |

Maternal toxic effects

no effects

Details on maternal toxic effects

no effects

Embryotoxic / teratogenic effects

no effects

Details on embryotoxic / teratogenic effects

no effects

Any other information on results incl. tables

There was no effect of isobutylene on the number, growth or survival of the foetuses in utero. There was no effect of isobutylene on foetal development. Although cleft sternebrae were observed only in foetuses in the isobutylene groups, the incidence of foetuses affected was small and not dose-related and there were no minor changes in the appearance or ossification of the sternebrae to indicate that this area of the skeleton was adversely affected by isobutylene. Also, there was no evidence for an adverse effect of isobutylene on other ossification centres of the skeleton. Isolated differences from control were considered to be incidental. Thus isobutylene at exposure concentrations of up to 8000 ppm, is considered not to have any adverse effect on foetal development.

Overall remarks, attachments

Overall remarks

Reach requires inhalation exposure units to be in mg/m3. Units of exposure in this paper are given as ppm, ppm have been converted into mg/m3using molecular weight of 56.11 and constant of 24.45 (molar vol of a gas at 25°C and 1 atm pressure).

Applicant's summary and conclusion

Conclusions

Isobutylene at exposure concentrations of 500, 2000 and 8000 ppm had no effect on female rats during gestation. There was no effect of isobutylene on the number, growth or survival of the foetuses in utero and no effect on foetal development.

Executive summary

Pregnant rats were exposed to isobutylene (2-methylpropene) at exposure concentrations of 500, 2000 and 8000 ppm (1147, 4589, 18359 mg/m3), from gestation day 5-21. Isobutylene had no effect on the females during gestation. There was no effect of isobutylene on the number, growth or survival of the foetuses in utero and no effect on foetal development. The NOECs for maternal and developmental toxicity were 8000 ppm (18359 mg/m3), the highest concentration tested.

Cross-reference to other study

none

11 Guidance on safe use

Additional information: Guidance on safe use, IUC4#1/Ch.1.8.1

| UUID |  | IUC5-e28038a5-94dc-4524-a235-af7770899ca1 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | Edlin Maskor / LOA Reach Services Team / Tervuren / Belgium |
| Date |  | 2010-06-16 20:57:06 BST |
| Remarks |  |  |

Administrative Data

First-aid measures

Product-Specific hazards: Extremely flammable liquefied gas An asphyxiant at high concentrations – oxygen depletion can be fatal. Contact with the liquid may result in frostbite. General advice : Take care to self-protect by avoiding becoming contaminated – use air supplied breathing apparatus where high concentrations are possible. Move contaminated patient(s) out of the dangerous area. Seek medical assistance - show the material safety data sheet or label if possible. Inhalation : Move to fresh air. Do not leave the victim unattended. Exposure to high concentrations may cause asphyxiation and the victim may be unaware. Keep patient warm and at rest. Seek immediate medical attention. If breathing is difficult, give oxygen if possible, or assisted ventilation. If unconscious place in recovery position. In the event of cardiac arrest, (no pulse), apply cardiopulmonary resuscitation. Skin contact : Liquid contact may cause frostbite. Do not remove clothing that adheres due to freezing. Immediately flush affected area with plenty of water – continue for at least 15 minutes. If there are signs of frostbite, (blanching or redness of skin or burning or tingling sensation), seek medical attention. Eye contact : Remove any contact lenses. Flush eyes with water thoroughly and continuously for at least 15 minutes. Keep eye wide open while rinsing. If there are signs of irritation or other symptoms seek medical attention. If eye irritation, pain, swelling, lachrimation or photophobia persists, the patient should be seen in a specialist health care facility. Ingestion : Is not considered a likely route of exposure – frostbite to the lips and mouth may occur if in contact with the liquid. Advice to Physician : A simple asphyxiant gas at normal temperatures and pressures – there is no specific antidote. In the event of contact with the liquid treat for frostbite.

Fire-fighting measures

Where possible stop the flow of gas. If the flow cannot be stopped allow the fire to burn out, whilst cooling containers and surroundings with a water spray. Suitable extinguishing media : LARGE FIRE: Use water spray, water fog or foam. DO NOT use direct water jet. SMALL FIRE: Dry powder or carbon dioxide (CO2) extinguisher, dry sand or fire fighting foam. Unsuitable Extinguishing Media : Do NOT use water jet. Simultaneous use of foam and water on the same surface is to be avoided as water destroys the foam. Combustion Products : Carbon monoxide, carbon dioxide and unburned hydrocarbons (smoke). Specific hazards during fire fighting : Vapour is denser than air – flashback may be possible over considerable distances. Cylinders or other containment vessels may explode under fire conditions - use water spray to cool unopened containers. Do not allow run-off from fire fighting to enter drains or water courses – may cause explosion hazard in drains and may reignite. Further information : Special protective equipment for fire-fighters : Wear an approved positive pressure self-contained breathing apparatus in addition to standard fire fighting gear.

Accidental release measures

Personal precautions : Wear personal protective equipment, including self contained breathing apparatus, unless the atmosphere is proved to be safe. Avoid breathing vapours. Ensure adequate ventilation and absence of sources of ignition. Beware of accumulation of vapours in low areas or contained areas, where explosive concentrations may occur. Environmental precautions : Land spillage : Prevent further leakage or spillage if safe to do so. Prevent spillage from entering drains or any place where accumulation may occur. Spillages in water or at sea : Prevent further leakage or spillage if safe to do so. If the spillage contaminates rivers, lakes or drains inform respective authorities. Methods for clean up : Contain spillage – ventilate area and allow to evaporate. Further accidental release measures : Spillages of liquid product will create a fire hazard and form an explosive atmosphere. Ensure all equipment is non sparking or electrically bonded. Avoid direct contact with released material. Stay upwind. Keep non-involved personnel away from the area of spillage. Ensure adequate ventilation, especially in confined areas.

Handling and storage

Advice on safe handling : Smoking, eating and drinking should be prohibited. Use only in well ventilated areas. Avoid all sources of ignition, oxidising agents, chlorine and hydrogen chloride or hydrogen fluoride.. Take precautionary measures against static discharges, use proper bonding and/or grounding procedures. Avoid contact with heat and ignition sources and oxidizing agents. Use only in a closed system. Use piping and equipment designed to withstand the pressures to be encountered. Use a check valve or other protective device to prevent reverse flow. Cleaning, inspection and maintenance of the internal structure of storage tanks must be done only by properly equipped and qualified personnel as defined by national, local or company regulations. Handle empty containers with care; vapour residue may be flammable. Do not pressurise, cut, weld, braze, solder, drill, or grind on containers. Dispose of rinse water in accordance with local and national regulations. The vapour is heavier than air, beware of accumulation in pits and confined spaces. Ensure that all relevant regulations regarding explosive atmospheres, and handling and storage facilities of flammable products are followed. Storage : Butene should be stored only in supplied cylinders or approved vessels. No smoking. Store in a designated cool and well-ventilated place. Cylinders should be secured vertical - and only transported in a secure position in a well ventilated vehicle or hand truck. Cylinders which have been are opened must be carefully resealed and kept upright. Vapour space above stored liquid may be flammable/explosive unless blanketed with inert gas. Storage installations should be designed with adequate bunds so as to prevent ground and water pollution in case of leaks or spills.

Exposure controls / personal protection

None established Occupational Exposure Limit Type of limit

Additional information: Guidance on safe use, IUC4#1/Ch.1.8.1

| UUID |  | IUC5-08c7c0f6-079e-4619-b7cd-fb094db9f250 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V1.0 Plug-In |
| Date |  | 2009-03-18 15:03:41 GMT |
| Remarks |  | Successfully migrated to IUCLID 5.2 format. |

Administrative Data

Exposure controls / personal protection

Occupational Exposure Limit Germany Occupational Exposure Limit Huels AG: Sicherheitsdatenblatt "Buten-1", Version 09, 17 Feb 1997 Occupational Exposure Limit MAK-Wert not established Occupational Exposure Limit Type of limit MAK (DE)

Additional information: Guidance on safe use, IUC4#1/Ch.1.8.1

| UUID |  | IUC5-2b5b16ca-fc60-4ef4-aa0b-0f8fdf0b8e8e |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V1.0 Plug-In |
| Date |  | 2009-03-18 15:04:08 GMT |
| Remarks |  | Successfully migrated to IUCLID 5.2 format. |

Administrative Data

Exposure controls / personal protection

Occupational Exposure Limit - Werte beziehen sich auf Buta-1,3-dien - nach Gefahr- stoffverordnung Occupational Exposure Limit Type of limit TRK (DE) 11 mg/m3

Additional information: Guidance on safe use, IUC4#2/Ch.1.8.1

| UUID |  | IUC5-be04dd1e-b0d5-4225-b6c1-93bfbf96d777 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V1.0 Plug-In |
| Date |  | 2009-03-18 09:55:34 GMT |
| Remarks |  | Successfully migrated to IUCLID 5.2 format. |

Administrative Data

Exposure controls / personal protection

Occupational Exposure Limit Type of limit TRK (DE) 5 ml/m3

Additional information: Guidance on safe use, IUC4#1/Ch.1.8.1

| UUID |  | IUC5-e7bbbc81-0544-4e9c-8d44-4adab31c87df |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V1.0 Plug-In |
| Date |  | 2009-03-18 15:05:07 GMT |
| Remarks |  | Successfully migrated to IUCLID 5.2 format. |

Administrative Data

Exposure controls / personal protection

Occupational Exposure Limit Germany Occupational Exposure Limit Huels AG: Sicherheitsdatenblatt "Buten-1", Version 09, 17 Feb 1997 Occupational Exposure Limit MAK-Wert not established Occupational Exposure Limit Huels AG Marl Occupational Exposure Limit Oxeno Olefinchemie GmbH Marl Occupational Exposure Limit Type of limit MAK (DE)

Additional information: Guidance on safe use, IUC4#2/Ch.1.8.1

| UUID |  | IUC5-d4ed0b9d-5335-4e48-b610-f448e6d9c6f5 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V1.0 Plug-In |
| Date |  | 2009-03-18 15:06:03 GMT |
| Remarks |  | Successfully migrated to IUCLID 5.2 format. |

Administrative Data

Exposure controls / personal protection

Occupational Exposure Limit Germany Occupational Exposure Limit Huels AG: Sicherheitsdatenblatt "Buten-1", Version 09, 17 Feb 1997 Occupational Exposure Limit MAK-Wert not established Occupational Exposure Limit Huels AG Marl Occupational Exposure Limit Type of limit MAK (DE)

12 Literature search

Additional information: Literature search - LOAT 2008

| UUID |  | IUC5-255693aa-7886-43fe-a521-7ba3059cd457 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | Thomas Penman / LOA Reach Services Team / Tervuren / Belgium |
| Date |  | 2010-07-14 22:42:54 BST |
| Remarks |  |  |

Administrative Data

Literature Search

| Date | 2008-11-16 |
| --- | --- |
| Remarks | Substance CAS No. 115-11-7 Substance Name 2-methylpropene Authoritative Reviews: IUCLID4 (PDF) (February 2000) SIDS Substance Search String: The open literature search was conducted using the following search strings for the environmental area of work: title substacne name + cas number AND (Environment OR Environmental OR Toxicology OR Toxicity OR Toxicant OR Ecotox* OR Pollution OR Acute OR Chronic OR Fish OR Invertebrate OR Algae OR NOEL OR NOAEL). The searches done on USEPA Ecotox, echemportal and ESIS were only done using the substance CAS No. Limited by Year: Searches were not limited by year as so few references found. IUCLID available dated 2000 Searched on Topic / Title / Keywords /Abstract: Searches undertaken in Topic Databases Searched: Toxline (which includes ANEUPL, BIOSIS, CIS, CRISP, DART, EMIC, FEDRIP, HEEP, HMTC, IPA, KEMI RISKLINE, MT GABS, NIOSH, NTIS, PESTAB, PPBIB, PubMed, TSCATS), Web of Science (WoS), USEPA Ecotox, echemportal, ESIS Search Valid Up to Date: November 08 The full search strategy is attacehd to Section 13 of this record. |

13 Assessment Reports

Additional information: Assessment Reports. SPARC

| UUID |  | IUC5-7627ee7e-eb81-4a4b-8435-8eff3dd2e385 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | Helen Philpott / LOA Reach Services Team / Tervuren / Belgium |
| Date |  | 2010-05-06 11:53:21 BST |
| Remarks |  |  |

Administrative Data

EU: REACH

Type of report

other: QMRF

Document

SPARC Final 30042010.pdf / 58.79 KB (application/pdf)

Additional information: Assessment Reports. HC5

| UUID |  | IUC5-a22e245c-6a2b-456c-a45e-51b649a45cc2 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | Helen Philpott / LOA Reach Services Team / Tervuren / Belgium |
| Date |  | 2010-05-06 11:53:22 BST |
| Remarks |  |  |

Administrative Data

EU: REACH

Type of report

other: QMRF

Document

HC5 Final 30042010.pdf / 62.42 KB (application/pdf)

Additional information: Assessment Reports. QPRF

| UUID |  | IUC5-a3b3663e-85f5-45b9-8853-5d835af8d42f |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | Helen Philpott / LOA Reach Services Team / Tervuren / Belgium |
| Date |  | 2010-05-06 11:53:23 BST |
| Remarks |  |  |

Administrative Data

EU: REACH

Type of report

other: QPRF

Document

QPRF_version_1.1 LOA Final 30042010.pdf / 25.04 KB (application/pdf)

Additional information: LOA Service Team Search strategy

| UUID |  | IUC5-2a38d002-dfe0-43ba-a94c-05745518e624 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | Thomas Penman / LOA Reach Services Team / Tervuren / Belgium |
| Date |  | 2010-07-14 22:43:00 BST |
| Remarks |  |  |

Administrative Data

EU: REACH

Type of report

other: Search strategy

Remarks

Added as no attachments are possible in Chapter 12 literature search

Document

LOA REACH Consortium Search Strategy.pdf / 27.37 KB (application/pdf)

Legal entity: LOA Reach Services Team

| UUID |  | IUC5-a537c06c-215a-4953-aba2-c1219a856aac |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V1.0 Plug-In |
| Date |  | 2008-09-23 04:51:11 BST |
| Remarks |  | Successfully migrated to IUCLID 5.2 format. |

General information

| Legal entity name | LOA Reach Services Team |
| --- | --- |
| Legal entity type | third party |

Contact information

Contact address

| Address flags | EU: REACH |
| --- | --- |
| Address | Sint Hubertuslaan 25 |
| Postal code | 3080 |
| Town | Tervuren |
| Country | Belgium |
| Phone | +3223050698 |
| E-mail | Sief.Manager@loa-reach.com |

Contact persons

| Person flags | EU: REACH |
| --- | --- |
| Organisation | LOA Reach Services Team |
| Title | Project Executive |
| First name | Mike |
| Last name | Penman |
| Phone | +32 23050698 |
| E-mail | Sief.Manager@loa-reach.com |
| Address | Sint Hubertuslaan 25 |
| Postal code | 3080 |
| Town | Tervuren |
| Country | Belgium |