SIDS Initial Assessment Report

For

CoCAM 5

Washington DC, United States

October 15-17, 2013

| Category and Chemical Names: | Phenylchlorosilane Category: Trichlorophenylsilane (TClPS) Dichloromethylphenylsilane (DClMPS) |
| --- | --- |
| 2. CAS Number(s): | TClPS: 98-13-5 DClMPS: 149-74-6 |
| 3. Sponsor: | United States Tala Henry, Ph.D. Director, Risk Assessment Division (7403M) U.S. Environmental Protection Agency (U.S. EPA) 1200 Pennsylvania Ave, N.W. Washington, DC 20460 Phone: 202-564-2959 |
| 4. Shared Partnership with: | Silicones Environmental Health and Safety Center (SEHSC): Bluestar Silicones Dow Corning Corporation Evonik Milliken & Company Momentive Performance Materials Shin-Etsu Silicones of America Wacker Silicones, A Division of Wacker Chemical Corporation Contact: Tracy Guerrero American Chemistry Council Director, Silicones, Environmental, Health, and Safety Center 700 2nd Street, NE Washington, DC, 20002 (202) 249-6196 www.sehsc.com |
| 5. Review and Quality Check Process Prior to CoCAM: | Industry prepared the documents, conducted an independent third party expert review then submitted the documents to U.S. EPA for submission to the OECD. The U.S. EPA conducted a review of this case. Discussions were held. Documents were then finalized for submission to the OECD. |
| 6. Date of Submission: | March, 2013 |
| 7. Date of Last Update: | July 6, 2015 |
| 8. Comments: | The summaries and evaluations contained in this assessment report may be based on unpublished proprietary data submitted for the purpose of the OECD assessment. In many cases, national/regional regulatory authorities will not accept a regulatory submission based on the summaries and evaluation of unpublished proprietary data contained in this [these] assessment report[s] unless they have received the data on which the summaries and evaluation are based, either from: - the owner of the data; or - a second party that has obtained permission from the owner of the data for this purpose; or alternatively - the applicant has received permission from the data owner that the summary and evaluation contained in this [these] assessment report[s] may be used in lieu of the data; or - following expiry of any period of exclusive use, mandatory compensation, where required, has been offered unless the period of protection for the proprietary data concerned has expired. Applicants who wish to use data in this assessment report should ensure that the use is consistent with the legal requirements in the country/region in which it is proposed to use the data. The phenylchlorosilanes category uses data from two hydrolysis products: - hydrogen chloride (HCl; CAS No. 7647-01-0) - http://www.chem.unep.ch/irptc/sids/OECDSIDS/64175.pdfdiphenylsilanediol (CAS No. 947-42-2) Two structurally analogous supporting substances are also used: - Diphenyldichlorosilane (CAS No. 80-10-4, DClDPS; presented at CoCAM 5) - Trimethoxyphenylsilane (TMPS, CAS No. 2996-92-1) Hydrolysis half-lives of the phenylchlorosilanes are supported using data from DClDPS as well as structurally similar chlorosilanes, dichloro(dimethyl)silane (CAS No. 75-78-5) and trichloro(methyl)silane (CAS No. 75-79-6). HCl and TMPS have been evaluated in the OECD Cooperative Chemicals Assessment Programme and can be found by searching the Existing Chemicals Database at the following website: http://www.oecd.org/env/hazard/data. |

SIDS INITIAL ASSESSMENT PROFILE

| Category name | Phenylchlorosilane Category |
| --- | --- |
| CAS No(s). | 98-13-5 149-74-6 |
| Chemical Name(s) | Trichlorophenylsilane (TClPS) Dichloromethylphenylsilane (DClMPS) |
| Structural Formula(s) | TClPS: DClMPS: |
| SUMMARY CONCLUSIONS OF THE SIAR Analogue/Category Rationale These chemicals can be represented by a similar general molecular formula: (Cl)xSiPh(Me)(3-x) Where Cl = chlorine, x = 2 or 3; Si = silicon [=1]; Ph = phenyl, [=1]; Me = CH3 Chlorosilanes, including the phenyl chlorosilanes, react rapidly when exposed to moisture or polar reagents (those that are protic and as such contain a dissociable H+), producing hydrogen chloride (HCl; CAS No. 7647-01-0) and the corresponding silanols (in general, siloxane oligomers and polymers at concentrations greater than 500 mg/L). Specifically, TClPS hydrolyses to form three moles of HCl and one mole of phenylsilanetriol, and DClMPS hydrolyses to form two moles of HCl and one mole of methylphenylsilanediol. The half-lives of the phenylchlorosilanes are expected to be < 1 minute based on data with supporting substance diphenyldichlorosilane (DClDPS, CAS No. 80-10-4); data are not available for TClPS or DClMPS. The assessment for TClPS focuses exclusively on the sponsored substance which contains up to 1% benzene as an impurity. As noted, the silanols resulting from initial hydrolysis can condense spontaneously to form highly cross-linked polymeric gels in uncontrolled environments. Exposure to parent chlorosilane is likely to be transient and observed toxicity in standard test systems will therefore, depending on the conditions of the system (e.g. pH and concentration of test material), likely be to hydrolysis products and condensed silanol material (at concentrations greater than 500 mg/L). Hydrolysis analogues. DClDPS hydrolyses rapidly in contact with water (half-life 0.2 minutes at pH 7 and 1.5 oC). Based on the rapid hydrolysis of DClDPS, similar rates of hydrolysis are expected for TClPS and DClMPS. Human Health and Aquatic Toxicity Analogues. (1) Due to their reactivity, the category members are expected to hydrolyse prior to exposure, or locally at the port of entry, to form HCl and a corresponding silanol hydrolysis product. The levels of chlorosilane required to generate concentrations near those tested for corresponding or analogous silanols would result in severely corrosive HCl concentrations. Therefore, data for HCl can be used to partially address the toxicity of the phenylchlorosilanes. The primary hazard for the sponsored phenylchlorosilanes is considered to be exposure to the hydrogen chloride hydrolysis product. (2) A previously assessed alkoxysilane, trimethoxy(phenyl)silane (TMPS; CAS No 2996-92-1) hydrolyses to form three moles of methanol and one mole of phenylsilanetriol (CAS No 3047-74-3, the same expected hydrolysis product as TClPS). TMPS hydrolyses more slowly at pH 7 (half-life ca. 0.4 hours), but under acidic conditions such as in the stomach following ingestion, much more rapid hydrolysis can be expected. While phenylsilanetriol is not the expected hydrolysis product of DClMPS, both phenylsilanetriol and methylphenylsilanediol are expected to be water soluble due to the hydroxy groups on the silicon, have low log Kow values, and are expected to be readily absorbed and excreted. Hydrolysis of the sponsored substances produces HCl, while TMPS produces methanol as a hydrolysis product. The contribution of methanol from TMPS to the toxicity assessment of the sponsored substances is expected to be negligible as compared to the effects of HCl. TMPS is considered to be a suitable analogue for human toxicity endpoints for DClMPS. HCl and TMPS were presented and agreed under the OECD Cooperative Chemicals Assessment Programme (http://www.oecd.org/env/hazard/data). The assessment for TClPS focuses exclusively on the sponsored substance which contains up to 1% benzene (CAS No. 71-43-2) as an impurity. Benzene has also previously been presented and agreed under the OECD HPV Chemicals Programme (http://www.oecd.org/env/hazard/data). The read across strategy for the phenylchlorosilanes follows: Endpoint; TClPS; DClMPS. Hydrolysis; DClDPS; DClDPS. Biodegradation; Data available; TClPS. Acute inhalation toxicity; HCl; HCl. Acute oral toxicity; DClMPS, TMPS, HCl; Data available. Skin, eye and respiratory tract irritation; HCl; HCl. Repeated dose toxicity: inhalation; TMPS, HCl; TMPS, HCl. Repeated dose toxicity: oral; TMPS; TMPS. Genetic toxicity in vitro: gene mutation; Data available; Data available. Genetic toxicity in vitro: chromosome aberration; TMPS, HCl; TMPS, HCl. Toxicity to fertility; TMPS, HCl; TMPS, HCl. Developmental toxicity; TMPS, HCl; TMPS, HCl. Physical-chemical Properties The category members are liquids with measured melting points of -49.4 (DClMPS) and -40 °C (TClPS), measured boiling points of 201.8 (TClPS) and 206 – 207 °C (DClMPS) and vapour pressures of 0.44 (TClPS; estimated) and 0.47 (DClMPS; extrapolated) hPa at 20 ºC. The calculated octanol-water partition coefficients (log Kow) are 3.60 (TClPS) and 3.8 (DClMPS; reliability = 4), and the estimated water solubilities are 48.7 (DClMPS) and 78.5 mg/L (TClPS) at 20 °C. The calculated water solubility and log Kow values may not be accurate because the substances are hydrolytically unstable. Human Health No data are available on the toxicokinetics, metabolism and distribution of the phenylchlorosilanes. However, these substances rapidly hydrolyse to HCl and the corresponding silanol hydrolysis products on contact with moisture. Damage to membranes caused by the corrosive nature of HCl might enhance the uptake of the sponsored substances or the silanol hydrolysis products. Hydrogen and chloride ions will enter the body’s natural homeostatic processes. HCl will rapidly dissociate and its effects are thought to be a result of pH change (local deposition of H+). The hydrophilic nature of phenylsilanediol or –triol may limit diffusion across certain membranes. The low molecular weight and high water solubility of the silanols suggest elimination via the kidneys in urine. Acute inhalation studies were not located for the phenylchlorosilanes. The acute inhalation toxicity of the phenylchlorosilanes is expected to be well-characterized by the effects of HCl exposure, rather than systemic effects of silanol hydrolysis products. The principal clinical signs are expected to be indicative of respiratory and ocular effects resulting from HCl exposure. Inhalation LC50 values for HCl were determined to be 4.2 - 4.7 mg/L for 1 hour for rats. The oral LD50 for one of the phenylchlorosilane category members, DClMPS, was >200 to <2000 mg/kg bw in rats [OECD TG 423]. For the supporting substance TMPS, in an acute toxicity study conducted according to OECD TG 425, a total of 7 female rats were administered TMPS by oral gavage including 4 animals at 2000 and 3 animals at 550 mg/kg bw in polyethylene glycol 300. When the single animal dosed at 2000 mg/kg bw in a limit test died spontaneously on study day 2, a main test with 6 animals was conducted. In the main test, all three animals dosed at 2000 mg/kg bw had to be killed in extremis on study days 4, 5 or 6 and all animals dosed at 550 mg/kg bw survived. Animals at 2000 mg/kg bw appeared unhealthy; effects on respiration, coordination and body weight loss were noted. Slight ruffled fur and slight sedation were observed in two of three animals dosed at 550 mg/kg bw; no clinical signs were observed in the third animal dosed at 550 mg/kg bw. There was no effect on body weight at 550 mg/kg bw. Three animals dosed at 2000 mg/kg bw showed a greater than 20% loss of body weight prior to being killed in extremis; no body weight was recorded at the spontaneous death of the first animal dosed at 2000 mg/kg bw that was found dead on test day 2. Macroscopic findings at 2000 mg/kg bw included light red congested lungs, black brown stomach distended with gas, tan discoloration of kidneys, and spleen reduced in size; there were no macroscopic findings at 550 mg/kg bw. The estimated LD50 was 1049 mg/kg bw. The acute oral LD50 values of HCl were determined to be 238 - 277 mg/kg bw for female rats. Irritation data are not available for the sponsored substances. The phenylchlorosilanes rapidly hydrolyse to HCl and the associated silanol. HCl is corrosive and highly irritating to the skin, eyes and respiratory tract. As such, the sponsored substances are expected to be corrosive to the skin, cause serious damage to the eyes and be highly irritating to the respiratory tract. Sensitization data are not available for the phenylchlorosilanes or the expected hydrolysis products. Repeated dose toxicity data are not available for the sponsored substances. Data from supporting substance TMPS and hydrolysis product HCl are used to fill the repeated-dose toxicity endpoint for the phenylchlorosilanes. TMPS was administered to four groups of 10 rats/sex/dose level by gavage daily at 0 (dried and deacidified corn oil), 100, 250 and 500 mg/kg bw/day. Males were exposed for 28 days (including 14 days prior to pairing) and females were exposed for 14 days prior to pairing, through the pairing and gestation periods until the F1 generation reached day 4 postpartum. Administration of TMPS at 500 mg/kg bw/day caused a reduction of food consumption in males during the first week of treatment and in females during the pre-pairing period up to day 14 of the gestation period. Reduced body weight was noted in males throughout the study and in females throughout the gestation period. Kidney weight was increased in males and thickened urinary bladder in males and females was seen at this dose. An increase of concentration of urea, bile acids and cholesterol was also noted in males at 500 mg/kg bw/day. Multifocal tubular degeneration/regeneration and transitional cell hyperplasia of kidney were noted in males and females. At 250 mg/kg bw/day, an increase in the urea and bile acid concentrations was observed. Multifocal tubular degeneration/regeneration and transitional cell hyperplasia of kidney were noted in males and females at this dose. The urinary bladder was observed to be thickened in males and females at all dose levels. This finding correlated with the histopathology examination showing perivascular lymphoid cell infiltration and transitional cell hyperplasia of the urinary bladder. Based on the findings in urinary bladder, a NOAEL (No Observed Adverse Effect Level) for systemic toxicity of TMPS could not be established. The LOAEL was 100 mg/kg bw/day. For the analogue substance TMPS, in a 4-week whole-body vapour inhalation (7 hr/day, 5 days/week) study conducted similar to OECD TG 412, there were no adverse treatment-related effects noted at any of the vapour concentrations administered. The NOAEC was determined to be 649 mg/m3 (highest concentration tested). In repeated dose toxicity studies of HCl by the inhalation route, local irritant effects were observed in the groups of rats and mice exposed to 0.015 mg/L and above for 90 days. The inhalation NOAEC for systemic toxicity for HCl, excluding the local effects of irritation, has been determined to be 0.030 mg/L, with a LOAEC of 0.075 mg/L. The toxicity of the phenylchlorosilanes is expected to be well characterized by the effects of HCl inhalation exposure, the prevalent route of the phenylchlorosilanes exposure. The sponsored substances did not induce gene mutations in bacterial cells in vitro [similar or equivalent to OECD TG 471], and the sponsored substance TClPS was negative for induction of gene mutations in mouse lymphoma cells [OECD TG 476]. Chromosomal aberration studies were not located for the sponsored substances. The supporting substance TMPS did not induce gene mutations in bacterial cells [similar to OECD TG 471], but did induce chromosomal aberrations in Chinese hamster V79 cells in vitro in the presence of metabolic activation [OECD TG 473]. Based on these results, TMPS is considered to be genotoxic in vitro. The hydrolysis product HCl did not induce gene mutations in bacterial cells. Positive results in the in vitro chromosome aberration test with HCl were considered to be the effect of low pH. Based on the available data, the phenylchlorosilanes are not expected to cause gene mutations in vitro; it may be clastogenic in vitro. No data are available for the carcinogenicity of the phenylchlorosilanes. Toxicity for reproduction data are not available for the sponsored substances. Data for supporting substance TMPS and hydrolysis product HCl are used to fill the reproductive toxicity endpoint for the phenylchlorosilanes. In the combined repeated-dose/reproductive/developmental toxicity screening test [OECD TG 422] with TMPS, the NOAEL for reproductive/developmental toxicity was 500 mg/kg bw/day (highest dose tested). The LOAEL for maternal toxicity was 100 mg/kg bw/day. Overall, TMPS did not show evidence of reproductive/developmental toxicity. No reliable studies have been reported regarding toxicity to reproduction and development in animals after oral, dermal or inhalation exposure to hydrogen chloride/hydrochloric acid. Because proton and chloride ion are the normal constituents in the body fluid of animal species, low concentrations of hydrogen chloride gas/mist or solution do not seem to cause adverse effects to animals. In fact, the cells of gastric glands secrete hydrochloric acid into the cavity of stomach and orally administered sulfuric acid, which results in pH change as well, did not cause developmental toxicity to laboratory animals. These facts indicate that hydrogen chloride/hydrochloric acid is not expected to have developmental toxicity. In addition, no effects on the gonads were observed in a 90-day inhalation repeated-dose study up to concentrations of 0.075 mg/L HCl. The toxicity of the phenylchlorosilanes is expected to be well-characterized by the effects of HCl inhalation exposure, the prevalent route of the phenylchlorosilanes exposure, and the phenylchlorosilanes are not expected to be reproductive toxicants. The sponsored phenylchlorosilanes possess properties indicating a hazard for human health [lethality from acute studies (oral and inhalation), corrosive and highly irritating to the skin, eyes and respiratory tract (based on hydrolysis product, HCl), repeated dose toxicity (based on analogue substance, TMPS), and genotoxic (clastogenic in vitro based on analogue substance, TMPS)]. Adequate screening-level data are available to characterize the human health hazard for the purposes of the OECD Cooperative Chemicals Assessment Programme. Environment Not all programs within EPI Suite have been validated for chemicals that contain the element Si, but recent upgrades to the Kow module, found in the current version of EPI Suite (v4.10), may improve estimates for silanes and siloxanes for this endpoint. However, there is still uncertainty associated with the calculated values and they should be used with caution whenever they are reported. The chlorine group is the most active functional group on these molecules and determines many aspects of the behaviour of the category members. The phenylchlorosilanes undergo rapid hydrolysis in the presence of moisture to form two or three moles of HCl and one mole of di- or tri- silanol, depending on the parent substance. An OECD TG 111 (Hydrolysis as a Function of pH) test was conducted at 1.5 ºC for supporting substance DClDPS; half-lives of less than one minute were reported at pH 4, 7, and 9. Likewise, TClPS and DClMPS are expected to hydrolyse rapidly under environmentally relevant conditions. In the atmosphere, indirect photo-oxidation by reaction with hydroxyl radicals is predicted to occur with half-lives of 2.7 to 5.5 days. Any potential for photodegradation might be superseded by hydrolysis of the parent compound depending on the concentration of water vapour in the air. The biodegradation of TClPS was determined in OECD TG 310; there was essentially no (1%) biodegradation of the test substance in 28 days. HCl is an inorganic compound and biodegradation tests are not applicable. Based on this information, the sponsored substances are not expected to be readily biodegradable. Due to rapid hydrolysis of the sponsored substances, any potential for biodegradation is likely to be of the hydrolysis products. Consequently, the only biodegradable materials in the test system will be silanols, and condensed silanol materials (high molecular weight polymers). At high concentrations (>500mg/L), the silanols will condense to form highly cross linked, high molecular weight polymers that are water insoluble and effectively nonbiodegradable. A level III fugacity model calculation with equal and continuous distributions to air, water and soil compartments suggests that the sponsored substances will distribute mainly to the air (47.6%) and soil (47.7%) compartments with minor distribution to the water and sediment compartments. Level III fugacity modeling using equal loading rates of 1000 kg/h each for air, soil and water predicts that the silanol hydrolysis products will distribute mainly to soil (ca. 83%), with a smaller fraction to water (ca. 16%) and negligible amounts to sediment and air. Based on the more realistic scenario of 100% release to air, the model predicts that the silanol hydrolysis products will be distributed mainly in air (96%) and water (ca. 4%). The sponsored substances are not expected to bioaccumulate in the aquatic environment based on rapid hydrolysis to silanols, with estimated BCFs of 3.2 L/kg wet-wt. The calculated bioconcentration factors for the sponsored substances were 110 (TClPS) - 1009 (DClMPS) L/kg wet-wt The following acute toxicity test results have been determined for aquatic species: Test substance; Species; Result (mg/L); Guideline; Test type. Fish, acute toxicity. Sponsored substances. TClPS; Oncorhynchus mykiss; 96-hr LC50 >100 (nominal; pH adjusted to 7.0); OECD TG 203; static. DClMPS. Supporting hydrolysis products DClDPS. HCl; Cyprinus carpio; 96-hr LC50 = 4.92 (pH = 4.3 ); OECD TG 203; semi-static. Aquatic invertebrate, acute toxicity. Sponsored substances. TClPS; Daphnia magna; 48-hr EC50 > 100 (nominal; pH adjusted to 7.0); OECD TG 202; static. DClMPS; Daphnia magna; 48-hr EC50 = 38 (without pH adjustment; nominal); >100 (with pH adjustment to 7.9; nominal); OECD TG 202; static. Supporting hydrolysis products DClDPS. HCl; Daphnia magna; 48-hr EC50 = 0.492 (pH= 5.3); OECD TG 202; not specified. Aquatic plants, acute toxicity. Sponsored substances. TClPS; Pseudokirchneriella subcapitata; 72-hr ErC50 and EbC50 > 100 (nominal; pH adjusted to 7.0); OECD TG 201; static. Supporting hydrolysis products DClDPS. HCl; Pseudokirchneriella subcapitata; 72-hr ErC50 = 0.492 (pH= 5.3); OECD TG 201; static. The sponsored phenylchlorosilanes possess properties indicating a hazard for the environment (acute aquatic toxicity values between < 1 and 100 mg/L). The hydrolysis product, HCl, has properties that can result in toxicity of < 1 mg/L to aquatic organisms in poorly buffered systems, mainly due to acidification of the test medium. The sponsored phenylchlorosilanes and their hydrolysis products have low potential for bioaccumulation and are not readily biodegradable. Adequate screening-level data are available to characterize the hazard for the environment for the purposes of the OECD Cooperative Chemicals Assessment Programme. Exposure Production and import volumes (metric tonnes) for 2010 are summarized below. Substance; United States; Europe; Japan. Production; Import; Production; Import; Production; Import. TClPS; 2268 - 9072; <454; 0; 0; 227 – 2268; 0. DClMPS; 454 - 3629; 0; 0; 0; 0; 0. Ranges are provided to protect confidential business information. TClPS is used in formulations up to 100% as intermediates for silicone oligomers and polymers. DClMPS is used in formulations at 100% as an intermediate (details not provided). No parent substance is expected to remain after end use. The phenylchlorosilanes are produced and processed in closed systems; commercial customers use the phenylchlorosilanes in closed systems. Due to the dynamic and exothermic nature of the processes incorporating chlorosilanes, many engineering controls are always in place to prevent occupational exposure such as water scrubber devices and related equipment; closed sampling loop; and local and general ventilation. Employees involved in chlorosilane production and application use personal protective equipment (PPE) such as safety glasses or goggles, steel-tipped shoes, flame-resistant clothing, hard hats, chemical resistant gloves, and respirator masks. Potential routes of exposure include inhalation and dermal exposure. There are no consumer uses of the phenylchlorosilanes. Environmental exposure is not expected. |

Note: 	This document may only be reproduced integrally. The conclusions in this document are intended to be mutually supportive, and should be understood and interpreted together.

SIDS Initial Assessment Report

# identity

## Identification of the Category

| CAS Name and Number: | Trichlorophenylsilane (TClPS; CAS No 98-13-5) Dichloromethylphenylsilane (DClMPS; CAS No 149-74-6) |
| --- | --- |
| IUPAC Name: | TClPS: Trichloro(phenyl)silane DClMPS: Dichloro-methyl-phenylsilane |
| Molecular Formula: | TClPS: C6 H8 Cl3 Si DClMPS: C7 H8 Cl2 Si |
| Structural Formula: | TClPS: DClMPS: |
| Molecular Weight: | TClPS: 211.6 DClMPS: 191.1 |
| Synonyms: | TClPS: Benzene, (trichlorosilyl)- Phenylsilicon trichloride Phenylsilicon trichloride Phenyltrichlorosilane Silane, phenyltrichloro- Silane, trichlorophenyl- Silicon phenyl trichloride Trichloro(phenyl)silane Trichlorophenylsilane DClMPS: Benzene, (dichloromethylsilyl)- Dichloro(methyl)(phenyl)silane Dichlor-fenyl-methylsilane Dichloromethylphenyl silane Dichloromethylphenylsilane Methylphenyldichlorosilane Phenylmethyldichlorosilane Silane, dichloromethylphenyl- |

## Purity/Impurities/Additives

Based on a survey conducted by the SEHSC, the purity for these substances is presented in Table 1 (Guerrero, 2012).

Table 1. 	Purity of the sponsored substances

| Substance | Purity (%) | Reported impurities |
| --- | --- | --- |
| TClPS | 97 to >98 | 0.1% Cyclohexyltrichlorosilane (CAS No. 98-12-4) 1% Phenylmethyldichlorosilane (CAS No. 149-74-6) <0.1 to 2% Phenyldichlorosilane (CAS No. 1631-84-1) <2% Dichlorodiphenylsilane (CAS No. 80-10-4) |
| DClMPS | 99 | 1% Benzene (CAS No. 98-13-5) |

## Physical-Chemical Properties

## Table 2	Summary of Physico-Chemical Properties

| Property | TClPS | DClMPS |
| --- | --- | --- |
| Physical state | Liquid | Liquid |
| Melting point (°C) | -40 (measured; DIPPR, 1965) | -49.4 (measured; Haas, 2011) |
| Boiling point (°C) | 201.8 at 1013 hPa (measured; DIPPR, 2006) | 206 - 207 at 1013 hPa (measured; Weast, 1981) |
| Liquid density | 1.25 relative at 20 °C (estimated; Peter Fisk Associates, 2009a,b; calculated) | 1.1866 relative at 20 °C (not specified; Weast, 1981) |
| Vapour pressure (hPa at 20 °C) | 0.44 (estimated using measured data; DIPPR, 2001) | 0.47 at 20 °C (extrapolated from measured data at higher temps; Weast, 1981) |
| Water solubility (mg/L) (a, b) | 78.5 (estimated; US EPA, 2012, EPISuite v4.10) | 48.7 (estimated; US EPA, 2012, EPISuite v4.10) |
| Partition coefficient n-octanol/water (log value)(a, b) | 3.60 (estimated; US EPA, 2012, EPISuite v4.10) | 3.8 (estimated; US EPA, 2012, EPISuite v4.10) |
| Henry’s law constant (Pa-m3/mole) (b) | Bond estimate: 33.3 (estimated; US EPA, 2012, EPISuite v4.10) | Bond estimate: 115 (estimated; US EPA, 2012, EPISuite v4.10) |

(a) These values may not be accurate because the substances are hydrolytically unstable.

(b) Reliability = 4; Not all programs within EPI Suite developed by the U.S. Environmental Protection Agency and Syracuse Research Corporation have been validated for chemicals that contain the element Si, but recent upgrades to the Kow module, found in the current version of EPI Suite (v4.10, released 2011), may improve estimates for silanes and siloxanes for this endpoint.  However, there is still uncertainty associated with the calculated values and they should be used with caution whenever they are reported.

## Category and Analogue Justification

These chemicals can be represented by a similar general molecular formula:

(Cl)xSiPh(Me)(3-x)

Where Cl = chlorine, x = 2 or 3;

Si = silicon [=1];

Ph = phenyl [ = 1];

Me = CH3

Chlorosilanes, including the phenyl chlorosilanes, react rapidly when exposed to moisture or polar reagents (those that are protic and as such contain a dissociable H+), producing hydrogen chloride (HCl; CAS No. 7647-01-0) and the corresponding silanols (in general, siloxane oligomers and polymers at concentrations greater than 500 mg/L; Bayer et al. 1991, Bazant et al. 1965, Noll 1968, and Tomanek 1990). Specifically, TClPS hydrolyses to form three moles of HCl and one mole of phenylsilanetriol, and DClMPS hydrolyses to form two moles of HCl and one mole of methylphenylsilanediol. The half-lives of the phenylchlorosilanes are expected to be < 1 minute based on data with supporting substance diphenyldichlorosilane (DClDPS); data are not available for TClPS or DClMPS. The assessment for TClPS focuses exclusively on the sponsored substance which contains up to 1% benzene as an impurity.

The levels of the phenylchlorosilanes required to generate significantly toxic concentrations of silanols would result in severely corrosive HCl concentrations.   The primary human health hazard for the phenylchlorosilanes is considered to be exposure to the hydrolysis product, HCl.

The silanols produced following phenylchlorosilane hydrolysis are provided in Table 3.

Sponsored substances

TClPS: TClPS hydrolyses to form three moles of HCl, and one mole of phenylsilanetriol (CAS No 3047-74-3).

DClMPS: DClMPS hydrolyses to form two moles of HCl, and one mole of methylphenylsilanediol (CAS No 3959-13-5).

Supporting substances

DClDPS: Supporting substance DClDPS hydrolyses rapidly in contact with water (half-life 0.2 minutes at pH 7 and 1.5 oC; Miller and Kozerski, 2001) to form one mole of diphenylsilanediol (CAS No 947-42-2) and 2 moles of HCl.  Based on the structural similarity of the sponsored substances and the rapid hydrolysis of DClDPS, similar rates of hydrolysis are expected for TClPS and DClMPS.

TMPS: TMPS (CAS No 2996-92-1) undergoes rapid hydrolysis in the presence of water with a measured hydrolysis half-life of 24 minutes at pH 7 and 25°C; hydrolysis is expected to produce 3 moles of methanol for each mole of phenylsilanetriol.  The silanol hydrolysis product is the same as that expected for sponsored substance TClPS and similar to the silanol hydrolysis product of DClMPS (see Table 3).

Table 3	Summary of silanol hydrolysis products

| Substance | Corresponding silanol hydrolysis product |
| --- | --- |
| Sponsored substance TClPS | Phenylsilanetriol (CAS No 3047-74-3) |
| Sponsored substance DClMPS | Methylphenylsilanediol (CAS No 3959-13-5) |
| Supporting substance DClDPS | Diphenylsilanediol (CAS No 947-42-2) |
| Supporting substance TMPS | Phenylsilanetriol (CAS No 3047-74-3) |

The category members are expected to hydrolyse rapidly to form hydrogen chloride (HCl) as one of the products.  Therefore, data for HCl can be used to partially address the toxicity of the phenylchlorosilanes.  The primary hazards for the sponsored phenylchlorosilanes are considered to be related to exposure to the hydrogen chloride hydrolysis product.

Table 4	Estimated water solubility and partition coefficient for hydrolysis products

| Property | Phenylsilanetriol (CAS No 3047-74-3)a | Methylphenylsilanediol (CAS No 3959-13-5)a |
| --- | --- | --- |
| Water solubility (mg/L) | 7.42E+04 (US EPA, 2013 | 1.479E+04 (US EPA, 2013) |
| Partition coefficient | -0.021 (pH 7, 22 oC; measured) (Safepharm, 2008) | 0.81 (US EPA, 2013) |

(a) Reliability = 4; The EPI Suite program (2012, v4.10) developed by the U.S. Environmental Protection Agency and Syracuse Research Corporation has not been validated for chemicals that contain silanes in their molecular structure.

Supporting substance trimethoxy(phenyl)silane (TMPS; CAS No 2996-92-1) hydrolyses to form three moles of methanol and one mole of phenylsilanetriol. TMPS hydrolyses more slowly at pH 7 (half-life ca. 0.4 hours), but under acidic conditions such as in the stomach following ingestion, much more rapid hydrolysis can be expected (RCC, 2008). TMPS is used to fill the repeated dose, in vitro mammalian genotoxicity, fertility and developmental toxicity endpoints.  Hydrolysis of the sponsored substances produces HCl, while TMPS produces methanol as a hydrolysis product. The contribution of methanol from TMPS to the toxicity assessment of the sponsored substances is expected to be negligible as compared to the effects of HCl. TMPS is considered to be a suitable analogue for human toxicity endpoints for DClMPS.

Table 5 provides the read across strategy for the phenylchlorosilanes.

Table 5.	 Read Across Strategy

| Endpoint | TClPS | DClMPS |
| --- | --- | --- |
| Hydrolysis | DClDPS | DClDPS |
| Biodegradation | TClPS | TClPS |
| Acute inhalation toxicity | HCl | HCl |
| Acute oral toxicity | DClMPS, TMPS, HCl | Data available |
| Skin, eye and respiratory tract irritation | HCl | HCl |
| Repeated dose toxicity: inhalation | TMPS, HCl | TMPS, HCl |
| Repeated dose toxicity: oral | TMPS | TMPS |
| Genetic toxicity in vitro: gene mutation | Data available | Data available |
| Genetic toxicity in vitro: chromosome aberration | TMPS, HCl | TMPS, HCl |
| Toxicity to fertility | TMPS, HCl | TMPS, HCl |
| Developmental toxicity | TMPS, HCl | TMPS, HCl |
| Aquatic toxicity to Fish, Daphnid and Algae | TClPS | TClPS |

† = Because aquatic studies with TClPS were conducted with pH adjustment to 7, supplemental data for HCl are used to provide effects of low pH resulting from hydrolysis of TClPS; ‡= Data for DClMPS are available for fish and daphnid endpoints (read across is only used for algae)

# general information on exposure

The phenylchlorosilanes react rapidly when exposed to moisture or polar reagents, producing HCl and silanols (siloxane oligomers and polymers at concentrations; e.g., greater than 500 mg/L) (Bayer et al. 1991, Bazant et al. 1965, Noll, 1968, and Tomanek, 1990).  This reaction is normally heterogeneous, highly exothermic, and potentially difficult to control, requiring close control of the reaction.  To prevent the rapid hydrolysis and subsequent loss of material or exposure of the reactants to atmospheric moisture, chlorosilanes are handled in closed pipes or containers.  An essential component is the reactor jacket and/or internal coils containing a heat-transfer fluid, which is also used for initial heating of the bed.  This system allows recovery of the reaction heat via steam, and greatly limits environmental exposure. Chlorosilanes are handled in closed systems as once they are exposed to moisture through accidental release they no longer have commercial value.  Exposure to chlorosilanes is not expected under normal use conditions for this reason.  If there is an accidental release the relevant routes of exposure are inhalation and dermal (Guerrero, 2012).

The silicone industry has a long history of safe transportation of chlorosilanes.  They are transported in rail cars or cargo tanks in accordance with U.S. Department of Transportation (U.S. DOT) Hazardous Materials Transportation Regulations  (49 CFR 171-180).  These regulations specify acceptable modes of transport, hazard placarding, container types and construction and handling practices.  U.S. DOT classifies chlorosilanes as Hazard Class 8, UN1760.  This classification requires “corrosive” labeling and packaging in United Nations (U.N.) certified containers.  Such containers meet rigorous performance standards as agreed to by the International Transportation Community.  The SEHSC prepared an ASTM manual (ASTM Stock #: MN L33), “Chlorosilane Emergency Response Guidelines,” and continues working with U.S. DOT, international transportation bodies, and chlorosilane producers to ensure the safe transportation of chlorosilanes (Guerrero, 2012).

## Production Volumes and Use Pattern

Table 6. 	Production and Import Volumes (metric tonnes) for 20101

| Substance | United States |  | Europe |  | Japan |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Production | Import | Production | Import | Production | Import |
| TClPS | 2268-9072 | <454 | 0 | 0 | 227-2268 | 0 |
| DClMPS | 454-3629 | 0 | 0 | 0 | 0 | 0 |

1 Guerrero, 2012 (Ranges are provided to protect confidential business information)

Use Pattern

TClPS is used in formulations up to 100% as intermediates for silicone oligomers and polymers.  DClMPS is used in formulations at 100% as an intermediate (details not provided).  No parent substance is expected to remain after end use.

## Environmental Exposure and Fate

Not all programs within EPI Suite developed by the U.S. Environmental Protection Agency and Syracuse Research Corporation have been validated for chemicals that contain silanes in their molecular structures (although some measured data are included in certain training sets), but recent upgrades to the Kow module, found in the current version of EPI Suite (v4.10), may improve estimates for silanes and siloxanes for this endpoint). However, there is still uncertainty associated with the calculated values and they should be used with caution whenever they are reported below.

### Sources of Environmental Exposure

The phenylchlorosilanes are handled in closed systems, and transported from the production site as the parent chemical, and then intended to be consumed during the generation of silicones.  Transfer of these materials is in closed pipe, drums, or tanks rather than in open systems to minimize loss of this material (through hydrolysis).  There are no intentional releases to the environment from manufacturing processes (Guerrero, 2012)

The phenylchlorosilanes are expected to hydrolyse rapidly; the half-lives are expected to be <1 minute based on data from the supporting substance, DClDPS.  Rapid hydrolysis limits exposure to the parent materials.  Hydrolysis of the phenylchlorosilanes results in the expected production of 2 or 3 moles of HCl and one mole of the corresponding phenylsilanediol or phenylsilanetriol.  Depending on pH and concentration in water, silanediols and silanetriols can condense to form highly cross-linked, high molecular weight polymers, further reducing the potential for exposure.  If the substance is slowly released such that the concentration of the resulting silanediol or silanetriol is not high enough to result in polymerization (<500 mg/L), the silanediol or silanetriol will exist largely as a monomer.  The monomers are known to be water soluble due to the hydroxy groups on the silicon.

### Photodegradation

The phenylchlorosilanes are not expected to undergo direct photolysis in air, but may undergo indirect photolysis through hydroxyl radical oxidation.  The hydroxyl radical reaction was calculated using a 12-hr day in AOPWIN® ver. 1.92a (USEPA, 2012 or 2013, EPI Suite v4.10).  The overall OH rate constants and estimated half-lives are provided in Table 7.  Photodegradation as a removal mechanism from air is unlikely as the phenylchlorosilanes are reactive and hydrolytically unstable, such that HCl and the likely hydrolysis products are assumed to be rapidly generated upon contact with atmospheric water vapour.

Table 7.	 Summary of Photodegradation

| Substance | Overall OH rate constant (cm3/molecule-sec) | Estimated half-life (days; 12-hr day; 1.5E6 OH/cm3) | References |
| --- | --- | --- | --- |
| TClPS | 2 E-12 | 5.5 | USEPA 2012 or 2013, EPI Suite (v4.10) |
| Hydrolysis product: phenylsilanetriol | 12.3 E-12 | 0.9 |  |
| DClMPS | 2.1 E-12 | 5.1 |  |
| Hydrolysis product: methylphenylsilanediol | 9.0 E-12 | 1.2 |  |

### Stability in Water

A hydrolysis study was conducted on supporting substance DClDPS. The hydrolysis mechanism is the same for all chlorosilanes.  The hydrolysis reactions of chlorosilanes proceed by nucleophilic attack of water at the Si atom, resulting in substitution of an –OH group for each –Cl atom attached to Si in the parent silane.  Due to the strong silicon-oxygen bonds formed during hydrolysis, this is a highly exothermic process.  The –Cl atoms are eliminated as HCl, which will ionize in water to form H+ and Cl- ions, leading to a pH reduction in unbuffered systems.

TClPS:  No hydrolysis study is available for TClPS. However, a reliable study according to OECD TG 111 is available for DClDPS.  These data suggest rapid hydrolysis for TClPS is expected.

DClMPS:  No hydrolysis study is available for DClMPS. However, a reliable study according to OECD TG 111 is available for DClDPS. These data suggest rapid hydrolysis for DClMPS is expected.

Supporting substance DClDPS:  An OECD  TG 111 (Hydrolysis as a Function of pH) study was conducted for DClDPS; with a t1/2 of less than a minute at pH 4, 7 and 9 and at 1.5°C (Miller and Kozerski, 2001).

Rapid hydrolysis has been shown for other chlorosilanes for which measured data is available (for example, the half-lives are <1 minute for dichloro(dimethyl)silane (CAS No. 75-78-5) and trichloro(methyl)silane (CAS No. 75-79-6).  Observed rates of hydrolysis were so rapid in all cases that it was not possible to distinguish among the different pH conditions) http://www.oecd.org/env/hazard/data. Therefore, it can be concluded that the sponsored substances will hydrolyse rapidly under environmentally relevant conditions and no further testing is necessary.

### Transport between Environmental Compartments

The Level III Fugacity model (Full Output) was used to evaluate the fate, transport and distribution of the phenylchlorosilanes and the associated hydrolysis products between environmental matrices.  The results of Level III fugacity modelling, using equal loading rates of 1000 kg/h each for air, soil and water (or 1000 kg/h for air only) are presented in Table 8.  The sponsored substances are expected to distribute mainly to the air (47.6%) and soil (47.7%) compartments with minor distribution to the water and sediment compartments. Level III fugacity modeling using equal loading rates of 1000 kg/h each for air, soil and water predicts that the silanol hydrolysis products will distribute mainly to soil (ca. 83%), with a smaller fraction to water (ca. 16%) and negligible amounts to sediment and air. Based on the more realistic scenario of 100% release to air, the model predicts that the silanol hydrolysis products will be distributed mainly in air (96%) and water (ca. 4%).  Fugacity modeling of HCl is not applicable (http://www.oecd.org/env/hazard/data).

Table 8. 	Summary of Level III fugacity data

| Substance | % Distribution (air/water/soil/sediment) | Reference |
| --- | --- | --- |
| TClPS | 47.6/4.8/47.7/<0.1 (100/<0.1/<0.1/<0.1)1 | USEPA 2012 or 2013, EPI Suite (v4.10) |
| Hydrolysis product: phenylsilanetriol | <0.1/16.3/83.2/0.6 (<0.1/3.9/96/0.1)1 |  |
| DClMPS | 47.6/4.8/47.7/<0.1 (100/<0.1/<0.1/<0.1)1 |  |
| Hydrolysis product: methylphenylsilanediol | <0.1/16.3/83.1/0.6 (<0.1/3.9/96/0.1)1 |  |

1 For 100% release to air.

### Biodegradation

One phenylchlorosilane biodegradation study is available for TClPS.  No measured data are available for the hydrolysis products.  At high concentrations (at concentrations greater than 500 mg/L), the silanols will condense to form highly cross linked, high molecular weight polymers that are water insoluble and effectively nonbiodegradable.

TClPS: The biodegradation of TClPS was determined in OECD Guideline 310 (Ready Biodegradability - CO2 in Sealed Vessels (Headspace Test) (Clarke, 2009).  There was essentially no (1%) biodegradation of the test substance in 28 days. The reference substance (benzoic acid, sodium salt) attained 85% degradation after 14 days, indicating the test was sensitive and valid. TClPS is considered to be not readily biodegradable.

DClMPS: The biodegradation of DClMPS was not determined due to rapid hydrolysis; any potential for biodegradation is likely to be of the hydrolysis products. A reliable study according to OECD TG 310 is available for TClPS, that TClPS is not readily biodegradable.  These data suggest that DClMPS is expected to be not readily biodegradable.

Hydrolysis products:

No measured data are available for the silanol hydrolysis products.  At high concentrations (at concentrations greater than 500 mg/L), the silanols will condense to form highly cross linked, high molecular weight polymers that are water insoluble and effectively nonbiodegradable.

HCl. No data located. HCl is an inorganic compound and biodegradation tests are not applicable.

### Bioaccumulation

The bioaccumulation potential of the phenylchlorosilanes was not located. The estimated BCFs using the BCFBAF Program (v3.01) are presented in Table 9. The sponsored substances are not expected to bioaccumulate in the aquatic environment based on rapid hydrolysis to silanols.  The calculated bioconcentration factors for the sponsored substances were 110 (TClPS) – 158.0 (DClMPS) L/kg wet-wt.  For the hydrolysis products, the estimated BCFs were 3.2 L/kg wet-wt.

Table 9. 	Summary of Bioaccumulation data

| Substance | Test | BCF (L/kg wet-wt) | Reference |
| --- | --- | --- | --- |
| TClPS | Modeling | 110.8 | USEPA 2012 or 2013, EPI Suite (v4.10) |
| Hydrolysis product: phenylsilanetriol |  | 3.162 |  |
| DClMPS |  | 158.9 |  |
| Hydrolysis product: methylphenylsilanediol |  | 3.162 |  |

## Human Exposure

### Occupational Exposure

The phenylchlorosilanes are produced and processed in closed systems; commercial customers use the phenylchlorosilanes in closed systems.  Due to the dynamic and exothermic nature of the processes incorporating chlorosilanes, many engineering controls are always in place to prevent occupational exposure such as water scrubber devices and related equipment; closed sampling loop; and local and general ventilation.  Employees involved in chlorosilane production and application use personal protective equipment (PPE) such as safety glasses or goggles, steel-tipped shoes, flame-resistant clothing, hard hats, chemical resistant gloves, and respirator masks.  Potential routes of exposure include inhalation and dermal exposure (Guerrero, 2012).

### Consumer Exposure

There are no consumer uses of the phenylchlorosilanes (Guerrero, 2012).

# human health hazards

## Effects on Human Health

Due to reactivity, the phenylchlorosilanes are expected to hydrolyse prior to exposure, or locally at the port of entry, to form HCl and corresponding silanol hydrolysis products (phenylsilanetriol (from TClPS) and methylphenylsilanediol (from DClMPS)).  In summary, for mammalian toxicity endpoints, data are provided for hydrogen chloride and TMPS (previously assessed in the OECD Cooperative Chemicals Assessment Programme).

TMPS hydrolyses and is expected to form three moles of methanol and one mole of phenylsilanetriol. TMPS hydrolyses more slowly at pH 7 (half-life ca. 0.4 hours), but under acidic conditions such as in the stomach following ingestion, much more rapid hydrolysis can be expected (RCC, 2008). TMPS is used to fill the repeated dose, in vitro mammalian genotoxicity, fertility and developmental toxicity endpoints.  It is of note that the oral NOAEL in rats (500 mg/kg bw/day) for methanol is greater than the dose that is expected to be generated from hydrolysis of TMPS in the stomach. Therefore the effects of TMPS following a dose of 100 mg/kg bw/day are not thought to be attributable to methanol.

The primary human health hazard for the phenylchlorosilanes is considered to be exposure to the hydrogen chloride hydrolysis product.  The levels of chlorosilane required to generate concentrations near those tested for the silanols would result in severely corrosive HCl concentrations.

### Toxicokinetics

There are no in vivo data on the toxicokinetics of the phenylchlorosilanes. The following summary has been prepared based on validated predictions of the physicochemical properties of the phenylchlorosilanes. These substances are moisture-sensitive, volatile liquids that are expected to hydrolyse rapidly in contact with water (half-life <1 minute at pH 7), generating HCl and phenylsilanediol or –triol. Human exposure can occur via the inhalation or dermal routes. Relevant inhalation exposure might be to the hydrolysis products if air is humid enough to lead to hydrolysis. The substance would also likely hydrolyse rapidly in contact with moist skin and the moisture rich environment of the respiratory tract. The resulting HCl hydrolysis product would be severely irritating or corrosive.

Absorption

Oral: Significant oral exposure is not expected for this corrosive substance.

Dermal: Based on the molecular weights of the parent and hydrolysis products (≤211 g.mol), absorption across the skin may be possible. However, the expected water solubility and predicted log Kow of the hydrolysis products phenylsilanediol or -triol (Table 4), suggest that they may be too hydrophilic to cross the lipid rich stratum corneum. Therefore dermal uptake is likely to be low. Since the other hydrolysis product, HCl is corrosive to the skin; damage to the skin might increase penetration. There are no reliable toxicity studies on the sponsored substances that could be used to evaluate the potential for dermal absorption.

Inhalation: The high water solubility of the phenylchlorosilanes hydrolysis products might lead to some of the phenylsilanediol or -triol being retained in the mucous of the lungs. As with dermal exposure, damage to membranes caused by the corrosive nature of the other hydrolysis product, HCl might enhance uptake of the parent silane and phenylsilanediol/triol hydrolysis product. There are no reliable toxicity studies on the sponsored substances that could be used to evaluate the potential for inhalation absorption.

Distribution

Much of the absorbed material is likely to be in the form of the hydrolysis products. The hydrophilic nature of phenylsilanediol or -triol will limit diffusion across membranes (including the blood-brain and blood-testes barriers) and its accumulation in fatty tissues.  Hydrogen and chloride ions will enter the body’s natural homeostatic processes.

Metabolism

The phenylchlorosilanes are rapidly hydrolysed to phenylsilanediol or -triol and HCl in the presence of water. Much of this may occur before absorption into the body depending on environmental conditions. There are no data regarding the metabolism of phenylsilanediol or –triol or diphenylsilanediol.  HCl will dissociate into hydrogen and chloride ions.

Excretion

The low molecular weight and high water solubility of phenylsilanediol or -triol or diphenylsilanediol suggest that it may well be rapidly eliminated via the kidneys in urine. Hydrogen and chloride ions will enter the body’s natural homeostatic processes.

Conclusion

No data are available on the toxicokinetics, metabolism and distribution of the phenylchlorosilanes.  However, these substances rapidly hydrolyse to HCl and the corresponding silanol hydrolysis products on contact with moisture. Damage to membranes caused by the corrosive nature of HCl might enhance the uptake of the sponsored substances or the silanol hydrolysis products. Hydrogen and chloride ions will enter the body’s natural homeostatic processes. HCl will rapidly dissociate and its effects are thought to be a result of pH change (local deposition of H+). The hydrophilic nature of phenylsilanediol or -triol will likely limit diffusion across certain membranes. The low molecular weight and high water solubility of the silanols suggest likely elimination via the kidneys in urine.

### Acute Toxicity

An acute oral toxicity study is available for the sponsored chemical, DClMPS.  Acute toxicity data are also presented for the supporting substance TMPS (oral) and the hydrolysis product HCl (inhalation and oral). No acute dermal toxicity data are available.

Studies in Animals

Inhalation

Sponsored substances:

No data located.

Hydrolysis product:

HCl: Inhalation LC50 values were determined to be 23.7-60.9 mg/L/5 min, 5.7-7.0 mg/L/30 min and 4.2-4.7 mg/L/60 min for rats, 20.9 mg/L/5 min, 3.9 mg/L/30 min and 1.7 mg/L/60 min for mice, depending on exposure period.

Oral

Sponsored substances:

TClPS: No data located

DClMPS: In an OECD 423 (Acute Oral toxicity - Acute Toxic Class Method) study, the test substance was administered by oral gavage to 3 male Crl:CD rats at 2000 mg/kg bw (neat) and to 3 male and 3 female rats at 200 mg/kg bw (corn oil vehicle) (LPT, 2002a); animals were observed for 14 days after dosing.  All animals administered 2000 mg/kg bw died within 30 minutes, and one male died at 200 mg/kg bw within 7 days.  Clinical signs observed in all animals at 2000 mg/kg bw included reduced mobility, ataxia, reduced muscle tone, dyspnea and ptosis; there were no clinical signs at 200 mg/kg bw. There were no effects on body weight and no findings at gross necropsy.  The oral LD50 was > 200 - <2000 mg/kg bw.

Supporting substance:

TMPS: In an acute toxicity study conducted according to OECD TG 425 (Acute Oral Toxicity – Up-and-Down Procedure), a total of 7 female rats were administered TMPS by oral gavage (including 4 animals at 2000 and 3 animals at 550 mg/kg bw) in polyethylene glycol 300.  When the single animal dosed at 2000 mg/kg bw in a limit-test died spontaneously on study day 2, a main test with 6 animals was conducted.  In the main test, all three animals dosed at 2000 mg/kg bw had to be killed in extremis on study days 4, 5 or 6 and all animals dosed at 550 mg/kg bw survived.  Animals at 2000 mg/kg bw appeared unhealthy; effects on respiration, coordination and body weight loss were noted. Slight ruffled fur and slight sedation were observed in two of three animals dosed at 550 mg/kg; no clinical signs were observed in the third animal dosed at 550 mg/kg bw. There was no effect on body weight at 550 mg/kg bw. Three animals dosed at 2000 mg/kg bw showed a greater than 20% loss of body weight prior to being killed in extremis; no body weight measurement was recorded at the spontaneous death of the first animal dosed at 2000 mg/kg bw that was found dead on test day 2. Macroscopic findings at 2000 mg/kg bw included light red congested lungs, black brown stomach distended with gas, tan discoloration of kidneys, and spleen reduced in size; there were no macroscopic findings at 550 mg/kg bw.  The estimated LD50 was 1049 mg/kg bw.

Hydrolysis product:

HCl: The acute oral LD50 values were determined to be 238-277 mg/kg bw for female rats.

Table 10. 	 Summary of Acute Toxicity Studies

| Species | Species/Sex | Exposure route | LD(C)50 | Reference |
| --- | --- | --- | --- | --- |
| Sponsored substances |  |  |  |  |
| DClMPS | Rat/male and female | Oral (gavage) | LD50 > 200 - <2000 mg/kg bw | LPT, 2002a |
| Supporting substance |  |  |  |  |
| TMPS | Rat/male and female | Oral (gavage) | LD50 =1049 mg/kg bw | http://www.oecd.org/env/hazard/data |
| Hydrolysis product |  |  |  |  |
| HCl | Rat/female | Oral (gavage) | LD50 = 238-277 mg/kg bw | http://www.oecd.org/env/hazard/data |
|  | Rats and mice | Inhalation | 1-hr LC50= 4.2-4.7 mg/L for rats, 1.7 mg/L for mice |  |

Conclusion

Acute inhalation studies were not located for the phenylchlorosilanes.  The acute inhalation toxicity of the phenylchlorosilanes is expected to be well-characterized by the effects of HCl exposure, rather than systemic effects of silanol hydrolysis products.  The principal clinical signs are expected to be indicative of respiratory and ocular effects resulting from HCl exposure. Inhalation LC50 values for HCl were determined to be 4.2-4.7 mg/L for 1 hour for rats. The oral LD50 for one of the phenylchlorosilane category members, DClMPS, was >200 to <2000 mg/kg bw in rats [OECD TG 423].  For the supporting substance TMPS, in an acute toxicity study conducted according to OECD TG 425, a total of 7 female rats were administered TMPS by oral gavage including 4 animals at 2000 and 3 animals at 550 mg/kg bw in polyethylene glycol 300. When the single animal dosed at 2000 mg/kg bw in a limit test died spontaneously on study day 2, a main test with 6 animals was conducted. In the main test, all three animals dosed at 2000 mg/kg bw had to be killed in extremis on study days 4, 5 or 6 and all animals dosed at 550 mg/kg bw survived. Animals at 2000 mg/kg bw appeared unhealthy; effects on respiration, coordination and body weight loss were noted. Slight ruffled fur and slight sedation were observed in two of three animals dosed at 550 mg/kg bw; no clinical signs were observed in the third animal dosed at 550 mg/kg bw. There was no effect on body weight at 550 mg/kg bw. Three animals dosed at 2000 mg/kg bw showed a greater than 20% loss of body weight prior to being killed in extremis; no body weight was recorded at the spontaneous death of the first animal dosed at 2000 mg/kg bw that was found dead on test day 2. Macroscopic findings at 2000 mg/kg bw included light red congested lungs, black brown stomach distended with gas, tan discoloration of kidneys, and spleen reduced in size; there were no macroscopic findings at 550 mg/kg bw. The estimated LD50 was 1049 mg/kg bw.  The acute oral LD50 values of HCl were determined to be 238-277 mg/kg bw for female rats.

### Irritation

Skin and eye irritation data are not available for the phenylchlorosilanes. Skin and eye irritation data and respiratory tract irritation data are available for HCl.

Skin Irritation

Studies in Animals

Hydrolysis product:

HCl. Hydrogen chloride is corrosive to the skin.

Eye Irritation

Studies in Animals

Hydrolysis product:

HCl. Severe effects can be expected from exposure to the eyes.

Respiratory Tract Irritation

Studies in Animals

Hydrolysis product:

HCl. Animals died from respiratory failure (alveolar emphysema, atelectasis, and oedema of the lungs) shortly after acute inhalation exposure.

Conclusion

Irritation data are not available for the sponsored substances. The phenylchlorosilanes rapidly hydrolyse to HCl and the associated silanol. HCl is corrosive and highly irritating to the skin, eyes and respiratory tract.  As such, the sponsored substances are expected to be corrosive to the skin, cause serious damage to the eyes and be highly irritating to the respiratory tract.

### Sensitisation

Sensitization data are not available for the phenylchlorosilanes or the expected hydrolysis products.

### Repeated Dose Toxicity

Repeated dose toxicity data for the phenylchlorosilanes are not available. Repeated oral and inhalation exposure to the supporting substance TMPS and inhalation data for the hydrolysis product HCl are available.

Studies in Animals

Inhalation

Supporting substance:

TMPS: In a 4-week whole-body vapour inhalation (7 hr/day, 5 days/week) study conducted with male and female Sprague-Dawley rats [similar to OECD TG 412 (Subacute Inhalation Toxicity: 28-Day Study)], there were no adverse treatment-related effects noted at any of the vapour concentrations administered. The NOAEC was determined to be 0.649 mg/L (highest concentration tested).

Hydrolysis product:

HCl: Several 90-day repeated dose inhalation toxicity studies, compliant with FDA-GLP, have been conducted resulting in local effects of irritation in groups of 0.015 mg/L and above.  The NOAECs for systemic toxicity have been determined to be 0.030 mg/L for rats and mice.

Oral

Supporting substance:

TMPS: In a combined repeated-dose/reproductive/developmental toxicity screening test [OECD TG 422], TMPS was administered to four groups of 10 rats/sex/dose level by gavage daily at 0 (dried and deacidified corn oil), 100, 250 and 500 mg/kg bw/day.  Males were exposed for 28 days (including 14 days prior to pairing) and females were exposed for 14 days prior to pairing, through the pairing and gestation periods until the F1 generation reached day 4 postpartum. Administration of TMPS at 500 mg/kg bw/day caused a reduction of food consumption in males during the first week of treatment and in females during the pre-pairing period up to day 14 of the gestation period. At this dose, reduced body weight was noted in males throughout the study and in females during the latter half of the gestation period. An increase in urea and cholesterol concentrations was seen in males. Multifocal tubular degeneration/regeneration and transitional cell hyperplasia of kidney were noted in males and females; and male kidney weights (absolute and relative) were increased. At 250 mg/kg bw/day, an increase in urea concentration was observed in males. Multifocal tubular degeneration/regeneration and transitional cell hyperplasia of kidney were noted in males and females at this dose. At all dose levels, the urinary bladder was observed to be thickened in males and females. This finding correlated with the histopathology examination showing perivascular lymphoid cell infiltration and transitional cell hyperplasia of the urinary bladder, along with dilation and bladder congestion.  Based on the findings in urinary bladder, a NOAEL for systemic toxicity of TMPS could not be established; the LOAEL was 100 mg/kg bw/day.

Table 11.	 Summary of Repeated Dose Toxicity

| Substance | Species, sex | Exposure doses and duration | Value (combined sexes) NOAEC/LOAEC | Reference |
| --- | --- | --- | --- | --- |
| Sponsored substances |  |  |  |  |
| TClPS | No data located |  |  |  |
| DClMPS | No data located |  |  |  |
| Supporting substance |  |  |  |  |
| TMPS | OECD 412; Rat/male and female | 0, 0.081, 0.406, and 0.649 mg/L (inhalation); 7 hr/d, 5 d/wk for 4 wks | NOAEC = 0.649 mg/L | http://www.oecd.org/env/hazard/data |
|  | OECD 422; Rat/male and female | 0, 100, 250 and 500 mg/kg bw/d (oral gavage); Males: 28 d, Females: 14 days prior to pairing, through pairing and gestation until F1 generation reached d 4 postpartum | LOAEL = 100 mg/kg bw/d |  |
| Hydrolysis product |  |  |  |  |
| HCl | Repeated dose inhalation; Rats and mice/males and females of both species | 0, 10, 20, or 50 ppm (0, 0.015, 0.030, 0.075 mg/L); (inhalation) 90 d, 6hr/d, 5 d/wk | NOAEC = 0.03 mg/L LOAEC = 0.075 mg/L | http://www.oecd.org/env/hazard/data |

Conclusion

Repeated dose toxicity data are not available for the sponsored substances.  Data from supporting substance TMPS and hydrolysis product HCl are used to fill the repeated-dose toxicity endpoint for the phenylchlorosilanes. TMPS was administered to four groups of 10 rats/sex/dose level by gavage daily at 0 (dried and deacidified corn oil), 100, 250 and 500 mg/kg bw/day. Males were exposed for 28 days (including 14 days prior to pairing) and females were exposed for 14 days prior to pairing, through the pairing and gestation periods until the F1 generation reached day 4 postpartum. Administration of TMPS at 500 mg/kg bw/day caused a reduction of food consumption in males during the first week of treatment and in females during the pre-pairing period up to day 14 of the gestation period. Reduced body weight was noted in males throughout the study and in females throughout the gestation period. Kidney weight was increased in males and thickened urinary bladder in males and females was seen at this dose. An increase of concentration of urea, bile acids and cholesterol was also noted in males at 500 mg/kg bw/day. Multifocal tubular degeneration/regeneration and transitional cell hyperplasia of kidney were noted in males and females. At 250 mg/kg bw/day, an increase in the urea and bile acid concentrations was observed. Multifocal tubular degeneration/regeneration and transitional cell hyperplasia of kidney were noted in males and females at this dose. The urinary bladder was observed to be thickened in males and females at all dose levels. This finding correlated with the histopathology examination showing perivascular lymphoid cell infiltration and transitional cell hyperplasia of the urinary bladder.  Based on the findings in urinary bladder, a NOAEL (No Observed Adverse Effect Level) for systemic toxicity of TMPS in rats could not be established.  The LOAEL was 100 mg/kg bw/day.  For the analogue substance TMPS, in a 4-week whole-body vapour inhalation (7 hr/day, 5 days/week) study conducted similar to OECD TG 412, there were no adverse treatment-related effects noted at any of the vapour concentrations administered.  The NOAEC was determined to be 0.649 mg/L (highest concentration tested). In repeated dose toxicity studies of HCl by the inhalation route, local irritant effects of HCl were observed in groups of rats and mice exposed to 0.015 mg/L and above.  The inhalation NOAEC for systemic toxicity for HCl, excluding the local effects of irritation, has been determined to be 0.030 mg/L, with a LOAEC of 0.075 mg/L.   The toxicity of the phenylchlorosilanes is expected to be well characterized by the effects of HCl inhalation exposure, the prevalent route of the phenylchlorosilanes exposure.

### Mutagenicity

Bacterial mutagenicity studies have been conducted with the phenylchlorosilanes.  An in vitro mouse lymphoma study has been conducted with TClPS.  In vitro bacterial and mammalian genotoxicity studies have been conducted with the supporting substance TMPS and the hydrolysis product HCl.

Studies in Animals

In vitro Studies - Gene mutations

Sponsored substances:

TClPS: (1) In a study conducted equivalent or similar to OECD 471 (Bacterial Reverse Mutation Assay), S. typhimurium TA98, TA100, TA1535, and TA1537 and E. coli WP2 were exposed to the test substance at concentrations of 0.078, 0.156, 0.312, 0.625, and 1.25 µg/plate in the presence and absence of metabolic activation (Aroclor 1254 induced rat liver S9) (Isquith, 1985). Cytotoxicity was observed at >1.25 µg/plate (data not shown). No mutagenic effects were observed for the test substance without and with metabolic activation. Appropriate concurrent vehicle and positive controls were included and the expected responses were observed. The results indicate that the test substance was not mutagenic under these test conditions.

(2) In an OECD 476 (In vitro Mammalian Cell Gene Mutation Test), mouse lymphoma L5178Y cells were exposed to the test substance in the presence or absence of metabolic activation (Phenobarbital/beta-naphthoflavone-induced rat liver S9) at concentrations of 200 to 900 μg/ml (4h with S9, 24h without S9) or 300 to 1000 μg/ml (4 h without S9) (Flanders, 2010).  Cytotoxicity was observed at 900 µg/ml. No increase in mutant frequency was observed at any concentration with and without activation (4 hour treatment) and without activation (4 or 24 hour treatment). Expected results were obtained with positive and vehicle controls. It is concluded that TClPS was negative for the induction of mutations in L5178Y cells under the conditions of the test.

DClMPS: In an OECD 471, bacterial reverse mutation assay, S. typhimurium TA98, TA100, TA102, TA1535 and TA1537 were exposed in the presence and absence of metabolic activation (rat liver; S9 mix induced with Aroclor 1254) to the test substance at concentrations of 10, 31.6, 100, 316 and 1000 µg/plate in a plate incorporation test and to concentrations of 1, 3.16, 10, 31.6, and 100 µg/plate in a pre-incubation test (Leuschner, 2002b). Cytotoxicity was observed at the highest concentration in each test. No mutagenic effects were observed for the test substance without and with metabolic activation. Appropriate concurrent vehicle and positive controls were included and the expected responses were observed. The results indicate that the test substance was not mutagenic under these test conditions.

Supporting substance:

TMPS: In a bacterial reverse mutation assay (EEC Directive, No. L251, Vol 27), TMPS did not induce gene mutations or cytotoxicity in bacterial cells (Salmonella typhimurium TA1535, TA1537, TA98 and TA100 and Escherichia coli WP2 uvrA), with or without metabolic activation at concentrations up to 5000 µg/plate.

Hydrolysis product:

HCl: HCl was negative in the Ames test.

In vitro Studies - Chromosomal aberrations

Sponsored substances:

TClPS: No data located.

DClMPS: No data located.

Supporting substance:

TMPS: TMPS induced chromosomal aberrations in Chinese hamster V79 cells in vitro in the presence of metabolic activation [OECD TG 473 (In Vitro Mammalian Chromosome Aberration Test)].

Hydrolysis product:

HCl. Positive results have been obtained in the in vitro chromosome aberration test with HCl; however, the positive result was considered to be the effect of low pH.

In vivo Studies - Chromosomal aberrations

Sponsored substances:

No data located

Table 12.	 Summary of Genotoxicity Data

| Substance | Gene Mutation |  | Chromosome Aberration |  |
| --- | --- | --- | --- | --- |
|  | Test/Result | Reference | Test/Result | Reference |
| Sponsored substances |  |  |  |  |
| TClPS | Similar to OECD TG 471 Bacterial reverse mutation assay/negative | Isquith, 1985 | No data located |  |
|  | OECD TG 476 In vitro Mammalian Cell Gene Mutation test/negative | Flanders, 2010 |  |  |
| DClMPS | OECD TG 471 Bacterial reverse mutation assay/negative | Leuschner, 2002b | No data located |  |
| Supporting substance |  |  |  |  |
| TMPS | EEC Directive, No. L251, Vol 27, bacterial reverse mutation assay/negative |  | OECD TG 473/positive | http://www.oecd.org/env/hazard/data |
| Hydrolysis product |  |  |  |  |
| HCl | Ames test/negative |  | In vitro chromosome aberration/positive | http://www.oecd.org/env/hazard/data |

Conclusion

The sponsored substances did not induce gene mutations in bacterial cells in vitro [similar or equivalent to OECD TG 471], and the sponsored substance TClPS was negative for induction of gene mutations in mouse lymphoma cells [OECD TG 476].  Chromosomal aberration studies were not located for the sponsored substances.  The supporting substance TMPS did not induce gene mutations in bacterial cells [similar to OECD 471], but did induce chromosomal aberrations in Chinese hamster V79 cells in vitro in the presence of metabolic activation [OECD TG 473]. Based on these results, TMPS is considered to be genotoxic in vitro. The hydrolysis product, HCl, did not induce gene mutations in bacterial cells.  Positive results in the in vitro chromosome aberration test with HCl were considered to be the effect of low pH.  Based on the available data, the phenylchlorosilanes are not expected to cause gene mutations in vitro; they may be clastogenic in vitro.

### Carcinogenicity

No data located

### Toxicity for Reproduction

Toxicity for reproduction data is not available for the phenylchlorosilanes. An oral gavage combined repeated-dose/reproductive/developmental toxicity screening test is available for the supporting substance TMPS and repeated-dose inhalation toxicity studies with histological examinations of male and female reproductive organs of mice and rats are available for the hydrolysis product HCl.

Studies in Animals

Effects on Fertility

Sponsored substances:

No data located

Supporting substance:

TMPS: In the combined repeated-dose/reproductive/developmental toxicity screening test [OECD TG 422] with TMPS, the NOAEL for reproductive/developmental toxicity was 500 mg/kg bw/day (highest dose tested). The LOAEL for maternal toxicity was 100 mg/kg bw/day. TMPS did not show evidence of reproductive toxicity.

Hydrolysis product:

HCl: No reliable studies have been reported regarding toxicity to reproduction and development in animals after oral, dermal or inhalation exposure to hydrogen chloride/hydrochloric acid.  Because proton and chloride ion are the normal constituents in the body fluid of animal species, lower concentration of hydrogen chloride gas/mist or solution does not seem to cause adverse effects to animals.  In addition, no effects on the gonads were observed in a 90-day inhalation repeated-dose study up to concentrations of 0.075 mg/L HCl.

Developmental Toxicity

Sponsored substances:

No data located

Supporting substance:

TMPS: In the combined repeated-dose/reproductive/developmental toxicity screening test [OECD TG 422] with TMPS, the NOAEL for reproductive/developmental toxicity was 500 mg/kg bw/day (highest dose tested). The LOAEL for maternal toxicity was 100 mg/kg bw/day. TMPS did not show evidence of developmental toxicity.

Hydrolysis product:

HCl. No reliable studies have been reported regarding toxicity to reproduction and development in animals after oral, dermal or inhalation exposure to hydrogen chloride/hydrochloric acid.  Because proton and chloride ion are the normal constituents in the body fluid of animal species, lower concentration of hydrogen chloride gas/mist or solution does not seem to cause adverse effects to animals.  In fact, the cells of gastric glands secrete hydrochloric acid into the cavity of stomach, and orally administered sulfuric acid, which results in pH change as well, did not cause developmental toxicity to laboratory animals.  These facts indicate that hydrogen chloride/hydrochloric acid is not expected to have developmental toxicity.

Table 13. 	Summary of Reproductive Toxicity

| Substance | Species, sex | Exposure doses and duration | Value (combined sexes) NOAEL(C)/LOAEL(C) | Reference |
| --- | --- | --- | --- | --- |
| Sponsored substances |  |  |  |  |
| TClPS | No data located |  |  |  |
| DClMPS | No data located |  |  |  |
| Supporting substance |  |  |  |  |
| TMPS | OECD TG 422; oral gavage; rat/male and female | 0, 100, 250 and 500 mg/kg bw/d; Males: 28 d, Females: 14 days prior to pairing, through pairing and gestation until F1 generation reached d 4 postpartum | NOAEL (reproductive/developmental effects) = 500 mg/kg bw (highest oral gavage dose tested) | http://www.oecd.org/env/hazard/data |
| Hydrolysis product |  |  |  |  |
| HCl | Repeated dose inhalation; rats and mice/males and females | 0, 10, 20, or 50 ppm (0, 0.015, 0.030, 0.075 mg/L)/90 d, 6hr/d, 5 d/wk | NOAEC (systemic toxicity) = 0.03 mg/L; NOAEC (histological effects on reproductive organs) = 0.075 mg/L | http://www.oecd.org/env/hazard/data |

Conclusion

Toxicity for reproduction data are not available for the sponsored substances.  Data for supporting substance TMPS and hydrolysis product HCl are used to fill the toxicity to reproduction toxicity endpoint for the phenylchlorosilanes. In the combined repeated-dose/reproductive/developmental toxicity screening test [OECD TG 422] with rats, the NOAEL for reproductive/developmental toxicity was 500 mg/kg bw/day (highest dose tested).  Overall, TMPS did not show evidence of reproductive/developmental toxicity.  No reliable studies have been reported regarding toxicity to reproduction and development in animals after oral, dermal or inhalation exposure to hydrogen chloride/hydrochloric acid.  Because proton and chloride ion are the normal constituents in the body fluid of animal species, low concentrations of hydrogen chloride gas/mist or solution do not seem to cause adverse effects to animals. In fact, the cells of gastric glands secrete hydrochloric acid into the cavity of the stomach and orally administered sulfuric acid, which results in pH change as well, did not cause developmental toxicity to laboratory animals.  These facts indicate that hydrogen chloride/hydrochloric acid is not expected to have developmental toxicity.  In addition, no effects on the gonads were observed in a 90-day inhalation repeated-dose study up to concentrations of 0.075 mg/L HCl. The toxicity of the phenylchlorosilanes is expected to be well characterized by the effects of HCl inhalation exposure, the prevalent route of the phenylchlorosilanes exposure, and the phenylchlorosilanes are not expected to be reproductive/developmental toxicants.

The sponsored phenylchlorosilanes possess properties indicating a hazard for human health [lethality from acute studies (oral and inhalation), corrosive and highly irritating to the skin, eyes and respiratory tract (based on the hydrolysis product, HCl) and repeated dose toxicity (based on the analogue substance, TMPS), and genotoxic (clastogenic in vitro based on analogue substance, TMPS)]. Adequate screening-level data are available to characterize the human health hazard for the purposes of the OECD Cooperative Chemicals Assessment Programme.

# hazards to the environment

## Aquatic Effects

Acute toxicity data for fish, daphnids and algae are available for TClPS and, with the exception of acute toxicity data for algae, DClMPS.   The phenylchlorosilanes undergo rapid hydrolysis, which occurs during testing; exposure to parent chlorosilane is likely to be transient and observed toxicity in standard test systems will therefore, depending on the conditions of the system (e.g. pH and concentration of test material), likely be due its hydrolysis products, HCl and the corresponding silanol hydrolysis product (condensed silanol material at concentrations greater than 500 mg/L). Acute toxicity data for fish, daphnids and algae are available for hydrolysis product, HCl.

Acute Toxicity Test Results

Fish

Sponsored substances:

TClPS: In an OECD Guideline 203 (Fish, Acute Toxicity Test), groups of rainbow trout (Oncorhynchus mykiss; one replicate of 7 fish/group) were exposed to nominal concentrations of TClPS at 0 or 100 mg/L (limit test) for 96 hours under static conditions (Springborn Smithers, 2009a).  Test solutions were stirred overnight, allowing TClPS to undergo hydrolysis to HCl and the corresponding silanol (phenylsilanetriol, CAS No. 3047-74-3).  Analytical monitoring of TClPS and its hydrolysis products was not conducted.  The pH of test solutions was adjusted to 7.0 with sodium hydroxide before fish were introduced; therefore, HCl produced from hydrolysis of TClPS is likely to have been neutralized. There were no deaths or behavioural signs of exposure-related toxicity.  The 96-hour LC50 was >100 mg/L (nominal).

DClMPS: No data located.

Hydrolysis products:

HCl. The hazard of hydrochloric acid to the environment is caused by the proton (pH effect).  For this reason, the effect of hydrochloric acid on aquatic organisms depends on the buffering capacity of the aquatic ecosystem.  Also, the variation in acute toxicity for aquatic organisms can be explained to a large extent by the variation in buffering capacity of the test medium.  For example, LC50 values for acute fish toxicity tests varied from 4.92 to 282 mg/L.  Several studies on acute toxicity of HCl to fish have been published.  The lowest reported 96-hour LC50 of 4.92 mg HCl/L (pH = 4.3) is from a semi-static test, conducted according to OECD TG 203, with Cyprinus carpio.

Aquatic invertebrates

Sponsored substances:

TClPS: In a study according to OECD Guideline 202 (Daphnia sp., Acute Immobilisation Test), groups of Daphnia magna (4 replicates of 5 daphnids/group) were exposed to solutions of TClPS for 48 hours under static conditions (Springborn Smithers, 2009b).  Nominal test concentrations of TClPS were 0 (control), 0 (solvent control), 2.0, 4.3, 9.4, 20.7, 45.5 and 100 mg/L.  Test solutions were stirred overnight, allowing TClPS to undergo hydrolysis to HCl and the corresponding silanol (phenylsilanetriol, CAS No. 3047-74-3).  The pH of test solutions was adjusted to 7.0 before Daphnia were introduced; therefore, HCl produced from hydrolysis of TClPS is likely to have been neutralized.  The measured concentration of the TClPS hydrolysis product phenylsilanetriol at test start and 48-hour test termination ranged from 93 to 104% of nominal TClPS concentrations.  A concentration-related increase in immobilisation was observed.  At 48 hours, 25% of daphnids at a nominal concentration of 100 mg/L were immobilised.  The 48-hour EC50 was >100 mg/L (nominal) and the NOEC was 9.4 mg/L (nominal).

DClMPS: In a study according to OECD Guideline 202 (Daphnia sp., Acute Immobilisation Test), groups of Daphnia magna (4 replicates of 5 daphnids/group) were exposed to solutions of DClMPS (with and without pH adjustment to 7.9) for 48 hours under static conditions (LPT, 2003).  Nominal concentrations of DClMPS were 0 (control), 2.15, 4.64, 10.0, 21.5, 46.4 and 100 mg/L.  Test solutions were shaken for 30 minutes, after which DClMPS was expected to have undergone hydrolysis to HCl and the corresponding silanol (methylphenylsilanediol, CAS No. 3959-13-5).  Under pH-adjusted conditions, HCl produced from hydrolysis of DClMPS is likely to have been neutralized.  In the test without pH adjustment, the pH range was 6.95-8.12 at nominal concentrations of 2.15 to 21.5 mg DClMPS/L, <5.6 at 46.4 mg DClMPS/L, and <3.0 at 100 mg DClMPS/L.  With pH adjustment, the pH range was 7.76-8.02.  Dissolved organic carbon (DOC) was measured to verify test substance concentrations for the first test (without pH adjustment).  Measured DOC at the highest test concentration (100 mg/L nominal) without pH adjustment was approximately 75% of the theoretical value at the start of the test and approximately 15% of the theoretical value after 48 hours.  At all other test concentrations, measured DOC was 105-167% of nominal concentrations.  The time-dependent decrease in DOC at the highest test concentration was attributed by the study report authors to formation of silanol condensation products that adsorbed to glass surfaces of the test vessel.  A concentration-related increase in immobilization was observed.  Percent immobilization after 48 hours at a nominal concentration of 100 mg/L with and without pH adjustment was 25% and 100%, respectively.  The 48-hour EC50 was 38 mg/L (nominal) without pH adjustment.  With pH adjustment, the 48-hour EC50 was > 100 mg/L (nominal).  With and without pH adjustment, the NOEC was < 2.15 mg/L (nominal).

Hydrolysis products:

HCl. The hazard of hydrochloric acid to the environment is caused by the proton (pH effect).  For this reason, the effect of hydrochloric acid on aquatic organisms depends on the buffering capacity of the aquatic ecosystem.  Also, the variation in acute toxicity for aquatic organisms can be explained to a large extent by the variation in buffering capacity of the test medium.  The result from a study conducted according to OECD TG 202 (conditions not specified) is a 48-hour EC50 of 0.492 mg HCl/L (pH = 5.3) for Daphnia magna.

Aquatic plants

Sponsored substances:

TClPS: In a study conducted according to OECD Guideline 201 (Freshwater Alga and Cyanobacteria, Growth Inhibition Test) under static conditions, Pseudokirchneriella subcapitata (3 replicates) were exposed to solutions of TClPS for 72 hours at nominal test concentrations of 0 (Control), 0 (Solvent control), 6.25, 12.5, 25.0, 50.0 and 100 mg TClPS/L (Springborn Smithers, 2009c).  Test solutions were stirred overnight, allowing TClPS to undergo hydrolysis to HCl and the corresponding silanol (phenylsilanetriol, CAS No. 3047-74-3).  Analytical monitoring of TClPS and its hydrolysis products was not conducted.  The pH of test solutions was adjusted to 7.0 before algae were introduced; therefore, HCl produced from hydrolysis of TClPS is likely to have been neutralized.  The pH was 7.30-7.65 at test initiation and 6.08-9.86 at the end of the test.  No significant effects were observed on cell density, growth rate and biomass.  The 72-hour ErC50 and EbC50 were >100 mg/L (nominal); the 72-hour NOEC was 100 mg/L (nominal).

DClMPS: No data located.

Hydrolysis products:

HCl. The hazard of hydrochloric acid to the environment is caused by the proton (pH effect). For this reason, the effect of hydrochloric acid on aquatic organisms depends on the buffering capacity of the aquatic ecosystem.  Also, the variation in acute toxicity for aquatic organisms can be explained to a large extent by the variation in buffering capacity of the test medium.  For Pseudokirchneriella subcapitata, the 72-hour EC50 and NOEC based on growth rate are 0.492 mg HCl/L (pH 5.3) and 0.097 mg HCl/L (pH = 6.0), respectively.  This study was conducted according to OECD TG 201 under static conditions.

Table 14	 Summary of Acute Aquatic Toxicity

| Test substance | Guideline; Test type | Species | Result (mg/L) | Reference |
| --- | --- | --- | --- | --- |
| Fish |  |  |  |  |
| Sponsored substances |  |  |  |  |
| TClPS | OECD TG 203; static | Rainbow trout (Oncorhynchus mykiss) | 96-hr LC50 >100 (nominal; pH adjusted to 7.0) | Springborn Smithers, 2009a |
| DClMPS | No data located |  |  |  |
| Supporting hydrolysis products |  |  |  |  |
| HCl | OECD TG 203; semi-static | Common carp (Cyprinus carpio) | 96-hr LC50 = 4.92 (pH = 4.3) | http://webnet.oecd.org/Hpv/UI/SIDS_Details.aspx?id=6E6AED5E-C43C-4930-A5FD-F3B4F5F558FF |
| Aquatic invertebrates |  |  |  |  |
| Sponsored substances |  |  |  |  |
| TClPS | OECD TG 202; static | Daphnia magna | 48-hr EC50 > 100 (nominal; pH adjusted to 7.0) | Springborn Smithers, 2009b |
| DClMPS | OECD TG 202; static | Daphnia magna | 48-hr EC50 = 38 (without pH adjustment; nominal); >100 (with pH adjustment to 7.9; nominal) | LPT, 2003 |
| Supporting hydrolysis products |  |  |  |  |
| HCl | OECD TG 202; not specified | Daphnia magna | 48-hr EC50 = 0.492 (pH = 5.3) | http://webnet.oecd.org/Hpv/UI/SIDS_Details.aspx?id=6E6AED5E-C43C-4930-A5FD-F3B4F5F558FF |
| Aquatic plants |  |  |  |  |
| Sponsored substances |  |  |  |  |
| TClPS | OECD TG 201; static | Pseudokirchneriella subcapitata | 72-hr ErC50 and EbC50 > 100 (nominal; pH adjusted to 7.0) | Springborn Smithers, 2009c |
| DClMPS | No data located |  |  |  |
| Supporting hydrolysis products |  |  |  |  |
| HCl | OECD TG 201; static | Pseudokirchneriella subcapitata | 72-hr ErC50 = 0.492 mg/L (pH = 5.3) | http://webnet.oecd.org/Hpv/UI/SIDS_Details.aspx?id=6E6AED5E-C43C-4930-A5FD-F3B4F5F558FF |

Toxicity to microorganisms

Sponsored substances: No data located.

The sponsored phenylchlorosilanes possess properties indicating a hazard for the environment (acute aquatic toxicity values between < 1 and 100 mg/L).  The hydrolysis product, HCl, has properties that can result in toxicity of < 1 mg/L to aquatic organisms in poorly buffered systems, mainly due to acidification of the test medium.  The sponsored phenylchlorosilanes and their hydrolysis products have low potential for bioaccumulation and are not readily biodegradable.  Adequate screening-level data are available to characterize the hazard for the environment for the purposes of the OECD Cooperative Chemicals Assessment Programme.

# references

Bayer AG, Goldschmidt, and Wacker-Chemie (1991) Silicones: Chemistry and Technology.  CRC Press, Boca Raton.  pp. 23-25.

Bazant, V., V. Chvalovsky, and J. Rathousky (1965) Organosilicon Compounds.  Academic Press, New York and London.  pp. 40-75.

Clarke N (2009) Assessment of ready biodegradability; CO2 in sealed vessels (CO2 headspace test) Harlan Laboratories Ltd, Shardlow Business Park, Shardlow, Derbyshire, DE72 2GD, UK 2581/0004 CES.

Design Institute for Physical Property Data (DIPPR) (1965) The AIChE DIPPR 801 Database American Institute of Chemical Engineers (AIChE), New York, N.Y., U.S.A.

Design Institute for Physical Property Data (DIPPR) American Institute of Chemical Engineers (AIChE) (2001) The AIChE DIPPR 801 Database Design Institute for Physical Property Data (DIPPR), New York, N.Y., U.S.A.

Design Institute for Physical Property Data (DIPPR) American Institute of Chemical Engineers (AIChE) (2006) Design Institute for Physical Property Data (DIPPR), New York, N.Y., U.S.A.

Dow Corning Corporation (1991b)  A 14-day repeated dose inhalation toxicity study with Dow Corning X2-2614 in albino rats     Dow Corning Corporation Health and Environmental Sciences, Midland, Michigan, USA  1991-I0000-36885  Dow Corning Corporation     1991-12-18.

Flanders L (2010) Trichloro(phenyl)silane Mouse Lymphoma Assay Harlan Laboratories Ltd Shardlow Business park, Shardlow, Derbyshire DE72 2GD UK Reconsile Project number 2697/0005 2010-01-29.

Guerrero, T. (2012) Senior SEHSC Scientific Programs Manager. Correspondence with SEHSC member companies.

Haas H  (2011)  Determination of physico-chemical properties of silane PM     Wacker Chemie AG, Corporate Analytics WL-CS-A/BGH Johannes-Hess-Strasse 24 D-84489 Burghausen  N-0002036594-P  Wacker Chemie AG S-B-TM/BGH Johannes-Hess-Strasse 24 D-84489 Burghausen     2011-03-24.

Isquith A (1985) Genetic Evaluation of Dow Corning Z-1216 Silane in Bacterial Reverse Mutation Assays Dow Corning Corporation Toxicology Department 1985-I0005-1370 Dow Corning 1985-07-15.

Leuschner (2002b) Mutagenicity Study of Silan PM in the Samonella Reverse Mutation Assay (In Vitro); LPT report No: 15425/40/02; September 4, 2002; Study Sponsor: Wacker Chemie AG.

LPT (2002a) Acute Toxicity of Silan PM by Oral Administration to CD® Rats; LPT report No. 15422/40/02; September 12, 2002; Test Sponsor: Wacker Chemie AG.

LPT (2003) Examination of Silan PM in an Acute Immobilisation Test in Daphnia Magna; LPT Report No. 15426/40/02; April 7, 2003; Sponsor: Wacker Chemie AG.

Miller, J. A.; Kozerski, G. E. (2001) "Hydrolysis Screening Studies of HPV Chlorosilanes"; final report submitted to the Silicones Environmental, Health and Safety Council of North America (SEHSC); Dow Corning Corporation Technical Report No. 2001-I0000-50823. Company study number 9587  2001-03-25.

Noll, W.  (1968)  Chemistry and Technology of Silicones.  Academic Press, New York and London.  pp. 94-95, 208-219.

Peter Fisk Associates (2009a) QSAR Method Reporting Format (QMRF) for the prediction of the density of organosilicon compounds. Reconsile.

Peter Fisk (2009b) QSAR Method Reporting Format (QMRF) for Trichloro(phenyl)silane.  Reconsile.

Peter Fisk Associates (2011a) QSAR Prediction Reporting Format (QPRF) for the vapour pressure of Dichloro(diphenyl)silane. Reconsile.

Peter Fisk Associates (2012) QSAR Model Reporting Format: KOWWIN v1.68 (EPI Suite) for the calculation of the n-octanol-water partition coefficient (log Kow) of organosilicon compounds. Reconsile. PFA.319.002.008. 2012-009-04.

RCC (2008) Hydrolysis determination of Trimethoxyphenylsilane at different pH values. RCC study number B23253. March 28, 2008.

Safepharm (2008) Phenylsilanetriol: Determination of partition coefficient and dissociation constant.  Report No 2581/0003. @008-11-24.

Springborn  Smithers (2009a) Trichloro(phenyl)silane: Acute toxicity to rainbow trout (Oncorhynchus mykiss) under static conditions. Springborn Smithers Laboratories (Europe) Seestrasse 21, Postfach CH-9326 Horn, Switzerland Reconsile Consortium, Avenue Edmond van Nieuwenhuyse 6, 1160 Bruxelles, Belgium Springborn Smithers Laboratories (Europe) Study # 1142.002.103 2010- 04-12.

Springborn Smithers (2009b) Trichloro(phenyl)silane: Acute toxicity to water fleas (Daphnia magna) under static conditions. Springborn Smithers Laboratories (Europe) Seestrasse 21, Postfach CH-9326 Horn, Switzerland Reconsile Consortium Springborn Smithers Laboratories (Europe) Study # 1142.002.110.

Springborn Smithers (2009c) Trichloro(phenyl)silane: Growth inhibition test with Pseudokirchneriella subcapitata (syn. Selenastrum capricornutum) under static conditions Springborn Smithers Laboratories (Europe) Seestrasse 21, Postfach CH-9326 Horn, Switzerland. Reconsile Springborn Smithers Study # 1142.002.430.

Tomanek, A.  (1990)  Silicones and Industry.  Hauser, Munich.  pp. 20-23.

U.S. Environmental Protection Agency (U.S. EPA) (2012 or 2013) Estimation Programs Interface Suite,™. for Microsoft® Windows, v4.10. United States Environmental Protection Agency, Washington, DC, USA.

Weast R C, Melvin J A  (1981)  CRC Handbook of Chemistry and Physics, 62nd Edition, 1981-1982  CRC Press, Boca Raton, Florida.

The hydrolysis of TMPS was performed at 50 oC at pH 4, 7 and 9.  The chromatograms of TMPS in the buffer solution pH 4 and pH 9 showed no peak at relevant retention times; no further testing was performed at pH 4 and pH 9.