Hidden comment: IUCLID LAYOUT VERSION 1.1 - NOV 2002

SIDS Dossier

OECD HPV Chemical Programme, SIDS Dossier, approved at CoCAM 1 (10/10/2011)

| Existing Chemical | : | ID: 25360-10-5 |
| --- | --- | --- |
| CAS No. | : | 25360-10-5 |
| EINECS Name | : | 1,1-dimethylheptanethiol |
| EC No. | : | 246-896-9 |
| Molecular Formula | : | C9H20S |

Producer related part

| Company | : | Mercaptans/Thiols Council |
| --- | --- | --- |
| Creation date | : | 17.11.2006 |

Substance related part

| Company | : | Mercaptans/Thiols Council |
| --- | --- | --- |
| Creation date | : | 17.11.2006 |

| Status | : |  |
| --- | --- | --- |
| Memo | : |  |

| Printing date | : | 21.06.2012 |
| --- | --- | --- |
| Revision date | : |  |
| Date of last update | : | 20.06.2012 |

| Number of pages | : | 3 |
| --- | --- | --- |

| Chapter (profile) | : | Chapter: 1, 2, 3, 4, 5, 6, 7, 8, 10 |
| --- | --- | --- |
| Reliability (profile) | : | Reliability: without reliability, 1, 2, 3, 4 |
| Flags (profile) | : | Flags: without flag, confidential, non confidential, WGK (DE), TA-Luft (DE), Material Safety Dataset, Risk Assessment, Directive 67/548/EEC, SIDS |

1.0.1	APPLICANT AND COMPANY INFORMATION

| Type | : | manufacturer |
| --- | --- | --- |
| Name | : | ARKEMA France |
| Contact person | : |  |
| Date | : |  |
| Street | : | 420 rue d'Estienne d'Orves |
| Town | : | 92705 Colombes |
| Country | : | France |
| Phone | : | +33149008080 |
| Telefax | : |  |
| Telex | : |  |
| Cedex | : |  |
| Email | : |  |
| Homepage | : |  |

| 19.01.2010 |  |  |
| --- | --- | --- |
| Type | : | manufacturer |
| Name | : | ARKEMA Inc. |
| Contact person | : |  |
| Date | : |  |
| Street | : | 2000 Market Street |
| Town | : | 19103 Philadelphia, PA |
| Country | : | United States |
| Phone | : |  |
| Telefax | : |  |
| Telex | : |  |
| Cedex | : |  |
| Email | : |  |
| Homepage | : |  |

| 19.01.2010 |  |  |
| --- | --- | --- |
| Type | : | manufacturer |
| Name | : | Chevron Phillips Chemical Company LP |
| Contact person | : |  |
| Date | : |  |
| Street | : | 10001 Six Pines Dr, Suite 4103 |
| Town | : | 77380 Woodlands, TX |
| Country | : | United States |
| Phone | : |  |
| Telefax | : |  |
| Telex | : |  |
| Cedex | : |  |
| Email | : |  |
| Homepage | : |  |

| 19.01.2010 |  |
| --- | --- |

1.0.2	LOCATION OF PRODUCTION SITE, IMPORTER OR FORMULATOR

1.0.3	IDENTITY OF RECIPIENTS

1.0.4	DETAILS ON CATEGORY/TEMPLATE

1.1.0	SUBSTANCE IDENTIFICATION

| IUPAC Name | : | 1,1-Dimethylheptanthiol |
| --- | --- | --- |
| Smiles Code | : | SC(CCCCCC)(C)C |
| Molecular formula | : | C9 H20 S1 |
| Molecular weight | : | 160.3 |
| Petrol class | : |  |

| 19.01.2010 |  |
| --- | --- |

1.1.1	GENERAL SUBSTANCE INFORMATION

| Purity type | : | typical for marketed substance |
| --- | --- | --- |
| Substance type | : | organic |
| Physical status | : | liquid |
| Purity | : | > 97 % w/w |
| Colour | : |  |
| Odour | : |  |

| 19.01.2010 |  |
| --- | --- |

1.1.2	SPECTRA

1.2		SYNONYMS AND TRADENAMES

| 1,1-Dimethylheptanthiol |  |  |
| --- | --- | --- |

| 17.01.2003 |  |  |
| --- | --- | --- |
| Sulfole 90 (registered trademark) |  |  |

| 13.10.2003 |  |  |
| --- | --- | --- |
| t-Nonyl mercaptan |  |  |

| 20.06.2012 |  |  |
| --- | --- | --- |
| tert-Nonyl mercaptan |  |  |

| 17.01.2003 |  |  |
| --- | --- | --- |
| Tert-nonyl thiol |  |  |

| 17.01.2003 |  |
| --- | --- |

1.3		IMPURITIES

| Purity | : | typical for marketed substance |
| --- | --- | --- |
| CAS-No | : |  |
| EC-No | : |  |
| EINECS-Name | : |  |
| Molecular formula | : |  |
| Value | : |  |

| Result | : | Purity information is considered Confidential Business Information |
| --- | --- | --- |
| 22.09.2009 |  |  |

1.4		ADDITIVES

1.5		TOTAL QUANTITY

| Remark | : | In the United States, the estimated production of t-Nonyl Mercaptan in 2006 was 450 - 4500 metric tons based on data from the Inventory Update Rule (IUR). The total annual global production estimates for the C8-C12 Aliphatic Thiols from the Mercaptans/Thiols Council is approximately 100 million pounds (45,000 metric tons). |
| --- | --- | --- |
| 21.09.2009 |  |  |

1.6.1	LABELLING

1.6.2	CLASSIFICATION

1.6.3	PACKAGING

1.7		USE PATTERN

1.7.1	DETAILED USE PATTERN

| Remark | : | Thiols possess a sulfhydryl group (-SH) that is instrumental in introducing a sulfur group into various biologically active molecules in the pharmaceutical and agrochemical industries. In the pesticide industry, thiols are used in the production of thiocarbamates for herbicides and thiophosphates for insecticides (Labat, 1992). Thiols in the C8-C12 carbon range are commonly used as intermediates in the organic synthesis and as polymerization chain transfer agents. The following are typical uses of t-nonyl mercaptan: - Lubricant additive intermediate - Polymerization chain transfer agent |
| --- | --- | --- |
| 03.12.2009 |  |  |

1.7.2	METHODS OF MANUFACTURE

| Remark | : | t-Nonyl mercaptan is produced by the addition of hydrogen sulfide to propylene trimer under acid catalysis conditions, followed by distillation to remove light and heavy components. |
| --- | --- | --- |
| 20.06.2012 |  |  |

1.8		REGULATORY MEASURES

1.8.1	OCCUPATIONAL EXPOSURE LIMIT VALUES

| Type of limit | : | MAK (DE) |
| --- | --- | --- |
| Limit value | : | 20 mg/m3 |

| 21.09.2009 |  |
| --- | --- |

1.8.2	ACCEPTABLE RESIDUES LEVELS

1.8.3	WATER POLLUTION

1.8.4	MAJOR ACCIDENT HAZARDS

1.8.5	AIR POLLUTION

1.8.6	LISTINGS E.G. CHEMICAL INVENTORIES

1.9.1	DEGRADATION/TRANSFORMATION PRODUCTS

1.9.2	COMPONENTS

1.10	SOURCE OF EXPOSURE

| Remark | : | Thiols belong to a family of organic sulfur compounds. In general, these compounds exist naturally (water, plants, soil) and are required for survival of all higher organisms. Some bacterial systems have been shown to utilize thiols as their direct source of sulfur, specifically degrading the materials by selective cleavage of the C-S linkage (Frost, 1994). Other microorganisms are capable of releasing lower thiols from larger sulfur compounds. n-Octyl, t-Nonyl and t-Dodecyl Mercaptans are primarily used in closed systems and environmental exposure is expected to be very limited during manufacturing. Residual mercaptans in finished products are not expected because of the processes used and the need to remove the thiols due to odor. Due to very low odor thresholds, even low concentrations of the Aliphatic Thiols can be detected and, therefore, in an industrial setting as intermediate, exposure is expected to be low. |
| --- | --- | --- |
| 17.01.2012 |  |  |

1.11	ADDITIONAL REMARKS

1.12	LAST LITERATURE SEARCH

| Type of search | : | Internal and External |
| --- | --- | --- |
| Chapters covered | : | 3, 4, 5 |
| Date of search | : | 06.04.2009 |

| Remark | : | Databases searched 1. ATSDR TOX PROFILES: <http://www.atsdr.cdc.gov/toxpro2.html> 2. ACGIH documentation: <http://www.acgih.org/TLV/> 3. HSDB: <http://toxnet.nlm.nih.gov/cgi-bin/sis/htmlgen?HSDB> 4. Toxline: <http://toxnet.nlm.nih.gov/cgi-bin/sis/htmlgen?TOXLINE> 5. TOXNET: <http://toxnet.nlm.nih.gov/index.html> 6. EUROPEAN CHEMICALS BUREAU (IUCLID): <http://ecb.jrc.it/esis/> 7. Bingham, E, Cohrssen, B, and Powell, C.H. 2001. Patty's Toxicology. Vol.7, 5th edition. 8. Internal company files (Arkema, Chevron Phillips, OTHER) 9. Syracuse Research Database: <http://www.syrres.com/esc/physdemo.htm> |
| --- | --- | --- |
| 19.01.2010 |  |  |

1.13	REVIEWS

2.1		MELTING POINT

| Value | : | -43.2 °C |
| --- | --- | --- |
| Sublimation | : |  |
| Method | : | other: measured |
| Year | : |  |
| GLP | : |  |
| Test substance | : |  |

| Test substance | : | t-nonyl mercaptan; purity not noted |
| --- | --- | --- |
| Reliability | : | (2) valid with restrictions |
| Flag | : | Critical study for SIDS endpoint |
| 17.01.2012 | (1) |  |
| Value | : | = -21.4 °C |
| Sublimation | : |  |
| Method | : | other: EPIWIN (v 4.00) MPBPVP Submodel (v 1.43) |
| Year | : | 2011 |
| GLP | : |  |
| Test substance | : |  |

| Remark | : | The EPIWIN model was run using the following measured physical chemical properties: boiling point of 188°C and vapor pressure of 0.53 mmHg. |
| --- | --- | --- |
| Test substance | : | t-nonyl mercaptan |
| Reliability | : | (2) valid with restrictions |
| Flag | : | Critical study for SIDS endpoint |
| 20.06.2012 | (13) |  |

2.2		BOILING POINT

| Value | : | = 188 °C at 1013 hPa |
| --- | --- | --- |
| Decomposition | : |  |
| Method | : | other: measured |
| Year | : |  |
| GLP | : | no |
| Test substance | : |  |

| Test substance | : | t-nonyl mercaptan; purity not noted |
| --- | --- | --- |
| Reliability | : | (2) valid with restrictions |
| Flag | : | Critical study for SIDS endpoint |
| 17.01.2012 | (1) |  |
| Value | : | = 195.1 °C at |
| Decomposition | : |  |
| Method | : | other: EPIWIN (v 4.00) MPBPVP Submodel (v 1.43) |
| Year | : | 2011 |
| GLP | : |  |
| Test substance | : |  |

| Remark | : | The EPIWIN model was run using the following measured physical chemical properties: melting point of -43.2°C and vapor pressure of 0.53 mmHg. |
| --- | --- | --- |
| Test substance | : | t-nonyl mercaptan |
| Reliability | : | (2) valid with restrictions |
| Flag | : | Critical study for SIDS endpoint |
| 20.06.2012 | (13) |  |

2.3		DENSITY

| Type | : |  |
| --- | --- | --- |
| Value | : | .853 g/cm³ at 15.5 °C |

| Test substance | : | t-nonyl mercaptan; purity not noted |
| --- | --- | --- |
| Reliability | : | (2) valid with restrictions |
|  |  | Data from Handbook |
| Flag | : | Critical study for SIDS endpoint |
| 20.06.2012 | (2) |  |

2.3.1	GRANULOMETRY

2.4		VAPOUR PRESSURE

| Value | : | .71 hPa at 25 °C |
| --- | --- | --- |
| Decomposition | : |  |
| Method | : | other (measured) |
| Year | : |  |
| GLP | : | no |
| Test substance | : |  |

| Test substance | : | t-nonyl mercaptan; purity not noted |
| --- | --- | --- |
| Reliability | : | (2) valid with restrictions |
|  |  | Data from Handbook |
| Flag | : | Critical study for SIDS endpoint |
| 17.01.2012 | (2) |  |
| Value | : | = .87 hPa at 25 °C |
| Decomposition | : |  |
| Method | : | other (calculated): EPIWIN (v 4.00) MPBPVP Submodel (v 1.43) |
| Year | : | 2011 |
| GLP | : |  |
| Test substance | : |  |

| Remark | : | The EPIWIN model was run using the following measured physical chemical properties: melting point of -43.2°C and boiling point of 188°C. |
| --- | --- | --- |
| Test substance | : | t-nonyl mercaptan |
| Reliability | : | (2) valid with restrictions |
| Flag | : | Critical study for SIDS endpoint |
| 20.06.2012 | (13) |  |

2.5		PARTITION COEFFICIENT

| Partition coefficient | : |  |
| --- | --- | --- |
| Log pow | : | = 4.59 at 25 °C |
| pH value | : |  |
| Method | : | other (calculated): EPIWIN (v 4.00) KOWWIN Submodel (v 1.68) |
| Year | : | 2011 |
| GLP | : |  |
| Test substance | : |  |

| Remark | : | The EPIWIN model was run using the following measured physical chemical properties: melting point of -43.2°C, boiling point of 188°C and vapor pressure of 0.53 mmHg. |
| --- | --- | --- |
| Test substance | : | t-nonyl mercaptan |
| Reliability | : | (2) valid with restrictions |
| Flag | : | Critical study for SIDS endpoint |
| 20.06.2012 | (13) |  |

2.6.1	SOLUBILITY IN DIFFERENT MEDIA

| Solubility in |  | : |  |
| --- | --- | --- | --- |
| Value |  | : | = 9.6 mg/l at 25 °C |
| pH | value | : |  |
|  | concentration | : | at °C |
| Temperature effects |  | : |  |
| Examine different pol. |  | : |  |
| pKa |  | : | at 25 °C |
| Description |  | : |  |
| Stable |  | : |  |
| Deg. product |  | : |  |
| Method |  | : | other: EPIWIN (v 4.00) WSKOW Submodel (v 1.42) |
| Year |  | : | 2011 |
| GLP |  | : |  |
| Test substance |  | : |  |

| Remark | : | The EPIWIN model was run using the following measured physical chemical properties: melting point of -43.2°C, boiling point of 188°C, and vapor pressure of 0.53 mmHg. |
| --- | --- | --- |
| Test substance | : | t-nonyl mercaptan |
| Reliability | : | (2) valid with restrictions |
| Flag | : | Critical study for SIDS endpoint |
| 20.06.2012 | (13) |  |

2.6.2	SURFACE TENSION

2.7		FLASH POINT

| Value | : | 63.8 °C |
| --- | --- | --- |
| Type | : |  |
| Method | : |  |
| Year | : |  |
| GLP | : | no |
| Test substance | : |  |

| Result | : | Flash point = 147 degree F |
| --- | --- | --- |
| Test substance | : | t-nonyl mercaptan; purity not noted |
| Reliability | : | (2) valid with restrictions |
|  |  | Data from Handbook |
| 19.01.2010 | (2) |  |

2.8		AUTO FLAMMABILITY

2.9		FLAMMABILITY

2.10	EXPLOSIVE PROPERTIES

2.11	OXIDIZING PROPERTIES

2.12	DISSOCIATION CONSTANT

2.13	VISCOSITY

2.14	ADDITIONAL REMARKS

3.1.1	PHOTODEGRADATION

| Type | : | air |
| --- | --- | --- |
| Light source | : |  |
| Light spectrum | : | nm |
| Relative intensity | : | based on intensity of sunlight |
| INDIRECT PHOTOLYSIS |  |  |
| Sensitizer | : | OH |
| Conc. of sensitizer | : | 1500000 molecule/cm³ |
| Rate constant | : | .0000000000398 cm³/(molecule*sec) |
| Degradation | : | 50 % after 3.2 hour(s) |
| Deg. product | : |  |
| Method | : | other (calculated) |
| Year | : |  |
| GLP | : |  |
| Test substance | : |  |

| Remark | : | Physical Property Inputs: Log Kow (octanol-water): ------ Boiling Point (deg C) : 188.00 Melting Point (deg C) : -43.15 Vapor Pressure (mm Hg) : 0.53 Water Solubility (mg/L): 9.6 Henry LC (atm-m3/mole) : ------ AOP Program (v1.92) Results: =========================== SMILES : SC(CCCCCC)(C)C CHEM : tert-Nonanethiol MOL FOR: C9 H20 S1 MOL WT : 160.32 ------------------- SUMMARY (AOP v1.92): HYDROXYL RADICALS (25 deg C) -------- Hydrogen Abstraction = 7.3029 E-12 cm3/molecule-sec Reaction with N, S and -OH = 32.5000 E-12 cm3/molecule-sec Addition to Triple Bonds = 0.0000 E-12 cm3/molecule-sec Addition to Olefinic Bonds = 0.0000 E-12 cm3/molecule-sec Addition to Aromatic Rings = 0.0000 E-12 cm3/molecule-sec Addition to Fused Rings = 0.0000 E-12 cm3/molecule-sec OVERALL OH Rate Constant = 39.8029 E-12 cm3/molecule-sec HALF-LIFE = 0.269 Days (12-hr day; 1.5E6 OH/cm3) HALF-LIFE = 3.225 Hrs |
| --- | --- | --- |
| Test substance | : | t-nonyl mercaptan |
| Conclusion | : | T1/2 = 3.2 Hrs; Overall OH Rate Constant (cm3/molecule-sec) =4.0 E-11 |
| Reliability | : | (2) valid with restrictions |
|  |  | Based on an acceptable computer model |
| Flag | : | Critical study for SIDS endpoint |
| 17.01.2012 | (13) |  |

3.1.2	STABILITY IN WATER

3.1.3	STABILITY IN SOIL

3.2.1	MONITORING DATA

3.2.2	FIELD STUDIES

3.3.1	TRANSPORT BETWEEN ENVIRONMENTAL COMPARTMENTS

| Remark | : | Henry's Law Constant: Henrys Law Constant (25 deg C) [HENRYWIN v3.20]: Bond Est : 2.52E-002 atm-m3/mole (2.55E+003 Pa-m3/mole) Group Est: Incomplete For Henry LC Comparison Purposes: Exper Database: none available User-Entered Henry LC: not entered Henrys LC [via VP/WSol estimate using User-Entered or Estimated values]: HLC: 1.165E-002 atm-m3/mole (1.180E+003 Pa-m3/mole) VP: 0.53 mm Hg (source: User-Entered) WS: 9.6 mg/L (source: User-Entered) |
| --- | --- | --- |
| Test substance | : | t-nonyl mercaptan |
| Conclusion | : | Henry's Law Constant = 2.52E-02 atm-m3/mole |
| Reliability | : | (2) valid with restrictions |
|  |  | Based on an acceptable computer model |
| Flag | : | Critical study for SIDS endpoint |
| 18.01.2012 | (13) |  |

3.3.2	DISTRIBUTION

| Media | : | other: air - water - soil - sediment |
| --- | --- | --- |
| Method | : | Calculation according Mackay, Level III |
| Year | : | 2011 |

| Remark | : | SMILES : SC(CCCCCC)(C)C CHEM : tert-Nonanethiol MOL FOR: C9 H20 S1 MOL WT : 160.32 ------------------------------ EPI SUMMARY (v4.00) -------------------------- Physical Property Inputs: Log Kow (octanol-water): ------ Boiling Point (deg C) : 188.00 Melting Point (deg C) : -43.15 Vapor Pressure (mm Hg) : 0.53 Water Solubility (mg/L): 9.6 Henry LC (atm-m3/mole) : ------ Level III Fugacity Model (Full-Output): ======================================= Chem Name : tert-Nonanethiol Molecular Wt: 160.32 Henry's LC : 0.0116 atm-m3/mole (calc VP/Wsol) Vapor Press : 0.53 mm Hg (user-entered) Log Kow : 4.59 (Kowwin program) Soil Koc : 949 (KOCWIN MCI method) Mass Amount Half-Life Emissions (percent) (hr) (kg/hr) Air 3.35 6.45 1000 Water 34.5 360 1000 Soil 61 720 1000 Sediment 1.16 3.24e+003 0 Fugacity Reaction Advection Reaction Advection (atm) (kg/hr) (kg/hr) (percent) (percent) Air 2.77e-011 1.95e+003 182 65.1 6.06 Water 6.76e-008 360 187 12 6.23 Soil 5.76e-008 318 0 10.6 0 Sediment 4.81e-008 1.35 0.126 0.0449 0.0042 Persistence Time: 181 hr Reaction Time: 206 hr Advection Time: 1.47e+003 hr Percent Reacted: 87.7 Percent Advected: 12.3 Half-Lives (hr), (based upon Biowin (Ultimate) and Aopwin): Air: 6.449 Water: 360 Soil: 720 Sediment: 3240 Biowin estimate: 2.931 (weeks ) Advection Times (hr): Air: 100 Water: 1000 Sediment: 5e+004 |
| --- | --- | --- |
| Test substance | : | t-nonyl mercaptan |
| Conclusion | : | t-nonyl mercaptan Mass Amount (percent) Air 3.35 Water 34.5 Soil 61 Sediment 1.16 |
| Reliability | : | (2) valid with restrictions |
|  |  | Based on an acceptable computer model |
| Flag | : | Critical study for SIDS endpoint |
| 17.01.2012 | (13) |  |
| Media | : | other: air |
| Method | : | Calculation according Mackay, Level III |
| Year | : | 2011 |

| Remark | : | SMILES : SC(CCCCCC)(C)C CHEM : tert-Nonanethiol MOL FOR: C9 H20 S1 MOL WT : 160.32 ------------------------------ EPI SUMMARY (v4.00) -------------------------- Physical Property Inputs: Log Kow (octanol-water): ------ Boiling Point (deg C) : 188.00 Melting Point (deg C) : -43.15 Vapor Pressure (mm Hg) : 0.53 Water Solubility (mg/L): 9.6 Henry LC (atm-m3/mole) : ------ Level III Fugacity Model (Full-Output): ======================================= Chem Name : tert-Nonanethiol Molecular Wt: 160.32 Henry's LC : 0.0116 atm-m3/mole (calc VP/Wsol) Vapor Press : 0.53 mm Hg (user-entered) Log Kow : 4.59 (Kowwin program) Soil Koc : 949 (KOCWIN MCI method) Mass Amount Half-Life Emissions (percent) (hr) (kg/hr) Air 99.2 6.45 1000 Water 0.191 360 0 Soil 0.596 720 0 Sediment 0.00645 3.24e+003 0 Fugacity Reaction Advection Reaction Advection (atm) (kg/hr) (kg/hr) (percent) (percent) Air 1.3e-011 915 85.1 91.5 8.51 Water 5.94e-012 0.0316 0.0164 0.00316 0.00164 Soil 8.91e-012 0.0493 0 0.00493 0 Sediment 4.23e-012 0.000118 1.11e-005 1.18e-005 1.11e-006 Persistence Time: 8.58 hr Reaction Time: 9.38 hr Advection Time: 101 hr Percent Reacted: 91.5 Percent Advected: 8.51 Half-Lives (hr), (based upon Biowin (Ultimate) and Aopwin): Air: 6.449 Water: 360 Soil: 720 Sediment: 3240 Biowin estimate: 2.931 (weeks ) Advection Times (hr): Air: 100 Water: 1000 Sediment: 5e+004 |
| --- | --- | --- |
| Test substance | : | t-nonyl mercaptan |
| Conclusion | : | t-nonyl mercaptan Mass Amount (percent) Air 99.2 Water 0.191 Soil 0.596 Sediment 0.00645 |
| Reliability | : | (2) valid with restrictions |
|  |  | Based on an acceptable computer model |
| Flag | : | Critical study for SIDS endpoint |
| 17.01.2012 | (13) |  |
| Media | : | other: water |
| Method | : | Calculation according Mackay, Level III |
| Year | : | 2011 |

| Remark | : | SMILES : SC(CCCCCC)(C)C CHEM : tert-Nonanethiol MOL FOR: C9 H20 S1 MOL WT : 160.32 ------------------------------ EPI SUMMARY (v4.00) -------------------------- Physical Property Inputs: Log Kow (octanol-water): ------ Boiling Point (deg C) : 188.00 Melting Point (deg C) : -43.15 Vapor Pressure (mm Hg) : 0.53 Water Solubility (mg/L): 9.6 Henry LC (atm-m3/mole) : ------ Level III Fugacity Model (Full-Output): ======================================= Chem Name : tert-Nonanethiol Molecular Wt: 160.32 Henry's LC : 0.0116 atm-m3/mole (calc VP/Wsol) Vapor Press : 0.53 mm Hg (user-entered) Log Kow : 4.59 (Kowwin program) Soil Koc : 949 (KOCWIN MCI method) Mass Amount Half-Life Emissions (percent) (hr) (kg/hr) Air 1.97 6.45 0 Water 94.8 360 1000 Soil 0.0118 720 0 Sediment 3.2 3.24e+003 0 Fugacity Reaction Advection Reaction Advection (atm) (kg/hr) (kg/hr) (percent) (percent) Air 5.89e-012 415 38.7 41.5 3.87 Water 6.74e-008 358 186 35.8 18.6 Soil 4.05e-012 0.0224 0 0.00224 0 Sediment 4.79e-008 1.34 0.125 0.134 0.0125 Persistence Time: 196 hr Reaction Time: 253 hr Advection Time: 873 hr Percent Reacted: 77.5 Percent Advected: 22.5 Half-Lives (hr), (based upon Biowin (Ultimate) and Aopwin): Air: 6.449 Water: 360 Soil: 720 Sediment: 3240 Biowin estimate: 2.931 (weeks ) Advection Times (hr): Air: 100 Water: 1000 Sediment: 5e+004 |
| --- | --- | --- |
| Test substance | : | t-nonyl mercaptan |
| Conclusion | : | t-nonyl mercaptan Mass Amount (percent) Air 1.97 Water 94.8 Soil 0.0118 Sediment 3.2 |
| Reliability | : | (2) valid with restrictions |
|  |  | Based on an acceptable computer model |
| Flag | : | Critical study for SIDS endpoint |
| 17.01.2012 | (13) |  |
| Media | : | other: soil |
| Method | : | Calculation according Mackay, Level III |
| Year | : | 2011 |

| Remark | : | SMILES : SC(CCCCCC)(C)C CHEM : tert-Nonanethiol MOL FOR: C9 H20 S1 MOL WT : 160.32 ------------------------------ EPI SUMMARY (v4.00) -------------------------- Physical Property Inputs: Log Kow (octanol-water): ------ Boiling Point (deg C) : 188.00 Melting Point (deg C) : -43.15 Vapor Pressure (mm Hg) : 0.53 Water Solubility (mg/L): 9.6 Henry LC (atm-m3/mole) : ------ Level III Fugacity Model (Full-Output): ======================================= Chem Name : tert-Nonanethiol Molecular Wt: 160.32 Henry's LC : 0.0116 atm-m3/mole (calc VP/Wsol) Vapor Press : 0.53 mm Hg (user-entered) Log Kow : 4.59 (Kowwin program) Soil Koc : 949 (KOCWIN MCI method) Mass Amount Half-Life Emissions (percent) (hr) (kg/hr) Air 1.72 6.45 0 Water 0.202 360 0 Soil 98.1 720 1000 Sediment 0.00681 3.24e+003 0 Fugacity Reaction Advection Reaction Advection (atm) (kg/hr) (kg/hr) (percent) (percent) Air 8.82e-012 622 57.9 62.2 5.79 Water 2.47e-010 1.31 0.681 0.131 0.0681 Soil 5.76e-008 318 0 31.8 0 Sediment 1.75e-010 0.00491 0.000459 0.000491 4.59e-005 Persistence Time: 337 hr Reaction Time: 358 hr Advection Time: 5.76e+003 hr Percent Reacted: 94.1 Percent Advected: 5.86 Half-Lives (hr), (based upon Biowin (Ultimate) and Aopwin): Air: 6.449 Water: 360 Soil: 720 Sediment: 3240 Biowin estimate: 2.931 (weeks) Advection Times (hr): Air: 100 Water: 1000 Sediment: 5e+004 |
| --- | --- | --- |
| Test substance | : | t-nonyl mercaptan |
| Conclusion | : | t-nonyl mercaptan Mass Amount (percent) Air 1.72 Water 0.202 Soil 98.1 Sediment 0.00681 |
| Reliability | : | (2) valid with restrictions |
|  |  | Based on an acceptable computer model |
| Flag | : | Critical study for SIDS endpoint |
| 17.01.2012 | (13) |  |

3.4		MODE OF DEGRADATION IN ACTUAL USE

3.5		BIODEGRADATION

| Type | : | aerobic |  |
| --- | --- | --- | --- |
| Inoculum | : | other: sewage |  |
| Concentration | : | 20 mg/l related to |  |
|  |  | related to |  |
| Contact time | : | 28 day(s) |  |
| Degradation | : | = 0 (±) % after 28 day(s) |  |
| Result | : | other: (note readily biodegradable) |  |
| Control substance | : | Aniline |  |
| Kinetic | : | % |
|  |  | % |
| Deg. product | : |  |  |
| Method | : | OECD Guide-line 301 F "Ready Biodegradability: Manometric Respirometry Test" |  |
| Year | : | 1992 |  |
| GLP | : | yes |  |
| Test substance | : |  |  |

| Remark | : | The test item, at a concentration of 20 mg/L, was exposed to sewage treatment micro-organisms with culture medium in sealed culture vessels in diffuse light at 21 ± 1ºC for 28 days. The test item was dissolved in silicone prior to dispersion in the test medium to aid dispersion of the test item in the test medium and to increase the surface area of the test item exposed to the test organisms. The degradation of the test item was assessed by the measurement of daily oxygen consumption values and Dissolved Organic Carbon (DOC) analysis on Days 0 to 28. Control solutions with inoculum and the reference item, aniline, together with a toxicity control were used for validation purposes. As biodegradation progresses, the micro-organisms convert oxygen to carbon dioxide, which is absorbed into the ethanolamine solution causing a net reduction in gas pressure within the sample flask. The pressure reduction triggers the electrolytic process, generating oxygen and restoring the pressure in the sample flask. The magnitude of the electrolyzing current and the duration of the current is proportional to the amount of oxygen supplied to the micro-organisms. |  |
| --- | --- | --- | --- |
| Result | : | The test item attained 0% degradation after 28 days, calculated from the oxygen consumption values, and therefore cannot be considered to be readily biodegradable under the terms and conditions of OECD TG 301F. The toxicity control (aniline and test item) attained 58% degradation after 14 days and 62% degradation after 28 days thereby confirming that the test item was not toxic to the sewage treatment micro-organisms used in the test. Aniline attained 67% degradation after 14 days and 71% degradation after 28 days thereby confirming the suitability of the inoculum and test conditions. The mean BOD (Biological Oxygen Demand) of the inoculated culture medium (control) was 26.69 mg O2/L after 28 days and therefore satisfied the validation criterion given in the OECD TG 301F. |  |
| Test substance | : | t-nonyl mercaptan; 99.29% purity |  |
| Conclusion | : | The test item attained 0% degradation after 28 days and therefore cannot be considered to be readily biodegradable under the terms and conditions of OECD TG 301F. |  |
| Reliability | : | (1) valid without restriction |  |
| Flag | : | Critical study for SIDS endpoint |  |
| 17.01.2012 | (6) |  |  |
| Type | : | aerobic |  |
| Inoculum | : | other: sewage |  |
| Concentration | : | 50 mg/l related to |  |
|  |  | related to |  |
| Contact time | : | 29 day(s) |  |
| Degradation | : | = 0 (±) % after 29 day(s) |  |
| Result | : | other: (not readily biodegradable) |  |
| Control substance | : | Aniline |  |
| Kinetic | : | % |
|  |  | % |
| Deg. product | : |  |  |
| Method | : | OECD Guide-line 301 F "Ready Biodegradability: Manometric Respirometry Test" |  |
| Year | : | 1992 |  |
| GLP | : | yes |  |
| Test substance | : |  |  |

| Remark | : | Initially, the test item at a concentration of 50 mg/L was exposed to sewage treatment micro-organisms with culture medium in sealed culture vessels in diffuse light at 21 ± 1ºC for 28 days. The test item was dissolved in silicone prior to dispersion in the test medium to aid dispersion of the test item in the test medium and to increase the surface area of the test item exposed to the test organisms. The degradation of the test item was assessed by the measurement of daily oxygen consumption values on Days 0 to 29. At the end of the initial test, a re-inoculation test was performed at a test concentration of 20 mg/L in sealed culture vessels for 28 days. The inoculum used in the re-inoculation test was taken from the test item vessels from the initial test. The degradation of the test item was assessed by the measurement of daily oxygen consumption values on Days 0 to 28. |
| --- | --- | --- |
| Result | : | Initial test The test item attained 0% degradation after 29 days, calculated from the oxygen consumption values, and therefore cannot be considered to be readily biodegradable under the terms and conditions of OECD TG 301F. The toxicity control attained 45% degradation after 14 days and 54% degradation after 29 days thereby confirming that the test item was not toxic to the sewage treatment micro-organism used in the test. Aniline attained 72% degradation after 14 days and 29 days thereby confirming the suitability of the inoculums and test conditions. The mean BOD of the inoculated culture medium (control) in the initial test was 38.00 mg O2/L after 28 days. Re-inoculation test The test item attained 0% degradation after 28 days. The toxicity control attained 46% degradation after 14 days and 74% degradation after 28 days thereby confirming that the test item was not toxic to the sewage treatment micro-organism used in the test. Aniline attained 64% degradation after 14 days and 72% after 28 days thereby confirming the suitability of the inoculums and test conditions. |
| Test substance | : | t-nonyl mercaptan; 99.29% purity |
| Conclusion | : | The test item at a concentration of 50 mg/L showed no degradation after 29 days in the initial test and therefore cannot be considered to be readily biodegradable under the OECD TG 301F. In the re-inoculation test, the test item, at a concentration of 20 mg/L, also showed no degradation after 28 days. |
| Reliability | : | (1) valid without restriction |
| Flag | : | Critical study for SIDS endpoint |
| 18.01.2012 | (5) |  |

3.6		BOD5, COD OR BOD5/COD RATIO

3.7		BIOACCUMULATION

| Remark | : | BCFBAF Program (v3.01) Results: ============================== SMILES : SC(CCCCCC)(C)C CHEM : tert-Nonanethiol MOL FOR: C9 H20 S1 MOL WT : 160.32 --------------------------------- BCFBAF v3.01 -------------------------------- Summary Results: Log BCF (regression-based estimate): 2.70 (BCF = 499 L/kg wet-wt) Biotransformation Half-Life (days) : 1.14 (normalized to 10 g fish) Log BAF (Arnot-Gobas upper trophic): 2.63 (BAF = 428 L/kg wet-wt) Log Kow (experimental): not available from database Log Kow used by BCF estimates: 4.59 Equation Used to Make BCF estimate: Log BCF = 0.6598 log Kow - 0.333 + Correction Correction(s): Value No Applicable Correction Factors Estimated Log BCF = 2.698 (BCF = 498.9 L/kg wet-wt) =========================================================== Whole Body Primary Biotransformation Rate Estimate for Fish: =========================================================== ------+-----+--------------------------------------------+---------+--------- TYPE ¦ NUM ¦ LOG BIOTRANSFORMATION FRAGMENT DESCRIPTION ¦ COEFF ¦ VALUE ------+-----+--------------------------------------------+---------+--------- Frag ¦ 1 ¦ Linear C4 terminal chain [CCC-CH3] ¦ 0.0341 ¦ 0.0341 Frag ¦ 1 ¦ Carbon with 4 single bonds & no hydrogens ¦ -0.2984 ¦ -0.2984 Frag ¦ 3 ¦ Methyl [-CH3] ¦ 0.2451 ¦ 0.7353 Frag ¦ 5 ¦ -CH2- [linear] ¦ 0.0242 ¦ 0.1209 L Kow¦ * ¦ Log Kow = 4.59 (KowWin estimate) ¦ 0.3073 ¦ 1.4119 MolWt¦ * ¦ Molecular Weight Parameter ¦ ¦ -0.4111 Const¦ * ¦ Equation Constant ¦ ¦ -1.5058 ============+============================================+=========+========= RESULT ¦ LOG Bio Half-Life (days) ¦ ¦ 0.0557 RESULT ¦ Bio Half-Life (days) ¦ ¦ 1.137 NOTE ¦ Bio Half-Life Normalized to 10 g fish at 15 deg C ¦ ============+============================================+=========+========= Biotransformation Rate Constant: kM (Rate Constant): 0.6098 /day (10 gram fish) kM (Rate Constant): 0.3429 /day (100 gram fish) kM (Rate Constant): 0.1928 /day (1 kg fish) kM (Rate Constant): 0.1084 /day (10 kg fish) Arnot-Gobas BCF & BAF Methods (including biotransformation rate estimates): Estimated Log BCF (upper trophic) = 2.630 (BCF = 426.6 L/kg wet-wt) Estimated Log BAF (upper trophic) = 2.631 (BAF = 427.7 L/kg wet-wt) Estimated Log BCF (mid trophic) = 2.721 (BCF = 525.6 L/kg wet-wt) Estimated Log BAF (mid trophic) = 2.733 (BAF = 541.2 L/kg wet-wt) Estimated Log BCF (lower trophic) = 2.741 (BCF = 551.2 L/kg wet-wt) Estimated Log BAF (lower trophic) = 2.791 (BAF = 617.9 L/kg wet-wt) Arnot-Gobas BCF & BAF Methods (assuming a biotransformation rate of zero): Estimated Log BCF (upper trophic) = 3.567 (BCF = 3688 L/kg wet-wt) Estimated Log BAF (upper trophic) = 4.345 (BAF = 2.211e+004 L/kg wet-wt) |
| --- | --- | --- |
| Test substance | : | t-nonyl mercaptan |
| Conclusion | : | BCF = 499 L/kg wet-wt |
| Reliability | : | (2) valid with restrictions |
|  |  | Based on an acceptable computer model |
| Flag | : | Critical study for SIDS endpoint |
| 17.01.2012 | (13) |  |

3.8		ADDITIONAL REMARKS

4.1		ACUTE/PROLONGED TOXICITY TO FISH

| Type | : | other: Acute |
| --- | --- | --- |
| Species | : |  |
| Exposure period | : | 96 hour(s) |
| Unit | : | mg/l |
| LC50 | : | = .1 |
| Method | : | other: ECOSAR (v. 1.00) |
| Year | : | 2011 |
| GLP | : |  |
| Test substance | : |  |

| Method | : | ECOSAR Program (v1.00) Input: ============================== SMILES : SC(CCCCCC)(C)C CHEM : tert-Nonanethiol CAS Num: ChemID1: ChemID2: ChemID3: MOL FOR: C9 H20 S1 MOL WT : 160.32 Log Kow: 4.59 (KowWin estimate) Melt Pt: -43.15 deg C Wat Sol: 9.6 mg/L (measured) |
| --- | --- | --- |
| Result | : | ECOSAR v1.00 Class(es) Found ------------------------------ Thiols and Mercaptans Predicted ECOSAR Class Organism Duration End Pt mg/L (ppm) ================ ======== ========= ====== ======== Thiols and Mercaptans: Fish 96-hr LC50 0.101 Thiols and Mercaptans: Daphnid 48-hr LC50 0.076 Thiols and Mercaptans: Green Algae 96-hr EC50 0.056 Thiols and Mercaptans: Fish ChV 0.011 ! Thiols and Mercaptans: Daphnid ChV 0.008 ! Thiols and Mercaptans: Green Algae ChV 0.051 =============== ======== ========= ====== ======== Neutral Organic SAR: Fish 96-hr LC50 0.788 (Baseline Toxicity) : Daphnid 48-hr LC50 0.622 Green Algae 96-hr EC50 0.788 Fish ChV 0.071 Daphnid ChV 0.091 Green Algae ChV 0.452 Note: * = asterisk designates: Chemical may not be soluble enough to measure this predicted effect. Note: ! = exclamation designates: The toxicity value was determined from a predicted SAR using established acute-to-chronic ratios and ECOSAR regression techniques which are documented in the supporting Technical Reference Manual. When possible, this toxicity value should be considered in a weight of evidence approach. Thiols and Mercaptans: --------------------- For All Acute and Chronic Toxicity Values: If the log Kow of the chemical is greater than the value listed below, or if the compound is solid and the LC50, EC50 or ChV exceeds the water solubility by 10X, no effects at saturation are predicted for these endpoints. ECOSAR v1.00 SAR Limitations: Maximum LogKow: 6.5 (Fish 96-hr LC50) Maximum LogKow: 5.0 (Daphnid LC50) Maximum LogKow: 6.4 (Green Algae EC50 Maximum LogKow: 8.0 (ChV) Maximum Mol Wt: 1000 Baseline Toxicity SAR Limitations: --------------------------------- Maximum LogKow: 5.0 (Fish 96-hr LC50; Daphnid LC50) Maximum LogKow: 6.4 (Green Algae EC50) Maximum LogKow: 8.0 (ChV) Maximum Mol Wt: 1000 |
| Test substance | : | t-nonyl mercaptan |
| Reliability | : | (2) valid with restrictions |
|  |  | Based on an acceptable computer model |
| Flag | : | Critical study for SIDS endpoint |
| 18.01.2012 | (13) |  |

4.2		ACUTE TOXICITY TO AQUATIC INVERTEBRATES

| Type | : | other: Acute |
| --- | --- | --- |
| Species | : |  |
| Exposure period | : | 48 hour(s) |
| Unit | : | mg/l |
| EC50 | : | = .08 |
| Method | : | other: ECOSAR (v. 1.00) |
| Year | : | 2011 |
| GLP | : |  |
| Test substance | : |  |

| Method | : | ECOSAR Program (v1.00) Input: ============================== SMILES : SC(CCCCCC)(C)C CHEM : tert-Nonanethiol CAS Num: ChemID1: ChemID2: ChemID3: MOL FOR: C9 H20 S1 MOL WT : 160.32 Log Kow: 4.59 (KowWin estimate) Melt Pt: -43.15 deg C Wat Sol: 9.6 mg/L (measured) |
| --- | --- | --- |
| Result | : | ECOSAR v1.00 Class(es) Found ------------------------------ Thiols and Mercaptans Predicted ECOSAR Class Organism Duration End Pt mg/L (ppm) Thiols and Mercaptans Daphnid 48-hr LC50 0.076 ECOSAR v1.00 Class(es) Found ------------------------------ Thiols and Mercaptans Predicted ECOSAR Class Organism Duration End Pt mg/L (ppm) =============== ========== ======= ====== ======== Thiols and Mercaptans: Fish 96-hr LC50 0.101 Thiols and Mercaptans: Daphnid 48-hr LC50 0.076 Thiols and Mercaptans: Green Algae 96-hr EC50 0.056 Thiols and Mercaptans: Fish ChV 0.011 ! Thiols and Mercaptans: Daphnid ChV 0.008 ! Thiols and Mercaptans: Green Algae ChV 0.051 =============== ========== ======= ====== ======== Neutral Organic SAR: Fish 96-hr LC50 0.788 (Baseline Toxicity) : Daphnid 48-hr LC50 0.622 Green Algae 96-hr EC50 0.788 Fish ChV 0.071 Daphnid ChV 0.091 Green Algae ChV 0.452 Note: * = asterisk designates: Chemical may not be soluble enough to measure this predicted effect. Note: ! = exclamation designates: The toxicity value was determined from a predicted SAR using established acute-to-chronic ratios and ECOSAR regression techniques which are documented in the supporting Technical Reference Manual. When possible, this toxicity value should be considered in a weight of evidence approach. Thiols and Mercaptans: --------------------- For All Acute and Chronic Toxicity Values: If the log Kow of the chemical is greater than the value listed below, or if the compound is solid and the LC50, EC50 or ChV exceeds the water solubility by 10X, no effects at saturation are predicted for these endpoints. ECOSAR v1.00 SAR Limitations: Maximum LogKow: 6.5 (Fish 96-hr LC50) Maximum LogKow: 5.0 (Daphnid LC50) Maximum LogKow: 6.4 (Green Algae EC50 Maximum LogKow: 8.0 (ChV) Maximum Mol Wt: 1000 Baseline Toxicity SAR Limitations: --------------------------------- Maximum LogKow: 5.0 (Fish 96-hr LC50; Daphnid LC50) Maximum LogKow: 6.4 (Green Algae EC50) Maximum LogKow: 8.0 (ChV) Maximum Mol Wt: 1000 |
| Test substance | : | t-nonyl mercaptan |
| Reliability | : | (2) valid with restrictions |
|  |  | Based on an acceptable computer model |
| Flag | : | Critical study for SIDS endpoint |
| 18.01.2012 | (13) |  |

4.3		TOXICITY TO AQUATIC PLANTS E.G. ALGAE

| Species | : | other algae: Green algae |
| --- | --- | --- |
| Endpoint | : |  |
| Exposure period | : | 96 hour(s) |
| Unit | : | mg/l |
| EC50 | : | = .06 |
| Method | : | other: ECOSAR (v. 1.00) |
| Year | : | 2011 |
| GLP | : |  |
| Test substance | : |  |

| Method | : | ECOSAR Program (v1.00) Input: ============================== SMILES : SC(CCCCCC)(C)C CHEM : tert-Nonanethiol CAS Num: ChemID1: ChemID2: ChemID3: MOL FOR: C9 H20 S1 MOL WT : 160.32 Log Kow: 4.59 (KowWin estimate) Melt Pt: -43.15 deg C Wat Sol: 9.6 mg/L (measured) |
| --- | --- | --- |
| Result | : | ECOSAR v1.00 Class(es) Found ------------------------------ Thiols and Mercaptans Predicted ECOSAR Class Organism Duration End Pt mg/L (ppm) =============== ========== ======= ====== ======== Thiols and Mercaptans: Fish 96-hr LC50 0.101 Thiols and Mercaptans: Daphnid 48-hr LC50 0.076 Thiols and Mercaptans: Green Algae 96-hr EC50 0.056 Thiols and Mercaptans: Fish ChV 0.011 ! Thiols and Mercaptans: Daphnid ChV 0.008 ! Thiols and Mercaptans: Green Algae ChV 0.051 =============== ========== ======= ====== ======== Neutral Organic SAR: Fish 96-hr LC50 0.788 (Baseline Toxicity) : Daphnid 48-hr LC50 0.622 Green Algae 96-hr EC50 0.788 Fish ChV 0.071 Daphnid ChV 0.091 Green Algae ChV 0.452 Note: * = asterisk designates: Chemical may not be soluble enough to measure this predicted effect. Note: ! = exclamation designates: The toxicity value was determined from a predicted SAR using established acute-to-chronic ratios and ECOSAR regression techniques which are documented in the supporting Technical Reference Manual. When possible, this toxicity value should be considered in a weight of evidence approach. Thiols and Mercaptans: --------------------- For All Acute and Chronic Toxicity Values: If the log Kow of the chemical is greater than the value listed below, or if the compound is solid and the LC50, EC50 or ChV exceeds the water solubility by 10X, no effects at saturation are predicted for these endpoints. ECOSAR v1.00 SAR Limitations: Maximum LogKow: 6.5 (Fish 96-hr LC50) Maximum LogKow: 5.0 (Daphnid LC50) Maximum LogKow: 6.4 (Green Algae EC50 Maximum LogKow: 8.0 (ChV) Maximum Mol Wt: 1000 Baseline Toxicity SAR Limitations: --------------------------------- Maximum LogKow: 5.0 (Fish 96-hr LC50; Daphnid LC50) Maximum LogKow: 6.4 (Green Algae EC50) Maximum LogKow: 8.0 (ChV) Maximum Mol Wt: 1000 |
| Test substance | : | t-nonyl mercaptan |
| Reliability | : | (2) valid with restrictions |
|  |  | Based on an acceptable computer model |
| Flag | : | Critical study for SIDS endpoint |
| 18.01.2012 | (13) |  |

4.4		TOXICITY TO MICROORGANISMS E.G. BACTERIA

4.5.1	CHRONIC TOXICITY TO FISH

4.5.2	CHRONIC TOXICITY TO AQUATIC INVERTEBRATES

4.6.1	TOXICITY TO SEDIMENT DWELLING ORGANISMS

4.6.2	TOXICITY TO TERRESTRIAL PLANTS

4.6.3	TOXICITY TO SOIL DWELLING ORGANISMS

4.6.4	TOX. TO OTHER NON MAMM. TERR. SPECIES

4.7		BIOLOGICAL EFFECTS MONITORING

4.8		BIOTRANSFORMATION AND KINETICS

4.9		ADDITIONAL REMARKS

5.0		TOXICOKINETICS, METABOLISM AND DISTRIBUTION

5.1.1	ACUTE ORAL TOXICITY

| Type | : | LD50 |
| --- | --- | --- |
| Value | : | = 5550 mg/kg bw |
| Species | : | rat |
| Strain | : | other:WBS/W |
| Sex | : | male |
| Number of animals | : | 20 |
| Vehicle | : | other: none |
| Doses | : | 3536, 5000, 7071 or 10000 mg/kg body weight |
| Method | : | other |
| Year | : |  |
| GLP | : | no |
| Test substance | : |  |

| Result | : | LD50 = 5550 mg/kg bw. At 5000 mg/kg bw, 2/5 rats died at days 2, and 3 post-dosing, at 7071 mg/kg bw 4/5 rats died at days 0, 2, 3, and 5 post dosing and at 10000 mg/kg bw, 5/5 rats died at days 0 (3), and 2 (2) post-dosing. Clinical signs: motor inactivity and uncoordination, distinct disorientation at 24 and 48 hours (all levels; not otherwise specified). All survivors showed acute losses in body weight for several days; these were recovered 5 or 6 days after treatment. Autopsies revealed probable hepatic and renal pathology. |
| --- | --- | --- |
| Test substance | : | t-nonyl mercaptan; purity not noted |
| Reliability | : | (2) valid with restrictions |
|  |  | Meets generally accepted scientific standards, well documented and acceptable for assessment. |
| Flag | : | Critical study for SIDS endpoint |
| 20.06.2012 | (12) |  |
| Type | : | LD50 |
| Value | : | 3700 mg/kg bw |
| Species | : | rat |
| Strain | : | Sprague-Dawley |
| Sex | : |  |
| Number of animals | : | 16 |
| Vehicle | : | other: undiluted |
| Doses | : | 2000, 3000, 4600 and 6800 mg/kg bw |
| Method | : | other: IBT protocol |
| Year | : | 1965 |
| GLP | : | no data |
| Test substance | : |  |

| Method | : | Young, albino rats, Sprague Dawley strain, weighing approximately 130 grams were selected as the test animals. Four rats were dosed with the undiluted test material, at each level. The animals were fasted for 16 hours and administration was made by gavage. The animals were observed for 14 days. The LD 50 value was calculated according to Thompson and Weil (1952). |
| --- | --- | --- |
| Result | : | The acute oral LD50 was calculated to be 3.7 g/kg of body weight. Gross toxic signs consisted of hypothermia, emaciation and loss of the righting reflex. Delayed deaths occurred from 3 to 12 days following intubation. At necropsy no gross pathological alterations were noted. |
| Test substance | : | t-nonyl mercaptan; purity not noted |
| Reliability | : | (4) not assignable |
|  |  | Study performed at Industrial Biotest |
| 18.01.2012 | (11) |  |

5.1.2	ACUTE INHALATION TOXICITY

| Type | : | LC50 |
| --- | --- | --- |
| Value | : | > 1.97 mg/l |
| Species | : | rat |
| Strain | : | Sprague-Dawley |
| Sex | : | male/female |
| Number of animals | : | 20 |
| Vehicle | : | other: none |
| Doses | : |  |
| Exposure time | : | 4 hour(s) |
| Method | : | OECD Guide-line 403 "Acute Inhalation Toxicity" |
| Year | : |  |
| GLP | : | yes |
| Test substance | : |  |

| Method | : | Route of administration: A single group of animals was exposed once to t-nonyl mercaptan vapour via whole body inhalation for 4-hours. The vapour concentration was the highest attainable at which the test substance was present as vapour only. Observation period: 14 days post exposure. Atmosphere concentration of t-nonyl mercaptan (measured): 1.97 mg/L. |
| --- | --- | --- |
| Result | : | Mortality: There were no deaths. Clinical signs: (a) During exposure: Signs of irritant effects including closing or partial closing of the eyes, disturbances to the respiratory pattern and adoption of an abnormal body posture. (b) During the observation period: Ataxia and slow, deep respiration immediately after exposure. Normal from Day 1 of the post-exposure observation period. Bodyweight: No treatment-related effects. Lung weight to bodyweight ratios: Within normal limits for all rats. Macroscopic pathology: No treatment-related findings. |
| Test substance | : | t-nonyl mercaptan; batch 2409, purity 97%; Pennwalt Corporation. |
| Conclusion | : | Estimation of the LC50 (4-hour): There were no deaths following exposure to t-nonyl mercaptan at a concentration of 1.97 mg/L (300 ppm). This was the highest concentration attainable at which the test substance was all present as vapour. |
| Reliability | : | (1) valid without restriction |
|  |  | GLP guideline study |
| Flag | : | Critical study for SIDS endpoint |
| 20.06.2012 | (10) |  |
| Type | : | LC50 |
| Value | : | > 1074 ppm |
| Species | : | rat |
| Strain | : | other: Crl:CD (SD) BR |
| Sex | : | male/female |
| Number of animals | : | 20 |
| Vehicle | : | other: none |
| Doses | : | 1074 ppm (7.04 mg/L; measured); 1558 ppm (10.21 mg/L; nominal) |
| Exposure time | : | 4 hour(s) |
| Method | : | other: Similar to OECD 403 and OPPTS 870.1300 |
| Year | : | 1987 |
| GLP | : | yes |
| Test substance | : |  |

| Result | : | LC50: The estimated median lethal concentration for a 4-hour exposure and 14-day post-exposure observation period was > 1074 ppm (> 7.04 mg/L). Clinical Observation: Effects related to exposure included ataxia, transient salivation, piloerection and wet fur. Chromodacryhorrhea and nasal discharge were observed on the day of exposure for treated and control animals. These changes were attributed to the procedure used for animal restraint. Body weights: A small and transient weight loss occurred in both control and test animals. Since both groups were affected similarly, the effect was attributed to animal restraint procedures. All animals had returned to at least their pre-exposure body weights by day 7 of the observation period. Pathology: No gross or histopathological evidence of any treatment-related changes were observed in selected tissues and organs of exposed animals. |
| --- | --- | --- |
| Test condition | : | Species/strain: Rats/male and female Crl:CD(SD)BR (180-200g) No. of animals/sex/dose: 5/sex/dose Route of administration: Inhalation (head only) Frequency of treatment: Single Dose/Conc. Levels: 0 and 1074 ppm (7.04 mg/L) measured [for the nominal concentration of 1558 ppm (10.21 mg/L)] Doses per time period: 4-hour exposure (Animals were actually exposed to test material for about 4.5 hours instead of 4 hours because of technical problems.) Control group and treatment: Yes. Concurrent room air Post exposure period - 14 days Clinical observations: Hourly during exposure and then for the remainder of the work day and once daily for 14 days. Body weights: Body weights were recorded before and after exposure, and on Days 7 and 14 and immediately before the terminal kill. A detailed macroscopic evaluation was performed at scheduled termination. The following tissues were fixed in formalin: kidneys, lungs, liver and all gross lesions. These tissues were embedded, sectioned, stained and examined by the study pathologist. |
| Test substance | : | t-nonyl mercaptan; purity not noted |
| Reliability | : | (1) valid without restriction |
|  |  | Comparable to Guideline study |
| 18.01.2012 | (9) |  |
| Type | : | LC50 |
| Value | : | > 1033 ppm |
| Species | : | rat |
| Strain | : |  |
| Sex | : | male/female |
| Number of animals | : | 10 |
| Vehicle | : |  |
| Doses | : | 1033 ppm (6.77 mg/L) |
| Exposure time | : |  |
| Method | : | other: according to Hazleton Laboratories, America protocol |
| Year | : | 1983 |
| GLP | : | no data |
| Test substance | : |  |

| Remark | : | Nominal concentration = 2.56 mg/L Analytical concentration = 1033 (+/- 85) ppm (6.77 +/- 0.58 mg/L) 5 animals /sex. There were no deaths. All rats appeared normal throughout the study. Upon gross pathology, a dilated renal pelvis was observed in two male rats. |
| --- | --- | --- |
| Reliability | : | (2) valid with restrictions |
|  |  | Adequate for interpretation based on consistency with other studies |
| 20.06.2012 | (7) |  |
| Type | : | LC50 |
| Value | : | > 7.9 mg/l |
| Species | : | rat |
| Strain | : | Sprague-Dawley |
| Sex | : |  |
| Number of animals | : | 4 |
| Vehicle | : | other: none (saturated atmosphere ) |
| Doses | : | 7.9 mg/L (nominal) |
| Exposure time | : | 4 hour(s) |
| Method | : |  |
| Year | : | 1965 |
| GLP | : | no data |
| Test substance | : |  |

| Method | : | Young, albino rats, Sprague Dawley strain, weighing approximately 250 grams were selected as the test animals. The animals were not fasted prior to exposure. A single group of four rats were exposed to a saturated air atmosphere of the test product for a single, four hour period. The nominal concentration was calculated to be 7.9 mg/L of air. The rats were observed for a total of 14 days. |
| --- | --- | --- |
| Result | : | No deaths occurred. No gross toxic signs were noted during the study. No gross pathological alterations were noted at necropsy. |
| Test substance | : | t-nonyl mercaptan; purity not noted |
| Reliability | : | (4) not assignable |
|  |  | Study performed at Industrial Biotest |
| 20.06.2012 | (11) |  |
| Type | : | other: Respiratory Tract Irritancy |
| Value | : | > 2.54 mg/l |
| Species | : | mouse |
| Strain | : |  |
| Sex | : |  |
| Number of animals | : |  |
| Vehicle | : |  |
| Doses | : |  |
| Exposure time | : |  |
| Method | : |  |
| Year | : | 1965 |
| GLP | : | no |
| Test substance | : |  |

| Remark | : | Four mice were used to test the potential of this material to act as a respiratory irritant. |
| --- | --- | --- |
| Result | : | A pattern of slight respiratory pauses was evident in Mouse #3 during both exposures. However, no patterns of respiratory pauses were evident in any of the other animals during the exposures. Based on these results, exposure to TNM at a nominal concentration of 2.54 mg/L of air via inhalation failed to produce upper respiratory irritancy in mice. |
| Test substance | : | t-nonyl mercaptan; purity not noted |
| Reliability | : | (4) not assignable |
|  |  | Study performed at Industrial Biotest |
| 18.01.2012 | (11) |  |
| Type | : | other: Respiratory tract irritation |
| Value | : |  |
| Species | : | mouse |
| Strain | : |  |
| Sex | : |  |
| Number of animals | : |  |
| Vehicle | : |  |
| Doses | : |  |
| Exposure time | : |  |
| Method | : | other: according to Hazleton Laboratories, America protocol |
| Year | : |  |
| GLP | : | no data |
| Test substance | : |  |

| Remark | : | Mouse Outbred SPF male mice (CD-1, COBS), Charles River Laboratories, Inc. Young adults (20-25 grams). Number per group =4. |
| --- | --- | --- |
| Result | : | Based on these results, exposure to Sulfole 90 at a nominal concentration of 2.54 mg/L of air via inhalation failed to produce upper respiratory irritancy in mice. |
| Test condition | : | Nominal concentration = 2.54 mg/L Analytical concentration = 1067.5 (+/- 3.54) ppm (6.99 +/- 0.02 mg/L) |
| Test substance | : | t-nonyl mercaptan; purity not noted |
| Reliability | : | (4) not assignable |
|  |  | Insufficient data for interpretation |
| 20.06.2012 | (8) |  |

5.1.3	ACUTE DERMAL TOXICITY

| Type | : | LD50 |
| --- | --- | --- |
| Value | : | > 2000 mg/kg bw |
| Species | : | rat |
| Strain | : | other: WBS/W |
| Sex | : | male |
| Number of animals | : | 6 |
| Vehicle | : | other: none |
| Doses | : |  |
| Method | : | other |
| Year | : |  |
| GLP | : | no |
| Test substance | : |  |

| Test substance | : | t-nonyl mercaptan; purity not noted |
| --- | --- | --- |
| Reliability | : | (2) valid with restrictions |
|  |  | Meets generally accepted scientific standards. |
| Flag | : | Critical study for SIDS endpoint |
| 20.06.2012 | (12) |  |
| Type | : | LD50 |
| Value | : | > 10200 mg/kg bw |
| Species | : | rabbit |
| Strain | : | other: New Zealand strain |
| Sex | : | male/female |
| Number of animals | : | 4 |
| Vehicle | : | other: undiluted |
| Doses | : | 4.6, 6.8 and 10.2 g/kg bw |
| Method | : | other: according to IBT protocol |
| Year | : | 1965 |
| GLP | : | no data |
| Test substance | : |  |

| Method | : | Young, albino rabbits, New Zealand strain, weighing approximately 2.5 kg were selected as the test animals. An area on each animal was shaved free of hair with electric clippers 24 hours prior to dosing. Two males and two females were dosed with the undiluted test material under an occlusive plastic sheeting, at each dose level. Applications were made at dose levels of 4.6, 6.8 and 10.2 g/kg of body weight and contact was maintained for 24 hours on shaved skin. The animals were observed for 14 days. The LD50 value was calculated using the method of Thompson and Weil (1952). |
| --- | --- | --- |
| Result | : | The acute percutaneous LD50 in albino rabbits is considered to be greater than 10.2 g/kg of body weight. No deaths occurred. Gross toxic signs consisted of moderate erythema and edema at the application site. Hyperactivity was observed for one hour after the start of application. |
| Test substance | : | t-nonyl mercaptan; purity not noted |
| Reliability | : | (4) not assignable |
|  |  | Study performed at Industrial Biotest |
| 18.01.2012 | (11) |  |

5.1.4	ACUTE TOXICITY, OTHER ROUTES

5.2.1	SKIN IRRITATION

| Species | : | rabbit |
| --- | --- | --- |
| Concentration | : | undiluted |
| Exposure | : | Occlusive |
| Exposure time | : | 4 hour(s) |
| Number of animals | : | 6 |
| Vehicle | : |  |
| PDII | : |  |
| Result | : | not irritating |
| Classification | : | not irritating |
| Method | : | other: 49 CFR 173.240 |
| Year | : | 1977 |
| GLP | : | no |
| Test substance | : |  |

| Test substance | : | t-nonyl mercaptan; purity not noted |
| --- | --- | --- |
| Reliability | : | (2) valid with restrictions |
|  |  | Meets generally accepted scientific standards, well documented and acceptable for assessment |
| Flag | : | Critical study for SIDS endpoint |
| 19.01.2010 | (12) |  |
| Species | : | rabbit |
| Concentration | : | undiluted |
| Exposure | : | Occlusive |
| Exposure time | : | 24 hour(s) |
| Number of animals | : | 4 |
| Vehicle | : | other: none |
| PDII | : | 6.1 |
| Result | : | corrosive |
| Classification | : |  |
| Method | : | Draize Test |
| Year | : | 1965 |
| GLP | : | no data |
| Test substance | : |  |

| Method | : | Undiluted test material was applied to the shaved backs of four albino rabbits at a volume of 0.5 mL on each test site. The material was confined to shaved, abraded and unabraded skin sites having an area of one square inch under a gauze patch. The entire trunk was enclosed with an occlusive plastic sheet and contact was maintained for 24 hours. The skin was examined for eschar, erythema, and edema on a graded scale of 0 to 4. |
| --- | --- | --- |
| Result | : | The 24 and 72 hour scores were averaged for abraded vs. non-abraded for the combined average irritation score. The primary irritation index was calculated to be 6.1. This material is identified as a skin irritant. |
| Test substance | : | t-nonyl mercaptan; purity not noted |
| Reliability | : | (4) not assignable |
|  |  | Study performed at Industrial Biotest |
| 17.01.2012 | (11) |  |

5.2.2	EYE IRRITATION

| Species | : | rabbit |
| --- | --- | --- |
| Concentration | : | undiluted |
| Dose | : | .1 ml |
| Exposure time | : | 24 hour(s) |
| Comment | : | not rinsed |
| Number of animals | : | 6 |
| Vehicle | : |  |
| Result | : | slightly irritating |
| Classification | : | not irritating |
| Method | : | other: 49 CFR 173.240 |
| Year | : |  |
| GLP | : | no |
| Test substance | : |  |

| Result | : | The only effect was a slight inflammatory reaction (without chemosis) in the conjunctivae which dissipated completely within 2 or 3 hours. Neither the cornea nor the iris was affected in any way. |
| --- | --- | --- |
| Test substance | : | t-nonyl mercaptan; purity not noted |
| Reliability | : | (2) valid with restrictions |
|  |  | Meets generally accepted scientific standards, well documented and acceptable for assessment |
| Flag | : | Critical study for SIDS endpoint |
| 16.06.2010 | (12) |  |
| Species | : | rabbit |
| Concentration | : | undiluted |
| Dose | : | .1 ml |
| Exposure time | : |  |
| Comment | : |  |
| Number of animals | : | 5 |
| Vehicle | : |  |
| Result | : | slightly irritating |
| Classification | : |  |
| Method | : | Draize Test |
| Year | : | 1965 |
| GLP | : | no |
| Test substance | : |  |

| Method | : | Undiluted test material was instilled into the right eye of five, adult, albino rabbits, New Zealand strain, at a volume of 0.1 mL. The left eye served as a control. Observations for irritation of the cornea, iris, palpebral conjunctiva was scored at 1, 24, 72, and 96 hours and at 7 days according to the method of Draize.* * J. Pharm. Exper. Therap., 82:4(1944). |
| --- | --- | --- |
| Result | : | No corneal damage was noted. Iritis and slight conjunctivitis were noted through 24 hours post instillation. All eyes appeared normal at 48 hours post instillation. Out of a maximum score of 110: 1 hour = 12.6; 24 hours = 5.0 48 hours to 7 days = 0.0 |
| Test substance | : | t-nonyl mercaptan; purity not noted |
| Reliability | : | (4) not assignable |
|  |  | Study performed at Industrial Biotest. |
| 18.01.2012 | (11) |  |

5.3		SENSITIZATION

| Type | : | Guinea pig maximization test |  |
| --- | --- | --- | --- |
| Species | : | guinea pig |  |
| Concentration | : | 1st: | Induction 1 % intracutaneous |
|  |  | 2nd: | Induction undiluted occlusive epicutaneous |
|  |  | 3rd: | Challenge 75 % occlusive epicutaneous |
| Number of animals | : | 30 |  |
| Vehicle | : | other: paraffin oil |  |
| Result | : | sensitizing |  |
| Classification | : |  |  |
| Method | : | OECD Guide-line 406 "Skin Sensitization" |  |
| Year | : | 1995 |  |
| GLP | : | yes |  |
| Test substance | : |  |  |

| Method | : | Thirty Dunkin-Hartley guinea-pigs (15 males and 15 females) were randomly placed into 2 groups: a control group (5 males and 5 females) and a treated group (10 males and 10 females). The sensitization potential of the test substance was evaluated after a 10-day induction period during which time the animals were treated with paraffin oil (control group) or the test substance (treated group). On day 1, in presence of Freund's complete adjuvant, 0.1 mL of the test substance at a concentration of 1 % (w/w) in the vehicle was administered by intradermal route. On day 8, 0.5 mL of the test substance in its original form was applied by cutaneous route during 48 hours by means of an occlusive dressing. After a period of 12 days without treatment, a challenge cutaneous application of 0.5 mL of the vehicle (left flank) and 0.5 mL of the test substance at a concentration of 75% (w/w) in the vehicle (right flank) were administered to all animals. The test substance and the vehicle were prepared on a dry gauze pad then applied to the skin and held in place for 24 hours by means of an occlusive dressing. Cutaneous reactions on the challenge application sites were then evaluated 24 and 48 hours after removal of the dressing. After the final scoring period, the animals were sacrificed and cutaneous samples were taken from the challenge application sites from the animals. No histological examination was performed on the cutaneous samples. The sensitivity of the guinea-pigs in CIT. experimental conditions were checked in a recent study with a positive sensitizer: Dinitro 2.4 Chlorobenzene. During induction period the test substance was applied at 0.1% (day 1) and 5% (day 8) concentrations. At cutaneous challenge application, 1% (w/w) was tested on the right flank. |
| --- | --- | --- |
| Result | : | No treatment-related clinical signs or deaths were noted during the study. Well-defined erythema was observed in 2/19 animals 24 and 48 hours after removal of the dressing. No effects of the treatment were noted in the control group. The guinea-pigs which were used in a recent study showed a satisfactory sensitization response in 95% animals using a positive sensitizer. |
| Test substance | : | t-nonyl mercaptan; ; EAP, batch 94-000606; purity 99%. |
| Conclusion | : | According to the maximization method established by Magnusson and Kligman, cutaneous reactions attributable to the sensitization potential of the test substance, t-NONYL MERCAPTAN, at the concentration of 75% (w/w) were observed in 10% of the guinea-pigs in the treated groups. t-Nonyl mercaptan was considered to be a weak sensitiser. |
| Reliability | : | (1) valid without restriction |
| Flag | : | Critical study for SIDS endpoint |
| 20.06.2012 | (3) |  |

5.4		REPEATED DOSE TOXICITY

5.5		GENETIC TOXICITY ‘IN VITRO‘

| Type | : | Salmonella typhimurium reverse mutation assay |
| --- | --- | --- |
| System of testing | : | Strains: TA 1535, TA 1537, TA 98, TA 100 and TA 102 |
| Test concentration | : |  |
| Cycotoxic concentr. | : | >= 100 ug/plate |
| Metabolic activation | : | with and without |
| Result | : | negative |
| Method | : | OECD Guide-line 471 |
| Year | : | 1983 |
| GLP | : | yes |
| Test substance | : |  |

| Method | : | A preliminary toxicity test was performed to define the doses to be used for the mutagenicity study. The test substance was then tested in two independent tests, with or without a metabolic activation system, the S9 mix, prepared from a liver microsomal fraction (S9) of rats induced with Aroclor 1254. The tests were performed according to the direct plate incorporation method except the second test with S9 mix, according to the preincubation method (one hour, 37 degree C). Five strains of bacteria Salmonella typhimurium: TA1535, TA1537, TA98, TA100 and TA102 were used. Each strain was exposed to five doses of the test substance (three plates/dose). After 48 to 72 hours of incubation at 37 degree C, the revertant colonies were scored. The test substance t-NONYL MERCAPTAN was dissolved in dimethylsulfoxide (DMSO). The doses of the positive controls were as follows: without S9 mix: 1 ug/plate of sodium azide (NaN3): TA1535 and TA100 strains, 50 ug/plate of 9-Aminoacridine (9AA): TA1537 strain, 0.5 ug/plate of 2-Nitrofluorene (2NF): TA98 strain, 0.5 ug/plate of Mitomycin C (MMC): TA102 strain. with S9 mix: 2 ug/plate of 2-Anthramine (2AM): TA1535, TA1537, TA98 and TA100 strains, 10 ug/plate of 2-Anthramine (2AM): TA102 strain. The selected doses were: without S9 mix: TA1535, TA98 strains: 3, 10, 30, 100, 300 ug/plate, for both tests TA1537, TA100 strains: 3, 10, 30, 100, 300 ug/plate, for the first test 12.5, 25, 50, 100, 200 ug/plate for the second test. A102 strain: 50, 150, 500, 1500, 5000 ug/plate, for both tests . with S9 mix: TA1535, TA 98 strains: 5, 15, 50, 150, 500 ug/plate, for both tests TA102 strain: 1, 5, 10, 50, 100 ug/plate, for the first test 15, 50, 150, 500, 1500 ug/plate for the second test TA1537 strain: 3, 10, 30, 100, 300 ug/plate, for the first test 12.5, 25, 50, 100, 200 ug/plate for the second test. TA 100 strain: 3, 10, 30, 100, 300 ug/plate, for both tests. |
| --- | --- | --- |
| Result | : | The number of revertants of the vehicle and positive controls was as specified in the acceptance criteria and within the range of the historical data. The top dose was selected according to the criteria specified in the international regulations. Since the test substance was toxic, the top dose was based on the level of toxicity: reduction in the number of revertants and/or slight moderate marked clearing of the bacterial lawn. The test substance did not induce any significant increase in the number of revertants, with or without S9 mix, in any of the 5 strains. |
| Test substance | : | t-nonyl mercaptan; ; EAP, batch 94-000606; purity 99%. |
| Conclusion | : | t-NONYL MERCAPTAN did not show mutagenic activity in this bacterial reverse mutation assay on Salmonella typhimurium. |
| Reliability | : | (1) valid without restriction |
|  |  | GLP guideline study |
| Flag | : | Critical study for SIDS endpoint |
| 18.01.2012 | (4) |  |

5.6		GENETIC TOXICITY ‘IN VIVO‘

5.7		CARCINOGENICITY

5.8.1	TOXICITY TO FERTILITY

5.8.2	DEVELOPMENTAL TOXICITY/TERATOGENICITY

5.8.3	TOXICITY TO REPRODUCTION, OTHER STUDIES

5.9		SPECIFIC INVESTIGATIONS

5.10	EXPOSURE EXPERIENCE

5.11	ADDITIONAL REMARKS

6.1		ANALYTICAL METHODS

6.2		DETECTION AND IDENTIFICATION

7.1		FUNCTION

7.2		EFFECTS ON ORGANISMS TO BE CONTROLLED

7.3		ORGANISMS TO BE PROTECTED

7.4		USER

7.5		RESISTANCE

8.1		METHODS HANDLING AND STORING

8.2		FIRE GUIDANCE

8.3		EMERGENCY MEASURES

8.4		POSSIB. OF RENDERING SUBST. HARMLESS

8.5		WASTE MANAGEMENT

8.6		SIDE-EFFECTS DETECTION

8.7		SUBSTANCE REGISTERED AS DANGEROUS FOR GROUND WATER

8.8		REACTIVITY TOWARDS CONTAINER MATERIAL

| (1) | Bayer AG. 1988. Internal Report |
| --- | --- |
| (2) | Bingham, E., Cohrssen, B. and Powell, C.H. (eds). 2001. Patty's Industrial Hygiene and Toxicology, 5th Edition, Volume 7. John Wiley & Sons: New York, USA, pp 681-765. |
| (3) | CIT. 1995. t-Nonyl Mercaptan, Skin sensitization test in guinea-pigs. Study no. 12342TSG. Unpublished study report of Arkema. |
| (4) | CIT. 1995. t-Nonyl Mercaptan: Reverse mutation assay on bacteria Salmonella Typhimurium. Study no. 12343 MMO. Unpublished study report of Arkema. |
| (5) | Harlan Laboratories, Ltd. 2011. Tert-nonyl mercaptan (CAS No: 25360-10-5): Assessment of ready biodegradability; manometric respiratory test. Project No. 1635/0082. Unpublished Study Report (Sponsored by Chevron Phillips Chemical Co. LP). |
| (6) | Harlan Laboratories, Ltd. 2011. Tert-nonyl mercaptan (CAS No: 25360-10-5): Assessment of ready biodegradability; manometric respiratory test. Project No. 41003804. Unpublished Study Report (Sponsored by Chevron Phillips Chemical Co. LP). |
| (7) | Hazleton Laboratories America, Inc. 1983. Acute inhalation study in rats, Sulfole 90. Project No. 652-137. Unpublished study report of Chevron Phillips Chemical Co. |
| (8) | Hazleton Laboratories America, Inc. May 13, 1983. Respiratory tract irritation study in mice, Sulfole 90. Phillips Petroleum Company. Project No. 652-138. |
| (9) | Hazleton UK. 1987. Acute Inhalation Toxicity of Tertiary Nonyl Mercaptan in Rats. Report No. 5470-543/4. Project No. 543/4. Unpublished study report of Arkema. |
| (10) | Huntingdon Research Centre Ltd. 1987. t-Nonyl Mercaptan, Acute inhalation toxicity in rats. Study no. PWT 44G/861708. Unpublished study report of Arkema. |
| (11) | Industrial Biotest Laboratories. May 12, 1965. Acute Toxicity Studies on Sulfole 90. Phillips Petroleum Company. |
| (12) | Pharmacology Research, Inc. 1977. t-Nonyl Mercaptan. Toxicology report. Unpublished study report of Arkema. |
| (13) | U.S. EPA. 2011. Estimation Programs Interface Suite™ for Microsoft® Windows, Version 4.10. United States Environmental Protection Agency, Washington, DC, USA. |

10.1	END POINT SUMMARY

10.2	HAZARD SUMMARY

10.3	RISK ASSESSMENT