SIDS Dossier

OECD HPV Chemical Programme, SIDS Dossier, approved at CoCAM 3 (16/10/2012)

| Printing Date | 2013-02-06 15:21:53 JST |
| --- | --- |

Restriction of specific regulatory purposes

Confidentiality

| Name | (JP_CoCAM3) 3,4-Dimethylaniline |
| --- | --- |
| Legal entity owner | National Institute of Health Sciences / Tokyo / Japan |

# Substance: (JP_CoCAM3) 3,4-Dimethylaniline

| UUID |  | IUC5-2265ada2-c834-493f-8afa-4ef1ecd1c189 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | mariko / National Institute of Health Sciences / Tokyo / Japan |
| Date |  | 2013-02-06 15:21:46 JST |
| Remarks |  |  |

# 0 Related Information

# 0.1 Templates

# 0.2 Categories

# 0.3 Mixtures

# 1 General Information

# 1.1 Identification

Substance identification

| Chemical name | (JP_CoCAM3) 3,4-Dimethylaniline |
| --- | --- |
| Legal entity | National Institute of Health Sciences / Tokyo / Japan |

Reference substance

| 3,4-xylidine / 3,4-dimethylaniline / 95-64-7 |  |
| --- | --- |
| EC number; EC name. 202-437-4; 3,4-xylidine. CAS number; CAS name. 95-64-7. IUPAC name. 3,4-dimethylaniline. |  |

Type of substance

| Composition | mono constituent substance |
| --- | --- |
| Origin | organic |

# 1.2 Composition

# 1.3 Identifiers

# 1.4 Analytical information

# 1.5 Joint submission

# 1.6 Sponsors

# 1.7 Suppliers

# 1.8 Recipients

# 1.9 Product and process oriented research and development

# 2 Classification & Labelling and PBT assessment

# 2.1 GHS

# 2.2 DSD - DPD

# 3 Manufacture, use and exposure

# 3.1 Technological process

Technological process

Methods of manufacture of substance; 3,4-Dimethylaniline is produced by either nitration of o-xylene followed by reduction or bromination of o-xylene followed by reaction with ammonia ..

# 3.2 Estimated quantities

Estimated quantities

Remarks. Annual volumes of production and import on 3,4-dimethylaniline in Japan were 200 tonnes and 200 tonnes respectively around 1997 ？ 1998 (MOE, Japan, 2009c). According to the notification based on the Chemical Substances Control Law in Japan, the total volume of production and import on this chemical in fiscal year 2009 was less than 100 tonnes (NITE, 2012). Worldwide production volume is not available..

# 3.3 Sites

# 3.4 Information on mixtures

# 3.5 Life Cycle description

Identified uses

Uses at industrial sites

Identified Use number. Identified Use name; 3,4-Dimethylaniline is used as a chemical intermediate for the synthesis of riboflavin.. Subsequent service life relevant for that use?; yes.

# 3.6 Uses advised against

# 3.7 Exposure Scenarios, exposure and risk assessment

# 3.7.2 Environmental assessment for aggregated sources

# 3.7.3 Generic exposure potential

# 3.8 Biocidal information

# 3.10 Application for authorisation of uses

# 4 Physical and chemical properties

# 4.1 Appearance/physical state/colour

## Endpoint study record: LIterature -1

| UUID |  | IUC5-38e87599-c17d-4925-8308-f2914a00b71c |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V3.0 Plug-In |
| Date |  | 2011-02-17 20:26:53 JST |
| Remarks |  | Successfully migrated to IUCLID 5.4 format. |

Administrative Data

| Purpose flag | supporting study; robust study summary |
| --- | --- |
| Reliability | 2 (reliable with restrictions) |
| Rationale for reliability incl. deficiencies | Reliable literature |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| review article or handbook | Lide D.R. (ed) | 2008 | CRC Handbook of Chemistry and Physics, version 2008 |  |  |  |  |  |  |

Data access

data published

Results and discussion

Form

other: plates

Substance type

organic

Applicant's summary and conclusion

Conclusions

3,4-Dimethylaniline is plates.

## Endpoint study record: Literature -2

| UUID |  | IUC5-ad8feddf-c89d-4f00-b630-1cd605bd27d5 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V3.0 Plug-In |
| Date |  | 2011-02-17 20:28:33 JST |
| Remarks |  | Successfully migrated to IUCLID 5.4 format. |

Administrative Data

| Purpose flag | key study; robust study summary |
| --- | --- |
| Reliability | 2 (reliable with restrictions) |
| Rationale for reliability incl. deficiencies | Reliable literature |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| review article or handbook | Seidel A. (ed) | 2004 | Kirk-Othmer Encyclopedia of Chemical Technology, fifth edition, volume 2 |  |  |  |  |  |  |

Data access

data published

Results and discussion

Physical state at 20°C and 1013 hPa

solid

Substance type

organic

Applicant's summary and conclusion

Conclusions

3,4-Dimethylaniline is solid.

# 4.2 Melting point/freezing point

## Endpoint study record: Literature -1

| UUID |  | IUC5-fe909342-ce08-4050-9b74-9ea9cdb93f08 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V3.0 Plug-In |
| Date |  | 2011-02-17 18:34:23 JST |
| Remarks |  | Successfully migrated to IUCLID 5.4 format. |

Administrative Data

| Purpose flag | key study; robust study summary |
| --- | --- |
| Reliability | 2 (reliable with restrictions) |
| Rationale for reliability incl. deficiencies | Reliable literature |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| review article or handbook | Lide D.R. (ed) | 2008 | CRC Handbook of Chemistry and Physics, version 2008 |  |  |  |  |  |  |

Data access

data published

Results and discussions

Melting / freezing point

| Melt./Freez. pt. | Atm. pressure | Decomposition | Decomp. temp. | Sublimation | Subl. temp. | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 51 °C |  |  |  |  |  |  |

Applicant's summary and conclusion

Conclusions

Melting point is 51 degree C.

## Endpoint study record: Literature -2

| UUID |  | IUC5-7d8ee76c-eae4-4c8e-b36b-b014911b40fd |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V3.0 Plug-In |
| Date |  | 2011-02-17 20:06:59 JST |
| Remarks |  | Successfully migrated to IUCLID 5.4 format. |

Administrative Data

| Purpose flag | supporting study; robust study summary |
| --- | --- |
| Reliability | 2 (reliable with restrictions) |
| Rationale for reliability incl. deficiencies | Reliable literature |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| review article or handbook | Speight J.G. | 2005 | Lange's Handbook of Chemistry, 16th edition |  |  |  |  |  |  |

Data access

data published

Results and discussions

Melting / freezing point

| Melt./Freez. pt. | Atm. pressure | Decomposition | Decomp. temp. | Sublimation | Subl. temp. | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 51 °C |  |  |  |  |  |  |

Applicant's summary and conclusion

Conclusions

Melting point is 51 degree C.

## Endpoint study record: Literature -3

| UUID |  | IUC5-4374a43d-c1fe-4ee3-bde7-982491954a1d |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V3.0 Plug-In |
| Date |  | 2011-02-17 20:46:06 JST |
| Remarks |  | Successfully migrated to IUCLID 5.4 format. |

Administrative Data

| Purpose flag | supporting study; robust study summary |
| --- | --- |
| Reliability | 2 (reliable with restrictions) |
| Rationale for reliability incl. deficiencies | Reliable literature |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| review article or handbook | Seidel A. (ed) | 2004 | Kirk-Othmer Encyclopedia of Chemical Technology, fifth edition, volume 2 |  |  |  |  |  |  |

Data access

data published

Results and discussions

Melting / freezing point

| Melt./Freez. pt. | Atm. pressure | Decomposition | Decomp. temp. | Sublimation | Subl. temp. | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 51 °C |  |  |  |  |  |  |

Applicant's summary and conclusion

Conclusions

Melting point is 51 degree C.

# 4.3 Boiling point

## Endpoint study record: Literature -1

| UUID |  | IUC5-87d9b81a-d323-4b07-be97-24cc3df74e50 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V3.0 Plug-In |
| Date |  | 2011-02-17 18:35:53 JST |
| Remarks |  | Successfully migrated to IUCLID 5.4 format. |

Administrative Data

| Purpose flag | key study; robust study summary |
| --- | --- |
| Reliability | 2 (reliable with restrictions) |
| Rationale for reliability incl. deficiencies | Reliable literature |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| review article or handbook | Lide D.R. (ed) | 2008 | CRC Handbook of Chemistry and Physics, version 2008 |  |  |  |  |  |  |

Data access

data published

Results and discussions

Boiling point

| Boiling pt. | Atm. pressure | Decomposition | Decomp. temp. | Remarks |
| --- | --- | --- | --- | --- |
| 228 °C |  |  |  |  |

Applicant's summary and conclusion

Conclusions

boiling point is 228 degree C.

## Endpoint study record: Literature -2

| UUID |  | IUC5-b2892881-3a55-485e-a211-5e0c546749de |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V3.0 Plug-In |
| Date |  | 2011-02-17 20:08:23 JST |
| Remarks |  | Successfully migrated to IUCLID 5.4 format. |

Administrative Data

| Purpose flag | supporting study; robust study summary |
| --- | --- |
| Reliability | 2 (reliable with restrictions) |
| Rationale for reliability incl. deficiencies | Reliable literature |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| review article or handbook | Speight J.G. | 2005 | Lange's Handbook of Chemistry, 16th edition |  |  |  |  |  |  |

Data access

data published

Results and discussions

Boiling point

| Boiling pt. | Atm. pressure | Decomposition | Decomp. temp. | Remarks |
| --- | --- | --- | --- | --- |
| 228 °C |  |  |  |  |

Applicant's summary and conclusion

Conclusions

Boiling point is 228 degree C.

## Endpoint study record: Literature -3

| UUID |  | IUC5-ebaa0a74-a9ba-42a3-964a-0f2325558746 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V3.0 Plug-In |
| Date |  | 2011-02-17 20:47:46 JST |
| Remarks |  | Successfully migrated to IUCLID 5.4 format. |

Administrative Data

| Purpose flag | supporting study; robust study summary |
| --- | --- |
| Reliability | 2 (reliable with restrictions) |
| Rationale for reliability incl. deficiencies | Reliable literature |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| review article or handbook | Seidel A. (ed) | 2004 | Kirk-Othmer Encyclopedia of Chemical Technology, fifth edition, volume 2 |  |  |  |  |  |  |

Data access

data published

Results and discussions

Boiling point

| Boiling pt. | Atm. pressure | Decomposition | Decomp. temp. | Remarks |
| --- | --- | --- | --- | --- |
| 226 °C |  |  |  |  |

Applicant's summary and conclusion

Conclusions

Boiling point is 226 degree C.

# 4.4 Density

## Endpoint study record: Literature -1

| UUID |  | IUC5-65848830-846c-4448-989a-806e2e5bc8cf |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V3.0 Plug-In |
| Date |  | 2011-02-17 18:40:03 JST |
| Remarks |  | Successfully migrated to IUCLID 5.4 format. |

Administrative Data

| Purpose flag | key study; robust study summary |
| --- | --- |
| Reliability | 2 (reliable with restrictions) |
| Rationale for reliability incl. deficiencies | Reliable literature |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| review article or handbook | Lide D.R. (ed) | 2008 | CRC Handbook of Chemistry and Physics, version 2008 |  |  |  |  |  |  |

Data access

data published

Results and discussion

Density

| Type | Density | Temp. | Remarks |
| --- | --- | --- | --- |
| density | 1.076 g/cm³ | 18 °C |  |

Applicant's summary and conclusion

Conclusions

Density is 1.076 g/m3 at 18 degree C.

# 4.6 Vapour pressure

## Endpoint study record: Literature

| UUID |  | IUC5-c1977d94-c1b5-49a4-acfb-9ab3d000959c |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | CERI_S.MIYACHI / Chemicals Evaluation and Research Institute (CERI) / Tokyo / Japan |
| Date |  | 2012-11-02 22:44:08 JST |
| Remarks |  |  |

Administrative Data

| Purpose flag | key study |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result | Study period | 1992 |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Reliable paper |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Dudley S. Jayasinghe, Bruce J. Brownawell, Hua Chen, and John C. Westall | 1992 | Determination of Henry's Constants of Organice Compounds of Low Volatility: Methylanilines in Methanol - Water |  |  |  |  |  |  |

Data access

data published

Materials and methods

GLP compliance

no

Results and discussions

Vapour pressure

3.72 Pa

25 °C

Applicant's summary and conclusion

Conclusions

Vapour pressure is 3.72 Pa at 25 degree C.

# 4.7 Partition coefficient

## Endpoint study record: Literature

| UUID |  | IUC5-c28574b9-a17b-4e85-b02f-e12ec02d5763 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | CERI_S.MIYACHI / Chemicals Evaluation and Research Institute (CERI) / Tokyo / Japan |
| Date |  | 2012-11-02 22:43:49 JST |
| Remarks |  |  |

Administrative Data

| Purpose flag | key study |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result | Study period | 1992 |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Reliable paper |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Dudley S. Jayasinghe, Bruce J. Brownawell, Hua Chen, and John C. Westall | 1992 | Determination of Henry's Constants of Organice Compounds of Low Volatility: Methylanilines in Methanol - Water |  |  |  |  |  |  |

Data access

data published

Materials and methods

Partition coefficient type

octanol-water

GLP compliance

no

Results and discussions

Partition coefficient

| Type | Partition coefficient | Temp. | pH | Remarks |
| --- | --- | --- | --- | --- |
| log Pow | 1.84 |  |  |  |

Applicant's summary and conclusion

Conclusions

Partition Coefficient between 1-octanol and water (Log Kow) is 1.84.

# 4.8 Water solubility

## Endpoint study record: QSAR -1

| UUID |  | IUC5-7a305651-080d-4b1a-a743-dde7279aed82 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | CERI_S.MIYACHI / Chemicals Evaluation and Research Institute (CERI) / Tokyo / Japan |
| Date |  | 2012-11-02 23:13:48 JST |
| Remarks |  |  |

Administrative Data

| Purpose flag | supporting study |  |  |
| --- | --- | --- | --- |
| Study result type | (Q)SAR |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Reliable QSAR method |  |  |

Materials and methods

Principles of method if other than guideline

Water solublity is calculated by WSKOW (version 1.41a) with melting point of 51 °C and log Kow of 1.84.

Results and discussions

Water solubility

| 2457 mg/L |  |
| --- | --- |
| Temp. | 25 °C |

Applicant's summary and conclusion

Interpretation of results

soluble (1000-10000 mg/L)

Conclusions

Water solubility is 2457 mg/L at 25 degree C.

## Endpoint study record: Literature

| UUID |  | IUC5-f7973f93-67df-436d-bd53-41a203b7986a |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | CERI_S.MIYACHI / Chemicals Evaluation and Research Institute (CERI) / Tokyo / Japan |
| Date |  | 2012-11-08 11:40:32 JST |
| Remarks |  |  |

Administrative Data

| Purpose flag | key study |
| --- | --- |
| Reliability | 2 (reliable with restrictions) |
| Rationale for reliability incl. deficiencies | Reliable handbook |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| review article or handbook |  |  | Beilstein |  |  |  |  |  |  |

Results and discussions

Water solubility

| 3800 vol% |  |
| --- | --- |
| Temp. | 22 °C |

Details on results

Orginal reference: Beale, Journal of the Chemical Society, JCSOA9, 4494, 4496 (1954) 0.38 % at 22 degree C

Applicant's summary and conclusion

Interpretation of results

soluble (1000-10000 mg/L)

Conclusions

Water solubililty is 3.8 g/L at 22 degree C.

# 4.21 Dissociation constant

## Endpoint study record: Literature -1

| UUID |  | IUC5-b4736924-4da4-4dca-9b68-0184f239e6f8 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V3.0 Plug-In |
| Date |  | 2011-02-17 21:06:08 JST |
| Remarks |  | Successfully migrated to IUCLID 5.4 format. |

Administrative Data

| Purpose flag | key study; robust study summary |
| --- | --- |
| Reliability | 2 (reliable with restrictions) |
| Rationale for reliability incl. deficiencies | Reliable literature |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| review article or handbook | Speight J.G. | 2005 | Lange's Handbook of Chemistry, 16th edition |  |  |  |  |  |  |

Data access

data published

Results and discussions

Dissociating properties

yes

Dissociation constant

| No. | pKa | at | Remarks |
| --- | --- | --- | --- |
|  | 5.17 | 25 °C |  |

Applicant's summary and conclusion

Conclusions

Dissociaton constant in water (pKa) is 5.17 at 25 degree C.

# 5 Environmental fate and pathways

# 5.1 Stability

# 5.1.1 Phototransformation in air

## Endpoint study record: AOPWIN

| UUID |  | IUC5-8e124b2b-99ae-474e-ad8e-c53de4e756d8 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V3.0 Plug-In |
| Date |  | 2012-06-29 13:17:20 JST |
| Remarks |  | Successfully migrated to IUCLID 5.4 format. |

Administrative Data

| Purpose flag | key study |  |  |
| --- | --- | --- | --- |
| Study result type | (Q)SAR | Study period | 2012 |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Reliable QSAR method |  |  |

Materials and methods

Principles of method if other than guideline

QSAR calculation by AOPWIN (version 1.92) Indirect photo-oxidation by OH radicals

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 95-64-7 |
| IUPAC name | 3,4-dimethylaniline |

Results and discussions

Any other information on results incl. tables

Overall OH rate constat = 200.3E-12 cm3/molecule.sec OH concentration = 1.5E6 OH/cm3 (12 hours irradiation) Half-life time = 0.053 days

Applicant's summary and conclusion

Conclusions

Half-life time of 3,4-dimethylaniline by reaction with hydroxyl radicals in air is calculated to be 0.053 days by AOPWIN (version 1.92).

# 5.2 Biodegradation

# 5.2.1 Biodegradation in water: screening tests

## Endpoint study record: Experiment

| UUID |  | IUC5-21498f00-d2c1-486e-a299-224b08fcb3cd |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V3.0 Plug-In |
| Date |  | 2012-02-27 13:35:54 JST |
| Remarks |  | Successfully migrated to IUCLID 5.4 format. |

Administrative Data

| Purpose flag | key study |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result | Study period | 17/12/1976 - 26/3/1977 |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Test was conducted similar to OECD test guideline |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | Chemicals Evaluation and Research Institute, Japan | 1977 | Study of Biodegradation on 3,4-Dimethylaniline |  | Chemicals Evaluation and Research Institute, Japan |  |  |  |  |

Data access

data published

Materials and methods

Test type

ready biodegradability

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| equivalent or similar to | OECD Guideline 302 C (Inherent Biodegradability: Modified MITI Test (II)) | no |

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS name | 3,4-dimethylaniline |

Details on test material

- Name of test material (as cited in study report): 3,4-Dimethylaniline - Substance type: Organic substance - Physical state: Solid - Analytical purity: > 99.89 % - Impurities (identity and concentrations): 2,3-xylidine(0.02%) - Purity test date: Not known - Lot/batch No.:Not known - Expiration date of the lot/batch: Not known - Storage condition of test material: Dark cool place

Study design

Oxygen conditions

aerobic

Inoculum or test system

activated sludge, non-adapted

Details on inoculum

Preparation of sludge:Sludge and surface water including surface soil were collected from ten different places in Japan which includes water treatment plants, rivers, lake and inner bays. These sludges and water/soil were mixed and cultivated with glucose and peptone as nutrient in a testing laboratory. Concentration of the activated sludge for testing: 100 mg/L as suspended solid

Duration of test (contact time)

4 wk

Initial test substance concentration

| Initial conc. | Based on |
| --- | --- |
| 30 mg/L | test mat. |

Parameter followed for biodegradation estimation

other: BOD

Details on analytical methods

DETAILS ON PRETREATMENT After the cultivation period, some of the test substance was taken out. Then this test solution was filtrated, and then analyzed with TOC and GC. IDENTIFICATION AND QUANTIFICATION OF PARENT COMPOUND GC - Separation method (e.g. HPLC, GC): GC - Conditions (column, mobile phase, etc.): Column: Apiezon/Chromosorb W C18, 2 m x 2 mm I.D. Carrier gas: Nitrogen gas Column temperature: 120 degree C - Detection method (e.g. ECD, UV, MS, ICP-AES, ICP-MS): FID - Internal or external calibration: External calibration - Method of confirmation of identity of measured compound: Elution time

Details on study design

TEST CONDITIONS - Solubilising agent (type and concentration if used): No - Suspended solids concentration: 100 mg/L - Continuous darkness: yes SAMPLING - Sampling frequency: BOD was continueously measured. GC analysis and TOC analysis were conducted after the cultivation period. - Sample storage before analysis: Analysis was conduted right after the sampling.

Results and discussions

% Degradation of test substance

| %Degr. | St. dev. | Parameter | Sampling time | Remarks |
| --- | --- | --- | --- | --- |
| 7.1 |  | other: BOD | 4 wk |  |

Overall remarks, attachments

Remarks on results including tables and figures

Bodegradability is as follows. -; Biodegradability. Result by BOD; 7.1 %. Result by HPLC; 4.8 %.

Applicant's summary and conclusion

Interpretation of results

under test conditions no biodegradation observed

Conclusions

3,4-Dimethylaniline was not biodegradated under the test condition.

# 5.3 Bioaccumulation

# 5.3.1 Bioaccumulation: aquatic / sediment

## Endpoint study record: Experiment

| UUID |  | IUC5-b0cfddac-4f85-43ff-a960-87049809654e |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V3.0 Plug-In |
| Date |  | 2012-07-02 14:04:25 JST |
| Remarks |  | Successfully migrated to IUCLID 5.4 format. |

Administrative Data

| Purpose flag | key study |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result | Study period | 13/8/1977-26/11/1977 |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Test was conducted squivalent to OECD test guideline. |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | Chemicals Evaluation and Research Institute, Japan | 1977 | Study of bio-concentration of 3,4-dimethylaniline |  | Chemicals Evaluation and Research Institute, Japan |  |  |  |  |

Data access

data published

Materials and methods

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| equivalent or similar to | OECD Guideline 305 (Bioconcentration: Flow-through Fish Test) |  |

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 95-64-7 |
| IUPAC name | 3,4-dimethylaniline |
| EC number | 202-437-4 |

Radiolabelling

no

Details on test material

- Name of test material (as cited in study report):3,4-dimethylaniline - Structural formula attached as image file (if other than submission substance): see Fig. - Substance type:Organic - Physical state:plate - Analytical purity:99.89% - Storage condition of test material: In dark cool place

Details on sampling

- Sampling intervals/frequency for test organisms: 2,3,4 and 6 week after starting point of study - Sample storage conditions before analysis: Analysis was conduted right after the sampling.

Details on analytical methods

DETAILS ON PRETREATMENT - Extraction (solvent used, method: e.g. liquid-liquid, SPE):liquid-liquid with hexane IDENTIFICATION AND QUANTIFICATION OF TEST SUBSTANCE/PRODUCT - Separation method (e.g. HPLC, GC):GC - Conditions (column, mobile phase, etc.): Column: PEG-20M 20% / Chromosorb-W AW-DMCS(80-100 mesh) i.d.; 2mm, length; 1m Carrier gas : Nitrogen Column temp.: 140 degree C - Detection method (e.g. ECD, UV, MS, ICP-AES, ICP-MS):FID - Internal or external calibration: External calibration - Method of confirmation of identity of measured compound:Elution time

Vehicle

no

Test organisms

Test organisms (species)

Cyprinus carpio

Details on test organisms

TEST ORGANISM - Length at study initiation (lenght definition, mean, range and SD):mean : approximately 10 cm - Weight at study initiation (mean and range, SD):mean : approximately 28 g ACCLIMATION - Acclimation period:21 days - Acclimation conditions (same as test or not): 25 degree C

Study design

Route of exposure

aqueous

Test type

flow-through

Water media type

freshwater

Total exposure / uptake duration

6 wk

Test conditions

Test temperature

25 degree C

Nominal and measured concentrations

-1st concentration group: Nominal = 1.0 mg/L, Measured=0.885-0.939(1-day - 6-week) -2nd concentration group: Nominal =0.1 mg/L, Measured=0.0883-0.0925(1-day - 6-week)

Details on test conditions

TEST SYSTEM - Test vessel: 100L - Renewal rate of test solution (frequency/flow rate):579L/d RANGE-FINDING / PRELIMINARY STUDY - Test concentrations: 2500 mg/L - Results used to determine the conditions for the definitive study: 48h-TLM value = 121 mg/L

Results and discussions

Bioaccumulation factor

| Conc. in environment / dose | Type | Value | Basis | Time of plateau | Calculation basis | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 1.0 mg/L | BCF | 1.9 — 3.3 | whole body w.w. |  |  |  |
| 0.1 mg/L | BCF | < 10 | whole body w.w. |  |  |  |

Any other information on results incl. tables

Bio-concentrated factor 2ｗ; 3ｗ; 4ｗ; 6ｗ. 1 mg/L; 2.2 2.3; 3.3 2.3; 2.5 3.1; 2.8 1.9. 0.1 mg/L; <10 <10; <10 <10; <10 <10; <10 <10.

Applicant's summary and conclusion

Conclusions

Bio-concentrated factor of 3,4-dimethylaniline is 1.9 - <10.

# 5.4 Transport and distribution

# 5.4.1 Adsorption / desorption

## Endpoint study record: QSAR -1

| UUID |  | IUC5-acbcf1c9-4131-407b-a215-b9b89d491f15 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V3.0 Plug-In |
| Date |  | 2011-02-17 22:04:29 JST |
| Remarks |  | Successfully migrated to IUCLID 5.4 format. |

Administrative Data

| Purpose flag | key study; robust study summary |  |  |
| --- | --- | --- | --- |
| Study result type | (Q)SAR |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Reliable QSAR method |  |  |

Materials and methods

Principles of method if other than guideline

Soil adsorption coefficeint is esmated by KOCWIN (version 2.00) with log Kow of 1.84.

Results and discussions

Adsorption coefficient

| Type | Value | Temperature | % Org. carbon | Remarks |
| --- | --- | --- | --- | --- |
| log Koc | 1.9 |  |  |  |

Applicant's summary and conclusion

Conclusions

Soil adsorption coefficeint (log Koc) of 1.9.

# 5.4.2 Henry's Law constant

## Endpoint study record: QSAR -1

| UUID |  | IUC5-a514d44a-0822-4303-848f-37ac36725bf2 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V3.0 Plug-In |
| Date |  | 2011-02-17 22:21:07 JST |
| Remarks |  | Successfully migrated to IUCLID 5.4 format. |

Administrative Data

| Purpose flag | supporting study; robust study summary |  |  |
| --- | --- | --- | --- |
| Study result type | (Q)SAR |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Reliable QSAR method |  |  |

Materials and methods

Principles of method if other than guideline

Henry's Law Constant is calculated by HENRYWIN (version 3.20) with bond contribution method.

Results and discussions

Henry's Law constant H

| H | 0.235 Pa m³/mol |
| --- | --- |
| Temp. (°C) | 25 |

Applicant's summary and conclusion

Conclusions

Henry's Law Constant is 0.235 Pa.m3/mol at 25 degree C.

## Endpoint study record: Estimation -1

| UUID |  | IUC5-929d2a0c-2ad4-46fe-aa8c-534b5c6d48d3 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V3.0 Plug-In |
| Date |  | 2011-02-17 22:36:03 JST |
| Remarks |  | Successfully migrated to IUCLID 5.4 format. |

Administrative Data

| Purpose flag | key study; robust study summary |  |  |
| --- | --- | --- | --- |
| Study result type | other: |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Estimation with reliable vapour pressure and water solubility. |  |  |

Materials and methods

Principles of method if other than guideline

Henry’s Law Constant is estimated with vapour pressure of 3.72 Pa 25 °C divided by water solubility of 2.5 g/L at 25 °C.

Results and discussions

Henry's Law constant H

| H | 0.18 Pa m³/mol |
| --- | --- |
| Temp. (°C) | 20 |

Applicant's summary and conclusion

Conclusions

Henry's Law Constant is 0.18 Pa.m3/mol at 25 degree C.

## Endpoint study record: Literature-1

| UUID |  | IUC5-1d794013-7f8c-49ed-a1fc-dfdf7462f9d5 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | CERI_S.MIYACHI / Chemicals Evaluation and Research Institute (CERI) / Tokyo / Japan |
| Date |  | 2012-11-06 18:01:57 JST |
| Remarks |  |  |

Administrative Data

| Purpose flag | key study |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result | Study period | 1992 |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Reliable paper |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Dudley S. Jayasinghe, Bruce J. Brownawell, Hua Chen, and John C. Westall | 1992 | Determination of Henry's Constants of Organice Compounds of Low Volatility: Methylanilines in Methanol - Water |  |  |  |  |  |  |

Data access

data published

Results and discussions

Henry's Law constant H

| H | 0.188 Pa m³/mol |
| --- | --- |
| Temp. (°C) | 25 |

Any other information on results incl. tables

Original valu in the literature: Log (Dimensionless Henry's Law Constant) = -4.12

Applicant's summary and conclusion

Conclusions

Henry's Law Constant is 0.188 Pa.m3/mol at 25 degree C.

# 5.4.3 Distribution modelling

## Endpoint study record: Calculation

| UUID |  | IUC5-04d0787f-0809-43ba-b412-e7acc32fa3db |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V3.0 Plug-In |
| Date |  | 2012-06-29 14:23:52 JST |
| Remarks |  | Successfully migrated to IUCLID 5.4 format. |

Administrative Data

| Purpose flag | key study |  |  |
| --- | --- | --- | --- |
| Study result type | estimated by calculation | Study period | 2012 |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Reliable calculation method |  |  |

Materials and methods

Model

Calculation according to Mackay, Level III

Calculation programme

EPIWIN (version 4.00)

Release year

2008

Media

other: Air-Water-Soil-Sediment

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 95-64-7 |
| IUPAC name | 3,4-dimethylaniline |

Any other information on materials and methods incl. tables

Input parameters are as follows. Melting point: 51 °C Boiling point: 228 °C Water solubility: 2500 mg/L Vapour pressure: 0.028 mmHg Partition coefficient between octanol and water: 1.84 Henry’s Law constant: 2.32×10-6atm.m3/mole at 25 °C

Results and discussions

Percent distribution in media

Any other information on results incl. tables

Distributions of 3,4 -dimethylaniline in each environmental compartment are as follows. 3,4-Dimethylaniline; Release to air, water and soil; Release to air; Release to water; Release to soil. Air compartment; 0.07 %; 26.6 %; 0.004 %; 0.002 %. Water compartment; 21.8 %; 10.8 %; 98.9 %; 2.87 %. Soil compartment; 77.9 %; 62.5 %; 0.01 %; 97.1 %. Sediment compartment; 0.24 %; 0.12 %; 1.1 %; 0.03 %.

Applicant's summary and conclusion

Conclusions

Fugacity level III calculations show that 3,4-dimethylaniline is mainly distributed to the water compartment (21.8 %) and soil compartment (77.9 %) if equally and continuously released to the air, soil and water.

# 5.5 Environmental data

# 5.5.1 Monitoring data

## Endpoint study record: Monitoring data.(MOE)

| UUID |  | IUC5-ed71a6bc-7f7d-40fe-86f6-1e84cc51d7d4 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | CERI_M.SEKIZAWA / Chemicals Evaluation and Research Institute (CERI) / Tokyo / Japan |
| Date |  | 2012-11-26 15:40:11 JST |
| Remarks |  |  |

Administrative Data

| Purpose flag | supporting study |  |  |
| --- | --- | --- | --- |
| Study result type | other: Environmental monitoring | Study period | 1976, 1977, 2005 |
| Reliability | 2 (reliable with restrictions) |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Japanese Ministry of Environment | 2011 | Report on Environmental Survey and Monitoring of Chemicals. |  |  |  |  |  |  |

Data access

data published

Materials and methods

Media

other: Surface water and Sediment

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 95-64-7 |
| IUPAC name | 3,4-dimethylaniline |

Results and discussions

Details on results

The results from the Report on Environmental Survey and Wildlife Monitoring of Chemicals compiled by the Ministry of the Environment are summarized in Table 1. This survey is carried out for water, bottom sediments, fishes, air, etc., at the sampling points set up around the country.

Any other information on results incl. tables

Table 1Results of the environmental survey and wildlife monitoring for 3,4-dimethylaniline Media; Fiscal Year; Number of detections/Number of samples; Number of detection stations/Number of sampling stations; Range of detection; Limit of detection. Surface water; 2005; 0/12; 0/4; -; 0.0072μg/L. Surface water; 1977; 0/6; 0/2; -; 1-20μg/L. Surface water; 1976; 0/68; 0/20; -; 0.06-0.7μg/L. Sediment; 2005; 0/9; 0/3; -; 0.0007μg/g dry. Sediment; 1977; 0/6; 0/2; -; 0.25-4μg/g dry. Sediment; 1976; 8/68; 4/20; 0.001-0.043μg/g dry; 0.001-0.004μg/g dry.

# 6 Ecotoxicological Information

# 6.1 Aquatic toxicity

# 6.1.1 Short-term toxicity to fish

## Endpoint study record: MOE, 2005a (Short-term toxicity to fish)

| UUID |  | IUC5-25eb84aa-7d17-41c6-8f02-b9a3bf2b09f9 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V3.0 Plug-In |
| Date |  | 2012-02-27 12:12:10 JST |
| Remarks |  | Successfully migrated to IUCLID 5.4 format. |

Administrative Data

| Purpose flag | key study; robust study summary |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result | Study period | 18 March 2005 - 20 July 2005 |
| Reliability | 1 (reliable without restriction) |  |  |
| Rationale for reliability incl. deficiencies | Nationl Guideline study under GLP conditions |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | KUREHA SPECIAL LABORATORY CO.,LTD. | 2005 | Fish (Oryzias latipes), Acute Toxicity Test of 3,4-Dimethylaniline |  | KUREHA SPECIAL LABORATORY CO.,LTD. |  | Ministry of the Environment, Japan | No. 2004-83 | 2005-07-20 |

Materials and methods

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | other guideline: Chemical Substances Control Law, Japan (equivalent to OECD Guideline 203) |  |

GLP compliance

yes : Chemical Substances Control Law, Japan (2003)

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 95-64-7 |
| EC number | 202-437-4 |
| EC name | 3,4-xylidine |
| IUPAC name | 3,4-dimethylaniline |

Details on test material

- Appearance: slightly bluish brown crystal - Purity: 100.0% - Lot/batch No.: CEN0677 - Storage condition of test material: In a refrigerator - Stability under the storage conditions: The test substance was stable under the storage conditions, because IR spectra of the test substance before and after the study were identical.

Analytical monitoring

yes

Details on sampling

- Frequency: at the start of the exposure (0 hour), before renewal of water (48 hours after exposure) and at the termination of the exposure - Sampling volume: 13mL

Details on analytical methods

PRETREATMENT OF TEST SOLUTION Samples were diluted with pure water. ANALYTICAL CONDITIONS - Instrument: HPLC-UV - Column: Mightysil RP-18GP (150mm×4.6mm I.D.) - Column temp.: 40 degree C - Eluent: Acetonitril/Pure water (65/35) - Flow rate: 1.0 mL/min - Measurement wavelength: UV 210 nm - Injection volume: 50 μL - Determination limit: 0.03 mg/L - Detection limit: 0.009 mg/L

Vehicle

no

Details on test solutions

1000mg of the test substance was dissolved with dilution water by stirring to produce a stock solution of 1000 mg/L . Test solution was prepared in each test vessel by mixing the appropriate volume of the stock solution with dilution water. Dilution water was used for the control group.

Test organisms

Test organisms (species)

Oryzias latipes

Details on test organisms

TEST ORGANISM - Source: Laboratory-bred - Length at study initiation: 2.5 cm (2.4-2.7 cm), n=10 - Weight at study initiation: 0.13 g (0.10-0.16 g), n=10 - Feeding during test: No ACCLIMATION - Acclimation period: 2005/3/1-2005/5/23 (10 months old) - Acclimation conditions: Same as test except exposure type (flow-through) - Type and amount of food: Type: Commercial fish food "TetraMin" Amount: ca. 2% of body weight/day, Not fed for 24 hours prior to the test - Health during acclimation: Mortality for 7 days prior to the test was 1.1 % OTHER - Confirming the reproducibility of the test conditions: Reference substance: Copper sulfate (II) pentahydrate (Reagent grade) Results: 96h-LC50: 0.27 mg/L (within the acceptable range to background data(0.31 ± 0.11 mg/L, n=19) in this laboratory)

Study design

Test type

semi-static

Water media type

freshwater

Limit test

no

Total exposure duration

96 h

Test conditions

Hardness

No data

Test temperature

23.8 - 24.2 degree C

pH

7.3 - 7.9

Dissolved oxygen

5.9 - 8.0 mg/L (higher than 60 % of the O2 saturation conc. (8.25 mg/L at 24.0 degree C))

Nominal and measured concentrations

- Nominal concentrations: 10, 18, 32, 56 and 100 mg/L - Measured concentrations (arithmetic mean): 9.64, 16.6, 30.4, 55.5 and 97.9 mg/L

Details on test conditions

TEST SYSTEM - Test vessel: - Type: no data - Material: Glass - Fill volume: 3L - Test solution volume: 3L/vessel - Aeration: No - Renewal rate of test solution: 48 hours after the start of exposure, the whole quantity - No. of organisms per vessel: 10 - No. of vessels per concentration (replicates): 1 - No. of vessels per control (replicates): 1 TEST MEDIUM / WATER PARAMETERS - Source/preparation of dilution water: Dechlorinated water (water from the water facility, removed the residual chlorine with the activated carbon and aerated sufficiently) - Hardness: 31 mg/L (as CaCO3) - pH: 7.8 OTHER TEST CONDITIONS - Adjustment of pH: No - Photoperiod: 16 hours light / 8 hours dark - Light intensity: Room light OBSERVATION AND MEASUREMENTS - Fish: Mortality and visible abnormality were observed at 0, 24, 48, 72 and 96 hours. - Test solutions: Temperature, pH and Dissolved oxygen concentration were measured at the start of the exposure and every 24 hours (before and after renewal of water).

Reference substance (positive control)

no

Any other information on materials and methods incl. tables

Table 1. Measured Concentration of the Test Substance in the Test Water Nominal Concentration (mg/L); Measured Concentration,mg/L (Percent of Nominal); MeanaMeasured Concentration (mg/L). 0 hour; 48hours; 96hours. new; old; new; old. Control; <0.03 －; <0.03 －; <0.03 －; <0.03 －; －. 10; 10.8 (108); 9.63 ( 96); 9.25 ( 93); 8.89 ( 89); 9.64. 18; 19.2 (107); 16.7 ( 93); 14.7 ( 82); 15.8 ( 88); 16.6. 32; 34.1 (107); 29.8 ( 93); 27.0 ( 84); 30.5 ( 95); 30.4. 56; 58.9 (105); 55.9 (100); 49.1 ( 88); 57.9 (103); 55.5. 100; 104 (104); 99.6 (100); 85.9 ( 86); 102 (102); 97.9. new: Freshly prepared test solutions, old: Test solutions after 48 hour exposure, a: Arithmetic mean Table 2. Temperature Nominal Concentration (mg/L); MeanaMeasured Concentration (mg/L); Temperature,℃. 0 Hour; 24 Hours; 48 Hours; 72 Hours; 96 Hours. new; old; new; old. Control; －; 24.1; 24.2; 24.0; 24.0; 24.0; 24.0. 10; 9.6; 24.1; 24.2; 24.0; 24.0; 24.0; 24.0. 18; 16.6; 24.0; 24.2; 24.0; 23.8; 24.0; 24.0. 32; 30.4; 24.0; 24.2; 24.0; 23.8; 24.0; 24.0. 56; 55.5; 24.0; 24.2; 24.0; 23.8; 24.0; 24.0. 100; 97.9; 24.0; 24.2; 24.0; 23.8; 24.0; 24.0. new : Freshly prepared test solution, old : Test solution after 48 hours exposure, a:Arithmetic Mean, -: Not calculated Table 3. Dissolved Oxygen Concentration Nominal Concentration (mg/L); MeanaMeasured Concentration (mg/L); Dissolved Oxygen Concentration, mg/L. 0 Hour; 24 Hours; 48 Hours; 72 Hours; 96 Hours. New; Old; New; Old. Control; －; 7.8; 6.4; 5.9; 8.0; 6.6; 6.3. 10; 9.6; 7.9; 6.3; 6.3; 8.0; 6.6; 6.4. 18; 16.6; 7.8; 6.5; 6.3; 8.0; 6.8; 6.5. 32; 30.4; 7.9; 6.4; 6.1; 8.0; 6.8; 6.5. 56; 55.5; 7.8; 6.6; 6.4; 8.0; 7.0; 6.7. 100; 97.9; 7.8; 6.6; 6.1; 8.0; 6.7; 5.9. new : Freshly prepared test solution, old : Test solution after 48 hours exposure, a:Arithmetic Mean, -: Not calculated Table 4. pH Values Nominal Concentration (mg/L); MeanaMeasured Concentration (mg/L); pH. 0 Hour; 24 Hours; 48 Hours; 72 Hours; 96 Hours. New; Old; New; Old. Control; －; 7.8; 7.5; 7.3; 7.9; 7.4; 7.3. 10; 9.6; 7.8; 7.5; 7.4; 7.9; 7.4; 7.4. 18; 16.6; 7.9; 7.5; 7.4; 7.9; 7.5; 7.4. 32; 30.4; 7.9; 7.5; 7.4; 7.9; 7.5; 7.4. 56; 55.5; 7.9; 7.5; 7.4; 7.9; 7.5; 7.5. 100; 97.9; 7.9; 7.5; 7.4; 7.9; 7.5; 7.4. new : Freshly prepared test solution, old : Test solution after 48 hours exposure, a:Arithmetic Mean, -: Not calculated

Results and discussions

Effect concentrations

| Duration | Endpoint | Effect conc. | Nominal/Measured | Conc. based on | Basis for effect | Remarks (e.g. 95% CL) |
| --- | --- | --- | --- | --- | --- | --- |
| 96 h | LC50 | > 97.9 mg/L | meas. (arithm. mean) | test mat. | mortality |  |

Details on results

- The highest concentration of 0% mortality at 96 hours: 55.5 mg/L - The lowest concentration of 100% mortality at 96 hours: > 97.9 mg/L - The No observed effect concentration at 96 hours: 16.6 mg/L - No toxicological symptom was observed at control, 10 mg/L and 18 mg/L groups. In contrast, at 32 mg/L group, abnormal swimming was observed. At 56 mg/L and 100 mg/L groups, abnormal swimming and innobile were observed. - There were no environmental factors which might have affected the reliability of the test.

Any other information on results incl. tables

Table 5. The Numbers of Dead Fish (Mortality) Nominal Concentration (mg/L); MeanaMeasured Concentration (mg/L); Cumulative Mortality(Percent Mortality). 24 Hours; 48 Hours; 72 Hours; 96 Hours. Control; －; 0( 0); 0( 0); 0( 0); 0( 0). 10; 9.64; 0( 0); 0( 0); 0( 0); 0( 0). 18; 16.6; 0( 0); 0( 0); 0( 0); 0( 0). 32; 30.4; 0( 0); 0( 0); 0( 0); 0( 0). 56; 55.5; 0( 0); 0( 0); 0( 0); 0( 0). 100; 97.9; 0( 0); 0( 0); 0( 0); 4( 40). a: Arithmetic mean, －: Not calculated Table 6. The Highest Concentration in 0% Mortality, the Lowest Concentration in 100% Mortality and No observed Effect Concentration Exposure Period (Hours); Highest Concentration in 0 % Mortality (mg/L); Lowest Concentration in 100 % Mortality (mg/L); No Observed Effect Concentration (mg/L). 24; 97.9; >97.9; 30.4*. 48; 97.9; >97.9; 30.4*. 72; 97.9; >97.9; 30.4*. 96; 55.5; >97.9; 30.4*. ＊:Fish at 30.4 mg/L were recovered from the toxicological symptom at the 3 hour Table 7. Observed Toxicological Symptoms Nominal Concentration (mg/L); MeanaMeasured Concentration (mg/L); Symptoms. Observation Time. 3 Hours; 6 Hours; 24 Hours; 48 Hours; 72 Hours; 96 Hours. Control; －; N; N; N; N; N; N. 10; 9.64; N; N; N; N; N; N. 18; 16.6; N; N; N; N; N; N. 32; 30.4; AS(2); N; N; N; N; N. 56; 55.5; AS(7); AS(4); AS(3); AS(2); AS(3); AS(1). 100; 97.9; AS(9)I(1); AS(8); AS(7)I(1); AS(3)I(4); AS(4)I(4); AS(5). N: No toxicological symptom was observed, AS: Abnormal swimming, I: Immobile, a: Arithmetic mean, －: Not calculated

# 6.1.3 Short-term toxicity to aquatic invertebrates

## Endpoint study record: MOE, 2005b (Short-term toxicity to aquatic invertebrates)

| UUID |  | IUC5-34c1a6de-8660-458e-a7a0-1114a5e36dce |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V3.0 Plug-In |
| Date |  | 2012-02-27 13:15:51 JST |
| Remarks |  | Successfully migrated to IUCLID 5.4 format. |

Administrative Data

| Purpose flag | key study; robust study summary |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result | Study period | 18 March 2005 - 20 July 2005 |
| Reliability | 1 (reliable without restriction) |  |  |
| Rationale for reliability incl. deficiencies | Nationl Guideline study under GLP conditions |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | KUREHA SPECIAL LABORATORY CO.,LTD. | 2005 | Daphnia magna, Acute Immobilisation Test of 3,4-Dimethylaniline |  | KUREHA SPECIAL LABORATORY CO.,LTD. |  | Ministry of the Environment, Japan | 2004-81 | 2005-07-20 |

Materials and methods

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | other guideline: Chemical Substances Control Law, Japan (equivalent to OECD Guideline 202) |  |

GLP compliance

yes (: Chemical Substances Control Law, Japan (2003) )

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 95-64-7 |
| EC number | 202-437-4 |
| EC name | 3,4-xylidine |
| IUPAC name | 3,4-dimethylaniline |

Details on test material

- Appearance: slightly bluish brown crystal - Purity: 100.0% - Lot/batch No.: CEN0677 - Storage condition of test material: In a refrigerator - Stability under the storage conditions: The test substance was stable under the storage conditions, because IR spectra of the test substance before and after the study were identical.

Analytical monitoring

yes

Details on sampling

- Frequency: at the start of the exposure (0 hour) and at the termination of the exposure (48 hours after exposure) - Sampling volume: 13 mL

Details on analytical methods

PRETREATMENT OF TEST SOLUTION Samples were diluted with pure water. ANALYTICAL CONDITIONS - Instrument: HPLC-UV - Column: Mightysil RP-18GP (150mm×4.6mm I.D.) - Column temp.: 40 degreeC - Eluent: Acetonitril/Pure water (65/35) - Flow rate: 1.0 mL/min - Measurement wavelength: UV 210 nm - Injection volume: 50 μL - Determination limit: 0.03 mg/L - Detection limit: 0.009 mg/L

Vehicle

no

Details on test solutions

20mg of the test substance was dissolved with dilution water to produce a stock solution of 100 mg/L . Test solution was prepared in each test vessel by mixing the appropriate volume of the stock solution with dilution water. Dilution water was used for the control group.

Test organisms

Test organisms (species)

Daphnia magna

Details on test organisms

TEST ORGANISM - Common name: Water flea - Source: Subcultured in the testing laboratory, Originally from the National Institute for Environmental Studies (Japan) on 5 February 1997 - Age at study initiation: less than 24 hours old - Feeding during test: No ACCLIMATION OF PARENTAL DAPHNIDS - Acclimation period: 2005/4/12-2005/5/9 (27 days-old) - Acclimation conditions: See below - Breeding water: Dilution water - Population density: 30 daphnids/2 L breeding water - Temperature: 20+/-1 degreeC - Photoperiod: 16-hr Light / 8-hr Dark, Room light - Type and amount of food: Type: Chlorella vulgaris Amount: 0.15-0.2 mgC(Organic Carbon)/animal/day - Presence of male and ephippia were not observed. - Health during acclimation: Mortality of parental daphnids for 2 weeks prior to the test were less than 5 %. OTHER - Confirming the reproducibility of the test conditions: Reference substance: pottasium dichromate, reagent chemical Results: 48h EC50: 0.97 mg/L (background data in this laboratory: 0.87+/-0.23 mg/L, n=14 )

Study design

Test type

static

Water media type

freshwater

Limit test

no

Total exposure duration

48 h

Test conditions

Hardness

246 - 252 mg/L (as CaCO3)

Test temperature

20.2 - 20.3 degreeC

pH

8.0

Dissolved oxygen

8.5 mg/L (greater than 60 % of the O2 saturation conc. (8.84 mg/L at 20.0 degreeC))

Nominal and measured concentrations

- Nominal concentrations: 0.56, 1.0, 1.3, 1.8 and 3.2 mg/L - Measured concentrations (arithmetic mean): 0.542, 0.960, 1.24, 1.73 and 3.10 mg/L

Details on test conditions

TEST SYSTEM - Test vessel: - Material: Glass - Fill volume: 100 mL - Test solution volume: 100 mL/vessel - No. of organisms per vessel: 5 - No. of vessels per concentration (replicates): 4 - No. of vessels per control (replicates): 4 TEST MEDIUM / WATER PARAMETERS - Source/preparation of dilution water: Elendt M4 Medium recommended by OECD Guideline 211 used as dilution water OTHER TEST CONDITIONS - Adjustment of pH: No - Photoperiod: 16 hours light / 8 hours dark - Light intensity: Room light OBSERVATION AND MEASUREMENTS - Daphnids: Immobility was observed at 24 and 48 hours. Daphnids were considered immobile if they were not able to swim within 15 seconds after gentle agitation of the test vessel. - Test solutions: Dissolved oxygen concentration, pH and water temperature of the test solutions were measured at the start and the end of the exposure. The concentration of Ca and Mg were measured by ICP method at the start and the end of the exposure.

Reference substance (positive control)

no

Any other information on materials and methods incl. tables

Table 1. Measured Concentrations of the Test Substance in the Test Water Nominal Concentration (mg/L); Measured Concentration (mg/L); MeanaMeasured Concentration (mg/L). 0 Hour new; Percent of Nominal; 48 hours old; Percent of Nominal. Control; <0.03; -; <0.03; -; －. 0.56; 0.549; 98; 0.534; 95; 0.542. 1.0; 0.980; 98; 0.940; 94; 0.960. 1.3; 1.26; 97; 1.22; 94; 1.24. 1.8; 1.76; 98; 1.69; 94; 1.73. 3.2; 3.18; 99; 3.02; 94; 3.10. new: Freshly prepared test solutions, old: Test solutions after 48 hour exposure a: Arithmetic mean, －: Not calculated Table 2. Temperature Nominal Concentration (mg/L); MeanaMeasured Concentration (mg/L); Temperature ,℃. 0 Hour new; 48 Hours old. Control; -; 20.3; 20.3. 0.56; 0.542; 20.3; 20.3. 1; 0.960; 20.3; 20.3. 1.3; 1.24; 20.3; 20.3. 1.8; 1.73; 20.3; 20.2. 3.2; 3.10; 20.3; 20.2. new: Freshly prepared test solutions, old: Test solutions after 48 hour exposure a: Arithmetic mean, －: Not calculated Table 3. Dissolved Oxygen Concentrations Nominal Concentration (mg/L); MeanaMeasured Concentration (mg/L); Dissolved Oxygen Concentration ,mg/L. 0 Hour new; 48 Hours old. Control; -; 8.5; 8.5. 0.56; 0.542; 8.5; 8.5. 1; 0.960; 8.5; 8.5. 1.3; 1.24; 8.5; 8.5. 1.8; 1.73; 8.5; 8.5. 3.2; 3.10; 8.5; 8.5. new: Freshly prepared test solutions, old: Test solutions after 48 hour exposure a: Arithmetic mean, －: Not calculated Table 4. pH Values Nominal Concentration (mg/L); MeanaMeasured Concentration (mg/L); pH. 0 Hour; 48 Hours. New; Old. Control; -; 8.0; 8.0. 0.56; 0.542; 8.0; 8.0. 1.0; 0.960; 8.0; 8.0. 1.3; 1.24; 8.0; 8.0. 1.8; 1.73; 8.0; 8.0. 3.2; 3.10; 8.0; 8.0. new: Freshly prepared test solutions, old: Test solutions after 48 hour exposure a: Arithmetic mean, －: Not calculated Table 5. Total Hardness (as CaCO3) Nominal Concentration (mg/L); MeanaMeasured Concentration (mg/L); Total Hardness(as CaCO3),mg/L. 0 Hour new; 48 Hours old. Control; -; 246; 248. 0.56; 0.542; 248; 249. 1.0; 0.960; 250; 250. 1.3; 1.24; 247; 246. 1.8; 1.73; 248; 246. 3.2; 3.10; 252; 250. new: Freshly prepared test solutions, old: Test solutions after 48 hour exposure a: Arithmetic mean, －: Not calculated

Results and discussions

Effect concentrations

| Duration | Endpoint | Effect conc. | Nominal/Measured | Conc. based on | Basis for effect | Remarks (e.g. 95% CL) |
| --- | --- | --- | --- | --- | --- | --- |
| 24 h | EC50 | > 3.1 mg/L | meas. (arithm. mean) | test mat. | mobility |  |
| 48 h | EC50 | 1.09 mg/L | meas. (arithm. mean) | test mat. | mobility | 95%CL: 0.959-1.22 mg/L, Logit |

Details on results

- The lowest concentration of 100% immobility: at 24 hours; higher than 3.10 mg/L, at 48 hours 3.10 mg/L - The Highest concentration of 0% immobility: at 24 hours; 1.24 mg/L, at 48 hours 0.542 mg/L - Immobility of control was 0 %. - There were no factors which might have affected the reliability of the test

Any other information on results incl. tables

Table 6. The Numbers of Immobile Daphnia Nominal Concentration (mg/l); MeanaMeasured Concentration (mg/l); Number of Daphnia; Cumulative Numbers of ImmobilizedDaphnia (Percent Immobility). 24 Hours; 48 Hours. 1; 2; 3; 4; Total; 1; 2; 3; 4; Total. Control; -; 20; 0; 0; 0; 0; 0( 0); 0; 0; 0; 0; 0( 0). 0.56; 0.542; 20; 0; 0; 0; 0; 0( 0); 0; 0; 0; 0; 0( 0). 1; 0.96; 20; 0; 0; 0; 0; 0( 0); 3; 2; 1; 1; 7( 35). 1.3; 1.24; 20; 0; 0; 0; 0; 0( 0); 3; 3; 2; 5; 13( 65). 1.8; 1.73; 20; 0; 1; 3; 2; 6( 30); 4; 5; 5; 5; 19( 95). 3.2; 3.1; 20; 2; 0; 2; 1; 5( 25); 5; 5; 5; 5; 20(100). a: Arithmetic mean, －： Not calculated

## Endpoint study record: Kuhn et al., 1989b (Short-term toxicity to aquatic invertebrates)

| UUID |  | IUC5-2b2bef6d-798f-4a0c-b7c4-536121cb3f9b |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V3.0 Plug-In |
| Date |  | 2012-04-26 14:57:37 JST |
| Remarks |  | Successfully migrated to IUCLID 5.4 format. |

Administrative Data

| Study result type | experimental result |  |  |
| --- | --- | --- | --- |
| Reliability | 4 (not assignable) |  |  |
| Rationale for reliability incl. deficiencies | Study details are not described. Only EC50 values are listed. |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Kuhn, R., M. Pattard, K.D. Pernak, and A. Winter | 1989 | Results of the Harmful Effects of Water Pollutants to Daphnia magna in the 21 Day Reproduction Test | Water Res. 23(4):501-510 |  |  |  |  |  |

Materials and methods

GLP compliance

no data

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test organisms

Test organisms (species)

Daphnia magna

Results and discussions

Effect concentrations

| Duration | Endpoint | Effect conc. | Nominal/Measured | Conc. based on | Basis for effect | Remarks (e.g. 95% CL) |
| --- | --- | --- | --- | --- | --- | --- |
| 24 h | EC50 | 2.9 mg/L | nominal | test mat. | no data |  |

# 6.1.4 Long-term toxicity to aquatic invertebrates

## Endpoint study record: MOE, 2005d (Long-term toxicity to aquatic invertebrates)

| UUID |  | IUC5-a6612e67-3f1b-457b-9668-5ae3b15a1a8d |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V3.0 Plug-In |
| Date |  | 2012-02-27 13:33:49 JST |
| Remarks |  | Successfully migrated to IUCLID 5.4 format. |

Administrative Data

| Purpose flag | key study; robust study summary |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result | Study period | 18 March 2005 - 20 July 2005 |
| Reliability | 1 (reliable without restriction) |  |  |
| Rationale for reliability incl. deficiencies | OECD Guideline study under GLP conditions |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | KUREHA SPECIAL LABORATORY CO.,LTD. | 2005 | Daphnia magna, Reproduction Test of 3,4-dimethylaniline |  | KUREHA SPECIAL LABORATORY CO.,LTD. |  | Ministry of the Environment, Japan | 2004-82 | 2005-07-20 |

Materials and methods

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | OECD Guideline 211 (Daphnia magna Reproduction Test) |  |

GLP compliance

yes

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 95-64-7 |
| EC number | 202-437-4 |
| EC name | 3,4-xylidine |
| IUPAC name | 3,4-dimethylaniline |

Details on test material

- Appearance: slightly bluish brown crystal - Purity: 100.0% - Lot/batch No.: CEN0677 - Storage condition of test material: In a refrigerator - Stability under the storage conditions: The test substance was stable under the storage conditions, because IR spectra of the test substance before and after the study were identical.

Analytical monitoring

yes

Details on sampling

- Frequency: At before and after renewal of water, total 6 times during exposure - Sampling volume: 13 mL

Details on analytical methods

PRETREATMENT OF TEST SOLUTION Samples were diluted with pure water. ANALYTICAL CONDITIONS - Instrument: HPLC-UV - Column: Mightysil RP-18GP (150mm×4.6mm I.D.) - Column temp.: 40 degreeC - Eluent: Acetonitril/Pure water (65/35) - Flow rate: 1.0 mL/min - Measurement wavelength: UV 210 nm - Injection volume: 50 μL - Determination limit: 0.02 mg/L - Detection limit: 0.0004 mg/L

Vehicle

no

Details on test solutions

20mg of the test substance was dissolved with dilution water to produce a stock solution of 100 mg/L . Test solution was prepared in each test vessel by mixing the appropriate volume of the stock solution with dilution water. Dilution water was used for the control group.

Test organisms

Test organisms (species)

Daphnia magna

Details on test organisms

TEST ORGANISM - Common name: Water flea - Source: Subcultured in the testing laboratory, Originally from the National Institute for Environmental Studies (Japan) on 5 February 1997 - Age at study initiation: less than 24 hours old - Feeding during test Food type: Chlorella vulgaris Amount: 0.15 - 0.2 mg C (Organic Carbon)/ day / animal ACCLIMATION OF PARENTAL DAPHNIDS - Acclimation period: 2005/5/23-2005/6/15 (23 days-old) - Acclimation conditions: See below - Breeding water: Dilution water - Population density: 30 daphnids/2 L breeding water - Temperature: 20+/-1 degreeC - Photoperiod: 16-hr Light / 8-hr Dark, Room light - Type and amount of food: Type: Chlorella vulgaris Amount: 0.15-0.2 mgC(Organic Carbon)/animal/day - Presence of male and ephippia were not observed. - Health during acclimation: Mortality of parental daphnids for 2 weeks prior to the test were less than 5 %.

Study design

Test type

semi-static

Water media type

freshwater

Limit test

no

Total exposure duration

21 d

Test conditions

Hardness

232 - 247 mg/L (as CaCO3)

Test temperature

20.2 - 20.4 degreeC

pH

7.6 - 8.0

Dissolved oxygen

7.5 - 8.4 mg/L

Nominal and measured concentrations

- Nominal concentrations: 0, 0.010, 0.032, 0.10, 0.32 and 1.0 mg/L - Measured concentrations (Arithmetic mean): 0, 0.0095, 0.0297, 0.0988, 0.324 and 1.03 mg/L

Details on test conditions

TEST SYSTEM - Test vessel: - Type: closed - Material: Glass - Fill volume: 110 mL - Test solution volume: 80 mL/vessel - No. of organisms per vessel: 1 - No. of vessels per concentration (replicates): 10 - No. of vessels per control (replicates): 10 TEST MEDIUM / WATER PARAMETERS - Source/preparation of dilution water: Elendt M4 Medium recommended by OECD Guideline 211 used as dilution water OTHER TEST CONDITIONS - Adjustment of pH: No - Photoperiod: 16 hours light / 8 hours dark - Light intensity: Room light OBSERVATION AND MEASUREMENTS - Parental daphnids: The numbers of the living and the dead parent daphnia and immobility were recorded daily during the exposure. - Juveniles: - No. of living and dead juveniles - No. of aborted eggs and ephippia (observed daily) - Time (day) to the first brood production Juveniles were removed after recording - Test solutions: Dissolved oxygen concentration, pH, water temperature and hardness of the test solutions were measured at the before and after renewal of water at once a week (total 3 times).

Reference substance (positive control)

no

Any other information on materials and methods incl. tables

Table 1-1 Measured Concentrations of the Test Substance in Test Water during 21-day Exposure Period Nominal Concentration (mg/L); Date →; Measured Concentration (mg/L); MeanaMeasured Concentration (mg/L); % of Nominal. 0; 2; 9; 12; 19; 21. new; old; new; old; new; old. Control; <0.002; <0.002; <0.002; <0.002; <0.002; <0.002; －; －. 0.010; 0.0096; 0.0096; 0.01; 0.0089; 0.0098; 0.0092; 0.0095; 95. 0.032; 0.0301; 0.0323; 0.0335; 0.0305; 0.0271; 0.025; 0.0297; 93. 0.10; 0.0988; 0.102; 0.105; 0.0966; 0.0996; 0.0907; 0.0988; 99. 0.32; 0.323; 0.33; 0.335; 0.328; 0.323; 0.306; 0.324; 101. 1.0; 0.991; 1.01; 1.06; 1.05; --; --; 1.03; 103. new: Freshly prepared test solution, old: Old test solution before renewal, a: Arithmetic mean, －: Not calculated, --: All daphnia were dead at this observation time Table 1-2 Measured Concentrations as a Percentage of Nominal Nominal Concentration (mg/L); MeanaMeasured Concentration (mg/L); Date →; Measured Concentration as Percentage of Nominal. 0; 2; 9; 12; 19; 21. new; old; new; old; new; old. Control; -; -; -; -; -; -; -. 0.010; 0.0095; 96; 96; 100; 89; 98; 92. 0.032; 0.0297; 94; 101; 105; 95; 85; 77. 0.10; 0.0988; 99; 102; 105; 97; 100; 91. 0.32; 0.324; 101; 103; 105; 103; 101; 96. 1.0; 1.03; 99; 101; 106; 105; --; --. new: Freshly prepared test solution, old: Old test solution before renewal, a: Arithmetic mean, －: Not calculated, --: All daphnia were dead at this observation time Table 2. Temperature Nominal Concentration (mg/L); MeanaMeasured Concentration (mg/L); Temperature(℃). Date→; 0; 2; 9; 12; 19; 21; Min.; Max.. new; old; new; old; new; old. Control; －; 20.4; 20.3; 20.3; 20.2; 20.4; 20.4; 20.2; 20.4. 0.010; 0.0095; 20.4; 20.3; 20.3; 20.2; 20.4; 20.3; 20.2; 20.4. 0.032; 0.0297; 20.4; 20.3; 20.3; 20.2; 20.4; 20.3; 20.2; 20.4. 0.10; 0.0988; 20.4; 20.3; 20.3; 20.2; 20.4; 20.3; 20.2; 20.4. 0.32; 0.324; 20.4; 20.3; 20.3; 20.2; 20.4; 20.3; 20.2; 20.4. 1.0; 1.03; 20.4; 20.3; 20.3; 20.2; --; --; 20.2; 20.4. Total; 20.2; 20.4. new: Freshly prepared test solution, old: Old test solution before renewal, a: Arithmetic mean, －: Not calculated, --: All daphnia were dead at this observation time Table 3. Dissolved Oxygen Concentration(D.O.) Nominal Concentration (mg/L); MeanaMeasured Concentration (mg/L); D.O. (mg/L). Date→; 0; 2; 9; 12; 19; 21; Min.; Max.. new; old; new; old; new; old. Control; －; 8.4; 8.2; 8.3; 7.7; 8.4; 7.7; 7.7; 8.4. 0.010; 0.0095; 8.4; 8.2; 8.3; 7.7; 8.4; 7.6; 7.6; 8.4. 0.032; 0.0297; 8.4; 8.1; 8.3; 7.7; 8.4; 7.7; 7.7; 8.4. 0.10; 0.0988; 8.4; 8.1; 8.3; 7.6; 8.4; 7.7; 7.6; 8.4. 0.32; 0.324; 8.4; 8.1; 8.3; 7.6; 8.4; 7.6; 7.6; 8.4. 1.0; 1.03; 8.4; 8.1; 8.3; 7.5; --; --; 7.5; 8.4. Total; 7.5; 8.4. new: Freshly prepared test solution, old: Old test solution before renewal, a: Arithmetic mean, －: Not calculated, --: All daphnia were dead at this observation time Table 4. pH Values Nominal Concentration (mg/L); MeanaMeasured Concentration (mg/L); pH. Date→; 0; 2; 9; 12; 19; 21; Min.; Max.. new; old; new; old; new; old. Control; －; 7.9; 7.9; 8.0; 7.8; 7.9; 7.7; 7.7; 8.0. 0.010; 0.0095; 7.9; 7.8; 8.0; 7.7; 7.9; 7.6; 7.6; 8.0. 0.032; 0.0297; 7.9; 7.8; 8.0; 7.7; 7.9; 7.7; 7.7; 8.0. 0.10; 0.0988; 7.9; 7.8; 8.0; 7.7; 7.9; 7.7; 7.7; 8.0. 0.32; 0.324; 7.9; 7.8; 8.0; 7.7; 7.9; 7.6; 7.6; 8.0. 1.0; 1.03; 7.9; 7.8; 8.0; 7.7; --; --; 7.7; 8.0. Total; 7.6; 8.0. new: Freshly prepared test solution, old: Old test solution before renewal, a: Arithmetic mean, －: Not calculated, --: All daphnia were dead at this observation time Table 5.Total Hardness(as CaCO3) Nominal Concentration (mg/L); MeanaMeasured Concentration (mg/L); Total Hardness(as CaCO3,mg/L). Date→; 0; 2; 9; 12; 19; 21; Min.; Max.. new; old; new; old; new; old. Control; －; 236; 241; 244; 244; 237; 242; 236; 244. 0.010; 0.0095; 236; 241; 245; 247; 239; 240; 236; 247. 0.032; 0.0297; 237; 243; 246; 242; 238; 245; 237; 246. 0.10; 0.0988; 234; 245; 245; 246; 241; 242; 234; 246. 0.32; 0.324; 235; 244; 247; 245; 239; 247; 235; 247. 1.0; 1.03; 232; 244; 247; 245; --; --; 232; 247. Total; 232; 247. new: Freshly prepared test solution, old: Old test solution before renewal, a: Arithmetic mean, －: Not calculated, --: All daphnia were dead at this observation time

Results and discussions

Effect concentrations

| Duration | Endpoint | Effect conc. | Nominal/Measured | Conc. based on | Basis for effect | Remarks (e.g. 95% CL) |
| --- | --- | --- | --- | --- | --- | --- |
| 21 d | EC50 | 0.0457 mg/L | meas. (arithm. mean) | test mat. | reproduction | 95%CL: 0.0412-0.0509 mg/L, Logit |
| 21 d | NOEC | 0.0095 mg/L | meas. (arithm. mean) | test mat. | reproduction |  |
| 21 d | LOEC | 0.0297 mg/L | meas. (arithm. mean) | test mat. | reproduction |  |

Details on results

- Mortality of parent animals: The mortality of the parent animals in the control was 0% at the end of the test, which meets the criteria for validity of the test (i.e. not more than 20%). - Time to first brood release or time to hatch: 7-9 days at the control group - The mean cumulative number of juveniles produced per parent animal: 145.5 at the control group (meets the criteria for validity of the test (i.e. ？60)) - Ephippia: None. A few aborted eggs and dead offsprings were observed at all exposure groups.

Any other information on results incl. tables

Table 6-1. Cumulative Numbers of Dead Parental Daphnia Nominal Concentration (mg/L); MeanaMeasured Concentration (mg/L); Days. 0; 1; 2; 3; 4; 5; 6; 7; 8; 9; 10; 11; 12; 13; 14; 15; 16; 17; 18; 19; 20; 21. Control; －; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0. 0.010; 0.0095; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0. 0.032; 0.0297; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0. 0.10; 0.0988; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0. 0.32; 0.324; 0; 0; 3; 3; 3; 3; 4; 4; 4; 4; 4; 4; 4; 4; 4; 4; 4; 4; 4; 4; 4; 4. 1.0; 1.03; 0; 0; 6; 7; 9; 9; 9; 9; 9; 9; 9; 9; 9; 10; 10; 10; 10; 10; 10; 10; 10; 10. a: Arithmetic mean, －: Not calculated Table 6-2. Mortality (%) of Parental Daphnia Nominal Concentration (mg/L); MeanaMeasured Concentration (mg/L); Days. 1; 2; 4; 7; 14; 21. Control; -; 0; 0; 0; 0; 0; 0. 0.010; 0.0095; 0; 0; 0; 0; 0; 0. 0.032; 0.0297; 0; 0; 0; 0; 0; 0. 0.10; 0.0988; 0; 0; 0; 0; 0; 0. 0.32; 0.324; 0; 30; 30; 40; 40; 40. 1.0; 1.03; 0; 60; 90; 90; 100; 100. a: Arithmetic mean, －: Not calculated Table 7.Time (Days) to First Brood Production No.; Nominal Concentration, mg/L (Meana Measured Concentration, mg/L ). Control; 0.010; 0.03; 0.10; 0.32; 1.0. (-); (0.0095); (0.0297); (0.0988); (0.324); (1.03). 1; 7; 8; 8; --; 18; D. 2; 8; 8; 8; --; D; D. 3; 8; 8; 8; 20; D; D. 4; 8; 8; 8; --; 11; D. 5; 8; 8; 8; 14; --; D. 6; 8; 8; 8; --; 8; D. 7; 8; 8; 8; 17; 14; D. 8; 8; 8; 8; --; 17; D. 9; 9; 8; 9; --; D; D. 10; 8; 8; 8; --; D; D. Min; 7; 8; 8; 14; 8; －. Max; 9; 8; 9; 20; 18; －. a: Arithmetic mean, －: Not calculated, D: Parental daphnia was dead, --: Brood production was not observed Table 8. Mean Cumulative Numbers of Juveniles Produced per Adult Alive for 21 Days (ΣF1/P) Nominal Concentration (mg/L); Days. 7; 8; 9; 10; 11; 12; 13; 14; 15; 16; 17; 18; 19; 20; 21. Control; 1.0; 8.0; 9.8; 16.6; 29.9; 33.3; 50.7; 63.3; 67.2; 85.8; 99.2; 103.3; 120.4; 140.9; 145.5. 0.010; 0.0; 8.5; 8.5; 10.4; 28.4; 28.4; 31.7; 61.1; 61.1; 65.4; 97.1; 97.1; 101.8; 138.3; 138.3. 0.032; 0.0; 6.7; 6.8; 8.1; 25.3; 25.3; 27.8; 55.6; 55.6; 58.5; 90.6; 90.6; 94.4; 131.0; 131.0. 0.10; 0.0; 0.0; 0.0; 0.0; 0.0; 0.0; 0.0; 0.2; 0.2; 0.2; 0.3; 0.3; 0.3; 0.4; 0.4. 0.32; 0.0; 0.3; 0.3; 0.3; 0.5; 0.5; 0.5; 1.0; 1.0; 1.0; 1.2; 1.3; 1.3; 1.8; 1.8. 1.0; -; -; -; -; -; -; -; -; -; -; -; -; -; -; -. －: Not calculated Table 9. Cumulative Numbers of Juveniles Produced per Adult Alive for 21 Days in Each Test Vessel and Result of Statistical Comparison of the Mean Values (by Dunnett’s Multicomparison Test) Vessel No.; Nominal Concentration, mg/L. (Meana Measured Concentration, mg/L). Control; 0.010; 0.032; 0.10; 0.32; 1.0. (-); (0.0095); (0.0297); (0.0988); (0.324); (1.03). 1; 141; 146; 140; 0; 1; D. 2; 146; 136; 127; 0; D; D. 3; 143; 144; 146; 1; D; D. 4; 141; 146; 128; 0; 3; D. 5; 129; 130; 132; 2; 0; D. 6; 152; 127; 132; 0; 4; D. 7; 131; 142; 135; 1; 1; D. 8; 154; 145; 130; 0; 2; D. 9; 178; 136; 106; 0; D; D. 10; 140; 131; 134; 0; D; D. Mean; 145.5; 138.3; 131; 0.4; 1.8; -. S.D.; 13.9; 7.2; 10.5; 0.7; 1.5; -. Inhibition rate(%); 4.9; 10; 99.7; 98.7; -. Significant difference; N.S; p<0.01; p<0.01; p<0.01; -. a: Arithmetic mean, －: Not calculated, D: Not included for calculation because the parental Daphnia was dead during 21-day testing period. N.S: Indicate no-significant differece

## Endpoint study record: Kuhn et al., 1989b (Long-term toxicity to aquatic invertebrates)

| UUID |  | IUC5-9050f1e1-c6e8-4594-afc0-25de8d090397 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | CERI_M.SEKIZAWA / Chemicals Evaluation and Research Institute (CERI) / Tokyo / Japan |
| Date |  | 2012-11-29 15:14:08 JST |
| Remarks |  |  |

Administrative Data

| Purpose flag | key study |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Test procedure according to national standards |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Kuhn, R., M. Pattard, K.D. Pernak, and A. Winter | 1989 | Results of the Harmful Effects of Water Pollutants to Daphnia magna in the 21 Day Reproduction Test | Water Res. 23(4):501-510 |  |  |  |  |  |
| other: | Kuhn | 1988 | Schadstoffwirkungen von Umweltchemikalien im Daphnien-Reproduktions-Test als Grundlage fr die Bewertung der Umweltgefhrlichkeit in Aquatischen Sys. | Forschungsbericht 10603052, Mrz |  |  |  |  |  |

Materials and methods

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | other guideline: Provisional procedure proposed by Federal Environmental Agency (Umweltbundesamt) |  |

GLP compliance

no data

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 95-64-7 |
| EC number | 202-437-4 |
| EC name | 3,4-xylidine |
| IUPAC name | 3,4-dimethylaniline |

Test organisms

Test organisms (species)

Daphnia magna

Details on test organisms

- IRCHA strain - 24-h old daphnid

Study design

Test type

semi-static

Water media type

freshwater

Total exposure duration

21 d

Test conditions

Test temperature

25+/-1 degreeC

Nominal and measured concentrations

nominal concentration: between 0.016 and 2.0 mg/L

Details on test conditions

- Renewal rate of test solution: 3 times a week - Dead parent animals or those incapable of swimming were removed. - The offspring were counted and the total number for each test vessel was recored. - The pH value and the oxygen concentration were measured in two test vessels per concentration level. - Photoperiod: from 7a.m. to 4p.m. - Light intensity: fluorescent lamp, Philips TL 40/25W

Results and discussions

Effect concentrations

| Duration | Endpoint | Effect conc. | Nominal/Measured | Conc. based on | Basis for effect | Remarks (e.g. 95% CL) |
| --- | --- | --- | --- | --- | --- | --- |
| 21 d | NOEC | 0.01 mg/L | meas. (not specified) | test mat. | reproduction |  |

Details on results

Because the chemical analysis showed a loss of test substance of more than 20%, the NOEC were derived based on measured concentrations.

# 6.1.5 Toxicity to aquatic algae and cyanobacteria

## Endpoint study record: MOE, 2005c (Toxicity to aquatic algae and cyanobacteria)

| UUID |  | IUC5-06a9e9b7-d739-4829-8142-0fe92d713825 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V3.0 Plug-In |
| Date |  | 2012-02-27 13:42:36 JST |
| Remarks |  | Successfully migrated to IUCLID 5.4 format. |

Administrative Data

| Purpose flag | key study; robust study summary |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result | Study period | 18 March 2005 - 20 July 2005 |
| Reliability | 1 (reliable without restriction) |  |  |
| Rationale for reliability incl. deficiencies | Nationl Guideline study under GLP conditions |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | KUREHA SPECIAL LABORATORY CO.,LTD. | 2005 | Alga (Pseudokirchneriella subcapitata), Growth Inhibition Test of 3,4-dimethylaniline |  | KUREHA SPECIAL LABORATORY CO.,LTD. |  | Ministry of the Environment, Japan | 2004-80 | 2005-07-20 |

Materials and methods

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| equivalent or similar to | OECD Guideline 201 (Alga, Growth Inhibition Test) |  |

GLP compliance

yes

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 95-64-7 |
| EC number | 202-437-4 |
| EC name | 3,4-xylidine |
| IUPAC name | 3,4-dimethylaniline |

Details on test material

- Appearance: slightly bluish brown crystal - Purity: 100.0% - Lot/batch No.: CEN0677 - Storage condition of test material: In a refrigerator - Stability under the storage conditions: The test substance was stable under the storage conditions, because IR spectra of the test substance before and after the study were identical.

Analytical monitoring

yes

Details on sampling

- Frequency: at the start of the exposure (0 hour) and at the termination of the exposure (72 hours after exposure) - Sampling volume: At the start of the exposure: Samples were taken from the container for preparation in each exposure level At the end of the exposure: Equal volume of samples were taken from each vessels per concentration, and removed algae by centrifugation.

Details on analytical methods

PRETREATMENT OF TEST SOLUTION Samples were diluted with pure water. ANALYTICAL CONDITIONS - Instrument: HPLC-UV - Column: Mightysil RP-18GP (150mm×4.6mm I.D.) - Column temp.: 40 degreeC - Eluent: Acetonitril/Pure water (65/35) - Flow rate: 1.0 mL/min - Measurement wavelength: UV 210 nm - Injection volume: 50 μL - Determination limit: 0.03 mg/L - Detection limit: 0.009 mg/L

Vehicle

no

Details on test solutions

25mg of the test substance was dissolved with dilution water to produce a stock solution of 100 mg/L . Test solution was prepared in each test vessel by mixing the appropriate volume of the stock solution with dilution water. Dilution water was used for the control group.

Test organisms

Test organisms (species)

Pseudokirchnerella subcapitata

Details on test organisms

TEST ORGANISM - Strain: ATCC22662 - Source: Originally from American Type Culture Collection on 13 November 1997 PRECULTURE - Period: from 2005/6/4 to 2005/6/7 - Culturing media and conditions: Same as test OTHER - Confirming the reproducibility of the test conditions: - Reference substance: 3,5-dichlorophenol (purity: 98.0%) - Results: 72-hr ErC50 = 0.86 mg/L ( background data (0.89 mg/L) in this laboratory)

Study design

Test type

static

Water media type

freshwater

Limit test

no

Total exposure duration

| 72 h |  |
| --- | --- |
| Remarks | Shake culture (100 rpm) |

Test conditions

Test temperature

23.0 degreeC

pH

At the start of exposure: 7.8 - 8.0 At the end of exposure: 8.0-8.9

Nominal and measured concentrations

- Nominal concentrations: 0, 1.8, 3.2, 5.6, 10 and 18 mg/L - Measured concentrations (Geometric mean): 0, 1.60, 2.94, 5.18, 9.46 and 16.9 mg/L

Details on test conditions

TEST SYSTEM - Test vessel: - Type: erlenmeyer flask with silicon rubber plug - Fill volume: 300 mL - Test solution volume: 100 mL/vessel - Initial cells density: 0.5 × 10^4 cells/mL - No. of vessels per concentration: 3 - No. of vessels per control: 6 GROWTH MEDIUM - Standard medium used: yes (Sterile OECD medium) OTHER TEST CONDITIONS - Sterile test conditions: yes - Photoperiod: Continuous - Light intensity: 60-120 uE/m2/s OBSERVATION - Cell density: Counted with a particle counter at 24, 48, and 72 h - Cell condition: Microscopic observation were conducted at 72 h - Test solutions: The pH of the test solution was measured at the start and the end of the exposure. The temperature in the incubator and light intensity were measured once a day during the exposure.

Reference substance (positive control)

no

Any other information on materials and methods incl. tables

Table 1. Measured Concentration of the Test Substance in Test Water Nominal Concentration (mg/L); Measured Concentration (mg/L). 0 Hour; Percent of Nominal; 72 Hours; Percent of Nominal; MeanaMeasured Concentration (mg/L). Control; <0.03; -; <0.03; -; -. 1.8; 1.80; 100; 1.42; 79; 1.600. 3.2; 3.25; 102; 2.66; 83; 2.94. 5.6; 5.43; 97; 4.95; 88; 5.18. 10; 9.32; 93; 9.60; 96; 9.46. 18; 17.0; 94; 16.8; 93; 16.9. a: Geometric mean, -: Not calculated Table 2. Daily Temperature, Light Intensity and Revolution at the Incubation Exposure Period (Hours); Temperature (℃); Light Intensity* (μE/m2/s); Revolution (rpm). 0; 23.0; 66-76; 100. 24; 23.0; 65-75; 100. 48; 23.0; 66-76; 100. 72; 23.0; 66-76; 100. *: Calibrated from light-measuring instruments in Lx (factor:0.013) Table 3. pH Values Nominal Concentration (mg/l); MeanaMeasured Concentration (mg/L); pH. 0 Hour; 72 Hours. Control; -; 1; 8.0; 8.8. 2; 8.8. 3; 8.7. 4; 8.7. 5; 8.8. 6; 8.4. 1.8; 1.60; 1; 7.9; 8.9. 2; 8.3. 3; 8.7. 3.2; 2.94; 1; 7.9; 8.7. 2; 8.8. 3; 8.7. 5.6; 5.18; 1; 7.8; 8.0. 2; 8.0. 3; 8.1. 10; 9.46; 1; 7.9; 8.0. 2; 8.0. 3; 8.0. 18; 16.9; 1; 7.8; 8.0. 2; 8.0. 3; 8.0. a: Geometric mean, -: Not calculated

Results and discussions

Effect concentrations

| Duration | Endpoint | Effect conc. | Nominal/Measured | Conc. based on | Basis for effect | Remarks (e.g. 95% CL) |
| --- | --- | --- | --- | --- | --- | --- |
| 72 h | EC50 | 8.59 mg/L | meas. (geom. mean) | test mat. | growth rate | 95%CL: 8.03-9.22 mg/L, Probit |
| 72 h | NOEC | 2.94 mg/L | meas. (geom. mean) | test mat. | growth rate |  |
| 72 h | EC50 | 4.62 mg/L | meas. (geom. mean) | test mat. | biomass | 95%CL: 4.37-4.88 mg/L, Probit |
| 72 h | NOEC | 2.94 mg/L | meas. (geom. mean) | test mat. | biomass |  |

Details on results

CELL GROWTH - Control group: Increased to 390 times of initial cell density at the end of exposure, meets the validity of the test (at least 16 times growth)

Any other information on results incl. tables

Table 4.Cell Densities of Pseudokirchneriella subcapitata during the 72-hour Exposure Nominal Concentration (MeanaMeasured Conc.) (mg/l); Vessel No.; Cell Density (×104cells/mL). 0 Hour; 24 Hours; 48 Hours; 72 Hours. Control (-); 1; 0.50; 3.78; 29.20; 191.00. 2; 0.50; 3.55; 30.15; 210.50. 3; 0.50; 3.87; 28.25; 186.50. 4; 0.50; 3.51; 26.65; 201.50. 5; 0.50; 3.39; 26.60; 193.50. 6; 0.50; 3.19; 24.70; 185.50. Average; 0.50; 3.55; 27.59; 194.75. SD; 0.00; 0.25; 1.99; 9.62. 1.8 (1.60); 1; 0.50; 3.48; 28.85; 189.00. 2; 0.50; 3.45; 27.40; 202.50. 3; 0.50; 3.16; 24.95; 171.50. Average; 0.50; 3.36; 27.07; 187.67. SD; 0.00; 0.18; 1.97; 15.54. 3.2 (2.94); 1; 0.50; 3.29; 27.15; 191.50. 2; 0.50; 3.34; 25.65; 183.00. 3; 0.50; 3.37; 24.35; 179.00. Average; 0.50; 3.33; 25.72; 184.50. SD; 0.00; 0.04; 1.40; 6.38. 5.6 (5.18); 1; 0.50; 3.04; 13.95; 78.95. 2; 0.50; 2.71; 7.09; 27.45. 3; 0.50; 2.97; 11.90; 51.20. Average; 0.50; 2.91; 10.98; 52.53. SD; 0.00; 0.17; 3.52; 25.78. 10 (9.46); 1; 0.50; 1.73; 2.35; 2.43. 2; 0.50; 2.12; 2.96; 2.96. 3; 0.50; 1.85; 2.23; 2.40. Average; 0.50; 1.90; 2.51; 2.60. SD; 0.00; 0.20; 0.39; 0.32. 18 (16.9); 1; 0.50; 1.57; 2.48; 2.28. 2; 0.50; 1.47; 1.74; 1.75. 3; 0.50; 1.50; 1.82; 2.54. Average; 0.50; 1.51; 2.01; 2.19. SD; 0.00; 0.05; 0.41; 0.40. SD : Standard deviation, a: Geometri mean, -: Not calculated Table 5. Percentage Inhibition of Pseudokirchneriella subcapitata Nominal Concentration (MeanaMeasured Conc.); Growth Rate; Area Under the Growth Curve. mg/L; Vessel No.; Rate μ(0-72); Inhibition (%)*1 Iμ(0-72); Area A(0-72); Inhibition (%)*1 IA(0-72). Control (-); 1; 1.9818; 3053.5. 2; 2.0142; 3304.8. 3; 1.9739; 2978.9. 4; 1.9996; 3111.8. 5; 1.9861; 3011.8. 6; 1.9721; 2865.4. Average; 1.988; 0.00; 3054.4; 0.00. SD; 0.0162; 147.8. 1.8 (1.60); 1; 1.9783; 3013.9. 2; 2.0013; 3140.4. 3; 1.9459; 2702.6. Average; 1.9752; 0.64; 2952.3; 3.34. SD; 0.0278; 225.3. 3.2 (2.94); 1; 1.9827; 2998.6. 2; 1.9675; 2861.8. 3; 1.9602; 2783.3. Average; 1.9701; 0.90; 2881.2; 5.67. SD; 0.0115; 108.9. 5.6 (5.18); 1; 1.6873; 1325.2. 2; 1.3352; 534.6. 3; 1.5430; 941.3. Average; 1.5218; 23.45**; 933.7; 69.43**. SD; 0.1770; 395.3. 10 (9.46); 1; 0.527; 97.1. 2; 0.5928; 127.4. 3; 0.5229; 96.7. Average; 0.5476; 72.46**; 107.1; 96.49**. SD; 0.0392; 17.6. 18 (16.9); 1; 0.5058; 94.6. 2; 0.4176; 68.0. 3; 0.5418; 80.2. Average; 0.4884; 75.43**; 80.9; 97.35**. SD; 0.0639; 13.3. *1 Values are the percent inhibition relative to the control. SD Standard deviation ** Significant difference p<0.01 a: Geometric mean -: Not calculated

## Endpoint study record: Kuhn and Pattard, 1990

| UUID |  | IUC5-41d5683f-d7f6-4d47-a58a-dad173c29c29 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | CERI_M.SEKIZAWA / Chemicals Evaluation and Research Institute (CERI) / Tokyo / Japan |
| Date |  | 2012-11-29 14:21:45 JST |
| Remarks |  |  |

Administrative Data

| Purpose flag | supporting study |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Test procedure according to national standards (DIN) |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Kuhn, R., and M. Pattard | 1990 | Results of the Harmful Effects of Water Pollutants to Green Algae (Scenedesmus subspicatus) in the Cell Multiplication Inhibition Test | Water Res. 24 (1):31-38 |  |  |  |  |  |

Materials and methods

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | other guideline: DIN 38 412, Part 9 (draft standard) |  |

GLP compliance

no data

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 95-64-7 |
| EC number | 202-437-4 |
| EC name | 3,4-xylidine |
| IUPAC name | 3,4-dimethylaniline |

Test organisms

Test organisms (species)

Desmodesmus subspicatus (reported as Scenedesmus subspicatus)

Details on test organisms

- Strain number: 8681 SAG

Study design

Test type

static

Water media type

freshwater

Total exposure duration

48 h

Test conditions

Nominal and measured concentrations

- Tested concentration range: 1.6-200 mg/L

Details on test conditions

- Test vessel: 300 ml Erlenmeyer flasks with metal caps - Photoperiod: constant lighting - Light intensity and quality: Osram L25/40 flourescent tubes, irradiance E=17.0 W/m2

Results and discussions

Effect concentrations

| Duration | Endpoint | Effect conc. | Nominal/Measured | Conc. based on | Basis for effect | Remarks (e.g. 95% CL) |
| --- | --- | --- | --- | --- | --- | --- |
| 48 h | EC10 | 17 mg/L |  | test mat. | growth rate |  |
| 48 h | EC50 | 50 mg/L |  | test mat. | growth rate |  |

# 6.1.7 Toxicity to microorganisms

## Endpoint study record: Arnold et al., 1990 (Toxicity to microorganisms)

| UUID |  | IUC5-01f40fd1-bbc1-40a5-af8f-719b28437ca8 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V3.0 Plug-In |
| Date |  | 2012-06-07 09:51:02 JST |
| Remarks |  | Successfully migrated to IUCLID 5.4 format. |

Administrative Data

| Study result type | experimental result |  |  |
| --- | --- | --- | --- |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Acceptable, well-documented publication report which meets basic scientific principles. |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Arnold, L.M., D.T.Lin, and T.M.Schultz | 1990 | QSAR for Methyl- and/or Chloro-Substituted Anilines and the Polar Narcosis Mechanism of Toxicity. | Chemosphere. 21, 183-191 |  |  |  |  |  |

Data access

data published

Materials and methods

Principles of method if other than guideline

The 48-h sublethal population growth assay under static conditions for Tetrahymena pyriformis.

GLP compliance

no data

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Details on test material

Purity >95%

Analytical monitoring

yes

Details on sampling

Sampling time: At 0 and 48 hours

Details on analytical methods

HPLC

Vehicle

no data

Test organisms

Test organisms (species)

Tetrahymena pyriformis

Study design

Test type

static

Total exposure duration

48 h

Test conditions

Nominal and measured concentrations

Nominal concenrations: 5- to 10-step graded concentration series using freshly prepared stock solutions.

Details on test conditions

Test substance was tested in duplicate for minimum of three replicates following range finding experiments. Clutures without test substance served as controls. Cultures were reared in 250 mL Erlenmeyer flasks containing 50 mL of proteose-peptone medium. Cell population levels were estimated spectrophotometrically as absorbance at 540 nm. Only replicates with control absorbance values of 0.6-0.9, equivalent to late log-growth-phase, were used in the analyses. The IGC50, 50% growth inhibitory concentration, and 95% fiducial limits were determined for test substance using probit analysis of Statistical Analysis Systems.

Results and discussions

Effect concentrations

| Duration | Endpoint | Effect conc. | Nominal/Measured | Conc. based on | Basis for effect | Remarks (e.g. 95% CL) |
| --- | --- | --- | --- | --- | --- | --- |
| 48 h | other: IGC50 | 235 mg/L |  | test mat. | growth inhibition |  |

Details on results

Log IGC50 -1 = -0.287 mmol/L MW=121.18 IGC50=1.936mmol/L=235 mg/L

## Endpoint study record: Kaiser and Palabrica, 1991 (Toxicity to microorganisms)

| UUID |  | IUC5-fb8ea39f-e334-41b6-b917-b875ea1241b3 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V3.0 Plug-In |
| Date |  | 2012-06-07 13:35:01 JST |
| Remarks |  | Successfully migrated to IUCLID 5.4 format. |

Administrative Data

| Study result type | experimental result |  |  |
| --- | --- | --- | --- |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Acceptable, well-documented publication report which meets basic scientific principles. |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Kaiser, K.L.E., and V.S. Palabrica | 1991 | Photobacterium phosphoreum Toxicity Data Index. | Water Pollut. Res. J. Can., 26(3), 361-431 |  |  |  |  |  |

Data access

data published

Materials and methods

Principles of method if other than guideline

Inhibition luminescence of marine luminescent bacteria

GLP compliance

no data

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Details on test material

Purity: 98%

Analytical monitoring

yes

Test organisms

Test organisms (species)

Photobacterium phosphoreum

Study design

Total exposure duration

| 30 min |  |
| --- | --- |
| Remarks | 5, 15 and 30 min |

Test conditions

Test temperature

15°C

pH

5-9

Results and discussions

Effect concentrations

| Duration | Endpoint | Effect conc. | Nominal/Measured | Conc. based on | Basis for effect | Remarks (e.g. 95% CL) |
| --- | --- | --- | --- | --- | --- | --- |
| 30 min | EC50 | 0.963 mg/L | meas. (not specified) | test mat. | other: luminescence inhibition |  |

Details on results

EC50: 50% reduction of the light emission relative to controls at the exposure time.

# 7 Toxicological information

# 7.1 Toxicokinetics, metabolism and distribution

# 7.1.1 Basic toxicokinetics

## Endpoint study record: Cauchon D & Krishnan K (1997)

| UUID |  | IUC5-92becedf-5761-470e-ae58-8c77508c6fe4 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | mariko / National Institute of Health Sciences / Tokyo / Japan |
| Date |  | 2013-02-06 15:20:49 JST |
| Remarks |  |  |

Administrative Data

| Reliability | 2 (reliable with restrictions) |
| --- | --- |
| Rationale for reliability incl. deficiencies | Study published in peer-reviewed journal. |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Cauchon D & Krishnan K | 1997 | In Vitro and In Vivo Evaluations of the Methaemoglobinaemic Potential of Xylidine Isomers in the Rat. | J. Appl. Toxicol., 17 (6) 397-404 |  |  |  |  |  |

Data access

data published

Materials and methods

Type of method

other: In vitro and in vivo

Objective of study

toxicokinetics

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | other guideline: No data | no data |

Principles of method if other than guideline

The methaemoglobinaemic potential of 3,4-xylidine was assessed in vitro, using red blood cells and hepatic post-mitochondrial fractions in a two-compartmental dialysis system; and in vivo, following a single oral dose of 4.8 mmol kg-1 (p.o.) of the test substance.

GLP compliance

no data

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 95-64-7 |
| EC number | 202-437-4 |
| EC name | 3,4-xylidine |

Details on test material

- Name of test material: 3,4-xylidine - Analytical purity: 98 - 99.5 %

Test animals

Species

rat

Strain

Sprague-Dawley

Sex

male

Details on test animals and environmental conditions

In vivo: TEST ANIMALS - Source: Charles River Canada Ltd. - Weight at study initiation: 200 - 220 g - Housing: Stainless steel - Diet: Ad libitum - Water: Ad libitum - Acclimation period: 4 - 7 days ENVIRONMENTAL CONDITIONS - Photoperiod (hrs dark / hrs light): 12 h dark/ 12 h light

Administration / exposure

Route of administration

other: In vivo: p.o.;

Vehicle

other: In vivo: Corn oil;

Duration and frequency of treatment / exposure

In vivo: Single application In vitro: for "intact" xylindines: 1 h; for "biotransformed" xylidines: 4 h.

Doses / concentrations

In vivo: 4.8 mmol/kg-1 (total volume 10 mL kg-1 body weight). In vitro: 1 mM for "intact" xylidines; 1, 0.3 or 0.6 mM for "biotransformed" xylidines.

No. of animals per sex per dose

In vivo: 6 males In vitro: Not applicable

Control animals

other: In vivo: Time zero methaemoglobin level served as the reference/control value for each animal; In vitro: Methanol

Details on study design

In vivo: Samples of blood were withdrawn from the tail vein of individual animals at 1, 2, 4 and 6 h following administration. Blood samples were also obtained before the administration of the test substance and analysed for methaemoglobin levels such that the level at time zero served as the reference/control value for each animal. Methaemoglobin levels relative to total haemoglobin were measured by a modification of Evelyn and Malloy's spectrophotometric technique. Samples of erythrocyte suspensions were haemolysed with distilled water and phosphate buffer containing Triton X-100. An aliquot of hte haemolysates was transferred to two cuvettes and absorbance at 630 nm was measured from cuvette 1 to generate reading R1. an aliquot of 20 % potassium ferricyanide was added to cuvette 2 and absorbance measured 5 min later to give reading R2. An aliquot of 10 % KCN was then added to both cuvettes C1 and C2 and the absorbances were measured 5 min later to obtain readings R3 and R4. The percentage of haemoglobin oxidised to methaemoglobin was calculated at 100 times the difference between R1 and R3 divided by R2 minus R4. In vitro: Methaemoglobinaemic potential of "intact" xylidines. To evaluate the methaemoglobinaemic potential of "intact" xylidines they were added directly to erythrocyte preparations. Aliquots of erythrocyte suspensions in glass vials were pre-incubated at 37 °C in a water bath for 10 min. An aliquot of each of each isomer solution prepared in methanol was added to the erythrocyte suspension and incubated for 1 h at 37 °C at the end of which the methaemoglobin levels were quantitated. Methaemoglobinaemic potential of "biotransformed" xylidines. To evaluate the methaemoglobinaemic potential of "biotransformed" xylidines the Dianrom® system was used, which consisted or a series of dialysis cells made form two tightly fitting Teflon® compartments (working volume of 1 mL per compartment) separated by a flat semi-permeable membrane (63 mm diameter, molecular weight cutoff 10000). Four sets of five each of these biocompartmental systems were mounted on a motorised support (providing constant rotation at 8 rpm) and immersed in a water bath at 37 °C. The semi-permeable membranes were conditioned by initially soaking them in distilled water for 30 min and then in phosphate buffer (pH 7.4) for at least 30 min prior to mounting them on dialysis cells. An aliquot of the erythrocyte suspension was injected into one of the compartments and an aliquot of the metabolising fraction was added to the other compartment. The metabolising fraction consisted of protein and cofactor solution. Following a pre-incubation for 10 min, the dialysis cells were removed from the water bath and an aliquot of the xylidine isomer solution prepared in methanol was quickly injected into compartment A (containing the erythrocytes) and placed into the bath for the duration of incubation (up to 4 h). At the end of the incubation period an aliquot of the erythrocyte suspension was removed for quantitation of the methaemoglobin levels.

Statistics

The data on methaemoglobin levels in each treatment group were submitted to Student's t-test with an acceptable level of significance of 0.05. The area under the percentage methaemoglobin vs time curve (AUCmetHb) was calculated according to the trapezoidal rule.

Any other information on materials and methods incl. tables

After 4 - 7 days acclimatization the rats were either used in the in vivo experiments or sacrificed by CO2 euthanasia to obtain liver and blood sampled for use in the in vitro experiments. The liver S9 fractions were prepared from individual rats immediately after their sacrifice by centrifuging the liver homogenates.

Results and discussions

Preliminary studies

No data

Pharmacokinetic studies

Details on absorption

No data

Details on distribution in tissues

No data

Details on excretion

No data

Metabolite characterisation studies

Details on metabolites

No data

Bioaccessibility

Any other information on results incl. tables

The test substance at 1 mM concentration failed to induce methaemoglobinaemia following a 1 h incubation with rat erythrocytes. However in the two-compartment in vitro system the same concentration (1 mM) induced significant methaemoglobulinaemia but only in the presence of active S9. At lower concentrations the effects were dramatically less potent (0.3 mM)or totally ineffective(0.06 mM) as inducers of methaemoglobinaemia. The percentage of methaemoglobin observed following incubation with 0.06 mM of the test substance was less than 5% suggesting that this blood concentration in vitro and in vivo is the NOAEL. The test substance failed to induce methaemoglobinaemia in vivo following a single oral dose. IN VITRO 3,4-xylidine at 1 mM concentration induced a significant (p<=0.05) increase in methaemoglobin in rat erythrocytes (1.73 ± 0.10 % compared with control value of 1.76 ± 0.03%). Although the increase was significant, the level achieved is not high enough to induce methaemoglobinaemia. As shown in Table 1, a concentration of 1 mM induced significant methaemoglobinaemia in the two compartment in vitro system. Here, the co-incubation of the rat liver S9 fraction with the erythrocytes should have facilitated the bioactivation of 3,4-xylidine and their subsequent interaction with haemoglobin. At a concentration of 0.03 mM, there was no difference with controls in the two compartment system (7.15±0.19% compared with control value of 3.90±0.10%). Similar results were found at a concentration of 0.06 mM (Table 2). Table 1: Time course of methaemoglobinaemia induced by 1mM 3,4-xylidine isomers in a two-compartment in vitro system Table 1: Time course of methaemoglobinaemia induced by 1mM 3,4-xylidine isomers in a two-compartment in vitro system ----------------------------------- Time Methaemoglobin level (hours) (as % haemoglobin)* ----------------------------------- 0.5 9.82±0.66 1 17.05±2.61 2 20.46±3.81 3 13.81±2.38 4 9.39±1.76 AUCmetHb** 56.83 ----------------------------------- *: The data represent mean±SEM (n=3) **: Area under the % methaemoglobin vs time curve (AUCmetHb) was calculated according to the trapezoidal rule ***: n=2 Table 2: Percentage methaemoglobin in rat erythrocytes at 1h and 2h following the introduction of 0.06 mM 2,6-xylidine in a two compartment in vitro system ----------------------------------------------------- Time Methaemoglobin level Control (hours) (as % haemoglobin)* ----------------------------------------------------- 1 5.84±0.37 2.90±0.27 2 3.34±0.23 2.56±0.13 ----------------------------------------------------- *: The data represent mean±SEM (n=3) IN VIVO In vivo, 3,4 xylidine did not induce methaemoglobinaemia after a single oral dose of 4.8 mmol/kg bw (Table 3). Table 3: Time course of methaemoglobinaemia in the rat following a single oral dose of 4.8 mmol/kg 2,6-xylidine ------------------------------------------- Time Methaemoglobin level (hours) (as % haemoglobin)* ------------------------------------------- 0 1.7±0.1 1 2.2±0.1 2 1.6±0.1 4 1.7±0.2 6 - ------------------------------------------- *: The data represent mean±SEM (n=3) for the time points at which blood was collected The quantitative differences between the in vivo and in vitro results may have been due to additional bioactivation pathways (N-hydroxylation or ring hydroxylation) mediated by high Km enzymes operative at the high incubation concentrations used in vitro.

Applicant's summary and conclusion

Conclusions

The methaemoglobinaemic potential of 3,4-xylidine was assessed in vitro, using red blood cells and hepatic post-mitocondrial fractions in a two-compartmental dialysis system; and in vivo, following a single oral dose of 4.8 mmol kg-1 (p.o.) of the test substance. The test substance at 1 mM concentration failed to induce methaemoglobinaemia following a 1 h incubation with rat erythtrocytes. However in the two-compartment in vitro system the same concentration(1 mM)induced significant methaemoglobulinaemia but only in the presence of active S9. At lower concentrations the effects were dramatically less potent(0.3 mM)or totally ineffective(0.06 mM) as inducers of methaemoglobinaemia. The percentage of methaemoglobin observed following incubation with 0.06 mM of the test substance was less than 5% suggesting that this blood concentration in vitro and in vivo is the NOAEL. The test substance failed to induce methaemoglobinaemia in vivo following a single oral dose.

## Endpoint study record: McLean S, Starmer GA & Thomas J (1969)

| UUID |  | IUC5-9f54ac30-d42f-454f-a30f-fa0c3f8ff14d |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | mariko / National Institute of Health Sciences / Tokyo / Japan |
| Date |  | 2013-02-06 15:20:49 JST |
| Remarks |  |  |

Administrative Data

| Reliability | 4 (not assignable) |
| --- | --- |
| Rationale for reliability incl. deficiencies | Insufficient detail in the report published in the literature. |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | McLean S, Starmer GA & Thomas J | 1969 | Methaemoglobin Formation by Aromatic Amines. | J. Pharm. Pharmac., 21, 441-450 |  |  |  |  |  |

Data access

data published

Materials and methods

Type of method

in vivo

Objective of study

toxicokinetics

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | other guideline: No data | no data |

Principles of method if other than guideline

Methaemoglobin formation induced in cats by substituted anilines was investigated in an attempt to correlate chemical structure with activity. Cats were used due to their sensitivity of methaemoglobin formation.

GLP compliance

no (Study pre-dates GLP)

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 95-64-7 |
| EC number | 202-437-4 |
| EC name | 3,4-xylidine |

Radiolabelling

no data

Details on test material

- Name of test material: 3,4-dimethylaniline

Test animals

Species

cat

Strain

no data

Sex

no data

Details on test animals and environmental conditions

TEST ANIMALS - Age at study initiation: > 24 weeks

Administration / exposure

Route of administration

intravenous

Vehicle

no data

Duration and frequency of treatment / exposure

5 h

Doses / concentrations

0.25 mmol/kg

No. of animals per sex per dose

5 animals, sex not specified.

Control animals

yes

Positive control

No data

Details on study design

Solution of 3,4-xylidine hydrochloride adjusted to pH 5.5 and made isotonic with sodium chloride. Injections made slowly into the femoral vein with a dose volume of approximately 10mL. Blood samples were taken from the femoral vein just before administration of a drug and at each hour afterwards for 5 hours. At least 5 cats and 2 dogs were used. The normal level of methaemoglobin was also measured in a population of 152 cats.

Details on dosing and sampling

No data

Statistics

No data

Results and discussions

Pharmacokinetic studies

Details on absorption

No data

Details on distribution in tissues

No data

Details on excretion

No data

Metabolite characterisation studies

Details on metabolites

Dimethylanilines formed less methaemoglobin than aniline. The level of methaemoglobin remained relatively constant over 5 h.

Bioaccessibility

Any other information on results incl. tables

Table 1: Formation of methaemoglobin after intravenous administration of 3,4-xylidine ------------------------------- Time Methaemoglobin (Hours) % ±S.E. ------------------------------- 1 11.8±2.1 2 14.1±2.5 3 15.6±301 4 16.6±3.7 5 12.2±3.3 Mean 14.1 Mean Maximum 18.0±3.4 ------------------------------- The normal level of methaemoglobin measured in a population of 152 cats was 1.1% (less than the sensitivity of the assay).

Applicant's summary and conclusion

Conclusions

Methaemoglobin formation induced in cats by substituted anilines was investigated in an attempt to correlate chemical structure with activity. Dimethylanilines formed less methaemoglobin than aniline. The level of methaemoglobin remained relatively constant over 5 h.

# 7.2 Acute Toxicity

# 7.2.1 Acute toxicity: oral

## Endpoint study record: Schafer EW & Bowles WA (1985)

| UUID |  | IUC5-efa60b37-a9e0-4720-b3f3-b2068b93432f |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | mariko / National Institute of Health Sciences / Tokyo / Japan |
| Date |  | 2013-02-06 15:20:49 JST |
| Remarks |  |  |

Administrative Data

| Study result type | experimental result |  |  |
| --- | --- | --- | --- |
| Reliability | 4 (not assignable) |  |  |
| Rationale for reliability incl. deficiencies | Insufficient detail. |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Schafer EQ Jr & Bowles WA. Jr | 1985 | Acute oral toxicity and repellency of 933 chemicals to house and deer mice. | Arch. Environ. Contam. Toxicol. 14:111 - 129. |  |  |  |  |  |

Data access

data published

Materials and methods

Test type

standard acute method

Limit test

no

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | other guideline: No data | no data |

Principles of method if other than guideline

Five individual bioassay repellency or toxicity variables were estimated or determined for deer mice and house mice.

GLP compliance

no (Study pre-dates GLP.)

Test materials

Test material identity

| Identifier | Identity |
| --- | --- |
| EC name | 3,4-xylidine |
| EC number | 202-440-0 |
| CAS number | 65-64-7 |

Details on test material

- Name of test material: 3,4-dimethylaniline

Test animals

Species

other: House mouse or deer mouse

Strain

no data

Sex

no data

Details on test animals and environmental conditions

Wild-trapped house and deer mice or domestically bred house mice were used in all test procedures.

Administration / exposure

Route of administration

oral: gavage

Vehicle

other: water, corn oil or 1 % carbopol

Details on oral exposure

No data

Doses

Repellency: 1 % or 2 % w/w

No. of animals per sex per dose

No data

Control animals

no data

Details on study design

Five bioassays were conducted resulting in six basic datasets as follows: Repellency: Three repellency test were conducted, two on deer mice and one on house mice. The initial test used five individually caged deer mice. Each was offered 25 white wheat seeds treated with 2 % (w/w) of the test substance daily for 3 days followed by 4 days of observation for gross sub-acute effects. An alternative less preferred food and water were available ad lib. Mortality and the number of wheat seeds consumed daily were recorded. The total number of treated seeds consumed by all mice over the three-day test period were subtracted from the total number available. The difference was converted into the percentage of seeds refused and the value was designated as the FR (Food Reduction). This was not repeated using Douglas fir seeds for this test substance. The third repellency test method used house mice and 25 white wheat seeds treated with 2 % of the test substance. The wheat seeds were offered to 10 individually caged mice for a five-day period along with the same alternative food used in the FR test. The test results, designated as REP were summarised as the percentage of mice refusing to eat an average of 13 or more treated seeds per day during the five-day test period. Two acute oral tests were also conducted on deer mice. The first, the approximate lethal dose (ALD) represented a range-finding study using approximately 6 animals and a graduated dosage scale. Using this single animals per level method, each succeeding treatment was 50 % higher than the preceding level and continued until mortality occurred. The test substance was administered by gavage using water, corn oil or 1 % carbopol as a carrier, followed by 3 days of observations for mortality. The second toxicity test (LD50) was conducted on a more limited basis in a similar manner, except that 2 to 4 animals were used per geometrically spaced dosage level. These tests generally required from 6 to 20 animals per experiment. The final set of toxicity data was derived from the FR value and the known average weight of individual seeds (50 mg) and the known average weight of each individual deer mouse (20 g). This calculated value , the LDfr represented the average amount of chemical in mg/kg/day that was ingested by each animal over the 3-day test period without killing more than 50 % of the test animals.

Statistics

The statistical method used to estimate the acute oral LD50 was that of Thompson (1948) and Thompson and Weil (1952).

Results and discussions

Effect levels

| Sex | Endpoint | Effect level | Based on | 95% CL | Remarks |
| --- | --- | --- | --- | --- | --- |
| male/female | other: LDfr | > 900 mg/kg/day | test mat. |  |  |

Mortality

No data

Clinical signs

No data

Body weight

No data

Gross pathology

No data

Other findings

LDfr (Amount of chemical ingested during the food reduction test which killed 50 % of the test mice): + 900 mg/kg/day. Deer mouse: Approximate lethal dose not specified Food Reduction test using 2 %-treated wheat seeds: 28.0 % Food Reduction test using Douglas fir seeds: Not specified House mouse (percentage of mice refusing to eat more than 50 % of 2 %-treated wheat): 100 %

Applicant's summary and conclusion

Interpretation of results

other: No data

Criteria used for interpretation of results

not specified

Conclusions

The LDfr (amount of chemical ingested during the food reduction test which killed 50 % of the test mice) was greater than 900 mg/kg/day.

## Endpoint study record: Vernot EH et al (1977)

| UUID |  | IUC5-885d482b-c98f-4461-b75d-39d737fe2b52 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | mariko / National Institute of Health Sciences / Tokyo / Japan |
| Date |  | 2013-02-06 15:20:49 JST |
| Remarks |  |  |

Administrative Data

| Reliability | 2 (reliable with restrictions) |
| --- | --- |
| Rationale for reliability incl. deficiencies | Acceptable, basic data given |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Vernot EH, MacEwen JD, Haun CC & Kinkead ER. | 1977 | Acute Toxicity and Skin Corrosion Data for Some Organic and Inorganic Compounds and Aqueous Solutions. | Toxicol. Appl. Pharmacol., 42, 417-423. |  |  |  |  |  |

Data access

data published

Materials and methods

Test type

standard acute method

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | other guideline: no data | no data |

Principles of method if other than guideline

Method: T03-03: Single oral dose toxicity was determined in which the LD50 and its 95 % confidence limts were estimated by the moving average technique.

GLP compliance

no (Study pre-dates GLP.)

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 67-64-7 |
| EC number | 202-440-0 |
| EC name | 3,4-xylidine |

Details on test material

IUCLID4 Test substance: as prescribed by 1.1 - 1.4

Test animals

Species

other: Rats and mice

Strain

other: Rats: Sprague-Dawley; mice: CF-1

Sex

male

Details on test animals and environmental conditions

TEST ANIMALS - Weight at study initiation: Rats: 200 - 300 g; mice: 22 - 28 g

Administration / exposure

Route of administration

oral: unspecified

Vehicle

no data

Details on oral exposure

No data

Doses

No data

No. of animals per sex per dose

No data

Control animals

no data

Details on study design

No data

Statistics

No data

Any other information on materials and methods incl. tables

Range-finding toxicity data: List VI. Amer. Ind. Hyg. Ass. J., 23, 95-107

Results and discussions

Preliminary study (if fixed dose study)

No data

Effect levels

| Sex | Endpoint | Effect level | Based on | 95% CL | Remarks |
| --- | --- | --- | --- | --- | --- |
| male | LD50 | 810 mg/kg bw | no data | 590 — 1120 | Male rats |
| male | LD50 | 710 mg/kg bw | no data | 520 — 960 | Male mice |

Mortality

No data

Clinical signs

No data

Body weight

No data

Gross pathology

No data

Applicant's summary and conclusion

Interpretation of results

other: No data

Criteria used for interpretation of results

not specified

Conclusions

The acute oral LD50 for the test substance in male rats was 810 (590 - 1120) mg/kg; in male mice it was 710 (520 - 960) mg/kg.

# 7.5 Repeated dose toxicity

# 7.5.1 Repeated dose toxicity: oral

## Endpoint study record: MHLW, Japan (1995)

| UUID |  | IUC5-dcadd249-3803-4fb7-84cc-8d28a70a30b3 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | mariko / National Institute of Health Sciences / Tokyo / Japan |
| Date |  | 2013-02-06 15:20:50 JST |
| Remarks |  |  |

Administrative Data

| Study result type | experimental result |  |  |
| --- | --- | --- | --- |
| Reliability | 1 (reliable without restriction) |  |  |
| Rationale for reliability incl. deficiencies | Study conducted in accordance with OECD TG 407 under GLP. |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | MHLW, Japan | 1995 | 3,4-dimethylaniline: Twenty-eight-day Repeat Dose Oral Toxicity Test of 3,4-Dimethylaniline in Rats | http://dra4.nihs.go.jp/mhlw_data/jsp/SearchPageENG.jsp | Biosafety Research Center/Foods, Drugs and Pesticides(An-pyo Center), 582-2 Shioshinden Aza Arahama, Fukude-cho, Iwata-gun,Shizuoka 437-12, Japan |  | Ministry of Health, Labour and Welfare of Japan | 2292 (115-028) |  |

Materials and methods

Test type

subacute

Limit test

no

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | OECD Guideline 407 (Repeated Dose 28-Day Oral Toxicity in Rodents) |  |

GLP compliance

yes

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 95-64-7 |

Details on test material

- Name of test material: 3,4-dimethylaniline - Physical state: White solid. - Analytical purity: 99.8%. - Stability under test conditions: Discolouration with light. - Storage condition of test material: Storage room at the testing laboratories.

Test animals

Species

rat

Strain

other: Crj:CD(SD)

Sex

male/female

Details on test animals and environmental conditions

TEST ANIMALS - Source: - Age at study initiation: - Weight at study initiation: - Fasting period before study: - Housing: - Diet (e.g. ad libitum): - Water (e.g. ad libitum): - Acclimation period: ENVIRONMENTAL CONDITIONS - Temperature (°C): - Humidity (%): - Air changes (per hr): - Photoperiod (hrs dark / hrs light): IN-LIFE DATES (Exposure group) From: Day 1 Up To: Day 29. (Recovery group) From: Day 1 To: Day 15.

Administration / exposure

Route of administration

oral: gavage

Vehicle

corn oil

Analytical verification of doses or concentrations

yes

Duration of treatment / exposure

Exposure period of 28 days, recovery period of 14 days.

Frequency of treatment

No data

Doses/concentrations

| 10 mg/kg/day |  |
| --- | --- |
| Basis | other: nominal in vehicle |

| 50 mg/kg/day |  |
| --- | --- |
| Basis | other: nominal in vehicle |

| 250 mg/kg/day |  |
| --- | --- |
| Basis | other: nominal in vehicle |

No. of animals per sex per dose

5 animals per sex per dose and the control group (exposure group). 5 animals per sex at 250 mg/kg/day and the control group (recovery group).

Control animals

yes, concurrent vehicle

Positive control

No.

Results and discussions

Effect levels

| Endpoint | Effect level | Based on | Sex | Basis for effect level / Remarks |
| --- | --- | --- | --- | --- |
| NOAEL | 10 mg/kg bw/day (nominal) | test mat. | male/female |  |

Results of examinations

Clinical signs and mortality

no effects

Food efficiency

not examined

Water consumption and compound intake (if drinking water study)

not examined

Ophthalmoscopic examination

not examined

Neurobehaviour

not examined

Histopathology: neoplastic

no effects

Details on results

MORTALITY There were no deaths. CLINICAL SIGNS Increased salivation was observed in both sexes at 250 mg/kg. This was seen in few animals in the week 1 during recovery period. BODY WEIGHT GAIN Males treated at 250 mg/kg showed a significant reduction in bodyweight gain compared to controls between week 2 to the end of the exposure period. Females treated at 250 mg/kg showed a significant reduction in bodyweight gain in Week 2 during recovery period. FOOD CONSUMPTION A significantly lower food intake was evident in males treated at 250 mg/kg but Week 1 only. A slight reduction in food intake was observed in females at at 250 mg/kg in Week 2 during recovery period. HAEMATOLOGY There was a significant decrease in haematocrit, haeboglobin and red blood cell counts and increase in platelet and teticulocyte in both sexes at 250 mg/kg/day. Furthermore, there was a significant increase in white blood cell counts in males and decrease in neutrophilic leukocyte ratio and increase in lymphocyte ratio in females at 250 mg/kg. No intergroup differences were found in males at the end of recovery period, however, a slight reduction in haematocrit was still seen in females. (see attached Table) CLINICAL CHEMISTRY There was a significant increase in total cholesterol at and above 50 mg/kg in males and at 250 mg/kg in females. Significant increase in GPT and total bilirubin at 250 mg/kg in both sexes and increase in albumin, A/G ratio and potassium in both sexes at 250 mg/kg, increase in glycemia and calcium and decrease in chlorine in females were also seen. No intergroup differences were found in males at the end of recovery period. (see attached Table) URINALYSIS There is a significant increase in the urinary volume, decrease in urine pH and urinary sediment (positive red blood cells) in both sexes at 250 mg/kg. Significant decrease in specific gravity was observed in females at the same dose. No intergroup differences were found at the end of recovery period. ORGAN WEIGHTS Animals treated with 250 mg/kg/day showed a significant increase in the liver and spleen weights, both absolute and relative to terminal bodyweight. Significant increase in relative testes weight was observed in males and decrease in relative adrenal weight in females at the same dose. No intergroup differences were found at the end of recovery period. (see attached Table) GROSS PATHOLOGY Significant effects were observed as only at 250 mg/kg bw/day as follows: Black coloured spleen and its tumefaction were seen in both sexes at 250 mg/kg at the end of exposure period. Enlargement was seen in the liver at 250 mg/kg in males and at and above 50 mg/kg in females. Although the incidence of effects at 50 mg/kg bw/day was not statistically significant, the effect in the liver was considered to be biologicaly significant based on the affinity of toxicological effects by 2,6-dimethylaniline.Black coloured liver was seen in males at 250 mg/kg. No no intergroup differences were found at the end of recovery period. (see attached Table) HISTOPATHOLOGY Significant effects were observed at only 250 mg/kg bw/day as follows: Haematopoiesis in the bone marrow, hyperemia, haematopoiesis and pigmentation in the spleen, enlargement and single cell necrosis of the hepatocytes, extramedullary hematopoiesis and pigmentation of the Kupffer cells in the liver were observed in both sexes at 250 mg/kg. There was a trend of increase in degeneration in tubular hyaline droplets in the kidneys at or above 50 mg/kg in males. The increase in lesion incidence at 50 mg/kg bw/day was not statistically significant, but group sizes were small and the increase could reasonably be interpreted as biologically significant. In the recovery group, there was a significant decrease in haematopoiesis, hyperemia in the spleen, haematopoiesis, enlargement of the hepatocytes, single cell necrosis, extramedullary hematopoiesis and tubular hyaline droplets in the kidneys, or these were not observed. However, pigmentation in the spleen were observed in all animals in the recovery group in both sexes. This was increased in females. Pigmentation of the Kupffer cells in the liver was also seen, particularly in all female animals. (see attached Table)

Overall remarks, attachments

Attached background material

| Attached document | Remarks |
| --- | --- |
| 95647_hemato.gif / 25.93 KB (image/gif) |  |
| 95647_SerumChem.gif / 27.69 KB (image/gif) |  |
| 95647_OrganWeigt.gif / 24.1 KB (image/gif) |  |
| 95647_Gross.gif / 8.72 KB (image/gif) |  |
| 95647_histo.gif / 16.31 KB (image/gif) |  |
| 95647_histo_female.gif / 16.06 KB (image/gif) |  |

Applicant's summary and conclusion

Conclusions

The NOAEL of 10 mg/kg bw/day was determined based on hyaline droplet in the kidney in males and effects on the liver (enlarged liver) in females. Although incidences at 50 mg/kg bw/day were not statistically significant, dose responsibility was observed. Therefore, it was considered that these effects were biologically significant. There is also an affinity of toxicological effects to 2,6-dimethylaniline (please see dossier of 2,6-dimethyl aniline)

# 7.6 Genetic toxicity

# 7.6.1 Genetic toxicity in vitro

## Endpoint study record: Kusakabe H et al (2002)

| UUID |  | IUC5-96643ba1-72e0-4dfc-94ec-8e34ca2c5afb |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | mariko / National Institute of Health Sciences / Tokyo / Japan |
| Date |  | 2013-02-06 15:20:50 JST |
| Remarks |  |  |

Administrative Data

| Purpose flag | supporting study |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result |  |  |
| Reliability | 4 (not assignable) |  |  |
| Rationale for reliability incl. deficiencies | Although the test has many aspects of OECD 473, there are key deficiencies such as positive and negative controls and information on the test conditions, as well as the specific solvent actually used to dissolve the test item. The deficiencies in the report are such as to reduce the confidence in the reliability to not assignable. |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Kusakabe H, Yamakage K, Wakuri S, Sasaki K, Nakagawa Y, Watanabe M, Hayashi M, Sofuni T, Ono H & Tanaka N | 2002 | Relevance of chemical structure and cytotoxicity to the induction of chromosome aberrations based on the testing results of 98 high production volume industrial chemicals. | Mutation Research 517 (2002) 187-198 |  |  |  |  |  |

Data access

data published

Materials and methods

Type of genotoxicity

chromosome aberration

Type of study

in vitro mammalian chromosome aberration test

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| equivalent or similar to | OECD Guideline 473 (In vitro Mammalian Chromosome Aberration Test) | yes (No negative or positive controls. Lack of certain details in the report, that are required in the guideline. Lack of specific solvent used for the test item.) |

Principles of method if other than guideline

Cells CHL/IU cells used in all CA tests were obtained from Japanese Cancer Research Resources Bank (JCRB). THe cells were cultured with Eagles minimum essential medium (MEM, Nissue Pharmaceutical, Tokyo, Japan) supplemented with 10% fetal calf serum or calf serum. The 2E04 cells were plated in 5 ml medium on 60 mm plate and cultured for 72 hr at 37°C in a humidified incubator (5% CO2) before the treatment with chemicals. Chemical treatment The test material was dissolved in either DMSO or acetone to at least 1000 mM or 500 mg/ml. In order to examine the metabolic activation of the chemical, the proliferating cells were treated with the chemical for 6 hr in serum-free MEM with S9 mix (S9(+)), or without S9 mix (S9(-)), then cultured a further 18 hr in the fresh MEM with serum. S9 and co-factors were mixed immediately before use, then applied to cultures to expose the cells to the S9 at 5% (v/v). Moreover, the cells were also treated with the chemicals for 24 and 48 hr continuously in the absence of S9 mix. Duplicate cultures were used for each dose. Growth inhibition test for cytotoxicity Preliminary growth inhibition test was conducted to determine the cytotoxicity of the chemicals. A maximum dose was set for the chromosomal aberration test at 50% or more of the cytotoxic dose determined by the growth inhibition test. When cytotoxicity was 50% or less, 5 mg/ml or 10 mM was set as the maximum dose. The other doses were sequential half-dilutions. Chromosome preparation Before harvesting, the cells were treated wit 0.1 µg/ml of colcemid for 2 hr, then chromosome specimens were made by the usual air-dry method. Chromosome analysis Chromosome specimens were stained with 3% Giemsa solution for 8 min. The number of cells with chromatid- and chromosome-type breaks and exchanges were scored per 200 cells at each dose. Polyploid cells were also scored per 800 cells at each dose. A polyploid cell was defined as a metaphase with more than 37 chromosomes, i.e. theoretical "hypertriploid cell", containing approximately three times the number of chromosomes than half the modal chromosome number (25) CHL/IU cells.

GLP compliance

no data

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 65-64-7 |
| EC number | 202-440-0 |
| EC name | 3,4-xylidine |

Details on test material

- Name of test material: No data

Confidential details on test material

Test material was supplied from an industrial company (not named in report) under management by Ministry of Health and Welfare in Japan.

Method

Species/strain

| Species/strain | other: Chinese Hamster lung cells |
| --- | --- |
| Details on mammalian cell lines (if applicable) | Cells obtained from Japanese Cancer Research Resources Bank (JRCB). |
| Additional strain characteristics | no data |
| Metabolic activation | with and without |
| Metabolic activation system | S9 mix at 5% v/v (after mixing with co-factors) |

Test concentrations

1000mM or 500 mg/ml

Vehicle

- Vehicle(s)/solvent(s) used: acetone or DMSO - Justification for choice of solvent/vehicle: limited solubility of test item in water

Controls

| Negative controls | no data |
| --- | --- |
| Solvent / vehicle controls | no data |
| True negative controls | no data |
| Positive controls | no data |

Details on test system and conditions

METHOD OF APPLICATION: in medium; Eagle's minimum essential medium (MEM) supplemented with 10% fetal calf serum or calf serum. DURATION - Preincubation period: 72 hr - Exposure duration: 6 hr - Expression time (cells in growth medium): 18 hr after exposure to chemical - Fixation time (start of exposure up to fixation or harvest of cells): 24 hr (includes 6 hr exposure + 18 hr in fresh MEM) STAIN: 3% Giemsa solution for 8 min NUMBER OF REPLICATIONS: 2 NUMBER OF CELLS EVALUATED:20000 DETERMINATION OF CYTOTOXICITY - Method: relative cell confluence or cell number to solvent control and also relative mitotic index; OTHER EXAMINATIONS: - Determination of polyploidy: Yes, scored per 800 cells at each dose.

Evaluation criteria

The number of cells with chromatid- and chromosome-type breaks and exchanges were scored per 200 cells at each dose. Polyploid cells were also scored per 800 cells at each dose. A polyploid cell was defined as a metaphase with more than 37 chromosomes, i.e. theoretical "hypertriploid cell", containing approximately three times the number of chromosomes than half the modal chromosome number (25) CHL/IU cells.

Statistics

No data

Results and discussions

Test results

| Species/strain | other: Chinese hamster lung cells |
| --- | --- |
| Metabolic activation | with and without |
| Test system | strain/cell type: Chinese hamster lung cells |
| Genotoxicity | negative |
| Cytotoxicity | no data (probably determined based on detail in report, but not specifically stated in the report) |
| Vehicle controls valid | not examined |
| Negative controls valid | not examined |
| Positive controls valid | not examined |

Additional information on results

ADDITIONAL INFORMATION ON CYTOTOXICITY: Preliminary growth inhibition test was conducted to determine the cytotoxicity of the chemicals. A maximum dose was set for the chromosomal aberration test at 50% or more of the cytotoxic dose determined by the growth inhibition test. When cytotoxicity was 50% or less, 5 mg/ml or 10 mM was set as the maximum dose. The other doses were sequential half-dilutions.

Any other information on results incl. tables

CA and cytotoxicity; Induction of structural CA in each system; Polyploidy (%). Short-term treatment; Continuous treatment. S9(+); S9(-); 24 hr; 48 hr. C; -; -; -; -; -. C = CA- negative at doses manifesting over 50% cytotoxicity - = no effect seen

Applicant's summary and conclusion

Interpretation of results

negative

Conclusions

3,4-Dimethylaniline is not clastogenic based on this paper.

## Endpoint study record: Nohmi T et al (1984)

| UUID |  | IUC5-e1eeac4e-5708-4f67-a208-f89d1a0fa829 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | mariko / National Institute of Health Sciences / Tokyo / Japan |
| Date |  | 2013-02-06 15:20:50 JST |
| Remarks |  |  |

Administrative Data

| Reliability | 4 (not assignable) |
| --- | --- |
| Rationale for reliability incl. deficiencies | Insufficient detail in the report published in the literature. |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Nohmi T, Yoshikawa K, Nakadate M, Miyata R & Ishidate M. | 1984 | Mutations in Salmonella Typhimurium and Inactivation of Bacillus Subtilis Transforming DNA Induced by Phenylhydroxylamine Derivatives. | Mutat. Res., 136, 159-168. |  |  |  |  |  |

Data access

data published

Materials and methods

Type of genotoxicity

DNA damage and/or repair

Type of study

Bacillus subtilis recombination assay

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | other guideline: No data | no data |

Principles of method if other than guideline

A bacterial reverse mutation test was performed using Salmonella typhimurium strains in the presence and absence of metabolic inhibition.

GLP compliance

no (Study pre-dates GLP)

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 65-64-7 |
| EC number | 202-440-0 |
| EC name | 3,4-xylidine |

Details on test material

- Name of test material: 3,4-dimethylaniline

Method

Target gene

Histidine

Species/strain

| Species/strain | S. typhimurium, other: TA98, TA100 |
| --- | --- |
| Additional strain characteristics | no data |
| Metabolic activation | with and without |
| Metabolic activation system | S9 from rat liver induced wiht Kanechlor KC-400 (polychlorinated biphenyl) ip, five days before they were killed. |

Test concentrations

No data

Vehicle

0.1 mL DMSO

Details on test system and conditions

Salmonella/microsome test: The test substance was added to an overnight culture of S. typhimurium and mixed with 1/15 M potassium phosphate buffer, pH 7.4 or S9 mix. After 20 min incubation at 37 °C, the reaction mixture was mixed with top agar and spread on minimal agar plates containing L-histidine and D-biotin. Plates were incubated for 2 days at 37 °C and the number of colonies counted. Induced mutation frequency: A suspension of S. typhimurium TA100 (about 5 x 10^8 cells/mL) were prepared from over night culture and mixed with various concentrations of the PHA derivatives. After incubation for 30 min at 37 °C with shaking, 0.1 mL of the sample was diluted 10^5-fold with phosphate buffer and then poured onto the semi-enriched gar plate for assaying surviving cells. The remaining sample was centrifuged and collected cells were mixed with top agar and then poured onto minimal agar plates. After 2 days at 37 °C, the number of revertants (M) and the surviving cells (N) were counted. The induced mutation frequency was calculated as the value of (M/10^N) minus (M0/10^6N), where M0 and M stand for the values in the untreated and chemical-treated cell culture respectively. Assay for loss of transforming DNA activity: Transforming DNA was prepared from vegetative cells of B. subtilis Marburg wild type. Competent cells of B. subtilis tryptophan-requiring mutant were prepared. The reaction mix consisting of the test substance (5 or 10 mM) in DMSO and transforming DNA were added to 0.1 M tris-HCL buffer pH 7.2 containing 0.1 M sodium chloride and 0.8 mM magnesium sulfate was incubated for 60 min at 37 °C. After incubation , the mixture was diluted 5-fold with tris-HCL buffer and 100 µL of the diluted solution which contained transforming DNA was mixed with the competent cell suspension. After incubation for 30 min at 37 °C the cells were mixed with soft agar and poured onto minimal agar. As a control, the test DNA was treated with DMSO prior to dilution and subsequent addition of the competent cells. The number of transformants was counted after incubation for 2 days at 37 °C and the percentage of the remaining activity of the transforming DNA was determined in each experiment by comparing the number of transformants produced by cells receiving control DNA and the number receiving treated DNA.

Statistics

No data

Results and discussions

Test results

| Species/strain | S. typhimurium TA 98 |
| --- | --- |
| Metabolic activation | with and without |
| Genotoxicity | negative |

| Species/strain | S. typhimurium TA 100 |
| --- | --- |
| Metabolic activation | without |
| Genotoxicity | negative |

| Species/strain | S. typhimurium TA 100 |
| --- | --- |
| Metabolic activation | with |
| Genotoxicity | positive (3.9 x 10 induced His+/µM) |

Additional information on results

Aniline derivatives were mutagenic in TA 100 only when S9 mix was present and they induced less than 10^3 revertants/µmole.

Any other information on results incl. tables

Mutagenicities*. TA100; TA98. Compound; -S9; +S9; -S9; +S9. 2,3-dimethylaniline; -; 4.4 x 10; —; -. 2,4-dimethylaniline; -; 1.2 x 10^2; -; -. 2,5-dimethylaniline; -; 4.2 x 10^2; -; -. 2,6-dimethylaniline; -; 2.4 x 10^2; -; -. 3,4-dimethylaniline; -; 3.9 x 10; -; -. 3,5-dimethylaniline; -; 9.3 x 10; -; -. * Mutagenicities were expressed as numbers of induced His+/umole. - Negative response

Applicant's summary and conclusion

Interpretation of results

positive with metabolic activation

Conclusions

Aniline derivatives were mutagenic in TA 100 only when S9 mix was present and they induced less than 10^3 revertants/µmole.

## Endpoint study record: Hartman CP et al (1979)

| UUID |  | IUC5-ff7294c1-fcdb-4e18-9ac1-604ad7697e02 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | mariko / National Institute of Health Sciences / Tokyo / Japan |
| Date |  | 2013-02-06 15:20:50 JST |
| Remarks |  |  |

Administrative Data

| Reliability | 4 (not assignable) |
| --- | --- |
| Rationale for reliability incl. deficiencies | Only tested in one strain at one dose. |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Hartman CP, Andrews AW & Chung K-T. | 1979 | Production of a Mutagen from Ponceau 3R by a Human Intestinal Anaerobe. | Infect. Immun., 23, 686-689. |  |  |  |  |  |

Data access

data published

Materials and methods

Type of genotoxicity

gene mutation

Type of study

bacterial reverse mutation assay (e.g. Ames test)

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | other guideline: No data | no data |

Principles of method if other than guideline

The Ames et al (1973) Salmonella mutagenicity test was used.

GLP compliance

no (Study pre-dates GLP.)

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 65-64-7 |
| EC number | 202-440-0 |
| EC name | 3,4-xylidine |

Details on test material

- Name of test material: 3,4-dimethylaniline

Method

Target gene

No data

Species/strain

| Species/strain | S. typhimurium TA 100 |
| --- | --- |
| Metabolic activation | with and without |
| Metabolic activation system | Rat liver S9 preparations |

Test concentrations

0.83 µmoles

Controls

| Negative controls | yes (Cells only) |
| --- | --- |
| Solvent / vehicle controls | yes (DMSO only) |
| True negative controls | no data |
| Positive controls | no data |
| Positive control substance | no data. |

Details on test system and conditions

IUCLID4 Type: Ames test

Evaluation criteria

No data

Statistics

No data

Results and discussions

Test results

| Species/strain | S. typhimurium TA 100 |
| --- | --- |
| Metabolic activation | with and without |
| Test system | all strains/cell types tested |
| Genotoxicity | negative |
| Cytotoxicity | no data |
| Vehicle controls valid | no data |
| Negative controls valid | no data |
| Positive controls valid | no data |

Additional information on results

No dimethylanilines showed any mutagenesis in dose-response curves from 1 to 1000 µg.

Any other information on results incl. tables

Mutagenicity of 3,4-dimethylaniline: Test conditions; TA 100 –S9; TA 100 + S9. Cells only; 64; 69. Dimethylsulfoxide (DMSO); 75; 67. 3,4-dimethylaniline (0.83 µmoles); 90; 85.

Applicant's summary and conclusion

Interpretation of results

negative

Conclusions

The test substance was assessed in a bacterial reverse mutation assay and was shown to be negative for mutagenicity with and without metabolic activation.

## Endpoint study record: Zimmer D et al (1980)

| UUID |  | IUC5-6093b567-4662-4265-a789-3c8a68b21f04 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | mariko / National Institute of Health Sciences / Tokyo / Japan |
| Date |  | 2013-02-06 15:20:50 JST |
| Remarks |  |  |

Administrative Data

| Reliability | 4 (not assignable) |
| --- | --- |
| Rationale for reliability incl. deficiencies | Insufficient detail in the report published in the literature. |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Zimmer D, Mazurek J, Petzold G & Bhuyan BK. | 1980 | Bacterial Mutagenicity and Mammalian Cell DNA Damage by Several Sustituted Anilines. | Mutat. Res., 77, 317-326. |  |  |  |  |  |

Data access

data published

Materials and methods

Type of genotoxicity

gene mutation

Type of study

bacterial reverse mutation assay (e.g. Ames test)

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | other guideline: No data | no data |

Principles of method if other than guideline

The test substance was assessed for mutagenicity using a modified Ames test.

GLP compliance

no (Study pre-dates GLP.)

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 65-64-7 |
| EC number | 202-440-0 |
| EC name | 3,4-xylidine |

Details on test material

IUCLID4 Test substance: as prescribed by 1.1 - 1.4 - Analytical purity: 97 - 99 %

Method

Target gene

No data

Species/strain

| Species/strain | S. typhimurium TA 100 |
| --- | --- |
| Metabolic activation | with and without |
| Metabolic activation system | Liver S9 fraction from Upjohn strain of Sprague-Dawley rats induced with Aroclor 1254. |

Test concentrations

No data

Controls

| Negative controls | no data |
| --- | --- |
| Solvent / vehicle controls | no data |
| True negative controls | no data |
| Positive controls | no data |
| Positive control substance | no data. |

Details on test system and conditions

IUCLID4 Type: Ames test

Evaluation criteria

The revertant count for each point was the average of duplicate plates scored on a modified Artek Model Colony Counter. A compound was considered mutagenic if it increased the number of revertants/plate over the control background level and showed a dose-response.

Results and discussions

Test results

| Species/strain | S. typhimurium TA 100 |
| --- | --- |
| Metabolic activation | with and without |
| Test system | all strains/cell types tested |
| Genotoxicity | positive (84 revertants/umole/plate) |
| Cytotoxicity | no data |
| Vehicle controls valid | no data |
| Negative controls valid | no data |
| Positive controls valid | no data |

Additional information on results

The test substance was weakly mutagenic in the test strains (84 revertants/umole/plate).

Applicant's summary and conclusion

Interpretation of results

positive

Conclusions

The test substance was assessed for mutagenicity using a modified Ames method with Salmonella typhimurium TA100. The results were weakly positive for mutagenicity.

## Endpoint study record: Zeiger et al (1988)

| UUID |  | IUC5-824c03be-dcd5-4540-8fdc-01baa270e318 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | mariko / National Institute of Health Sciences / Tokyo / Japan |
| Date |  | 2013-02-06 15:20:51 JST |
| Remarks |  |  |

Administrative Data

| Study result type | experimental result |  |  |
| --- | --- | --- | --- |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Study published in peer-reviewed journal. |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Zeiger E, Anderson B, Haworth S, Lawlor T, Mortelmans K. | 1988 | Salmonella mutagenicity tests: IV. Results from the testing of 300 chemicals. | Environ Mol Mutagen. 1988;11 Suppl 12:1-157. |  |  |  |  |  |

Data access

data published

Materials and methods

Type of genotoxicity

gene mutation

Type of study

bacterial reverse mutation assay (e.g. Ames test)

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Details on test material

Purity: ~99%

Method

Species/strain

| Species/strain | other: S. typhimurium TA 1535, TA 1537, TA 98, TA97 and TA 100 |
| --- | --- |
| Metabolic activation | with and without |
| Metabolic activation system | S9 (male SD rats and male Syrian hamster livers) |

Test concentrations

0, 3, 10, 33, 100, 333, 666, 1000, or 1666 μg/plate

Vehicle

DMSO

Controls

| Solvent / vehicle controls | yes (DMSO) |
| --- | --- |
| Positive controls | yes |

Details on test system and conditions

The preincubation assay was performed as described previously [Haworth et al., 1983], with some differences, as described below. The test chemical (0.05 ml), Salmonella culture (0.10 ml), and S-9 mix or buffer (0.50 ml) were incubated at 37°C, without shaking, for 20 min. Chemicals known or suspected to be volatile were incubated in capped tubes. The top agar was added and the contents of the tubes were mixed and poured onto the surface of petri dishes containing Vogel-Bonner medium [Vogel and Bonner, 1956]. The histidine-independent (his+) colonies arising on these plates were counted following two days incubation at 37°C. Plates were machine counted (New Brunswick, Edison, NJ; Artek, Farmingdale, NY) unless precipitate was present which interfered with the count, or the color of the test chemical on the plate reduced the contrast between the colonies and the background agar. At the discretion of the investigators, plates with low numbers of colonies were counted by hand. Variations in the protocol among the tested chemicals reflect the evolution of the protocol originally described by Haworth et al. [1983]. Testing in strains TA97, TA98, TA100, and TA1535, with some additional testing in strain TA1537; 10% S-9 was used. The first test of a chemical was without activation and with 10% S-9 in the S-9 mix. If a positive result was obtained the test was repeated. Haworth S, Lawlor T, Mortelmans K, Speck W, Zeiger E (1983): Salmonella mutagenicity results for 250 chemicals. Environ Mutagen 5(Suppll):3-142. Vogel HJ, Bonner DM (1956): Acetylornithinase of E. coli: Partial purification and some properties. J BioI Chern 218:97-106.

Evaluation criteria

A chemical was judged mutagenic (+) or weakly mutagenic (+W) if it produced a reproducible dose-related reponse over the solvent control in replicate trials. A chemical was Judged questionable (?) if the results of individual trials were not ,reproducible, if increases in his + revertants did not meet the criteria for a "+W"response, or if only single doses produced increases in his+ revertants in repeat trials. Chemicals were judged nonmutagenic (-) if they did not meet the criteria for a mutagenic or questionable response.

Results and discussions

Test results

| Species/strain | other: S. typhimurium TA97 and TA 100 |
| --- | --- |
| Genotoxicity | positive |
| Cytotoxicity | yes |
| Positive controls valid | yes |

| Species/strain | other: S. typhimurium TA 1535, TA 1537, TA 98, |
| --- | --- |
| Genotoxicity | negative |
| Cytotoxicity | yes |
| Positive controls valid | yes |

Additional information on results

Cytotoxicity was observed at 1666 μg/plate in TA98, TA100 and TA97. Slight cytotoxicity was observed in TA97 at 1000 μg/plate and TA1535 at 1666 μg/plate.

Overall remarks, attachments

Attached background material

| Attached document | Remarks |
| --- | --- |
| Ames Table.pdf / 277.3 KB (application/pdf) | Environ Mol Mutagen. 1988;11 Suppl 12:1-157. |

Illustration (picture/graph)

Applicant's summary and conclusion

Interpretation of results

positive with metabolic activation

Conclusions

Positive results were obtained with Salmonella strains TA100 and TA97

# 7.6.2 Genetic toxicity in vivo

## Endpoint study record: Comet Assay (Hayashi et al., 2000)

| UUID |  | IUC5-47d2a839-3bcb-4ed3-a86c-b838a64ac394 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | mariko / National Institute of Health Sciences / Tokyo / Japan |
| Date |  | 2013-02-06 15:20:51 JST |
| Remarks |  |  |

Administrative Data

| Study result type | experimental result |  |  |
| --- | --- | --- | --- |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Meets generally accepted scientific standards |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | Hayashi, M., Suzuki, T., Ohara, A. | 2000 | Establishment of methods for gene mutation analysis using transgenic mice. In: Research report on studies on establishment of new methods for evaluation of carcinogenicity studies using animals and its implication. | (Grant-in-Aid for Cancer Research from the Ministry of Health, Labour and Welfare, Japan), in Japanese. (unpublished) |  |  |  |  |  |

Materials and methods

Type of genotoxicity

DNA damage and/or repair

Type of study

single cell gel/comet assay in rodents for detection of DNA damage

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Details on test material

Supplier: Wako Pure Chemical Industries, Ltd.

Test animals

Species

mouse

Strain

other: ddY

Sex

male

Details on test animals and environmental conditions

(same animals to the Micronucleus/bone marrow (Hayashi et al.,2000))

Administration / exposure

Route of administration

oral: gavage

Vehicle(s)

olive oil

Details on exposure

Each of dimethylanilines was orally administered at 200 mg/kg bw (10 ml/kg) four times at weakly intervals, and bone marrow, liver, lung and kidneys were isolated 3 hours after the last dosing.

Post exposure period

3 and 24 h

Doses / concentrations

| 200 mg/kg bw/weak |  |
| --- | --- |
| Basis | actual ingested |

No. of animals per sex per dose

Three males/group

Control animals

yes, concurrent vehicle

Positive control(s)

no positive control

Examinations

Tissues and cell types examined

Bone marrow, liver, kidney and lung were taken and examined.

Details of tissue and slide preparation

Nuclei isolated by homogenisation were placed on slides and placed in chilled lysing solution (2.5 M NaCl, 0.1 M EDTA, 0.01 M Tris, 10% DMSO, 1% TritonX-100) and kept at 4 °C in the dark for more than 60 min. Electrophoresis was conducted at less than 10 °C in the dark for 20 min at approximately 300 mA (1 V/cm) (pH=13). After the electrophoresis, slides were neutralized with buffer (400 mM Tris; pH7.5) for three times (10 min each). DNA was stained with EtBr and observed.

Evaluation criteria

The different type of migration was classified in 5 categories as follows (500 cells/group were recorded). Type 1: without tail (no damage) Type 2: with a small tail (light genetic damage) Type 3: with tail with evident migration Type 4: with definite tale with a consistent amount of fragments Type 5: Almost all the DNA is present in the tail (severe genetic damage)

Results and discussions

Test results

| Sex | male |
| --- | --- |
| Genotoxicity | positive |

Any other information on results incl. tables

Positive results were obtained in bone marrow, lung, kidney, and liver.

Overall remarks, attachments

Illustration (picture/graph)

Applicant's summary and conclusion

Conclusions

Positive results were obtained in lung, kidney, and liver at 3 h after the last dosing but not observed at 24 h after the last dosing.

## Endpoint study record: Micronucleus/bone marrow (Hayashi et al., 2000)

| UUID |  | IUC5-9ea738ab-9bd4-48c3-adbd-c2bbc8cca2f9 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | mariko / National Institute of Health Sciences / Tokyo / Japan |
| Date |  | 2013-02-06 15:20:51 JST |
| Remarks |  |  |

Administrative Data

| Study result type | experimental result |  |  |
| --- | --- | --- | --- |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Meets generally accepted scientific standards |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | Hayashi, M., Suzuki, T., Ohara, A. | 2000 | Establishment of methods for gene mutation analysis using transgenic mice. In: Research report on studies on establishment of new methods for evaluation of carcinogenicity studies using animals and its implication. | (Grant-in-Aid for Cancer Research from the Ministry of Health, Labour and Welfare, Japan), in Japanese. (unpublished) |  |  |  |  |  |

Materials and methods

Type of genotoxicity

chromosome aberration

Type of study

micronucleus assay

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Details on test material

Supplier: Wako Pure Chemical Industries, Ltd.

Test animals

Species

mouse

Strain

other: ddY

Sex

male

Details on test animals and environmental conditions

same animals to the comet assay (Hayashi et al.,2000)

Administration / exposure

Route of administration

oral: gavage

Vehicle(s)

olive oil

Frequency of treatment

once

Post exposure period

24 h

Doses / concentrations

| 200 mg/kg bw |  |
| --- | --- |
| Basis | actual ingested |

No. of animals per sex per dose

3 animals/group

Control animals

yes, concurrent no treatment

Positive control(s)

no positive control

Examinations

Tissues and cell types examined

Polychromatic erythrocytes from the bone marrow

Details of tissue and slide preparation

Cells for specimen were collected 24 h after the administration.

Results and discussions

Test results

| Sex | male |
| --- | --- |
| Genotoxicity | negative |

Any other information on results incl. tables

Table 1.Micronucleus Induction in Bone Marrow (Male) Chemical; Dose; MNPCE*s/1000 PCEs; Mean±SD; PCE/NCE**. Control; 0 mg/kg; 1; 2; 1; 1.3±0.5; 1.1±0.0. 2,3-DMA; 200 mg/kg; 0; 1; 1; 0.7±0.5; 0.8±0.2. 2,4-DMA; 200 mg/kg; 0; 0; 2; 0.7±0.9; 1.1±0.2. 2,5-DMA; 200 mg/kg; 0; 0; 0; 0.0±0.0; 1.1±0.4. 2,6-DMA; 200 mg/kg; 1; 2; 1; 1.6±0.9; 1.0±0.1. 3,4-DMA; 200 mg/kg; 0; 2; 1; 1.3±0.5; 1.1±0.1. 3,5-DMA; 200 mg/kg; 0; 0; 2; 0.7±0.9; 0.9±0.2. * micronucleated polychromatic erythrocytes ** normochromatic erythrocytes

Applicant's summary and conclusion

Interpretation of results

negative

Conclusions

Numbers of micronuleated cells were not increased in any treatment.

## Endpoint study record: Gene mutation (RA from 2,5-, 2,6- and 3,5-dimethylaniline)

| UUID |  | IUC5-eb9884c9-6c2d-4d6b-9a19-9741ab0d866e |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | mariko / National Institute of Health Sciences / Tokyo / Japan |
| Date |  | 2013-02-06 15:20:51 JST |
| Remarks |  |  |

Administrative Data

| Study result type | read-across based on grouping of substances (category approach) |  |  |
| --- | --- | --- | --- |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | Hayashi, M., Suzuki, T., Ohara, A. | 2000 | Establishment of methods for gene mutation analysis using transgenic mice. In: Research report on studies on establishment of new methods for evaluation of carcinogenicity studies using animals and its implication. | (Grant-in-Aid for Cancer Research from the Ministry of Health, Labour and Welfare, Japan), in Japanese. (unpublished) |  |  |  |  |  |

Materials and methods

Test materials

Details on test material

Read accross from 2,5-, 2,6- and 3,5-dimethylaniline

# 7.7 Carcinogenicity

## Endpoint study record: Morris (1957)

| UUID |  | IUC5-1e5c90aa-9b4c-4f13-8da6-33e848f1d1ce |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | mariko / National Institute of Health Sciences / Tokyo / Japan |
| Date |  | 2013-02-06 15:20:51 JST |
| Remarks |  |  |

Administrative Data

| Study result type | experimental result |  |  |
| --- | --- | --- | --- |
| Reliability | 3 (not reliable) |  |  |
| Rationale for reliability incl. deficiencies | only abstract is available |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | Morris, HP, Lombard, LS, Wagner BP, Weisburger, JH | 1957 | Pituitary tumors in rats ingesting diets containing p-fluoroacetanilide, o-hydroxyacetanilide, and 2,4-dimethylaniline | Proceedings of the American association for cancer research (1954-1970), 2 P234 |  |  |  |  |  |

Data access

data published

Materials and methods

Test materials

Details on test material

a chemical name of title is 2,4-dimethylaniline but it is 3,4-dimetylaniline in the text.

Test animals

Species

rat

Strain

other: Buffalo

Sex

female

Administration / exposure

Route of administration

oral: feed

Details on exposure

3,4-dimethylaniline was fed for 9 1/2 months

Duration of treatment / exposure

3,4-dimethylaniline was fed for 9 1/2 months

Doses / concentrations

136 mg/kg bw/day

No. of animals per sex per dose

18 females

Results and discussions

Results of examinations

Details on results

A 10-15 % incidence of pituitary gland adenomas was noted in controls. Sixty per cent developed adenomas of the pituitary gland. Few tumors of other organs were observed. These included cortical adenomas of the adrenal gland in two rats and a leiomyoma of the uterus in another. All these three animals had pituitary gland adenomas.

Applicant's summary and conclusion

Conclusions

A 10-15 % incidence of pituitary gland adenomas was noted in controls. Sixty per cent developed adenomas of the pituitary gland. Few tumors of other organs were observed. These included cortical adenomas of the adrenal gland in two rats and a leiomyoma of the uterus in another. All these three animals had pituitary gland adenomas.

Executive summary

3,4-dimethylaniline was fed for 9 1/2 months. All animals were autopsied when death appeared imminent or by 19 1/2 months. Controls were fed the basic diet only. A 10-15 % incidence of pituitary gland adenomas was noted in controls. Sixty per cent developed adenomas of the pituitary gland. Few tumors of other organs were observed. These included cortical adenomas of the adrenal gland in two rats and a leiomyoma of the uterus in another. All these three animals had pituitary gland adenomas.

## Endpoint study record: RA from 2,6-dimethylaniline

| UUID |  | IUC5-259d2df4-e44c-4396-8c09-356e9dae6c58 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | mariko / National Institute of Health Sciences / Tokyo / Japan |
| Date |  | 2013-02-06 15:20:51 JST |
| Remarks |  |  |

Administrative Data

| Study result type | read-across based on grouping of substances (category approach) |  |  |
| --- | --- | --- | --- |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | NTP | 1990 | Toxicology and Carcinogenesis Studies of 2,6-Xylidine (2,6-Dimethylaniline) (CAS No. 87-62-7) in Charles River CD Rats (Feed Studies). | National Toxicology Program Technical Report, TR 278. |  |  |  |  |  |

Materials and methods

Test materials

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 87-62-7 |

# 7.8 Toxicity to reproduction

# 7.8.1 Toxicity to reproduction

## Endpoint study record: RA from 2,6-dimethylaniline

| UUID |  | IUC5-0783b188-72e7-48e1-9850-64f9b40b9e72 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | mariko / National Institute of Health Sciences / Tokyo / Japan |
| Date |  | 2013-02-06 15:20:52 JST |
| Remarks |  |  |

Administrative Data

| Study result type | read-across based on grouping of substances (category approach) |  |  |
| --- | --- | --- | --- |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | MHLW, Japan. | 2005 | Combined Repeated Dose and Reproductive/Developmental Toxicity Screening Test of 2,6-Dimethylaniline by Oral Administration in Rats | http://dra4.nihs.go.jp/mhlw_data/jsp/SearchPageENG.jsp |  |  |  |  |  |

Materials and methods

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

no

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 87-62-7 |

# 7.8.2 Developmental toxicity / teratogenicity

## Endpoint study record: RA from 2,6-dimethylaniline

| UUID |  | IUC5-dda4497a-7c1b-4f14-ab90-c214834431c8 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | mariko / National Institute of Health Sciences / Tokyo / Japan |
| Date |  | 2013-02-06 15:20:52 JST |
| Remarks |  |  |

Administrative Data

| Study result type | read-across based on grouping of substances (category approach) |  |  |
| --- | --- | --- | --- |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | MHLW, Japan. | 2005 | Combined Repeated Dose and Reproductive/Developmental Toxicity Screening Test of 2,6-Dimethylaniline by Oral Administration in Rats | http://dra4.nihs.go.jp/mhlw_data/jsp/SearchPageENG.jsp |  |  |  |  |  |

Materials and methods

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

no

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 87-62-7 |

# 7.9 Specific investigations

# 7.9.3 Specific investigations: other studies

## Endpoint study record: Seiler JP (1977)

| UUID |  | IUC5-035f5ca8-b672-47d5-99ad-f5a569aca730 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | mariko / National Institute of Health Sciences / Tokyo / Japan |
| Date |  | 2013-02-06 15:20:52 JST |
| Remarks |  |  |

Administrative Data

| Study result type | experimental result |  |  |
| --- | --- | --- | --- |
| Reliability | 3 (not reliable) |  |  |
| Rationale for reliability incl. deficiencies | Not performed to GLP or guideline. Deficiencies in reporting. |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Seiler JP | 1977 | Inhibition of testicular DNA synthesis by chemical mutagens and carcinogens. Preliminary results in the validation of a novel short term test | Mutation Research, 46(1977) 305-310 |  |  |  |  |  |

Data access

data published

Materials and methods

Type of effects studied

other: Inhibition of testicular DNA synthesis

Type of method

in vivo

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | other guideline: No data | no data |

Principles of method if other than guideline

The measurement of the inhibition of testicular DNA synthesis by chemical carcinogens and mutagens in male mice is considered as a simple and effective in vivo mammalian screening test and gives positive restuls with all types of carcinogens, whereas genetically inactive compounds do not suppress the DNA even at toxic doses. To evaluate the test system a series of experiments designed to validate the procedure was performed with 100 chemicals. Three groups of compounds were investigated. 3,4 dimethylaniline was included in the chemicals tested and was grouped among other chemicals in which preliminary data from Salmonella microsome tests gave rise to suspicions.

GLP compliance

no (Study pre-dates GLP)

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 95-68-1 |
| EC number | 202-440-0 |
| EC name | 3,4-xylidine |

Details on test material

- Name of test material: 3,4-dimethylaniline

Test animals

Species

mouse

Strain

no data

Sex

no data

Details on test animals and environmental conditions

TEST ANIMALS - Weight at study initiation: 23-28g

Administration / exposure

Route of administration

intraperitoneal

Vehicle

no data

Details on exposure

The compound was dissolved by ultrasonic treatment in a suitable carrier with the addition of up to 10% DMSO

Analytical verification of doses or concentrations

no data

Duration of treatment / exposure

No data

Frequency of treatment

No data

Post exposure period

No data

No. of animals per sex per dose

3-4 animals were used.

Control animals

yes

Examinations

Any other information on materials and methods incl. tables

Incorportation of[3H] thymidineinto testicular DNA was expressed as cpm per µg DNA. The mean values were then tested for statistically significant deviations from the concurrent controls by a simplified t test, where only the mean and the extreme values of the sample are needed for the calculation.

Results and discussions

Details on results

Table 1 shows the results obtained for 3,4 dimethylaniline.

Any other information on results incl. tables

Table 1: Inhibition of thymidine incorporation into testicular DNA Dose (mg/kg) and mode of application; Mutagenicity/ carcinogenicitya; Incorporation of tritiated thymidine (% of concurrent control)b. 100 ip; (+/0); 35.1c. a+ = mutagenic/carcinogenic activity, 0 = no data bStatistical significance of deviation from control (=100%) by attest c P < 0.001

Applicant's summary and conclusion

Conclusions

The conclusion of the report is focused on the validity of the assay itself rather than the individual test results. Results conlcude that the assay is a valuable tool in predicting mutagenicity/carcinogenicity but the assay is not very sensitive as large amounts up to the LD50 have to be given to animals to elicit a response. It is therefore nearly impossible to detect the mutagenic or carcinogenic activity of an impurity within an otherwise inactive compound as does the Ames test. Analytical work is thus necessary to detect and identify impurities.It has to be stressed that the assay cannot reveal the true mechanism of cancer induction: but with a correlation between carcinogenic activity and the biological activity in such as test as close t0 100% as possible it may be used as a predictive tool that could enable cancer prevention.

## Endpoint study record: Jones CR & Sabbioni G (2003)

| UUID |  | IUC5-cd695ffa-86f7-4877-a14a-1d42fcb6a6df |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | mariko / National Institute of Health Sciences / Tokyo / Japan |
| Date |  | 2013-02-06 15:20:52 JST |
| Remarks |  |  |

Administrative Data

| Study result type | experimental result |  |  |
| --- | --- | --- | --- |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Meets generally accepted scientific standards with aceptable restrictions. |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Jones CR & Sabbioni G | 2003 | Identification of DNA adducts using HPLC/MS/MS following in vitro and in vivo experiments with arylamines and nitoarenes. | Chem. Res. Toxicol. 16: 1251 - 1263. |  |  |  |  |  |

Data access

data submitter is data owner

Materials and methods

Type of effects studied

toxicogenomics

Type of method

other: In vitro and in vivo.

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | other guideline: No data | no data |

Principles of method if other than guideline

Female wistar rats were given a single oral dose of the test substance and their analogous nitro derivatives by oral gavage 24 h prior to sacrifice. Hepatic NA and in vitro modified DNA were hydrolysed enzymatically to individual 12'-deoxyribonucleosides. Adducts were determined using HPLC/MS/MS by comparison to synthesised standards. The hydrolysis efficiency was monitored by HPLC with UV detection.

GLP compliance

no data

Test materials

Test material identity

| Identifier | Identity |
| --- | --- |
| EC name | 3,4-xylidine |
| CAS number | 65-64-7 |
| EC number | 202-440-0 |

Details on test material

- Name of test material: 3,4-dimethylaniline

Test animals

Species

rat

Strain

Wistar

Sex

female

Details on test animals and environmental conditions

TEST ANIMALS - Source: Zentralinstitut fur Versucgstierzucht (Hannover, Germany) - Weight at study initiation: 200 - 225 g

Administration / exposure

Route of administration

oral: gavage

Vehicle

other: 1,2-propanediol

Details on exposure

No data

Analytical verification of doses or concentrations

no data

Details on analytical verification of doses or concentrations

No data

Duration of treatment / exposure

Single application

Frequency of treatment

Single application

Post exposure period

24 h

Doses / concentrations

| 0.5 mmol/kg |  |
| --- | --- |
| Basis | actual ingested |

No. of animals per sex per dose

Two

Control animals

yes, concurrent vehicle

Examinations

Examinations

After 24 h the animals were anaesthetised with ether and blood (4 - 6 m) was drawn into EDTA tubes by cardiac puncture. The livers were removed, washed with saline solution, then frozen with liquid nitrogen and stored at -20 °C. To confirm the negative results animal experiments were repeated with female Wistar rats at the University of Newcastle. Isolation of erythrocytes from whole blood: Freshly drawn heparinised blood was centrifuged to separate the erythrocytes from teh plasma phase and stored at -20 °C. The erythrocytes were washed with equal volumes of saline solution (0.9 % NaCl) and then stored at -20 °C. Isolation and purification of Hb: An aliquot of thawed erythrocytes was added to ice cold water and maintained at 4 °C for 20 min to facilitate lysis of cellular membranes. The membranes were removed by centrifucation. The supernatant was decanted cto a new tube and precipitated wht EtOH then wasehd with EtOH and EtOH-diethyl ester before drying in a dsiccator over silica gel until a constant weight was attained. The Hb was stored at -20 °C until analysis. Determination of Hb adducts: The corresponding internal standard was added to the Hb, gently shaken for 1 h at room temperature and the hydrolysate was extracted with hexane and analysis performed on a Hewlett-Packard chromatograph equipped with autosampler and interfaced to a mass spectrometer.

Results and discussions

Details on results

DNA adducts were not detected in rats dosed with the test substance.

Applicant's summary and conclusion

Conclusions

DNA adducts were not detected in rats dosed with the test substance.

# 7.10 Exposure related observations in humans

# 7.10.2 Epidemiological data

## Endpoint study record: Gan J et al (2004)

| UUID |  | IUC5-75398d1b-1594-4af9-99c7-fdff72963d94 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | mariko / National Institute of Health Sciences / Tokyo / Japan |
| Date |  | 2013-02-06 15:20:53 JST |
| Remarks |  |  |

Administrative Data

| Study result type | experimental result |  |  |
| --- | --- | --- | --- |
| Reliability | other: Not applicable to an epidmeiology study. |  |  |
| Rationale for reliability incl. deficiencies | Not applicable to an epidmeiology study. |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Gan J, Skipper PL, Gag-Dominguez M, Arakawa K, Ross RK, Yu MC & Tannenbaum SR. | 1994 | Alkylaniline-hemoglobin adducts and risk of non-smoking-related bladder cancer. | Journal of the National Cancer Institute 96 (19) 1425 - 1431. |  |  |  |  |  |

Data access

data published

Materials and methods

Study type

case control study (retrospective)

Endpoint addressed

carcinogenicity

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | other guideline: Not applicable | not applicable |

Principles of method if other than guideline

A population-based case-control study was performed in Los Angeles County, California involving 298 case subjects with bladder cancer and 308 control subjects who were matched on age, sex, race/ethnicity and neighbourhood of residence. In-person interviews provided information on tobacco smoking and other potential risk factors for bladder cancer. To assess arylamine exposure, levels of arylamine-hemoglobin adducts of nine selected alkylanilines were measured in peripheral blood collected from study subjects. Analysis of covariance and conditional logistic regression methods were used to analyse the relationship between arylamine-hemoglobin adducts and bladder cancer.

GLP compliance

no data

Test materials

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 95-68-1 |
| EC number | 202-437-4 |
| EC name | 3,4-xylidine |

Details on test material

- Name of test material: 2,3-dimethylaniline.

Method

Type of population

other: Bladder cancer patients

Ethical approval

no data

Details on study design

HYPOTHESIS TESTED (case control study): METHOD OF DATA COLLECTION - Type: Interview - Details: Structured interviews were conducted by study personnel in each subject's home. During the interview, subjects were asked to provide information up to two years before the diagnosis of bladder cancer for case subjects or for two years before the diagnosis of cancer o the index case subject for the matched control subject (i.e. reference year). Information was requested on demographic characteristics, lifetime use of tobacco products and alcohol, usual dietary habits, lifetime occupational history, previous medical conditions and previous medication use. STUDY POPULATION - Total population (Total no. of persons in cohort from which the subjects were drawn): Figure not specified; enrolled case subjects represented approximately 75 % of the total number of eligible patients. - Selection criteria: Subjects with bladder cancer identified through the Los Angeles County Cancer Surveillance Program (a population-based Surveillance, Epidemiology and End Results (SEER) program cancer registry of Los Angeles County. - Total number of subjects participating in study: 1679 - Sex/age/race: Non-Asian race/ethnicity, 25-64 years of age diagnosed with histologically confirmed cancer between January 1st 1987 and April 30th 1996. - Smoker/nonsmoker: - Total number of subjects at end of study: - Matching criteria: - Other: COMPARISON POPULATION - Type: Other comparison group: An invariant procedure was followed to recruit a control subject from the same neighbourhood of residence as the index case at the time of cancer diagnosis who matched the index case by age (within 5 years), sex, race/ethnicity. - Details: HEALTH EFFECTS STUDIED - Disease(s): Bladder cancer - ICD No.: - Year of ICD revision: - Diagnostic procedure: - Other health effects: OTHER DESCRIPTIVE INFORMATION ABOUT STUDY: A matched control could not be found for 5% (n = 86) of enrolled case subjects. For 21 index case patients the control subject was not matched by race/ethnicity. All subjects provided written informed consent (separate forms for interview and blood/urine donation) that had been approved by the institutional review boards of the University of Southern California and the Massachusetts Institute of Technology. Beginning in January 1992, all case subjects (n = 1044) and their matched control subjects (n = 979) were asked to provide a blood sample at the end of the in-person interview. Blood was obtained and successfully analysed from 761 (73 %) case subjects from 770 (79 %) control subjects. All subjects who donated blood were asked detailed questions on their use of their use of tobacco products during the preceding 2 months. All subjects were asked to collect an over night urine sample (end with the first morning void) after consuming two cups of instant coffee (approximately 70 mg of caffeine) in the previous afternoon (between 3 and 6 pm). Caffeine tablets were offered as an alternative to non-coffee drinkers. From the 1536 subjects who donated blood, 606 were selected for the study of alkylaniline-hemoglobin adducts. This subset of study participants provided the last 606 samples to be analysed . The analytical method was not available for the first 930 subjects. Of the 606 subjects, 298 were case subjects and 308 were control subjects. The study included 190 matched pairs, 108 unmatched case subjects and 118 unmatched control subjects.

Exposure assessment

no data

Details on exposure

No data

Statistical methods

The distributions of the alkylaniline-hemoglobin adduct levels in the study population were markedly skewed; therefore formal statistical testing was performed on logarithmically transformed values of adduct levels, and geometric (as opposed to arithmetic) mean values were presented. Pair-wise Pearson correlation coefficients (on logarithmically transformed values) were calculated for the nine adducts under study to examine the extent for which they were associated with The analysis of covariance method was used to compare adduct levels between case and control subjects, with an adjustment for the effects of cigarette smkoing at the time of blood draw on adduct levels. The analysis of covariance method was also used to compare adduct levels between exclusive users of permanent hair dyes and non-users of permanent hair dyes, with an adjustment for the effect of cigarette smoking at the time of blood draw on adduct levels. Additional adjustments were also made for other risk factors for bladder cancer identified in the Los Angeles Bladder Cancer Study. These covariates included educational level, lifetime history of cigarette smoking, regular use of non-steriodal anti-inflammatory drugs, use of permanent hair dyes, dietary intake of carotenoids, 4-ABP-hemoglobib adducts, NAT2 phenotype and GSTM1 genotype. Results of analyses with and without adjustment for ethnicity/race were simillar therefore race/ethnicity was not retained as a covariate. From all alkylanilines that had a statistically significant univariate association with bladder cancer risk, a stepwise regression method was used to identify the set of alkylanilines that were independent predictors of risk. The backward and forward selection methods yielded the same set of independent risk predictors. Conditional logic regression models were used to examine the relationship between quartiles of adduct levels and bladder cancer risk. Odds ratios were used to estimate relative risks. All P-values were two-sided.

Any other information on materials and methods incl. tables

Laboratory analyses: Blood specimens were collected in heparinized tubes and were fractionated into plasma, buffy coat and erythrocytes, all of which were stored at -80 °C before analyses. Erythrocytes, identifiable only by their code numbers were sent on dry ice to the Massachusetts Institute of Technology for arylamine-haemoglobin adduct analysis. Haemoglobin adducts of nine alkylanilines were subjected to quantitative analysis.

Results and discussions

Results

With the exception of 2,6-DMA, all alkylaniline-haemoglobin adducts showed positive and relatively close correlations with each other. Mean levels of alkylaniline-haemoglobin adducts were higher in smokers than in non smokers, with the exception of 2,6-DMA alkylaniline-haemoglobin adducts. Adducts of alkylaniline were higher in case studies than in control subjects. Levels of 2,3-DMA-, 2,4-DMA-, 2,6-DMA- and 3,5-DMA- adducts were statistically higher in non-smoking case subjects than in smoking control subjects. 2,6-DMA and 3,5-DMA were identified as independent predictors of bladder cancer and there was a statistically significant increase in risk associated with an increase in adduct levels for each of these. The increased risk persisted when the analysis was restricted to nonsmokers blood draw. Among non-smokers, individuals in the highest quartiles of 2,6-DMA- and 3,5-DMA-haemoglobin adduct levels were three to five times more likely to develop bladder cancer than individuals in the lowest quartiles. Relative risk for 2,6-DMA was 5.5 (95% CI = 2.8 - 10.7); relative risk for 3,5-DMA was 3.1 % (CI =1.6 - 6.0). These increased risks persisted after adjustment for other risk factors identified in the study population. It had been speculated previously that the carcinogenic agents in hair dyes may be arylamines, therefore the levels of nine-alkylaniline-haemoglobin adducts according to sex were assessed, with adjustments for age at blood draw and smoking status at the time of blood draw. Only levels of 3,5-DMA demonstrated a statistically significant difference (P = 0.04) with levels higher in women than in men (adjusted geometric mean27.8 pg/g of haemoglobin in women versus23.3 pg/g of haemoglobin in men). The levels of 3,5-DMA-haemoglobin adduct according to use of permanent hair dye, adjusting for the number of cigarettes smoked per day were assessed. The adjusted geometric mean level of 3,5-DMA-haemoglobin adducts was higher in exclusive users of permanent dye than in non-users. The difference was statistically significant when the analysis was restricted to women.

Confounding factors

No data

Strengths and weaknesses

No data

Applicant's summary and conclusion

Conclusions

A population-base case-control study was performed. Levels of all arylamine-haemoglobin adducts, with the exception of 2,6-DMA were higher in smokers than in non-smokers and levels of arylamine-haemoglobin adducts were higher in case subjects than in control subjects. Arylamine-haemoglobin adducts of 2,6-DMA and 3,5-DMA were independently statistically significantly (P<0.001) associated with bladder cancer risk after adjusting for cigarette smoking at the time of blood collection, lifetime smoking history and other potential risk factors. These adducts were also independently associated with bladder cancer risk when only non-smokers were considered.

## Endpoint study record: Bryant MS et al (1988)

| UUID |  | IUC5-52a83914-2613-43ed-97a5-6b019b24537d |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | mariko / National Institute of Health Sciences / Tokyo / Japan |
| Date |  | 2013-02-06 15:20:53 JST |
| Remarks |  |  |

Administrative Data

| Study result type | experimental result |  |  |
| --- | --- | --- | --- |
| Reliability | other: Not applicable for epidemiology data |  |  |
| Rationale for reliability incl. deficiencies | Not applicable for epidemiology data |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Bryant MS, Vineis P, Skipper PL & Tannenbaum | 1988 | Hemoglobin adducts of aromatic amines: Associations with smoking status and type of tobacco | Proc. Natl. Acad. Sci. USA. 85: 9788-9791 |  |  |  |  |  |

Data access

data published

Materials and methods

Study type

cohort study (retrospective)

Endpoint addressed

carcinogenicity

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | other guideline: No data | no data |

Principles of method if other than guideline

Hemoglobin adducts of 15 aromatic amines (one of which included 3,4 dimethylaniline) were determined in nonsmokers and smokers of blond or black tobacco cigarettes living in Turin, Italy. Subjects were all males age 55 or less and were representative of the population previously examined in a case/control study of bladder cancer.

GLP compliance

no data

Test materials

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 95-64-7 |
| EC number | 202-437-4 |
| EC name | 3,4-xylidine |

Details on test material

- Name of test material: 3,4 dimethylaniline

Method

Type of population

general

Details on study design

HYPOTHESIS TESTED (if cohort or case control study): The study describes results of a study conducted on residents in the city of Turin, Italy, in which hemoglobin adducts of 4-aminobiphenyl and other aromatic amines were measured in a population previously studied with regard to bladder cancer risk. A putative etiologic role for this class of compounds in the bladder carcinogenicity of tobacco smoke could thus be examined. METHOD OF DATA COLLECTION - Type:Questionnaire / Clinical tests : - Details: Male volunteers were recruited at a blood donor center in the city of Turin, Italy to donate an additional 10 ml blood sample and fill out a questionnaire on their smoking history and other parameters such as occupation. During two collection periods, 87 samples were collected from 25 non-smokers, 40 blond-tobacco smokers and 3 mixed blond/black-tobacco smokers. SETTING: Blood donor center STUDY POPULATION - Total population: 25 nonsmokers and 61 smokers - Selection criteria: Subjects were representative of the resource population previosuly used in a case/control study for bladder cancer. - Sex/age: Males age 55 or less - Smoker/nonsmoker: Smokers and non-smokers - Matching criteria: The mixed blond/black tobacco smokers were grouped with the blond-tobacco users. HEALTH EFFECTS STUDIED - Disease: Bladder cancer

Exposure assessment

no data

Statistical methods

Adjustments of adduct levels by age of the subjects or by amount smoked as continuous variables were done with the least-squares method using a linear model. Correlations with smoking status were determined by comparing the 25 nonsmokers were the 61 smokers. Statistical significance was determined with a Wilcoxon two-sample test. Correlations with tobacco type were determined by comparing the 42 smokers of blond tobacco with the 18 smokers of black tobacco

Any other information on materials and methods incl. tables

Measurement of aromatic amine-hemoglobin adducts: Erythrocytes were washed with saline, frozen and then coded later for analysis. All analyses were performed without prior knowledge of sample identity.

Results and discussions

Results

Table 1 shows the P values for the association of adduct levels with either smoking status (smoker versus non smoker) or tobacco type (black tobacco versus blond tobacco) with 3,4-dimethylaniline.

Confounding factors

None of the subjects was involved in occupations known to be associated with aromatic amine exposure. Furthermore, no occupation was associated with any of the three comparison groups (nonsmokers and blond or black tobacco smokers). Thus occupation is unlikely to be a confounding factor.

Strengths and weaknesses

No data

Any other information on results incl. tables

Table 1: Hemoglobin adduct levels of 3,4 dimethylaniline: Associations with smoking status and tobacco type Mean adduct level, pg of amine per g of Hb (SEM); Pfor association. Non smokers; Smokers. Blond tobacco; Black tobacco; Smoking status; Tobacco type. 47 (11); 46 (11); 65 (19); 0.71; 0.045.

Applicant's summary and conclusion

Conclusions

The results of the study indicate that measurement of the hemoglobin adducts may be useful in examining the biochemical basis for increased risk of bladder cancer in smokers.

# 14 Information requirements

# 14.2 Alternative name request

# Reference substance: 3,4-xylidine

| UUID |  | ECB5-944a4c04-2fa8-4dc5-923e-b3cd048e325d |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V3.0 Plug-In |
| Date |  | 2007-05-10 18:00:00 JST |
| Remarks |  | Successfully migrated to IUCLID 5.4 format. |

General information

| Reference substance name | 3,4-xylidine |
| --- | --- |

EC inventory

| EC number | 202-437-4 | CAS number | 95-64-7 |
| --- | --- | --- | --- |
| EC name | 3,4-xylidine |  |  |
| Molecular formula | C8H11N |  |  |

Reference substance information

CAS information

| CAS number | 95-64-7 |
| --- | --- |

IUPAC name

| 3,4-dimethylaniline |  |
| --- | --- |

Synonyms

| Name | Benzenamine, 3,4-dimethyl- |
| --- | --- |

Molecular and structural information

| Molecular formula | C8H11N |
| --- | --- |
| Molecular weight range | 121.1796 |
| SMILES notation | Cc1ccc(N)cc1C |
| InChI | InChI=1/C8H11N/c1-6-3-4-8(9)5-7(6)2/h3-5H,9H2,1-2H3 |
| Structural formula |  |

# Legal entity: National Institute of Health Sciences

| UUID |  | IUC4-b036ff75-0f3c-323b-b200-ed5f46cf5101 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V3.0 Plug-In |
| Date |  | 2009-01-09 13:32:30 JST |
| Remarks |  | Successfully migrated to IUCLID 5.4 format. |

General information

| Legal entity name | National Institute of Health Sciences |
| --- | --- |

Identifiers

Other IT system identifiers

| Flags | IT system | ID | Remarks |
| --- | --- | --- | --- |
|  | LEO | 10767 |  |
|  | IUCLID4 | 16558402024DIV750 |  |

Contact information

Contact address

| Address | 1-18-1 kamiyoga |
| --- | --- |
| Address | Setagaya-ku |
| Postal code | 158-8501 |
| Town | Tokyo |
| Country | Japan |

Contact persons

| Organisation | National Institute of Health Sciences |
| --- | --- |
| Department | Division of Risk Assessment |
| Title | Dr. |
| First name | Akihiko |
| Last name | Hirose |
| Address | 1-18-1 Kamiyoga |
| Address | Setagaya-ku |
| Postal code | 158-8501 |
| Town | Tokyo |
| Country | Japan |