SIDS Dossier

OECD HPV Chemical Programme, SIDS Dossier, approved at CoCAM 3 (16/10/2012)

| Printing Date | 2013-02-06 14:23:55 JST |
| --- | --- |

Restriction of specific regulatory purposes

Confidentiality

| Name | (JP_CoCAM3) 2,5-Dimethylaniline |
| --- | --- |
| Legal entity owner | National Institute of Health Sciences / Tokyo / Japan |

# Substance: (JP_CoCAM3) 2,5-Dimethylaniline

| UUID |  | IUC5-c9ca88a4-a76b-4e51-adfc-e0937be5decf |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | mariko / National Institute of Health Sciences / Tokyo / Japan |
| Date |  | 2013-02-06 14:23:46 JST |
| Remarks |  |  |

# 0 Related Information

# 0.1 Templates

# 0.2 Categories

# 0.3 Mixtures

# 1 General Information

# 1.1 Identification

Substance identification

| Chemical name | (JP_CoCAM3) 2,5-Dimethylaniline |
| --- | --- |
| Legal entity | National Institute of Health Sciences / Tokyo / Japan |

Reference substance

| 2,5-xylidine / 2,5-dimethylaniline / 95-78-3 |  |
| --- | --- |
| EC number; EC name. 202-451-0; 2,5-xylidine. CAS number; CAS name. 95-78-3. IUPAC name. 2,5-dimethylaniline. |  |

Type of substance

| Composition | mono constituent substance |
| --- | --- |
| Origin | organic |

# 1.2 Composition

# 1.3 Identifiers

# 1.4 Analytical information

# 1.5 Joint submission

# 1.6 Sponsors

# 1.7 Suppliers

# 1.8 Recipients

# 1.9 Product and process oriented research and development

# 2 Classification & Labelling and PBT assessment

# 2.1 GHS

# 2.2 DSD - DPD

# 3 Manufacture, use and exposure

# 3.1 Technological process

Technological process

Methods of manufacture of substance; 2,5-Dimethylaniline is commercially produced by the following method: 1) nitrating and reducing commercial xylene to an isomeric mixture of xylidines, 2) after 2,4-xylidine is removed by formation of the acetate salt, 2,5-xylidine hydrochloride is precipitated by the addition of hydrochloric acid and this can be converted to 2,5-xylidine by the addition of base..

# 3.2 Estimated quantities

Estimated quantities

Remarks. No detailed information is obtained on 2,5-dimethylaniline on the production and import volume in Japan. Production and/or import volume on 2,5-dimethylaniline in USA was less than 500,000 pounds (227 tonnes) in 2006 according to IUR information by US EPA (US EPA, 2012). Worldwide production volume is not available..

# 3.3 Sites

# 3.4 Information on mixtures

# 3.5 Life Cycle description

Identified uses

Uses at industrial sites

Identified Use number. Identified Use name; 2,5-Dimethylaniline is used as a chemical intermediate for the synthesis of Direct Yellow 51, Solvent Red 22, Solvent Red 26 and used in the manufacture of p-xyloquinone; Direct Violet 7.. Subsequent service life relevant for that use?; yes.

# 3.6 Uses advised against

# 3.7 Exposure Scenarios, exposure and risk assessment

# 3.7.2 Environmental assessment for aggregated sources

# 3.7.3 Generic exposure potential

# 3.8 Biocidal information

# 3.10 Application for authorisation of uses

# 4 Physical and chemical properties

# 4.1 Appearance/physical state/colour

## Endpoint study record: Literature -1

| UUID |  | IUC5-295ccdbe-f393-4748-9b4a-3debe1c5e151 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V3.0 Plug-In |
| Date |  | 2011-02-17 20:24:56 JST |
| Remarks |  | Successfully migrated to IUCLID 5.4 format. |

Administrative Data

| Purpose flag | supporting study; robust study summary |
| --- | --- |
| Reliability | 2 (reliable with restrictions) |
| Rationale for reliability incl. deficiencies | Reliable handbook |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| review article or handbook | Lide D.R. (ed) | 2008 | CRC Handbook of Chemistry and Physics, version 2008 |  |  |  |  |  |  |

Data access

data published

Results and discussion

Form

liquid

Colour

Yellow

Substance type

organic

Applicant's summary and conclusion

Conclusions

2,5-Dimethylaniline is a light yellow liquid.

## Endpoint study record: Literature -2

| UUID |  | IUC5-d8fca215-5628-4645-b5e0-2e3c8aac032b |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V3.0 Plug-In |
| Date |  | 2011-02-17 20:24:45 JST |
| Remarks |  | Successfully migrated to IUCLID 5.4 format. |

Administrative Data

| Purpose flag | supporting study; robust study summary |
| --- | --- |
| Reliability | 2 (reliable with restrictions) |
| Rationale for reliability incl. deficiencies | Reliable literature |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| review article or handbook | Seidel A. (ed) | 2004 | Kirk-Othmer Encyclopedia of Chemical Technology, fifth edition, volume 2 |  |  |  |  |  |  |

Data access

data published

Results and discussion

Physical state at 20°C and 1013 hPa

liquid

Form

other: oily

Substance type

organic

Applicant's summary and conclusion

Conclusions

2,5-Dimethylaniline is oily liquid.

## Endpoint study record: Experiment

| UUID |  | IUC5-3abc70a3-99d5-47a7-a463-401f835736cf |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | CERI_S.MIYACHI / Chemicals Evaluation and Research Institute (CERI) / Tokyo / Japan |
| Date |  | 2012-11-05 11:01:37 JST |
| Remarks |  |  |

Administrative Data

| Purpose flag | key study |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result | Study period | 20/6/1988 - 22/6/1988 |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Reliable test report |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | Chemicals Evaluation and Research Institutet, Japan | 1988 | Measurement of physical-chemical property on 2,5-dimethylaniline |  | Chemicals Evaluation and Research Institutet, Japan | 81025K |  |  |  |

Materials and methods

GLP compliance

no

Results and discussion

Physical state at 20°C and 1013 hPa

liquid

Colour

Coulourless, tranparent

Substance type

organic

Applicant's summary and conclusion

Conclusions

2,5-Dimethylaniline is colourless, tranparent liquid.

# 4.2 Melting point/freezing point

## Endpoint study record: Literature -1

| UUID |  | IUC5-5c9d33ea-17d5-460a-ac31-22064f8685e4 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V3.0 Plug-In |
| Date |  | 2011-02-17 17:59:33 JST |
| Remarks |  | Successfully migrated to IUCLID 5.4 format. |

Administrative Data

| Purpose flag | key study; robust study summary |
| --- | --- |
| Reliability | 2 (reliable with restrictions) |
| Rationale for reliability incl. deficiencies | Reliable literature |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| review article or handbook | Lide D.R. (ed) | 2008 | CRC Handbook of Chemistry and Physics, version 2008 |  |  |  |  |  |  |

Data access

data published

Results and discussions

Melting / freezing point

| Melt./Freez. pt. | Atm. pressure | Decomposition | Decomp. temp. | Sublimation | Subl. temp. | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 15.5 °C |  |  |  |  |  |  |

## Endpoint study record: Literature -2

| UUID |  | IUC5-ee5612bf-4ba6-4e65-ad34-0986c6a7bc6f |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V3.0 Plug-In |
| Date |  | 2011-02-17 19:59:55 JST |
| Remarks |  | Successfully migrated to IUCLID 5.4 format. |

Administrative Data

| Purpose flag | supporting study; robust study summary |
| --- | --- |
| Reliability | 2 (reliable with restrictions) |
| Rationale for reliability incl. deficiencies | Reliable literature |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| review article or handbook | Speight J.G. | 2005 | Lange's Handbook of Chemistry, 16th edition |  |  |  |  |  |  |

Data access

data published

Results and discussions

Melting / freezing point

| Melt./Freez. pt. | Atm. pressure | Decomposition | Decomp. temp. | Sublimation | Subl. temp. | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 15.5 °C |  |  |  |  |  |  |

Applicant's summary and conclusion

Conclusions

Melting point is 15.5 degree C.

## Endpoint study record: Literature -3

| UUID |  | IUC5-6692e59f-5362-4941-83ff-91aadda291a3 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V3.0 Plug-In |
| Date |  | 2011-02-17 20:42:20 JST |
| Remarks |  | Successfully migrated to IUCLID 5.4 format. |

Administrative Data

| Purpose flag | supporting study; robust study summary |
| --- | --- |
| Reliability | 2 (reliable with restrictions) |
| Rationale for reliability incl. deficiencies | Reliable literature |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| review article or handbook | Seidel A. (ed) | 2004 | Kirk-Othmer Encyclopedia of Chemical Technology, fifth edition, volume 2 |  |  |  |  |  |  |

Data access

data published

Results and discussions

Melting / freezing point

| Melt./Freez. pt. | Atm. pressure | Decomposition | Decomp. temp. | Sublimation | Subl. temp. | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 15.5 °C |  |  |  |  |  |  |

Applicant's summary and conclusion

Conclusions

Melting point is 15.5 degree C.

# 4.3 Boiling point

## Endpoint study record: Literature -1

| UUID |  | IUC5-c32f934c-b448-4003-926c-83fc5756a58b |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V3.0 Plug-In |
| Date |  | 2011-02-17 17:59:20 JST |
| Remarks |  | Successfully migrated to IUCLID 5.4 format. |

Administrative Data

| Purpose flag | key study; robust study summary |
| --- | --- |
| Reliability | 2 (reliable with restrictions) |
| Rationale for reliability incl. deficiencies | Reliable literature |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Lide D.R. (ed) | 2008 | CRC Handbook of Chemistry and Physics, version 2008 |  |  |  |  |  |  |

Data access

data published

Results and discussions

Boiling point

| Boiling pt. | Atm. pressure | Decomposition | Decomp. temp. | Remarks |
| --- | --- | --- | --- | --- |
| 214 °C |  |  |  |  |

Applicant's summary and conclusion

Conclusions

Boiling point is 215 degree C.

## Endpoint study record: Literature -2

| UUID |  | IUC5-7ddf622b-77be-4cf2-b2ac-753889635b8c |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V3.0 Plug-In |
| Date |  | 2011-02-17 20:01:22 JST |
| Remarks |  | Successfully migrated to IUCLID 5.4 format. |

Administrative Data

| Purpose flag | supporting study; robust study summary |
| --- | --- |
| Reliability | 2 (reliable with restrictions) |
| Rationale for reliability incl. deficiencies | Reliable literature |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| review article or handbook | Speight J.G. | 2005 | Lange's Handbook of Chemistry, 16th edition |  |  |  |  |  |  |

Data access

data published

Results and discussions

Boiling point

| Boiling pt. | Atm. pressure | Decomposition | Decomp. temp. | Remarks |
| --- | --- | --- | --- | --- |
| 214 °C |  |  |  |  |

Applicant's summary and conclusion

Conclusions

Boiling point is 214 degree C.

## Endpoint study record: Literature -3

| UUID |  | IUC5-c9519537-61fc-4b7a-9d12-855b9a510878 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V3.0 Plug-In |
| Date |  | 2011-02-17 20:43:45 JST |
| Remarks |  | Successfully migrated to IUCLID 5.4 format. |

Administrative Data

| Purpose flag | supporting study; robust study summary |
| --- | --- |
| Reliability | 2 (reliable with restrictions) |
| Rationale for reliability incl. deficiencies | Reliable literature |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| review article or handbook | Seidel A. (ed) | 2004 | Kirk-Othmer Encyclopedia of Chemical Technology, fifth edition, volume 2 |  |  |  |  |  |  |

Data access

data published

Results and discussions

Boiling point

| Boiling pt. | Atm. pressure | Decomposition | Decomp. temp. | Remarks |
| --- | --- | --- | --- | --- |
| 213.5 °C |  |  |  |  |

Applicant's summary and conclusion

Conclusions

Boiling point is 213.5 degree C.

# 4.4 Density

## Endpoint study record: Literature -1

| UUID |  | IUC5-cd449d46-2217-4da1-a300-4e613d3cc925 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V3.0 Plug-In |
| Date |  | 2011-02-17 18:04:07 JST |
| Remarks |  | Successfully migrated to IUCLID 5.4 format. |

Administrative Data

| Purpose flag | key study; robust study summary |
| --- | --- |
| Reliability | 2 (reliable with restrictions) |
| Rationale for reliability incl. deficiencies | Reliable literature |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| review article or handbook | Lide D.R. (ed) | 2008 | CRC Handbook of Chemistry and Physics, version 2008 |  |  |  |  |  |  |

Data access

data published

Results and discussion

Density

| Type | Density | Temp. | Remarks |
| --- | --- | --- | --- |
| density | 0.979 g/cm³ | 21 °C |  |

Applicant's summary and conclusion

Conclusions

Density is 0.9790 g/cm3 at 21 degree C.

# 4.6 Vapour pressure

## Endpoint study record: Literature -1

| UUID |  | IUC5-01311cea-70bb-4c7b-9d24-ad0569c00016 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | CERI_S.MIYACHI / Chemicals Evaluation and Research Institute (CERI) / Tokyo / Japan |
| Date |  | 2012-11-05 10:20:46 JST |
| Remarks |  |  |

Administrative Data

| Purpose flag | key study; robust study summary |
| --- | --- |
| Reliability | 2 (reliable with restrictions) |
| Rationale for reliability incl. deficiencies | Reliable literature |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| review article or handbook | Howard P.H. and Meylan W.M. (ed) | 1997 | Handbook of Physical Properties of Organic Chemicals |  |  |  |  |  |  |

Results and discussions

Vapour pressure

| 20 Pa |  |
| --- | --- |
| 20 °C |  |
| Remarks | 1.5E-1 mmHg |

Overall remarks, attachments

Remarks on results including tables and figures

Original reference is “Weber RC et al; Vapor Pressure Distribution of Selected Organic Chemicals. USEPA-600/2-81-021 p. 16 (1981)”

Applicant's summary and conclusion

Conclusions

Vapour puressure is 20.0 Pa at 20 degree C.

# 4.7 Partition coefficient

## Endpoint study record: Literature -1

| UUID |  | IUC5-4bdb50be-54fe-4d9a-a604-e7d408fdeaf8 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V3.0 Plug-In |
| Date |  | 2011-02-17 21:35:07 JST |
| Remarks |  | Successfully migrated to IUCLID 5.4 format. |

Administrative Data

| Purpose flag | supporting study; robust study summary |
| --- | --- |
| Reliability | 2 (reliable with restrictions) |
| Rationale for reliability incl. deficiencies | Reliable literature |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| review article or handbook | Hansch C., Leo A. and Hekman D. | 1995 | Exploring QSAR ？ Hydrophobic, Electronic, and Steric Constants |  |  |  |  |  |  |

Data access

data published

Materials and methods

Partition coefficient type

octanol-water

Results and discussions

Partition coefficient

| Type | Partition coefficient | Temp. | pH | Remarks |
| --- | --- | --- | --- | --- |
| log Pow | 1.83 |  | 7.4 |  |

Applicant's summary and conclusion

Conclusions

Partition coefficient between otanol and water (log Kow) is 1.83.

## Endpoint study record: Experiment

| UUID |  | IUC5-6ba0366c-2450-4882-bc56-b77d6aa09fd1 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | CERI_S.MIYACHI / Chemicals Evaluation and Research Institute (CERI) / Tokyo / Japan |
| Date |  | 2012-11-05 10:04:04 JST |
| Remarks |  |  |

Administrative Data

| Purpose flag | key study |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result | Study period | 14/10/1988 - 23/10/1988 |
| Reliability | 1 (reliable without restriction) |  |  |
| Rationale for reliability incl. deficiencies | Test was conducted according to OECD test guideline in compliance with GLP. |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | Chemicals Evaluation and Research Institutet, Japan | 1988 | Measurement of partition coefficient between 1-octanol and water on 2,5-dimethylaniline |  | Chemicals Evaluation and Research Institutet, Japan | 81025K |  |  |  |

Data access

data published

Materials and methods

Partition coefficient type

octanol-water

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | OECD Guideline 107 (Partition Coefficient (n-octanol / water), Shake Flask Method) | no |

Type of method

shake-flask method

GLP compliance

yes (incl. certificate)

Test materials

Details on test material

- Name of test material (as cited in study report): 2,5-dimethylaniline - Analytical purity: > 97 % - Impurities (identity and concentrations): Unknown - Lot/batch No.: FBY01 - Expiration date of the lot/batch: Not known - Stability under test conditions: Confirmed by the IR spectra before and after the experiment - Storage condition of test material: Dark cool place

Any other information on materials and methods incl. tables

Condition 1: Octanol 5 mL/Water 45 mL Condition 2: Octanol 10 mL/Water 40 mL Condition 3: Octanol 20 mL/Water 30 mL

Results and discussions

Partition coefficient

| Type | Partition coefficient | Temp. | pH | Remarks |
| --- | --- | --- | --- | --- |
| log Pow | 1.91 | 25 °C | 6.2 — 6.4 |  |

Any other information on results incl. tables

Results are as follows. Condition; Log Kow; Average of Log Kow; pH in the water phase. Condition -1; a; 2.00; 1.91; 6.4. b; 1.99; 6.4. Condition -2; a; 1.91; 6.4. b; 1.90; 6.4. Condition -3; a; 1.84; 6.2. b; 1.84; 6.3. Total average of Log Kow: 1.91

Applicant's summary and conclusion

Conclusions

Partition Coefficient between 1-octanol and water (Log Kow) is 1.91.

# 4.8 Water solubility

## Endpoint study record: Literature -1

| UUID |  | IUC5-7d8b5369-6e77-434b-8387-af7aae12276d |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V3.0 Plug-In |
| Date |  | 2011-02-17 18:06:43 JST |
| Remarks |  | Successfully migrated to IUCLID 5.4 format. |

Administrative Data

| Purpose flag | supporting study; robust study summary |
| --- | --- |
| Reliability | 2 (reliable with restrictions) |
| Rationale for reliability incl. deficiencies | Reliable literature |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| review article or handbook | Lide D.R. (ed) | 2008 | CRC Handbook of Chemistry and Physics, version 2008 |  |  |  |  |  |  |

Data access

data published

Results and discussions

Water solubility

| 6.6 g/L |  |
| --- | --- |
| Temp. | 20 °C |

Applicant's summary and conclusion

Interpretation of results

soluble (1000-10000 mg/L)

Conclusions

Water solubility is 6.6 g/L at 20 degree C.

## Endpoint study record: Experiment

| UUID |  | IUC5-0425c0ea-f4cb-4e36-83e4-94758765079a |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | CERI_S.MIYACHI / Chemicals Evaluation and Research Institute (CERI) / Tokyo / Japan |
| Date |  | 2012-11-02 22:26:28 JST |
| Remarks |  |  |

Administrative Data

| Purpose flag | key study; robust study summary |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result | Study period | 20/6/1998 - 22/6/1998 |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Test was conducted according to an equivalent protocol of OECD test guideline. |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | Chemicals Evaluation and Research Institutet, Japan | 1988 | Measurement of physical-chemical property on 2,5-dimethylaniline |  | Chemicals Evaluation and Research Institutet, Japan | 81025K |  |  |  |

Materials and methods

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| equivalent or similar to | OECD Guideline 105 (Water Solubility) |  |

Type of method

flask method

GLP compliance

no

Test materials

Details on test material

- Name of test material (as cited in study report): 2,5-dimethylaniline - Analytical purity: > 97 % - Lot/batch No.: FBY01

Details on methods

0.4 g of the test item and 40 mL of purified water are added to a test vessel (n=2). Then this test vessel was shaken for 12 hours at 30 +/- 1 degree C. Then this test vessel was shaken for 24 hours with 25 +/- 1 degree C. Then the concentration of the test item in the test water was decieded after the filtration by HPLC.

Results and discussions

Water solubility

| 5.6 g/L |  |
| --- | --- |
| Temp. | 25 °C |

Details on results

Result a: 5.5 g/L b: 5.7 g.L Average: 5.6 g/L

Applicant's summary and conclusion

Interpretation of results

soluble (1000-10000 mg/L)

Conclusions

Water solubility is 5.6 g/L at 25 degree C.

# 4.21 Dissociation constant

## Endpoint study record: Literature -1

| UUID |  | IUC5-4bf90e09-bb27-4744-85dc-9f4953c666dc |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V3.0 Plug-In |
| Date |  | 2011-02-17 21:03:30 JST |
| Remarks |  | Successfully migrated to IUCLID 5.4 format. |

Administrative Data

| Purpose flag | supporting study; robust study summary |
| --- | --- |
| Reliability | 2 (reliable with restrictions) |
| Rationale for reliability incl. deficiencies | Reliable literature |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| review article or handbook | Speight J.G. | 2005 | Lange's Handbook of Chemistry, 16th edition |  |  |  |  |  |  |

Data access

data published

Results and discussions

Dissociating properties

yes

Dissociation constant

| No. | pKa | at | Remarks |
| --- | --- | --- | --- |
|  | 4.53 | 25 °C |  |

Applicant's summary and conclusion

Conclusions

Dissociaton constant in water (pKa) is 4.53 at 25 degree C.

## Endpoint study record: Experiment

| UUID |  | IUC5-5f68dd80-7dc4-48eb-8654-5075a3f2af84 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | CERI_S.MIYACHI / Chemicals Evaluation and Research Institute (CERI) / Tokyo / Japan |
| Date |  | 2012-11-05 10:00:05 JST |
| Remarks |  |  |

Administrative Data

| Purpose flag | key study |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result | Study period | 11/10/1988 - 23/10/1988 |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Test was conducted according to an equivalent protocol of OECD test guideline. |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | Chemicals Evaluation and Research Institutet, Japan | 1988 | Measurement of dissociation constant in water on 2,5-dimethylaniline |  | Chemicals Evaluation and Research Institutet, Japan | 81025K |  |  |  |

Materials and methods

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| equivalent or similar to | OECD Guideline 112 (Dissociation Constants in Water) (Titration method) |  |

GLP compliance

no

Test materials

Details on test material

- Name of test material (as cited in study report): 2,5-dimethylaniline - Analytical purity: > 99 % - Lot/batch No.: FBY01

Results and discussions

Dissociating properties

yes

Dissociation constant

| No. | pKa | at | Remarks |
| --- | --- | --- | --- |
|  | 4.51 | 25 °C |  |

Overall remarks, attachments

Remarks on results including tables and figures

Dissociation constant a: 4.48 b: 4.50 c: 4.55 Average: 4.51

Applicant's summary and conclusion

Conclusions

Dissociation constant of 2,5-dimethylaniline in water is 4.51 at 25 degree C.

# 5 Environmental fate and pathways

# 5.1 Stability

# 5.1.1 Phototransformation in air

## Endpoint study record: AOPWIN

| UUID |  | IUC5-31c689c4-bde4-469c-8529-4d990a406d98 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V3.0 Plug-In |
| Date |  | 2012-06-29 13:13:27 JST |
| Remarks |  | Successfully migrated to IUCLID 5.4 format. |

Administrative Data

| Purpose flag | key study |  |  |
| --- | --- | --- | --- |
| Study result type | (Q)SAR | Study period | 2012 |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Reliable QSAR method |  |  |

Materials and methods

Principles of method if other than guideline

QSAR calculation by AOPWIN (version 1.92) Indirect photo-oxidation by OH radicals

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 95-78-3 |
| IUPAC name | 2,5-dimethylaniline |

Results and discussions

Any other information on results incl. tables

Overall OH rate constat = 200.3E-12 cm3/molecule.sec OH concentration = 1.5E6 OH/cm3 (12 hours irradiation) Half-life time = 0.053 days

Applicant's summary and conclusion

Conclusions

Half-life time of 2,5-dimethylaniline by reaction with hydroxyl radicals in air is calculated to be 0.053 days by AOPWIN (version 1.92).

# 5.2 Biodegradation

# 5.2.1 Biodegradation in water: screening tests

## Endpoint study record: Experiment

| UUID |  | IUC5-b808d308-5e0f-4b31-9d4f-241dcadb36e1 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V3.0 Plug-In |
| Date |  | 2012-07-02 14:28:45 JST |
| Remarks |  | Successfully migrated to IUCLID 5.4 format. |

Administrative Data

| Purpose flag | key study; robust study summary |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result | Study period | 11th July 1986 - 13th September 1986 |
| Reliability | 1 (reliable without restriction) |  |  |
| Rationale for reliability incl. deficiencies | The test was conducted accoriding to OECD Test guideline in compliance with GLP. |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | Chemicals Evaluation and Research Institute, Japan | 1989 | Study of Biodegradation on 2,5-Dimethylaniline |  | Chemicals Evaluation and Research Institute, Japan | 21025 |  |  |  |

Data access

data published

Materials and methods

Test type

ready biodegradability

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | OECD Guideline 301 C (Ready Biodegradability: Modified MITI Test (I)) | no |

GLP compliance

yes (incl. certificate)

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| IUPAC name | 2,5-dimethylaniline |

Details on test material

- Name of test material (as cited in study report):2,5-dimethylaniline - Structural formula attached as image file (if other than submission substance): see Fig. - Substance type:Organic substance - Physical state:Not known - Analytical purity:>97% - Impurities (identity and concentrations):Not Known - Purity test date:Not known - Lot/batch No.:FBY01 - Expiration date of the lot/batch:Not kown - Stability under test conditions:Confirmed by the IR spectra before and after the experiment - Storage condition of test material:In cool placce

Study design

Oxygen conditions

aerobic

Inoculum or test system

activated sludge, non-adapted

Details on inoculum

Preparation of sludge:Sludge and surface water including surface soil were collected from ten different places in Japan which includes water treatment plants, rivers, lake and inner bays. These sludges and water/soil were mixed and cultivated with glucose and peptone as nutrient in a testing laboratory. Concentration of the activated sludge for testing: 30 mg/L as suspended solid

Duration of test (contact time)

28 d

Initial test substance concentration

| Initial conc. | Based on |
| --- | --- |
| 100 mg/L | test mat. |

Parameter followed for biodegradation estimation

other: BOD

Details on analytical methods

DETAILS ON PRETREATMENT 10 mL of the test solution was taken out and centrifuged with 1000 g for 10 miniutes. Then, the supernatant of this solution was employed for the quantitative analysis for HPLC and TOC. HPLC conditions Column: Inertsil ODS-2, 15 cm x 4.6 mm I.D. Eluent: Methanol/purified water 6/4 v/v Wavelength: 234 nm TOC conditions Instrument: TOC analyzer, TOC-500 (Shimadzu) Furnace temperature: 680 degree C

Details on study design

TEST CONDITIONS - Composition of medium: A solution: B solution: C solution: D solution: 3 mL of A solution, 3 mL of B solution, 3 mL of C solution and 3 mL of D solution was added to the purified water and fill up to 1 L. Then pH of this solution was adjusted to 7.0 - Solubilising agent (type and concentration if used): No - Test temperature: 25 +/- 1 degree C - pH: At the start of the cultivation period: 7.0 At the end of the cultivation period: 7.0 - 7.2 - pH adjusted: No - Suspended solids concentration: 30 mg/L - Continuous darkness: yes SAMPLING - Sampling frequency: BOD was continueously measured. HPLC analysis and TOC analysis were conducted after the cultivation period. - Sample storage before analysis: Analysis was conduted right after the sampling. CONTROL AND BLANK SYSTEM - Inoculum blank: Yes

Reference substance

aniline

Results and discussions

% Degradation of test substance

| %Degr. | St. dev. | Parameter | Sampling time | Remarks |
| --- | --- | --- | --- | --- |
| 0 — 1 |  | other: Biodegradability based on BOD | 28 d |  |

Details on results

Biodegradability of Reference substance (Aniline) After 7 days: 61 % based on BOD After 14 days: 75 % based on BOD

Overall remarks, attachments

Remarks on results including tables and figures

Test results are as follows (water + test substance); (Sludge + test substance)-1; (Sludge + test substance)-2; (Sludge + test substance) -3; Theoretical amount. BOD; 0.0 mg; 0.0 mg; 0.7 mg; 0.0 mg; 93.6 mg. DOC residue; 23.4 mgC; 23.3 mgC; 23.2 mgC; 23.3 mgC; 23.9 mgC. 98 %; 97 %; 97 %; 97 %; -. Test substance residue; 29.7 mg; 29.8 mg; 29.9 mg; 30.1 mg; 30.2 mg. 98 %; 99 %; 99 %; 100 %; -. Biodegradability is as follows. -; (Sludge + test substance)-1; (Sludge + test substance)-2; (Sludge + test substance) -3. Biodegradability by BOD; 0 %; 1 %; 0 %. Biodegradability by TOC; 0 %; 1 %; 0 %. Biodegradability by HPLC; 0 %; 0 %; 0 %.

Applicant's summary and conclusion

Validity criteria fulfilled

yes

Interpretation of results

under test conditions no biodegradation observed

Conclusions

2,5-Dimethylaniline was not biodegradated under the test condition.

# 5.3 Bioaccumulation

# 5.3.1 Bioaccumulation: aquatic / sediment

## Endpoint study record: Experiment

| UUID |  | IUC5-06dfba53-e1f0-4d3d-b60c-a6a24bb7a727 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V3.0 Plug-In |
| Date |  | 2012-07-02 14:02:37 JST |
| Remarks |  | Successfully migrated to IUCLID 5.4 format. |

Administrative Data

| Purpose flag | key study |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result | Study period | 9/11/1989-22/1/1990 |
| Reliability | 1 (reliable without restriction) |  |  |
| Rationale for reliability incl. deficiencies | Test was conducted accrding to OECD test guideline in compliance with GLP. |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | Chemicals Evaluation and Research Institute, Japan | 1990 | Study of bio-concentration of 2,5-dimethylaniline (K-1025) with carp |  | Chemicals Evaluation and Research Institute, Japan | 51025 |  |  |  |

Data access

data published

Materials and methods

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| equivalent or similar to | OECD Guideline 305 (Bioconcentration: Flow-through Fish Test) | no |

GLP compliance

yes (incl. certificate)

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 95-78-3 |
| IUPAC name | 2,5-dimethylaniline |
| EC number | 202-451-0 |

Details on test material

- Name of test material (as cited in study report): 2,5-dimethylaniline - Analytical purity: >97% - Lot/batch No.: FBY01

Details on analytical methods

DETAILS ON PRETREATMENT - Extraction: SPE, Sep-PakC18 Pre-cleaning : Acetonitrile 10 mL, water 10 mL Elution: Acetonitrile 10 mL (Solution for HPLC-analysis)

Test organisms

Test organisms (species)

Cyprinus carpio

Details on test organisms

TEST ORGANISM - Length at study initiation (lenght definition, mean, range and SD): 9.4 cm - Weight at study initiation (mean and range, SD):20.8g

Study design

Route of exposure

aqueous

Test type

flow-through

Water media type

freshwater

Total exposure / uptake duration

6 wk

Test conditions

Test temperature

25 degree C

Dissolved oxygen

5.5-6.8 mg/L

Nominal and measured concentrations

-1st concentration group: Nominal = 1 mg/L, Measured=0.955-0.970(1-day - 6-week) -2nd concentration group: Nominal = 0.1 mg/L, Measured=0.0950-0.101(1-day - 6-week)

Any other information on materials and methods incl. tables

Aquarium volume: 100L, 1155 L/day

Results and discussions

Bioaccumulation factor

| Conc. in environment / dose | Type | Value | Basis | Time of plateau | Calculation basis | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 1 mg/L | BCF | 1.5 — 3.2 | whole body w.w. |  |  |  |
| 0.1 mg/L | BCF | < 3.8 | whole body w.w. |  |  |  |

Any other information on results incl. tables

2ｗ; 3ｗ; 4ｗ; 6ｗ. 1 mg/L; 2.6 3.2; 2.0 1.7; 2.6 2.4; 1.5 1.8. 0.1 mg/L; <3.8 <3.8; <3.8 <3.8; <3.8 <3.8; <3.8 <3.8.

Applicant's summary and conclusion

Conclusions

Bio-concentration factor is <3.8.

# 5.4 Transport and distribution

# 5.4.1 Adsorption / desorption

## Endpoint study record: QSAR -1

| UUID |  | IUC5-e12553b3-ebfb-46a2-84da-1fe1b5171055 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V3.0 Plug-In |
| Date |  | 2011-02-17 22:00:23 JST |
| Remarks |  | Successfully migrated to IUCLID 5.4 format. |

Administrative Data

| Purpose flag | key study; robust study summary |  |  |
| --- | --- | --- | --- |
| Study result type | (Q)SAR |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Reliable QSAR method |  |  |

Materials and methods

Principles of method if other than guideline

Soil adsorption coefficeint is esmated by KOCWIN (version 2.00) with log Kow of 1.91.

Results and discussions

Adsorption coefficient

| Type | Value | Temperature | % Org. carbon | Remarks |
| --- | --- | --- | --- | --- |
| log Koc | 2 |  |  |  |

Applicant's summary and conclusion

Conclusions

Soil adsorption coefficeint (log Kow) is 2.0.

# 5.4.2 Henry's Law constant

## Endpoint study record: QSAR -1

| UUID |  | IUC5-5d052b70-a99f-4e3b-a806-472ad6603c2d |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V3.0 Plug-In |
| Date |  | 2011-02-17 22:18:59 JST |
| Remarks |  | Successfully migrated to IUCLID 5.4 format. |

Administrative Data

| Purpose flag | supporting study; robust study summary |  |  |
| --- | --- | --- | --- |
| Study result type | (Q)SAR |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Reliable QSAR method |  |  |

Materials and methods

Principles of method if other than guideline

Henry's Law Constant is calculated by HENRYWIN (version 3.20) with bond contribution method.

Results and discussions

Henry's Law constant H

| H | 0.235 Pa m³/mol |
| --- | --- |
| Temp. (°C) | 25 |

Applicant's summary and conclusion

Conclusions

Henry's Law Constant is 0.235 Pa.m3/mol at 25 degree C.

## Endpoint study record: Esimation -1

| UUID |  | IUC5-c25971df-f31e-4161-a7a5-4c5a71b1a834 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V3.0 Plug-In |
| Date |  | 2011-02-17 22:31:50 JST |
| Remarks |  | Successfully migrated to IUCLID 5.4 format. |

Administrative Data

| Purpose flag | key study; robust study summary |  |  |
| --- | --- | --- | --- |
| Study result type | other: |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Estimation with reliable vapour pressure and water solubility. |  |  |

Materials and methods

Principles of method if other than guideline

Henry’s Law Constant is estimated with vapour pressure of 20 Pa 25 °C divided by water solubility of 5.6 g/L at 25 °C.

Results and discussions

Henry's Law constant H

| H | 0.43 Pa m³/mol |
| --- | --- |
| Temp. (°C) | 25 |

Applicant's summary and conclusion

Conclusions

Henry's Law Constant is 0.43 Pa.m3/mol at 25 degree C.

# 5.4.3 Distribution modelling

## Endpoint study record: Calculation

| UUID |  | IUC5-cea75d2a-2d15-47a5-b780-69f7f3af9027 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V3.0 Plug-In |
| Date |  | 2012-06-29 14:18:10 JST |
| Remarks |  | Successfully migrated to IUCLID 5.4 format. |

Administrative Data

| Purpose flag | key study |  |  |
| --- | --- | --- | --- |
| Study result type | estimated by calculation | Study period | 2012 |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Reliable calculation method |  |  |

Materials and methods

Model

Calculation according to Mackay, Level III

Calculation programme

EPIWIN (version 4.00)

Release year

2008

Media

other: Air-Water-Soil-Sediment

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 95-78-3 |
| IUPAC name | 2,5-dimethylaniline |

Any other information on materials and methods incl. tables

Input parameters are as follows. Melting point: 15.5 °C Boiling point: 214 °C Water solubility: 5600 mg/L Vapour pressure: 0.15 mmHg Partition coefficient between octanol and water: 1.91 Henry’s Law constant: 2.32×10-6atm.m3/mole at 25 °C

Results and discussions

Percent distribution in media

Any other information on results incl. tables

Distributions of 2,5 -dimethylaniline in each environmental compartment are as follows. 2,5-Dimethylaniline; Release to air, water and soil; Release to air; Release to water; Release to soil. Air compartment; 0.07 %; 26.6 %; 0.004 %; 0.002 %. Water compartment; 21.8 %; 10.8 %; 98.9 %; 2.87 %. Soil compartment; 77.9 %; 62.5 %; 0.01 %; 97.1 %. Sediment compartment; 0.24 %; 0.12 %; 1.1 %; 0.03 %.

Applicant's summary and conclusion

Conclusions

Fugacity level III calculations show that 2,5-dimethylaniline is mainly distributed to the water compartment (21.8 %) and soil compartment (77.9 %) if equally and continuously released to the air, soil and water.

# 5.5 Environmental data

# 5.5.1 Monitoring data

## Endpoint study record: Monitoring data.(MOE)

| UUID |  | IUC5-2e174dea-1b66-4522-a410-b1c23cdb1b26 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | CERI_M.SEKIZAWA / Chemicals Evaluation and Research Institute (CERI) / Tokyo / Japan |
| Date |  | 2012-11-26 15:38:24 JST |
| Remarks |  |  |

Administrative Data

| Purpose flag | supporting study |  |  |
| --- | --- | --- | --- |
| Study result type | other: Environmental monitoring | Study period | 1976 |
| Reliability | 2 (reliable with restrictions) |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Japanese Ministry of Environment | 2011 | Report on Environmental Survey and Monitoring of Chemicals. |  |  |  |  |  |  |

Data access

data published

Materials and methods

Media

other: Surface water and Sediment

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 95-78-3 |
| IUPAC name | 2,5-dimethylaniline |

Results and discussions

Details on results

The results from the Report on Environmental Survey and Wildlife Monitoring of Chemicals compiled by the Ministry of the Environment are summarized in Table 1. This survey is carried out for water, bottom sediments, fishes, air, etc., at the sampling points set up around the country.

Any other information on results incl. tables

Table 1Results of the environmental survey and wildlife monitoring for 2,5-dimethylaniline Media; Fiscal Year; Number of detections/Number of samples; Number of detection stations/Number of sampling stations; Range of detection; Limit of detection. Surface water; 1976; 0/68; 0/20; -; 0.2-0.5μg/L. Sediment; 1976; 2/68; 1/20; 0.006-0.027μg/g dry; 0.001-0.004μg/g dry.

# 6 Ecotoxicological Information

# 6.1 Aquatic toxicity

# 6.1.1 Short-term toxicity to fish

## Endpoint study record: MOE, 2009d(Short-term toxicity to fish)

| UUID |  | IUC5-601b5d2f-06b3-4989-8e67-51df426c7a77 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | CERI_M.SEKIZAWA / Chemicals Evaluation and Research Institute (CERI) / Tokyo / Japan |
| Date |  | 2012-11-28 11:39:06 JST |
| Remarks |  |  |

Administrative Data

| Purpose flag | key study; robust study summary |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result | Study period | 31 January 2008 - 23 June 2009 |
| Reliability | 1 (reliable without restriction) |  |  |
| Rationale for reliability incl. deficiencies | Nationl Guideline study under GLP conditions |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | KUREHA SPECIAL LABORATORY CO.,LTD. | 2009 | Fish (Oryzias latipes), Acute Toxicity Test of 2,5-Dimethylaniline |  | KUREHA SPECIAL LABORATORY CO.,LTD. |  | Ministry of the Environment, Japan | No. 2007-74 | 2009-06-23 |

Materials and methods

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | other guideline: Chemical Substances Control Law, Japan (equivalent to OECD Guideline 203) |  |

GLP compliance

yes : Chemical Substances Control Law, Japan (2005)

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 95-78-3 |
| EC number | 202-451-0 |
| EC name | 2,5-xylidine |
| IUPAC name | 2,5-dimethylaniline |

Details on test material

- Appearance: brownish yellow transparent liquid - Purity: 99.3% - Lot/batch No.: A0234724 - Storage condition of test material: In a refrigerator - Stability under the storage conditions: The test substance was stable under the storage conditions, because IR spectra of the test substance before and after the study were identical.

Analytical monitoring

yes

Details on sampling

- Frequency: at the start of the exposure (0 hour), before renewal of water (48 hours after exposure) and at the termination of the exposure

Details on analytical methods

PRETREATMENT OF TEST SOLUTION Samples were diluted with eluting solution (50-200 times). ANALYTICAL CONDITIONS - Instrument: HPLC-UV - Column: Inertsil WP300 C4 5 μm (150mm×4.6mm I.D.) - Column temp.: 40 degree C - Eluent: Acetonitril/20mmol/L : phosphate buffer solution(pH8) (30/70) - Flow rate: 1.0 mL/min - Measurement wavelength: UV 230 nm - Injection volume: 20 μL - Determination limit: 0.02 mg/L - Detection limit: 0.004 mg/L

Vehicle

no

Details on test solutions

1500mg of the test substance was dissolved with dilution water by stirring to produce a stock solution of 500 mg/L . Test solution was prepared in each test vessel by mixing the appropriate volume of the stock solution with dilution water. Dilution water was used for the control group.

Test organisms

Test organisms (species)

Oryzias latipes

Details on test organisms

TEST ORGANISM - Source: Laboratory-bred - Length at study initiation: 3.0 cm (2.8-3.0 cm), n=10 - Weight at study initiation: 0.30 g (0.22-0.36 g), n=10 - Feeding during test: No ACCLIMATION - Acclimation period: 2008/3/10-2009/2/16 (20 months old) - Acclimation conditions: Same as testb except exposure type (flow-through) - Type and amount of food: Type: Commercial fish food "TetraMin" Amount: ca. 2% of body weight/day, Not fed for 24 hours prior to the test - Health during acclimation: Mortality for 7 days prior to the test was 2.1 % OTHER - Confirming the reproducibility of the test conditions: Reference substance: Copper sulfate (II) pentahydrate (Reagent grade) Results: 96h-LC50: 0.34 mg/L (within the acceptable range to background data(0.30 ± 0.10 mg/L, n=35) in this laboratory)

Study design

Test type

semi-static

Water media type

freshwater

Limit test

no

Total exposure duration

96 h

Test conditions

Hardness

No data

Test temperature

23.8 - 24.4？C

pH

7.4 - 7.9

Dissolved oxygen

7.2 - 8.2 mg/L (？ 60 % of the O2 saturation conc. (8.25 mg/L at 24.0 ？C))

Nominal and measured concentrations

- Nominal concentrations: 28, 44, 69 and 110 mg/L - Measured concentrations (arithmetic mean): 27.7, 44.3, 68.6 and 110 mg/L

Details on test conditions

TEST SYSTEM - Test vessel: - Type: closed (with lid) - Material: Glass - Fill volume: 5L - Test solution volume: 5L/vessel - Aeration: No - Renewal rate of test solution: 48 hours after the start of exposure, the whole quantity - No. of organisms per vessel: 10 - No. of vessels per concentration (replicates): 1 - No. of vessels per control (replicates): 1 TEST MEDIUM / WATER PARAMETERS - Source/preparation of dilution water: Dechlorinated water (water from the water facility, removed the residual chlorine with the activated carbon and aerated sufficiently) - Hardness: 33 mg/L (as CaCO3) - pH: 7.9 OTHER TEST CONDITIONS - Adjustment of pH: No - Photoperiod: 16 hours light / 8 hours dark - Light intensity: Room light OBSERVATION AND MEASUREMENTS - Fish: Mortality and visible abnormality were observed at 0, 3, 6, 24, 48, 72 and 96 hours. - Test solutions: Temperature, pH and Dissolved oxygen concentration were measured at the start of the exposure and every 24 hours (before and after renewal of water).

Reference substance (positive control)

no

Any other information on materials and methods incl. tables

Table 1. Measured Concentration of the Test Substance in the Test Water Nominal Concentration (mg/L); Measured Concentration,mg/L (Percent of Nominal); MeanaMeasured Concentration (mg/L). 0 hour; 48hours; 96hours. new; old; new; old. Control; <0.02 (－); <0.02 (－); <0.02 (－); <0.02 (－); －. 28; 27.3 (98); 28.2(101); 27.6(99); 27.8(99); 27.7. 44; 42.7(97); 44.1(100); 43.8(100); 46.4(105); 44.3. 69; 66.7(97); 68.8 (100); 69.3(100); 69.6(101); 68.6. 110; 107(97); 110(100); 111(101); 110(100); 110. new : Freshly prepared test solutions old : Test solutions after 48 hours exposure a : Arithmetic Mean - : Not calculated Table 2. Temperature Nominal Concentration (mg/L); MeanaMeasured Concentration (mg/L); Temperature,℃. 0 Hour; 24 Hours; 48 Hours; 72 Hours; 96 Hours. new; old; new; old. Control; －; 24.0; 24.3; 24.4; 23.9; 24.2; 24.0. 28; 27.7; 24.0; 24.2; 24.3; 23.9; 24.0; 23.9. 44; 44.3; 24.0; 24.2; 24.3; 24.0; 24.0; 23.9. 69; 68.6; 24.0; 24.2; 24.3; 23.8; 24.0; 23.9. 110; 110.0; 24.0; 24.2; 24.2; 23.9; 24.0; 23.9. new : Freshly prepared test solutions old : Test solutions after 48 hours exposure a : Arithmetic Mean - : Not calculated Table 3. Dissolved Oxygen Concentration Nominal Concentration (mg/L); MeanaMeasured Concentration (mg/L); Dissolved Oxygen Concentration, mg/L. 0 Hour; 24 Hours; 48 Hours; 72 Hours; 96 Hours. New; Old; New; Old. Control; －; 8.2; 7.7; 7.2; 8.2; 7.6; 7.4. 28; 27.7; 8.2; 7.8; 7.2; 8.2; 7.9; 7.6. 44; 44.3; 8.2; 7.7; 7.4; 8.2; 7.9; 7.7. 69; 68.6; 8.2; 7.8; 7.5; 8.2; 7.9; 7.7. 110; 110.0; 8.2; 7.7; 7.4; 8.2; 7.9; 7.7. new : Freshly prepared test solutions old : Test solutions after 48 hours exposure a : Arithmetic Mean - : Not calculated Table 4. pH Values Nominal Concentration (mg/L); MeanaMeasured Concentration (mg/L); pH. 0 Hour; 24 Hours; 48 Hours; 72 Hours; 96 Hours. New; Old; New; Old. Control; －; 7.9; 7.5; 7.4; 7.8; 7.6; 7.4. 28; 27.7; 7.9; 7.6; 7.4; 7.8; 7.6; 7.5. 44; 44.3; 7.9; 7.6; 7.4; 7.8; 7.6; 7.4. 69; 68.6; 7.9; 7.6; 7.4; 7.8; 7.6; 7.5. 110; 110.0; 7.9; 7.6; 7.4; 7.8; 7.6; 7.4. new : Freshly prepared test solutions old : Test solutions after 48 hours exposure a : Arithmetic Mean - : Not calculated

Results and discussions

Effect concentrations

| Duration | Endpoint | Effect conc. | Nominal/Measured | Conc. based on | Basis for effect | Remarks (e.g. 95% CL) |
| --- | --- | --- | --- | --- | --- | --- |
| 96 h | LC50 | > 110 mg/L | meas. (arithm. mean) | test mat. | mortality |  |

Details on results

- The highest concentration of 0% mortality at 96 hours: 110 mg/L - The lowest concentration of 100% mortality at 96 hours: > 110 mg/L - The No observed effect concentration at 96 hours: 44 mg/L - No toxicological symptom was observed at control, 27.7 mg/L and 44.3 mg/L groups. In contrast, at 68.6 mg/L and 110 mg/L groups, abnormal swimming were observed. - There were no environmental factors which might have affected the reliability of the test.

Any other information on results incl. tables

Table 5. The Numbers of Dead Fish (Percent Mortality) Nominal Concentration (mg/L); MeanaMeasured Concentration (mg/L); Cumulative Mortality(Percent Mortality). 24 Hours; 48 Hours; 72 Hours; 96 Hours. Control; －; 0/10 ( 0); 0/10 ( 0); 0/10 ( 0); 0/10 ( 0). 28; 27.7; 0/10 ( 0); 0/10 ( 0); 0/10 ( 0); 0/10 ( 0). 44; 44.3; 0/10 ( 0); 0/10 ( 0); 0/10 ( 0); 0/10 ( 0). 69; 68.6; 0/10 ( 0); 0/10 ( 0); 0/10 ( 0); 0/10 ( 0). 110; 110; 0/10 ( 0); 0/10 ( 0); 0/10 ( 0); 0/10 ( 0). a : Arithmetic Mean - : Not calculated Table 6. Observed Toxic Symptoms Nominal Concentration (mg/L); Meana Measured Concentration (mg/L); Symptoms. Observation Time. 3 Hours; 6 Hours; 24 Hours; 48 Hours; 72 Hours; 96 Hours. Control; －; N; N; N; N; N; N. 28; 27.7; N; N; N; N; N; N. 44; 44.3; N; N; N; N; N; N. 69; 68.6; AS( 5); AS( 5); AS( 3); AS( 3); AS( 6); AS( 7). 110; 110; AS( 9); AS(10); AS(10); AS(10); AS(10); AS(10). N: No toxic symptom was observed －: Not calculated a: Arithmetic mean AS: Abnormal swimming

# 6.1.3 Short-term toxicity to aquatic invertebrates

## Endpoint study record: MOE, 2009e (Short-term toxicity to aquatic invertebrates)

| UUID |  | IUC5-dbc15fd5-6ab5-424c-aebd-bfd347d08d7c |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V3.0 Plug-In |
| Date |  | 2012-02-26 15:49:42 JST |
| Remarks |  | Successfully migrated to IUCLID 5.4 format. |

Administrative Data

| Purpose flag | key study; robust study summary |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result | Study period | 31 January 2008 - 23 June 2009 |
| Reliability | 1 (reliable without restriction) |  |  |
| Rationale for reliability incl. deficiencies | Nationl Guideline study under GLP conditions |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | KUREHA SPECIAL LABORATORY CO.,LTD. | 2009 | Daphnia magna, Acute Immobilisation Test of 2,5-Dimethylaniline |  | KUREHA SPECIAL LABORATORY CO.,LTD. |  | Ministry of the Environment, Japan | 2007-72 | 2009-06-23 |

Materials and methods

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | other guideline: Chemical Substances Control Law, Japan (equivalent to OECD Guideline 202) | no |

GLP compliance

yes : Chemical Substances Control Law, Japan (2005)

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 95-78-3 |
| EC number | 202-451-0 |
| EC name | 2,5-xylidine |
| IUPAC name | 2,5-dimethylaniline |

Details on test material

- Appearance: brownish yellow transparent liquid - Purity: 99.3% - Lot/batch No.: A0234724 - Storage condition of test material: In a refrigerator - Stability under the storage conditions: The test substance was stable under the storage conditions, because IR spectra of the test substance before and after the study were identical.

Analytical monitoring

yes

Details on sampling

- Frequency: at the start of the exposure (0 hour) and at the termination of the exposure (48 hours after exposure)

Details on analytical methods

PRETREATMENT OF TEST SOLUTION Samples were diluted with eluting solution (10-50 times). ANALYTICAL CONDITIONS - Instrument: HPLC-UV - Column: Inertsil WP300 C4, 5 μm (150 mm×4.6 mm I.D.) - Column temp.: 40 degreeC - Eluent: Acetonitril/20mmol/L : phosphate buffer solution(pH8) (30/70) - Flow rate: 1.0 mL/min - Measurement wavelength: UV 230 nm - Injection volume: 20 μL - Determination limit: 0.02 mg/L - Detection limit: 0.004 mg/L

Vehicle

no

Details on test solutions

500mg of the test substance was dissolved with dilution water by stirring to produce a stock solution of 500 mg/L . Test solution was prepared in each test vessel by mixing the appropriate volume of the stock solution with dilution water. Dilution water was used for the control group.

Test organisms

Test organisms (species)

Daphnia magna

Details on test organisms

TEST ORGANISM - Common name: Water flea - Source: Subcultured in the testing laboratory, Originally from the National Institute for Environmental Studies (Japan) on 5 February 1997 - Age at study initiation: less than 24 h-old - Feeding during test: No ACCLIMATION OF PARENTAL DAPHNIDS - Acclimation period: 2009/1/24-209/2/17 (24 days-old) - Acclimation conditions: See below - Breeding water: Dilution water - Population density: 30 daphnids/2 L breeding water - Temperature: 20+/-1 degreeC - Photoperiod: 16-hr Light / 8-hr Dark, Room light - Type and amount of food: Type: Chlorella sp. Amount: 0.15-0.20 mgC(Organic Carbon)/animal/day - Juveniles were removed three times a week. - Presence of male and ephippia were not observed. - Health during acclimation: Mortality of parental daphnids for 2 weeks prior to the test were 0 %. OTHER - Confirming the reproducibility of the test conditions: Reference substance: pottasium dichromate, reagent chemical Results: 48h EC50: 0.82 mg/L (background data in this laboratory: 0.86 +/- 0.20 mg/L, n=21 )

Study design

Test type

static

Water media type

freshwater

Limit test

no

Total exposure duration

48 h

Test conditions

Hardness

249 - 254 mg/L (as CaCO3)

Test temperature

20.0 - 20.6 degreeC

pH

7.8 - 7.9

Dissolved oxygen

8.6 - 8.7 mg/L (greater than 60 % of the O2 saturation conc. (8.84 mg/L at 20.0 degreeC))

Nominal and measured concentrations

- Nominal concentrations: 6.0, 7.7, 10, 13, 17, 22, 28 and 36 mg/L - Measured concentrations (arithmetic mean): 5.84, 7.48, 9.72, 11.3, 16.7, 21.7, 27.3 and 35.2 mg/L

Details on test conditions

TEST SYSTEM - Test vessel: - Type: closed - Material: Glass - Fill volume: 110 mL - Test solution volume: 100 mL/vessel - No. of organisms per vessel: 5 - No. of vessels per concentration (replicates): 4 - No. of vessels per control (replicates): 4 TEST MEDIUM / WATER PARAMETERS - Source/preparation of dilution water: Elendt M4 Medium recommended by OECD Guideline 211 used as dilution water - Hardness: 251 mg/L (as CaCO3) - pH: 7.9 OTHER TEST CONDITIONS - Adjustment of pH: No - Photoperiod: 16 hours light / 8 hours dark - Light intensity: Room light OBSERVATION AND MEASUREMENTS - Daphnids: Immobility was observed at 24 and 48 hours. Daphnids were considered immobile if they were not able to swim within 15 seconds after gentle agitation of the test vessel. - Test solutions: Dissolved oxygen concentration, pH and water temperature of the test solutions were measured at the start and the end of the exposure. The concentration of Ca and Mg were measured by ICP method at the start and the end of the exposure.

Reference substance (positive control)

no

Any other information on materials and methods incl. tables

Table 1. Measured Concentration of the Test Substance in the Test Water Nominal Concentration (mg/L); Measured Concentration (mg/L) (Percent of Nominal); MeanaMeasured Concentration (mg/L). 0 Hour new; 48 Hours old. Control; <0.02; (-); <0.02; (-); －. 6.0; 5.74; (96); 5.93; (99); 5.84. 7.7; 7.36; (96); 7.60; (99); 7.48. 10; 9.53; (95); 9.91; (99); 9.72. 13; 11.00; (85); 11.5; (88); 11.3. 17; 16.4; (96); 17; (100); 16.7. 22; 21.4; (97); 22; (100); 21.7. 28; 26.8; (96); 27.7; (99); 27.3. 36; 34.4; (96); 35.9; (100); 35.2. new: Freshly prepared test solutions old: Test solutions after 48 hours exposure a: Arithmetic mean －: Not calculated Table 2. Temperature Nominal Concentration (mg/L); MeanaMeasured Concentration (mg/L); Temperature ,℃. 0 Hour new; 48 Hours old. Control; -; 20.2; 20.0. 6.0; 5.84; 20.4; 20.0. 7.7; 7.48; 20.4; 20.0. 10; 9.72; 20.4; 20.0. 13; 11.3; 20.4; 20.0. 17; 16.7; 20.5; 20.0. 22; 21.7; 20.6; 20.2. 28; 27.3; 20.6; 20.2. 36; 35.2; 20.6; 20.3. new: Freshly prepared test solutions old: Test solutions after 48 hours exposure a: Arithmetic mean －: Not calculated Table 3. Dissolved Oxygen Concentrations Nominal Concentration (mg/L); MeanaMeasured Concentration (mg/L); Dissolved Oxygen Concentration ,mg/L. 0 Hour new; 48 Hours old. Control; -; 8.7; 8.6. 6.0; 5.84; 8.7; 8.6. 7.7; 7.48; 8.7; 8.6. 10; 9.72; 8.7; 8.6. 13; 11.3; 8.6; 8.6. 17; 16.7; 8.6; 8.6. 22; 21.7; 8.6; 8.6. 28; 27.3; 8.6; 8.6. 36; 35.2; 8.6; 8.6. new: Freshly prepared test solutions old: Test solutions after 48 hours exposure a: Arithmetic mean －: Not calculated Table 4. pH Values Nominal Concentration (mg/L); MeanaMeasured Concentration (mg/L); pH. 0 Hour; 48 Hours. New; Old. Control; -; 7.9; 7.8. 6.0; 5.84; 7.8; 7.8. 7.7; 7.48; 7.8; 7.8. 10; 9.72; 7.8; 7.8. 13; 11.3; 7.8; 7.8. 17; 16.7; 7.8; 7.8. 22; 21.7; 7.8; 7.8. 28; 27.3; 7.8; 7.9. 36; 35.2; 7.8; 7.9. new: Freshly prepared test solutions old: Test solutions after 48 hours exposure a: Arithmetic mean －: Not calculated Table 5. Total Hardness (as CaCO3) Nominal Concentration (mg/L); MeanaMeasured Concentration (mg/L); Total Hardness(as CaCO3),mg/L. 0 Hour new; 48 Hours old. Control; -; 251; 252. 6.0; 5.84; 254; 251. 7.7; 7.48; 251; 253. 10; 9.72; 254; 252. 13; 11.3; 249; 251. 17; 16.7; 251; 250. 22; 21.7; 251; 251. 28; 27.3; 250; 251. 36; 35.2; 249; 251. new: Freshly prepared test solutions old: Test solutions after 48 hours exposure a: Arithmetic mean －: Not calculated

Results and discussions

Effect concentrations

| Duration | Endpoint | Effect conc. | Nominal/Measured | Conc. based on | Basis for effect | Remarks (e.g. 95% CL) |
| --- | --- | --- | --- | --- | --- | --- |
| 24 h | EC50 | 27 mg/L | meas. (arithm. mean) | test mat. | mobility | 95%CL: 23-32 mg/L, Probit analysis |
| 48 h | EC50 | 18 mg/L | meas. (arithm. mean) | test mat. | mobility | 95%CL: 17-20 mg/L, Probit analysis |

Details on results

- The lowest concentration of 100% immobility: at 24 hours; higher than 35 mg/L, at 48 hours 27 mg/L - The Highest concentration of 0% immobility: at 24 hours; 11 mg/L, at 48 hours 11 mg/L - Immobility of control was 0 %. - There were no factors which might have affected the reliability of the test

Any other information on results incl. tables

Table 6. The Numbers of Immobile Daphnia Nominal Concentration (mg/L); MeanaMeasured Concentration (mg/L); Number of Daphnia; Cumulative Numbers of ImmobilizedDaphnia (Percent Immobility). 24 Hours; 48 Hours. 1; 2; 3; 4; Total; 1; 2; 3; 4; Total. Control; -; 20; 0; 0; 0; 0; 0( 0); 0; 0; 0; 0; 0( 0). 6.0; 5.84; 20; 0; 0; 0; 0; 0( 0); 0; 0; 0; 0; 0( 0). 7.7; 7.48; 20; 0; 0; 0; 0; 0( 0); 0; 0; 0; 0; 0( 0). 10; 9.72; 20; 0; 0; 0; 0; 0( 0); 0; 0; 0; 0; 0( 0). 13; 11.3; 20; 0; 0; 0; 0; 0( 0); 0; 0; 0; 0; 0( 0). 17; 16.7; 20; 0; 1; 2; 0; 3( 15); 2; 1; 2; 0; 5( 25). 22; 21.7; 20; 1; 3; 2; 3; 9( 45); 4; 5; 3; 5; 17( 85). 28; 27.3; 20; 4; 2; 1; 2; 9( 45); 5; 5; 5; 5; 20(100). 36; 35.2; 20; 5; 3; 3; 3; 14( 70); 5; 5; 5; 5; 20(100). a: Arithmetic mean -: Not calculated

# 6.1.4 Long-term toxicity to aquatic invertebrates

## Endpoint study record: MOE, 2009g (Long-term toxicity to aquatic invertebrates)

| UUID |  | IUC5-cf822de6-6011-4945-ab02-9c60c1837b56 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V3.0 Plug-In |
| Date |  | 2012-02-27 11:07:53 JST |
| Remarks |  | Successfully migrated to IUCLID 5.4 format. |

Administrative Data

| Purpose flag | key study |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result | Study period | 31 January 2008 - 23 June 2009 |
| Reliability | 1 (reliable without restriction) |  |  |
| Rationale for reliability incl. deficiencies | OECD Guideline study under GLP conditions |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | KUREHA SPECIAL LABORATORY CO.,LTD. | 2009 | Daphnia magna, Reproduction Test of 2,5-dimethylaniline |  | KUREHA SPECIAL LABORATORY CO.,LTD. |  | Ministry of the Environment, Japan | 2007-73 | 2009-06-24 |

Materials and methods

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | OECD Guideline 211 (Daphnia magna Reproduction Test) |  |

GLP compliance

yes

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 95-78-3 |
| EC number | 202-451-0 |
| EC name | 2,5-xylidine |
| IUPAC name | 2,5-dimethylaniline |

Details on test material

- Appearance: brownish yellow transparent liquid - Purity: 99.3% - Lot/batch No.: A0234724 - Storage condition of test material: In a refrigerator - Stability under the storage conditions: The test substance was stable under the storage conditions, because IR spectra of the test substance before and after the study were identical.

Analytical monitoring

yes

Details on sampling

- Frequency: At before and after renewal of water, total 6 times during exposure

Details on analytical methods

PRETREATMENT OF TEST SOLUTION Samples were diluted with eluting solution (10-50 times). ANALYTICAL CONDITIONS - Instrument: HPLC-UV - Column: Inertsil WP300 C4, 5 μm (150 mm×4.6 mm I.D.) - Column temp.: 40 degreeC - Eluent: Acetonitril/20mmol/L : phosphate buffer solution(pH8) (30/70) - Flow rate: 1.0 mL/min - Measurement wavelength: UV 230 nm - Injection volume: 20 μL - Determination limit: 0.02 mg/L - Detection limit: 0.004 mg/L

Vehicle

no

Details on test solutions

100mg of the test substance was dissolved with dilution water by stirring to produce a stock solution of 500 mg/L . Test solution was prepared in each test vessel by mixing the appropriate volume of the stock solution with dilution water. Dilution water was used for the control group.

Test organisms

Test organisms (species)

Daphnia magna

Details on test organisms

TEST ORGANISM - Common name: Water flea - Source: Subcultured in the testing laboratory, Originally from the National Institute for Environmental Studies (Japan) on 5 February 1997 - Age at study initiation: less than 24 hour-old - Feeding during test Food type: Chlorella sp. Amount: 0.15 - 0.20 mg C (Organic Carbon)/ day / animal ACCLIMATION OF PARENTAL DAPHNIDS - Acclimation period: 2009/2/22-209/3/16 (22 days-old) - Acclimation conditions: See below - Breeding water: Dilution water - Population density: 30 daphnids/2 L breeding water - Temperature: 20+/-1 degreeC - Photoperiod: 16-hr Light / 8-hr Dark, Room light - Type and amount of food: Type: Chlorella sp. Amount: 0.15-0.20 mgC(Organic Carbon)/animal/day - Presence of male and ephippia were not observed. - Health during acclimation: Mortality of parental daphnids for 2 weeks prior to the test were 3.3 %. OTHER - Confirming the reproducibility of the test conditions: Reference substance: pottasium dichromate, reagent chemical Results: 48h EC50: 0.82 mg/L (background data in this laboratory: 0.86 +/- 0.20 mg/L, n=21 )

Study design

Test type

semi-static

Water media type

freshwater

Limit test

no

Total exposure duration

21 d

Test conditions

Hardness

246 - 256 mg/L (as CaCo3)

Test temperature

20.0 - 20.9 degreeC

pH

7.4 - 8.0

Dissolved oxygen

7.7 - 8.8 mg/L

Nominal and measured concentrations

- Nominal concentrations: 0, 0.046, 0.10, 0.22, 0.46, 1.0, 2.2 and 4.6 mg/L - Measured concentrations (Time-weighted mean): 0, 0.0424, 0.0958, 0.211, 0.447, 0.981, 2.18 and 4.54 mg/L

Details on test conditions

TEST SYSTEM - Test vessel: - Type: closed - Material: Glass - Fill volume: 110 mL - Test solution volume: 80 mL/vessel - No. of organisms per vessel: 1 - No. of vessels per concentration (replicates): 10 - No. of vessels per control (replicates): 10 TEST MEDIUM / WATER PARAMETERS - Source/preparation of dilution water: Elendt M4 Medium recommended by OECD Guideline 211 used as dilution water - Hardness: 251 mg/L (as CaCO3) - pH: 7.8 OTHER TEST CONDITIONS - Adjustment of pH: No - Photoperiod: 16 hours light / 8 hours dark - Light intensity: Room light OBSERVATION AND MEASUREMENTS - Parental daphnids: The numbers of the living and the dead parent daphnia and immobility were recorded daily during the exposure. - Juveniles: - No. of living and dead juveniles - No. of aborted eggs and ephippia (observed daily) - Time (day) to the first brood production Juveniles were removed after recording - Test solutions: Dissolved oxygen concentration, pH, water temperature and hardness of the test solutions were measured at the before and after renewal of water at once a week (total 3 times).

Reference substance (positive control)

no

Any other information on materials and methods incl. tables

Table 1-1 Measured Concentrations of the Test Substance in Test Water during 21-day Exposure Period (Semi-Static Conditions) Nominal Concentration (mg/L); Date →; Measured Concentration (mg/L); MeanaMeasured Concentration (mg/L). 0; 2; 9; 11; 18; 21. new; old; new; old; new; old. Control; N.D.; N.D.; N.D.; N.D.; N.D.; N.D.; -. 0.046; 0.0433; 0.0434; 0.0453; 0.0417; 0.0457; 0.0369; 0.0424. 0.10; 0.0943; 0.0962; 0.0996; 0.0973; 0.100; 0.0892; 0.0958. 0.22; 0.221; 0.215; 0.216; 0.208; 0.215; 0.197; 0.211. 0.46; 0.466; 0.453; 0.452; 0.441; 0.456; 0.423; 0.447. 1.0; 1; 0.993; 0.978; 0.967; 0.996; 0.957; 0.981. 2.2; 2.22; 2.19; 2.16; 2.16; 2.24; 2.13; 2.18. 4.6; 4.57; 4.55; 4.44; 4.51; 4.66; 4.50; 4.54. new: Freshly prepared test solution old: Old test solution before renewal -: Not calculated a:Time-Weighted Mean Table 1-2 Measured Concentrations as a Percentage of Nomina Nominal Concentration (mg/L); MeanaMeasured Concentration (mg/L); Date →; Measured Concentration as Percentage of Nominal. 0; 2; 9; 11; 18; 21. new; old; new; old; new; old. Control; -; -; -; -; -; -; -. 0.046; 0.0424; 93; 93; 98; 91; 100; 80. 0.10; 0.0958; 94; 96; 100; 97; 100; 89. 0.22; 0.211; 100; 98; 98; 95; 98; 90. 0.46; 0.447; 101; 98; 98; 96; 99; 92. 1.0; 0.981; 100; 99; 98; 97; 100; 96. 2.2; 2.18; 101; 100; 98; 98; 102; 97. 4.6; 4.54; 99; 99; 97; 98; 101; 98. new: Freshly prepared test solution old: Old test solution before renewal -: Not calculated a:Time-Weighted Mean Table 2. Temperature Nominal Concentration (mg/L); MeanaMeasured Concentration (mg/L); Temperature(℃). Date→; 0; 2; 9; 11; 18; 21; Min.; Max.. new; old; new; old; new; old. Control; -; 20.8; 20.8; 20.0; 20.0; 20.8; 20.6; 20.0; 20.8. 0.046; 0.0424; 20.8; 20.8; 20.1; 20.0; 20.8; 20.6; 20.0; 20.8. 0.10; 0.0958; 20.8; 20.8; 20.1; 20.0; 20.8; 20.4; 20.0; 20.8. 0.22; 0.211; 20.8; 20.8; 20.2; 20.0; 20.8; 20.4; 20.0; 20.8. 0.46; 0.447; 20.9; 20.8; 20.2; 20.0; 20.8; 20.4; 20.0; 20.9. 1.0; 0.981; 20.9; 20.9; 20.3; 20.2; 20.8; 20.4; 20.2; 20.9. 2.2; 2.18; 20.9; 20.9; 20.4; 20.2; 20.8; 20.4; 20.2; 20.9. 4.6; 4.54; 20.9; 20.9; 20.4; 20.3; 20.9; 20.3; 20.2; 20.9. Total; 20.0; 20.9. new: Freshly prepared test solution, old: Old test solution before renewal, -: Not calculated, a:Time-Weighted Mean Table 3. Dissolved Oxygen Concentration(D.O.) Nominal Concentration (mg/L); MeanaMeasured Concentration (mg/L); D.O. (mg/L). Date→; 0; 2; 9; 11; 18; 21; Min.; Max.. new; old; new; old; new; old. Control; -; 8.6; 8.6; 8.8; 8.1; 8.8; 7.8; 7.8; 8.8. 0.046; 0.0424; 8.6; 8.5; 8.8; 8.2; 8.8; 7.8; 7.8; 8.8. 0.10; 0.0958; 8.6; 8.6; 8.8; 8.1; 8.8; 7.7; 7.7; 8.8. 0.22; 0.211; 8.6; 8.6; 8.7; 8.0; 8.8; 7.7; 7.7; 8.8. 0.46; 0.447; 8.6; 8.5; 8.8; 8.0; 8.8; 7.7; 7.7; 8.8. 1.0; 0.981; 8.6; 8.6; 8.7; 8.0; 8.8; 7.7; 7.7; 8.8. 2.2; 2.18; 8.6; 8.5; 8.8; 7.9; 8.8; 7.8; 7.8; 8.8. 4.6; 4.54; 8.6; 8.6; 8.7; 8.3; 8.8; 8.5; 8.3; 8.8. Total; 7.7; 8.8. new: Freshly prepared test solution, old: Old test solution before renewal, -: Not calculated, a:Time-Weighted Mean Table 4. pH Values Nominal Concentration (mg/L); MeanaMeasured Concentration (mg/L); pH. Date→; 0; 2; 9; 11; 18; 21; Min.; Max.. new; old; new; old; new; old. Control; -; 7.8; 8.0; 7.9; 7.5; 7.9; 7.4; 7.4; 8.0. 0.046; 0.0424; 7.8; 7.9; 7.9; 7.6; 7.9; 7.4; 7.4; 7.9. 0.10; 0.0958; 7.8; 8.0; 7.9; 7.5; 7.9; 7.4; 7.4; 8.0. 0.22; 0.211; 7.8; 7.9; 7.9; 7.5; 7.9; 7.4; 7.4; 7.9. 0.46; 0.447; 7.8; 8.0; 7.9; 7.5; 7.9; 7.4; 7.4; 8.0. 1.0; 0.981; 7.8; 7.9; 7.9; 7.5; 7.9; 7.4; 7.4; 7.9. 2.2; 2.18; 7.8; 8.0; 7.9; 7.5; 7.9; 7.4; 7.4; 8.0. 4.6; 4.54; 7.8; 8.0; 7.9; 7.6; 7.9; 7.7; 7.6; 8.0. Total; 7.4; 8.0. new: Freshly prepared test solution, old: Old test solution before renewal, -: Not calculated, a:Time-Weighted Mean Table 5.Total Hardness(as CaCO3) Nominal Concentration (mg/L); MeanaMeasured Concentration (mg/L); Total Hardness(as CaCO3,mg/L). Date→; 0; 2; 9; 11; 18; 21; Min.; Max.. new; old; new; old; new; old. Control; -; 251; 252; 247; 250; 251; 254; 247; 254. 0.046; 0.0424; 250; 250; 248; 249; 254; 255; 248; 255. 0.10; 0.0958; 248; 252; 251; 248; 253; 251; 248; 253. 0.22; 0.211; 247; 253; 249; 247; 253; 251; 247; 253. 0.46; 0.447; 250; 252; 246; 249; 254; 255; 246; 255. 1.0; 0.981; 249; 254; 247; 247; 254; 253; 247; 254. 2.2; 2.18; 252; 256; 251; 248; 256; 253; 248; 256. 4.6; 4.54; 249; 253; 249; 249; 253; 256; 249; 256. Total; 246; 256. new: Freshly prepared test solution, old: Old test solution before renewal, -: Not calculated, a:Time-Weighted Mean

Results and discussions

Effect concentrations

| Duration | Endpoint | Effect conc. | Nominal/Measured | Conc. based on | Basis for effect | Remarks (e.g. 95% CL) |
| --- | --- | --- | --- | --- | --- | --- |
| 21 d | EC50 | 2.6 mg/L | meas. (TWA) | test mat. | reproduction | 95%CL: 2.1-3.3 mg/L, Logit |
| 21 d | NOEC | 0.096 mg/L | meas. (TWA) | test mat. | reproduction |  |
| 21 d | LOEC | 0.21 mg/L | meas. (TWA) | test mat. | reproduction |  |

Details on results

- Mortality of parent animals: The mortality of the parent animals in the control was 0% at the end of the test, which meets the criteria for validity of the test (i.e. not more than 20%). Although death of the parent animals were observed in the 0.10 and 0.46 mg/L, death were not caused by the substance because of no dose-response. - Time to first brood release or time to hatch: 8-9 days at the control group - The mean cumulative number of juveniles produced per parent animal: 142.2 at the control group (meets the criteria for validity of the test (i.e. ？60)) - Ephippia: None. A few aborted eggs and dead offsprings were observed at all exposure groups.

Any other information on results incl. tables

Table 6-1. Cumulative Numbers of Dead Parental Daphnia Nominal Concentration (mg/L); MeanaMeasured Concentration (mg/L); Days. 0; 1; 2; 3; 4; 5; 6; 7; 8; 9; 10; 11; 12; 13; 14; 15; 16; 17; 18; 19; 20; 21. Control; -; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0. 0.046; 0.0424; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0. 0.10; 0.0958; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 1. 0.22; 0.211; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0. 0.46; 0.447; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 1; 1; 1; 1; 1; 2. 1.0; 0.981; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0. 2.2; 2.18; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0. 4.6; 4.54; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0. a:Time-Weighted Mean, -: Not calculated Table 6-2. Mortality (%) of Parental Daphnia Nominal Concentration (mg/L); MeanaMeasured Concentration (mg/L); Days. 1; 2; 4; 7; 14; 21. Control; -; 0; 0; 0; 0; 0; 0. 0.046; 0.0424; 0; 0; 0; 0; 0; 0. 0.10; 0.0958; 0; 0; 0; 0; 0; 10. 0.22; 0.211; 0; 0; 0; 0; 0; 0. 0.46; 0.447; 0; 0; 0; 0; 0; 20. 1.0; 0.981; 0; 0; 0; 0; 0; 0. 2.2; 2.18; 0; 0; 0; 0; 0; 0. 4.6; 4.54; 0; 0; 0; 0; 0; 0. a:Time-Weighted Mean, -: Not calculated Table 7. Time (Days) to First Brood Production No.; Nominal Concentration, mg/L (MeanaMeasured Concentration ,mg/L). Control; 0.046; 0.10; 0.22; 0.46; 1.0; 2.2; 4.6. (-); (0.0424); (0.0958); (0.211); (0.447); (0.981); (2.18); (4.54). 1; 9; 8; 8; 8; 8; 8; 8; 12. 2; 8; 8; 9; 9; 9; 8; 9; 12. 3; 8; 8; 8; 8; 8; 8; 9; 12. 4; 8; 8; 8; 8; 9; 8; 9; 9. 5; 9; 8; 8; 9; 8; 9; 9; 8. 6; 8; 8; 8; 8; 8; 8; 8; 12*. 7; 8; 8; 8; 8; 8; 8; 8; 9*. 8; 8; 8; 8; 8; 8; 9; 9; 12. 9; 9; 8; 8; 9; 8; 8; 8; 9. 10; 9; 9; 8; 8; 8; 9; 8; 12. Min.; 8; 8; 8; 8; 8; 8; 8; 8. Max.; 9; 9; 9; 9; 9; 9; 9; 12. a:Time-Weighted Mean, -: Not calculated, *:Dead offsprings were observed before living offsprings prodused Table 8. Mean Cumulative Numbers of Juveniles Produced per Adult Alive for 21 Days (ΣF1/P) Nominal Concentration (mg/L); Days. 7; 8; 9; 10; 11; 12; 13; 14; 15; 16; 17; 18; 19; 20; 21. Control; 0.0; 7.1; 10.8; 10.8; 29.1; 33.4; 33.4; 57.8; 67.2; 67.2; 93.7; 104.5; 104.5; 125.5; 142.2. 0.046; 0.0; 10.2; 11.1; 11.1; 31.8; 33.6; 33.6; 62.1; 64.7; 64.7; 93.2; 96.5; 96.5; 125.7; 132.7. 0.10; 0.0; 8.9; 10.4; 10.4; 29.9; 32.2; 32.2; 60.2; 64.7; 64.7; 86.3; 96.1; 96.1; 120.3; 131.1. 0.22; 0.0; 6.8; 10.2; 10.2; 23.1; 31.1; 31.1; 50.3; 61.2; 61.2; 73.4; 89.8; 89.8; 104.2; 122.9. 0.46; 0.0; 7.6; 9.5; 9.5; 27.3; 32.0; 32.0; 52.5; 62.5; 62.5; 77.4; 88.4; 88.4; 106.8; 119.0. 1.0; 0.0; 4.5; 8.6; 8.6; 20.8; 27.7; 27.7; 44.8; 57.6; 57.6; 70.9; 86.5; 86.5; 100.5; 117.7. 2.2; 0.0; 4.5; 8.0; 8.0; 17.4; 23.9; 23.9; 36.4; 47.4; 47.4; 60.8; 74.0; 74.0; 88.3; 102.0. 4.6; 0.0; 0.3; 1.0; 1.0; 1.1; 3.1; 3.1; 3.1; 7.0; 7.0; 7.0; 15.3; 15.9; 15.9; 20.9. a: Arithmetic mean Table 9. Cumulative Numbers of Juveniles Produced per Adult Alive for 21 Days in Each Test Vessel and Result of Statistical Comparison of the Mean Values (by Dunnett’s Multicomparison Test) Vessel No.; Nominal Concentration, mg/L. MeanaMeasured Concentration*1, mg/L. Control; 0.046; 0.10; 0.22; 0.46; 1.0; 2.2; 4.6. (-); (0.0424); (0.0958); (0.211); (0.447); (0.981); (2.18); (4.54). 1; 133; 125; 132; 132; 111; 112; 109; 5. 2; 148; 149; 112; 110; 112; 111; 80; 35. 3; 150; 135; 125; 127; 123; 104; 96; 17. 4; 152; 142; 137; 112; 93; 135; 103; 5. 5; 135; 128; 150; 118; 142; 108; 97; 28. 6; 143; 124; D; 138; 114; 138; 101; 23. 7; 147; 132; 133; 138; D; 124; 100; 13. 8; 155; 133; 133; 121; D; 94; 101; 43. 9; 134; 144; 126; 120; 134; 134; 115; 30. 10; 125; 115; 132; 113; 123; 117; 118; 10. Mean; 142.2; 132.7; 131.1; 122.9; 119.0; 117.7; 102; 20.9. S.D.; 9.9; 10.3; 10.2; 10.4; 15.1; 14.7; 10.7; 13. Inhibition rate(%); 6.7; 7.8; 13.6; 16.3; 17.2; 28.3; 85.3. Significant difference; N.S; N.S; p < 0.01; p < 0.01; p < 0.01; p < 0.01; p < 0.01. D: Not included for calculation because the parental Daphnia was dead during a 21-day testing period. N.S: Indicate no-significant difference, *1 : Time-Weighted Mean

# 6.1.5 Toxicity to aquatic algae and cyanobacteria

## Endpoint study record: MOE, 2009f (Toxicity to aquatic algae and cyanobacteria)

| UUID |  | IUC5-e1714409-9f09-4841-b3c5-db8d6fcdb540 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | CERI_M.SEKIZAWA / Chemicals Evaluation and Research Institute (CERI) / Tokyo / Japan |
| Date |  | 2012-11-29 15:27:28 JST |
| Remarks |  |  |

Administrative Data

| Purpose flag | key study; robust study summary |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result | Study period | 31 January 2008 - 23 June 2009 |
| Reliability | 1 (reliable without restriction) |  |  |
| Rationale for reliability incl. deficiencies | National Guideline study under GLP conditions |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | KUREHA SPECIAL LABORATORY CO.,LTD. | 2009 | Alga (Selenastrum capricornutum), Growth Inhibition Test of 2,5-dimethylaniline |  | KUREHA SPECIAL LABORATORY CO.,LTD. |  | Ministry of the Environment, Japan | 2007-71 | 2009-06-23 |

Materials and methods

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| equivalent or similar to | OECD Guideline 201 (Alga, Growth Inhibition Test) |  |

GLP compliance

yes

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 95-78-3 |
| EC number | 202-451-0 |
| EC name | 2,5-xylidine |
| IUPAC name | 2,5-dimethylaniline |

Details on test material

- Appearance: brownish yellow transparent liquid - Purity: 99.3% - Lot/batch No.: A0234724 - Storage condition of test material: In a refrigerator - Stability under the storage conditions: The test substance was stable under the storage conditions, because IR spectra of the test substance before and after the study were identical.

Analytical monitoring

yes

Details on sampling

- Frequency: at the start of the exposure (0 hour), at 48 hours after and at the termination of the exposure (72 hours after exposure)

Details on analytical methods

PRETREATMENT OF TEST SOLUTION Samples were diluted with eluting solution (2-200 times). ANALYTICAL CONDITIONS - Instrument: HPLC-UV - Column: Inertsil WP300 C4, 5 μm (150 mm×4.6 mm I.D.) - Column temp.: 40 degreeC - Eluent: Acetonitril/20mmol/L : phosphate buffer solution(pH8) (30/70) - Flow rate: 1.0 mL/min - Measurement wavelength: UV 230 nm - Injection volume: 20 μL - Determination limit: 0.02 mg/L - Detection limit: 0.004 mg/L

Vehicle

no

Details on test solutions

500mg of the test substance was dissolved with dilution water by stirring to produce a stock solution of 500 mg/L . Test solution was prepared in each test vessel by mixing the appropriate volume of the stock solution with dilution water. Dilution water was used for the control group.

Test organisms

Test organisms (species)

Pseudokirchnerella subcapitata

Details on test organisms

TEST ORGANISM - Strain: ATCC22662 - Source: Originally from American Type Culture Collection on 13 November 1997 PRECULTURE - Period: from 2009/2/7 to 2009/2/10 - Culturing media and conditions: Same as test OTHER - Confirming the reproducibility of the test conditions: - Reference substance: 3,5-dichlorophenol (purity: 98.0%) - Results: 72-hr ErC50 = 2.13 mg/L ( background data (2.32 +/- 0.14 mg/L) in this laboratory)

Study design

Test type

static

Water media type

freshwater

Limit test

no

Total exposure duration

| 72 h |  |
| --- | --- |
| Remarks | Shake culture (100 rpm) |

Test conditions

Test temperature

23.0 - 23.2 degreeC

pH

Control group: 7.8 (at the start of exposure), 7.9 (48 hours after), 8.6-8.9 (at the end of exposure) Exposure groups: 7.8-7.9 (at the start of exposure), 7.9-8.0 (48 hours after), 8.0-8.8 (at the end of exposure)

Nominal and measured concentrations

- Nominal concentrations: 0, 1.0, 2.2, 4.6, 10, 22, 46 and 100 mg/L - Measured concentrations (Geometric mean): 0, 0.929, 2.03, 4.28, 9.48, 20.6, 43.5 and 93.8 mg/L

Details on test conditions

TEST SYSTEM - Test vessel: - Type: erlenmeyer flask with silicon rubber plug (air permeability) - Fill volume: 300 mL - Test solution volume: 100 mL/vessel - Initial cells density: 0.5 × 10^4 cells/mL - No. of vessels per concentration: 3 - No. of vessels per control: 6 GROWTH MEDIUM - Standard medium used: yes (Sterile OECD medium) OTHER TEST CONDITIONS - Sterile test conditions: yes - Photoperiod: Continuous - Light intensity: 60-120 uE/m2/s OBSERVATION - Cell density: Counted with a particle counter at 24, 48, and 72 h - Cell condition: Microscopic observation were conducted at 24, 48, and 72 h - Test solutions: The pH of the test solution was measured at the start, 48 hours after and the end of the exposure. The temperature in the incubator and light intensity were measured once a day during the exposure.

Reference substance (positive control)

no

Any other information on materials and methods incl. tables

Table 1. Measured Concentration of the Substance in the Test Water Nominal Concentration (mg/L); Measured Concentration (mg/L) (Percent of Nominal); MeanaMeasured Concentration (mg/L). 0 Hour; 48 Hours; 72 Hours; 0-48-72 Hour. Control; <0.02; (-); <0.02; (-); <0.02; (-); －. 1.0; 0.966; (97); 0.915; (92); 0.900; (90); 0.929. 2.2; 2.10; (95); 2.01; (91); 1.94; (88); 2.03. 4.6; 4.42; (96); 4.26; (93); 4.09; (89); 4.28. 10; 9.75; (98); 9.47; (95); 9.01; (90); 9.48. 22; 21.2; (96); 20.4; (93); 19.8; (90); 20.6. 46; 45.0; (98); 43.0; (93); 42.0; (91); 43.5. 100; 97.9; (98); 92.9; (93); 88.4; (88); 93.8. a: Geometric mean, －: Not calculated Table 2. Daily Temperature, Light Intensity and Revolution at the Incubation Exposure Period (Hours); Temperature (oC); Light Intensity*(μE/m2/s); Revolution. Range of measured intensity; Average; (rpm). 0; 23.0; 66-79; 69; 101. 24; 23.2; 66-79; 69; 101. 48; 23.2; 66-79; 69; 101. 72.0; 23.2; 66-79; 69; 101. *: Calibrated from Light-measuring instruments in Lx (factor: 0.013) Table 3. pH Values Nominal Concentration (mg/l); MeanaMeasured Concentration (mg/l); pH. 0 Hour; 48 Hours; 72 Hours. Control; -; 1; 7.8; 7.9; 8.6. 2; 8.7. 3; 8.9. 4; 8.7. 5; 8.8. 6; 8.6. 1.0; 0.929; 1; 7.8; 7.9; 8.6. 2; 8.6. 3; 8.6. 2.2; 2.03; 1; 7.8; 8.0; 8.8. 2; 8.5. 3; 8.7. 4.6; 4.28; 1; 7.9; 8.0; 8.7. 2; 8.5. 3; 8.5. 10; 9.48; 1; 7.8; 8.0; 8.3. 2; 8.4. 3; 8.2. 22; 20.6; 1; 7.8; 8.0; 8.0. 2; 8.0. 3; 8.0. 46; 43.5; 1; 7.8; 7.9; 8.0. 2; 8.0. 3; 8.0. 100; 93.8; 1; 7.8; 7.9; 8.0. 2; 8.0. 3; 8.0. a: Geometric mean (0-48-72hr), -: Not calculated

Results and discussions

Effect concentrations

| Duration | Endpoint | Effect conc. | Nominal/Measured | Conc. based on | Basis for effect | Remarks (e.g. 95% CL) |
| --- | --- | --- | --- | --- | --- | --- |
| 72 h | EC50 | 30 mg/L | meas. (geom. mean) | test mat. | growth rate | 95%CL: 27-34 mg/L, probit |
| 72 h | NOEC | 2.03 mg/L | meas. (geom. mean) | test mat. | growth rate |  |

Details on results

CELL GROWTH - Control group: Increased to 297 times of initial cell density at the end of exposure, meets the validity of the test (at least 16 times growth)

Any other information on results incl. tables

Table 4. Cluculated Biomass (Dry Cell Weight) of Pseudokirchneriella subcapitata during the 72 hours Exposure Nominal Concentration (MeanaMeasured Conc.) (mg/l); Vessel No.; Calculated Biomass [Dry Cell Weight (mg/mL)]. 0 Hour; 24 Hours; 48 Hours; 72 Hours. Control (-); 1; 0.00010; 0.00056; 0.00440; 0.02881. 2; 0.00010; 0.00058; 0.00472; 0.02921. 3; 0.00010; 0.00060; 0.00439; 0.02901. 4; 0.00010; 0.00058; 0.00445; 0.03237. 5; 0.00010; 0.00057; 0.00473; 0.03013. 6; 0.00010; 0.00059; 0.00452; 0.02881. Average; 0.00010; 0.00058; 0.00454; 0.02972. SD; 0.00000; 0.00001; 0.00015; 0.00139. 1.0 (0.929); 1; 0.00010; 0.00056; 0.00437; 0.02799. 2; 0.00010; 0.00060; 0.00454; 0.02901. 3; 0.00010; 0.00058; 0.00437; 0.02881. Average; 0.00010; 0.00058; 0.00443; 0.02860. SD; 0.00000; 0.00002; 0.00010; 0.00054. 2.2 (2.03); 1; 0.00010; 0.00055; 0.00425; 0.02555. 2; 0.00010; 0.00060; 0.00443; 0.02799. 3; 0.00010; 0.00060; 0.00414; 0.02748. Average; 0.00010; 0.00058; 0.00427; 0.02701. SD; 0.00000; 0.00003; 0.00015; 0.00129. 4.6 (4.28); 1; 0.00010; 0.00053; 0.00362; 0.00038. 2; 0.00010; 0.00057; 0.00384; 0.02240. 3; 0.00010; 0.00058; 0.00383; 0.02341. Average; 0.00010; 0.00056; 0.00376; 0.02321. SD; 0.00000; 0.00003; 0.00012; 0.00073. 10 (9.48); 1; 0.00010; 0.00049; 0.00222; 0.01221. 2; 0.00010; 0.00053; 0.00286; 0.01486. 3; 0.00010; 0.00053; 0.00262; 0.01178. Average; 0.00010; 0.00052; 0.00257; 0.01295. SD; 0.00000; 0.00002; 0.00032; 0.00167. 22 (20.6); 1; 0.00010; 0.00040; 0.00090; 0.00306. 2; 0.00010; 0.00040; 0.00124; 0.00365. 3; 0.00010; 0.00045; 0.00122; 0.00420. Average; 0.00010; 0.00042; 0.00112; 0.00364. SD; 0.00000; 0.00003; 0.00019; 0.00057. 46 (43.5); 1; 0.00010; 0.00022; 0.00033; 0.00063. 2; 0.00010; 0.00022; 0.00043; 0.00083. 3; 0.00010; 0.00025; 0.00041; 0.00116. Average; 0.00010; 0.00023; 0.00039; 0.00087. SD; 0.00000; 0.00002; 0.00005; 0.00027. 100 (93.8); 1; 0.00010; 0.00013; 0.00014; 0.00025. 2; 0.00010; 0.00014; 0.00016; 0.00019. 3; 0.00010; 0.00015; 0.00017; 0.00020. Average; 0.00010; 0.00014; 0.00016; 0.00021. SD; 0.00000; 0.00001; 0.00002; 0.00003. SD : Standard deviation, -: Not calculated, a: Geometric mean (0-48-72h) Table 5. Percentage Growth Inhibition in Biomass of Pseudokirchneriella subcapitata Nominal Concentration (MeanaMeasured Conc.) (mg/l); Vessel No.; Growth Rate in Biomass. Rate μ(0-72hr); Inhibition (%)*1 Iμ(0-72hr). Control; 1; 1.8878. 2; 1.8924. 3; 1.8901. 4; 1.9266. 5; 1.9027. 6; 1.8878. Average; 1.8979; 0.00. SD; 0.0151. 1.0 (0.929); 1; 1.8781. 2; 1.8901. 3; 1.8878. Average; 1.8853; 0.66NS. SD; 0.0064. 2.2 (2.03); 1; 1.8477. 2; 1.8781. 3; 1.8720. Average; 1.8659; 0.66NS. SD; 0.0161. 4.6 (4.28); 1; 1.8244. 2; 1.8039. 3; 1.8186. Average; 1.8156; 4.34**. SD; 0.0106. 10 (9.48); 1; 1.6016. 2; 1.6671. 3; 1.5897. Average; 1.6195; 14.67**. SD; 0.0417. 22 (20.6); 1; 1.1403. 2; 1.1991. 3; 1.2459. Average; 1.1951; 37.03. SD; 0.0529. 46 (43.5); 1; 0.6135. 2; 0.7054. 3; 0.8170. Average; 0.7120; 62.4800. SD; 0.1019. 100 (93.8); 1; 0.3054. 2; 0.2140. 3; 0.2310. Average; 0.2501; 86.82. SD; 0.0486. *1: Values are the percent inhibition relative to the control., SD: Standard deviation, -: Not calculated, NS: No significant diferrence, a: Geometric mean (0-48-72h), *: Significant difference p<0.05, **: Significant diferrence p<0.01

# 6.1.7 Toxicity to microorganisms

## Endpoint study record: Arnold et al., 1990 (Toxicity to microorganisms)

| UUID |  | IUC5-e691c1d7-a827-4111-af42-e5104e11dc89 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V3.0 Plug-In |
| Date |  | 2012-06-06 15:33:46 JST |
| Remarks |  | Successfully migrated to IUCLID 5.4 format. |

Administrative Data

| Study result type | experimental result |  |  |
| --- | --- | --- | --- |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Acceptable, well-documented publication report which meets basic scientific principles. |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Arnold, L.M., D.T.Lin, and T.M.Schultz | 1990 | QSAR for Methyl- and/or Chloro-Substituted Anilines and the Polar Narcosis Mechanism of Toxicity. | Chemosphere. 21, 183-191 |  |  |  |  |  |

Data access

data published

Materials and methods

Principles of method if other than guideline

The 48-h sublethal population growth assay under static conditions for Tetrahymena pyriformis.

GLP compliance

no data

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Details on test material

Purity >95%

Analytical monitoring

yes

Details on sampling

Sampling time: At 0 and 48 hours

Details on analytical methods

HPLC

Vehicle

no data

Test organisms

Test organisms (species)

Tetrahymena pyriformis

Study design

Test type

static

Total exposure duration

48 h

Test conditions

Nominal and measured concentrations

Nominal concenrations: 5- to 10-step graded concentration series using freshly prepared stock solutions.

Details on test conditions

Test substance was tested in duplicate for minimum of three replicates following range finding experiments. Clutures without test substance served as controls. Cultures were reared in 250 mL Erlenmeyer flasks containing 50 mL of proteose-peptone medium. Cell population levels were estimated spectrophotometrically as absorbance at 540 nm. Only replicates with control absorbance values of 0.6-0.9, equivalent to late log-growth-phase, were used in the analyses. The IGC50, 50% growth inhibitory concentration, and 95% fiducial limits were determined for test substance using probit analysis of Statistical Analysis Systems.

Results and discussions

Effect concentrations

| Duration | Endpoint | Effect conc. | Nominal/Measured | Conc. based on | Basis for effect | Remarks (e.g. 95% CL) |
| --- | --- | --- | --- | --- | --- | --- |
| 48 h | other: IGC50 | 269 mg/L |  | test mat. | growth inhibition |  |

Details on results

Log IGC50 -1 = -0.346 mmol/L MW=121.18 IGC50=2.22mmol/L=269 mg/L

## Endpoint study record: Kaiser and Palabrica, 1991 (Toxicity to microorganisms)

| UUID |  | IUC5-7a516ba0-25ec-4c7d-8398-e1a8621f2223 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V3.0 Plug-In |
| Date |  | 2012-06-07 13:20:49 JST |
| Remarks |  | Successfully migrated to IUCLID 5.4 format. |

Administrative Data

| Study result type | experimental result |  |  |
| --- | --- | --- | --- |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Acceptable, well-documented publication report which meets basic scientific principles. |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Kaiser, K.L.E., and V.S. Palabrica | 1991 | Photobacterium phosphoreum Toxicity Data Index. | Water Pollut. Res. J. Can., 26(3), 361-431 |  |  |  |  |  |

Data access

data published

Materials and methods

Principles of method if other than guideline

Inhibition luminescence of marine luminescent bacteria

GLP compliance

no data

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Details on test material

Purity: 99%

Analytical monitoring

yes

Test organisms

Test organisms (species)

Photobacterium phosphoreum

Study design

Total exposure duration

| 30 min |  |
| --- | --- |
| Remarks | 5, 15 and 30 min |

Test conditions

Test temperature

15°C

pH

5-9

Results and discussions

Effect concentrations

| Duration | Endpoint | Effect conc. | Nominal/Measured | Conc. based on | Basis for effect | Remarks (e.g. 95% CL) |
| --- | --- | --- | --- | --- | --- | --- |
| 30 min | EC50 | 22.1 mg/L | meas. (not specified) | test mat. | other: luminescence inhibition |  |

Details on results

EC50: 50% reduction of the light emission relative to controls at the exposure time.

# 7 Toxicological information

# 7.1 Toxicokinetics, metabolism and distribution

# 7.1.1 Basic toxicokinetics

## Endpoint study record: Lindstrom HV et al (1963)

| UUID |  | IUC5-c7a09d6e-c29b-4165-95a1-d5e1f4142825 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | mariko / National Institute of Health Sciences / Tokyo / Japan |
| Date |  | 2013-02-06 14:22:42 JST |
| Remarks |  |  |

Administrative Data

| Study result type | experimental result |  |  |
| --- | --- | --- | --- |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Meets generally accepted scientific standards with acceptable restrictions. |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Lindstrom HV, Hansen WH, Nelson AA & Fitzhugh OG. | 1963 | The metabolism of FD&C Red No. 1. II. The fate of 2,5-paraxylidine and 2,6-metaxylidine in rats and observations on the toxicity of xylidine isomers. | Journal of Pharmacology and Experimental Therapeutics 142: 257 - 264. |  |  |  |  |  |

Data access

data published

Materials and methods

Type of method

in vivo

Objective of study

metabolism

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | other guideline: No data | no data |

GLP compliance

no data

Test materials

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 95-78-3 |
| EC name | 202-451-0 |
| EC name | 2,5-xylidine |

Radiolabelling

no

Details on test material

- Name of test material: 2,5-xylidine

Test animals

Species

rat

Strain

Osborne-Mendel

Sex

male

Details on test animals and environmental conditions

TEST ANIMALS - Weight at study initiation: 250 - 300 g

Administration / exposure

Route of administration

oral: gavage

Vehicle

water

Details on exposure

No data

Duration and frequency of treatment / exposure

8 days

Doses / concentrations

200 mg/kg

No. of animals per sex per dose

No data

Control animals

no data

Positive control

No data

Details on study design

No data

Details on dosing and sampling

The animals were used no more than 8 days since the urine was found to become quite dark after that time and it was required that the metabolic behaviour was as normal as possible, therefore the collections were limited to collection of hte lighter urine. Previous studies have suggested that the development of dark colour in the urine was associated with disturbances which can complicate the fate of xylidines in the animal and make it more difficult to characterise the normal metabolism.

Statistics

No data

Any other information on materials and methods incl. tables

Isolation and identification of amino-xylols: The plate-like crystals that were obtained were compared with synthetic materials as to melting point and mixed melting point values and to their infra red and ultraviolet absorption spectra. These tests demonstrated that the amino-xylols from 2,5-xylidine were identical in structure to 4-hydroxy-2,5-dimethylaniline. Characterisation of aminotoluic acids: In earlier work, the 4-methyl group in 2,4-xylidine was shown to be oxidised to a carboxyl group, resulting in the formation of aminotoluic acid. Similar oxidations in 2,5-xylidine were deemed possibe when concentrated urine samples from rats fed this isomer was tested by diazolitization and coupling after paper chromatography. There were spots on the paper strips which gave colours characteristic of aromatic carboxy acids. However, attempts to isolate such oxidation products from urine were unsuccessful due to their presence only in traces. Paper chromatographic and paper electrographic techniques requiring small amounts of material were utilised to identify these metabolites. In electrophoresis the amphoteric nature of aminotoluic acids makes possible their characterisation due to migrations in different directions in acid and basic solutions. Hydrolysed urine samples were extracted with diethyl ether at pH 7.5tp remove amino-xylols as much as possible. The residual hydrolysates were purified using ion-exchange resin and extracted into n-butanol., with sodium sulfate utilised to saturate the water phase. 2,5-paraxylidine was metabolised in a small measure to 4-methyl2-amino-benzoic acid and to 4-methyl-3-aminobenzoic acid. Extraction of original xylidines: Paper chromatographs of 2,5-xylidine show that this amine has an Rt value near 0.96 and gives a pale blue colour upon diazotization and coupling on paper strips. Chromatographs from urines of animals fed this xylidine yield spots with the same Rt values and colours, but do so only after urine hydrolysis and the authors conclude that this may indicate that a certain amount of these xylidines is not oxidised in the rat, but is merely conjugated at the amino group.

Results and discussions

Pharmacokinetic studies

Details on absorption

No data

Details on distribution in tissues

No data

Details on excretion

No data

Metabolite characterisation studies

Metabolites identified

yes

Details on metabolites

Isolation and identification of amino-xylols: The plate-like crystals that were obtained were compared with synthetic materials as to melting point and mixed melting point values and to their infra red and ultraviolet absorption spectra. These tests demonstrated that the amino-xylols from 2,5-xylidine were identical in structure to 4-hydroxy-2,5-dimethylaniline. Characterisation of aminotoluic acids: In earlier work, the 4-methyl group in 2,4-xylidine was shown to be oxidised to a carboxyl group, resulting in the formation of aminotoluic acid. Similar oxidations in 2,5-xylidine were deemed possibe when concentrated urine samples from rats fed this isomer was tested by diazolitization and coupling after paper chromatography. There were spots on the paper strips which gave colours characteristic of aromatic carboxy acids. However, attempts to isolate such oxidation products from urine were unsuccessful due to their presence only in traces. Paper chromatographic and paper electrographic techniques requiring small amounts of material were utilised to identify these metabolites. In electrophoresis the amphoteric nature of aminotoluic acids makes possible their characterisation due to migrations in different directions in acid and basic solutions. Hydrolysed urine samples were extracted with diethyl ether at pH 7.5tp remove amino-xylols as much as possible. The residual hydrolysates were purified using ion-exchange resin and extracted into n-butanol., with sodium sulfate utilised to saturate the water phase. 2,5-paraxylidine was metabolised in a small measure to 4-methyl2-amino-benzoic acid and to 4-methyl-3-aminobenzoic acid. Extraction of original xylidines: Paper chromatographs of 2,5-xylidine show that this amine has an Rt value near 0.96 and gives a pale blue colour upon diazotization and coupling on paper strips. Chromatographs from urines of animals fed this xylidine yield spots with the same Rt values and colours, but do so only after urine hydrolysis and the authors conclude that this may indicate that a certain amount of these xylidines is not oxidised in the rat, but is merely conjugated at the amino group.

Applicant's summary and conclusion

Conclusions

The metabolism of the 2,5-xylidine was investigated in Osborne-Mendel rats. Results showed that the test substance was oxidised mainly to amino-xylols.

## Endpoint study record: Magnusson G et al (1979)

| UUID |  | IUC5-b4cf3de6-6eb7-4b3c-a213-0e810e476134 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | mariko / National Institute of Health Sciences / Tokyo / Japan |
| Date |  | 2013-02-06 14:22:42 JST |
| Remarks |  |  |

Administrative Data

| Reliability | 2 (reliable with restrictions) |
| --- | --- |
| Rationale for reliability incl. deficiencies | Study published in peer-reviewed journal. |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Magnusson G, Majeed SK, Down WH, Sacharin RM, Jorgeson W | 1979 | Hepatic Effects of Xylidine Isomers in Rats. | Toxicology, 12, 63-74 |  |  |  |  |  |

Data access

data published

Materials and methods

Type of method

in vivo

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | other guideline: No data | no data |

Principles of method if other than guideline

The effect of repeated (4 weeks) oral administration of 2,5-xylidine at the dose levels of 400 -500 mg/kg/day on the morphology and microsomal drug metabolising enzyme activity of the liver was studied in rats.

GLP compliance

no (Study pre-dates GLP.)

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 95-78-3 |
| EC number | 202-451-0 |
| EC name | 2,5-xylidine |

Radiolabelling

no data

Details on test material

IUCLID4 Test substance: as prescribed by 1.1 - 1.4

Test animals

Species

rat

Strain

other: Sprague-Dawley derived CFY strain

Sex

male/female

Details on test animals and environmental conditions

TEST ANIMALS - Source: Anglia Laboratory Animals, Alconbury, Huntingdon UK. - Age at study initiation: 8 weeks - Weight at study initiation: Males: 200g; females: 170 g - Fasting period before study: - Housing: 5 rats/ polycarbonate cage. - Individual metabolism cages: No - Diet: Ad libitum - Water: Ad libitum ENVIRONMENTAL CONDITIONS - Temperature (°C): 23 - 27 °C - Humidity (%): 45 - 55 % - Photoperiod (hrs dark / hrs light): 12h light/12 h dark

Administration / exposure

Route of administration

oral: gavage

Vehicle

no data

Duration and frequency of treatment / exposure

28 day(s)

Doses / concentrations

Males: 400 mg/kg bw/day for the first week then 500 mg/kg bw/week Females: 400 mg/kg bw/day for the first week then 500 mg/kg bw/week

No. of animals per sex per dose

Males: 10; Females: 10

Control animals

yes

Details on study design

Dogs: During the experiment the dogs were observed for clinical effects of the compound. Before the start and once a week during the experiment the haematological and blood chemistry examinations were performed. Rats: During the experiment the rats were observed daily for clinical effects. At termination haematological and blood chemistry examinations were performed.

Details on dosing and sampling

No data

Statistics

Statistical significance between the values was calculated by Bartlett's t-test.

Any other information on materials and methods incl. tables

ANIMALS: Strain: sprague-Dawley Age: 8 weeks Weight: Males - approximately 200 g; Females - approximately 170 g EXPOSURE: Control: saline (5mL/kg) Dose volume: 5 mL/kg animals/sex/group: 5 OBSERVATIONS: Rats were weighed once per week and observed daily for clinical signs. NECROPSY: Post mortem examination was performed on all animals at the end of the study, including any that died during the study. MORPHOLOGY: Tissue samples of liver were fixed for examination. BIOCHEMISTRY: Microsomal suspensions of the liver were prepared for assay of cytochrome P-450 concentration and activity of aniline hydroxylase and p-nitrophenol glucuronyltransferase.

Results and discussions

Preliminary studies

No data

Pharmacokinetic studies

Details on absorption

No data

Details on distribution in tissues

No data

Details on excretion

No data

Metabolite characterisation studies

Metabolites identified

not measured

Details on metabolites

No data

Bioaccessibility

Any other information on results incl. tables

No mortalities due to test substance administration occurred during the study. A reduction in body weight gain was observed in all treated rats, especially the males, but this was not significant. There was no difference in food consumption between control and treated groups. Livers were enlarged, but had a normal gross appearance. Relative liver weight was significantly increased compared to controls in both male and female rats (p<0.05 and p<0.01, respectively). Microscopic examination revealed a minimal fine droplet fatty change of the livers of comparable degree in both control and treated groups. Furthermore, there was an apparent enlargement of the hepatocytes, which was significant in the centrilobular region (p<0.01) for both males and females. Staining with PAS demonstrated a centrilobular decrease in liver glycogen. This decrease was greater in males than in females. No mortalities due to test substance administration occurred during the study. A reduction in body weight gain was observed in all treated rats, especially the males, but this was not significant. There was no difference in food consumption between control and treated groups. Livers were enlarged, but had a normal gross appearance. Relative liver weight was significantly increased compared to controls in both male and female rats (p

Applicant's summary and conclusion

Conclusions

The effect of repeated (4 weeks) oral administration of 2,5-xylidine at the dose levels of 400 - 500 mg/kg/day on the morphology and microsomal drug metabolising enzyme activity of the liver was studied in rats. No mortalities due to test substance administration occurred during the study. A reduction in body weight gain was observed in all treated rats, especially the males, but this was not significant. There was no difference in food consumption between control and treated groups. Livers were enlarged, but had a normal gross appearance. Relative liver weight was significantly increased compared to controls in both male and female rats (p<0.05 and p<0.01, respectively). Microscopic examination revealed a minimal fine droplet fatty change of the livers of comparable degree in both control and treated groups. Furthermore, there was an apparent enlargement of the hepatocytes, which was significant in the centrilobular region (p<0.01) for both males and females. Staining with PAS demonstrated a centrilobular decrease in liver glycogen. This decrease was greater in males than in females. Glucose-6-phosphatase activity was evenly distributed in control animals but in treated animals there was a centrilobular decrease in enzyme activity. The hepatic microsomal protein content was increased in males, but not in females. The hepatic microsomal cytochrome P-450 content was increased in males and females. Total aniline hydroxylase activity was not increased. Glucuronyltransferase activity was increased, but not significantly, over controls.

## Endpoint study record: Cauchon D & Krishnan K (1997)

| UUID |  | IUC5-233201fc-86ed-4f0d-898f-8e6c8aa1b55c |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | mariko / National Institute of Health Sciences / Tokyo / Japan |
| Date |  | 2013-02-06 14:22:42 JST |
| Remarks |  |  |

Administrative Data

| Reliability | 2 (reliable with restrictions) |
| --- | --- |
| Rationale for reliability incl. deficiencies | Study published in peer-reviewed journal. |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Cauchon D & Krishnan K | 1997 | In Vitro and In Vivo Evaluations of the Methaemoglobinaemic Potential of Xylidine Isomers in the Rat. | J. Appl. Toxicol., 17 (6) 397-404 |  |  |  |  |  |

Data access

data published

Materials and methods

Type of method

other: In vitro and in vivo

Objective of study

toxicokinetics

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | other guideline: No data | no data |

Principles of method if other than guideline

The methaemoglobinaemic potential of 2,5-xylidine was assessed in vitro, using red blood cells and hepatic post-mitochondrial fractions in a two-compartmental dialysis system; and in vivo, following a single oral dose of 4.8 mmol kg-1 (p.o.) of the test substance.

GLP compliance

no data

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 95-78-3 |
| EC number | 202-451-0 |
| EC name | 2,5-xylidine |

Details on test material

- Name of test material: 2,5-xylidine - Analytical purity: 98 - 99.5 %

Test animals

Species

rat

Strain

Sprague-Dawley

Sex

male

Details on test animals and environmental conditions

In vivo: TEST ANIMALS - Source: Charles River Canada Ltd. - Weight at study initiation: 200 - 220 g - Housing: Stainless steel - Diet: Ad libitum - Water: Ad libitum - Acclimation period: 4 - 7 days ENVIRONMENTAL CONDITIONS - Photoperiod (hrs dark / hrs light): 12 h dark/ 12 h light

Administration / exposure

Route of administration

other: In vivo: p.o.;

Vehicle

other: In vivo: Corn oil;

Duration and frequency of treatment / exposure

In vivo: Single application In vitro: for "intact" xylindines: 1 h; for "biotransformed" xylidines: 4 h.

Doses / concentrations

In vivo: 4.8 mmol/kg-1 (total volume 10 mL kg-1 body weight). In vitro: 1 mM for "intact" xylidines; 1, 0.3 or 0.6 mM for "biotransformed" xylidines.

No. of animals per sex per dose

In vivo: 6 males In vitro: Not applicable

Control animals

other: In vivo: Time zero methaemoglobin level served as the reference/control value for each animal; In vitro: Methanol

Details on study design

In vivo: Samples of blood were withdrawn from the tail vein of individual animals at 1, 2, 4 and 6 h following administration. Blood samples were also obtained before the administration of the test substance and analysed for methaemoglobin levels such that the level at time zero served as the reference/control value for each animal. Methaemoglobin levels relative to total haemoglobin were measured by a modification of Evelyn and Malloy's spectrophotometric technique. Samples of erythrocyte suspensions were haemolysed with distilled water and phosphate buffer containing Triton X-100. An aliquot of hte haemolysates was transferred to two cuvettes and absorbance at 630 nm was measured from cuvette 1 to generate reading R1. an aliquot of 20 % potassium ferricyanide was added to cuvette 2 and absorbance measured 5 min later to give reading R2. An aliquot of 10 % KCN was then added to both cuvettes C1 and C2 and the absorbances were measured 5 min later to obtain readings R3 and R4. The percentage of haemoglobin oxidised to methaemoglobin was calculated at 100 times the difference between R1 and R3 divided by R2 minus R4. In vitro: Methaemoglobinaemic potential of "intact" xylidines. To evaluate the methaemoglobinaemic potential of "intact" xylidines they were added directly to erythrocyte preparations. Aliquots of erythrocyte suspensions in glass vials were pre-incubated at 37 °C in a water bath for 10 min. An aliquot of each of each isomer solution prepared in methanol was added to the erythrocyte suspension and incubated for 1 h at 37 °C at the end of which the methaemoglobin levels were quantitated. Methaemoglobinaemic potential of "biotransformed" xylidines. To evaluate the methaemoglobinaemic potential of "biotransformed" xylidines the Dianrom® system was used, which consisted or a series of dialysis cells made form two tightly fitting Teflon® compartments (working volume of 1 mL per compartment) separated by a flat semi-permeable membrane (63 mm diameter, molecular weight cutoff 10000). Four sets of five each of these biocompartmental systems were mounted on a motorised support (providing constant rotation at 8 rpm) and immersed in a water bath at 37 °C. The semi-permeable membranes were conditioned by initially soaking them in distilled water for 30 min and then in phosphate buffer (pH 7.4) for at least 30 min prior to mounting them on dialysis cells. An aliquot of the erythrocyte suspension was injected into one of the compartments and an aliquot of the metabolising fraction was added to the other compartment. The metabolising fraction consisted of protein and cofactor solution. Following a pre-incubation for 10 min, the dialysis cells were removed from the water bath and an aliquot of the xylidine isomer solution prepared in methanol was quickly injected into compartment A (containing the erythrocytes) and placed into the bath for the duration of incubation (up to 4 h). At the end of the incubation period an aliquot of the erythrocyte suspension was removed for quantitation of the methaemoglobin levels.

Statistics

The data on methaemoglobin levels in each treatment group were submitted to Student's t-test with an acceptable level of significance of 0.05. The area under the percentage methaemoglobin vs time curve (AUCmetHb) was calculated according to the trapezoidal rule.

Any other information on materials and methods incl. tables

After 4 - 7 days acclimatization the rats were either used in the in vivo experiments or sacrificed by CO2 euthanasia to obtain liver and blood sampled for use in the in vitro experiments. The liver S9 fractions were prepared from individual rats immediately after their sacrifice by centrifuging the liver homogenates.

Results and discussions

Preliminary studies

No data

Pharmacokinetic studies

Details on absorption

No data

Details on distribution in tissues

No data

Details on excretion

No data

Bioaccessibility

Any other information on results incl. tables

The test substance at 1 mM concentration failed to induce methaemoglobinaemia following a 1 h incubation with rat erythrocytes. However in the two-compartment in vitro system the same concentration (1 mM) induced significant methaemoglobulinaemia but only in the presence of active S9. At lower concentrations the effects were dramatically less potent (0.3 mM)or totally ineffective(0.06 mM) as inducers of methaemoglobinaemia. The percentage of methaemoglobin observed following incubation with 0.06 mM of the test substance was less than 5% suggesting that this blood concentration in vitro and in vivo is the NOAEL. The test substance failed to induce methaemoglobinaemia in vivo following a single oral dose. IN VITRO 2,5-xylidine at 1 mM concentration induced a significant. Table 1: Time course of methaemoglobinaemia induced by 1mM 2,5-xylidine isomers in a two-compartment in vitro system Time (hours); Methaemoglobin level (as % haemoglobin)*. 0.5; 7.96±0.96. 1; 31.36±7.44. 2; 12.43±1.59. 3; 10.58±0.84. 4; 11.99±3.54. AUCmetHb**; 49.42. *: The data represent mean±SEM (n=3) **: Area under the % methaemoglobin vs time curve (AUCmetHb) was calculated according to the trapezoidal rule ***: n=2 Table 2: Percentage methaemoglobin in rat erythrocytes at 1h and 2h following the introduction of 0.06 mM 2,6-xylidine in a two compartment in vitro system Time (hours); Methaemoglobin level (as % haemoglobin)*; Control. 1; 3.10±0.27; 2.90±0.27. 2; 2.90±0.32; 2.56±0.13. *: The data represent mean±SEM (n=3) IN VIVO In vivo, 2,5 xylidine did not induce methaemoglobinaemia after a single oral dose of 4.8 mmol/kg bw (Table 3). Table 3: Time course of methaemoglobinaemia in the rat following a single oral dose of 4.8 mmol/kg 2,6-xylidine Time (hours); Methaemoglobin level (as % haemoglobin)*. 0; 1.8±0.3. 1; 1.0±0.2. 2; 1.4±0.1. 4; 1.0±0.1 6. *: The data represent mean±SEM (n=3) for the time points at which blood was collected The quantitative differences between the in vivo and in vitro results may have been due to additional bioactivation pathways (N-hydroxylation or ring hydroxylation) mediated by high Km enzymes operative at the high incubation concentrations used in vitro.

Applicant's summary and conclusion

Conclusions

The methaemoglobinaemic potential of 2,5-xylidine was assessed in vitro, using red blood cells and hepatic post-mitocondrial fractions in a two-compartmental dialysis system; and in vivo, following a single oral dose of 4.8 mmol kg-1 (p.o.) of the test substance. The test substance at 1 mM concentration failed to induce methaemoglobinaemia following a 1 h incubation with rat erythtrocytes. However in the two-compartment in vitro system the same concentration(1 mM)induced significant methaemoglobulinaemia but only in the presence of active S9. At lower concentrations the effects were dramatically less potent(0.3 mM)or totally ineffective(0.06 mM) as inducers of methaemoglobinaemia. The percentage of methaemoglobin observed following incubation with 0.06 mM of the test substance was less than 5% suggesting that this blood concentration in vitro and in vivo is the NOAEL. The test substance failed to induce methaemoglobinaemia in vivo following a single oral dose.

## Endpoint study record: McLean S, Starmer GA & Thomas J (1969)

| UUID |  | IUC5-1cdd8266-a570-48ac-8a6c-5b8c90e2baeb |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | mariko / National Institute of Health Sciences / Tokyo / Japan |
| Date |  | 2013-02-06 14:22:43 JST |
| Remarks |  |  |

Administrative Data

| Reliability | 4 (not assignable) |
| --- | --- |
| Rationale for reliability incl. deficiencies | Insufficient detail in the report published in the literature. |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | McLean S, Starmer GA & Thomas J | 1969 | Methaemoglobin Formation by Aromatic Amines. | J. Pharm. Pharmac., 21, 441-450 |  |  |  |  |  |

Data access

data published

Materials and methods

Type of method

in vivo

Objective of study

toxicokinetics

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | other guideline: No data | no data |

Principles of method if other than guideline

Methaemoglobin formation induced in cats by substituted anilines was investigated in an attempt to correlate chemical structure with activity. Cats were used due to their sensitivity of methaemoglobin formation.

GLP compliance

no (Study pre-dates GLP)

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 95-78-3 |
| EC number | 202-451-0 |
| EC name | 2,5-xylidine |

Radiolabelling

no data

Details on test material

- Name of test material: 2,5-dimethylaniline

Test animals

Species

cat

Strain

no data

Sex

no data

Details on test animals and environmental conditions

TEST ANIMALS - Age at study initiation: > 24 weeks

Administration / exposure

Route of administration

intravenous

Vehicle

no data

Duration and frequency of treatment / exposure

5 h

Doses / concentrations

0.25 mmol/kg

No. of animals per sex per dose

5 animals, sex not specified.

Control animals

yes

Positive control

No data

Details on study design

Solution of 2,5-xylidine hydrochloride adjusted to pH 5.5 and made isotonic with sodium chloride. Injections made slowly into the femoral vein with a dose volume of approximately 10mL. Blood samples were taken from the femoral vein just before administration of a drug and at each hour afterwards for 5 hours. At least 5 cats and 2 dogs were used. The normal level of methaemoglobin was also measured in a population of 152 cats.

Details on dosing and sampling

No data

Statistics

No data

Results and discussions

Preliminary studies

No data

Pharmacokinetic studies

Details on absorption

No data

Details on distribution in tissues

No data

Details on excretion

No data

Metabolite characterisation studies

Metabolites identified

no data

Details on metabolites

Dimethylanilines formed less methaemoglobin than aniline. The level of methaemoglobin remained relatively constant over 5 h.

Bioaccessibility

Any other information on results incl. tables

Table 1: Formation of methaemoglobin after intravenous administration of 2,5-xylidine Time (hours); Methaemoglobin % ±S.E.. 1; 30.7±5.0. 2; 35.2±7.7. 3; 30.0±4.6. 4; 28.6±2.6. 5; 21.1±4.3. Mean; 29.1. Mean Maximum; 36.3±7.1. The normal level of methaemoglobin measured in a population of 152 cats was 1.1% (less than the sensitivity of the assay).

Applicant's summary and conclusion

Conclusions

Methaemoglobin formation induced in cats by substituted anilines was investigated in an attempt to correlate chemical structure with activity. Dimethylanilines formed less methaemoglobin than aniline. The level of methaemoglobin remained relatively constant over 5 h.

# 7.2 Acute Toxicity

# 7.2.1 Acute toxicity: oral

## Endpoint study record: Vernot EH et al (1977)

| UUID |  | IUC5-c33e8ea6-158f-473e-960a-8e84d9386283 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | mariko / National Institute of Health Sciences / Tokyo / Japan |
| Date |  | 2013-02-06 14:22:43 JST |
| Remarks |  |  |

Administrative Data

| Purpose flag | supporting study |
| --- | --- |
| Reliability | 2 (reliable with restrictions) |
| Rationale for reliability incl. deficiencies | The report published in the literature. |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Vernot EH, MacEwen JD, Haun CC & Kinkead ER. | 1977 | Acute Toxicity and Skin Corrosion Data for Some Organic and Inorganic Compounds and Aqueous Solutions. | Toxicol. Appl. Pharmacol., 42, 417-423. |  |  |  |  |  |

Data access

data published

Materials and methods

Test type

standard acute method

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | other guideline: no data | no data |

Principles of method if other than guideline

Method: T03-03: Single oral dose toxicity was determined in which the LD50 and its 95 % confidence limts were estimated by the moving average technique.

GLP compliance

no (Study pre-dates GLP.)

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 95-78-3 |
| EC number | 202-451-0 |
| EC name | 2,5-xylidine |

Details on test material

IUCLID4 Test substance: as prescribed by 1.1 - 1.4

Test animals

Species

other: Rats and mice

Strain

other: Rats: Sprague-Dawley; mice: CF-1

Sex

male

Details on test animals and environmental conditions

TEST ANIMALS - Weight at study initiation: Rats: 200 - 300 g; mice: 22 - 28 g

Administration / exposure

Route of administration

oral: unspecified

Vehicle

no data

Details on oral exposure

No data

Doses

No data

No. of animals per sex per dose

No data

Control animals

no data

Details on study design

No data

Statistics

No data

Any other information on materials and methods incl. tables

Range-finding toxicity data: List VI. Amer. Ind. Hyg. Ass. J., 23, 95-107

Results and discussions

Preliminary study (if fixed dose study)

No data

Effect levels

| Sex | Endpoint | Effect level | Based on | 95% CL | Remarks |
| --- | --- | --- | --- | --- | --- |
| male | LD50 | 1300 mg/kg bw | no data | 940 — 2140 | Male rats |
| male | LD50 | 840 mg/kg bw | no data | 470 — 1490 | Male mice |

Mortality

No data

Clinical signs

No data

Body weight

No data

Gross pathology

No data

Applicant's summary and conclusion

Interpretation of results

other: No data

Criteria used for interpretation of results

not specified

Conclusions

The acute oral LD50 for the test substance in male rats was 1300 (940 - 2140) mg/kg; in male mice it was 840 (470 - 1490) mg/kg.

# 7.5 Repeated dose toxicity

# 7.5.1 Repeated dose toxicity: oral

## Endpoint study record: MHLW (2011)

| UUID |  | IUC5-48d9209c-46be-48a9-a1f2-d00679df029d |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | mariko / National Institute of Health Sciences / Tokyo / Japan |
| Date |  | 2013-02-06 14:22:43 JST |
| Remarks |  |  |

Administrative Data

| Purpose flag | key study |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result |  |  |
| Reliability | 1 (reliable without restriction) |  |  |
| Rationale for reliability incl. deficiencies | The study was conducted in accordance with OECD Test Guidelines and under GLP. |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | MHLW (Ministry of Health, Labour and Welfare), Japan | 2011 | Twenty-eight-day Repeat Dose Oral Toxicity Test of 2,5-dimethylaniline in Rats | Unpublished | Safety Research Institute for Chemical Compounds Co., Ltd. |  |  |  |  |

Materials and methods

Test type

subacute

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | other guideline: Guideline for 28-Day Repeated Dose Toxicity Test in Mammalian Species (Chemical Substances Control Law of Japan) and OECD Guideline 407 |  |

GLP compliance

yes (incl. certificate)

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Details on test material

- Name of test material (as cited in study report): 2,5-Dimethylaniline - Analytical purity:= 99.3% - Lot No.: A0234724 - Stability under test conditions: The stability of test material was identified by analysis of the remainder.

Test animals

Species

rat

Strain

other: Crl: CD(SD)

Sex

male/female

Details on test animals and environmental conditions

TEST ANIMALS - Source: Charles River Japan, Inc. - Age at study initiation: 5 weeks old - Weight at study initiation: male (140-168 g), female (111-139 g) - Housing: single animal was housed in a metallic cage with wire mesh bottoms - Diet: MF , Oriental Yeast Co., Ltd., Japan (ad libitum) - Water: Tap water (ad libitum) - Acclimation period:7 days ENVIRONMENTAL CONDITIONS - Temperature (°C): 21-25 - Humidity (%): 43-57 - Air changes (per hr):10-15 - Photoperiod (hrs dark / hrs light): 12 hr dark/12 hr light

Administration / exposure

Route of administration

oral: gavage

Vehicle

corn oil

Analytical verification of doses or concentrations

yes

Duration of treatment / exposure

28 days

Frequency of treatment

once a day

Doses/concentrations

| 0, 12, 60 and 300 mg/kg bw/day |  |
| --- | --- |
| Basis | actual ingested |

No. of animals per sex per dose

6 animals/sex/dose (treatment groups) 6 animals/sex/dose for 0 and 300 mg/kg bw/day (recovery groups)

Control animals

yes, concurrent vehicle

Details on study design

Dose was selected based on a dose-finding study, in which 3 males and 3 females were dosed at 0 (corn oil), 35, 150, 600 mg/kg bw/day for 14 days, increases in urine specific gravity and the amount of urine with the fall of pH, the anemia tendency, a decrease in the body weight gain, and hypertrophy of the liver and spleen with increases in the weight of the liver and kidney were observed in males or females at 150 or 600 mg/kg bw/day. Moreover, the dosing period in the main study was longer than that in the dose-finding study. Therefore, the highest dose for the main study was set as 300 mg/kg bw/day. Middle and low dose levels were set as common ration of 5. Rats (12/sex/group) were treated with 2,5-dimethylaniline via oral gavage at 0, 12, 60 or 300 mg/kg bw/day. Six/sex/group at 0 and 300 mg/kg bw/day were examined after a 14-day recovery period.

Examinations

Observations and examinations performed and frequency

CAGE SIDE OBSERVATIONS: Yes - Time schedule: All males and females were observed every day during the administration period and the recovery period. DETAILED CLINICAL OBSERVATIONS: Yes The functional observational battery testing (FOB) was performed on all animals. Among the measures in the FOB, detailed clinical observations were made before the initiation of dosing and once a week thereafter. Sensory motor reflexes, forelimb and hindlimb grip strengths, and motor activity were measured at Week 4 of administration period and Week 2 of recovery period BODY WEIGHT: - Time schedule for examinations: Body weights were measured on days 1, 4, 7, 10, 14, 17, 21 and 28 of administration period, and on days 7 and 14 of recovery period. FOOD CONSUMPTION: - Time schedule for examinations: Food consumption of all rats were measured on days 1, 4, 7, 10, 14, 17, 21 and 28 of administration period and on days 7 and 14 of administration. Water CONSUMPTION: - Time schedule for examinations: Water consumption of all rats were measured on days 1, 7, 14, 21 and 28 of administration period and on days 7 and 14 of administration. HAEMATOLOGY: - Time schedule for collection of blood: Hematological examinations were performed on the blood samples obtained just prior to necropsy. - Anaesthetic used for blood collection: Yes - Animals fasted: Yes - How many animals:6 /sex/dose CLINICAL CHEMISTRY: Yes - Time schedule for collection of blood: Clinical chemistry examinations were performed on the blood samples obtained just prior to necropsy. - Animals fasted: Yes - How many animals: 6/sex/dose OPHTHALMOSCOPIC EXAMINATION: No URINALYSIS: Yes - Time schedule for collection of urine: Urinary samples were collected in the fourth week of the administration period and in the second week of the recovery period. NEUROBEHAVIOURAL EXAMINATION: No

Sacrifice and pathology

GROSS PATHOLOGY: Yes HISTOPATHOLOGY: Yes (adrenal, bone, bone marrow, cerebellum, cerebrum, epididymis, esophagus, eye, harderian gland, heart, intestine, kidney, liver, lung, lymph node, medullary, ovary, pancreas, parathyroid, pituitary, prostate, sciatic nerve, seminal vesicle including coagulating gland, spinal cord, spleen, stomach, testis, thymus, thyroid, tongue, trachea, urinary bladder, uterus and vagina) Histopathological examinations were performed for all animals at 0 and 300 mg/kg bw/day, and in addition, forstomach, liver, kidney, spleen and marrow in the femur for all animals at 12 and 60 mg/kg bw/day Organ weight: adrenal glands, brain, epididymidis, heart, kidneys, liver, ovaries, pituitary, spleen, testes, thymus, thyroid and uterus were weighed at necropsy.

Statistics

Continuous data were analyzed by the Bartlett’s test for homogeneity of distribution. When homogeneity was recognized, one-way ANOVA was applied for comparison of mean values, and then if significant, Dunnett test was conducted. If not homogenous, the data were analyzed using the Kruskal-Wallis test, and then if significant, Mann-Whitney U-test was conducted. For quantitative urinalysis data, Kruskal-Wallis test and Mann-Whitney U-test were performed.

Results and discussions

Results of examinations

Clinical signs and mortality

yes

Body weight and weight gain

yes

Food consumption and compound intake (if feeding study)

yes

Food efficiency

no data

Water consumption and compound intake (if drinking water study)

yes

Ophthalmoscopic examination

no data

Haematology

yes

Clinical chemistry

yes

Urinalysis

yes

Neurobehaviour

no data

Organ weights

yes

Gross pathology

yes

Histopathology: non-neoplastic

yes

Histopathology: neoplastic

no effects

Details on results

CLINICAL SIGNS AND MORTALITY On day 5 in the administration period, lacrimation and staggering gait were observed in two males and four females at 300 mg/kg bw/day. In one female, salivation, staggering gait, and soil of perigenital and perioral fur were observed at 300 mg/kg bw/day, but the frequency was only occasional (not considered toxicological effects). DETAILED CLINICAL OBSERVATIONS: no effects BODY WEIGHT AND WEIGHT GAIN In the administration period, significant decreases in the body weight and weight gain were observed in males at 300 mg/kg bw/day from day 4 in the administration period, and a significant decrease in the body weight on day 7 was observed in females at 300 mg/kg bw/day. In the recovery period, an increase in the body weight gain was observed in males at 300 mg/kg bw/day. FOOD CONSUMPTION AND COMPOUND INTAKE (if feeding study) At 300 mg/kg bw/day in the administration period, a significant decrease in the food consumption was observed in males on days 4-28 and in females on days 4 and 7. WATER CONSUMPTION AND COMPOUND INTAKE (if drinking water study) In the administration period, an increase in the water intake was observed in males on days 7 and 21 at 60 mg/kg bw/day, and in males and females on days 7-28 at 300 mg/kg bw/day. In the recovery period, an increase in the water intake was observed in males on days7 and 14 and in females on day 14 at 300 mg/kg bw/day. In the administration period, a decrease in the water intake was observed in males on day 28 and an increase in females on days 7 and 14 at 12 mg/kg bw/day (not considered toxicological effects). HAEMATOLOGY In the administration period, changes were found in males at 300 mg/kg bw/day and in females at 60 mg/kg bw/day or more. In the recovery period, changes were found in males and females at 300 mg/kg bw/day. (See attached tables.) CLINICAL CHEMISTRY In the administration period, changes were found in males at 60 mg/kg bw/day or more and in females at 300 mg/kg bw/day. In the recovery period, changes were found in males and females at 300 mg/kg bw/day. (See attached tables.) In male, α1-globulin fraction was significantly decreased also at 12 mg/kg bw/day. This change observed was not considered toxicological effect, because no related effects were observed at 12 mg/kg bw/day. URINALYSIS In the administration period, the followings were observed at 300 mg/kg bw/day: An increase in the urine volume and decreases in the urine protein and specific gravity in males and females, and a decrease in pH in males. In the recovery period, a decrease in the specific gravity was observed in females at 300 mg/kg bw/day. ORGAN WEIGHTS In the administration period, increases in relative weights of the liver, kidney and spleen were observed in males and females at 300 mg/kg bw/day. At 300 mg/kg bw/day, the changes in the heart, brain, pituitary gland, testis, prostate and seminal vesicle were not considered toxicological effects because of a decrease in the body weight at necropsy of the administration period. In the recovery period, increases in relative weight of the spleen in males and females and of the liver in females were observed at 300 mg/kg bw/day. At 300 mg/kg bw/day, the changes in the liver, brain, pituitary gland, testis and prostate were not considered toxicological effects because of a decrease in the body weight at necropsy of the recovery period (See attached tables.) GROSS PATHOLOGY In the administration period, gross pathological findings were observed in males at 60 mg/kg bw/day or more and in females at 300 mg/kg bw/day. In the recovery period, gross pathological findings were observed in males and females at 300 mg/kg bw/day. (See attached tables.) HISTOPATHOLOGY: NON-NEOPLASTIC At the histopathological examination, squamous cell hyperplasia and hyperkeratosis of the forestomach were observed in both sexes, and moderate or severe eosinophilic body of proximal tubular epithelium in the kidney were observed in males at 60 mg/kg bw/day and more. At 300 mg/kg bw/day, there were centrilobular hepatocyte hypertrophy in the liver, edema and necrosis of the kidney, and congestion and moderate hemosiderin deposit in the spleen in both sexes. Please refer to attached tables for the other findings. The effects were recovered during the 2 week recovery phase.

Overall remarks, attachments

Attached background material

| Attached document | Remarks |
| --- | --- |
| 95783_chemistry.pdf / 317.63 KB (application/pdf) |  |
| 95783_gross.pdf / 14.1 KB (application/pdf) |  |
| 95783_haemato.pdf / 91.16 KB (application/pdf) |  |
| 95783_histo.pdf / 19 KB (application/pdf) |  |
| 95783_organweight.pdf / 75.43 KB (application/pdf) |  |

Applicant's summary and conclusion

Conclusions

In a 28-day repeated dose oral toxicity study in rats, at 300 mg/kg bw/day, staggering gait and lacrimation were observed in both sexes. Body weight and food consumption were significantly low while water intake was significantly increased (both the administration and recovery periods) in both sexes. In the urinary examination, there were a significant increase in the urine volume, and significant decreases in the specific gravity and protein level in both sexes and pH in males at 300 mg/kg bw/day. There were significant decreases in the RBC, haematocrit and haemoglobin and increases in methemoglobin and reticulocyte count in both sexes. In males, significantly decreased MCHC and lymphocyte ratio, and increased segmented leukocyte ratio were observed at 300 mg/kg bw/day. At 60 mg/kg bw/day and more, significantly longer PT was observed in females. In clinical chemistry, there were significant decreases in α1-globulin fraction and glucose, and there were significant increases in α2-globulin fraction, β-globulin fraction, T-bill, TG, BUN and IP in males at 60 or 300 mg/kg bw/day. There were significant decreases in α1-globulin fraction, and there were significant increases in α2-globulin fraction, ALT, T-bill, T-cho and UN in females at 300 mg/kg bw/day. At necropcy, thickening of focal mucosa in the forestomach was observed in males at 60 mg/kg bw/day and more and in females at 300 mg/kg bw/day. In the kidney, yellowish white discoloration of bilateral papilla was observed in both sexes at 300 mg/kg bw/day. Relative weights of the liver, kidney and spleen were significantly increased at 300 mg/kg bw/day in both sexes. At the histopathological examination, squamous cell hyperplasia and hyperkeratosis of the forestomach were observed in both sexes, and moderate or severe eosinophilic body of proximal tubular epithelium in the kidney were observed in males at 60 mg/kg bw/day and more. At 300 mg/kg bw/day, there were centrilobular hepatocyte hypertrophy in the liver, edema and necrosis of the kidney, and congestion and moderate hemosiderin deposit in the spleen in both sexes. The NOAEL of this study was considered to be 12 mg/kg bw/day in both sexes based on haematological and biochemical effects and effects on the kidney and forestomach.

## Endpoint study record: Russfield AB (1973)

| UUID |  | IUC5-714151f8-f7f8-48a3-8a49-452878a8aa5c |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | mariko / National Institute of Health Sciences / Tokyo / Japan |
| Date |  | 2013-02-06 14:22:43 JST |
| Remarks |  |  |

Administrative Data

| Study result type | experimental result |  |  |
| --- | --- | --- | --- |
| Reliability | 4 (not assignable) |  |  |
| Rationale for reliability incl. deficiencies | Abstract only. |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Russfield AB, Boger F, Homburder F, Weisburger EK & Weisburger JH. | 1973 | Effect of structure of seven methyl anilines on toxicity and on incidnece of subcutaneous and liver tumors in Charles River Rats. | Ed. Proc 32: 833; Abstract No. 3467. |  |  |  |  |  |

Data access

data published

Materials and methods

Test type

chronic

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | other guideline: No data | no data |

Principles of method if other than guideline

The test substance was administered to groups of 50 male Sprague Dawley rats in a two-year feeding study.

GLP compliance

no (Study pre-dates GLP.)

Test materials

Test material identity

| Identifier | Identity |
| --- | --- |
| EC name | 2,5-xylidine |
| CAS number | 95-78-3 |
| EC number | 202-451-0 |

Details on test material

- Name of test material: 2,5-xylidine

Test animals

Species

rat

Strain

Sprague-Dawley

Sex

male

Details on test animals and environmental conditions

TEST ANIMALS - Source: Charles River.

Administration / exposure

Route of administration

oral: feed

Vehicle

no data

Details on oral exposure

No data

Analytical verification of doses or concentrations

no data

Details on analytical verification of doses or concentrations

No data

Duration of treatment / exposure

Two tears

Frequency of treatment

No data

Doses/concentrations

| No data |  |
| --- | --- |
| Basis | no data |

No. of animals per sex per dose

50 males

Control animals

yes

Details on study design

No data

Positive control

No data

Examinations

Observations and examinations performed and frequency

No data

Sacrifice and pathology

No data

Other examinations

No data

Statistics

No data

Results and discussions

Results of examinations

Clinical signs and mortality

no data

Body weight and weight gain

no data

Food consumption and compound intake (if feeding study)

no data

Food efficiency

no data

Water consumption and compound intake (if drinking water study)

no data

Ophthalmoscopic examination

no data

Haematology

no data

Clinical chemistry

no data

Urinalysis

no data

Neurobehaviour

no data

Organ weights

no data

Gross pathology

no data

Histopathology: non-neoplastic

no data

Histopathology: neoplastic

no data

Details on results

Subcutaneous fibromas of fibrosarcomas occurred in 16 % of controls and in 24 % of rats fed the test substance. Two cholangiocarninomas were found in the 111 controls. Hepatomas also developed.

Applicant's summary and conclusion

Conclusions

The test substance was administered to groups of 50 male Sprague Dawley rats in a two-year feeding study. Subcutaneous fibromas of fibrosarcomas occurred in 16 % of controls and in 24 % of rats fed the test substance. Two cholangiocarninomas were found in the 111 controls. Hepatomas also developed.

## Endpoint study record: Magnusson G et al (1971)

| UUID |  | IUC5-4e6042e8-2a55-408b-bf10-b9268785dd5b |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | mariko / National Institute of Health Sciences / Tokyo / Japan |
| Date |  | 2013-02-06 14:22:44 JST |
| Remarks |  |  |

Administrative Data

| Reliability | 2 (reliable with restrictions) |
| --- | --- |
| Rationale for reliability incl. deficiencies | Meets generally accepted scientific standards with acceptable restrictions. |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Magnusson G, Bodin N-O & Hansson E. | 1971 | Hepatic changes in dogs and rats induced by xylidine isomers. | Acta. Path. Microbiol. Scand. Section A 79, 639 - 648. |  |  |  |  |  |

Data access

data published

Materials and methods

Test type

subacute

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | other guideline: No data | no data |

Principles of method if other than guideline

Male and female beagle dogs received daily oral doses of 2, 10 and 50 mg/kg and male and female Sprague Dawley rats received 20, 100 and 500-700 mg/kg daily for 4 weeks. Dogs and rats were sacrificed at the end of the experiment and subjected to complete autopsy.

GLP compliance

no (Study pre-dates GLP.)

Test materials

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 95-78-3 |
| EC name | 2,5-xylidine |

Details on test material

- Name of test material: 2,5-xylidine

Test animals

Species

other: Dogs and rats

Strain

other: Beagle dogs and Sprague-Dawley rats

Sex

male/female

Details on test animals and environmental conditions

TEST ANIMALS - Age at study initiation: Dogs: 10 - 12 months; rats: 2 months - Weight at study initiation: Dogs: 8.3 - 14.0 kg; male rats: 272 - 310 g, female rats 210 - 234 g. - Diet: Ad libitum - Water: Ad libitum

Administration / exposure

Route of administration

other: Dogs: capsule; rats: stomach tube

Vehicle

no data

Analytical verification of doses or concentrations

no data

Duration of treatment / exposure

4 weeks

Frequency of treatment

Once daily

Doses/concentrations

| Dogs: 0, 2, 10 or 50 mg/kg |  |
| --- | --- |
| Basis | actual ingested |

| Rats: 0, 20, 100 or 500 mg/kg. After 2 weeks of treatment the high dose group was increased to 700 mg/kg. |  |
| --- | --- |
| Basis | actual ingested |

No. of animals per sex per dose

Dogs: 1/sex/dose Rats: 5/sex/group

Control animals

yes

Examinations

Observations and examinations performed and frequency

CAGE SIDE OBSERVATIONS: Yes - Time schedule: Dogs: Daily DETAILED CLINICAL OBSERVATIONS: Yes - Time schedule: Dogs: Daily BODY WEIGHT: Yes FOOD CONSUMPTION AND COMPOUND INTAKE: - Food consumption for each animal determined and mean daily diet consumption calculated as g food/kg body weight/day: No - Compound intake calculated as time-weighted averages from the consumption and body weight gain data: No FOOD EFFICIENCY: - Body weight gain in kg/food consumption in kg per unit time X 100 calculated as time-weighted averages from the consumption and body weight gain data: No WATER CONSUMPTION AND COMPOUND INTAKE: No OPHTHALMOSCOPIC EXAMINATION: No HAEMATOLOGY: Yes - Time schedule for collection of blood: Once a week - Anaesthetic used for blood collection: No data - Animals fasted: No data - How many animals: No data - Parameters: Dogs: Haematocrit, haemoglobin concentration, red and white blood cell counts, differential leukocyte count. Rats: Haematocrit, haemoglobin concentration. CLINICAL CHEMISTRY: Yes - Time schedule for collection of blood: Once a week - Animals fasted: No data - How many animals: No data - Parameters: Dogs: Serum concentrations of glucose, bilirubin, urea-nitrogen, total bilirubin. Rats: Serum concentrations of urea-nitrogen and ornithine carbamyltransferase. URINALYSIS: No NEUROBEHAVIOURAL EXAMINATION: No

Sacrifice and pathology

GROSS PATHOLOGY: Yes all animals were subjected to a full autopsy. HISTOPATHOLOGY: Yes; the liver and kidneys were examined microscopically.

Other examinations

Dogs: The rate of clearance of sulphobromophthalein was determined by measuring the dye retention 30 min after administration of 5 mg/kg bw.

Results and discussions

Results of examinations

Details on results

CLINICAL SIGNS AND MORTALITY Dogs: At the 10 and 50 mg/kg doses there was vomiting which occurred from 0.5 h to 4 h after treatment. The vomiting was most frequent at the highest dose levels. The dogs receiving the highest dose displayed general poor condition. Rats: One rat died after treatment at the high dose level (Day 25). BODY WEIGHT AND WEIGHT GAIN Dogs: The dogs receiving the highest dose showed a decreased body weight. Rats: Decreased body weight was observed at the high dose level in male rats. HAEMATOLOGY Dogs: Results were within normal ranges. Rats: At the high dose the harmoglobin concentration was decreased. The haematocrit also showed a decrease though less pronounced than the haemoglobin. CLINICAL CHEMISTRY Dogs: Hyperbilirubinaemia and hypoproteiaemia were observed in both dogs receiving 50 mg/kg. Other clinical chemistry results were within normal ranges. Rats: An increased value for OCT was observed in five rats. At the low dose level one rat had increased OCT values. All values for urea-nitrogen were within normal ranges. GROSS PATHOLOGY Dogs: The livers were slightly enlarged at the high dose level. The dogs treated with 50 mg/kg had fatty livers. The livers were yellow in colour had a soft consistency and a greasy cut surface. The bile was dark and thick. The tissues of the dogs were icteric. Dogs treated with 10 mg/kg have pale livers. The kidneys had a normal appearance in all dogs. Rats: The livers of the treated rats were enlarged at the high dose level. HISTOPATHOLOGY: Dogs: In sections with slight to moderate fatty change, small fat deposits were distributed in the peripherolobular cells. In severe lesions a mixture of large and small fat droplets were diffusely distributed in the lobules. In embedded sections of severely damaged livers the cells appeared empty due to the great number of fat droplets. Some isolated liver cells were necrotic or exhibited necrobiotic nuclear changes. Bile thrombi and bile pigments were also observed when the hepatic lesions were severe. Fatty degeneration of the liver was observed at the medium and high dose levels. The fatty degeneration was considered slight at the high dose level for the female dog, as moderate at the medium dose level with the male dog and as severe at the high dose level with regard to the male dog. The kidneys had a normal appearance in all dogs. Rats: Fine fat droplets were observed in the perilobular areas of the liver of 1 female rat. A hyperkeratosis of the forestomach was demonstrated at the high dose level. OTHER FINDINGS Dogs: The rate of clearance of sulphobromophthalein in dogs was increased at the highest dose levels and in the female receiving 10 mg/kg.

Applicant's summary and conclusion

Conclusions

Rat One rat died after treatment at the high dose level (day 25). Decreased body weight was observed at 500-700 mg/kg bw/day in male rats. An increased value for ornithine carbamoyltransferase was observed in five rats. At the low dose level one rat had increased ornithine carbamoyltransferase values. At 500-700 mg/kg bw/day, the hemoglobin concentration was decreased. The livers of the treated rats were enlarged at 500-700 mg/kg bw/day. Fine fat droplets were observed in the perilobular areas of the liver of one female rat. A hyperkeratosis of the forestomach was demonstrated at the high dose level. The NOAEL in rats was set to 100 mg/kg bw/day based on effects on the blood and liver at the highest dose. Dog Dogs of the 10 and the 50 mg/kg dose groups vomited which occurred from 0.5 h to 4 h after treatment. The vomiting was most frequent at the highest dose levels. The dogs receiving the highest dose displayed general poor condition and a decreased body weight. At the high dose the hemoglobin concentration was decreased. Hyperbilirubinaemia and hypoproteiaemia were observed in both dogs receiving 50 mg/kg. The livers of dogs at the high dose level were slightly enlarged. The dogs treated with 10 and 50 mg/kg had fatty livers and with 10 mg/kg bw/day had pale livers. The kidneys had a normal appearance in all dogs. The NOAEL in dogs was set to 2 mg/kg bw/day based on the clinical signs (vomiting) and effects on the liver.

# 7.6 Genetic toxicity

# 7.6.1 Genetic toxicity in vitro

## Endpoint study record: Florin I et al (1980)

| UUID |  | IUC5-a4fa5e3b-03aa-42ba-a580-d1b680b539a3 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | mariko / National Institute of Health Sciences / Tokyo / Japan |
| Date |  | 2013-02-06 14:22:44 JST |
| Remarks |  |  |

Administrative Data

| Reliability | 4 (not assignable) |
| --- | --- |
| Rationale for reliability incl. deficiencies | Insufficient detail in the report published in the literature. |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Florin I, Rutberg L, Curvall M & Enzell CR. | 1980 | Screening of tobacco smoke using constituents for mutagenicity using the ames' test. | Toxicology, 15, 219-232. |  |  |  |  |  |

Data access

data published

Materials and methods

Type of genotoxicity

gene mutation

Type of study

bacterial reverse mutation assay (e.g. Ames test)

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | other guideline: No data | no data |

Principles of method if other than guideline

To clarify the mutagenic activity of individual smoke components, 239 compounds representative of gaseous and semi-volatile phases of tobacco smoke were assayed for mutagenicity towards 4 histidine-requiring mutants of Salmonella typhimurium. All compounds were tested qualitatively both with and without metabolic activation using a liver fraction (S9) from Aroclor 1254 or methylcholanthrene induced rats.

GLP compliance

no (Study pre-dates GLP)

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 95-78-3 |
| EC number | 202-451-0 |
| EC name | 2,5-xylidine |

Details on test material

IUCLID4 Test substance: as prescribed by 1.1 - 1.4

Method

Target gene

Histidine

Species/strain

| Species/strain | S. typhimurium TA 100 |
| --- | --- |
| Additional strain characteristics | other: The presence of the rfa-mutation was checked by crystal violet inhibition. The presence of the plasmid pKM101 in strains TA 98 and TA 100 was checked by resistence to ampicillin. |
| Metabolic activation | with and without |
| Metabolic activation system | Aroclor 1254 or methylcholanthrene induced rats. |

Test concentrations

Spot test: 3 µmol/plate Quantitative test: 0.03, 0.3, 3, 30 µmol/plate)

Controls

| Positive controls | yes |
| --- | --- |
| Positive control substance | N-ethyl-N-nitro-N-nitrosoguanidine. |
| Remarks | Without metabolic activation |

| Positive controls | yes |
| --- | --- |
| Positive control substance | other: 2-aminoanthracene. |
| Remarks | With metabolic activation |

Details on test system and conditions

METHOD OF APPLICATION: In medium. Initially cultures were grown in Difco nutrient broth. Since this medium is suspected to have a weak mutagenic effect, it was substituted for Oxoid nutrient broth No. 2 in later experiments. Revertants were scored on glucosenminimal salts medium supplemented with 0.05 µm histidine and 0.05 µm biotin. Plates used for viable counts also contained histidine and biotin. DURATION - Pre-incubation period: No data - Exposure duration: No data - Expression time (cells in growth medium): No data - Selection time (if incubation with a selection agent): No data - Fixation time (start of exposure up to fixation or harvest of cells): No data SELECTION AGENT: Histidine NUMBER OF REPLICATIONS: No data NUMBER OF CELLS EVALUATED: No data OTHER EXAMINATIONS: The chemical was tested in spot tests using all four strains, with and without S9. A quantitative test was then performed with strains TA1535 and TA1537. The number of spontaneous revertants was measured.

Results and discussions

Test results

| Species/strain | S. typhimurium TA 100 |
| --- | --- |
| Metabolic activation | with and without |
| Test system | all strains/cell types tested (Salmonella typhimurium strains TA98, TA100, TA1535 and TA1537) |
| Genotoxicity | negative |

Additional information on results

The test substance was tested quantitatively with TA100 and was toxic at all tested concentrations.

Applicant's summary and conclusion

Interpretation of results

negative

Conclusions

The test substance was assessed for mutagenicity in four S. typhimurium strains with and without metabolic activation. The test substance was negative for mutagenicity but was toxic at all tested concentrations.

## Endpoint study record: Nohmi T et al (1984)

| UUID |  | IUC5-2010e4f6-7b84-4623-9b4c-0ba97d979228 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | mariko / National Institute of Health Sciences / Tokyo / Japan |
| Date |  | 2013-02-06 14:22:44 JST |
| Remarks |  |  |

Administrative Data

| Reliability | 4 (not assignable) |
| --- | --- |
| Rationale for reliability incl. deficiencies | Insufficient detail in the report published in the literature. |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Nohmi T, Yoshikawa K, Nakadate M, Miyata R & Ishidate M. | 1984 | Mutations in Salmonella Typhimurium and Inactivation of Bacillus Subtilis Transforming DNA Induced by Phenylhydroxylamine Derivatives. | Mutat. Res., 136, 159-168. |  |  |  |  |  |

Data access

data published

Materials and methods

Type of genotoxicity

DNA damage and/or repair

Type of study

Bacillus subtilis recombination assay

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | other guideline: No data | no data |

Principles of method if other than guideline

A bacterial reverse mutation test was performed using Salmonella typhimurium strains in the presence and absence of metabolic inhibition.

GLP compliance

no (Study pre-dates GLP)

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 95-78-0 |
| EC number | 202-451-0 |
| EC name | 2,5-xylidine |

Details on test material

- Name of test material: 2,5-dimehtylaniline

Method

Target gene

Histidine

Species/strain

| Species/strain | S. typhimurium, other: TA98, TA100 |
| --- | --- |
| Additional strain characteristics | no data |
| Metabolic activation | with and without |
| Metabolic activation system | S9 from rat liver induced wiht Kanechlor KC-400 (polychlorinated biphenyl) ip, five days before they were killed. |

Test concentrations

No data

Vehicle

0.1 mL DMSO

Details on test system and conditions

Salmonella/microsome test: The test substance was added to an overnight culture of S. typhimurium and mixed with 1/15 M potassium phosphate buffer, pH 7.4 or S9 mix. After 20 min incubation at 37 °C, the reaction mixture was mixed with top agar and spread on minimal agar plates containing L-histidine and D-biotin. Plates were incubated for 2 days at 37 °C and the number of colonies counted. Induced mutation frequency: A suspension of S. typhimurium TA100 (about 5 x 10^8 cells/mL) were prepared from over night culture and mixed with various concentrations of the PHA derivatives. After incubation for 30 min at 37 °C with shaking, 0.1 mL of the sample was diluted 10^5-fold with phosphate buffer and then poured onto the semi-enriched gar plate for assaying surviving cells. The remaining sample was centrifuged and collected cells were mixed with top agar and then poured onto minimal agar plates. After 2 days at 37 °C, the number of revertants (M) and the surviving cells (N) were counted. The induced mutation frequency was calculated as the value of (M/10^N) minus (M0/10^6N), where M0 and M stand for the values in the untreated and chemical-treated cell culture respectively. Assay for loss of transforming DNA activity: Transforming DNA was prepared from vegetative cells of B. subtilis Marburg wild type. Competent cells of B. subtilis tryptophan-requiring mutant were prepared. The reaction mix consisting of the test substance (5 or 10 mM) in DMSO and transforming DNA were added to 0.1 M tris-HCL buffer pH 7.2 containing 0.1 M sodium chloride and 0.8 mM magnesium sulfate was incubated for 60 min at 37 °C. After incubation , the mixture was diluted 5-fold with tris-HCL buffer and 100 µL of the diluted solution which contained transforming DNA was mixed with the competent cell suspension. After incubation for 30 min at 37 °C the cells were mixed with soft agar and poured onto minimal agar. As a control, the test DNA was treated with DMSO prior to dilution and subsequent addition of the competent cells. The number of transformants was counted after incubation for 2 days at 37 °C and the percentage of the remaining activity of the transforming DNA was determined in each experiment by comparing the number of transformants produced by cells receiving control DNA and the number receiving treated DNA.

Statistics

No data

Results and discussions

Test results

| Species/strain | S. typhimurium TA 98 |
| --- | --- |
| Metabolic activation | with and without |
| Genotoxicity | negative |

| Species/strain | S. typhimurium TA 100 |
| --- | --- |
| Metabolic activation | without |
| Genotoxicity | negative |

| Species/strain | S. typhimurium TA 100 |
| --- | --- |
| Metabolic activation | with |
| Genotoxicity | positive (3.9 x 10 induced His+/µM) |

Additional information on results

Aniline derivatives were mutagenic in TA 100 only when S9 mix was present and they induced less than 10^3 revertants/µmole.

Any other information on results incl. tables

Mutagenicities*. TA100; TA98. Compound; -S9; +S9; -S9; +S9. 2,3-dimethylaniline; -; 4.4 x 10; —; -. 2,4-dimethylaniline; -; 1.2 x 10^2; -; -. 2,5-dimethylaniline; -; 4.2 x 10^2; -; -. 2,6-dimethylaniline; -; 2.4 x 10^2; -; -. 3,4-dimethylaniline; -; 3.9 x 10; -; -. 3,5-dimethylaniline; -; 9.3 x 10; -; -. * Mutagenicities were expressed as numbers of induced His+/umole. - Negative response

Applicant's summary and conclusion

Interpretation of results

positive with metabolic activation

Conclusions

Aniline derivatives were mutagenic in TA 100 only when S9 mix was present and they induced less than 10^3 revertants/µmole.

## Endpoint study record: Hartman CP et al (1979)

| UUID |  | IUC5-5b4f00c8-f411-4799-aed1-8bb7b89e5a0c |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | mariko / National Institute of Health Sciences / Tokyo / Japan |
| Date |  | 2013-02-06 14:22:44 JST |
| Remarks |  |  |

Administrative Data

| Reliability | 4 (not assignable) |
| --- | --- |
| Rationale for reliability incl. deficiencies | Only tested in one strain at one dose. |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Hartman CP, Andrews AW & Chung K-T. | 1979 | Production of a Mutagen from Ponceau 3R by a Human Intestinal Anaerobe. | Infect. Immun., 23, 686-689. |  |  |  |  |  |

Data access

data published

Materials and methods

Type of genotoxicity

gene mutation

Type of study

bacterial reverse mutation assay (e.g. Ames test)

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | other guideline: No data | no data |

Principles of method if other than guideline

The Ames et al (1973) Salmonella mutagenicity test was used.

GLP compliance

no (Study pre-dates GLP.)

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 95-78-3 |
| EC number | 202-451-0 |
| EC name | 2,5-xylidine |

Details on test material

- Name of test material: 2,5-dimethylaniline

Method

Target gene

No data

Species/strain

| Species/strain | S. typhimurium TA 100 |
| --- | --- |
| Metabolic activation | with and without |
| Metabolic activation system | Rat liver S9 preparations |

Test concentrations

0.83 µmoles

Controls

| Negative controls | yes (Cells only) |
| --- | --- |
| Solvent / vehicle controls | yes (DMSO only) |
| True negative controls | no data |
| Positive controls | no data |
| Positive control substance | no data. |

Details on test system and conditions

IUCLID4 Type: Ames test

Evaluation criteria

No data

Statistics

No data

Results and discussions

Test results

| Species/strain | S. typhimurium TA 100 |
| --- | --- |
| Metabolic activation | with and without |
| Test system | all strains/cell types tested |
| Genotoxicity | negative |
| Cytotoxicity | no data |
| Vehicle controls valid | no data |
| Negative controls valid | no data |
| Positive controls valid | no data |

Additional information on results

No dimethylanilines showed any mutagenesis in dose-response curves from 1 to 1000 µg.

Any other information on results incl. tables

Mutagenicity of 2,5-dimethylaniline: Test conditions; TA 100 –S9; TA 100 + S9. Cells only; 64; 69. Dimethylsulfoxide (DMSO); 75; 67. 2,5-dimethylaniline (0.83 µmoles); 85; 90.

Applicant's summary and conclusion

Interpretation of results

negative

Conclusions

The test substance was assessed in a bacterial reverse mutation assay and was shown to be negative for mutagenicity with and without metabolic activation.

## Endpoint study record: Zimmer D et al (1980)

| UUID |  | IUC5-5923fa63-9ec8-4e1f-9bc9-138957724e11 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | mariko / National Institute of Health Sciences / Tokyo / Japan |
| Date |  | 2013-02-06 14:22:44 JST |
| Remarks |  |  |

Administrative Data

| Reliability | 4 (not assignable) |
| --- | --- |
| Rationale for reliability incl. deficiencies | Insufficient detail in the report published in the literature. |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Zimmer D, Mazurek J, Petzold G & Bhuyan BK. | 1980 | Bacterial Mutagenicity and Mammalian Cell DNA Damage by Several Sustituted Anilines. | Mutat. Res., 77, 317-326. |  |  |  |  |  |

Data access

data published

Materials and methods

Type of genotoxicity

gene mutation

Type of study

bacterial reverse mutation assay (e.g. Ames test)

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | other guideline: No data | no data |

Principles of method if other than guideline

The test substance was assessed for mutagenicity using a modified Ames test.

GLP compliance

no (Study pre-dates GLP.)

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 95-78-3 |
| EC number | 202-451-0 |
| EC name | 2,5-xylidine |

Details on test material

IUCLID4 Test substance: as prescribed by 1.1 - 1.4 - Analytical purity: 97 - 99 %

Method

Target gene

No data

Species/strain

| Species/strain | S. typhimurium TA 100 |
| --- | --- |
| Metabolic activation | with and without |
| Metabolic activation system | Liver S9 fraction from Upjohn strain of Sprague-Dawley rats induced with Aroclor 1254. |

Test concentrations

No data

Controls

| Negative controls | no data |
| --- | --- |
| Solvent / vehicle controls | no data |
| True negative controls | no data |
| Positive controls | no data |
| Positive control substance | no data. |

Details on test system and conditions

IUCLID4 Type: Ames test

Evaluation criteria

The revertant count for each point was the average of duplicate plates scored on a modified Artek Model Colony Counter. A compound was considered mutagenic if it increased the number of revertants/plate over the control background level and showed a dose-response.

Results and discussions

Test results

| Species/strain | S. typhimurium TA 100 |
| --- | --- |
| Metabolic activation | with and without |
| Test system | all strains/cell types tested |
| Genotoxicity | positive (58 revertants/umole/plate) |
| Cytotoxicity | no data |
| Vehicle controls valid | no data |
| Negative controls valid | no data |
| Positive controls valid | no data |

Additional information on results

The test substance was weakly positive in the test strains (58 revertants/umole/plate).

Applicant's summary and conclusion

Interpretation of results

positive

Conclusions

The test substance was assessed for mutagenicity using a modified Ames method with Salmonella typhimurium TA100. The results were weakly positive for mutagenicity.

## Endpoint study record: Zeiger et al (1988)

| UUID |  | IUC5-bac08b85-9ba3-48da-97d5-c9aaf8542464 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | mariko / National Institute of Health Sciences / Tokyo / Japan |
| Date |  | 2013-02-06 14:22:44 JST |
| Remarks |  |  |

Administrative Data

| Study result type | experimental result |  |  |
| --- | --- | --- | --- |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Study published in peer-reviewed journal. |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Zeiger E, Anderson B, Haworth S, Lawlor T, Mortelmans K. | 1988 | Salmonella mutagenicity tests: IV. Results from the testing of 300 chemicals. | Environ Mol Mutagen. 1988;11 Suppl 12:1-157. |  |  |  |  |  |

Data access

data published

Materials and methods

Type of genotoxicity

gene mutation

Type of study

bacterial reverse mutation assay (e.g. Ames test)

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Details on test material

Purity: ~98%

Method

Species/strain

| Species/strain | other: S. typhimurium TA 1535, TA 98, TA97 and TA 100 |
| --- | --- |
| Metabolic activation | with and without |
| Metabolic activation system | S9 (male SD rats and male Syrian hamster livers) |

| Species/strain | S. typhimurium TA 1537 |
| --- | --- |
| Metabolic activation | with |
| Metabolic activation system | S9 (male SD rats and male Syrian hamster livers) |

Test concentrations

0, 33, 100, 333, 1000, or 1666 μg/plate

Vehicle

DMSO

Controls

| Solvent / vehicle controls | yes |
| --- | --- |
| Positive controls | yes |

Details on test system and conditions

The preincubation assay was performed as described previously [Haworth et al., 1983], with some differences, as described below. The test chemical (0.05 ml), Salmonella culture (0.10 ml), and S-9 mix or buffer (0.50 ml) were incubated at 37°C, without shaking, for 20 min. Chemicals known or suspected to be volatile were incubated in capped tubes. The top agar was added and the contents of the tubes were mixed and poured onto the surface of petri dishes containing Vogel-Bonner medium [Vogel and Bonner, 1956]. The histidine-independent (his+) colonies arising on these plates were counted following two days incubation at 37°C. Plates were machine counted (New Brunswick, Edison, NJ; Artek, Farmingdale, NY) unless precipitate was present which interfered with the count, or the color of the test chemical on the plate reduced the contrast between the colonies and the background agar. At the discretion of the investigators, plates with low numbers of colonies were counted by hand. Variations in the protocol among the tested chemicals reflect the evolution of the protocol originally described by Haworth et al. [1983]. Testing in strains TA97, TA98, TA100, and TA1535, with some additional testing in strain TA1537; 10% S-9 was used. The first test of a chemical was without activation and with 10% S-9 in the S-9 mix. If a positive result was obtained the test was repeated. Vogel HJ, Bonner DM (1956): Acetylornithinase of E. coli: Partial purification and some properties. J BioI Chern 218:97-106.

Evaluation criteria

A chemical was judged mutagenic (+) or weakly mutagenic (+W) if it produced a reproducible dose-related reponse over the solvent control in replicate trials. A chemical was Judged questionable (?) if the results of individual trials were not ,reproducible, if increases in his + revertants did not meet the criteria for a "+W"response, or if only single doses produced increases in his+ revertants in repeat trials. Chemicals were judged nonmutagenic (-) if they did not meet the criteria for a mutagenic or questionable response.

Results and discussions

Test results

| Species/strain | other: S. typhimurium TA 1537, TA 98, TA97 and TA 100 |
| --- | --- |
| Genotoxicity | positive (with S9) |
| Cytotoxicity | yes |
| Positive controls valid | yes |

Additional information on results

Slight cytotoxicity was observed in TA 100, TA1535, TA 97 and TA98 at 1666 μg/plate.

Overall remarks, attachments

Attached background material

| Attached document | Remarks |
| --- | --- |
| Ames Table.pdf / 277.3 KB (application/pdf) | Environ Mol Mutagen. 1988;11 Suppl 12:1-157. |

Illustration (picture/graph)

Applicant's summary and conclusion

Interpretation of results

positive with metabolic activation

Conclusions

Positive results were obtained in Salmonella strains TA100, TA1537, TA97, and TA98 when incubated with rat or hamster S9 mixture.

## Endpoint study record: RA from 2,3-, 2,4, 2,6-, 3,4- and 3,5-dimethylaniline (chromosome aberration)

| UUID |  | IUC5-0eea2632-a654-45bc-b841-717550d34279 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | mariko / National Institute of Health Sciences / Tokyo / Japan |
| Date |  | 2013-02-06 14:22:45 JST |
| Remarks |  |  |

Administrative Data

| Study result type | read-across based on grouping of substances (category approach) |  |  |
| --- | --- | --- | --- |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MHLW, Japan |  |  | http://dra4.nihs.go.jp/mhlw_data/jsp/SearchPageENG.jsp |  |  |  |  |  |

Materials and methods

Type of genotoxicity

chromosome aberration

Test materials

Details on test material

Read across form 2,3-, 2,4-, 2,6-, 3,4- and 3,5-dimethylaniline

# 7.6.2 Genetic toxicity in vivo

## Endpoint study record: Micronucleus/peripheral blood (Hayashi et al., 2000)

| UUID |  | IUC5-82bcd255-772b-4caf-bcc1-07ca2c99e64f |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | mariko / National Institute of Health Sciences / Tokyo / Japan |
| Date |  | 2013-02-06 14:22:45 JST |
| Remarks |  |  |

Administrative Data

| Study result type | experimental result |  |  |
| --- | --- | --- | --- |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Meets generally accepted scientific standards |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | Hayashi, M., Suzuki, T., Ohara, A. | 2000 | Establishment of methods for gene mutation analysis using transgenic mice. In: Research report on studies on establishment of new methods for evaluation of carcinogenicity studies using animals and its implication. | (Grant-in-Aid for Cancer Research from the Ministry of Health, Labour and Welfare, Japan), in Japanese. (unpublished) |  |  |  |  |  |

Materials and methods

Type of genotoxicity

chromosome aberration

Type of study

micronucleus assay

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Details on test material

Supplier: Wako Pure Chemical Industries, Ltd.

Test animals

Species

mouse

Strain

other: Muta

Sex

male

Details on test animals and environmental conditions

- Source: Covance Research Products - Age at study initiation: 7-8 weeks old - Acclimation period: 1-2 weeks same animals to gene mutation (Hayashi et al., 2000)

Administration / exposure

Route of administration

oral: gavage

Vehicle(s)

olive oil

Frequency of treatment

once

Post exposure period

48 h

Doses / concentrations

| 100 mg/kg bw |  |
| --- | --- |
| Basis | actual ingested |

No. of animals per sex per dose

5 animals/group

Control animals

yes, concurrent vehicle

Positive control(s)

benzo(a)pyrene

Examinations

Tissues and cell types examined

Polychromatic erythrocytes from the peripheral blood

Details of tissue and slide preparation

Cells for specimen were collected 48 h after the administration.

Results and discussions

Test results

| Sex | male |
| --- | --- |
| Genotoxicity | negative |
| Vehicle controls valid | yes |

Any other information on results incl. tables

Table 1.Micronucleus Induction in Peripheral Blood (Male) Chemical; Dose; MNRET*s/1000RETs; Mean±SD. Control; 0 mg/kg; 1; 2; 1; 2; 3; 1.6±0.9. 2,5-DMA; 100 mg/kg; 1; 2; 1; 0; 1; 1.0±0.7. 2,6-DMA; 100 mg/kg; 3; 1; 1; 2; 1; 1.6±0.9. 3,5-DMA; 100 mg/kg; 3; 2; 2; 5; 3; 3.0±1.2. *Micronucleated reticulocytes

Applicant's summary and conclusion

Interpretation of results

negative

Conclusions

Numbers of micronuleated cells were not increased in any treatment.

## Endpoint study record: Comet Assay (Hayashi et al., 2000)

| UUID |  | IUC5-daa90ed1-d63e-4d12-8ea0-6dcf6e2b9cc1 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | mariko / National Institute of Health Sciences / Tokyo / Japan |
| Date |  | 2013-02-06 14:22:45 JST |
| Remarks |  |  |

Administrative Data

| Study result type | experimental result |  |  |
| --- | --- | --- | --- |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Meets generally accepted scientific standards |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | Hayashi, M., Suzuki, T., Ohara, A. | 2000 | Establishment of methods for gene mutation analysis using transgenic mice. In: Research report on studies on establishment of new methods for evaluation of carcinogenicity studies using animals and its implication. | (Grant-in-Aid for Cancer Research from the Ministry of Health, Labour and Welfare, Japan), in Japanese. (unpublished) |  |  |  |  |  |

Materials and methods

Type of genotoxicity

DNA damage and/or repair

Type of study

single cell gel/comet assay in rodents for detection of DNA damage

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Details on test material

Supplier: Wako Pure Chemical Industries, Ltd.

Test animals

Species

mouse

Strain

other: ddY

Sex

male

Details on test animals and environmental conditions

(same animals to the Micronucleus/bone marrow (Hayashi et al.,2000))

Administration / exposure

Route of administration

oral: gavage

Vehicle(s)

olive oil

Details on exposure

Each of dimethylanilines was orally administered at 200 mg/kg bw (10 ml/kg) four times at weakly intervals, and bone marrow, liver, lung and kidneys were isolated 3 hours after the last dosing.

Post exposure period

3 and 24 h

Doses / concentrations

| 200 mg/kg bw/weak |  |
| --- | --- |
| Basis | actual ingested |

No. of animals per sex per dose

Three males/group

Control animals

yes, concurrent vehicle

Positive control(s)

no positive control

Examinations

Tissues and cell types examined

Bone marrow, liver, kidney and lung were taken and examined.

Details of tissue and slide preparation

Nuclei isolated by homogenisation were placed on slides and placed in chilled lysing solution (2.5 M NaCl, 0.1 M EDTA, 0.01 M Tris, 10% DMSO, 1% TritonX-100) and kept at 4 °C in the dark for more than 60 min. Electrophoresis was conducted at less than 10 °C in the dark for 20 min at approximately 300 mA (1 V/cm) (pH=13). After the electrophoresis, slides were neutralized with buffer (400 mM Tris; pH7.5) for three times (10 min each). DNA was stained with EtBr and observed.

Evaluation criteria

The different type of migration was classified in 5 categories as follows (500 cells/group were recorded). Type 1: without tail (no damage) Type 2: with a small tail (light genetic damage) Type 3: with tail with evident migration Type 4: with definite tale with a consistent amount of fragments Type 5: Almost all the DNA is present in the tail (severe genetic damage)

Results and discussions

Test results

| Sex | male |
| --- | --- |
| Genotoxicity | positive |

Overall remarks, attachments

Illustration (picture/graph)

Applicant's summary and conclusion

Conclusions

Positive results were obtained in lung, kidney, and liver at 3 h after the last dosing but not observed at 24 h after the last dosing.

## Endpoint study record: Micronucleus/bone marrow (Hayashi et al., 2000)

| UUID |  | IUC5-c9e21518-b746-4d20-836d-35b1acfe0181 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | mariko / National Institute of Health Sciences / Tokyo / Japan |
| Date |  | 2013-02-06 14:22:45 JST |
| Remarks |  |  |

Administrative Data

| Study result type | experimental result |  |  |
| --- | --- | --- | --- |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Meets generally accepted scientific standards |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | Hayashi, M., Suzuki, T., Ohara, A. | 2000 | Establishment of methods for gene mutation analysis using transgenic mice. In: Research report on studies on establishment of new methods for evaluation of carcinogenicity studies using animals and its implication. | (Grant-in-Aid for Cancer Research from the Ministry of Health, Labour and Welfare, Japan), in Japanese. (unpublished) |  |  |  |  |  |

Materials and methods

Type of genotoxicity

chromosome aberration

Type of study

micronucleus assay

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Details on test material

Supplier: Wako Pure Chemical Industries, Ltd.

Test animals

Species

mouse

Strain

other: ddY

Sex

male

Details on test animals and environmental conditions

same animals to the comet assay (Hayashi et al.,2000)

Administration / exposure

Route of administration

oral: gavage

Vehicle(s)

olive oil

Frequency of treatment

once

Post exposure period

24 h

Doses / concentrations

| 200 mg/kg bw |  |
| --- | --- |
| Basis | actual ingested |

No. of animals per sex per dose

3 animals/group

Control animals

yes, concurrent no treatment

Positive control(s)

no positive control

Examinations

Tissues and cell types examined

Polychromatic erythrocytes from the bone marrow

Details of tissue and slide preparation

Cells for specimen were collected 24 h after the administration.

Results and discussions

Test results

| Sex | male |
| --- | --- |
| Genotoxicity | negative |

Any other information on results incl. tables

Table 1.Micronucleus Induction in Bone Marrow (Male) Chemical; Dose; MNPCE*s/1000 PCEs; Mean±SD; PCE/NCE**. Control; 0 mg/kg; 1; 2; 1; 1.3±0.5; 1.1±0.0. 2,3-DMA; 200 mg/kg; 0; 1; 1; 0.7±0.5; 0.8±0.2. 2,4-DMA; 200 mg/kg; 0; 0; 2; 0.7±0.9; 1.1±0.2. 2,5-DMA; 200 mg/kg; 0; 0; 0; 0.0±0.0; 1.1±0.4. 2,6-DMA; 200 mg/kg; 1; 2; 1; 1.6±0.9; 1.0±0.1. 3,4-DMA; 200 mg/kg; 0; 2; 1; 1.3±0.5; 1.1±0.1. 3,5-DMA; 200 mg/kg; 0; 0; 2; 0.7±0.9; 0.9±0.2. * micronucleated polychromatic erythrocytes ** normochromatic erythrocytes

Applicant's summary and conclusion

Interpretation of results

negative

Conclusions

Numbers of micronuleated cells were not increased in any treatment.

## Endpoint study record: Gene mutation (Hayashi et al., 2000)

| UUID |  | IUC5-c4796351-1703-4481-98f4-22e58229ceed |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | mariko / National Institute of Health Sciences / Tokyo / Japan |
| Date |  | 2013-02-06 14:22:45 JST |
| Remarks |  |  |

Administrative Data

| Study result type | experimental result |  |  |
| --- | --- | --- | --- |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Meets generally accepted scientific standards |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | Hayashi, M., Suzuki, T., Ohara, A. | 2000 | Establishment of methods for gene mutation analysis using transgenic mice. In: Research report on studies on establishment of new methods for evaluation of carcinogenicity studies using animals and its implication. | (Grant-in-Aid for Cancer Research from the Ministry of Health, Labour and Welfare, Japan), in Japanese. (unpublished) |  |  |  |  |  |

Materials and methods

Type of genotoxicity

gene mutation

Type of study

transgenic animal mutagenicity assay

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Details on test material

Supplier: Wako Pure Chemical Industries, Ltd.

Test animals

Species

mouse

Strain

other: Muta

Sex

male

Details on test animals and environmental conditions

Same animals to Micronucleus/peripheral blood (Hayashi et al., 2000)

Administration / exposure

Route of administration

oral: gavage

Vehicle(s)

olive oil

Details on exposure

Each of the dimethylanilies was orally administered at 100 mg/kg bw four times at weakly intervals

Frequency of treatment

once a weak

Doses / concentrations

| 100 mg/kg bw |  |
| --- | --- |
| Basis | actual ingested |

No. of animals per sex per dose

4-5/group

Control animals

yes, concurrent vehicle

Positive control(s)

benzo(a)pyrene

Examinations

Tissues and cell types examined

The nasal tissues, bone marrow, and liver

Details of tissue and slide preparation

The DNA was extracted from the nasal tissues, bone marrow, and liver on day 7 after the last dosing. Lambda pahge was prepared by the in vitro packaging method, and lacZ was infected by E.coli C gale E-, and cII was infected by E.coli G1225(hfl-).

Evaluation criteria

Numbers of total plaques and numbers of mutant plaques were counted, and the mutation frequency (MF=number of mutants/total number of plaques) was calculated. In addition, the cII sequence was analyzed by an ABI 310 Genetic Analyzer.

Results and discussions

Test results

| Sex | male |
| --- | --- |
| Genotoxicity | positive |

Additional information on results

2,5- Dimethylanilines increased mutation frequency of lacZ and cII genes in the nasal tissue more than double. 2,5-Dimethylaniline also increased mutation frequency of lacZ gene in the bone marrow. In the liver, there was no increase in the frequency of lacZ or cII gene mutation in mice 2,5-Dimethylaniline cause transitions from AT to GC (2% [control] vs 18%]) and transversions from GC to TA (0% [control]vs 12% ).

Overall remarks, attachments

Illustration (picture/graph)

Applicant's summary and conclusion

Interpretation of results

positive

Conclusions

2,5- Dimethylanilines increased gene mutation in the nasal tissue and bone marrow.

# 7.7 Carcinogenicity

## Endpoint study record: Weisburger EK et al (1978)

| UUID |  | IUC5-0c4494d8-edce-447c-ab99-1f84c9d358b1 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | mariko / National Institute of Health Sciences / Tokyo / Japan |
| Date |  | 2013-02-06 14:22:45 JST |
| Remarks |  |  |

Administrative Data

| Purpose flag | supporting study |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | This study was reported as a part of the program by the National Cancer Institute, USA. Although there are some deficiencies in reporting, content of the study is reliable. |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Weisburger EK, Russfield AB, Homburger F, Weisburger JH, Boger E, Van Dongen CG & Chu K | 1978 | Testing of twenty-one environmental aromatic amines or derivatives for long-term toxicity or carcinogenicity | J. Environ. Pathol. Toxicol. 2(2) 325-356 |  |  |  |  |  |

Data access

data published

Materials and methods

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | other guideline: No data | no data |

Principles of method if other than guideline

Twenty one aromatic amines or derivatives were tested for long-term toxicity by dietary administration to male Charles River rats and male and female HaM/ICR mice. 2,5-xylidine was included in the group of substances tested.

GLP compliance

no (Study pre-dates GLP)

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 95-78-3 |
| EC name | 2,5-xylidine |

Details on test material

- Name of test material: 2,5-Xylidine - Analytical purity: Report states that most of the compounds tested were 97-99% pure

Test animals

Species

other: Rats and mice

Strain

other: Rats: Sprague-Dawley, Mice: ICR

Sex

male/female

Details on test animals and environmental conditions

TEST ANIMALS - Source: Charles River - Age at study initiation: 4-6 weeks - Acclimation period: Animals were acclimatized to the basal diet of powdered Purina laboratory chow for 2 weeks ENVIRONMENTAL CONDITIONS - Temperature: 23 ± 2°C

Administration / exposure

Route of administration

oral: feed

Vehicle

other: Steroid suspending vehicle. The composition was : Polysorbate 80, 0.4%; carboxymethylcellulose, 0.5%; benzyl alcohol, 0.9%, all dissolved in 0.9% aqueous solution of sodium chloride

Duration of treatment / exposure

No data

Frequency of treatment

No data

Post exposure period

No data

No. of animals per sex per dose

25

Control animals

yes, concurrent no treatment

Details on study design

Toxicity determinations: The LD50 value was determined by the intraperitoneal route in steroid suspending vehicle in order to determine a maximally tolerated oral dose. This was accomplished through 30 –day feeding studies in groups of animals, each consisting of 5 male and 5 female mice and 5 male rats at 5 dose levels. Based on food consumption, body weight changes and mortality during the 30 day feeding study and the following 2 week observation period on control diet to detect any delayed effects, the dose levels for long term bioassays were chosen. Long term studies: The test compound was fed at two levels, the higher level being the maximum tolerated dose, the lower, one-half that level. The 25 male and 25 female mice at each dietary level were housed in groups of 5 animals per plastic cage. The 25 male rats at each level were housed 2 to 3 per plastic cage when they were small; later 1 or 2 were housed per cage. The rats and mice were kept in separate air-conditioned animal rooms at 23 ± 2°C and observed daily, including weekends. Subgroups were phased in over a period of 8 months. Each group consisted of 1 untreated dietary control group (25 male rats, 25 male mice, 25 female mice) and 5 groups of rats and mice receiving 5 of the compounds at each of the 2 dose levels, or 150 animals per compound plus controls. These subgroups of 825 animals were started every few weeks as animals and chemicals became available. For the chronic studies the 4-6 week old animals were acclimated to the basal diet of powdered Purina laboratory chow for 2 weeks, randomized and assigned to experimental and control groups. They were given a diet containing the compound and continued for a period of 18 months. Mice were held 3 months longer and rats 6 months longer on a control diet before termination of the study. Control animals received Purina chow only. Body weights were followed carefully and served as a guide to continued dosage. Of the weight gain was below that observed in the corresponding controls by 10% or more, or if death occurred from toxicity. The dosage was lowered. The dosages and the times when changes were made in the dietary levels are shown in table 2.

Examinations

Sacrifice and pathology

Animals which died during the first 6 months of the study were discarded without necropsy. A complete gross necropsy was done on all animals which died after 6 months on test or were killed at the end of the experimental period. Histological examination was done on all grossly abnormal organs, tumor masses, lung spleen, liver, kidney, adrenal, heart, bladder, stomach, intestines, reproductive organs and rat pituataries.

Statistics

Statistical analysis of tumors found was performed using the Fisher Exact Test. The tumor incidence in a control group, matched or pooled was compared to the incidence in each treated group. If one applies the Bonferroni correction for the treated groups sharing the same controls then a tumor incidence is statistically significant if the p value from the Fisher Exact test is 0.025. Tumors in treated groups with p values of 0.05 or less are indicated in table 3. Tumors in treated groups with p values of 0.05 or less for both matched and pooled controls are deemed to be statistically significant

Results and discussions

Results of examinations

Clinical signs and mortality

no data

Body weight and weight gain

no data

Food consumption and compound intake (if feeding study)

no data

Food efficiency

no data

Water consumption and compound intake (if drinking water study)

no data

Ophthalmoscopic examination

no data

Haematology

no data

Clinical chemistry

no data

Urinalysis

no data

Neurobehaviour

no data

Organ weights

no data

Gross pathology

no data

Histopathology: non-neoplastic

no data

Histopathology: neoplastic

yes

Details on results

2,5-xylidine led to an increase in subcutaneous fibromas and fibrosarcoma in male rats relative to pooled controls only. The increase in hepatic tumors in male mice were not significant, but the incidence of vascular tumors was elevated when compared with that of pooled controls.

Any other information on results incl. tables

Table 1: LD50 values obtained in mice and rats for 2,5-Xylidine: Mice; Rats. M; F; M; F. 800; 872; 770; 848. Table 2: Dose levels in diet and changes Animals; Low; High; Time on level (months); Low; High; Time on level (months); Low; High; Time on level (months). Rats; 6000; 12000; 5; 3000; 6000; 13. Mice (M); 6000; 12000; 18. Mice (F); 6000; 12000; 18. Table 3: Tumors observed in male rats Tumor; High; Low; Control. Simultaneous; Pooled. Subcutaneous fibromas and fibrosarcomas; 9/17a; 7/17a; 8/17; 18/111. aDenotes significant difference (p < 0.025) from incidence in all pooled controls only Table 4: Tumors observed in male mice Tumor; High; Low; Control. Simultaneous; Pooled. Vascular; 7/19; 5/18b; 2/16b; 5/99. Liver; 1/10a; 4/18; 1/16; 5/99. aHepatomas bDenotes significant difference (p < 0.025) from incidence in pooled controls only Table 5: Tumors observed in female mice Tumor; High; Low; Control. Simultaneous; Pooled. Livera; 2/20; 5/16b,c,d; 0/13; 1/102. aHepatomas bDenotes significant difference (p < 0.025) from incidence in all simultaneous controls only cDenotes significant difference (p < 0.025) from incidence in all pooled controls only dp between 0.025 and 0.05

Applicant's summary and conclusion

Conclusions

2,5-xylidine led to an increase in subcutaneous fibromas and fibrosarcoma in male rats relative to pooled controls only. The increase in hepatic tumors in male mice were not significant, but the incidence of vascular tumors was elevated when compared with that of pooled controls.

# 7.8 Toxicity to reproduction

# 7.8.1 Toxicity to reproduction

## Endpoint study record: RA from 2,6-dimethylaniline

| UUID |  | IUC5-07454052-9571-4b0b-a1c8-c6d9fb3ccadb |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | mariko / National Institute of Health Sciences / Tokyo / Japan |
| Date |  | 2013-02-06 14:22:46 JST |
| Remarks |  |  |

Administrative Data

| Study result type | read-across based on grouping of substances (category approach) |  |  |
| --- | --- | --- | --- |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | MHLW, Japan. | 2005 | Combined Repeated Dose and Reproductive/Developmental Toxicity Screening Test of 2,6-Dimethylaniline by Oral Administration in Rats | http://dra4.nihs.go.jp/mhlw_data/jsp/SearchPageENG.jsp |  |  |  |  |  |

Materials and methods

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

no

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 87-62-7 |

# 7.8.2 Developmental toxicity / teratogenicity

## Endpoint study record: RA from 2,6-dimethylaniline

| UUID |  | IUC5-8361a231-df46-45f9-9da3-e51e0a333b55 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | mariko / National Institute of Health Sciences / Tokyo / Japan |
| Date |  | 2013-02-06 14:22:46 JST |
| Remarks |  |  |

Administrative Data

| Study result type | read-across based on grouping of substances (category approach) |  |  |
| --- | --- | --- | --- |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | MHLW, Japan. | 2005 | Combined Repeated Dose and Reproductive/Developmental Toxicity Screening Test of 2,6-Dimethylaniline by Oral Administration in Rats | http://dra4.nihs.go.jp/mhlw_data/jsp/SearchPageENG.jsp |  |  |  |  |  |

Materials and methods

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

no

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 87-62-7 |

# 7.9 Specific investigations

# 7.9.3 Specific investigations: other studies

## Endpoint study record: Seiler JP (1977)

| UUID |  | IUC5-6b6a1a33-cec9-4890-9703-4048e44c4754 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | mariko / National Institute of Health Sciences / Tokyo / Japan |
| Date |  | 2013-02-06 14:22:46 JST |
| Remarks |  |  |

Administrative Data

| Study result type | experimental result |  |  |
| --- | --- | --- | --- |
| Reliability | 3 (not reliable) |  |  |
| Rationale for reliability incl. deficiencies | Not performed to GLP or guideline. Deficiencies in reporting. |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Seiler JP | 1977 | Inhibition of testicular DNA synthesis by chemical mutagens and carcinogens. Preliminary results in the validation of a novel short term test | Mutation Research, 46(1977) 305-310 |  |  |  |  |  |

Data access

data published

Materials and methods

Type of effects studied

other: Inhibition of testicular DNA synthesis

Type of method

in vivo

Principles of method if other than guideline

The measurement of the inhibition of testicular DNA synthesis by chemical carcinogens and mutagens in male mice is considered as a simple and effective in vivo mammalian screening test and gives positive restuls with all types of carcinogens, whereas genetically inactive compounds do not suppress the DNA even at toxic doses. To evaluate the test system a series of experiments designed to validate the procedure was performed with 100 chemicals. Three groups of compounds were investigated. 2,5 dimethylaniline was included in the chemicals tested and was grouped among other chemicals in which preliminary data from Salmonella microsome tests gave rise to suspicions.

GLP compliance

no (Study pre-dates GLP)

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| EC name | 2,5-xylidine |
| CAS number | 95-78-0 |
| EC number | 202-451-0 |

Details on test material

- Name of test material: 2,5-dimethylaniline

Test animals

Species

mouse

Strain

no data

Sex

no data

Details on test animals and environmental conditions

TEST ANIMALS - Weight at study initiation: 23-28g

Administration / exposure

Route of administration

oral: unspecified

Vehicle

no data

Details on exposure

The compound was dissolved by ultrasonic treatment in a suitable carrier with the addition of up to 10% DMSO

Duration of treatment / exposure

No data

Frequency of treatment

No data

Post exposure period

No data

No. of animals per sex per dose

3-4 animals were used.

Control animals

yes

Further details on study design

No data

Examinations

Examinations

No data

Positive control

No data

Any other information on materials and methods incl. tables

Incorportation[3H] thymidineof into testicular DNA was expressed as cpm per µg DNA. The mean values were then tested for statistically significant deviations from the concurrent controls by a simplified t test, where only the mean and the extreme values of the sample are needed for the calculation.

Results and discussions

Details on results

Table 1 shows the results obtained for 2,5 dimethylaniline.

Any other information on results incl. tables

Table 1: Inhibition of thymidine incorporation into testicular DNA Dose (mg/kg) and mode of application; Mutagenicity/ carcinogenicitya; Incorporation of tritiated thymidine (% of concurrent control)b. 100 po; (+/0); 47.9c. a+ = mutagenic/carcinogenic activity, 0 = no data bStatistical significance of deviation from control (=100%) by attest c P < 0.01

Applicant's summary and conclusion

Conclusions

The conclusion of the report is focused on the validity of the assay itself rather than the individual test results. Results conlcude that the assay is a valuable tool in predicting mutagenicity/carcinogenicity but the assay is not very sensitive as large amounts up to the LD50 have to be given to animals to elicit a response. It is therefore nearly impossible to detect the mutagenic or carcinogenic activity of an impurity within an otherwise inactive compound as does the Ames test. Analytical work is thus necessary to detect and identify impurities.It has to be stressed that the assay cannot reveal the true mechanism of cancer induction: but with a correlation between carcinogenic activity and the biological activity in such as test as close t0 100% as possible it may be used as a predictive tool that could enable cancer prevention.

## Endpoint study record: Marques MM et al (1997)

| UUID |  | IUC5-7a9b84f5-31fb-4991-b9e0-3a9174c98b7e |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | mariko / National Institute of Health Sciences / Tokyo / Japan |
| Date |  | 2013-02-06 14:22:46 JST |
| Remarks |  |  |

Administrative Data

| Study result type | experimental result |  |  |
| --- | --- | --- | --- |
| Reliability | 4 (not assignable) |  |  |
| Rationale for reliability incl. deficiencies | Insufficient detail in the published reference. |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Marques, MM, Mourato LLG, Amorim MT, Santos MA, Melchior WB & Beland FA. | 1997 | Effect of substitution site upon the oxidation potentials of alkylanilines, the mutagenicities of N-hydroxyalkylanilines and the conformations of alkylaniline-DNA adducts. | Chem.Res. Toxicol. 10: 1266 - 1274. |  |  |  |  |  |

Data access

data published

Materials and methods

Type of effects studied

other: DNA adduct formation

Type of method

in vitro

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | other guideline: No data | no data |

Principles of method if other than guideline

The ability of dimethylaniline constituents of tobacco smoke to undergo oxidation at the nitrogen atom was analysed. The mutagenicity of the corresponding N-hydroxylalkylanilines and the conformational properties of the DNA adducts from their N-acetoxy derivatives were investigated.

GLP compliance

no data

Test materials

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 95-78-3 |
| EC number | 202-451-0 |
| EC name | 2,5-xylidine |

Details on test material

- Name of test material: 2,5-dimethylaniline

Test animals

Species

other: S. typhimurium

Strain

other: TA100

Sex

no data

Details on test animals and environmental conditions

No data

Administration / exposure

Route of administration

other: Not applicable; the study was performed in vitro

Vehicle

no data

Details on exposure

Mutagenicity assays were conducted in the absence of exogenous metabolic activation using Salmonella typhimurium TA100. Triplicate plates were used for each dose.

Analytical verification of doses or concentrations

no data

Details on analytical verification of doses or concentrations

No data

Duration of treatment / exposure

No data

Frequency of treatment

No data

Post exposure period

No data

Doses / concentrations

| 0, 10, 30, 60 and 100 µg/plate |  |
| --- | --- |
| Basis | other: Acutal added to in vitro assay. |

No. of animals per sex per dose

Not applicable; an in vitro study.

Control animals

no data

Further details on study design

The N-hydroxylamines were synthesized in 50 - 60 % yields by reduction of the corresponding nitroarenes at 50 °C with zinc dust in aqueous ammonium chloride containing 30 % ethanol. DNA modification reactions: Chemical modification of DNA with each arylamine was conducted by reacting the corresponding N-acetoxyarylamine with salmon testes DNA. Following standard enzymatic hydrolysis of the DNA to nucleosides, the HPLC retention times and UV spectra of arylamine-DNA adducts were compared to those of the corresponding dG adducts.

Examinations

Examinations

No data

Positive control

No data

Results and discussions

Details on results

Mutagenicity of N-hydroxylarylamines in S. typhimurium TA100. The relationship between the mutagenicity and the location of the alkyl substituent in the N-hydroxyarylamines was analysed using S. typhimurium TA100. N-hydroxyarylamines with only m-alkyl substituents (N-hydroxy-3,5-dimethylaniline) tended to give the weakest mutagenic response. The introduction of p-alkyl substituent increased the mutagenicity3,5-dimethyl versus 3,4-dimethyl. N-hydroxy-2,4-dimethylaniline was more mutagenic than its 2,3- and 2,5-dimethylanalogues. it should be noted that part of the increase in activity observed for N-hydroxy-2,4-dimethylaniline can be attributed to a higher adduct forming efficiency of its N-acetoxy derivative (9 %) compared to that of the N-(acetoxy)-2,3- and 2,5-dimethylaniline isomers (4 % and 2 %, respectively). Synthesis and characterisation of alkylaniline-DNA adducts: N-acetoxyarylamines were used as electrophilic synthon for the preparation of alkylaniline-DNA adducts. Due to their instability, these reactive intermediates were prepared in situ from the corresponding N-hydroxylarylaminesby reaction at low temperature with acetyl cyanide or pivaloyl cyanide in dry THF. The N-(acetoxy)arylamines were then reacted immediately with dG, dGnp (n = 3',5' and 3',5') and DNA. Adducts formed upon reaction of the N-(acetoxy)arylamines with DNA were identified after enzymatic hydrolysis of hte modified DNA to nucleosides. A single major adduct, which was characterised as a dG-C8-Ar species on the basis of spectroscopic evidence, was detected in all instances. In conclusion, using S. typhimurium TA100 as the tester strain, increased mutagenicities in N-hydroxyarylamines containing p-alkyl substituents was found. In all instances N-(acetoxy)arylamines generated from the N-hydroxyarylamines reacted with dG, dG nucleotides and DNA to yield C8-substituted dG derivatives as the major adducts. This was consistent with previous observations with carcinogenic aromatic amines, such as 2,aminofluorene and 4-aminobiphenyl. Taken together the data suggests that the conformational properties of the DNA adducts, in particular their ability to adopt syn conformations, may be determinant factors for the genotoxic responses elicited by certain alkylanilines.

Applicant's summary and conclusion

Conclusions

Using S. typhimurium TA100 as the tester strain, increased mutagenicities in N-hydroxyarylamines containing p-alkyl substituents was found.

## Endpoint study record: Thelestam M et al (1980)

| UUID |  | IUC5-4e02160b-d8c2-4561-b1a0-6de29e7b572d |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | mariko / National Institute of Health Sciences / Tokyo / Japan |
| Date |  | 2013-02-06 14:22:46 JST |
| Remarks |  |  |

Administrative Data

| Reliability | 2 (reliable with restrictions) |
| --- | --- |
| Rationale for reliability incl. deficiencies | Study published in peer reviewed journal. |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Thelestam M, Curvall M & Enzell CR. | 1980 | Effect of Tobacco Smoke Compounds on the Plasma Membrane of Cultured Human Lung Fibroblasts. | Toxicology, 15, 203-217. |  |  |  |  |  |

Data access

data published

Materials and methods

Type of effects studied

mechanistic studies

Type of method

in vitro

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | other guideline: no data | no data |

Principles of method if other than guideline

Human diploid embryonic lung fibroblasts (line MRC-5) were cultivated in Eagle's medium in polystyrene wells to a cell density of 1E05 cells/cm2 (approx. 7E05 cells/well). The cells in confluent monolayers were labelled with [3H]uridine to obtain a low molecular weight cytoplasmic marker consisting of uridine nucleotides. Labelled cultures wereincubated for 30 min at 37 deg C with test compound diluted to 25 mM in tris-buffered saline. The solution containing leaked radioactive marker was removed and centrifuged and radioactivity in 0.1 mL aliquots of the supernatant was measured. A maximal release of the radioactivity was obtained by treating control cells for 30 minutes with 0.06M sodium borate buffer and scraping with a rubber policeman. This treatment ruptured the cell membranes leaving the nuclei intact.

GLP compliance

no (Study pre-dates GLP.)

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 95-78-3 |
| EC number | 202-451-0 |
| EC name | 2,5-xylidine |

Details on test material

- Name of test material: 2,5-xylidine

Test animals

Species

human

Strain

other: Not applicable

Sex

no data

Details on test animals and environmental conditions

Not applicable; this is an in vitro test.

Administration / exposure

Route of administration

other: In vitro administration

Details on exposure

Human diploid embryonic lung fibroblasts (line MRC-5) were cultivated in Eagle's medium in polystyrene wells to a cell density of 1E05 cells/cm2 (approx. 7E05 cells/well). The cells in confluent monolayers were labelled with [3H]uridine to obtain a low molecular weight cytoplasmic marker consisting of uridine nucleotides. Labelled cultures wereincubated for 30 min at 37 deg C with test compound diluted to 25 mM in tris-buffered saline. The solution containing leaked radioactive marker was removed and centrifuged and radioactivity in 0.1 mL aliquots of the supernatant was measured. A maximal release of the radioactivity was obtained by treating control cells for 30 minutes with 0.06M sodium borate buffer and scraping with a rubber policeman. This treatment ruptured the cell membranes leaving the nuclei intact.

Analytical verification of doses or concentrations

no data

Duration of treatment / exposure

30 min

Frequency of treatment

Single exposure

Post exposure period

None

Doses / concentrations

| 25 mM |  |
| --- | --- |
| Basis | other: Added to the cell cultures |

No. of animals per sex per dose

Not applicable; this is an in vitro test.

Control animals

other: Not applicable; this is an in vitro test.

Further details on study design

Type: other: Effect on membrane permeability

Results and discussions

Details on results

Nucleotide release = 18 % . This result shows that 2,5-xylidine caused some damage to the cell membrane.

Applicant's summary and conclusion

Conclusions

The test substance was assessed for its ability to specifically detect primary damage to the cell membrane. The test substance caused 18 % nucleotide release. This result shows that 2,5-xylidine caused some damage to the cell membrane.

## Endpoint study record: Pettersson B et al (1982)

| UUID |  | IUC5-ae83283f-8fd4-4d2a-983a-a7a9c5e4ad8c |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | mariko / National Institute of Health Sciences / Tokyo / Japan |
| Date |  | 2013-02-06 14:22:46 JST |
| Remarks |  |  |

Administrative Data

| Reliability | 2 (reliable with restrictions) |
| --- | --- |
| Rationale for reliability incl. deficiencies | Study published in peer reviewed journal. |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Pettersson B, Curvall M & Enzell CR. | 1982 | Effects of Tobacco Smoke Compounds on the Ciliary Activity of the Embryo Chicken Trachea In Vitro. | Toxicology, 23, 41-55. |  |  |  |  |  |

Data access

data published

Materials and methods

Type of effects studied

mechanistic studies

Type of method

in vitro

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | other guideline: No data | no data |

Principles of method if other than guideline

The ciliotoxicity of the test subtsance at 5 mM was investigated using tracheal organ cultures and inverted microscopy.

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 95-78-3 |
| EC number | 202-451-0 |
| EC name | 2,5-xylidine |

Details on test material

IUCLID4 Test substance: as prescribed by 1.1 - 1.4

Test animals

Species

other: chicken

Strain

no data

Sex

no data

Administration / exposure

Route of administration

other: Not applicable; test was performed in vitro.

Vehicle

no data

Analytical verification of doses or concentrations

no data

Duration of treatment / exposure

60 min

Frequency of treatment

Not applicable; the study was performed using organ cultures in vitro.

Post exposure period

Not applicable; the study was performed using organ cultures in vitro.

Doses / concentrations

| 5 mM |  |
| --- | --- |
| Basis | nominal conc. |

No. of animals per sex per dose

Not applicable; the study was performed using organ cultures in vitro.

Further details on study design

Type: other: Ciliotoxicity Chicken tracheal organ cultures were prepared asceptically from 16 to 17-day old chicken embryos. Trachea rings were prepared and used within 5 - 10 days. The ciliary activity was studied at 37 °C by means of inverted microscopy using a magnification of 250. One tracheal ring was placed in a perspex testing chamber (vol 3.1 mL) containing the medium admixed with an ethanol or DMSO solution of the test substance. After addition of the tracheal ring the chamber was closed to external air. The microscope was connected to a tv camera and recording equipment allowing automatic recording of the ciliaryactivity. The ciliary activity was displaye continuously on the monitor and recorded for 10 s every min. The time to complete cessation of ciliary activity was determined. The test was repeated on at least 3 separate occassions. Each experiment involved rings from different tracheal preparations. At the magnification used, it was not possible to display the entire cross-section of the tracheal ring. Instead, one part of the ring having well-exposed cilia was selected and studied. Both solvents used were non-toxic to the cilia at the concentration used in teh experiments (1.6 % v/v).

Results and discussions

Details on results

The time to ciliostasis for the test substance was > 60 min and was therefore considered non-toxic to cilia.

Applicant's summary and conclusion

Conclusions

The ciliotoxicity of the test subtsance at 5 mM was investigated using tracheal organ cultures and inverted microscopy. The time to ciliostatis was > 60 min and was therefore considered non- toxic to the cilia.

## Endpoint study record: Pettersson B et al (1980)

| UUID |  | IUC5-07605f28-084c-4544-87c7-9856d57bf974 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | mariko / National Institute of Health Sciences / Tokyo / Japan |
| Date |  | 2013-02-06 14:22:47 JST |
| Remarks |  |  |

Administrative Data

| Reliability | 2 (reliable with restrictions) |
| --- | --- |
| Rationale for reliability incl. deficiencies | Study published in peer reviewed journal. |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Pettersson B, Curvall M & Enzell CR. | 1980 | Effect of Tobacco Smoke Compounds on the Noradrenaline Induced Oxidative Metabolism in Isolated Brown Fat Cells. | Toxicology, 18, 1-15. |  |  |  |  |  |

Data access

data published

Materials and methods

Type of effects studied

mechanistic studies

Type of method

in vitro

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | other guideline: no data | no data |

Principles of method if other than guideline

The effect on the cell metabolism of the test substance was investigated by measuring the inhibition of noradrenaline induced respiration in isolated hamster brown fat cells.

GLP compliance

no (Study pre-dates GLP.)

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 95-78-3 |
| EC number | 202-451-0 |
| EC name | 2,5-xylidine |

Details on test material

- Name of test material: 2,5-xylidine - Analytical purity: > 97 %

Test animals

Species

hamster

Strain

other: Brown

Sex

no data

Details on test animals and environmental conditions

Not applicable; in vitro study.

Administration / exposure

Route of administration

other: not applicable; in vitro study.

Vehicle

no data

Details on exposure

Isolated brown fat cells were prepared from adult hamsters.

Analytical verification of doses or concentrations

no data

Duration of treatment / exposure

5 min followed by 5 min with noradrenaline.

Frequency of treatment

Single application

Post exposure period

No data

Doses / concentrations

| 1 mM |  |
| --- | --- |
| Basis | other: applied to cells |

No. of animals per sex per dose

Not applicable, this is an in vitro study. The cells were used at a final concentration of 10^5 cells/mL.

Control animals

yes

Further details on study design

Type: other: Inhibition of noradrenaline induced oxidative metabolism

Examinations

Examinations

The oxygen consumption rate of the cells were measured at 37 °C using a Clark-type oxygen electrode fitted in a perspex vessel of 1 mL volume. The output of the electrode was amplified and continuously registered ona recorded. After addition of a buffer solution the vessel was sealed to external air except for a narrow channel through which subsequent additions were made. Sufficient cells were introduced to give a final concentration of 10^5 cells/mL and the test substances, dissolved in ethanol or DMSO were incubated with thecells for exactly 5 min during which period the oxygen consumption was registered. After this preincubation, noradrenaline was added and the oxygen consumption of the cells was registered for a further 5 min. The noradrenaline concentration was approximately twice the dose known to induce maximal respiratory rate. Excess noradrenaline was used to avoid inactivation of noradrenaline per se by the test substance. The toxicity of a substance was determined by comparing the noradrenaline induced oxygen consumption in the presence of the test compound to that observed for the control (solvent blank); the inhibition is expressed as a percentage.

Positive control

An ethanol solution of harman and an ethanol control.

Results and discussions

Details on results

The toxicity of a substance is determined by comparing the noradrenaline induced oxygen consumption in the presence of the test compound to that observed to the control. The inhibition registered is expressed as a percentage. At 1 mM, 2,6-xylidine produced 34 % inhibition.

Applicant's summary and conclusion

Conclusions

The effect on the cell metabolism of the test substance was investigated by measuring the inhibition of noradrenaline induced respiration in isolated hamster brown fat cells. The toxicity of a substance is determined by comparing the noradrenaline induced oxygen consumption in the presence of the test compound to that observed to the control. The inhibition registered is expressed as a percentage. At 1 mM, 2,5-xylidine produced 34 % inhibition.

## Endpoint study record: Curvall M et al (1984)

| UUID |  | IUC5-4b1f1923-1c16-4a43-9f86-dcc064d0fd57 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | mariko / National Institute of Health Sciences / Tokyo / Japan |
| Date |  | 2013-02-06 14:22:47 JST |
| Remarks |  |  |

Administrative Data

| Reliability | 3 (not reliable) |
| --- | --- |
| Rationale for reliability incl. deficiencies | Insufficient details of results. |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Curvall M, Enzell CR & Pettersson B. | 1984 | An Evaluation of the Utility of Four In Vitro Short Term Tests for Predicting the Cytotoxicity of Individual compounds Derived from Tobacco Smoke. | Cell Biol. Toxicol., 1, 173-193. |  |  |  |  |  |

Data access

data published

Materials and methods

Type of effects studied

cytotoxicity

Type of method

in vitro

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | other guideline: No data | no data |

Principles of method if other than guideline

Biological data on the test compounds effect on cell multiplication, oxidative metabolism, ciliary activity and membrane permeability were studied.

GLP compliance

no (Study pre-dates GLP)

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 95-78-3 |
| EC number | 202-451-0 |
| EC name | 2,5-xylidine |

Details on test material

IUCLID4 Test substance: as prescribed by 1.1 - 1.4

Test animals

Species

other: In vitro study on ascetis scrcoma BP, brown fat cells from adult hamsters, human diploid embryonic lung fibroblasts and chicken embryo tracheas.

Strain

other: Not applicable

Sex

no data

Details on test animals and environmental conditions

Not applicable; the tests were performed in vitro.

Administration / exposure

Route of administration

other: Not applicable

Vehicle

no data

Details on exposure

Inhibition of cell growth: Ascites sarcoma BP 8 cells were grown as stationary suspension cultures in Hams F-10 medium with fetal calf serum (15%), penecillin (96 IU/mL) and streptomycin 83 ug/mL). Substance in ethanol or DMSO added to 3 mL aliqouts of cell suspension (0.4E04 cells/mL). After incubation for 48 h the cell density was determined. The inhibitory effect is expressed as the ratio between the growth rate of the treated cell culture and the control and given as a percentage. The substance was tested at a final concentration of 1 mM and the test was performed in duplicate. Inhibition of oxidative mechanism: Inhibition of the oxidative metabolism in isolated brown fat cells was determined by measuring the oxygen consumption. Brown adipocytes were isolated by collagenase treatment of brown fat from adult hamsters (Mesocricetus auratus). The toxicity was determined by comparing the norepinephrine induced oxygen consumption, after addition of the substance, to that of the control and expressing the ratio as a percentage. A final concentration of 1 mM was tested. At least five experiments were performed on different cell preparations. Membrane damage: Plasma membrane damage was assessed as leakage of a cytoplasmic nucleotide marker from prelabelled cells. Human diploid embryonic lung fibroblasts (line MRC-5) between the 20th and 35th generation were used. Cells in confluent monolayers were labelled with [3H]-uridine to obtain a low molecular weight cytoplasmic marker, consisting of uridine nucleotides. The cells were rinsed three times with Hank's balanced salt solution and treated for 30 mintues at 37 deg C with the substance (25 mM solution). The solution was centrifuged and the released radioactivity was measured. The results were expressed as a percentage of the maximal release. The test was run in duplicate. Ciliotoxicity: Ciliotoxicity was measured as time to ciliostasis in the tracheas of unborn chickens. The substance was tested at a 5 mM concentration and examined on at least 3 different occassions, each experiment involving rings from different tracheal preparations.

Analytical verification of doses or concentrations

no data

Duration of treatment / exposure

Inhibition of cell growth: 48 h Membrane damage: 30 min

Frequency of treatment

Single application

Post exposure period

Not applicable

Doses / concentrations

| 1 mM or 5mM |  |
| --- | --- |
| Basis | other: test substance |

No. of animals per sex per dose

Not applicable; the tests were performed in vitro.

Control animals

other: Not applicable; the tests were performed in vitro.

Results and discussions

Details on results

Inhibition was scored as between 1 - 10 % in the tests for inhibition of cell growth, inhibition of oxidative metabolism and membrane damage. No ciliotoxicity was observed.

Applicant's summary and conclusion

Conclusions

Biological data on the test compounds effect on cell multiplication, oxidative metabolism, ciliary activity and membrane permeability were studied. Inhibition was scored as between 1 - 10 % in the tests for inhibition of cell growth, inhibition of oxidative metabolism and membrane damage. No ciliotoxicity was observed.

# 7.10 Exposure related observations in humans

# 7.10.2 Epidemiological data

## Endpoint study record: Gan J et al (2004)

| UUID |  | IUC5-c27f43a1-5254-434c-adc2-aabec3e75ebc |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | mariko / National Institute of Health Sciences / Tokyo / Japan |
| Date |  | 2013-02-06 14:22:47 JST |
| Remarks |  |  |

Administrative Data

| Reliability | other: Not applicable for epidemiological data |
| --- | --- |
| Rationale for reliability incl. deficiencies | Not applicable for epidemiological data |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Gan J, Skipper PL, Gag-Dominguez M, Arakawa K, Ross RK, Yu MC & Tannenbaum SR. | 1994 | Alkylaniline-hemoglobin adducts and risk of non-smoking-related bladder cancer. | Journal of the National Cancer Institute 96 (19) 1425 - 1431. |  |  |  |  |  |

Materials and methods

Study type

case control study (retrospective)

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | other guideline: Not applicable | not applicable |

Principles of method if other than guideline

A population-based case-control study was performed in Los Angeles County, California involving 298 case subjects with bladder cancer and 308 control subjects who were matched on age, sex, race/ethnicity and neighbourhood of residence. In-person interviews provided information on tobacco smoking and other potential risk factors for bladder cancer. To assess arylamine exposure, levels of arylamine-hemoglobin adducts of nine selected alkylanilines were measured in peripheral blood collected from study subjects. Analysis of covariance and conditional logistic regression methods were used to analyse the relationship between arylamine-hemoglobin adducts and bladder cancer.

GLP compliance

no data

Test materials

Test material identity

| Identifier | Identity |
| --- | --- |
| EC name | 2,5-xylidine |
| CAS number | 95-78-3 |
| EC number | 202-451-0 |

Details on test material

- Name of test material: 2,3-dimethylaniline.

Method

Type of population

other: Bladder cancer patients

Ethical approval

no data

Details on study design

HYPOTHESIS TESTED (case control study): METHOD OF DATA COLLECTION - Type: Interview - Details: Structured interviews were conducted by study personnel in each subject's home. During the interview, subjects were asked to provide information up to two years before the diagnosis of bladder cancer for case subjects or for two years before the diagnosis of cancer o the index case subject for the matched control subject (i.e. reference year). Information was requested on demographic characteristics, lifetime use of tobacco products and alcohol, usual dietary habits, lifetime occupational history, previous medical conditions and previous medication use. STUDY POPULATION - Total population (Total no. of persons in cohort from which the subjects were drawn): Figure not specified; enrolled case subjects represented approximately 75 % of the total number of eligible patients. - Selection criteria: Subjects with bladder cancer identified through the Los Angeles County Cancer Surveillance Program (a population-based Surveillance, Epidemiology and End Results (SEER) program cancer registry of Los Angeles County. - Total number of subjects participating in study: 1679 - Sex/age/race: Non-Asian race/ethnicity, 25-64 years of age diagnosed with histologically confirmed cancer between January 1st 1987 and April 30th 1996. - Smoker/nonsmoker: - Total number of subjects at end of study: - Matching criteria: - Other: COMPARISON POPULATION - Type: Other comparison group: An invariant procedure was followed to recruit a control subject from the same neighbourhood of residence as the index case at the time of cancer diagnosis who matched the index case by age (within 5 years), sex, race/ethnicity. - Details: HEALTH EFFECTS STUDIED - Disease(s): Bladder cancer - ICD No.: - Year of ICD revision: - Diagnostic procedure: - Other health effects: OTHER DESCRIPTIVE INFORMATION ABOUT STUDY: A matched control could not be found for 5% (n = 86) of enrolled case subjects. For 21 index case patients the control subject was not matched by race/ethnicity. All subjects provided written informed consent (separate forms for interview and blood/urine donation) that had been approved by the institutional review boards of the University of Southern California and the Massachusetts Institute of Technology. Beginning in January 1992, all case subjects (n = 1044) and their matched control subjects (n = 979) were asked to provide a blood sample at the end of the in-person interview. Blood was obtained and successfully analysed from 761 (73 %) case subjects from 770 (79 %) control subjects. All subjects who donated blood were asked detailed questions on their use of their use of tobacco products during the preceding 2 months. All subjects were asked to collect an over night urine sample (end with the first morning void) after consuming two cups of instant coffee (approximately 70 mg of caffeine) in the previous afternoon (between 3 and 6 pm). Caffeine tablets were offered as an alternative to non-coffee drinkers. From the 1536 subjects who donated blood, 606 were selected for the study of alkylaniline-hemoglobin adducts. This subset of study participants provided the last 606 samples to be analysed . The analytical method was not available for the first 930 subjects. Of the 606 subjects, 298 were case subjects and 308 were control subjects. The study included 190 matched pairs, 108 unmatched case subjects and 118 unmatched control subjects.

Details on exposure

No data

Statistical methods

The distributions of the alkylaniline-hemoglobin adduct levels in the study population were markedly skewed; therefore formal statistical testing was performed on logarithmically transformed values of adduct levels, and geometric (as opposed to arithmetic) mean values were presented. Pair-wise Pearson correlation coefficients (on logarithmically transformed values) were calculated for the nine adducts under study to examine the extent for which they were associated with The analysis of covariance method was used to compare adduct levels between case and control subjects, with an adjustment for the effects of cigarette smkoing at the time of blood draw on adduct levels. The analysis of covariance method was also used to compare adduct levels between exclusive users of permanent hair dyes and non-users of permanent hair dyes, with an adjustment for the effect of cigarette smoking at the time of blood draw on adduct levels. Additional adjustments were also made for other risk factors for bladder cancer identified in the Los Angeles Bladder Cancer Study. These covariates included educational level, lifetime history of cigarette smoking, regular use of non-steriodal anti-inflammatory drugs, use of permanent hair dyes, dietary intake of carotenoids, 4-ABP-hemoglobib adducts, NAT2 phenotype and GSTM1 genotype. Results of analyses with and without adjustment for ethnicity/race were simillar therefore race/ethnicity was not retained as a covariate. From all alkylanilines that had a statistically significant univariate association with bladder cancer risk, a stepwise regression method was used to identify the set of alkylanilines that were independent predictors of risk. The backward and forward selection methods yielded the same set of independent risk predictors. Conditional logic regression models were used to examine the relationship between quartiles of adduct levels and bladder cancer risk. Odds ratios were used to estimate relative risks. All P-values were two-sided.

Any other information on materials and methods incl. tables

Laboratory analyses: Blood specimens were collected in heparinized tubes and were fractionated into plasma, buffy coat and erythrocytes, all of which were stored at -80 °C before analyses. Erythrocytes, identifiable only by their code numbers were sent on dry ice to the Massachusetts Institute of Technology for arylamine-haemoglobin adduct analysis. Haemoglobin adducts of nine alkylanilines were subjected to quantitative analysis.

Results and discussions

Results

With the exception of 2,6-DMA, all alkylaniline-haemoglobin adducts showed positive and relatively close correlations with each other. Mean levels of alkylaniline-haemoglobin adducts were higher in smokers than in non smokers, with the exception of 2,6-DMA alkylaniline-haemoglobin adducts. Adducts of alkylaniline were higher in case studies than in control subjects. Levels of 2,3-DMA-, 2,4-DMA-, 2,6-DMA- and 3,5-DMA- adducts were statistically higher in non-smoking case subjects than in smoking control subjects. 2,6-DMA and 3,5-DMA were identified as independent predictors of bladder cancer and there was a statistically significant increase in risk associated with an increase in adduct levels for each of these. The increased risk persisted when the analysis was restricted to nonsmokers blood draw. Among non-smokers, individuals in the highest quartiles of 2,6-DMA- and 3,5-DMA-haemoglobin adduct levels were three to five times more likely to develop bladder cancer than individuals in the lowest quartiles. Relative risk for 2,6-DMA was 5.5 (95% CI = 2.8 - 10.7); relative risk for 3,5-DMA was 3.1 % (CI =1.6 - 6.0). These increased risks persisted after adjustment for other risk factors identified in the study population. It had been speculated previously that the carcinogenic agents in hair dyes may be arylamines, therefore the levels of nine-alkylaniline-haemoglobin adducts according to sex were assessed, with adjustments for age at blood draw and smoking status at the time of blood draw. Only levels of 3,5-DMA demonstrated a statistically significant difference (P = 0.04) with levels higher in women than in men (adjusted geometric mean27.8 pg/g of haemoglobin in women versus23.3 pg/g of haemoglobin in men). The levels of 3,5-DMA-haemoglobin adduct according to use of permanent hair dye, adjusting for the number of cigarettes smoked per day were assessed. The adjusted geometric mean level of 3,5-DMA-haemoglobin adducts was higher in exclusive users of permanent dye than in non-users. The difference was statistically significant when the analysis was restricted to women.

Confounding factors

No data

Strengths and weaknesses

No data

Applicant's summary and conclusion

Conclusions

A population-base case-control study was performed. Levels of all arylamine-haemoglobin adducts, with the exception of 2,6-DMA were higher in smokers than in non-smokers and levels of arylamine-haemoglobin adducts were higher in case subjects than in control subjects. Arylamine-haemoglobin adducts of 2,6-DMA and 3,5-DMA were independently statistically significantly (P<0.001) associated with bladder cancer risk after adjusting for cigarette smoking at the time of blood collection, lifetime smoking history and other potential risk factors. These adducts were also independently associated with bladder cancer risk when only non-smokers were considered.

## Endpoint study record: Bryant MS et al (1988)

| UUID |  | IUC5-2cdd60da-48e3-40b1-88a3-df34fac8e217 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | mariko / National Institute of Health Sciences / Tokyo / Japan |
| Date |  | 2013-02-06 14:22:47 JST |
| Remarks |  |  |

Administrative Data

| Study result type | experimental result |  |  |
| --- | --- | --- | --- |
| Reliability | other: Not applicable for epidemiology data |  |  |
| Rationale for reliability incl. deficiencies | Not applicable for epidemiology data |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Bryant MS, Vineis P, Skipper PL & Tannenbaum | 1988 | Hemoglobin adducts of aromatic amines: Associations with smoking status and type of tobacco | Proc. Natl. Acad. Sci. USA. 85: 9788-9791 |  |  |  |  |  |

Data access

data published

Materials and methods

Study type

cohort study (retrospective)

Endpoint addressed

carcinogenicity

Principles of method if other than guideline

Hemoglobin adducts of 15 aromatic amines (one of which included 2,5 dimethylaniline) were determined in nonsmokers and smokers of blond or black tobacco cigarettes living in Turin, Italy. Subjects were all males age 55 or less and were representative of the population previously examined in a case/control study of bladder cancer.

Test materials

Test material identity

| Identifier | Identity |
| --- | --- |
| EC name | 2,5-xylidine |
| CAS number | 95-78-3 |
| EC number | 202-451-0 |

Details on test material

- Name of test material: 2,5 dimethylaniline

Method

Type of population

general

Details on study design

HYPOTHESIS TESTED (if cohort or case control study): The study describes results of a study conducted on residents in the city of Turin, Italy, in which hemoglobin adducts of 4-aminobiphenyl and other aromatic amines were measured in a population previously studied with regard to bladder cancer risk. A putative etiologic role for this class of compounds in the bladder carcinogenicity of tobacco smoke could thus be examined. METHOD OF DATA COLLECTION - Type:Questionnaire / Clinical tests : - Details: Male volunteers were recruited at a blood donor center in the city of Turin, Italy to donate an additional 10 ml blood sample and fill out a questionnaire on their smoking history and other parameters such as occupation. During two collection periods, 87 samples were collected from 25 non-smokers, 40 blond-tobacco smokers and 3 mixed blond/black-tobacco smokers. SETTING: Blood donor center STUDY POPULATION - Total population: 25 nonsmokers and 61 smokers - Selection criteria: Subjects were representative of the resource population previosuly used in a case/control study for bladder cancer. - Sex/age: Males age 55 or less - Smoker/nonsmoker: Smokers and non-smokers - Matching criteria: The mixed blond/black tobacco smokers were grouped with the blond-tobacco users. HEALTH EFFECTS STUDIED - Disease: Bladder cancer

Exposure assessment

no data

Statistical methods

Adjustments of adduct levels by age of the subjects or by amount smoked as continuous variables were done with the least-squares method using a linear model. Correlations with smoking status were determined by comparing the 25 nonsmokers were the 61 smokers. Statistical significance was determined with a Wilcoxon two-sample test. Correlations with tobacco type were determined by comparing the 42 smokers of blond tobacco with the 18 smokers of black tobacco

Any other information on materials and methods incl. tables

Measurement of aromatic amine-hemoglobin adducts: Erythrocytes were washed with saline, frozen and then coded later for analysis. All analyses were performed without prior knowledge of sample identity.

Results and discussions

Results

Table 1 shows the P values for the association of adduct levels with either smoking status (smoker versus non smoker) or tobacco type (black tobacco versus blond tobacco) with 2,5-dimethylaniline.

Confounding factors

None of the subjects was involved in occupations known to be associated with aromatic amine exposure. Furthermore, no occupation was associated with any of the three comparison groups (nonsmokers and blond or black tobacco smokers). Thus occupation is unlikely to be a confounding factor.

Strengths and weaknesses

No data

Any other information on results incl. tables

Table 1: Hemoglobin adduct levels of 2,5 dimethylaniline: Associations with smoking status and tobacco type Mean adduct level, pg of amine per g of Hb (SEM); Pfor association. Non smokers; Smokers. Blond tobacco; Black tobacco; Smoking status; Tobacco type. 50 (6); 67 (10); 70 (14); 0.45; 0.63.

Applicant's summary and conclusion

Conclusions

The results of the study indicate that measurement of the hemoglobin adducts may be useful in examining the biochemical basos for increased risk of bladder cancer in smokers.

# 14 Information requirements

# 14.2 Alternative name request

# Reference substance: 2,5-xylidine

| UUID |  | ECB5-458bf9d6-b577-4178-93ad-836832788e64 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V3.0 Plug-In |
| Date |  | 2007-05-10 18:00:00 JST |
| Remarks |  | Successfully migrated to IUCLID 5.4 format. |

General information

| Reference substance name | 2,5-xylidine |
| --- | --- |

EC inventory

| EC number | 202-451-0 | CAS number | 95-78-3 |
| --- | --- | --- | --- |
| EC name | 2,5-xylidine |  |  |
| Molecular formula | C8H11N |  |  |

Reference substance information

CAS information

| CAS number | 95-78-3 |
| --- | --- |

IUPAC name

| 2,5-dimethylaniline |  |
| --- | --- |

Synonyms

| Name | Benzenamine, 2,5-dimethyl- |
| --- | --- |

Molecular and structural information

| Molecular formula | C8H11N |
| --- | --- |
| Molecular weight range | 121.1796 |
| SMILES notation | Cc1ccc(C)c(N)c1 |
| InChI | InChI=1/C8H11N/c1-6-3-4-7(2)8(9)5-6/h3-5H,9H2,1-2H3 |
| Structural formula |  |

# Legal entity: National Institute of Health Sciences

| UUID |  | IUC4-b036ff75-0f3c-323b-b200-ed5f46cf5101 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V3.0 Plug-In |
| Date |  | 2009-01-09 13:32:30 JST |
| Remarks |  | Successfully migrated to IUCLID 5.4 format. |

General information

| Legal entity name | National Institute of Health Sciences |
| --- | --- |

Identifiers

Other IT system identifiers

| Flags | IT system | ID | Remarks |
| --- | --- | --- | --- |
|  | LEO | 10767 |  |
|  | IUCLID4 | 16558402024DIV750 |  |

Contact information

Contact address

| Address | 1-18-1 kamiyoga |
| --- | --- |
| Address | Setagaya-ku |
| Postal code | 158-8501 |
| Town | Tokyo |
| Country | Japan |

Contact persons

| Organisation | National Institute of Health Sciences |
| --- | --- |
| Department | Division of Risk Assessment |
| Title | Dr. |
| First name | Akihiko |
| Last name | Hirose |
| Address | 1-18-1 Kamiyoga |
| Address | Setagaya-ku |
| Postal code | 158-8501 |
| Town | Tokyo |
| Country | Japan |