SIDS Dossier

OECD HPV Chemical Programme, SIDS Dossier, approved at SIAM 32 (19/04/2011)

| Printing Date | 2012-10-02 14:56:40 EDT |
| --- | --- |

Restriction of specific regulatory purposes

EU: REACH, other:

Confidentiality

| Name | 75-31-0_LEAD_master_isopropylamine |
| --- | --- |
| Legal entity owner | BASF SE |

Substance: 75-31-0_LEAD_master_isopropylamine

| UUID |  | IUC5-5fa2bc19-5a90-41bd-94b0-a4b75fa82881 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | Collette / Epona Associates, LLC / Willington / United States |
| Date |  | 2012-08-13 17:13:05 EDT |
| Remarks |  |  |

0 Related Information

0.1 Templates

0.2 Categories

0.3 Mixtures

1 General Information

1.1 Identification

Substance identification

| Chemical name | 75-31-0_LEAD_master_isopropylamine |
| --- | --- |
| Public name | Isopropylamine |
| EU: REACH |  |
| Legal entity | BASF SE |
| EU: REACH |  |

Role in the supply chain

| EU: REACH |  |
| --- | --- |
| Role: | Manufacturer; Importer |

Reference substance

isopropylamine

EC number; EC name. 200-860-9; isopropylamine. CAS number; CAS name. IUPAC name.

Type of substance

| Composition | mono constituent substance |
| --- | --- |
| Origin | organic |

Trade names

| EU: REACH |  |
| --- | --- |
| Name | Monoisopropylamin |
| Name | 2-Propanamine (9CI) |
| Name | 2-Aminopropane |
| Name | 2-Propylamine |
| Name | Monoisopropylamine |
| Name | sec-Propylamine |
| Name | 1-Methylethylamine |
| Name | Isopropylamine (8CI) |

Contact person

| Person flags | EU: REACH |
| --- | --- |

1.2 Composition

1.3 Identifiers

1.4 Analytical information

1.5 Joint submission

1.6 Sponsors

1.7 Suppliers

1.8 Recipients

1.9 Product and process oriented research and development

2 Classification and Labelling

2.1 GHS

2.2 DSD - DPD

3 Manufacture, use and exposure

3.1 Technological process

3.2 Estimated quantities

3.3 Sites

3.4 Form in the supply chain

3.5 Identified uses

3.6 Uses advised against

3.7 Waste from production and use

3.8 Exposure estimates

3.9 Biocidal information

3.10 Application for authorisation of uses

4 Physical and chemical properties

4.1 Appearance/physical state/colour

Endpoint summary: Appearance/physical state/colour

| UUID |  | IUC5-bbeb04e5-0ad0-42f6-8292-97afdf67b3c1 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:57:54 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

EU: REACH

Short description of key information

Liquid

Key value for chemical safety assessment

Physical state at 20ºC and 1013 hPa

liquid

Endpoint study record: OXEA Company Data (+2010 Appearance RL2)

| UUID |  | IUC5-0e3ce6b0-c870-4f06-be22-3a41a9fc04e7 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-12-30 09:48:11 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Purpose flag | key study; robust study summary; used for classification; used for MSDS |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Comparable to guideline study with acceptable restrictions (e.g. non-GLP) |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| other company data | Nicole Müller | 2010 | Analysenbericht REACH i-Propylamin CAS 75-31-0 |  | Analytik und Produktkontrolle (internal) | 20081104151000 | OXEA | T10243 | 2010-02-09 |

Data access

data submitter is data owner

Data protection claimed

yes, but willing to share

Materials and methods

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | other guideline: RCH/AL 287 Visuelle Beurteilung von Flüssigkeiten |  |

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |
| EC name | Isopropylamine |
| EC number | 200-860-9 |

Details on test material

- Analytical purity (GC): 99,94% [m/m] - Source: Tank 111 - Date of sampling: 2009-03-09 - Lot/LIMS-ID: 20081104151000 - Purity test date: 02. March 2009

Results and discussion

Physical state at 20ºC and 1013 hPa

liquid

Colour

Colourless, Clear

Odour

ammonia-like

Substance type

organic

Applicant's summary and conclusion

Conclusions

Substance is a clear, colourless organic liquid with ammonia like odour

Executive summary

Substance is a clear, colourless organic liquid with ammonia like odour

4.2 Melting point/freezing point

Endpoint summary: Melting point/freezing point

| UUID |  | IUC5-a185266b-c8df-42f2-aaab-e44fc768f3a0 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:57:54 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

EU: REACH

Short description of key information

<-90 °C

Key value for chemical safety assessment

| Melting / freezing point at 101 325 Pa | -90 °C |
| --- | --- |

Endpoint study record: OXEA Company Data (+2010 PourPoint RL2)

| UUID |  | IUC5-988121af-0342-4ebc-8676-f6dea9ea29e9 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-12-30 09:48:19 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Purpose flag | key study; robust study summary; used for classification; used for MSDS |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Comparable to guideline study with acceptable restrictions (e.g. non-GLP) |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| other company data | Nicole Müller | 2010 | Analysenbericht REACH i-Propylamin CAS 75-31-0 |  | Analytik und Produktkontrolle (internal) | 20081104151000 | OXEA | T10243 | 2010-02-09 |

Data access

data submitter is data owner

Data protection claimed

yes, but willing to share

Materials and methods

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| equivalent or similar to | other guideline: DIN ISO 3016 / ASTM D97 |  |

Type of method

pour point

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |
| EC name | Isopropylamine |
| EC number | 200-860-9 |

Details on test material

- Analytical purity (GC): 99,94% [m/m] - Source: Tank 111 - Date of sampling: 2009-03-09 - Lot/LIMS-ID: 20081104151000 - Purity test date: 02. March 2009

Results and discussions

Melting / freezing point

| Melt./Freez. pt. | Atm. pressure | Decomposition | Decomp. temp. | Sublimation | Subl. temp. | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| < -90 °C | 1013 hPa |  |  |  |  |  |

Applicant's summary and conclusion

Conclusions

Melting point is below -20° C.

Executive summary

Endpoint could not be determined, enpoint of instrument reached, value is lower -90 °C.

4.3 Boiling point

Endpoint summary: Boiling point

| UUID |  | IUC5-3ba41d1e-a536-41ad-9f06-c1af07700441 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:57:55 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

EU: REACH

Short description of key information

32 °C

Key value for chemical safety assessment

| Boiling point at 101 325 Pa | 32 °C |
| --- | --- |

Endpoint study record: OXEA Company Data (+2010 Boiling point RL2)

| UUID |  | IUC5-3cc1ad19-f765-4ce1-996a-5c8771b0d1a9 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-12-30 09:48:32 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Purpose flag | key study; robust study summary; used for classification; used for MSDS |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Comparable to guideline study with acceptable restrictions (e.g. non-GLP) |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| other company data | Nicole Müller | 2010 | Analysenbericht REACH i-Propylamin CAS 75-31-0 |  | Analytik und Produktkontrolle (internal) | 20081104151000 | OXEA | T10243 | 2010-02-09 |

Data access

data submitter is data owner

Data protection claimed

yes, but willing to share

Materials and methods

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| equivalent or similar to | OECD Guideline 103 (Boiling point/boiling range) |  |

Type of method

photocell detection

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |
| EC name | Isopropylamine |
| EC number | 200-860-9 |

Details on test material

- Analytical purity (GC): 99,94% [m/m] - Source: Tank 111 - Date of sampling: 2009-03-09 - Lot/LIMS-ID: 20081104151000 - Purity test date: 02. March 2009

Results and discussions

Boiling point

| Boiling pt. | Atm. pressure | Decomposition | Decomp. temp. | Remarks |
| --- | --- | --- | --- | --- |
| 32 °C | 1013 hPa |  |  |  |

Applicant's summary and conclusion

Conclusions

Boiling point is 305 K.

Executive summary

Mean of two measurements corrected to 1013 hPa.

Endpoint study record: BASF AG BRU 67.12.Boiling point

| UUID |  | IUC5-7973350d-14a6-4194-a8a9-4d8af537c914 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-12-30 09:48:24 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Purpose flag | supporting study |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result |  |  |
| Reliability | 4 (not assignable) |  |  |
| Rationale for reliability incl. deficiencies | Scientifically verified data without data for analytical purity |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | BASF AG | 1967 | Dampfdruck von Monoisopropylamin von 30 - 150 °C |  | BASF AG, Ammoniaklaboratotium | BRU 67.12 | BASF SE, D-67056 Ludwigshafen |  | 1967-05-17 |

Data access

data submitter has Letter of Access

Materials and methods

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | other guideline: no data |  |

Principles of method if other than guideline

other: measured

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |
| EC number | 200-860-9 |
| IUPAC name | propan-2-amine |

Results and discussions

Boiling point

| Boiling pt. | Atm. pressure | Decomposition | Decomp. temp. | Remarks |
| --- | --- | --- | --- | --- |
| 31 °C | 1013.25 hPa |  |  | extrapolated |

4.4 Density

Endpoint summary: Density

| UUID |  | IUC5-691f190a-b7c0-48e9-953c-e1237f7aaa25 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:57:56 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

EU: REACH

Short description of key information

687.1 kg/m3

Key value for chemical safety assessment

Relative density at 20°C

687.1

Endpoint study record: OXEA Company Data (+2010 Density RL2)

| UUID |  | IUC5-bd83e98f-2a5b-43a7-80da-5512408b0a9f |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-12-30 09:48:51 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Purpose flag | key study; robust study summary; used for classification; used for MSDS |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Comparable to guideline study with acceptable restrictions (e.g. non-GLP) |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| other company data | Nicole Müller | 2010 | Analysenbericht REACH i-Propylamin CAS 75-31-0 |  | Analytik und Produktkontrolle (internal) | 20081104151000 | OXEA | T10243 | 2010-02-09 |

Data access

data submitter is data owner

Data protection claimed

yes, but willing to share

Materials and methods

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| equivalent or similar to | other guideline: DIN 51757 / ASTM D 4052 |  |

Type of method

oscillating densitimeter (Anton Paar DMA 4500)

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |
| EC name | Isopropylamine |
| EC number | 200-860-9 |

Details on test material

- Analytical purity (GC): 99,94% [m/m] - Source: Tank 111 - Date of sampling: 2009-03-09 - Lot/LIMS-ID: 20081104151000 - Purity test date: 02. March 2009

Results and discussion

Density

| Type | Density | Temp. | Remarks |
| --- | --- | --- | --- |
| density | 687.1 kg/m³ | 20 °C |  |

Applicant's summary and conclusion

Conclusions

Density is 687,1 kg/m³ at 20°C

Executive summary

Density is 687,1 kg/m³ at 20°C , liquid, purity is 99,94 %[m/m]. Measured with oscillating densitimeter, referenced to water.

4.5 Particle size distribution (Granulometry)

Endpoint summary: Particle size distribution (Granulometry)

| UUID |  | IUC5-91c5e17c-5d32-44b1-aa25-82866e3d4e72 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:58:14 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

EU: REACH

Short description of key information

not applicable

Discussion

Substance is marketed or used in a non solid or granular form.

Endpoint study record: OXEA Company Data (+2008 Waiver Granulometry RL1)

| UUID |  | IUC5-16f5daf6-ef8a-4437-82d0-66242cdc0558 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:52:05 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| EU: REACH |  |
| --- | --- |
| Purpose flag | used for classification; used for MSDS |
| Data waiving | other justification |
| Justification for data waiving | In accordance with column 2 of REACH annex VII, the study does not need to be conducted if the substance is marketed or used in a non solid or granular form” |

Materials and methods

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Applicant's summary and conclusion

Conclusions

study scientifically unjustified

4.6 Vapour pressure

Endpoint summary: Vapour pressure

| UUID |  | IUC5-23962b12-e1a7-4673-a567-d8dc6cf27f4b |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:57:56 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

EU: REACH

Short description of key information

631 hPa

Key value for chemical safety assessment

| Vapour pressure | 631 hPa | at the temperature of | 20 °C |
| --- | --- | --- | --- |

Endpoint study record: OXEA Company Data (+2010 Dampfdruck RL2)

| UUID |  | IUC5-dd762a1f-8d5e-43f3-8821-6b1d64c25d39 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-12-30 09:49:06 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Purpose flag | key study; robust study summary; used for classification |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Comparable to guideline study with acceptable restrictions (e.g. non-GLP) |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| other company data | Nicole Müller | 2010 | Analysenbericht REACH i-Propylamin CAS 75-31-0 |  | Analytik und Produktkontrolle (internal) | 20081104151000 | OXEA | T10243 | 2010-02-09 |

Data access

data submitter is data owner

Data protection claimed

yes, but willing to share

Materials and methods

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | other guideline: DIN-EN 13016-2 |  |

Type of method

static method MiniVap (Fa. Grabner-Instruments)

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |
| EC name | Isopropylamine |
| EC number | 200-860-9 |

Details on test material

- Analytical purity (GC): 99,94% [m/m] - Source: Tank 111 - Date of sampling: 2009-03-09 - Lot/LIMS-ID: 20081104151000 - Purity test date: 02. March 2009

Results and discussions

Vapour pressure

631 hPa

20 °C

Applicant's summary and conclusion

Conclusions

Vapour pressure = 631 hPa at 20° C.

Executive summary

Good fit to data from DIPPR Database

Endpoint study record: BASF AG BRU 67.12.Vapour pressure

| UUID |  | IUC5-55ad8723-9433-46ef-9554-97ba116999be |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-12-30 09:48:57 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Purpose flag | supporting study |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result |  |  |
| Reliability | 4 (not assignable) |  |  |
| Rationale for reliability incl. deficiencies | Scientifically verified data without data for analytical purity |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | BASF AG | 1967 | Dampfdruck von Monoisopropylamin von 30 - 150 °C |  | BASF AG, Ammoniaklaboratorium | BRU 67.12 | BASF SE, D-67056 Ludwigshafen |  | 1967-05-17 |

Data access

data submitter has Letter of Access

Materials and methods

Principles of method if other than guideline

other (measured)

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Details on test material

Name of test material (as cited in study report): isopropylamine Analytical purity: no data

Results and discussions

Vapour pressure

1038 hPa

31.5 °C

Any other information on results incl. tables

VP = 1038hPa at 31.5°C ,1180hPa at 35°C, 1448hPa at 41°C, 1738hPa at 46.5 °C ....

4.7 Partition coefficient

Endpoint summary: Partition coefficient

| UUID |  | IUC5-60e077e7-40fc-4f46-9102-beea5b6de84c |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:57:57 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

EU: REACH

Short description of key information

Log Pow -0.5

Key value for chemical safety assessment

| Log Kow (Log Pow) | -0.5 | at the temperature of | 20 °C |
| --- | --- | --- | --- |

Endpoint study record: OXEA Company Data (+2009 Partition coefficient RL1)

| UUID |  | IUC5-5f28b3a5-6af2-49bf-bac7-23b53351d737 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-12-30 09:49:13 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Purpose flag | key study; robust study summary; used for classification; used for MSDS |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result | Study period | Study initiation date: 29 June 2009 Study completion date: 19 October 2009 |
| Reliability | 1 (reliable without restriction) |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | R. Enste-Diefenbach | 2009 | FINAL REPORT Determination of the Physical-chemical Properties of Isopropylamine |  | AQura GmbH Gebäude 145, PB 15 Paul-Baumann-Straße 1, D-45772 Marl, Germany | AN-ASB 0439 | OXEA | T10202 | 2009-10-18 |

Data access

data submitter is data owner

Data protection claimed

yes, but willing to share

Materials and methods

Partition coefficient type

octanol-water

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | OECD Guideline 117 (Partition Coefficient (n-octanol / water), HPLC Method) |  |
| according to | EU Method A.8 (Partition Coefficient) |  |

Type of method

HPLC method

GLP compliance

yes

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Results and discussions

Partition coefficient

| Type | Partition coefficient | Temp. | pH | Remarks |
| --- | --- | --- | --- | --- |
| log Pow | -0.5 | 25 °C | 7.5 |  |
| Pow | 0.3 | 25 °C | 7.5 |  |

Applicant's summary and conclusion

Conclusions

Partition coefficient n-octanol / water (Pow): Pow =0,3 / log Pow = -0,5

Executive summary

The partition coefficient n-octanol / water (Pow) of the test item was determined according to OECD guideline no. 117 [adopted on 13 April 2004] and EU test method A.8 [Directive 92/69/EEC, Official Journal L 383 A 1992] with the HPLC method. Partition coefficient n-octanol / water (Pow): Pow = 0,3 / log Pow = -0,5

4.8 Water solubility

Endpoint summary: Water solubility

| UUID |  | IUC5-fda93b56-d59b-4d70-9a25-f3db2d886a2d |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | Collette / Epona Associates, LLC / Willington / United States |
| Date |  | 2012-05-29 11:21:12 EDT |
| Remarks |  |  |

Administrative Data

EU: REACH

Short description of key information

miscible in all proportions

Key value for chemical safety assessment

| Water solubility | 421.8 g/L | at the temperature of | 20 °C |
| --- | --- | --- | --- |

Discussion

Miscible in all proportions. The key value for chemical safety assesment was estimated as the content of a 50% solution isopropylamine and water ((density of isopropylamine + density water)/4). It was calculated based on the density of water and isopropylamine. The 50% level was chosen as at higher isopropylamine concentration it is the solubility of water in isopropylamine.

Endpoint study record: OXEA Company Data (+2009 Water solubility RL1)

| UUID |  | IUC5-7259f3ef-6b00-43dd-bd68-9fea9250551b |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | Collette / Epona Associates, LLC / Willington / United States |
| Date |  | 2012-05-29 11:24:08 EDT |
| Remarks |  |  |

Administrative Data

| Purpose flag | key study; robust study summary; used for classification; used for MSDS |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result | Study period | Study initiation date: 29 June 2009 Study completion date: 19 October 2009 |
| Reliability | 1 (reliable without restriction) |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | R. Enste-Diefenbach | 2009 | FINAL REPORT Determination of the Physical-chemical Properties of Isopropylamine |  | AQura GmbH Gebäude 145, PB 15 Paul-Baumann-Straße 1, D-45772 Marl, Germany | AN-ASB 0439 | OXEA | T10202 | 2009-10-18 |

Data access

data submitter is data owner

Data protection claimed

yes, but willing to share

Materials and methods

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| equivalent or similar to | OECD Guideline 105 (Water Solubility) |  |
| equivalent or similar to | EU Method A.6 (Water Solubility) |  |

Type of method

flask method

GLP compliance

yes

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Results and discussions

Water solubility

| > 50 [m/m] |  |
| --- | --- |
| Temp. | 20 °C |
| pH | 14 |
| Remarks | 421 g/L: estimated as the content of a 50% solution isopropylamine and water ((density of isopropylamine + density water)/4). It was calculated based on the density of water and isopropylamine. |

Details on results

The water solubility of the test item was determined according to OECD guideline no. 105 [adopted on 27 July 1995] and EU test method A.6 [Directive 92/69/EEC, Official Journal L 383 A 1992] with the modified flask method.

Applicant's summary and conclusion

Interpretation of results

miscible

Conclusions

The test item is miscible in a 1 : 1 ratio mixture (w/w) with water (at 20°C).

Executive summary

The water solubility of the test item was determined according to OECD guideline no. 105 [adopted on 27 July 1995] and EU test method A.6 [Directive 92/69/EEC, Official Journal L 383 A 1992] with the modified flask method. The test item is miscible in a 1 : 1 ratio mixture (w/w) with water (at 20°C).

Endpoint study record: Stephenson (+1993, J.Chem.Eng.Data, RL2)

| UUID |  | IUC5-a83238ac-5b01-4c08-853e-2bc5dfb49a29 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-12-30 09:49:35 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Purpose flag | supporting study |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | detailed representation |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | Richard M. Stephenson | 1993 | Mutual Solubility of Water and Aliphatic Amines | J. Chem. Eng. Data 1993,38,625-629 | Department of Chemical Engineering, Box U .. 222, University of Connecticut, Storrs, Connecticut 06269 |  |  | T04957 |  |

Data access

data published

Materials and methods

Type of method

analysis (GC)

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Results and discussions

Water solubility

| miscible in all proportions, |  |
| --- | --- |
| Temp. | 20 °C |

Applicant's summary and conclusion

Interpretation of results

miscible

Conclusions

miscible in all proportions,

Executive summary

Substance is miscible in all proportions with water

4.10 Surface tension

Endpoint summary: Surface tension

| UUID |  | IUC5-10e6bf58-ca4e-4003-b761-4d15cfe7efd7 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:57:58 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

EU: REACH

Short description of key information

68.5 mN/m 5 at 20 °C and 1 g/L concentration

Key value for chemical safety assessment

| Surface tension | 68.5 | in mN/m at 20°C and concentration in mg/L | 1000 |
| --- | --- | --- | --- |

Endpoint study record: OXEA Company Data (+2009 Surface Tension RL1)

| UUID |  | IUC5-11c5ee8a-29ae-43be-bd74-6804c55f86fc |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:51:30 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| EU: REACH |  |  |  |
| --- | --- | --- | --- |
| Purpose flag | key study; robust study summary; used for classification; used for MSDS |  |  |
| Study result type | experimental result | Study period | Study initiation date: 29 June 2009 Study completion date: 19 October 2009 |
| Reliability | 1 (reliable without restriction) |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | R. Enste-Diefenbach | 2009 | FINAL REPORT Determination of the Physical-chemical Properties of Isopropylamine |  | AQura GmbH Gebäude 145, PB 15 Paul-Baumann-Straße 1, D-45772 Marl, Germany | AN-ASB 0439 | OXEA | T10202 | 2009-10-18 |

Data access

data submitter is data owner

Data protection claimed

yes, but willing to share

Materials and methods

Type of method

ring method

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | OECD Guideline 115 (Surface Tension of Aqueous Solutions) |  |
| according to | EU Method A.5 (Surface Tension) |  |

GLP compliance

yes

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Results and discussions

Surface tension

| 68.5 mN/m |  |
| --- | --- |
| Temp. | 20 °C |
| Concentration | 1 g/L |

Applicant's summary and conclusion

Conclusions

Surface tension (at 20°C): 68,5 mN/m.

Executive summary

The surface tension of aqueous solutions of the test item was determined according to OECD guideline no. 115 [adopted on 27 July 1995] and EU test method A.5 [Directive 92/69/EEC, Official Journal L 383 A 1992] with the ring method. Surface tension (at 20°C): 68,5 mN/m

4.11 Flash point

Endpoint summary: Flash point

| UUID |  | IUC5-c1bac532-88ba-4945-badb-c6302ce54370 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:57:58 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

EU: REACH

Short description of key information

<-30 °C

Key value for chemical safety assessment

| Flash point at 101 325 Pa | -30 °C |
| --- | --- |

Endpoint study record: BASF AG 73/0114.Flash point

| UUID |  | IUC5-bb303ea5-2908-4863-9fc4-f6f17796fc34 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:51:20 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| EU: REACH |  |  |  |
| --- | --- | --- | --- |
| Purpose flag | weight of evidence; robust study summary; used for MSDS |  |  |
| Study result type | experimental result |  |  |
| Reliability | 4 (not assignable) |  |  |
| Rationale for reliability incl. deficiencies | Scientifically verified data without data for analytical purity |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | BASF AG, | 1973 |  |  | BASF AG, Department of Safety Engineering | TLM-SIK 73.0114 | BASF SE |  | 1973-05-10 |

Data access

data submitter has Letter of Access

Materials and methods

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| equivalent or similar to | ISO No., other: Abel-Pensky |  |

Type of method

closed cup

Principles of method if other than guideline

Method: other: Abel-Pensky

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |

Details on test material

- Name of test material (as cited in study report): isopropylamine - Analytical purity: purity no further data

Results and discussions

Flash point

| < -30 °C |  |
| --- | --- |
| at | 1013 hPa |

Endpoint study record: OXEA Company Data (+2010 Flammpunkt RL2)

| UUID |  | IUC5-5c7115e9-af86-48f7-9cba-d0ac60a1a17c |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:51:31 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| EU: REACH |  |  |  |
| --- | --- | --- | --- |
| Purpose flag | weight of evidence; robust study summary; used for classification; used for MSDS |  |  |
| Study result type | experimental result |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Comparable to guideline study with acceptable restrictions (e.g. non-GLP) |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| other company data | Nicole Müller | 2010 | Analysenbericht REACH i-Propylamin CAS 75-31-0 |  | Analytik und Produktkontrolle (internal) | 20081104151000 | OXEA | T10243 | 2010-02-09 |
| study report | R. Enste-Diefenbach | 2009 | FINAL REPORT Determination of the Physical-chemical Properties of Isopropylamine |  | AQura GmbH Gebäude 145, PB 15 Paul-Baumann-Straße 1, D-45772 Marl, Germany | AN-ASB 0439 | OXEA | T10202 | 2009-10-18 |

Data access

data submitter is data owner

Data protection claimed

yes, but willing to share

Materials and methods

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| equivalent or similar to | ISO 2719:2002 (Determination of flash point - Pensky-Martens closed cup method) |  |

Type of method

closed cup

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |
| EC name | Isopropylamine |
| EC number | 200-860-9 |

Details on test material

- Analytical purity (GC): 99,94% [m/m] - Source: Tank 111 - Date of sampling: 2009-03-09 - Lot/LIMS-ID: 20081104151000 - Purity test date: 02. March 2009

Results and discussions

Flash point

| <= -25 °C |  |
| --- | --- |
| at | 1013 hPa |

Applicant's summary and conclusion

Conclusions

Result is below detection limit of Instrument ( <= 20 °C)

Executive summary

Result is below detection limit of Instrument ( <= 20 °C)

4.12 Auto flammability

Endpoint summary: Auto flammability

| UUID |  | IUC5-6aa7059e-fda9-409b-af8d-1eabe2e6f7cb |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:57:59 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

EU: REACH

Short description of key information

355 °C

Key value for chemical safety assessment

| Autoflammability / Self-ignition temperature at 101 325 Pa | 355 °C |
| --- | --- |

Endpoint study record: BASF AG 73/0114.Auto flammability

| UUID |  | IUC5-f504f486-dc8f-4304-9ba2-c17d802a77cc |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:51:20 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| EU: REACH |  |  |  |
| --- | --- | --- | --- |
| Purpose flag | supporting study; used for MSDS |  |  |
| Study result type | experimental result |  |  |
| Reliability | 4 (not assignable) |  |  |
| Rationale for reliability incl. deficiencies | Scientifically verified data without data for analytical purity |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | BASF AG | 1973 |  |  | BASF AG, Department of Safety Engineering | TLM-SIK 73/0114 | BASF SE |  | 1973-05-10 |

Data access

data submitter has Letter of Access

Materials and methods

Principles of method if other than guideline

Method: other: DIN 51 794

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |

Details on test material

- Name of test material (as cited in study report): isopropylamine - Analytical purity: purity no further data

Results and discussions

Autoflammability / Self-ignition temperature

| 320 °C |  |
| --- | --- |
| at | 1013 hPa |

Endpoint study record: OXEA Company Data (+2009 Auto ignition RL1)

| UUID |  | IUC5-79f2c39a-2270-425b-a274-49b58005fb14 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:51:31 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| EU: REACH |  |  |  |
| --- | --- | --- | --- |
| Purpose flag | key study; robust study summary; used for classification; used for MSDS |  |  |
| Study result type | experimental result | Study period | Study initiation date: 29 June 2009 Study completion date: 19 October 2009 |
| Reliability | 1 (reliable without restriction) |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | R. Enste-Diefenbach | 2009 | FINAL REPORT Determination of the Physical-chemical Properties of Isopropylamine |  | AQura GmbH Gebäude 145, PB 15 Paul-Baumann-Straße 1, D-45772 Marl, Germany | AN-ASB 0439 | OXEA | T10202 | 2009-10-18 |

Data access

data submitter is data owner

Data protection claimed

yes, but willing to share

Materials and methods

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | EU Method A.15 (Auto-Ignition Temperature (Liquids and Gases)) |  |
| according to | other guideline: DIN 51 794 (covers a temperature range from 75 to 650 °C) |  |

GLP compliance

yes

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Results and discussions

Autoflammability / Self-ignition temperature

| 355 °C |  |
| --- | --- |
| at | 1016 hPa |

Applicant's summary and conclusion

Conclusions

Auto-ignition temperature = 355 °C

Executive summary

The auto-ignition temperature of the test item was determined according to EU test method A.15 [Directive 92/69/EEC, Official Journal L 383 A 1992] and DIN 51 794. Auto-ignition temperature: (1016 hPa) 355 °C

4.13 Flammability

Endpoint summary: Flammability

| UUID |  | IUC5-8e1ea003-a57e-4a15-833c-705160a48e24 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:58:15 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

EU: REACH

Short description of key information

Highly flammable The substance has no pyrophoric properties and does not liberate flammable gases on contact with water.

Key value for chemical safety assessment

| Flammability | highly flammable |
| --- | --- |

Discussion

Flammability derived from flash point. Based on chemical structure pyrophoric properties and flammability in contact with water are not predicted.

Endpoint study record: OXEA Compana Data (+2009 Waiver Flammability liquid RL1)

| UUID |  | IUC5-a17d5146-6dc4-47de-84b4-a4ff3975abd1 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:52:05 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| EU: REACH |  |
| --- | --- |
| Purpose flag | used for classification; used for MSDS |
| Data waiving | other justification |
| Justification for data waiving | In accordance with chapter R.7.1.10 of REACH guidance 3.2 R7a the flammability of a liquid is described by flash point and boiling point. |

Endpoint study record: OXEA Compana Data (+2009 Waiver Flammability pyrophoric & water contact RL1)

| UUID |  | IUC5-147be862-050a-47be-bd78-79e98dcb24d0 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:52:06 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| EU: REACH |  |
| --- | --- |
| Purpose flag | used for classification; used for MSDS |
| Data waiving | other justification |
| Justification for data waiving | In accordance with chapter R.7.1.10.4 of REACH guidance 3.2 R7a the absence of structural alerts will mean that testing is not necessary for pyrophoric properties and flammability of contact with water. This is the case for this substance. |

4.14 Explosiveness

Endpoint summary: Explosiveness

| UUID |  | IUC5-568ed58f-7773-4757-a4e0-3ac84231be05 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:58:16 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

EU: REACH

Short description of key information

non explosive

Discussion

There are no chemical groups associated with explosive properties present in the molecule.

Endpoint study record: OXEA Company Data (+2008 Waiver Explosiveness RL1)

| UUID |  | IUC5-150374d2-865b-4505-af9c-82761ffd96e2 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:52:06 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| EU: REACH |  |
| --- | --- |
| Purpose flag | used for classification; used for MSDS |
| Data waiving | other justification |
| Justification for data waiving | Justification for data waiving: In accordance with column 2 of REACH annex VII, the study does not need to be conducted as there are no chemical groups associated with explosive properties present in the molecule. |

Materials and methods

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Applicant's summary and conclusion

Interpretation of results

non explosive

Conclusions

study scientifically unjustified

4.15 Oxidising properties

Endpoint summary: Oxidising properties

| UUID |  | IUC5-e5876cb3-f740-47d5-845d-cf8a579d882f |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:58:16 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

EU: REACH

Short description of key information

no oxidising properties

Discussion

The Substance is incapable of reacting exothermically with combustible materials on the basis of the chemical structure.

Endpoint study record: OXEA Compana Data (+2008 Waiver Oxidising RL1)

| UUID |  | IUC5-4be0dfb8-5c93-4afb-9e3c-5081360afe8f |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:52:07 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| EU: REACH |  |
| --- | --- |
| Purpose flag | used for classification; used for MSDS |
| Data waiving | other justification |
| Justification for data waiving | Justification for data waiving: In accordance with column 2 of REACH annex VII, the study does not need to be conducted as the substance is highly flammable |

Materials and methods

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Applicant's summary and conclusion

Interpretation of results

no oxidising properties

Conclusions

study scientifically unjustified

4.17 Stability in organic solvents and identity of relevant degradation products

Endpoint summary: Stability in organic solvents and identity of relevant degradation products

| UUID |  | IUC5-587351f2-27df-4f05-9e1c-4776f9366a76 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:58:00 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

EU: REACH

Short description of key information

not applicable

Discussion

The stability of the substance is not considered as critical.

Endpoint study record: OXEA Company Data (+2008 Waiver Stability RL1)

| UUID |  | IUC5-ecdc6138-8047-4801-90e7-c39a64cb85ff |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:51:32 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| EU: REACH |  |  |  |
| --- | --- | --- | --- |
| Purpose flag | used for classification; used for MSDS |  |  |
| Data waiving | other justification |  |  |
| Justification for data waiving | Justification for data waiving: In accordance with column 1 of REACH annex IX, the study does not need to be conducted as the stability of the substance is not considered to be critical |  |  |
| Study result type | other: expert statement |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| other: expert statement | Michael Messerschmidt | 2008 | Evaluation Stability Alkylamines.doc |  | Analytik und Produktkontrolle (internal) |  | OXEA | T10020 | 2008-12-01 |

Data access

data submitter is data owner

Data protection claimed

yes, but willing to share

Materials and methods

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Applicant's summary and conclusion

Conclusions

study scientifically unjustified

Executive summary

The substance is stable in common organic solvents; solvents containing a carbonyl moiety may lead to imines formation with primary amines.

4.20 pH

Endpoint study record: OXEA Company Data (+2010 pH RL2)

| UUID |  | IUC5-e7aab334-2bde-4415-bc32-ea61af45da20 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:51:32 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| EU: REACH |  |  |  |
| --- | --- | --- | --- |
| Purpose flag | key study; robust study summary; used for classification; used for MSDS |  |  |
| Study result type | experimental result |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Comparable to guideline study with acceptable restrictions (e.g. non-GLP) |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| other company data | Nicole Müller | 2010 | Analysenbericht REACH i-Propylamin CAS 75-31-0 |  | Analytik und Produktkontrolle (internal) | 20081104151000 | OXEA | T10243 | 2010-02-09 |

Data access

data submitter is data owner

Data protection claimed

yes, but willing to share

Materials and methods

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | other guideline: DIN 19268 |  |

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |
| EC name | Isopropylamine |
| EC number | 200-860-9 |

Details on test material

- Analytical purity (GC): 99,94% [m/m] - Source: Tank 111 - Date of sampling: 2009-03-09 - Lot/LIMS-ID: 20081104151000 - Purity test date: 02. March 2009

Results and discussions

pH value

| pH value | Temperature | Concentration | Remarks |
| --- | --- | --- | --- |
| 13.1 | 25 °C | ca. 50 vol% |  |

Applicant's summary and conclusion

Conclusions

pH = 13,1

Executive summary

Mean of two measurements

4.21 Dissociation constant

Endpoint summary: Dissociation constant

| UUID |  | IUC5-70bbb346-b82c-490c-b001-587a71a35927 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:58:00 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

Short description of key information

pKa = 10.8 at 23.2 °C

Key value for chemical safety assessment

| pKa at 20°C | 10.8 |
| --- | --- |

Endpoint study record: OXEA Company Data (+2009 Dissociation constant RL1)

| UUID |  | IUC5-eba0907b-6fa3-4c8e-b97b-67cf49036b96 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-12-30 09:49:48 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Purpose flag | key study; robust study summary; used for classification; used for MSDS |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result | Study period | Study initiation date: 29 June 2009 Study completion date: 19 October 2009 |
| Reliability | 1 (reliable without restriction) |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | R. Enste-Diefenbach | 2009 | FINAL REPORT Determination of the Physical-chemical Properties of Isopropylamine |  | AQura GmbH Gebäude 145, PB 15 Paul-Baumann-Straße 1, D-45772 Marl, Germany | AN-ASB 0439 | OXEA | T10202 | 2009-10-18 |

Data access

data submitter is data owner

Data protection claimed

yes, but willing to share

Materials and methods

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | OECD Guideline 112 (Dissociation Constants in Water) |  |

GLP compliance

yes

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Results and discussions

Dissociating properties

yes

Dissociation constant

| No. | pKa | at | Remarks |
| --- | --- | --- | --- |
| #1 | 10.8 | 23.5 °C |  |

Applicant's summary and conclusion

Conclusions

Dissociation constant (at 23.5 °C): pKB = 10.8

Executive summary

The dissociation constant of the test item was determined according to OECD guideline no. 112 [adopted on 12 May 1981] with the titration method. Dissociation constant (at 23.5 °C): pKB = 10.8

4.22 Viscosity

Endpoint summary: Viscosity

| UUID |  | IUC5-f3132514-f98f-4d42-b193-160f17b968a3 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:58:01 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

EU: REACH

Short description of key information

0.47 mm²/s (static)

Key value for chemical safety assessment

| Viscosity at 20°C | 0.47 mm²/s (static) |
| --- | --- |

Endpoint study record: OXEA Company Data (+2009 Viscosity RL1)

| UUID |  | IUC5-70376a60-cba0-46ff-a1b0-36043b5b6acf |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:51:33 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| EU: REACH |  |  |  |
| --- | --- | --- | --- |
| Purpose flag | key study; robust study summary; used for classification; used for MSDS |  |  |
| Study result type | experimental result | Study period | Study initiation date: 29 June 2009 Study completion date: 19 October 2009 |
| Reliability | 1 (reliable without restriction) |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | R. Enste-Diefenbach | 2009 | FINAL REPORT Determination of the Physical-chemical Properties of Isopropylamine |  | AQura GmbH Gebäude 145, PB 15 Paul-Baumann-Straße 1, D-45772 Marl, Germany | AN-ASB 0439 | OXEA | T10202 | 2009-10-18 |

Data access

data submitter is data owner

Data protection claimed

yes, but willing to share

Materials and methods

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | OECD Test Guideline 114 (Viscosity of Liquids) |  |

Type of method

capillary method

Principles of method if other than guideline

The determination of the kinematic viscosity was carried out by the capillary method with a viscosimeter according to Ubbelohde.

GLP compliance

yes

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Results and discussions

Viscosity

| 0.47 mm²/s (static) |  |
| --- | --- |
| Temp. | 20 °C |

Applicant's summary and conclusion

Conclusions

Kinematic viscosity at 20°C [mm^2/ s] 0,47

Executive summary

The kinematic viscosity of the test item was determined according to OECD guideline no. 114 [adopted on 12 May 1981] with the capillary viscosimeter method. Kinematic viscosity at 20°C [mm^2/s] 0,47

4.23 Additional physico-chemical information

Endpoint study record: BASF AG73/0114.Additional physico-chemical information

| UUID |  | IUC5-1ad8c8b3-391a-4804-b72a-a79632a7364a |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:51:21 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| EU: REACH |  |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result |  |  |
| Reliability | 4 (not assignable) |  |  |
| Rationale for reliability incl. deficiencies | Scientifically verified data |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BASF |  | BASF AG, Department of Productsafety, Sicherheitstechnische Kennzahlen |  | BASF AG | TLM-SIK 73/0114 | BASF SE |  | 1973-05-10 |

Data access

data submitter has Letter of Access

Data protection claimed

yes

Materials and methods

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |

Details on test material

- Name of test material (as cited in study report): isopropylamine - Analytical purity: purity no further data

Results and discussions

Results

Memo: Explosive Limits

Any other information on results incl. tables

lower limit:4.2% - upper limit: 13.9 vol.%

5 Environmental fate and pathways

Endpoint summary: Environmental fate and pathways

| UUID |  | IUC5-ae01435d-1350-4c6d-9fb9-d6f4af996a5c |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:58:02 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

EU: REACH

Discussion

Summary "Environmental fate and pathways" Isopropylamine has no intrinsic persistent or bioaccumulating properties: In valid standard ready biodegradability studies, 70 - 80 % (COD) and 95 % (DOC) of test substance has been biodegraded (BASF 1997) (Arkema 1992).Although test conditions were according to inherent standards,the high degradation rate after 5d of > 83 % confirms ready biodegradation (Hoechst 1980). Based on model calculation, its half-life time is supposed to be about 10 h in air due to OH radical-induced photolysis. No bioaccumulating potential is expected in aquatic organisms, given the low logPow of < 1 and the short residence time in the environment. There is no substantial adsorption potential to soil, based on the very low partition coefficient as well as ready biodegradability. This is confirmed by experimental data obtained with n-butylamine, a structure-analogous short-chain alkylamine (Koc = 15 - 107, depending on the soil type) [Oepen et al. 1991]. With a pKa of 10.6 - 10.8, isopropylamine as a strong base will exist predominantly in its protonated form in the environment, and the protonated form of isopropylamine is not expected to volatilize from water or moist soil surfaces.

5.1 Stability

5.1.1 Phototransformation in air

Endpoint study record: Key.K2.Phototransformation in air Epiwin 2010

| UUID |  | IUC5-44f67ff7-98c8-4d0d-bc70-55ab39eb340b |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | Collette / Epona Associates, LLC / Willington / United States |
| Date |  | 2012-05-30 07:57:09 EDT |
| Remarks |  |  |

Administrative Data

| Purpose flag | key study |  |  |
| --- | --- | --- | --- |
| Study result type | estimated by calculation |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Accepted calculation method |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| other: EPIWin calculation |  | 2010 |  | U.S. Environmental Protection Agency (U.S. EPA). Estimation Programs Interface Suite,™. for Microsoft® Windows, v4.00. United States Environmental Protection Agency, Washington, DC, USA. |  |  |  |  |  |

Materials and methods

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| other: Epiwin description | SMILES : C(C)(C)N CHEM : Isopropylamine MOL FOR: C3 H9 N1 MOL WT : 59.11 |

Study design

Estimation method (if used)

PHOTOCHEMICAL REACTION WITH OH RADICALS - Concentration of OH radicals: 1.5E6 OH/cm3 - Degradation rate constant: 39.3766 E-12 cm3/molecule-sec - Temperature for which rate constant was calculated: 25 deg C - Computer programme: AOP Program (v1.92) - Other: OTHER ESTIMATION METHOD:

Any other information on materials and methods incl. tables

SMILES : C(C)(C)N CHEM : Isopropylamine MOL FOR: C3 H9 N1 MOL WT : 59.11 ------------------------------ EPI SUMMARY (v4.00) -------------------------- Physical Property Inputs: Log Kow (octanol-water): -0.50 Boiling Point (deg C) : 31.80 Melting Point (deg C) : -90.00 Vapor Pressure (mm Hg) : 473 Water Solubility (mg/L): 4.218E+005 Henry LC (atm-m3/mole) : ------

Results and discussions

Any other information on results incl. tables

AOP Program (v1.92) Results: =========================== ------------------- SUMMARY (AOP v1.92): HYDROXYL RADICALS (25 deg C) -------- Hydrogen Abstraction = 18.3766 E-12 cm3/molecule-sec Reaction with N, S and -OH = 21.0000 E-12 cm3/molecule-sec Addition to Triple Bonds = 0.0000 E-12 cm3/molecule-sec Addition to Olefinic Bonds = 0.0000 E-12 cm3/molecule-sec Addition to Aromatic Rings = 0.0000 E-12 cm3/molecule-sec Addition to Fused Rings = 0.0000 E-12 cm3/molecule-sec OVERALL OH Rate Constant = 39.3766 E-12 cm3/molecule-sec HALF-LIFE = 0.272 Days (12-hr day; 1.5E6 OH/cm3) HALF-LIFE = 3.260 Hrs ------------------- SUMMARY (AOP v1.91): OZONE REACTION (25 deg C) ----------- ****** NO OZONE REACTION ESTIMATION ****** (ONLY Olefins and Acetylenes are Estimated) Experimental Database: NO Structure Matches Fraction sorbed to airborne particulates (phi): 2.76E-009 (Junge-Pankow, Mackay avg) 3.37E-009 (Koa method) Note: the sorbed fraction may be resistant to atmospheric oxidation

Endpoint study record: ChemID (+2005_Phototransformation in air, RL4)

| UUID |  | IUC5-06ba0e94-6972-4049-9745-a2498201dddd |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-12-30 09:49:59 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Study result type | estimated by calculation |  |  |
| --- | --- | --- | --- |
| Reliability | 4 (not assignable) |  |  |
| Rationale for reliability incl. deficiencies | Secondary source |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| other: Online database | ChemIDplus (2005) | 2005 | 2-Propylamine [CAS No. 75-31-0] | Nat. Library of Medicine (NLM), Specialized Information Service (SIS), 25 July 2005 [http://chem.sis.nlm.nih.gov/chemidplus/] |  |  |  |  |  |

Data access

data published

Materials and methods

Principles of method if other than guideline

Method: other (calculated)

GLP compliance

no data

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |
| EC number | 200-860-9 |
| EC name | isopropylamine |

Study design

Details on test conditions

Sensitiser (for indirect photolysis): OH

Duration of test at given test condition

| Temp. | 25 °C |
| --- | --- |

Results and discussions

% Degradation of test substance

| %Degr. | St. dev. | Sampling time | Test condition |
| --- | --- | --- | --- |
| 50 |  | 10 h |  |

Transformation products

no data

Any other information on results incl. tables

Rate constant (for indirect photolysis): 0.0000000000394 cm³/(molecule*sec) Degradation in % (for indirect photolysis): ca. 50 after 10 hour(s)

Overall remarks, attachments

Overall remarks

"Vapor-phase isopropylamine is degraded in the atmosphere by reaction with photochemically-produced hydroxyl radicals(SRC); the half-life for this reaction in air is estimated to be 10 hours(SRC) from its estimated rate constant of 3.9^-11 cm3/(molecule*sec) at 25°C(SRC), determined from a structure estimation method (Meylan, Howard 1993)." (HSDB 2005)

5.1.2 Hydrolysis

Endpoint study record: WAIVER_Hydrolysis

| UUID |  | IUC5-e5bc6385-c97d-4c49-b0e8-fbf0f514ab5b |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:51:34 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| EU: REACH |  |
| --- | --- |
| Data waiving | other justification |
| Justification for data waiving | In accordance with column 2 of Annex IX (required in Section 9.2.2.1) of REACH Regulation 1907/2006: no study needed for isopropylamine, as it is readily biodegradable and lacks hydrolysable groups. |

Materials and methods

Test materials

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |
| EC name | Isopropylamine |
| EC number | 200-860-9 |

5.2 Biodegradation

Endpoint summary: Biodegradation

| UUID |  | IUC5-6c5ec02d-11b6-4baf-9cc4-d871fc2afa44 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:58:02 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

EU: REACH

Discussion

In valid standard ready biodegradability studies, 70 - 80 % (COD) and 95 % (DOC) of test substance has been biodegraded (BASF 1997) (Arkema 1992).Although test conditions were according to inherent standards,the high degradation rate after 5d of > 83 % confirms ready biodegradation (Hoechst 1980).

5.2.1 Biodegradation in water: screening tests

Endpoint summary: Biodegradation in water: screening tests

| UUID |  | IUC5-0b1f411a-8b0c-402f-964f-1e6bd9ccfb40 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:58:03 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

EU: REACH

Short description of key information

Isopropylamin is according to OECD Guideline 301 F (BASF 1997) ready biodegradable.

Key value for chemical safety assessment

| Biodegradation in water | readily biodegradable |
| --- | --- |

Endpoint study record: Key.BASFAG96/0626/26/1.Biodegradation in water: screening tests

| UUID |  | IUC5-6528268f-9783-48ef-91e6-b0cfdb34d9c9 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-12-30 09:50:13 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Purpose flag | key study; robust study summary; used for MSDS |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result |  |  |
| Reliability | 1 (reliable without restriction) |  |  |
| Rationale for reliability incl. deficiencies | GLP-guideline study |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | BASF AG | 1997 |  |  | Department of Ecology | 96/0626/26/1 | BASF AG |  | 1997-07-27 |

Data access

data submitter has Letter of Access

Data protection claimed

yes

Materials and methods

Test type

ready biodegradability

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | OECD Guideline 301 F (Ready Biodegradability: Manometric Respirometry Test) |  |

GLP compliance

yes

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |
| EC name | isopropylamine |

Details on test material

- Name of test material (as cited in study report): Isopropylamine waterfree - Analytical purity: 99.9% - Lot/batch No.: 019727 - Storage condition of test material: in refrigerator

Study design

Oxygen conditions

aerobic

Inoculum or test system

activated sludge, domestic (adaptation not specified)

Details on inoculum

- Source of inoculum/activated sludge (e.g. location, sampling depth, contamination history, procedure): Laboratory STP treating domestic and synthetic sewage

Duration of test (contact time)

35 d

Initial test substance concentration

| Initial conc. | Based on |
| --- | --- |
| 100 mg/L | test mat. |

Parameter followed for biodegradation estimation

O2 consumption

Details on study design

- Temperature of incubation °C: room temperature - Dosing procedure: Direct addition to test medium - Sampling frequency: continuously for 35 days - Appropriate controls and blank system used: yes [blank values (BC), biodegradation of a reference substance (RS), inhibition of the inoculum (IH) and abiotic elimination (PC)] - Analytical method used to measure biodegradation: BOD

Results and discussions

% Degradation of test substance

| %Degr. | St. dev. | Parameter | Sampling time | Remarks |
| --- | --- | --- | --- | --- |
| 70 — 80 |  | O2 consumption | 28 d |  |

Details on results

RS-Freetext: test device: - Sapromat-system reference control: - reference substance: aniline - concentration: 100 mg/L related to reference substance - kinetic of reference substance: 8 day(s) 3 % BOD/ThOD 9 day(s) 15 % BOD/ThOD 12 day(s) 46 % BOD/ThOD 13 day(s) 62 % BOD/ThOD 28 day(s) 82 % BOD/ThOD 35 day(s) 83 % BOD/ThOD inhibition control: - substances: aniline + isopropylamine - concentration: each 100 mg/L related to substance - kinetic of inhibition control: 7 day(s) 0 % BOD/ThOD 14 day(s) 2 % BOD/ThOD 18 day(s) 24 % BOD/ThOD 21 day(s) 56 % BOD/ThOD 28 day(s) 72 % BOD/ThOD 35 day(s) 74 % BOD/ThOD assay to examine physico-chemical elimination: - concentration: 100 mg/L related to test substance - w/o inoculum, w mercury chloride to avoid biodegradation - kinetic of physico-chemical elimination: 7 day(s): 0 % BOD/ThOD 14 day(s): 0 % BOD/ThOD 21 day(s): 0 % BOD/ThOD 28 day(s): 1 % BOD/ThOD NO3-N and N02-N concentration at test end (after 35 day(s)): - assay NO2-N NO3-N (mg/L) (mg/L) TS1 0.0 1.61 TS2 0.0 2.99 TS3 0.0 1.61 TS4 0.0 0.92 TS5 0.0 18.4 TS6 0.0 25.3 TS7 0.0 9.2 biodegradation (%) in relation to the ThOD corrected for oxygen demand due to nitrification at test end (after 35 day(s)): - assay BOD/ThOD BOD/ThOD % (1) % (2) TS1 75 72 TS2 77 72 TS3 78 75 TS4 61 59 TS5 126 91 TS6 124 77 TS7 99 82 (1) = nitrification not considered; (2) corrected for nitrification nitrification in the replicates TS1 to TS4 was regarded as negligible. At least these assays were considered to evaluate the biodegradability of isopropylamine (70-80 % BOD/ThOD after 28 days). Biodegradation % BOD/ThOD - after 28 day(s): mean value (TS1-TS4): 72 % (nitrification not considered) mean value (TS5-TS7): 109 % (nitrification not considered) - at test end (after 35 day(s)): mean value (TS1-TS4): 73 % (nitrification not considered) mean value (TS1-TS4): 70 % (corrected for nitrification) mean value (TS5-TS7): 116 % (nitrification not considered) mean value (TS5-TS7): 83 % (corrected for nitrification) pH values at test start and test end (after 35 day(s)): - assay pH pH day 0 day 35 BC1 7.6 7.7 BC2 7.6 7.7 RS 7.6 9.1 IH 10.0 9.0 TS1 10.0 7.4 TS2 10.0 7.4 TS3 9.9 7.4 TS4 10.0 7.4 TS5 9.9 6.3 TS6 10.1 6.6 TS7 10.1 7.0 PC 10.1 7.4 Kinetic of test substance (in %): < 10 after 8 day(s) > 50 after 10 day(s) ca. 60 after 12 day(s) = 70 ... 80 after 28 day(s) = 70 ... 80 after 35 day(s)

Applicant's summary and conclusion

Interpretation of results

readily biodegradable

Endpoint study record: Arkema 1992/K2/Ready Biodegradation

| UUID |  | IUC5-cda33df7-18ea-425c-b9a1-a8493f0db286 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-12-30 09:50:07 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Purpose flag | supporting study; robust study summary |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result | Study period | no data |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Study according to international standards |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | Boutonnet JC | 1992 | isopropylamine - Biodégradabilité aérobie ultime selon la norme ISO 7827 (1984) |  | Centre d'application de Levallois | 35806/J.C. BOUTONNET/CK | Arkema France | 737 | 1992-01-22 |

Data access

data submitter has Letter of Access

Data protection claimed

yes

Materials and methods

Test type

ready biodegradability

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | ISO 7827 (Evaluation in an Aqueous Medium of the "Ultimate" Aerobic Biodegradability of Organic Compounds - Method by Anlaysis of Dissolved Organic Carbon (DOC)) | no data |

GLP compliance

no data

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |
| EC name | isopropylamine |

Details on test material

Name of test material: isopropylamine

Confidential details on test material

no data

Details on properties of test surrogate or analogue material

not relevant

Study design

Oxygen conditions

aerobic

Inoculum or test system

no data

Details on inoculum

no data

Duration of test (contact time)

21 d

Initial test substance concentration

| Initial conc. | Based on |
| --- | --- |
| 30 mg/L | DOC |

Parameter followed for biodegradation estimation

DOC removal

Details on analytical methods

no data

Details on study design

no data

Reference substance

benzoic acid, sodium salt

Results and discussions

% Degradation of test substance

| %Degr. | St. dev. | Parameter | Sampling time | Remarks |
| --- | --- | --- | --- | --- |
| 95 |  | DOC removal | 21 d |  |

BOD5 / COD results

Results with reference substance

no data

Applicant's summary and conclusion

Validity criteria fulfilled

yes

Interpretation of results

readily biodegradable

Conclusions

- Difference between the extreme values of the test flasks at the end of the test < 20 %. - Percentage of elimination of DOC in the reference substance at day 14 is > 70 %. - The substance is not inhibitory (degradation > 35 %).

Executive summary

The aerobic biodegradation of isopropylamine has been examinated through ISO 7827 (1984) guideline. DOC removal was examined during 21 days from an initial concentration of 30.0 mg/L. The results have shown that 95% of the substance is removed after 21 days. The substance is readily biodegradable. The substance is not inhibitory (deg > 35 %).

Endpoint study record: Zahn-Wellens Hoechst AG (+1980, OECD302B, SS, RL2)

| UUID |  | IUC5-79c29f9f-0728-4ba6-aae0-52e96d8167a5 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-12-30 09:50:32 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Purpose flag | supporting study; robust study summary |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Basic data given, comparable to guideline study (pre-guideline) |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| other company data | Wellens, H. | 1980 | Abwasserbiologische Untersuchung von Isopropylamin | unpublished report | Abteilung Reinhaltung von Wasser und Luft |  | Hoechst AG | T02467 | 1980-04-16 |
| publication | Zahn R and Wellens H | 1980 | Pruefung der biologischen Abbaubarkeit im Standversuch - Weitere Erfahrungen und neue Einsatzmöglichkeiten. | Z. Wasser Abwasser Forsch., 13(1), 1-7 |  |  |  | T02014 |  |
| publication | Zahn R and Wellens H | 1974 | Ein einfaches Verfahren zur Prüfung der biologischen Abbaubarkeit von Produkten und Abwasserinhaltsstoffen | Chemiker-Zeitung 98(5): 228-232 |  |  |  | T04679 |  |

Data access

data submitter is data owner

Data protection claimed

yes, but willing to share

Materials and methods

Test type

inherent biodegradability

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| equivalent or similar to | OECD Guideline 302 B (Inherent biodegradability: Zahn-Wellens/EMPA Test) | no data |

Principles of method if other than guideline

Zahn-Wellens-Test, DIN 38412, part 25, details from Zahn R and Wellens H 1974

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |
| EC number | 200-860-9 |
| EC name | isopropylamine |

Details on test material

Isopropylamine [CAS No. 75-31-0], not further specified

Study design

Oxygen conditions

aerobic

Inoculum or test system

activated sludge, industrial, non-adapted

Details on inoculum

no details Biomass concentration: approx. 1 g dry mass/L (according to Zahn, Wellens 1980)

Duration of test (contact time)

14 d

Initial test substance concentration

| Initial conc. | Based on |
| --- | --- |
| ca. 1000 mg/L | COD |
| ca. 400 mg/L | test mat. |

Parameter followed for biodegradation estimation

O2 consumption

Details on study design

TEST CONDITIONS - Composition of medium: mineral medium: 215 mg urea (alternatively 385 mg NH4Cl) and 89 mg NaH2PO4 x H2O in 2 L of water - Additional substrate: no - Solubilising agent (type and concentration if used): no - Test temperature: 22 ± 3°C - pH: 6.5 - 7.5 - pH adjusted: if necessary - Aeration of dilution water: yes - Suspended solids concentration: 1 g dry substance (activated sludge)/L - Continuous darkness: no data TEST SYSTEM - Culturing apparatus: 4 L cylindric glas vessel (beaker) - Number of culture flasks/concentration: 1 - Method used to create aerobic conditions: stirring and introduction of compressed air passed through a cotton wool strainer and a wash-bottle - Measuring equipment: COD or TOC analyzer - Test performed in closed vessels due to significant volatility of test substance: no data - Test performed in open system: no data SAMPLING - Sampling frequency: daily or other time intervals - Sampling method: withdrawal of sample from test vessel and filtration of sample through paper filter (Schleicher und Schüll, Blauband Nr. 589) that had been washed three times with distilled water CONTROL AND BLANK SYSTEM - Inoculum blank: yes

Any other information on materials and methods incl. tables

More details on the method may be found in: Zahn, Wellens (1974): Ein einfaches Verfahren zur Prüfung der biologischen Abbaubarkeit von Produkten und Abwasserinhaltsstoffen Chemiker-Zeitung 98, 228-232 (Publ. in German language, not yet available)

Results and discussions

% Degradation of test substance

| %Degr. | St. dev. | Parameter | Sampling time | Remarks |
| --- | --- | --- | --- | --- |
| > 95 |  | O2 consumption | 10 d |  |
| ca. 83 |  | O2 consumption | 5 d |  |

BOD5 / COD results

Any other information on results incl. tables

COD-Elimination by non-biological processes (unspecified): 10 - 20 %

Applicant's summary and conclusion

Validity criteria fulfilled

not applicable

Interpretation of results

readily biodegradable

Conclusions

Test conditions were according to inherent standards, but due to the high degradation rate after 5d the conclusion of ready biodgradation is justified.

5.2.2 Biodegradation in water and sediment: simulation tests

Endpoint study record: WAIVER_Biodegradation in water and sediment

| UUID |  | IUC5-803b147e-358f-4585-b44a-c832e13a9a5d |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:51:35 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| EU: REACH |  |
| --- | --- |
| Data waiving | other justification |
| Justification for data waiving | In accordance with column 2 of Annex VII (required in Section 9.2.1.2 and 9.2.1.4) of REACH Regulation 1907/2006: no study needed for isopropylamine, as it is readily biodegradable. |

Materials and methods

Test materials

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |

5.2.3 Biodegradation in soil

Endpoint study record: WAIVER_Biodegradation in soil

| UUID |  | IUC5-ddcc3e5b-c237-4f47-9e32-fdcd89fa47f8 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:51:36 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| EU: REACH |  |
| --- | --- |
| Data waiving | other justification |
| Justification for data waiving | In accordance with column 2 of Annex IX (required in Section 9.2.1.3) of REACH Regulation 1907/2006: no study needed for isopropylamine, as it is readily biodegradable. |

Materials and methods

Test materials

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |

5.3 Bioaccumulation

5.3.1 Bioaccumulation: aquatic / sediment

Endpoint summary: Bioaccumulation: aquatic / sediment

| UUID |  | IUC5-05a2eb4f-e576-46cb-9dbf-27a4d517d1a4 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:58:03 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

EU: REACH

Short description of key information

No bioaccumulating potential is expected in aquatic organisms, given the low logPow of < 1 and the short residence time in the environment.

Endpoint study record: Key. K2.Bioaccumulation: aquatic / sediment EPIWIN 2010

| UUID |  | IUC5-898d92ef-521b-4b87-9bcc-8eb23c806119 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | Collette / Epona Associates, LLC / Willington / United States |
| Date |  | 2012-05-30 08:20:16 EDT |
| Remarks |  |  |

Administrative Data

| Purpose flag | key study |  |  |
| --- | --- | --- | --- |
| Study result type | estimated by calculation |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Accepted calculation method |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| other: EPIWin calculation |  | 2010 |  | U.S. Environmental Protection Agency (U.S. EPA). Estimation Programs Interface Suite,™. for Microsoft® Windows, v4.00. United States Environmental Protection Agency, Washington, DC, USA. |  |  |  |  |  |

Materials and methods

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| other: Epiwin description | SMILES : C(C)(C)N CHEM : Isopropylamine MOL FOR: C3 H9 N1 MOL WT : 59.11 |

Test conditions

Any other information on materials and methods incl. tables

SMILES : C(C)(C)N CHEM : Isopropylamine MOL FOR: C3 H9 N1 MOL WT : 59.11 ------------------------------ EPI SUMMARY (v4.00) -------------------------- Physical Property Inputs: Log Kow (octanol-water): -0.50 Boiling Point (deg C) : 31.80 Melting Point (deg C) : -90.00 Vapor Pressure (mm Hg) : 473 Water Solubility (mg/L): 4.218E+005 Henry LC (atm-m3/mole) : ------

Results and discussions

Bioaccumulation factor

| Conc. in environment / dose | Type | Value | Basis | Time of plateau | Calculation basis | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
|  | BCF | 3.162 L/kg |  |  |  |  |

Any other information on results incl. tables

BCFBAF Program (v3.00) Results: ============================== --------------------------------- BCFBAF v3.00 -------------------------------- Summary Results: Log BCF (regression-based estimate): 0.50 (BCF = 3.16 L/kg wet-wt) Biotransformation Half-Life (days) : 0.073 (normalized to 10 g fish) Log BAF (Arnot-Gobas upper trophic): -0.04 (BAF = 0.918 L/kg wet-wt) Log Kow (experimental): 0.26 Log Kow used by BCF estimates: -0.50 (user entered) Equation Used to Make BCF estimate: Log BCF = 0.50 Correction(s): Value Correction Factors Not Used for Log Kow < 1 Estimated Log BCF = 0.500 (BCF = 3.162 L/kg wet-wt) =========================================================== Whole Body Primary Biotransformation Rate Estimate for Fish: =========================================================== ------+-----+--------------------------------------------+---------+--------- TYPE ¦ NUM ¦ LOG BIOTRANSFORMATION FRAGMENT DESCRIPTION ¦ COEFF ¦ VALUE ------+-----+--------------------------------------------+---------+--------- Frag ¦ 1 ¦ Aliphatic amine [-NH2 or -NH-] ¦ 0.4067 ¦ 0.4067 Frag ¦ 2 ¦ Methyl [-CH3] ¦ 0.2451 ¦ 0.4902 Frag ¦ 1 ¦ -CH- [linear] ¦ -0.1912 ¦ -0.1912 L Kow¦ * ¦ Log Kow = -0.50 (user-entered ) ¦ 0.3073 ¦ -0.1537 MolWt¦ * ¦ Molecular Weight Parameter ¦ ¦ -0.1516 Const¦ * ¦ Equation Constant ¦ ¦ -1.5058 ============+============================================+=========+========= RESULT ¦ LOG Bio Half-Life (days) ¦ ¦ -1.1366 RESULT ¦ Bio Half-Life (days) ¦ ¦ 0.07301 NOTE ¦ Bio Half-Life Normalized to 10 g fish at 15 deg C ¦ ============+============================================+=========+========= Biotransformation Rate Constant: kM (Rate Constant): 9.494 /day (10 gram fish) kM (Rate Constant): 5.339 /day (100 gram fish) kM (Rate Constant): 3.002 /day (1 kg fish) kM (Rate Constant): 1.688 /day (10 kg fish) Arnot-Gobas BCF & BAF Methods (including biotransformation rate estimates): Estimated Log BCF (upper trophic) = -0.037 (BCF = 0.9182 L/kg wet-wt) Estimated Log BAF (upper trophic) = -0.037 (BAF = 0.9182 L/kg wet-wt) Estimated Log BCF (mid trophic) = -0.022 (BCF = 0.9502 L/kg wet-wt) Estimated Log BAF (mid trophic) = -0.022 (BAF = 0.9502 L/kg wet-wt) Estimated Log BCF (lower trophic) = -0.019 (BCF = 0.957 L/kg wet-wt) Estimated Log BAF (lower trophic) = -0.019 (BAF = 0.957 L/kg wet-wt) Arnot-Gobas BCF & BAF Methods (assuming a biotransformation rate of zero): Estimated Log BCF (upper trophic) = -0.033 (BCF = 0.9268 L/kg wet-wt) Estimated Log BAF (upper trophic) = -0.033 (BAF = 0.9273 L/kg wet-wt)

Endpoint study record: WAIVER_Bioaccumulation

| UUID |  | IUC5-6b302ca7-b395-49c9-9228-d5d38c1c89e8 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:51:37 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| EU: REACH |  |
| --- | --- |
| Data waiving | other justification |
| Justification for data waiving | In accordance with column 2 of Annex IX (required in Section 9.3) of REACH Regulation 1907/2006: no study needed for isopropylamine, as the logPow value is significantly below 3 (logPow = 0.21 – 0.26). |

Materials and methods

Test materials

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |

5.4 Transport and distribution

Endpoint summary: Transport and distribution

| UUID |  | IUC5-2ae94a3c-70ca-41fc-be92-ef17c0fd24bc |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:58:04 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

EU: REACH

Discussion

There is no substantial adsorption potential to soil, based on the very low partition coefficient as well as ready biodegradability. Experimental Koc values obtained with n-butylamine, a structure-analogous short-chain alkylamine [CAS no. 109-73-9] ranged from 15 – 107, depending on the soil type, which confirms the low adsorption potential of these strong bases to soil. With a pKa of 10.6 – 10.8, isopropylamine will exist predominantly in its protonated form in the environment, and the protonated form of isopropylamine is not expected to volatilize from water or moist soil surfaces..

5.4.1 Adsorption / desorption

Endpoint study record: von Oepen (+1991, READACROSS butylamine, Adsorption, KS, RL2)

| UUID |  | IUC5-73e616fd-44a8-44da-a4bd-8f57e149526a |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-12-30 09:50:48 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Purpose flag | key study; robust study summary |  |  |
| --- | --- | --- | --- |
| Study result type | read-across from supporting substance (structural analogue or surrogate) |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Guideline study, limited documentation |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Von Oepen B, Kördel W, Klein W | 1991 | Sorption of nonpolar and polar compounds to soils: Processes, measurements and experience with the applicability of the modified OECD-Guideline 106 | Chemosphere, 22, 285-304 |  |  |  | T04905 | 1990-12-31 |

Data access

data published

Materials and methods

Study type

adsorption/desorption

Media

soil

Type of method

batch equilibrium method

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | OECD Guideline 106 (Adsorption - Desorption Using a Batch Equilibrium Method) | yes ( Mass balance performed at a time point when less than 75% desorption was found.) |

Principles of method if other than guideline

Comparative study including 50 chemicals

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

no

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 109-73-9 |
| EC number | 203-699-2 |
| EC name | butylamine |

Radiolabelling

no

Details on test material

- Name of test material (as cited in study report): n-butylamine - Substance type: organic - Physical state: liquid - Analytical purity: no data - Lot/batch No.: 047

Study design

HPLC method

Details on study design: HPLC method

GC or HPLC in the supernatant of the soil-water batches

Batch equilibrium or other method

Analytical monitoring

yes

Details on sampling

- Concentrations: 0.15, 0.5, 5, and 15 mg/L - Sampling interval: 0.5, 1, 1.5, 5, 24, and 72 h - Freundlich isotherm determined after 16 h (sorption eqiuilibrium)

Details on analytical methods

no data

Details on matrix

Three types of soil examined: Podsol (acidic forest soil, pH 2.8); Alfisol (a neutral agricultural soil, pH 6.7); sublimnic soil (sediment from Lake Constance, pH 7.1)

Details on test conditions

TEST CONDITIONS - Buffer: no data - pH: no data - Suspended solids concentration:10 g dw/50 mL test solution - other: TEST SYSTEM - Type, size and further details on reaction vessel: not reported - Water filtered (i.e. yes/no; type of size of filter used, if any): not reported - Amount of soil: 10 g dw - Test performed in open system: yes - Method of preparation of test solution: no data - Are the residues from the adsorption phase used for desorption: yes

Results and discussions

Adsorption coefficient

| Type | Value | Temperature | % Org. carbon | Remarks |
| --- | --- | --- | --- | --- |
| Koc | 15 — 107 |  |  |  |
| log Koc | >= 1.2 — 2.1 |  |  |  |

Results: Batch equilibrium or other method

Adsorption and desorption constants

Adsorption: Koc = 15 (Podsol) Koc = 105 (Alfisol) Koc = 107 (limnic soil/sediment) Desorption: no data reported.

Any other information on results incl. tables

For all test substances the sorption equilibrium was reached within 16 h. Adsorption was reversible to great extent: the mass balance resulted in a recovery of > 80 %. The adsorption behavior was variable for polar substances; therefore the Koc appears to be of little predictive value for other soils, according to the authors.

5.4.2 Henry's Law constant

Endpoint study record: Key.Epiwin 2010, RL2

| UUID |  | IUC5-6968341f-abc1-4f86-80fe-e994ae7463a6 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | Collette / Epona Associates, LLC / Willington / United States |
| Date |  | 2012-07-02 11:32:37 EDT |
| Remarks |  |  |

Administrative Data

| Purpose flag | key study |  |  |
| --- | --- | --- | --- |
| Study result type | estimated by calculation |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | scientifically acceptable method |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| other: EPIWin calculation |  | 2010 |  | U.S. Environmental Protection Agency (U.S. EPA). Estimation Programs Interface Suite,™. for Microsoft® Windows, v4.00. United States Environmental Protection Agency, Washington, DC, USA. |  |  |  |  |  |
| other: Online database | ChemIDplus (2005) | 2005 | 2-Propylamine [CAS No. 75-31-0] | Nat. Library of Medicine (NLM), Specialized Information Service (SIS), 25 July 2005 [http://chem.sis.nlm.nih.gov/chemidplus/] |  |  |  |  |  |
| other: Online database | HSDB (2005) | 2005 | 2-Propylamine [CAS No. 75-31-0] | Hazardous Substances Databank, database of the National Library of Medicine's TOXNET system, 29 June 2005 (http://toxnet.nlm.nih.gov) |  |  |  |  |  |

Data access

data published

Materials and methods

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| other: Epiwin description | SMILES : C(C)(C)N CHEM : Isopropylamine MOL FOR: C3 H9 N1 MOL WT : 59.11 |

Any other information on materials and methods incl. tables

SMILES : C(C)(C)N CHEM : Isopropylamine MOL FOR: C3 H9 N1 MOL WT : 59.11 ------------------------------ EPI SUMMARY (v4.00) -------------------------- Physical Property Inputs: Log Kow (octanol-water): -0.50 Boiling Point (deg C) : 31.80 Melting Point (deg C) : -90.00 Vapor Pressure (mm Hg) : 473 Water Solubility (mg/L): 4.218E+005 Henry LC (atm-m3/mole) : ------

Results and discussions

Henry's Law constant H

| H | ca. 1.35 Pa m³/mol |
| --- | --- |
| Temp. (°C) | 25 |

Any other information on results incl. tables

HENRYWIN (v3.20) Program Results: ============================= Bond Est : 1.34E-005 atm-m3/mole (1.35E+000 Pa-m3/mole) Group Est: Incomplete SMILES : C(C)(C)N CHEM : Isopropylamine MOL FOR: C3 H9 N1 MOL WT : 59.11 --------------------------- HENRYWIN v3.20 Results -------------------------- Experimental Database Structure Match: Name : ISOPROPYLAMINE CAS Num : 000075-31-0 Exp HLC : 4.51E-05 atm-m3/mole (4.57 Pa-m3/mole) Temper : 25 deg C Exp Ref : VP/WSOL ----------+---------------------------------------------+---------+---------- CLASS ¦ BOND CONTRIBUTION DESCRIPTION ¦ COMMENT ¦ VALUE ----------+---------------------------------------------+---------+---------- HYDROGEN ¦ 7 Hydrogen to Carbon (aliphatic) Bonds ¦ ¦ -0.8377 HYDROGEN ¦ 2 Hydrogen to Nitrogen Bonds ¦ ¦ 2.5670 FRAGMENT ¦ 2 C-C ¦ ¦ 0.2326 FRAGMENT ¦ 1 C-N ¦ ¦ 1.3010 ----------+---------------------------------------------+---------+---------- RESULT ¦ BOND ESTIMATION METHOD for LWAPC VALUE ¦ TOTAL ¦ 3.263 ----------+---------------------------------------------+---------+---------- HENRYs LAW CONSTANT at 25 deg C = 1.34E-005 atm-m3/mole = 5.46E-004 unitless = 1.35E+000 Pa-m3/mole --------+-----------------------------------------------+------------+-------- ¦ GROUP CONTRIBUTION DESCRIPTION ¦ COMMENT ¦ VALUE --------+-----------------------------------------------+------------+-------- ¦ 2 CH3 (X) ¦ ¦ -1.24 ¦ 1 NH2 (C) ¦ ¦ 4.15 ¦ MISSING Value for: CH (C)(C)(N) --------+-----------------------------------------------+------------+-------- RESULT ¦ GROUP ESTIMATION METHOD for LOG GAMMA VALUE ¦ INCOMPLETE ¦ 2.91 --------+-----------------------------------------------+------------+-------- For Henry LC Comparison Purposes: Exper Database: 4.51E-05 atm-m3/mole (4.57E+000 Pa-m3/mole) User-Entered Henry LC: not entered Henrys LC [via VP/WSol estimate using User-Entered or Estimated values]: HLC: 8.722E-005 atm-m3/mole (8.837E+000 Pa-m3/mole) VP: 473 mm Hg (source: User-Entered) WS: 4.22E+005 mg/L (source: User-Entered) Bond Est : 1.34E-005 atm-m3/mole (1.35E+000 Pa-m3/mole) Group Est: Incomplete SMILES : C(C)(C)N CHEM : Isopropylamine MOL FOR: C3 H9 N1 MOL WT : 59.11 --------------------------- HENRYWIN v3.20 Results -------------------------- Experimental Database Structure Match: Name : ISOPROPYLAMINE CAS Num : 000075-31-0 Exp HLC : 4.51E-05 atm-m3/mole (4.57 Pa-m3/mole) Temper : 25 deg C Exp Ref : VP/WSOL ----------+---------------------------------------------+---------+---------- CLASS ¦ BOND CONTRIBUTION DESCRIPTION ¦ COMMENT ¦ VALUE ----------+---------------------------------------------+---------+---------- HYDROGEN ¦ 7 Hydrogen to Carbon (aliphatic) Bonds ¦ ¦ -0.8377 HYDROGEN ¦ 2 Hydrogen to Nitrogen Bonds ¦ ¦ 2.5670 FRAGMENT ¦ 2 C-C ¦ ¦ 0.2326 FRAGMENT ¦ 1 C-N ¦ ¦ 1.3010 ----------+---------------------------------------------+---------+---------- RESULT ¦ BOND ESTIMATION METHOD for LWAPC VALUE ¦ TOTAL ¦ 3.263 ----------+---------------------------------------------+---------+---------- HENRYs LAW CONSTANT at 25 deg C = 1.34E-005 atm-m3/mole = 5.46E-004 unitless = 1.35E+000 Pa-m3/mole --------+-----------------------------------------------+------------+-------- ¦ GROUP CONTRIBUTION DESCRIPTION ¦ COMMENT ¦ VALUE --------+-----------------------------------------------+------------+-------- ¦ 2 CH3 (X) ¦ ¦ -1.24 ¦ 1 NH2 (C) ¦ ¦ 4.15 ¦ MISSING Value for: CH (C)(C)(N) --------+-----------------------------------------------+------------+-------- RESULT ¦ GROUP ESTIMATION METHOD for LOG GAMMA VALUE ¦ INCOMPLETE ¦ 2.91 --------+-----------------------------------------------+------------+-------- For Henry LC Comparison Purposes: Exper Database: 4.51E-05 atm-m3/mole (4.57E+000 Pa-m3/mole) User-Entered Henry LC: not entered Henrys LC [via VP/WSol estimate using User-Entered or Estimated values]: HLC: 8.722E-005 atm-m3/mole (8.837E+000 Pa-m3/mole) VP: 473 mm Hg (source: User-Entered) WS: 4.22E+005 mg/L (source: User-Entered) Bond Est : 1.34E-005 atm-m3/mole (1.35E+000 Pa-m3/mole) Group Est: Incomplete SMILES : C(C)(C)N CHEM : Isopropylamine MOL FOR: C3 H9 N1 MOL WT : 59.11 --------------------------- HENRYWIN v3.20 Results -------------------------- Experimental Database Structure Match: Name : ISOPROPYLAMINE CAS Num : 000075-31-0 Exp HLC : 4.51E-05 atm-m3/mole (4.57 Pa-m3/mole) Temper : 25 deg C Exp Ref : VP/WSOL ----------+---------------------------------------------+---------+---------- CLASS ¦ BOND CONTRIBUTION DESCRIPTION ¦ COMMENT ¦ VALUE ----------+---------------------------------------------+---------+---------- HYDROGEN ¦ 7 Hydrogen to Carbon (aliphatic) Bonds ¦ ¦ -0.8377 HYDROGEN ¦ 2 Hydrogen to Nitrogen Bonds ¦ ¦ 2.5670 FRAGMENT ¦ 2 C-C ¦ ¦ 0.2326 FRAGMENT ¦ 1 C-N ¦ ¦ 1.3010 ----------+---------------------------------------------+---------+---------- RESULT ¦ BOND ESTIMATION METHOD for LWAPC VALUE ¦ TOTAL ¦ 3.263 ----------+---------------------------------------------+---------+---------- HENRYs LAW CONSTANT at 25 deg C = 1.34E-005 atm-m3/mole = 5.46E-004 unitless = 1.35E+000 Pa-m3/mole --------+-----------------------------------------------+------------+-------- ¦ GROUP CONTRIBUTION DESCRIPTION ¦ COMMENT ¦ VALUE --------+-----------------------------------------------+------------+-------- ¦ 2 CH3 (X) ¦ ¦ -1.24 ¦ 1 NH2 (C) ¦ ¦ 4.15 ¦ MISSING Value for: CH (C)(C)(N) --------+-----------------------------------------------+------------+-------- RESULT ¦ GROUP ESTIMATION METHOD for LOG GAMMA VALUE ¦ INCOMPLETE ¦ 2.91 --------+-----------------------------------------------+------------+-------- For Henry LC Comparison Purposes: Exper Database: 4.51E-05 atm-m3/mole (4.57E+000 Pa-m3/mole) User-Entered Henry LC: not entered Henrys LC [via VP/WSol estimate using User-Entered or Estimated values]: HLC: 8.722E-005 atm-m3/mole (8.837E+000 Pa-m3/mole) VP: 473 mm Hg (source: User-Entered) WS: 4.22E+005 mg/L (source: User-Entered) Bond Est : 1.34E-005 atm-m3/mole (1.35E+000 Pa-m3/mole) Group Est: Incomplete SMILES : C(C)(C)N CHEM : Isopropylamine MOL FOR: C3 H9 N1 MOL WT : 59.11 --------------------------- HENRYWIN v3.20 Results -------------------------- Experimental Database Structure Match: Name : ISOPROPYLAMINE CAS Num : 000075-31-0 Exp HLC : 4.51E-05 atm-m3/mole (4.57 Pa-m3/mole) Temper : 25 deg C Exp Ref : VP/WSOL ----------+---------------------------------------------+---------+---------- CLASS ¦ BOND CONTRIBUTION DESCRIPTION ¦ COMMENT ¦ VALUE ----------+---------------------------------------------+---------+---------- HYDROGEN ¦ 7 Hydrogen to Carbon (aliphatic) Bonds ¦ ¦ -0.8377 HYDROGEN ¦ 2 Hydrogen to Nitrogen Bonds ¦ ¦ 2.5670 FRAGMENT ¦ 2 C-C ¦ ¦ 0.2326 FRAGMENT ¦ 1 C-N ¦ ¦ 1.3010 ----------+---------------------------------------------+---------+---------- RESULT ¦ BOND ESTIMATION METHOD for LWAPC VALUE ¦ TOTAL ¦ 3.263 ----------+---------------------------------------------+---------+---------- HENRYs LAW CONSTANT at 25 deg C = 1.34E-005 atm-m3/mole = 5.46E-004 unitless = 1.35E+000 Pa-m3/mole --------+-----------------------------------------------+------------+-------- ¦ GROUP CONTRIBUTION DESCRIPTION ¦ COMMENT ¦ VALUE --------+-----------------------------------------------+------------+-------- ¦ 2 CH3 (X) ¦ ¦ -1.24 ¦ 1 NH2 (C) ¦ ¦ 4.15 ¦ MISSING Value for: CH (C)(C)(N) --------+-----------------------------------------------+------------+-------- RESULT ¦ GROUP ESTIMATION METHOD for LOG GAMMA VALUE ¦ INCOMPLETE ¦ 2.91 --------+-----------------------------------------------+------------+-------- For Henry LC Comparison Purposes: Exper Database: 4.51E-05 atm-m3/mole (4.57E+000 Pa-m3/mole) User-Entered Henry LC: not entered Henrys LC [via VP/WSol estimate using User-Entered or Estimated values]: HLC: 8.722E-005 atm-m3/mole (8.837E+000 Pa-m3/mole) VP: 473 mm Hg (source: User-Entered) WS: 4.22E+005 mg/L (source: User-Entered) ============================= Bond Est : 1.34E-005 atm-m3/mole (1.35E+000 Pa-m3/mole) Group Est: Incomplete SMILES : C(C)(C)N CHEM : Isopropylamine MOL FOR: C3 H9 N1 MOL WT : 59.11 --------------------------- HENRYWIN v3.20 Results -------------------------- Experimental Database Structure Match: Name : ISOPROPYLAMINE CAS Num : 000075-31-0 Exp HLC : 4.51E-05 atm-m3/mole (4.57 Pa-m3/mole) Temper : 25 deg C Exp Ref : VP/WSOL ----------+---------------------------------------------+---------+---------- CLASS ¦ BOND CONTRIBUTION DESCRIPTION ¦ COMMENT ¦ VALUE ----------+---------------------------------------------+---------+---------- HYDROGEN ¦ 7 Hydrogen to Carbon (aliphatic) Bonds ¦ ¦ -0.8377 HYDROGEN ¦ 2 Hydrogen to Nitrogen Bonds ¦ ¦ 2.5670 FRAGMENT ¦ 2 C-C ¦ ¦ 0.2326 FRAGMENT ¦ 1 C-N ¦ ¦ 1.3010 ----------+---------------------------------------------+---------+---------- RESULT ¦ BOND ESTIMATION METHOD for LWAPC VALUE ¦ TOTAL ¦ 3.263 ----------+---------------------------------------------+---------+---------- HENRYs LAW CONSTANT at 25 deg C = 1.34E-005 atm-m3/mole = 5.46E-004 unitless = 1.35E+000 Pa-m3/mole --------+-----------------------------------------------+------------+-------- ¦ GROUP CONTRIBUTION DESCRIPTION ¦ COMMENT ¦ VALUE --------+-----------------------------------------------+------------+-------- ¦ 2 CH3 (X) ¦ ¦ -1.24 ¦ 1 NH2 (C) ¦ ¦ 4.15 ¦ MISSING Value for: CH (C)(C)(N) --------+-----------------------------------------------+------------+-------- RESULT ¦ GROUP ESTIMATION METHOD for LOG GAMMA VALUE ¦ INCOMPLETE ¦ 2.91 --------+-----------------------------------------------+------------+-------- For Henry LC Comparison Purposes: Exper Database: 4.51E-05 atm-m3/mole (4.57E+000 Pa-m3/mole) User-Entered Henry LC: not entered Henrys LC [via VP/WSol estimate using User-Entered or Estimated values]: HLC: 8.722E-005 atm-m3/mole (8.837E+000 Pa-m3/mole) VP: 473 mm Hg (source: User-Entered) WS: 4.22E+005 mg/L (source: User-Entered)

Overall remarks, attachments

Overall remarks

HENRYWIN (v3.20) Program Results: ================================== Bond Est : 1.34E-005 atm-m3/mole (1.35E+000 Pa-m3/mole) Group Est: Incomplete SMILES : C(C)(C)N CHEM : Isopropylamine MOL FOR: C3 H9 N1 MOL WT : 59.11 --------------------------- HENRYWIN v3.20 Results -------------------------- Experimental Database Structure Match: Name : ISOPROPYLAMINE CAS Num : 000075-31-0 Exp HLC : 4.51E-05 atm-m3/mole (4.57 Pa-m3/mole) Temper : 25 deg C Exp Ref : VP/WSOL ----------+---------------------------------------------+---------+---------- CLASS ¦ BOND CONTRIBUTION DESCRIPTION ¦ COMMENT ¦ VALUE ----------+---------------------------------------------+---------+---------- HYDROGEN ¦ 7 Hydrogen to Carbon (aliphatic) Bonds ¦ ¦ -0.8377 HYDROGEN ¦ 2 Hydrogen to Nitrogen Bonds ¦ ¦ 2.5670 FRAGMENT ¦ 2 C-C ¦ ¦ 0.2326 FRAGMENT ¦ 1 C-N ¦ ¦ 1.3010 ----------+---------------------------------------------+---------+---------- RESULT ¦ BOND ESTIMATION METHOD for LWAPC VALUE ¦ TOTAL ¦ 3.263 ----------+---------------------------------------------+---------+---------- HENRYs LAW CONSTANT at 25 deg C = 1.34E-005 atm-m3/mole = 5.46E-004 unitless = 1.35E+000 Pa-m3/mole --------+-----------------------------------------------+------------+-------- ¦ GROUP CONTRIBUTION DESCRIPTION ¦ COMMENT ¦ VALUE --------+-----------------------------------------------+------------+-------- ¦ 2 CH3 (X) ¦ ¦ -1.24 ¦ 1 NH2 (C) ¦ ¦ 4.15 ¦ MISSING Value for: CH (C)(C)(N) --------+-----------------------------------------------+------------+-------- RESULT ¦ GROUP ESTIMATION METHOD for LOG GAMMA VALUE ¦ INCOMPLETE ¦ 2.91 --------+-----------------------------------------------+------------+-------- For Henry LC Comparison Purposes: Exper Database: 4.51E-05 atm-m3/mole (4.57E+000 Pa-m3/mole) User-Entered Henry LC: not entered Henrys LC [via VP/WSol estimate using User-Entered or Estimated values]: HLC: 8.722E-005 atm-m3/mole (8.837E+000 Pa-m3/mole) VP: 473 mm Hg (source: User-Entered) WS: 4.22E+005 mg/L (source: User-Entered)

5.4.3 Distribution modelling

Endpoint study record: Key. K2.Distribution modelling Epiwin2012

| UUID |  | IUC5-fa2c7544-6091-448a-9e3c-49c07cd2f1cf |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | Collette / Epona Associates, LLC / Willington / United States |
| Date |  | 2012-07-20 13:42:57 EDT |
| Remarks |  |  |

Administrative Data

| Purpose flag | key study |  |  |
| --- | --- | --- | --- |
| Study result type | estimated by calculation |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Accepted calculation method |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| other: EPIWin calculation |  | 2012 |  | U.S. Environmental Protection Agency (U.S. EPA). Estimation Programs Interface Suite,™. for Microsoft® Windows, v4.10. United States Environmental Protection Agency, Washington, DC, USA. |  |  |  |  |  |

Materials and methods

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| other: Epiwin decsription | Chem Name : Isopropylamine Molecular Wt: 59.11 |

Any other information on materials and methods incl. tables

Level III Fugacity Model (Full-Output): ======================================= Chem Name : Isopropylamine Molecular Wt: 59.11 Henry's LC : 1.34e-005 atm-m3/mole (user-entered; Bond estimate) Vapor Press : 473 mm Hg (user-entered) Log Kow : -0.5 (user-entered) Soil Koc : 19.5 (KOCWIN MCI method)

Results and discussions

Percent distribution in media

Air (%)

0.877

Water (%)

36.1

Soil (%)

62.9

Sediment (%)

0.0956

Any other information on results incl. tables

Level III Fugacity Model (Full-Output): ======================================= Mass Amount Half-Life Emissions (percent) (hr) (kg/hr) Air 0.877 6.52 1000 Water 36.1 360 1000 Soil 62.9 720 1000 Sediment 0.0956 3.24e+003 0 Fugacity Reaction Advection Reaction Advection (atm) (kg/hr) (kg/hr) (percent) (percent) Air 4.05e-011 1.04e+003 98.1 34.8 3.27 Water 4.58e-010 777 404 25.9 13.5 Soil 1.15e-008 677 0 22.6 0 Sediment 4.13e-010 0.229 0.0214 0.00762 0.000713 Persistence Time: 373 hr Reaction Time: 448 hr Advection Time: 2.23e+003 hr Percent Reacted: 83.3 Percent Advected: 16.7 Half-Lives (hr), (based upon Biowin (Ultimate) and Aopwin): Air: 6.518 Water: 360 Soil: 720 Sediment: 3240 Biowin estimate: 3.093 (weeks) Advection Times (hr): Air: 100 Water: 1000 Sediment: 5e+004

Endpoint study record: HSDB 2005, RL4

| UUID |  | IUC5-2b388502-4d3e-4206-b4fa-2d05eadcaf94 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-12-30 09:51:12 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Study result type | estimated by calculation |  |  |
| --- | --- | --- | --- |
| Reliability | 4 (not assignable) |  |  |
| Rationale for reliability incl. deficiencies | Secondary source |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| other: Online database | ChemIDplus (2005) | 2005 | 2-Propylamine [CAS No. 75-31-0] | Nat. Library of Medicine (NLM), Specialized Information Service (SIS), 25 July 2005 [http://chem.sis.nlm.nih.gov/chemidplus/] |  |  |  |  |  |
| other: Online database | HSDB (2005) | 2005 | 2-Propylamine [CAS No. 75-31-0] | Hazardous Substances Databank, database of the National Library of Medicine's TOXNET system, 29 June 2005 (http://toxnet.nlm.nih.gov) |  |  |  |  |  |

Materials and methods

Model

other: calculation

Media

water - air

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |
| EC number | 200-860-9 |
| EC name | isopropylamine |

Overall remarks, attachments

Overall remarks

Henry's Law constant = approx. 4.6 Pa*m3/mol (77314 Pa*m3/16920 mol) "The Henry's Law constant of isopropylamine is estimated as 4.5X10exp-5 atm*m3/mol(SRC) from itsvapor pressure of 580 mm Hg at 25 deg C(1) and water solubility of 1X10exp+6 mg/l(2) at 20°C.This Henry's Law constant indicates that the neutral form of isopropylamine is expected to volatilizefrom water surfaces(3). Based on this Henry's Law constant, the volatilization half-life from a model river (1 m deep,flowing 1 m/sec, wind velocity of 3 m/sec)(2) is estimated as approximately 17 hours(SRC). The volatilization half-life from a model lake (1 m deep, flowing 0.05 m/sec, wind velocityof 0.5 m/sec)(2) is estimated as 182 hours(SRC). With a pKa of 10.63(4), isopropylamine will exist predominantly in its protonated form in theenvironment, and the protonated form of isopropylamine is not expected to volatilize from water ormoist soil surfaces(SRC)." (HSDB 2005) References: (1) Daubert TE, Danner RP; Physical and Thermodynamic Properties of Pure Chemicals Data CompilationWashington,DC: Taylor and Francis, (1989) (2) Yalkowsky SH, Dannenfelser RM; Aquasol Database of Aqueous Solubility. Version 5. Collegeof Pharmacy, University of Arizona - Tucson, AZ (1992) (3) Lyman WJ et al; Handbook of Chemical Property Estimation Methods. Washington,DC:Amer Chem Soc pp. 15-1 to 15-29 (1990) (4) Perrin CL, Fabian MA; Anal Chem 68: 2127-34 (1996)] -----------------

6 Ecotoxicological Information

Endpoint summary: Ecotoxicological Information

| UUID |  | IUC5-3cec9c38-55fe-49ef-b0a4-04d4fe80b289 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:58:05 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

EU: REACH

PNEC aqua (freshwater)

| PNEC aqua (freshwater) |  | 19 µg/L |  |
| --- | --- | --- | --- |
| Assessment factor | 1000 | Extrapolation method | assessment factor |

Justification for (no) PNEC freshwater derivation

There are short-term results from three trophic levels, the alga test with the lowest EC50 of 18.9 mg/L, rounded to 19 mg/L.

PNEC aqua (marine water)

| PNEC aqua (marine water) |  | 1.9 µg/L |  |
| --- | --- | --- | --- |
| Assessment factor | 10000 | Extrapolation method | assessment factor |

Justification for (no) PNEC marine water derivation

No data available. The PNEC has been derived from freshwater data: There are short-term results from three trophic levels, the alga test with the lowest EC50 is 18.9 mg/L.

PNEC aqua (intermittent releases)

| PNEC aqua (intermittent releases) |  | 0.19 mg/L |  |
| --- | --- | --- | --- |
| Assessment factor | 100 | Extrapolation method | assessment factor |

Justification for (no) PNEC intermittent releases derivation

The PNEC is based on the lowest EC50 (alga) of 18.9 mg/L, divided by 100.

PNEC STP

| PNEC STP |  | 30 mg/L |  |
| --- | --- | --- | --- |
| Assessment factor | 20 | Extrapolation method | assessment factor |

Justification for (no) PNEC STP derivation

PNEC derivation following TGD: An assessment factor of 10 can be applied to the NOEC or EC10 of a standard respiration inhibition test (e.g. performed acc. to OECD TG 209). The available respiration inhibition data on activated sludge was generated according to the standard procedures. The lowest toxicity value was quoted as EC20, which was approx. 600 mg/L. The NOEC/EC10 is estimated to be 300 mg/L (additional factor of 2).

PNEC sediment (freshwater)

| PNEC sediment (freshwater) |  | 1.9 mg/kg sediment dw |
| --- | --- | --- |
| Extrapolation method | partition coefficient |  |

Justification for (no) PNEC sediment derivation

Estimation is based on the PNECaqua (freshwater) and the Koc of ~100 (dry weight) for pH neutral soil, experimentally obtained with n-butylamine (readacross) ==> PNEC(soil): 0.019 mg/L *100 = 1.9 mg/kg sed dw.

PNEC sediment (marine water)

| PNEC sediment (marine water) |  | 0.19 mg/kg sediment dw |
| --- | --- | --- |
| Extrapolation method | partition coefficient |  |

Justification for (no) PNEC marine sediment derivation

Estimation is based on the PNECaqua (marine) and the Koc of ~100 (dry weight) for pH neutral soil, experimentally obtained with n-butylamine (readacross) ==> PNEC(soil): 0.0019 mg/L *100 = 0.19 mg/kg sed dw.

PNEC soil

| PNEC soil |  | 1.9 mg/kg soil dw |
| --- | --- | --- |
| Extrapolation method | partition coefficient |  |

Justification for (no) PNEC soil derivation

Estimation is based on the PNECaqua (freshwater) and the Koc of ~100 (dry weight) for pH neutral soil, experimentally obtained with n-butylamine (readacross) ==> PNEC(soil): 0.019 mg/L *100 = 1.9 mg/kg soil dw. [in compliance with Guidance R.10, Equation R.10-5, p. 40]

PNEC oral

No potential for bioaccumulation

Justification for (no) PNEC oral derivation

not relevant, as no accumulation in the food chain is expected due to the intrinsic properties of the TS.

Discussion

The lowest LC/EC50 values of acute aquatic toxicity range between 10 and 100 mg/L for isopropylamine (fish, daphnia, alga). The lowest EC10 and NOEC are 2.6 and 1.25 mg/L, respectively, in alga. The amine is readily biodegradable and lacks bioaccumulating potential (log Pow <<3).

Conclusion on classification

Environmental classification justification

No classification for environmental hazards is required because of ready biodegradability and lack of bioaccumulation potential.

6.1 Aquatic toxicity

Endpoint summary: Aquatic toxicity

| UUID |  | IUC5-983323f7-16f1-48b8-84ac-5733cf91d224 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:58:05 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

EU: REACH

Discussion

Summary of acute aquatic toxicity data (Note: Data relate to test solutions without additional neutralisation) Test; Source; Species; EC/LC50 [mg/L]; EC/LCx [mg/L]; # Reliability. ## Fish. Fish 96 h, DIN 38412, part 15 / OECD 203; Hoechst AG 1979a; Oncorhynchus mykiss; 40; LD0 = 25; 2. Fish 96 h, DIN 38412, part 15 / OECD 203; Hoechst AG 1979b; Lepomis gibbosus; 88; LD0 = 25; 2. ## Daphnia. Daphnia, 48 h, OECD 202 / EU C.2; BASF AG 1988; Daphnia magna; 47.4; EC0 = 31.2; 2. Daphnia, 48 h, ASTM 1986 / EPA 1986, 1987; Chester et a. 1994; Daphnia magna; 91.6; EC10 ~50; 2. ## Alga. Alga, 72 h, DIN 38412, part 9; BASF AG 1988; Scenedesmus subspicatus; 18.9; ECr10 ~2.6; 2. Alga, 96 h, EPA 1986; Chester et al. 1994; Scenedesmus capricornutum; <62.5; ECr96 = 62.5; 2.

6.1.1 Short-term toxicity to fish

Endpoint summary: Short-term toxicity to fish

| UUID |  | IUC5-2661f0b7-35e5-405d-92b0-c377054acec5 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:58:06 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

EU: REACH

Short description of key information

Oncorhynchus mykiss: LC50(96 h) = 40 mg/L; LC0 = 25 mg/L (nominal) Lepomis gibbosus: LC50(96 h) = 88 mg/L; LC0 = 25 mg/L (nominal)

Key value for chemical safety assessment

| LC50 for freshwater fish | 40 mg/L |
| --- | --- |

Discussion

Two valid standard tests with fish showed consistent results with the LC0 and LC50 values ranging distinctly above 10 mg/L. No data on marine fish have been located. It is assumed that marine fish respond without substantial difference from freshwater fish.

Endpoint study record: Hoechst AG (+1979a, LC50, 96 h, fish, KS, RL2)

| UUID |  | IUC5-9b9543f4-27bb-4689-b0cd-1170113477de |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-12-30 09:51:18 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Purpose flag | key study; robust study summary; used for classification; used for MSDS |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Guideline study without detailed documentation |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | Markert and Weigand | 1979 | Akute Toxizitaet von Isopropylamin an Regenbogenforellen (Salmo gairderii) | unpublished | Hoechst AG; Pharma Forschung Toxikologie | 773/79 | OXEA | T01344 | 1979-12-06 |

Data access

data submitter is data owner

Data protection claimed

yes, but willing to share

Materials and methods

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | other guideline: DIN 38412 Teil 15 |  |
| equivalent or similar to | OECD Guideline 203 (Fish, Acute Toxicity Test) |  |

Principles of method if other than guideline

Bestimmung der Wirkung von Wasserinhaltsstoffen auf Fische, DIN 38412 Teil 15

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |
| EC number | 200-860-9 |
| EC name | isopropylamine |

Details on test material

Isopropylamine [CAS No. 75-31-0], pure, not further specified

Analytical monitoring

no

Vehicle

no

Test organisms

Test organisms (species)

Salmo gairdneri (new name: Oncorhynchus mykiss)

Study design

Test type

static

Water media type

freshwater

Limit test

no

Total exposure duration

96 h

Post exposure observation period

no

Test conditions

Hardness

Total: 9.5°d Carbonate: 6.4°d

Test temperature

12 +/- 1°C

pH

7.9 - 8.4 (without test substance) intermittent increase up to 10.2 due to addition of test substance (no further details given)

Dissolved oxygen

> 8 mg/L

Salinity

NaHCO3: 192 mg/L, CaSO4*2H2O:120 mg/L MgSO4: 120 mg/L KCl: 8 mg/L

Nominal and measured concentrations

0, 1, 10, 16, 25, 40, 63, 80, 100 mg/L (nominal)

Details on test conditions

Number of fish: 10 per test concentration TEST SYSTEM - Test vessel: - Type: open - Material, size, headspace, fill volume: Glass, 40 cm x 25 cm x 30 cm filled with 20 L water - Aeration: by glass capillaries, ca. 150 ml air per minute - Renewal rate of test solution (frequency/flow rate): static - No. of organisms per vessel: 5 or 10 in first and second experiment, respectively - No. of vessels per concentration (replicates): 2 or 1 in first and second experiment, respectively - No. of vessels per control (replicates): 2 or 1 in first and second experiment, respectively - Biomass loading rate: ca. 33 g/ 20 L (1.65 g/ L) TEST MEDIUM / WATER PARAMETERS - Source/preparation of dilution water: Deionised water - Total organic carbon: - Chlorine: Dechlorinated - Alkalinity: see above (pH) - Ca/Mg ratio: see above (Salinity) - Conductivity: no data - Intervals of water quality measurement: 24h OTHER TEST CONDITIONS - Adjustment of pH: no - Photoperiod: 12h - Light intensity: ca. 700 Lux EFFECT PARAMETERS MEASURED (with observation intervals if applicable) : mortality TEST CONCENTRATIONS - Spacing factor for test concentrations: ca. 1.6 (second experiment) - Justification for using less concentrations than requested by guideline: - Range finding study - Test concentrations: First Experiment 0, 1, 10, 100 mg/ L (full test)

Reference substance (positive control)

no

Any other information on materials and methods incl. tables

Oxygen: 8.1 - 9.5 mg/L The pH was not neutralised and initially increased in concentration-related manner (at maximum pH 10.2 at 100 mg/L). But the pH normalised during the test period and was in the range of the control values by the end of the test. (No detailed data tables documented.) ----------------

Results and discussions

Effect concentrations

| Duration | Endpoint | Effect conc. | Nominal/Measured | Conc. based on | Basis for effect | Remarks (e.g. 95% CL) |
| --- | --- | --- | --- | --- | --- | --- |
| 48 h | LC100 | 63 mg/L | nominal | test mat. | mortality |  |
| 48 h | LC50 | 40 mg/L | nominal | test mat. | mortality |  |
| 48 h | LC0 | 25 mg/L | nominal | test mat. | mortality |  |
| 96 h | LC100 | 63 mg/L | nominal | test mat. | mortality |  |
| 96 h | LC50 | 40 mg/L | nominal | test mat. | mortality |  |
| 96 h | LC0 | 25 mg/L | nominal | test mat. | mortality |  |
| 96 h | EC0 | 10 mg/L | nominal | test mat. | behaviour |  |

Any other information on results incl. tables

MORTALITY No deaths were observed at 25 mg/L. Test number of animals concentration dead/alive [96 h] ===================================== 0 mg/L 0/20 . 40 mg/L 5/10 63 mg/L 10/10 80 mg/L 10/10 100 mg/L 10/10 ===================================== Signs of Intoxication No toxicity and no symptoms were evident at the concentrations of 1 and 10 mg/L.All others: dark discoloration, apathy, imbalance,dorsal position at the bottom of the vessel. Deceased animals showed a wide-opened mouth and swollen gills. The surving animals showed no visible abnormalities. -----------------

Applicant's summary and conclusion

Validity criteria fulfilled

yes (however, no analysis performed)

Cross-reference to other study

Hoechst AG (+1979b, DIN, OECD203, SS, RL2)

Endpoint study record: Hoechst AG (+1979b, LC50, 96 h, fish, SS, RL2)

| UUID |  | IUC5-954b0c54-5ddf-4354-84f3-ac1532aa4722 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-12-30 09:51:24 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Purpose flag | supporting study; robust study summary; used for MSDS |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Guideline study without detailed documentation |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | Markert and Weigand | 1979 | Akute Toxizitaet von Isopropylamin an Sonnenbarschen (Lepomis gibbosus) | unpublished | Hoechst AG; Pharma Forschung Toxikologie | 774/79 | OXEA | T01345 | 1979-12-06 |

Data access

data submitter is data owner

Data protection claimed

yes, but not willing to share

Materials and methods

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| equivalent or similar to | OECD Guideline 203 (Fish, Acute Toxicity Test) |  |

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |
| EC number | 200-860-9 |
| EC name | isopropylamine |

Details on test material

Isopropylamine [CAS No. 75-31-0], pure, not further specified

Analytical monitoring

no

Vehicle

no

Test organisms

Test organisms (species)

Lepomis gibbosus

Study design

Test type

static

Water media type

freshwater

Limit test

no

Total exposure duration

96

Test conditions

Hardness

Total: 9.5°d (due to disolved salts) Carbonate: 6.4

Test temperature

22 +/-1°C

pH

7.5 - 8.5 (without test substance) Initially increased after application of test substance in groups 16 - 100mg/L; intermittent increase up to 10.1 due to addition of test substance at 100 mg/ml

Dissolved oxygen

> 7 mg/L

Salinity

NaHCO3: 192 mg/L, CaSO4*2H2O:120 mg/L MgSO4: 120 mg/L KCl: 8 mg/L

Nominal and measured concentrations

0, 10, 16, 25, 40, 63, 71, 80, 90, 100 mg/l (nominal)

Details on test conditions

Number of fish: 10 per test concentration TEST SYSTEM - Test vessel: - Type: open - Material, size, headspace, fill volume: Glass, 40 cm x 25 cm x 30 cm filled with 20 L water - Aeration: by glass capillaries, ca. 150 mL air per minute - Renewal rate of test solution (frequency/flow rate): static - No. of organisms per vessel: 5 in first and 10 in second and third experiment, respectively - No. of vessels per concentration (replicates): 2 in first and 1 in second and third experiment, respectively - No. of vessels per control (replicates): 2 in first and 1 in second and third experiment, respectively - Biomass loading rate: ca. 28 g/ 20 L (1.4 g/ L) TEST MEDIUM / WATER PARAMETERS - Source/preparation of dilution water: Deionised water - Total organic carbon: no data - Chlorine: Dechlorinated - Alkalinity: see above (pH) - Ca/Mg ratio: see above (Salinity) - Conductivity: no data - Intervals of water quality measurement: 24h OTHER TEST CONDITIONS - Adjustment of pH: no - Photoperiod: 12h - Light intensity: ca. 700 Lux EFFECT PARAMETERS MEASURED (with observation intervals if applicable) : mortality TEST CONCENTRATIONS - Spacing factor for test concentrations: ca. 1.6 (=< 63 mg/L), 1.1 (>= 71 mg/L) - Justification for using less concentrations than requested by guideline: - Range finding study - Test concentrations: First Experiment 0, 10, 100 mg/ L (full test)

Reference substance (positive control)

no

Any other information on materials and methods incl. tables

The pH was not neutralised and initially increased in concentration-related manner (pH 10.1 at 100 mg/L). But the pH normalised during the test period and was in the range of the control values by the end of the test. (No detailed data tables documented.)

Results and discussions

Effect concentrations

| Duration | Endpoint | Effect conc. | Nominal/Measured | Conc. based on | Basis for effect | Remarks (e.g. 95% CL) |
| --- | --- | --- | --- | --- | --- | --- |
| 48 h | LC50 | 97 mg/L | nominal | test mat. | mortality |  |
| 48 h | LC0 | 25 mg/L | nominal | test mat. | mortality |  |
| 96 h | LC100 | >= 100 mg/L | nominal | test mat. | mortality |  |
| 96 h | LC50 | 88 mg/L | nominal | test mat. | mortality |  |
| 96 h | LC0 | 25 mg/L | nominal | test mat. | mortality |  |
| 96 h | EC0 | 25 mg/L | nominal | test mat. | behaviour |  |

Any other information on results incl. tables

MORTALITY No deaths were observed at 25 mg/L. Test number of animals concentration dead/alive [96 h] ====================================== 0 mg/L 0/30 . 40 mg/L 1/10 63 mg/L 0/10 71 mg/L 1/10 80 mg/L 1/10 90 mg/L 6/10 100 mg/L 10/10 ====================================== SIGNS of INTOXICATION No toxicity was evident at the concentrations of 10 - 25 mg/L. All others: dark discoloration, apathy, imbalance, reduced respiration rate, narcosis. The surving animals showed no visible abnormalities.

Applicant's summary and conclusion

Validity criteria fulfilled

yes (however, no analysis performed)

Cross-reference to other study

Hoechst AG (+1979a, DIN, OECD203, KS, RL2)

Endpoint study record: EPIWIN, 2010, Short-term toxicity to fish

| UUID |  | IUC5-d9d0cade-03e5-44d1-8126-ee5213334a3d |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | Collette / Epona Associates, LLC / Willington / United States |
| Date |  | 2012-07-02 15:19:52 EDT |
| Remarks |  |  |

Administrative Data

| Purpose flag | supporting study |  |  |
| --- | --- | --- | --- |
| Study result type | estimated by calculation |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Accepted calculation method |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| other: modelling |  | 2010 |  | U.S. Environmental Protection Agency (U.S. EPA) Estimation Programs Interface Suite,™. for Microsoft® Windows, v4.00. United States Environmental Protection Agency, Washington, DC, USA. |  |  |  |  |  |

Materials and methods

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test conditions

Any other information on materials and methods incl. tables

ECOSAR Program (v1.00): ============================== SMILES : C(C)(C)N CHEM : Isopropylamine MOL FOR: C3 H9 N1 MOL WT : 59.11 Log Kow: 0.27 (KowWin estimate) Melt Pt: -90.00 deg C Wat Sol: 4.218E+005 mg/L (measured)

Results and discussions

Effect concentrations

| Duration | Endpoint | Effect conc. | Nominal/Measured | Conc. based on | Basis for effect | Remarks (e.g. 95% CL) |
| --- | --- | --- | --- | --- | --- | --- |
| 96 h | LC50 | 114 mg/L |  |  |  |  |

Details on results

ECOSAR Program (v1.00) Results: ============================== ECOSAR v1.00 Class(es) Found ------------------------------ Aliphatic Amines Predicted ECOSAR Class Organism Duration End Pt mg/L (ppm) ============================================= Aliphatic Amines : Fish 96-hr LC50 114.031 ============================================= Neutral Organic SAR : Fish 96-hr LC50 1324.745 (Baseline Toxicity) Aliphatic Amines: ---------------- For Fish 96-hr LC50: For aliphatic amines with log Kow greater than 7.0, a test duration of greater than 96 hrs may be required for proper expression of toxicity. Also, if the toxicity value obtained by the use of this equation exceeds the water solubility (measured or estimated), mortalities greater than 50% would not be expected in a saturated solution during an exposure period of 96 hrs. ECOSAR v1.00 SAR Limitations: ---------------------------- Maximum LogKow: 6.0 (Fish LC50) Maximum Mol Wt: 1000 Baseline Toxicity SAR Limitations: --------------------------------- Maximum LogKow: 5.0 (Fish 96-hr LC50) Maximum Mol Wt: 1000

Endpoint study record: Protic (+1989, 96 h, RL4)

| UUID |  | IUC5-b006a36e-3728-401b-8c6f-bcda7c6a1d3f |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-12-30 09:51:30 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Purpose flag | supporting study |  |  |
| --- | --- | --- | --- |
| Study result type | (Q)SAR |  |  |
| Reliability | 4 (not assignable) |  |  |
| Rationale for reliability incl. deficiencies | 4c: Original reference not yet available |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Protic M. and Sabljic A. | 1989 | Quantitative structure-activity relationships of acute toxicity of commercial chemicals on fathead minnows: effect of molecular size | Aquatic Toxicology, 14, 47-64 |  |  |  | T04313 |  |

Data access

data published

Materials and methods

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| no guideline available | other guideline: (Q)SAR calculation |  |

GLP compliance

no data

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |
| EC number | 200-860-9 |
| EC name | isopropylamine |

Details on test material

no data

Test organisms

Test organisms (species)

Pimephales promelas

Study design

Test type

static

Water media type

freshwater

Total exposure duration

96 h

Results and discussions

Effect concentrations

| Duration | Endpoint | Effect conc. | Nominal/Measured | Conc. based on | Basis for effect | Remarks (e.g. 95% CL) |
| --- | --- | --- | --- | --- | --- | --- |
| 96 h | LC50 | 310 mg/L |  |  |  |  |

Applicant's summary and conclusion

Validity criteria fulfilled

no data

Endpoint study record: Tonogai (+1982, LC50, 48 h, RL4)

| UUID |  | IUC5-e5921947-dd02-41b3-83a1-b7011b0c8908 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-12-30 09:51:38 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Purpose flag | supporting study |
| --- | --- |
| Reliability | 4 (not assignable) |
| Rationale for reliability incl. deficiencies | Short communication, limited documentation, insufficient for assessment of results (1000 mg/L) |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Tonogai YS,Ogawa S, Ito Y and Iwaida M | 1982 | Actual Survey on TLM (Median Tolerance Limit) Values of Environmental Pollutants, Especially on Amines, Nitriles, Aromatic Nitrogen Compounds. | J. Toxicol. Sci. 7, 193-203 |  |  |  | T02717 |  |

Data access

data published

Materials and methods

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | other guideline: Japan Industrial Standards |  |

Principles of method if other than guideline

Method: Procedure of Japan Industrial Standards ((Japanese Industrial Standards Committee, 1971)

GLP compliance

no data

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |
| EC number | 200-860-9 |
| EC name | isopropylamine |

Details on test material

Isopropylamine [CAS No. 75-31-0], not further specified

Analytical monitoring

no

Vehicle

no

Details on test solutions

Chemical substances were dissolved in water and neutralized with 0.01 N NaOH or HCl if necessary

Test organisms

Test organisms (species)

Oryzias latipes

Details on test organisms

- Fish were acclimated for 10 days in tap water prior to the experiment - 10 fish were used per trial, length about 2 cm, weight, about 0.2 g - Test volume: 2 L (deion. water, 25 °C)

Study design

Test type

static

Water media type

freshwater

Total exposure duration

48 h

Test conditions

Details on test conditions

- Ten fish were kept in 2 L of deionized water at 25 °C - The TLm (mean lethal concentration) was determined at 24 and 48 hours

Results and discussions

Effect concentrations

| Duration | Endpoint | Effect conc. | Nominal/Measured | Conc. based on | Basis for effect | Remarks (e.g. 95% CL) |
| --- | --- | --- | --- | --- | --- | --- |
| 48 h | other: TLm | 1000 mg/L |  |  |  |  |

6.1.2 Long-term toxicity to fish

Endpoint study record: WAIVER_Long-term toxicity to fish

| UUID |  | IUC5-41f83986-255a-4b3c-ad29-55842bc5eddf |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:51:41 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| EU: REACH |  |  |  |
| --- | --- | --- | --- |
| Data waiving | other justification |  |  |
| Justification for data waiving | In accordance with column 2 of Annex IX (required in Section 9.1) of REACH Regulation 1907/2006: no study indicated for isopropylamine, given its low acute aquatic toxicity, ready biodegradability, and lack of a bioaccumulating potential ==> not classified as hazardous to the environment |  |  |
| Study result type | other: expert judgement |  |  |
| Reliability | 1 (reliable without restriction) |  |  |
| Rationale for reliability incl. deficiencies | Acceptable, well-documented statement which meets basic scientific principles |  |  |

Materials and methods

Test materials

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |
| EC number | 200-860-9 |
| EC name | isopropylamine |

Overall remarks, attachments

Overall remarks

Collection of arguments - one comprehensive Ames test including two sources of metabolic enzyme systems based on the preincubation test variant - one

6.1.3 Short-term toxicity to aquatic invertebrates

Endpoint summary: Short-term toxicity to aquatic invertebrates

| UUID |  | IUC5-a428a9c3-3a24-4234-bd19-e763f6182649 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:58:07 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

EU: REACH

Short description of key information

Daphnia magna: EC0(48 h) = 32 mg/L; EC10(48 h) = ~50 mg/L; EC50(48 h) = 47.4 and 91.6 mg/L (nominal)

Key value for chemical safety assessment

| EC50/LC50 for freshwater invertebrates | 47 mg/L |
| --- | --- |

Discussion

Two valid standard tests with daphnia showed consistent results with the EC0 and EC50 values ranging distinctly above 10 mg/L. No data is available on marine species. It is assumed that marine species respond without substantial difference from daphnia.

Endpoint study record: Key.BASFAG1/0176/2/88-0176/88.Short-term toxicity to aquatic invertebrates

| UUID |  | IUC5-4adcae0a-65f1-475c-9f90-46250b78bb78 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-12-30 10:02:24 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Purpose flag | key study; robust study summary; used for MSDS |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Non GLP-guideline study without analytical monitoring |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | BASF AG | 1988 |  |  | Department of Ecology |  | BASF AG | 1/0176/2/88-0176/88 | 1988-04-19 |

Data access

data submitter has Letter of Access

Data protection claimed

yes

Materials and methods

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | other guideline: EU directive 79/831/EEC, Annex V, Part C |  |

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |
| EC name | isopropylamine |

Details on test material

- Name of test material (as cited in study report): Isopropylamine - Analytical purity: > 99% - Composition of test material, percentage of components: waterfree

Analytical monitoring

no

Vehicle

no

Test organisms

Test organisms (species)

Daphnia magna

Details on test organisms

- age of animals: 2-24 h

Study design

Test type

static

Water media type

freshwater

Limit test

no

Total exposure duration

48 h

Post exposure observation period

none

Test conditions

Hardness

water-hardness: 2.8 mmol/L

Test temperature

20-22 °C (292-294 K)

pH

range of pH at start: 8.4 (1.95 mg/L) - 10.95 (250 mg/L); range of pH after 48 h: 7.94 (3.9 mg/L) - 9.06 (250 mg/L)

Dissolved oxygen

range of O2 (mg/L) at start: 8.44 (3.9 mg/L) - 8.6 (control); range of O2 (mg/L) after 48 h: 7.98 (62.4 mg/L) - 8.39 (250 mg/L)

Nominal and measured concentrations

- test concentrations: 1.95, 3.9, 7.8 15.6, 31.2, 62.4, 125, 250 mg/L (nominal)

Details on test conditions

Test conditions: - dilution water: source: tap water; pretreatment steps: (1) 6 µm- and charcoal-filtration (2) H2SO4 was added to reduce alkalinity up to pH 4.3 (3) distilled water was added to reduce water-hardness (4) water was aerated (oil-free air) until saturated with oxygen (5) water was stored for at least 24 h for stabilization. Specifications measured at test start: water-hardness: 2.8 mmol/L, alkalinity up to pH 4.3: 0.83 mmol/L, pH: 8.0, conductivity: 640 µSiemens/cm - water solubility: >500 mg/L at 21 °C (293 K) - illumination: diffuse light - test volume: 10 ml - test vessels: test tubes (glass) with flat bottom (nominal volume 20 ml) - replicates: 4 per concentration - volume/animal: 2 ml - number of animals/vessel: 5 - total number of animals/conc.: 20 - observation times: visually after 0, 3, 6, 24 and 48 h - observation parameters: swimming ability, pH, oxygen - test was performed in non-sealed vessels

Reference substance (positive control)

no data

Any other information on materials and methods incl. tables

Procedures to determine EC-values after 48 h: - EC50: according to Spearman-Kaerber - EC0: highest concentration tested at which <= 10 % of the animals were immobile - EC100: lowest tested concentration at which 100 % of the animals were immobile;

Results and discussions

Effect concentrations

| Duration | Endpoint | Effect conc. | Nominal/Measured | Conc. based on | Basis for effect | Remarks (e.g. 95% CL) |
| --- | --- | --- | --- | --- | --- | --- |
| 48 h | EC0 | 31.2 mg/L | nominal | test mat. | mobility |  |
| 48 h | EC50 | 47.4 mg/L | nominal | test mat. | mobility | 95%C.L. = 41.58 - 53.96 mg/L |
| 48 h | EC100 | 125 mg/L | nominal | test mat. | mobility |  |

Any other information on results incl. tables

Number of mobile test animals after exposure (48 h) to various test concentrations: concentration (mg/L) mobile Daphnids 1.9520 3.920 7.820 15.620 31.219 62.4 3 125.0 0 250.0 0 control20 effect values after 48 h: EC0 = 31.2 mg/L EC50 = 47.37 mg/L 95 % confidence limits: 41.58 - 53.96 mg/L EC100 = 125 mg/L effect values after 24 h: EC0 = 31.2 mg/L EC50 = 49.04 mg/L 95 % confidence limits: 43.87 - 54.81 mg/L EC100 = 125 mg/L remarks in the report: Toxicity at 62.5 - 250 mg/L may be due to the high pH (up to 11.95)

Applicant's summary and conclusion

Validity criteria fulfilled

yes (O2 >3 mg/L, no mortality in control )

Endpoint study record: Chester (+1994, 48h, daphnia, RL2)

| UUID |  | IUC5-cf295c62-f27d-4050-a1c4-4001c8489344 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-12-30 10:02:34 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Purpose flag | supporting study; robust study summary |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Guideline study without detailed documentation |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Chester NA, Haley MV, Landis WG | 1994 | The Aquatic Toxicity of Isopropylamine: Comparison of Experimentally Derived Values with Structure Activity Predictions | Animal Test Alternatives, pp. 37-46 | Chemical Research Development and Engineering Center, MD/USA |  |  | T04894 |  |

Data access

data published

Materials and methods

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | other guideline: ASTM 1986: Guide for Conducting Acute Toxicity Tests With Fishes, Macroinvertebrates, and Amphibians, Standard E729, American Society for Testing and Materials, Philadelphia |  |
| according to | other guideline: EPA 1986: Part 797, Environmental effects testing guidelines, subpart 3, aquatic guidelines, CFR 40, part 700 to end. U.S. Government Printing Office, Washington, D.C., 1986. |  |
| according to | other guideline: EPA 1987: User's Guide: Procedures for Conducting Daphnia magna Toxicity Bioassays, EPA-600/8-87/011. U.S. Environmental Protection Agency, Las Vegas, NV 1987. |  |

GLP compliance

no data

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |
| EC number | 200-860-9 |
| EC name | isopropylamine |

Details on test material

isopropylamine, purity not specified

Analytical monitoring

no

Vehicle

no

Details on test solutions

STOCK AND TEST SOLUTION PREPARATION: - A buffered (pH 7.0) and a non-buffered test were included. - The control treatment included adjustment of the pH (pH 8.3-11) DILUTION WATER: - pH: 7.0 (buffered), 7.3-10.3 (unbuffered) - Adjustment of pH: separate test with buffered solutions (pH 7.0) and control with adapted pH

Test organisms

Test organisms (species)

Daphnia magna

Details on test organisms

TEST ORGANISM - Source: test laboratory, originally University Washington (Dr. Frieda Taub) - Age at study initiation (mean and range, SD): no data - Method of breeding: Daphnia reared in the laboratory according to: Goulden, C.E., Comotto, R.M., Hendrickson, J.A., Jr., Hornig, L.L., and Johnson, K.L. 1982: Procedures and recommendations for the culture and use of Daphnia in bioassay studies, Aquatic toxicology and Hazard Assessment: Fifth Conference, ASTM STP-766 (J.G. Pearson, R.B. Foster, and W.E. Bishop, eds.), American Society for Testing and Materials, Philadelphia, 1982

Study design

Test type

static

Water media type

freshwater

Limit test

no

Total exposure duration

48 h

Test conditions

Test temperature

20 +/- 1 °C

pH

first assay: 7.3-10.3 second assay: adjusted to 7.0 with 10% HCl, resulting in solution pH ranging from 7.1-7.4

Nominal and measured concentrations

Unbuffered concentrations (extracted graphically): 50, 100, 150, 200, and 250 mg/L (nominal) Buffered concentrations (extracted graphically): ca. 31, 63, 125, 250, and 500 mg/L (nominal)

Details on test conditions

TEST SYSTEM - Test vessel: no data - Type of flow-through (e.g. peristaltic or proportional diluter): no - No. of organisms per vessel: no data - No. of vessels per concentration (replicates): no data - No. of vessels per control (replicates): no data - No. of vessels per vehicle control (replicates): no data TEST MEDIUM / WATER PARAMETERS no data OTHER TEST CONDITIONS - Adjustment of pH: first assay: 7.3-10.3 second assay: adjusted to 7.0 with 10% HCl, resulting in solution pH ranging from 7.1-7.4 CONTROLS: - No isopropylamine was added, pH was adjusted from pH 8.3-11.0 with 0.5 M NaOH - Photoperiod: 16 h light (315 ft-candles)/8 hour dark - Light intensity: no data EFFECT PARAMETERS MEASURED (with observation intervals if applicable): mortality TEST CONCENTRATIONS - Spacing factor for test concentrations: 2 (buffered) - Range finding study: no data

Reference substance (positive control)

no

Results and discussions

Effect concentrations

| Duration | Endpoint | Effect conc. | Nominal/Measured | Conc. based on | Basis for effect | Remarks (e.g. 95% CL) |
| --- | --- | --- | --- | --- | --- | --- |
| 48 h | EC50 | 197.6 — 204 mg/L | nominal | test mat. | mortality | Mean EC (according to Tab. 1 and Fig. 1); buffered, 95% CI: 167.4-230.6 mg/L |
| 48 h | EC50 | 91.6 mg/L | nominal | test mat. | mortality | Unbuffered, 95% CI: 77.1-104.2 mg/L; pH not given |
| 48 h | EC10 | ca. 50 mg/L | nominal | test mat. | mortality | graphically derived from dose-response curve of unbuffered test medium (Fig. 1) |
| 48 h | EC100 | ca. 150 mg/L | nominal | test mat. | mortality | graphically derived from dose-response curve of unbuffered test medium (Fig. 1) |

Details on results

Mortality in control treatment at pH 8.3-11: 0 - 35%

Reported statistics and error estimates

The EC50 values were calculated using probit analysis and were verified with graphically tabulated EC50 values derived from mortality and concentration values.

Applicant's summary and conclusion

Validity criteria fulfilled

no data

Endpoint study record: EPIWIN 2010, Short-term toxicity to aquatic invertebrates

| UUID |  | IUC5-44906c06-3ae6-4e0b-a7c9-0b8b4504ee66 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | Collette / Epona Associates, LLC / Willington / United States |
| Date |  | 2012-07-03 07:57:12 EDT |
| Remarks |  |  |

Administrative Data

| Purpose flag | supporting study |  |  |
| --- | --- | --- | --- |
| Study result type | estimated by calculation |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Accepted calculation method |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| other: modelling |  | 2010 |  | U.S. Environmental Protection Agency (U.S. EPA) Estimation Programs Interface Suite,™. for Microsoft® Windows, v4.00. United States Environmental Protection Agency, Washington, DC, USA. |  |  |  |  |  |

Materials and methods

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test conditions

Any other information on materials and methods incl. tables

ECOSAR Program (v1.00): ============================== SMILES : C(C)(C)N CHEM : Isopropylamine MOL FOR: C3 H9 N1 MOL WT : 59.11 Log Kow: 0.27 (KowWin estimate) Melt Pt: -90.00 deg C Wat Sol: 4.218E+005 mg/L (measured)

Results and discussions

Effect concentrations

| Duration | Endpoint | Effect conc. | Nominal/Measured | Conc. based on | Basis for effect | Remarks (e.g. 95% CL) |
| --- | --- | --- | --- | --- | --- | --- |
| 48 h | EC50 | 8 mg/L |  |  |  |  |

Details on results

ECOSAR Program (v1.00) Results: ============================== ECOSAR v1.00 Class(es) Found ------------------------------ Aliphatic Amines Predicted ECOSAR Class Organism Duration End Pt mg/L (ppm) ============================================= Aliphatic Amines : Daphnid 48-hr LC50 8.046 =========================== ================== ======== ====== ========== Neutral Organic SAR (Baseline Toxicity) : Daphnid 48-hr LC50 620.180 Aliphatic Amines: ---------------- For Daphnid 48-hr LC50: For aliphatic amines with log Kow greater than 5.0, a test duration of greater than 48 hrs may be required for proper expression of toxicity. Also, if the toxicity value obtained by the use of this equation exceeds the water solubility (measured or estimated), significant mortalities would not be expected in a saturated solution during an exposure period of 48 hrs. ECOSAR v1.00 SAR Limitations: ---------------------------- Maximum LogKow: 5.0 (Daphnid LC50) Maximum Mol Wt: 1000 Baseline Toxicity SAR Limitations: --------------------------------- Maximum LogKow: 5.0 (Daphnid 96-hr LC50) Maximum Mol Wt: 1000

6.1.4 Long-term toxicity to aquatic invertebrates

Endpoint study record: WAIVER_Long-term toxicity to aquatic invertebrates

| UUID |  | IUC5-6c84802e-933b-4a56-82b5-52f729882625 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:51:42 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| EU: REACH |  |  |  |
| --- | --- | --- | --- |
| Data waiving | other justification |  |  |
| Justification for data waiving | In accordance with column 2 of Annex IX (required in Section 9.1) of REACH Regulation 1907/2006: no study indicated for isopropylamine, given its low acute aquatic toxicity, ready biodegradability, and lack of a bioaccumulating potential ==> not classified as hazardous to the environment |  |  |
| Study result type | no data |  |  |

Materials and methods

Test materials

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |
| EC number | 200-860-9 |
| EC name | isopropylamine |

6.1.5 Toxicity to aquatic algae and cyanobacteria

Endpoint summary: Toxicity to aquatic algae and cyanobacteria

| UUID |  | IUC5-ee092482-5954-4392-b8c2-34d53e13f5e9 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:58:07 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

EU: REACH

Short description of key information

In fresh-water algae, the EC10 and EC50 were 2.6 and 18.9 mg isopropylamine/L, respectievely. The NOEC was 1.25 mg/L. No data is available for marine algae.

Key value for chemical safety assessment

| EC50/LC50 for freshwater algae | 18.9 mg/L |
| --- | --- |
| EC10/LC10 or NOEC for freshwater algae | 1.25 mg/L |

Discussion

Isopropylamine starts inhibiting algal growth at concentrations of greater than 1 mg/L with the EC50 ranging significantly above 10 mg/L and the NOEC close to 1 mg/L (BASF 1988). It is assumed that marine algae respond without substantial difference from freshwater algae.

Endpoint study record: Key.BASFAG2/0176/88.Toxicity to aquatic algae and cyanobacteria

| UUID |  | IUC5-760ebd8d-318c-4346-ad9e-ba63de643e6f |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-12-30 10:04:26 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Purpose flag | key study; robust study summary; used for MSDS |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Guideline study with acceptable restrictions (e.g. no analytical verification of test concentrations, non-GLP) |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | BASF AG | 1988 | Test protocol algae test with monoisopropylamine |  | Laboratory for Ecology | 2/0176/88 | BASF AG |  | 1988-10-20 |
| other company data | ECT Oekotoxikologie GmbH | 2007 | Statistical re-evaluation with ToxRatPro (v2.09): Alga, Growth inhibition test (OECD 201, EEC, ISO). Algae test Project No. 2/0176/88 |  | ECT Oekotoxikologie GmbH |  | BASF AG Department for Product Safety |  | 2007-10-30 |

Data access

data submitter has Letter of Access

Data protection claimed

yes

Materials and methods

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | other guideline: German industrial standard test guideline DIN 38412, part 9 |  |

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |
| EC name | isopropylamine |

Details on test material

- Name of test material (as cited in study report): Isopropylamine - Analytical purity: no data

Analytical monitoring

no

Vehicle

no

Test organisms

Test organisms (species)

Scenedesmus subspicatus (new name: Desmodesmus subspicatus)

Study design

Test type

static

Water media type

freshwater

Limit test

no

Total exposure duration

72 h

Test conditions

Test temperature

20 °C

pH

see below "Any other information on results ...."

Dissolved oxygen

---

Nominal and measured concentrations

0 (control), 0.156, 0.313, 0.625, 1.25, 2.5, 5, 10, and 20 mg/L (nominal)

Reference substance (positive control)

no data

Any other information on materials and methods incl. tables

Pre-culture: - medium: according to DIN 38412, part 9; OECD medium - culture flasks: 250-ml-Erlenmeyer flasks - culture volume: 100 ml Test culture: - No. of algae in test flasks at test start: 10000 cells/ml - coloration of test substance: clear liquid - test volume: 10 ml - tubes were incubated for 72 h at 20 °C - samples were taken at regular intervals (0, 24, 48, 72, and 96 h) - measurements: fluorescence Test conditions: - replicates: 5 per concentration and control; 2 blank per concentration and control (w/o cells)

Results and discussions

Effect concentrations

| Duration | Endpoint | Effect conc. | Nominal/Measured | Conc. based on | Basis for effect | Remarks (e.g. 95% CL) |
| --- | --- | --- | --- | --- | --- | --- |
| 72 h | NOEC | 1.25 mg/L | nominal | test mat. | growth rate | Based on re-calculation: 0.63 mg/L (see below) |
| 72 h | LOEC | 2.5 mg/L | nominal | test mat. | growth rate | Based on re-calculation: 1.25 mg/L (see below) |
| 72 h | EC10 | 2.6 mg/L | nominal | test mat. | growth rate |  |
| 72 h | EC20 | 5.1 mg/L | nominal | test mat. | growth rate |  |
| 72 h | EC50 | 18.9 mg/L | nominal | test mat. | growth rate |  |

Any other information on results incl. tables

The following nominal effect concentrations related to yield (EyC), growth rate (ErC), and biomass integral (EbC) were recalculated according to OECD TG 201 (adopted 23 Mar 2006) using the fluorescence values for inhibition of biomass from the original report. Since no calibration curve data were available, fluorescence data were equated with cell numbers (95 % confidence intervals in brackets): Yield (mg/l) ------------------ 72 h 96 h EyC10: 0.887 (0.725-1.05) 0.221 (0.088-0.370) EyC20: 1.51 (1.30-1.71) 0.414 (0.214-0.618) EyC50: 4.13 (3.78-4.51) 1.376 (0.994-1.90) NOEyC: 0.625 0.313 LOEyC: 1.25 0.625 Growth rate (mg/l) ------------------ 72 h 96 h ErC10: 2.56 (2.41-2.70) 0.833 (0.604-1.708) ErC20: 5.09 (4.90-5.28) 2.26 (1.84-2.67) ErC50: 18.9 (18.2-19.6) 15.0 (12.7-18.2) NOErC: 0.625 0.313 LOErC: 1.25 0.625 Biomass integral (mg/l) ----------------------- 72 h 96 h EbC10: 1.13 (0.739-1.52) 0.439 (0.309-0.576) EbC20: 1.99 (1.47-2.49) 0.846 (0.656-1.04) EbC50: 5.84 (4.94-6.97) 2.95 (2.56-3.42) NOEbC: 1.25 0.313 LOEbC: 2.5 0.625 Original effect values as given in the report (nominal concentrations in mg/l): 72 h 96 h EC20: 1.48 0.47 EC50: 4.13 1.2 EC90: n.d. 14.2 n.d.: not determined pH at test start (0 h) and at test end (72 h) --------------------------------------------- stock solution: - pH 10.6 (conductivity: 73 µS/cm) Conc[mg] 0 h 72 h uninoculated uninoculated inoculated 0 (control) 8.1 8.5 9.7 0.156 8.0 8.4 9.8 0.313 8.0 8.3 9.7 0.625 8.0 8.3 9.6 1.25 8.1 8.2 9.2 2.50 8.2 8.2 9.2 5 8.3 8.2 8.8 10 8.6 8.2 8.6 20 9.0 8.3 8.6 No data were given on oxygen and temperature development during the test.

Endpoint study record: Chester (+1994, 96h, alga, RL2)

| UUID |  | IUC5-87dfe712-6776-497c-ae20-f08bb6376972 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-12-30 10:04:33 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Purpose flag | supporting study; robust study summary |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Guideline study without detailed documentation |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Chester NA, Haley MV, Landis WG | 1994 | The Aquatic Toxicity of Isopropylamine: Comparison of Experimentally Derived Values with Structure Activity Predictions | Animal Test Alternatives, pp. 37-46 | Chemical Research Development and Engineering Center, MD/USA |  |  | T04894 |  |

Data access

data published

Materials and methods

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | other guideline: EPA 1986: Part 797, Environmental effects testing guidelines, subpart 3, aquatic guidelines, CFR 40, part 700 to end. U.S. Government Printing Office, Washington, D.C., 1986 | yes ( exposure time 96 h instead of 72 h according to OECD 201) |

Principles of method if other than guideline

- Endpoint: 50% inhibition of biomass growth after 96 h

GLP compliance

no data

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |
| EC number | 200-860-9 |
| EC name | isopropylamine |

Details on test material

isopropylamine, purity not specified

Analytical monitoring

no

Vehicle

no

Details on test solutions

- pH start: buffered (7.0), unbuffered (7.5-11)

Test organisms

Test organisms (species)

Scenedesmus capricornutum

Details on test organisms

TEST ORGANISM (new name: Pseudokirchnerella subcapitata) - Source (laboratory, culture collection): test laboratory - Method of cultivation: grown in a semiflow-through culture apparatus on T82MV medium

Study design

Test type

static

Water media type

freshwater

Limit test

no

Total exposure duration

96 h

Test conditions

Test temperature

not reported

pH

- pH start: buffered (7.0), unbuffered (>=7.5): - pH (first assay): 7.5- 11.0+ - pH (second assay): adjusted to 7.0 with 10% HCl, resulting in solution pH ranging from 7.1-7.4

Nominal and measured concentrations

Concentrations (extracted graphically): 63, 125, 250, 500, 1000 mg/L (nominal)

Details on test conditions

TEST METHODS: - Controls: yes, pH adjusted and unadjusted - Test vessels were inoculated with approximately 8.0 x10^4 cells/mL of the algal stock. - Two separate assays were again conducted to assess the effect of pH on toxicity (buffered test pH was 7.0; unadjusted solutions ranged from 7.5-11.0). EVALUATION CRITERIA: - The percentage growth inhibition was calculated from the area under the growth curve and was plotted against concentration (according to OECD 201). - The EC50 values were calculated using probit analysis and were verified graphically fromleast-square regression lines and equation of the dose-response curves.

Reference substance (positive control)

no

Results and discussions

Effect concentrations

| Duration | Endpoint | Effect conc. | Nominal/Measured | Conc. based on | Basis for effect | Remarks (e.g. 95% CL) |
| --- | --- | --- | --- | --- | --- | --- |
| 96 h | EC50 | < 62.5 mg/L | nominal | test mat. | biomass ( [Area under the growth curve (see OECD 201)] ) | unbuffered, pH not given |
| 96 h | EC90 | ca. 62.5 mg/L | nominal | test mat. | biomass ( [Area under the growth curve (see OECD 201)]) | unbuffered, pH not given |
| 96 h | EC50 | 118 mg/L | nominal | test mat. | biomass ( [Area under the growth curve (see OECD 201)]) | Buffered Test. Results determined by probit analysis, (95% CI: 73-166 mg/L) |
| 96 h | EC90 | > 500 mg/L | nominal | test mat. | biomass ( [Area under the growth curve (see OECD 201)]) | graphically derived from dose-response curve of buffered test solution (Fig. 2) |

Details on results

- No individual results were included. - The lowest isopropylamine concentration tested (62.5 mg/L, unbuffered) inhibited algal growth at > 92%, prohibiting EC50 calculation. - Graphic estimations: 120.3 mg/L (buffered) - The pH values of the test solutions and their influence on results are not reported.

Reported statistics and error estimates

EC50 values were derived graphically from a least-square regression line and equation as well as by probit analysis.

Applicant's summary and conclusion

Validity criteria fulfilled

no data

Endpoint study record: EPIWIN 2010,Toxicity to aquatic algae and cyanobacteria

| UUID |  | IUC5-e114b464-5ec0-43e6-af61-b97132f794fb |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | Collette / Epona Associates, LLC / Willington / United States |
| Date |  | 2012-07-02 15:34:11 EDT |
| Remarks |  |  |

Administrative Data

| Purpose flag | supporting study |  |  |
| --- | --- | --- | --- |
| Study result type | estimated by calculation |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Accepted calculation method |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| other: modelling |  | 2010 |  | U.S. Environmental Protection Agency (U.S. EPA) Estimation Programs Interface Suite,™. for Microsoft® Windows, v4.00. United States Environmental Protection Agency, Washington, DC, USA. |  |  |  |  |  |

Materials and methods

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test conditions

Any other information on materials and methods incl. tables

ECOSAR Program (v1.00): ============================== SMILES : C(C)(C)N CHEM : Isopropylamine MOL FOR: C3 H9 N1 MOL WT : 59.11 Log Kow: 0.27 (KowWin estimate) Melt Pt: -90.00 deg C Wat Sol: 4.218E+005 mg/L (measured)

Results and discussions

Effect concentrations

| Duration | Endpoint | Effect conc. | Nominal/Measured | Conc. based on | Basis for effect | Remarks (e.g. 95% CL) |
| --- | --- | --- | --- | --- | --- | --- |
| 96 h | EC50 | 1.97 mg/L |  |  |  |  |

Details on results

ECOSAR Program (v1.00) Results: ============================== ECOSAR v1.00 Class(es) Found ------------------------------ Aliphatic Amines Predicted ECOSAR Class Organism Duration End Pt mg/L (ppm) ============================================= Aliphatic Amines : Green Algae 96-hr EC50 1.968 ============================================= Neutral Organic SAR (Baseline Toxicity) : Green Algae 96-hr EC50 131.482 Aliphatic Amines: ---------------- For Green Algae Acute Toxicity Values: If the log Kow of the chemical is greater than 7, or if the compound is solid and the EC50 exceeds the water solubility by 10X, no effects at saturation are predicted for these endpoints. ECOSAR v1.00 SAR Limitations: ---------------------------- Maximum LogKow: 7.0 (Green Algae EC50) Maximum Mol Wt: 1000 Baseline Toxicity SAR Limitations: --------------------------------- Maximum LogKow: 6.4 (Green Algae EC50) Maximum Mol Wt: 1000

6.1.7 Toxicity to microorganisms

Endpoint summary: Toxicity to microorganisms

| UUID |  | IUC5-d6c9ecda-dffe-436e-b156-dfbab7fc24a2 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:58:08 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

EU: REACH

Short description of key information

In a respiration assay (OECD 209) using active sludge, an EC20 of about 600 mg isopropylamine/L has been identified. The NOEC/EC10 is estimated to be 300 mg/L. The EC50 was > 1000 mg/L.

Key value for chemical safety assessment

| EC50/LC50 | 1000 mg/L |
| --- | --- |
| EC10/LC10 or NOEC | 300 mg/L |

Discussion

Isopropylamine shows very low inhibitory activity towards an bacteria active sludge mixture.

Endpoint study record: BASFAG96/0626/08/1.Toxicity to microorganisms

| UUID |  | IUC5-e1bfd9b2-a2d2-4aeb-873a-32d5fcc3ee07 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-12-30 10:04:42 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Purpose flag | key study; robust study summary; used for MSDS |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result |  |  |
| Reliability | 1 (reliable without restriction) |  |  |
| Rationale for reliability incl. deficiencies | GLP-guideline study |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | BASF AG | 1996 |  |  | Department of Ecology | 96/0626/08/1 |  |  | 1996-11-26 |

Data access

data submitter has Letter of Access

Data protection claimed

yes

Materials and methods

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | OECD Guideline 209 (Activated Sludge, Respiration Inhibition Test) |  |

GLP compliance

yes

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |
| EC name | isopropylamine |

Details on test material

- Name of test material (as cited in study report): Isopropylamine - Analytical purity: no data (see 6.1.5) - Composition of test material, percentage of components: waterfree

Analytical monitoring

no

Vehicle

no

Test organisms

Test organisms (species)

activated sludge, domestic

Details on inoculum

Activated sludge from a laboratory wastewater plant treating municipal and artificial sewage. Concentration of dry substance 1 g/L .

Study design

Test type

static

Water media type

freshwater

Limit test

no

Total exposure duration

30 min

Test conditions

Test temperature

20 ± 2 °C

Nominal and measured concentrations

500 and 1000 mg/L (nominal)

Details on test conditions

Type: aquatic

Reference substance (positive control)

yes (3,5-dichlorophenol)

Results and discussions

Effect concentrations

| Duration | Endpoint | Effect conc. | Nominal/Measured | Conc. based on | Basis for effect | Remarks (e.g. 95% CL) |
| --- | --- | --- | --- | --- | --- | --- |
| 30 min | other: EC20 | ca. 600 mg/L | nominal | test mat. | respiration rate |  |
| 30 min | EC50 | > 1000 mg/L | nominal | test mat. | growth inhibition |  |
| 30 min | other: EC80 | > 1000 mg/L | nominal | test mat. | growth inhibition |  |

Results with reference substance (positive control)

- Results with reference substance valid? yes - Relevant effect levels: EC20 approx. 1.5 mg/L, EC50 approx. 10 mg/L, EC80 approx. 80 mg/L

Endpoint study record: Hoechst AG 1980_ETAD; RL3

| UUID |  | IUC5-b67c6667-911d-4871-8284-8834e73296b8 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-12-30 10:04:50 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Study result type | experimental result |  |  |
| --- | --- | --- | --- |
| Reliability | 3 (not reliable) |  |  |
| Rationale for reliability incl. deficiencies | Abstract / short communication/ documentation insufficient for assessment |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| other company data | Wellens, H. | 1980 | Abwasserbiologische Untersuchung von Isopropylamin | unpublished report | Abteilung Reinhaltung von Wasser und Luft |  | Hoechst AG | T02467 | 1980-04-16 |

Data access

data submitter is data owner

Data protection claimed

yes, but willing to share

Materials and methods

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | ETAD Fermentation Tube Method |  |

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |
| EC number | 200-860-9 |
| EC name | isopropylamine |

Details on test material

Isopropylamine [CAS No. 75-31-0], not further specified

Analytical monitoring

no

Test organisms

Test organisms (species)

anaerobic bacteria from a domestic water treatment plant

Study design

Test type

static

Water media type

no data

Test conditions

Details on test conditions

Type: aquatic

Results and discussions

Effect concentrations

| Duration | Endpoint | Effect conc. | Nominal/Measured | Conc. based on | Basis for effect | Remarks (e.g. 95% CL) |
| --- | --- | --- | --- | --- | --- | --- |
| 24 h | EC0 | > 2500 mg/L |  |  |  |  |

Any other information on results incl. tables

EC0 relates to toxic threshold.

6.2 Sediment toxicity

Endpoint study record: WAIVER_Sediment toxicity

| UUID |  | IUC5-94fb03b6-dfa2-4fec-91be-a422fe78d9dd |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:51:44 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| EU: REACH |  |
| --- | --- |
| Data waiving | other justification |
| Justification for data waiving | In accordance with column 2 of Annex X (required in Section 9.5.1) of REACH Regulation 1907/2006: no long-term study in sediment is needed for isopropylamine, since the chemical is readily biodegradable and has no potential to bioaccumulate or to adsorb to sediment. Furthermore, exposure of sediment is unlikely to occur. |

Materials and methods

Test materials

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |

6.3 Terrestrial toxicity

6.3.1 Toxicity to soil macroorganisms except arthropods

Endpoint study record: WAIVER_Toxicity to soil macroorganisms except arthropods

| UUID |  | IUC5-f31cf4c8-b7ca-4fd4-9bbc-a7bf38f18bba |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:51:44 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| EU: REACH |  |
| --- | --- |
| Data waiving | other justification |
| Justification for data waiving | In accordance with column 2 of Annex IX and X (required in Section 9.4) of REACH Regulation 1907/2006: no studies on terrestrial organisms are needed for isopropylamine since the chemical is readily biodegradable and has no potential to bioaccumulate or to adsorb to soil. Furthermore, exposure of sediment is unlikely to occur. |

Materials and methods

Test materials

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |

6.3.2 Toxicity to terrestrial arthropods

Endpoint study record: WAIVER_Toxicity to terrestrial arthropods

| UUID |  | IUC5-70182e6a-b1c8-4738-9941-5c7b29acf87c |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:51:45 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| EU: REACH |  |
| --- | --- |
| Data waiving | other justification |
| Justification for data waiving | In accordance with column 2 of Annex IX and X (required in Section 9.4) of REACH Regulation 1907/2006: no studies on terrestrial organisms are needed for isopropylamine, since the chemical is readily biodegradable and has no potential to bioaccumulate or to adsorb to soil. Furthermore, exposure of sediment is unlikely to occur. |

Materials and methods

Test materials

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |

6.3.3 Toxicity to terrestrial plants

Endpoint study record: WAIVER_Toxicity to terrestrial plants

| UUID |  | IUC5-8df06889-3a5f-442f-ba25-e86f88008ab9 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:51:46 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| EU: REACH |  |
| --- | --- |
| Data waiving | other justification |
| Justification for data waiving | In accordance with column 2 of Annex X (required in Section 9.4) of REACH Regulation 1907/2006: no studies on terrestrial plants are needed for isopropylamine, since the chemical is readily biodegradable and has no potential to bioaccumulate or to adsorb to soil. Furthermore, exposure of soil is unlikely to occur. |

Materials and methods

Test materials

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |

6.3.4 Toxicity to soil microorganisms

Endpoint study record: WAIVER_Toxicity to soil microorganisms

| UUID |  | IUC5-8389a51a-51ab-46d6-8637-50300b284296 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:51:46 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| EU: REACH |  |
| --- | --- |
| Data waiving | other justification |
| Justification for data waiving | In accordance with column 2 of Annex IX and X (required in Section 9.4) of REACH Regulation 1907/2006: no studies on terrestrial microorganisms are needed for isopropylamine, since the chemical is readily biodegradable and has no potential to bioaccumulate or to adsorb to soil. Furthermore, exposure of soil is unlikely to occur. |

Materials and methods

Test materials

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |

6.3.5 Toxicity to birds

Endpoint study record: WAIVER_Toxicity to birds

| UUID |  | IUC5-e364a361-3328-48ad-88fb-97463dcb5995 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:51:47 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| EU: REACH |  |
| --- | --- |
| Data waiving | other justification |
| Justification for data waiving | In accordance with column 2 of Annex X (required in Section 9.6.1) of REACH Regulation 1907/2006: no long-term or reproductive studies on birds are needed for isopropylamine, since the chemical is readily biodegradable and has no potential to bioaccumulate or to adsorb to soil. Furthermore, exposure of soil is unlikely to occur, and a mammalian dataset is available. |

Materials and methods

Test materials

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |

7 Toxicological information

Endpoint summary: Toxicological information

| UUID |  | IUC5-5e20cecf-9c38-4667-94ba-7c7d1ff8d023 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:58:09 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

EU: REACH

Workers

Acute / short-term exposure - systemic effects

Dermal DN(M)EL

| No-threshold effect and/or no dose-response information available |  |  |  |
| --- | --- | --- | --- |
| Most sensitive endpoint | irritation / corrosion (eye and skin) |  |  |

Justification for (no) DN(M)EL derivation / applied assessment factors

An acute dermal toxicity study indicates an LD0 of about 400 mg/kg in rats, but the data are insufficient to establish a NOAEL for the purpose of a DNEL derivation.

Inhalation DN(M)EL

| No-threshold effect and/or no dose-response information available |  |  |  |
| --- | --- | --- | --- |
| Most sensitive endpoint | irritation (respiratory tract) |  |  |

Justification for (no) DN(M)EL derivation / applied assessment factors

An acute inhalation toxicity study indicates an 4-h LC50 of 8.7 mg/l in rats, but the data is insufficient to establish a NOAEC for the purpose of a DNEL derivation.

Acute / short-term exposure - local effects

Dermal DN(M)EL

| No-threshold effect and/or no dose-response information available |  |  |  |
| --- | --- | --- | --- |
| Most sensitive endpoint | irritation / corrosion (eye and skin) |  |  |

Justification for (no) DN(M)EL derivation / applied assessment factors

Isopropylamine is severely irritating: therefore, no DNEL for local effects derivable.

Inhalation DN(M)EL

| DNEL (Derived No Effect Level) |  | 25 mg/m³ |
| --- | --- | --- |
| Most sensitive endpoint | irritation (respiratory tract) |  |

Justification for (no) DN(M)EL derivation / applied assessment factors

Considered as nasal irritation threshold in humans (10 mL/m3 = 25 mg/m3), cited in literature (see below) = 15-min short-term MAK value (Germany).

Long-term exposure - systemic effects

Dermal DN(M)EL

| DNEL (Derived No Effect Level) |  | 4.8 mg/kg bw/day |
| --- | --- | --- |
| Most sensitive endpoint | repeated dose toxicity |  |

Justification for (no) DN(M)EL derivation / applied assessment factors

Starting point: NOAEC (90d inh., rat) = 500 mg/m3 for systemic effects: corresponding to a dose in rat of NOAEC(inhal.) x0.38 m3/(kg*d) = 190 mg/(kg*d); adjustment to long-term exposure in animals (*1/2) and worker´s conditions: 190 mg/(kg*d) * 1/(4 x 5 x 2) = 4.8 mg/(kg*d) ((including the allometric scaling factor of 1/4 and intraspecies factor for workers of 1/5). It is assumed that absorption via skin is identical to absorption via inhalation.

Inhalation DN(M)EL

| DNEL (Derived No Effect Level) |  | 25 mg/m³ |  |
| --- | --- | --- | --- |
| Dose descriptor starting point | NOAEC | Most sensitive endpoint | repeated dose toxicity |

Justification for (no) DN(M)EL derivation / applied assessment factors

Starting point: NOAEC (90d inh., rat) = 500 mg/m3 for systemic effects (highest tested concentration), factors according to guidance R.8 for adjustment to chronic exposure (*1/2) and workers´conditions: 500 mg/m3 *6h/8h *6.7m3/10m3 * 1/(5 x 2) = 25 mg/m3.

Long-term exposure - local effects

Dermal DN(M)EL

| No-threshold effect and/or no dose-response information available |  |  |  |
| --- | --- | --- | --- |
| Most sensitive endpoint | irritation / corrosion (eye and skin) |  |  |

Justification for (no) DN(M)EL derivation / applied assessment factors

Isopropylamine is severely irritating: therefore, no DNEL for local effects derived.

Inhalation DN(M)EL

| DNEL (Derived No Effect Level) |  | 12 mg/m³ |
| --- | --- | --- |
| Most sensitive endpoint | irritation (respiratory tract) |  |

Justification for (no) DN(M)EL derivation / applied assessment factors

This value corresponds to the German MAK.

Discussion

Isopropylamineis assumed to be readily bioavailable by all potential routes of exposure. Exposure may mainly occur by inhalation and by skin contact. Based on general experience and knowledge with this class of substances, isopropylamine is readily eliminated from the organism either metabolised by oxidative desamination, thus resulting in acetone, or excreted from the body in unchanged form. Overall, isopropylamine will not accumulate and is not expected to form harmful intermediates (Beard and Noe 1981; Cavender et al. 2000). The primary and most relevant effect is its irritating/corrosive effect on skin and mucous membranes. This severe local insult limits exposure to doses that potentially may induce systemic lesions; on the other hand, no significant systemic and reproduction toxicity was observed after inhalation of concentrations as high as 500 mg/m3 and 1000 mg/m3, respectively, in rats. It is reasonable to establish a DNEL for inhalation solely, related to the local effects observed in the respiratory tract. The data compiled below indicate that the DNEL has to be established below 10 ppm (25 mg/m3), as 10 ppm is reported to be an irritating exposure level for affected workers.Furthermore, this value is in a range where no sensory irritation was seen in mice (see Table below).Corresponding DNELs for potential systemic intoxication would be greater than those for local effects. In synopsis of all available data, it is proposed that in principle the latest MAK value of 5 mL/m3 (12 mg/m3) should be adopted as a DNEL for repeated and prolonged inhalation, while 10 mL/m3 (25 mg/m3) correspond to the MAK short-term exposure limit. Overview of effects data following exposure to isopropylamine Olfactory / nasal effects markers; Exposure concentrations; # Reference; Section IUCLID5. mL/m3; mg/m3. Odor threshold (human); 1.2; ~3; Amoore and Hautala 1983; 7.10.3. Nasal irritation threshold (human); 10; 25; Beard and Noe 1981; 7.10.3. RD50 (mouse, 15 min) *); 157; 390; Gagnaire et al. 1989, 1993; 7.2.2. Estimate from RD50: RD0 (mouse, 15 min) *); ~15; ~37. NOAEC (systemic effects, rat, 90 d); 215; 500; Monsanto 1988; 7.5.3. NOAEC (histopathological changes, rat, 28d); 43; 100; Dudek et al. 1991; 7.5.3.

General population

Long-term exposure - local effects

Discussion

No data derived, considered to be not relevant, as the general public is no target population.

7.1 Toxicokinetics, metabolism and distribution

7.1.1 Basic toxicokinetics

Endpoint summary: Basic toxicokinetics

| UUID |  | IUC5-884e411d-7f98-402f-80e0-2f531fcde25f |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:58:10 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

EU: REACH

Short description of key information

No particular data available. Isopropylamine is expected to be readily eliminated from the organism either metabolised by oxidative desamination, thus resulting in acetone, or excreted from the body in unchanged form.

Key value for chemical safety assessment

Bioaccumulation potential

no bioaccumulation potential

Discussion

Isopropylamineis assumed to be readily bioavailable by all potential routes of exposure. Exposure may mainly occur by inhalation and by skin contact. It can be concluded that ingested isopropylamine is no proper substrate of MAO, but partly may undergo methyl- or N-oxidation. But a substantial part is expected to be excreted unchanged into the urine. If oxidative deamination occurs, acetone will be formed. This is usually exhaled as well as excreted into the urine. Small amounts of acetone are normally formed in the intermediary metabolism through spontaneous or enzymatic decarboxylation of acetoacetate. ----------- Overall, isopropylamine will not accumulate and is not expected to form harmful intermediates (Cavender et al. 2000).

Endpoint study record: Cavender et al. (+2000, Review, RL4)

| UUID |  | IUC5-6e1b7a6f-9b72-4f61-aa8b-a4928ca16b3d |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-12-30 10:05:15 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Purpose flag | weight of evidence |  |  |
| --- | --- | --- | --- |
| Study result type | other: review, handbook |  |  |
| Reliability | 4 (not assignable) |  |  |
| Rationale for reliability incl. deficiencies | Secondary literature |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| review article or handbook | Cavender FL, Bingham E, Cohrssen B, Powell CH (editors) | 2000 | Aliphatic and Alicyclic Amines, in: | Patty´s Industrial Hygiene and Toxicology, J. Wiley & Sons 2000 |  |  |  | T02713 | 1999-12-31 |

Data access

data published

Materials and methods

Type of method

in vivo

Objective of study

other: metabolism, excretion

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |
| EC number | 200-860-9 |
| EC name | isopropylamine |

Test animals

Species

other: mammalian species

Results and discussions

Metabolite characterisation studies

Metabolites identified

no

Details on metabolites

assumed to be excreted unchanged in the urine as well as partly as acetone after oxidative desamination and oxidation.

Overall remarks, attachments

Overall remarks

A. Excerpt from Cavender et al. 2000: "In animals, the lower aliphatic amines are mainly metabolized to the corresponding carboxylic acid and urea; the intermediate compounds have been shown by in vitro experiments to be the corresponding aldehyde and ammonia. MAOs oxidatively deaminate primary, secondary, and tertiary amines. However, compounds that have a substituted methyl group on the alpha carbon are not metabolized by MAOs (14). To serve as a substrate, the amine group must be attached to an unsubstituted methylene group (alpha-carbon hydrogen prerequisite). For example, MAO cannot catalyze the oxidation of aniline, amphetamine, or methylamine. Ethylamine and propylamine have also been demonstrated to be poor substrates for MAO. The rate of enzyme oxidation activity increases with an increase in aliphatic chain length to an optimum of C5 or C6, followed by a decline in activity to a maximum at 13 methylene groups (14a). For example, butyl-, amyl-, isoamyl-, and heptylamines are readily metabolized to aldehyde and ammonia (15, 16), but compounds of longer chain lengths, for example, octadecylamine, may inhibit the enzyme. Deamination rate also diminishes with secondary and tertiary amines (51a)." ------------------ B. Excerpt from Cavender et al. 2000: "MAO is involved only to a limited extent in the metabolism of aliphatic amines (16), whereas microsomal enzymes can be responsible for multiple amine biotransformations. Microsomal versus mitochrondrial substrate affinity depends not only upon structural characteristics, but also upon the degree of lipophilicity. Microsomal enzyme-catalyzed aliphatic and alicyclic amine biotransformation reactions can include deamination, methylation, N-dealkylation, N-oxidation, N-acetylation, cyclization, N-hydroxylation, and nitrosation (19 - 21)." C. Excerpt from Cavender et al. 2000: "In summary, the principal routes of biotransformation of aliphatic and alicyclic amines are N-dealkylation or oxidative deamination and N-oxidation (30a-30b). Primary aliphatic amines are susceptible to N-oxidation only if the alpha-carbon is subtituted. This prevents the formation of a carbinolamine intermediate, and N-oxidation occurs, resulting in formation of primary hydroxylamines, which can then undergo further oxidation to nitroso and oxime metabolites."

Applicant's summary and conclusion

Interpretation of results

no bioaccumulation potential based on study results

Conclusions

It can be concluded that ingested isopropylamine is no proper substrate of MAO, but partly may undergo methyl or N-oxidation. But a substantial part is expected to be excreted unchanged in the urine. If oxidative deamination occurs, acetone will be formed. This is usually exhaled as well as excreted into the urine. Small amounts of acetone are normally formed in the intermediary metabolism through spontaneous or enzymatic decarboxylation of acetoacetate. -----------

7.2 Acute Toxicity

Endpoint summary: Acute Toxicity

| UUID |  | IUC5-9594931a-8320-4c2c-8683-33f3fc4bf93f |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:58:10 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

EU: REACH

Short description of key information

LD50(oral, rat) <173 mg/kg bw (undiluted) / mean LD50(oral, rat) = 550 - 820 mg/kg bw (diluted in water or olive oil); mean LC50(inh., rat) = 8.7 mg/L; LD50(dermal, rat) > 400 mg/kg bw (diluted, 10 % in water)

Key value for chemical safety assessment

Acute toxicity: oral

| Effect level | LD50 | in mg/kg bw | 170 |
| --- | --- | --- | --- |

Acute toxicity: inhalation

| Effect level | LC50 | in mg/m³ air | 8700 |
| --- | --- | --- | --- |

Discussion

As a primary short-chain amine, isopropylamine reacts strongly alkaline and is corrosive. Therefore, no dermal animal test was indicated.

7.2.1 Acute toxicity: oral

Endpoint study record: Monsanto.Acute toxicity: oral.1985

| UUID |  | IUC5-b77d1ae3-6afd-4181-b101-aca1fbd08210 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | Collette / Epona Associates, LLC / Willington / United States |
| Date |  | 2012-06-14 11:48:43 EDT |
| Remarks |  |  |

Administrative Data

| Purpose flag | key study |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result |  |  |
| Reliability | 1 (reliable without restriction) |  |  |
| Rationale for reliability incl. deficiencies | GLP, similar to guideline study |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | Walker, B, | 1985 | Acute Oral Toxicity (LD50) Study in Rats |  | International Research and Development Corporation Mattawan, Michigan |  | Monsanto |  | 1985-07-17 |

Materials and methods

Test type

standard acute method

Limit test

no

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | other guideline: not specified |  |

GLP compliance

yes

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| Common name | Isopropylamine |

Details on test material

Isopropylamine lot 8494

Test animals

Species

rat

Strain

Sprague-Dawley

Sex

male/female

Details on test animals and environmental conditions

197-265 grams at study initiation; Maintained in accordance with the recommendations contained in the D .H .E .W . Publication No . 78-23 (N .I .H .) entitled "Guide for the Care and Use of Laboratory Animals" and conditioned for a period of at least 10 days prior to study initiation . Anima room with controlled temperature, humidity and light (12 hours light and 12 hours dark) . Diet and water freely available ; diet withheld approximately 8 hours prior to dosing and approximately 3-4 hours after dosing .

Administration / exposure

Route of administration

oral: gavage

Vehicle

unchanged (no vehicle)

Details on oral exposure

The test article was administered by oral gavage using a 100 mcl, clean, glass syringe fitted with a 16 gauge snub-tipped dosing needle .

Doses

70, 118, 200 and 338 mg/kg bw

No. of animals per sex per dose

5/sex

Control animals

no

Details on study design

Mortality: Observed 1, 2 1/2 and 4 hours after administration on the first day; twice daily thereafter for 13 days . Pharmacotoxic Signs : Observed 1, 2 1/2 and 4 hours after administration an the first day ; once daily thereafter for 13 days . Body Weights : immediately prior to test article administration, on Day 8 and at study termination (Day 15) . Necropsy: Performed on all animals that died on study and those sacrificed at termination by qualified Acute Department personnel according to IRDC Standard Operating Procedure Vo . 44-1-3 . No tissues were saved .

Results and discussions

Effect levels

| Sex | Endpoint | Effect level | Based on | 95% CL | Remarks |
| --- | --- | --- | --- | --- | --- |
| male | LD50 | 111 mg/kg bw |  |  |  |
| female | LD50 | 126 mg/kg bw |  |  |  |
| male/female | LD50 | 122 mg/kg bw |  |  |  |

Mortality

Dose Male Female (mg/kg bw) 70 1 0 118 5 3 200 1 4 338 5 5

Clinical signs

Decreased activity and staining around the nose and mouth were observed in most rats at all dosage levels . Rats in the 118 and 338 mg/kg groups also exhibited high incidence of labored breathing . These signs were noted in rats that died on study and those surviving to termination .

Body weight

No effect

Gross pathology

Various degrees of congestion, ranging from mild to severe, were observed in the stomach non-glandular and/or glandular mucosa and lungs of most rats that died on study in the 118, 200 and 338 mg/kg groups . No other prevalent findings were observed . In general, rats sacrificed at termination exhibited no visible abnormalities .

Applicant's summary and conclusion

Conclusions

The LD50 value (95% confidence limits) For combined male and female rats was 122 (105-142) mg/kg bw.

Endpoint study record: BASF AG VI/391.Acute toxicity: oral.rat,SS

| UUID |  | IUC5-802c4664-2cc8-41b0-905b-5530e6812efd |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-12-30 10:05:20 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Purpose flag | supporting study; robust study summary |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Acceptable, well documented report which meets basic scientific principles. |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | BASF AG | 1960 | Industrial Hygiene Orientating Investigation | unpublished data | BASF AG, Department of Toxicology | VI/391 | BASF SE |  | 1960-02-04 |

Data access

data submitter has Letter of Access

Data protection claimed

yes

Materials and methods

Principles of method if other than guideline

BASF-Test, see details in remarks on material and methods.

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| Common name | isopropylamine |

Details on test material

- Name of test material (as cited in study report): Monoisopropylamin - Physical state: liquid

Test animals

Species

rat

Strain

no data

Sex

male/female

Details on test animals and environmental conditions

TEST ANIMALS Weight at study initiation: 136 - 190 g

Administration / exposure

Route of administration

oral: gavage

Vehicle

other: olive oil

Details on oral exposure

VEHICLE - Concentration in vehicle: 1, 10 and 20 %

Doses

20, 79, 316, 501, 794, 1260, 5010 cm³/kg bw ( 14, 55, 218, 346, 548, 869, 3457 mg/kg bw - conversion in mg/kg is based on the density: 0.69 g/cm³)

No. of animals per sex per dose

14 and 55 mg/kg bw: 1 (no gender stated) 218 mg/kg bw: 5 (no gender stated) 346 and 548 mg/kg bw: 10 (no gender stated) 869 mg/kg bw: 5 (no gender stated) 3457 mg/kg bw: 2 (no gender stated)

Control animals

no data

Details on study design

- Duration of observation period following administration: 7 days - Frequency of observations: several times on the day of application and daily thereafter - Necropsy of survivors performed: yes - Other examinations performed: clinical signs

Any other information on materials and methods incl. tables

BASF-Test: Animals were treated by single gavage application with an aqueous solution of the test substance. The animals were observed for mortality and for clinical symptoms of toxicity. At the end of the observation period of 7 days, the surviving animals were sacrificed for the purpose of necropsy; animals that died during the observations period also were subjected to necropsy.

Results and discussions

Effect levels

| Sex | Endpoint | Effect level | Based on | 95% CL | Remarks |
| --- | --- | --- | --- | --- | --- |
| male/female | LD50 | 552 mg/kg bw |  |  | Conversion in mg/kg bw is based on the density d: 0.69 g/cm3 |

Mortality

see details in remarks on results

Clinical signs

3457 mg/kg bw: 1 animal showed 2 min post application dyspnoea, convulsions, gasping. The second animal showed intermittend respiration and slight abdominal position. 869 mg/kg bw: staggering, slightly accelerated respiration, slight abdominal position; On the next morning lacrimation, ruffled fur, blood crusted eyes. 548 mg/kg bw: 24 h post application ruffled fur. 346 mg/kg bw: intermittent respiration, blood crusted noses, ruffled fur.

Body weight

no data

Gross pathology

Animals that died: several animals showed autolysis in the abdominal cavity. Sacrificed animals: 5 animals of the 548 mg/kg bw group showed adhesion of the stomach with the peritoneum, fluffy-yellowish stomach content (residual test substance )

Any other information on results incl. tables

Mortality: Dose (mg/kg bw); No. animals; Conc. (%); 1 h; 24 h; 48 h; 7 days. 3457; 2; 20; 1/2; 2/2; 2/2; 2/2. 869; 5; 10; 0/5; 4/5; 5/5; 5/5. 548; 10; 10; 0/10; 0/10; 1/10; 3/10. 346; 10; 10; 0/10; 3/10; 3/10; 3/10. 218; 5; 10; 0/5; 0/5; 0/5; 0/5. 55; 1; 1; 0/1; 0/1; 0/1; 0/1. 14; 1; 1; 0/1; 0/1; 0/1; 0/1. The test substance caused systemic toxicity (including mortality) in a dose dependent manner.

Endpoint study record: Myers, Ballentyne (+1997, rat, key, RL2)

| UUID |  | IUC5-3dcdcbd6-b934-4f14-883e-89ad769d6dca |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-12-30 10:05:28 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Purpose flag | key study; robust study summary; used for classification; used for MSDS |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Basic data given, comparable to guideline studies |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Myers, C.; Ballantyne, B. | 1997 | Comparative Acute Toxicity and Primary Irritation of Various Classes of Amines | Toxic Substances Mechanisms., 16, 151-193 |  |  |  | T02627 |  |

Data access

data published

Materials and methods

Test type

up-and-down procedure

Limit test

no

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| equivalent or similar to | OECD Guideline 425 (Acute Oral Toxicity: Up-and-Down Procedure) | yes (only males tested; screening) |

GLP compliance

no data

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |
| EC number | 200-860-9 |
| EC name | isopropylamine |

Details on test material

Isopropylamine [CAS No. 75-31-0], production quality samples from Union Carbide Corp. (not further specified)

Test animals

Species

rat

Strain

Wistar

Sex

male

Details on test animals and environmental conditions

TEST ANIMALS - Source: no data - Age at study initiation: 3 - 4 wk - Weight at study initiation: 90 -120 g - Fasting period before study: unfasted - Housing: no data - Acclimation period: no data ENVIRONMENTAL CONDITIONS: no data

Administration / exposure

Route of administration

oral: gavage

Vehicle

water

Details on oral exposure

Application by gavage through a commercial ball-end needle attached to a calibrated syringe (page 153)

Doses

no details, dose range with spacing factor 2

No. of animals per sex per dose

5 males

Control animals

no data

Statistics

Moving average method according to Thompson (1947) und Weil (1983)

Results and discussions

Effect levels

| Sex | Endpoint | Effect level | Based on | 95% CL | Remarks |
| --- | --- | --- | --- | --- | --- |
| male | LD50 | < 0.25 mL/kg bw |  |  | undiluted |
| male | LD50 | < 173 mg/kg bw |  |  | undiluted; transformation from volume (ml/kg bw) to mass (mg/kg bw) |
| male | LD50 | 1.07 mL/kg bw |  | >= 0.72 — <= 1.59 | 10% solution |
| male | LD50 | ca. 730 mg/kg bw |  | >= 491 — <= 1084 | 10% solution; transformation from volume (ml/kg bw) to mass (mg/kg bw) |

Mortality

Time to death: 7 min. - 4d (undiluted), 4h - 14d (10% solution)

Clinical signs

lethargy, labored breathing, piloerection, prostration

Gross pathology

lung hemorrhage; kidney and adrenal congestion; gastric and intestinal hemorrhage, ulceration Most surviving animals without significant gross pathology (page 163)

Applicant's summary and conclusion

Interpretation of results

toxic

Criteria used for interpretation of results

EU

Endpoint study record: Smyth (+1951, rat, RL4)

| UUID |  | IUC5-411365ae-7905-4ef5-9aa8-f4a7922fc745 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-12-30 10:05:32 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Study result type | experimental result |  |  |
| --- | --- | --- | --- |
| Reliability | 4 (not assignable) |  |  |
| Rationale for reliability incl. deficiencies | Only Abstract / short communication; documentation insufficient for assessment |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Smyth HF, Carpenter CP, Weil CS | 1951 | Range Finding Toxicity Data: List IV. Arch. Ind. Hyg. Occup. Med., 4, 119-122 | Arch. Ind. Hyg. Occup. Med., 4, 119-122 | Mellon Institute of Industrial Research, University of Pittsburgh |  |  | T01614 |  |
| publication | Smyth HF, Carpenter CP, Weil CS | 1949 | Range-finding toxicity data: list III | J. Ind. Hyg. Toxicol. 31, 60-62 | Mellon Institute of Industrial Research, University of Pittsburgh |  |  | T02249 |  |
| publication | Smyth HF, Carpenter CP | 1944 | The Place of the Range Finding Test in the Industrial Toxicology Laboratory | J. Ind. Hyg. Toxicol., 26, 269-273 | Mellon Institute of Industrial Research, University of Pittsburgh |  |  | T01688 |  |

Data access

data published

Materials and methods

Test type

standard acute method

Limit test

no

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| no guideline followed |  |  |
| equivalent or similar to | OECD Guideline 401 (Acute Oral Toxicity) | yes (only male animal tested, screening) |

Principles of method if other than guideline

Smyth et al. (1944) [T01688], Screening, 14 d observation, with modification according to Smyth et al. (1949) [T02249]

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |
| EC number | 200-860-9 |
| EC name | isopropylamine |

Details on test material

Isopropylamine [CAS No. 75-31-0], not further specified

Test animals

Species

rat

Strain

Wistar

Sex

male

Details on test animals and environmental conditions

TEST ANIMALS - Source: no data - Age at study initiation: 4 - 5 wk - Weight at study initiation: 90 -120 g - Fasting period before study: unfasted ENVIRONMENTAL CONDITIONS: no data

Administration / exposure

Route of administration

oral: gavage

Vehicle

water

Details on oral exposure

VEHICLE - Concentration in vehicle: no data - Amount of vehicle (if gavage): no data

Doses

4 doses in a geometrical series (factor 2)

No. of animals per sex per dose

5 males

Control animals

no data

Statistics

Moving average method according to Thompson (1947)

Results and discussions

Effect levels

| Sex | Endpoint | Effect level | Based on | 95% CL | Remarks |
| --- | --- | --- | --- | --- | --- |
| male | LD50 | 820 mg/kg bw |  | 670 — 1070 | diluted |

Applicant's summary and conclusion

Cross-reference to other study

Myers , Ballentyne 1997; key, RL2

7.2.2 Acute toxicity: inhalation

Endpoint study record: Hoffman_Celanese (+1990, rat, key, RL1)

| UUID |  | IUC5-f024bd0a-3ea6-4cda-9c7c-1453bbaad57e |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-12-30 10:05:44 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Purpose flag | key study; robust study summary; used for classification; used for MSDS |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result |  |  |
| Reliability | 1 (reliable without restriction) |  |  |
| Rationale for reliability incl. deficiencies | GLP guideline study |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| other company data | Hoffman, G.M., Newton, P.E. | 1990 | An Acute Inhalation Toxicity Study of C-899 in the Rat | Hoechst Celanese Corp., unpublished study | Bio/dynamics Inc., Department of Toxicology | 89-8213 | OXEA | T02439 | 1990-12-03 |

Data access

data submitter is data owner

Data protection claimed

yes, but willing to share

Materials and methods

Test type

standard acute method

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| equivalent or similar to | OECD Guideline 403 (Acute Inhalation Toxicity) |  |
| according to | EPA OTS 798.1150 (Acute inhalation toxicity) |  |

GLP compliance

yes

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |
| EC number | 200-860-9 |
| EC name | isopropylamine |

Details on test material

Isopropylamine [CAS No. 75-31-0]; supplied by Hoechst Celanese Corp. (C-899; not further specified); 100% active ingredient

Test animals

Species

rat

Strain

Sprague-Dawley

Sex

male/female

Details on test animals and environmental conditions

TEST ANIMALS - Source: Charles River Breeding Laboratories, Inc., Raleigh, North Carolina, USA - Age at study initiation: males 8 - 9 wk, females 10 - 11 wk - Weight at study initiation: males: 242 g - 335g, females: 208g - 236g - Housing: doubly housed during acclimation period, individually housed in Plexiglass chamber (100 L) during exposure - Diet , water: ad libitum - Acclimation period: 1 to 3 wk ENVIRONMENTAL CONDITIONS during acclimation period - Temperature (°C): 19 - 24°C - Humidity (%): 40 - 70% - Air changes (per hr): no data - Photoperiod: 12hrs dark / 12hrs light) ENVIRONMENTAL CONDITIONS during exposure - Temperature (°C): 21 - 26°C - Humidity (%): 46 - 65% - Air flow rate: 20 L/min (= complete air change every 5 min)

Administration / exposure

Route of administration

inhalation: vapour

Type of inhalation exposure

whole body

Vehicle

other: unchanged (no vehicle)

Details on inhalation exposure

GENERATION OF TEST ATMOSPHERE / CHAMBER DESCRIPTION - Exposure apparatus: Plexiglass exposure chamber with glass front - Exposure chamber volume: 100 L - Method of holding animals in test chamber: individually - Source and rate of air: 20 L/min, the air exchange was about once per every 5 min. - Method of conditioning air: by passing air through a midget impinger containing the TS - System of generating particulates/aerosols: - Temperature: actual 21-26°C - Relative humidity: 64-65% - pressure: no information given TEST ATMOSPHERE - Brief description of analytical method used: MIRAN Ambient Air Analyser; test atmosphere was drawn through Balston DFU filter via Teflon line into the Analyser - Samples taken from breathing zone: yes (hourly sampling) - Particle size measurements were performed at hourly intervals

Analytical verification of test atmosphere concentrations

yes

Duration of exposure

4 h

Concentrations

5.3, 2.8, 5.1, and 6.5 mg/L (analytical) [7.1, 3.4, 5.9, and 7.2 mg/L (nominal)], no aerosol The nominal concentration was calculated from the consumption of TS from the reservoir divided by the total amount of air.

No. of animals per sex per dose

5

Control animals

no

Details on study design

- Duration of observation period following administration: 14 days; detailed observations once daily for survivers; Assessment of viability twice daily - Necropsy of survivors performed: yes (at post-exposure day 15) - Examinations of dead and surviving animals of nasal passages, trachea, orifices, cranial cavity, brain and spinal cord, viscera, cervical tissues and organs

Statistics

Calculation of median lethal concentration, 95% CL (method according to Litchfield and Wilcoxon)

Results and discussions

Effect levels

| Sex | Endpoint | Effect level | Based on | 95% CL | Exp. duration | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| male/female | LC50 | ca. 8.7 mg/L air (analytical) |  | >= 6.5 — <= 12 | 4 h |  |
| male/female | LC50 | ca. 3540 ppm |  |  | 4 h | calculated from formula 1 ppm = 2.46mg/m^3 |
| female | LC50 | 7.7 mg/L air (analytical) |  |  | 4 h |  |
| male | LC50 | ca. 9.4 mg/L air (analytical) |  |  | 4 h |  |

Mortality

see table below

Clinical signs

Labored breathing, lacrimation, nasal discharge, reduced activity, closed eyes. Respiratory distress and nasal discharge persisted the first week post-exposure, but moist and dry rales were observed in most of the survivors throughout 14 d.

Body weight

Transient losses were prominent.

Gross pathology

Nasal tubinates and lungs were reddened and swollen with lung emphysema present in a few animals.

Any other information on results incl. tables

Group Mean ExposureNominal [mg/L] Mortality tration Concentration No.dead/No.exposed ---------------------------------------------------------------- I* 5.3 7.1 5/5 2/5 7/10 II 2.8 3.4 0/5 0/5 0/10 III 5.1 5.9 2/5 0/5 2/10 IV 6.5 7.2 1/5 2/5 3/10 ================================================================ * Group I was excluded from evaluation due to technical problems during exposure The LD50 of 8.7 mg/L (95% C.I. = 6.5 - 12 mg/L) is the combined extrapolated value. -----------------

Endpoint study record: Smyth (+1951, rat, RL4)

| UUID |  | IUC5-a9b66c1e-35b6-4629-9a9a-36d15566660f |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-12-30 10:05:53 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Study result type | experimental result |  |  |
| --- | --- | --- | --- |
| Reliability | 4 (not assignable) |  |  |
| Rationale for reliability incl. deficiencies | Only Abstract / short communication; documentation insufficient for assessment |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Smyth HF, Carpenter CP, Weil CS | 1951 | Range Finding Toxicity Data: List IV. Arch. Ind. Hyg. Occup. Med., 4, 119-122 | Arch. Ind. Hyg. Occup. Med., 4, 119-122 | Mellon Institute of Industrial Research, University of Pittsburgh |  |  | T01614 |  |
| review article or handbook | Cavender FL, Bingham E, Cohrssen B, Powell CH (editors) | 2000 | Aliphatic and Alicyclic Amines, in: | Patty´s Industrial Hygiene and Toxicology, J. Wiley & Sons 2000 |  |  |  | T02713 | 1999-12-31 |

Data access

data published

Materials and methods

Test type

other: Screening at two dose levels

Limit test

no

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| no guideline followed |  |  |

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |
| EC number | 200-860-9 |
| EC name | isopropylamine |

Details on test material

Isopropylamine [CAS No. 75-31-0], not further specified

Test animals

Species

rat

Strain

Wistar

Sex

male

Administration / exposure

Route of administration

inhalation: vapour

Type of inhalation exposure

whole body

Vehicle

other: air

Analytical verification of test atmosphere concentrations

no

Duration of exposure

4 h

Concentrations

4000 and 8000 ppm (nominal)

No. of animals per sex per dose

6

Control animals

no

Any other information on materials and methods incl. tables

An air stream was passed through a fritted glass disc immersed at a depth of at least one inch in approx. 50 ml of the test chemical at ambient temperature. The air concentrations are reported to be 4000 and 8000 ppm. -----------

Results and discussions

Effect levels

| Sex | Endpoint | Effect level | Based on | 95% CL | Exp. duration | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| male | other: Acute inhalation hazard test | > 9.8 — < 19.6 mg/L air |  |  | 4 h |  |
| male | LC0 | ca. 9.8 mg/L air (nominal) |  |  | 4 h |  |
| male | LC100 | ca. 19.6 mg/L air (nominal) |  |  | 4 h |  |

Mortality

at 4000 ppm = 9.8 mg/l, 4 h: Mortality 0/6; at 8000 ppm = 19.6 mg/l, 4 h: Mortality 6/6

Clinical signs

no data

Body weight

no data

Gross pathology

no data

Applicant's summary and conclusion

Cross-reference to other study

Hoffman_Celanese (+1990, rat, key, RL1)

Endpoint study record: Gagnaire (+1989 / 1993, sensory irritation, inhal, mouse, RL2)

| UUID |  | IUC5-1428d786-7de4-468f-aaa3-078da76a3cfd |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-12-30 10:05:58 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Purpose flag | key study; robust study summary |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Comparable to guideline study with acceptable restrictions (limited documentation) Key study for sensory nasal irritation |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Gagnaire F, Azim S, Simon P, Cossec B, Bonnet P and de Ceaurriz J | 1993 | Sensory and pulmonary irritation of aiphatic amines in mice: a structure-activity relationship study. | J. Appl. Toxicol., 13, 129-135 (1993) | Institut National de Recherche et de Sécurité |  |  | T01726 |  |
| publication | Gagnaire R, Azim S, Simon P, Bonnet P, de Ceaurriz J | 1989 | Nasal irritation and pulmonary toxicity of aliphatic amines. | J. Appl. Toxicol., 9 (5), 301-304 (1989) | Institut National de Recherche et de Sécurité |  |  | T04692 |  |

Data access

data published

Materials and methods

Test type

other: special acute respiratory reflex test

Limit test

yes

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | other guideline: American Society for Testing and Materials (ASTM), Standard Test Method for Estimating Sensory Irritancy of Airborne Chemicals, Annual Book of ASTM Standards Designation E 981-84 |  |

Principles of method if other than guideline

Comparative study including a series of primary, secondary and tertiary amines: Determination of sensory irritation potency by oronasal exposure, key parameter: RD50 (RD50 = the exposure concentration resulting in a 50-% decrease in the respiratory rate).

GLP compliance

no data

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |
| EC number | 200-860-9 |
| EC name | isopropylamine |

Details on test material

- Name of test material (as cited in study report): isopropylamine - Substance type: organic - Physical state: liquid - Analytical purity: no data, "high purity"

Test animals

Species

mouse

Strain

other: OF1

Sex

male

Details on test animals and environmental conditions

TEST ANIMALS - Source: Domaine des Oncins, Saint Germain sur l´Arbresle - Age at study initiation: - Weight at study initiation: 20 - 25 g - Fasting period before study: unfasted - Housing: 10 - Diet: ad libitum - Water: ad libitum - Acclimation period: 7 d ENVIRONMENTAL CONDITIONS: controlled, air-conditioned (no details)

Administration / exposure

Route of administration

inhalation: vapour

Type of inhalation exposure

nose only

Vehicle

other: unchanged (no vehicle)

Details on inhalation exposure

see below: "Any other information on test materials and methods"

Analytical verification of test atmosphere concentrations

yes (Adsorption of the test substance to silica gel followed by desorption under simultaneous derivatisation (m-toluene derivatives). Direct analysis: reverse-phase HPLC. )

Duration of exposure

15 min

Concentrations

Several concentrations measured from 69 - 197 ppm (analytical)

No. of animals per sex per dose

6 per test concentration

Control animals

other: internal control = each test animal before exposure

Details on study design

Before, during and after exposure respiratory rate was determined: Observation period: 60 min exposure, measurement 1x/15 min, 30 min recovery.

Any other information on materials and methods incl. tables

TEST PRINCIPLE and PROCEDURE The sensory irritation potency on the upper airways was evaluated by measuring the reflex expiratory bradypnea in exposed mice: The breathing frequency was used as an index of upper respiratory tract irritation by air-borne exposure to chemicals. Mice were restrained in a body plethysmographs with nose-only exposure. The breathing frequency was monitored before,during and after exposure. The maximum decrease during 15 min was recorded and the RD50 calculated (RD50 = the exposure concentration resulting in a 50% decrease in the respiratory rate). TEST ATMOSPHERE The exposure concentrations were generated by passing carrier air through liquid amine and by appropriate dilution with additional air.

Results and discussions

Effect levels

| Sex | Endpoint | Effect level | Based on | 95% CL | Exp. duration | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| male | other: Respiratory irritation, RD50 | 0.39 mg/L air (analytical) |  |  | 15 min |  |
| male | other: Respiratory irritation, RD50 | 157 ppm |  |  | 15 min |  |

Mortality

none

Clinical signs

suppression of respiration rate

Any other information on results incl. tables

The RD50value for isopropylamine was 157 ml/m3 (ppm) which corresponds to approx. 0.39 mg/L. Note: Calculation based on MW = 59 and molar volume of 24 L (20 °C): 1 ml/m3= 2.46 mg/m3

Applicant's summary and conclusion

Conclusions

In mice, isopropylamine is an acute sensory irritant when inhaled: effective exposure levels in mice = ~157 mL/m3 = ~390 mg/m3 that produce a 50% suppression of the respiration rate.

Endpoint study record: International Research and Development Corporation (1992).Inhal.rats(75-31-0).K2

| UUID |  | IUC5-fe67d636-7aff-4c48-8346-cf987400a127 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-12-30 10:06:07 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Purpose flag | supporting study; robust study summary |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | No guideline followed, GLP, test method sufficiently described |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | International Research and Development Corporation | 1992 | Acute inhalation toxicity evaluation on isopropylamine in rats |  | International Reasearch and Development Corporation | IRDC 214-057 | Taminco | EXT/94-081 | 1992-12-13 |

Data access

data submitter has Letter of Access

Data protection claimed

yes

Materials and methods

Test type

other: no data

Limit test

no

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| equivalent or similar to | OECD Guideline 403 (Acute Inhalation Toxicity) | yes ( Selection of exposure concentration limited: no 4-h value obtained ) |

GLP compliance

yes

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |
| EC number | 200-860-9 |

Details on test material

- Name of test material (as cited in study report): isopropylamine, anhydrous - Physical state: clear liquid - Analytical purity: 99.8% - Stability under test conditions: chemically stable and insensitive to light - Storage condition of test material: normal indoor ambient condition

Test animals

Species

rat

Strain

other: Charles River Crl:cd BR VAF/Plus; Sprague-Dawley derived

Sex

male/female

Details on test animals and environmental conditions

TEST ANIMALS - Source: Charles River Laboratories, Inc., Portage, Michigan - Age at study initiation: approx. 49-70 days - Weight at study initiation: males 214-300 and females 167-220 grams on day of exposure - Fasting period before study: Diet and water freely available except during exposure - Housing: individual stainless steel wire mesh cages - Diet (e.g. ad libitum): certified pelleted rodent chow #5002, Purina Mills, Inc, St-Louis, Missouri. Diet and water freely available except during exposure - Water (e.g. ad libitum): ad libitum ENVIRONMENTAL CONDITIONS - Temperature (°C), Humidity (%), Photoperiod (hrs dark / hrs light): maintained in accordance with the recommendations contained in the D.H.E.W. Publication entitled "Guide for the care and use of Laboratory animals"

Administration / exposure

Route of administration

inhalation

Type of inhalation exposure

whole body

Vehicle

other: no data

Details on inhalation exposure

GENERATION OF TEST ATMOSPHERE / CHAMBER DESCRIPTION - Method of holding animals in test chamber:150L plexi-glass and glass aquarium - Source and rate of air:air flow between 50 L/min, temperature 20-24°C, relative humidity 30-60 % -chamber air is supplied by either the HVAC system, which is separate from the general laboratory system, or by in-house compressed air. TEST ATMOSPHERE - Brief description of analytical method used: gas-phase infrared spectrophotometry

Analytical verification of test atmosphere concentrations

yes

Duration of exposure

| 20 min | Remarks | 6, 20, 60 min |
| --- | --- | --- |

Concentrations

6 min exposure were conducted at concentrations of: 13,700; 16,400; 16,600; 17,300 and 19,100 ppm 20 min exposure were conducted at concentrations of 5,180; 6,160; 7,050; 7,110; 8,060 and 9,580 ppm 60 min exposure were conducted at concentrations of 2,750; 3,360; 4,200; 4,780; 4,810 and 5,090 ppm

No. of animals per sex per dose

5 male and 5 female albino rats

Control animals

no data

Details on study design

- Duration of observation period following administration: 14 days - Frequency of observations and weighing: at 7 and 14 days post-exposure - Necropsy of survivors performed: yes - Other examinations performed: clinical signs, body weight,organ weights, histopathology, other:

Statistics

LC50 was analysed by one of the following methods: -A simplified method of evaluation dose-effect experiments T. Litchfield, Jr. and F. Wilcoxon, J. Pharmacol. and Expt. Therap. Vol 96, 1949. -The determination of the dosage-mortality curve from small numbers. C.I. Bliss, Quart. J. Pharmacol., Vol. 11, 1938

Results and discussions

Effect levels

| Sex | Endpoint | Effect level | Based on | 95% CL | Exp. duration | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| no data | LC50 | 18200 ppm |  | 16500 — 20000 | 6 min |  |
| no data | LC50 | 6920 ppm |  | 6500 — 7370 | 20 min |  |
| no data | LC50 | 4770 ppm |  | 4340 — 5240 | 60 min |  |

Mortality

6 min exposure: concentrations of 13,700; 16,400; 16,600; 17,300 and 19,100 ppm resulted in deaths of 10, 40, 30, 20, 70%, respectively 20 min exposure: concentrations of 5,180; 6,160; 7,050; 7,110; 8,060 and 9,580 ppm resulted in deaths of 0, 30, 50, 70, 70 and 100%, respectively 60 min exposure: concentrations of 2,750; 3,360; 4,200; 4,780; 4,810 and 5,090 ppm resulted in deaths of 0, 10, 30, 50, 40 and 70%, respectively

Clinical signs

6 min exposure: death, gasping, labored breath, rales and corneal opacity 20 min exposure: death, gasping, rales, labored breath, and corneal opacity 60 min exposure: death, gasping, labored breath, rales and corneal opacity

Body weight

6 min exposure: body weight gain was normal for post-exposure week 1 for females exposed to 13,700. Body weight gain was depressed for post-exposure week 1 for both males and females in all other groups. During the second post-exposure week, both males and females in all groups gained weight at a normal rate. 20 min exposure: body weight gain was normal for females exposed to 5,180; 6,160 and 7,050 ppm. Body weight gain was depressed for post-exposure week 1 at all other exposure levels. Females exposed to 8,060 ppm continued to have depressed body weight during the second post-exposure week while males and females in all other groups gained weight at a normal rate. 60 min exposure: body weight gain was normal for females exposed to 4,780 ppm but was depressed for post-exposure week 1 for all groups. Additionally, the males exposed to 2,750 ppm continued to have depressed body weight gain for the second post-exposure week. All others gained weight at a normal rate for week 2.

7.2.3 Acute toxicity: dermal

Endpoint study record: BASF AG 92/360.Acute toxicity: dermal.rat,KS

| UUID |  | IUC5-6bda4901-a875-4ff1-9e47-972c56ef4e3d |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-12-30 10:06:16 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Purpose flag | key study; robust study summary |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result |  |  |
| Reliability | 1 (reliable without restriction) |  |  |
| Rationale for reliability incl. deficiencies | Guideline study (OECD 402) |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | BASF AG | 1993 | Report on the Study of acute dermal Toxicity of Isopropylamine in Rats. | unpublished data | BASF AG, Department of Toxicology | 11A0360/921099 | BASF SE |  | 1993-12-02 |

Data access

data submitter has Letter of Access

Data protection claimed

yes

Materials and methods

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | OECD Guideline 402 (Acute Dermal Toxicity) |  |

GLP compliance

yes

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| Common name | isopropylamine |

Details on test material

- Name of test material (as cited in study report): Isopropylamine - Physical state: liquid - Analytical purity: 99.9 % - Lot/batch No.: 561211 - Storage condition of test material: refrigerator

Test animals

Species

rat

Strain

Wistar

Sex

male/female

Details on test animals and environmental conditions

TEST ANIMALS - Source: Dr. K. THOMAE GmbH, Biberach, Germany - Age at study initiation: young adult animals - Weight at study initiation: Animals of comparable weight (200 - 300 g) - Housing: singly in stainless stell wire mesh cages, Type DK-III - Diet: KLIBA-Labordiaet 343, Klingenthalmuehle AG, Switzerland, ad libitum - Water: water ad libitum - Acclimation period: at least 1 week ENVIRONMENTAL CONDITIONS - Temperature (°C): 20 - 24 °C - Humidity (%): 30 - 70 % - Photoperiod (hrs dark / hrs light): 12/12

Administration / exposure

Type of coverage

semiocclusive

Vehicle

other: aqua bidest

Details on dermal exposure

TEST SITE - Area of exposure: 50 cm2 - Type of wrap if used: Four layers of absorbent gauze REMOVAL OF TEST SUBSTANCE - Washing (if done): rinsing with warm water - Time after start of exposure: 24 h TEST MATERIAL - Concentration (if solution): 10 %

Duration of exposure

24 h

Doses

400 mg/kg bw

No. of animals per sex per dose

5

Control animals

no data

Details on study design

- Duration of observation period following administration: 14 days - Frequency of observations and weighing: Several times at the day of application and at least once daily therafter. - Frequency of weighing: Shortly before application, weekly thereafter and at the end of the study - Necropsy of survivors performed: yes - Other examinations performed: clinical signs, body weight

Results and discussions

Effect levels

| Sex | Endpoint | Effect level | Based on | 95% CL | Remarks |
| --- | --- | --- | --- | --- | --- |
| male/female | LD50 | > 400 mg/kg bw | act. ingr. (10% dilution in water because of corrosivity) |  |  |
| male/female | LD0 | 400 mg/kg bw | act. ingr. (10% dilution in water because of corrosivity) |  |  |

Mortality

No mortality occurred.

Clinical signs

On the day of application the animals showed the following syptoms: dyspnoea, apathy, staggering, spastic gait, tremor, piloerection, exophthalmos and poor general state.

Body weight

mean body weight: male: 248 g, 298 g, 327 g (day 0, day 7, day 13); female: 218 g, 233 g, 247 g (day 0, day 7, day 13);

Gross pathology

Sacrificed animals: Organs: no pathologic findings noted. Skin: full thickness necrosis (4 males/4 females), superficial necrosis (1 female), superficial crust formation (1 male)

Any other information on results incl. tables

The acute dermal LD50 of the test material was found to be greater than 400 mg/kg bw for the male und female animals. There is indication that the test material causes local irritation to exposed tissues.

Endpoint study record: Smyth (+1951, rabbit, RL3)

| UUID |  | IUC5-bf57055d-446e-4130-9e2e-32f8de0c56ac |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-12-30 10:06:28 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Study result type | experimental result |  |  |
| --- | --- | --- | --- |
| Reliability | 3 (not reliable) |  |  |
| Rationale for reliability incl. deficiencies | Only Abstract / short communication; documentation insufficient for assessment |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Smyth HF, Carpenter CP, Weil CS | 1951 | Range Finding Toxicity Data: List IV. Arch. Ind. Hyg. Occup. Med., 4, 119-122 | Arch. Ind. Hyg. Occup. Med., 4, 119-122 | Mellon Institute of Industrial Research, University of Pittsburgh |  |  | T01614 |  |
| review article or handbook | Cavender FL, Bingham E, Cohrssen B, Powell CH (editors) | 2000 | Aliphatic and Alicyclic Amines, in: | Patty´s Industrial Hygiene and Toxicology, J. Wiley & Sons 2000 |  |  |  | T02713 | 1999-12-31 |

Data access

data published

Materials and methods

Test type

standard acute method

Principles of method if other than guideline

Smyth et al., Screening test

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |
| EC number | 200-860-9 |
| EC name | isopropylamine |

Details on test material

Isopropylamine [CAS No. 75-31-0], not further specified

Test animals

Species

rabbit

Strain

New Zealand White

Sex

male

Administration / exposure

Type of coverage

occlusive

Duration of exposure

24h

Doses

no data

No. of animals per sex per dose

4

Control animals

no data

Any other information on materials and methods incl. tables

About 1/10 of skin area was treated under occlusive conditions for 24 h.

Results and discussions

Effect levels

| Sex | Endpoint | Effect level | Based on | 95% CL | Remarks |
| --- | --- | --- | --- | --- | --- |
| male | LD50 | 380 mg/kg bw |  |  |  |

Any other information on results incl. tables

LD50 is given as 550 ml/kg bw (S.D. 300 - 1020 ml/kg).

Endpoint study record: Bioresearch (1975). acute_dermal (75-31-0)

| UUID |  | IUC5-76be8b74-dd30-49d8-87bd-dc96368cc5cd |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-12-30 10:06:35 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Purpose flag | weight of evidence |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result | Study period | 1975 |
| Reliability | 3 (not reliable) |  |  |
| Rationale for reliability incl. deficiencies | Not GLP, test concentration not clearly defined, no standard protocol, poor description of experimental conditions, no data on controls or historical controls |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | Biosearch | 1975 | 50% MIPA - Acute dermal toxicity - Rabbits |  | Bioserch Inc, Philadelphia, PA | MX-75-026 | Air Products & Chemcals Inc. |  | 1975-10-12 |

Data access

data submitter has Letter of Access

Data protection claimed

yes

Materials and methods

Test type

fixed dose procedure

Limit test

yes

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| no guideline followed |  |  |

Principles of method if other than guideline

10 albino rabbits 2,5-3,5 kg, 5 animals with intact skin and 5 animals with abraded skin, 24h exposure, 14d observation period for signs of toxicty or mortality

GLP compliance

no data

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |
| EC number | 200-860-9 |

Details on test material

- Name of test material (as cited in study report): 50% MIPA, no further information

Test animals

Species

rabbit

Strain

other: Albino

Sex

no data

Details on test animals and environmental conditions

TEST ANIMALS - Weight at study initiation: 2,5 - 3,5 kg ENVIRONMENTAL CONDITIONS - no information

Administration / exposure

Type of coverage

occlusive

Vehicle

unchanged (no vehicle)

Details on dermal exposure

TEST SITE - Area of exposure: no data - Type of wrap if used: "impervious material" REMOVAL OF TEST SUBSTANCE -no data - removed after 24h TEST MATERIAL - 1000 mg/kg

Duration of exposure

24h

Doses

1 (1000 mg/kg) solvent not mentioned not clear if 50% ww or vv

No. of animals per sex per dose

no data on sex of animals

Control animals

no

Details on study design

- Duration of observation period following administration: 14 days - Frequency of observations: daily - Necropsy of survivors performed: no - Other examinations performed: none

Results and discussions

Effect levels

| Sex | Endpoint | Effect level | Based on | 95% CL | Remarks |
| --- | --- | --- | --- | --- | --- |
| no data | LD50 | > 1000 mg/kg bw |  |  |  |

Mortality

No mortality observed

Clinical signs

Not reported

Body weight

Not reported

Gross pathology

Not reported

Other findings

Not reported

Applicant's summary and conclusion

Interpretation of results

other:

Criteria used for interpretation of results

not specified

Conclusions

LD50 > 1000 mg/kg for 50% MIPA (presumably 500 mg/kg of pure MIPA?)

7.3 Irritation / corrosion

Endpoint summary: Irritation / corrosion

| UUID |  | IUC5-b49b381a-0b1c-4156-adb1-5f3502c47029 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:58:11 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

EU: REACH

Key value for chemical safety assessment

Skin irritation / corrosion

corrosive

Eye irritation

corrosive

Respiratory irritation

irritating

Discussion

As a primary short-chain alkylamine, isopropylamine reacts strongly alkaline, which explains its strongly irritating properties.

7.3.1 Skin irritation / corrosion

Endpoint study record: BASF AG VI/391.Skin irritation / corrosion.rabbit

| UUID |  | IUC5-95b2ff96-1721-485f-8a1e-272f8348ed99 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-12-30 10:06:47 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Purpose flag | supporting study; robust study summary |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Acceptable, well documented report which meets basic scientific principles. |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | BASF AG | 1960 | Industrial Hygiene Orientatin Investigation | unpublished data | BASF AG, Department of Toxicology | VI/391 | BASF SE |  | 1960-02-04 |

Data access

data submitter has Letter of Access

Data protection claimed

yes

Materials and methods

Type of method

in vivo

Principles of method if other than guideline

See details in remarks on material and methods.

GLP compliance

no

Test materials

Test material identity

| Identifier | Identity |
| --- | --- |
| Common name | isopropylamine |

Details on test material

- Name of test material (as cited in study report): Isopropylamin

Test animals

Species

rabbit

Strain

Vienna White

Test system

Type of coverage

no data

Preparation of test site

no data

Vehicle

unchanged (no vehicle)

Duration of treatment / exposure

1 min, 5 min and 15 min

Observation period

3 weeks

Number of animals

2

Control animals

other: untreated skin site of the same animal served as control

Details on study design

TEST SITE - Area of exposure: 2.5 x 2.5 cm REMOVAL OF TEST SUBSTANCE - Washing (if done): rinsed with 50 % Lutrol - Time after start of exposure: 1 min, 5 min or 15 min

Any other information on materials and methods incl. tables

BASF-Test. Two animals were treated for 1, 5 and 15 min using occlusive conditions. An application site of 2.5x2.5 cm was covered with the liquid test substance. After the application time (1, 5, 15 min) the skin was washed with Lutrol (50%). The animals were observed for3 weeksand skin changes were recorded daily.

Results and discussions

Irritation / corrosion results

| Irritation parameter | Basis | Time point | Score | Max. score | Reversibility | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| other: necrosis score, 1, 5 or 15 min exposure | mean | 24 h - 48 h - 72 h | 2 | 3 | not reversible |  |

Any other information on results incl. tables

The application of the test substance for 1, 5 or 15 min caused brown-blackish hard necrosis.The appearing effects are considered as full thickness necrosis. At the end of the 3 week observation period scar formation was observed. A 20% preparation in water was also tested. The skin effects were less pronounced. The 1 and 5 min application caused slight erythema which was reversible within 48 h and 7 days, respectively. The 15 min application caused slight edema, slight erythema and red-brown necrosis of parchment-like consistency. After 2 weeks scar formation was observed.

Endpoint study record: Siglin_Celanese (+1989, 100%, rabbit, key, RL1)

| UUID |  | IUC5-01e9be31-5d47-4279-9b27-baceff607bdf |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-12-30 10:06:54 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Purpose flag | key study; robust study summary; used for classification; used for MSDS |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result |  |  |
| Reliability | 1 (reliable without restriction) |  |  |
| Rationale for reliability incl. deficiencies | GLP guideline study |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | Siglin, J.C. | 1989 | Dermal Corrosivity of C-899 isopropylamine in Rabbits (IMO) | Hoechst Celanese Corp., unpublished study | Springborn Life Sciences Inc. (SLS), Spencerville, Ohio, USA | 3206.24 | OXEA | T02440 | 1989-05-01 |

Data access

data submitter is data owner

Data protection claimed

yes, but willing to share

Materials and methods

Type of method

in vivo

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| equivalent or similar to | OECD Guideline 404 (Acute Dermal Irritation / Corrosion) | yes (Exposure time) |

GLP compliance

yes

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test materials

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |
| EC number | 200-860-9 |
| EC name | isopropylamine |

Details on test material

Isopropylamine [CAS No. 75-31-0]; supplied by Hoechst Celanese Corp. (C-899; not further specified)

Test animals

Species

rabbit

Strain

New Zealand White

Details on test animals and environmental conditions

TEST ANIMALS - Source: Mohican Valley Rabbitry, Loudonville, Ohio, USA - Age at study initiation: young adults - Weight at study initiation: 1.9 - 2.2 kg - Housing: individually housed in stainless steel cages - Diet, water: ad libitum - Acclimation period: > 5d ENVIRONMENTAL CONDITIONS - Temperature: 65 - 78°F - relative Humidity: 35-70% - Air changes (per hr): no information given - Photoperiod: 12 hrs dark / 12 hrs light

Test system

Type of coverage

occlusive

Preparation of test site

shaved

Vehicle

unchanged (no vehicle)

Amount/concentration applied

amount (volume): 0.5 ml

Duration of treatment / exposure

3 min (and 1 h, not further considered)

Observation period

3 min, 1, 24, 48 and 72h and 7 d

Number of animals

3

Details on study design

TEST SITE - Area of exposure: right flank - Type of wrap : 1.5 in x1.5 in (= 3.81 cm x 3.81 cm) gauze patch wrapped with a double layer plastic wrap REMOVAL OF TEST SUBSTANCE - Washing: wiped with gauze soaked with distilled water - Time after start of exposure: 3 min SCORING SYSTEM for erythema and edema respectively from 0 (no effect) to 4 (severe effect) according to Draize (1959)

Results and discussions

Irritation / corrosion results

| Irritation parameter | Basis | Time point | Score | Max. score | Reversibility | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| erythema score | animal #1 | 72 h | 4 | 4 | not reversible ( within 7d) | Necrosis (after 3 min), eschar formation (after 24 h) |
| erythema score | animal #2 | 72 h | 4 | 4 | not reversible ( within 7 d) | Necrosis (after 3 min), eschar formation (after 24 h) |
| erythema score | animal #3 | 72 h | 4 | 4 | not reversible ( within 7 d) | Necrosis (after 3 min), eschar formation (after 24 h) |
| edema score | animal #1 | 3 min | 2 | 4 | fully reversible within: 7 d | necrotic centre, scored at edge of test site |
| edema score | animal #2 | 3 min | 2 | 4 | not fully reversible within: 7 d | scored within and around test site |
| edema score | animal #3 | 3 min | 2 | 4 | fully reversible (7 d) | scored within and around test site |
| edema score | animal #1 | 72 h | 1 | 4 | fully reversible within: 7 d | scored at edge of test site |
| edema score | animal #2 | 72 h | 2 | 4 | not fully reversible within: 7 d | scored at edge of test site |
| edema score | animal #3 | 72 h | 1 | 4 | fully reversible within: 7 d | scored at edge of test site |

Irritant/corrosive response data

The treatment produced immediate necrosis and subsequently eschar within 24 h. The scores mentioned above relate to symptoms around the necrotic centre.

Applicant's summary and conclusion

Interpretation of results

Category 1A (corrosive)

Endpoint study record: Smyth (+1951, rabbit, RL4)

| UUID |  | IUC5-5ae26e12-b028-45b6-a425-ae2c11c016c1 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-12-30 10:07:02 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Study result type | experimental result |  |  |
| --- | --- | --- | --- |
| Reliability | 4 (not assignable) |  |  |
| Rationale for reliability incl. deficiencies | No guideline study, different scoring system |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Smyth HF, Carpenter CP, Weil CS | 1951 | Range Finding Toxicity Data: List IV. Arch. Ind. Hyg. Occup. Med., 4, 119-122 | Arch. Ind. Hyg. Occup. Med., 4, 119-122 | Mellon Institute of Industrial Research, University of Pittsburgh |  |  | T01614 |  |
| review article or handbook | Cavender FL, Bingham E, Cohrssen B, Powell CH (editors) | 2000 | Aliphatic and Alicyclic Amines, in: | Patty´s Industrial Hygiene and Toxicology, J. Wiley & Sons 2000 |  |  |  | T02713 | 1999-12-31 |

Data access

data published

Materials and methods

Type of method

in vivo

Principles of method if other than guideline

Smyth Carpenter / Draize Test

GLP compliance

no

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test materials

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |
| EC number | 200-860-9 |
| EC name | isopropylamine |

Details on test material

Isopropylamine [CAS No. 75-31-0], not further specified

Test animals

Species

rabbit

Test system

Type of coverage

open

Preparation of test site

shaved

Vehicle

unchanged (no vehicle)

Amount/concentration applied

undiluted

Duration of treatment / exposure

24 hour(s)

Any other information on materials and methods incl. tables

Primary skin irritation on rabbits records in a 10-grade ordinal series the most severe reaction on the clipped skin of any of the five albino rabbits within 24 h of application of 0.01 mL of undiluted sample or of dilutions in water, propylene glycol, or kerosene. Grade 1 in the table indicates the least visible capillary injection from undiluted chemical. Grade 6 indicates necrosis when undiluted, and Grade 10 indicates necrosis from a 0.01% solution. (from Cavender et al. 2000). --------------

Results and discussions

Any other information on results incl. tables

Grade 6 irritation following application of 10 µl of the TS: Severe erythema, edema, and necrosis.[Note: For methodology: see also Smyth, H.F. et al. (1962): Range-finding toxicity data: List VI. Amer. Ind. Hyg. Assoc. J., 23, 95-107 [T00840a] --------------

Applicant's summary and conclusion

Interpretation of results

corrosive

Conclusions

The results are not evaluable as to the grade of irritation effects after 4 h exposure.

7.3.2 Eye irritation

Endpoint study record: BASF AG VI/391.Eye irritation.rabbit,SS

| UUID |  | IUC5-e8e111d7-9dd0-4b72-91d4-15ccbd433491 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-12-30 10:07:06 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Purpose flag | supporting study; robust study summary |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result | Study period | 02. Oct 1956 - 07. Nov 1956 |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Acceptable, well documented report which meets basic scientific principles. |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | BASF AG | 1960 | Industrial Hygiene Orientating Investigation | unpublished data | BASF AG, Department of Toxicology | VI/391 | BASF SE |  | 1960-02-04 |

Data access

data submitter has Letter of Access

Data protection claimed

yes

Materials and methods

Type of method

in vivo

Principles of method if other than guideline

see details in remarks on material and methods.

GLP compliance

no

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test materials

Test material identity

| Identifier | Identity |
| --- | --- |
| Common name | isopropylamine |

Details on test material

- Name of test material (as cited in study report): isopropylamine

Test animals

Species

rabbit

Strain

Vienna White

Test system

Vehicle

unchanged (no vehicle)

Amount/concentration applied

TEST MATERIAL - Amount(s) applied (volume or weight with unit): 1 drop (ca. 50 µl)

Duration of treatment / exposure

single application, eyes were not washed out

Observation period

19 days

Number of animals

3

Control animals

other: single application, eyes were not washed out

Any other information on materials and methods incl. tables

50 µL of the test substance were applied to the conjunctival sac of one eye in 3 animals. The animals were observed after 10 min, 1 and 3 h on the day of treatment and up to 19 days afterwards. Findings were recorded daily. The eyes were not washed out after 24 hours as specified in OECD Guideline 405. The report describes findings after 1 and 24 hours and after 8 days. For final evaluation, the findings after 48 and 72 hours from the raw data were taken into account.

Results and discussions

Overall irritation / corrosion results

| Irritation parameter | Basis | Time point | Score | Max. score | Reversibility | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| cornea score | mean | 24 h - 48 h - 72 h | 3 | 4 | not reversible | complete loss of the eye |

Any other information on results incl. tables

The application of the test substance caused severe corrosion of the mucous membranes and the cornea predominantly expressed by erythema, chemosis and corneal opacity. At the end of the observation period of 19 days the mucous membranes and the cornea were completly destroyed. Loss of the eye.

Applicant's summary and conclusion

Interpretation of results

corrosive

Endpoint study record: Blaszcak , Auletta (+1985, rabbit, RL2)

| UUID |  | IUC5-0390e589-9bdd-45fc-ab78-f883ef4c30cd |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-12-30 10:07:12 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Purpose flag | key study; robust study summary; used for classification; used for MSDS |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Only short communication available, comparable to guideline study with acceptable restrictions (limited documentation) |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | Blaszcak D.L., Auletta C.S. | 1985 | Acute Toxicity and Irritation Studies - C-899; Eye irritation project No. 6001-85, | Hoechst Celanese Corp., unpublished study | Bio/dynamics Inc., Department of Toxicology | 6001-85 | OXEA | T02442 | 1985-10-22 |

Data access

data submitter is data owner

Data protection claimed

yes, but willing to share

Materials and methods

Type of method

in vivo

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| equivalent or similar to | OECD Guideline 405 (Acute Eye Irritation / Corrosion) | yes (only one animal) |

GLP compliance

no data

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test materials

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |
| EC number | 200-860-9 |
| EC name | isopropylamine |

Details on test material

Isopropylamine [CAS No. 75-31-0], not further specified

Test animals

Species

rabbit

Strain

no data

Test system

Vehicle

unchanged (no vehicle)

Amount/concentration applied

0.1 ml undiluted applied

Duration of treatment / exposure

24 h

Observation period

24 h

Number of animals

1

Control animals

no

Details on study design

not rinsed

Results and discussions

Overall irritation / corrosion results

| Irritation parameter | Basis | Time point | Score | Max. score | Reversibility | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| overall irritation score | animal #1 | 24 h | 80 |  | not reversible | corrosive |

Any other information on results incl. tables

At 24 h the animal exhibited moderate to severe conjunctival irritation (redness, chemosis, discharge, and necrosis) and corneal opacity, stippling and ulceration (The iris could not be evaluated). The animal was killed after 24 h for humane reasons. Total score: 80

Applicant's summary and conclusion

Interpretation of results

corrosive

Criteria used for interpretation of results

EU

Endpoint study record: Smyth (+1951, rabbit, RL2)

| UUID |  | IUC5-a6f337b0-cda9-4d40-88ed-2fe9e1005bfa |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-12-30 10:07:19 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Purpose flag | supporting study; robust study summary |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Meets generally accepted scientific standards, sufficiently documented and acceptable for assessment |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Smyth HF, Carpenter CP, Weil CS | 1951 | Range Finding Toxicity Data: List IV. Arch. Ind. Hyg. Occup. Med., 4, 119-122 | Arch. Ind. Hyg. Occup. Med., 4, 119-122 | Mellon Institute of Industrial Research, University of Pittsburgh |  |  | T01614 |  |
| review article or handbook | Cavender FL, Bingham E, Cohrssen B, Powell CH (editors) | 2000 | Aliphatic and Alicyclic Amines, in: | Patty´s Industrial Hygiene and Toxicology, J. Wiley & Sons 2000 |  |  |  | T02713 | 1999-12-31 |
| publication | Carpenter CP, Smyth HF | 1946 | Chemical burns of the rabbit cornea | Am. J. Ophth. 29 (1946) 1363-1372 | Mellon Institute, Pittsburgh, Pennsylvania |  |  | T02576 |  |

Data access

data published

Materials and methods

Type of method

in vivo

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| no guideline followed |  |  |

Principles of method if other than guideline

according to Carpenter et al. 1946 (T02576)

GLP compliance

no

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test materials

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |
| EC number | 200-860-9 |
| EC name | isopropylamine |

Test animals

Species

rabbit

Strain

other: albino

Test system

Vehicle

water

Amount/concentration applied

Concentration: 1 % active substance Amount applied: 0.5 ml

Observation period

24

Number of animals

=< 5 (Carpenter et al., 1946 T02576)

Details on study design

not rinsed

Any other information on materials and methods incl. tables

Eyes of various animals were treated with a defined volume of different dilutions or undiluted material. Changes of the cornea were evaluated based on a specific grading system: grade 1 in the table indicates a very small area of necrosis resulting from 0.5 ml of undiluted chemical in the eye (=<5 % of cornea with necrosis), grade 5 indicates a so-called severe burn from 0.005 ml (undiluted): 63 - 87 % of cornea with necrosis), and grade 10 indicates a severe burn from 0.5 ml of a 1% solution (63 - 87 % of cornea with necrosis). ( according to Carpenter et al. 1946) ----------------- Excerpt from Cavender et al. 2000: "Eye injury in rabbits records the degree of corneal necrosis from various volumes and concentrations of chemical as detailed by Carpenter and Smyth. Grade 1 in the table indicates at most a very small area of necrosis resulting from 0.5 mL of undiluted chemical in the eye; Grade 5 indicates a so-called severe burn from 0.05 mL (note: must read 0.005 mL), and Grade 10 indicates a severe burn from 0.5 mL of a 1% solution in water or propylene glycol." ----------------

Results and discussions

Irritant/corrosive response data

A 1% solution in water resulted in severe necrosis.

Any other information on results incl. tables

The TS caused an irritation score of 10 on a 10-grade scale (method described in: Carpenter et al. 1946, Chemical Burns of the Rabbit Cornea, Am. J. Ophth 29, 1363ff; T02576).

Applicant's summary and conclusion

Interpretation of results

corrosive

7.4 Sensitisation

Endpoint summary: Sensitisation

| UUID |  | IUC5-cee87144-98fe-4f70-a50e-48db41728328 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:58:12 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

EU: REACH

Skin sensitisation

Short description of key information

Isopropylamine gave no evidence of a skin-sensitising potential in the guinea-pig maximisation assay as well as in the mouse-ear swelling test (MEST).

Key value for chemical safety assessment

Skin sensitisation

not sensitising

7.4.1 Skin sensitisation

Endpoint study record: Elf Atochem (+1994, Arkema, GPMT, KS, RL1)

| UUID |  | IUC5-7d365e5e-88e1-46d5-b9ea-b27409431985 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-12-30 10:07:31 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Purpose flag | key study; robust study summary; used for classification; used for MSDS |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result | Study period | Oct./Nov. 1993 |
| Reliability | 1 (reliable without restriction) |  |  |
| Rationale for reliability incl. deficiencies | GLP guideline study |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | Clouzeau J. | 1994 | Monosisopropylamine - Skin sensitization test in guinea-pigs (Maximization method of Magnusson B and Kligman AM) | Elf Atochem, unpublished report | Centre International de Toxicologie (C.I.T.), France | 10931 TSG | Arkema | T10010 | 1994-01-04 |

Data access

data submitter has Letter of Access

Data protection claimed

yes

Materials and methods

Type of method

in vivo

Type of study

Guinea pig maximisation test

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | OECD Guideline 406 (Skin Sensitisation) |  |

GLP compliance

yes

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |
| EC number | 200-860-9 |
| EC name | isopropylamine |

Details on test material

- Name of test material (as cited in study report): Monoisopropylamine - molecular formula: C3H9N - molecular weight: 59.09 - Substance type: organic - Physical state: liquid - Analytical purity: 99.7 % - Purity test date: - Lot/batch No.: 9301P0355 -Origin: Elf Atochem - Storage condition of test material: room temperature, protected from light

Test animals

Species

guinea pig

Strain

Dunkin-Hartley

Sex

male/female

Details on test animals and environmental conditions

TEST ANIMALS - Source: Centre dÉlevage Lebeau, France - Age at study initiation: no data - Weight at study initiation: 432 +-34 g (m); 440 +-22 g) (f) - Housing: single - Diet: ad libitum - Water: ad libitum - Acclimation period: >=5 d before start ENVIRONMENTAL CONDITIONS - Temperature (°C): 22 +-1 - Humidity (%): 50 +-20 - Air changes (per hr): no technically forced exchange - Photoperiod (hrs dark / hrs light): 12 / 12

Test system

Traditional sensitisation test

Route of induction exposure

intradermal and epicutaneous

Route of challenge exposure

epicutaneous, occlusive

Vehicle

water

Concentration

0.1 % (id induction) 1 % (epicutaneous induction) 10 % = maximum non-irritant concentration (epicutaneous challenge)

No. of animals per dose

5 m + 5 f (pos. and neg. control groups); 10 m + 10 f (test group)

Details on study design (Traditional tests)

RANGE FINDING TESTS: Minimum Irritant Concentration for id and epicutaneous application MAIN STUDY A. INDUCTION EXPOSURE - No. of exposures: 2x (day 1 and 8) - Exposure period: 48 h (dermal) following 24h treatment with 0.5 mL 10% laurylsulphate - Test groups: FCA/saline, FCA + 0.1% TS/saline, 0.1% TS/saline (intradermal); 0.5 mL 1% TS/saline (dermal) - Control group: FCA/saline, FCA/saline + saline, saline (intradermal); 0.5 mL saline (dermal) - Site: scapular region - Frequency of applications: 1x (id), 1x (dermal) - Duration: 8 d - Concentrations: 0.1 % (id), 1 % (dermal) B. CHALLENGE EXPOSURE - No. of exposures: 1 (dermal) - Day(s) of challenge: day 22 - Exposure period: 24 h - Test groups: - Control group: - Site: right flank (TS) / left flank (saline) - Concentrations: 10 % - Evaluation (hr after challenge): 24 and 48 h C. OTHER Clinical observation, body-weight control until day 25 Necropsy on day 25

Positive control substance(s)

yes (dinitro 2,4-chlorobenzene)

Results and discussion

Positive control results

Challenge with 0.1 % DNCB: erythema scores 2 for 4/5 animals, score 1 for 1/5 animals (reading 24 h p.a.) Challenge with 0.5 % DNCB: erythema scores 4 for 2/5 animals, score 3 for 3/5 animals (reading 24 h p.a.)

Traditional sensitisation test

Results of test (except LLNA)

| Reading | Hours after challenge | Group | Dose level | No. with + reactions | Total no. in group | Clinical observations |
| --- | --- | --- | --- | --- | --- | --- |
| 1st reading | 24 | test group | 10 % | 0 | 20 | none |
| 2nd reading | 48 | test group | 10 % | 0 | 20 | none |
| 1st reading | 24 | negative control | 10 % | 0 | 10 | none |
| 2nd reading | 48 | negative control | 10 % | 0 | 10 |  |

LLNA

Any other information on results incl. tables

Pre-Test intradermal: 0.1 % were irritant, 1 % necrotic after 24 h post-application dermal: 10 % not irritant after 24 h post-application, 25 % were necrotic. Main test No clinical signs, no mortalities, normal body-weight gain. After induction, irritation in the control group and necrosis in treated group were observed. No cutaneous reaction (scores 0) was observed at 24 and 48 h after dermal challenge at the maximum mon-irritant concentration.

Applicant's summary and conclusion

Interpretation of results

not sensitising

Endpoint study record: Blaszcak, Auletta (+1987, MEST, RL3)

| UUID |  | IUC5-169b1033-0bac-477d-8793-cbc12136cd8c |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-12-30 10:07:36 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Study result type | experimental result |  |  |
| --- | --- | --- | --- |
| Reliability | 3 (not reliable) |  |  |
| Rationale for reliability incl. deficiencies | Unsuitable test system (not suitable to detect weak sensitisers), challenge dose seems too high |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | Blaszcak D.L., Auletta C.S. | 1987 | Mouse Ear Swelling Test (Sensitization Assay) - Test material C-899 | Hoechst Celanese Corp., unpublished study | Bio/dynamics Inc., Department of Toxicology | 6766-86 | OXEA | T02443 | 1987-05-26 |
| other company data | Brower, D. | 1988 | Receipt of toxicity test final report - 11 alkylamines. Interoffice Memorandum / Summary Report | Hoechst-Celanese Corp., unpublished | Bio/dynamics Inc., Department of Toxicology | MDB: 34-88 | OXEA | T01250 | 1988-03-02 |

Data access

data submitter is data owner

Data protection claimed

yes, but willing to share

Materials and methods

Type of method

in vivo

Type of study

Mouse ear swelling test

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| no guideline available |  |  |

Principles of method if other than guideline

based on "Gad SC, Dunn BJ, and Dobbs DW 1986: Development and validation of an alternative dermal sensitization test: Mouse Ear Swelling Test (MEST). Toxicol. Appl. Pharmacol., 84, 93-114"

GLP compliance

yes

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |
| EC number | 200-860-9 |
| EC name | isopropylamine |

Test animals

Species

mouse

Strain

CF-1

Details on test animals and environmental conditions

TEST ANIMALS - Source: Charles River Breeding laboratories, Inc., Kingston, New York , USA - Age at study initiation: - Weight at study initiation: - Diet, water: ad libitum ENVIRONMENTAL CONDITIONS - Temperature (°F): 67 - 70°F - relative Humidity (%): 30 -70% - Photoperiod: 12hrs dark / 12hrs light

Test system

Traditional sensitisation test

Route of induction exposure

other: see details below

Route of challenge exposure

other: see details below

Vehicle

other: 70% ethanol

Concentration

see below

No. of animals per dose

10

Details on study design (Traditional tests)

1st application: Induction 0.5 % active substance open epicutaneous 2nd application: Induction 50 other: %FCA intracutaneous 3rd application: Challenge 50 % active substance open epicutaneous

LLNA

Any other information on materials and methods incl. tables

Mouse Ear Swelling Test (MEST) Induction: topical application to the clipped abdomen Induction: intradermal application of Freund´s Complete Adjuvans (FCA) Challenge: topical to the dorsal and ventral surface of the ear Based on a range-finding to identify a non-corrosive concentration, 1% dilution was selected for the induction and 10% dilution for the challenge. FCA was applied as 50% aqueous dilution on two sites of the abdomen. The test sites were tape-stripped to remove the horny layer. Dinitrochlorobenzene (DNCB) served as positive control substance (10 animals). Day 0: Induction with 20 µl FCA i.d. and with 100 µl TS topically to the test sites. Days 1, 2, and 3: Induction with 100 µl TS topically to the test sites. Day 10: Challenge with 10 µl TS and vehicle on the ears. Day 17: Re-challenge with 10 µl TS and vehicle on the ears, but the application was switched, i.e. the ear previously treated with the TS was now treated with the vehicle and vice versa. An irritation control group (5 animals) correspondingly treated with vehicle and FCA was run simultaneously. The reading of reactions was done 24 and 48 hours after challenge. ---------------

Results and discussion

LLNA

Any other information on results incl. tables

Mouse Ear Sweelling Test (MEST) POSITIVE CONTROL: 60 and 50 % of the animals showed a positive response (at least 20 % increase in ear thickness) after 24 and 48 h, respectively. TEST GROUP: No animals challenged with the TS showed a positive reaction at challenge (0, 5, 11, and 17 % incremental increase in thickness observed), and 10 % of the animals (= 1/10) at re-challenge (0, 5, 10, and 20 % incremental increase in thickness observed). There were no responders in the irritation control group (0, 5, 11, and 16 % incremental increase in thickness observed). ------------------

Applicant's summary and conclusion

Interpretation of results

not sensitising

Conclusions

The mouse-ear swelling test (MEST) serves as a screen to detect potent sensitisers (moderate and strong ones). Weak sensitisers are unlikely to be detected in this test. According to EPA guideline OPPTS 870.2600, a negative result in the MEST does not indicate that the substance is a non-sensitizer, and should be confirmed in an accepted test using guinea pigs and or LLNA if appropriate. The test provided no evidence of a sensitising potential of isopropylamine. Also all other amines tested gave no positive response. The functionality of the test system was demonstrated by using an appropriate positive control substance which was clearly positive. Despite the limited number of animals used and the test restrictions, the result can be weighed in relation to the positive control, the dose used to provoke an effect and the negative result obtained with three structure-related primary amines (cyclohexylamine, hexylamine, n-octylamine). Hence, isopropylamine is not considered to be a potent sensitiser and unlikely to elicit a skin allergic reaction. ---------------

7.5 Repeated dose toxicity

Endpoint summary: Repeated dose toxicity

| UUID |  | IUC5-c1344f5d-79f7-4235-8206-e81af44f4457 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:58:12 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

EU: REACH

Short description of key information

Isopropylamine produced nasal inflammation in rats exposed to 500 mg/m3 for 13 weeks, while 100 mg/m3 showed no adverse effects.

Key value for chemical safety assessment

Repeated dose toxicity: inhalation

| Effect level | NOAEC | 100 mg/m³ |
| --- | --- | --- |
| Test type | subchronic |  |
| Species | rat |  |

Target organ

respiratory: nose

Discussion

In a valid inhalation guideline study on rats, the highest concentration, 500 mg/m3 (LOAEC), produced nasal inflammation (females), a slight decrease in body weight of < 7 % (males), and a slight decrease in serum-glucose level (females). The systemic effects are considered to be of no pathological relevance. Because of the highly irritating activity of isopropylamine, oral and dermal application was not indicated.

7.5.1 Repeated dose toxicity: oral

Endpoint study record: Waiver_Repeated dose toxicity: oral

| UUID |  | IUC5-fa4c34c6-69a1-4b4b-941c-ef1a55fab61b |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:51:55 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| EU: REACH |  |
| --- | --- |
| Data waiving | other justification |
| Justification for data waiving | In accordance with column 2 of Annex VIII and IX (required in Section 8.6.1 and 8.6.2) of REACH Regulation 1907/2006: the oral route is not considered to be the relevant route of potential exposure. |

7.5.2 Repeated dose toxicity: inhalation

Endpoint study record: Monsanto (1988, rat, RL1)

| UUID |  | IUC5-5092ed32-2235-4f1a-a201-63bff4f19eac |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-12-30 10:07:53 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Purpose flag | key study; robust study summary; used for classification; used for MSDS |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result |  |  |
| Reliability | 1 (reliable without restriction) |  |  |
| Rationale for reliability incl. deficiencies | GLP guideline study |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | Heydens WF, Thake DC | 1988 | Inhalation fertility, reproduction, and subchronic toxicity studies with isoproyplamine in Sprague- Dawley rats | unpublished | Monsanto Agricultural Company - Environmental Health Laboratory/Missouri, USA | EHL 86040, 86041, 86042, 86089 / ML-86-076 | Monsanto Corporation | T10168 | 1988-08-22 |

Data access

data submitter has Letter of Access

Data protection claimed

yes

Materials and methods

Test type

subchronic

Limit test

no

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| equivalent or similar to | OECD Guideline 413 (Subchronic Inhalation Toxicity: 90-Day) |  |

GLP compliance

yes

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |
| EC number | 200-860-9 |
| EC name | isopropylamine |

Test animals

Species

rat

Strain

Sprague-Dawley

Sex

male/female

Administration / exposure

Route of administration

inhalation: vapour

Vehicle

air

Analytical verification of doses or concentrations

yes

Duration of treatment / exposure

13 weeks

Frequency of treatment

6 h/d, 5d/wk

Doses/concentrations

| 20, 101, and 499 mg/m3 |  |
| --- | --- |
| Basis | analytical conc. |

No. of animals per sex per dose

15

Control animals

yes, concurrent no treatment

Results and discussions

Effect levels

| Endpoint | Effect level | Based on | Sex | Basis for effect level / Remarks |
| --- | --- | --- | --- | --- |
| NOAEC | 100 mg/m³ air |  | male/female | histopathology (nose) |
| NOAEC | 500 mg/m³ air (nominal) | test mat. | male/female | overall effects (excluding local effects) |

Results of examinations

Body weight and weight gain

yes (not considered as toxicologically relevant)

Haematology

no effects

Clinical chemistry

yes (not considered as toxicologically relevant)

Gross pathology

no effects

Histopathology: non-neoplastic

yes (treatment-related)

Details on results

CLINICAL SIGNS AND MORTALITY BODY WEIGHT AND WEIGHT GAIN: at 500 mg/m3 (males): slight reduction in body weight throughout (< 7%) FOOD CONSUMPTION FOOD EFFICIENCY WATER CONSUMPTION OPHTHALMOSCOPIC EXAMINATION HAEMATOLOGY CLINICAL CHEMISTRY at 500 mg/m3 (females): slight decrease in serum-glucose level URINALYSIS NEUROBEHAVIOUR ORGAN WEIGHTS GROSS PATHOLOGY HISTOPATHOLOGY: NON-NEOPLASTIC at 500 mg/m3 (females): inflammation of nasal mucosa HISTOPATHOLOGY: NEOPLASTIC (if applicable) HISTORICAL CONTROL DATA (if applicable) OTHER FINDINGS

Endpoint study record: Dudek (+1991, rat, 28 d, RL4)

| UUID |  | IUC5-09057efb-ef28-4673-8654-110b53286f4e |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-12-30 10:08:03 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Purpose flag | supporting study |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result |  |  |
| Reliability | 4 (not assignable) |  |  |
| Rationale for reliability incl. deficiencies | Abstract / short communication |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Dudek B.R. et al. | 1991 | One month inhalation study of isopropylamine (IPA) in rats | Toxicologist, 11, No. 1, 88 |  |  |  |  |  |
| other: database | GESTIS - Stoffdatenbank | 2005 | Isopropylamin [CAS-No. 75-31-0] | GESTIS - Stoffdatenbank, Gefahrstoffinformationssystem der gewerblichen Berufsgenossenschaften [http://biade.itrust.de/biade/lpext.dll/Infobase/] |  |  |  | T02711 | 2005-04-03 |
| review article or handbook | Greim, H.; Bury, D.; Klimisch, H.J.; Oeben-Negele, M.; Ziegler-Skylakakis, K. | 1998 | Toxicity of aliphatic amines: structure-activity relationship | Chemosphere, 36, 271-295 |  |  |  | T01201 |  |

Data access

data published

Materials and methods

Test type

subacute

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| no guideline available |  |  |

Principles of method if other than guideline

no data

GLP compliance

no data

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |
| EC number | 200-860-9 |
| EC name | isopropylamine |

Details on test material

no data

Test animals

Species

rat

Strain

Sprague-Dawley

Sex

male/female

Administration / exposure

Route of administration

inhalation

Analytical verification of doses or concentrations

no data

Duration of treatment / exposure

28d, (20 exposures)

Frequency of treatment

5 d/wk, 6 h/d

Doses/concentrations

| 0; 0.1; 0.5 and 1.35 mg/L air (approx. 41, 204, and 550 ppm) |  |
| --- | --- |
| Basis | no data |

No. of animals per sex per dose

15

Control animals

yes

Details on study design

Post-exposure period: none

Results and discussions

Effect levels

| Endpoint | Effect level | Based on | Sex | Basis for effect level / Remarks |
| --- | --- | --- | --- | --- |
| NOEC | 0.1 mg/L air (nominal) |  | male/female | = 43 ppm |
| LOAEC | 0.5 mg/L air (nominal) |  | male/female | Corneal opacities and degeneration of the nasal epithelium were seen at the higher doses. |

Results of examinations

Any other information on results incl. tables

Dose-related irritation of ocular and nasal mucous membranes associated with inflammatory degenerative changes of the nasal epithelium, and - additionally - turbidity of the cornea at the higher concentration. Also a reduction in body-weight gain was noted in the top-dose group. The local epithelial lesions were confirmed by histological examination (GESTIS 2005). ---------------- Note: Greim et al. 1998

7.5.3 Repeated dose toxicity: dermal

Endpoint study record: Waiver_Repeated dose toxicity: dermal

| UUID |  | IUC5-214652bc-8561-4a52-8519-ef1d59ecb0aa |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:51:56 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| EU: REACH |  |
| --- | --- |
| Data waiving | other justification |
| Justification for data waiving | In accordance with column 2 of Annex VIII and IX (required in Section 8.6.1 and 8.6.2) of REACH Regulation 1907/2006: the dermal route is not considered to be the relevant route of potential exposure, as the substance is highly corrosive. |

7.6 Genetic toxicity

Endpoint summary: Genetic toxicity

| UUID |  | IUC5-fbd66d50-4cf5-4ad2-9712-146512990cbc |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:58:13 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

EU: REACH

Short description of key information

Isopropylamine did not show mutagenic activity either in the bacterial reverse mutation assay (Ames test) or in a mammalian cell culture assay for chomosomen aberration and gene mutation (HPRT).

Key value for chemical safety assessment

Genetic toxicity

negative

7.6.1 Genetic toxicity in vitro

Endpoint study record: Zeiger (+1987, Ames, KS, RL2)

| UUID |  | IUC5-5b601414-ae20-4207-9103-dda8fe784cfa |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-12-30 10:08:07 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Purpose flag | key study; robust study summary; used for MSDS |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Comparable to guideline study, part of NTP/USA |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Zeiger, E., Anderson, B., Haworth, S., Lawlor, T., Mortelmans, K., Speck, W. | 1987 | Salmonella mutagenicity test: III. Results from the testing of 255 chemicals | Environ. Mutagen., 9(Suppl. 9), 1-110 |  |  |  | T02620 |  |

Data access

data published

Materials and methods

Type of genotoxicity

gene mutation

Type of study

bacterial reverse mutation assay (e.g. Ames test)

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| equivalent or similar to | OECD Guideline 471 (Bacterial Reverse Mutation Assay) | yes (preincubation assay, two different sources of a microsomal metabolic system; 4 instead of 5 tester strains.) |

Principles of method if other than guideline

Ames-Test modified acc. to Haworth et al. (1983): Environ. Mutagen. 5, Suppl. 1, 3-142

GLP compliance

no data

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |
| EC number | 200-860-9 |
| EC name | isopropylamine |

Details on test material

Isopropylamine [CAS No. 75-31-0], purity 99% (Aldrich Chemicals) (No. 159 in publication)

Method

Species/strain

| Species/strain | S. typhimurium TA 1535, TA 1537, TA 98 and TA 100 |
| --- | --- |
| Metabolic activation | with and without |
| Metabolic activation system | Metabolic activation systems were derived from Arochlor-induced livers of male SD rats and male Syrian hamsters. |

Test concentrations

0.010, 0.033, 0.100, 0.333, 1.0, 3.333 mg/plate

Vehicle

- Vehicle(s)/solvent(s) used: water - Justification for choice of solvent/vehicle: Test substance well soluble in water

Controls

| Negative controls | no (corresponds to vehicle control) |
| --- | --- |
| Solvent / vehicle controls | yes (distilled water) |
| True negative controls | no |
| Positive controls | yes |
| Positive control substance | other: -S9: Sodium azide (TA1535, TA100); 9-aminoacridine (TA1537); 4-nitro-o-phenylenediamine (TA98) // +S9: 2-aminoanthracene (all strains). |

Details on test system and conditions

METHOD OF APPLICATION: preincubation, agar plate DURATION - Preincubation period: 20 min - Exposure duration: 2 days at 37 °C NUMBER OF REPLICATIONS: 3 DETERMINATION OF CYTOTOXICITY - relative total growth (background lawn)

Evaluation criteria

Mutagenic (+) or weakly mutagenic (+w) if a reproducible, dose-related increase in revertants over the corresponding solvent controls in replicate trials was seen. Questionable (?) if a reproducible increase in revertants did not meet the criteria of either "+" or "+w", or if single doses produced an increase in repeat trials.

Results and discussions

Test results

| Species/strain | S. typhimurium TA 1535, TA 1537, TA 98 and TA 100 |
| --- | --- |
| Metabolic activation | with and without |
| Test system | all strains/cell types tested |
| Genotoxicity | negative |
| Cytotoxicity | yes >= 3.333 mg/plate (see Report, Appendix 2, p. 79/80) |
| Vehicle controls valid | yes |
| Negative controls valid | not examined |
| Positive controls valid | yes |

Additional information on results

--

Applicant's summary and conclusion

Interpretation of results

negative

Endpoint study record: Molinier (+1994, ChrAb in vitro, KS, RL1)

| UUID |  | IUC5-b6525555-67a1-458f-aaa0-292842dc48da |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-12-30 10:08:14 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Purpose flag | key study; robust study summary |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result |  |  |
| Reliability | 1 (reliable without restriction) |  |  |
| Rationale for reliability incl. deficiencies | GLP guideline study |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | Molinier B | 1994 | In vitro mammalian cytogenetic test in cultured human lymphocytes. | unpublished report | Centre International de Toxicologie (C.I.T.), France | 10932 MLH | Arkema | T10009 | 1994-03-24 |

Data access

data submitter has Letter of Access

Data protection claimed

yes

Materials and methods

Type of genotoxicity

chromosome aberration

Type of study

in vitro mammalian chromosome aberration test

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | OECD Guideline 473 (In vitro Mammalian Chromosome Aberration Test) | no ( However, in the presence of S9-mix, selection of the dose levels was based on pH and not on the mitotic index, since the use of non-physiological pH was not recommended.) |

GLP compliance

yes

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS name | 75-31-0 |
| EC number | 200-860-9 |
| EC name | isopropylamine |

Details on test material

- Name of test material (as cited in study report): Monoisopropylamine - molecular fourmula: C3H9N - molecular weight: 59.09 - Substance type: organic - Physical state: liquid - Analytical purity: 99.7 % - Purity test date: - Lot/batch No.: 9301P0355 -Origin: Elf Atochem - Storage condition of test material: room temperature, protected from light

Method

Species/strain

| Species/strain | lymphocytes: human |
| --- | --- |
| Details on mammalian cell lines (if applicable) | - Donators: two healthy humans with no recent X-ray exposure and believed to be free of viral infection - Time: on day of of culture - Culture: whole blood in sterile tubes containing heparin |
| Metabolic activation | with and without |
| Metabolic activation system | liver microsomal fraction (S9) of male SD rats induced with Arochlor 1254 |

Test concentrations

-S9 mix: 30, 100, and 300 µg/mL +S9 mix: 250, 500, and 1000 µg/mL Doses used for evaluation: -S9-mix: 30, 100, and 300 µg/mL +S9-mix: 250, 500, and 1000 µg/mL

Vehicle

distilled water

Controls

| Negative controls | no |
| --- | --- |
| Solvent / vehicle controls | yes (water control) |
| True negative controls | no |
| Positive controls | yes |
| Positive control substance | other: mitomycin C (- S9); cyclophosphamide (+S9) |

Details on test system and conditions

METHOD OF APPLICATION: in medium DURATION - Exposure duration: 24 and 48 h (-S9); 2 h (+S9) - Fixation time (start of exposure up to fixation or harvest of cells): 24 and 48 h(+/-S9) SPINDLE INHIBITOR (cytogenetic assays): Colcemid, applied 2 h before harvest NUMBER OF REPLICATIONS: 2 per dose and interval NUMBER OF CELLS EVALUATED: 200 metaphases per dose and interval DETERMINATION OF CYTOTOXICITY - Method: mitotic index, evaluated on 1000 cells each OTHER EXAMINATIONS: - Determination of polyploidy: yes - Determination of endoreplication: yes - Other: gaps, chromatid / chromosomal breaks and exchanges, multiple aberrations and pulverisation

Evaluation criteria

Validity: - Aberrant cell frequencies (excluding gaps) in the controls fell within the range of historical data; - the positive controls induced statistically significant increases in the aberrent cell frequencies that fall within the historical contrl ranges. Evaluation criteria: - a statistically significant increase in the aberrant cell frequency for at least one of the doses, for any harvest time.

Statistics

X2 test (significance level of P = 0.05) to compare mean aberration frequencies with the controls.

Any other information on materials and methods incl. tables

The cells were harvested at about 1.5 normal cell cycle times after the beginning of treatment and approximately 24 h later.

Results and discussions

Test results

| Species/strain | lymphocytes: human |
| --- | --- |
| Metabolic activation | with and without |
| Test system | strain/cell type: human lymphocytes |
| Genotoxicity | negative |
| Cytotoxicity | yes at >=300µg/mL (-S9), no cytotoxicity (+S9) |
| Vehicle controls valid | yes |
| Negative controls valid | not examined |
| Positive controls valid | yes |

Additional information on results

TEST-SPECIFIC CONFOUNDING FACTORS - Effects of pH: cytotoxicity RANGE-FINDING/SCREENING STUDIES: COMPARISON WITH HISTORICAL CONTROL DATA: ADDITIONAL INFORMATION ON CYTOTOXICITY: - At 300 and 1000 µg/ml, the pH values were 8 and 10, respectively. - 5000 µg/ml were hemolytic to the cells. MITOTIC INDEX: - With metabolic activation (Report, Table 3 and 4): 1000 µg/ml (2 treatment / 24 h harvest): 87 % of the control 1000 µg/ml (2 treatment / 48 h harvest): 93 % of the control - Without metabolic activation (Report, Table 1 and 2): 300 µg/ml (24 h treatment/24 h harvest): 66 % of the control 1000 µg/ml (24 h treatment/24 h harvest): 0 % of the control 300 µg/ml (48 treatment/48 h harvest): 76 % of the control 1000 µg/ml (48 h treatment/48 h harvest): 45 % of the control

Any other information on results incl. tables

The aberrant cell frequencies in the negative controls were within the range of the historical data (0.4 +/-0.5 %), gaps excluded. The aberrant cell frequencies in the positive controls were significantly higher than that of the negative controls (P 0.001). Overview of aberrant cell frequency (from Report, Table 1 and 2): ## Number of aberrations; % aberrant cells. Doses [µg/mL]; + gaps; - gaps; + gaps; - gaps. ## Without metabolic activation: 24 / 24 h or 48 / 48 h exposure / harvest time. 24 h; 48 h; 24 h; 48 h; 24 h; 48 h; 24 h; 48 h. 0; 3; 3; 1; 2; 1.5; 1.5; 1.5; 1.0. 30; 4; 6; 2; 2; 2.0; 3.0; 1.0; 1.0. 100; 4; 1; 0; 1; 2.0; 0.5; 0.0; 0.5. 300; 1; 3; 1; 2; 0.5; 1.5; 0.5; 1.0. MMC [0.2]; 42; -; 36; -; 19; -; 16; -. MMC = mitomycin Overview of aberrant cell frequency (from Report, Table 3 and 4): ## Number of aberrations; % aberrant cells. Doses [µg/mL]; + gaps; - gaps; + gaps; - gaps. ## With metabolic activation: 2 / 24 h or 2 / 48 h exposure / harvest time. 24 h; 48 h; 24 h; 48 h; 24 h; 48 h; 24 h; 48 h. 0; 2; 2; 1; 2; 1.0; 1.0; 0.5; 1.0. 250; 5; 2; 3; 1; 2.5; 1.0; 1.5; 0.5. 500; 4; 5; 2; 1; 2.0; 2.5; 1.0; 0.5. 1000; 1; 2; 0; 1; 0.5; 1.0; 0.0; 0.5. CPA [50]; 60; -; 50; -; 21.5; -; 19.5; -. CPA = cyclophosphamide

Applicant's summary and conclusion

Interpretation of results

negative without metabolic activation

Endpoint study record: Covance (+2010, HPRT, KS, RL1)

| UUID |  | IUC5-77ad86c8-e622-4679-9ba8-dede85736390 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-12-30 10:08:22 EST |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| Purpose flag | key study; robust study summary; used for classification; used for MSDS |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result |  |  |
| Reliability | 1 (reliable without restriction) |  |  |
| Rationale for reliability incl. deficiencies | GLP guideline study |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | Stone V | 2010 | Isopropylamine - Mutation at the hprt locus of mouse lymphoma L5178Y cells (MLA) using the Microtitre fluctuation technique | unpublished | Covance Laboratories Ltd, North Yorkshire, England | 8218072 | REACH Consortium of Alkylamine Producers | T10356 | 2010-04-30 |

Data access

data submitter is data owner

Data protection claimed

yes, but willing to share

Materials and methods

Type of genotoxicity

gene mutation

Type of study

mammalian cell gene mutation assay

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | OECD Guideline 476 (In vitro Mammalian Cell Gene Mutation Test) ( as of 1997) |  |

Principles of method if other than guideline

HPRT assay for the detection of mutations at the hypoxanthine-guanine phosphoribosyl transferase (hprt) locus (6-thioguanine [6TG] resistance) in mouse lymphoma cells using a fluctuation protocol according to Cole et al. 1983. Reference: Cole J, Arlett C F, Green M H L, Lowe J and Muriel W 1983: A comparison of the agar cloning and microtitration techniques for assaying cell survival and mutation frequency in L5178Y mouse lymphoma cells. Mutation Research, 111, 317-386

GLP compliance

yes (incl. certificate)

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| EC number | 200-860-9 |
| CAS number | 75-31-0 |
| EC name | isopropylamine |

Details on test material

- Name of test material (as cited in study report): Isopropylamine (MIPA) - Substance type: organic - Physical state: liquid - Analytical purity: 99.91 % (GC) [see Report. Appendix 7: OXEA, Specification of 30 Oct. 2009] - Impurities (identity and concentrations): Water (0.06 %), n-i-propylidene-i-propylamine (0.014%) - Expiration date of the lot/batch: Sep. 2010 - Stability under test conditions: stable - Storage condition of test material: stored at 2 - 8 °C under desiccant in the dark

Method

Target gene

Induction of a forward mutation in the X-linked hprt locus: Resistance to the toxic analogue 6-thioguanine (6TG) results from lack of hypoxanthine-guanine phosphoribosyl transferase (HPRT) activity. Thus, the hprt-negative mutants are unable to use 6TG and survive in its presence.

Species/strain

| Species/strain | mouse lymphoma L5178Y cells |
| --- | --- |
| Details on mammalian cell lines (if applicable) | - Type and identity of media: RPMI media (containing antibiotics and varying concentrations of horse serum, heat-inactivated, from 0 - 20 %) - Properly maintained: yes - Periodically checked for Mycoplasma contamination: yes - Periodically "cleansed" against high spontaneous background: yes, - Other procedures: purge of TK-negative mutants |
| Additional strain characteristics | other: TK proficient (TK+) |
| Metabolic activation | with and without |
| Metabolic activation system | post-mitochondrial fraction from livers of male SD rats previously induced with Arochlor-1254 |

Test concentrations

Experiment 1: 0, 80, 120, 210, 240, and 270 µg/mL (evaluated) Experiment 2: 0, 100, 150, 175, 200, 250, 300, and 400 µg/mL (evaluated)

Vehicle

- Vehicle(s)/solvent(s) used: water - Justification for choice of solvent/vehicle: sufficient water solubility of the test substance

Controls

| Negative controls | no |
| --- | --- |
| Solvent / vehicle controls | yes ( water as vehicle) |
| True negative controls | no |
| Positive controls | yes |
| Positive control substance | other: 4-nitroquinoline-N-oxide (-S9); benzo(a)pyrene (+S9) |

Details on test system and conditions

METHOD OF APPLICATION: in medium, incubation in centrifuge tubes, culture flasks, and wells of microtiter plates, depending on the operation step DURATION - Preincubation period: no data, pre-culture in RPMI 10 after thawing up to an appropriate cell density - Exposure duration: 3 h - Incubation temperature: 37 +- 1 °C - Expression time (cells in growth medium): 7 d - Selection time (if incubation with a selection agent): 12 d SELECTION AGENT (mutation assays): 6-thioguanine (6-TG) NUMBER OF REPLICATIONS: 2 NUMBER OF CELLS EVALUATED: Selection of 6-TG resistance: 4 microtiter plates of 90 wells at 2 x10^4 cells per well each (= 384 x2 x10^4 cells per test concentration) DETERMINATION OF CYTOTOXICITY - Method: Relative survival in the presence and absence of test substance (by visual counting of viable clones in microtiter plates) DETERMINATION of VIABILITY (after expression period) - Method: visual counting of viable clones in microtiter plates DETERMINATION of 6-TG RESISTANCE (after expression period) - Method: visual inspection of microtiter plates for the number wells containing clones.

Evaluation criteria

ACCEPTANCE CRITERIA The assay was considered valid if the following criteria were met: 1. the mutant frequencies in the negative (vehicle) control cultures fell within the normal range (not more than three times the historical mean value) 2. at least one concentration of each of the positive control chemicals induced a clear increase in mutant frequency (the difference between the positive and negative control mutant frequencies was greater than half the historical mean value). EV ALUATION CRITERIA For valid data, the test article was considered to induce forward mutation at the hprt locus in mouse lymphoma L5178Y cells if: 1. the mutant frequency at one or more concentrations was significantly greater than that of the negative control (p < 0.05) 2. there was a significant concentration﷓relationship as indicated by the linear trend analysis (p < 0.05) 3. the effects described above were reproducible. Results that only partially satisfied the assessment criteria described above were considered on a case-by-case basis.

Statistics

Statistical significance of mutant frequencies was carried out according to the UKEMS guidelines. Thus the control log mutant frequency (LMF) was compared with the LMF from each treatment concentration, and secondly the data were checked for a linear trend in mutant frequency with test article treatment. These tests require the calculation of the heterogeneity factor to obtain a modified estimate of variance. Reference: Robinson et al. 1990: Statistical evaluation of bacterial/mammalian fluctuation tests. In Statistical Evaluation of Mutagenicity Test Data (Ed D J Kirkland), Cambridge University Press, pp 102 – 140.

Any other information on materials and methods incl. tables

# ANALYSIS OF RESULTS All calculations were performed by computer using validated software. From the zero term of the Poisson distribution the probable number of clones/well (P) on microtitre plates in which there are EW empty wells (without clones) out of a total of TW wells is given by: P = -ln (EW/TW) Plating efficiency (PE)in any given culture is therefore: PE = P/Noof cells plated per well and as an average of 1.6 cells per well were plated on all survival and viability plates, PE = P/1.6 Percentage relative survival (% RS)in each test culture was therefore determined by comparing plating efficiencies in test and control cultures thus: % RS = [PE(test)/PE(control)] x 100 To take into account any loss of cells during the 3 hour treatment period, percentage relative survival values for each concentration of test article were adjusted as follows: Adjusted % RS =; % RS x Post-treatment cell concentration for test article treatment. Post-treatment cell concentration for vehicle control. All percentage relative survival (% RS) values were adjusted as described above. Mutant frequency (MF)is usually expressed as "mutants per 106viable cells". In order to calculate this, the plating efficiencies of both mutant and viable cells in the same culture were calculated: MF = [PE (mutant)/PE (viable)] x 106 From the formulae given and with the knowledge that 2 x 104cells were plated/well for mutation to 6TG resistance, PE (mutant) = P (mutant)/2 x 104 PE (viable) = P (viable)/1.6 where, in each case, P = -1n (EW/TW) Therefore, MF = [P (mutant)/2 x 104] x [1.6/P (viable)] x 106

Results and discussions

Test results

| Species/strain | mouse lymphoma L5178Y cells |
| --- | --- |
| Metabolic activation | with and without |
| Test system | strain/cell type: mouse lymphoma L5178Y cells |
| Genotoxicity | negative |
| Cytotoxicity | yes (dose-related: The highest concentrations to provide >10 % RS were 148 µg/mL +S9 and 296 µg/mL -S9.) |
| Vehicle controls valid | yes |
| Negative controls valid | not examined |
| Positive controls valid | yes |

Additional information on results

TEST-SPECIFIC CONFOUNDING FACTORS - Effects of pH: marked increases of >= 1 pH unit at >= 240 µg/mL - Effects of osmolality: no - Evaporation from medium: no - Precipitation: only at the highest concentration (which turned out to be too toxic) RANGE-FINDING/SCREENING STUDIES: for cytotoxicity testing up to solublity limit of 500 µg/mL COMPARISON WITH HISTORICAL CONTROL DATA: Acceptance criterion, Control/Historical ratio > 0.5, fulfilled for positive controls (See Appendices)

Any other information on results incl. tables

Summary of mutation data[mean of two replicate cultures]: Experiment 1 (3 hour treatment in the absence and presence of S-9) [for individual replicates, see Appendix 2, Table 10 (-S9), and Appendix 3, Table 18 (+S9)] Treatment (µg/mL); -S-9; Treatment (µg/mL); +S-9. % RS; MF§; % RS; MF§. 0; 100; 1.28; 0; 100!; 1.26. 80; 97; 0.23; NS; 40; 85; 1.79; NS. 120; 81; 1.22; NS; 200; 66; 2.11; NS. 210; 36; 2.66; NS; 240; 58; 0.51; NS. 240; 30; 1.62; NS; 280; 45; 1.94; NS. 270; 19; 0.78; NS; 300; 33; 2.20; NS. 350; 17; 0.39; NS. Linear trend; NS; Linear trend; NS. NQO; B[a]P. 0.1; 44; 19.72; 2; 54; 10.78. 0.15; 34; 40.90; 3; 20; 48.09. Experiment 2 (3 hour treatment in the absence and presence of S-9) [for individual replicates, see Appendix 4, Table 26 (-S9), and Appendix 5, Table 34 (+S9)] Treatment (µg/mL); -S-9; Treatment (µg/mL); +S-9. % RS; MF§; % RS; MF§. 0; 100; 5.14; 0; 100; 6.41. 100; 98; 4.46; NS; 200; 90; 7.21; NS. 150; 82; 4.28; NS; 250; 93; 3.43; NS. 175; 70; 4.92; NS; 300; 60; 4.95; NS. 200; 56; 4.65; NS; 325; 53; 5.67; NS. 250; 36; 2.97; NS; 350; 32; 4.64; NS. 300; 28; 5.45; NS; 375; 20; 5.70; NS. 400; 3; 6.88; NS. Linear trend; NS; Linear trend; NS. NQO; B[a]P. 0.1; 59; 54.25; 2; 66; 80.64. 0.15; 58; 66.29; 3; 39; 104.52. § = 6TG resistant mutants/106viable cells 7 days after treatment % RS =Percent relative survival adjusted by post treatment cell counts NS = not significant ! =Based on one replicate only

Applicant's summary and conclusion

Interpretation of results

negative

Conclusions

Isopropylamine (MIPA) did not induce mutation at the hprt locus of L5178Y mouse lymphoma cells under test conditions employed in this study. Thisincluded treatments up to highly toxic concentrations in two independent experiments, in the absence and presence of a rat liver metabolic activation system.

7.7 Carcinogenicity

Endpoint study record: WAIVER_Carcinogenicity

| UUID |  | IUC5-ed48bec1-93df-48d2-8886-7e90b959041a |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:51:59 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| EU: REACH |  |
| --- | --- |
| Data waiving | study scientifically unjustified |
| Justification for data waiving | A carcinogenicity study is not warranted as all in vitro mutagenicity studies are negative. There is no hint for any carcinogenic potential. |

7.8 Toxicity to reproduction

Endpoint summary: Toxicity to reproduction

| UUID |  | IUC5-15a38449-2f84-4e9a-ba94-44ac3aa31d3e |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:58:14 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

EU: REACH

Effects on fertility

Short description of key information

NOAEC = 500 mg/m3: No adverse effect on reproductive performance in the P-generation and no adverse effects on development of the progeny of the P-generation (F1)

Key value for chemical safety assessment

| Effect level for fertility (inhalation exposure) | NOAEC | 500 mg/m³ |  |
| --- | --- | --- | --- |

Discussion

A valid one-generation study in rats failed to reveal an impairment of fertility in either gender and gave no evidence of adverse developmental effects in the offspring. The exposure concentration over prolonged a period of time was limited by the highly irritating activity of isopropylamine. For the same reasons, repeated oral and dermal application was not indicated.

Developmental toxicity / teratogeniticy

Short description of key information

In the offspring of rats, no adverse developmental or teratogenic effects were observed after intrauterine exposure to isopropylamine.

Key value for chemical safety assessment

| Effect level for developmental toxicity (inhalation exposure) | NOAEC | 1000 mg/m³ |  |
| --- | --- | --- | --- |

Discussion

In a valid GLP guideline study, the embryonal and foetal development was not impaired following inhalation exposure of pregnant rats to high levels of isopropylamine. Maternal toxicity was observed at 1000 mg/m3,based on effects on bodyweight, treatment-related clinical signs viz. nasal discharge, rales, laboured breathing, sneezing and fur staining/encrustation and macroscopically observed reduced abdominal fat. Hence, the NOAEC(maternal) was 500 mg/m3, while the NOAEC(offspring) was 1000 mg/m3. Because of the highly irritating activity of isopropyamine, oral and dermal application was not indicated.

7.8.1 Toxicity to reproduction

Endpoint study record: WAIVER_Toxicity to reproduction

| UUID |  | IUC5-54f76dbd-51e1-4a79-bd29-1326fcf59b99 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:52:00 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| EU: REACH |  |  |  |
| --- | --- | --- | --- |
| Data waiving | study scientifically unjustified |  |  |
| Justification for data waiving | In accordance with Annex XI of REACH Regulation 1907/2006, a reproductive toxicity study over two generations is not indicated for isopropylamine, because the mammalian dataset that includes a one-generation study provides no evidence of a reproductive potential of this chemical (low systemic toxicity, no mutagenic potential, no potential to bioaccumulate). |  |  |
| Study result type | other: one-generation study |  |  |

Materials and methods

Test materials

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |

Endpoint study record: Monsanto (+1988, ACC 2008, OECD 415, rat, KS, RL2)

| UUID |  | IUC5-d6e84ca5-e0b4-41f9-92de-4e4adc76d7e2 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | Collette / Epona Associates, LLC / Willington / United States |
| Date |  | 2012-06-26 14:33:37 EDT |
| Remarks |  |  |

Administrative Data

| Purpose flag | key study; robust study summary; used for classification; used for MSDS |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | GLP study, similar to OECD Test Guidelines with acceptable deviations |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | Heydens WF, Thake DC | 1988 | Inhalation fertility, reproduction, and subchronic toxicity studies with isoproyplamine in Sprague- Dawley rats | unpublished | Monsanto Agricultural Company - Environmental Health Laboratory/Missouri, USA | EHL 86040, 86041, 86042, 86089 / ML-86-076 | Monsanto Corporation | T10168 | 1988-08-22 |

Data access

data submitter has Letter of Access

Data protection claimed

yes

Materials and methods

Test type

one-generation study

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| equivalent or similar to | OECD Guideline 415 (One-Generation Reproduction Toxicity Study) | yes (Dosing of females only until gd 21 No information on termination of male rats, no dosing during lactation until weaning; pathology of P-generation limited?) |

Principles of method if other than guideline

Toxicity to fertility

GLP compliance

yes

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |
| EC number | 200-860-9 |
| EC name | isopropylamine |

Details on test material

- Name of test material (as cited in study report): Isopropylamine (2-aminopropane) - Substance type: organic - Physical state: clear liquid - Analytical purity: 99.77 % - Lot/batch No.: LP-606

Test animals

Species

rat

Strain

Sprague-Dawley

Sex

male/female

Details on test animals and environmental conditions

TEST ANIMALS - Age at study initiation: (P) 54 d wks; (F1) x wks - Weight at study initiation: (P) Males: 241 - 300 g; Females: 163 - 207 g - Fasting period before study: ENVIRONMENTAL CONDITIONS - Temperature (°C): - Humidity (%): - Air changes (per hr): - Photoperiod (hrs dark / hrs light):

Administration / exposure

Route of administration

inhalation: vapour

Type of inhalation exposure (if applicable)

whole body

Vehicle

air

Details on mating procedure

- M/F ratio per cage: 1:1 - Length of cohabitation: maximally 7 d - Proof of pregnancy: vaginal plug / sperm in vaginal smear referred to as [day 0 / day 1] of pregnancy - Female reproduction (males unexposed): After 7 days of unsuccessful pairing replacement of first male by another male with proven fertility. - Male fertility (females unexposed): After 7 days of unsuccessful pairing replacement of first female by another unexposed female

Analytical verification of doses or concentrations

yes

Duration of treatment / exposure

- Duration of test: up to 15 weeks (males) - Duration of test: 17-18 weeks (females)

Frequency of treatment

untreated + treated males: Six hours per day, 5 days per week treated and untreated females: Six hours per day, 5 days per week (premating and mating period) treated females: Six hours per day, 7 days per week (during gestation period)

Doses / concentrations

| 20, 100, and 499 mg/m3 |  |
| --- | --- |
| Basis | analytical conc. (- Distribution: range 1.8-10% - Stability: range 11-15%) |

| 21, 109 and 526 mg/m3 |  |
| --- | --- |
| Basis | nominal conc. (average) |

No. of animals per sex per dose

Twenty male rats per treatment - Twenty-five female rats per treatment

Control animals

yes

Examinations

Parental animals: Observations and examinations

ADMINISTRATION / EXPOSURE: - Premating period: 10.5 weeks - Route of administration: inhalation - Post exposure period: from day 21 of gestation until day 21 of lactation - Exposure duration: 6 hours/day, 5 days/week (after mating 7 days/week until day 20 of gestation) - Preparation of particles: using a Laskin-style nebulizer - Air changes: 12/hour STANDARDIZATION OF LITTERS: - On day 4 of lactation litters were culled to 4 pups/sex PARAMETERS ASSESSED IN PARENTS: - Detailed observations and body weight were assessed on a weekly basis - Mortality/clinical observations: daily during and after exposure - Body weight females: weekly until mating, thereafter on day 0, 6, 13 and 20 of gestation and on day 0, 4, 7, 14 and 21 of lactation - Number of live pups at delivery - After termination, females were examined and the number of implantations and the number of corpora lutea were determined ORGANS EXAMINED AT NECROPSY (MACROSCOPIC AND MICROSCOPIC): - Gross necropsy of dams and pups on day 21 of lactation - Histopathology on tissues of pups of control and high dose group

Litter observations

OFFSPRING: - Body weight: day 0, 4, 7 and 14 of lactation (litter weight by sex) and on day 21 (individual weights) - External examination on day 0, 4, 7, 14 and 21 of lactation ORGANS EXAMINED AT NECROPSY (MACROSCOPIC AND MICROSCOPIC): - Gross necropsy of dams and pups on day 21 of lactation - Histopathology on tissues of pups of control and high dose group

Statistics

- Dunnett's test, (uncorrected) chi-square test

Results and discussions

Effect levels

| Endpoint | Generation | Sex | Effect level | Based on | Basis for effect level / Remarks |
| --- | --- | --- | --- | --- | --- |
| NOAEC (highest concentration tested) | P | male/female | 500 mg/m³ air |  | No significant effects on mating or fertility parameters |
| NOAEC (direct exposure: P-generation / indirect (intrauterine) exposure: embryos/foetus) | F1 | male/female | 500 mg/m³ air |  | based on neonatal and perinatal mortality and body weight |

Results of examinations: parental animals

Clinical signs (parental animals)

yes incidentally fur discolouration and focal hair loss (not treatment-related)

Body weight and food consumption (parental animals)

yes (In males during the majority of the study, no significant effects in females)

Test substance intake (parental animals)

not examined not applicable (inhalation)

Reproductive function: estrous cycle (parental animals)

not examined

Reproductive function: sperm measures (parental animals)

not examined

Reproductive performance (parental animals)

no effects

Gross pathology (parental animals)

no effects

Histopathology (parental animals)

no effects

Details on results (parental animals)

PATERNAL TOXIC EFFECTS BY DOSE LEVEL: - Two males in the high dose group died (week 1 and first week of mating) - Males in the high dose group had a significantly lower body weight during the majority of the study - No significant differences appeared in mating and fertility parameters - Maternal mortality: none - Body weight: no treatment related effects - Clinical signs: incidentally fur discolouration and focal hair loss (not treatment-related) - Mating success: 23/25, 25/25, 23/25 and 25/25 at 0, 20, 100 and 500 mg/m3, respectively - Number pregnant per dose level: 21, 19, 17 and 19 at 0, 20, 100 and 500 mg/m3, respectively - Gestational length: 22-23 days - Gross pathology incidence and severity: no treatment related effects - The number of corpora lutea, implantations and resorptions was not affected by treatment

Results of examinations: offspring

Viability (offspring)

no effects

Clinical signs (offspring)

no data

Body weight (offspring)

no effects

Sexual maturation (offspring)

not examined

Gross pathology (offspring)

no effects

Histopathology (offspring)

no effects

Details on results (offspring)

- Mean litter size (day 0): 14, 13, 14 and 14 at 0, 20, 100 and 500 mg/m3, respectively - Mean litter size (day 21): 8, 8, 8 and 8 at 0, 20, 100 and 500 mg/m3, respectively - Sex ratio: no treatment related effects - Body weight (day 0): 6.3, 6.6, 6.5 and 6.2 g at 0, 20, 100 and 500 mg/m3, respectively - Body weight (day 21): 55, 58, 57 and 56 g at 0, 20, 100 and 500 mg/m3, respectively - External abnormalities: none - Histopathology: no treatment related effects

Any other information on results incl. tables

The mating, fertility and litter data of this study (Monsanto 1988, Table 9, attached) do not indicate that there are statistically significant differences regarding the fertility index (= No. PREGNANT/NO. CO-HOUSED) or the pregnancy index (= No. PREGNANT/NO. WITH CONFIRMED COPULATION). The ratio numbers of pregnant females/mating success calculated by NL do not indicate a relationship to dose. Table 9: Summary of Mating, Fertility and Litter data Generation: F0; CONTROL; LOW; MID; HIGH. Number co-housed; 25; 25; 25; 25. Number with confirmed copulation; 23; 25; 23; 25. Number pregnant; 21; 19; 17; 19. Fertility Index (%); 84; 76; 68; 76. Pregnancy Index (%); 91; 76; 74; 76. Number of Live Pups: Day 0 Day 4 (pre-cull) Day 7 Day 14 Day 21; 13.6 (2.0) 13.5 (2.0) 8.0 (0.0) 7.9 (0.4) 7.9 (0.4); 12.7 (2.2) 12.6 (2.1) 7.9 (0.2) 7.9 (0.2) 7.9 (0.2); 13.9 (4.2) 13.8 (4.2) 7.6 (1.2) 7.6 (1.2) 7.5 (1.2); 13.6 (3.8) 13.3 (3.9) 7.6 (1.8) 7.6 (1.8) 7.6 (1.8). Values expressed as mean (S.D.) Significantly different from control value: *p<0.05, **p<0.01 Fertility Index = No. Pregnant/No. Co-housed Pregnancy Index = No. pregnant/No. with confirmed copulation

Applicant's summary and conclusion

Conclusions

No effects on mating or fertility parameters were observed in Sprague-Dawley rats after exposure to an aerosol of isopropylamine at analytical concentrations up to 499 mg/m^3, resulting in a NOAEL of 499 mg/m^3.

7.8.2 Developmental toxicity / teratogenicity

Endpoint study record: Monsanto (+1988, ACC 2008, OECD , rat, KS, RL1)

| UUID |  | IUC5-2973c076-f895-450c-9ec7-3edc5d3a8234 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | Collette / Epona Associates, LLC / Willington / United States |
| Date |  | 2012-06-13 14:24:36 EDT |
| Remarks |  |  |

Administrative Data

| Purpose flag | key study; robust study summary; used for classification; used for MSDS |  |  |
| --- | --- | --- | --- |
| Study result type | experimental result |  |  |
| Reliability | 1 (reliable without restriction) |  |  |
| Rationale for reliability incl. deficiencies | GLP study with well-documented methodology |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| study report | Kier LD, Thake DC | 1988 | Teratology study of isopropylamine administered by inhalation to rats | unpublished report | Monsanto Agricultural Company - Environmental Health Laboratory/Missouri, USA | EHL 86081 / ML-86-400 | Monsanto Corporation | T10171 | 1988-05-18 |
| study report | Kier LD | 1988 | Teratology study of isopropylamine administered by inhalation to rats | EPA TSCA 8(e) Audit Summary Memo: indicating to OTS 0522377 |  | ML-86-400 | Monsanto Corp. | T04642 | 1988-05-18 |

Data access

data submitter has Letter of Access

Data protection claimed

yes

Materials and methods

Limit test

no

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| equivalent or similar to | OECD Guideline 414 (Prenatal Developmental Toxicity Study) ( note: not mentioned in ACC IUC5) | yes (Food consumption was not measured, while a deviation from standard reproductive toxicity testing, considered an acceptable deviation.) |

GLP compliance

yes

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |
| EC number | 200-860-9 |
| IUPAC name | propan-2-amine |

Details on test material

- Name of test material (as cited in study report): Isopropylamine (2-aminopropane) - Substance type: organic - Physical state: clear liquid - Analytical purity: 99.77 % - Lot/batch No.: LP-606

Test animals

Species

rat

Strain

Sprague-Dawley

Details on test animals and environmental conditions

- Age (males): 10 weeks - Weight at day 0 of gestation (females): 214-282 g - Weight at day 0 of gestation (males): 317-389 g - Number of animals: 25 females/group - Thirty air changes per hour

Administration / exposure

Route of administration

inhalation: vapour

Type of inhalation exposure (if applicable)

whole body

Vehicle

air

Analytical verification of doses or concentrations

yes

Details on analytical verification of doses or concentrations

- Method: Gas analyser - Sampling time: at least 5 times/day (treatment); the control was checked for the presence of test material twice during the study

Details on mating procedure

- Mating: 1 female / 1 male - Day 0 of gestation: presence of vaginal plug

Duration of treatment / exposure

- Gestation days 6-15

Frequency of treatment

- Six hours per day

Duration of test

- Twenty days, Cesarean section on gestation day 20

Doses / concentrations

| 50, 500, 1000 mg/m3 |  |
| --- | --- |
| Basis | nominal conc. |

| 50, 499, and 1000 mg/m3 |  |
| --- | --- |
| Basis | analytical conc. stable and homogeneous (within 4% difference between chamber locations) |

Control animals

yes

Examinations

Maternal examinations

- Mortality: twice daily - Clinical observations: on gestation days 0 and 6-20 (on treatment days after exposure) - Body weight gain: gestation days 0, 6, 10, 13, 16 and 20 - Food consumption: not measured ORGANS EXAMINED AT NECROPSY (MACROSCOPIC AND MICROSCOPIC): - Macroscopy: findings dams recorded - Microscopy: not performed

Ovaries and uterine content

- Examination of uterine content: number of corpora lutea; relative position and number of live and dead foetuses and early and late resorptions

Fetal examinations

- Examination of fetuses: sex; weight; external, visceral (1/2 of each litter) and skeletal (1/2 of each litter) findings

Statistics

- Dunnett's test; Mann-Whitney U test; Fisher's Exact test

Results and discussions

Effect levels

| Endpoint | Effect type | Effect level | Based on | Basis for effect level / Remarks |
| --- | --- | --- | --- | --- |
| NOAEC | teratogenicity | 1000 mg/m³ air |  | NOAEC (teratogenicity): 1000 mg/m3 based on the absence of any significantly increased malformation or variation. |
| NOAEC | maternal toxicity | 500 mg/m³ air |  | based on the effects on bodyweight, treatment-related clinical signs viz. nasal discharge, rales, laboured breathing, sneezing and fur staining/encrustation and macroscopically observed reduced abdominal fat. |

Maternal toxic effects

yes

Details on maternal toxic effects

- Mortality and day of death: no deaths - Body weight(gain): decreased at 1000 mg/m3 on gestation days 10, 13, 16 and 20 and during day 6-20; at 500 mg/m3, lower gains during gestation days 6-10 and overall 6-20 - Number early delivery: 1 female at 500 mg/m3 - Number of resorptions (early/late): at 500 mg/m3: slightly increased (1.0/dam vs 0.6/dam in the control) - Number of implantations: no differences - Pre and post implantation loss: Pre-implantation loss no differences; post-implantation loss, at 500 mg/m3 slightly increased (1.0/dam vs 0.6/dam in the control) - Number of corpora lutea: no differences - Duration of pregnancy: scheduled sacrifice on gestation day 20 - Gross pathology incidence and severity: at 1000 mg/m3: reduced abdominal fat in 9 females; at 50 mg/m3: reduced abdominal fat in 2 females Incidental hydronephosis, kidney stones and dilated ureter were observed without relationship to treatment. - Organ weight/histopathology: no da - Clinical signs: at 1000 mg/m3: rales, laboured breathing and vaginal discharge (one female). An increased incidence of sneezing, fur staining/encrustration and nasal discharge; at 500 mg/m3: an increased incidence of sneezing - Number mated/pregnant at 0, 50, 500 and 1000 mg/m3: 25/22, 25/25, 25/24, 25/24

Embryotoxic / teratogenic effects

no effects

Details on embryotoxic / teratogenic effects

- Sex ratio: no differences - Body weight gain: no differences - External abnormalities: no treatment-related differences - Visceral abnormalities: no treatment-related differences - Skeletal abnormalities: reduced ossification of the 13th rib and bent ribs at the high dose group without clear evidence of a dose-response (not significant)

7.9 Specific investigations

7.9.3 Specific investigations: other studies

Endpoint study record: Gagnaire (+1989/1993, RD50, Respiratory irritation, mouse, RL2)

| UUID |  | IUC5-fd5570d1-cd7d-4cd2-ab5d-4ddc92f3bee9 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:52:01 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| EU: REACH |  |  |  |
| --- | --- | --- | --- |
| Purpose flag | key study; robust study summary |  |  |
| Study result type | experimental result |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | meets generally accepted scientific standards, sufficiently documented and acceptable for assessment |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Gagnaire F, Azim S, Bonnet P, Guenier JP, and De Ceaurriz J | 1989 | Nasal irritation and pulmonary toxicity of aliphatic amines in mice | J Appl Toxicol 9, 301-304 | Institut National de Recherche et de Sécurité |  |  | T04692 | 1988-12-31 |
| publication | Gagnaire F, Azim S, Simon P, Cossec B, Bonnet P, and De Ceaurriz J | 1993 | Sensory and pulmonary irritation of aliphatic amines in mice: a structure-activity relationship study | J Appl Toxicol 13, 129-135 | Institut Nationale de Recherche et de Sécurité |  |  | T01726 | 1992-12-31 |

Data access

data published

Materials and methods

Type of effects studied

other: Respiratory irritation

Type of method

in vivo

Endpoint addressed

respiratory irritation

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| according to | other guideline: American Society for Testing and Materials (ASTM), Standard Test Method for Estimating Sensory Irritancy of Airborne Chemicals, Annual Book of ASTM Standards, Designation E 981-84 |  |

Principles of method if other than guideline

Determination of sensory irritation potency by oronasal exposure, key parameter: RD50 (RD50 = the exposure concentration resulting in a 50-% decrease in the respiratory rate).

GLP compliance

no data

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |
| EC number | 200-860-9 |
| EC name | isopropylamine |

Details on test material

high-purity liquid

Test animals

Species

mouse

Strain

other: OF1

Sex

male

Details on test animals and environmental conditions

TEST ANIMALS - Source: IFFA Credo, Domaine des Onciens, Saint-Germain sur l'Arbresle, France - Age at study initiation: - Weight at study initiation: 20 - 25 g - Fasting period before study: unfasted - Housing: in groups of 10 - Diet., water: ad libitum - Acclimation period: 7 d controlled ENVIRONMENTAL CONDITIONS (no detailed information given)

Administration / exposure

Route of administration

inhalation

Vehicle

unchanged (no vehicle)

Details on exposure

GENERATION OF TEST ATMOSPHERE / CHAMBER DESCRIPTION (see details below) - Exposure apparatus: 200 L inhalation chamber - Method of holding animals in test chamber: Mice were restrained in a body plethysmographs with nose-only exposure. - Source and rate of air: - Method of conditioning air: - System of generating particulates/aerosols: - Temperature, humidity, pressure in air chamber: - Air flow rate: - Air change rate: - Method of particle size determination: - Treatment of exhaust air: TEST ATMOSPHERE - Brief description of analytical method used: HPLC ( sampling by silica gel in glass tubes; after derivatisation resulting m-Toluene derivatives were analysed by reversed-phase chromatography with UV detection) - Samples taken from breathing zone: yes

Analytical verification of doses or concentrations

yes

Duration of treatment / exposure

Exposure period: 60 min

Frequency of treatment

1x, 15 min

Post exposure period

Recovery period: 30 min

Doses / concentrations

| Several concentrations measured from 69 - 197 ppm |  |
| --- | --- |
| Basis | analytical conc. |

No. of animals per sex per dose

6 per test concentration

Control animals

other: internal control = each test animal before exposure

Further details on study design

Observation period: 60 min exposure, 30 min recovery, before, during and after exposure respiratory rate was determined

Examinations

Any other information on materials and methods incl. tables

TEST PRINCIPLE and PROCEDURE Comparative inhalation study including various amines: The sensory irritation potency on the upper airways was evaluated by measuring the reflex expiratory bradypnea in exposed mice: The breathing frequency was used as an index of upper respiratory tract irritation by air-borne exposure to chemicals. Mice were restrained in a body plethysmographs with nose-only exposure. Breathing frequency was monitored before, during and after exposure. The maximum decrease during 15 min was recorded and the RD50calculated (RD50= the exposure concentration resulting in a 50% decrease in the respiratory rate). TEST ATMOSPHERE The exposure concentrations were generated by passing carrier air through liquid amine and by appropriate dilution with additional air. ANALYSIS The exposure concentration was analysed by adsorption of test substance to silica gel followed by desorption under simultaneous derivatisation (m-toluene derivatives). Direct analysis: reverse-phase HPLC. -----------------------

Results and discussions

Any other information on results incl. tables

The RD50value for isopropylamine was 157 ml/m3 (ppm) which corresponds to approx. 0.39 mg/L. Note: Calculation based on MW = 59 and molar volume of 24 L (20 °C): 1 ml/m3= 2.46 mg/m3 ---------------------

Applicant's summary and conclusion

Conclusions

The authors conclude that isopropylamine like related amines mainly produce sensory upper airway irritation. -------------

Endpoint study record: HSDB 2005, RL4

| UUID |  | IUC5-9f90162c-02fc-4657-bb0c-5ba0da66d0e8 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:52:02 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| EU: REACH |  |
| --- | --- |
| Reliability | 4 (not assignable) |
| Rationale for reliability incl. deficiencies | Secondary source (database) |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| secondary source | Privitera, P.J. et al. | 1982 |  | J. Pharmacol. Exp. Ther., 222(1), 116-121 (cited from HSDB 2005) |  |  |  |  |  |
| other: Online database | HSDB (2005) | 2005 | 2-Propylamine [CAS No. 75-31-0] | Hazardous Substances Databank, database of the National Library of Medicine's TOXNET system, 29 June 2005 (http://toxnet.nlm.nih.gov) |  |  |  |  |  |

Data access

data published

Materials and methods

Type of effects studied

biochemical or cellular interactions

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |
| EC number | 200-860-9 |
| EC name | isopropylamine |

Overall remarks, attachments

Overall remarks

Excerpt from HSDB 2005: " In dogs, following iv infusion of 2.5 mg/kg/min the positive chronotropic responses to preganglionic cardioaccelerator nerve stimulation were reduced, whereas the responses to postganglionic nerve stimulation were essentially unchanged. Isopropylamine has both ganglion stimulating and blocking properties and is similar in its action to the classical nicotinic ganglion stimulant drugs. " --------------- " Previous studies ... have demonstrated that the acute sympathomimetic effects of isopropylamine are due to stimulation of autonomic ganglia. The present studies were designed to determine whether isopropylamine produced ganglionic blockade after its initial stimulation of autonomic ganglia. Infusion of isopropylamine (2.5 mg/kg/min) produced an initial increase in arterial pressure and heart rate which was followed by a prolonged hypotension and bradycardia; lower doses produced only a hypotensive response. After infusion of isopropylamine, the positive chronotropic responses to preganglionic cardioaccelerator nerve stimulation were significantly reduced, whereas the responses to postganglionic nerve stimulation were essentially unchanged. Similarly, the negative chronotropic responses to peripheral vagal stimulation were significantly reduced after isopropylamine administration. Moreover, isopropylamine reduced the cardiovascular responses to iv injections of the ganglion nicotinic stimulant, dimethylphenylpiperazinium iodide. Studies on the disposition of isopropylamine indicated that there was significant accumulation of isopropylamine in all tissues as compared to plasma. When the relationship between plasma isopropylamine and the decreases in mean arterial pressure was examined, a significant positive correlation (r=0.87) was found. These results indicate that isopropylamine has both ganglion stimulation and blocking properties and is similar in its action to the classical nicotinic ganglion stimulant drugs such as dimethylphenylpiperazinium iodide and tetramethylammonium." --------------------

7.10 Exposure related observations in humans

7.10.3 Direct observations: clinical cases, poisoning incidents and other

Endpoint study record: Amoore-Hautala (+1983, direct observations, odor threshold, RL2)

| UUID |  | IUC5-80b9b0ef-d159-45a4-8d5d-0f60aed71af9 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:52:02 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| EU: REACH |  |  |  |
| --- | --- | --- | --- |
| Purpose flag | weight of evidence; robust study summary |  |  |
| Study result type | estimated by calculation |  |  |
| Reliability | 2 (reliable with restrictions) |  |  |
| Rationale for reliability incl. deficiencies | Study based on scientific principles, basic data given |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| publication | Amoore JE, Hautala E | 1983 | Odor as an aid to chemical safety: Odor thresholds compared with threshold limit values and volatilities for 214 industrial chemicals in air and water dilution. | Appl. Toxicol. 3, 272-290 |  |  |  | T04893 |  |

Data access

data published

Materials and methods

Study type

other: Expertise on odor thresholds as safeguards for toxic substances

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| no guideline required |  |  |

Principles of method if other than guideline

Literature search and data evaluation based on olfactory and physico-chemical properties, comparision with TLVs if available.

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |
| EC number | 200-860-9 |
| EC name | isopropylamine |

Method

Type of population

occupational

Route of exposure

inhalation

Reason of exposure

unintentional, occupational

Results and discussions

Clinical signs

The mean odour threshold derived as average from two published sources (not specified) is reported to be 1.2 ppm +- 2.8 (standard error, SE) in air and 4.9 ppm in water (one source). (Note: The high SE indicates highly variable values found in literature.)

Endpoint study record: Patty (+1981, direct observations, workers, vapor, inhal, RL4)

| UUID |  | IUC5-f508378d-bd26-4b27-a638-57c2e7092309 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-08 02:52:03 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

Administrative Data

| EU: REACH |  |  |  |
| --- | --- | --- | --- |
| Purpose flag | weight of evidence; robust study summary |  |  |
| Study result type | other: review |  |  |
| Reliability | 4 (not assignable) |  |  |
| Rationale for reliability incl. deficiencies | Only short abstract referring to non-verifiable data |  |  |

Data source

Reference

| Reference type | Author | Year | Title | Bibliographic source | Testing laboratory | Report no. | Owner company | Company study no. | Report date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| review article or handbook | Beard RR, Noe JT | 1981 | Aliphatic and alicyclic amines | in: Patty's Industrial Hygiene and Toxicology, Vol. 2B (Toxicology) (Clayton GD, Clayton FE, eds.), John Wiley & Sons, Chap. 44, 3135-3173; 3rd edition, 1981 |  |  |  | T01289 | 1980-12-31 |
| review article or handbook | Proctor NH, Hughes JP | 1978 | Chemical hazards of the workplace | 2nd. ed., p292, Lippincott, Philadelphia (cited from Beard and Noe 1981, T01289) |  |  |  |  |  |

Data access

data published

Materials and methods

Study type

other: Health report on exposed workers

Endpoint addressed

respiratory irritation

Test guideline

| Qualifier | Guideline | Deviations |
| --- | --- | --- |
| no guideline available |  |  |

Principles of method if other than guideline

no data

GLP compliance

no

Test materials

Identity of test material same as for substance defined in section 1 (if not read-across)

yes

Test material identity

| Identifier | Identity |
| --- | --- |
| CAS number | 75-31-0 |
| EC number | 200-860-9 |
| EC name | isopropylamine |

Method

Type of population

occupational

Route of exposure

inhalation

Reason of exposure

unintentional, occupational

Exposure assessment

no data

Results and discussions

Clinical signs

Humans briefly exposed to 10 to 20 ppm of isopropylamine (= 24.6 - 49.2 mg/m3) experienced irritation of the nose and throat. Workers complained of transient visual disturbances (halos around lights) after exposure to the vapor for 8 h, probably owing to mild corneal edema, which usually cleared within 3 - 4 h. (Report p. 3154).

Legal entity: BASF SE

| UUID |  | IUC4-fa6b5320-73c6-367e-9073-49dcfd348866 |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-09-29 10:26:05 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

General information

| Legal entity name | BASF SE |
| --- | --- |
| Legal entity type | company |

Reference substance: isopropylamine

| UUID |  | ECB5-d650e964-6568-403f-8ccd-d11fea1d1f7c |
| --- | --- | --- |
| Dossier UUID |  | 0 |
| Author |  | XML Transformation V2.0 Plug-In |
| Date |  | 2010-06-16 09:43:03 EDT |
| Remarks |  | Successfully migrated to IUCLID 5.3 format. |

General information

| Reference substance name | isopropylamine |
| --- | --- |

EC inventory

| EC number | 200-860-9 | CAS number | 75-31-0 |
| --- | --- | --- | --- |
| EC name | isopropylamine |  |  |
| Molecular formula | C3H9N |  |  |